| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #51 |
| Source:* | Bob Flynn, Exacta GSS, bob.flynn@exacctagss.com Andreas Kraft, DT, A.Kraft@telekom.de Miguel Angel Reina Ortega, ETSI, MiguelAngel.ReinaOrtega@etsi.org |
|  | 6 September 2021 |
| Reason for Change/s:* | Clarification for <timeSeries> CREATE procedure |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-xxxx MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001, 4.11.0 |
| Clauses * | 10.2.4.29, 10.2.4.23, 9.6.36 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

During review of timeSeries functionality the following issues have been identified

- There is no definition of a missing data point. Provided below.

- What should happen if missing data detect time = 0. This is a bad request based on definitions, specifically this value must be greater than periodicIntervalDelta.

- Stop / restart timer process. Previously, for simplicity, this was not possible, the <timeSeries> resource had to be deleted.  We think this is too restrictive for reasonable use of the <timeSeries> resource and propose procedures below.

### **********************  Start of Change 1   *****************************

#### 10.2.4.29	Procedure for Time Series Data Detecting and Reporting

In the case that the periodicInterval is set and the missingDataDetect is TRUE, the Hosting CSE shall monitor the Time Series Data based on its periodicInterval. When the Hosting CSE detects a missing data point, the dataGenerationTime of the missing data point is inserted into the missingDataList attribute and the missingDataCurrentNr shall be increased by one. When the missingDataCurrentNr exceeds the missingDataMaxNr, the oldest dataGenerationTime shall be removed from missingDataList to enable the insertion of the new missing data point information and missingDataCurrentNr shall be set to the value of missingDataMaxNr.

When an AE wants to be informed of the number of missing data points in a given renewable time duration, the AE should send a create <subscription> request and set the missingData condition tag in the eventNotificationCriteria conditions to specify the reporting policy. This enables the AE to keep track of the number of missing data points and the corresponding timestamps over a predefined but renewable duration (i.e. the "window duration" of the missingData condition).

When the Hosting CSE reports missing data points, it shall check the missingData condition in the subscription resources created for that purpose.

A missing data point is detected when a <timeSeriesInstance> with a dataGenerationTime value matching the expected dataGenerationTime is not present in the Hosting CSE at the missing data detection time.

Expected dataGenerationTime is defined as a range of the dataGenerationTime of the first received <timeSeriesInstance> resource plus (N * periodicInterval) +/- periodicIntervalDelta.

Missing data detection time is defined as expected dataGenerationTime + missingDataDetectTimer.

When the first missing data point is detected (i.e. a detection of the first discontinuous time-stamp) following the creation of a subscription, the Hosting CSE shall start a timer associated with that subscription and start counting the number of missing data points. The timer is set according to the "window duration" in the subscription's missingData  condition. The “window duration” is fixed with respect to the first missing data point. The reporting policy is governed by the rules below:

- If the total number of missing data points becomes equal to the "minimum specified missing number of the Time Series Data" specified in the subscription's missingData condition before the timer expires, a NOTIFY request shall be sent including the "number of missing data points" that have been detected since the start of the subscription's timer. The missing data points counter shall continue counting while the “window duration” timer continues to run (since it did not expire). A similar NOTIFY request shall be sent for each subsequent missing data point detected until the timer expires (see next bullet for behavior when the timer expires). The “window duration” timer does not stop before it expires.

- If the timer expires, the missing data points counter is reset back to 0. The “window duration” timer is restarted upon detection of next missing data point.

- The reset of the timer and the missing data points counter upon timer expiry shall continue until such time as the subscription is cancelled or terminated.

- If no missing data points have been detected at all during the life time of a subscription, then no timer shall be started at all. But once a timer is started triggered by the first missing data point, then the above rules in the previous bullets shall apply.

Figure 10.2.4.29-1 depicts the above rules.

Figure 10.2.4.29-1: Time Series Data Detecting and Reporting Mechanism

T1: 	When the first missing data point is detected, the timer is started and the missing data points counter is set to a value of 1.

T2: 	The NOTIFY Request is sent when the value of the missing data points counter becomes equal to the value in the missingData attribute.

T3: 	The NOTIFY Request is sent again because the missing data points counter is greater than the value in the missingData attribute.

T4: 	At the end of the "window duration" the missing data points counter is reset back to 0.

T5: 	The "window duration" timer is restarted since a missing data point is detected and the missing data points counter is set to a value of 1.

Figure 10.2.4.29-1: Time Series Data Detecting and Reporting Mechanism

The missing data detection process can be paused by updating the missingDataDetect to false. The Hosting CSE will retain the current state of missingDataList and missingDataCurrentNr. The missing data detection process can be restarted by updating the missingDataDetect to true. When the missingDataDetect is set to true the Hosting CSE will clear the missingDataList and missingDataCurrentNr.

While this process is paused the parameters of the <timeSeries> resource can be updated. If any of the parameters (missingDataDetectTimer, missingDataMaxNr, periodicIntervalDelta, periodicInterval) related to the missing data detection process is updated while the data detection process is paused the Hosting CSE will clear the missingDataList and missingDataCurrentNr.

### ********************* End of Change 1 *********************************

### **********************  Start of Change 2   *****************************

#### 10.2.4.23	Update <timeSeries>

This procedure shall be used for updating the attributes in a <timeSeries> resource.

Table 10.2.4.23-1: <timeSeries> UPDATE

| < timeSeries> UPDATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-2 apply with the specific details for: Content: attributes of the <timeSeries> resource as defined in clause 9.6.36 which need be updated. |
| Processing at Originator before sending Request | According to clause 10.1.4. |
| Processing at Receiver | According to clause 10.1.4 with the following additions: If missingDataDetect is modified to false the Hosting CSE will pause the missing data detection process. If missingDataDetect is modified to true the Hosting CSE will clear the missingDataList and missingDataCurrentNr and restart the missing data detection process. If any parameters related to the missing data detection process are updated while the data detection process is paused the Hosting CSE will clear the missingDataList and missingDataCurrentNr. |
| Information in Response message | According to clause 10.1.4. |
| Processing at Originator after receiving Response | According to clause 10.1.4. |
| Exceptions | According to clause 10.1.4 with these following additions. An error will be generated if any of the following attributes are modified while the value of missingDataDetect is true: missingDataDetectTimer, missingDataMaxNr, periodicIntervalDelta, periodicInterval. |

### ********************* End of Change 2 *********************************

### **********************  Start of Change 3   *****************************

### 9.6.36	Resource Type timeSeries

The <timeSeries> resource represents a container for Time Series Data instances. It is used to share information with other entities and potentially to track, detect and report the missing data in Time Series. A <timeSeries> resource has no associated content. It has only attributes and child resources.

Table 9.6.36-1: Child resources of <timeSeries> resource

| Child Resources of <timeSeries> | Child Resource Type | Multiplicity | Description | <timeSeriesAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <timeSeriesInstance> | 0..n | See clause 9.6.37 | <timeSeriesInstance>, <timeSeriesInstanceAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| la | <latest> | 1 | See clause 9.6.27 | None |
| ol | <oldest> | 1 | See clause 9.6.28 | None |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.61 | None |

The <timeSeries> resource shall contain the attributes specified in table 9.6.36-2.

Table 9.6.36-2: Attributes of <timeSeries> resource

| Attributes of <timeSeries> | Multiplicity | RW/ RO/ WO | Description | <timeSeriesAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| holder | 0..1 | RW | See clause 9.6.1.3. | NA |
| location | 0..1 | RW | See clause 9.6.1.3. | OA |
| resourceMappingRules | 0..1 | RW | See clause 9.6.1.3 | OA |
| maxNrOfInstances | 0..1 | RW | Maximum number of direct child <timeSeriesInstance> resources in the <timeSeries> resource. | OA |
| maxByteSize | 0..1 | RW | Maximum size in bytes of data that is allocated for the <timeSeries> resource for all direct child<timeSeriesInstance> resources. | OA |
| maxInstanceAge | 0..1 | RW | Maximum age of a direct child <timeSeriesInstance> resource in the <timeSeries> resource. The value is expressed in seconds. | OA |
| currentNrOfInstances | 1 | RO | Current number of direct child <timeSeriesInstance> resource in the <timeSeries> resource. It is limited by the maxNrOfInstances. The currentNrOfInstances attribute of the <timeSeries> resource shall be updated on successful creation or deletion of direct child < timeSeriesInstance > resource of <timeSeries > resource. | NA |
| currentByteSize | 1 | RO | Current size in bytes of data stored in all direct child <timeSeriesInstance> resources of a <timeSeries> resource. It is limited by the maxByteSize. The currentByteSize attribute of the <timeSeries> resource shall be updated on successful creation or deletion of direct child < timeSeriesInstance > resource of <timeSeries > resource. | NA |
| periodicInterval | 0..1 | RW | If the Time Series Data is periodic, this attribute shall contain the expected amount of time between two instances of Time Series Data. | OA |
| periodicIntervalDelta | 0..1 | RW | If the Time Series Data is periodic, this attribute contains a +/- delta value relative to periodicInterval for the purpose of detecting missing data. The value of this attribute shall be less than or equal to (periodicInterval/2). If the attribute is omitted the hosting CSE can use a local policy to determine a default value. | OA |
| missingDataDetect | 1 | RW | Indicates whether the Receiver shall detect the missing Time Series Data if it is periodic.The default value is false. | NA |
| ontologyRef | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is stored in the child <timeSeriesInstance> resources of the present <timeSeriesData> resource (see note). | OA |
| missingDataMaxNr | 0..1 | RW | Maximum number of entries in the missingDataList if the periodicInterval is set and the missingDataDetect is TRUE. | OA |
| missingDataList | 0..1(L) | RO | The list of the dataGenerationTime value representing the missing Time Series Data in descending order by time if the periodicInterval is set and the missingDataDetect is TRUE. | NA |
| missingDataCurrentNr | 0..1 | RO | Current number of the missing Time Series Data in the missingDataList. | NA |
| missingDataDetectTimer | 0..1 | RW | The missingDataDetectTimer is a duration after which a <timeSeriesInstance> shall be considered missing by the hosting CSE. If periodicIntervalDelta is present, the value of this attribute shall be greater than periodicIntervalDelta. | OA |
| contentInfo | 0..1 | WO | This attribute contains information to understand the contents of the content attribute of <timeSeriesInstance>. It shall be composed of two mandatory components consisting of an Internet Media Type (as defined in the IETF RFC 6838 [i.36]) and an encoding type. In addition, an optional content security component may also be included. The format of this attribute is defined in oneM2M TS-0004 [3]. This attribute should be used to represent the content information of the content attribute of child <timeSeriesInstance> resources so that AEs can understand the content. | OA |
| NOTE: The access to this URI is out of scope of oneM2M. |  |  |  |  |

### ********************* End of Change 3 *********************************