# 6	oneM2M feature descriptions

## 6.1	Semantic routing table for ASD

### 6.1.1	Overview

The SRT is used by a Hosting CSE, that hosts a resource to handle an ASD request, to find CSEs that execute queries in the ASD. Since the ASDs include SPARQL queries for the distributed RDF triples [i.19] over multiple CSEs, SRTs are maintained by the data from RDF triples from pertaining oneM2M resource instances (i.e. <semanticDescriptor> resource). This clause describes how it looks like and how a SRT gets populated among CSEs.

### 6.1.2	Data sources for SRT

As mentioned, the targeted data for the advanced semantic discovery is semantic data (i.e. RDF triples) that are contained in <semanticDescriptor> resources in different CSEs. Therefore the final goal of an ASD from the Originator is to find CSEs that would have ASD matching semantic resources and get results from them, and it can be done by a single CSEs not those distributed CSEs.

By the specification, a <semanticDescriptor> resource can be created as a child resource of different parent resources (e.g. AE, container). This parent-child resource relationship explains that a <semanticDescriptor> resource describes or annotates the parent resource. From the example resource tree in figure 6.1.2-1, if an ASD got a match for the <semanticDescriptor> resource of a container, then the data the Originator is looking for is contained in the container.

It is worthwhile to note that in oneM2M system, each resource including the semantic resource has ownership. This is because not all semantic resources get propagated in SRTs. There would be policies defining which semantic resources gets propagated to whom by which condition or event. Those policies can be given by the owners of the resources. There are several concepts that define the ownership such as holder (owner), creator and resource hierarchy. This topic is elaborated in clause 6.1.4.

Figure 6.1.2-1: A resource tree with semanticDescriptor resources

### 6.1.3	Elements of routing records

#### 6.1.3.1	Mandatory elements

##### 6.1.3.1.1	CSE identifier

Since the major objective to have SRTs is to select CSE(s) to perform the semantic queries in ASDs, there is the CSE-ID along with the RDF resource summary. When there is a match with the summary, the associated CSE-ID is used to send subsequent ASD requests. The To parameter values of subsequent ASDs consist of CSE-ID and ASD request handling resource identifier. Further details are described in clause 6.2.4.

##### 6.1.3.1.2	RDF resource summary

Here the summary can be defined as the concise information, about the triple data, stored in a particular CSE, which determines the possibility of a valid (not-null) response retrieval based on a specific SPARQL query. Different levels of granularity can be involved in the summary, depending on the ASD criteria and fragmentation. The goal of this summary is to identify the candidate CSEs that can return the response for a given SPARQL query. This will provide a non-exhaustive list of CSEs, which lead to faster ASD execution, restricting from redundant query fanouts. However, the summary should be able to accurately represent the actual triple data, stored in the CSEs. There can be different approaches for defining a summary, which can vary based on ASD policies.

One approach of defining summary is to involve the type definitions of the triple data stored. These type definitions can include the definitions from the ontologies, which the particular CSE is utilizing to define the triple data or any other schema involved in representing the triples. This will organize the information based on the types or ontologies which the target CSEs utilize and the hosting CSE will be able perform selection using this information. Using this approach the entries of SRT, in worst possible scenario, can be as big as the size of the ontologies or the schema using by the target CSEs.

Second approach involves including some selected part of the triple data in the SRT. This can involve any information which may be considered as focal point of the triple data to be queried. In this case the hosting CSE will create an anticipation or generalization about the triple data stored in the target CSE and will perform actions accordingly. The advantage of this approach is that the entries stored in the SRT can be made concise by selecting minimal information. However, the process of anticipation or generalization can become a challenge when the data is more diverse across the CSEs, having triples annotated in complex structures and are distributed among many CSEs.

Last approach involves definition of meta information which provides the mapping with the actual triples defined in the CSE. This mapping should be unanimously identified by all the CSEs in the network so that any hosting CSE can interpret the entries in the SRT and perform selection accordingly. The mapping can involve the URIs, keywords or any other identifiers, which can accurately represent a valid semantic resource in the database. This approach provides a lot of flexibility in defining the summary, however the mapping should support all the groups (identified by ontologies), and therefore, the triple data stored in the CSE in the considered network. Hence explicit mapping is required with each group of triple data, which can be a challenge as the number of groups (ontologies) increases. In addition, the mapping needs to be propagated among all the CSEs for unanimous interpretation of summaries in the SRT.

For this use case, the first option is considered to represent a summary in the SRT. This is because the considered environment is distributed, having different ontologies. Also, the size of the SRT is compromised as to support wide range of diversity in the triple data. Table 6.1.3.1.2-1 shows the example summary of the triple data involving three different CSE as defined in the column CSE-ID. In the column 'RDF Resource Summary', each entry includes the list of URIs, representing RDF Class or RDF Property [i.20]. Using this structure, two types of information can be extracted. First one is the URIs of the ontologies, identified as the prefix of the URI, used by CSE to represent its triple data. Second is the types of the RDF resources, currently involved to represent the RDF triple data. In this example total three prefixes are used which are 'http://www.xyz.org/ontology/base-v1', 'http://www.abc.org/ontology/city-v1' and 'http://www.onem2m.org/ontology/Base_Ontology', representing three different ontologies. The postfix (value after '#' in the URI) determines the type or RDF resource. The general convention of identifying the type is such that the postfix with a capitalized initial letter indicates that the URI represents an RDF Class and an RDF Property otherwise. In the example the postfix 'Device', 'Service' and 'Function' represents a RDF Class URI and postfix 'location' and 'manufacturer', represents the RDF Property URI.

Table 6.1.3.1.2-1: Example summary in the SRT

| CSE-ID | RDF Resource Summary |
| --- | --- |
| CSE2 | http://www.xyz.org/ontology/base-v1#Device http://www.xyz.org/ontology/base-v1#Service http://www.xyz.org/ontology/base-v1#location |
| CSE3 | http://www.abc.org/ontology/city-v1#Device http://www.abc.org/ontology/city-v1#Function http://www.abc.org/ontology/city-v1#manufacturer |
| CSE4 | http://www.onem2m.org/ontology/Base_Ontology#Device http://www.xyz.org/ontology/base-v1#location |

#### 6.1.3.2	Optional elements

The cardinality per each type in the summary can be kept in the Semantic Routing Table. When the cardinality information presents, the SRT hosting CSE should take into account the cardinality when it selects CSE(s) for semantic routing. Bigger cardinality means there would be more matching semantic data, so the CSE would prefer the associated CSE over others.

In the similar sense, the number of hops between the SRT hosting CSE and the remote CSE can be referred to select CSEs for ASD fan-out. When a CSE is in closer than others in terms of hops, the ASD response could be returned quicker than others generally.

Different relationship types of Semantic Discovery Agreement can also be stored and used by the CSE. For example, when fanning-out an ASD to the Provider CSE (e.g. Customer-Provider relationship), there could be more information to find better ASD handling CSEs as candidates. The types with the routing examples are illustrated in ETSI TR 103 715 [i.2] and ETSI TR 103 716 [i.3].

Including the optional elements of the SRT, table 6.1.3.1.2-1 in the present document is aligned with the original tables in ETSI TR 103 715 [i.2] and ETSI TR 103 716 [i.3] from the information point of view.

### 6.1.4	SRT update

A SRT needs to be updated so a SRT can find proper target CSEs that execute ASDs. When there is newly available RDF resource summary on a CSE, it can be propagated to other CSEs so those CSEs have up-to-date information to distribute ASDs.

In general, a SRT can be maintained by two methods which is depicted in the figure 6.1.4-1:

Explicit update by a CSE that hosts <semanticDescriptor> resources

Subscription set by a SRT hosting CSE and notification by a <semanticDescriptor> resource hosting CSE

The CSE4 who has something to advertise into the other CSE's SRT can explicitly sends an Update request to the CSE1 so the CSE1 updates its SRT.

Note that the SRT in the diagram is a temporal virtual resource name for ASD routing table maintenance. In oneM2M specifications, virtual resources represent a specific feature without pertaining resource representation. Therefore, it is handy to define a set of procedures without defining a REST resource especially when there could be different implementations. One benefit to exploit a virtual resource concept here is that list update gets easier. When there is a long list of data in a resource attribute, to update such as adding or deleting item(s) from the original list, the Update request need to contain the entire items having new and old items. With the virtual resource, the Originator can include an item to add or delete with the indication whether it is adding an item or deleting it.

Figure 6.1.4-1: SRT update with virtual resource

Alternatively, subscription/notification feature can be used. It is used by the SRT Hosting CSEs, not the semantic data Hosting CSEs. Therefore the two different options can be used in different use cases - who initiates the SRT updates. In figure 6.1.4-1, the CSE1 subscribes an event to keep updating its own SRT regarding CSE3's semantic data by creating a <subscription> resource with corresponding notification event criteria. By the existing subscription/notification mechanism in oneM2M systems, there is a single resource (i.e. subscribed-to resource) for event subscription. In this case, the <SRT> virtual resource is the subscribed-to resource so a <subscription> resource creation request is configured to have the To parameter as the resource identifier such as "CSE1/base/SRT".

Another benefit to have SRT as a virtual resource is that CSE1 knows the request target since virtual resource names are given in clause 7.3 of the present document. On the other hand, if the SRT is represented in a normal resource, the resource name is given by the entity who creates that so there is the discovery first to get access to the SRT. Also in case of normal resource, there would be more than one resource instances.

oneM2M provides different event types for notifications, but to trigger notifications for new semantic data summary there needs a new event type definition to the notificationEventType of a subscription resource. The new event type "Update to semantic data summary". Basically, new summary data can be notified per event. There could be filtering per semantic discovery agreement between (e.g. CSE1 and CSE3).

Whatever the methods, there is no limitation for CSEs involving the propagation. It could be done between Registrar and Registry in zero hop. Also, could happen between remote CSEs (e.g. CSE1 and CSE4 in 3 hops). Note that the term and definition hop is defined in oneM2M TS-0001 [i.5].

Figure 6.1.4-2: SRT update examples

Once a new semantic data summary gets noticed by a CSE, then it can be propagated by an update or notification. Noticing new semantic summary involves a couple of aspects. Firstly, there are more than one <semanticDescriptor> resources. Different resources could have duplicated summary (e.g. RDF Class URI), so each manipulation on a <semanticDescriptor> resource needs to be monitored by the CSE. For the duplicated summary, from the Hosting CSE perspective, it is not required to propagate that data.

Secondly, different AEs create their own <semanticDescriptor> resources and they would have different policies on SRT updates. This means not all RDF triples in <semanticDescriptor> resources get summarized and sends to other CSEs. Based on the AE's policies that are set a priori on a CSE (e.g. new semanticDataPropagationPolicy attribute of a <AE> resource), summarization methods (clause 6.1.3.1.2) could be chosen and specific group of CSEs can get summary updates involving semantic discovery agreement (e.g. P2P CSEs).

## 6.2	ASD Handling

### 6.2.1	Overview

The core element of an ASD request is the embedded SPARQL query, which aims at retrieving the requested triple data from the considered CSEs. The SPARQL query could be executed on the selected CSEs if following considerations are fulfilled:

The SPARQL query should contain proper definitions of prefixes involved.

The SPARQL query should contain at least one Class type (RDF Class) or Property type (RDF Property) as a condition to be matched in the WHERE clause.

The first consideration is focused on retrieving the list of ontologies involved in the query. Based on these prefixes, and routing table, the hosting CSE will be able to select the target CSE, where the query will be sent for execution. The second consideration is related to the case when the ASD request specifies RDF Summaries to be matched in the triple data. As described in clause 6.1.3.1.2, the entries in the routing table will include the Class or Property URI if their corresponding instances are populated in the database. Therefore, the URI of the RDF Class or Property type in the query will be used to select the target CSEs which contain the respective triple data. In addition, it is also an important factor in SPARQL query fragmentation, which is described in clause 6.2.3.

### 6.2.2	ASD request handling with criteria matching

The ASD can be performed with different matching criteria in the SRT, as each criterion has different impact such as the search complexity, query fan-out, response time etc. Three matching criteria can be possible to select the target CSEs. These criteria can be selected by the Originator or by the local policies of CSEs. These criteria can be considered as a selection to have a compromise, mainly between the lookup complexity and the query fan-out.

The first matching criteria is to select the target CSEs based on the Ontology URI. All the prefixes from the query are extracted and searched in the SRT. Then those CSEs are selected which contain all the URIs extracted from the query. In this case, more CSEs will be selected than by using the other matching criteria, as the CSE are selected only based on the ontologies, of which the instances are available. There is an important point to note that the CSE will not be selected if it does not contain any instance of required ontology, even if the CSE contains, can access or can interpret that ontology. This emphasizes the preference of querying the instances as triple data in the ASD than the ontologies, as ontologies are assumed to be accessible on the Web.

The second criteria involve matching the summary of the triple data available in the CSE. In this case, the summary involves the URI of a (RDF) Class type or Property type, of which at least one instance is available in the database. The URIs of all the Class and Property types are extracted from the query and matched with the summary given in SRT. Those CSEs, will be selected of which, the summary matches with the Class and Property types, extracted from the query. Here note that the summary does not ensure that the valid triple data will be returned in response. Rather it filters out the irrelevant CSEs based on the query and selects those which matches with the query parameters. Nonetheless, there can be case when the CSE is selected based on the summary and it returns null value upon the query execution.

The third criteria involve matching summary with semantic inferencing features, which allows a hosting CSE to select the target CSE by performing inferencing techniques on the ontologies involved in the query. This requires that hosting CSE is able to access the ontology and perform inferencing. The target of inferencing is to extract sufficient information to select a target CSE. Note that, these criteria do not require or consider the ability of target CSE to be able to perform inferencing. This process is limited to the selection of target CSEs and the query execution and response are dependent on the target CSEs' ability to interpret the query and perform inferencing.

Figure 6.2.2-1(a) shows the example of ASD query with request of matching ontologies. Here the ontology URI is found in both entries of CSE2 and CSE4 so the ASD is sent to both of the CSEs. Whereas in figure 6.2.2-1(b), the ASD query involves matching summary, in which the matching Class and property types exist only in CSE2. Hence in this case the ASD request is sent to CSE2 only. This emphasizes the above defined considerations for SPARQL query.

Figure 6.2.2-1: ASD handling based on matching ontologies and matching summary

Figure 6.2.2-2 shows the example of ASD query where the hosting CSE performs inferencing on the Class hierarchy of an ontology in order to select the target CSE. The SPARQL query involves a statement in the WHERE clause, specifying a type 'xyz:Device' (which after dereferencing using the prefix given in the query, becomes 'http://www.xyz.org/ontology/base-v1#Device'). Even though it can be seen in the SRT that CSE2 contains URIs, from the same ontology, as specified in the SPARQL query, no such RDF Class or Property URI can be completely matched with 'xyz:Device'. This is the case where inferencing techniques are required, in order to inquire, at a higher granularity level, whether CSE2 is a suitable candidate for executing the given SPARQL query. In this case, using the prefix URI (http://www.xyz.org/ontology/base-v1), the hosting CSE1 accesses the xyz ontology to perform inference in order to interpret the class hierarchies involving Classes 'xyz:Device' and 'xyz:SmartDevice'. After inferencing the ontology, it can be realized that 'xyz:SmartDevice' is a sub-class (rdfs:subClassOf [i.20]) of 'xyz:Device', which can be matched with the first entry of CSE2 of the SRT (as by using the Property rdfs:subClassOf, the class hierarchy in an ontology defines taxonomy of same concept, classifying generic to specific concepts, when the hierarchy is traversed from super Class to sub Class). Based on this interpretation CSE2 can be selected as a target CSE for query execution.

Figure 6.2.2-2: ASD handling based on matching summary with inferencing

### 6.2.3	ASD Fragmentation

In order to control the request fanout, hosting CSE has to determine the most suitable candidate CSEs that contain the triple data matching with the ASD request. Since the data is distributed across CSEs in oneM2M network topology using multiple ontologies for triple data, there will be cases when a single SPARQL query may not return complete desired response from the candidate CSEs. Therefore, SPARQL query fragmentation is required, such that only sub-section of the query is sent to the candidate CSEs, which contain the relevant triple data. However, the hosting CSE has to make sure that even after the fragmentation of the SPARQL query, the logic defined in the original un-fragmented query remains intact, so that the expected response remains the same and can be aggregated.

Under this consideration, there can be different ways through which, a SPARQL query can be fragmented. One of these is fragmenting by using 'UNION' operator. In this case, the set of condition statements, grouped using 'UNION' operator, can be separated and redefined in a separate subsequent SPARQL query. Based on each subsequent query, different candidate CSEs can be selected from the SRT lookup. Table 6.2.3-1 shows the example of query fragmentation based on 'UNION' operator.

Table 6.2.3-1: SPARQL Query Fragmentation based on 'UNION' operator

| Original SPARQL Query | Fragmented Subsequent SPARQL Queries |
| --- | --- |
| PREFIX base : <http://www.onem2m.org/ontology/Base_Ontology> PREFIX xyz : <http://www.xyz.org/ontology/base-v1> PREFIX abc : <http://www.abc.org/ontology/city-v1> SELECT ?device WHERE { { ?device a base:Device. ?device xyz:location city:SophiaAntipolis. } UNION { ?device a xyz:Device. ?device xyz:location xyz:ETSI. } UNION { ?device a abc:Device. ?device abc:manufacturer ?manufacturer ?manufacturer abc:name company:Merci } } | PREFIX base : <http://www.onem2m.org/ontology/Base_Ontology> PREFIX xyz : <http://www.xyz.org/ontology/base-v1> SELECT ?device WHERE { ?device a base:Device. ?device xyz:location city:SophiaAntipolis. } |
|  | PREFIX xyz : <http://www.xyz.org/ontology/base-v1> SELECT ?device WHERE { ?device a xyz:Device. ?device xyz:location xyz:ETSI. } |
|  | PREFIX abc : <http://www.abc.org/ontology/base-v1> SELECT ?device WHERE { ?device a abc:Device. ?device abc:manufacturer ?manufacturer ?manufacturer abc:name company:Merci } |

However, a SPARQL query can be defined in various different structures than the one defined above. Which may or may not ensure a valid fragmentation. Therefore, in addition to the considerations specified in clause 6.2.2, following considerations should be adhered, while defining a fragmentable SPARQL query:

The query should define common response to be returned by the CSEs, so that the result can be aggregated.

The WHERE clause in the query should contain sub-clauses, which are connected together using 'UNION' operator.

Each sub-clause should contain compulsory conditions to be matched in the triple data hosted by a CSE. These sub-clauses can be further fragmented if needed.

It should be ensured that the logic of the query remains intact. That is, the logic of each subsequent query should not be changed if it defragmented back to the original query.

It should be ensured that the response to be returned by each subsequent query should have same final requested query variables, to ensure data aggregation.

The final consideration is demonstrated in table 6.2.3-1, where the original as well as subsequent SPARQL queries have same query variable '?device'. This will ensure consistency in the response so that the returned list of devices is aggregated as if it was executed using the original unfragmented query. The advantage of this fragmentation is that it will simplify the complex queries and will lead to smaller query execution time in a target CSE.

### 6.2.4	Subsequent ASD Initiation

#### 6.2.4.1	SPARQL query and target CSE

The subsequent ASD request can be defined and initiated after the Query fragmentation performed. In this case, a subsequent ASD request will be generated based on each subsequent SPARQL query. Each subsequent ASD request will have its own list of target CSEs where it will be sent.

Figure 6.2.4.1-1 shows the subsequent ASD request initiation based on the example in table 6.2.3-1. Here, the outlined information in red, green, and blue, highlights the matched values as well as the target CSE for respective subsequent queries. Unlike the other subsequent queries, the query highlighted in green has three conditions, which are not completely matched with the corresponding RDF resource summary. In this case, a matching threshold can be considered to select the particular CSE for forwarding. If the number of conditions matched are equal or above the threshold, then the CSE should be considered. The threshold value is dependent on the hosting CSE's policy of target selection and may vary from on CSE to another. Here it can be observed that subsequent query is forwarded to only relevant CSEs. If instead, the original unfragmented query was used, then each CSE would execute the complete SPARQL query, searching for irrelevant triples, and may cause increase in the execution time.

Following the current specification oneM2M TS-0001 [i.5], SPARQL queries will be carried in semanticsFilter condition of the Filter Criteria parameter. A CSE-ID for a subsequent ASD is used to build the To parameter value. If an ASD targets CSE2, for instance, the To parameter value will be "CSE2/CSEBaseName/asd".

In this example, the "CSEBaseName" is the resource instance name of <CSEBase> resource of CSE2. oneM2M provides a universal short name which is fixed in the specification as "-". So the example above can be re-written as "CSE2/-/asd". Also "ASD" represents the virtual resource, which will also be the fixed resource name in the specification, that handles ASD.

Figure 6.2.4.1-1: Subsequent ASD Initiation with matching summary

In a case there can be no match in a SRT for the ASD query. In this case, by default, the ASD handling CSE can fan-out the ASD to the list of CSEs in its SRT. However, if the Originator explicitly includes the ASDMulticast indicator as false, then there will be direct response back to the Originator with empty content.

#### 6.2.4.2	Other parameters

In ASD request handling procedures, other than the Filter Criteria for SPARQL query and the To parameter for subsequent ASDs, there involves other request parameters as well. Those are defined in clause 6.2.5 and those parameter settings from the original ASD will be referred to set the new value in the subsequent ASD.

### 6.2.5	ASD request parameter handling

#### 6.2.5.1	Overview

Apart from the SPARQL query mentioned above, other request parameters can be included in an ASD request. Those are needed to indicate scope of CSEs, time limitation, result type, etc.

#### 6.2.5.2	ASD handling indicator

This new optional parameter indicates the matching criteria of RDF resource summary in the SRT. When the ASD handling CSE selects target CSE(s) from its SRT, the query in the ASD request can be matched in ontology or type level. Also semantic inferencing can be performed to select target CSE(s). Default value is "type".

#### 6.2.5.3	ASD target limit

This new optional parameter indicates the scope of targeted CSEs for advanced semantic discovery fan-out. This would avoid the discovery handling overhead for CSEs. There would be the system default value which is used when it is not provided in ASD requests.

The scope can be defined as the number of CSEs or hops from the first target CSE.

#### 6.2.5.4	Subsequent ASD allowed

This new optional parameter indicates whether there could be subsequent ASD requests to other CSEs by the ASD handling CSE. Firstly, the Originator of ASD can choose whether this is allowed or not. If this is allowed so there can be several ASD fan-outs, the Originator basically says it is okay to wait for some time but expects a better result. Also, when an ASD handling CSE decides there are not enough target CSEs that it has chosen, it can allow the target CSEs to send out subsequent ASD request again. It may take other parameter values for consideration such as timestamps in the following sub-clauses.

#### 6.2.5.5	ASD multicast allowed

This new optional parameter indicates when there is no match in an SRT, the ASD handling CSE fans-out the ASD request to all CSEs in its SRT. Default value is true.

#### 6.2.5.6	Request expiration timestamp

The Request Expiration Timestamp request parameter defines the request validity time. When a request is received later than the timestamp, the request gets rejected. This existing parameter can be used to decide whether to perform the fan-out to other CSEs or execute the query locally for response.

#### 6.2.5.7	Result expiration timestamp

The Result Expiration Timestamp request parameter defines the timestamp for the result validity by the Originator. When a response is received by the Originator after the timestamp, the result in the response gets ignored. This existing parameter can be used to set a timer to wait the fan-out semantic discovery responses. When the advanced semantic discovery request gets fanned-out, the original Result Expiration Timestamp value can be used to set the new Result Expiration Timestamp value, that will be earlier than the original one for aggregation, in the new requests.

#### 6.2.5.8	Response type

The Response Type request parameter defines the communication mode between the Originator and the Hosting CSE. A complex discovery query would require a quiet long time to get the response back. When a non-blocking mode for this parameter is used, the Originator can get the response by its result retrieval (i.e. non-blocking synchronous mode) or by a notification (i.e. non-blocking asynchronous mode). This existing parameter can be used to by the Originator when the discovery is expected to take some time.

#### 6.2.5.9	Event category

The Event Category request parameter defines the request handling priorities. oneM2M TS-0001 [i.5] defines several category values and also other values can be defined for implementation of the specification. This existing parameter can be used to set the priority among advanced semantic discovery requests.

#### 6.2.5.10	Group Request Identifier

The Group Request Identifier request parameter is used to detect and avoid duplicated handling during group fan-outs. This parameter can be also used for the ASD fan-out to detect and avoid unwanted ASD fan-out loops.

### 6.2.6	Semantic Query Execution

Upon receiving the ASD request, the CSE will execute the SPARQL query in its local database. The CSE can execute the SPARQL query, which have stored the triple data and can interpret and execute query based on SPARQL 1.1 protocols. Selection of target CSE is performed using the SRT. Based on the SRT, if the hosting CSE is selected, it can execute the query in its local database as well as send the ASD or subsequent ASD to the other selected CSE using the SRT. The CSE will handle only the ASDs which are targeted towards it.

### 6.2.7	Result aggregation

The result aggregation involves the aggregation of SPARQL result and the corresponding oneM2M resources. The considerations specified in clause 6.2.4, simplifies the process of SPARQL result aggregation as it only involves merging of the results. However, the response can be aggregated either in multiple steps by different CSEs, or can be aggregated directly by the hosting CSE. In the former case aggregation depends upon different factors such as network topologies, SDAs, etc. For this case same example in figure 6.2.4.1-1 can be considered where the aggregation will be performed in two steps. In first step, the results of CSE3 and CSE4 can be aggregated either at the IN-CSE, or it can be aggregated at CSE2 along with the result of CSE2. Also, in this case the topology plays an important role as the CSEs involved in executing subsequent ASD queries lie on the path. In alternate case, the result will be aggregated directly at the hosting CSE.

Figure 6.2.7-1: SPARQL result example of subsequent query

Figure 6.2.7-1 shows the example of the triples matched based on each subsequent query. Although multiple triples are matched based on different query parameters, the final query variable of each query is same, which in this case is '?device', containing the instance URIs of Devices, represented using three different ontologies. The result of each CSE is then returned in terms of this query variable, which are then aggregated at the hosting CSE1. Figure 6.2.7-2 shows the aggregation of these results of the above mentioned subsequent query execution. This is the simplified case involving only one query variable, where the aggregated result is generated by merging each result. However, there can be cases where different query parameters and processes such as grouping, adds to the complexity of aggregation process. Nonetheless, it can be handled by the CSE and the same process can be followed.

Figure 6.2.7-2: SPARQL result aggregation example

## 6.3	Semantic discovery agreement

### 6.3.1	Relationships

Making a semantic discovery agreement between CSEs is to agree on the discovery fan-out scope and routing information access. A semantic discovery relationship in this context serves as defining a relation type for the fan-out scope and routing table access control. For interoperability, a relationship type needs to be specified in the specification, so different implementation can use the same type values for the same handling procedure.

During the semantic discovery routing, the relationships are used to limit the scope of discovery request forwarding or routing. For instance, a semantic discovery request from the CSE1 can be fanned-out to all the CSEs in the same M2M Service Provider domain.

Figure 6.3.1-1: Example of semantic discovery relationship

### 6.3.2	Access Control

For routing record updates and propagations that are described in clause 6.1.4, a CSE needs to access (e.g. RETRIEVE for <subscription> resource creation) to other CSEs' resources. The relationships between CSEs can be used for access decision for the resources having semantic discovery routing information.

Figure 6.3.2-1: Example of semantic discovery access control

## 6.4	Semantic discovery routing recommendation

There has been a candidate solution for Semantic Recommendation System in clause 7.2.7 of ETSI TR 103 715 [i.2]. It assumes a simple query involving a single type information, sensor type in the example, and suggested a P2P network based algorithm. However, the queries possibly supported by an ASD which includes SPARQL queries could be much more complicated than that. There can be different algorithms for many different aspects, so it seems to be more implementation specific so far. This is for further study to find standardizable solution on this topic.