	Doc# SDS-2021-0222-TS-0004-attribute_level_acps_for_responses_R4
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS #51

	Source:*
	Dale Seed, Convida, seed.dale @convidawireless.com

	Date:*
	2021-09-15

	Reason for Change/s:*
	Add attribute level ACP check to responses 

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active - WI-0077 - Attribute Based Access Control Policy 

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 V4.6.0 

	Clauses *
	7.3.3.12

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	N/A

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This CR adds support to filter responses such that they only include attributes allowed by attribute level ACPs.

-----------------------Start of change 1-------------------------------------------
7.3.3.12
Create a success response

The Hosting CSE shall create a success response primitive with a Response Status Code indicating:

· "CREATED" in case of Create operation;

· "OK" in case of Retrieve operation;

· "UPDATED" in case of Update operation;

· "DELETED" in case of Delete operation; and

· "OK" in case of Notify operation.

The Hosting CSE shall include Request Identifier parameter in the response primitive. The Release Version Indicator parameter shall be included unless the primitive is being sent to a Release 1 entity.
If the request includes a M2M Service User parameter, the Hosting CSE shall include a M2M Service User parameter in the response primitive configured with the same value as in the request.

If the Result Content is not given in the Request, the default value for Delete is 0 (nothing), for Create/Retrieve/Update it is 1 (all attributes).
The Hosting CSE shall include the Content parameter in a Retrieve Response.
If the access control rule(s), used by the Hosting CSE to grant the Originator of the request access to the targeted resource, include accessControlAttributes parameter(s), then the Hosting CSE shall include only those attributes in the Content parameter of the response which are present in the accessControlAttributes parameter(s) of these rules as defined in clause 7.1.5 in oneM2M TS-0003 [7].
For Create/Update operation, if Result Content is modified-attributes then the Content parameter shall only be present if attributes which were not in the request Content  parameter were added, modified or deleted or if any attributes were set to values different from the values specified in the request Content parameter. The deleted attributes to be returned shall be set to NULL values in the response Content parameter.
The Hosting CSE shall not include the Content parameter in a Create/Update/Delete Response if Result Content is set to 0 (nothing).
The information of the Content parameter shall comply to the value of the Result Content request parameter of the corresponding Request.
More details can be found in clause 7.2.1.2 (Response primitive format).
The response content serialization shall use the type indicated in the received request. If a content serialization cannot be determined from the request then the serialization of the response shall use one of the serialization types specified in the contentSerialization attribute of the originator.
The Hosting CSE may include To, From, Originating Timestamp, Result Expiration Timestamp, Event Category parameters.
If the received request includes a Primitive Profile Identifier parameter the Hosting CSE shall apply the referenced <primitiveProfile> resource to the response, provided that all the following conditions are true:

· The referenced <primitiveProfile> resource is present and is retrievable by the request originator;
· The primitive profile’s IDList, resourceTypes, operations, resourceIDs and releaseVersions attributes match the received request;
· The applicability attribute includes the value “RESPONSES_FROM_CSE”.
To apply the <primitiveProfile> resource to the response, the Hosting CSE shall add, replace or delete any applicable response parameters or attributes defined in the additions or deletions attributes of the referenced <primitiveProfile> resource. This may result in the Hosting CSE deleting mandatory and/or optional parameters and attributes that the Hosting CSE would otherwise normally include in a response sent to the request Originator. When applying the profile to the Content parameter of the response, the Hosting CSE shall only apply the profile to the representation of the resource returned in the content of the response (if any). If the Content parameter of the response includes nested representations of child resource(s) and their descendant(s), the Hosting CSE shall not apply the profile to these child resource(s) or descendant(s).

If it is constructing an aggregated response, the Hosting CSE shall apply the primitive profile processing rules defined above for a non-aggregated response to each individual response primitive within the aggregated response. For each individual response primitive within the aggregated response, the Hosting CSE shall evaluate the response, its corresponding request and any applicable primitive profile to determine whether to apply a profile to that response. The Hosting CSE shall perform these steps for each individual response primitive independently of the other responses within the aggregated response. In addition, the Hosting CSE shall also apply the primitive profile processing rules defined above for a non-aggregated response to the response parameters of the aggregated response. The Hosting CSE shall evaluate the aggregated response, its corresponding request and any applicable primitive profile to determine whether to apply a profile to the response parameters of the aggregated response.

-----------------------End of change 1-------------------------------------------

© 2021 oneM2M Partners
                                                                                                   Page 30 (of 31)



[image: image1.png]