	Doc# SDS-2023-0025R01-OGC-to-oneM2M_Configuration_IPE_towards_STA
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS 58

	Source:*
	Andreas Neubacher, Deutsche Telekom, andreas.neubacher@magenta.at
Ingo Friese, Deutsche Telekom, Ingo.Friese@telekom.de

	Date:*
	2023-02-15

	Reason for Change/s:*
	Additional input, elaborating on configuration of the OGC/STA Server (direction oneM2M IPE towards OGC/STA Server)

	CR  against:  Release*
	Rel.5

	CR  against:  WI*
	 FORMCHECKBOX 
 Active WI-0100  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TR-0065 V0.1.0

	Clauses *
	New content 6.3.2.2

	Type of change: *
	 FORMCHECKBOX 
 Editorial change

 FORMCHECKBOX 
 Bug Fix or Correction

 Change to existing feature or functionality

 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Other TS/TR(s) impacted
	non

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES 
  NO 
This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2020 (do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar.
Use change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
R01:

· Provides CR introduction
· Clarifies the usage of a reference ID for a STA Datastream
Additional input, elaborating on configuration of the OGC/STA Server (direction oneM2M IPE towards OGC/STA Server) when operating OGC/SensorThings API IPE


-----------------------Start of change 1-------------------------------------------
6.3.2.2 Communication direction IPE towards OGC / STA Server

As described in the “Generic approach” detailed in Clause 6.2.2 the IPE receives a <contentInstance> included in a <Notification> message from the oneM2M CSE. The IPE copies the ‘content’ attribute of every incoming <contentInstance> to the ‘result’ attribute of a new formed “Observation”.
In an exemplary setup (Figure 6.3.2.2.-1) the “Observation” is posted from the IPE to the OGC / STA Server using HTTP. But before an “Observation” can be sent to the OGC/STA Server, the IPE needs to know the destination “DataStream”. According to the OGC data model [i.7] a “DataStream” needs at least a “Thing” entity that it belongs to (see Figure 5.1-1).

Configuration steps: The IPE requires a destination-“DataStream” in order to send an “Observation”. In case  there is no associated “DataStream” on the OGC / STA Server yet, because the “Thing” doesn’t yet exist, the  IPE needs to create a new “Thing”and an associated “DataStream” on the OGC / STA Server. When a “DataStream” is created in the OGC / STA Server the IPE gets back an Id as a reference (e.g. {“@iot.id:3635353”}).
This reference is later needed to send an “Observation” to the relateted “DataStream”. That’s why the reference has to be stored when an entity of OGC data model has been created. The IPE might also create additional optional entities of the OGC data model like e.g. “Location” or “Sensor” when needed. 
The creation of entities like “DataStream” and ”Thing” requires a number of mandatory properties that have to be known at configuration time, eg. ‘name’ and ‘description’. These property fields might be defined “by hand” or they could be automatically derived e.g. from the “Label” or “ResourceName” property of the regarded oneM2M <AE> or <container> during IPE configuration. The OGC / STA procedures for creating OGC entities are described in SensorThing API documentation [i.8].
When “Thing” and “DataStream” entities are created the IPE is able to send “Observations” to the OGC / STA Server as HTTP POST messages. The interested STA Client can now subscribe to the destination “Datastream” on the MQTT Broker of the OGC/STA Server and thus getting every “Observation” forwarded from the IPE.
[image: image1.png]
Figure 6.3.2.2.-1: Message flow from IPE to OGC / STA Server to OGC Client

-----------------------End of change 1---------------------------------------------

-----------------------Start of Changes to References Clause -------------

2.2
Informative references
[i.8]
API documentation for the Open Geospatial Consortium (OGC) SensorThings international standard
https://developers.sensorup.com/docs/#introduction
-----------------------End of Changes to References  -------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
OGC /�STA Server





IPE





    





OGC / STA�Client





MQTT�Broker





“Observation“�as�HTTP-Post





“Observation“�published�over MQTT








© 2020 oneM2M Partners
                                                                                                   Page 2 (of 4)



