	SDS-2023-0079-TS-0008_Introduce_no-response_option
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS 59.1

	Source:*
	Abhijan Bhattacharyya , TCS, abhijan.bhattacharyya@tcs.com 

	Date:*
	2023-04-28

	Reason for Change/s:*
	See the introduction

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0008 – V4.4.0

	Clauses *
	5.1, 5.2.0, 5.2.4

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Other TS/TR(s) impacted
	None

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2020 (do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar.
Use change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This CR introduces how the ‘no-response’ option of CoAP can be used in oneM2M systems to avoid transmission of unnecessary responses from the request/response layer of the serving CoAP endpoint.

In change 2, section 5.2.4 is proposed which defines how oneM2M systems are going to react to the "no-response" option as specified by RFC 7967 since this modifies the usual CoAP semantics, just as Block-wise transfer does.
-----------------------Start of change 1-------------------------------------------
5.0
Introduction

The clause describes which features need to be supported in CoAP layer and introduces a message format and several features of CoAP used in this protocol binding specification.
5.1
Required Features
This clause explicitly specifies the required features of the CoAP layer for oneM2M to properly bind oneM2M primitives into CoAP messages:
· The 4-byte binary CoAP message header is defined in section 3 of IETF RFC 7252 [1].

· Confirmable (CON), Non-confirmable (NON), Acknowledgement (ACK) and Reset (RST) messages shall be supported. The Reset message is used to send an error message in response to a malformed Confirmable message in CoAP layer.

· GET, PUT, POST and DELETE methods shall be supported. oneM2M primitives map to these methods.

· The CoAP Response Codes specified in clause 6.2.4 shall be supported for oneM2M Response Status Code parameter mapping.

· The Uri-Host, Uri-Port, Uri-Path, and Uri-Query shall be supported.

· The Content-Type Option shall be used to indicate the media types of the payload.

· Block-wise transfers feature may be supported to carry large payloads.

· The Caching feature may be supported.
· The ‘no-response’ option may be used along with PUT, POST, DELETE request methods to avoid transmission of unnecessary responses from the request/response layer of the serving CoAP endpoint.    
5.2
Introduction to CoAP

5.2.0
Introduction

This clause describes a message format, caching and block-wise transfers and no-response features which may be used to map oneM2M primitives to CoAP messages.
5.2.1
Message Format
This clause specifies details about the CoAP [1] message format:

· CoAP message occupies the data section of one UDP datagram.

· CoAP message format supports a 4-byte fixed-size header.

· Fixed-size header is followed by a Token value of length 0 to 8 bytes.

· The Token value is followed by a sequence of zero or more CoAP Options in TLV format.

· CoAP Options are followed by the payload part.

For more details on the CoAP message format and the supported header fields, refer to IETF RFC 7252 [1].
-----------------------End of change 1---------------------------------------------

-----------------------Start of change 2-------------------------------------------

5.2.4
No-response option

CoAP no-response option [6] may be used when CoAP client endpoints are not interested in the execution status of the request to a server endpoint. In such cases CoAP client endpoints may use this option in such requests. This is particularly useful in group communication and many resource update or resource creation scenarios as described in detail in [6]. If required, a client endpoint may use the feature for ‘granular control on response suppression’, as specified in Section 2.1 of [6], to indicate the particular response type which it intends to be suppressed by the server. All the special considerations regarding flow control, token reuse, etc. as specified in Section 3 of [6] must be followed. This option is applicable for both ‘piggybacked’ and ‘separate’ responses. Refer to IETF RFC 7967 [6] for further details.
-----------------------End of change 2---------------------------------------------

-----------------------Start of Changes to References Clause -------------

2.1
Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non‑specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]
IETF RFC 7252: "The Constrained Application Protocol (CoAP)".
[2]
oneM2M TS-0004: " Service Layer Core Protocol Specification".
[3]
IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP)".
[4]
oneM2M TS-0003: "Security solutions".
[5]
IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".
[6]
IETF RFC 7967: “Constrained Application Protocol (CoAP) Option for No Server Response”.
2.2
Informative references
 [i.1]
oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf) 
-----------------------End of Changes to References  -------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 2 (of 4)



[image: image1.png]