| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #59 |
| Source:* | Andreas Kraft, DT, Andreas.Kraft@t-systems.com Andreas Neubacher, DT, Andreas.Neubacher@magenta.at Bob Flynn, Exacta GSS, bob.flynn@exactagss.com |
| Date:* | 2023-05-17 |
| Reason for Change/s:* | New event evaluation mode for crossResourceSubscription – TS-0004 |
| CR against: Release* | Release 5 |
| CR against: WI* | Active WI-xxxx MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 v.4.14.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

The <crossResourceSubscription> is used to listen to events from one or more <subscription> resources in a specified time window (sliding or fixed). If all the specified <subscription> resources emit events within such a time window, the <crossResourceSubscription> resource itself sends a notification to one or more receivers. This can be used to send a notification only if certain events happen in a given time window.

What is currently missing is the opposite: React and send a notification in case all or some expected events from the monitored <subscription> resource have NOT been received within a time window. Use case for this could be a dead-man-switch or the monitoring of device health / heartbeat.

For this the following three use cases are used.

- Send a notification when ALL notifications have been received within a time window. This is the current behaviour.

- Send a notification when NO notification has been received in a time window. This can only be valid for a periodic window because the correct start of a sliding window cannot be determined, or it is not even started. This is a new proposed behaviour.

- Send a notification when ONLY SOME notifications have been received in a time window. This also can only be valid for a periodic window because a sliding window is only started when a notification is received. This is a new proposed behaviour.

The following table shows the proposed set of options for handling the different event sets.

|  | Number of events received in a periodic time window | TS-0001 and TS-0004 impacts |
| --- | --- | --- |
| ALL_EVENTS_PRESENT | N events received | Slide, periodic |
| ALL_OR_SOME_EVENTS_PRESENT | 1..N events received | Slide, periodic |
| ALL_OR_SOME_EVENTS_MISSING | 0..N-1 events received | Periodic |
| ALL_EVENTS_MISSING | 0 events received | Periodic |
| SOME_EVENTS_MISSING | 1..N-1 events received | Slide, periodic |

This CR proposes the introduction of a new optional attribute that enables the functionality in the <crossResourceSubscription> resource type. Another CR SDS-2023-0086 proposes the necessary updates of the procedures in TS-0001.

- Change 1: Added enum m2m:timeWindowInterpretation

- Change 2: Added timeWindowInterpretation attribute to <crossResourceSubscription> definition and procedures

- Change 3: Updated notification procedure in clause 7.5.1.2.18

- Change 4: Added short name for timeWindowInterpretation to clause 8.2.3

R01

- Change 3: Changes to text and procedure order according to discussions during SDS#59.2

### **********************  Start of Change 1   *****************************

##### 6.3.4.2.92	m2m:timeWindowInterpretation

Used for the timeWindowInterpretation attribute of the <crossResourceSubscription> resource.

Table 6.3.4.2.92-1: Interpretation of m2m:eventEvaluationMode

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | ALL_EVENTS_PRESENT |  |
| 2 | ALL_OR_SOME_EVENTS_PRESENT |  |
| 3 | ALL_OR_SOME_EVENTS_MISSING |  |
| 4 | ALL_EVENTS_MISSING |  |
| 5 | SOME_EVENTS_MISSING |  |

### ********************* End of Change 1 *********************************

### **********************  Start of Change 2   *****************************

### 7.4.58	Resource Type <crossResourceSubscription>

#### 7.4.58.1	Introduction

The <crossResourceSubscription> resource is used to support cross-resource subscription and cross-resource notification. A cross-resource subscription is made on multiple target resources. A cross-resource notification shall be generated if and only if expected events on all target resources occur within a designated time window. When notifications from all target resources occur within a specified time window the Hosting CSE shall issue a cross-resource notification.

The detailed description can be found in clause 9.6.58 in oneM2M TS-0001 [6].

Table 7.4.58.1-1: Data type definition of <crossResourceSubscription> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| crossResourceSubscription | CDT-crossResourceSubscription.xsd |  |

Table 7.4.58.1-2: Universal/Common Attributes of <crossResourceSubscription> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| custodian | O | O |
| creator | O | NP |

Table 7.4.58.1-3: Resource Specific Attributes of <crossResourceSubscription> resource

| Attribute Name |  | Request Optionality |  |  |  | Data Type |  | Default Value and Constraints |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Create |  | Update |  |  |  |  |  |
| regularResourcesAsTarget |  | O |  | NP |  | m2m:listOfURIs |  | No default. At least one of these must be present |  |
| subscriptionResourcesAsTarget |  | O |  | NP |  | m2m:listOfURIs |  |  |  |
| regularResourcesAsTargetSubscriptions |  | NP |  | NP |  | m2m:listOfURIs |  | No default |  |
| timeWindowType |  | M |  | O |  | m2m:timeWindowType |  | No default |  |
| timeWindowSize |  | M |  | O |  | xs:duration |  | No default |  |
| eventEvaluationMode |  | O |  | O |  | m2m:eventEvaluationMode |  | Default value is ALL_EVENTS_PRESENT |  |
| eventNotificationCriteriaSet |  | O |  | NP |  | m2m:eventNotificationCriteriaSet |  | Default behaviour is notification on Update_of_Resource |  |
| notificationEventCat |  | O |  | NP |  | m2m:eventCat |  | No default |  |
| expirationCounter |  | O |  | O |  | xs:positiveInteger |  | No default |  |
| notificationURI |  | M |  | O |  | list of xs:anyURI |  | No default |  |
| subscriberURI |  | O |  | NP |  | xs:anyURI |  | No default |  |
| notificationStatsEnable |  | O |  | O |  | xs:boolean |  | false |  |
| notificationStatsInfo |  | NP |  | NP |  | m2m:setOfNotificationStatsInfo |  | No default |  |

Table 7.4.58.1-4: Child Resources of <crossResourceSubscription> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <notificationTargetMgmtPolicyRef> | [variable] | 0..n | Clause 7.4.30 |
| <notificationTargetSelfReference> | ntsr | 1 | Clause 7.4.33 |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <schedule> | notificationSchedule | 0..1 | Clause 7.4.9 |

#### 7.4.58.2	<crossResourceSubscription> resource specific procedures for CRUD operations

##### 7.4.58.2.0	Introduction

This clause describes <crossResourceSubscription> resource specific primitive behaviour for CRUD operations.

##### 7.4.58.2.1	Create

Originator:

The following are changes to the Originator procedures described in clause 7.2.2.1:

- Orig-1.0 When composing a request primitive, the Originator shall include regularResourcesAsTarget and/or subscriptionResourcesAsTarget attributes in the resource representation of the <crossResourceSubscription> in the content of the primitive. If regularResourcesAsTarget attribute is included, eventNotificationCriteriaSet attribute shall be included. If eventNotificationCriteriaSet contains only one eventNotificationCriteria, this eventNotificationCriteria shall be applied to all regular resources included in regularResourcesAsTarget attribute; otherwise, eventNotificationCriteriaSet shall contain the same number of eventNotificationCriteria elements as the number of regular target resources contained in regularResourcesAsTarget and each eventNotificationCriteria element shall be sequentially applied to corresponding target resource as listed in the regularResourcesAsTarget.

Receiver:

The following are changes to the receiver procedures described in clause 7.2.2.2:

Recv-6.5: The following steps are in addition to the generic Create procedures defined in clause 7.3.3.5:

a)	The request shall be rejected with a "BAD_REQUEST" Response Status Code if at least one of regularResourcesAsTarget or subscriptionResourcesAsTarget attributes is not present in the request.

b) 	If subscriptionResourcesAsTarget is included, the Hosting CSE shall retrieve and update each <subscription> resource indicated in subscriptionResourcesAsTarget by issuing an UPDATE request to the <subscription> resource host as follows:

i)	In the UPDATE request, the receiver shall use the From parameter from the current CREATE request.

ii)	The associatedCrossResourceSub attribute shall be updated by adding the resource identifier of the <crossResourceSubscription> resource being created.

iii)	The notificationURI attribute shall be updated by adding the resource identifier of the <crossResourceSubscription> resource being created. The Hosting CSE shall properly reply to a potential subscription verification request.

iv)	If any <subscription> for a target resource cannot be successfully updated or if the <crossResourceSubscription> CREATE Request timeout is exceeded, the receiver shall send an unsuccessful response with a "CROSS_RESOURCE_OPERATION_FAILURE" Response Status Code to the Originator; the Hosting CSE shall also remove itself from any already successfully associated <subscription> resources using the procedures in clause 7.4.8.2.4 and also delete any already-created <subscription> resources at other target resources.

  - If regularResourcesAsTarget is included, the Hosting CSE shall send a CREATE <subscription> request message to each target resource indicated by regularResourcesAsTarget.

        - The request shall be rejected with a “BAD_REQUEST” Response Status Code if the eventNoficationCriteriaSet attribute is not present in the request. If present, the eventNotificationCriteriaSet attribute shall contain either one eventNotificationCriteria or the same number as the number of regular target resources contained in regularResourcesAsTarget attribute. Otherwise, the request shall be rejected with “BAD_REQUEST”.

        - In the new CREATE <subscription> request, the receiver shall use the From of the current CREATE request. For this new <subscription>:

1)	The eventNotificationCriteria attribute shall use the corresponding entry from the eventNotificationCriteriaSet attribute of the <crossResourceSubscription> that is being created. If the eventNotificationCriteriaSet attribute contains only one entry, the eventNotificationCriteria attribute shall use that entry.

2)	The notificationURI attribute shall be set to the resource identifier of the <crossResourceSubscription> that is being created. The Hosting CSE shall properly reply to a potential subscription verification request.

3)	The associatedCrossResourceSub attribute shall be set to the resource identifier of the <crossResourceSubscription> that is being created.

4)	The notificationEventCat attribute shall be set to the value of the notificationEventCat attribute of the <crossResourceSubscription> resource, if present. Otherwise, it shall not be set.

5)	The expirationTime attribute shall be set to value of the expirationTime attribute of the <crossResourceSubscription> resource. This is the value that was included in the <crossResourceSubscription> CREATE request or, if a value was not included in that request, it is the value provided by the Hosting CSE for the <crossResourceSubscription> resource.

        - If any <subscription> for a target resource cannot be successfully created or if the <crossResourceSubscription> CREATE Request timeout is exceeded, the receiver shall send an unsuccessful response with a "CROSS_RESOURCE_OPERATION_FAILURE" Response Status Code to the Originator; the receiver shall also delete already-created <subscription> resources at other target resources that were created based on the presence of regularResourcesAsTarget.

        - Upon successful creation of each <subscription> resource, the resource identifier of the recently created <subscription> shall be added to the regularResourcesAsTargetSubscriptions attribute at the position of the corresponding entry in regularResourcesAsTarget.

d)	Once the <crossResourceSubscription> resource is created, the Hosting CSE shall start the time window if the timeWindowType=PERIODICWINDOW; if timeWindowType=SLIDINGWINDOW, the Hosting CSE shall start the time window after the first notification is received from a Target Resource Hosting CSE.

- If the notificationStatsEnable attribute is set to true, the Hosting CSE shall start recording notification statistics in the notificationStatsInfo attribute once the <crossResourceSubscription> resource is created.

- The request shall be rejected with a “BAD_REQUEST” Response Status Code if the eventEvaluationMode attribute is set to “ALL_OR_SOME_EVENTS_MISSING” or “ALL_EVENTS_MISSING”, and timeWindowType is set to “SLIDINGWINDOW”.

##### 7.4.58.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.58.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

The following are changes to the receiver procedures described in clause 7.2.2.2.

- Recv-6.5: The following steps are in addition to the generic Update procedures defined in clause 7.3.3.7:

- If the timeWindowSize attribute is updated and/or the timeWindowType attribute is updated, the receiver shall restart the time window as described in clause 7.4.58.2.1.

- If the notificationStatsEnable attribute in the request is false and the notificationStatsEnable attribute in the resource is true, the Hosting CSE shall stop collecting notification statistics for the <crossResourceSubscription> resource. The Hosting CSE shall maintain the current value of the notificationStatsInfo attribute.

- If the notificationStatsEnable attribute in the request is true and the notificationStatsEnable attribute in the resource is either true or false, the Hosting CSE shall set the value of the notificationStatsEnable attribute in the resource to true, delete any values stored in the notificationStatsInfo attribute of the resource and then start recording notification statistics.

2) The request shall be rejected with a “BAD_REQUEST” Response Status Code if the eventEvaluationMode attribute would be either “ALL_OR_SOME_EVENTS_MISSING” or “ALL_EVENTS_MISSING”, and timeWindowType would be “SLIDINGWINDOW” after the UPDATE request.

##### 7.4.58.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

The following are changes to the receiver procedures described in clause 7.2.2.2:

Recv-6.5: The following steps are in addition to the generic Delete procedures defined in clause 7.3.3.8:

  - The Hosting CSE shall stop the time window timer if running.

  - If this procedure was caused by a subscription deletion notification (see clause 7.5.1.2.4), then the following steps shall not be performed on the <subscription> indicated in the subscription deletion notification.

  - The Hosting CSE shall delete the previously-created <subscription> child resource of each target resource indicated in the regularResourcesAsTarget attribute. Target subscriptions are indicated in regularResourcesAsTargetSubscriptions attribute of the <crossResourceSubscription>. If this procedure is triggered by a DELETE operation, then the Receiver shall use the From of the current request for these requests. Otherwise, the Receiver shall use the creator of this <crossResourceSubscription> as its From parameter.

  - The Hosting CSE shall retrieve and update each <subscription> resource indicated in the subscriptionResourcesAsTarget attribute using the procedure in clause 7.4.8.2.3 to remove the resource identifier of this <crossResourceSubscription> from the <subscription> resource’s associatedCrossResourceSub and notificationURI attributes. If this procedure is triggered by a DELETE operation, then the Receiver shall use the From of the current request for these requests. Otherwise, the Receiver shall use the creator of this <crossResourceSubscription> as its From parameter.

  - Any failures in steps c) or d) shall be ignored.

  - The Hosting CSE shall send a Notify request for Cross Resource Subscription Deletion using the procedures in clause 7.5.1.2.20.

### ********************* End of Change 2 *********************************

### **********************  Start of Change 3   *****************************

##### 7.5.1.2.18	Cross-Resource Notification

When the <crossResourceSubscription> Hosting CSE receives a notification from the Host of a <subscription> indicated in regularResourcesAsTarget or subscriptionResourcesAsTarget the <crossResourceSubscription> Hosting CSE shall perform the following steps:

- The Hosting CSE shall send a notification response to the <subscription> resource Hosting CSE.

- Aggregate notifications using the time window mechanism indicated by the timeWindowType attribute of the <crossResourceSubscription> resource to determine if a cross-resource notification shall be issued:

a)	The Hosting CSE shall store the received notifications until the current time window expires.

7)	When the current time window expires, the Hosting CSE shall perform the following steps:

a)Start a new time window:

i)	If timeWindowType is PERIODICWINDOW then a new time window shall be started when the current time window expires.

ii)	If timeWindowType is SLIDINGWINDOW then a new time window shall be started when the next notification is received.

- The hosting CSE shall check the following conditions to determine if a notification shall be generated:

  - when the eventEvaluationMode attribute is missing or set to “ALL_EVENTS_PRESENT” and notifications from all target <subscription> resources occurred within the required time window, or

  - when the eventEvaluationMode attribute is set to “ALL_OR_SOME_EVENTS_PRESENT” and notifications from at least one <subscription> resource occurred within the required time window, or

  - when the eventEvaluationMode attribute is set to “ALL_OR_SOME_EVENTS_MISSING” and notifications from none or many, but not all, <subscription> resources occurred within the required time window, or

  - when the eventEvaluationMode attribute is set to “ALL_EVENTS_MISSING” and no notification from the target <subscription> resources occurred within the required time window, or

  - when the eventEvaluationMode attribute is set to “SOME_EVENTS_MISSING” and notifications from at least one, but not all, <subscription> resources occurred within the required time window,

If any of the above conditions is true, then the hosting CSE shall compose a cross-resource notification in a notification data object with type m2m:notification:

- Set the subscriptionReference element to the URI of the <crossResourceSubscription> resource.

- If the expirationCounter attribute is set, then it shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter equals to zero ('0'), the corresponding <crossResourceSubscription> resource shall be deleted as described in 7.4.58.2.4.

- Send the notification to the notificationURI using the procedure defined in clause 7.5.1.2.2.

- "Wait for Response primitive" procedure.

- Discard stored notifications

The Subscriber or Notification Targets which receive cross-resource notifications from the Hosting CSE shall perform the following steps in order:

- "Create a success response" procedure defined in clause 7.3.3.12.

- "Send the Response primitive" procedure.

### ********************* End of Change 3 *********************************

### **********************  Start of Change 4   *****************************

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

Table 8.2.3-1: Resource attribute short names (1/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| accessControlPolicyIDs | All except accessControlPolicy, contentInstance | acpi |
| announcedAttribute | All announceable resources | aa |
| announceTo | All announceable resources | at |
| announceSyncType | All announceable resources | ast |
| creationTime | All | ct |
| expirationTime | All except contentInstance, CSEBase | et |
| labels | All (optional) | lbl |
| lastModifiedTime | All | lt |
| link | All | lnk |
| parentID | All | pi |
| resourceID | All | ri |
| resourceType | All | ty* |
| stateTag | container, contentInstance, delivery, request, flexContainer | st |
| resourceName | All | rn |
| privileges | accessControlPolicy | pv |
| selfPrivileges | accessControlPolicy | pvs |
| authorizationDecisionResourceIDs | accessControlPolicy | adri |
| authorizationPolicyResourceIDs | accessControlPolicy | apri |
| authorizationInformationResourceIDs | accessControlPolicy | airi |
| App-ID | AE | api |
| AE-ID | AE | aei |
| AE-IDList | AEContactListPerCSE | ail |
| appName | AE | apn |
| pointOfAccess | AE, CSEBase, remoteCSE | poa |
| ontologyRef | AE, container, contentInstance, semanticDescriptor. flexContainer, timeSeries, reasoningRules | or |
| nodeLink | AE, CSEBase, remoteCSE, flexContainer | nl |
| contentSerialization | AE | csz |
| registrationStatus | AE | regs |
| trackRegistrationPoints | AE | trps |
| sessionCapabilities | AE | scp |
| activityPatternElements | AE, remoteCSE | ape |
| triggerEnable | AE, remoteCSE | tren |
| enableTimeCompensation | AE, remoteCSE | etc |
| creator | container, contentInstance, eventConfig, group, pollingChannel, statsCollect, statsConfig, subscription, semanticDescriptor, notificationTargetPolicy, flexContainer, timeSeries, crossResourceSubscription, backgroundDataTransfer | cr |
| maxNrOfInstances | container, timeSeries, flexContainer | mni |
| maxByteSize | container, timeSeries, flexContainer | mbs |
| maxInstanceAge | container, timeSeries, flexContainer | mia |
| currentNrOfInstances | container, timeSeries, flexContainer | cni |
| location | CSEBase, remoteCSE, AE, container, flexContainer, timeSeries, deviceInfo | loc |
| custodian | All except contentInstance, timeSeriesInstance, flexContainerInstance, accessControlPolicy, pollingChannel | cstn |

Table 8.2.3-2: Resource attribute short names (2/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| currentByteSize | container,timeSeries, flexContainer | cbs |
| locationID | container | li |
| disableRetrieval | container | disr |
| contentInfo | contentInstance, timeSeries | cnf |
| contentSize | contentInstance, timeSeriesInstance, flexContainer, flexContainerInstance | cs |
| contentRef | contentInstance | conr |
| containerDefinition | flexContainer | cnd |
| primitiveContent | request | pc* |
| content | contentInstance, timeSeriesInstance | con |
| deletionCnt | contentInstance | dcnt |
| cseType | CSEBase, remoteCSE | cst |
| CSE-ID | CSEBase, remoteCSE, service SubscribedNode, AEContactListPerCSE | csi |
| supportedResourceType | CSEBase | srt |
| notificationCongestionPolicy | CSEBase | ncp |
| currentTime | CSEBase | ctm |
| source | delivery | sr |
| target | delivery, request | tg |
| lifespan | delivery | ls |
| eventCat | delivery | ec |
| deliveryMetaData | delivery | dmd |
| aggregatedRequest | delivery | arq |
| eventID | eventConfig, statsCollect | evi |
| eventType | eventConfig | evt |
| evenStart | eventConfig | evs |
| eventEnd | eventConfig | eve |
| operationType | eventConfig | opt |
| dataSize | eventConfig | ds |
| eventResourceTypes | eventConfig | erts |
| eventResourceIDs | eventConfig | eris |
| execStatus | execInstance | exs |
| execResult | execInstance | exr |
| execDisable | execInstance | exd |
| execTarget | execInstance, mgmtCmd | ext |
| execMode | execInstance, mgmtCmd | exm |
| execFrequency | execInstance, mgmtCmd | exf |
| execDelay | execInstance, mgmtCmd | exy |
| execNumber | execInstance, mgmtCmd | exn |
| execReqArgs | execInstance, mgmtCmd | exra |
| execEnable | mgmtCmd | exe |
| memberType | group | mt |
| specializationType | group | spty |
| currentNrOfMembers | group | cnm |
| maxNrOfMembers | group | mnm |
| memberIDs | group, backgroundDataTransfer | mid |
| membersAccessControlPolicyIDs | group | macp |
| memberTypeValidated | group | mtv |
| consistencyStrategy | group | csy |
| semanticSupportIndicator | group | ssi |
| notifyAggregation | group | nar |
| groupName | group, subscription | gn |
| locationSource | locationPolicy | los |
| locationUpdatePeriod | locationPolicy | lou |
| locationTargetID | locationPolicy | lot |
| locationServer | locationPolicy | lor |
| locationContainerID | locationPolicy | loi |
| locationContainerName | locationPolicy | lon |
| locationStatus | locationPolicy | lost |
| authID | locationPolicy | aid |
| retrieveLastKnownLocation | locationPolicy | rlkl |
| locationUpdateEventCriteria | locationPolicy | luec |
| locationInformationType | locationPolicy | lit |
| geographicalTargetArea | locationPolicy | gta |
| geofenceEventCriteria | locationPolicy | gec |
| description | mgmtCmd, mgmtObj, all management resources from firmware, ontology, ontologyMappingAlgorithm | dc |
| cmdType | mgmtCmd | cmt |
| mgmtDefinition | mgmtObj, all management resources from firmware | mgd |
| objectIDs | mgmtObj | obis |

Table 8.2.3-3: Resource attribute short names (3/6)

| Attribute Name |  | Occurs in |  | Short Name |  |
| --- | --- | --- | --- | --- | --- |
| objectPaths |  | mgmtObj |  | obps |  |
| mgmtSchema |  | mgmtObj |  | mgs |  |
| nodeID |  | node |  | ni |  |
| hostedCSELink |  | node |  | hcl |  |
| mgmtClientAddress |  | node |  | mgca |  |
| hostedAELinks |  | node |  | hael |  |
| hostedServiceLinks |  | node |  | hsl |  |
| networkID |  | node |  | nid |  |
| roamingStatus |  | node |  | rms |  |
| nodeType |  | node |  | nty |  |
| requestAggregation |  | pollingChannel |  | rqag |  |
| CSEBase |  | remoteCSE |  | cb* |  |
| M2M-Ext-ID |  | remoteCSE, AE, locationPolicy, triggerRequest |  | mei |  |
| M2M-Ext-IDs |  | nwMonitoringReq |  | meis |  |
| Trigger-Recipient-ID |  | remoteCSE, triggerRequest, AE |  | tri |  |
| requestReachability |  | remoteCSE |  | rr |  |
| triggerReferenceNumber |  | remoteCSE, AE |  | trn |  |
| descendantCSEs |  | remoteCSE |  | dcse |  |
| multicastCapability |  | remoteCSE |  | mtcc |  |
| originator |  | request, flexContainerInstance |  | org |  |
| metaInformation |  | request |  | mi |  |
| requestStatus |  | request |  | rs |  |
| operationResult |  | request |  | ors |  |
| operation |  | request |  | op* |  |
| requestID |  | request |  | rid |  |
| scheduleElement |  | schedule |  | se |  |
| networkCoordinated |  | schedule |  | nco |  |
| deviceIdentifier |  | serviceSubscribedNode |  | di |  |
| ruleLinks |  | serviceSubscribedNode |  | rlk |  |
| niddRequired |  | serviceSubscribedNode |  | nrq |  |
| statsCollectID |  | statsCollect |  | sci |  |
| collectingEntityID |  | statsCollect |  | cei |  |
| collectedEntityID |  | statsCollect |  | cdi |  |
| devStatus |  | areaNwkDeviceInfo |  | ss |  |
| statsRuleStatus |  | statsCollect |  | srs |  |
| statModel |  | statsCollect |  | sm |  |
| collectPeriod |  | statsCollect |  | cp |  |
| eventNotificationCriteria |  | subscription |  | enc |  |
| expirationCounter |  | subscription, crossResourceSubscription |  | exc |  |
| notificationURI |  | subscription, crossResourceSubscription |  | nu |  |
| groupID |  | subscription |  | gpi |  |
| notificationForwardingURI |  | subscription |  | nfu |  |
| batchNotify |  | subscription |  | bn |  |
| rateLimit |  | subscription |  | rl |  |
| preSubscriptionNotify |  | subscription |  | psn |  |
| pendingNotification |  | subscription |  | pn |  |
| notificationStoragePriority |  | subscription |  | nsp |  |
| latestNotify |  | subscription |  | ln |  |
| notificationContentType |  | subscription |  | nct |  |
| notificationEventCat |  | subscription, crossResourceSubscription |  | nec |  |
| subscriberURI |  | subscription, crossResourceSubscription |  | su |  |
| version |  | firmware, software, token, softwareCampaign |  | vr |  |
| URL |  | firmware, software, softwareCampaign |  | url |  |
| update |  | firmware |  | ud |  |
| updateStatus |  | firmware |  | uds |  |
| install |  | software |  | in |  |
| uninstall |  | software |  | un |  |
| installStatus |  | software |  | ins |  |
| activate |  | software, m2mServiceSubscriptionProfile |  | act |  |
| deactivate |  | software |  | dea |  |
| activeStatus |  | software, areaNwkInfo |  | acts |  |
| memAvailable |  | memory |  | mma |  |
| memTotal |  | memory |  | mmt |  |

Table 8.2.3-4: Resource attribute short names (4/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| areaNwkType | areaNwkInfo | ant |
| listOfDevices | areaNwkInfo | ldv |
| devId | areaNwkDeviceInfo | dvd |
| devType | areaNwkDeviceInfo | dvt |
| areaNwkId | areaNwkDeviceInfo | awi |
| sleepInterval | areaNwkDeviceInfo | sli |
| sleepDuration | areaNwkDeviceInfo | sld |
| listOfNeighbors | areaNwkDeviceInfo | lnh |
| batteryLevel | battery | btl |
| batteryStatus | battery | bts |
| deviceLabel | deviceInfo | dlb |
| manufacturer | deviceInfo | man |
| model | deviceInfo | mod |
| deviceType | deviceInfo | dty |
| fwVersion | deviceInfo | fwv |
| swVersion | deviceInfo | swv |
| hwVersion | deviceInfo | hwv |
| manufacturerDetailsLink | deviceInfo | mfdl |
| manufacturingDate | deviceInfo | mfd |
| subModel | deviceInfo | smod |
| deviceName | deviceInfo | dvnm |
| osVersion | deviceInfo | osv |
| country | deviceInfo | cnty |
| systemTime | deviceInfo | syst |
| supportURL | deviceInfo | spur |
| presentationURL | deviceInfo | purl |
| protocol | deviceInfo | ptl |
| capabilityName | deviceCapability | can |
| attached | deviceCapability | att |
| capabilityActionStatus | deviceCapability | cas |
| enable | deviceCapability, allJoynSvcObject | ena |
| disable | deviceCapability | dis |
| currentState | deviceCapability | cus |
| reboot | reboot | rbo |
| factoryReset | reboot | far |
| logTypeId | eventLog | lgt |
| logData | eventLog | lgd |
| logStatus | eventLog | lgst |
| logStart | eventLog | lga |
| logStop | eventLog | lgo |
| firmwareName | firmware | fwn |
| softwareName | software, softwareCampaign | swn |
| cmdhPolicyName | cmdhPolicy | cpn |
| mgmtLink | cmdhPolicy, activeCmdhPolicy, cmdhDefaults, cmdhNetworkAccessRules, cmdhNwAccessRule | cmlk |
| activeCmdhPolicyLink | activeCmdhPolicy | acmlk |
| order | cmdhDefEcValue, cmdhLimits | od |
| defEcValue | cmdhDefEcValue | dev |
| requestOrigin | cmdhDefEcValue, cmdhLimits | ror |
| requestContext | cmdhDefEcValue, cmdhLimits | rct |
| requestContextNotification | cmdhDefEcValue, cmdhLimits | rctn |
| requestCharacteristics | cmdhDefEcValue, cmdhLimits | rch |
| applicableEventCategories | cmdhNetworkAccessRules | aecs |
| applicableEventCategory | cmdhEcDefParamValues, cmdhBuffer | aec |
| defaultRequestExpTime | cmdhEcDefParamValues | dqet |
| defaultResultExpTime | cmdhEcDefParamValues | dset |
| defaultOpExecTime | cmdhEcDefParamValues | doet |
| defaultRespPersistence | cmdhEcDefParamValues | drp |
| defaultDelAggregation | cmdhEcDefParamValues | dda |
| limitsEventCategory | cmdhLimits | lec |
| limitsRequestExpTime | cmdhLimits | lqet |
| limitsResultExpTime | cmdhLimits | lset |
| limitsOpExecTime | cmdhLimits | loet |
| limitsRespPersistence | cmdhLimits | lrp |
| limitsDelAggregation | cmdhLimits | lda |
| targetNetwork | cmdhNwAccessRule | ttn |

Table 8.2.3-5: Resource attribute short names (5/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| minReqVolume | cmdhNwAccessRule | mrv |
| spreadingWaitTime | cmdhNwAccessRule | swt |
| backOffParameters | cmdhNwAccessRule | bop |
| otherConditions | cmdhNwAccessRule | ohc |
| maxBufferSize | cmdhBuffer | mbfs |
| storagePriority | cmdhBuffer | sgp |
| applicableCredIDs | serviceSubscribedAppRule | apci |
| allowedApp-IDs | serviceSubscribedAppRule | aai |
| allowedAEs | serviceSubscribedAppRule | aae |
| allowedRole-IDs | serviceSubscribedAppRule | ari |
| notificationTargetURI | notificationTargetMgmtPolicyRef | ntu |
| notificationPolicyID | notificationTargetMgmtPolicyRef | npi |
| action | notificationTargetPolicy | acn |
| policyLabel | notificationTargetPolicy | plbl |
| rulesRelationship | notificationTargetPolicy | rrs |
| creator | notificationTargetPolicy | cr |
| deletionRules | policyDeletionRules | dr |
| deletionRulesRelation | policyDeletionRules | drr |
| dynamicAuthorizationConsultationIDs | All resources having an accessControlPolicyID attribute | daci |
| dynamicAuthorizationEnabled | dynamicAuthorizationConsultation | dae |
| dynamicAuthorizationPoA | dynamicAuthorizationConsultation | dap |
| dynamicAuthorizationLifetime | dynamicAuthorizationConsultation | dal |
| descriptorRepresentation | semanticDescriptor | dcrp |
| semanticOpExec | semanticDescriptor | soe |
| descriptor | semanticDescriptor | dsp |
| relatedSemantics | semanticDescriptor | rels |
| semanticValidated | semanticDescriptor | svd |
| validationEnable | semanticDescriptor | vlde |
| periodicInterval | timeSeries | pei |
| missingDataDetect | timeSeries | mdd |
| missingDataMaxNr | timeSeries | mdn |
| missingDataList | timeSeries | mdlt |
| missingDataCurrentNr | timeSeries | mdc |
| missingDataDetectTimer | timeSeries | mdt |
| dataGenerationTime | timeSeriesInstance | dgt |
| sequenceNr | timeSeriesInstance | snr |
| roleID | role | rlid |
| roleName | role | rlnm |
| tokenLink | role | rltl |
| tokenID | token | tkid |
| tokenObject | token | tkob |
| issuer | token, role | tkis |
| holder | token, role | tkhd |
| notBefore | token, role | tknb |
| notAfter | token, role | tkna |
| tokenName | token | tknm |
| audience | token | tkau |
| permissions | token | tkps |
| extension | token | tkex |
| e2eSecInfo | CSEBase, remoteCSE, AE | esi |
| supportedReleaseVersions | CSEBase, remoteCSE, AE | srv |

Table 8.2.3-6: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| direction | allJoynApp | dir |
| objectPath | allJoynSvcObject | ajop |
| interfaceIntrospectXmlRef | allJoynInterface | ajir |
| input | allJoynMethodCall | inp |
| callStatus | allJoynMethodCall | clst |
| output | allJoynMethodCall | out |
| currentValue | allJoynProperty | crv |
| requestedValue | allJoynProperty | rqv |
| decision | authorizationDecision | dec |
| status | authorizationDecision, authorizationPolicy, authorizationInformation, m2mServiceSubscriptionProfile | sus |
| to | authorizationDecision, authorizationPolicy | to* |
| from | authorizationDecision, authorizationInformation | fr* |
| requestedResourceType | authorizationDecision | rrt |
| operation | authorizationDecision | op* |
| filterUsage | authorizationDecision | fu |
| roleIDs | authorizationDecision, authorizationInformation | rids* |
| tokenIDs | authorizationDecision, authorizationInformation | tids* |
| tokens | authorizationDecision | tkns* |
| requestTime | authorizationDecision | rtm |
| originatorLocation | authorizationDecision | olo |
| originatorIP | authorizationDecision | oip |
| policies | authorizationPolicy | ps |
| combiningAlgorithm | authorizationPolicy | ca |
| ontologyFormat | ontology | ontf |
| ontologyContent | ontology | ontc |
| sourceOntology | ontologyMapping | sont |
| targetOntology | ontologyMapping | tont |
| mappingPolicy | ontologyMapping | mpol |
| mappingAlgorithmLinks | ontologyMapping | mpal |
| mappingResultFormat | ontologyMapping | mprf |
| mappingResult | ontologyMapping | mpr |
| executable | ontologyMappingAlgorithm | exec |
| algorithmType | ontologyMappingAlgorithm | algt |
| mappingThreshold | ontologyMappingAlgorithm | mpth |
| memberFilter | semanticMashupJobProfile | mbft |
| smiID | semanticMashupJobProfile | miid |
| inputDescriptor | semanticMashupJobProfile | iptd |
| outputDescriptor | semanticMashupJobProfile | uptd |
| functionDescriptor | semanticMashupJobProfile | fucd |
| smjpID | semanticMashupInstance | mjid |
| smjpInputParameter | semanticMashupInstance, semanticMashupResult | jpin |
| memberStoreType | semanticMashupInstance | mst |
| mashupMember | semanticMashupInstance | msm |
| resultGenType | semanticMashupInstance | rgt |
| periodForResultGen | semanticMashupInstance | prg |
| mashupResultFormat | semanticMashupResult | mrf |
| mashupResult | semanticMashupResult | mrt |
| ruleRepresentation | reasoningRules | rrep |
| ruleRepresentationFormat | reasoningRules | rrepf |
| reasoningType | reasoningJobInstance | rtyp |
| reasoningMode | reasoningJobInstance | rmod |
| reasoningPeriod | reasoningJobInstance | rper |
| factSet | reasoningJobInstance | rfst |
| ruleSet | reasoningJobInstance | rrst |
| resultRepresentation | reasoningJobInstance | rsrp |
| resultRepresentationFormat | reasoningJobInstance | rsrpf |
| numberImpactedCSEs | AEContactList | nic |
| externalGroupID | LocalMulticastGroup, remoteCSE, nwMonitoringReq, AE | egid |
| multicastAddress | LocalMulticastGroup | mad |
| multicastGroupFanoutTarget | LocalMulticastGroup | mgft |
| memberList | LocalMulticastGroup | mli |
| responseTarget | LocalMulticastGroup | rstt |
| responseTimeWindow | LocalMulticastGroup | rstw |
| TMGI | LocalMulticastGroup | tmgi |
| sessionOriginatorID | multimediaSession | soi |
| acceptedSessionDescriptions | multimediaSession | asd |
| offeredSessionDescriptions | multimediaSession | osd |
| sessionState | multimediaSession | sst |
| triggerPurpose | triggerRequest | tpe |
| triggerStatus | triggerRequest | tst |
| triggerValidityTime | triggerRequest | tvt |
| triggerInfoAE-ID | triggerRequest | tiae |
| triggerInfoAddress | triggerRequest | tia |
| triggerInfoOperation | triggerRequest | tio |
| targetedResourceType | triggerRequest | tirt |
| triggerReference | triggerRequest | trf |
| regularResourcesAsTarget | crossResourceSubscription | rrat |
| subscriptionResourcesAsTarget | crossResourceSubscription | srat |
| regularResourcesAsTargetSubscriptions | crossResourceSubscription | rrats |
| timeWindowType | crossResourceSubscription | twt |
| timeWindowSize | crossResourceSubscription | tws |
| eventNotificationCriteriaSet | crossResourceSubscription | encs |
| eventEvaluationMode | crossResourceSubscription | eem |
| associatedCrossResourceSub | subscription | acrs |
| volumePerNode | backgroundDataTransfer | vpn |
| numberOfNodes | backgroundDataTransfer | non |
| desiredTimeWindow | backgroundDataTransfer | dtw |
| transferSelectionGuidance | backgroundDataTransfer | tsg |
| geographicInformation | backgroundDataTransfer | ggi |
| groupLink | backgroundDataTransfer | gli |
| transactionLockTime | transactionMgmt, transaction | tltm |
| transactionExecuteTime | transactionMgmt, transaction | text |
| transactionCommitTime | transactionMgmt, transaction | tct |
| transactionExpirationTime | transactionMgmt | tept |
| transactionMode | transactionMgmt | tmd |
| transactionLockType | transactionMgmt, transaction | tltp |
| transactionControl | transactionMgmt, transaction | tctl |
| transactionState | transactionMgmt, transaction | trst |
| transactionMaxRetries | transactionMgmt | tmr |
| transactionMgmtHandling | transactionMgmt | tmh |
| requestPrimitives | transactionMgmt | rqps |
| responsePrimitives | transactionMgmt | rsps |
| transactionID | transaction | tid |
| sessionEndpoints | e2eQosSession | eqse |
| e2eQosRequirements | e2eQosSession | eqsr |
| e2eQosPolicies | e2eQosSession | eqsp |
| e2eQosStatus | e2eQosSession | eqss |
| beaconRequester | timeSyncBeacon | bcnr |
| beaconCriteria | timeSyncBeacon | bcnc |
| beaconInterval | timeSyncBeacon | bcni |
| beaconThreshold | timeSyncBeacon | bcnt |
| beaconNotificationURIs | timeSyncBeacon | bcnu |
| monitorEnable | nwMonitoringReq | mnen |
| monitorStatus | nwMonitoringReq | mnst |
| failureReason | nwMonitoringReq | frsn |
| geographicArea | nwMonitoringReq | geoa |
| congestionLevel | nwMonitoringReq | cgnl |
| congestionStatus | nwMonitoringReq | cgns |
| numberOfDevices | nwMonitoringReq | ndev |
| notificationStatsEnable | subscription, crossResourceSubscription | nse |
| notificationStatsInfo | subscription, crossResourceSubscription | nsi |
| IDList | primitiveProfile | idl |
| resourceTypes | primitiveProfile | rtys |
| resourceIDs | primitiveProfile | rsds |
| releaseVersions | primitiveProfile | rvs |
| additions | primitiveProfile | adds |
| deletions | primitiveProfile | dels |
| applicability | primitiveProfile | appl |
| processStatus | processManagement | prst |
| processControl | processManagement | prct |
| currentState | processManagement | cust |
| activateConditions | processManagement | atcos |
| endConditions | processManagement | encos |
| initialState | processManagement | inst |
| stateActive | state | sact |
| stateAction | state | stac |
| stateTransitions | state | sttrs |
| actionPriority | action | apy |
| subjectResourceID | action | sri |
| evalCriteria | action, dependency | evc |
| evalMode | action | evm |
| evalControlParam | action | ecp |
| dependencies | action | dep |
| objectResourceID | action | orc |
| actionPrimitive | action | apv |
| input | action | ipu |
| actionResult | action | air |
| sufficient | dependency | sfc |
| referencedResourceID | dependency | rri |
| M2M-Sub-ID | m2mServiceSubscriptionProfile | msui |
| M2M-SS-ID | m2mServiceSubscriptionProfile | mssi |
| M2M-User-ID | serviceSubscribedUserProfile | mui |
| allowedUsers | serviceSubscribedAppRule | allu |
| activationTime | m2mServiceSubscriptionProfile | actt |
| deactivationTime | m2mServiceSubscriptionProfile | dact |
| serviceSubscriptionDuration | m2mServiceSubscriptionProfile | ssd |
| currentNumAEs | m2mServiceSubscriptionProfile | cna |
| currentNumNodes | m2mServiceSubscriptionProfile | cnn |
| maxNumAEs | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mna |
| maxNumNodes | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnns |
| maxNumBytes | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnb |
| maxRequestRate | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mrr |
| maxNumContainers | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnc |
| maxNumInstancesPerContainer | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnis |
| maxNumTimeSeries | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnt |
| maxNumInstancesPerTimeSeries | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnit |
| maxMembersPerGroup | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mmg |
| maxNotificationRate | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnr |
| maxNumFlexContainers | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnf |
| maxNumInstancesPerFlexContainer | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnif |
| defaultAccessControlPrivileges | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | dacp |
| campaignEnabled | softwareCampaign | swce |
| campaignStatus | softwareCampaign | swcs |
| softwareTargets | softwareCampaign | swts |
| softwareTriggerCriteria | softwareCampaign | swtc |
| softwareOperation | softwareCampaign | swop |
| softwareLinks | softwareCampaign | swlk |
| aggregatedSoftwareStatus | softwareCampaign | asws |
| individualSoftwareStatuses | softwareCampaign | isws |
| NOTE: * marked short names have been already assigned in Table 8.2.2-1. |  |  |

### ********************* End of Change 4 *********************************