| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 60.1 |
| Source:* | Miguel Angel Reina Ortega, ETSI, MiguelAngel.ReinaOrtega@etsi.org Bob Flynn, Exacta GSS, bob.flynn@exacctagss.com Andreas Kraft, DT, A.Kraft@telekom.de |
| Date:* | 2023-09-19 |
| Reason for Change/s:* | New announce access control policy IDs |
| CR against: Release* | Rel-5 |
| CR against: WI* | Active < WI-0077> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v5.3.0 |
| Clauses * | 9.6.1.3.2, 9.6.26.1, 9.6.26.3, 10.2.13.2, 10.2.13.5 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes a new attribute for all anounceable resources so that an originator can manage access rights for the announced resource separately from its own access rights. This was discussed in contribution SDS-2022-0020-Discussion_on_announcement_issues item 1.

## ----------------------- Start of Change 1--------------------------------------------

##### 9.6.1.3.2	Common attributes

The following attributes are commonly used in multiple, but not all, resource types which are normal, not virtual or announced. Common attributes for announced resource types are independently defined in clause 9.6.26.3.

NOTE:	The list of attributes in table 9.6.1.3.2-1 is not exhaustive.

Table 9.6.1.3.2-1: Common Attributes

| Attribute Name | Description |
| --- | --- |
| accessControlPolicyIDs | The attribute contains a list of identifiers for <accessControlPolicy> resources. The privileges defined in the <accessControlPolicy> resources that are referenced determine who is allowed to access the resource containing this attribute for a specific purpose (e.g. Retrieve, Update, Delete, etc.). For an Update operation to a resource, it is forbidden to change the accessControlPolicyIDs attribute in the same request to Update other attributes of the targeted resource, i.e a request to Update the accessControlPolicyIDs attribute shall be the only attribute in the UPDATE request. To update the accessControlPolicyIDs attribute, a Hosting CSE shall check whether the Originator has Update privilege in any current selfPrivileges of the <accessControlPolicy> resources which this attribute references. To update any attribute other than the accessControlPolicyIDs attribute, a Hosting CSE shall check whether the Originator has Update privilege in any privileges, of the <accessControlPolicy> resources which the accessControlPolicyIDs attribute references. If a resource type does not have an accessControlPolicyIDs attribute definition, then the accessControlPolicyIDs for that resource is governed in a different way, for example, the accessControlPolicy associated with the parent may apply to a child resource that does not have an accessControlPolicyIDs attribute definition, or the privileges for access are fixed by the system. Refer to the corresponding resource type definitions and procedures to see how access control is handled in such cases. If a resource type does have an accessControlPolicyIDs attribute definition, but the (optional) accessControlPolicyIDs attribute value is not set in a resource instance, then the Hosting CSE shall apply the concept of the default access policy. The Hosting CSE shall first check whether the resource has a custodian attribute configured and if so, the default policy shall provide unrestricted access only to the custodian. If the custodian attribute is not configured, then the default policy shall provide unrestricted access only to the Originator of the successful resource creation request. All other entities shall be denied to access the resource. For that purpose, the Hosting CSE shall keep that Originator information of the resource. Note that how to keep that information is implementation specific. The default access policy is not applied to a resource which has a value assigned to the accessControlPolicyIDs attribute. All resources are accessible if and only if the privileges (i.e. configured as privileges or selfPrivileges attribute of <accessControlPolicy> resource) allow it, therefore all resources shall have an associated accessControlPolicyIDs attribute, either explicitly (setting the attribute in the resource itself) or implicitly (either by using the parent privileges or the system default policies). Which means that the system shall provide default access privileges in case that the Originator does not provide a specific accessControlPolicyIDs during the creation of the resource. |
| expirationTime | Time/date after which the resource will be deleted by the Hosting CSE. This attribute can be provided by the Originator, and in such a case it will be regarded as a hint to the Hosting CSE on the lifetime of the resource. The Hosting CSE shall configure the expirationTime value. If the Hosting CSE configures the new expirationTime attribute value rather than the Originator suggested value, the new value can be sent back to the Originator depending on the Result Content value. The lifetime of the resource can be extended by providing a new value for this attribute in an UPDATE operation. Or by deleting the attribute value, e.g. by updating the attribute with NULL when doing a full UPDATE, in which case the Hosting CSE can decide on a new value. If the Originator does not provide a value in the CREATE operation the system shall assign an appropriate value depending on its local policies and/or M2M service subscription agreements. A resource is known as 'obsolete' when the resource contains the attribute "expirationTime" and the lifetime of this resource has reached the value of this attribute. If the 'obsolete' resource had a reference to an Application Entity Resource ID, the Hosting CSE shall send a NOTIFY request to the IN-CSE, requesting to delete the entry from the <AEContactList> resource. |
| stateTag | An incremental counter of modification on the resource. When a resource is created, this counter is set to 0, and it will be incremented on every modification of the resource (see notes 1 and 2). |
| announceTo | This attribute may be included in a CREATE or UPDATE Request in which case it contains a list of addresses/CSE-IDs where the resource is to be announced. For the case that CSE-IDs are provided, the announced-to CSE shall decide the location of the announced resources based on the rules described in clause 9.6.26. For the original resource, this attribute shall only be present if it has been successfully announced to other CSEs. This attribute maintains the list of the resource addresses to the successfully announced resources. Updates on this attribute will trigger new resource announcement or de-announcement. If the announceTo attribute includes resource address(s), the present document does not provide any means for validating these address(s) for announcement purposes. It is the responsibility of the Hosting-CSE referenced by the resource address(s) to validate the access privileges of the originator of the Request that triggers the announcement. |
| announcedAttribute | This attribute shall only be present at the original resource if some Optional Announced (OA) type attributes have been announced to other CSEs. This attribute maintains the list of the announced Optional Attributes (OA type attributes) in the original resource. Updates to this attribute will trigger new attribute announcement if a new attribute is added or de-announcement if the existing attribute is removed. |
| announceSyncType | This attribute indicates the types of synchronization for resource announcement. Possible values are as follows: - Uni-directional synchronization: Announced resource(s) is updated if the original resource is updated - Bi-directional synchronization: Announced resource(s) is updated if the original resource is updated and vice versa This attribute is presented in both the original resource and the announced resource(s). The absence of this attribute implies that uni-directional synchronization is the type of supported synchronization for resource announcement. |
| announceAccessControlPolicyIDs | If announceTo attribute is present in a CREATE Request, this attribute shall also be present. If announceTo attribute is present in an UPDATE Request, this attribute shall also be present either in the request or in the to-be-updated resource. The attribute contains a list of identifiers for <accessControlPolicy> resources. The privileges defined in the <accessControlPolicy> resources that are referenced determine who is allowed to access the announced resource for a specific purpose (e.g. Retrieve, Update, Delete, etc.). See clause x.x.x.x |
| labels | Tokens used to add meta-information to resources. This attribute is optional. The value of the labels attribute is a list of individual labels, each of them being: - Either a standalone label-key, used as a simple "tag", that can be used for example for discovery purposes when looking for particular resources that one can "tag" using that label-key. - Or a composite element made of a label-key and a label-value, separated by a special character defined in oneM2M TS-0004 [3]. The list of allowed characters in a label (and in label-keys and label-values) and separator characters is defined in oneM2M TS-0004 [3], clause 6.3.3. |
| e2eSecInfo | Present in a resource representing an AE or CSE. Indicates the end-to-end security capabilities supported by the AE or CSE. May indicate supported end-to-end security frameworks. May also contains a certificate or credential identifier used by the AE or CSE. May include random values for use in end-to-end security protocols. The details of this attributes are described in oneM2M TS-0003 [2]. This attribute is optional and if not present it means that the represented entity does not support oneM2M end-to-end security procedures. |
| dynamicAuthorizationConsultationIDs | This attribute contains a list of identifiers of <dynamicAuthorizationConsultation> resources. The information defined in a <dynamicAuthorizationConsultation> resource is used by a CSE for initiating consultation-based dynamic authorization requests. Consultation-based dynamic authorization is only performed for a targeted resource if and only if it is linked to an enabled <dynamicAuthorizationConsultation> resource. If the attribute is not set or has a value that does not correspond to a valid <dynamicAuthorizationConsultation> resource(s), or it refers to an <dynamicAuthorizationConsultation> resource(s) that is not reachable, then the dynamicAuthorizationConsultationIDs associated with the parent may apply to the child resource if present, or a system default <dynamicAuthorizationConsultation> may apply if present. |
| creator | The AE-ID or CSE-ID of the entity which created the resource containing this attribute. |
| location | This attribute contains the geo-coordinates of entities or things represented by its resource types (e.g. AE, container). Longitude and Latitude are shall be included as a coordinate and optionally altitude may also be included. The representation format shall follow the definition in the GeoJSON format [16]. This attribute can be used for geo-query (Clause 10.2.6) with relevant filter conditions (clause 8.1.2). |
| custodian | The AE-ID, M2M-User-ID or CSE-ID of the entity which owns the resource containing this attribute. |
| NOTE 1: In order to enable detection of overflow, the counter needs to be capable of expressing sufficiently long numbers. NOTE 2: This attribute has the scope to allow identifying changes in resources within a time interval that is lower than the one supported by the attribute lastModifiedTime (e.g. less than a second or millisecond). This attribute can also be used to avoid race conditions in case of competing modifications. |  |

-------------------------------------------------- End of Change 1---------------------------------------

## ----------------------- Start of Change 2--------------------------------------------

#### 9.6.26.1	Overview

A resource can be announced to one or more remote CSEs to inform the remote CSEs of the existence of the original resource. An announced resource can have a limited set of attributes and a limited set of child resources from the original resource. The announced resource includes a link to the original resource hosted by the original resource-Hosting CSE.

In case that the original resource is deleted, all announced resources for the original resource shall be deleted, except for <AEAnnc> resources that were created during the registration of an AE with AE-ID-Stem starting with "S", which shall not be deleted. If the announced resource is not deleted promptly (e.g. the announced resource is not reachable), the announced resource can be deleted later either by the original resource Hosting CSE or by the expiration of the announced resource itself. The original resource shall store the list of links for the announced resources for those purposes.

Synchronization between the attributes announced by the original resource and the announced resource shall be the responsibility of the original resource Hosting CSE and announced resource Hosting CSE depending on the value of the announceSyncType attribute. There shall not be any synchronization for unannounced children created at the original resource and the announced resource. The access control policy for the announced resource shall synchronize with the announceAccessControlPolicyIDs attribute of the original resource.

The original resource shall have at least announceTo and announceAccessControlPolicyIDs attributes present if the resource itself has been announced. If any of the Optional Announced (OA) attributes are also announced, then announcedAttribute attribute shall also be present. An AE or other CSE can request the original resource Hosting CSE for announcing the original resource to the list of CSE-IDs or the address(es) listed in the announceTo attribute in the announcing request. An Update to the announceTo attribute will trigger new resource announcement(s) or the de-announcement(s) of the announced resource. After a successful announcement procedure the attribute announceTo contains only the list of address(es) of the announced resources.

In order to announce an attribute marked as OA (see clause 9.5.0), the attribute shall be included in the announcedAttribute attribute list at the original resource. The attributes included in the announcedAttribute attribute are announced to the announced resource. On successful announcement of the resource, such attributes shall be created at the announced resource; otherwise they shall not be present in the announced resource. Update to the announcedAttribute attribute in the original resource will trigger new attribute announcement or the de-announcement of the announced attribute(s). Synchronization between the value of the announced attributes at the original resource and the announced resource is the responsibility of the original resource Hosting CSE and announced resource Hosting CSE depending on the value of the announceSyncType attribute. If an attribute in the original resource has an identifier type (i.e. CSE-ID, AE-ID, resource identifier), the original resource Hosting CSE shall convert the value of this attribute to an identifier format (see table 7.2-1) required by the Receiver CSE.

If an announced resource is updated by an entity other than the original resource Hosting CSE and the announceSyncType attribute of the announced resource has a value of "Bi-directional synchronization", then the announced resource Hosting CSE shall update the original resource to synchronize the values of the announced resource attributes to the original resource. In this case, the original resource Hosting CSE shall UPDATE all other announced resource(s) listed in the announceTo attribute. If an updated announced attribute has an identifier type (i.e. CSE-ID, AE-ID, resource identifier) the announced resource Hosting CSE shall convert the value of this attribute to an identifier format (see table 7.2-1) required by the original resource Hosting CSE.

An announced resource may have child resources. In general, a child resource of an announced resource shall be of one of the resource types that are specified as possible child resource types for the original resource or of one of their associate announced resource types. However, for specific announced resource types, specific exceptions apply regarding which child resource types can occur. The details on which child resources are specified for each announced resource type are summarized in table 9.6.26.1-1.

Child resources of the original resource can be announced independently as needed. In this case, the child resources at the announced resource shall be of the child resource's associated announced type. When a child resource at the announced resource is created locally at the remote CSE, the child resource shall be of ordinary - i.e. not-announced - child resource type.

When a Hosting CSE of an original resource is initiating an announcement, it shall first check if the parent resource is announced to the announcement target CSE by checking the announceTo attribute of the parent resource and if so, create the announced resource as a child of the announced parent resource. If the parent resource is not announced, the Hosting CSE shall check if <CSEBaseAnnc> exists into the announcement target CSE. If it does not exist, the Hosting CSE shall create a <CSEBaseAnnc> resource as a child of the <CSEBase> resource of the announcement target CSE. The Hosting CSE shall then create the announced resource as a child resource of the <CSEBaseAnnc> resource.

When a Hosting CSE of an original resource is initiating an announcement, the From parameter of the announce request shall contain either a SP-relative-CSE-ID of the Hosting CSE of the original resource if the announcement target CSE resides in the same SP domain or an Absolute-CSE-ID of the Hosting CSE of the original resource if the announcement target CSE resides in a different SP domain.

If an attribute is marked as RO and also marked as MA or OA, then only the attribute of the original resource shall be interpreted as RO. The corresponding attribute of the announced resource shall be always writable to the original resource hosting CSE to allow it to properly announce and de-announce the attribute and keep the announced attribute synchronized with the original one. Only the original resource Hosting CSE shall be allowed to update and delete the announced attribute which is created by the original resource Hosting CSE.

Table 9.6.26.1-1: Announced Resource Types

| Announced Resource Type | Short Description | Child Resource Types | Clause |
| --- | --- | --- | --- |
| accessControlPolicyAnnc | Announced variant of accessControlPolicy | subscription | 9.6.2 |
| actionAnnc | Announced variant of action | dependencyAnnc, subscription, transaction | 9.6.61 |
| AEAnnc | Announced variant of AE | subscription, container, containerAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc semanticDescriptor, semanticDescriptorAnnc, timeSeries, timeSeriesAnnc | 9.6.5 |
| containerAnnc | Announced variant of container | container, containerAnnc, flexContainer, flexContainerAnnc, contentInstance, contentInstanceAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc, timeSeries, timeSeriesAnnc | 9.6.6 |
| contentInstanceAnnc | Announced variant of contentInstance | semanticDescriptor, semanticDescriptorAnnc | 9.6.7 |
| CSEBaseAnnc | Announced variant of CSEBase | container, containerAnnc, dynamicAuthorizationConsultationAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, scheduleAnnc, semanticDescriptorAnnc, semanticMashupJobProfileAnnc, timeSeries, timeSeriesAnnc, remoteCSEAnnc, nodeAnnc, mgmtObjAnnc, AEAnnc, locationPolicyAnnc | 9.6.3 |
| dynamicAuthorizationConsultationAnnc | Announced variant of dynamicAuthorizationConsultation | None specified | 9.6.40 |
| flexContainerAnnc | Announced variant of flexContainer | container, containerAnnc, flexContainer, flexContainerAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc, timeSeries, timeSeriesAnnc | 9.6.35 |
| groupAnnc | Announced variant of group | subscription, semanticDescriptor, semanticDescriptorAnnc | 9.6.13 |
| locationPolicyAnnc | Announced variant of locationPolicy | None specified | 9.6.10 |
| mgmtObjAnnc | Announced variant of mgmtObj | subscription | 9.6.15 |
| nodeAnnc | Announced variant of node | mgmtObjAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc, scheduleAnnc | 9.6.18 |
| remoteCSEAnnc | Announced variant of remoteCSE | container, containerAnnc, dynamicAuthorizationConsultationAnnc, flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, scheduleAnnc, semanticDescriptorAnnc, semanticMashupJobProfileAnnc, timeSeries, timeSeriesAnnc, remoteCSEAnnc, nodeAnnc, mgmtObjAnnc, AEAnnc, locationPolicyAnnc | 9.6.4 |
| scheduleAnnc | Announced variant of schedule | None specified | 9.6.9 |
| semanticDescriptorAnnc | Announced variant of semanticDescriptor | Subscription | 9.6.30 |
| semanticMashupInstanceAnnc | Announced variant of semanticMashupInstance | None specified | 9.6.54 |
| semanticMashupJobProfileAnnc | Announced variant of semanticMashupJobProfile | None specified | 9.6.53 |
| timeSeriesAnnc | Announced variant of timeSeries | timeSeriesInstance, timeSeriesInstanceAnnc, subscription, semanticDescriptor, semanticDescriptorAnnc | 9.6.36 |
| timeSeriesInstanceAnnc | Announced variant of timeSeriesInstance | None specified | 9.6.37 |
| timeSyncBeaconAnnc | Announced variant of timeSyncBeacon | subscription | 9.6.69 |

-------------------------------------------------- End of Change 2---------------------------------------

## ----------------------- Start of Change 3--------------------------------------------

#### 9.6.26.3	Common Attributes for Announced Resources

Table 9.6.26.3-1 lists the common attributes for the announced resources.

Table 9.6.26.3-1: Commonly Used Attributes for Announced Resources

| Attribute Name | Mandatory /Optional | Description |
| --- | --- | --- |
| accessControlPolicyIDs | Conditionally Mandatory | The list of identifiers (either an ID or a URI) of an <accessControlPolicy> resource announced by the original resource See clause 9.6.1.3.2 for further information on this attribute. This attribute shall be set to the value of the announceAccessControlPolicyIDs attribute of the original resource. |
| stateTag | Conditionally Mandatory | An incremental counter of modification on the resource. See clause 9.6.1.3.2 for information on this attribute. |
| labels | Conditionally Mandatory | Tokens used as keys for discovering resources as announced by the original resource. See clause 9.6.1.3 for further information on this attribute. The attribute is conditionally mandatory, which means that the attribute shall exist in the announced resource if it is present in the original resource. |
| registrationStatus | Optional | Only optional for announced <AE> resource. Denotes status of the announced AE registration. If ACTIVE, the announced <AE> resource and all its child resources may be discoverable. If INACTIVE, the announced <AE> registration and all its child resources shall not be discoverable. The attribute is conditionally mandatory, which means that the attribute shall exist in the announced resource if it is present in the original resource. |

-------------------------------------------------- End of Change 3---------------------------------------

## ----------------------- Start of Change 4--------------------------------------------

#### 10.2.13.2	Procedure for AE and CSE to initiate Creation of an Announced Resource

This clause describes the procedure for an AE or a CSE to initiate the creation of an announced resource.

Figure 10.2.13.2-1 depicts how creation of an announced resource is initiated (clause 10.2.13.2) and the announced resource is created on an announcement target CSE (clause 10.2.13.5).

Figure 10.2.13.2-1: Announced resource CREATE procedures

The Originator of a Request for initiating resource announcement can be either an AE or a CSE. Two methods are supported for initiating the creation of an announced resource:

CREATE: The Originator can initiate the creation of an announced resource during the creation of the original resource by providing announceTo attribute in the CREATE Request.

UPDATE: The Originator can initiate the creation of an announced resource by using the UPDATE Request to update the announceTo attribute at the original resource.

Table 10.2.13.2-1: Initiate Resource Announcement: UPDATE or CREATE

| Initiate Resource Announcement: CREATE or UPDATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 are applicable as indicated in that table. In addition, for the case of the CREATE procedure for a specific resource is described in clause 10.2. The Originator suggests the address(es) or the CSE-ID(s) to which the resource will be announced in the Content parameter. |
| Processing at the Originator before sending Request | Content: contains address where the resource needs to be announced (within announceTo attribute): The Originator provides either the address(es) for the announced resource or the list of CSE-IDs of the remote CSEs where the original resource needs to be announced by including such information within the announceTo attribute of the UPDATE or CREATE Request. - In the case of a CREATE Request, the announceAccessControlPolicyIDs attribute shall be present. - In the case of an UPDATE Request, the announceAccessControlPolicyIDs attribute shall be present either in the request or in the to-be-updated resource. |
| Processing at the Receiver | Once the Originator has been successfully authorized, the Receiver (which shall be the original resource Hosting CSE) shall grant the Request after successful validation of the Request: If the Request provides address(es) for the announced resource that are not already stored in the announceTo attribute or for a newly created announceTo attribute, the Receiver shall announce the resource to the provided address(es). If the Request provides a list of CSE-IDs of the remote CSEs that are not already stored in the announceTo attribute or for the newly created announceTo attribute, the announced resource target location shall be the announced parent resource or the <CSEBaseAnnc> resource representing the Receiver at the announcement target CSE. In order to determine the target location, the Receiver shall: - Check if the parent resource is announced by checking the announceTo attribute of the parent resource and if so, create the announced resource as a child of the announced parent resource. - If the parent resource is not announced, the Receiver shall check if <CSEBaseAnnc> existsinto the announcement target CSE. If it does not exist, the Receiver shall create a <CSEBaseAnnc> resource as a child of the <CSEBase> resource of the announcement target CSE. The Receiver shall then create the announced resource as a child of the <CSEBaseAnnc> resource. |
| Information in Response message | On successful completion of resource announcement as in clause 10.2.3.5, the Receiver shall provide all parameters defined in table 8.1.3-1 that are applicable as indicated in that table in the Response message: The Receiver shall provide the address(es) of the announced resource to the Originator which replace the given CSE-ID or URI in the content of the announceTo attribute in the original resource and by providing it in the UPDATE or CREATE Response message depending on the type of the Request. |
| Processing at Originator after receiving Response | According to clause 10.1.2 in case of CREATE Request. According to clause 10.1.4 in case of UPDATE Request. |
| Exceptions | All exceptions described in the basic procedures (clause 10.1.2) are applicable. If the parent resource of a contentInstance, or a timeSeriesInstance or a flexContainerInstance is not announced then the announcement shall fail. If the announceAccessControlPolicyIDs attribute is not present in the CREATE Request, then the announcement shall fail. If the announceAccessControlPolicyIDs attribute is not present in the UPDATE Request nor in the to-be-updated resource, then the announcement shall fail. |

-------------------------------------------------- End of Change 4---------------------------------------

## ----------------------- Start of Change 5--------------------------------------------

#### 10.2.13.5	Procedure for original resource Hosting CSE to Create an Announced Resource

This clause explains the resource announcement procedure that shall be used by the original resource Hosting CSE to announce the original resource to the remote CSE(s).

See figure 10.2.13.2-1 for the graphical explanation.

The Originator of this Request shall be the original resource Hosting CSE. The Originator shall request to create the announced resource by using CREATE Request.

Table 10.2.13.5-1: Resource Hosting CSE to Announce Resource: CREATE

| Resource Announcement: CREATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 are applicable as indicated in that table. Content: contains MA attributes and OA attributes that are included in announcedAttribute attribute. |
| Processing at the Originator before sending Request | Other details for the information in the Request message shall be as follows: - Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute at the original resource shall be provided in the CREATE Request. Such attributes shall have the same value as for the original resource. - resourceType which shall be set to the appropriate tag that identifies the <Annc> resource. - expirationTime provided by the Originator equal to the one for the original resource. - The link attribute of the announced resource shall have the address of the original resource in SP-relative Resource-ID format or Absolute Resource-ID format. - The labels attribute of the announced resource shall have the same value as for the original resource. - The accessControlPolicyIDs attribute shall always be provided in the CREATE Request and shall be set to the value of the announceAccessControlPolicyIDs attribute of the original resource. - If an attribute that is to be announced has an identifier type (i.e. CSE-ID, AE-ID, resource identifier), the Originator shall convert the value of the original resource attribute to an identifier format (see table 7.2-1) required by the Receiver. |
| Processing at the Receiver | Once the Originator has been successfully authorized, the Receiver shall grant the Request after successful validation of the Request. The Receiver shall perform as follows: - The basic procedure (clause 10.1.2) for the Receiver of the CREATE Request apply. - The created announced resource shall include the common attributes specified in clause 9.6.26.1. The created announced resource shall contain the additional attributes that are provided by the Originator; i.e. attributes marked with MA and the attributes marked with OA that are included in the announcedAttribute attribute. - The created announced resource shall set the accessControlPolicyIDs attribute to the value received in the Request message, and shall set the labels attribute (if present) and the link attribute to the value received in the Request message. - Respond to the Originator with the CREATE Response. In this Response, the address of the successfully announced resource shall be provided. |
| Information in Response message | All parameters defined in table 8.1.3-1 are applicable as indicated in that table with the specific details for: Content: address where the announced resource is created according to clause 10.1.2. |
| Processing at Originator after receiving Response | The Originator after receiving the Response from the Receiver shall perform the following steps: - If the announced resource has been successfully created, the announceTo attribute of the original resource shall be updated to include the address for the successfully announced resource at the Receiver. The announcedAttribute attribute shall be updated as well to represent the successfully announced attributes as received in the Response. - For the attributes marked as MA and for the attributes marked as OA that are included in the announcedAttribute attribute, the Originator shall further take the responsibility to keep their values synchronized at the announced resource by using UPDATE operation (clause 10.1.4). |
| Exceptions | All exceptions described in the basic procedures (clause 10.1.2) are applicable. |

-------------------------------------------------- End of Change 5---------------------------------------

## ----------------------- Start of Change 6--------------------------------------------

This change is to be applied to all anounceable resources in the corresponding resource attributes table:

| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| --- | --- | --- | --- | --- |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| announceAccessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA (See clause 9.6.26.3) |

-------------------------------------------------- End of Change 6---------------------------------------