| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #66 |
| Source:* | Andreas Kraft, andreas.kraft@exactagss.com |
| Date:* | 2024-08-29 |
| Reason for Change/s:* | Clarifying <schedule> child-resource procedure for <crossResourceSubscription> |
| CR against: Release* | Rel 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004, v.4.21.0 |
| Clauses * | 7.5.1.2.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The CR proposes to add the <crossResourceSubscription> resource type to “7.5.1.2.2 Notification for <subscription> resources”, step 3.0.

The <schedule> resource is a child resource of the <crossResourceSubscription>, but the behaviour for it is not described. Since the effect, ie. creating notifications under certain circumstances, is the same as for <subscription>, the proposed change is just to add the <crossResourceSubscription> to the text of 7.5.1.2.2.

When it comes to the notification handling the <crossResourceSubscription> procedures are pointing to the respective <subscription> procedures in the specifications.

R01:

- Re-wording the changes in Change 1, step 3.

- Removing superfluous text after Change 1 which was included by accident.

### -----------------------Start of change 1-------------------------------------------

##### 7.5.1.2.2	Notification for <subscription> resources

When the notification message is forwarded or aggregated by transit CSEs, the Originator or a transit CSE shall check whether there are notification policies to enforce between subscription resource Hosting CSE and the notification target. In that case, the transit CSE as well as the Originator shall process Notify request primitive(s) by using the corresponding policy and send processed Notify request primitive(s) to the next CSE with notification policies related to the enforcement so that the transit CSE is able to enforce the policy defined by the subscriber. The notification policies related to the enforcement at this time is verified by using the subscription reference in the Notify request primitive. In the notification policies, the latestNotify attribute is only enforced in the transit CSE as well as the Originator.

If Event Category parameter is set to "latest" in the notification request primitive, the transit CSE as well as Originator shall cache the most recent Notify request. That is, if a new Notify request is received by the CSE with a subscription reference that has already been buffered for a pending Notify request, the newer Notify request will replace the buffered older Notify request.

Originator:

When an event is generated, the Originator shall execute the following steps in order:

Step 1.0	Check the eventNotificationCriteria attribute of the <subscription> resource associated with the modified resource:

If the eventNotificationCriteria attribute is set, then the Originator shall check whether the corresponding event matches with the event criteria. If multiple matching conditions of different types (i.e. different condition tags) are present in the eventNotificationCriteria attribute, then the combined condition shall be derived by applying the logical operation specified by the filterOperation condition. By default the logical AND operation shall be used if the filterOperation condition is not present.

If notificationEventType is not set within the eventNotificationCriteria attribute and the operationMonitor is also not present, the Originator shall use the default setting of "Update_of_Resource" to compare against the event.

If the notificationEventType has the value "Create_of_Direct_Child_Resource" or "Delete of Direct Child Resource" and the childResourceType condition is also present, then the matching event shall only be detected if one of the child resource types present in the list has been created or deleted, respectively. If the childResourceType condition is not present then a matching event is generated whenever any child resource is created or deleted.

If the notificationEventType has either an explicit or default value of "Update_of_Resource" and the attribute condition is also present then the matching event shall only be detected if one of the attributes in the list has been updated. If the attribute condition is not present then a matching event is generated whenever any attribute has been updated.

If the event matches, go to the step 2.0. Otherwise, the Originator shall discard the corresponding event.

If the eventNotificationCriteria attribute is not configured, the Originator shall use the default setting of "Update_of_Resource" for the notificationEventType and then continue with the step 2.0.

Step 2.0	The Originator shall check the notification policy as described in the below steps, but the notification policy may be checked in different order. After checking the notification policy in step 2.0 (i.e. from step 2.1 to step 2.6), then continue with step 3.0.

Step 2.1	The Originator shall determine the type of the notification per the notificationContentType attribute. The possible values of for notificationContentType attribute are "Modified Attributes", "All Attributes", "ResourceID", "Trigger Payload" or “TimeSeries notification”. This attribute may be used jointly with the notificationEventType attribute in the eventNotificationCriteria to determine if it is the attributes/resourceID of the subscribed-to resource or the attributes/resourceID of the child resource of the subscribed-to resource that shall be returned in the content of the notification:

If the value of notificationContentType is set to "Modified Attributes", the Notify request primitive shall include the partial resource containing modified attribute(s) only (Refer to clause 7.2.1.2 for response content description).

If the value of notificationContentType is set to "All Attributes", the Notify request primitive shall include the complete resource with all attributes (Refer to clause 7.2.1.2 for response content description).

If the value of notificationContentType is set to "ResourceID", the Notify request primitive shall include the URI of the resource (Refer to clause 7.2.1.2 for response content description).

If the value of notificationContentType is set to "Trigger Payload", the Notify request primitive shall include the trigger payload (Refer to clause 9.2.1 for trigger payload description).

If the value of notificationContentType is set to "TimeSeries notification", the Notify request primitive shall include a timeSeriesNotification (Refer to clause 6.3.5.69 for timeSeriesNotification description).

- In addition to the procedure described above, if the primitiveProfileID attribute of the <subscription> resource is configured with the resource identifier of a <primitiveProfile> resource, then the Originator shall apply the <primitiveProfile> resource to the request parameters of the Notify request primitive by adding, replacing or deleting any applicable request parameters defined in the additions or deletions attributes of the referenced <primitiveProfile> resource.

- If notificationContentType is set to "Modified Attributes" or "All Attributes", the Originator shall also apply the referenced <primitiveProfile> resource to the representation of the subscribed-to resource or the child resource of the subscribed-to resource that is included in the content of the notification, if applicable. Before doing this the Originator shall make the following checks:

- Check that the primitive profile’s resourceTypes and resourceIDs attributes match the subscribed-to resource or the child resource of the subscribed-to resource that is included in the content of the notification.

- Check that the primitive profile’s operations attribute includes the Notify operation and the releaseVersions attribute matches the release version indicator of the notification.

- Check that the applicability attribute includes the value “NOTIFICATIONS_FROM_CSE”.

- If all these checks are successful, the Originator shall apply the <primitiveProfile> resource to the resource attributes included in the content of the notification by adding, replacing or deleting resource attributes defined in the additions and deletions attribute of the <primitiveProfile> resource.

Step 2.2	Check the notificationEventCat attribute:

If the notificationEventCat attribute is set, the Notify request primitive shall employ the Event Category parameter as given in the notificationEventCat attribute. Then continue with the step 2.3.

If the notificationEventCat attribute is not configured, then continue with step 2.3.

Step 2.3	Check the latestNotify attribute:

If the latestNotify attribute is set, the Originator shall assign Event Category parameter of value "latest" of the notifications generated pertaining to the subscription created.

Step 2.4	Check the batching notifications policy and the rateLimit attribute:

See details in oneM2M TS-0001 [6], clause 10.2.10.7.

If both the batchNotify and primitiveProfileID attributes of the <subscription> resource are configured, the Originator shall attempt to apply the referenced <primitiveProfile> resource to the individual notifications embedded within an aggregated notification using the same procedure as described in Step 2.1. In addition, the Originator shall also apply the primitive profile to the request parameters of the aggregated notification request primitive by adding, replacing or deleting any applicable request parameters defined in the additions or deletions attributes of the referenced <primitiveProfile> resource

NOTE:	The use of some attributes such as preSubscriptionNotify is not supported in the present document.

Step 2.5	Check the notificationURI attribute:

The Originator shall fetch the notificationURI attribute and set the value to the To parameter of the Notify request. When the notificationURI attribute contains more than one target, the Originator shall generate each Notify request per target.

If the notificationURI attribute includes the notification serialization indication, in form of key-value pair, e.g. "ct=json", after the delimiter "?", the Originator shall serialize the notification for the notification target in that serialization type. The delimiter with the serialization indication shall be removed when the target is set to the To parameter of the Notify request. Then continue with step 3.0.

Step 3.0	The Originator shall check the notification and reachability schedules, but the notification schedules may be checked in different order:

If the <subscription> or <crossResourceSubscription> includes a <schedule> child resource named notificationSchedule, the Originator shall check the time periods given in the scheduleElement attribute of that <schedule> child resource.

Also, the Originator shall check the reachability schedule associated with the Receiver by exploring its <schedule> resource. If reachability schedules are not present in a Node then that Entity is considered to be always reachable.

If notificationSchedule and reachability schedule indicate that message transmission is allowed, then proceed with step 5.0. Otherwise, proceed with step 4.0.

In particular, if the notificationEventCat attribute is set to 'immediate' and the notificationSchedule resource does not allow transmission, then go to step 5.0 and send the corresponding Notify request primitive by temporarily ignoring the Originator's notification schedule.

Step 4.0	Check the pendingNotification attribute:

If the pendingNotification attribute is set, then the Originator shall cache pending Notify request primitives according to the pendingNotification attribute. The possible values are 'sendLatest' and 'sendAllPending'. If the value of pendingNotification is set to 'sendLatest', the most recent Notify request primitive shall be cached by the Originator and it shall set the Event Category parameter to "latest". If pendingNotification is set to 'sendAllPending', all Notify request primitives shall be cached by the Originator. If the pendingNotification attribute is not configured, the Originator shall discard the corresponding Notify request primitive. Any cached Notify request primitives are sent to the Receiver once message transmission becomes possible (see the step 6.0).

Step 5.0	Check the expirationCounter attribute:

If the expirationCounter attribute is set, then it shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter equals to zero('0'), the corresponding <subscription> resource shall be deleted. Then end the 'Compose Notify Request Primitive' procedure.

If the expirationCounter attribute is not configured, then end the 'Compose Notify Request Primitive' procedure.

When message transmission becomes possible, the Originator shall execute the following steps in order:

Step 6.0	If the pendingNotification attribute is set, the Originator shall send any cached Notify request primitives and then continue with the step 7.0

Step 7.0	Check the expirationCounter attribute:

If the expirationCounter attribute is set, then its value shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter meets zero, the corresponding <subscription> resource shall be deleted. Then end the 'Compose Notify Request Primitive' procedure.

If the expirationCounter attribute is not configured, then end the 'Compose Notify Request Primitive' procedure.

Receiver:

When the Hosting CSE receives a Notify request primitive, the Hosting CSE shall check validity of the primitive parameters. In case the Receiver is a transit CSE which forwards or aggregates Notify request primitives before sending to the subscriber or other transit CSEs, upon receiving the Notify request primitive with the Event Category parameter set to "latest", the Receiver shall identify the latest Notify request primitive with the same subscription reference while storing Notify request primitives locally. When the Receiver as a transit CSE needs to send pending Notify request primitives, it shall send the latest Notify request primitive. When the Receiver as a transit CSE needs to send Notify request primitives, it shall use one of the serializations specified in the subscriber or other transit CSE contentSerialization attribute. If there is no contentSerialization value specified the transit CSE may use any serialization format.

### -----------------------End of change 1---------------------------------------------