| HANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TDE 43.2 |
| Source:* | Bob Flynn (Convida wireless); bob.flynn@chordant.io |
| Date:* | 2020-01-28 |
| Reason for Change/s:* | MgmtObj Test Purposes |
| CR against: Release* | Rel-3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018 v3.2.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution attempts to create test purposes for Device Management operations in TS-0001/TS-0004.

This contribution addresses test purposes for [firmware].

Relevant text from TS-0001

## D.2	Resource firmware

The [firmware] resource is used to share information regarding the firmware on the device. The [firmware] resource is a specialization of the <mgmtObj>resource.

The [firmware] resource shall contain the child resources specified in table D.2-1.

Table D.2-1: Child resources of [firmware] resource

| Child Resources of [firmware] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 |

The [firmware] resource shall contain the attributes specified in table D.2-2.

Table D.2-2: Attributes of [firmware] resource

| Attributes of [firmware] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1(L) | RW | See clause 9.6.1.3. |
| mgmtDefinition | 1 | WO | See clause 9.6.15. Has fixed value "firmware" to indicate the resource is for firmware management. |
| objectIDs | 0..1 (L) | WO | See clause 9.6.15. |
| objectPaths | 0..1 (L) | WO | See clause 9.6.15. |
| description | 0..1 | RW | See clause 9.6.15. |
| version | 1 | RW | The version of the firmware. This attribute is a specialization of [objectAttribute] attribute. |
| name | 1 | RW | The name of the firmware to be used on the device. This attribute is a specialization of [objectAttribute] attribute. |
| URL | 1 | RW | The URL from which the firmware image can be downloaded. This attribute is a specialization of [objectAttribute] attribute. |
| update | 1 | RW | The action that downloads and installs a new firmware in a single operation. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| updateStatus | 1 | RW | Indicates the status of the update. This attribute is a specialization of [objectAttribute] attribute. |

Relevant text from TS-0004

# D.2	Resource [firmware]

## D.2.1	Introduction

The detailed description of the [firmware] resource can be found in clause D.2 of the oneM2M TS-0001 [6].

Table D.2.1-1: Data Type Definition of [firmware]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| firmware, firmwareAnnc | CDT-firmware-V3_15_0.xsd |  |

Table D.2.1-2: Resource specific attributes of [firmware]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 | 1001 (firmware) |
| objectIDs | O | NP | See clause 7.4.15 |  |
| objectPaths | O | NP | See clause 7.4.15 |  |
| description | O | O | See clause 7.4.15 |  |
| version | M | O | xs:string |  |
| firmwareName | M | O | xs:string |  |
| URL | M | O | xs:anyURI |  |
| update | M | O | xs:boolean |  |
| updateStatus | NP | O | m2m:actionStatus |  |

## D.2.2	Resource specific procedure on CRUD operations

### D.2.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in clause 7.2.2.

### D.2.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific step after generic procedure defined in clause 7.2.2.2.

May start to download the firmware image from the location indicated by attribute URL in the firmware resource.

### D.2.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operations to be performed after Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

If the attribute update is present in the request and set to true, download the new firmware image from the address indicated by attribute URL of the firmware resource if it is not already downloaded else use the downloaded firmware image to update the current using firmware. The Receiver may need to update the fwVersion attribute of the [deviceInfo] resource.

WRF – This clause describes “Reciever” actions. The “Reciever” is the Host CSE (by convention of TS-0004). This clause states that the Host CSE will check for “update = true”. The Hosting CSE will download the specified firmware and then update the firmware. The Hosting CSE will (may) then update the fwVersion of [deviceInfo].

All of this is supposed to be done by the device that this MO represents.  The current wording does not describe any concept of this device.

### D.2.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.2.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific step after generic procedure defined in clause 7.2.2.2:

Delete the downloaded firmware image locally.

WRF – A normal resource is “deleted” when this primitive is received. This description indicates that the Hosting CSE should delete a downloaded firmware image. This is not uninstalling a firmware image or De-Activating.

Isssues:

IUT can be a CSE or device (native oneM2M or nodn with IPE). But TS-0001/TS-0004 do not address the device. It addresses the CSE as a device (CSE as a managed device with DM client capabilities) – Is this intended?

It is not clear that the CSE has (or should have) any responsibilities beyond normal <resource> handling if the CSE is not the managed device.

These are some potential test purposes written for the “device”, even though this is not appropriate based on the existing specifications.

- When a [firmware] resource is created, the “device” should download the indicated firware version.

- When a [firmware] resource is updated, if “update=true” the device will download and install the FW.

- When a [firmware] resource is deleted the device should delete the specified “image”.

### -----------------------Start of new text 1-------------------------------------------

| TP Id |  | TP/oneM2M/AE/DM/FW/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT downloads a firmware image when a [firmware] mgmtObj is created. |  |
| Reference |  | TS-0004, clause D.2.2.1 |  |
| Parent Release |  | Release 2 |  |
| Config Id |  | CFG?? |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being registered to a CSE and the CSE having a <node> resource that represents the IUT at NODE_RESOURCE_ADDRESS and the AE1 being registered to a CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the CSE receives a valid CREATE request from AE1 containing To set to NODE_RESOURCE_ADDRESS and From set to AE1_ID and Resource Type set to 13 (mgmtObj) and Content containing a valid resource representation } |  | CSE AE1 |
|  | then { the IUT downloads a firmware image from the location specified in URL } |  | IUT Image repository |

| TP Id |  | TP/oneM2M/AE/DM/FW/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT installs a firmware image when a [firmware] mgmtObj is updated. |  |
| Reference |  | TS-0004, clause D.2.2.2 |  |
| Parent Release |  | Release 2 |  |
| Config Id |  | CFG?? |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being registered to a CSE and the CSE having a [firmware] resource at FW_RESOURCE_ADDRESS and the CSE having a [deviceinfo] resource at DEVICEINFO _RESOURCE_ADDRESS and the AE1 being registered to a CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the CSE receives a valid UPDATE request from AE1 containing To set to FW_RESOURCE_ADDRESS and From set to AE1_ID and Content containing a update attribute set to true and fwVersionset to NEW_FW_VERSION } |  | CSE AE1 |
|  | then { the IUT installs a firmware image and the IUT sends a valid UPDATE request To set to DEVICEINFO_RESOURCE_ADDRESS Content containing attribute fwVersion set to NEW_FW_VERSION } |  | IUT CSE |

| TP Id |  | TP/oneM2M/AE/DM/FW/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes a firmware image when a [firmware] mgmtObj is deleted. |  |
| Reference |  | TS-0004, clause D.2.2.2 |  |
| Parent Release |  | Release 2 |  |
| Config Id |  | CFG?? |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being registered to a CSE and the CSE having a [firmware] resource at FW_RESOURCE_ADDRESS and the AE1 being registered to a CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the CSE receives a valid DELETE request from AE1 containing To set to FW_RESOURCE_ADDRESS and From set to AE1_ID } |  | CSE AE1 |
|  | then { the IUT uninstalls a fwVersion firmware image } |  | IUT ??? |

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of new text 2-------------------------------------------

### -----------------------End of change 2-------------------------------------------