| HANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TDE 43.3 |
| Source:* | Bob Flynn (Convida wireless); bob.flynn@chordant.io |
| Date:* | 2020-02-04 |
| Reason for Change/s:* | HAIM Test Purposes |
| CR against: Release* | Rel-3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0013 v3.0.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution attempts to create new test objectives for the HAIM models defined in TS-0023.

Four test objectives are defined:

TD/M2M/NH/01 – Create HAIM Light Device Model

TD/M2M/NH/02 – Read status of a light device

TD/M2M/NH/03 – Turn the Light Device ON/OFF

TD/M2M/NH/04 – Toggle the Light Device

R01

Add reference. [X]

Nodn abbreviation, IPE, HAIM

### -----------------------Start of new text 1-------------------------------------------

#### 8.5 	HAIM Device Model

#### 8.5.1	HAIM Light Device Creation

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_01 |
| Objective: |  |  | AE1 creates a HAIM Light Device Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for binarySwitch |
| 7 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource/resource name of deviceLight fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 9 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |
| Note |  | Optional: Repeat steps 5-10 for additional deviceLight Modules |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.5.2	HAIM Light Device Status Read

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_02 |
| Objective: |  |  | AE2 reads the status of a HAIM Light Device Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27, 5.3.12 |
| Pre-test conditions: |  |  | AE2 has created an application resource <AE> on registrar CSE AE1 has created a HAIM Light Device model |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 sends a request to retrieve a <flexContainer> for binarySwitch |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <deviceLight> resource/binarySwitch fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 4 |  | IOP Check | AE2 indicates successful operation |
| Note |  | Optional: Repeat steps 1-4 for additional deviceLight Modules |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.5.3	HAIM Light Device Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_03 |
| Objective: |  |  | AE2 turns the binarySwitch of a HAIM Light Device Model “ON” or “OFF” |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27 |
| Pre-test conditions: |  |  | AE2 has created an application resource <AE> on registrar CSE AE1 has created a HAIM Light Device model |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/ URI of <deviceLight> resource/binarySwitch fr = AE-ID rqi = (token-string) pc = Serialized representation of <flexContainer> resource with new value for powerState |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is updates in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| Note |  | Optional: Repeat steps 1-5 for additional device states and settings |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.5.4	HAIM Light Device Toggle Action

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_04 |
| Objective: |  |  | AE2 toggles the state of a HAIM Light Device Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27 |
| Pre-test conditions: |  |  | AE2 has created an application resource <AE> on registrar CSE AE1 has created a HAIM Light Device model |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/ URI of <deviceLight> resource/binarySwitch/toggle fr = AE-ID rqi = (token-string) pc = Serialized representation of <flexContainer> resource for toggle action |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is updates in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE2 indicates successful operation. Check that the powerState of the binarySwitch is updated. |
| Note |  | Optional: Repeat steps 1-5 for additional device states and settings |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of new text 2-------------------------------------------

#### 7.1.1.3	M2M_CFG_10

oneM2M entities model

Note: For HAIM Model tests, clause 8.5, AE1 represents a native oneM2M device that implements the HAIM models or a combined IPE(AE) plus a non-oneM2M device node (nodn) .

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of new text 3-------------------------------------------

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0001: "Functional Architecture- Release 3".

[2]	oneM2M TS-0004 "Service Layer Core protocol Specification - Release 3".

[3]	oneM2M TS-0008: "CoAP Protocol Binding Release 3".

[4]	oneM2M TS-0009: "HTTP Protocol Binding - Release 3".

[5]	oneM2M TS-001: "MQTT Protocol Binding - Release 3".

[6]	oneM2M TS-0015: "Testing Framework".

[7]	oneM2M TS-0011: "Common Terminology".

[8]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[9]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[10]	oneM2M TS-0005: "Management Enablement (OMA) - Release 3".

[11]	oneM2M TS-0006: "Management Enablement (BBF) - Release 3".

[12]	oneM2M TS-0003: "Security Solutions - Release 3".

[13]	oneM2M TS-0034: "Semantics Support - Release 3".

[14]	oneM2M TS-0023: " Home Appliances Information Model and Mapping – Release 3".

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of new text 4-------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACP	Access Control Policy

AE	Application Entity

AE-ID	Application Entity Identifier

APP-ID	Application Identifier

BBF	BroadBand Forum

CFG	Configuration

CoAP	Constrained Application Protocol

CSE	Common Services Entity

CSE-ID	Common Service Entity Identifier

DM	Device Management

DTLS	Datagram Transport Layer Security

DUT	Device Under Test

FQDN	Fully Qualified Domain Name

HAIM	Home Appliances Information Model

HTTP	HyperText Transfer Protocol

IN	Infrastructure Node

IN-CSE	CSE which resides in the Infrastructure Node

IOP	Interoperability

IP	Internet Protocol

IPE	Interworking Proxy Entity

JSON	JavaScript Object Notation

LWM2M	Lightweight M2M

M2M	Machine to Machine

MA	Mandatory Announced

Mca	Reference Point for M2M Communication with AE

Mcc	Reference Point for M2M Communication with CSE

MH	Multi Hop

MO	Management Object

MQTT	Message Queuing Telemetry Transport

NB	Non-Blocking

NH	No Hop

NODN	Non oneM2M Device Node

OMA	Open Mobile Alliance

PRO	Protocol

PSK	Pre-Shared Key

RFC	Request for Comments

RP	Reference Point

RPC	Remote Procedure Calls

RQI	Request-ID

SE	Security

SH	Single Hop

SP	Service Provider

SUT	System Under Test

TCP	Transmission Control Protocol

TD	Test Description

TLS	Transport Layer Security

UDP	User Datagram Protocol

URI	Uniform Resource Identifier

XML	eXtensible Markup Language

### -----------------------End of change 4-------------------------------------------