| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0018-V-2.16.0 |
| Document Name: | Test Suite Structure and Test Purposes |
| Date: | 2020-04-21 |
| Abstract: | The Test Suite Structure and Test Purposes document for conformance testing consists of: - Defining the test suite structure by grouping the test purposes according to different criteria; - Specifing test purposes for conformance test. A test purpose is an informal description of the expected test behaviour. |
| Template Version:23 February 2015 (Do not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2019, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Conventions	7

5	Prerequisites and Test Configurations	7

5.1	Test Configurations	7

6	Test Suite Structure (TSS)	8

6.1	Test groups	8

6.1.0	Overview	8

6.1.1	Functional Entity Groups	8

6.1.2	Common Services Functions Subgroups	9

6.1.3	oneM2M Resource Primitives specific Operations	9

6.2	Test Suite Structure (TSS) for oneM2M	9

7	Test Purposes (TP)	10

7.1	Introduction	10

7.1.1	TP definition conventions	10

7.1.2	TP Identifier naming conventions	10

7.1.3	Rules for the behaviour description	11

7.1.4	ICS reference	11

7.2	Test Purposes for oneM2M Service Primitives	14

7.2.1	Group AE (AE)	14

7.2.1.1	General Capability (GEN)	14

7.2.1.1.1	CREATE Operation	14

7.2.1.1.2	UPDATE Operation	16

7.2.1.1.3	RETRIEVE Operation	18

7.2.1.1.4	DELETE Operation	19

7.2.1.2	Registration (REG)	21

7.2.1.2.1	CREATE Operation	21

7.2.1.2.2	DELETE Operation	22

7.2.1.3	Data Management and Repository (DMR)	23

7.2.1.3.1	CREATE Operation	23

7.2.1.3.2	UPDATE Operation	26

7.2.1.3.3	RETRIEVE Operation	28

7.2.1.3.4	DELETE Operation	30

7.2.1.4	Subscription and Notification (SUB)	32

7.2.1.4.1	CREATE Operation	32

7.2.1.4.2	NOTIFY Operation	34

7.2.1.5	Communication Management and Delivery Handling (CMDH)	35

7.2.1.5.1	Resource pollingChannel (PCH)	35

7.2.2	Group CSE (CE)	36

7.2.2.1	General Capability (GEN)	36

7.2.2.1.1	CREATE Operation	36

7.2.2.1.2	UPDATE Operation	40

7.2.2.1.3	RETRIEVE Operation	44

7.2.2.1.4	DELETE Operation	48

7.2.2.2	Registration (REG)	51

7.2.2.2.1	RETRIEVE Operation	51

7.2.2.2.2	CREATE Operation	56

7.2.2.2.3	DELETE Operation	78

7.2.2.2.4	UPDATE Operation	81

7.2.2.3	Data Management and Repository Function (DMR)	84

7.2.2.3.1	RETRIEVE Operation	84

7.2.2.3.2	UPDATE Operation	106

7.2.2.3.3	CREATE Operation	139

7.2.2.3.4	DELETE Operation	167

7.2.2.3.5	BASIC Operation	181

7.2.2.4	Subscription and Notification (SUB)	183

7.2.2.4.1	CREATE Operation	183

7.2.2.4.2	DELETE Operation	189

7.2.2.4.3	UPDATE Operation	192

7.2.2.4.4	NOTIFY Operation	201

7.2.2.5	Group Management (GMG)	209

7.2.2.5.1	CREATE Operation	209

7.2.2.5.2	UPDATE Operation	218

7.2.2.5.3	RETRIEVE Operation	232

7.2.2.5.4	BASIC OPERATION	233

7.2.2.6	Discovery (DIS)	241

7.2.2.6.1	RETRIEVE Operation	241

7.2.2.7	Communication Management and Delivery Handling(CMDH)	250

7.2.2.7.1	Resource pollingChannel (PCH)	250

7.2.2.7.2	Response Type (RT)	256

7.2.2.7.2.1	nonBlockingRequestSynch (NBS)	256

7.2.2.7.2.2	nonBlockingRequestAsynch (NBA)	259

7.2.2.7.2.3	BlockingRequest (BR)	264

7.2.2.8	Security(SEC)	265

7.2.2.8.1	CREATE Operation	265

7.2.2.8.2	UPDATE Operation	269

7.2.2.8.3	BASIC OPERATION	271

7.2.2.9	FlexContainer	285

7.2.2.9.1	CREATE Operation	285

7.2.2.9.2	UPDATE Operation	286

Annex A (informative):	Conformance Test Requirement	289

A.1	MQTT Protocol Conformance Test Requirement	289

Annex B (informative):	TP template	291

History	292

1	Scope

The present document specifies oneM2M test suite structure and test purposes that are designed to evaluate the conformity of oneM2M implementations to the oneM2M specifications. It also specifies:

guidelines for description of test behaviours and definition of test purpose structure;

notations to define a test purpose for conformance testing; and

test configurations for conformance testing.

2	References

2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0001: "Functional Architecture".

[2]	oneM2M TS-0004: "Service Layer Core Protocol Specification".

[3]	oneM2M TS-0015: "Testing Framework".

[4]	oneM2M TS-0017: "Implementation Conformance Statements".

[5]	oneM2M TS-0010: "MQTT-protocol-binding".

2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

3	Definition of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in oneM2M TS-0015 [3] Testing Framework and the following apply:

valid <OPERATION> Request: oneM2M Request primitive that only contains all mandatory primitive parameters with correct values which are not required to be explicitly specified in the test purpose

NOTE:	If specific values need to be specified, parameters need to be explicitly indicated in the test purpose. Operation parameter is set to <OPERATION>.

valid Response: oneM2M Response primitive that only contains all mandatory primitive parameters with correct values which are not required to be explicitly specified in the test purpose

NOTE:	If specific values need to be specified, parameters need to be explicitly indicated in the test purpose. Request Identifier parameter contains the Request Identifier value of the corresponding Request.

valid <ATTRIBUTE_NAME>: correct value for the <ATTRIBUTE_NAME> attribute which is not required to be explicitly specified in the test purpose

3.2	Symbols

Void.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in oneM2M TS-0001 [1] and the following apply:

ADN	Application Dedicated Node

ADN-AE	AE which resides in the Application Dedicated Node

AE	Application Entity

AE-ID	Application Entity-IDentifier

APP-ID	APPlication IDentifier

ASN	Application Service Node

BR	BlockingRequest

CMDH	Communication Management and Delivery Handling

CSE	Common Service Entity

CSE-ID	Common Service Entity Identifier

CSF	Common Service Function

DIS	DIScovery

DMR	Data Management and Repository

DUT	Device Under Test

FLXC	FLeXContainer

FQDN	Fully Qualified Domain Name

GEN	GENeral capability

GMG	Group ManaGement

ICS	Implementation Conformance Statements

IN	Infrastructure Node

IN-CSE	CSE which resides in the Infrastructure Node

IP	Internet Protocol

IUT	Implementation Under Test

MA	MAndatory

MN	Middle Node

MN-CSE	CSE which resides in the Middle Node

NA	Not Applicable

NBA	NonBlockingrequestAsynch

NBR	NonBlockingRequest

NBS	NonBlockingrequestSynch

PCH	Request Message Polling

PICS	Protocol Implementation Conformance Statements

PX_POA	Protocol implementation eXtra information for testing_ PointOfAccess

PX_SRT	Protocol implementation eXtra information for testing_ SupportedResourceType

REG	REGistration

REQ	REQuest

RT	Response Type

RW	Read Write

SCEF	Service Capability Exposure Function

SEC	SECurity

SUB	SUBscription and Notification

SUT	System Under Test

TP	Test Purposes

TS	Technical Specification

TSS	Test Suite Structure

TTCN	Testing and Test Control Notation

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

5	Prerequisites and Test Configurations

5.1	Test Configurations

Test configurations are defined to test different entities such as CSE and AE etc.

Figure 5.1-1 and Figure 5.1-2 show CSE test configurations where CSE as a IUT can be tested against TTCN-3 Test System, which is acting as AE or AE and CSE respectively. Test system may include as well other entities which are necessary for each configuration, i.e. a second AE (AE2) for receiving notifications These entities are represented with dashed rectangles.

Figure 5.1-3 shows AE test configuration (CF03) for ADN-AE testing scenario where ADN-AE is IUT and TTCN-3 Test System is acting as a CSE.

Figure 5.1-4 depicts test configurations between two CSEs, where one CSE is acting as a Test System, the other is SUT.

Figure 5.1-1: Test configuration 1 (CF01)

Figure 5.1-2: Test configuration 2 (CF02)

Figure 5.1-3: Test configuration 3 (CF03)

Figure 5.1-4: Test configuration 4 (CF04)

6	Test Suite Structure (TSS)

6.1	Test groups

6.1.0	Overview

The test groups are organized in 2 levels. The first level defines the functional entity of oneM2M. The second level defines the common services functions which are represented in oneM2M functional architecture.

6.1.1	Functional Entity Groups

The functional entity groups identify the types of entity which are specified in oneM2M functional architecture (oneM2M TS-0001 [1]):

AE

CSE

6.1.2	Common Services Functions Subgroups

The common services functions subgroups identify the general procedures in each common services functions specified in oneM2M functional architecture (oneM2M TS-0001 [1]):

Registration

Data Management and Repository

Subscription and Notification

Group Management

Discovery

Location

Device Management

Communication Management and Delivery Handling

Security

6.1.3	oneM2M Resource Primitives specific Operations

A group of resource type specific procedures are defined in oneM2M functional architecture (oneM2M TS-0001 [1]) and service layer core protocol (oneM2M TS-0004 [2]). Those procedures can be potentially abstracted with requirements for generating test purposes for conformance testing purpose. Those resource type specific procedures are the complementary procedures to the general procedures defined in oneM2M Common Service Functions (CSF) and will use for subgroups of structures for oneM2M tests. A list of those procedures are including:

pollingChannel

non-blocking communication

retargeting

FlexContainer

6.2	Test Suite Structure (TSS) for oneM2M

The Test Suite Structure (TSS) for oneM2M will be present as below:

Group 1: General Capability (GEN):

Subgroup 1.1:	Protocol Bindings

Subgroup 1.2:	Serializations

Group 2: AE(AE):

Subgroup 2.1:	General Capability (GEN)

Subgroup 2.2:	Registration(REG)

Subgroup 2.3:	Data Management and Repository(DMR)

Subgroup 2.4:	Subscription and Notification(SUB)

Subgroup 2.5:	Communication Management and Delivery Handling(CMDH)

Subgroup 2.5.1:	Request Message Polling(PCH)

Group 3: CSE(CE):

Subgroup 3.1:	General Capability (GEN)

Subgroup 3.2:	Registration(REG)

Subgroup 3.3:	Data Management and Repository(DMR)

Subgroup 3.4:	Subscription and Notification(SUB)

Subgroup 3.5:	Group Management(GMG)

Subgroup 3.6:	Discovery(DIS)

Subgroup 3.7:	Communication Management and Delivery Handling(CMDH)

Subgroup 3.7.1:	Request Message Polling(PCH)

Subgroup 3.7.2:	Response Type(RT)

Subgroup 3.8:	Security(SEC)

Subgroup 3.9:	FlexContainer (FLXC)

7	Test Purposes (TP)

7.1	Introduction

7.1.1	TP definition conventions

The TP definition is constructed according to Test Suite Structure & Test Purposes(TSS&TP) in Testing Framework (oneM2M TS-0015 [3]).

7.1.2	TP Identifier naming conventions

The identifier of the TP is constructed according to table 7.1.2-1.

Table 7.1.2-1: TP naming convention for oneM2M

| Name | Option | Interpretation |
| --- | --- | --- |
| TP/<root>/<ent>/<gr>[/<sgr>][/<opr>]/<nnn>_<pmu> |  |  |
| <root> = root | oneM2M | oneM2M |
| <ent> = entity | AE | Application Entity |
|  | CSE | Common Services Entity |
| <gr> = group | GEN | Common Service Function |
|  | REG |  |
|  | DMR |  |
|  | SUB |  |
|  | GMG |  |
|  | DIS |  |
|  | SEC |  |
|  | FLXC |  |
| <sgr> = sub-group | ACP | AccessControlPolicy |
| <opr> = operation | CRE | Create |
|  | UPD | Update |
|  | RET | Retrieve |
|  | DEL | Delete |
|  | NTF | Notify |
| <nnn> = sequential number | 001 to 999 | 023 |
| <pmu> = permutation part | May be number, short name of attribute or resource type depend on the permutated part in the TP. If there are more than one permutated variables, they should be separated by slash '/'. | Enumerated type attribute and its value is represented as <VARIABLE_1>/<VALUE>, e.g. TP/oneM2M/CSE/DMR/RET/020_RCN/0 When resourceType attribute is involved in a TP Id, the resourceType shall be put in first order followed by any other attribute(s), if any, <RESOURCE_TYPE>/<VARIABLE_1>/<VARIABLE_2> e.g. TP/oneM2M/CSE/DMR/CRE/015_CNT/ACPI |

7.1.3	Rules for the behaviour description

The description of the TP is built according to oneM2M TS-0015 [3] Testing Framework.

In addition, the following rules apply:

Primitive parameters shall use their Parameter name as indicated in oneM2M TS-0004 [2] table 8.2.2-1.

Resource attributes shall use their Attribute name as indicated in oneM2M TS-0004 [2] table 8.2.3 followed by "attribute".

When Content request parameter contains a resource representation, such representation shall contain all mandatory attributes and could contain some optional attributes. Only those attributes and/or child resources required by the test purpose will be explicitly indicated.

When Content response parameter contains a resource representation, such representation shall contain attributes and/or child resources according to the Result Content parameter of the corresponding request. Only those attributes and/or child resources required by the test purpose will be explicitly indicated.

7.1.4	ICS reference

Table 7.1.4-1: Node

| Item | Name of field | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | ASN (Application Service Node) | 5.1.2 | o.1 |  |
| 2 | ADN (Application Dedicated Node) | 5.1.2 | o.1 |  |
| 3 | IN (Infrastructure Node) | 5.1.2 | o.1 |  |
| 4 | MN (Middle Node) | 5.1.2 | o.1 |  |
| o.1: Exactly one item shall be supported |  |  |  |  |

Table 7.1.4-2: Functional entity type

| Prerequisite: A.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Name of field | Reference | Status | Support |
| 1 | AE | 5.1.1 | c.1 |  |
| 2 | CSE | 5.1.1 | c.2 |  |
| c.1: If A.1-1 or A.1-2 then m else o c.2: If A.1-2 x else m |  |  |  |  |

Table 7.1.4-3: Mnemonics for PICS reference

| Mnemonic |  | PICS item |  |  |
| --- | --- | --- | --- | --- |
| PICS_AE |  | TS-0017 [4], clause A.5.1.1/1 |  |  |
| PICS_CSE |  | TS-0017 [4], clause A.5.1.1/2 |  |  |
| PICS_ASN_CSE |  | TS-0017 [4], clause A.5.1.2/1 |  |  |
| PICS_ADN |  | TS-0017 [4], clause A.5.1.2/7 |  |  |
| PICS_IN_CSE |  | TS-0017 [4], clause A.5.1.2/3 |  |  |
| PICS_MN_CSE |  | TS-0017 [4], clause A.5.1.2/2 |  |  |
| PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  | TS-0017 [4], clause A.5.2.1/2 |  |  |
| PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  | TS-0017 [4], clause A.5.2.1/1 |  |  |
| PICS_SP_RELATIVE_RESOURCE_ID |  | TS-0017 [4], clause A.5.2.1 |  |  |
| PICS_ABSOLUTE_RESOURCE_ID |  | TS-0017 [4], clause A.5.2.1 |  |  |
| PICS_ACP_SUPPORT |  | TS-0017 [4], clause A.5.3.1/10 |  |  |
| PICS_CB_CST | TS-0017 [4], clause A.5.7.1/3 |  |  |
| PICS_CB_NL | TS-0017 [4], clause A.5.7.1/4 |  |  |
| PICS_CSR_RN | TS-0017 [4], clause A.5.7.2/10 |  |  |
| PICS_CSR_ET | TS-0017 [4], clause A.5.7.2/11 |  |  |
| PICS_CSR_LBL | TS-0017 [4], clause A.5.7.2/2 |  |  |
| PICS_CSR_POA | TS-0017 [4], clause A.5.7.2/6 |  |  |
| PICS_CSR_NL | TS-0017 [4], clause A.5.7.2/9 |  |  |
| PICS_CSR_CST | TS-0017 [4], clause A.5.7.2/5 |  |  |
| PICS_CSR_RR | TS-0017 [4], clause A.5.7.2/12 |  |  |
| PICS_AE_LBL | TS-0017 [4], clause A.5.7.3/2 |  |  |
| PICS_AE_APN | TS-0017 [4], clause A.5.7.3/5 |  |  |
| PICS_AE_POA | TS-0017 [4], clause A.5.7.3/6 |  |  |
| PICS_AE_NL | TS-0017 [4], clause A.5.7.3/8 |  |  |
| PICS_AE_CSZ | TS-0017 [4], clause A.5.7.3/9 |  |  |
| PICS_CNT_ACPI | TS-0017 [4], clause A.5.7.5/1 |  |  |
| PICS_CNT_MNI | TS-0017 [4], clause A.5.7.5/6 |  |  |
| PICS_CNT_MBS | TS-0017 [4], clause A.5.7.5/7 |  |  |
| PICS_CNT_MIA | TS-0017 [4], clause A.5.7.5/8 |  |  |
| PICS_CNT_OR | TS-0017 [4], clause A.5.7.5/10 |  |  |
| PICS_CNT_LI | TS-0017 [4], clause A.5.7.5/9 |  |  |
| PICS_CNT_RN | TS-0017 [4], clause A.5.7.5/11 |  |  |
| PICS_CNT_ET | TS-0017 [4], clause A.5.7.5/12 |  |  |
| PICS_CNT_LBL | TS-0017 [4], clause A.5.7.5/2 |  |  |
| PICS_CNT_CR | TS-0017 [4], clause A.5.7.5/5 |  |  |
| Mnemonic |  |  | PICS item |
| PICS_CIN_CNF |  |  | TS-0017 [4], clause A.5.7.6/5 |
| PICS_CIN_RN |  |  | TS-0017 [4], clause A.5.7.6/7 |
| PICS_CIN_ET |  |  | TS-0017 [4], clause A.5.7.6/8 |
| PICS_CIN_LBL |  |  | TS-0017 [4], clause A.5.7.6/1 |
| PICS_CIN_CR |  |  | TS-0017 [4], clause A.5.7.6/4 |
| PICS_ACP_LBL |  |  | TS-0017 [4], clause A.5.7.4/1 |
| PICS_SUB_ACPI |  |  | TS-0017 [4], clause A.5.7.7/2 |
| PICS_SUB_LBL |  |  | TS-0017 [4], clause A.5.7.7/1 |
| PICS_SUB_ENC |  |  | TS-0017 [4], clause A.5.7.7/3 |
| PICS_SUB_EXC |  |  | TS-0017 [4], clause A.5.7.7/4 |
| PICS_SUB_GPI |  |  | TS-0017 [4], clause A.5.7.7/5 |
| PICS_SUB_NFU |  |  | TS-0017 [4], clause A.5.7.7/6 |
| PICS_SUB_BN |  |  | TS-0017 [4], clause A.5.7.7/7 |
| PICS_SUB_RL |  |  | TS-0017 [4], clause A.5.7.7/8 |
| PICS_SUB_PN |  |  | TS-0017 [4], clause A.5.7.7/10 |
| PICS_SUB_NSP |  |  | TS-0017 [4], clause A.5.7.7/11 |
| PICS_SUB_LN |  |  | TS-0017 [4], clause A.5.7.7/12 |
| PICS_SUB_NCT |  |  | TS-0017 [4], clause A.5.7.7/13 |
| PICS_SUB_NEC |  |  | TS-0017 [4], clause A.5.7.7/14 |
| PICS_SUB_CR |  |  | TS-0017 [4], clause A.5.7.7/15 |
| PICS_SUB_SU |  |  | TS-0017 [4], clause A.5.7.7/16 |
| PICS_GRP_LBL |  |  | TS-0017 [4], clause A.5.7.8/2 |
| PICS_GRP_ACPI |  |  | TS-0017 [4], clause A.5.7.8/1 |
| PICS_GRP_MACP |  |  | TS-0017 [4], clause A.5.7.8/6 |
| PICS_GRP_GN |  |  | TS-0017 [4], clause A.5.7.8/9 |
| Mnemonic |  |  | PICS item |
| PICS_AE |  |  | TS-0017 [4], clause A.5.1.1/1 |
| PICS_CSE |  |  | TS-0017 [4], clause A.5.1.1/2 |
| PICS_ASN_CSE |  |  | TS-0017 [4], clause A.5.1.2/1 |
| PICS_ADN |  |  | TS-0017 [4], clause A.5.1.2/7 |
| PICS_IN_CSE |  |  | TS-0017 [4], clause A.5.1.2/3 |
| PICS_MN_CSE |  |  | TS-0017 [4], clause A.5.1.2/2 |

7.2	Test Purposes for oneM2M Service Primitives

7.2.1	Group AE (AE)

7.2.1.1	General Capability (GEN)

7.2.1.1.1	CREATE Operation

TP/oneM2M/AE/GEN/CRE/001

| TP Id |  | TP/oneM2M/AE/GEN/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the creation of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS } |  |  |
| ted behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid Container CREATE Request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | CSE IUT |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/CRE/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/CRE/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/CRE/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/AE/GEN/CRE/002

| TP Id |  | TP/oneM2M/AE/GEN/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the creation of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to HIERARCHICAL_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | CSE IUT |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/CRE/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/CRE/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/CRE/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

7.2.1.1.2	UPDATE Operation

TP/oneM2M/AE/GEN/UPD/001

| TP Id |  | TP/oneM2M/AE/GEN/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the update of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing container resource containing valid labels attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT CSE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/UPD/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/UPD/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/UPD/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/AE/GEN/UPD/002

| TP Id |  | TP/oneM2M/AE/GEN/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the update of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing container resource containing valid labels attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing Container resource containing valid labels attribute } |  | IUT CSE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/UPD/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/UPD/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/UPD/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

7.2.1.1.3	RETRIEVE Operation

TP/oneM2M/AE/GEN/RET/001

| TP Id |  | TP/oneM2M/AE/GEN/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the retrieval of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/RET/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/RET/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/RET/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/AE/GEN/RET/002

| TP Id |  | TP/oneM2M/AE/GEN/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the retrieval of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/RET/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/RET/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/RET/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

7.2.1.1.4	DELETE Operation

TP/oneM2M/AE/GEN/DEL/001

| TP Id |  | TP/oneM2M/AE/GEN/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the deletion of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform DELETE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid DELETE request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/DEL/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/DEL/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/DEL/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/AE/GEN/DEL/002

| TP Id |  | TP/oneM2M/AE/GEN/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the deletion of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform DELETE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid DELETE request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/DEL/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/DEL/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/DEL/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

7.2.1.2	Registration (REG)

7.2.1.2.1	CREATE Operation

TP/oneM2M/AE/REG/CRE/001

| TP Id | TP/oneM2M/AE/REG/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an AE initial registration request with no AE-ID-STEM provided when it is started |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case C, and clause 9.6.19 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUT having got a valid APP-ID } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSE_RESOURCE_ADDRESS and Resource Type set to 2 (AE) and Content containing AE resource representation } | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing Resource Type set to 2 (AE) and To set to CSE_RESOURCE_ADDRESS and From set to empty and Content containing AE resource representation } | CSE IUT |

TP/oneM2M/AE/REG/CRE/002

| TP Id |  | TP/oneM2M/AE/REG/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a registration CREATE Request with the value of the attribute ATTRIBUTE_NAME of the AE resource |  |
| Reference |  | TS-0004 [2], clause 7.4.5.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUT having got a valid APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 2 (AE) and Content containing AE resource containing ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing AE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT CSE |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/REG/CRE/002_RN | TS-0004 [2], clause 7.4.5.1 | resourceName |
| TP/oneM2M/AE/REG/CRE/002_ET | TS-0004 [2], clause 7.4.5.1 | expirationTime |
| TP/oneM2M/AE/REG/CRE/002_LBL | TS-0004 [2], clause 7.4.5.1 | labels |
| TP/oneM2M/AE/REG/CRE/002_APN | TS-0004 [2], clause 7.4.5.1 | appName |
| TP/oneM2M/AE/REG/CRE/002_API | TS-0004 [2], clause 7.4.5.1 | App-ID |
| TP/oneM2M/AE/REG/CRE/002_POA | TS-0004 [2], clause 7.4.5.1 | pointOfAccess |
| TP/oneM2M/AE/REG/CRE/002_OR | TS-0004 [2], clause 7.4.5.1 | ontologyRef |
| TP/oneM2M/AE/REG/CRE/002_NL | TS-0004 [2], clause 7.4.5.1 | nodeLink |
| TP/oneM2M/AE/REG/CRE/002_RR | TS-0004 [2], clause 7.4.5.1 | requestReachability |
| TP/oneM2M/AE/REG/CRE/002_CSZ | TS-0004 [2], clause 7.4.5.1 | contentSerialization |

7.2.1.2.2	DELETE Operation

TP/oneM2M/AE/REG/DEL/001

| TP Id | TP/oneM2M/AE/REG/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends AE deregistration request to CSE |  |
| Reference | TS-0001 [1], clause 10.1.4.2.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being in the "initial state" the IUT having registered to CSE and the IUT having privileges to perform DELETE operation on the resource AE to CSE } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to AE_RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } | CSE IUT |

7.2.1.3	Data Management and Repository (DMR)

7.2.1.3.1	CREATE Operation

TP/oneM2M/AE/DMR/CRE/001

| TP Id |  | TP/oneM2M/AE/DMR/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Container creation request when it is triggered |  |
| Reference |  | TS-0001 [1], clauses 10.1.1.1 and 10.2.4.1, TS-0004 [2], clauses 7.2.2.1 and 7.4.6.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | CSE IUT |

TP/oneM2M/AE/DMR/CRE/002

| TP Id |  | TP/oneM2M/AE/DMR/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a ContentInstance creation request when it is triggered |  |
| Reference |  | TS-0001 [1], clauses 10.1.1.1 and 10.2.4.1, TS-0004 [2], clauses 7.2.2.1 and 7.4.7.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and Content containing contentInstance resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and From set to AE_ID and Content containing contentInstance resource representation } |  | CSE IUT |

TP/oneM2M/AE/DMR/CRE/003

| TP Id |  | TP/oneM2M/AE/DMR/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a ContentInstance creation request with optional attribute ATTRIBUTE_NAME |  |
| Reference |  | TS-0001 [1], clauses 10.1.1.1 and 10.2.4.1, TS-0004 [2], clauses 7.2.2.1 and 7.4.7.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS through preconfiguration request and the IUT having privileges to perform CREATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and Content containing ContentInstance resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and From set to AE_ID and Content containing ContentInstance resource containing valid ATTRIBUTE_NAME attribute } |  | CSE IUT |

| TP Id | PICS Selection | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/CRE/003_CNF | PICS_CIN_CNF | contentInfo |
| TP/oneM2M/AE/DMR/CRE/003_RN | PICS_CIN_RN | resourceName |
| TP/oneM2M/AE/DMR/CRE/003_ET | PICS_CIN_ET | expirationTime |
| TP/oneM2M/AE/DMR/CRE/003_LBL | PICS_CIN_LBL | labels |
| TP/oneM2M/AE/DMR/CRE/003_CR | PICS_CIN_CR | creator |

TP/oneM2M/AE/DMR/CRE/004

| TP Id |  | TP/oneM2M/AE/DMR/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Container creation request with optional attribute ATTRIBUTE_NAME when it is triggered |  |
| Reference |  | TS-0001 [1], clauses 10.1.1.1 and 10.2.4.1, TS-0004 [2], clauses 7.2.2.1 and 7.4.6.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | CSE IUT |

| TP Id | PICS Selection | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/CRE/004_ACPI | PICS_CNT_ACPI | accessControlPolicyIDs |
| TP/oneM2M/AE/DMR/CRE/004_MNI | PICS_CNT_MNI | maxNrOfInstances |
| TP/oneM2M/AE/DMR/CRE/004_MBS | PICS_CNT_MBS | maxByteSize |
| TP/oneM2M/AE/DMR/CRE/004_MIA | PICS_CNT_MIA | maxInstanceAge |
| TP/oneM2M/AE/DMR/CRE/004_OR | PICS_CNT_OR | ontologyRef |
| TP/oneM2M/AE/DMR/CRE/004_LI | PICS_CNT_LI | locationID |
| TP/oneM2M/AE/DMR/CRE/004_RN | PICS_CNT_RN | resourceName |
| TP/oneM2M/AE/DMR/CRE/004_ET | PICS_CNT_ET | expirationTime |
| TP/oneM2M/AE/DMR/CRE/004_LBL | PICS_CNT_LBL | labels |
| TP/oneM2M/AE/DMR/CRE/004_CR | PICS_CNT_CR | creator |

7.2.1.3.2	UPDATE Operation

TP/oneM2M/AE/DMR/UPD/001

| TP Id |  | TP/oneM2M/AE/DMR/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the AE resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered containing a RW attribute ATTRIBUTE_NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and Content containing AE resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing AE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT CSE |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/001_ET | TS-0004 [2], clause 7.4.5.1 | expirationTime |
| TP/oneM2M/AE/DMR/UPD/001_LBL | TS-0004 [2], clause 7.4.5.1 | labels |
| TP/oneM2M/AE/DMR/UPD/001_APN | TS-0004 [2], clause 7.4.5.1 | appName |
| TP/oneM2M/AE/DMR/UPD/001_POA | TS-0004 [2], clause 7.4.5.1 | pointOfAccess |
| TP/oneM2M/AE/DMR/UPD/001_OR | TS-0004 [2], clause 7.4.5.1 | ontologyRef |
| TP/oneM2M/AE/DMR/UPD/001_NL | TS-0004 [2], clause 7.4.5.1 | nodeLink |
| TP/oneM2M/AE/DMR/UPD/001_RR | TS-0004 [2], clause 7.4.5.1 | requestReachability |
| TP/oneM2M/AE/DMR/UPD/001_CSZ | TS-0004 [2], clause 7.4.5.1 | contentSerialization |

TP/oneM2M/AE/DMR/UPD/002

| TP Id |  | TP/oneM2M/AE/DMR/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the <container> resource |  |
| Reference |  | TS-0004 [2], clause 7.4.6.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered containing the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | IUT CSE |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/002_ET | TS-0004 [2], clause 7.4.6.2.3 | expirationTime |
| TP/oneM2M/AE/DMR/UPD/002_LBL | TS-0004 [2], clause 7.4.6.2.3 | labels |
| TP/oneM2M/AE/DMR/UPD/002_MNI | TS-0004 [2], clause 7.4.6.2.3 | maxNrOfInstances |
| TP/oneM2M/AE/DMR/UPD/002_MBS | TS-0004 [2], clause 7.4.6.2.3 | maxByteSize |
| TP/oneM2M/AE/DMR/UPD/002_MIA | TS-0004 [2], clause 7.4.6.2.3 | maxInstanceAge |

7.2.1.3.3	RETRIEVE Operation

TP/oneM2M/AE/DMR/RET/001

| TP Id |  | TP/oneM2M/AE/DMR/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE Request on the TARGET_RESOURCE_ADDRESS to CSE |  |
| Reference |  | TS-0001 [1], clause 10.1.2, TS-0004 [2], clause 7.2.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the CSE having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the IUT having privileges to perform RETRIEVE operation on resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to TARGET_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/RET/001_CB | TS-0001 [1], clause 10.2.3.2 | 5 (CSEBase) |
| TP/oneM2M/AE/DMR/RET/001_AE | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/AE/DMR/RET/001_CNT | TS-0001 [1], clause 10.2.4.2 | 3 (Container) |

TP/oneM2M/AE/DMR/RET/002

| TP Id |  | TP/oneM2M/AE/DMR/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE Request of oldest virtual resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.23.1, TS-0004 [2], clause 7.4.28.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest and From set to AE_ID } |  | IUT CSE |

TP/oneM2M/AE/DMR/RET/003

| TP Id |  | TP/oneM2M/AE/DMR/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE Request of latest virtual resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.22.1, TS-0004 [2], clause 7.4.27.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/latest } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/latest and From set to AE_ID } |  | IUT CSE |

TP/oneM2M/AE/DMR/RET/004

| TP Id |  | TP/oneM2M/AE/DMR/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a <container> resource RETRIEVE Request to CSE |  |
| Reference |  | TS-0004 [2], clause 7.4.6.2.2 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on container resource } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

TP/oneM2M/AE/DMR/RET/005

| TP Id |  | TP/oneM2M/AE/DMR/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a <contentInstance> resource RETRIEVE Request to CSE |  |
| Reference |  | TS-0004 [2], clause 7.4.7.2.2 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT CSE |

7.2.1.3.4	DELETE Operation

TP/oneM2M/AE/DMR/DEL/001

| TP Id |  | TP/oneM2M/AE/DMR/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a DELETE Request of oldest virtual resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.23.2, TS-0004 [2], clause 7.4.28.2.5 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest } |  | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest and From set to AE_ID } |  | IUT CSE |

TP/oneM2M/AE/DMR/DEL/002

| TP Id |  | TP/oneM2M/AE/DMR/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a DELETE Request of latest virtual resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.22.2, TS-0004 [2], clause 7.4.27.2.5 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest } |  | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/latest and From set to AE_ID } |  | IUT CSE |

TP/oneM2M/AE/DMR/DEL/003

| TP Id | TP/oneM2M/AE/DMR/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <container> resource DELETE request to CSE |  |
| Reference | TS-0004 [2], clause 7.4.6.2.4 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and and the IUT being switched on and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform DELETE operation on container resource } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } | CSE IUT |

TP/oneM2M/AE/DMR/DEL/004

| TP Id | TP/oneM2M/AE/DMR/DEL/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <contentInstance> resource DELETE request to CSE |  |
| Reference | TS-0004 [2], clause 7.4.7.2.4 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS and From set to AE_ID } | CSE IUT |

7.2.1.4	Subscription and Notification (SUB)

7.2.1.4.1	CREATE Operation

TP/oneM2M/AE/SUB/CRE/001

| TP Id |  | TP/oneM2M/AE/SUB/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a subscription creation request |  |
| Reference |  | TS-0001 [1], clauses 10.1.1.1 and 10.2.4.1, TS-0004 [2], clauses 7.2.2.1 and 7.4.8.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT being switched off and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing Resource Type set to 23 (subscription) and To set to AE_RESOURCE_ADDRESS and Content containing subscription resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing Resource Type set to 23 (subscription) and To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing subscription resource representation } |  | CSE IUT |

TP/oneM2M/AE/SUB/CRE/002

| TP Id |  | TP/oneM2M/AE/SUB/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a subscription creation request with optional attribute ATTRIBUTE_NAME when it is triggered |  |
| Reference |  | TS-0001 [1], clauses 10.1.1.1 and 10.2.11.2, TS-0004 [2], clauses 7.2.2.1 and 7.4.8.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and Content containing subscription resource containing notificationURI attribute and valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE_ID and Content containing subscription resource containing notificationURI attribute and valid ATTRIBUTE_NAME attribute } |  | CSE IUT |

| TP Id | PICS Selection | Reference | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/AE/SUB/CRE/002_ACPI | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | accessControlPolicyIDs |
| TP/oneM2M/AE/SUB/CRE/002_ENC | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | eventNotificationCriteria |
| TP/oneM2M/AE/SUB/CRE/002_EXC | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | expirationCounter |
| TP/oneM2M/AE/SUB/CRE/002_NFU | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | notificationForwardingURI |
| TP/oneM2M/AE/SUB/CRE/002_NCT | PICS_SUB_NCT | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | notificationContentType |
| TP/oneM2M/AE/SUB/CRE/002_NEC | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | notificationEventCat |
| TP/oneM2M/AE/SUB/CRE/002_SU | PICS_SUB_SU | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8.1 | subscriberURI |

7.2.1.4.2	NOTIFY Operation

TP/oneM2M/AE/SUB/NTF/001

| TP Id |  | TP/oneM2M/AE/SUB/NTF/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify Response to the hosting CSE when receiving a Notify request containing a single notification |  |
| Reference |  | TS-0001 [1], clause 6.1.12, TS-0004 [2], clause 7.5.1.2 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT having been registered and the IUT having created subscription resource under the CSE and the IUT being reachable through a URL ACCESSIBLE_URL } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a NOTIFY Request from hosting CSE containing Content containing notification message representation } |  | IUT CSE |
|  | then { the IUT sends a valid NOTIFY Response to the hosting CSE containing Response Status Code set to RESPONSE_STATUS_CODE } |  | CSE IUT |

TP/oneM2M/AE/SUB/NTF/002

| TP Id |  | TP/oneM2M/AE/SUB/NTF/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify Response to the hosting CSE when receiving a Notify request containing aggregated notifications |  |
| Reference |  | TS-0001 [1], clause 6.2.12, TS-0004 [2], clause 7.5.1.2 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT having been registered and the IUT having created subscription resource under the CSE and the IUT being reachable through a URL ACCESSIBLE_URL } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a NOTIFY Request from hosting CSE containing Content containing aggregatedNotification message representation } |  | IUT CSE |
|  | then { the IUT sends a valid Response to the hosting CSE containing Response Status Code set to RESPONSE_STATUS_CODE } |  | CSE IUT |

7.2.1.5	Communication Management and Delivery Handling (CMDH)

7.2.1.5.1	Resource pollingChannel (PCH)

TP/oneM2M/AE/PCH/001

| TP Id | TP/oneM2M/AE/PCH/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which performs polling sends the Notify request to <pollingChannelURI> Hosting CSE after receiving response using polling channel |  |
| Reference | TS-0001 [1], clause 10.2.13.8 TS-0004 [2], clause 7.4.22.2.5 |  |
| Config Id | CF03 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to a CSE1 and the IUT having a <pollingChannel> resource and the IUT having sent a retrieve Request to the CSE To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and the CSE having received the Request from Originator containing To set to AE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a polling Response from the CSE containing Response Status Code set to 2000 (OK) } | IUT CSE |
|  | then { the IUT sends a NOTIFY Request to the CSE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and Content containing Response Status Code set to RESPONSE_STATUS_CODE } | IUT CSE |
| NOTE: Based on figure 10.2.13.1-1 in TS-0001 [1] step 004 and 005. |  |  |

TP/oneM2M/AE/PCH/002

| TP Id | TP/oneM2M/AE/PCH/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an RETRIEVE Request to <pollingChannelURI> resource See the Note below. |  |
| Reference | TS-0001 [1], clause 10.2.13.6, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the AE having a child <pollingChannel> resource } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to POLLINGCHANNELURI _RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to POLLINGCHANNELURI _RESOURCE_ADDRESS From set to AE_ID } | IUT CSE |
| NOTE: Based on figure 10.2.5.12-1 in TS-0001 [1] step 001. |  |  |

7.2.2	Group CSE (CE)

7.2.2.1	General Capability (GEN)

7.2.2.1.1	CREATE Operation

TP/oneM2M/CSE/GEN/CRE/001

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/CRE/002

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/CRE/003

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using shortcut structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/003_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/CRE/004

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using structured resource identifier with hybrid addressing. |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to HYBRID_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |
| TP/oneM2M/CSE/GEN/CRE/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus “/fopt”. |  |

7.2.2.1.2	UPDATE Operation

TP/oneM2M/CSE/GEN/UPD/001

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/UPD/002

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/UPD/003

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using shortcut structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/003_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/UPD/004

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using structured resource identifier with hybrid addressing. |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to HYBRID_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |
| TP/oneM2M/CSE/GEN/UPD/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus “/fopt”. |  |

7.2.2.1.3	RETRIEVE Operation

TP/oneM2M/CSE/GEN/RET/001

| TP Id |  | TP/oneM2M/CSE/GEN/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content containing container resource representation } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/RET/002

| TP Id |  | TP/oneM2M/CSE/GEN/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content containing container resource representation } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/RET/003

| TP Id |  | TP/oneM2M/CSE/GEN/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using shortcut structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content containing container resource representation } |  | IUT  AE |

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/003_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/RET/004

| TP Id |  | TP/oneM2M/CSE/GEN/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using structured resource identifier with hybrid addressing. |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to HYBRID_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content containing container resource representation } |  | IUT  AE |

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |
| TP/oneM2M/CSE/GEN/RET/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus “/fopt”. |  |

7.2.2.1.4	DELETE Operation

TP/oneM2M/CSE/GEN/DEL/001

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/DEL/002

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/DEL/003

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using shortcut structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/003_CSR/ | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

TP/oneM2M/CSE/GEN/DEL/004

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using structured resource identifier with hybrid addressing. |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to HYBRID_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |
| TP/oneM2M/CSE/GEN/DEL/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus “/fopt”. |  |

7.2.2.2	Registration (REG)

7.2.2.2.1	RETRIEVE Operation

TP/oneM2M/CSE/REG/RET/001

| TP Id |  | TP/oneM2M/CSE/REG/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource and responds with all attributes that have multiplicity equals to 1. |  |
| Reference |  | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource representation } |  | IUT AE |

TP/oneM2M/CSE/REG/RET/002

| TP Id |  | TP/oneM2M/CSE/REG/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource with the optional ATTRIBUTE. |  |
| Reference |  | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource and the IUT having a CSEBase resource containing the valid ATTRIBUTE attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource containing valid ATTRIBUTE attribute } |  | IUT AE |

| TP Id | PICS Selection | Reference | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/002_CST | PICS_CB_CST | TS-0001 9.6.3 | cseType |
| TP/oneM2M/CSE/REG/RET/002_NL | PICS_CB_NL | TS-0001 9.6.3 | nodeLink |

TP/oneM2M/CSE/REG/RET/003

| TP Id |  | TP/oneM2M/CSE/REG/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <AE> resource with attributes multiplicity equals to 1. |  |
| Reference |  | TS-0001 [1], clause 10.2.1 and clause 9.6.5, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the AE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing AE resource representation } |  | IUT AE |

TP/oneM2M/CSE/REG/RET/004

| TP Id |  | TP/oneM2M/CSE/REG/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <AE> resource with optional ATTRIBUTE. |  |
| Reference |  | TS-0001 [1], clause 10.2.1.2 and clause 9.6.5, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing valid ATTRIBUTE attribute and the AE having privileges to perform RETRIEVE operation on the AE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing AE resource containing ATTRIBUTE attribute } |  | IUT AE |

| TP Id | PICS Selection | Reference | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/004_LBL | PICS_AE_LBL | TS-0001 [1], clause 9.6.5 | labels |
| TP/oneM2M/CSE/REG/RET/004_APN | PICS_AE_APN | TS-0001 [1], clause 9.6.5 | appName |
| TP/oneM2M/CSE/REG/RET/004_POA | PICS_AE_POA | TS-0001 [1], clause 9.6.5 | pointOfAccess |
| TP/oneM2M/CSE/REG/RET/004_NL | PICS_AE_NL | TS-0001 [1], clause 9.6.5 | nodeLink |
| TP/oneM2M/CSE/REG/RET/004_CSZ | PICS_AE_CSZ | TS-0001 [1], clause 9.6.5 | contentSerialization |

TP/oneM2M/CSE/REG/RET/005

| TP Id |  | TP/oneM2M/CSE/REG/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource including the cseType attribute |  |
| Reference |  | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource containing cseType attribute set to 1 (IN_CSE) } |  | IUT AE |

TP/oneM2M/CSE/REG/RET/006

| TP Id |  | TP/oneM2M/CSE/REG/RET/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <remoteCSE> resource. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4 TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the ORIGINATOR having privileges to perform RETRIEVE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from ORIGINATOR containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing remoteCSE resource representation } |  | IUT ORIGINATOR |

TP/oneM2M/CSE/REG/RET/007

| TP Id |  | TP/oneM2M/CSE/REG/RET/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <remoteCSE> resource with optional ATTRIBUTE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4, TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the ORIGINATOR having privileges to perform RETRIEVE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from ORIGINATOR containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing remoteCSE resource containing ATTRIBUTE attribute } |  | IUT ORIGINATOR |

| TP Id | PICS Selection | Reference | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/007_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4 | labels |
| TP/oneM2M/CSE/REG/RET/007_CST | PICS_CSR_CST | TS-0001 [1], clause 9.6.4 | cseType |
| TP/oneM2M/CSE/REG/RET/007_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4 | pointOfAccess |
| TP/oneM2M/CSE/REG/RET/007_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4 | nodeLink |

TP/oneM2M/CSE/REG/RET/008

| TP Id |  | TP/oneM2M/CSE/REG/RET/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource and responds with supportedResourceTypes attribute containing a list of the supported resources and pointOfAccess containing the list of physical addresses to be used by Registree to connect to this CSE (e.g. IP address, FQDN). |  |
| Reference |  | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource containing supportedResourceType set to PX_SRT pointOfAccess set to PX_POA } |  | IUT AE |

TP/oneM2M/CSE/REG/RET/009

| TP Id |  | TP/oneM2M/CSE/REG/RET/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> retrieve request on TARGET_REMOTE_CSE_ADDRESS |  |
| Reference |  | TS-0001 [1], clause 10.2.2.2, TS-0004 [2], clause 7.4.5.2.3 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT CSE |

TP/oneM2M/CSE/REG/RET/010

| TP Id |  | TP/oneM2M/CSE/REG/RET/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> retrieve request |  |
| Reference |  | TS-0001 [1], clause 10.2.2.2, TS-0004 [2], clause 7.4.5.2.3 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing remoteCSE resource representation } |  | IUT CSE |

7.2.2.2.2	CREATE Operation

TP/oneM2M/CSE/REG/CRE/001

| TP Id |  | TP/oneM2M/CSE/REG/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned SP_RELATIVE_AE_ID |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case b, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_SELECTION |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to SP_RELATIVE_AE_ID and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource representation } |  | IUT AE |

| TP Id | PARENT_RELEASE | SP_RELATIVE_ AE_ID | PICS_SELECTION |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/001_CAE | Release 1 | Starting with "C" | PICS_CSE |
| TP/oneM2M/CSE/REG/CRE/001_SAE | Release 2 | Starting with "S" | PICS_IN_CSE |

TP/oneM2M/CSE/REG/CRE/002

| TP Id |  | TP/oneM2M/CSE/REG/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned AE_ID_STEM of AE-ID-Stem format |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case d, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_SELECTION |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID_STEM and Resource Type set to 2 (AE) and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource representation } |  | IUT AE |

| TP Id | PARENT_RELEASE | AE_ID_STEM | PICS_SELECTION |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/002_CAE | Release 1 | Starting with "C" | PICS_CSE |
| TP/oneM2M/CSE/REG/CRE/002_SAE | Release 2 | Starting with "S" | PICS_IN_CSE |

TP/oneM2M/CSE/REG/CRE/003

| TP Id |  | TP/oneM2M/CSE/REG/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned S-AE-ID-STEM |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case b, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing From set to S-AE-ID-STEM and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT AE |
|  | then{ the IUT sends a valid CREATE Request to IN-CSE containing From set to SP-RELATIVE-AE-ID and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels attribute set to "Credential-ID:None" } |  | IUT IN-CSE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Valid AE-ID-Stem starting with "S". |  |  |  |

TP/oneM2M/CSE/REG/CRE/004

| TP Id |  | TP/oneM2M/CSE/REG/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (allowed App-ID, not allowed C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 step 3 and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT not allowing to register an AE containing allowed App-ID attribute indicating APP-ID and no t allowed AE-ID attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to C-AE-ID-STEM and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT AE |
|  | then { the IUT does not create the AE resource and the IUT sends a valid Response containing Response Status Code set to 4107 (SECURITY_ASSOCIATION_REQUIRED) } |  | IUT AE |

TP/oneM2M/CSE/REG/CRE/005

| TP Id |  | TP/oneM2M/CSE/REG/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, S-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, and clause 9.6.19 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE and and the IUT allowing to register an AE containing App-ID attribute set to APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to "S" and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT AE |
|  | then { the IUT sends a valid CREATE Request to the IN-CSE containing Resource Type set to 10002 (AEAnnc) and From set to SP-RELATIVE-CSE-ID followed by '/S' and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and valid link attribute and labels attribute indicating 'Credential-ID:None' } |  | IUT IN-CSE |

TP/oneM2M/CSE/REG/CRE/006

| TP Id |  | TP/oneM2M/CSE/REG/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided character 'S' in AE_ID_Stem |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing From set to character 'S' and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT AE |
|  | then{ the IUT sends a valid CREATE Request to IN_CSE containing From set to SP-RELATIVE-CSE-ID followed by '/S' and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID } |  | IUT IN-CSE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{AE_ID_Stem} AE_ID_Stem = "S". |  |  |  |

TP/oneM2M/CSE/REG/CRE/007

| TP Id |  | TP/oneM2M/CSE/REG/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an response of AEANNC create request during AE registration with provided character 'S' in AE_ID_Stem |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE and the IUT having received a valid AE create request containing From set to "S" and the IUT having sent a valid AEANNC create to IN_CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when{ the IUT receives a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID } |  | IUT <- IN-CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing ResourceID attribute set to S-AE-ID-STEM and AE-ID attribute set to S-AE-ID-STEM } |  | IUT AE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM} S-AE-ID-STEM = Valid AE-ID-Stem assigned by IN-CSE. |  |  |  |

TP/oneM2M/CSE/REG/CRE/008

| TP Id |  | TP/oneM2M/CSE/REG/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer request to the IN-CSE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case b, step 5 and clause 9.6.19 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" the IUT having registered to the IN-CSE and the AE having already been registered containing AE-ID set to S-AE-ID-STEM and the AE having deregistered } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to S-AE-ID-STEM and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid UPDATE Request to the IN-CSE containing From set to SP-RELATIVE-AE-ID and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and link attribute set to SP-RELATIVE-AE-ID and labels attribute indicating 'Credential-ID:None' } |  | IUT IN-CSE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE. |  |  |  |

TP/oneM2M/CSE/REG/CRE/009

| TP Id |  | TP/oneM2M/CSE/REG/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case b, step 5-8 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the MN-CSE and the AE having already been registered containing AE-ID set to S-AE-ID-STEM and the AE having deregistered } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from the MN-CSE containing To set to AEANNC_RESOURCE_ADDRESS and From set to SP-RELATIVE-AE-ID and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels attribute indicating 'Credential-ID:None' } |  | IUT MN-CSE |
|  | then { the IUT updates the AEANNC_RESOURCE_ADDRESS resource and the IUT sends a valid Response to MN-CSE containing Response Status Code set to 2004 (UPDATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels attribute indicating 'Credential-ID:None' } |  | IUT MN-CSE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE. |  |  |  |

TP/oneM2M/CSE/REG/CRE/010

| TP Id |  | TP/oneM2M/CSE/REG/CRE/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer response to the AE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case b, step 8 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the MN-CSE and the AE having already been registered containing AE-ID set to S-AE-ID-STEM and the AE having deregistered and the AE having sent a registration containing From set to S-AE-ID-STEM and the IUT having sent a valid AEAnnc UPDATE Request to IN-CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Response from IN-CSE containing Response Status Code set to 2004 (UPDATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels attribute indicating 'Credential-ID:None' } |  | IUT IN-CSE |
|  | then { the IUT sends a valid Response to the AE containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing AE-ID attribute set to S-AE-ID-STEM } |  | IUT AE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE. |  |  |  |

TP/oneM2M/CSE/REG/CRE/011

| TP Id |  | TP/oneM2M/CSE/REG/CRE/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and asking AE_ID_Stem by indicating AE_ID_Stem starting with character 'C'. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case c, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to 'C' and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing ResourceID attribute indicating a value starting with 'C' } |  | IUT AE |

TP/oneM2M/CSE/REG/CRE/012

| TP Id |  | TP/oneM2M/CSE/REG/CRE/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration with the optional attribute OPTIONAL_ATTRIBUTE provided |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 and clause 9.6.5-2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to AE_ID and Content containing AE resource containing valid OPTIONAL_ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing valid OPTIONAL_ATTRIBUTE attribute } |  | IUT AE |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/012_AE/LBL | PICS_AE_LBL | TS-0001 [1], clause 9.6.5-2 | labels |
| TP/oneM2M/CSE/REG/CRE/012_AE/APN | PICS_AE_APN | TS-0001 [1], clause 9.6.5-2 | AppName |
| TP/oneM2M/CSE/REG/CRE/012_AE/POA | PICS_AE_POA | TS-0001 [1], clause 9.6.5-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/012_AE/NL | PICS_AE_NL | TS-0001 [1], clause 9.6.5-2 | nodeLink |
| TP/oneM2M/CSE/REG/CRE/012_AE/OR | PICS_AE_OR | TS-0001 [1], clause 9.6.5-2 | ontologyRef |

TP/oneM2M/CSE/REG/CRE/013

| TP Id |  | TP/oneM2M/CSE/REG/CRE/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <remoteCSE> resource with OPTIONAL_ATTRIBUTE. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4, TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation containing OPTIONAL_ATTRIBUTE attribute } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  | IUT CSE |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/013_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4 | labels |
| TP/oneM2M/CSE/REG/CRE/013_CST | PICS_CSR_CST | TS-0001 [1], clause 9.6.4 | cseType |
| TP/oneM2M/CSE/REG/CRE/013_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/013_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4 | nodeLink |

TP/oneM2M/CSE/REG/CRE/014

| TP Id |  | TP/oneM2M/CSE/REG/CRE/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the request to the remoteCSE according to pointOfAccess in the <remoteCSE> resource. |  |
| Reference |  | TS-0001 [1], clause 9.6.4 and clause 9.3.2.2.3, TS-0004 [2], clause 6.2.3 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created the remoteCSE resource containing POINT_OF_ACCESS attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid REQUEST from Originator containing To set to RESOURCE_ADDRESS and From set to Originator_ID } |  | IUT Originator |
|  | then { the IUT sends the REQUEST to POINT_OF_ACCESS } |  | IUT CSE |

TP/oneM2M/CSE/REG/CRE/015

| TP Id |  | TP/oneM2M/CSE/REG/CRE/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the request to the CSE according to pointOfAccess in the <remoteCSE> resource and forward the response back to the originator. |  |
| Reference |  | TS-0001 [1], clause 9.6.4 and clause 9.3.2.2.3, TS-0004 [2], clause 6.2.3 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE , PICS_SP_RELATIVE_RESOURCE_ID, PICS_ABSOLUTE_RESOURCE_ID |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received a valid REQUEST from Originator and The IUT having sent the REQUEST to the remoteCSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RESPONSE from the remoteCSE } |  | IUT CSE |
|  | then { the IUT sends the RESPONSE to the Originator } |  | IUT Originator |

TP/oneM2M/CSE/REG/CRE/016

| TP Id |  | TP/oneM2M/CSE/REG/CRE/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (not allowed App-ID) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 step 3 and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT not allowing to register an AE containing not allowed App-ID attribute indicating APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT AE |
|  | then { the IUT does not create the AE resource and the IUT sends a valid Response containing Response Status Code set to 4107 (SECURITY_ASSOCIATION_REQUIRED) } |  | IUT AE |

TP/oneM2M/CSE/REG/CRE/017

| TP Id |  | TP/oneM2M/CSE/REG/CRE/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a create request of <AE> resource that does not include the MANDATORY_ATTRIBUTE. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 2(AE) and Content containing AE resource containing no MANDATORY_ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD Request) } |  | IUT AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/017_API | TS-0001 [1], clause 9.6.5 | App-ID |
| TP/oneM2M/CSE/REG/CRE/017_RR | TS-0001 [1], clause 9.6.5 | requestReachability |

TP/oneM2M/CSE/REG/CRE/018

| TP Id |  | TP/oneM2M/CSE/REG/CRE/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <remoteCSE> resource with attributes multiplicity equals to 1. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  | IUT CSE |

TP/oneM2M/CSE/REG/CRE/019

| TP Id |  | TP/oneM2M/CSE/REG/CRE/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <remoteCSE> resource with attributes multiplicity equals to 1 without the preconfigured CSE-ID. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and no From and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  | IUT CSE |

TP/oneM2M/CSE/REG/CRE/020

| TP Id |  | TP/oneM2M/CSE/REG/CRE/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and asking AE_ID_Stem by indicating AE_ID_Stem starting with character 'S'. |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing From set to 'S' and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing ResourceID attribute set to S-AE-ID-STEM } |  | IUT AE |
| NOTE: S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE. |  |  |  |

TP/oneM2M/CSE/REG/CRE/021

| TP Id |  | TP/oneM2M/CSE/REG/CRE/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the create request of <CSEBase> resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.3.1 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and Resource Type set to 5 (CSEBase) and From set to AE-ID and Content containing CSEBase resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/REG/CRE/022

| TP Id |  | TP/oneM2M/CSE/REG/CRE/022 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, C-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case C and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE containing App-ID attribute indicating APP-ID and AE-ID attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to 'C' and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT AE |
|  | then { the IUT creates the AE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

TP/oneM2M/CSE/REG/CRE/023

| - TP Id |  | TP/oneM2M/CSE/REG/CRE/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects registration of already registered AE (C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 step 4 and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state"and the IUT having registered the AE with AE-ID attribute set to C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and To set to TARGE_CSE_RESOURCE_ADDRESS and From set to C-AE-ID-STEM and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT does not create the AE resource and the IUT sends a valid Response containing Response Status Code set to 4105 (CONFLICT) } |  | IUT AE |

TP/oneM2M/CSE/REG/CRE/024

| TP Id |  | TP/oneM2M/CSE/REG/CRE/024 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with attributes multiplicity equals to 1 |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | IUT CSE |

TP/oneM2M/CSE/REG/CRE/025

| TP Id |  | TP/oneM2M/CSE/REG/CRE/025 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with attributes multiplicity equals to 1 |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation and the IUT creates the <remoteCSE> resource } |  | IUT CSE |

TP/oneM2M/CSE/REG/CRE/026

| TP Id |  | TP/oneM2M/CSE/REG/CRE/026 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | IUT CSE |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/026_RN | PICS_CSR_RN | TS-0001 [1], clause 9.6.4-2 | resourceName |
| TP/oneM2M/CSE/REG/CRE/026_ET | PICS_CSR_ET | TS-0001 [1], clause 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/CRE/026_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/CRE/026_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/026_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |
| TP/oneM2M/CSE/REG/CRE/026_CST | PICS_CSR_CST | TS-0001 [1], clause 9.6.4-2 | cseType |

TP/oneM2M/CSE/REG/CRE/027

| TP Id |  | TP/oneM2M/CSE/REG/CRE/027 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with cseType attribute set to 'MN_CSE' |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and the IUT creates a <remoteCSE> resource } |  | IUT CSE |

TP/oneM2M/CSE/REG/CRE/028

| TP Id |  | TP/oneM2M/CSE/REG/CRE/028 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the CSE having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE and the IUT creates a <remoteCSE> resource } |  | IUT CSE |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/028_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/CRE/028_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/028_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |

TP/oneM2M/CSE/REG/CRE/029

| TP Id |  | TP/oneM2M/CSE/REG/CRE/029 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (preprovisioned S-AE-ID-STEM provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, step 5-8 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from the CSE containing Resource Type set to 10002 (AEAnnc) and From set to SP-RELATIVE-AE-ID and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels indicating 'Credential-ID:None' } |  | IUT MN-CSE |
|  | then { the IUT creates the AEANNC_RESOURCE_ADDRESS resource and the IUT sends a valid Response to CSE containing Response Status Code set to 2001 (CREATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels indicating 'Credential-ID:None' } |  | IUT MN-CSE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM} S-AE-ID-STEM = Valid AE-ID-Stem starting with "S". |  |  |  |

TP/oneM2M/CSE/REG/CRE/030

| TP Id |  | TP/oneM2M/CSE/REG/CRE/030 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer response to the AE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case b, step 8 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the MN-CSE and the AE having sent a registration containing From set to S-AE-ID-STEM and the IUT having sent a valid AEAnnc CREATE Request to IN-CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from IN-CSE containing Response Status Code set to 2001 (CREATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels attribute indicating 'Credential-ID:None' } |  | IUT IN-CSE |
|  | then { the IUT sends a valid Response to the AE containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing AE-ID attribute set to S-AE-ID-STEM } |  | IUT AE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM }. S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE. |  |  |  |

TP/oneM2M/CSE/REG/CRE/031

| TP Id |  | TP/oneM2M/CSE/REG/CRE/031 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (S-AE-ID-STEM not provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from MN-CSE containing Resource Type set to 10002 (AEAnnc) and From set to SP-RELATIVE-AE-ID and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels indicating 'Credential-ID:None' } |  | IUT MN-CSE |
|  | then { the IUT creates the AEANNC_RESOURCE_ADDRESS resource and the IUT sends a valid Response to CSE containing Response Status Code set to 2001 (CREATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID* and labels indicating 'Credential-ID:None' } |  | IUT MN-CSE |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{AE-ID-STEM} SP-RELATIVE-AE-ID* = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM} AE-ID-STEM = "S" S-AE-ID-STEM = Valid AE-ID-Stem starting with "S". |  |  |  |

TP/oneM2M/CSE/REG/CRE/032

| TP Id |  | TP/oneM2M/CSE/REG/CRE/032 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 with AE-ID-Stem being empty |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing From set to empty and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource representation } |  | IUT AE |

7.2.2.2.3	DELETE Operation

TP/oneM2M/CSE/REG/DEL/001

| TP Id |  | TP/oneM2M/CSE/REG/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the delete request of <CSEBase> resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.3.4 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/REG/DEL/002

| TP Id |  | TP/oneM2M/CSE/REG/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> delete request on TARGET_REMOTE_CSE_ADDRESS, and deletes the <remoteCSE> resource |  |
| Reference |  | TS-0001 [1], clause 10.2.2.4 and clause 9.6.4, TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the CSE and the CSE originator having privileges to perform DELETE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from CSE containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to CSE_ID and no Content } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT deletes the remoteCSE resource } |  | IUT CSE |

TP/oneM2M/CSE/REG/DEL/003

| TP Id |  | TP/oneM2M/CSE/REG/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration. |  |
| Reference |  | TS-0001 [1], clause 10.1.4.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform DELETE operation on the resource AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the AE_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

TP/oneM2M/CSE/REG/DEL/004

| TP Id |  | TP/oneM2M/CSE/REG/DEL/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> delete request on TARGET_REMOTE_CSE_ADDRESS |  |
| Reference |  | TS-0001 [1], clause 10.2.2.4, TS-0004 [2], clause 7.4.5.2.5 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform DELETE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESSS } |  | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT CSE |

TP/oneM2M/CSE/REG/DEL/005

| TP Id |  | TP/oneM2M/CSE/REG/DEL/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration when AE-ID is starting with "S" |  |
| Reference |  | TS-0001 [1], clause 10.1.4.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing From set to "S" and the AE having privileges to perform DELETE operation on the resource AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the AE_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

TP/oneM2M/CSE/REG/DEL/006

| TP Id |  | TP/oneM2M/CSE/REG/DEL/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration when AE-ID is starting with "S" |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing From set to "S" and the AE having privileges to perform DELETE operation on the resource AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid UPDATE Request containing Content containing AEANNC_RESOURCE_ADDRESS resource containing link attribute set to "INACTIVE" } |  | IUT  IN-CSE |

TP/oneM2M/CSE/REG/DEL/007

| TP Id |  | TP/oneM2M/CSE/REG/DEL/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the response from IN-CSE for the AE de-registration when AE-ID is starting with "S" |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing From set to "S" and the AE having privileges to perform DELETE operation on the resource AE and the AE having sent a valid DELETE Request at AE_RESOURCE_ADDRESS resource and the IUT having sent a valid UPDATE Request to IN-CSE containing Content containing AEANNC_RESOURCE_ADDRESS resource containing link attribute set to empty } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from IN-CSE containing Response Status Code set to 2004 (UPDATED) } |  | IUT  IN-CSE |
|  | then { the IUT deletes the AE_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

7.2.2.2.4	UPDATE Operation

TP/oneM2M/CSE/REG/UPD/001

| TP Id |  | TP/oneM2M/CSE/REG/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update request of <CSEBase> resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.3.3 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and Content containing CSEBase resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/REG/UPD/002

| TP Id |  | TP/oneM2M/CSE/REG/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> update request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.2.2.3, TS-0004 [2], clause 7.4.5.2.4 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS containing a RW OPTIONAL_ATTRIBUTE attribute set to VALUE_1 and the CSE having privileges to perform UPDATE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and Content containing remoteCSE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE_2 } |  | IUT CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing remoteCSE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE_2 } |  | IUT CSE |

| TP Id | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/002_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/UPD/002_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/002_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |

TP/oneM2M/CSE/REG/UPD/003

| TP Id |  | TP/oneM2M/CSE/REG/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> update request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.2.2.3, TS-0004 [2], clause 7.4.5.2.4 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS containing a RW OPTIONAL_ATTRIBUTE attribute set to VALUE_1 and the IUT having privileges to perform UPDATE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and Content containing remoteCSE resource containing attribute OPTIONAL_ATTRIBUTE set to VALUE_2 } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and Content containing remoteCSE resource containing attribute OPTIONAL_ATTRIBUTE set to VALUE_2 } |  | IUT CSE |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/003_ET | PICS_CSR_ET | TS-0001 [1], clause 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/UPD/003_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/UPD/003_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/003_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |
| TP/oneM2M/CSE/REG/UPD/003_RR | PICS_CSR_RR | TS-0001 [1], clause 9.6.4-2 | requestReachability |

7.2.2.3	Data Management and Repository Function (DMR)

7.2.2.3.1	RETRIEVE Operation

TP/oneM2M/CSE/DMR/RET/001

| TP Id |  | TP/oneM2M/CSE/DMR/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/001_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/001_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/001_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/001_CIN | Release 1 | TS-0001 [1], clause 10.2.4.2 | 4 (contentInstance) |
| TP/oneM2M/CSE/DMR/RET/001_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/001_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/RET/002

| TP Id |  | TP/oneM2M/CSE/DMR/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference |  | TS-0001 [1], clause 10.1.2 item 1) |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/RET/003

| TP Id |  | TP/oneM2M/CSE/DMR/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a resource TARGET_RESOURCE_ADDRESS under AE without having privileges for the RETRIEVE operation |  |
| Reference |  | TS-0001 [1], clause 10.1.2 item 2) |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/003_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/003_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/003_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/003_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |

TP/oneM2M/CSE/DMR/RET/004

| TP Id |  | TP/oneM2M/CSE/DMR/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("To" option) |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_ATTRIBUTE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT AE |
| NOTE: TARGET_ATTRIBUTE_ADDRESS is represented as TARGET_RESOURCE_ADDRESS#ATTRIBUTE where '#' is present between TARGET_RESOURCE_ADDRESS and ATTRIBUTE. ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/004_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/004_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/004_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/004_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/004_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/RET/005

| TP Id |  | TP/oneM2M/CSE/DMR/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("Content" option) |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT AE |
| NOTE: ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/005_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/005_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/005_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/005_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/005_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/RET/006

| TP Id |  | TP/oneM2M/CSE/DMR/RET/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containin ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_1 attribute and valid ATTRIBUTE_2 attribute and no other attribute } |  | IUT AE |
| NOTE: ATTRIBUTE_1 indicates common attribute for all RESOURCE_TYPE resources listed in the table below. ATTRIBUTE_2 indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/006_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/006_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/006_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/006_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/006_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/RET/007

| TP Id |  | TP/oneM2M/CSE/DMR/RET/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource ("single" attribute case) |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource containing no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |
| NOTE: labels attribute was chosen, since it is a common attribute for all types of resource. It can be replaced by any common attribute. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/007_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/007_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/007_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/RET/007_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/RET/007_PCH/LBL | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChallenge) | labels |

TP/oneM2M/CSE/DMR/RET/008

| TP Id |  | TP/oneM2M/CSE/DMR/RET/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attributes of the TARGET_RESOURCE_ADDRESS resource ("multiple" attributes case) |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource containing no ATTRIBUTE_1 attribute and no ATTRIBUTE_2 attribute and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/008_CNT/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_SUB/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_ACP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_GRP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_PCH/LBL/PI | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) | labels | parentID |

TP/oneM2M/CSE/DMR/RET/009

| TP Id |  | TP/oneM2M/CSE/DMR/RET/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the RETRIEVE Request of a latest resource target to a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 [1], clause 10.2.22.1, TS-0004 [2], clause 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no direct child contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/RET/010

| TP Id | TP/oneM2M/CSE/DMR/RET/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the RETRIEVE Request of the oldest contentInstance target to a <container> resource |  |
| Reference | TS-0001 [1], clause 10.2.23.1, TS-0004 [2], clause 7.4.29.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform RETRIEVE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing contentInstance resource containing resourceName attribute set to CONTENT_INSTANCE_1 } | IUT AE |

TP/oneM2M/CSE/DMR/RET/011

| TP Id |  | TP/oneM2M/CSE/DMR/RET/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the RETRIEVE Request of an oldest resource target to a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 [1], clause 10.2.23.1, TS-0004 [2], clause 7.4.29.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no direct child contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/RET/012

| TP Id |  | TP/oneM2M/CSE/DMR/RET/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the RETRIEVE Request of the latest contentInstance target to a <container> resource |  |
| Reference |  | TS-0001 [1], clause 10.2.22.1, TS-0004 [2], clause 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform RETRIEVE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a Response containing Response Status Code set to 2000 (OK) and Content containing contentInstance resource containing resourceName attribute set to CONTENT_INSTANCE_2 } |  | IUT AE |

TP/oneM2M/CSE/DMR/RET/013

| TP Id |  | TP/oneM2M/CSE/DMR/RET/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/013_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/013_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/013_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/013_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/013_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/013_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |

TP/oneM2M/CSE/DMR/RET/014

| TP Id |  | TP/oneM2M/CSE/DMR/RET/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a resource TARGET_RESOURCE_ADDRESS under CSEBase without having privileges for the RETRIEVE operation |  |
| Reference |  | TS-0001 [1], clause 10.1.2 item 2) |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/014_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/014_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/014_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/014_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/014_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/014_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |

TP/oneM2M/CSE/DMR/RET/015

| TP Id |  | TP/oneM2M/CSE/DMR/RET/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("To" option) under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_ATTRIBUTE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT AE |
| NOTE: TARGET_ATTRIBUTE_ADDRESS is represented as TARGET_RESOURCE_ADDRESS # ATTRIBUTE. ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/015_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/015_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/015_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/015_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/015_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/015_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |

TP/oneM2M/CSE/DMR/RET/016

| TP Id |  | TP/oneM2M/CSE/DMR/RET/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("Content" option) under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT AE |
| NOTE: ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/016_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/016_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/016_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/016_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/016_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/016_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |

TP/oneM2M/CSE/DMR/RET/017

| TP Id |  | TP/oneM2M/CSE/DMR/RET/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_1 attribute and valid ATTRIBUTE_2 attribute and no other attribute } |  | IUT AE |
| NOTE: ATTRIBUTE_1 indicates common attribute for all RESOURCE_TYPE resources listed in the table below. ATTRIBUTE_2 indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/017_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/017_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/017_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/017_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/017_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/017_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |

TP/oneM2M/CSE/DMR/RET/018

| TP Id |  | TP/oneM2M/CSE/DMR/RET/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource ("single" attribute case) under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource containing no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/018_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/018_AE/LBL | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) | labels |
| TP/oneM2M/CSE/DMR/RET/018_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/018_CSR/LBL | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) | labels |
| TP/oneM2M/CSE/DMR/RET/018_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/RET/018_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels |

TP/oneM2M/CSE/DMR/RET/019

| TP Id |  | TP/oneM2M/CSE/DMR/RET/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource ("multiple" attributes case) under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the <CSEBase> resource containing no ATTRIBUTE_1 attribute and no ATTRIBUTE_2 attribute and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/019_ACP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_AE/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_CNT/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_CSR/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_SUB/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_GRP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels | announceTo |

TP/oneM2M/CSE/DMR/RET/020

| TP Id |  | TP/oneM2M/CSE/DMR/RET/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a RETRIEVE Request target to TARGET_RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to RESULT_CONTENT } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) and no Content } |  | IUT  AE |

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/020_RCN/0 | TS-0004 [2], clause 6.3.4.2.7 | 0 (Nothing) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/2 |  | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/3 |  | 3 (attributes and hierarchichal address) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/9 |  | 9 (modified attributes) |

TP/oneM2M/CSE/DMR/RET/021

| TP Id |  | TP/oneM2M/CSE/DMR/RET/021 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to 1 (attributes) } |  | IUT  AE |  |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references } |  | IUT  AE |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/021_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/021_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/021_ACP | Release 2 | 1 (accessControlPolicy) |

7.2.2.3.2	UPDATE Operation

TP/oneM2M/CSE/DMR/UPD/001

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/001_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_PCH/LBL | Release 2 | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | labels |

TP/oneM2M/CSE/DMR/UPD/002

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT adds successfully the attribute ATTRIBUTE_NAME to the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing no optional RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 } |  | IUT AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 } |  | IUT AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/002_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_GRP/LBL | Release 1 | PICS_GRP_LBL | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_PCH/LBL | Release 2 |  | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | labels |

TP/oneM2M/CSE/DMR/UPD/003

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes successfully the attribute ATTRIBUTE_NAME from the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing an optional RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to NULL } |  | IUT AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to NULL } |  | IUT AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/003_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_GRP/LBL | Release 1 | PICS_GRP_LBL | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_PCH/LBL | Release 2 |  | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | labels |

TP/oneM2M/CSE/DMR/UPD/004

| TP Id | TP/oneM2M/CSE/DMR/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the value of the attribute ATTRIBUTE_NAME_1, and creates the ATTRIBUTE_NAME_2 and deletes the ATTRIBUTE_NAME_3 of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME_1 attribute set to VALUE_1 and no optional RW ATTRIBUTE_NAME_2 attribute and an optional RW ATTRIBUTE_NAME_3 attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME_1 attibute set to VALUE_2 and ATTRIBUTE_NAME_2 attribute set to VALUE_3 and ATTRIBUTE_NAME_3 attribute set to NULL } | IUT AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME_1 attribute set to VALUE_2 and ATTRIBUTE_NAME_2 attribute set to VALUE_3 and ATTRIBUTE_NAME_3 attribute set to NULL } | IUT AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME_1 | ATTRIBUTE_NAME_2 | ATTRIBUTE_NAME_3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/004_CNT/ET/MNI/LBL | Release 1 | PICS_CNT_MNI and PICS_CNT_LBL | TS-0001 [1], clause 10.2.4.3 | 3 (container) | expirationTime | maxNrOfInstances | labels |
| TP/oneM2M/CSE/DMR/UPD/004_ACP/PVS/PV/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | selfPrivileges | privileges | labels |
| TP/oneM2M/CSE/DMR/UPD/004_SUB/ET/LBL/EXC | Release 1 | PICS_SUB_LBL and PICS_SUB_EXC | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | expirationTime | labels | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/004_GRP/ET/GN/LBL | Release 1 | PICS_GRP_GN and PICS_GRP_LBL | TS-0001 [1], clause 10.2.7.4 | 9 (group) | expirationTime | groupName | labels |

TP/oneM2M/CSE/DMR/UPD/005

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update an attribute of a TARGET_RESOURCE_ADDRESS resource when the resource does not exist |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing valid representation resource containing valid attribute } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/UPD/006

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update the attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource without having privileges for the UPDATE operation |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having no privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/006_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |

TP/oneM2M/CSE/DMR/UPD/007

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update a non-RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a non-RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/007_CNT/CT | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_ACP/CT | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_SUB/CT | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_GRP/CT | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_PCH/CT | Release 2 | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | creationTime |

TP/oneM2M/CSE/DMR/UPD/008

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete a mandatory RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a mandatory RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to NULL } |  | IUT AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/008_ACP/PVS | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | selfPrivileges |
| TP/oneM2M/CSE/DMR/UPD/008_SUB/NU | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | notificationURI |
| TP/oneM2M/CSE/DMR/UPD/008_GRP/MNI | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | maxNrOfInstances |

TP/oneM2M/CSE/DMR/UPD/009

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update a RW attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource with an UNACCEPTABLE_VALUE |  |
| Reference |  | TS-0001 [1], clause 10.1.3 item 3) |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a valid RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing not supported ATTRIBUTE_NAME attribute } |  | IUT AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/009_CNT/EXC | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_ACP/EXC | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_SUB/MNI | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/009_GRP/EXC | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_PCH/EXC | Release 2 | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | expirationCounter |

TP/oneM2M/CSE/DMR/UPD/010

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the stateTag attribute of a RESOURCE_TYPE resource is increased when an update operation has been performed on its child resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing CHILD_RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having privileges to perform UPDATE operation on the TARGET_CHILD_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID Content containing CHILD_RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE |
|  | then { the IUT increments the stateTag attribute of RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/010_CNT | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) |

TP/oneM2M/CSE/DMR/UPD/011

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE Request of an existing RESOURCE_TYPE resource with error "OPERATION_NOT_ALLOWED" |  |
| Reference |  | TS-0001 [1], clause 10.2.19.4, TS-0004 [2], clause 7.3.2.1 and clause 7.4.8.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource containing a RESOURCE_TYPE resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE representation } |  |  |
|  | then { the IUT does not update the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/011_CNT/CIN | Release 1 | TS-0001 [1], clause 10.2.4.3 | 4 (contentInstance) | 3 (container) |

TP/oneM2M/CSE/DMR/UPD/012

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE Request of a latest resource as a direct child of a <container> resource with error "OPERATION_NOT_ALLOWED" |  |
| Reference |  | TS-0001 [1], clause 10.2.22, TS-0004 [2], clause 7.4.28.2.4 and clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing a contentInstance resource containing valid creationTime attribute set to the most recent resource creation time } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT does not update the CONTAINER_RESOURCE_ADDRESS/la resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/UPD/013

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE Request of an oldest resource as a direct child of a <container> resource with error "OPERATION_NOT_ALLOWED" |  |
| Reference |  | TS-0001 [1], clause 10.2.23, TS-0004 [2], clause 7.4.29.2.4 and clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing contentInstance resource(s) } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT does not update the CONTAINER_RESOURCE_ADDRESS/ol resource and the IUT sends a Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/UPD/014

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the optional attribute OPTIONAL_ATTRIBUTE of the RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/ACPI | Release 1 | PICS_CNT_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | container | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/ET | Release 1 | PICS_CNT_ET | TS-0001 [1], clause 9.6.1.3.1-1 | container | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | container | labels |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MNI | Release 1 | PICS_CNT_MNI | TS-0001 [1], clause 9.6.6-2 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MBS | Release 1 | PICS_CNT_MBS | TS-0001 [1], clause 9.6.6-2 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MIA | Release 1 | PICS_CNT_MIA | TS-0001 [1], clause 9.6.6-2 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/OR | Release 1 | PICS_CNT_OR | TS-0001 [1], clause 9.6.6-2 | container | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | accessControlPolicy | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | accessControlPolicy | labels |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ACPI | Release 1 | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | subscription | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | subscription | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | subscription | labels |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ENC | Release 1 | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8-2 | subscription | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/EXC | Release 1 | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8-2 | subscription | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/GPI | Release 1 | PICS_SUB_GPI | TS-0001 [1], clause 9.6.8-2 | subscription | groupID |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NFU | Release 1 | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8-2 | subscription | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/BN | Release 1 | PICS_SUB_BN | TS-0001 [1], clause 9.6.8-2 | subscription | batchNotify |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/RL | Release 1 | PICS_SUB_RL | TS-0001 [1], clause 9.6.8-2 | subscription | rateLimit |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/PN | Release 1 | PICS_SUB_PN | TS-0001 [1], clause 9.6.8-2 | subscription | pendingNotification |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NSP | Release 1 | PICS_SUB_NSP | TS-0001 [1], clause 9.6.8-2 | subscription | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/LN | Release 1 | PICS_SUB_LN | TS-0001 [1], clause 9.6.8-2 | subscription | latestNotify |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NCT | Release 1 | PICS_SUB_NCT | TS-0001 [1], clause 9.6.8-2 | subscription | notificationContentType |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NEC | Release 1 | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8-2 | subscription | notificationEventCat |

TP/oneM2M/CSE/DMR/UPD/015

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the optional attribute OPTIONAL_ATTRIBUTE of the RESOURCE_TYPE resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource containing no OPTIONAL_ATTRIBUTE attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT AE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/015_CSR/LBL | Release 1 | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | remoteCSE | labels |
| TP/oneM2M/CSE/DMR/UPD/015_CSR/POA | Release 1 | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | remoteCSE | pointOfAccess |
| TP/oneM2M/CSE/DMR/UPD/015_CSR/NL | Release 1 | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | remoteCSE | nodeLink |
| TP/oneM2M/CSE/DMR/UPD/015_AE/LBL | Release 1 | PICS_AE_LBL | TS-0001 [1], clause 9.6.5-2 | AE | labels |
| TP/oneM2M/CSE/DMR/UPD/015_AE/ACPI | Release 1 | PICS_AE_ACPI | TS-0001 [1], clause 9.6.5-2 | AE | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_AE/APN | Release 1 | PICS_AE_APN | TS-0001 [1], clause 9.6.5-2 | AE | AppName |
| TP/oneM2M/CSE/DMR/UPD/015_AE/POA | Release 1 | PICS_AE_POA | TS-0001 [1], clause 9.6.5-2 | AE | pointOfAccess |
| TP/oneM2M/CSE/DMR/UPD/015_AE/OR | Release 1 | PICS_AE_OR | TS-0001 [1], clause 9.6.5-2 | AE | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 9.6.6-2 | container | labels |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/ACPI | Release 1 | PICS_CNT_ACPI | TS-0001 [1], clause 9.6.6-2 | container | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/MNI | Release 1 | PICS_CNT_MNI | TS-0001 [1], clause 9.6.6-2 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/MBS | Release 1 | PICS_CNT_MBS | TS-0001 [1], clause 9.6.6-2 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/MIA | Release 1 | PICS_CNT_MIA | TS-0001 [1], clause 9.6.6-2 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/OR | Release 1 | PICS_CNT_OR | TS-0001 [1], clause 9.6.6-2 | container | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/LI | Release 1 | PICS_CNT_LI | TS-0001 [1], clause 9.6.6-2 | container | locationID |
| TP/oneM2M/CSE/DMR/UPD/015_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | labels |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/ACPI | Release 1 | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.8-2 | subscription | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 9.6.8-2 | subscription | labels |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/ENC | Release 1 | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8-2 | subscription | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/EXC | Release 1 | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8-2 | subscription | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/GPI | Release 1 | PICS_SUB_GPI | TS-0001 [1], clause 9.6.8-2 | subscription | groupID |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/NFU | Release 1 | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8-2 | subscription | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/BN | Release 1 | PICS_SUB_BN | TS-0001 [1], clause 9.6.8-2 | subscription | batchNotify |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/RL | Release 1 | PICS_SUB_RL | TS-0001 [1], clause 9.6.8-2 | subscription | rateLimit |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/PN | Release 1 | PICS_SUB_PN | TS-0001 [1], clause 9.6.8-2 | subscription | pendingNotification |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/NSP | Release 1 | PICS_SUB_NSP | TS-0001 [1], clause 9.6.8-2 | subscription | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/LN | Release 1 | PICS_SUB_LN | TS-0001 [1], clause 9.6.8-2 | subscription | latestNotify |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/NEC | Release 1 | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8-2 | subscription | notificationEventCat |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/LBL | Release 1 | PICS_GRP_LBL | TS-0001 [1], clause 9.6.13-2 | group | labels |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/ACPI | Release 1 | PICS_GRP_ACPI | TS-0001 [1], clause 9.6.13-2 | group | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/MACP | Release 1 | PICS_GRP_MACP | TS-0001 [1], clause 9.6.13-2 | group | membersAccessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/GN | Release 1 | PICS_GRP_GN | TS-0001 [1], clause 9.6.13-2 | group | groupName |

TP/oneM2M/CSE/DMR/UPD/016

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the attribute MANDATORY_ATTRIBUTE of the RESOURCE_TYPE resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing MANDATORY_ATTRIBUTE attribute set to VALUE } |  | IUT AE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing MANDATORY_ATTRIBUTE attribute set to VALUE } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | MANDATORY_ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/016_CSR/ET | Release 1 | TS-0001 [1], clause 9.6.4-2 | remoteCSE | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_CSR/RR | Release 1 | TS-0001 [1], clause 9.6.4-2 | remoteCSE | requestReachability |
| TP/oneM2M/CSE/DMR/UPD/016_AE/ET | Release 1 | TS-0001 [1], clause 9.6.5-2 | AE | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_AE/RR | Release 1 | TS-0001 [1], clause 9.6.5-2 | AE | requestReachability |
| TP/oneM2M/CSE/DMR/UPD/016_CNT/ET | Release 1 | TS-0001 [1], clause 9.6.6-2 | container | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_ACP/ET | Release 1 | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_ACP/PV | Release 1 | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | privileges |
| TP/oneM2M/CSE/DMR/UPD/016_ACP/PVS | Release 1 | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | selfPrivileges |
| TP/oneM2M/CSE/DMR/UPD/016_SUB/ET | Release 1 | TS-0001 [1], clause 9.6.8-2 | subscription | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_SUB/NU | Release 1 | TS-0001 [1], clause 9.6.8-2 | subscription | notificationURI |
| TP/oneM2M/CSE/DMR/UPD/016_SUB/NCT | Release 1 | TS-0001 [1], clause 9.6.8-2 | subscription | notificationContentType |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/ET | Release 1 | TS-0001 [1], clause 9.6.13-2 | group | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/MNM | Release 1 | TS-0001 [1], clause 9.6.13-2 | group | maxNrOfMembers |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/MID | Release 1 | TS-0001 [1], clause 9.6.13-2 | group | memberIDs |

TP/oneM2M/CSE/DMR/UPD/019

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a UPDATE Request target to TARGET_RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to RESULT_CONTENT Content and RESOURCE_TYPE resource containing valid attribute } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) and no Content } |  | IUT  AE |

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/2 | TS-0004 [2], clause 6.3.4.2.7 | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/3 |  | 3 (attributes and hierarchichal address) |

TP/oneM2M/CSE/DMR/UPD/020

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns nothing of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 0 (nothing) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 0 (nothing) Content and RESOURCE_TYPE resource containing valid attribute } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and no Content } |  | IUT  AE |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/020_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/020_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/UPD/020_ACP | Release 2 | 1 (accessControlPolicy) |

TP/oneM2M/CSE/DMR/UPD/021

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/021 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 1 (attributes) Content and RESOURCE_TYPE resource containing valid attribute } |  | IUT  AE |  |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references } |  | IUT  AE |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/021_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/021_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/UPD/021_ACP | Release 2 | 1 (accessControlPolicy) |

TP/oneM2M/CSE/DMR/UPD/022

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/022 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only modified attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 9 (modified attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 9 (modified attributes) Content and RESOURCE_TYPE resource containing valid ATTRIBUTE attribute set to VALUE } |  | IUT  AE |  |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing modified attributes and no ATTRIBUTE attribute and no hierarchichal address and no child resources and no child resource references } |  | IUT  AE |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/022_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/022_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/UPD/022_ACP | Release 2 | 1 (accessControlPolicy) |

7.2.2.3.3	CREATE Operation

TP/oneM2M/CSE/DMR/CRE/001

| TP Id | TP/oneM2M/CSE/DMR/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource on the TARGET_RESOURCE_ADDRESS of type PARENT_RESOURCE_TYPE with resource name not provided |  |
| Reference | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to RESOURCE_TYPE and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing no resourceName attribute } | IUT AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing Valid resourceName attribute } | IUT AE |

| TP Id |  | PARENT_RELEASE |  | Reference |  | RESOURCE_TYPE |  | PARENT_RESOURCE_TYPE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/CB |  | Release 1 |  | TS-0001 [1], clause 10.2.4.1 |  | 3 (container) |  | 5 (CSEBase) |  |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/AE |  | Release 1 |  | TS-0001 [1], clause 10.2.4.1 |  | 3 (container) |  | 2 (AE) |  |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/CNT |  | Release 1 |  | TS-0001 [1], clause 10.2.4.1 |  | 3 (container) |  | 3 (container) |  |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/CB |  | Release 1 |  | TS-0001 [1], clause 10.2.21.1 |  | 1 (accessControlPolicy) |  | 5 (CSEBase) |  |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/AE |  | Release 1 |  | TS-0001 [1], clause 10.2.21.1 |  | 1 (accessControlPolicy) |  | 2 (AE) |  |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CB |  | Release 1 |  | TS-0001 [1], clause 10.2.11.2 |  | 23 (subscription) |  | 5 (CSEBase) |  |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/AE |  | Release 1 |  | TS-0001 [1], clause 10.2.11.2 |  | 23 (subscription) |  | 2 (AE) |  |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CNT |  | Release 1 |  | TS-0001 [1], clause 10.2.11.2 |  | 23 (subscription) |  | 3 (container) |  |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/ACP |  | Release 1 |  | TS-0001 [1], clause 10.2.11.2 |  | 23 (subscription) |  | 1 (accessControlPolicy) |  |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CSR |  | Release 1 |  | TS-0001 [1], clause 9.6.4-1 |  | 23 (subscription) |  | 16 (remoteCSE) |  |
| TP/oneM2M/CSE/DMR/CRE/001_CIN/CNT |  | Release 1 |  | TS-0001 [1], clause 10.2.4.1 |  | 4 (contentInstance) |  | 3 (container) |  |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/CB |  | Release 1 |  | TS-0001 [1], clause 10.2.7.2 |  | 9 (group) |  | 5 (CSEBase) |  |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/AE |  | Release 1 |  | TS-0001 [1], clause 10.2.7.2 |  | 9 (group) |  | 2 (AE) |  |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/GRP |  | Release 1 |  | TS-0001 [1], clause 10.2.11.2 |  | 23 (subscription) |  | 9 (group) |  |
| TP/oneM2M/CSE/DMR/CRE/001_PCH/AE |  | Release 2 |  | TS-0001 [1], clause 10.2.13.2 |  | 15 (pollingChannel) |  | 2 (AE) |  |

TP/oneM2M/CSE/DMR/CRE/002

| TP Id | TP/oneM2M/CSE/DMR/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with a non-existing resource name provided by AE |  |
| Reference | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the AE containing no child resource containing resourceName attribute set to NAME } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource containing resourceName attribute set to NAME } | IUT AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/002_CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/002_ACP | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/002_SUB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/002_GRP | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/002_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/002_FLXC | Release 2 | TS-0001 [1], clause 10.2.29.1 | 28 (flexContainer) |

TP/oneM2M/CSE/DMR/CRE/003

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource with an existing resource name provided by AE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the AE containing a child resource having resourceName attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource containing resourceName attribute set to NAME } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4105 (CONFLICT) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/003_CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/003_ACP | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/003_SUB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/003_GRP | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/003_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/CRE/004

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource when AE has no privileges |  |
| Reference |  | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having no privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource } |  | IUT AE |
|  | then { the IUT does not create the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/004_CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/004_ACP | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/004_SUB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/004_GRP | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/004_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/CRE/005

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of container resource when the RW ATTRIBUTE is provided with an invalid value |  |
| Reference |  | TS-0001 [1], clause 10.2.4.1, TS-0004 [2], clause 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing an invalid RW ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/005_MNI | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/005_MBS | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/005_MIA | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | maxInstanceAge |

TP/oneM2M/CSE/DMR/CRE/006

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns a default value to RW ATTRIBUTE in the response of a RESOURCE_TYPE resource CREATE Request where no a RW ATTRIBUTE is provided by AE |  |
| Reference |  | TS-0001 [1], clause 10.2.4.1, TS-0004 [2], clause 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Content containing RESOURCE_TYPE resource representation } |  | IUT AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute } |  | IUT AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/006_CNT/MNI | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/006_CNT/MBS | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/006_CNT/MIA | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | container | maxInstanceAge |

TP/oneM2M/CSE/DMR/CRE/007

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Checks that the IUT accepts a newly created RESOURCE_TYPE when the currentNrOfInstances exceeds the field value set in maxNrOfInstances in the PARENT_RESOURCE_TYPE resource by removing enough of the oldest RESOURCE_TYPE resources to allow the creation of the new RESOURCE_TYPE resource |  |
| Reference |  | TS-0001 [1], clause 10.2.19.2, TS-0004 [2], clause 7.4.8.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource containing currentNrOfInstances attribute set to MAX_NUMBER_OF_INSTANCES and maxNrOfInstances attribute set to MAX_NUMBER_OF_INSTANCES and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Content containing RESOURCE_TYPE resource representation } |  | IUT AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT removes the oldest RESOURCE_TYPE resources and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

| TP Id | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/007_CNT/CIN | container | contentInstance |

TP/oneM2M/CSE/DMR/CRE/008

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Checks that the IUT accepts a newly created RESOURCE_TYPE resource when the currentByteSize exceeds the field value set in maxByteSize in the PARENT_RESOURCE_TYPE resource by removing enough of the oldest RESOURCE_TYPE resources to allow the creation of the new RESOURCE_TYPE resource.. |  |
| Reference |  | TS-0001 [1], clause 10.2.19.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource containing maxByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and currentByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Content containing RESOURCE_TYPE resource with size less than MAX_BYTE_SIZE_OF DATA } |  | IUT AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT removes the oldest contentInstance resources and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

| TP Id | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/008_CNT/CIN | container | contentInstance |

TP/oneM2M/CSE/DMR/CRE/009

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT increaments and then copies the field value of attribute stateTag in parent container resource when contentInstance resource is created as the direct child of the parent container |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2, TS-0004 [2], clause 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource CONTAINER_RESOURCE_ADDRESS containing stateTag attribute and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 4 (contentInstance) and Content containing contentInstance resource representation } |  | IUT AE |
|  | then { the IUT increments stateTag attribute of CONTAINER_RESOURCE_ADDRESS resource andthe IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/CRE/010

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of a container resource named "la" as a direct child of a container resource |  |
| Reference |  | TS-0001 [1], clause 10.2.22, TS-0004 [2], clause 7.4.28.2.2 and clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing Container resource containing resourceName attribute set to "la" } |  | IUT AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/CRE/011

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of a container resource named "ol" as a direct child of a container resource |  |
| Reference |  | TS-0001 [1], clause 10.2.23, TS-0004 [2], clause 7.4.29.2.2 and clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing resourceName attribute set to "ol" } |  | IUT AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/CRE/012

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute OPTIONAL_ATTRIBUTE provided |  |
| Reference |  | TS-0001 [1], clause 10.1.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing valid OPTIONAL_ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/ACPI | Release 1 | PICS_CNT_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | 3 (container) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/ET | Release 1 | PICS_CNT_ET | TS-0001 [1], clause 9.6.1.3.1-1 | 3 (container) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/MNI | Release 1 | PICS_CNT_MNI | TS-0001 [1], clause 9.6.6-2 | 3 (container) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/MBS | Release 1 | PICS_CNT_MBS | TS-0001 [1], clause 9.6.6-2 | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/MIA | Release 1 | PICS_CNT_MIA | TS-0001 [1], clause 9.6.6-2 | 3 (container) | maxInstanceAge |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/OR | Release 1 | PICS_CNT_OR | TS-0001 [1], clause 9.6.6-2 | 3 (container) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/CR | Release 1 | PICS_CNT_CR | TS-0001 [1], clause 9.6.6-2 | 3 (container) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/ET | Release 1 | PICS_CIN_ET | TS-0001 [1], clause 9.6.1.3.1-1 | 4 (contentInstance) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/LBL | Release 1 | PICS_CIN_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 4 (contentInstance) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/CR | Release 1 | PICS_CIN_CR | TS-0001 [1], clause 9.6.7-2 | 4 (contentInstance) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/CNF | Release 1 | PICS_CIN_CNF | TS-0001 [1], clause 9.6.7-2 | 4 (contentInstance) | contentInfo |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/OR | Release 1 | PICS_CIN_OR | TS-0001 [1], clause 9.6.7-2 | 4 (contentInstance) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/012_ACP/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | 1 (accesControlPolicy) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 1 (accesControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/ACPI | Release 1 | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | 23 (subscription) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | 23 (subscription) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/ENC | Release 1 | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/EXC | Release 1 | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | expirationCounter |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/GPI | Release 1 | PICS_SUB_GPI | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | groupID |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NFU | Release 1 | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/BN | Release 1 | PICS_SUB_BN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | batchNotify |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/RL | Release 1 | PICS_SUB_RL | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | rateLimit |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/PSN | Release 1 | PICS_SUB_PSN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | preSubscriptionNotify |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/PN | Release 1 | PICS_SUB_PN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | pendingNotification |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NSP | Release 1 | PICS_SUB_NSP | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/LN | Release 1 | PICS_SUB_LN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | latestNotify |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NCT | Release 1 | PICS_SUB_NCT | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationContentType |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NEC | Release 1 | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationEventCat |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/CR | Release 1 | PICS_SUB_CR | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/SU | Release 1 | PICS_SUB_SU | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | subscriberURI |

TP/oneM2M/CSE/DMR/CRE/013

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of a RESOURCE_TYPE resource with error "NOT_ACCEPTABLE" when contentSize exceeds maxByteSize of the PARENT_RESOURCE_TYPE |  |
| Reference |  | TS-0001 [1], clause 10.2.19, TS-0004 [2], clause 7.4.7.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource having maxByteSize attribute set to MAX_BYTE_SIZE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing valid content with size larger than MAX_BYTE_SIZE } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | IUT AE |

| TP Id | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/013_CNT/CIN | container | contentInstance |

TP/oneM2M/CSE/DMR/CRE/014

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates the currentByteSize attribute in a PARENT_RESOURCE_TYPE when a new RESOURCE_TYPE is created. |  |
| Reference |  | TS-0001 [1], clause 10.2.19.2, TS-0004 [2], clause 7.4.8.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource at PARENT_RESOURCE_ADDRESS containing maxByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and currentByteSize attribute set to 0 and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Content containing RESOURCE_TYPE resource containing valid content attribute with size equal to DATA_SIZE } |  | IUT AE |
|  | then { the IUT increments currentByteSize attribute of PARENT_RESOURCE_ADDRESS resource by DATA_SIZE and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED)} |  | IUT AE |

| TP Id | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/014_CNT/CIN | container | contentInstance |

TP/oneM2M/CSE/DMR/CRE/015

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully after a CREATE request an empty content of created resource when the ResultContent set to 0 (Nothing) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, clause 8.1.3 and clause 10.1.1.1, TS-0004 [2], clause 6.3.4.2.7 |  |
| Parent Release |  | PARENT_RELEASE |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Result Content set to 0 (nothing) and Content containing RESOURCE_TYPE resource representation } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and no Content } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/015_CNT | Release 2 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/015_GRP | Release 2 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/015_ACP | Release 2 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/015_SUB | Release 2 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/015_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/CRE/016

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully after a CREATE request only hierarchichal address of the created resource when the Result Content is set to 2 (hierarchical address) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, clause 8.1.3 and clause 10.1.1.1, TS-0004 [2], clause 6.3.4.2.7 |  |
| Parent Release |  | PARENT_RELEASE |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Result Content set to 2 (hierarchical address) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing Hierarchical address of the RESOURCE_TYPE resource } |  | IUT  AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/016_CNT | Release 2 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/016_GRP | Release 2 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/016_ACP | Release 2 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/016_SUB | Release 2 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/016_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/CRE/017

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully after a CREATE request only attributes and hierarchichal address of the created resource when Result Content is set 3 (attributes and hierarchichal address) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, clause 8.1.3 and clause 10.1.1.1, TS-0004 [2], clause 6.3.4.2.7 |  |
| Parent Release |  | PARENT_RELEASE |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Result Content set to 3 (attributes and hierarchichal address) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing resource element containing Hierarchical address of the RESOURCE_TYPE resource and RESOURCE_TYPE resource representation } |  | IUT  AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/017_CNT | Release 2 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/017_GRP | Release 2 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/017_ACP | Release 2 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/017_SUB | Release 2 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/017_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/CRE/019

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/019 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 1 (attributes) Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |  |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references } |  | IUT  AE |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/019_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/019_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/019_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/019_PCH | Release 2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/019_SUB | Release 2 | 23 (subscription) |

TP/oneM2M/CSE/DMR/CRE/020

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/020 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only modified attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 9 (modified attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 1 (attributes) Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |  |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing modified attributes } |  | IUT  AE |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/020_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/020_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/020_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/020_PCH | Release 2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/020_SUB | Release 2 | 23 (subscription) |

7.2.2.3.4	DELETE Operation

TP/oneM2M/CSE/DMR/DEL/001

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a RESOURCE_TYPE resource |  |
| Reference |  | TS-0001 [1], clause 10.1.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/001_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/001_ACP | Release 1 | TS-0001 [1], clause 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/001_SUB | Release 1 | TS-0001 [1], clause 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/001_CIN | Release 1 | TS-0001 [1], clause 10.2.4.1 | 4 (contentInstance) |
| TP/oneM2M/CSE/DMR/DEL/001_GRP | Release 1 | TS-0001 [1], clause 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/001_PCH | Release 2 | TS-0001 [1], clause 10.2.13.5 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/DEL/002

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS without having privileges for the DELETE operation |  |
| Reference |  | TS-0001 [1], clause 10.1.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having no privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT does not delete the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/002_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/002_ACP | Release 1 | TS-0001 [1], clause 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/002_SUB | Release 1 | TS-0001 [1], clause 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/002_GRP | Release 1 | TS-0001 [1], clause 10.2.7.5 | 9 (group) |

TP/oneM2M/CSE/DMR/DEL/003

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference |  | TS-0001 [1], clause 10.1.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT does not delete the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/DEL/004

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when AE tries to delete a child resource of a RESOURCE_TYPE resource when this resource has been already deleted |  |
| Reference |  | TS-0001 [1], clause 10.1.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS and the AE having deleted the TARGET_RESOURCE_ADDRESS resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/004_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/004_ACP | Release 1 | TS-0001 [1], clause 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/004_SUB | Release 1 | TS-0001 [1], clause 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/004_GRP | Release 1 | TS-0001 [1], clause 10.2.7.5 | 9 (group) |

TP/oneM2M/CSE/DMR/DEL/005

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the stateTag attribute of a RESOURCE_TYPE resource is increased when a child resource is deleted |  |
| Reference |  | TS-0001 [1], clause 10.1.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing a child resource and the AE having privileges to perform DELETE operation on the TARGET_CHILD_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT increments the stateTag attribute of the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/005_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |

TP/oneM2M/CSE/DMR/DEL/006

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT decreases the field value of attribute currentNrOfInstances and currentByteSize of parent RESOURCE_TYPE when a latest RESOURCE_TYPE resource is deleted successfully |  |
| Reference |  | TS-0001 [1], clause 10.2.19.5, TS-0004 [2], clause 7.4.8.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a PARENT_RESOURCE_TYPE resource containing currentNrOfInstances attribute set to NR_OF_INSTANCES and currentByteSize attribute set to BYTE_SIZE_OF_DATA and a RESOURCE_TYPE resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT decreases the currentNrOfInstances attribute of PARENT_RESOURCE_TYPE resource and the IUT decreases the currentByteSize attribute of PARENT_RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/006_CNT/CIN | Release 1 | TS-0001 [1], clause 10.2.4.4 | 4 (contentInstance) | 3 (container) |

TP/oneM2M/CSE/DMR/DEL/007

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the DELETE Request of the latest contentInstance resource target to a container resource |  |
| Reference |  | TS-0001 [1], clause 10.2.22.2, TS-0004 [2], clause 7.4.28.2.5 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform DELETE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT deletes the CONTENT_INSTANCE_2_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/DEL/008

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the DELETE request of a latest resource in a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 [1], clause 10.2.22.2, TS-0004 [2], clause 7.4.28.2.5 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/DEL/009

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the DELETE Request of the oldest contentInstance resource target to a container resource |  |
| Reference |  | TS-0001 [1], clause 10.2.23.2, TS-0004 [2], clause 7.4.29.2.5 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform DELETE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT deletes the CONTENT_INSTANCE_1_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT AE |

TP/oneM2M/CSE/DMR/DEL/010

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the DELETE request of an oldest resource in a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 [1], clause 10.2.23.2, TS-0004 [2], clause 7.4.29.2.5 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

TP/oneM2M/CSE/DMR/DEL/011

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a DELETE Request target to TARGET_RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to RESULT_CONTENT no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) and no Content } |  | IUT  AE |

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/2 | TS-0004 [2], clause 6.3.4.2.7 | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/3 |  | 3 (attributes and hierarchichal address) |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/9 |  | 9 (modified attributes) |

TP/oneM2M/CSE/DMR/DEL/012

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns nothing of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 0 (nothing) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 0 (nothing) no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and no Content } |  | IUT  AE |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/012_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/012_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/012_ACP | Release 2 | 1 (accessControlPolicy) |

TP/oneM2M/CSE/DMR/DEL/013

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/013 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to 1 (attributes) no Content } |  | IUT  AE |  |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references } |  | IUT  AE |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/013_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/013_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/013_ACP | Release 2 | 1 (accessControlPolicy) |

7.2.2.3.5	BASIC Operation

TP/oneM2M/CSE/DMR/001

| TP Id |  | TP/oneM2M/CSE/DMR/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the request to the hosting CSE when the To parameter contains the hosting CSE-ID which is known by the IUT (a descendant CSE or its Registrar CSE) |  |
| Reference |  | TS-0001 [1], clause 8.2.1.1 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to a Hosting CSE and the IUT having registered the AE and the Hosting CSE containing a resource representation on TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on the Hosting CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from the AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a OPERATION Request to the Hosting CSE } |  | IUT  CSE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DMR/001_CRE | CREATE |
| TP/oneM2M/CSE/DMR/001_RET | RETRIEVE |
| TP/oneM2M/CSE/DMR/001_UPD | UPDATE |
| TP/oneM2M/CSE/DMR/001_DEL | DELETE |

TP/oneM2M/CSE/DMR/002

| TP Id |  | TP/oneM2M/CSE/DMR/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the response from the hosting CSE after retargeting an OPERATION Request |  |
| Reference |  | TS-0001 [1], clause 8.2.1.1 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the Hosting CSE and the IUT having registered the AE and the Hosting CSE containing a resource representation on TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on the Hosting CSE and the IUT having received a valid OPERATION Request from the AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and the IUT sends the OPERATION Request to the Hosting CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Response from the Hosting CSE } |  | IUT  CSE |
|  | then { the IUT sends a valid Response to the AE } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DMR/002_CRE | CREATE |
| TP/oneM2M/CSE/DMR/002_RET | RETRIEVE |
| TP/oneM2M/CSE/DMR/002_UPD | UPDATE |
| TP/oneM2M/CSE/DMR/002_DEL | DELETE |

7.2.2.4	Subscription and Notification (SUB)

7.2.2.4.1	CREATE Operation

TP/oneM2M/CSE/SUB/CRE/001

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the target subscribed-to RESOURCE_TYPE resource is not subscribable. |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT having a resource PARENT_RESOURCE_ADDRESS not being subscribable RESOURCE_TYPE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE-ID and Content containing subscription resource representation } |  | IUT AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 5203 (TARGET_NOT_SUBSCRIBABLE) and no Content } |  | IUT AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/SUB/CRE/001_CIN | TS-0001 [1], clause 9.6.7 | 4 (contentInstance) |
| TP/oneM2M/CSE/SUB/CRE/001_SUB | TS-0001 [1], clause 9.6.8 | 23 (subscription) |

TP/oneM2M/CSE/SUB/CRE/002

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the originator does not have privileges for retrieving the subscribed-to resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute pointing to accessControlPolicy resource containing privileges attribute containing accessControlRule attribute containing accessControlOriginators attribute indicating AE-ID and accessControlOperations attribute not indicating RETRIEVE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE-ID and Content containing subscription resource representation } |  | IUT AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) and no Content } |  | IUT AE |

TP/oneM2M/CSE/SUB/CRE/003

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT cannot send the Notify request to the notificationURI |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT not being able to send the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE-ID and Content containg subscription resource containing notificationURI attribute set to NOTI_URI_ADDRESS } |  | IUT AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 5204 (SUBSCRIPTION_VERIFICATION_INITIATION_FAILED) and no Content } |  | IUT AE |

TP/oneM2M/CSE/SUB/CRE/004

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT stores Originator ID in the notification creator attribute when a <SUBSCRIPTION> creation request which needs verification is received and the notificationURI is not the Originator. |  |
| Reference |  | TS-0001 [1], clause 10.2.12.1, TS-0004 [2], clause 7.4.9.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and Content containing subscription resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid NOTIFY Request containing Content containing notification message containing verificationRequest attribute set to TRUE and creator attribute set to AE1_RESOURCE_ADDRESS } |  | IUT  AE2 |

TP/oneM2M/CSE/SUB/CRE/005

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when notificationEventType attribute is set to "Create_of_Direct_Child_Resource" and a create operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 3 (Create_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing CHILD_RESOURCE_TYPE resource containing resourceName attribute set to NAME } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and the IUT sends a NOTIFY Request contaning Content containing Notification message containing notificationEvent attribute containing representation attribute containing CHILD_RESOURCE_TYPE resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/CRE/006

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a subscription resource with the eventNotificationCriteria attribute which includes a CONDITION_TAG condition provided |  |
| Reference |  | TS-0001 [1], clause 9.6.8 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to subscription and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing supscription resource containing eventNotificationCriteria attribute containing valid CONDITION_TAG condition } |  | IUT AE |
|  | then { the IUT creates the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

| TP Id | Reference | CONDITION_TAG |
| --- | --- | --- |
| TP/oneM2M/CSE/SUB/CRE/006_CRB | TS-0001 [1], clause 9.6.8 | createdBefore |
| TP/oneM2M/CSE/SUB/CRE/006_CRA | TS-0001 [1], clause 9.6.8 | createdAfter |
| TP/oneM2M/CSE/SUB/CRE/006_MS | TS-0001 [1], clause 9.6.8 | modifiedSince |
| TP/oneM2M/CSE/SUB/CRE/006_US | TS-0001 [1], clause 9.6.8 | unmodifiedSince |
| TP/oneM2M/CSE/SUB/CRE/006_STS | TS-0001 [1], clause 9.6.8 | stateTagSmaller |
| TP/oneM2M/CSE/SUB/CRE/006_STB | TS-0001 [1], clause 9.6.8 | stateTagBigger |
| TP/oneM2M/CSE/SUB/CRE/006_EXB | TS-0001 [1], clause 9.6.8 | expireBefore |
| TP/oneM2M/CSE/SUB/CRE/006_EXA | TS-0001 [1], clause 9.6.8 | expireAfter |
| TP/oneM2M/CSE/SUB/CRE/006_SZA | TS-0001 [1], clause 9.6.8 | sizeAbove |
| TP/oneM2M/CSE/SUB/CRE/006_SZB | TS-0001 [1], clause 9.6.8 | sizeBelow |
| TP/oneM2M/CSE/SUB/CRE/006_ET | TS-0001 [1], clause 9.6.8 | eventType |
| TP/oneM2M/CSE/SUB/CRE/006_OM | TS-0001 [1], clause 9.6.8 | operationMonitor |
| TP/oneM2M/CSE/SUB/CRE/006_ATR | TS-0001 [1], clause 9.6.8 | attribute |

7.2.2.4.2	DELETE Operation

TP/oneM2M/CSE/SUB/DEL/001

| TP Id |  | TP/oneM2M/CSE/SUB/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the AE2_RESOURCE_ADDRESS of the subscriberURI attribute when the SUBSCRIPTION_RESOURCE_ADDRESS <subscription> resource is deleted |  |
| Reference |  | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription resource containing subscriberURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE1 containing To set to SUBSCRIPTION_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing subscriptionDeletion attribute set to TRUE and subscriptionReference attribute set to AE2_RESOURCE_ADDRESS } |  | IUT  AE1 IUT AE2 |

TP/oneM2M/CSE/SUB/DEL/002

| TP Id |  | TP/oneM2M/CSE/SUB/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventNotificationCriteria/notificationEventType attribute is set to "Delete_of_Resource" and an delete operation has been performed the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 2 (Delete_of_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing a valid representation attribute } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/DEL/003

| TP Id |  | TP/oneM2M/CSE/SUB/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventNotificationCriteria/notificationEventType attribute is set to "Delete_of_Direct_Child_Resource" and an delete operation has been performed on a child resource of the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing a CHILD_RESOURCE_ADDRESS child resource and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 4 (Delete_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE1 containing To set to CHILD_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing CHILD_RESOURCE_TYPE resource} |  | IUT  AE1 IUT  AE2 |

7.2.2.4.3	UPDATE Operation

TP/oneM2M/CSE/SUB/UPD/001

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends Notify request to the subscriber resource when an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT AE1 IUT AE2 |

TP/oneM2M/CSE/SUB/UPD/002

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to "modified attributes" and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 2 (modified attributes) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/UPD/003

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to "ResourceID" and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 3 (ResourceID) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing subscribed-to ResourceID } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/UPD/004

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT decreases the expirationCounter attribute of a subscription resource when the Hosting CSE of the subscribed-to resource successfully sends the notification request to subscriber resource(s) |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to MAX_NUMBER_NOTIFICATION and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT decreases the expirationCounter attribute of the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/UPD/005

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes the subscription resource when the the expirationCounter meets zero |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to 1 and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT decreases the expirationCounter attribute of the subscription resource and the IUT deletes the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/UPD/006

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT does no't send a Notify request to the subscriber resource when the "attribute" condition tag of eventNotificationCriteria attribute is set to LIST_OF_ATTRIBUTE and does no't contain the ATTRIBUTE_NAME attribute updated. |  |
| Reference |  | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to LIST_OF_ATTRIBUTE containing no ATTRIBUTE_NAME attribute and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT does no't send a NOTIFY Request } |  | IUT  AE1 |

TP/oneM2M/CSE/SUB/UPD/007

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends batched notifications to the subscriber resource when the number value of the batchNotify attribute is set to NUMBER and when this number have been reached |  |
| Reference |  | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing number attribute set to NUMBERand duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (NUMBER– 1) valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing a valid ATTRIBUTE_NAME attribute } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends NUMBER times a NOTIFY Request contaning Content containing notification message containing a valid notificationEvent attribute } |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/UPD/008

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the number value of the batchNotify attribute is set to NUMBER and when this number has been reached |  |
| Reference |  | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing number attribute set to NUMBERand duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (NUMBER– 1) valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing a valid ATTRIBUTE_NAME attribute } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing the latest notification message containing a valid notificationEvent attribute} |  | IUT  AE1 IUT  AE2 |

TP/oneM2M/CSE/SUB/UPD/009

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the "attribute" condition tag of eventNotificationCriteria attribute is set to ATTRIBUTE_NAME and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

7.2.2.4.4	NOTIFY Operation

TP/oneM2M/CSE/SUB/NTF/001

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response containing Response Status Code set to 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) } |  | IUT AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) and no Content } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/002

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE |  |
| Reference |  | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response containing Response Status Code set to 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) } |  | IUT AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) and no Content } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/003

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends batched notifications to the subscriber resource when the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference |  | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing duration attrinbute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to subscribed-to resource and timer has not reached TIME_LIMIT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the timer expires after the TIME_LIMIT duration value } |  | IUT  AE1 |
|  | then { The IUT sends a NOTIFY Request contaning Content containing notification message containing a valid notificationEvent attribute } |  | IUT  AE2 |

TP/oneM2M/CSE/SUB/NTF/004

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference |  | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to subscribed-to resource and timer has not reached TIME_LIMIT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the timer expires after the TIME_LIMIT duration value } |  | IUT  AE1 |
|  | then { The IUT sends a NOTIFY Request contaning Content containing the latest notification message containing a valid notificationEvent attribute } |  | IUT  AE2 |

TP/oneM2M/CSE/SUB/NTF/005

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest cached notification after IUT escapes from connectionless state when the pendingNotification is set to 1 (sendLatest) |  |
| Reference |  | TS-0001 [1], clause 9.6.8 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and pendingNotification attribute set to 1 (sendLatest) and the AE2 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT being in the "connectionless state" with AE2 and the IUT having frequently received from AE1 a valid UPDATE Requests to subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT escapes from the "connectionless state" } |  | NA |
|  | then { The IUT sends a valid NOTIFY Request contaning Content containing the latest notification message } |  | IUT  AE2 |

TP/oneM2M/CSE/SUB/NTF/006

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the notifications of cached notification messages after IUT escapes from connectionless state when the pendingNotification is set to 2 (sendAllPending) |  |
| Reference |  | TS-0001 [1], clause 9.6.8 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and pendingNotification attribute set to 2 (sendAllPending) and the AE2 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT being in the "connectionless state" with AE2 and the IUT having received from AE1 a NUMBER of valid UPDATE Requests to subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT escapes from the "connectionless state" } |  | NA |
|  | then { The IUT sends NUMBER times a valid NOTIFY Request contaning Content containing notification message containing a valid notificationEvent attribute } |  | IUT  AE2 |

TP/oneM2M/CSE/SUB/NTF/007

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the notification to the AE according to pointOfAccess in the <AE> resource. |  |
| Reference |  | TS-0001 [1], clause 9.6.5 and clause 9.3.2.3.1, TS-0004 [2], clause 7.3.3.9 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created the AE resource containing POINT_OF_ACCESS attribute and REQUEST_REACHABILITY set to true } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid NOTIFY from CSE containing To set to AE_RESOURCE_ADDRESS From set to CSE_ID } |  | IUT CSE |
|  | then { the IUT sends the valid NOTIFY to POINT_OF_ACCESS } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/008

| TP Id | TP/oneM2M/CSE/SUB/NTF/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the notification to the AE according to pointOfAccess in the <AE> resource and forward the response back to the CSE. |  |
| Reference | TS-0001 [1], clause 9.6.5 and clause 9.3.2.3.1, TS-0004 [2], clause 7.3.3.9 |  |
| Config Id | CF02 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received a valid NOTIFY from CSE and The IUT having sent the NOTIFY to the AE } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid NOTIFY RESPONSE from the AE } | IUT AE |
|  | then { the IUT sends the valid NOTIFY RESPONSE to the CSE } | IUT CSE |

7.2.2.5	Group Management (GMG)

7.2.2.5.1	CREATE Operation

TP/oneM2M/CSE/GMG/CRE/001

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the group resource when member ID exceed max number of members. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to LIST_OF_MEMBER_RESOURCE_IDS and maxNrOfMembers attribute set to MAX_NR_MEMBERS less than the number of memerIDs set in memberIDs attribute } |  | IUT AE |
|  | then { the IUT does not create the group resource and the IUT sends a valid Response containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT AE |

TP/oneM2M/CSE/GMG/CRE/002

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the group resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT registered with remote CSE CSE1 and the AE having created a resource at MEMBER_RESOURCE_ADDRESS_1 at another CSE1 containing resourceType set to RESOURCE_TYPE and the IUT not having privileges to perform RETRIEVE operation on the MEMBER_RESOURCE_ADDRESS_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberType attribute set to RESOURCE_TYPE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT AE |
|  | then { the IUT does not create the group resource and the IUT sends a valid Response containing Response Status Code set to 5105 (RECEIVER_HAS NO_PRIVILEGE) } |  | IUT AE |

TP/oneM2M/CSE/GMG/CRE/003

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects the presence of duplicate member IDs during the creation of the <group> resource and removes the duplicate member IDs prior to creation of the <group> resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having created a resource at MEMBER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS, MEMBER_RESOURCE_ADDRESS } |  | IUT AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS } |  | IUT AE |
| NOTE: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type. |  |  |  |

TP/oneM2M/CSE/GMG/CRE/004

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT validates the resource type during the creation of the <group> resource when memberType attribute is not 'mixed'. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS and memberType attribute set to RESOURCE_TYPE } |  | IUT AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) Content containing group resource containing memberTypeValidated attribute set to TRUE } |  | IUT AE |
| NOTE: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type. |  |  |  |

TP/oneM2M/CSE/GMG/CRE/005

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is SET_MIXED. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS and memberType attribute set to RESOURCE_TYPE_2 and consistencyStrategy attribute set to SET_MIXED } |  | IUT AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberType attribute set to MIXED } |  | IUT AE |
| NOTE: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type. |  |  |  |

TP/oneM2M/CSE/GMG/CRE/006

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_MEMBER,. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and consistencyStrategy attribute set to ABANDON_MEMBER } |  | IUT AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT AE |
| NOTE: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type. |  |  |  |

TP/oneM2M/CSE/GMG/CRE/007

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the group resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_GROUP. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and consistencyStrategy attribute set to ABANDON_GROUP } |  | IUT AE |
|  | then { the IUT does not create the group resource and the IUT sends a valid Response containing Response Status Code set to 4110 (GROUP_MEMBER_TYPE_INCONSISTENT) } |  | IUT AE |
| NOTE: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type. |  |  |  |

7.2.2.5.2	UPDATE Operation

TP/oneM2M/CSE/GMG/UPD/001

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update of the group resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT registered with remote CSE CSE1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 containing resourceType set to RESOURCE_TYPE and the IUT having created a <group> resource at GROUP_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the GROUP_RESOURCE_ADDRESS and the AE having created a resource at MEMBER_RESOURCE_ADDRESS_2 at another CSE1 and the IUT not having privileges to perform RETRIEVE operation on MEMBER_RESOURCE_ADDRESS_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT sends a valid valid Response containing Response Status Code set to 5105 (RECEIVER_HAS_NO_PRIVILEGE) } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/002

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects the presence of duplicate member IDs during an update of the <group> resource and removes the duplicate member IDs prior to updating the <group> resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 and the IUT having created a <group> resource at GROUP_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and the AE having privileges to perform UPDATE operation on the GROUP_RESOURCE_ADDRESS and the IUT having created a resource at MEMBER_RESOURCE_ADDRESS_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to GROUP_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/003

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT validates the resource type during an UPDATE of the <group> resource when memberType attribute is not 'mixed'. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing group resource containing memberTypeValidated attribute set to TRUE } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/004

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the <group> resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is SET_MIXED. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to SET_MIXED and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberType attribute set to MIXED } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/005

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the group resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_MEMBER. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/006

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the update of the group resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_GROUP. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_GROUP and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT does not update the group resource and the IUT sends a valid Response containing Response Status Code set to 4110 (GROUP_MEMBER_TYPE_INCONSISTENT) } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/007

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects when the number of memberIDs exceeds the limitation of maxNrOfMembers. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and maxNrOfMembers attribute set to 2 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_3 having resourceType attribute set to RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_3 } |  | IUT AE |
|  | then { the IUT does not update the group resource and the IUT sends a valid Response containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/008

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects when the value provided for maxNrOfMembers attribute is less than the value of the currentNrOfMembers attribute. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and maxNrOfMembers attribute set to 2 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute set to RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing maxNrOfMembers attribute set to 1 } |  | IUT AE |
|  | then { the IUT does not update the group resource and the IUT sends a valid Response containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/009

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles validation of the resource type during an UPDATE of the <group> resource when memberIDs contains a sub-group on a temporarily unreachable Hosting CSE. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the remoteCSE having a group resource at MEMBER_RESOURCE_ADDRESS_2 containing memberType attribute set to RESOURCE_TYPE_1 and the remoteCSE having onlineStatus set to False } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing group resource containing memberTypeValidated attribute set to FALSE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |

TP/oneM2M/CSE/GMG/UPD/010

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles validation of the resource type during an UPDATE of the group resource when memberIDs contains a sub-group on a a previously unreachable Hosting CSE that has become reachable |  |
| Reference |  | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to FALSE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the AE having a subscription to TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the remoteCSE having a group resource at MEMBER_RESOURCE_ADDRESS_2 containing memberType attribute set to RESOURCE_TYPE_2 and the remoteCSE having onlineStatus set to False } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from remoteCSE containing To set to REMOTE_CSE_ADDRESS and From set to CSE-ID and Content containing remoteCSE resource containing onlineStatus attribute set to TRUE } |  | IUT AE |
|  | then { the IUT sends a NOTIFY Request containing Content containing group resource containing memberTypeValidated attribute set to TRUE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT AE |

7.2.2.5.3	RETRIEVE Operation

TP/oneM2M/CSE/GMG/RET/001

| TP Id |  | TP/oneM2M/CSE/GMG/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT performs a RETRIEVE request for each resource in memberIDs with a relative address appended to fanOutPoint that includes a virtual resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform RETRIEVE operation and memberType attribute set to 3 (container) and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to container and and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to container and and MEMBER_RESOURCE_ADDRESS1 and MEMBER_RESOURCE_ADDRESS2 containing a child resource containing resourceType attribute set to contentInstance } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt/la and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing TaggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1/la, Response for MEMBER_RESOURCE_ADDRESS2/la } |  | IUT AE |

7.2.2.5.4	BASIC OPERATION

TP/oneM2M/CSE/GMG/001

| TP Id |  | TP/oneM2M/CSE/GMG/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT allows a <group>/fopt OPERATION when the Originator has OPERATION permission specified in membersAccessControlPolicyIDs attribute in the group resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing aggregatedResponse containing Response for MEMBER_RESOURCE_ADDRESS1, Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/001_CRE | CREATE |
| TP/oneM2M/CSE/GMG/001_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/001_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/001_DEL | DELETE |

TP/oneM2M/CSE/GMG/002

| TP Id |  | TP/oneM2M/CSE/GMG/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT denies a <group>/fopt OPERATION when the Originator does not have OPERATION permission specified in membersAccessControlPolicyIDs in the group resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/002_CRE | CREATE |
| TP/oneM2M/CSE/GMG/002_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/002_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/002_DEL | DELETE |

TP/oneM2M/CSE/GMG/003

| TP Id |  | TP/oneM2M/CSE/GMG/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT allows a <group>/fopt OPERATION when the Originator has OPERATION permission specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the <group> resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to empty and accessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set 2000 (OK) and Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1, Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/003_CRE | CREATE |
| TP/oneM2M/CSE/GMG/003_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/003_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/003_DEL | DELETE |

TP/oneM2M/CSE/GMG/004

| TP Id |  | TP/oneM2M/CSE/GMG/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT rejects a <group>/fopt OPERATION when the Originator does not have OPERATION permission specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the group resource. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to empty and accessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/004_CRE | CREATE |
| TP/oneM2M/CSE/GMG/004_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/004_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/004_DEL | DELETE |

TP/oneM2M/CSE/GMG/005

| TP Id |  | TP/oneM2M/CSE/GMG/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT performs an OPERATION request for each resource in memberIDs with no relative address appended to it. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE privileges to perform OPERATION and memberType attribute set to AE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType set to AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000(OK) and Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/005_CRE | CREATE |
| TP/oneM2M/CSE/GMG/005_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/005_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/005_DEL | DELETE |

TP/oneM2M/CSE/GMG/006

| TP Id |  | TP/oneM2M/CSE/GMG/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT perfomrs an OPERATION request for each resource in memberIDs with a relative address appended to it. |  |
| Reference |  | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE and memberType attribute set to AE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing a child resource containing resourceType attribute set to container and resourceName attribute set to NAME and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing a child resource containing resourceType attribute set to container and resourceName attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt/NAME and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000(OK) and Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/006_CRE | CREATE |
| TP/oneM2M/CSE/GMG/006_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/006_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/006_DEL | DELETE |

TP/oneM2M/CSE/GMG/007

| TP Id |  | TP/oneM2M/CSE/GMG/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT denies a <group>/fanOutPoint RETRIEVE when there is no memberID in <group> resource. |  |
| Reference |  | TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE to perform all operations and no memberID attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4109 (NO_MEMBER) } |  | IUT AE |

7.2.2.6	Discovery (DIS)

7.2.2.6.1	RETRIEVE Operation

TP/oneM2M/CSE/DIS/001

| TP Id |  | TP/oneM2M/CSE/DIS/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list all discovered resource addresses |  |
| Reference |  | TS-0001 [1], clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation } |  |  |

TP/oneM2M/CSE/DIS/002

| TP Id |  | TP/oneM2M/CSE/DIS/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully appropriate list of discovered resource when the filter criteria is provided in the request |  |
| Reference |  | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation on the child resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing filterUsage set to 1 (Discovery Criteria) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation } |  | IUT AE |

TP/oneM2M/CSE/DIS/003

| TP Id |  | TP/oneM2M/CSE/DIS/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns the empty address list when no result matching with filter criteria is discovered |  |
| Reference |  | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation on the child resource and FILTER_CRITERIA_CONDITIONS not matching child resource } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList element set to empty list } |  |  |

TP/oneM2M/CSE/DIS/004

| TP Id |  | TP/oneM2M/CSE/DIS/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list of discovered resource addresses with Non-hierarchical addressing form when the Desired Identifier Result Type is provided in the request |  |
| Reference |  | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Desired Identifier Result Type set to 2 (unstructured) and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation containing unstructured addresses of discovered resources } |  |  |

TP/oneM2M/CSE/DIS/005

| TP Id |  | TP/oneM2M/CSE/DIS/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a discovery requests to the resource TARGET_RESOURCE_ADDRESS when AE has no privilege to perform the discovery operation on the resource TARGET_RESOURCE_ADDRESS |  |
| Reference |  | TS-0001 [1], clause 10.2.6.2 and clause 7.2.3.14, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing no child resources and the AE having no privileges to perform DISCOVERY operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList element set to empty list } |  | IUT AE |

TP/oneM2M/CSE/DIS/006

| TP Id |  | TP/oneM2M/CSE/DIS/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE sends requests to discover the resource TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference |  | TS-0001 [1], clause 10.2.6.2, TS-0004 [2], clause 7.3.3.14 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a containe resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  |  |

TP/oneM2M/CSE/DIS/007

| TP Id |  | TP/oneM2M/CSE/DIS/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds the originator with an error when the originator sends a request including an invalid format of filter criteria to discover the resource TARGET_RESOURCE_ADDRESS |  |
| Reference |  | TS-0001 [1], clause 10.2.6, TS-0004 [2], clause 7.3.3.6 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions |  | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS not being valid format } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

TP/oneM2M/CSE/DIS/008

| TP Id |  | TP/oneM2M/CSE/DIS/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds the originator with a success response when the originator sends a request to discover the resource TARGET_RESOURCE_ADDRESS including two conflicted filter criteria of different type FILTER_CRITERIA_CONDITION_1 and FILTER_CRITERIA_CONDITION_2 (E.G. createBefore < createdAfter) |  |
| Reference |  | TS-0001 [1], clause 6.2.5, TS-0004 [2], clause 7.3.3.17 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation on the child resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) and having conflict conditions } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList element set to empty list } |  | IUT AE |

| TP Id | REQ Reference | FILTER_CRITERIA _CONDITION_1 | FILTER_CRITERIA _CONDITION_2 |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DIS/008_CRB/CRA | REQ-0004-07241 | createdBefore | createdAfter |
| TP/oneM2M/CSE/DIS/008_MS/US | REQ-0004-07242 | unmodifiedSince | modifiedSince |
| TP/oneM2M/CSE/DIS/008_STS/STB | REQ-0004-07243 | stateTagSmaller | stateTagBigger |
| TP/oneM2M/CSE/DIS/008_EXB/EXA | REQ-0004-07244 | expireBefore | expireAfter |

TP/oneM2M/CSE/DIS/009

| TP Id |  | TP/oneM2M/CSE/DIS/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns the empty address list when resources match the filter criteria but they do not include DISCOVERY permission. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release |  | Release 1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE not having privileges to perform DISCOVERY operation on the child resource and FILTER_CRITERIA_CONDITIONS matching the child resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing filterUsage set to 1 (Discovery Criteria) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList element set to empty list } |  | IUT AE |

TP/oneM2M/CSE/DIS/010

| TP Id |  | TP/oneM2M/CSE/DIS/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list of discovered resource addresses with hierarchical addressing form when the Desired Identifier Result Type is provided in the request |  |
| Reference |  | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Desired Identifier Result Type set to 1 (structured) and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation containing structured addresses of discovered resources } |  | IUT AE |

TP/oneM2M/CSE/DIS/011

| TP Id |  | TP/oneM2M/CSE/DIS/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list of discovered resource addresses with hierarchical addressing form when the Desired Identifier Result Type is not provided in the request |  |
| Reference |  | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and no Desired Identifier Result Type parameter and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation containing structured addresses of discovered resources } |  | IUT AE |

7.2.2.7	Communication Management and Delivery Handling(CMDH)

7.2.2.7.1	Resource pollingChannel (PCH)

TP/oneM2M/CSE/PCH/001

| TP Id | TP/oneM2M/CSE/PCH/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which hosts <pollingChannel> resource forwards a Notify request successfully to a target AE, once the IUT receives a polling request from AE (retrieve request to <pollingChannelURI> resource) See the Note below. |  |
| Reference | TS-0001 [1], clause 10.2.13.6, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE1 and the AE having a child <pollingChannel> resource and the IUT having received a retrieve request from the AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a NOTIFY Request from the CSE containing To set to AE_RESOURCE_ADDRESS and From set to CSE_ID } | IUT CSE |
|  | then { the IUT sends a RETRIEVE Response to the AE containing Response Status Code set to 2000 (OK) Content containing the request received from the CSE } | IUT AE |
| NOTE: Based on figure 10.2.5.12-1 in TS-0001 [1] step 001, 002 and 003 in order. In this case, IUT is pending AE's polling request since there was no request to AE. |  |  |

TP/oneM2M/CSE/PCH/002

| TP Id | TP/oneM2M/CSE/PCH/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which hosts <pollingChannel> resource forwards a Notify request successfully to a target AE, once the IUT receives a polling request from the AE |  |
| Reference | TS-0001 [1], clause 10.2.13.6, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the AE having a child <pollingChannel> resource and the IUT having received a NOTIFY Request from CSE containing To set to AE_RESOURCE_ADDRESS and From set to CSE_ID } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a retrieve Request from the AE containing To set to POLLINGCHANNELURI _RESOURCE_ADDRESS } | IUT AE |
|  | then { the IUT sends a Response to the AE containing Response Status Code set to 2000 (OK) Content containing the Request received from the CSE } | IUT AE |
| NOTE: Based on figure 10.2.13.1-1 in TS-0001 [1] step 002 is followed by 001 and 003. The CSE's request to AE is pended and will be sent as soon as AE sends polling request to CSE. |  |  |

TP/oneM2M/CSE/PCH/003

| TP Id | TP/oneM2M/CSE/PCH/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT performs both forwarding the response to the CSE and sending response to AE after receiving a Notify Request from the AE sent to the <pollingChannelURI> resource |  |
| Reference | TS-0001 [1], clause 10.2.13.8, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the AE1 having a child <pollingChannel> resource and the IUT having received a retrieve Request from the AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and the IUT having received a NOTIFY Request from the CSE containing To set to AE_RESOURCE_ADDRESS and From set to CSE_ID and the IUT sends a Response to the AE containing Response Status Code set to 2000 (OK) Content containing the Request received from the CSE } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a NOTIFY Request from the AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and Content containing Response Status Code set to RESPONSE_STATUS_CODE } | IUT AE |
|  | then { the IUT sends a Response to the CSE Response Status Code set to RESPONSE_STATUS_CODE the IUT sends the valid NOTIFY Response to the AE } | IUT CSE IUT AE |
| NOTE: Based on figure 10.2.13.1-1 in TS-0001 [1] step 005 and 006. |  |  |

TP/oneM2M/CSE/PCH/004

| TP Id | TP/oneM2M/CSE/PCH/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which performs polling sends the Notify request to <pollingChannelURI> Hosting CSE after receiving response using polling channel |  |
| Reference | TS-0001 [1], clause 10.2.13.8, TS-0004 [2], clause 7.4.22.2.5 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to a CSE and the IUT having a <pollingChannel> resource and the IUT having sent a retrieve Request to the CSE To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and the CSE having received the Request containing To set to CSE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a polling Response from the CSE containing Response Status Code set to 2000 (OK) } | IUT CSE1 |
|  | then { the IUT sends a NOTIFY Request to the CSE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and Content containing Response Status Code set to RESPONSE_STATUS_CODE } | IUT CSE1 |
| NOTE: Based on figure 10.2.13.1-1 in TS-0001 [1] step 004 and 005. |  |  |

TP/oneM2M/CSE/PCH/005

| TP Id | TP/oneM2M/CSE/PCH/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a <pollingChannel> OPERATION of the AE when AE-ID is not same as the AE-ID of the parent resource |  |
| Reference | TS-0001 [1], clause 10.2.13.2, TS-0004 [2], clause 7.4.21 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE1 and the IUT having registered an AE2 and the IUT having created a pollingChannel resource on the AE1 } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid OPERATION Request from the AE2 containing To set to POLLING_CHANNEL_RESOURCE_ADDRESS and From set to AE2_ID } | IUT AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } | IUT AE2 |

| TP Id | Reference | OPERATION |
| --- | --- | --- |
| TP/oneM2M/CSE/PCH/005_RET | TS-0001 10.2.13.3 | RETRIEVE |
| TP/oneM2M/CSE/PCH/005_UPD | TS-0001 10.2.13.4 | UPDATE |
| TP/oneM2M/CSE/PCH/005_DEL | TS-0001 10.2.13.5 | DELETE |

TP/oneM2M/CSE/PCH/006

| TP Id | TP/oneM2M/CSE/PCH/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the response with a status to the CSE1 when the request expires according to its Request Expiration Timestamp |  |
| Reference | TS-0001 [1], clause 10.2.13.6 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the AE having a child <pollingChannel> resource and the IUT having received a RETRIEVE Request from AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and Request Expiration Timestamp set to VALUE } |  |
| Expected behaviour | Test events | Direction |
|  | when { Request Expiration Timestamp expires } | IUT |
|  | then { and the IUT sends a Response to the AE containing Response Status Code set to 4008 (REQUEST_TIMEOUT) } | IUT AE |

7.2.2.7.2	Response Type (RT)

7.2.2.7.2.1	nonBlockingRequestSynch (NBS)

TP/oneM2M/CSE/RT/NBS/001

| TP Id |  | TP/oneM2M/CSE/RT/NBS/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a Non-Blocking Synchronous Request OPERATION if the IUT does not support the <request> resource |  |
| Reference |  | TS-0001 [1], clause 8.2.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_SYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 1 (nonBlockingRequestSynch) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5206 (NON_BLOCKING_REQUEST_NOT_SUPPORTED) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBS/001_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBS/001_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBS/001_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBS/001_DEL | DELETE |

TP/oneM2M/CSE/RT/NBS/002

| TP Id |  | TP/oneM2M/CSE/RT/NBS/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds to a Non-Blocking Synchronous Request OPERATION with the reference of a <request> resource provided within the Content parameter of the Response |  |
| Reference |  | TS-0001 [1], clause 8.2.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_SYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 1 (nonBlockingRequestSynch) } |  | IUT AE |
|  | then { and the IUT sends a valid Response containing Response Status Code set to 1001 (ACCEPTED for n onBlockingRequestSynch) and Content containing URI attribute set to REQUEST_RESOURCE_ADDRESS } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBS/002_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBS/002_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBS/002_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBS/002_DEL | DELETE |

TP/oneM2M/CSE/RT/NBS/003

| TP Id |  | TP/oneM2M/CSE/RT/NBS/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the <request> resource after receiving a Non-Blocking Synchronous Request OPERATION and it contains the expected result in operationResult attribute |  |
| Reference |  | TS-0001 [1], clause 8.2.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_SYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having received a Non-Blocking Synchronous Request OPERATION and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type <request> under the AE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and ResponseType set to 3 (blockingRequest) no Content } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing request resource representation containing operationResult attribute containing expected response to OPERATION } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBS/003_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBS/003_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBS/003_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBS/003_DEL | DELETE |

7.2.2.7.2.2	nonBlockingRequestAsynch (NBA)

TP/oneM2M/CSE/RT/NBA/001

| TP Id |  | TP/oneM2M/CSE/RT/NBA/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds to a Non-Blocking Asynchronous Request OPERATION without a reference to a resource containing the context of the request if the IUT does not support the <request> resource |  |
| Reference |  | TS-0001 [1], clause 8.2.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 2 (nonBlockingRequestAsynch) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 1002 (ACCEPTED for nonBlockingRequestAsynch) and no Content } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBA/001_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBA/001_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBA/001_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBA/001_DEL | DELETE |

TP/oneM2M/CSE/RT/NBA/002

| TP Id |  | TP/oneM2M/CSE/RT/NBA/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds to a Non-Blocking Asynchronous Request OPERATION with the reference of a <request> resource provided within the Content parameter of the Response |  |
| Reference |  | TS-0001 [1], clause 8.2.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 2 (nonBlockingRequestAsynch) } |  | IUT AE |
|  | then { and the IUT sends a valid Response containing Response Status Code set to 1002 (ACCEPTED for nonBlockingRequestAsynch) and Content containing URI attribute set to REQUEST_RESOURCE_ADDRESS } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBA/002_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBA/002_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBA/002_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBA/002_DEL | DELETE |

TP/oneM2M/CSE/RT/NBA/003

| TP Id |  | TP/oneM2M/CSE/RT/NBA/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the result of the requested Non-Blocking Asynchronous OPERATION as notification to the Originator when no notification target list is provided |  |
| Reference |  | TS-0001 [1], clause 8.1.2, 8.2.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 2 (nonBlockingRequestAsynch) NotificationURI not set } |  | IUT AE |
|  | then { the IUT sends a valid Response (Ack) and the IUT sends a valid NOTIFY Request containing Content containing ResponsePrimitive representation indicating the response of the original Non-Blocking Asynchronous OPERATION containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/RT/NBA/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/RT/NBA/003_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/RT/NBA/003_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/RT/NBA/003_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/RT/NBA/004

| TP Id |  | TP/oneM2M/CSE/RT/NBA/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT does not send the result of the requested Non-Blocking Asynchronous OPERATION as notification to the Originator when an empty notification target list is provided |  |
| Reference |  | TS-0001 [1], clause 8.1.2, 8.2.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 2 (nonBlockingRequestAsynch) and NotificationURI set to empty list } |  | IUT AE |
|  | then { the IUT sends a valid Response (Ack) and the IUT does not send a NOTIFY Response } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBA/004_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBA/004_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBA/004_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBA/004_DEL | DELETE |

TP/oneM2M/CSE/RT/NBA/005

| TP Id |  | TP/oneM2M/CSE/RT/NBA/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the result of the requested Non-Blocking Asynchronous OPERATION as notification to the notification targets indicated in responseType/notificationURI attribute of the request |  |
| Reference |  | TS-0001 [1], clauses 8.1.2, 8.2.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Response Type parameter containing responseTypeValue set to 2 (nonBlockingRequestAsynch) and NotificationURI set to AE1_RESOURCE_ADDRESS and AE2_RESOURCE_ADDRESS } |  | IUT AE |
|  | then { the IUT sends a valid Response (Ack) and the IUT sends a valid NOTIFY Request containing to AE1 Content containing ResponsePrimitive representation indicating the response of the original Non-Blocking Asynchronous OPERATION containing Response Status Code set to RESPONSE_STATUS_CODE and the IUT sends a valid NOTIFY Request containing to AE2 Content containing ResponsePrimitive representation indicating the response of the original Non-Blocking Asynchronous OPERATION containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE1 IUT AE1 IUT AE2 |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/RT/NBA/005_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/RT/NBA/005_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/RT/NBA/005_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/RT/NBA/005_DEL | DELETE | 2002 (DELETED) |

7.2.2.7.2.3	BlockingRequest (BR)

TP/oneM2M/CSE/RT/BR/001

| TP Id |  | TP/oneM2M/CSE/RT/BR/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully to a Blocking Request OPERATION |  |
| Reference |  | TS-0001 [1], clause 8.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 3 (container) and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and ResponseType set to 3 (blockingRequest) } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/RT/BR/001_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/RT/BR/001_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/RT/BR/001_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/RT/BR/001_DEL | DELETE | 2002 (DELETED) |

7.2.2.8	Security(SEC)

7.2.2.8.1	CREATE Operation

TP/oneM2M/CSE/SEC/ACP/CRE/001

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a accessControlPolicy resource with privileges attribute having multiple access control rules |  |
| Reference |  | TS-0001 [1], clause 9.6.2-2 and clause 10.2.21 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing privileges attribute containing accessControlRule attribute containing ACCESS_CONTROL_RULE_1 and ACCESS_CONTROL_RULE_2 } |  | IUT AE |
|  | then { the IUT creates the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

TP/oneM2M/CSE/SEC/ACP/CRE/002

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a accessControlPolicy resource with selfPrivileges attribute having multiple access control rules |  |
| Reference |  | TS-0001 [1], clause 9.6.2-2 and clause 10.2.21 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing selfPrivileges attribute containing accessControlRule attribute containing ACCESS_CONTROL_RULE_1 and ACCESS_CONTROL_RULE_2 } |  | IUT AE |
|  | then { the IUT creates the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

TP/oneM2M/CSE/SEC/ACP/CRE/003

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a accessControlPolicy resource with privileges attribute having empty rules |  |
| Reference |  | TS-0001 [1], clause 9.6.2 and clause 10.2.21 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing privileges attribute set to empty list } |  | IUT AE |
|  | then { the IUT creates the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT AE |

TP/oneM2M/CSE/SEC/ACP/CRE/004

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the creation of a accessControlPolicy resource with selfPrivileges attribute having empty rules |  |
| Reference |  | TS-0001 [1], clause 9.6.2 and clause 10.2.21 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing selfPrivileges attribute set to empty list } |  | IUT AE |
|  | then { the IUT does not create the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

7.2.2.8.2	UPDATE Operation

TP/oneM2M/CSE/SEC/ACP/UPD/001

| TP Id | TP/oneM2M/CSE/SEC/ACP/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the accessControlPolicyIDs attribute of the <AE> resource when the AE has privileges for UPDATE operation in any selfPrivileges of the <accessControlPolicy> resources which this attribute originally indicates. |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing accessControlPolicyID attribute set to ACP_ID_1 and the AE having privileges to perform UPDATE operation on the AE_RESOURCE_ADDRESS in selfPrivileges } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID Content containing AE resource containing accessControlPolicyIDs attribute set to ACP_ID_2 } | IUT AE |
|  | then { the IUT updates the accessControlPolicyIDs attribute of AE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing AE resource containing accessControlPolicyID attribute set to ACP_ID_2 } | IUT AE |

TP/oneM2M/CSE/SEC/ACP/UPD/002

| TP Id | TP/oneM2M/CSE/SEC/ACP/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries an UPDATE operation on the accessControlPolicyIDs attribute without having privileges for such operation in any selfPrivileges of the <accessControlPolicy> resources which this attribute originally indicates. |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing accessControlPolicyID attribute set to ACP_ID_1 and the AE having no privileges to perform UPDATE operation on the AE_RESOURCE_ADDRESS in selfPrivileges } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID Content containing AE resource containing accessControlPolicyID attribute set to ACP_ID_2 } | IUT AE |
|  | then { the IUT does not update the accessControlPolicyIDs attribute of AE resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } | IUT AE |

7.2.2.8.3	BASIC OPERATION

TP/oneM2M/CSE/SEC/ACP/001

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when an ORIGINATOR tries an OPERATION without having privileges for performing the OPERATION on the TARGET_RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the ORIGINATOR having no privileges to perform an OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to TARGET_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT ORIGINATOR |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/001_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/001_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/001_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/001_DEL | DELETE |

TP/oneM2M/CSE/SEC/ACP/002

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on its <AE> resource which has multiple accessControlPolicyID attribute |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2-1 and clause 9.6.2.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <container> resource containing accessControlPolicyID attribute containing ACP_ID_1 and ACP_ID_2 and the AE having privileges to perform an OPERATION on itself given by ACP_ID_2 only } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/002_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/002_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/002_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/002_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/003

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when an allowed DOMAIN tries an OPERATION on a AE_RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the DOMAIN having privileges to perform an OPERATION on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to DOMAIN } |  | IUT DOMAIN |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT DOMAIN |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/003_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/003_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/003_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/004

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when any ORIGINATOR tries an OPERATION on a AE_RESOURCE_TYPE resource for which all originators are allowed |  |
| Reference |  | TS-0001 [1], clause 9.6.2.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and any originator (*) having privileges to perform an OPERATION on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to ORIGINATOR } |  | IUT ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT ORIGINATOR |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/004_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/004_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/004_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/004_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/005

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource and that the request is within accessControlTimeWindow context. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within TIME_WINDOW } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID within TIME_WINDOW } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/005_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/005_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/005_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/005_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/006

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlTimeWindow context. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within TIME_WINDOW } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID out of TIME_WINDOW } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id |  | OPERATION |  |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/006_CRE | CREATE |  |
| TP/oneM2M/CSE/SEC/ACP/006_RET | RETRIEVE |  |
| TP/oneM2M/CSE/SEC/ACP/006_UPD | UPDATE |  |
| TP/oneM2M/CSE/SEC/ACP/006_DEL | DELETE |  |

TP/oneM2M/CSE/SEC/ACP/007

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource and that the request is within accessControlLocationRegion context. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a child locationPolicy resource containing locationContainerID attribute set to CONTAINER_ID and a child container resource containing locationID attribute set to LOCATION_POLICY_ID and contentInstance resource containing content attribute set to LOCATION and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within LOCATION_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and LOCATION within LOCATION_RESTRICTION } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/007_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/007_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/007_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/007_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/008

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlLocationRegion context. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within LOCATION_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID out of LOCATION_RESTRICTION } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/008_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/008_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/008_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/008_DEL | DELETE |

TP/oneM2M/CSE/SEC/ACP/009

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on a TARGET_RESOURCE_TYPE resource and that the request is within accessControlIpAddresses context. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within IP_ADDRESS_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and AE_IP_ADDRESS within IP_ADDRESS_RESTRICTION } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/009_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/009_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/009_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/009_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/010

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlIpAddresses context. |  |
| Reference |  | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within IP_ADDRESS_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and AE_IP_ADDRESS out of IP_ADDRESS_RESTRICTION } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/010_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/010_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/010_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/010_DEL | DELETE |

TP/oneM2M/CSE/SEC/ACP/011

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on its child <container> resource whose accessControlPolicyID attribute is not set and AE has privileges for such OPERATION on its associated accessControlPolicy resource |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <container> resource containing no accessControlPolicyID attribute and the AE having privileges to perform an OPERATION on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/011_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/011_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/011_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/011_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/012

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when an ORIGINATOR tries an OPERATION on a <Container> resource with no accessControlPolicyID associated and such ORIGINATOR having no privileges for performing the OPERATION on the <container> resource. |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_ACP_SUPPORT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <container> resource containing no accessControlPolicyID attribute and the ORIGINATOR having no privileges to perform an OPERATION on the <container> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT ORIGINATOR |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/012_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/012_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/012_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/012_DEL | DELETE |

TP/oneM2M/CSE/SEC/ACP/013

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the creator AE tries an OPERATION on the AE resource whose accessControlPolicyIDs attribute is not set. |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the <AE> resource having no accessControlPolicyIDs attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/013_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/013_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/013_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/013_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SEC/ACP/014

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when an AE2 tries an OPERATION on the AE1 resource whose accessControlPolicyIDs attribute is not set. |  |
| Reference |  | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the <AE> resource of AE1 having no accessControlPolicyIDs attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE1_RESOURCE_ADDRESS and From set to AE2_ID } |  | IUT  AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE2 |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/014_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/014_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/014_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/014_DEL | DELETE |

7.2.2.9	FlexContainer

7.2.2.9.1	CREATE Operation

TP/oneM2M/CSE/FLXC/CRE/001

| TP Id | TP/oneM2M/CSE/FLXC/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the <flexContainer> resource when the resource representation does not comply with the schema definition. |  |
| Reference | TS-0001 [1], clause 10.2.29, TS-0004 [2], clause 7.4.29.2.1 |  |
| Parent Release | Release 2 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE, |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives an invalid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 28 (flexContainer) and Content containing an invalid resource representation } | IUT AE |
|  | then { the IUT does not create the flexContainer resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } | IUT AE |

7.2.2.9.2	UPDATE Operation

TP/oneM2M/CSE/FLXC/UPD/001

| TP Id |  | TP/oneM2M/CSE/FLXC/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT increments the field value of attribute stateTag in the <flexContainer> resource when a custom attribute of the resource is modified. |  |
| Reference |  | TS-0001 [1], clause 9.6.35 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource FLEXCONTAINER_RESOURCE_ADDRESS containing stateTag attribute and the AE having privileges to perform UPDATE operation on the resource FLEXCONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to FLEXCONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing flexContainer resource containing ATTRIBUTE_NAME attribute set to VALUE } |  | IUT AE |
|  | then { the IUT increments the stateTag attribute of FLEXCONTAINER_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | IUT AE |

TP/oneM2M/CSE/FLXC/UPD/002

| TP Id |  | TP/oneM2M/CSE/FLXC/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT does not increment the field value of attribute stateTag in the <flexContainer> resource when a common attribute of the resource is modified. |  |
| Reference |  | TS-0001 [1], clause 9.6.35 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource FLEXCONTAINER_RESOURCE_ADDRESS containing stateTag attribute and the AE having privileges to perform UPDATE operation on the resource FLEXCONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to FLEXCONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing flexContainer resource containing ATTRIBUTE_NAME attribute set to VALUE } |  | IUT AE |
|  | then { the IUT does not increment the stateTag attribute of FLEXCONTAINER resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | IUT AE |

TP/oneM2M/CSE/FLXC/UPD/003

| TP Id | TP/oneM2M/CSE/FLXC/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update of the <flexContainer> resource when the resource representation does not comply with the schema definition. |  |
| Reference | TS-0001 [1], clause 10.2.29, TS-0004 [2], clause 7.4.29.2.3 |  |
| Parent Release | Release 2 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE, |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform UPDATE operation on the resource FLXC_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives an invalid UPDATE request from AE containing To set to FLXC_RESOURCE_ADDRESS and From set to AE_ID and Content containing flexContainer resource containing an invalid resource representation } | IUT AE |
|  | then { the IUT does not update the flexContainer resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } | IUT AE |

Annex A (informative):
Conformance Test Requirement

A.1	MQTT Protocol Conformance Test Requirement

| Test Items |  | TS Clause | Obligation | Condition for Optional Item | Checking point | Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| Commonly Repeated Items |  | TS-0010 [5], clause 5.1 | Mandatory |  | "QoS=1" | Fail when QoS is other than "1" |
|  |  | TS-0010 [5], clause 5.1 | Mandatory |  | "Retain" flag set as "0" | Fail if DUT sends with "Retain" flag=1 |
| Connection Items | Connect | TS-0010 [5], clause 6.3 | Mandatory |  | Prefix A:: or C:: is added to the ID | Fail if Prefix A:: or C:: are not added to the ID |
|  |  | TS-0010 [5], clause 6.3 | Mandatory |  | if Clean session (ex: Retain) flag is set to "False" in CONNECT | Fail when clean session is "True" when DUT want to be connected |
|  |  | TS-0010 [5], clause 6.3 | Mandatory |  | if "Will Flag" is not set in CONNECT | Fail when will flag is enabled |
|  | Disconnect | TS-0010 [5], clause 6.3 | Mandatory |  | if Clean session (ex: Retain) flag is set to "True" in CONNECT | Fail when clean session is "False" when DUT want to be disconnected |
| Subscription Items |  | TS-0010 [5], clause 5.1 | Mandatory |  | if the message is set to Durable | Fail is the message is not Durable |
|  |  | TS-0010 [5], clause 6.4.4 | Conditional | (for Initial Registration) when Originator does no't knows it's ID | if DUT's Initial ID assigning /oneM2M/reg_req/<originator>/<receiver> oneM2M/reg_resp/<originator>/<receiver> | Fail if DUT's ID assigning is other than this |
| Publish Item |  | TS-0010 [5], Figure 6.4.5-2 | Conditional | when Originator knows it's ID | if DUT's Initial ID assigning oneM2M/req/<Originator>/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010 [5], Figure 6.4.5-2 | Conditional | When Originator uses wild card | if DUT's Initial ID assigning oneM2M/req/+/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | MQTT V3.1.1 | Conditional |  | if the has proper Primitive contents ("op", "fr", "to", "ri", "pc", "ty") | Fail if any of primitive contents feature wrong |
| Reception Item |  | TS-0010 [5], Figure 6.4.5-2 | Conditional | when Originator knows it's ID | if DUT's Initial ID assigning oneM2M/resp/<Originator>/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010 [5], Figure 6.4.5-2 | Conditional | When Originator uses wild card | if DUT's Initial ID assigning oneM2M/resp/<Originator>/+ | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010 [5], Figure 6.4.5-2 | Conditional |  | server Delete received contents properly not storing data | Fail if Receiver stores message |
|  |  | TS-0010 [5], Figure 6.4.5-2 | Conditional |  | If the receiver sends PubACK when receives messages | Fail if the receiver does no't send PubACK |
|  |  | MQTT V3.1.1 | Mandatory |  | if the has proper Primitive contents ("op", "fr", "to", "ri", "pc", "ty") | Fail if any of primitive contents feature wrong |

<PAGE BREAK>

Annex B (informative):
TP template

| TP Id |  |  |  |
| --- | --- | --- | --- |
| Test objective |  |  |  |
| Reference |  |  |  |
| Config Id |  |  |  |
| PICS Selection |  |  |  |
| Initial conditions |  |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { } |  | IUT AE |
|  | then { } |  | IUT AE |

History

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.2.0.0 | 2017-11-13 | Incorporated agreed documents from TST#31.2 - TST-2017-0247 – Preparation of TS-0018 v2_0_0 |
| V.2.1.0 | 2018-01-17 | Incorporated agreed documents from TST#32.2, TST#33 - TST-2017-0284-TS-0018_Changes_from_STF531 - TST-2017-0285-TS-0018_Changes_from_Interop5 - TST-2018-0004R01-Proposal_for_new_Testcase |
| V.2.2.0 | 2018-03-16 | Incorporated agreed documents from TST#33.2, TST#34 - TST-2018-0030R01-TS-0018_New_TPs_for_nncries - TST-2018-0033R01-New_TPs_for_PollingChannel - TST-2018-0034R03-3GPPTriggeringTPs - TST-2018-0046-TS-0018_Changes_from_STF531_R2 - TST-2018-0048-TS-0018_Changes_from_STF531_for_announcement_R2 |
| V.2.3.0 | 2018-05-22 | Incorporated agreed documents from TST#34.2 - TST-2018-0064-TS-0018_Changes_from_STF531_R2 - TST-2018-0065-TS-0018_ANNC_CRE_008_R2 - TST-2018-0066R01-TS-0018_New_TPs_for_timeSeries_permutations |
| V.2.4.0 | 2018-06-12 | Incorporated agreed documents from TST#35 - TST-2018-0074R04-LocationPolicy - TST-2018-0075-ANNC_CREATE - TST-2018-0080-TS-0018_New_TPs_for_non-blocking_requests - TST-2018-0081-TS-0018_Changes_from_STF531_R2 |
| V.2.5.0 | 2018-07-16 | Incorporated agreed documents from TST#35.1a, TST#35.2 - TST-2018-0099-TS-0018_Changes_from_STF531_R2 - TST-2018-0103R01-TS-0018_pollingChannel_TP_correction-STF531 |
| V.2.6.0 | 2018-08-08 | Incorporated agreed documents from TST#36 - TST-2018-0125R01-TS-0018_New_TPs_Rel-2_addressing_shortcut - TST-2018-0126R01-TS-0018_Changes_from_STF531_R2 |
| V.2.7.0 | 2018-11-06 | Incorporated agreed documents from TST#36.1, TST#37 - TST-2018-0133-TS-0018_new_TP_for_retargeting - TST-2018-0134-TS-0018_modifying_pollingChannel_TP - TST-2018-0131R02-TS-0018_Changes_for_TPs_R2 - TST-2018-0146-TS-0018_Changes_for_retargeting_TPs_R2 |
| V.2.8.0 | 2018-12-07 | Incorporated agreed documents from TST#38 - TST-2018-0165-TS-0018_New_TPs_Rel-2_hybrid_addressing - TST-2018-0168-TS-0018_Changes_for_discovery_TPs_R2 |
| V.2.9.0 | 2019-02-18 | Incorporated agreed documents from TDE#38.2 - TDE-2019-0009-TS-0018_Changes_for_TPs_R2 |
| V.2.10.0 | 2019-02-20 | Incorporated agreed documents from TDE#39 - TDE-2019-0012R01-TS-0018_New_TPs_for_rcn_R2 - TDE-2019-0013-TS-0018_Changes_for_TPs_R2 - TDE-2019-0021-ResponseStatusCodeUpdate(R2) |
| V.2.11.0 | 2019-02-22 | Incorporated agreed documents from TDE#39 - TDE-2019-0025-TS-0018_Changes_for_rcn_TPs_R2 - TDE-2019-0029R03-TS-0018_TP_FlexContainers - TDE-2019-0036-TS-0018_Rel-2_hybrid_addressing_TP_changes - TDE-2019-0037-TS-0018_Rel-2_shortcut_addressing_TP_changes |
| V.2.12.0 | 2019-04-24 | Incorporated agreed documents from TDE#39.1 - TDE-2019-0041R01-TS-0018_New_TPs_for_registration_R2 - TDE-2019-0043R01-TS-0018_Changes_for_registration_TPs_R2 |
| V.2.13.0 | 2019-05-24 | Incorporated agreed documents from TDE#39.2, TDE#40 - TDE-2019-0069R01-TS-0018_New_TPs_for_registration_R2 - TDE-2019-0087R01-TS-0018-New_TPs_for_completing_release_2 |
| V2.13.1 | July 2019 | Pre-processing done before ratified e-mail: mailto:edithelp@etsi.org |
| V.2.14.0 | 2019-09-27 | Incorporated agreed documents from TDE#41.1 - TDE-2019-0127-TS-0018_TP_changes_for_RCN_R2 - TDE-2019-0128-TS-0018_TP_changes_for_ANNC_R2 - TDE-2019-0145R01-TS-0018_Changes_for_group_TPs_R2 - TDE-2019-0147R01-TS-0018_Changes_for_discovery_TPs_R2 - TDE-2019-0156-TS-0018-Remove_resource_types_to_align_with_Rel-2_testing_spec - TDE-2019-0158-TS-0018_New_TPs_for_discovery_R2 |
| V.2.15.0 | 2020-03-10 | Incorporated agreed documents from TDE#44 - TDE-2020-0012-TS-0018_Changes_for_generic_TPs_R2 |
| V.2.16.0 | 2020-05-12 | Incorporated agreed documents from TDE#44.1, TDE#45 - TDE-2020-0019R01-TS-0018_Changes_for_discovery_TPs_R2 - TDE-2020-0038-GroupMembersPrivilegeCheck_R2 |