| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | TDE #49 |
| Source:* | TF-oneM2M |
| Date:* | 2020-12-10 |
| Contact:* | Miguel Angel Reina Ortega, ETSI, MiguelAngel.ReinaOrtega@etsi.org |
| Reason for Change/s:* | New TPs for primitiveProfile release 4 |
| CR against: Release* | Rel-4 |
| CR against: WI* | Active <Work Item number> MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018-V-4.1.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR is a mirror CR? YES NO if YES, please indicate the document number of the original CR: |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

New TPs for primitiveProfile release 4.

R01 – Corrections to the TPs according to the discussion during presentation of the original contribution

R02 – Corrections to the TPs according to the discussion during presentation of R01

R03 – Corrections to the TPs according to CRs: SDS-2020-0129R07-TS-0001_primitive_profile_maintenance_R4 and SDS-2020-0131R0613-TS-0004-primitiveProfile_procedures_R4

----------------------- Start of change 1 -----------------------

TP/oneM2M/CSE/PP/001

| TP Id |  | TP/oneM2M/CSE/PP/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT applies the referenced <primitiveProfile> over the request messages by adding the applicable parameters/attributes when the request message contains a valid Primitive Profile Identifier parameter and the referenced profile is accessible to the Hosting CSE. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource at PRIMITIVE_PROFILE_ADDRESS containing additions attribute having list of attributes to be added and AE having Retrieve privileges for the <primitiveProfile> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { The IUT applies the profile by adding attributes defined in additions attribute and the IUT sends a valid Response to the AE } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/001_CRE | CREATE |
| TP/oneM2M/CSE/ PP /001_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP /001_UPD | UPDATE |
| TP/oneM2M/CSE/ PP /001_DEL | DELETE |

TP/oneM2M/CSE/PP/002

| TP Id |  | TP/oneM2M/CSE/PP/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT applies the referenced <primitiveProfile> over the request messages by deleting the applicable parameters/attributes when the request message contains a valid PrimitiveProfileIdentifier parameter and the referenced profile is accessible to the Hosting CSE. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> at PRIMITIVE_PROFILE_ADDRESS containing deletions attribute having list of attributes to be deleted and AE having Retrieve privileges for the <primitiveProfile> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { The IUT applies the profile by deleting attributes defined in deletions and the IUT sends a valid Response to the AE } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/002_CRE | CREATE |
| TP/oneM2M/CSE/ PP /002_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP /002_UPD | UPDATE |
| TP/oneM2M/CSE/ PP /002_DEL | DELETE |

TP/oneM2M/CSE/PP/003

| TP Id |  | TP/oneM2M/CSE/PP/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT applies the referenced <primitiveProfile> over the request messages by replacing the applicable parameters/attributes when the request message contains a valid PrimitiveProfileIdentifier parameter and the referenced profile is accessible to the Hosting CSE. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> at PRIMITIVE_PROFILE_ADDRESS containing additions attribute having list of attributes to be replaced and AE having Retrieve privileges for the <primitiveProfile> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { The IUT applies the profile by replacing attributes defined in additions attribute and the IUT sends a valid Response to the AE } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/003_CRE | CREATE |
| TP/oneM2M/CSE/ PP /003_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP /003_UPD | UPDATE |
| TP/oneM2M/CSE/ PP /003_DEL | DELETE |

TP/oneM2M/CSE/PP/004

| TP Id |  | TP/oneM2M/CSE/PP/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the request when the <primitiveProfile> resource referenced by the PrimitiveProfileIdentifier parameter is not present |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the retrieve privilege for <primitiveProfile> resource to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT not having <primitiveProfile> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid OPERATION Request from AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { the IUT does not apply the primitive profile and the IUT sends a valid Response containing Response Status Code set to 41XX (PRIMITIVE_PROFILE_NOT_ACCESSIBLE) } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/004_CRE | CREATE |
| TP/oneM2M/CSE/ PP/004_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP/004_UPD | UPDATE |
| TP/oneM2M/CSE/ PP/004_DEL | DELETE |

TP/oneM2M/CSE/PP/005

| TP Id |  | TP/oneM2M/CSE/PP/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the request when the originator does not have retrieve privileges for <primitiveProfile> resource referenced by the PrimitiveProfileIdentifier parameter. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a <primitiveProfile> at PRIMITIVE_PROFILE_ADDRESS and the AE not having the retrieve privilege for <primitiveProfile> resource to perform OPERATION on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid OPERATION Request from AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { the IUT does not apply the primitive profile and the IUT sends a valid Response containing Response Status Code set to 41XX (PRIMITIVE_PROFILE_NOT_ACCESSIBLE) } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/005_CRE | CREATE |
| TP/oneM2M/CSE/ PP/005_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP/005_UPD | UPDATE |
| TP/oneM2M/CSE/ PP/005_DEL | DELETE |

TP/oneM2M/CSE/PP/006

| TP Id |  | TP/oneM2M/CSE/PP/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully processes the request without applying the profile when the attributes in the request do not match with the referenced <primitiveProfile> resource attributes.. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] clause 7.3.3.1, 6.6.3.5 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the retrieve privilege for resource <primitiveProfile> to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource at PRIMITIVE_PROFILE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid OPERATION Request from AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID not matching with the <primitiveProfile> resource attributes } |  | AE  IUT |
|  | then { the IUT does not apply the primitive profile and sends the request to TARGET_RESOURCE_ADDRESS when the process is completed } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/006_CRE | CREATE |
| TP/oneM2M/CSE/ PP /006_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP /006_UPD | UPDATE |
| TP/oneM2M/CSE/ PP /006_DEL | DELETE |

TP/oneM2M/CSE/PP/007

| TP Id |  | TP/oneM2M/CSE/PP/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of <primitiveProfile> resource request when the request parameters or resource attributes defined in the additions and deletions attributes of the referenced <primitiveProfile> resource are not valid. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 60? (primitiveProfile) To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

TP/oneM2M/CSE/PP/008

| TP Id |  | TP/oneM2M/CSE/PP/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update request of <primitiveProfile> resource when the request parameters or resource attributes defined in the additions and deletions attributes of the referenced <primitiveProfile> resource are not valid. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on PRIMITIVE_PROFILE_ADDRESS and the IUT having a <primitiveProfile> resource at PRIMITIVE_PROFILE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid UPDATE Request from AE containing a valid Primitive Profile Identifier and To set to PRIMITIVE_PROFILE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT AE |

TP/oneM2M/CSE/PP/009

| TP Id |  | TP/oneM2M/CSE/PP/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a <primitiveProfile> resource when a mutual exclusivity check is failed on the names of the attributes or parameters specified in the additions and, deletions attributes. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid CREATE Request from AE containing Resource Type set to 60? (primitiveProfile) and Content containing primitiveProfile resource containing additions attribute and deletions attribute not being mutually exclusive } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

TP/oneM2M/CSE/PP/010

| TP Id |  | TP/oneM2M/CSE/PP/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update request of <primitiveProfile> resource when a mutual exclusivity check is failed on the names of the attributes or parameters specified in the additions and deletions attributes. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource at PRIMITIVE_PROFILE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid UPDATE Request from AE containing To set to PRIMITIVE_PROFILE_ADDRESS and Content containing additions attribute and deletions attribute not being mutually exclusive } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT AE |

TP/oneM2M/CSE/PP/011

| TP Id |  | TP/oneM2M/CSE/PP/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a <primitiveProfile> resource when a request contains non permissible or mandatory parameters specified in the additions and deletions attributes. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid CREATE Request from AE containing To set to PRIMITIVE_PROFILE_ADDRESS and Content containing primitiveProfile resource containing additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  | AE  IUT |
|  | then { The IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

TP/oneM2M/CSE/PP/012

| TP Id |  | TP/oneM2M/CSE/PP/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE operation on resource <primitiveProfile> when a request contains non permissible or mandatory parameters specified in the additions and deletions attributes. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> at PRIMITIVE_PROFILE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid UPDATE Request from AE containing To set to PRIMITIVE_PROFILE_ADDRESS and Content containing primitiveProfile resource containing additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

TP/oneM2M/CSE/PP/013

| TP Id |  | TP/oneM2M/CSE/PP/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT successfully applies the <primitiveProfile> to the notification (Notify request primitive) in case of nonBlockingRequestAsynch request having valid attribute PrimitiveProfileIdentifier |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS0004 [2] clause 7.5.1.2.5 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a resource representation at TARGET_RESOURCE_ADDRESS and the IUT having received a valid nonBlockingRequestAsynch request containing a valid Primitive Profile Identifier and the IUT having a <primitiveProfile> at PRIMITIVE_PROFILE_ADDRESS containing additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The requested operation is completed } |  | IUT |
|  | then { The IUT applies the <primitiveProfile> to Notify request primitive by adding ADD_PARAMETER and deleting DEL_PARAMETER and the IUT sends a valid NOTIFY request containing content representation } |  | IUT  AE |

TP/oneM2M/CSE/PP/014

| TP Id |  | TP/oneM2M/CSE/PP/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT successfully applies the <primitiveProfile> to the response in case of nonBlockingRequestSynch request having valid Primitive Profile Identifier |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS0004 [2] clause 7.2.2.2 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having the <request> resource at REQUEST_RESOURCE_ADDRESS and the IUT having received a valid nonBlockingRequestSynch request containing a valid Primitive Profile Identifier and the IUT having a <primitiveProfile> at PRIMITIVE_PROFILE_ADDRESS containing additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT receives a valid RETRIEVE Request from AE containing To set to REQUEST_RESOURCE_ADDRESS } |  | IUT |
|  | then { the IUT applies the referenced <primitiveProfile> to the response by adding ADD_PARAMETER and deleting DEL_PARAMETER and the IUT sends a valid Response containing Content containing Response Status Code set to 2XXX and Response Primitive representation } |  | IUT |

TP/oneM2M/CSE/PP/015

| TP Id |  | TP/oneM2M/CSE/PP/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT successfully applies the <primitiveProfile> to the notification request generated for the <subscription> resource |  |
| Reference |  | TS-0001 [1], clause 9.6.8, 9.6.73, 10.2.25, TS-0004 [2] clause 7.3.3.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a valid <subscription> resource containing a primitiveProfileID attribute set to PRIMITIVE_PROFILE_ID and AE having subscribed to the <subscription> resource and the IUT having a valid <primitiveProfile> resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { The IUT generates a valid notification request for a the <subscription> resource containing primitiveProfileID } |  | IUT |
|  | then { The IUT applies the <primitiveProfile> resource referenced in the primitiveProfileID attribute to the generated notification by adding ADD_PARAMETER and deleting DEL_PARAMETER and the IUT sends a valid NOTIFY request containing content representation } |  | IUT |

TP/oneM2M/CSE/PP/016

| TP Id |  | TP/oneM2M/CSE/PP/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the request to the hosting CSE without applying the <primitiveProfile> when the To parameter contains the hosting CSE-ID which is known by the IUT |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] clause 7.3.3.1 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state” and the IUT having registered to a Hosting CSE (H-CSE) and the IUT having registered the AE and the Hosting CSE having create a RESOURCE on TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on the Hosting CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | AE  IUT |
|  | then { The IUT forwards the OPERATION Request to Hosting CSE without applying the <primitiveProfile> resource referenced in the Primitive Profile Identifier } |  | IUT  H-CSE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/PP/016_CRE | CREATE |
| TP/oneM2M/CSE/ PP /016_RET | RETRIEVE |
| TP/oneM2M/CSE/ PP /016_UPD | UPDATE |
| TP/oneM2M/CSE/ PP /016_DEL | DELETE |

TP/oneM2M/CSE/PP/017

| TP Id |  | TP/oneM2M/CSE/PP/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT applies the referenced <primitiveProfile> over the response messages by adding the applicable parameters/attributes when the originating AE specified a valid PrimitiveProfileIdentifier parameter in the corresponding request. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] clause 7.3.3.12 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and a valid additions attribute having list of attributes to be added and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid response from a resource containing To set to AE_ID and From set to TARGET_RESOURCE_ADDRESS } |  | IUT |
|  | then { the IUT applies the referenced primitive profile by adding attributes defined in additions attribute and the IUT sends a valid Response to the AE containing valid RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/017_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ PP /017_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/ PP /017_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ PP /017_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/PP/0018

| TP Id |  | TP/oneM2M/CSE/PP/0018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT applies the referenced <primitiveProfile> over the response messages by deleting the applicable parameters/attributes when the originating AE specified a valid PrimitiveProfileIdentifier parameter in the corresponding request. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] clause 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and deletions attribute having list of attributes to be deleted and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid response from a resource containing To set to AE_ID and From set to TARGET_RESOURCE_ADDRESS } |  | IUT |
|  | then { the IUT applies the referenced primitive profile by deleting attributes defined in deletions attribute and the IUT sends a valid Response to the AE containing valid RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/018_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ PP /018_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/ PP /018_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ PP /018_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/PP/019

| TP Id |  | TP/oneM2M/CSE/PP/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT applies the referenced <primitiveProfile> over the response messages by replacing the applicable parameters/attributes when the originating AE specified a valid PrimitiveProfileIdentifier parameter in the corresponding request. |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and additions attribute having list of attributes to be replaced and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid response from a resource containing To set to AE_ID and From set to TARGET_RESOURCE_ADDRESS } |  | IUT |
|  | then { the IUT applies the referenced primitive profile by replacing attributes defined in additions attribute and the IUT sends a valid Response to the AE containing valid RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/019_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ PP /019_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/ PP /019_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ PP /019_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/PP/020

| TP Id |  | TP/oneM2M/CSE/PP/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT successfully applies the <primitiveProfile> to the representation of the requested resource returned in the Content parameter of the response and If the Content parameter of the response includes nested representations of child resource(s) and their descendant(s), the Hosting CSE shall not apply the profile to these child resource(s) or descendant(s). |  |
| Reference |  | TS-0001 [1], clause 9.6.73, 10.2.25, TS-0004 [2] clause 7.3.3.12 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and modifications attribute having list of attributes to be modified and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid response from a resource containing To set to AE_ID and From set to TARGET_RESOURCE_ADDRESS and Result Content set to 4 (attributes and child resources) and Content containing Resource representation containing nested representations of child resources and their descendants } |  | IUT |
|  | then { the IUT applies the referenced primitive profile on the requested resource representation and ignore the child resource(s) or descendant(s) of the resource representation. and the IUT sends a valid Response to the AE containing valid RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/ PP /020_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/ PP /020_DEL | DELETE | 2002 (DELETED) |

## ---------------------- End of change 1 -----------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?