| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TDE # |
| Source:* | TF-oneM2M |
| Date:* | 2021-04-22 |
| Reason for Change/s:* | New TPs for Primitive Profile release 4 |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018 V4.3.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

New TPs for Primitive Profile Release 4

### -----------------------Start of change 1-------------------------------------------

TP/oneM2M/CSE/PP/0021

| TP Id |  | TP/oneM2M/CSE/PP/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of <primitiveProfile> resource when the request parameters or resource attributes defined in the additions attribute have complex data type definition |  |
| Reference |  | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.2.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 60? (primitiveProfile) To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing primitiveProfile resource containing additions attribute indicating ADD_PARAMTER having complex data type } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

TP/oneM2M/CSE/PP/022

| TP Id |  | TP/oneM2M/CSE/PP/022 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a <primitiveProfile> resource when any parameters specified in the additions attribute not matching with the defined data types for the parameters |  |
| Reference |  | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 60? (primitiveProfile) and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing primitiveProfile resource containing additions attribute indicating ADD_PARAMTER not matching the defined data types } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

TP/oneM2M/CSE/PP/023

| TP Id |  | TP/oneM2M/CSE/PP/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update request of a <primitiveProfile> resource when any parameters specified in the additions attribute not matching with the defined data types for the parameters |  |
| Reference |  | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on PRIMITIVE_PROFILE_ADDRESS and the IUT having a <primitiveProfile> resource at PRIMITIVE_PROFILE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to PRIMITIVE_PROFILE_ADDRESS and From set to AE_ID and Content containing primitiveProfile resource containing additions attribute indicating ADD_PARAMTER not matching the defined data types } |  | AE  IUT |
|  | then { the IUT sends a valid Response containing Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD REQUEST) } |  | IUT  AE |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?