| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TDE # 50.3 |
| Source:* | TF-oneM2M |
| Date:* | 2021-07-28 |
| Reason for Change/s:* | New TPs for Flex Container release 4 |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018 V4.3.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

New TPs for Flex Container Release 4.

### -----------------------Start of change 1-------------------------------------------

7.2.2.13.1	CREATE Operation

TP/oneM2M/CSE/FLXC/CRE/002

| TP Id |  | TP/oneM2M/CSE/FLXC/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE request of <flexContainer> resource when the containerDefinition attribute does not include the schema value to validate the received resource representation |  |
| Reference |  | TS-0001 [1], clause 10.1.2 and 10.2.4.16, TS-0004 [2], clause 7.4.37.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE the AE having privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID Content containing flexContainer resource containing containerDefinition set to UNAVAILABLE_SCHEMA } |  | IUT AE |
|  | then { the IUT does not create the flexContainer resource and the IUT sends a valid Response containing Response Status Code set to 4125 (SPECIALIZATION_SCHEMA_NOT_FOUND) } |  | AE IUT |

TP/oneM2M/CSE/FLXC/CRE/003

| TP Id |  | TP/oneM2M/CSE/FLXC/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE request of <flexContainer> resource when value of contentSize attribute is greater than maxByteSize attribute |  |
| Reference |  | TS-0001 [1], clause 10.1.2 and 10.2.4.16, TS-0004 [2], clause 7.4.37.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE the AE having privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID and Content containing flexContainer resource containing maxNrOfInstances set to non-zero value and a valid maxByteSize and a valid CUSTOM_ATTRIBUTE with size bigger than maxByteSize } |  | IUT AE |
|  | then { the IUT does not create the flexContainer resource and the IUT sends a valid Response containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | AE IUT |

TP/oneM2M/CSE/FLXC/CRE/004

| TP Id |  | TP/oneM2M/CSE/FLXC/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully creates the <flexContainer> resource and its child <flexcontainerInstance> resource when ATTRIBUTE attribute of <flexContainer> resource is present with a non-zero value in the request |  |
| Reference |  | TS-0001 [1], clause 10.1.2 and 10.2.4.16, TS-0004 [2], clause 7.4.37.2.1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the AE having privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID Content containing flexContainer resource containing a valid ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT creates the <flexContainer> resource and the IUT creates the child <flexContainerInstance> resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | AE IUT |

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/CRE/004/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/CRE/004/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/CRE/004/MIA | maxInstanceAge |

7.2.2.13.2	UPDATE Operation

TP/oneM2M/CSE/FLXC/UPD/004

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE request of <flexContainer> resource when the size value of the customAttribute exceeds the value specified in the maxByteSize attribute |  |
| Reference |  | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a valid maxByteSize attribute and the AE having privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID flexcontainer resource containing a valid CUSTOM_ATTRIBUTE with size greater than maxByteSize } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | AE IUT |

TP/oneM2M/CSE/FLXC/UPD/005

| TP Id |  | TP/oneM2M/CSE/FLXC/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT updates the <flexContainer> resource and creates its new child <flexContainerInstance> resource when request contains a custom attribute of the targeted <flexContainer> resource |  |
| Reference |  | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a maxNrOfInstances attribute set to non-zero value and a maxByteSize attribute set to non-zero value and a maxInstanceAge attribute set to non-zero value and the AE having privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID Content containing flexcontainer resource containing a valid CUSTOM_ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT updates the <flexContainer> resource and the IUT creates the child <flexContainerInstance> resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | AE IUT |

TP/oneM2M/CSE/FLXC/UPD/006

| TP Id |  | TP/oneM2M/CSE/FLXC/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates the set of <flexContainerInstance> children resources, currentNrOfInstances and currentByteSize attributes of the <flexContainer> resource when the UPDATE request creates, modifies or deletes ATTRIBUTE attribute of the <flexContainer> resource |  |
| Reference |  | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing children <flexContainerInstance> resources and the AE having privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID and Content containing flexContainer resource containing a valid ATTRIBUTE attribute } |  | IUT AE |
|  | then { the IUT updates the set of <flexContainerInstances> resources according to the modified <flexContainer> attributes and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing flexContainer resource containing currentNrOfInstances and currentByteSize attributes indicating modified values } |  | AE IUT |

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/UPD/006/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/UPD/006/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/UPD/006/MIA | maxInstanceAge |

TP/oneM2M/CSE/FLXC/UPD/007

| TP Id |  | TP/oneM2M/CSE/FLXC/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes all child <flexContainerInstance> resources and the currentNrOfInstances, currentByteSize, maxNrOfInstances, maxByteSize and maxInstanceAge attributes of the <flexContainer> when UPDATE request updates ATTRIBUTE of the <flexContainer> resource with a value of zero |  |
| Reference |  | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing children <flexContainerInstance> resources and the AE having privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing Resource Type set to 28 (flexContainer) To set to TARGET _RESOURCE_ADDRESS and From set to AE_ID Content containing <flexContainer> resource containing ATTRIBUTE attribute set to ZERO } |  | IUT AE |
|  | then { the IUT deletes all <flexContainerInstance> resources and the IUT deletes currentNrOfInstances, currentByteSize, maxNrOfInstances, maxByteSize and maxInstanceAge attributes of <flexContainer> and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | AE IUT |

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/UPD/007/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/UPD/007/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/UPD/007/MIA | maxInstanceAge |

7.2.2.13.3	RETRIEVE Operation

TP/oneM2M/CSE/FLXC/RET/001

| TP Id |  | TP/oneM2M/CSE/FLXC/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT performs the RETRIEVE request for the latest <flexContainerInstance> resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3, 10.2.4.17, TS-0004 [2], clause 7.4.37.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 < creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AE having privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing Resource Type set to 58 (flexContainerInstance) and To set to TARGET _RESOURCE_ADDRESS/la and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a Response containing Response Status Code set to 2000 (OK) and Content containing flexContainerInstance resource containing resourceName attribute set to FLEX_CONTAINER_INSTANCE_2 } |  | IUT AE |

TP/oneM2M/CSE/FLXC/RET/002

| TP Id |  | TP/oneM2M/CSE/FLXC/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT performs the RETRIEVE request for the oldest <flexContainerInstance> resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3, 10.2.4.17, TS-0004 [2], clause 7.4.37.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 < creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AE having privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing Resource Type set to 58 (flexContainerInstance) and To set to TARGET _RESOURCE_ADDRESS/ol and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a Response containing Response Status Code set to 2000 (OK) and Content containing flexContainerInstance resource containing resourceName attribute set to FLEX_CONTAINER_INSTANCE_1 } |  | IUT AE |

7.2.2.13.4	DELETE Operation

TP/oneM2M/CSE/FLXC/DEL/001

| TP Id |  | TP/oneM2M/CSE/FLXC/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT performs the DELETE request for the latest <flexContainerInstance> resource |  |
| Reference |  | TS-0001 [1], clause 10.1.5, 10.2.4.19, TS-0004 [2], clause 7.4.37.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 < creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AE having privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing Resource Type set to 58 (flexContainerInstance) To set to TARGET _RESOURCE_ADDRESS/la and From set to AE_ID } |  | IUT AE |
|  | then { the IUT deletes the FLEX_CONTAINER_INSTANCE_2 resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and } |  | IUT AE |

TP/oneM2M/CSE/FLXC/DEL/002

| TP Id |  | TP/oneM2M/CSE/FLXC/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT performs the DELETE request for the oldest <flexContainerInstance> resource |  |
| Reference |  | TS-0001 [1], clause 10.1.5, 10.2.4.19, TS-0004 [2], clause 7.4.37.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 < creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AE having privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing Resource Type set to 58 (flexContainerInstance) To set to TARGET _RESOURCE_ADDRESS/ol and From set to AE_ID } |  | IUT AE |
|  | then { the IUT deletes the FLEX_CONTAINER_INSTANCE_1 resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and } |  | IUT AE |

7.2.2.13.5	BASIC Operation

TP/oneM2M/CSE/FLXC/001

| TP Id |  | TP/oneM2M/CSE/FLXC/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the OPERATION request for the latest <flexContainerInstance> resource if targeted resource does not exist |  |
| Reference |  | TS-0001 [1], clause 10.2.4.17, 10.2.4.19, TS-0004 [2], clause 7.4.37.2.2, 7.4.37.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing no child flexContainerInstance resource and the AE having privileges to perform OPERATION on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 58 (flexContainerInstance) To set to TARGET _RESOURCE_ADDRESS/la and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/FLXC/001/RET | Retrieve |
| TP/oneM2M/CSE/FLXC/001/DEL | Delete |

TP/oneM2M/CSE/FLXC/002

| TP Id |  | TP/oneM2M/CSE/FLXC/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the OPERATION request for the oldest <flexContainerInstance> resource if targeted resource does not exist |  |
| Reference |  | TS-0001 [1], clause 10.2.4.17, 10.2.4.19, TS-0004 [2], clause 7.4.37.2.2, 7.4.37.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform operation on TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing Resource Type set to 58 (flexContainerInstance) To set to TARGET _RESOURCE_ADDRESS/ol and From set to AE_ID } |  | IUT AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/FLXC/002/RET | Retrieve |
| TP/oneM2M/CSE/FLXC/002/DEL | Delete |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?