l

| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0045-Developer_Guide_Deploying_Semantics-V0_0_1 |
| Document Name: | Developer Guide: Deploying Semantics |
| Date: | 2022-03-09 |
| Abstract: | This developer guide is to describe how developer can quickly implement semantic functionality of the release 3. The intended work is about a basic scenario describing the semantic annotation using SAREF and oneM2M Base ontologies using the oneM2M semantic descriptor resources and semantic discovery and semantic queries |
| Template Version: January 2017 (Do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this report.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

© 2022, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions	4

4	Conventions	5

5	Motivation	5

6	System Description	6

6.1	Use case	6

6.2	Custom Model	6

6.3	Semantic Modelling	7

6.4	Smart Device Modelling	7

7	Semantic Annotation using oneM2M Base Ontology	7

7.1	Describing oneM2M API with oneM2M Base Ontology	7

7.2	Custom Model annotation	7

7.3	Semantic Model annotation	7

7.4	SDT model annotation	7

8	Semantic Queries	7

8.1	Discovery Queries	7

8.2	Interoperability Queries	8

9	Procedures	8

9.1	Introduction	8

9.2	Call Flows	8

History	8

# 1	Scope

The present document provides a simple use case for guiding application developers to model physical devices in oneM2M and adding semantic annotations that will enable interoperability of devices that are modelled in oneM2M:

Description of a physical device that is to be modelled in oneM2M,

Description of methods that can be used to model the device using oneM2M resources and procedures,

The semantic annotation of the devices using the oneM2M base ontology,

The semantic queries that can be used to discover device capabilities and enable interoperability,

The call flows for implementation of the use case with a focus on the semantic aspects.

# 2	References

## 2.1	Normative references

Normative references are not applicable in the present document.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at: http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	oneM2M TS-0011: "Common Terminology".

[i.3]	oneM2M TS-0012: "Base Ontology".

[i.4]	oneM2M TS-0030: "Generic Interworking".

[i.5]	oneM2M TS-0001: "Functional Architecture".

[i.6]	oneM2M TS-0004: "Service Layer Core Protocol".

[i.7]	oneM2M TS-0009: "HTTP Protocol Binding".

[i.8] 	ETSI TS® 103 783: SAREF: SDT interoperability and oneM2M base ontology alignment

# 3	Definitions

For the purposes of the present document, the terms and definitions given in oneM2M TS-0011 [i.2] apply.

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# 5	Motivation

The assumption of many existing oneM2M applications is that they interact with other oneM2M applications through known resource structures. They either create the resources themselves or are configured to use specific resources. Information is typically stored in containers, sometimes as base64-encoded content instances, with the implicit assumption that applications have a-priori knowledge of the syntax and semantics of this information.

Depending on a-priori knowledge of the structures and data works well for small-scale vertical deployments of IoT devices. When the deployment evolves to include new devices, the existing applications change to reflect the new additions. However, in larger systems of IoT devices where the IoT devices may be a part of a legacy deployment or more than a single vertical solution, changes to all existing applications may become impractical. To enable growth and diversity of IoT devices in large heterogenous settings, applications need to be able to discover the structure and meaning of data from devices and how to use the services of the devices. In oneM2M Release 1 support for discovery of resources based on specific attribute values and the use of labels was defined. The agreement of a fixed set of labels (using a-priori knowledge) can be a viable solution for small deployments.

Figure 1: Semantic understanding of device and data in IoT deployments

For medium or large deployments of heterogeneous IoT devices a more expressive approach for describing and discovering IoT devices is provided by oneM2M. Each type of device in a heterogeneous deployment can model services and data in the oneM2M Service Layer using different structures and syntax of data. For example, temperature sensors may report measurements using different units such as Celsius, Fahrenheit and Kelvin. Additionally, those IoT devices may measure different aspects, such as indoor temperature, outdoor temperature, refrigerator temperature, etc., and the representation of the measurement may differ as well.

Figure 2: Meaningfulness of data from IoT devices

With semantic annotations in oneM2M all the different aspects of IoT devices can be described using RDF triples, which is a standard semantic format. The vocabulary used for a semantic description can be defined according to an ontology such as SAREF. With semantic discovery, applications can describe precisely what information they need or can deal with. This is powered by specifying a semantic filter using the SPARQL query language. The SPARQL filter is matched against the respective semantic annotations of each resource within the discovery scope. This feature in oneM2M helps applications to properly handle the data from the IoT devices.

Besides differences in the data from an IoT device in oneM2M the information model of devices can be modelled in a variety of ways. As with most IoT platforms, oneM2M supports custom information models that are defined for a specific use case and work well in small scale or single vertical scenarios. Another method that oneM2M defines to model devices is based on the semantic description of a device that is mapped to a resource structure (see TS-0030).  A third approach to modelling devices in oneM2M is the use of Smart Device Templates (see TS-0023).

With all these options available to model a device the ability to have a-priori knowledge of a device model becomes less likely as IoT deployments scale beyond small vertical use cases. The oneM2M Base Ontology addresses this and enables developers of these different models to make them interoperable if the appropriate semantic annotations are made and semantic filtering is used to discover the appropriate API for a model. The focus of the remainder of this developer guide is to demonstrate this process.

# 6	System Description

## 6.1	Use case

Describe a washing machine

## 6.2	Custom Model

Custom washing machine model

## 6.3	Semantic Modelling

A model based on semantic description (ref TS-0012).

## 6.4	Smart Device Modelling

Washing machine using SDT.

# 7	Semantic Annotation using oneM2M Base Ontology

## 7.1	Describing oneM2M API with oneM2M Base Ontology

Describe the specific base ontology triples needed

## 7.2	Custom Model annotation

Show the specific annotations of this model

## 7.3	Semantic Model annotation

Show the specific annotations of this model

## 7.4	SDT model annotation

Show the specific annotations of this model

# 8	Semantic Queries

This section describes the semantic queries and how the responses can be used to achieve interoperability

Generally, these queries are executed by an application that is designed to use the IoT devices. In this use case the application is a smart phone-based washing machine control application.

## 8.1	Discovery Queries

## 8.2	Interoperability Queries

# 9	Procedures

## 9.1	Introduction

- Implement each of the models and implementation of basic queries that a washing machine application would use to control any of the washing machine models described.

- This will show each model being created, the application being created, the queries issued by the application to control the washing machine.

- These call flows are described an implemented using python in a jupyter notebook that can be downloaded at oneM2M gitlab.

## 9.2	Call Flows

Description of the call flows

# History

| Publication history |  |  |
| --- | --- | --- |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.0 | 2022-03-09 | SKELETON |