| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TDE 55 |
| Source:* | TF-oneM2M |
| Date:* | 2022-07-06 |
| Reason for Change/s:* | New TPs for crossResourceSubscription release 4 |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018 V4.6.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

New TPs for crossResourceSubscription Release 4 according to contributions SDS-2021-087 and SDS-2021-088 latest revisions.

### ----------------------Start of change 1-------------------------------------------

TP/oneM2M/CSE/SUB/NTF/xx1

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx1 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a crossResourceSubscription CREATE request if neither regularResourcesAsTarget nor subscriptionResourcesAsTarget attributes are present |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing crossResourceSubscription resource containing no regularResourcesAsTarget attribute and no subscriptionResourcesAsTarget attribute } |  | IUT AE |
|  | then { The IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/xx2

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx2 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a crossResourceSubscription CREATE request if the eventNotificationCriteriaSet attribute is set to EVENT_NOTIFICATION_CRITERIA_SET and regularResourcesAsTarget attribute is set |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource on CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing crossResourceSubscription resource containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS and eventNotificationCriteriaSet attribute set to EVENT_NOTIFICATION_CRITERIA_SET } |  | IUT AE |
|  | then { The IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

| TP Id | EVENT_NOTIFICATION_CRITERIA_SET |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/xx2_ENCS_NP | Attribute not present |
| TP/oneM2M/CSE/SUB/NTF/xx2_ENCS_IV | {ENC_1, ENC_2} |

TP/oneM2M/CSE/SUB/NTF/xx3

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx3 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a subscription CREATE request to the target resource indicated in the reqularResourcesAsTarget attribute of a crossResourceSubscription CREATE request |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing crossResourceSubscription resource containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS and eventNotificationCriteriaSet attribute set to {EVENT_NOTIFICATION_CRITERIA} } |  | IUT AE |
|  | then { The IUT sends a valid CREATE request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing subscription resource containing eventNotificationCriteria attribute set to EVENT_NOTIFICATION_CRITERIA and notificationURI attribute set to CROSS_RESOURCE_SUBSCRIPTION_ID and associatedCrossResourceSub attribute set to CROSS_RESOURCE_SUBSCRIPTION_ID } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/xx4

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx4 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a crossResourceSubscription CREATE request if any subscription for a target resource indicated in regularResourcesAsTarget attribute is not successfully created |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the “initial state” and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having received a valid CREATE request from AE containing Content containing crossResourceSubscription resource containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS and the IUT having sent a valid CREATE request to CSE containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing subscription resource representation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid response from CSE containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT CSE |
|  | then { The IUT sends a valid Response to AE containing Response Status Code set to 4xxx (CROSS_RESOURCE_OPERATION_FAILURE) } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/xx5

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx5 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a subscription UPDATE request to the subscription resource indicated in the subscriptionResourcesAsTarget attribute of a crossResourceSubscription CREATE request |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a subscription resource at SUBSCRIPTION_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the resource SUBSCRIPTION_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing crossResourceSubscription resource containing subscriptionResourcesAsTarget attribute containing SUBSCRIPTION_RESOURCE_ADDRESS } |  | IUT AE |
|  | then { The IUT sends a valid UPDATE request to CSE containing To set to SUBSCRIPTION_RESOURCE_ADDRESS and From set to AE_ID and Content containing subscription resource containing notificationURI attribute including CROSS_RESOURCE_SUBSCRIPTION_ID and associatedCrossResourceSub attribute including CROSS_RESOURCE_SUBSCRIPTION_ID } |  | IUT CSE |

TP/oneM2M/CSE/SUB/NTF/xx6

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx6 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT records notification statistics in the notificationStatsInfo attribute of the crossResourceSubscription resource if the notificationStatsEnable attribute of the crossResourceSubscription CREATE request is set to TRUE |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing notificationStatsEnable set to TRUE and regularResourcesAsTarget attribute having CONTAINER_RESOURCE_ADDRESS and the IUT having sent a crossResourceSubscription notification to AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS From set to AE_ID } |  | IUT AE |

|  | then { The IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing crossResourceSubscription resource containing a notificationStatsInfo attribute containing one entry } | IUT AE |
| --- | --- | --- |

TP/oneM2M/CSE/SUB/NTF/xx7

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/xx7 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a crossResourceSubscription UPDATE request if ATTRIBUTE attribute is present |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing regularResourcesAsTarget attribute having CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS and From set to AE_ID and Content containing crossResourceSubscription resource containing a valid ATTRIBUTE attribute } |  | IUT AE |
|  | then { The IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT AE |

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/xx7_RRAT | regularResourcesAsTarget |
| TP/oneM2M/CSE/SUB/NTF/xx7_SRAT | subscriptionResourcesAsTarget |
| TP/oneM2M/CSE/SUB/NTF/xx7_ENCS | eventNotificationCriteriaSet |

TP/oneM2M/CSE/SUB/NTF/x08

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/x08 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT restarts the time window for a crossResourceSubscripton resource if ATTRIBUTE is properly updated |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubsription resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS timeWindowType attribute set to TIME_WINDOW_TYPE_1 and timeWindowSize attribute set to DURATION_1 and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 and the IUT having updated CROSS_RESOURCE_SUBS_ADDRESS resource with ATTRIBUTE attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request (Req1) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_1 and From set to AE_ID and Content containing container resource containing valid labels attribute and the IUT receives a valid UPDATE Request (Req2) from AE1 containing To set to CONTAINER_RESOURCE_ADDRESS_2 and From set to AE_ID and Content containing container resource containing valid labels attribute and timer has not reached NEW_TIME_LIMIT } |  | IUT AE1 IUT AE1 |
|  | then { the IUT sends a valid Response to AE1 for Req1 and the IUT sends a valid Response to AE1 for Req2 and the IUT sends a NOTIFY Request to AE2 generated from crossResourceSubsription on CROSS_RESOURCE_SUBS_ADDRESS } |  | IUT AE1 IUT AE1 IUT AE2 |

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/x08_TWT | TIME_WINDOW_TYPE_2 |
| TP/oneM2M/CSE/SUB/NTF/x08_TWS | DURATION_2 |

TP/oneM2M/CSE/SUB/NTF/x09

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/x09 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT restarts recording notification statistics in the notificationStatsInfo attribute of the crossResourceSubscription resource if the notificationStatsEnable attribute of the crossResourceSubscription UPDATE request is set to TRUE |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing notificationStatsEnable set to TRUE and regularResourcesAsTarget attribute having CONTAINER_RESOURCE_ADDRESS and the IUT having sent a crossResourceSubscription notification to AE and the IUT having updated the crossResourceSubscription resource containing notificationStatsEnable set to TRUE and and the IUT having sent a crossResourceSubscription notification to AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS From set to AE_ID } |  | IUT AE |
|  | then { The IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing crossResourceSubscription resource containing a notificationStatsInfo attribute containing one entry } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/x10

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/x10 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT stops recording notification statistics in the notificationStatsInfo attribute of the crossResourceSubscription resource if the notificationStatsEnable attribute of the crossResourceSubscription UPDATE request is set to FALSE and keeps the current value of the notificationStatsInfo |  |
| Reference |  | TS-0001 [1] Clause 9.6.58, TS-0004 [2] Clause 7.4.58.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing notificationStatsEnable set to TRUE and regularResourcesAsTarget attribute having CONTAINER_RESOURCE_ADDRESS and the IUT having sent a crossResourceSubscription notification to AE and the IUT having updated the crossResourceSubscription resource containing notificationStatsEnable set to FALSE and and the IUT having sent a crossResourceSubscription notification to AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS From set to AE_ID } |  | IUT AE |
|  | then { The IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing crossResourceSubscription resource containing a notificationStatsInfo attribute containing one entry } |  | IUT AE |

TP/oneM2M/CSE/SUB/NTF/x11

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/x11 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriberURI attribute when the crossResourceSubscription resource is deleted |  |
| Reference |  | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing regularResourcesAsTarget attribute having CONTAINER_RESOURCE_ADDRESS subscriberURI set to AE_RESOURCE_ADDRESS and the IUT having received a valid DELETE Request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS and From set to AE_ID and the IUT having sent a valid DELETE request to CSE containing To set to SUBSCRIPTION_CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid response from CSE containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning To set to Content containing notification message containing subscriptionDeletion attribute set to TRUE and subscriptionReference attribute set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS } |  | IUT  AE IUT AE |

TP/oneM2M/CSE/SUB/NTF/x12

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/x12 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes the subscription resources created during the crossResourceSubscription creation when the crossResourceSubscription resource is deleted |  |
| Reference |  | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a container resource at CONTAINER_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing regularResourcesAsTarget attribute having CONTAINER_RESOURCE_ADDRESS subscriberURI set to AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid DELETE Request to CSE containing To set to SUBSCRIPTION_CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

TP/oneM2M/CSE/SUB/NTF/x13

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/x13 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates the subscription resources indicated in the subscriptionResourcesAsTarget attribute to remove the resource identifier of the crossResourceSubscription from the subscriptio’s associatedCrossResourceSub and notificationURI attributes when the crossResourceSubscription resource is deleted |  |
| Reference |  | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to CSE containing a subscription resource at SUBSCRIPTION_RESOURCE_ADDRESS containing notificationURI set to AE_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the resource SUBSCRIPTION_RESOURCE_ADDRESS and the IUT having created a crossResourceSubscription resource containing subscriptionResourcesAsTarget attribute having SUBSCRIPTION_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CROSS_RESOURCE_SUBSCRIPTION_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid UPDATE Request to CSE containing To set to SUBSCRIPTION_CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing subscription resource containing associatedCrossResourceSub set to NULL and notificationURI set to AE_RESOURCE_ADDRESS } |  | IUT  CSE |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?