	Doc#  TDE-2022-0045R01-TP-SSN_Create.DOC
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	TDE 55

	Source:*
	Rahul Kumar, C-DOT, krahul@cdot.in
Anupama Chopra, C-DOT, anupama@cdot.in 
Poornima Shandilya, C-DOT, poornima@cdot.in

	Date:*
	2022-07-14

	Reason for Change/s:*
	See the introduction

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0018 V4.6.0

	Clauses *
	7.2.2.18.4.2

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Other TS/TR(s) impacted
	<TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR>

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES 
  NO 
This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2020 (do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar.
Use change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This CR proposes to add TPs for <ServiceSubscribedNode> resource as per changes in TS-0001 Section 10.2.12.6 Create <serviceSubscribedNode> procedure. 
	Table 10.2.12.6-1: <serviceSubscribedNode> CREATE

<serviceSubscribedNode> CREATE 

Information in Request message

All parameters defined in table 8.1.2-3 apply with the specific details for:

To: The Receiver or Hosting CSE shall be an IN-CSE
Content: The resource content shall provide the information as defined in clause 9.6.20
Processing at Originator before sending Request

According to clause 10.1.2
Processing at Receiver

According to clause 10.1.2
Information in Response message

All parameters defined in table 8.1.3-1 apply with the specific details for:

Content: Address of the created <serviceSubscribedNode> resource, according to clause 10.1.2
Processing at Originator after receiving Response

According to clause 10.1.2
· After a successful creation of the <serviceSubscribedNode> resource, the Originator shall use a CREATE operation request to create a <node> resource. The request shall address a <CSEBase> of a Hosting CSE. The Originator shall set nodeID attribute of this <node> resource to a value equal to nodeID attribute of the <serviceSubscribedNode> resource.
· The Originator shall use a CREATE operation request to create a [registration] resource. The request shall address the above-mentioned <node> resource. The Originator shall set M2M-Sub-ID attribute of [registration] resource to a value equal to M2M-Sub-ID attribute of the <m2mServiceSubscriptionProfile> resource which is the parent of this <serviceSubscribedNode> resource.
· If the node which is represented by this <node> resource intends to use Security Association then the Originator shall use a CREATE operation request to create an [authenticationProfile] resource. The request shall address the above-mentioned <node> resource. The Originator shall set M2M-Sub-ID attribute of the [authenticationProfile] resource to a value equal to the M2M-Sub-ID attribute of  the <m2mServiceSubscriptionProfile> resource which is the parent of this  <serviceSubscribedNode> resource.
Exceptions

According to clause 10.1.2



-----------------------Start of change 1-------------------------------------------
7.2.1.8
Security (SEC)
7.2.1.8.1
Authentication
7.2.1.8.1.1
ESPrim (ESP)
7.2.1.8.1.1.1
NOTIFY Operation
TP/oneM2M/AE/SEC/ESP/NTF/001

	TP Id
	TP/oneM2M/AE/SEC/ESP/NTF/001

	Test objective
	Check that the IUT sends a request for receiverESPrimRandObject, if the e2eSecInfo attribute of the Receiver’s <remoteCSE> resource in CSE2 does not include a sharedReceiverESPrimRandObject

	Reference
	TS-0003 8.4.2 – step B.2D, TS-0004 7.5.1.2.11

	Config Id
	CF03

	PICS Selection
	PICS_AE

	Initial conditions
	with {

the IUT being registered 
and the IUT being switched on 
and the IUT having established a pairwiseESPrimKey with the Receiver (CSE1) 
and the IUT having sent a RETRIEVE Request on the e2eSecInfo attribute in resource 

TARGET_REMOTE_CSE_ADDRESS

}

	Expected behaviour
	Test events
	Direction

	
	when {

the IUT receives a valid Response from CSE2 containing
 

Response Status Code set to 2000 (OK) and


Content containing

<remoteCSE>resource containing



  e2eSecInfo attribute containing




no sharedReceiverESPrimRandObject parameter

}
	IUT ( CSE2

	
	then {

the IUT sends a valid NOTIFY Request to Receiver CSE(CSE1) containing 

           To set to TARGET_RECEIVER_ADDRESS  and


From set to AE_ID and


Content containing



securityInfo element containing




securityInfoType element set to 3  

                      (receiverESPrimRandObject Request)
}
	CSE1 ( IUT


..
..

…

7.2.1.9
Service Subscription Management(SSM)
7.2.1.9.1
Service Subscribed Node(SSN)

7.2.1.9.1.1
CREATE Operation
TP/oneM2M/AE/SSM/SSN/CRE/001
	TP Id
	TP/oneM2M/AE/SSM/SSN/CRE/001

	Test objective
	Check that the IUT sends a <node> resource creation request, after the creation of <serviceSubscribedNode> resource

	Reference
	TS-0001 [1], clauses 10.2.12.6, TS-0004 [2], clause 7.4.20.2.1

	Config Id
	CF03

	Parent Release
	Release 4

	PICS Selection
	PICS_AE

	Initial conditions
	with {

the IUT being registered  
      the IUT being switched on      
      and the IUT having  created a serviceSubscribedNode resource
      containing
           nodeID attribute set to NODE_ID

and the IUT having privileges to perform CREATE OPERATION on the resource
               CSEBASE_RESOURCE_ADDRESS               
}

	Expected behaviour
	Test events
	Direction

	
	when {

the IUT is triggered to send a valid CREATE Request
      containing 


To set to CSEBASE_RESOURCE_ADDRESS and


From set to AE-ID and 
            Content containing 

node resource representation containing 




    nodeID attribute set to NODE_ID 
}
	NA

	
	then {

the IUT sends a valid CREATE Request containing 


To set to CSEBASE_RESOURCE_ADDRESS and


From set to AE-ID and 
            Content containing 

node resource representation containing 




    nodeID attribute set to NODE_ID 
}
	IUT  CSE


TP/oneM2M/AE/SSM/SSN/CRE/002
	TP Id
	TP/oneM2M/AE/SSM/SSN/CRE/002

	Test objective
	Check that the IUT sends a  [Registration]  resource create request, after the creation of <serviceSubscribedNode> resource and <node> resource.

	Reference
	TS-0001 [1], clauses 10.2.12.6, TS-0004 [2], clause 7.4.20.2.1

	Config Id
	CF03

	Parent Release
	Release 4

	PICS Selection
	PICS_AE

	Initial conditions
	with {

the IUT being registered
      the IUT being switched on
      and the IUT having created a m2mServiceSubscriptionProfile resource 
      containing
          M2M-Sub-ID attribute set to M2M_SUB_ID
          resourceID set to RESOURCE_ID
      and the IUT having  created a serviceSubscribedNode resource
      containing
            parentID attribute set to RESOURCE_ID
           nodeID attribute set to NODE_ID
      and the IUT having created a  <node> resource 
      containing
            nodeID attribute set to NODE_ID

and the IUT having privileges to perform CREATE OPERATION on the <node> resource
               TARGET_RESOURCE_ADDRESS
}

	Expected behaviour
	Test events
	Direction

	
	when {

the IUT is triggered to send a valid CREATE Request
      containing 


To set to TARGET_RESOURCE_ADDRESS and
           From set to AE-ID
            Content containing 

registration resource representation containing 




    M2M-Sub-ID attribute set to M2M_SUB_ID
}
	NA

	
	then {

the IUT sends a valid CREATE Request

      containing 

     To set to TARGET_RESOURCE_ADDRESS and
           From set to AE-ID
            Content containing 

registration resource representation containing 




    M2M-Sub-ID attribute set to M2M_SUB_ID
}
	IUT  CSE


TP/oneM2M/AE/SSM/SSN/CRE/003
	TP Id
	TP/oneM2M/AE/SSM/SSN/CRE/003

	Test objective
	Check that the IUT sends a [authenticationProfile]  resource create request, after the creation of <serviceSubscribedNode> resource and <node> resource, if the <node> resource intends to use Security Association.

	Reference
	TS-0001 [1], clauses 10.2.12.6, TS-0004 [2], clause 7.4.20.2.1

	Config Id
	CF03

	Parent Release
	Release 4

	PICS Selection
	PICS_AE

	Initial conditions
	with {

the IUT being registered
      the IUT being switched on
      and the IUT having created a m2mServiceSubscriptionProfile resource 
      containing
          M2M-Sub-ID attribute set to M2M_SUB_ID
          resourceID set to RESOURCE_ID
      and the IUT having  created a serviceSubscribedNode resource
      containing
            parentID attribute set to RESOURCE_ID
           nodeID attribute set to NODE_ID
      and the IUT having created a  <node> resource 
      containing
            nodeID attribute set to NODE_ID

and the IUT having privileges to perform CREATE OPERATION on the <node> resource
               TARGET_RESOURCE_ADDRESS
}

	Expected behaviour
	Test events
	Direction

	
	when {

the IUT is triggered to send a valid CREATE Request 
       containing 


To set to TARGET_RESOURCE_ADDRESS and
            From set to AE-ID
            Content containing 

authenticationProfile resource representation containing 




    M2M-Sub-ID attribute set to M2M_SUB_ID
}
	NA

	
	then {

the IUT sends a valid CREATE Request 

       containing 


To set to TARGET_RESOURCE_ADDRESS and
            From set to AE-ID
            Content containing 

authenticationProfile resource representation containing 




    M2M-Sub-ID attribute set to M2M_SUB_ID
}
	IUT  CSE


-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 9 (of 9)



[image: image1.png]