| OneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-Requirements-V-0.5.0 |
| Document Name: | oneM2M Requirements Technical Specification |
| Date: | <2013-08-09> |
| Abstract: | This document contains the technical requirements for OneM2M release 1. |

oneM2M IPR and copyright statements

“Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of an agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

# Contents

Contents	2

6.	1	Scope	3

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols, abbreviations and acronyms	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

3.4	Acronyms	6

4	Conventions	7

5	Introduction to the M2M ecosystem	8

5.1	Functional roles description	8

6	Functional Requirements	10

6.1	Overall System Requirements	10

6.2	Management Requirements	15

6.3	Abstraction & Semantics Requirements	16

6.3.1	 Abstraction Requirements	16

6.3.2	 Semantics Requirements	16

6.4	Security Requirements	17

6.5	Charging Requirements	18

6.5	Operational Requirements	19

7	Non-Functional Requirements (non-normative)	19

Proforma copyright release text block	20

Annex <A>(Informative/Normative): Title of annex (style H9)	21

Annex <y>:Bibliography	22

History	23

# 1	Scope

This present document contains an informative functional role model and  normative technical requirements for OneM2M.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TR XXXX, “oneM2M Definitions and Acronyms”

Editor’s Note: to be replaced by a reference to the oneM2M Definitions document

[x]	“<number> <source> <Title> <specific version>”

[y]	“<number> <source> <Title>”

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M TR XXXX: ”oneM2M Drafring Rules”

Editor’s Note: to be replaced by a reference to the oneM2M Drafting Rules document

# 3	Definitions, symbols, abbreviations and acronyms

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in “oneM2M Definitions and Acronyms” [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [1].

M2M Application: applications that run the service logic and use Service Capabilities accessible via open interfaces.

M2M Application Service: an M2M Application Service is realized through the service logic of an M2M Application and is operated by the end user or an M2M Application Service Provider.

M2M Application Service Provider: is an entity (e.g. a company) that provides M2M Application Services to the User.

M2M Service: is the set of functionalities that a M2M Service Capability Layer makes available through the standardized interfaces.

Editor’s Note: further description of M2M Service is FFS

M2M Service Provider: is an entity (e.g. a company) that provides M2M Services of the M2M System to a M2M Application Service Provider or to the User.

M2M Service Capability Set (MSCS): is the entirety of Service Capabilities in the M2M System that are operated by an M2M Service Provider.

M2M System: The M2M System is the system under development by the oneM2M global initiative that enables deployable M2M solutions.

Editor’s Note: further description of M2M System is FFS

Service Capabilities: Service Capabilities provide functions that are to be shared by different applications. Service Capabilities expose functionalities through a set of open interfaces.

Editor’s Note: further description of Service Capabilities is FFS

Underlying Network: networks, busses and other technology assisting in data transport.

Editor’s Note: further  description of the Underlying Network is FFS

Network Operator: is an entity (e.g. a company) that operates a Underlying Network.

User: The actual user of the products or services of an M2M solution.

Editor’s Note: further description of the User is FFS

Dynamic Device/Gateway Context: Dynamic metrics, which may impact the M2M operations of M2M Devices/Gateways.

Static Device/Gateway Context: Static metrics, which may impact the M2M operations of M2M Devices/Gateways.

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

## 3.4	Acronyms

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# 4	Conventions

The key words “shall”, “shall not”, “should”, “should not”, “may”, “need not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.Error: Reference source not found].

# 5	Introduction to the M2M ecosystem

Editor’s Note: This section should contain an informative description of the M2M System to assist the reader understanding the requirements contained in the current document

## 5.1	Functional roles description

Figure 1: Functional Roles in the M2M Ecosystem

- The User (individual or company – aka: end-user)
fulfils all of the following criteria

- Uses an M2M solution

Editor’s Note: further description of the User is FFS

- The Application Service Provider
fulfils all of the following criteria

- Provides an M2M Application Service

- Operates M2M Applications

- The M2M Service Provider
fulfils all of the following criteria

- Provides M2M Services to  Application Service Providers

- Operates an M2M Service Capability Set(MSCS)

- The Network Operator
fulfils all of the following criteria

- Provides Connectivity and related services for M2M Service Providers

- Operates an Underlying Network. Such an Underlying Network could e.g. be a telecom network.

Any of the above functional roles may coincide with any of the other roles. These functional roles do not imply business roles or architectural assumptions.

# 6	Functional Requirements

## 6.1	Overall System Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| OSR-001 | The M2M System shall be able to allow communication between M2M Applications in the Network Domain, and the M2M Applications in the Device Domain, by using multiple communication means based on IP Access. |  |
| OSR-002 | The M2M system shall support communication means that can accommodate devices with constrained computing (e.g. small CPU, memory, battery) or communication capabilities (e.g. 2G wireless modem, certain WLAN node) as well as rich computing (e.g. large CPU, memory) or communication (e.g. 3/4G wireless modem, wireline) capabilities. |  |
| OSR-003 | The M2M system shall support the ability to maintain M2M session in coordination with application session for those M2M applications that require it. Editor’s Note: M2M session need to defined. |  |
| OSR-004 | M2M system shall support the ability to support session-less application communications for those M2M applications that require it. |  |
| OSR-005 | The M2M system shall be able to expose the services offered by telecommunications networks to Applications (e.g. SMS, USSD, localization, subscription configuration, authentication (e.g. Generic Bootstrapping Architecture), etc),subject to restriction based on Network Operator’s policy. |  |
| OSR-006 | The M2M System shall be able to reuse the services offered by underlying networks to M2M Applications and/or M2M Service Layer, by means of open access models (e.g. OMA, GSMA OneAPI framework). An example of available services is: - IP Multimedia communications - Messaging - Location - Charging and billing services - Device information and profiles - Configuration and management of devices - Triggering, monitoring of devices - Small data transmission - Group management The set of features or APIs to be supported depends on the M2M Service Capabilities and access to available APIs. |  |
| OSR-007 | The M2M system shall provide a mechanism for applications to interact with the applications and data/information belonging to another M2M system, subject to permission. |  |
| OSR-008 | The M2M System shall provide the capability for M2M Applications to communicate with a device (i.e. application in the device) without the need for the M2M Applications to be aware of the network technology and the specific communication protocol of the device. |  |
| OSR-009 | The M2M System shall support the ability for single or multiple M2M Applications to interact with a single or multiple M2M Devices/Gateways (application in the device/gateway). Editor’s Note: The relation M2M Network Application to M2M Device/Gateway can be 1:1, 1:n, n:1 and/or n:m |  |
| OSR-010 | The M2M system shall support mechanisms for confirmed delivery of a message to its addressee to those M2M Applications requesting reliable delivery to dectect failure of message within a given time interval. |  |
| OSR-011 | The M2M System shall be able to request different communication paths, from the Underlying Network based on Underlying Network Operator and/or M2M Service Provider policies, routing mechanisms for transmission failures or request from M2M Applications. |  |
| OSR-012 | The M2M System shall support communications between M2M Applications and Devices supporting M2M services by means of continuous or non-continuous connectivity. |  |
| OSR-013 | The M2M System shall be aware of the delay tolerance acceptable by the M2M Application and shall schedule the communication accordingly or request the underlying network to do it, based on policies criteria. |  |
| OSR-014 | The M2M System shall be able to communicate with M2M Devices, behind an M2M Gateway that supports heterogeneous M2M area networks. |  |
| OSR-015 | The M2M system shall support different communication patterns including infrequent communications, small data transfer, large file transfer, streamed communication. |  |
| OSR-016 | The M2M System shall provide the capability to notify M2M Applications of the availability of, and changes to, available M2M Application/management information on the M2M Device/Gateway, including changes to the M2M Area Network. |  |
| OSR-017 | The M2M System shall be capable of offering access to different sets of M2M services to M2M Application providers. The minimum set of services are: - Connectivity management - Device management (service level management) - Application Data management The M2M System shall facilitate different deployment scenarios by offering access to different sets of services independently, to a group of those or to all of them. |  |
| OSR-018 | The M2M System shall be capable of offering M2M Services to M2M Devices roaming across Underlying cellular Networks,subject to restriction based on Network Operator’s policy. |  |
| OSR-019 | The M2M System shall support the capabilities for data repository (i.e. to collect/store) and data transfer from one or more M2M Devices or M2M Gateways, in ways requested by the M2M Application Infrastructure as listed below: - action initiated either by an M2M Device, M2M Gateway, M2M Services Infrastructure, or M2M Application Infrastructure - when triggered by schedule or event; - for specified data For delivery to one or more M2M Gateways, M2M Services Infrastructure, or M2M Application Infrastructure |  |
| OSR-020 | The M2M System shall be able to support policies and their management regarding the aspects of storage and retrieval of data/information. |  |
| OSR-021 | The M2M system shall be able to provide mechanisms to enable sharing of data among multiple M2M Applications. |  |
| OSR-022 | When some of the components of M2M System are not available (e.g. WAN connection lost), the M2M System shall be able to support the normal operation of components of the M2M System that are available. |  |
| OSR-023 | The M2M System shall be capable of identifying the M2M Services to be used by M2M service subscriptions. Note: M2M Service subscriptions are not Application subscriptions (e.g., Home Energy Management). Editor’s Note: M2M Service Subscription needs to be defined. |  |
| OSR-024 | The M2M System shall be capable of identifying the devices used by M2M service subscriptions. |  |
| OSR-025 | The M2M System shall be capable of identifying the M2M Applications used by M2M service subscriptions. |  |
| OSR-026 | If provided by the Underlying Network, the M2M System shall be capable of associating the device used by M2M Service Subscriptions with the device identifiers offered by the Underlying Network and the device. |  |
| OSR-027 | The M2M System shall provide a generic mechanism to support transparent exchange of information between the M2M Application and the Underlying network, subject to restriction based on M2M Service Provider’s policy and/or Network Operator ‘s policy. Editor’s Note: Transparent exchange of information implies information that is mainly interpreted by the M2M application and the underlying network provider. |  |
| OSR-028 | The M2M system shall be able to trigger a series of commands to actuators on behalf of applications. |  |
| OSR-029 | The M2M system shall be able to support sending common command(s) to each actuator or sensor via a group. |  |
| OSR-030 | The M2M system shall be able to support the management (i.e. addition, removal, retrieval and update) of the membership of a group. |  |
| OSR-031 | The M2M system shall be able to support a group as a member of another group. |  |
| OSR-032 | The M2M System shall be able to support event categorization (e.g., normal, urgency) associated with data for M2M applications when collecting, storing and reporting that data. Note: Based on the event categories and via interworking with underlying networks, the M2M System can support differentiated services (by providing Quality-of-Service) requested by M2M applications. Editor’s Note: ‘event categorization’ needs to be defined. |  |
| OSR-033 | Based on the Dynamic Device/Gateway Context of the M2M Gateway and/or Device and the defined event categorization, the M2M System shall provide the capability to dynamically adjust the scheduling of reporting and notification of the M2M Device/Gateway. Note: for example, if the battery of Gateway is remained only 10% or below, the Gateway notifies the M2M service platform of the status. The M2M Application in the Infrastructure node will adjust the scheduling of reporting and notification based on the event categorization associated with each message. Consequently, the M2M Gateway operates longer. |  |
| OSR-034 | The M2M System shall support seamless replacement of M2M Devices as well as M2M Gateways (e.g., redirecting traffic, connection, recovery, etc.). |  |
| OSR-035 | The M2M System shall support the exchange of non-Application related relevant information (e.g. Device/Gateway classes) between M2M Device/Gateway and M2M infrastructure for the purpose of efficient communication facilitation. Capability for M2M device to report device class to M2M service infrastructure and for M2M service infrastructure to inform device of the M2M platform capabilities |  |
| OSR-036 | The oneM2M system should provide mechanisms to accept requests from M2M application service providers for compute/analytics services. |  |
| OSR-037 | The M2M System shall enable a M2M Application to request to send data, in a manner independent of the Underlying Network, to the M2M Applications of a group of M2M Devices and M2M Gateways in geographic areas that are specified by the M2M Application. |  |
| OSR-038 | The M2M system shall support the inclusion of M2M Application’s QoS preference in service requests to Underlying Networks. |  |
| OSR-039 | The M2M system shall be able to authorize service requests with QoS preference at service level, but shall pass M2M Application’s QoS preference in service requests to Underlying Network for authorization and granting or negotiation of the service QoS requests. |  |
| OSR-040 | The M2M system shall be able to leverage multiple communication mechanisms (such as USSD or SMS) when available in the underlying networks. |  |
| OSR-041 | The M2M System shall provide a mechanism, which supports the addition of new M2M Services without impact to oneM2M interface interoperability |  |
| OSR-042 | The M2M System shall be able to support M2M Gateways and M2M Devices that offer communication services to M2M Application to buffer incoming messages for communicating data to another M2M Gateway/Device/Infrastructure and support forwarding them at a later time depending on communication policies and based on service preference associated with the messages. |  |
| OSR-043 | The M2M system shall enable an M2M Application to send a communication request with the following service preference: - QoS parameters, including delay tolerance, for initiating the delivery of data - categorizing communication requests into different levels of priority or QoS classes |  |
| OSR-044 | The M2M System shall be able to support concurrent processing of messages within M2M Gateways and/or M2M Devices from different sources with awareness for the service preference associated with the messages while observing the provisioned communication policies. |  |
| OSR-045 | The M2M System shall be able to maintain context associated with M2M sessions (e.g. security context or network connectivity context during the interruption of the session). |  |
| OSR-046 | The M2M System shall be able to support different QoS-levels specifying parameters, such as guaranteed bitrate, delay, delay variation, loss ratio and error rate, etc. |  |
| OSR-047 | The M2M system shall be able to verify that members of a group support a common set of functions. |  |
| OSR-048 | The M2M System shall support communication with devices which are reachable based on defined time schedules (e.g. periodic) as well as devices which are reachable in an unpredictable and spontaneous manner. |  |
| OSR-049 | The M2M System shall be able to be aware of the reachability status and to utilize reachability schedules generated by either the M2M Device or the Infrastructure Domain. |  |
| OSR-050 | The M2M system shall be able to support capability for the M2M application to select communication protocol acknowledgment mechanisms. |  |
| OSR-051 | The M2M system shall be able to support mechanism for the M2M Devices and/or Gateways to report their geographical location information to M2M Applications. Note: Geographical location information can be more than simply longitude and latitude. |  |
| OSR-052 | The M2M system shall provide a M2M Service that allows M2M Devices and/or Gateways to share their own or other M2M Devices’ geographical location information. Note: Geographical location information can be more than simply longitude and latitude. |  |
| OSR-053 | The M2M system shall be able to provide the capability for an M2M Application to selectively share data (e.g. access control) When sharing data among applications. |  |
| OSR-054 | If communication over one communication channel provided by the underlying network can only be triggered by one side (infrastructure domain or field domain), and alternative channel(s) is (are) available in the other direction, the M2M System shall be able to use the alternative channel(s) to trigger bidirectional communication on the first channel. |  |
| OSR-055 | Depending on availability of suitable interfaces provided by the Underlying Network the M2M System shall be able to request the Underlying Network to broadcast / multicast data to a group of M2M Devices in a specified area. |  |
| OSR-056 | The M2M System shall be able to select an appropriate Underlying Network to broadcast or multicast data depending on the network’s broadcast/multicast support and the connectivity supported by the targeted group of M2M Devices/Gateways. |  |
| OSR-057 | The M2M System shall provide a means that enables the interoperability of interfaces among different releases. Note: “Means” above does not imply only technical mechanisms. |  |
| OSR-058 | The M2M System shall be able to support a M2M Application, M2M Device, or M2M Gateway to obtain access to resources of another M2M Application, M2M Device, or M2M Gateway. |  |
| OSR-059 | The M2M System shall be able to provide the capability of M2M Applications to exchange data with one or more authorized M2M Applications which are not known. |  |
| OSR-060 | The M2M System shall enable discovery of usable M2M Applications on an M2M Gateway or at an M2M Device . |  |
| OSR-061 | The M2M System shall enable discovery of M2M Gateways and M2M Devices available to an M2M Application for data exchange. |  |
| OSR-062 | The M2M System shall be able to provide time stamps as needed by common service functions. |  |
| OSR-063 | The M2M system shall be able to support Role-based access control based on service subscriptions. |  |
| OSR-064 | The M2M System should support time synchronization with an external clock source. |  |
| OSR-065 | M2M Devices and M2M Gateways may support time synchronization within the M2M System. |  |
| OSR-066 | The M2M System shall enable means of testing the connectivity towards a set of M2M Applications. |  |
| OSR-067 | The M2M System shall be able to manage the scheduling of M2M Service Layer connectivity and messaging between the Infrastracture Domain and M2M Devices/Gateways. |  |
| OSR-068 | The M2M System shall be able to aggregate messages depending on message delay tolerance and/or category. |  |
| OSR-069 | The oneM2M System shall be able to provide mechanisms to accept M2M Applications for distribution, from M2M application service providers. |  |
| OSR-070 | The oneM2M System shall be able to support the placement and operation of M2M Applications in selected M2M nodes per criteria requested by M2M Applications service providers, subject to access rights. |  |
| OSR-071 | The oneM2M System shall be able to take operational and management action as requested by M2M Applications. |  |
| OSR-072 | When available from an underlying network, the M2M System shall be able to provide the capability to retrieve and report the information regarding whether a M2M Device is authorized to access Underlying Network services. |  |
| OSR-073 | When available from the Underlying Network, the M2M System shall be able to maintain the M2M Service Operational Status of a M2M Device and update it when the Underlying Network connectivity service status changes. |  |
| OSR-074 | The M2M System shall be able to provide the capability to notify an authorized M2M Application when the M2M Service Administrative State or M2M Service Operational Status of a M2M Device changes, if that M2M Application has subscribed for such notifications. |  |
| OSR-075 | The M2M System should be able to enable an authorized M2M Application to set the M2M Service Administrative State of a M2M Device. |  |

Table 1 Overall System Requirements

## 6.2	Management Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| MGR-001 | The M2M System shall be able to support management and configuration of M2M Gateways/ Devices including resource constrained M2M Devices. |  |
| MGR-002 | The M2M System shall provide the capability to discover the M2M Area Networks including information about devices on those networks and the parameters (e.g. topology, protocol) of those networks.. |  |
| MGR-003 | The M2M System shall be able to provide the capability to maintain and describe the management information model of devices and parameters (e.g. topology, protocol) of M2M Area Networks. |  |
| MGR-004 | The M2M System shall support common means to manage devices enabled by different management technologies (e.g. OMA DM, BBF TR069). |  |
| MGR-005 | The M2M System shall provide the capability to manage multiple devices in a grouped manner. |  |
| MGR-006 | The M2M System shall provide the capability for provisioning and configuration of devices in M2M Area Networks . |  |
| MGR-007 | The M2M System shall provide the capability for monitoring and diagnostics of M2M Gateways/Devices in M2M Area Networks . |  |
| MGR-008 | The M2M System shall provide the capability for software management of devices in M2M Area Networks. |  |
| MGR-009 | The M2M System shall provide the capability for rebooting and/or resetting of M2M Gateways/Devices and other devices in M2M Area Networks. |  |
| MGR-010 | The M2M System shall provide the capability for authorizing devices to access M2M Area Networks. |  |
| MGR-011 | The M2M System shall provide the capability for modifying the topology of devices in M2M Area Networks,subject to restriction based on management policy of M2M Area Network. |  |
| MGR-012 | Upon detection of a new device the M2M Gateway shall be able to be provisioned by the M2M Service Infrastructure with an appropriate configuration which is required to handle the detected device. |  |
| MGR-013 | The M2M System shall be able to identify and manage M2M Service status of M2M Devices. |  |
| MGR-014 | The M2M System shall be able to retrieve events and information logged by M2M Gateways/ Devices and other devices in M2M Area Networks. |  |
| MGR-015 | The M2M System shall be able to support firmware management (e.g. update) of M2M Gateways/ Devices and other devices in M2M Area Networks. |  |
| MGR-016 | The M2M System shall be able to retrieve information related to the Static and Dynamic Device/Gateway Context for M2M Gateways/Devices as well as Device Context for other devices in M2M Area Networks. |  |
| MGR-017 | The M2M system shall support the capability to map M2M service subscription role(s) to roles used within technology specific Device Management protocols. |  |

Table 2 Management Requirements

## 6.3	Abstraction & Semantics Requirements

### 6.3.1	 Abstraction Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| ABR-001 | The M2M system shall provide a generic structure for data representation. |  |
| ABR-002 | The M2M system shall be able to provide translation mechanisms among Information Models (including meta-data) used by M2M Applications, M2M Devices/Gateways, and other devices. Editor’s Note: need definition for Information Model and Meta-data. |  |
| ABR-003 | The M2M System shall provide capabilities to represent Virtual Devices and Things, (which are not necessarily physical devices.) |  |

Table 3 Abstraction Requirements

### 6.3.2	 Semantics Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| SMR-001 | The M2M System shall provide capabilities to manage semantic descriptions of resources and M2M Applications, e.g, create, retrieve, update, delete, associate/link. Editor’s Note: Resources may be changed the meaning after the discussion of WG2. Editor’s Note: If M2M Application is part of Resource, M2M Application need to be removed. |  |
| SMR-002 | The M2M System shall support a common modeling language for semantic descriptions (including relationships between Things) in order to make them available to M2M Applications. |  |
| SMR-003 | The M2M System shall be able to provide interworking capabilities between different modeling languages for semantic descriptions. |  |
| SMR-004 | The M2M System shall provide capabilities to discover M2M Resources based on semantic descriptions. |  |
| SMR-005 | The M2M System shall support the capability to access semantic descriptions which are outside of the M2M System. |  |
| SMR-006 | The M2M System shall be able to support capabilities for performing M2M data Analytics based on semantic descriptions from M2M Applications/Systems. |  |
| SMR-007 | The M2M System shall be able to provide capabilities for performing Semantic Mash-up using M2M data from M2M Applications/Systems (e.g. to create Virtual Devices, offer new M2M services, etc.) |  |

Table 4 Semantics Requirements

## 6.4	Security Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| SER-001 | The M2M System shall incorporate protection against threats to its availability such as Denial of Service attacks. |  |
| SER-002 | The M2M System shall be able to ensure the confidentiality of data. |  |
| SER-003 | The M2M System shall be able to ensure the integrity of data. |  |
| SER-004 | In case where the M2M devices support USIM / UICC and the underlying networks support network layer security, the M2M system shall be able to leverage device’s USIM / UICC credentials and network’s security capability e.g. 3GPP GBA for establishing the M2M services and applications level security through interfaces to underlying network. |  |
| SER-005 | In case where the M2M devices support USIM/UICC and the underlying networks support network layer security, and when the M2M system is aware of underlying network’s bootstrapping capability e.g. 3GPP GBA, the M2M system shall be able to expose this capability to M2M services and applications through API. |  |
| SER-006 | In case where the M2M devices support USIM / UICC and the underlying networks support network layer security, the M2M system shall be able to leverage device’s USIM / UICC credentials when available to bootstrap m2m security association. |  |
| SER-007 | When some of the components of M2M System are not available (e.g. WAN connection lost), the M2M System shall be able to support the confidentiality and the integrity of data between authorized components of the M2M System that are available. |  |
| SER-008 | The M2M system shall support countermeasures against unauthorized access toM2M services and M2M application services. |  |
| SER-009 | The M2M system shall be able to support mutual authentication for interaction with underlying networks, M2M services and M2M application services. |  |
| SER-010 | The M2M System shall be able to support mechanisms for protection against misuse, cloning, replacement or theft of security credentials. |  |
| SER-011 | The M2M System shall protect the use of the identity of a stakeholder within the M2M system against discovery and misuse by other stakeholders. |  |
| SER-012 | The M2M System shall be able to support countermeasures against Impersonation attacks and Replay attacks. |  |
| SER-013 | The M2M service shall be able to provide the mechanism for integrity-checking on boot, periodically on run-time, and on software upgrades for software/hardware/firmware component(s) on M2M Device(s). |  |
| SER-014 | The M2M System shall be able to provide configuration data to an authenticated and authorized M2M Application in the M2M Gateway/Device. |  |
| SER-015 | - The M2M System shall be able to support mechanisms to provide User identity to authorized and authenticated M2M Applications (e.g., police center) when the M2M System has the User’s consent. Editor’s Note: Definition of User needs to be more clarified to avoid possible misunderstanding of User identification data report. |  |
| SER-016 | The M2M System shall be able to support non repudiation within the M2M service layer and in its authorized interactions with the network and application layers. |  |
| SER-017 | The M2M System shall be able to mitigate identified threats pertaining to the M2M Service layer. Editor’s note: This will be refined after the Security Analysis. |  |
| SER-018 | The M2M system shall enable a stakeholder to use a resource or service and be accountable for that use without exposing its identity to other stakeholders. Editor’s note: “Stakeholder” needs to be defined. |  |
| SER-019 | The M2M system shall be able to use service-level credentials present inside the M2M device for establishing the M2M services and applications level security. |  |
| SER-020 | The M2M system shall enable legitimate M2M Service Providers to provision their own credentials into the M2M devices/gateways. |  |
| SER-021 | The M2M System shall be able to remotely and securely provision M2M security credentials in M2M Devices and/or M2M Gateways. |  |
| SER-022 | The M2M System shall enable M2M Application Service Providers to authorize interactions involving their M2M applications on supporting entities (e.g. Devices/ Gateways/ Service infrastructure). Editor’s note: Whether other stakeholders need to be involved is FFS. |  |
| SER-023 | If HSM is supported the M2M Device should use HSM (Hardware Security Module) to provide security level consistent with the requirement supported by the M2M System. Editor note: clarification regarding security level is needed. |  |
| SER-024 | The M2M system shall enable M2M applications to use different and segregated security environments. |  |
| SER-025 | The M2M system shall be able to prevent stakeholders to identify and/or observe the actions of other stakeholders in the M2M system, e.g. access to resources and services. Note: The above requirement does not cover whatever is outside of the M2M system, e.g. underlying networks. Editor’s note: “Stakeholder” needs to be defined or replaced by a more accurate term. |  |
| SER-026 | The M2M system shall be able to provide mechanism for the protection of confidentiality of the geographical location information. Note: Geographical location information can be more than simply longitude and latitude. |  |

Table 5 Security Requirements

## 6.5	Charging Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| CHG-001 | The M2M system shall support collection of charging specific information related to the individual services facilitated by the system (e.g. Data Management, Device Management and/or Connectivity management), and concurrently with the resource usage. The format of the recorded information shall be fully specified including mandatory and optional elements. Editor Note: this requirement requires further work on the wording. |  |
| CHG-002a | The M2M System shall support mechanisms to facilitate correlation of charging information (e.g. of a User) collected for M2M Services, M2M Application Services and services provided by underlying network operators. |  |
| CHG-002b | The M2M system shall provide means to coordinate charging data records for data usages with differentiated QoS from the underlying network. |  |
| CHG-003 | The M2M system shall be able to reuse existing charging mechanisms of underlying networks. |  |
| CHG-004 | The M2M system shall support transfer of the charging information records to the Billing Domain of the M2M Service Provider, for the purpose of: - subscriber billing - inter-provider billing - provider-to-subscriber accounting including additional functions like statistics. |  |
| CHG-005 | The M2M system should support generation of charging events for the purpose of requesting resource usage authorization from the real time credit control system where the subscriber account is located. The information contained in the charging events and the relevant chargeable events shall be fully specified including mandatory and optional elements. Editor’t Note: A chargeable event is any activity, a provider may want to charge for, which utilizes the resources and related M2M services offered by such provider. A charging event is the set of charging information needed by the credit control system for resource authorization. |  |

Table 6 Charging Requirements

## 6.5	Operational Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| OPR-001 | The M2M System shall provide the capability for monitoring and diagnostics of M2M Applications. |  |
| OPR-002 | The M2M System shall provide the capability for software management of M2M Applications. |  |
| OPR-003 | The M2M System shall be capable of configuring the execution state an M2M Application (start, stop, restart). |  |
| OPR-004 | When suitable interfaces are provided by the Underlying Network, the M2M System shall have the ability to schedule traffic via the Underlying Network based on instructions received from the Underlying Network. |  |
| OPR-005 | The M2M System shall be able to exchange information with M2M Applications related to usage and traffic characteristics of M2M Devices or M2M Gateways by the M2M Application. This information includes the following features for a M2M Device: - Time controlled devices to send or receive data only during defined time intervals Note: “Low mobility” and “Time controlled” are equivalent to the MTC Features specified in [i.x] (section 7.2 of 3GPP TS 22.368) |  |
| OPR-006 | Depending on availability of suitable interfaces provided by the Underlying Network the M2M System shall be able to provide information related to usage and traffic characteristics of M2M Devices or M2M Gateways to the Underlying Network. |  |

Table 7 Charging Requirements

# 7	Non-Functional Requirements (non-normative)

This section is intended to gather high-level principles and guidelines that shall govern the design of the oneM2M system. Such principles and guidelines are fundamental to the design of the M2M system. But as they cannot necessarily be expressed as requirements per se, they shall be introduced and expressed in this section.

| Requirement ID | Description | Release |
| --- | --- | --- |
| NFR-001 | Continua Health Alliance is incorporating a RESTful approach to its design. To support CHA, oneM2M should consider RESTful styles and approaches while designing the oneM2M architecture. |  |
| NFR-002 | The M2M system should communicate using protocols that are efficient in terms of amount of exchanged information over amount of exchanged data measured in bytes. |  |

Table 8 Non-Functional Requirements

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annex <A>(Informative/Normative): Title of annex (style H9)

<Text>

A.1	First clause of the annex (style H1)

<Text>

A.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

The following text is to be used when appropriate:

## Annex <y>:Bibliography

The annex entitled “Bibliography” is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: “<Title>”.

OR

<Publication>: “<Title>”.

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 11 Mar 2013 | Rapporteur Input – Skeleton Draft |
| V.0.0.2 | 11 Mar 2013 | Implement the agreed contributions until TP#3: oneM2M-REQ-2012-0054R07 oneM2M-REQ-2013-0180R03 oneM2M-REQ-2013-0195R01 oneM2M-REQ-2013-0143R02 oneM2M-REQ-2013-0154R03 oneM2M-REQ-2013-0155R03 oneM2M-REQ-2013-0228R02 oneM2M-REQ-2013-0198R03 oneM2M-REQ-2013-0162R02 oneM2M-REQ-2013-0170R04 oneM2M-REQ-2013-0134R02 |
| V0.0.3 | 24 Apr 2013 | Implement the agreed contributions: oneM2M-REQ-2013-0174R05 oneM2M-REQ-2013-0187R07 oneM2M-REQ-2013-0269R02 oneM2M-REQ-2013-0148R04 oneM2M-REQ-2013-0149R03 Note: no changes are required by 149R03. It only states that the current charging requirements covers the overlapped proposals. oneM2M-REQ-2013-0247 Note: Operational Section was added oneM2M-REQ-2013-0252R02 oneM2M-REQ-2013-0268R01 oneM2M-REQ-2013-0285R02 oneM2M-REQ-2013-0310R01 oneM2M-REQ-2013-0271R02 oneM2M-REQ-2013-0291R03 oneM2M-REQ-2013-0280R02 |
| V0.1.0 | 20 May 2013 | Implement the agreed contributions: oneM2M-REQ-2013-0179R08 |
| V0.2.0 | 03 Jun 2013 | Implement the agreed contributions: oneM2M-REQ-2013-0297R04 oneM2M-REQ-2013-0274R01 oneM2M-REQ-2013-0273R02 |
| V0.3.0 | 17 Jun 2013 | Implement the agreed contributions: oneM2M-REQ-2013-0274R02 |
| V0.4.0 | 24 Jun 2013 | Implement the agreed contributions: oneM2M-REQ-2013-0157R06 oneM2M-REQ-2013-0368R01 oneM2M-REQ-2013-0267R05 oneM2M-REQ-2013-0275R03 oneM2M-REQ-2013-0320R02 oneM2M-REQ-2013-0287R05 oneM2M-REQ-2013-0329R01 oneM2M-REQ-2013-0189R08 oneM2M-REQ-2013-0311R02 oneM2M-REQ-2013-0331R02 oneM2M-REQ-2013-0341R01 oneM2M-REQ-2013-0343R04 oneM2M-REQ-2013-0344R05 oneM2M-REQ-2013-0363R02 oneM2M-REQ-2013-0372R01 oneM2M-REQ-2013-0376R01 oneM2M-REQ-2013-0173R05 oneM2M-REQ-2013-0313R02 oneM2M-REQ-2013-0375 oneM2M-REQ-2013-0345R01 |
| V0.5.0 | 09 Aug 2013 | Implement the agreed contributions: oneM2M-REQ-2013-0357R05 oneM2M-REQ-2013-0358R01 oneM2M-REQ-2013-0359R04 oneM2M-REQ-2013-0335R03 - oneM2M-REQ-2013-0366R02 - oneM2M-REQ-2013-0371R01 oneM2M-REQ-2013-0138R05 oneM2M-REQ-2013-0316R03 oneM2M-REQ-2013-0333R03 oneM2M-REQ-2013-0337R05 oneM2M-REQ-2013-0338R02 oneM2M-REQ-2013-0349R02 oneM2M-REQ-2013-0350R05 oneM2M-REQ-2013-0374R02 oneM2M-REQ-2013-0378R01 oneM2M-REQ-2013-0381R03 oneM2M-REQ-2013-0392R01 oneM2M-REQ-2013-0394R02 oneM2M-REQ-2013-0397R01 oneM2M-REQ-2013-0384R03 oneM2M-REQ-2013-0360R03 oneM2M-REQ-2013-0403R01 oneM2M-REQ-2013-0404R01 |