| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0001 - V-0.6.1 |
| Document Name: | oneM2M Functional Architecture Baseline Draft |
| Date: | 2014-May-05 |
| Abstract: | This document specifies the functional architecture for the oneM2M Services Platform. This version of the document is an update over v0.6.0 and incorporates contributions accepted during ARC#10bis (AdHoc) meeting on April 30, 2014; May 1, 2014 and May 2, 2014. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

- About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

- More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

- Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

1	Scope	16

2	References	17

2.1	Normative references	17

2.2	Informative references	17

3	Definitions, symbols and abbreviations	19

3.1	Definitions	19

3.2	Symbols	20

3.3	Abbreviations & Acronyms	20

4	Conventions	22

5	Architecture Model	23

5.1	General Concepts	23

5.2	Architecture Reference Model	23

5.2.1	Functional Architecture	23

5.2.2	Reference Points	24

5.2.2.1	Mca Reference Point	24

5.2.2.2	Mcc Reference Point	24

5.2.2.3	Mcn Reference Point	24

5.2.2.4	Mcc' Reference Point	25

5.2.2.5 	Mch Reference Point	25

6	Configurations supported by oneM2M Architecture	26

6.1	Relationships among oneM2M Entities	26

6.2	Common Services Functions	28

6.2.1	Application and Service Layer Management	28

6.2.1.1	General Concepts	28

6.2.1.2	Detailed Descriptions	28

6.2.1.2.1	Software Management Function	29

6.2.2	Communication Management and Delivery Handling	29

6.2.2.1	General Concepts	29

6.2.2.2	Detailed Descriptions	30

6.2.3	Data Management and Repository	30

6.2.3.1	General Concepts	30

6.2.3.2	Detailed Descriptions	30

6.2.4	Device Management	31

6.2.4.1	General Concepts	31

6.2.4.1.1	Device Management Architecture	31

6.2.4.1.2	Management Server Interaction	32

6.2.4.1.3	Management Client Interaction	34

6.2.4.1.4	Device Management Resource Lifecycle	34

6.2.4.2	Detailed Descriptions	35

6.2.4.2.1	Device Configuration Function	36

6.2.4.2.2	Device Diagnostics and Monitoring Function	36

6.2.4.2.3	Device Firmware Management Function	36

6.2.4.2.4	Device Topology Management Function	36

6.2.5	Discovery	37

6.2.5.1	General Concepts	37

6.2.5.2	Detailed Descriptions	37

6.2.6	Group Management	37

6.2.6.1	General Concepts	37

6.2.6.2	Detailed Descriptions	37

6.2.7	Location	38

6.2.7.1	General Concepts	38

6.2.7.2	Detailed Descriptions	38

6.2.8	Network Service Exposure, Service Execution and Triggering	38

6.2.8.1	General Concepts	38

6.2.8.2	Detailed Descriptions	39

6.2.9	Registration	39

6.2.9.1	General Concepts	39

6.2.9.2	Detailed Descriptions	39

6.2.10	Security	41

6.2.10.1	General Concepts	41

6.2.10.2	Detailed Descriptions	41

6.2.11	Service Charging and Accounting	42

6.2.11.1	General Concepts	42

6.2.11.2	Detailed Descriptions	42

6.2.12	Subscription and Notification	43

6.2.12.1	General Concepts	43

6.2.12.2	Detailed Descriptions	43

6.3	Security Aspects	44

6.4	Inter-M2M SP Communication	44

6.4.1	Inter M2M SP Communication for oneM2M Compliant Nodes	44

6.4.1.1	Public Domain Names and CSEs	45

6.4.2	Inter M2M SP Generic Procedures	45

6.4.2.1	Actions of the Originating M2M Node in the Originating Domain	46

6.4.2.2 	Actions of the Receiving CSE in the Originating Domain	46

6.4.2.3	Actions in the IN of the Target Domain	46

6.4.3. 	DNS Provisioning for Inter-M2M SP Communication	46

6.4.3.1 	Inter-M2M SP Communication Access Control Policies	46

6.5	M2M Service Subscription	46

7	M2M Identification and Addressing	48

7.1	M2M Identifiers	48

7.1.1	M2M Service Provider Identifier (M2M-SP-ID)	48

7.1.2	Application Entity Identifier (AE-ID)	48

7.1.3	Application Identifier (App-ID)	48

7.1.4	CSE Identifier (CSE-ID)	48

7.1.5	M2M Node Identifier (M2M-Node-ID)	48

7.1.6	M2M Service Subscription Identifier (M2M-Sub-ID)	49

7.1.7	M2M Request Identifier (M2M-Request-ID)	49

7.1.8	M2M External Identifier (M2M-Ext-ID)	49

7.1.9	Underlying Network Identifier (UNetwork-ID)	50

7.1.10	Trigger Recipient Identifier (Trigger-Recipient-ID)	50

7.1.11	M2M Service Identifier (M2M-Serv-ID)	50

7.2	M2M Identifiers lifecycle and characteristics	51

7.3	Addressing an Application Entity	52

7.3.1	Application Entity Addressing	52

7.3.2	Application Entity Reachability	52

7.3.2.1	CSE Point of Access (CSE-PoA)	52

7.3.2.2	Locating Application Entities	52

7.3.2.3	Usage of CSE-PoA by the M2M System	52

7.3.2.3.1	CSE-PoA related to CSEs associated with a Fixed Network	53

7.3.2.3.2	CSE-PoA related to CSEs associated with Mobile Networks	53

7.3.2.3.3	CSE-PoA to CSEs associated with multiple Underlying Networks	53

7.3.3	 Notification Re-targeting	53

7.3.3.1	Application Entity Point of Access (AE-PoA)	53

8	Description and Flows of Reference Points	55

8.1	General Communication Flow Scheme on Mca and Mcc Reference Points	55

8.1.1	Description	55

8.1.2	Request	55

8.1.2.1	Summary of Request Message Parameters	60

8.1.3	Response	62

8.1.4	Summary of Response Message Parameters	63

8.2	Procedures for Accessing Resources	64

8.2.1	Accessing Resources in CSEs - Blocking Requests	64

8.2.1.1 	M2M Requests Routing Policies	70

8.2.2	Accessing Resources in CSEs - Non-Blocking Requests	70

8.2.2.1	Response with Reference to Result of Requested Operation	70

8.2.2.2	Synchronous Case	70

8.2.2.3	Asynchronous Case	73

8.3	Description and Flows on Mcn Reference Point	73

8.4	Device Triggering	74

8.4.1	Definition and scope	74

8.4.2	General Procedure for Device Triggering	74

8.4.2.1	Triggering procedure for targeting ASN/MN-CSE	74

8.5	Location Request	76

8.5.1	Definition and Scope	76

8.5.2	General Procedure for Location Request	76

8.6 	Connection Request	78

8.7 	Device Management	78

9	Resource Management	79

9.1	General Principles	79

9.2	Resources	79

9.2.1	Normal Resources	80

9.2.2	Virtual Resources and Attributes	80

9.3	Resource Addressing	80

9.3.1	Generic Principles	80

9.3.1.1	Support for Multiple CSEs within a single Node	81

9.3.2 	Resource addressing via a FQDN URI	81

9.4	Resource Structure	82

9.4.1	Links between Resources	82

9.4.2	Link Types	82

9.5	Resource Type Specification Conventions	83

9.5.1	Handling of Unsupported Resources/Attributes/Sub-resources within the M2M System	86

9.6	Resource Types	87

9.6.1	Common Attributes	91

9.6.2	Resource Type accessControlPolicy	93

9.6.2.1	originatorPrivileges	95

9.6.2.2	contexts	95

9.6.2.3	operationFlags	95

9.6.3	Resource Type CSEBase	96

9.6.4	Resource Type remoteCSE	98

9.6.5	Resource Type AE	101

9.6.6	Resource Type container	104

9.6.7	Resource Type contentInstance	106

9.6.8	Resource Type subscription	108

9.6.9	Resource Type schedule	111

9.6.10	Resource Type locationPolicy	112

9.6.11	Resource Type delivery	114

9.6.12	Resource Type request	116

9.6.13	Resource Type group	118

9.6.14	Resource Type fanOutPoint	120

9.6.15	Resource Type mgmtObj	121

9.6.16	Resource Type mgmtCmd	124

9.6.17	Resource Type execInstance	125

9.6.18	Resource Type node	127

9.6.19	Resource Type m2mServiceSubscription	130

9.6.20	Resource Type nodeInfo	131

9.6.21	Resource Type pollingChannel	133

9.6.22	Resource Type statsConfig	133

9.6.23	Resource Type eventConfig	134

9.6.24	Resource Type statsCollect	136

9.6.25	Resource Announcement	138

9.6.25.1	Common Attributes for Announced Resources	139

10	Information Flows	140

10.1.1	CREATE (C)	140

10.1.1.1	 Non-registration related CREATE procedure	140

10.1.1.2	Registration related CREATE procedure	142

10.1.1.2.1	 CSE Registration procedure	142

10.1.1.2.2	 Application Entity Registration procedure	144

10.1.2	RETRIEVE (R)	144

10.1.3	UPDATE (U)	145

10.1.4	DELETE (D)	146

10.1.5	NOTIFY (N)	147

10.2	Resource Type-Specific Procedures	149

10.2.1	<AE> Resource	149

10.2.1.1	Create <AE>, aka Application Registration	149

10.2.1.2	Retrieve <AE>	150

10.2.1.3	Update <AE>	150

10.2.1.4	Delete <AE>, aka De-Register	151

10.2.2	<remoteCSE> Resource	151

10.2.2.1	Create <remoteCSE>, aka CSE Registration	151

10.2.2.2	Retrieve <remoteCSE>	152

10.2.2.3	Update <remoteCSE>, aka Re-registration	152

10.2.2.4	Delete <remoteCSE>, aka CSE De-Registration	153

10.2.3	<container> Resource	153

10.2.3.1	Create <container>	153

10.2.3.2	Retrieve <container>	154

10.2.3.3	Update <container>	155

10.2.3.4	Delete <container>	155

10.2.4	Access to Remotely Hosted Resources via <delivery>	156

10.2.4.1	Introduction to usage of <delivery> resource type	156

10.2.4.2	Create <delivery>	159

10.2.4.3	Retrieve <delivery>	161

10.2.4.4	Update <delivery>	161

10.2.4.5	Delete <delivery>	162

10.2.5	Resource Discovery Procedure	162

10.2.5.1	Introduction	162

10.2.5.2	Discovery procedure via Retrieve Operation	162

10.2.6	Group Management Procedures	163

10.2.6.1	Introduction	163

10.2.6.2	Create <group>	163

10.2.6.3	Retrieve <group>	164

10.2.6.4	Update <group>	165

10.2.6.5	Delete <group>	166

10.2.6.6	Create <fanOutPoint>	166

10.2.6.7	Retrieve <fanOutPoint>	168

10.2.6.8	Update <fanOutPoint>	169

10.2.6.9	Delete <fanOutPoint>	170

10.2.6.10	Subscribe and Un-Subscribe <fanOutPoint> of a group	171

10.2.6.11	Aggregate the notifications by group	172

10.2.7	mgmtObj Management Procedures	173

10.2.7.1	Introduction	173

10.2.7.2	Create <mgmtObj>	173

10.2.7.3	Retrieve <mgmtObj>	174

10.2.7.4	Update <mgmtObj>	175

10.2.7.5	Delete <mgmtObj>	176

10.2.7.6	Execute <mgmtObj>	177

10.2.8	External Management Operations through <mgmtCmd>	178

10.2.8.1	Introduction	178

10.2.8.2	Create <mgmtCmd>	178

10.2.8.3	Retrieve <mgmtCmd>	179

10.2.8.4	Update <mgmtCmd>	180

10.2.8.5	Delete <mgmtCmd>	181

10.2.8.6	Execute <mgmtCmd>	182

10.2.8.7	Cancel <execInstance>	183

10.2.8.8	Retrieve <execInstance>	184

10.2.8.9	Delete <execInstance>	185

10.2.9	Supported Attribute Discovery Procedure	186

10.2.9.1	Introduction	186

10.2.9.2	Supported Attributes Discovery procedure via Retrieve Operation	186

10.2.10	Location Management Procedures	187

10.2.10.1	Procedure related to <locationPolicy> resource	187

10.2.10.1.1	Create <locationPolicy>	187

10.2.10.1.2	Retrieve <locationPolicy>	188

10.2.10.1.3	Update <locationPolicy>	189

10.2.10.1.4	Delete <locationPolicy>	189

10.2.10.2	Procedure when the <container> and <contentInstance> resource contain location information	190

10.2.10.2.1	Procedure for <container> resource that stores the location information	190

10.2.10.2.2	Procedure for <contentInstance> resource that stores location information	190

10.2.11	<subscription> Resource Procedures	190

10.2.11.1	Introduction	190

10.2.11.2	Create <subscription>	190

10.2.11.3	Retrieve <subscription>	191

10.2.11.4	Update <subscription>	192

10.2.11.5	Delete <subscription>	192

10.2.12	Notification Procedures for Resource Subscription	193

10.2.13	Polling Channel Management Procedures	195

10.2.13.1	Introduction	195

10.2.13.2	Create <pollingChannel>	196

10.2.13.3	Retrieve <pollingChannel>	196

10.2.13.4	Update <pollingChannel>	197

10.2.13.5	Delete <pollingChannel>	197

10.2.13.6	Re-targeting Requests to Polling Channel	197

10.2.13.7	Long Polling on Polling Channel	198

10.2.14	<node> Resource Procedure	198

10.2.14.1	Create <node>	198

10.2.14.2	Retrieve <node>	199

10.2.14.3	Update <node>	200

10.2.14.4	Delete <node>	200

10.2.15	Service Charging and Accounting Procedures	201

10.2.15.1	Introduction	201

10.2.15.1.1	Service Event-based Statistics Collection for Applications	201

10.2.15.2 	CREATE <statsConfig>	202

10.2.15.3	RETRIEVE <statsConfig>	203

10.2.15.4 	UPDATE <statsConfig>	204

10.2.15.5 	DELETE <statsConfig>	204

10.2.15.6 	CREATE <statsCollect>	205

10.2.15.7	RETRIEVE <statsCollect>	206

10.2.15.8	UPDATE <statsCollect>	206

10.2.15.9	DELETE <statsCollect>	207

10.2.15.10	Service Statistics Collection Record	207

10.2.16	<m2mServiceSubscription> Resource	208

10.2.16.1	Create <m2mServiceSubscription>	208

10.2.16.2	Retrieve <m2mServiceSubscription>	209

10.2.16.3	Update <m2mServiceSubscription>	209

10.2.16.4	Delete <m2mServiceSubscription>	210

10.2.17	<nodeInfo> Resource	210

10.2.17.1	Create <nodeInfo>	210

10.2.17.2	Retrieve <nodeInfo>	211

10.2.17.3	Update <nodeInfo>	211

10.2.17.4	Delete <nodeInfo>	212

10.2.18	Resource Announcement Procedure	212

10.2.18.1 	Procedure for AE and CSE to initiate Creation of an Announced Resource	212

10.2.18.2 	Procedure at AE or CSE to Retrieve information from an Announced Resource	213

10.2.18.3 	Procedure for AE and CSE to initiate Deletion of an Announced Resource	214

10.2.18.4	Procedure for original resource Hosting CSE to Create an Announced Resource	215

10.2.18.5	Procedure for original resource Hosting CSE to Delete an Announced Resource	217

10.2.18.6	Procedure for AE and CSE to initiate creation or deletion of Announced Attribute	217

10.2.18.7	Procedure for original resource Hosting CSE to Announce Attribute	218

10.2.18.8	Procedure for original resource Hosting CSE to delete Announced Attribute	219

11	Trust Enabling Architecture	220

11.1	Enrolling M2M Nodes and M2M Applications for oneM2M Services	220

11.2 	M2M Initial Provisioning Procedures	220

11.2.1	M2M Node Enrolment and Service Provisioning	220

11.2.1	M2M Application Enrolment	221

11.3	M2M Operational Security Procedures	221

11.3.1	Identification of CSE and AE	222

11.3.2	Authentication of CSE and AE	222

11.3.3	M2M Security Association Establishment	222

11.3.4	M2M Authorization Procedure	223

12	Information Recording	224

12.1	M2M Infrastructure Node (IN) Information Recording	224

12.1.1	Information Recording Triggers	224

12.1.2	M2M Recorded Information Elements	224

12.1.2.1	Unit of Recording	224

12.1.2.2	Information Elements within an M2M Event Record	224

12.1.3	Identities Associations in Support of Recorded Information	227

12.2	Offline Charging	227

12.2.1	Architecture	227

12.2.2	Filtering of Recorded Information for Offline Charging	228

12.2.3	Examples of Charging Scenarios	228

12.2.3.1	Example Charging Scenario 1 – Data Storage Resource Consumption	228

12.2.3.2	Example Charging Scenario 2 – Data transfer	228

12.2.3.3	Example Charging Scenario 3 – Connectivity	228

12.2.4	 Correlation between charging information of M2M IN and charging in access network	228

12.2.5	  Definition of Charging Information	228

12.2.5.1	Triggers for Charging Information	228

12.2.5.2	Charging Messages over Mch Reference Point	229

12.2.5.3	Structure of the Accounting Message Formats	229

12.2.5.3.1	Accounting-Request Message	229

12.2.5.3.2	Accounting-Answer Message	229

Proforma copyright release text block	231

Annex A	  (Informative): Mapping of Requirements with CSFs	232

Annex B	  (Informative): oneM2M System and 3GPP MTC Release-11 Underlying Network Interworking	236

B.1	3GPP MTC Release-11 Underlying Network Introduction	236

B.2	3GPP Release-11 MTC Functionality	236

B.3	ASN-CSE initiated connectivity establishment	238

B.3.1	Use of DHCP and DNS	238

B.3.2	Pre-configuration	238

B.4	Serving IN-CSE initiated connectivity establishment	239

B.5	Idle mode Management	239

B.6	Procedure	239

B.6.1	General	239

B.6.1.1	ASN/MN-CSE Initiated Connectivity Establishment Procedure	240

B.6.1.2	IN-CSE initiated connectivity establishment procedure over Tsp	241

Annex C	  (Informative): Interworking between oneM2M System and 3GPP2 Underlying Networks	244

C.1	General Concepts	244

C.2	M2M Communication Models	244

C.3	3GPP2 Architectural Reference Model for M2M	246

C.4	Communication between oneM2M Service Layer and 3GPP2 Underlying Network	246

C.5	 Information Flows	247

C.5.1	Discovery of M2M Devices	247

C.5.2	Tsp Interface Call Flow	247

C.5.2	Point to Point Device Triggering	249

C.5.3	Broadcast Device Triggering	249

Annex D	  (Normative) <mgmtObj> Resource Instances Description	250

D.1	oneM2M Management Functions	250

D.2	Resource firmware	251

D.3	Resource software	253

D.4	Resource memory	255

D.5	Resource areaNwkInfo	256

D.6	Resource areaNwkDeviceInfo	258

D.7	Resource Type battery	259

D.8	Resource Type deviceInfo	261

D.9	Resource Type deviceCapability	263

D.10	Resource Type reboot	265

D.11	Resource Type eventLog	266

D.12	Resource cmdhPolicy	268

D.12.1	Resource cmdhDefaults	270

D.12.2	Resource cmdhDefEcValue	272

D.12.3	Resource cmdhEcDefParamValues	276

D.12.4	Resource cmdhLimits	278

D.12.5	Resource cmdhNetworkAccessRules	283

D.12.6	Resource cmdhNwAccessRule	285

D.12.7	Resource cmdhBuffer	289

Annex E	  (Informative) CSE Minimum Provisioning	292

Annex F	  (Informative) Interworking / Integration of non-oneM2M solutions and protocols	293

F.1	Introduction	293

F.2	Interworking with non-oneM2M solutions through specialized interworking applications	293

F.3	Interworking versus integration of non-oneM2M solutions	296

F.4	Entity-relation representation of non-IP based M2M Area Network	296

F.4.1	Responsibilities of Interworking Proxy Application Entity (IPE)	297

Annex G	  (Informative) List of M2M Services	298

Annex H	  (Informative) Object Identifier Based M2M Device Identifier	299

H.1.	Overview of Object Identifier	299

H.2.	OID Based M2M Device Identifier	299

H.2.1	M2M Device Indication ID – (higher arc)	300

H.2.2.	Manufacturer ID – (x)	300

H.2.3.	Model ID – (y)	300

H.2.4.	Serial Number ID – (z)	300

H.2.5.	Expanded ID – (a)	300

H.3.	Example of M2M device ID based on OID	300

Annex I	  (Informative) Functionalities for the Subsequent Architecture TS Release	302

I.1	Service Session Management Text for Removal	302

6.2.12	Service Session Management	302

6.2.12.1	General Concepts	302

6.2.12.2	Detailed Descriptions	302

6.3	Functional Entities Comprising Enablers	304

Annex Z	  (Informative) Bibliography	306

History	307

# 1	Scope

The present document describes the end-to-end oneM2M functional architecture, including the description of the functional entities and associated reference points.

oneM2M functional architecture focuses on the Service Layer aspects and takes Underlying Network-independent view of the end-to-end services. The Underlying Network is used for the transport of data and potentially for other services.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[TS0003]	oneM2M Security Solutions Technical Specification

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M T2-0002: "Requirements TS ...".

[i.2]	oneM2M T2-0004: "Core Protocols TS ...".

[i.3]	oneM2M T2-0003: "Security Solutions TS ...".

[i.4]	BBF TR-069 CPE WAN Management Protocol Issue: 1 Amendment 4, July 2011

[i.5]	OMA-TS-DM_Protocol-V1_3 “OMA Device Management Protocol”

[i.6]	OMA-AD-LightweightM2M-V1_0 “OMA Lightweight Machine to Machine Architecture”

[i.7]	OMA-TS-MLP-V3_4-20130226-C: “Mobile Location Protocol”, Version 3.4

[i.8]	OMA-TS-REST-NetAPI_TerminalLocation-V1_0-20130924-A: “RESTful Network API for Terminal Location”, Version 1.0

[i.9]	IETF RFC 1035: "Domain names - Implementation and specification".

[i.10]	IETF RFC 3588: "Diameter Base Protocol".

[i.11]	IETF RFC 3596: "DNS Extensions to Support IP Version 6".

[i.12]	IETF RFC 3986: "Uniform Resource Identifier (URI): General Syntax".

[i.13]	IETF RFC 4006: "Diameter Credit-Control Application".

[i.14]	IETF RFC 6874: "Representing IPV6 Zone Identifiers in Address Literals and Uniform Resources Identifiers".

[i.15]	IETF RFC 6895: "Domain Name System (DNS) IANA Considerations".

[i.16]	GSMA-IR.67: "DNS/ENU Guidelines for Service Providers & GRX/IPX Providers".

[i.17]	3GPP TS 23.682: "Architecture Enhancements to facilitate communications with packet data networks and applications". Release-11

[i.18]	3GPP 32.240  “Telecommunication management; Charging management; Charging Architecture and Principles".

[i.19]	3GPP 32.299  “Telecommunication management; Charging management; Diameter charging application".

[i.20]	3GPP2 X.P0068, "Network Enhancements for Machine to Machine (M2M)"

[i.21]	JNI (Java Native Interface <XXXX>

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Application Layer: The Application Layer comprises oneM2M Applications and related business and operational logic.

Attribute: Stores information pertaining to the resource.

Child Resource: It is a sub-resource of another resource that is its parent resource. The parent resource contains references to the child resources(s).

Common Services Layer: The Common Services Layer consists of oneM2M service functions that enable oneM2M Applications (e.g., management, discovery and policy enforcement).

Common Services Function: A Common Services Function (CSF) is an informative architectural construct which conceptually groups together a number of sub-functions. Those sub-functions are implemented as normative resources and procedures. A set of CSFs is contained in the CSE.

Device Entity (6.2.9.2)

- Device Management Resource

Execution Environment: An Execution Environment is a logical entity that represents an environment capable of running software modules.

Hosting CSE: A Hosting CSE is the CSE where the addressed resource is hosted.

- M2M Service Provider Domain: An M2M Service Provider Domain is the part of the M2M System that is associated with a specific M2M Service Provider.

Managed Entity: A Managed Entity may be either an M2M Device, M2M Gateway, or a device in the M2M Area Network or the M2M Application Layer or M2M Service Layer software components.

- Management Adapter

- Management Client Management Resource

- Management Server

Network Services Layer: The Network Services Layer provides transport, connectivity and service functions.

Node: A functional entity containing one of the following:

- one or more M2M Applications;

- one CSE and zero or more M2M Applications.

Originator: The Originator is the actor that initiates a Request. An Originator can either be an Application or a CSE.

Receiver: The Receiver is the actor that receives the Request. A Receiver can either be a CSE or an Application.

Registree: A Registree is an AE or CSE that registers with another CSE.

Registrar CSE: A Registrar CSE is the CSE where an Application or another CSE has registered.

- Request (7.1.7)

- Response (7.1.7

Resource: A resource is a uniquely addressable entity in oneM2M architecture. A resource is transferred and manipulated using CRUDN operations. A resource can contain child resource(s) and attribute(s).

- Role

Service Charging and Accounting: A set of functionalities within the M2M Service Layer that enable configuration of information collection and charging policies, collection of Charging Records based on the policies, and correlation of Charging Records to users of M2M common services.

Service Charging Record: Formatted collection of information about a chargeable operation.

Service Layer Offline Charging: Charging mechanism where charging information does not affect, in real-time, the service rendered.

Service Layer Online Charging: Charging mechanism where charging information can affect, in real-time, the service rendered, including real time credit control

Software Package: A Software Package is an entity that can be deployed on the Execution Environment.  It can consist of entities such as software modules, configuration files, or other entities.

Software Module: A Software Module is  ....

Structured Data: Structured Data is data that either has a structure according to a specified Information Model or are otherwise organized in a defined manner.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations & Acronyms

For the purposes of the present document, the following abbreviations and acronyms apply:

3GPP	3rd Generation Partnership Project

A/AAAA	Authentication, Authorization, Accounting and Auditing

ADN	Application Dedicated Node

ADN-AE	AE which resides in the Application Dedicated Node

AE	Application Entity

AID CSF	Addressing and Identification CSF

API	Application Programming Interface

ASM	Application and Service Layer Management

ASM CSF	Application and Service Layer Management CSF

ASN	Application Service Node

ASE-AE	Application Entity that is registered with the CSE in the Application Service Node

ASN-CSE	CSE which resides in the Application Service Node

CDR	Charging Data Record

CF	Configuration Function

CMDH 	Communication Management and Delivery Handling

CSE	Common Services Entity

CSE-PoA	CSE Point of Access

CSF	Common Services Function

DCF	Device Configuration Function

DDMF	Device Diagnostics and Monitoring Function

DFMF	Device Firmware Management Function

DHCP	Dynamic Host Control Protocol

DIS CSF	Discovery CSF

DTMF	Device Topology Management Function

DMF	Diagnostic and Monitoring Function

DNS	Domain Name Server

DMG CSF	Device Management CSF

DMR	Data Management and Repository

EF	Enabler Function

GMG CSF	Group Management CSF

GPS	Global Positioning System

GSMA	GSM Association (Global System for Mobile Communication Association)

IMEI	International Mobile Equipment Identity

IN	Infrastructure Node

IN-AE	Application Entity that is registered with the CSE in the Infrastructure Node

IN-CSE	CSE which resides in the Infrastructure Node

IP	Internet Protocol

JNI	Java Native Interface

LOC CSF	Location CSF

M2M	Machine to Machine

MN	Middle Node

MN-CSE	CSE which resides in the Middle Node

MSISDN	Mobile Subscriber Integrated Services Digital Network-Number

MTC	Machine Type Communications

OMA	Open Mobile Alliance

NSE	Network Service Entity

NSSE CSF	Network Service Exposure, Service Execution and Triggering CSF

OID	Object Identifier

PPP	Point to Point Protocol

REG CSF	Registration CSF

RPC	Remote Procedure Calls

SCA CSF	Service Charging and Accounting CSF

SCS	Services Capability Server

SDO	Standards Developing Organization

SEC CSF	Security CSF

SMF	Software Monitoring Function

SMS	Short Messaging System

SP	Service Provider

SSM 	Service Session Management

SSM CSF	Service Session Management CSF

SUB CSF	Subscription and Notification CSF

Tsp	(7.3.2.3.2)

Tsms	(7.3.2.3.2)

- UE	User Equipment

URI	Universal Resource Identifier

URN	Universal Resource Name

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in this document are to be interpreted as described in the oneM2M Drafting Rules [Error: Reference source not found].

To improve readability:

- The information elements of oneM2M Request/Response messages will be referred to as parameters. Parameter abbreviations will be written in bold italic.

- The information elements of resources will be referred to as attributes and child resources. Attributes will be written in italics.

# 5	Architecture Model

## 5.1	General Concepts

Figure 5.1-1 depicts the oneM2M Layered Model for supporting end-to-end (E2E) M2M Services. This layered model comprises three layers: Application Layer, Common Services Layer and the Underlying Network Services Layer.

- Figure 5.1-1: oneM2M Layered Model

## 5.2	Architecture Reference Model

### 5.2.1	Functional Architecture

Figure 5.2.1-1 illustrates the oneM2M functional architecture.

- Figure 5.2.1-1: oneM2M Functional Architecture

NOTE: Other reference points for charging and management aspects are specified.

- See clause 12.2.1. for Mch reference point

- See clause xxxx for XXX reference point

The oneM2M functional architecture in figure 5.2.1-1 comprises of the following functions:

- Application Entity (AE): Application Entity represents an instantiation of Application logic for end-to-end M2M solutions. Each Application Entity is identified with a unique AE-ID (see clause 7.1.2). Examples of the Application Entities can be an instance of a fleet tracking application, a remote blood sugar monitoring application, a power metering application, or a controlling application.

- Common Services Entity (CSE): A Common Services Entity represents an instantiation of a set of "common service functions" of the M2M environments. Such service functions are exposed to other entities through reference points Mca and Mcc. Reference point Mcn is used for accessing Underlying Network Service Entities. Each Common Service Entity is identified with a unique CSE-ID (see clause 7.1.4).

Examples of service functions offered by CSE include: Data Management, Device Management, M2M Subscription Management, and Location Services. Such "sub-functions" offered by a CSE may be logically and informatively conceptualized as Common Services Functions (CSFs). The normative Resources which implement the service functions in a CSE can be mandatory of optional.,

- Underlying Network Services Entity (NSE): An Underlying Network Services Entity provides services to the CSEs. Examples of such services include device management, location services and device triggering. No particular organization of the NSEs is assumed.

NOTE:	Underlying Networks provide data transport services between entities in the oneM2M System. Such data transport services are not included in the NSE.

### 5.2.2	Reference Points

The following reference points are supported by the Common Services Entity (CSE). The "Mc(-) nomenclature is based on the mnemonic "M2M communications".

NOTE:	Information exchange between two M2M nodes assumes the usage of the transport and connectivity services of the Underlying Network Services Entity, which are considered to be the basic services.

#### 5.2.2.1	Mca Reference Point

Communication flows between an Application Entity (AE) and a Common Services Entity (CSE) cross the Mca reference point. These flows enable the AE to use the services supported by the CSE, and for the CSE to communicate with the AE.

NOTE:	The AE and the CSE it invokes could or could not be co-located within the same physical entity.

#### 5.2.2.2	Mcc Reference Point

Communication flows between two Common Services Entities (CSEs) cross the Mcc reference point. These flows enable a CSE to use the services supported by another CSE.

#### 5.2.2.3	Mcn Reference Point

Communication flows between a Common Services Entity (CSE) and the Underlying Network Services Entity (NSE) cross the Mcn reference point. These flows enable a CSE to use the supported services (other than transport and connectivity services) provided by the Underlying NSE.

#### 5.2.2.4	Mcc' Reference Point

Communication flows between two Common Services Entities (CSEs) in infrastructure nodes that are oneM2M compliant and that resides in different M2M SP domains cross the Mcc' reference point. These flows enable a CSE of an infrastructure node residing in the Infrastructure Domain of an M2M Service Provider to communicate with a CSE of another infrastructure node residing in the Infrastructure Domain of another M2M Service Provider to use its supported services, and vice versa.

Mcc' extends the reachability of services offered over the Mcc reference point, or a subset thereof.

The trigger for these communication flows may be initiated elsewhere in the oneM2M network.

#### 5.2.2.5 	Mch Reference Point

Communication flows which transfer CDRs generated by the IN to an external charging server cross the Mch reference point. The Mch reference point may be mapped to reference points of other specifications. E.g., for a 3GPP Underlying Network, the Mch reference point maps to the Rf reference point enabling a 3GPP charging server to be used for oneM2M CDRs.

# 6	Configurations supported by oneM2M Architecture

## 6.1	Relationships among oneM2M Entities

Figure 6.1-1 illustrates the relationships among oneM2M entities supported by the oneM2M architecture as shown in Figure 5.2.1-1. The illustration does not constrain the multiplicity of the entities nor require that all relationships shown are present.

- Figure 6.1-1: Configurations supported by oneM2M Architecture

Nodes: Two types of oneM2M Nodes are supported:

- A CSE-Capable Node is a functional entity that contains at least one oneM2M Common Services Entity and zero or more oneM2M Application Entities. The ASN and MN are examples of CSE-Capable Nodes.

- A Non-CSE-Capable Node is a functional entity that contains one or more oneM2M Application Entities and no oneM2M Common Services Entity. The ADN is an example of a Non-CSE-Capable Node.

NOTE 1:	CSEs resident in different Nodes could not be identical and are dependent on the services supported by the CSE in that Node.

- Description of Nodes:

oneM2M architecture enables the following types of Nodes. As functional objects, such Nodes may or may not be mapped to physical objects.

- Application Service Node (ASN):

An Application Service Node is a Node that contains one Common Services Entity and contains at least one Application Entity. There may be zero or more ASNs in the Field Domain of an M2M System.

An Application Service Node may communicate over a Mcc reference point with either:

- exactly one Middle Node;

- or exactly one Infrastructure Node.

An Application Service Node communicates over Mcn with NSEs.

Example of physical mapping: an Application Service Node could reside in an M2M Device.

- Application Dedicated Node (ADN):

An Application Dedicated Node is a Node that contains at least one Application Entity and does not contain a Common Services Entity. There may be zero or more ADNs in the Field Domain of an M2M System.

An Application Dedicated Node communicates with a Middle Node or an Infrastructure Node over a Mca reference point.

Example of physical mapping: an Application Dedicated Node could reside in a constrained M2M Device.

- Middle Node (MN):

A Middle Node is a Node that contains one Common Services Entity and contains zero or more Application Entities. There may be zero or more Middle Nodes in the Field Domain of an M2M System.

A Middle Node communicates with:

- either an IN or another MN over an Mcc reference point; plus at least

- an IN/MN/ASN over an Mcc reference point; or

- an ADN over a Mca reference point.

A Middle Node communicates over Mcn with NSEs.

Example of physical mapping: a Middle Node could reside in an M2M Gateway.

- Infrastructure Node (IN):

An Infrastructure Node is a Node that contains one Common Services Entity and contains zero or more Application Entities. There is one logical Infrastructure Node in the Infrastructure Domain per oneM2M Service Provider of an M2M System. It may contain CSE functions not applicable to other node types.

An Infrastructure Node communicates over respective Mcc reference points with:

- one or more Middle Node(s); and/or

- one or more Application Service Node(s).

An Infrastructure Node communicates with one or more Application Dedicated Nodes over respective Mca reference points.

An Infrastructure Node communicates over Mcn with NSEs, and over Mcc' with Infrastructure Nodes of other M2M Systems.

Example of physical mapping: an Infrastructure Node could reside in an M2M Service Infrastructure.

## 6.2	Common Services Functions

This clause describes the service functions provided by the Common Services Layer in the M2M System. Such service functions reside within a CSE and are referred to as Common Services Functions (CSFs). The CSFs provide services to the AEs via the Mca reference point and to other CSEs via the Mcc reference point. CSEs interact with the NSE via the Mcn reference point. An instantiation of a CSE in a Node comprises a subset of the CSFs from the CSFs described in the present document.

The CSE descriptions are provided for the understanding of the oneM2M Architecture functionalities and are informative. The CSFs contained inside the CSE can interact with each other but these interactions are not specified in the present document.

- Figure 6.2-1: Common Services Functions

### 6.2.1	Application and Service Layer Management

#### 6.2.1.1	General Concepts

The Application and Service Layer Management (ASM) CSF provide management of the AEs and CSEs on the ADNs, ASNs, MNs and INs. This includes functions to configure, troubleshoot and upgrade the functions of the CSE, as well as to upgrade the AEs.

#### 6.2.1.2	Detailed Descriptions

The ASM CSF provides management capabilities for CSEs and the  AEs.

- Figure 6.2.1.2-1: Management Layers and Function

The ASM CSF utilizes the functions provided by the Device Management (DMG) CSF for interaction with the Management Server.

The management functions include:

- Configuration Function (CF): This function enables the configuration of the capabilities and features of the CSE (e.g., CMDH policies).

- Software Management Function (SMF): This function provides lifecycle management for software components and associated artifacts (e.g., configuration files) for different entities such as CSE and AE.

##### 6.2.1.2.1	Software Management Function

The Software Management Function (SMF) provides the capability to manage software components (e.g., Software Package, Software Module) for AEs and CSEs.

The ASM CSF provides the capability to manage the lifecycle of the Software Packages for a CSE or an AE. AE Software Packages may be deployed on any Node that supports the AE; including those on the Middle Nodes, Application Dedicated Nodes and Application Service Nodes.

The lifecycle of a Software Package consists of states (e.g., Installing, Installed, Updating, Uninstalling and Uninstalled) that transition when an action (e.g., Download, Install, Update and Remove) is applied to the Software Package.

When a Software Package is installed into an execution environment the software component that is capable of executing in the Execution Environment is called a Software Module. The lifecycle of a Software Module consists of states (e.g., Idle, Starting, Active, Stopping) that transition when an action (e.g., Start, Stop) is applied to the Software Module.

### 6.2.2	Communication Management and Delivery Handling

#### 6.2.2.1	General Concepts

The Communication Management and Delivery Handling (CMDH) CSF provides communications with other CSEs, AEs and NSEs.

The CMDH CSF decides at what time to use which communication connection for delivering communications (e.g., CSE-to-CSE communications) and, when needed and allowed, to buffer communication requests so that they can be forwarded at a later time. This processing in the CMDH CSF is carried out per the provisioned CMDH policies and delivery handling parameters that can be specific to each request for communication.

For communication using the Underlying Network data transport services, the Underlying Network can support the equivalent delivery handling functionality. In such case the CMDH CSF uses the Underlying Network, and it may act as a front end to access the Underlying Network equivalent delivery handling functionality.

#### 6.2.2.2	Detailed Descriptions

The service that AEs or CSEs can request from the CMDH CSF is to transport some data to a specific target (CSE or AE), according to given delivery parameters while staying within the constraints of provisioned communication management and delivery handling policies.

The content of the data provided by the Originator is not be visible to the CMDH CSF and does not influence its behaviour. Consequently, the CMDH CSF is not aware of the specific operation requested at the target entity, including the parameters passed to the operation at the destination CSF. This means that all attributes intended to be delivered to the destination entity (e.g., which CSF is the destination on the target entity, what that CSF does with the data etc.) are hidden to the CMDH CSF.

The target entity may be reached either directly or via the CSE(s) of a Middle Node(s).

As part of the delivery request, the CMDH CSF can be provided with acceptable delivery parameters for the Originator (e.g., acceptable expiration time for delivery).

The functions supported by the CMDH CSF are as follows:

- Ability for the M2M Service Provider to derive CMDH policies describing details for the usage of the specific Underlying Network(s). These policies may be based on the M2M Service Subscription associated with Application and Service Entities (AEs and CSEs) in the Field Domain and on the agreements on usage of Underlying Network communication resources. CMDH Policies can be provisioned into the respective CSEs in the Field Domain.

- For the delivery of communication, ability to select appropriate communication path to use at any given time in line with  provisioned CMDH policies and  with CMDH-related parameters  set by the Originator of requests,  and when needed and allowed, how long to buffer communication requests so that they can be forwarded at a later time. This policy-driven use of communication resources allows an M2M Service Provider to control which Originators of requests are allowed to consume – possibly costly – communication resources at certain times.

- For the delivery of communication, ability to be aware of the availability of the Underlying Networks.

- Ability to manage the proper use of buffers for store-and-forward processing through use of CMDH policies.

### 6.2.3	Data Management and Repository

#### 6.2.3.1	General Concepts

One of the purposes of CSEs is to enable AEs to exchange data with each other.

The Data Management and Repository (DMR) CSF is responsible for providing data storage and mediation functions. It includes the capability of collecting data for the purpose of aggregating large amounts of data, converting this data into a specified format, and storing it for analytics and semantic processing. The data can be either raw data transparently retrieved from an M2M Device, or processed data which is calculated and/or aggregated by M2M entities.

NOTE: Collection of large amounts of data is known as the Big Data Repository and is not part of this document.

#### 6.2.3.2	Detailed Descriptions

The DMR CSF provides the capability to store data such as Application data, subscriber information, location information, device information, semantic information, communication status, access permission, etc. The data stored by the DMR CSF enables management of the data and provides the foundation of Big Data.

The following are examples of DMR CSF functionalities:

- Ability to store data in an organized fashion so it is discernible. This includes storage of contextual information such as data types, semantic information, time stamps, location, etc., to complement the data stored in order to access and search the data based on aset of parameters. This is part of data semantics capability which is not part of the present document

- Provides the means to aggregate data received from different entities.

- Ability to grant access to data from remote CSEs and AEs based on defined access control policies, and trigger data processing based on data access.

- Ability to provide the means to perform data analytics on large amount of data to allow service providers to provide value-added services.

### 6.2.4	Device Management

#### 6.2.4.1	General Concepts

The Device Management (DMG) CSF provides management of device capabilities on MNs (e.g., M2M Gateways), ASNs and ADNs (e.g., M2M Devices), as well as devices that reside within an M2M Area Network. Application Entities (AE) can manage the device capabilities on those Nodes by using the services provided by the DMG CSF alleviating the need for the AE to have knowledge of the management technology specific protocols or data models. While the AE does not require an understanding of the management technology specific protocols or data models, this information is provided to the AE so that an AE can utilize this information for administrative purposes (e.g., diagnostics, troubleshooting).

##### 6.2.4.1.1	Device Management Architecture

In order to manage the CSE and device capabilities of the MNs, ASNs and ADNs, the DMG can utilize existing device management technologies (e.g., TR-069 [i.4], OMA-DM [i.5], and LWM2M [i.6]) in addition to management of Management Resources across the Mcc reference point. When the device management technology is used to manage the MN, ASN or ADN, the DMG of the IN translates or adapts the management related requests from other CSEs or from AEs to the device management commands of the corresponding device management technology.

In order to perform the translation and adaptation functions, the DMG has a functional component termed the Management Adapter (Figure 6.2.4.1.1-1). The Management Adapter in the DMG of the IN (IN-DMG-MA) performs the adaptation between the DMG and Management Servers; while the Management Adapter in the DMG of the MN  (MN-DMG-MA) and ASN (ASN-DMG-MA) performs translation and adaptation between the DMG and the Management Client. Only one Management Adapter is shown in the DMG although it can interact with Management Server using different management technologies.

The interface between Management Server and Management Client (figure 6.2.4.1.1-1) is the "mc" interface which is subject to the device management technology that is used (e.g., TR069 [i.4] or LWM2M [i.6]). The "mc" interface is technology dependent and is outside the scope of this specification.

The DMG in the CSE of the MN has the same functionality as the DMG in the CSE of the ASN In addition, the DMG in the MN can be used to manage devices in the M2M Area Network. In this case, the DMG is deployed with proxy functionality that interacts with the Proxy Management Client using the "mp" interface. The "mp" interface is technology dependent and is outside the scope of this specification.

The Management Server and Management Client can be implemented as an entity external to the Node or they can be implemented as an entity embedded within the Node (Figure 6.2.4.1.1-1). The Management Server and the Management Client are located on the boundary of the Node to indicate this situation as well as to depict that an IN can utilize multiple Management Servers from various M2M and Network Service Providers.

- Figure 6.2.4.1.1-1: Device Management Architecture

##### 6.2.4.1.2	Management Server Interaction

# 6.2.4.1.2.1	Overview

The DMG CSF in the IN has the capability to utilize Management Servers from existing device management technologies (e.g., TR-069 [i.4], OMA DM [i.5], LWM2M [i.6]) to implement the Device Management functions. The IN-DMG-MA communicates with the Management Server using the "ms" interface that is provided by the Management Server. Note that "ms" interface is outside the scope of this specification. The IN-DMG-MA takes the following roles:

- Protocol Translation between DMG and the Management Server:

- After the DMG receives the requests from the request Originator, the IN-DMG-MA translates the requests from the request Originator to requests with associated identifiers that can be understood by the Management Server. Likewise the IN-DMG-MA translates events from the Management Server and delivers the events to M2M Entities (e.g., AE, CSE) that are subscribed to the event. When the Management Server is embedded within the IN-DMG, the Management Adapter translates the request and accepts events in the protocol understood by the Management Client.

- Interaction with the Management Server:

- By using "ms" interface, the IN-DMG-MA can communicate with the Management Server. This is for delivering the requests from the request Originator to the Management Server, or receiving information from the Management Server that will be notified to subscribing M2M Entities (e.g., AE, CSE). The communication between the IN-DMG-MA and the Management Server requires an establishment of a session. The establishment of a session between the IN-DMG-MA and Management Server provides security dimensions for Access Control, Authentication, Non-repudiation, Data confidentiality, Communication security, Data integrity and Privacy. The IN-DMG-MA can utilize a policy that defines when a session between the IN-DMG-MA and Management Server is established and torn down.

- Management Server selection:

- When the IN-DMG-MA communicates with multiple Management Servers that have different level of access control privileges to resources from the Management Server the IN-DMG-MA selects the proper Management Server that has the access control privileges to perform the management requests. The access control policy information for resources from Management Servers may be discovered using the "ms" interface.

- Discovery of external management objects:

- When the IN-DMG-MA maintains information (i.e., metadata, values) of the external management objects managed by a Management Server using the “ms” interface, the IN-DMG-MA will be capable of discovering and keep up to date the external management object's information that are managed by the IN-DMG and a Management Server.

A Management Server can be located in the Underlying Network using the Mcn reference point as depicted in figure 6.2.4.1.2.1-1 or the Management Server can be located in the M2M Service Layer as depicted in figure 6.2.4.1.2.1-2.

- Figure 6.2.4.1.2.1-1: Management Server in Underlying Network

- Figure 6.2.4.1.2.1-2: Management Server in M2M Service Layer

The "ms" interface is functionally the same interface regardless if the Management Server resides in the Underlying Network or the Service Layer. However, the access control privileges that the Management Server has for resources from the management technology can be different depending whether the Management Server resides in the Underlying Network or in the Services Layer. For example in Figure 6.2.4.1.2.1-1, the Management Server in the Underlying Network controls access of the exposed resources from the management technology while, in the Figure 6.2.4.1.2.1-2, the Management Server in the M2M Service Layer controls access to the resources.

# 6.2.4.1.2.2	Management Server - Access Permissions

When an operation on a M2M Service Layer Resource is performed and if the access to the Resource is granted and the operation for the Resource utilizes a Management Server external to the service layer, the IN-DMG CSF selects one or more among the authenticated Management Servers necessary to access the requested resources. The procedure for the selection of Management Servers is implementation specific and outside the scope of the present document.

The DMG CSF management functions that cause impacts to the Underlying Network utilize access permissions that are delegated from the provider of the network service layer.

6.2.4.1.2.3	Management Server – External management object discovery

An IN-DMG-MA discovers information of the external management objects managed by a Management Server using the “ms” interface. The discovery of this information includes the:

- M2M devices, devices in the M2M Area Network and M2M Applications to which the Management Server has access.

- The metadata associated with the external management objects associated the M2M devices, devices in the M2M Area Network and M2M Applications. This metadata includes items such as the supported data/object model.

The IN-DMG-MA is capable of being kept up-to-date of the changes in the M2M Devices, devices in the M2M Area Network and M2M Applications or the metadata of the external management objects associated with those entities. In addition, the IN-DMG-MA can maintain the value associated external management objects, associated the M2M devices, devices in the M2M Network and M2M Applications.

##### 6.2.4.1.3	Management Client Interaction

# 6.2.4.1.3.1	Overview

The DMG CSF in the MN or ASN can use the Management Client from existing management technologies (e.g., TR-069 [i.12], OMA DM [i.14], LWM2M [i.15]) to implement the Device Management functions. The MN-DMG-MA or ASN-DMG-MA communicates with the Management Client using the "la" interface (e.g., DM-7, 8, 9 ClientAPI in OMA DM [Error: Reference source not found]) that is provided by the Management Client. Note that the "la" interface is outside the scope of this specification. The MN-DMG-MA or ASN-DMG-MA takes the following roles:

- Interaction with the Management Client:

- By using "la" interface, the Management Adapter can communicate with the Management Client to discover the external management objects supported by the Management Client.

- Mapping between the DMG and Management Client:

- After the Management Adapter discovers the external management objects supported by the Management Client; the Management Adapter performs the mapping between the external management objects to resources. The DMG in the MN or ASN can create those resources in the IN-CSE, and the resources can be used by the IN-AE to manage the device capabilities pertaining to the MN or ASN.

- Figure 6.2.4.1.3.1-1: Management Client Interaction using "Ia" interface

##### 6.2.4.1.4	Device Management Resource Lifecycle

# 6.2.4.1.4.1	Resource Attributes from Device Management Resources

The lifecycle of a Device Management Resource is implemented using the resource management information defined in clause 9.1 through clause 9.5 and the corresponding procedures to Create, Retrieve, Update and Delete the resources are defined in clause 10. This clause describes additional resource management and procedures for Device Management Resources.

# 6.2.4.1.4.2	Overview

Clauses 9.1 through 9.5 define resource management information that is applicable to any type of resource, including Device Management Resources. In addition a Device Management Resource also maintains information and relationships that are specific to Device Management Resources. This information is used to:

- Manage external management objects via a Management Server which requires the information necessary to identify and access the Management Server.

- Invoke the security mechanism of the Management Server in order to authorize access to the external management objects.

# 6.2.4.1.4.3	Procedures for Creation, Update and Deletion of Device Management Resources

Clause 10 defines the procedures to Create, Update and Delete resources. These procedures are also applicable to Device Management Resources in addition to the procedures Device Management Resources are Created, Updated or Deleted:

- By administrative means using the Mca reference point.

- Directly by a CSE based on a discovery or another event within the CSE.

- Indirectly by the Management Server or Management Client when an event (such as firmware update, or fault notification) occurs within the Management Server or Client.

Regardless of the Create, Update or Delete operation, the Originator of the operation will be authorized to perform the operation. In addition, at most one Management Server is able to Create, Delete or Update addressable elements of a Management Resource.

#### 6.2.4.2	Detailed Descriptions

The DMG CSF provides capabilities for the purpose of managing M2M Devices/Gateways as well as devices in M2M Area Networks.

- Figure 6.2.4.2-1: Device Management Entities and Functions

Such capabilities include:

- Device Configuration Function (DCF): This function includes the configuration of the capabilities of the M2M Device, M2M Gateway or device in the M2M Area Network.

- Device Diagnostics and Monitoring Function (DDMF): This function includes the troubleshooting through the use of diagnostic tests and retrieval of operational status and statistics associated with the M2M Device, M2M Gateway or device in the M2M Area Network.

- Device Firmware Management Function (DFMF): This function provides the software lifecycle management for firmware components and associated artifacts for the M2M Device, M2M Gateway or device in the M2M Area Network.

- Device Topology Management Function (DTMF): This function provides the management of the topology of the M2M Area Network. A M2M Area Network is comprised of ADNs and other devices in the M2M Area Network.

##### 6.2.4.2.1	Device Configuration Function

The Device Configuration Function (DCF) provides the configuration of device capabilities that are necessary to support M2M Services and AEs in M2M Devices, M2M Gateways or devices in a M2M Area Network.

These device configuration capabilities include:

- Discovery of a device's management objects and attributes.

- Ability to enable or disable a device capability.

- Provisioning configuration parameters of a device.

##### 6.2.4.2.2	Device Diagnostics and Monitoring Function

The Device Diagnostics and Monitoring Function (DDMF) permits the troubleshooting of device capabilities that are necessary to support M2M Services and AEs in M2M Devices, M2M Gateways or devices in a M2M Area Network.

These device diagnostic and monitoring capabilities include:

- Configuration of diagnostics and monitoring parameters on the device.

- Retrieval of device information that identifies a device and its model and manufacturer.

- Retrieval of device information for the software and firmware installed on the device.

- Retrieval of information related to a battery within the device.

- Retrieval of information associated with the memory in use by a device.

- Retrieval of the event logs from a device.

- Device reboot diagnostic operation.

- Device factory reset diagnostic operation.

##### 6.2.4.2.3	Device Firmware Management Function

The Device Firmware Management Function (DFMF) provides lifecycle management for firmware associated with a device.

Device firmware is comprised of firmware modules and artefacts (e.g., configuration files) that are maintained on a device. A device can maintain more than one firmware image and the capability to manage individual firmware images. The firmware lifecycle includes actions to download, update or remove a firmware image. In addition firmware could be downloaded and updated within the same action.

##### 6.2.4.2.4	Device Topology Management Function

The Device Topology Management Function (DTMF) is a function that is specific to M2M Gateways where a M2M Gateway maintains zero or more M2M Area Networks.

These device topology management capabilities include:

- Configuration of the topology of the M2M Area Network.

- Retrieval of information related to the devices attached to the M2M Area Network.

- Retrieval of information that describes the transport protocol associated with the M2M Area Network.

- Retrieval of information that describes the characteristics associated with online/offline status of devices in the M2M Area Network.

### 6.2.5	Discovery

#### 6.2.5.1	General Concepts

The Discovery (DIS) CSF searches information about applications and services as contained in attributes and resources. The result of a discovery request from an Originator depends upon the filter criteria and is subject to access control policy allowed by M2M Service Subscription. An Originator could be an AE or another CSE. The scope of the search could be within one CSE, or in more than one CSE. The discovery results are returned back to the Originator.

#### 6.2.5.2	Detailed Descriptions

The DIS CSF uses the Originator provided filter criteria (e.g., a combination of keywords, identifiers, location and semantic information) that can limit the scope of information returned to the Originator.

The discovery request indicates the address (e.g., URI) of the resource where the discovery is to be performed. Upon receiving such request, the DIS CSF discovers, identifies, and returns the matching information regarding discovered resources according to the filter criteria.

A successful response includes the discovered information or address(es) (e.g., URI(s)) pertaining to the discovered resources. In the latter case the Originator can retrieve the resources using such discovered address. Based on the policies or Originator request, the CSE which received the discovery request can forward the request to other registered ASN-CSEs, MN-CSEs or IN-CSEs.

### 6.2.6	Group Management

#### 6.2.6.1	General Concepts

The Group Management (GMG) CSF is responsible for handling group related requests. The request is sent to manage  a group and its membership as well as for the bulk operations supported by the group. When adding or removing members to/from a group, it is necessary to validate whether the group member complies with the purpose of the group. Bulk operations includes read, write, subscribe, notify, device management, etc. Whenever a request or a subscription is made via the group, the group is responsible for aggregating its responses and notifications. The members of a group can have the same role with regards to access control policy control towards a resource. In this case, access control is facilitated by grouping. When the Underlying Network provides broadcasting and multicasting capability, the GMG CSF is able to utilize such capability.

#### 6.2.6.2	Detailed Descriptions

The GMG CSF enables the M2M System to perform bulk operations on multiple devices, applications or resources that are part of a group. In addition, the GMG CSF supports bulk operations to multiple resources of interest and aggregates the results. It facilitates access control based on grouping. When needed and available, the GMG CSF can leverage the existing capabilities of the Underlying Network including broadcasting/multicasting.

When facilitating access control using a group, only members with the same access control policy towards a resource are included in the same group. Also, only M2M AEs or CSEs which have a common role with regards to access control policy are included in the same group. This is used as a representation of the role when facilitating role based access control.

The service functions supported by the GMG CSF are as follows:

- Handles the requests to create, query, update, and delete a group. An AE or a CSE may request the creation/retrieve/update/deletion of a group as well as the addition and deletion of members of the group.

- Creates one or more groups in CSEs in any of the Nodes in M2M System for a particular purpose (e.g., facilitation of access control, device management, fan-out common operations to a group of devices, etc.).

- Handles the requests to retrieve the information (e.g., URI, metadata, etc.) of a group and its associated members.

- Manages group membership and handles requests to add or remove members to and from a group's member list. A member may belong to one or more groups. A group may be a member of another group. When new members are added to a Group, the GMG CSF validates if the member complies with the purpose of the group.

- Leverages the capabilities of other CSFs in order to fulfill the functionalities supported  by the GMG CSF service functions. Examples include: Security CSF for authentication and authorization.

- Forwards requests to all members in the group. In case the group contains another group as a member, the forwarding process is done recursively, i.e., the nested group forwards the request to its members. After forwarding the request to all members in the group, the GMG CSF generates an aggregated response by aggregating the corresponding responses from the Group members.

- Supports subscriptions to individual groups. Subscriptions to a group is made only if the subscriber is interested in all members of the group. If subscription to a group is made, the GMG CSF aggregates the notifications from the group members, and notifies the subscriber with the aggregated notification. Responses and event notifications relevant to a subscription may be selectively filtered by filtering criteria.

### 6.2.7	Location

#### 6.2.7.1	General Concepts

The Location (LOC) CSF allows AEs to obtain geographical location information of Nodes (e.g., ASN, MN) for location-based services. Such location information requests can be from an AE residing on either a local Node or a remote Node.

NOTE:	Geographical location information can include more than simply the longitude and the latitude information.

#### 6.2.7.2	Detailed Descriptions

The LOC CSF obtains and manages geographical location information based on the requests from AEs residing on either a local Node or a remote Node. The LOC CSF interacts with any of the following:

- a location server in the Underlying Network,

- a GPS module in an M2M device, or

- information for inferring location stored in other Nodes.

In order to update the location information, an AE can configure an attribute (e.g., update period). Based on such defined attributes, the LOC CSF can update the location information using one of the location retrieval mechanisms listed above.

NOTE:	The location technology (e.g., Cell-ID, assisted-GPS, and fingerprint) used by the Underlying Network depends on its capabilities.

The functions supported by the LOC CSF are as follows:

- Requests other Nodes to share and report their own or other Nodes' geographical location information with the requesting AEs.

- Provides means for protecting the confidentiality of geographical location information.

### 6.2.8	Network Service Exposure, Service Execution and Triggering

#### 6.2.8.1	General Concepts

The Network Service Exposure, Service Execution and Triggering (NSSE) CSF manages communications with the Underlying Networks for accessing network service functions over the Mcn reference point. The NSSE CSF uses the available/supported methods for service "requests" on behalf of AEs. The NSSE CSF shields other CSFs and AEs from the specific technologies and mechanisms supported by the Underlying Networks.

NOTE:	The NSSE CSF provides adaptation for different sets of network service functions supported by various Underlying Networks.

The network service functions provided by the Underlying Network include service functions such as, but not limited to, device triggering, small data transmission, location notification, policy rules setting, location queries, IMS services, device management. Such services do not include the general transport services.

#### 6.2.8.2	Detailed Descriptions

The NSSE CSF manages communication with the Underlying Networks for obtaining network service functions on behalf of other CSFs, remote CSEs or AEs. The NSSE CSF uses the Mcn reference point for communicating with the Underlying Networks.

The M2M System allows the Underlying Networks to control network service procedures and information exchange over the Underlying Networks while providing such network services. For example, for the 3GPP networks, the Underlying Network can choose to provide the network services based on control plane signalling mechanisms.

Other CSFs in a CSE that need to use the services offered by the Underlying Network use the NSSE CSF.

The service functions supported by the NSSE CSF are as follows:

- The NSSE CSF shields other CSFs and AEs from the specific technology and mechanisms supported by the Underlying Networks.

NOTE:	The NSSE CSF provides adaptation for different sets of network service functions supported by various Underlying Networks.

- The NSSE CSF maintains the necessary connections and/or sessions over the Mcn reference point, between the CSE and the Underlying Network when local CSFs are in need of a network service.

- The NSSE CSF provides information to the CMDH CSF related to the Underlying Network so the CMDH CSF can include that information to determine proper communication handling.

### 6.2.9	Registration

#### 6.2.9.1	General Concepts

The Registration (REG) CSF processes a request from an AE or another CSE to register with a Registrar CSE in order to allow the registered entities to use the services offered by the Registrar CSE.

#### 6.2.9.2	Detailed Descriptions

Registration is the process of delivering AE or CSE information to another CSE in order to use M2M Services.

An AE on an ASN, an MN or an IN performs registeration locally with the corresponding CSE in order to use M2M services offered by that CSE. An AE on an ADN performs registration with the CSE on an MN or an IN in order to use M2M services offered by that CSE. An IN-AE performs registration with the corresponding CSE on an IN in order to use M2M services offered by that IN CSE. An AE can have interactions with its Registrar CSE (when it is the target CSE) without the need to have the Registrar CSE register with other CSEs.

The CSE on an ASN performs registration with the CSE in the MN in order to be able to use M2M Services offered by the CSE in the MN. As a result of successful ASN-CSE registration with the MN-CSE, the CSEs on the ASN and the MN establish a relationship allowing them to exchange information.

The CSE on an MN performs registration with the CSE of another MN in order to be able to use M2M Services offered by the CSE in the other MN. As a result of successful MN-CSE registration with the other MN-CSE, the CSEs on the MNs establish a relationship allowing them to exchange information.

The CSE on an ASN or on an MN perform registration with the CSE in the IN in order to be able to use M2M Services offered by the CSE in the IN. As a result of successful ASN/MN registration with the IN-CSE, the CSEs on ASN/MN and IN establish a relationship allowing them to exchange information.

Following a successful registration of an AE to a CSE, the AE is able to access, assuming access privilege is granted, the resources in all the CSEs that are potential targets of request from the Registrar CSE.

The capabilities supported by the REG CSF are as follows:

- Ability for AEs to register to their associated CSE, as per Table 6.2.9.2-1,

- Ability for CSE to register to the other CSE, as per Table 6.2.9.2-1,

- Ability for an ASN-CSE/MN-CSE to register association of its M2M-Ext-ID (if available) with its CSE-ID, (see clause 7.1.8).

- Ability for an ASN-CSE/MN-CSE to register association of its Trigger-Recipient-ID (if available) with its CSE-ID, (see clause 7.1.8). When Trigger-Recipient-ID is not present, it is assumed that the CSE is not able to receive triggers.

NOTE: Such registrations are applicable to a single M2M Service Provider Domain.

Rapporteur Note:  Need discussions on the placement of the following text. This was not handled completely during the ARC#10.bis (AdHoc meeting)

The following Table shows which oneM2M entity types can register with which other entity types.

Table 6.2.9.2-1: Entity Registration

| Originator (Registree) | Receiver (Registrar) | Registration Procedure |
| --- | --- | --- |
| ADN-AE | MN-CSE, IN-CSE | AE registration procedure see clause TBD |
| ASN-AE | ASN-CSE |  |
| MN-AE | MN-CSE |  |
| IN-AE | IN-CSE |  |
| ASN-CSE | MN-CSE, IN-CSE | CSE registration procedure see clause TBD |
| MN-CSE | MN-CSE, IN-CSE |  |

The Originator (Registree) in Table 6.2.9.2-1 requests the registration and the Receiver (Registrar) is responsible for verifying the request, and checking the authentication and authorization of the Originator in order to establish a peer relationship.

[6.2.9.2.c] Editor’s Note: Authorization above might only be needed for AEs, this is FFS in Security WG

- An AE can not be registered to more than one CSE (ASN-CSE, MN-CSE or IN-CSE).

- An ASN-CSE can be registered to at most one other CSE (MN-CSE or IN-CSE).

- An MN-CSE can be registered to at most one other IN-CSE.

An MN-CSE can support only a single registration towards another MN-CSE or an IN-CSE. A concatenation (registration chain) of multiple uni-directional registrations shall not form a loop. E.g., Two MN-CSEs A and B, can not register with each other. Three MN-CSEs A, B and C, where A registers to B, and B registers to C, then C can not register to A.

Registration information shall be able to include:

- Identifier.

- Reachability schedules; which are elements of a node's policy, and specify when messaging can occur between nodes. Reachability schedules can be used in conjunction with other policy elements. When reachability schedules are not present in a node then that node is expected to be always reachable.

[6.2.9.2.e] Editor's note: Further Registration information is FFS.

### 6.2.10	Security

#### 6.2.10.1	General Concepts

The Security (SEC) CSF comprises the following functionalities:

- Sensitive data handling;

- Security administration;

- Security association establishment;

- Access control inlcuding identification, authentication and authorization;

- Identity management.

Sensitive data handling functionality in the SEC CSF protects the local credentials on which security relies during storage and manipulation. Sensitive data handling functionality performs other sensitive functions such as security algorithms. This functionality is able to support several cryptographically separated security environments.

Security management capabilities are provided by the Security Administration functionality as specified in TS-0003 [TS0003].

NOTE: ASM and DMG CSFs do not include security management capabilities of the SEC CSF.

Security administration functionality enables services such as the following:

- Creation and administration of dedicated security environment supported by Sensitive Data Handling functionality;

- Post-provisioning of a root credential protected by the security environment;

- Provisioning and administration of subscriptions related to M2M Common Services and M2M Application Services.

Security association establishment functionality establishes security association between corresponding M2M Nodes, in order to provide services such as confidentiality and integrity.

Access control functionality authorizes services and specific operations (e.g., Read/Update) on resources identified and authenticated entities, according to provisioned access control policies and assigned roles.

While unique identifier of an entity are used for authentication and identity management, this functionality provides pseudonyms which serve as temporary identifiers which cannot be linked to the true identity of either the associated entity or its user.

#### 6.2.10.2	Detailed Descriptions

The functionalities supported by the SEC CSF are as follows:

- Sensitive data handling:

- Provides the capability to protect the local credentials on which security relies during storage and manipulation.

- Extends sensitive data handling functionality to other sensitive data used in the M2M Systems such as subscription related information, access control policies and personal data pertaining to individuals.

- Performs other sensitive functions as well, such as security algorithms running in cryptographically separated secure environments.

- Security administration :

- Creates and administers dedicated secure environment supported by sensitive data handling functionality

- Post-provisions master credentials protected by the secure environment.

NOTE:	The secure environment can also be pre-provisioned with a master credentials prior to deployment; therefore this capability is not always required. Post-provisioning is required when secure remote provisioning needs to be performed or re-initiated after deployment.

- Provisioning and administration of subscriptions related to M2M Services and M2M application services. Besides the associated master credentials, a subscription includes other information classified as sensitive data such as authorization roles and identifiers for access control management.

- Security association establishment:

- Establishes security associations between corresponding M2M Nodes in order to provide specific security services (e.g., confidentiality, integrity, or support for application level signature generation and verification) involving specified security algorithms and sensitive data. This involves key derivation based on provisioned master credentials. This functionality of the SEC CSF is mandatory when security is supported.

- Access control:

- Authorizes services and specific operations (e.g. Read/Update) on resources to identified and authenticated entities, according to provisioned access control policies and assigned roles. This functionality is mandatory when any services relying on authorization and access control are present. Among other usages, the services of this functionality may be applied to personal information as a means to preserve privacy.

- Identity protection:

- Provides pseudonyms to be used instead of the unique identifiers of an entity to serve as temporary identifiers not linkable to the true identity of either the associated entity or its user.

Detailed functionalities are described in the Security Solutions Technical Specification [TS0003].

### 6.2.11	Service Charging and Accounting

#### 6.2.11.1	General Concepts

The Service Charging and Accounting (SCA) CSF provides charging functions for the Service Layer. It supports different charging models which also include online real time credit control. The SCA CSF manages service layer charging policies and configuration capturing service layer chargeable events, generating charging records and charging information. The SCA CSF can interact with the charging System in the Underlying Network also. The SCA CSF in the IN-CSE handles the charging information.

#### 6.2.11.2	Detailed Descriptions

The SCA CSF performs information recording corresponding to a chargeable event based on the configured charging policies. The SCA CSF sends the charging information transformed from the specific recorded information to the billing domain by the use of a standard or proprietary interface for charging purposes.

The SCA CSF supports "independent service layer charging" and "correlated charging with the Underlying Network" charging system.  For independent service layer charging, only charging functions in the M2M service layer are involved. For correlated charging, charging functions in both the service layer and the Underlying Network are involved.

The SCA CSF supports one or multiple charging models, such as the following:

- Subscription based charging: A service subscriber is charged based on service layer subscriptions.

- Event based charging: Charging is based on service layer chargeable events. A chargeable event refers to the discrete transactions. For example, an operation on data (Create, Update, Retrieve) can be an event. Chargeable event can also be timer based. Chargeable events are configurable to initiate information recording. More than one chargeable event can be simultaneously configured and triggered for information recording.

The Service Layer charging system consists of the following logical functions:

- Charging management function: This function handles charging related policies, configurations, inter-node charging function communications and interacting with the charging system in the Underlying Network. Charging related policies.

- Charging triggering function: This function resides in the service layer. It captures the chargeable event and generates recorded information for charging. Recorded information may contain mandatory and optional elements.

- Offline charging function: This function handles offline charging related operations. Offline charging does not affect services provided in real time. Charging triggering information is generated at the CSFs where the chargeable transaction happens. The offline charging function generates service charging records based on recorded information. A service charging record is a formatted collection of information about a chargeable event (e.g. amount of data transferred) for use in billing and accounting.

NOTE: Charging triggering and offline charging function are based on charging policies. The system may record information for other purposes such as for event logging. Some of such information may be applicable for charging purposes.

### 6.2.12	Subscription and Notification

#### 6.2.12.1	General Concepts

The Subscription and Notification (SUB) CSF provides notifications pertaining to a subscription that tracks changes on a resource (e.g., deletion of a resource). A subscription to a resource is initiated by an AE or a CSE, and is granted by the Hosting CSE subject to access control policies. During an active resource subscription, the Hosting CSE sends a notification per modification of the resource to the address(es) where the resource subscriber wants to receive it.

#### 6.2.12.2	Detailed Descriptions

The SUB CSF manages subscriptions to resources, subject to access control policies, and sends corresponding notifications to the address(es) where the resource subscribers want to receive them. An AE or a CSE is the subscription resource subscriber. AEs and CSEs subscribe to resources other CSEs. A subscription Hosting CSE sends notifications to the address(es) specified by the resource subscriber for modifications on a resource. The scope of a resource subscription includes tracking changes of attribute(s) and child resource(s) of the subscribed-to resource. It does not include tracking the change of attribute(s) of the child resource(s). Filter criteria conditions can be used to constrain the scope of tracking.

A subscription is represented as resource subscription in the CSE resource structure.

The functions supported by the SUB CSF are as follows:

- Inclusion of the resource subscriber ID, the Hosting CSE-ID and subscribed-to resource address(es) per resource subscription request. It may also include other criteria (e.g., resource modifications of interest and notification policy) and the address(es) where to send the notifications.

- Ability to subscribe to a single resource via a single subscription, or subscribe to multiple resources via a single subscription when they are grouped and represented as a single group resource.

## 6.3	Security Aspects

The oneM2M Security Analysis Technical Report [i.3] differentiates security domains related to the transport layer (Underlying Network), service layer (M2M common services) and Application Layer. It also considers possible trust scenarios involving these different security domains, and investigates countermeasures to threats that potentially affect the security of the M2M System.

Each of the security domains may provide their own set of security capabilities. The oneM2M security solution shall provide configurable security services through an API for upper security domains to leverage, or enable the use of the exposed security features of other security domains when appropriate.

As a result, beyond providing security solutions that protect the integrity of the M2M Service Layer, the oneM2M architecture exposes, through its APIs, further security services that are made available to M2M Applications. This enables M2M Applications to benefit from security solutions deployed in the M2M Service Architecture, without adding redundant and/or proprietary security solutions.

NOTE:	It remains the responsibility of M2M Application Service Providers to perform their own risk assessment process to identifying the specific threats affecting them and derive their actual security needs.

Security aspects are described in oneM2M Security Solutions Technical Specification [TS0003].

## 6.4	Inter-M2M SP Communication

### 6.4.1	Inter M2M SP Communication for oneM2M Compliant Nodes

To enable M2M entities (e.g., CSE, AE) in different M2M Service Provider (SP) domains to communicate, configuration within the M2M domain determines if such a communication is allowed. If allowed, the M2M System shall support routing of the traffic across the originating M2M SP domain and within the target M2M SP domain.

Communication between different M2M SPs which occurs over the reference point Mcc', is subject to business agreements. The offered functionality is typically a subset of the functionality offered over the Mcc reference point.

The following Table shows which oneM2M entity types may register with which other entity types across the Mcc’ reference point.

Table 6.4.1-1 - Inter M2M SP Entity Registration

| Originator (Registree) | Receiver in different M2M SP domain (Registrar) | Registration Procedure |
| --- | --- | --- |
| IN-CSE | IN-CSE | Inter M2M SP registration procedure see clause TBD |

An IN-CSE is allowed to register to a single IN-CSE of multiple different M2M SP domains in the M2M System.

Any interM2M SP communication in support of a request originating from one M2M SP domain shall be processed and forwarded through the Infrastructure Node of the originating M2M domain towards the Infrastructure Node of the target M2M SP domain and finally forwarded to its target CSE, if different from the Infrastructure Node. Hence the Infrastructure Node in both M2M domains shall be the exit and entry points, respectively, for all inter M2M SP communication traffic.

In this configuration approach, public DNS shall be used to support traffic routing for inter M2M SP communication in accordance with [i.16]. This relies on public domain names being allocated to communicating CSE entities within the oneM2M architecture, and to whom access across domains is permitted through policies.  To that effect, an M2M SP supporting inter- M2M SP communication shall ensure that the public domain names for the CSEs whose functionality is available across domains are held in its public DNS and shall always point to the IP address associated with the Infrastructure Node for the domain (being the entry point) for accessibility purposes.

The M2M SP could optionally also have additional policies (example: black list or white list) that governs accessibility from other domains to CSE functionality located within its own domain. These policies are however out of scope of this document.

The public domain names of CSEs to whom access from other domains is allowed by policies, shall be created in the DNS of the M2M SP by the Infrastructure Node at registration time of these CSEs, and shall be removed at de-registration. DNS entries for CSEs can also be created/removed for registered CSEs at any time by the M2M SP through administrative means to handle dynamic policies.

#### 6.4.1.1	Public Domain Names and CSEs

To enable the usage of public DNSs as described above, there is a need for a naming convention for public names for CSEs.  This naming convention facilitates the creation of the necessary entries of the public domain names of CSEs in the DNS by the infrastructure node.

CSEs public domain names shall be a sub-domain of the Infrastructure Node’s public domain name. This naming convention allows the Infrastructure Node to include the needed DNS entry corresponding to the CSE to whom access from other domains is allowed. This would typically occur when the CSE registers with the Infrastructure Node, subject to policies, or administratively.

Accordingly, the structure of the public domain of the CSEs in IN/MN/ASN shall follow the following naming convention, which relies on the CSE identifier (CSE-ID) as part of the naming convention to facilitate the DNS entry creation:

- Infrastructure Node CSE public domain name: <Infrastructure Node CSE Identifier>.<M2M Service Provider domain name>.

- Middle Node CSE public domain name: <Middle Node CSE Identifier>.<Infrastructure Node public domain name>.

- Application Service Node  CSE public domain name: <Application Service Node CSE Identifier>.<Infrastructure Node public domain name>.

Both the MN-CSE and the ASN-CSE public domain names are sub-domains of the Infrastructure Node public domain name.

The A/AAAA records in the DNS, as per [i.9], [i.11], and [i.15] shall consist of the public domain name of the CSE and the IP address of the M2M Infrastructure Node, since the M2M Infrastructure Node is the entry point of the M2M Service Provider domain name where it belongs to.

Note that entries in the public domain names of the three nodes depicted above do not imply that the actual CSE-Identifier allocated for that node has to be used in the DNS entry. Rather any name, including indeed the CSE Identifier for the node, can be used there as long as the entry resolves to the intended Node.

Example:

These 3 host entries are valid entries in the DNS:

- MN-CSEID.IN-CSEID.m2m.myoperator.org

- node1.node2.m2m.myoperator.org

- MN-CSEID.node22.m2m.myoperator.org

### 6.4.2	Inter M2M SP Generic Procedures

This clause describes the behaviour of the M2M Nodes in support of inter-M2M SP procedures.

#### 6.4.2.1	Actions of the Originating M2M Node in the Originating Domain

The originator in the originating domain can be any M2M node such as ADN, an MN, or an ASN, and shall send a request to the Registrar CSE to retrieve a resource located in another M2M SP domain.

The originator shall use any of the options defined in clause 9.3.1 to identify the target host and resource for that purpose.

#### 6.4.2.2 	Actions of the Receiving CSE in the Originating Domain

The receiving CSE in the originating domain shall check if the addressed resource is locally available. If the addressed resource is not locally available, then the request shall be forwarded to the next hop.

If the receiving CSE is on an IN, it shall check if the addressed resource is locally available within its domain.  If the addressed resource is not located within its own domain, then the IN shall perform a DNS lookup by using the target hostname provided in the RETRIEVE request. A successful DNS lookup shall return to the origin IN in the originating domain the IP address of the M2M IN residing in the target M2M SP domain.

Subsequently, the IN in the originating domain shall forward the request to the IN of the target domain.

#### 6.4.2.3	Actions in the IN of the Target Domain

The IN is the entry point of the target M2M SP domain. The IN shall check if the addressed resource is a local resource. If it is not a local resource it shall forward the request to the appropriate CSE, after identifying the hosting CSE within its domain, using the pointOfAccess attribute.

Once the request reaches the target Hosting-CSE, the CSE shall apply the access control policies applicable to the request.  Consequently, the Hosting-CSE shall forward the response for the incoming request following the same path of the incoming request.

### 6.4.3. 	DNS Provisioning for Inter-M2M SP Communication

As specified previously, any M2M SP supporting inter-M2M SP communication shall ensure that the public domain names for the CSEs whose functionality is available across domains are held in the M2M SP’s DNS and shall always point to the IP address associated with the Infrastructure domain CSE (being the entry point) for accessibility purposes.

This implies that the IN-CSE shall be responsible for creating the appropriate entry in the DNS for a successfully registered CSE in the IN-CSE, if the M2M SP policies do allow access to the CSE across multiple M2M domains. Similarly the IN-CSE shall be responsible for deleting the appropriate entry in the M2M SP’s DNS for a successfully de-registered CSE in the IN-CSE if the M2M SP policies do allow access to the CSE across multiple M2M domains

#### 6.4.3.1 	Inter-M2M SP Communication Access Control Policies

Additional M2M SP policies that further restrict access to CSEs to requests originating from configured M2M SPs only, can complement the DNS entries created by the IN-CSE.  These policies are out of scope of this document.

## 6.5	M2M Service Subscription

The M2M Service Subscription defines the technical part of the contract between an M2M Subscriber (typically an M2M Application Service Provider) and an M2M Service Provider. Each M2M Service Subscription shall have a unique identifier, the M2M-Sub-ID, as specified in clause 7.1.11. An M2M Service Subscription establishes a link between one or more AEs; one or more M2M Nodes, one or more roles associated with an M2M Service Subscription as well as subscriber defined groups used for access control policies.

An M2M Service is the marketable service offered to M2M service subscriber (organizations or individuals). Examples of services include device management, location, data exchange, etc.

In each M2M Service, one or multiple M2M Service role(s) shall be defined by the M2M Service Provider. An M2M Service subscription role is defined as a set of privileges pertaining to a resource types which are associated with M2M Service. The M2M Subscriber subscribes as one or multiple roles within the M2M Services, depending upon which role(s) the M2M Subscribers are interested in.

An example of the set of M2M Services and M2M Service subscription roles is provided in Annex G.

It shall be possible for an M2M Service Provider to define their own services and roles.

An M2M Service Subscription governs an M2M subscription, the M2M Nodes where AEs can reside and execute, the M2M Service roles and subscriber defined groups used for access control policies.  An example of the Subscriber defined group is “all node applications within this service subscription with App-ID set to a specific value”. Access control policy resource configuration shall comply with the M2M Service Subscription. For authorization of accessing the resource, access control policy is applied and can be based on M2M Service subscription roles.

How to authorize the request operation based on M2M Service Subscription resource are defined in TS-0003 [TS0003].

An M2M Service Subscription shall be used for the following purposes:

- Serve as a basis for authorization for resource operations

- Serve as the basis for charging.

- Identify which Nodes are part of this M2M Service Subscription

# 7	M2M Identification and Addressing

## 7.1	M2M Identifiers

This clause provides a list of identifiers required for the purpose of interworking within the oneM2M architectural model.

An identifier provides unique information or name, tag or label which has a consistent meaning when applied. For example, a CSE-ID for an MN is used for the purpose of CSE identification within an MN. A CSE-ID for an ASN is used for CSE identification within an ASN.

One identifier shall not be assigned to two or more different entities. An identifier is used to reference M2M entities during their lifetime. Such identifiers provide information for association with other identifiers.

It is assumed that the Application Identifier (App-ID) and the CSE Identifier (CSE-ID) have been assigned initially and are known before an M2M System boots up.

.

### 7.1.1	M2M Service Provider Identifier (M2M-SP-ID)

An M2M Service Provider shall be uniquely identified by the M2M Service Provider Identifier (M2M-SP-ID). This is a static value assigned to the Service Provider.

### 7.1.2	Application Entity Identifier (AE-ID)

An Application Entity Identifier (AE-ID) uniquely identifies an AE resident on an M2M Node, or an AE that requests to interact with an M2M Node. An AE-ID shall identify an Application Entity for the purpose of all interactions within the M2M System.

It is the responsibility of the M2M Service Provider to ensure that the AE-ID is globally unique. The AE-ID shall include the Application ID (see clause 7.1.3).

### 7.1.3	Application Identifier (App-ID)

This is equivalent to the application name and is not guaranteed to be globally unique on its own.

This identifier may be supported via a single or multiple registration authorities/entities. The definition and assignment of such an identifier is out of scope of this document.

NOTE:  Detailed format and structure of the App-ID is not specified in the present document.  Who assigns the App-ID is not specified in the present document.

### 7.1.4	CSE Identifier (CSE-ID)

A CSE shall be identified by a globally unique identifier, the CSE-ID, when instantiated within an M2M node in the M2M System.

The CSE-ID shall identify the CSE for the purpose of all interactions from/to the CSE within the M2M System.

### 7.1.5	M2M Node Identifier (M2M-Node-ID)

An M2M node, hosting a CSE and/or Application(s) shall be identified by a globally unique identifier, the M2M-Node-ID.

The M2M System shall allow the M2M Service Provider to set the CSE-ID and the M2M-Node-ID to the same value.

The M2M-Node-ID enables the M2M Service Provider to bind a CSE-ID to a specific M2M node.

Examples of allocating a globally unique M2M-Node-ID include the use of Object Identity (OID) and IMEI. For details on OID, see Annex H.

### 7.1.6	M2M Service Subscription Identifier (M2M-Sub-ID)

The M2M-Sub-ID enables the M2M Service Provider to bind application(s), M2M nodes, CSEs and services identified by service identifiers to a particular M2M service subscription.

The M2M Service Subscription Identifier has the following characteristics:

- belongs to the M2M Service Provider;

- identifies the subscription to an M2M Service Provider;

- enables communication with the M2M Service Provider;

- can differ from the M2M Underlying Network Subscription Identifier.

There can be multiple M2M Service Subscription Identifiers per M2M Underlying Network subscription.

### 7.1.7	M2M Request Identifier (M2M-Request-ID)

The M2M-Request-ID tracks a Request initiated by an AE over the Mca reference point, and by a CSE over the Mcc reference point, if applicable, end to end. It is also included in the Response to the Request over the Mca or Mcc reference points.

To enable an AE to track Requests and corresponding Responses over the Mca reference point, AEs shall include a distinct M2M Request Identifier per request over the Mca Reference point to the CSE for any initiated request.

The CSE shall make such M2M Request Identifier received from the AE globally unique by appending its CSE-ID to it.

If the CSE creates an M2M Request Identifier, then the CSE shall maintain a binding between the M2M Request Identifier received from the AE and the M2M Request Identifier it created in its interactions towards other peer CSEs. The CSE shall include the M2M Request Identifier received from the AE in its Response to the AE. This binding shall be maintained by the CSE until the Request message sequence is completed. Note that the Request initiated by the CSE could be the result of an application Request, or a request initiated autonomously by the CSE to fulfil a service.

In case the receiving CSE is not reachable over the underlying network, the IN-CSE initiates procedure for "waking up" the Node hosting the receiving CSE by using procedures such as device triggering over the Mcn reference point. For Device Triggering, the triggering reference number used to co-relate device triggering response is independent of the M2M Request Identifier. An IN-CSE may use the same value of an M2M-Request-Identifier in an incoming request for the triggering reference number in its interaction with the underlying network.

A CSE receiving a Request from a peer CSE shall include the received M2M Request Identifier in all additional Requests unspanned (i.e.,1:1) it has to generate (including propagation of the incoming Request) and that are associated with the incoming Request, where applicable.

Note that the M2M Request Identifier can be made globally unique by including the CSE-ID in combination with any random number.

### 7.1.8	M2M External Identifier (M2M-Ext-ID)

The M2M-Ext-ID is used by an M2M Service Provider (M2M SP) when services targeted to a CSE, identified by a CSE-ID, are requested from the Underlying Network.

The M2M External Identifier allows the Underlying Network to identify the M2M Device associated with the CSE-ID for the service request. To that effect, the Underlying Network maps the M2M-Ext-ID to the Underlying Network Identifier it allocated to the target M2M Device. In addition, the M2M SP shall maintain the association between the CSE-ID, the M2M-Ext-ID and the identity of the Underlying Network.

Both pre-provisioned and dynamic plug and play association between the CSE-ID with the M2M-Ext-ID should be supported.

NOTE 1:	For each CSE-ID, there shall be only one M2M-Ext-ID for a specific UNetwork-ID. Hence an M2M SP interworking with multiple Underlying Networks shall have a different M2M-Ext-IDs associated with the same CSE-ID, one per Underlying Network and  shall select the appropriate M2M-Ext-ID for any service request it initiates towards an Underlying Network.

NOTE 2:	The mapping by the Underlying Network of the M2M-Ext-ID to the M2M Device is access network specific.

### 7.1.9	Underlying Network Identifier (UNetwork-ID)

The UNetwork-ID is used for identifying an Underlying Network. UNetwork-ID is a static value and unique within a M2M Service Provider domain.

One or more Underlying Networks may be available at an M2M Node offering different sets of capabilities, availability schedules etc. Based on the "policy" information at the Node and the capabilities offered by the available Underlying Networks, appropriate Underlying Network can be chosen by using UNetwork-ID. For example, based on "policy", scheduling of traffic triggered by a certain event category in certain time periods may be allowed over Underlying Network "WLAN" but may not be allowed over Underlying Network "2G Cellular".

### 7.1.10	Trigger Recipient Identifier (Trigger-Recipient-ID)

The Trigger-Recipient-ID is used when device triggering services are requested from the Underlying Network, to identify an instance of an ASN/MN-CSE on an execution environment, to which the trigger is routed. For example, when 3GPP device triggering is used, the Trigger-Recipient-ID maps to the Application-Port-Identifier [3GPP 23.682].

### 7.1.11	M2M Service Identifier (M2M-Serv-ID)

The M2M-Serv-ID is an identifier of a M2M Service offered by an M2M SP. It is an essential part of the M2M Service Subscription which stores a set of M2M-Serv-IDs pertaining to the set of subscribed services. Beyond the set of services depicted in this specification it shall be possible for an M2M Service Provider to offer other services. Those will be identified by means of M2M SP specific M2M-Serv-IDs.

## 7.2	M2M Identifiers lifecycle and characteristics

- Table 7.2-1: M2M Identifiers lifecycle and characteristics

| Identifier | Assigned by | Assigned to | Assigned during | Lifetime | Uniqueness | Used during | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M2M Service Provider Identifier | Out of scope | AE, CSE | Out of scope | Out of scope | Global | Provisioning |  |
| Application Entity Identifier | AE or Registrar CSE | AE | AE start-up | Application Entity Registration | Global | - Application Entity Registration - Security Context Establishment - All other operations initiated by the AE | Security requirements apply for Security Context Establishment |
| Application Identifier | Out of scope | Out of scope | Pre-provisioned | Out of scope | Specific to M2M service deployment | - Application Entity registration |  |
| CSE Identifier | M2M SP | CSE | Security Provisioning | Life of the CSE | Global | - Information flows (clause 10) - Security Context Establishment | Security requirements apply for Security Context Establishment |
| M2M Node Identifier | Out of Scope | M2M Node hosting CSE | Pre-provisioned | Life of the M2M Node | Global | - Device Management | Needs to be Read Only |
| M2M Subscription Identifier | M2M SP, Out of Scope | Application Entities, and one or more CSEs belonging to the same M2M subscriber | At service signup | Life of the M2M Service Subscription with the MM SP | Global | - Charging and Information Recorded - Role based access control - Authentication | Multiple CSEs can be allocated the same M2M Subscription Identifier |
| M2M-Request-ID | Mcc: CSE Mca: Application Entity | A request initiated by an AE or CSE | Mcc: When a request is initiated by a CSE, or handling of a request received by a CSE. Mca: When a request is initiated by an AE | Equal to the lifetime of the Request and its corresponding Response | Mcc: Global Mca: Local or global | Requests and corresponding responses |  |
| External Identifier | Manual mode: Jointly between the Underlying Network provider and M2M SP, where applicable. Plug-And- Play: FFS | M2M Node belonging to a CSE that wants to utilize services of the Underlying Network. | Manual Mode: Administrative Agreement. Plug-And- Play: FFS | Life of the CSE. | Local or global, decided by the specific Underlying Network provider | Requests initiated by a CSE over the Mcn reference point, where applicable. |  |
| Underlying Network Identifier | M2M SP | Underlying Networks | Pre-provisioned | Life of the Underlying Network | Local to M2M SP domain | UL Network selection |  |
| Trigger Recipient Identifier | Execution Environment | ASN/MN-CSE | ASN/MN-CSE start-up or wake-up | Life of the CSE | Execution Environment-wide | Device Triggering procedures, where applicable |  |
| M2M Service Identifier | M2M Service Provider, Out of Scope | A service defined by the M2M Service Provider which consists of a set of functions defined by this document. | Out of Scope | Out of Scope | Local to the M2M Service Provider | For M2M Service Subscription |  |

## 7.3	Addressing an Application Entity

### 7.3.1	Application Entity Addressing

In M2M communication, the goal of M2M addressing is to reach the CSE with which the target AE is registered, and ultimately the target AE on the M2M Node on which the target AE is resident. This principle applies to all Application Entities.

Reachability and routing from/to AEs on M2M Nodes is associated with the CSEs with which these AEs are registered, and the connectivity of such CSEs to the Underlying Networks. Reaching an AE shall be performed through reaching the CSE the AE is registered with. A CSE-PoA (CSE Point of Access) shall provide the set of information needed to reach a CSE from an Underlying Network perspective. Typically a CSE-PoA contains information that is resolved into a network address.

### 7.3.2	Application Entity Reachability

#### 7.3.2.1	CSE Point of Access (CSE-PoA)

The CSE-PoA shall be used by the M2M System to communicate with a CSE on an M2M Node. Once communication with a CSE is achieved, an AE registered with that CSE can be reached as long as the AE can be uniquely identified.

The information included in the CSE-PoA as well as the refresh of the CSE-PoA, depends on the characteristics of the Underlying Network and an M2M Node’s transport capabilities.

#### 7.3.2.2	Locating Application Entities

Locating an AE is a two-step process as follows:

- Step 1: There is a need to locate the CSE where the AE is registered. Locating the CSE shall be accomplished as follows:

- For AEs associated with ASNs/MNs/INs, the CSE-PoA of the ASN-CSE/MN-CSE/IN-CSE where the AE is registered shall be used.

- For AEs associated with ADNs, the CSE-PoA of the MN-CSE/IN-CSE where the ADN is registered shall be used.

- Step 2: The CSE shall locate the appropriate AE using its Application Entity Identifier (AE-ID).

#### 7.3.2.3	Usage of CSE-PoA by the M2M System

The CSE-PoA holds the information used by the M2M System to locate routing information for a CSE. This information shall be provided by the CSE at registration time. However, the routing information related to a CSE (and ultimately to the target AE) in an M2M System depends on the characteristic of the Underlying Network. This impacts the criteria for updating the CSE-PoA by the registered CSE, in addition to the regular CSE registration updates. The information to be conveyed as CSE-PoA needs to support Underlying Network specifics.

In general the addressing and routing information related to a CSE can be achieved when a static public IP address is assigned to and M2M Node and direct DNS address translation or dynamic DNS address translation is used.

In those circumstances, the CSE-PoA for a registered CSE shall have a URI conforming to RFC 3986 [i.12] as follows:

- URI = scheme:/fullyqualifieddomainname/path/; or

- URI = scheme://ip-address/path/.

The following clauses specify the information to be conveyed in the CSE-PoA by a registered CSE for various types of Underlying Networks, as well as the criteria for updating the CSE-PoA for the registered CSEs, in addition to the normal CSE registration refresh.

##### 7.3.2.3.1	CSE-PoA related to CSEs associated with a Fixed Network

In this case the CSE-PoA for a registered CSE shall have a URI as described above. If the IP address is private, then the address is usually built based on the address of the related PPP protocol which is a public IP address. This in turn is mapped to the corresponding private address.

##### 7.3.2.3.2	CSE-PoA related to CSEs associated with Mobile Networks

If the IP address for the registered CSE cannot be reliably used, and cannot be included in the CSE-PoA, then the CSE-PoA for the registered CSE shall include appropriate information as required by the respective Underlying Networks and supported by oneM2M.

Each Underlying Network shall need to specify the means for allowing an M2M SP to fetch the IP address associated with a CSE attaching to that Underlying Network and consequently the information to be included in the CSE-PoA for the registered CSE.

In the event that the M2M SP has connections to multiple Underlying Networks, there is a need to establish a binding between the registered CSE and the associated Underlying Network. That binding may be established through CSEs explicitly identifying the Underlying Network at registration/update time. Otherwise the M2M SP may derive the identity of the Underlying Network, e.g., by using the link over which the registration arrived, store it and bind it to the registration information.

In the scenarios an M2M Node in mobile networks is not reachable by the previously known IP address and it supports SMS, the originating CSE can make use of SMS for device triggering mechanism to wake up the M2M Node to renew the IP addresses or perform specific functionalities.

To support this option, the CSE-PoA shall, on Mcn interface to the Underlying Networks supporting such an SMS for device triggering mechanism, include identification information of the CSE (such as the external identifier as defined by 3GPP TS 23.682 [i.17] in the case of Tsp-based triggering, or MSISDN or any identifier used by triggering network APIs), and send the request to the Underlying Network via the mechanisms supported, such as Tsp, Tsms, Network APIs.

Annex B shows the 3GPP defined interfaces for machine type communication interfaces and example device triggering flows.

##### 7.3.2.3.3	CSE-PoA to CSEs associated with multiple Underlying Networks

When an M2M Node attaches to a fixed network, the CSE-PoA for a registered CSE shall conform to the procedures associated with the fixed network.

When an M2M Node attaches to a mobile network, the CSE-PoA for a registered CSE shall conform to the procedures associated that mobile network.

If an M2M Node is already attached to an Underlying Network and attaches to another Underlying Network, the CSE may update its PoA information at the remote CSE.

### 7.3.3		Notification Re-targeting

#### 7.3.3.1	Application Entity Point of Access (AE-PoA)

A Notify request to an AE is sent by targeting <AE> resource on a Hosting CSE. If the Hosting CSE verifies access control privilege of the Originator, the Hosting CSE shall re-target the request to the address specified as AE-PoA (i.e., pointOfAccess attribute of <AE> resource). The AE-PoA may be initially configured in the <AE> resource when the AE registers to the Registrar CSE. If the <AE> resource does not contain an AE-PoA, an active communication link, if available, can be used for re-targeting. If neither of them is available, the request cannot be re-targeted to the AE.

Figure 7.3.3-1: Re-targeting a notification request to an AE

# 8	Description and Flows of Reference Points

## 8.1	General Communication Flow Scheme on Mca and Mcc Reference Points

Procedures involving CSEs and AEs are driven by the exchange of messages across reference points according to the message flows described in this clause.

Depending on the message operation, procedures may manipulate information in a standardized resource structure as described in clause 9. Access and manipulation of the resources is subject to their associated privileges.

### 8.1.1	Description

Figure 8.1.1-1 shows the general flow that governs the information exchange within a procedure, which is based on the use of Request and Response scheme. The scheme applies to communications such as:

- between an AE and a CSE (Mca reference point); and

- among CSEs (Mcc reference point).

The communications can be initiated either by AEs or by CSEs depending upon the operation in the Request message.

- Figure 8.1.1-1:  General Flow

### 8.1.2	Request

The Request from an Originator to a Receiver includes the following parameters:

- to: URI of the target resource for the operation. The to parameter shall conform to clause 9.3.1

NOTE 1:  The to target resource needs to be known by the Originator. It can be known either by pre-provisioning (clause 11.2) or by discovery (clause 10.2.5 for discovery). Discovery of CSEBase is not supported in this release of the document. It is assumed knowledge of CSEBase is by pre-provisioning only.

NOTE 2: 	The term target resource always refers to the resource which is addressed for the specific operation. For example the to of a Create operation for a resource “example” would be “/m2m.provider.com/exampleBase” the to parameter for the operation Retrieve of the same resource “example” is “/m2m.provider.com/exampleBase/example”.

- fr:  Identifier representing the Originator.

NOTE 3:	The fr parameter can be used by the Receiver to check the Originator identity for access privilege verification.

- cn: resource content to be transferred.

- role: optional, required when role based access control is applied <associated text and procedure TBD>

[8.1.2.0a] Editor's Note:  Need to figure out the impact of Role in different procedures.

- op: operation to be executed: Create (C), Retrieve (R), Update (U), Delete (D), Notify (N)

The op parameter shall indicate the operation to be executed at the Receiver:

- Create (C): to is the URI of the target resource where the new resource (parent resource)

- Retrieve (R): an existing to addressable resource is read and provided back to the Originator.

- Update (U): the content of an existing to addressable resource is replaced with  the new content as in cn parameter.

- Delete (D): an existing to addressable resource and all its sub-resources are deleted from the Resource storage.

- Notify (N):  information to be sent to the Receiver, processing on the Receiver is not indicated by the Originator.

The ty parameter shall be present in Request for the following operations:

- Create: ty is the type of the resource to be created.

- cn: resource content to be transferred.

The cn parameter shall be present in Request for the following operations:

- Create: cn is the content of the new resource with the resource type ty.

- Update: cn is the content to be replaced in an existing resource. For the Update operation that is used for Execute operation, cn does not exist.

- Retrieve: cn is the filter to be applied for discovery purposes.

- Notify: cn is the notification information.

Note that the cn parameter can also be empty.

Other allowed parameters shall be as follows:

- nm:  optional name of the resource to be created.

Example usage of the name includes a name that the Originator of the Create resource wishes to be used as the identifier of the newly created resource. For creating a container with the name “myContainer” the request will provide the parameter nm with the value “myContainer” and the created resource would be: /<CSEBase>/myContainer.

- ot: optional originating timestamp of when the message was built.

Example usage of the originating timestamp includes: to measure and enable operation (e.g., message logging, correlation, message prioritization/scheduling, accept performance requests, charging, etc.) and to measure performance (distribution and processing latency, closed loop latency, SLAs, analytics, etc.)

- rqet: optional request message expiration timestamp.

Example usage of the request expiration timestamp includes to indicate when request messages (including delay-tolerant) should expire due to their staleness being no longer of value, and to inform message scheduling/prioritization. When a request has set request expiration timestamp to a specific time and the Request demands an operation on a Hosting CSE that is not the CSE currently processing the request, then the current CSE shall keep on trying to deliver the Request to the Hosting CSE until the request expiration timestamp time, in line with provisioned policies.

- rset: optional result message expiration timestamp.

Example usage of the result expiration timestamp includes to indicate when result messages (including delay-tolerant) should expire due to expected staleness of the result, being no longer of value, and to inform message scheduling/prioritization. It can be used to set the maximum allowed total request/result message sequence round trip deadline.

- rt: optional response message type: Indicates  what the response to the issued request shall contain and when the response is sent to the Originator:

- nonBlockingRequestSynch: In case the request is accepted by the Local CSE, the Local CSE responds after acceptance with an Acknowledgement confirming that the Local CSE will further process the request. The Local CSE includes in the response to an accepted request a reference that can be used to access the status of the request and the result of the requested operation at a later time. Processing of Non-Blocking Requests is defined in clause 8.2.2 and in particular for the synchronous case in clause 8.2.2.2.

- nonBlockingRequestAsynch: In case the request is accepted by the Local CSE, the Local CSE responds after acceptance with an Acknowledgement confirming that the Local CSE will further process the request. The result of the requested operation needs to be sent as a notification to the Originator. Processing of Non-Blocking Requests is defined in clause 8.2.2 and in particular for the asynchronous case in clause 8.2.2.3.

- blockingRequest: In case the request is accepted by the Local CSE, the Local CSE responds with the result of the requested operation after completion of the requested operation. Processing of Blocking Requests is defined in clause 8.2.1.

Example usage of the response type set to nonBlockingRequestSynch: An Originator that is optimized to minimize communication time and energy consumption wants to express a Request to the local CSE and get an acknowledgement on whether the Request got accepted. After that the Originator may switch into a less power consuming mode and retrieve a Result of the requested Operation at a later time.

Further example usage of response type set to nonBlockingRequestSynch includes when the result content is extremely large, or when the result consists of multiple content parts from a target group which are to be aggregated asynchronously over time.

- rc: optional result content: Indicates what are the expected components of the result of the requested operation. The Originator of a request may not need to get back a result of an operation at all. This shall be indicated in the rc parameter. Which exact settings of rc are possible depends on the requested operation specified in op. Possible values of rc are:

- attributes: Representation of the requested resource shall be returned as content, without the URI(s) of the child resource(s). This is the default value. For example, if the request is to retrieve a <container> resource, the URI(s) of the <contentInstance> child-resource(s) is not provided. This setting is not valid for a Notify operation.

- attributes+child-resources: Representation of the requested resource, along with the URI(s) of the child resource(s), possibly limited by a maximum number of retrieved links, shall be returned as content. For example, if the request is to retrieve a <container> resource, the <container> resource and the URI(s) of the <contentInstance> child-resource(s) are provided. This setting is not valid for a Notify operation

- child-resource: URI(s) of the child resources, possibly limited by a maximum number of retrieved URI(s), without any representation of the actual requested resource shall be returned as content. For example, if the request is to retrieve a <container> resource, only the URI(s) of the <contentInstance> child-resource(s) is provided. This setting is not valid for  a Notify operation

- nothing: Nothing shall be returned as operational result content. This setting is not valid for a retrieve operation. This setting is the default when a Notification was requested by the op parameter.

- announced-resource: Representation of the announced resource at local CSE shall be returned without the URI(s) of the child resource(s). If such information is not available at the local CSE, information at original resource Hosting CSE as per attribute setting shall be returned. This option shall not be used if the optional parameter fc (filter criteria) is included. This setting is not valid for a Notify operation.

[8.1.2.f0] Editor's Note: Example usage of all rc enumerated values needed. What is the use case for "nothing"?

[8.1.2.f1] Editor's Note: Do we need to support use case for "no response expected" in this release.

- rp: optional response persistence: indicates the duration for which the address containing the responses is to persist.

Example usage of response persistence includes requesting sufficient persistence for analytics to process the response content aggregated asynchronously over time. If a result expiration timestamp is specified then the response persistence should last beyond the  result expiration time.

- ri: request Identifier

Example usage of request identifier includes enabling the correlation between a Request and one of the many received Responses.

- oet: optional operation execution time: indicates the time when the specified operation op is to be executed by the target CSE. A target CSE shall execute the specified operation of a Request having its operational execution time indicator set, starting at the operational execution time. If the execution time has already passed or if the indicator is not set, then the specified operation shall be immediately executed, unless the request expiration time, if set, has been reached.

Example usage of operational execution time includes asynchronous distribution of flows, which are to be executed synchronously at the operational execution time.

NOTE 4:	Time-based flows could not supported depending upon time services available at CSEs.

- ec: optional event category: Indicates the event category that should be used to handle this request. Event categories are impacting how Requests to access remotely hosted resources are processed in the CMDH CSF. Selection and scheduling of connections via CMDH are driven by policies that can differentiate event categories.

Example usage of "event category" set to specific value X: When the request is demanding an operation to be executed on a Hosting CSE that is different from the Registrar CSE, the request may be stored in the CSE that is currently processing the request on the way to the Hosting CSE until it is allowed by provisioned policies for that event category X to use a communication link to reach the next CSE on a path to the Hosting CSE or until the request expiration timestamp is expired.

- da: optional delivery aggregation on/off: Use CRUD operations of <delivery> resources to express forwarding of one or more original requests to the same target CSE(s).

NOTE 5:	Since da is optional, there could be a default value to be used when not present in the Request. This parameter could not be exposed to AEs via Mca.

Example usage of delivery aggregation set on: The CSE processing a request shall use aggregation of requests to the same target CSE by requesting CREATE of a <delivery> resource on the next CSE on the path to the target CSE.

- gid: optional group request identifier: Identifier added to the group request that is to be fanned out to each member of the group.

- fc:  optional filter criteria: conditions for filtered retrieve operation are described in Table 8.1.2-1.

- Disrestype: Optional Discovery result type. This parameter applies to discovery related requests (see clause 10.2.5) to indicate the preference of the Originator for the format of returned information in the result of the operation. This parameter shall take on one of the following values reflecting the options in clause 9.3.1:

- Hierarchical URI option

- Non-hierarchical URI option

- CSE-ID, and resource identifier option

- Example usage of the discovery result type set to Non-hierarchical URI option. The request originator is only willing to receive non-hierarchal URIs for discovered resources.

- The absence of the parameter implies that the result shall be in Hierarchical URI format.

[8.1.2.g]  Editors Note: This may be impacted by the management of access control policy which will be enhanced to take into account Hierarchical and non-Hierarchical URI.

Rapportur Note: Discussions during ARC#10bis.  Contribution expected for the resolution of the above stated Editor's Note.

Table 8.1.2-1: filterCriteria conditions

| Condition tag | Multiplicity | Matching condition |
| --- | --- | --- |
| createdBefore | 0..1 | The creationTime attribute of the resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically before the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the resource is chronologically after the specified value. |
| labels | 0..n | The labels attributes of the resource matches the specified value. |
| resourceType | 0..n | The resourceType attribute of the resource is the same as the specified value. It also allows discriminating between normal and announced resources. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> resource is smaller than the specified value. |
| contentType | 0..n | The typeOfContent attribute of the <contentInstance> resource matches the specified value |
| limit | 0..1 | Limitation the number of matching resources to the specified value. |
| attribute | 0..n | Attribute token name to be provided to reduce the scope of filtering to resource attributes. |

Example usage of filter criteria conditions in a HTTP query: an HTTP GET operation can be requested applying also a filter in the query part of the request itself:

GET /root?label=one&label=two&createdBefore=2014-01-01T00:00:00&limit=128

The example discovers a maximum of 128 resources matching the following logical condition: createdBefore < 2014-01-01T00:00:00 AND (label = one OR label = two).

Once the Request is delivered, the Receiver shall analyze the Request to determine the target resource.

If the target resource is addressing another M2M node, the Receiver shall route the request appropriately.

If the target resource is addressing the Receiver, it shall:

- Check the existence of to addressed resource.

- Identify the resource type by ty.

- Check the privileges for fr Originator to perform the requested operation.

- Perform the requested operation (using cn content when provided) according to the provided request parameters as described above.

- Depending on the request result content, respond to the Originator with indication of successful or unsuccessful operation results. In some specific cases (e.g., limitation in the binding protocol or based on application indications), the Response could be avoided.

The message flow procedure started with an Originator Request message shall be considered closed when either:

- A Request message is expired according to the rqet (request expiration timestamp).

- A Response message is delivered to the Originator.

#### 8.1.2.1	Summary of Request Message Parameters

Table 8.1.2.1-1summarises the parameters specified in clause 8.1.2 for the Request message, showing any differences as applied to C, R, U, D or N operations. "M" indicates mandatory, "O" indicates optional, "N/A" indicates "not applicable".

- Table 8.1.2.1-1: Summary of Request Message Parameters

| Request message parameter \ Operation | Create: a new resource is created | Retrieve: an existing resource is read and the result is provided | Update: the content of an existing resource is replaced with new content | Delete: an existing resource and all its sub-resources are deleted | Notify: the content / address is transferred |
| --- | --- | --- | --- | --- | --- |
| Operation (op) - operation to be executed | M | M | M | M | M |
| To (to) – the address of the target resource on the target CSE | M | M | M | M | M |
| From (fr) – the identifier of the message Originator | M | M | M | M | M |
| Request Identifier (ri) – uniquely identifies a Request message | M | M | M | M | M |
| Resource Type (ty) - of resource to be created | M | N/A | N/A | N/A | N/A |
| Name (nm) - of resource to be created | O | N/A | N/A | N/A | N/A |
| Content (cn) - to be transferred | O | O | O | N/A | M |
| Originating Timestamp (ot) - when the message was built | O | O | O | O | O |
| Request Expiration Timestamp (rqet) – when the request message expires | O | O | O | O | O |
| Result Expiration Timestamp (rset) -when the result message expires | O | O | O | O | O |
| Operational Execution Time (oet) - the time when the specified operation is to be executed by the target CSE. | O | O | O | O | O |
| Response Type (rt) – Registrar CSE response shall either indicate that the Request was accepted, or include the operation result | O | O | O | O | O |
| Result Destination (rd) – either a reference to where the result is stored, or the result is sent to the Originator | O | O | O | O | N/A |
| Result Persistence (rp) - the duration for which the reference containing the responses is to persist | O | O | O | O | N/A |
| Result Content (rc) – the expected components of the result | O | O | O | O | N/A |
| Event Category (ec) – indicates how and when the system should deliver the message | O | O | O | O | O |
| Delivery Aggregation (da) - aggregation of requests to the same target CSE is to be used | O | O | O | O | O |
| Group Request Identifier (gid) - Identifier added to the group request that is to be fanned out to each member of the group. | O | O | O | O | O |
| Filter Criteria (fc) – conditions for filtered retrieve operation | N/A | O | N/A | N/A | N/A |
| Discovery Result Type (Disrestype) - format of information returned for Discovery operation | N/A | O | N/A | N/A | N/A |

### 8.1.3	Response

The Response received by the Originator of a Request accessing resources over the Mca and Mcc reference points shall contain mandatory and may contain optional parameters. Certain parameters may be mandatory or optional depending upon the Requested operation (CRUDN) or the mandatory response code. In this clause, the mandatory parameters are detailed first, following by those that are conditional, followed by those that are optional:

Mandatory Parameters:

- rs: response code: This parameter indicates whether the operation was successful, unsuccessful or is an acknowledgement

- A “successful” code indicates to the Originator that the Requested operation has been executed successfully by the Hosting CSE.

- An “unsuccessful” code indicates to the Originator that the Requested operation has not been executed successfully by the Hosting CSE.

- An “acknowledgement” indicates to the Originator that the Request has been received and accepted by the attached CSE, i.e., by the CSE that received the Request from the issuing Originator directly, but the Request operation has not been executed yet. The success or failure of the execution of the Requested operation is to be conveyed later.

- Details of successful, unsuccessful, and acknowledge codes are provided in the oneM2M Core Protocol Technical Specification (TS-0004)  [i.2].

- ri: Request Identifier.  The ri in the Response shall match the ri in the corresponding Request.

Conditional Parameters:

- cn: resource content:

- If rs is “successful” then

- The cn parameter may be present in a Response in the following cases:

- Create: cn is the address and/or the content of the created resource.

- Update: cn is the content replaced in an existing resource.

- Delete: cn is the content actually deleted.

- The cn parameter shall be present in a Response in the following cases:

- Retrieve: cn is the retrieved resource content or aggregated contents of discovered resources.

If present in the Request, result contents rc, indicates which components of the result of the requested operation are to be included in the Response.

- If rs is “unsuccessful” then cn may be present in a Response to provide more error information.

- If rs is “acknowledgment” then cn is not present

- ra: address for the temporary storage of end node Responses. Present only for non-blocking as indicated by the Request parameter rt.

Optional parameters:

- to: ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request, "rd" information can be used.

- fr: ID of the Receiver.

- ot: originating timestamp of when the message was built.

- rset: result expiration timestamp. The Receiver shall echo the result expiration timestamp if set in the Request message, or may set the result expiration timestamp itself.

Example usage of the Receiver setting the result expiration timestamp is when the value of the delivery time is dependent upon some changing Receiver context e.g., Result message deadline for aircraft position based upon velocity.

- cs: status codes (e.g., authorization timeout, etc.).

### 8.1.4	Summary of Response Message Parameters

Table 8.1.4-1 summarises the parameters specified in clauses 1.3 for the Response messages, showing any differences as applied to successful C, R, U, D or N operations, and unsuccessful operations. "M" indicates mandatory, "O" indicates optional, "N/A" indicates "not applicable".

- Table 8.1.4-1: Summary of Response Message Parameters

| Response message parameter \ success or not | Rs = Ack | Rs = successful: Op = Create: a new resource is created | Rs= successful: Op= Retrieve: an existing resource is read and the result is provided | RS= successful. Op= Update: the content of an existing resource is replaced with new content | RS= successful. Op = Delete: an existing resource and all its sub-resources are deleted |  | Rs= successful Op= Notify: the content / address is transferred |  | RS = unsuccessful Op = C,R,U,D or N |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Response Code (rs) - successful, unsuccessful, ack | M | M | M | M |  | M |  | M |  | M |
| Request Identifier (ri) – uniquely identifies a Request message | M | M | M | M |  | M |  | M |  | M |
| Content (cn) - to be transferred | N/A | O (The address and/or the content of the created resource) | M (the retrieved resource content or aggregated contents of discovered resources) | O (The content replaced in an existing resource) |  | O (The content actually deleted) |  | N/A |  | O (Additional error info) |
| To (to) –the identifier of the Originator | O | O | O | O |  | O |  | O |  | O |
| From (fr) – the identifier of the Receiver | O | O | O | O |  | O |  | O |  | O |
| Originating Timestamp (ot) - when the message was built | O | O | O | O |  | O |  | O |  | O |
| Result Expiration Timestamp (rset) – when the message expires | O | O | O | O |  | O |  | N/A |  | O |
| Status codes (cs) - (e.g., authorization timeout, etc.) | O | O | O | O |  | O |  | O |  | O |
| Response Address (ra) - address for the temporary storage of end node Responses. | O | O | O | O |  | O |  | N/A |  | N/A |

## 8.2	Procedures for Accessing Resources

This clause describes the procedures for accessing the resources. The term "hop" in the descriptions here refers to the number of CSEs that forward a request from the Originator CSE to the Hosting CSE.

### 8.2.1	Accessing Resources in CSEs - Blocking Requests

For the procedures described herein, the addressed resource can be stored in different CSEs. Table 8.2.1-1 describes the possible scenarios, where the addressed resource may be on the Registrar CSE or on a CSE located elsewhere in the oneM2M System.

In this sub-clause - for simplicity - it is assumed that the Originator of a Request can always wait long enough to get a Response to the Request after the requested operation has finished. This implies potentially long or unknown blocking times (time for which a pending Request has not been responded to) for the Originator of a Request.

For scenarios that avoid such possibly long blocking times, clause 8.2.2 specifies mechanisms to handle synchronous and asynchronous resource access procedures via returning appropriate references.

- Table 8.2.1-1: Accessing Resources in different CSEs

| Traversals across Mca/Mcc Reference Points | Description | Reference |
| --- | --- | --- |
| No Hops | - The Originator of the Request accesses a resource. - The Originator of the Request can be an AE or a CSE. - Registrar CSE and Hosting CSE are the same entity. - The CSE shall check the Access Control Privileges for accessing the resource. - Depending on the expected result content, the CSE shall respond to the Originator of the Request, either with a Success or Failure Response | Figure 8.2.1-1 |
| 1 Hop | - The Originator of the Request accesses a resource. - The Originator of the Request may be an AE or a CSE. - Registrar CSE and Hosting CSEs are different entities. - Registrar CSE shall forward the Request to the Hosting CSE if the Registrar CSE is registered with the Hosting CSE, for accessing the resource - Hosting CSE shall check the Access Control Privileges for accessing the resource and depending on the expected result content respond with a Success or Failure Response. | Figure 8.2.1-2 |
| Multi Hops | - The Originator of the Request accesses a resource. - The Originator of the Request may be an AE or a CSE. - Registrar CSE, Transit CSE(s) and the Hosting CSE are different entities. - Registrar CSE shall: - Forward the Request to a Transit-1 CSE (e.g., MN-CSE) that the Registrar CSE is registered with, if configured through policies to do so or; - Forward the request to an IN-CSE if the Registrar CSE is registered with IN-CSE and if configured through policies to do so. - Transit-1 CSE shall: - Forward the request to the Hosting CSE if it is registered with the Hosting CSE or - Forward the Request to another Transit-2 CSE (e.g., another MN-CSE) that the Transit-1 CSE is registered with. or - Forward the request to an IN-CSE if the Transit-1 CSE is registered with the IN-CSE. - The IN-CSE shall: - Forward the request to another IN-CSE if the resource belongs to another M2M SP or - Forward the request to the Hosting CSE if the latter is known (e.g. announcements) by the IN-CSE. - Hosting CSE shall check the Access Control Privileges for accessing the resource and depending on the expected result content respond with a Success or Failure Response. | Figure 8.2.1-3 |

- Figure 8.2.1-1: Originator accesses a resource on the Registrar CSE (No Hops)

- Figure 8.2.1-2: AE/CSE accesses a resource at the Hosting CSE (One Hop)

- Figure 8.2.1-3: Originator accesses a resource at the Hosting CSE (Multi Hops)

#### 8.2.1.1 	M2M Requests Routing Policies

CSEs can use policies to govern routing of M2M requests to the next hop towards its target. Routing, through these policies, can be based, for example, on the target CSE, target M2M domain, specific types of resources if applicable, priority of a request, etc.

These policies are not defined in this release of this document. It is the responsibility of M2M SP and the CSE administrator to ensure the appropriateness of these policies for routing purposes

### 8.2.2	Accessing Resources in CSEs - Non-Blocking Requests

#### 8.2.2.1	Response with Reference to Result of Requested Operation

In case the Originator of a Request has asked for only a response with an Acknowledgement of the Request and a reference to the result of the requested operation - see rt parameter in clause 8.1.2 - it is necessary to provide a prompt response to the Originator with a reference to an internal resource on the Registrar CSE or another specified reference, so that the Originator can retrieve the outcome of the requested operation at a later time. The details of such an internal resource are defined in clause 9.6.11. The reference is provided in the response to the Request. The abbreviation "Req-Ref" is used for simplicity in the figures of the following clauses.

Two different cases to allow the Originator of a request to retrieve the result of a requested operation are defined in the following two clauses.

#### 8.2.2.2	Synchronous Case

In the synchronous case, it is assumed that the Originator of a Request is not able to receive asynchronous messages, i.e., all exchange of information between Originator and Registrar CSE needs to be initiated by the Originator.

In that case the information flow depicted in figure 8.2.2.2-1 is applicable. For the flow depicted in figure 8.2.2.2-1 it is assumed that completion of the requested operation happens before the Originator is trying to retrieve the result of the requested operation with a second request referring to the "Req-Ref" provided in the Response to the original Request.

Another variation of the information flow for the synchronous case is depicted in figure 8.2.2.2-2. In this variation it is assumed that the requested operation completes after the second request but before the third request sent by the Originator.

Equivalent information flows are valid also for cases where the target resource of the requested operation is not hosted on the Registrar CSE. From an Originator's perspective there is no difference as the later retrieval of the result of a requested operation would always be an exchange of Request/Response messages between the Originator and the Registrar CSE using the reference to the original request.

Figure 8.2.2.2-1: Non-blocking access to resource in synchronous mode
(Hosting CSE = Registrar CSE), requested operation completed before second request

Figure 8.2.2.2-2: Non-blocking access to resource in synchronous mode
(Hosting CSE = Registrar CSE), requested operation completed after the second
but before the third request

#### 8.2.2.3	Asynchronous Case

In the asynchronous case, it is assumed that the Originator of a Request is able to receive notification messages, i.e., the Registrar CSE could send an unsolicited message to the Originator at an arbitrary time to send the result to a notification target. The possible mechanisms for the notification to reach the Originator are the same as in the case of a notification after a subscription.

In that case the information flow depicted in figure 8.2.2.3-1 is applicable. In this case it is assumed that the Originator of the Request provided a reference - notificationReference - for notification when the result of the requested operation is available.

Equivalent information flows are valid also for cases where the target resource of the requested operation is hosted on the Registrar CSE. From an Originator's perspective there is no difference as the later notification of the result of a requested operation would always be an exchange of request/response messages between the Originator and the Registrar CSE using reference to the original Request.

Figure 8.2.2.3-2: Non-blocking access to resource in notification mode 
(Hosting CSE not equal to Registrar CSE), Originator provided reference for notification

## 8.3	Description and Flows on Mcn Reference Point

Communications between the CSEs and the NSEs across the Mcn reference point and includes:

- The CSE(s) accessing network service functions provided by Underlying Networks, and

- Optimizing network service processing for Underlying Networks.

Such services normally are more than just the general transport services.

Communications which pass over the Mcn reference point to Underlying Networks include:

- Messaging services that are widely deployed by Applications and network operators using a number of existing mechanisms.

- Network APIs defined by other SDOs (e.g., OMA and GSMA) are used by network operators for their services.

- Interworking for services and security aspects for MTC (Machine Type Communications) has been defined by 3GPP.

Examples of service requests from a CSE towards the Underlying Networks are:

- Connection requests with/without QoS requirements.

- Payments, messages, location, bearer information, call control and other network capabilities (e.g., by using GSMA oneAPI, network APIs supporting protocols defined by other SDOs, or proprietary network APIs).

- Device triggering.

- Device management.

- Management information exchange such as charging/accounting records, monitoring and management data exchange.

- Location request.

## 8.4	Device Triggering

### 8.4.1	Definition and scope

Device Triggering is a means by which a node in the infrastructure domain (e.g., IN-CSE) sends information to a node in the field domain (e.g., ASN-CSE) to perform a specific task, e.g., to wake up the device, to establish communication from the field domain towards the infrastructure domain, or when IP address for the device is not available or reachable by the infrastructure domain.

Underlying Network functionality is used to perform device triggering for example, using alternate means of communication (e.g., SMS) with the Field Node.

NOTE: Device Triggering is applicable for the entities which are registered with IN-CSE.

Each Underlying Network type may provide different way of performing a device triggering, for example 3GPP has defined a dedicated interface for requesting device triggering. The normative references for applicable interfaces are as follows: 3GPP TS 23.682 [i.17], 3GPP2 X.S0068 [i.20]. Access specific mechanisms are covered in the Annexes B and C.

### 8.4.2	General Procedure for Device Triggering

This clause covers different scenarios for device triggering.

#### 8.4.2.1	Triggering procedure for targeting ASN/MN-CSE

This case describes the scenario where IN-CSE targets an ASN/MN-CSE (which is registered with the IN-CSE) for the Device Triggering request.

The Figure 8.4.2 1-1 shows the general procedure for Device Triggering and, if required, for establishment of connectivity between IN-CSE and the Field Node.

Figure 8.4.2 1-1: Device Triggering general procedure for CSE

NOTE:     The IN and ASN/MN are assumed to be connected through the same Underlying Network.

NOTE:     The Device Triggering Handler is a functional entity that receives the device triggering request, and it is dependent on the Underlying Network. The Device Triggering Handler is out of scope of this specification.

Pre-condition

The CSE which is the target of the device triggering has to be registered with the IN-CSE.

The CSE-PoA for the ASN/MN-CSE already contains either an IP address or none.

[optional] Step-1: Request to targeted ASN/MN-CSE

The IN-AE requests to perform one of the CRUD operations on a resource residing on the ASN/MN-CSE, the request is sent via the Mca reference point to the IN-CSE.

Step-2: Underlying network selection

The IN-CSE selects the Underlying Network and the mechanism to deliver the triggering request to the Underlying Network according to the configuration for connected Underlying Networks.

For example for 3GPP access network IN-CSE can use Tsp, Tsms and GSMA OneAPI, but the preferred mechanism is Tsp.

Step-3: Device Triggering request

IN-CSE issues the device triggering request to the selected Underlying Network.

NOTE: The Underlying Network dependent Device Triggering procedure for 3GPP and 3GPP2 systems are described in Annexes B and Annex C respectively.

Some information provided to the selected Underlying Network for performing device triggering includes:

- M2M-Ext-ID associated with the ASN/MN-CSE as the target of the triggering request (see 7.1.8).

- Trigger-Recipient-ID associated with the ASN/MN-CSE (see clause 7.1.10). For example when 3GPP Underlying Network is used this identifier could map to Application-Port-ID.

- IN-CSE ID which could be used by the Underlying Network to authorize the IN-CSE for device triggering.

NOTE: The above Trigger-Recipient-ID is sent at registration.

[8.4.2.1.a]  Editor’s Note: M2M-Ext-ID is set on registration, how the IN-CSE gets it is FFS.

Step-4: Underlying Network Specific Device Triggering procedure

Device Triggering processing procedure is performed between the Underlying Network and the target Node which hosts the ASN/MN-CSE.

Step-5: Device Triggering response

The IN-CSE receives a response for the Device Triggering request via the Mcn reference point.

Step-6: ASN/MN-CSE Receives Device Trigger

[optional] Step-7 Connection establishment

In case that it is required by the Device Triggering request, connectivity is established between the ASN/MN-CSE and the IN-CSE and the renewal of the CSE-PoA might be needed.

## 8.5	Location Request

### 8.5.1	Definition and Scope

Location Request is a means by which a CSE requests the geographical or physical location information of a target CSE or AE hosted in a M2M Node to the location server located in the Underlying Network over Mcn reference point. This clause describes only the case of location request when the attribute locationSource is set to Network Based.

### 8.5.2	General Procedure for Location Request

This procedure describes a scenario wherein an AE or a CSE sends a request to obtain the location information of a target AE or CSE hosted in an M2M Node to the location server NSE, and the location server responses to the CSE with location information.

The Figure 8.5.2.1-1 shows the general procedure for Location Request.

Figure 8.5.2 1-1: General Procedure for Location Request

NOTE: Detailed descriptions for Step-1 to the Step-3 are described in the clause 10.2.10. 1.

Step-1: Create <locationPolicy>

The Originator requests to CREATE <locationPolicy> resource at the Registrar CSE. The locationSource attribute of the <locationPolicy> resource shall be set to ‘Network-Based’ and the value for locationTargetID and locationServer attributes shall be set properly set for the Location Request.

Step-2: Local Processing for creating <locationPolicy> resource

After verifying the privileges and the given attributes, the Registrar CSE shall create the <locationPolicy> resource. Linked <container> resource can be created after successful creation of <locationPolicy> resource.

Step-3: Response for creating <locationPolicy>

The Registrar CSE shall respond with a Response message.

Step-4: Location Request

The Registrar CSE issues Location Request to the selected Underlying Network. For doing this, the local CSE shall transform the location configuration information received from the Originator into Location Request that is acceptable for the Underlying Network. For example, the Location Request can be one of existing location acquisition protocols such as OMA Mobile Location Protocol [i.7] or OMA RESTful NetAPI for Terminal Location [i.8]. Additionally, the Registrar CSE shall provide default values for other parameters (e.g., required quality of position) in the Location Request according to local policies.

NOTE 1:    The Location Request can be triggered by the given conditions, e.g.,: 1) when the locationUpdatePeriod attribute has expired, or if the locationUpdatePeriod attribute is not given from the Step-1, 2) the <locationPolicy> is created or 3) the linked <container> has been retrieved.

Step-5: Performing Location Procedure

The Underlying Network specific procedures are performed. This may involve getting location information from the target device or the network node. These procedures are outside the scope of oneM2M specifications.

Step-6: Location Response

The NSE responds to the local CSE with location information if the local CSE is authorized. If not, the NSE sends an error code back to the local CSE.

Step-7: Local Processing after Location Response

The received response shall be contained in the <container> resource that is related the <locationPolicy> resource.

NOTE 2:  Please see the clause 10.2.10.2 for detail information.

NOTE 3: For notification regarding the location response towards the Originator, the subscription mechanism is used

## 8.6 	Connection Request

Connection request service is not defined in the present document.

## 8.7 	Device Management

See clause 6.2.4 for a detailed description on the interaction with a Device Management

# 9	Resource Management

All entities in the oneM2M System, such as AEs, CSEs, "data", etc., are represented as "resources". A "resource structure" is specified as a representation of the "resources". Such "resources" are uniquely addressable. Procedures for accessing such resources are also specified.

## 9.1	General Principles

The following are the general principles for the design of the resource model.

- The "type" of each resource shall be specified. New resource types shall be supported as the need for them is identified.

- The root of the resource structure in a CSE shall be assigned an absolute address. See clause 9.3.1 for additional information.

- The attributes for all resource type shall be specified.

- Each resource type can have multiple instances.

- All resource and associated attributes shall be uniquely addressable via their associated Universal Resource Identifiers (URI), consisting of:

- a hierarchical URI based on the chain of child-parent relations;

- a non-hierarchical URI made of a unique identifier addressable via the CSEBase.

- Examples:

- Root example "myCSE" (well known, common to hierarchical and non-hierarchical).

EXAMPLE 1:	The following two examples address the same resource:

- //myCSE/123(hierarchical).

- //myCSE/234 (hierarchical).

EXAMPLE 2:	The following two examples address the same resource:

- //myCSE/myApplication/987 (hierarchical).

- //myCSE/234 (non-hierarchical).

- The non-hierarchical URI is always stored in the parentID attribute, that is a mandatory attribute.

- In case of hierarchical URI, the relationship parent-child and vice-versa is determined in the hierarchical URI. When created via the non-hierarchical URI, the parent child-relation is determined by the parentID attribute.

- Both hierarchical and non-hierarchical URIs shall be supported by all CSEs.

## 9.2	Resources

This clause introduces the resources used in a CSE. A resource scheme is used for modelling the resource structure and associated relationships. Clause 9.5 provides guidelines on how to describe a resource. This document identifies three categories of resources:

- Normal resources (clause 9.2.1)

- Virtual resources (clause 9.2.2)

- Announced resources (clause 9.2.3)

### 9.2.1	Normal Resources

Normal resources include the complete set of representations of data which constitutes the base of information to be managed.

Unless qualified as either "virtual" or "announced", the resource types in this document are normal resources.

### 9.2.2	Virtual Resources and Attributes

A virtual resource or a virtual attribute is used to trigger processing and/or retrieve results, but they do not have a permanent representation in a CSE.

- 9.2.3	Announced Resource

An announced resource is a resource at a remote CSE that is linked to the original resource that has been announced, and it maintains some of the characteristics of the original resource.

Resource announcement can facilitate resource discovery. The announced resource at a remote CSE can also be used for creating child resources at the remote CSE that are not present as children of the original resource or are not announced children of the original resource.

The following are the resource specification guidelines for the resource announcement:

- In order to support announcement of resources, an additional column in the resource template (clause 9.5.1), shall specify the attributes to be announced for inclusion in the associated announced resource type.

- For each announced <resourceType>, the addition of suffix "Annc" to the original <resourceType> shall be used to indicate its associated announced resource type. For example, resource <containerAnnc> shall indicate the announced resource type for resource type <container>; <groupAnnc> shall indicate announced resource type for resource type <group>, etc.

## 9.3	Resource Addressing

### 9.3.1	Generic Principles

There are three different means for addressing a resource within the oneM2M resource structure. They are as follows:

- The resource can be addressed by a URI, over Mca, Mcc, and Mcc',  with the actual path portion of the URI defining the entire relationship for the target resource within the resource structure. This is a structured representation of the resources within a CSE where the parent relationship chain is embedded in the resource address. Example:  IN-CSEID.m2m.myoperator.org/CSERoot/myAppX/myContainerY

- The resource can be addressed by a URI, over Mca, Mcc, and Mcc', with the actual path portion of the URI only indicating the actual target resource within the resource structure, but without explicitly embedding the parent-child relationship. Hence the actual parent relationship chain is not known a priori and the hosting CSE needs to resolve the logical location of the target resource in the chain of relationship within the resource structure. Example:  "node2.m2m.myoperator.org/CSERoot/myContainerY” where the same container of the previous example is directly addressed

- The resource can be addressed via two parameters over Mca;  namely the CSE-ID of the host where the resource is located, and/or the resource identifier of the actual target resource. This case is applicable only to intra-domain routing. To proxy this request onwards over the Mcc interface, if applicable, the Registrar CSE can proxy the request to the IN node including these two parameters ; optionally the Registrar CSE shall use the CSE-ID to generate the complete target host name in accordance with the rules define in clause 6.4.1.1.

- Note in this scenario it is assumed that the actual node identifiers are used in the host name to enable the generation of the host name. Furthermore, it is assumed in this case that resource identifiers allocated by the host are unique, otherwise the resource identifier full path has to be provided. If the resource identifier full path is provided then there is no need for the CSE-ID to be sent to the IN. The CSEBase is used to identify the CSE-ID

- This case is an optimization of the second case, since the host name included in the URI has to be generated, as an option, in this case by the Registrar CSE before it can send the request to the target CSE (via the IN node). Example: the same resource addressed in the previous bullet will be provided by the requesting entity to the receiving CSE by means of the identifiers “CSE4-CSEID” and “myContainerY” and then it will be resolved to "CSE4-CSEID.IN-CSEID.m2m.myoperator.org/CSERoot/myContainerY”

These 3 methods shall all be supported by all M2M nodes, notably the Registrar CSEs receiving requests, before they proxy these requests any further, where applicable.

#### 9.3.1.1	Support for Multiple CSEs within a single Node

For nodes that support multiple CSEs, the CSEBase, which is the first element in the path, shall be equivalent to the CSE-ID. This allows the host to easily distinguish the different CSEs.

Note in this case the host name resolves to the same IP address.

#### 9.3.2 	Resource addressing via a FQDN URI

Resources shall be uniquely addressable via FQDN URI as one of the methods in clause 9.3.1. The following are examples of addressable resources.

- /<CSEBase>/Entity Instance 1

- /<CSEBase>/Entity Instance n

- /<CSEBase>/Application Instance 1

- /<CSEBase>/Application Instance n

- /<CSEBase>/Container1/Container2

The <CSEBase> shall contain all resources of the hosting node. A <CSEBase> is the root of all other resources it contains. The <CSEBase> shall represent the first element of the path representing a resource in an absolute URI, as per [i.12].

The authority part of the absolute URI, as per [i.14] and [i.12] shall be either an Infrastructure Node public domain name, a Middle Node public domain name or an Application Service Node public domain name and shall conform to the naming convention described in clause 6.4.1.1.

## 9.4	Resource Structure

### 9.4.1	Links between Resources

- Figure 9.4.1-1: Topological Resource Structure at a CSE

[9.4.1.a]  Editor's Note:  The resource structure picture needs to be updated as the resource structure evolves.

As represented in the figure above, resources in the oneM2M System may be linked with each other via unidirectional or bidirectional links.

NOTE:	A resource can also be linked using an attribute containing the linked resource ID.

A link shall contain the following information:

- linkedResourceURI: The target linked resource, usually given by using the URI of that resource.

- linkType: The type of the link, describing the relationship that the current resource has with the other linked resource (only in one direction, i.e., from this resource to the other linked resource).

A link may also contain the following information:

- linkedResourceType: The type of the target linked resource.

[9.4.1.b]  Editor's Note: Links management (creation, deletion, etc.) is FFS.

### 9.4.2	Link Types

The following links are defined:

Table 9.4.2-1: Link Types

| Linked Resource Type (link destination) | Linking Resource Type (link origin) | Linking method | Description |
| --- | --- | --- | --- |
| accessControlP | Several (node, application, remoteCSE, container) | Attribute named “accessControlPolicyID” | See clause 9.6.2 |
| node | CSEBase | Attribute named “nodeID” OR Child resource of type “node” | See clause 9.6.18 |
| CSEBase or remoteCSE | node | Attribute named “hostedCSEID” OR parent resource of type CSEBase | See clause 9.6.3 See clause 9.6.4 |
| a parent resource of any resourceType | a child resource of any resourceType | Attribute named “parentID” | See clause 9.6.1 |
| a child resource of any resourceType | a parent resource of any resourceType | Child resource of a specific type | See clause 9.6.5 |

[9.4.2.a]  Editor’s Note: There are more links to define in this Table to reflect the current state of the TS.

## 9.5	Resource Type Specification Conventions

Rapporteur Note:  Should we move this clause under Conventions (clause 4).

The following conventions are be used for the specification of resources.

Resources are specified by a tabular notation and the associated graphical representation as follows:

- The resources are specified in association with a CSE. The resources are the representation in the CSE of the components and elements within the M2M System. Other CSEs, AEs, application data representing sensors, commands, etc. are known to the CSE by means of their resource representation. Resource, Child Resource and Attributes are defined in clause 3.1 and are restated below for readability.

- Resource: A Resource is a uniquely addressable entity in oneM2M architecture. A resource is transferred and manipulated using  CRUDN operations (see clause 10.1). A resource can contain child resource(s) and attribute(s).

- Child Resource: It is a sub-resource of another resource that is its parent resource. The parent resource contains references to the child resources(s).

- Attribute: Stores information pertaining to the resource itself.

- The set of attributes, which are common to all resources, are not detailed in the graphical representation of a resource.

- When an instance of a resource is created, a URI is assigned. URI is used to uniquely address the resource.

- Resources and attributes are addressed in the same manner, e.g., by the use of the URI.

Rapporteur Note:  Move the above two bullets to some other clause - such clause TBD. These do not belong to the conventions,

- Resource names and attribute names are strings in lower case. In case of a composed name, the subsequent word(s) start with a capital letter; e.g., accessControlPolicy, creationTime, expirationTime.

- Resource type names and attribute names are written in italic form in this document.

- A string delimited with '<' and '>' e.g., '<resourceType>' is a placeholder for the type of a resource.

- A string delimited with '[' and ']' e.g. '[resourceName]' is a placeholder for the name of a resource or an attribute.

The resources are specified as shown below:

Figure 9.5-1: <resourceType> representation convention

The resource specification provides the graphical representation for the resource as the Figure 9.5-1. The graphical representation of a resource shows the multiplicity of the attributes and child resources. The set of attributes, which are common to all resources are not detailed in the graphical representation of a resource. The following graphical representations are used for representing the attributes and child resources:

- Square boxes are used for the resources;

- Square boxes with round corners are used for attributes.

Child resources in a <resourceType> are detailed as shown in Table 9.5-1.

The child resource table for an announce-able <resourceType> resource shall include an additional column titled '<resourceTypeAnnc> Child Resource Types', indicating the type of announced resources. See the clause 9.6.25 for further details.

An announced resource may have child resources, such child resources can be of type normal or announced. When a resource is announced, the associated child resources are announced independently of the original resource, as needed by the resource announcing CSE. The child resources at the announced resource are of the child resource Announced type.

When child resources at the announced resource are created locally by the remote CSE, the child resources are of normal child resource type.

- Table 9.5-1: Child Resources of <resourceType>

| Child Resource Name of <resourceType> | Child Resource Type | Multiplicity | Description | <resourceTypeAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| <Fill in the name of Child Resource1 if a fixed name is required or [variable] if no fixed name is required> | <Fill in the type of Child Resource1> | <Fill in Multiplicity> | See clause <XRef> <clause> where the type of this child resource is described. | <Fill the child resource type for the announced resource. It can be none or <crTypeAnnc> or <crType>; where the <crType> is the child resource type of the original Child Resource1. |
| <Fill in the name of Child ResourceN if a fixed name is required or [variable] if no fixed name is required> | <Fill in the type of Child ResourceN> | <Fill in Multiplicity> | See clause <XRef> <clause> where the type of this child resource is described. | <Fill the child resource type for the announced resource. It can be none or <crTypeAnnc> or <crType>; where the <crType> is the child resource type of the original Child ResourceN. |

Attributes in a <resourceType> are detailed as shown in Table 9.5-2

The attributes table for announce-able <resourceType> resource includes an additional column titled 'Attribute for <resourceTypeAnnc>', indicating the attributes that are to be announced for that <resourceType>. See the clause 9.6.25 for further details.

- Table 9.5-2: Attributes of <resourceType>

| Attribute Name of <resourceType> | Multiplicity | RW/ RO/ WO | Description | <resourceTypeAnnc> (MP/OP/NP) |
| --- | --- | --- | --- | --- |
| <Fill in name of Common Attribute1> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute - to be moved later to a common attribute clause. | <Fill in MA or OA or NA> |
| <Fill in name of Common AttributeN> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute - to be moved later to a common attribute clause. | <Fill in MA or OA or NA> |
| <Fill in name of Resource Specific Attribute1> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute - to be moved later to a central attribute table that also defines the type of the attribute, allowed ranges etc. | <Fill in MA or OA or NA> |
| <Fill in name of Resource-Specific AttributeN> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute - to be moved later to a central attribute table that also defines the type of the attribute, allowed ranges etc. | <Fill in MA or OA or NA> |

In case of misalignment of the graphical representation of a resource and the associate tabular representation, tabular representation shall take precedence.

The access modes for attributes can assume the following values:

- Read/Write (RW): all operations are allowed for the attribute (Create/Update/Retrieve/Delete/Notify).

- Read Only (RO): the value of the attribute is set by the Hosting CSE when the resource is Created. Such an attribute can only be read.

- Write Once (WO): the value of the attribute is set when the resource is Created based on information from the Originator. Such an attribute can then only be read.

The multiplicity, both for the child resources and the attributes can have the following values:

- A value of "0" indicates that the child resource/attribute is not present.

- A value of "1" indicates that the child resource/attribute is present.

- A value of "0..1" indicates that the child resource/attribute could not be present. If present, it can have an instance of one only.

- A value of "0..n" indicates that the child resource/attribute could not be present. If present, multiple instances are supported.

- A value of "1..n" indicates that the child resource/attribute is always present. It has at least one instance and can have multiple instances.

The attributes for <resourceTypeAnnc> in the attribute table can have the following set of values:

- MA (Mandatory Accounced): Such attributes in the original resource are announced to the announced resources. The content of such announced attributes is the same as the content of the original resource.

- OA (Optional Announced): Such attributes in the original resource may be announced to the announced resources depending on the contents of the announcedAttribute attribute at the original resource. The content of such announced attributes is of the same as the content of the original attributes.

- NA (Not Announced): Such attributes are not announced to the announced resources.

### 9.5.1	Handling of Unsupported Resources/Attributes/Sub-resources within the M2M System

Any CSE shall respond to a received request targeted to it and that includes resource(s), resource attribute(s) or sub-resource(s) that are unsupported by the target CSE, by sending an appropriate error code back to the request originator.

When a CSE is not the target entity of a received request that includes resource(s), resource attribute(s) or sub-resource(s) that are unsupported by the CSE, the CSE shall attempt to forward the received request to the targeted entity. If the CSE cannot forward the received request for any reason, the forwarding CSE shall respond to the received request by sending an appropriate error code back to the request originator.oneM2M specifications include both mandatory and optional functionalities of interfaces between oneM2M entities. Thus, the implemented functionality of the interfaces may not include all the functionalities of the specifications.

## 9.6	Resource Types

Table 9.6-1 introduces the defined normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6-1 lists the supported <resourceTypes>. An addition of suffix "Annc" to such <resourceTypes> indicates the associated announced resource type.

Table 9.6-1 Resource Summary

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing the resources. . | subscription | AE, remoteCSE, CSEBase | 9.6.2 |
| contentInstance | Represents a data instance in the container resource. | subscription | container | 9.6.7 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the registrar CSE. | subscription, container, group, accessControlPolicy, mgmtObj, commCapabilities, pollingChannel | remoteCSE, CSEBase | 9.6.5 |
| cmdhBuffer | Defines CMDH buffer usage limits | subscription | cmdhPolicy | D.12.7 |
| cmdhDefaults | Defines CMDH default values | cmdhDefEcValue, cmdhEcDefParamValues subscription | cmdhPolicy | D.12.1 |
| cmdhEcDefParamValues | Represent a specific set of default values for the CMDH related parameters | subscription | cmdhDefaults | D.12.3 |
| cmdhDefEcValue | Defines a value for the ec (event category) parameter of an incoming request when it is not defined. | subscription | cmdhDefaults | D.12.2 |
| cmdhLimits | Defines limits for CMDH related parameter values | subscription | cmdhPolicy | D.12.4 |
| cmdhNetworkAccessRules | Defines rules for the usage of underlying networks | cmdhNwAccessRule subscription | cmdhPolicy | D.12.5 |
| cmdhNwAccessRule | Defines a rule for the usage of underlying networks | schedule subscription | cmdhNetworkAccessRules | D.12.6 |
| cmdhPolicy | A set of rules defining which CMDH parameters will be used by default | cmdhDefaults, cmdhLimits, cmdhNetworkAccessRules, cmdhBuffer subscription | None | D.12 |
| container | Shares data instances among entities. Used as a mediator that takes care of buffering the data to exchange "data" between AEs and/or CSEs. The exchange of data between AEs (e.g., an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule. | container, contentInstance, subscription, | application, container, remoteCSE, CSEBase | 9.6.6 |
| CSEBase | The structural root for all the resources that are residing on a CSE. It shall store information about the CSE itself. | remoteCSE, node, application, container, group, accessControlPolicy, subscription, mgmtObj, mgmtCmd, locationPolicy, statsConfig | None | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription | None documented | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription | statsConfig | 9.6.23 |
| execInstance | The Execution Instance resource contains all execution instances of the same management command mgmtCmd. | subscription | mgmtCmd | 9.6.17 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group. | fanOutPoint subscription | Application, remoteCSE, CSEBase | 9.6.13 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referred from container, the contentInstances of the container provides location information. | subscription | CSEBase | 9.6.10 |
| fanOutPoint | Virtual resource containing target for group requests It is used for addressing bulk operations to all the resources that belong to a group. | None | group | 9.6.14 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols. | execInstance subscription | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node | parameters subscription | remoteCSE, CSEBase | 9.6.15 Annex D |
| m2mServiceSubscription | Data pertaining to the M2M Service Subscription | nodeInfo subscription | None documented | 9.6.19 |
| node | Represents specific Node information | schedule, mgmtObj subscription | CSEBase, remoteCSE | 9.6.18 |
| nodeInfo | Node information | subscription | m2mServiceSubscription | 9.6.20 |
| parameters | Provides a mechanism to describe the management object in a generic way to easily import information from existing management protocols. | parameters subscription | mgmtObj, parameters | 9.6.16 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | None | remoteCSE, application | 9.6.21 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource. | application, container, group, accessControlPolicy, subscription, mgmtObj, pollingChannel, node | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription | None documented | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription | node, subscription, cmdhNwAccessRule | 9.6.39 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription | CSEBase (in IN-CSE) | 9.6.24 |
| statsConfig | Stores configuration of statistics for applications | eventConfig subscription | CSEBase (in IN-CSE) | 9.6.22 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribe-to resource. | schedule | accessControlPolicy, application, cmdhBuffer, cmdhDefaults, cmdhEcDefParamValues, cmdhDefEcValue, cmdhLimits, cmdhNetworkAccessRules, cmdhNwAccessRule, cmdhPolicy, container, CSEBase, delivery, eventConfig, execInstance, group, contentInstance, locationPolicy, mgmtCmd, mgmtObj, m2mServiceSubscription, node, nodeInfo, parameters, remoteCSE, request, schedule, statsCollect, statsConfig | 9.6.8 |

### 9.6.1	Common Attributes

Many of the attributes of the resources described in the present document are common. Such attributes are described here once in order to avoid duplicating the description for every resource that contains it.

Attributes that are only used in one or two resource types are described only in the clause for that resource type.

- Table 9.6.1-1: Common Attributes

| Common Attribute | Description |
| --- | --- |
| resourceType | Resource Type. This Write Once (at creation time then cannot be changed) resourceType attribute identifies the type of resources as specified in clause 9.6. Each resource shall have a resourceType attribute. |
| parentID | The system shall assign the value to this attribute according to the parameters given in the CREATE Request. It establishes the parent-child relationship by identification of the parent of this child resource. Such identifier shall use the non-hierarchical URI representation. For example, an AE resource with the identifier ”myAE1” which has been created under the resource “…//example.com/oneM2M/myCSE”, the value of the parentID attribute will contain “…//parentID”. |
| accessControlPolicyIDs | The attribute contains a list of identifiers (either an ID or a URI depending if it is a local resource or not) of an <accessControlPolicy> resource. The privileges defined in the <accessControlPolicy> resource that are referenced determine who is allowed to access the resource containing this attribute for a specific purpose (e.g., Retrieve, Update, Delete, etc.). If a resource type does not have an accessControlPolicyIDs attribute definition, then the accessControlPolicy for that resource is governed in a different way, for example, the accessControlPolicy associated with the parent may apply to a child resource that does not have an accessControlPolicyIDs attribute definition, or the privileges for access are fixed by the system. Refer to the corresponding resourceType and procedures to see how permissions are handled in such cases. If a resource type does have an accessControlPolicyIDs attribute definition, but the (optional) accessControlPolicyIDs attribute is not set, or it is set to a value that does not correspond to a valid, existing <accessControlPolicy> resource, or it refers to an <accessControlPolicy> resource that is not reachable (e.g., because it is located on a remote CSE that is offline or not reachable), then the system default access permissions shall apply. All resources are accessible only if the privileges from the Access Control Policy grants it, therefore all resources shall have an associated AccessControlPolicyIDs attribute, either explicitly (setting the attribute in the resource itself) or implicitly (either by using the parent privileges or the system defaults). Which means that the system shall provide a default access privileges in case that the Originator does not provide a specific AccessControlPolicyIDs during the creation of the resource, Default access grants the configures privileges to the originator (e.g., depending on the prefix of URI of the resource). This attribute is absent from the resource in some cases, especially if the resource shall have the same privileges of the parent resource; such an attribute is therefore not needed. To update this attribute, a Hosting CSE shall check whether an Originator has Update permission in selfPrivileges of the <accessControlPolicy> resource which this attribute originally indicates. |
| creationTime | Time/date of creation of the resource. This attribute is mandatory for all resources and the value is assigned by the system at the time when the resource is locally created. Such an attribute cannot be changed. |
| expirationTime | Time/date after which the resource will be deleted by the hosting CSE. This attribute can be provided by the Originator, and in such a case it will be regarded as a hint to the hosting CSE on the lifetime of the resource. The hosting CSE can however decide on the real expirationTime. If the hosting CSE decides to change the expirationTime attribute value, this is communicated back to the Originator. The lifetime of the resource can be extended by providing a new value for this attribute in an UPDATE operation. Or by deleting the attribute value, e.g., by not providing the attribute when doing a full UPDATE, in which case the hosting CSE can decide on a new value. This attribute shall be mandatory. If the Originator does not provide a value in the CREATE operation the system shall assign an appropriate value depending on its local policies and/or M2M service subscription agreements. |
| lastModifiedTime | Last modification time/date of the resource. This attribute shall be mandatory and its value is assigned automatically by the system each time that the addressed target resource is modified by means of the UPDATE operation. |
| stateTag | An incremental counter of modification on the resource. When a resource is created, this counter is set to 0, and it will be incremented on every modification of the resource. NOTE: In order to enable detection of overflow, the counter needs to be capable of expressing sufficiently long numbers. . NOTE: This attribute has the scope to allow identifying changes in resources within a time interval that is lower than the one supported by the attribute lastModifiedTime (e.g., less than a second or millisecond). This attribute can also be used to avoid race conditions in case of competing modifications. Modifications (e.g., update/delete) can be made on the condition that this attribute has a given value. |
| labels | Tokens used as keys for discovering resources. This attribute is optional and if not present it means that the resource cannot be found by means of discovery procedure which uses labels as key parameter of the discovery. |
| link | This attribute shall be present only on the announced resource. This attribute shall provide the link (URI) to the original resource. |
| announceTo | This attribute shall only be present on the original resource if it has been announced to other CSEs. This attribute maintains the list of URIs to the announced resources. Updates on this attribute will trigger new resource announcement or de-announcement. In case of new resource announcement, either the exact URI to the announced resource that is to be created can be provided, or the CSE-ID of the remote CSE can be provided. In the later case, the original resource-hosting CSE will decide the exact location of the announced resource in the remote CSE identified by the CSE-ID. |
| announcedAttribute | This attributes shall only be present on the original resource if some Optional Announced (OA) type attributes have been announced to other CSEs. This attribute maintains the list of the announced Optional Attributes (OA type attributes) in the original resource. Updates to this attribute will trigger new attribute announcement if a new attribute is added or de-announcement if the existing attribute is removed. |

### 9.6.2	Resource Type accessControlPolicy

Access control policy is defined as "white lists" or privileges, i.e., each privilege defines "allowed" entities (defined as originatorPrivileges) for certain access modes (defined as privilegesFlags). Sets of privileges are handled such that the resulting privileges for a group of  privileges are the sum of the individual privileges; i.e., an action is permitted if it is permitted by some/any privileges in the set. The selfPrivilege attribute lists those that have the rights to Read/Update/Delete for the resource <accessControlPolicy> itself.

Each privilege may also refer to a role described in a <serviceSubscription>.

All privileges defined in an access control policy are also contextualized to: location, time windows and IP address.

By setting an accessControlPolicyID attribute on a resource, the privileges for accessing that resource are then defined by the privileges defined in the <accessControlPolicy> resource.

Figure 9.6.2-1: Structure of <accessControlPolicy> resource

[9.6.2.b]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attributes.

The <accessControlPolicy> resource shall contain the child resource in Table 9.6.2-1.

- Table 9.6.2-1: Child resources of <accessControlPolicy> resource

| Child Resource Name of <accessControlPolicy> | Child Resource Type | Multiplicity | Description | <accessControlPolicyAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |

[9.6.2.c]  Editor's Note: Contribution ARC-2014-0089R03 does not provide the table for the <subscription> child-resource. The contributor to verify the accuracy of the proposal.

The <accessControlPolicy> resource shall contain the attributes in Table 9.6.2-2.

- Table 9.6.2-2: Attributes of <accessControlPolicy> resource

| Attribute Name of <accessControlPolicy> | Multiplicity | RW/ RO/ WO | Description | <accessControlPolicyAnnc> Attribute |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| privileges | 1 | RW | The list of privileges defined by this <accessControlPolicy> resource. These privileges are applied to resources referencing this <accessControlPolicy> resource using the accessControlPolicyID attribute. | MA |
| selfPrivileges | 1 | RW | Defines the list of privileges for the <accessControlPolicy> resource itself. | MA |

The privileges could then be generalised to actions (which might be granting access, but might also be more specific, like granting access to a subset, i.e., filtering part of the data). The privileges could be generalised to conditions, which may include things like the identity of the requestor, everybody except specified identities, but it might also include time based conditions, etc.

The access granting mechanism based on access control policy operates by matching the request originator with originator privilege list stored in an <accessControlPolicy> resource. When a positive match is found then the requested operation (e.g., RETRIEVE) is checked with a set of allowed operations associated to the matching privilege holder; if this check fail, then the request will be denied. These sets are called privilege flags.

Selfprivileges and privileges are both lists of originator privilege associated with privilege flags applied respectively to <accessControlPolicy> resource itself (self privileges) and to all other resource types addressing the <accessControlPolicy> resource with the accessControlPolicyID common attribute (privileges).

All privileges defined in an access control policy are also contextualized to: location, time windows and IP address before granting the access

Each privilege in the selfprivileges and privileges lists can be also constituted by one role. Such role is identified by a role name and a URL addressing a <m2mServiceSubscription> resource where such role is defined. In case the originator is presenting itself with a specific role, the access control policy operates by matching the request originator with the list belonging to such specific role as described in the Service Subscription resource (achieved by partial addressing).

Each privilege in the privileges and selfprivileges lists is constituted by the following elements

Table 9.6.2-3: Privileges types

| Name | Description |
| --- | --- |
| originatorPrivileges | See clause 9.6.2.1 |
| contexts | See clause 9.6.2.2 |
| operationFlags | See clause 9.6.2.3 |

#### 9.6.2.1	originatorPrivileges

The following originator privilege types shall be considered for access control policy check.

Table 9.6.2.1-1: originatorPrivileges types

| Name | Description |
| --- | --- |
| Domain | FQDN domain |
| originator identifier | CSE-ID or AE-ID which represent a originator identity |
| Token | Access token usually provided as query parameter |
| All | All originators |
| Role | A role name associated with the URL the a Service Subscription resource where such role is defined |

#### 9.6.2.2	contexts

The following originator contexts types shall be considered for access control policy check.

Table 9.6.2.2-1: Context types

| Name | Description |
| --- | --- |
| Context | Defines the context in which every privileges of the present access control policy resource applies, e.g., time windows, location, IP address. Editor’s note: the detailed definition for the context is FFS. |

#### 9.6.2.3	operationFlags

The following privilege flags shall be considered for access control policy check in association to an originator privilege.

Table 9.6.2.3-1: PrivilegeFlags

| Name | Description |
| --- | --- |
| RETRIEVE | Privilege to retrieve the content of an addressed resource |
| CREATE | Privilege to create a child resource |
| UPDATE | Privilege to update the content of an addressed resource |
| DELETE | Privilege to delete an addressed resource |
| DISCOVER | Privilege to discover the resource |
| NOTIFY | Privilege to receive a notification |

[9.6.2.3.a]  Editor’s Note:  A non-verbose  privilege flag description could be the following: <pf>RcUeDN</pf> that means: RETRIEVE allowed, CREATE not allowed, UPDATE allowed, DELETE not allowed, DISCOVER allowed, NOTIFY allowed.

### 9.6.3	Resource Type CSEBase

A <CSEBase> resource shall represent a CSE. This <CSEBase> resource shall be the root for all the resources that are residing on the CSE.

Figure 9.6.3-1: Structure of <CSEBase> resource

[9.6.3.a0] Editor's Note; The picture above needs to be updated as per the new structure of CSEBase resource.

The <CSEBase> resource shall contain the child resources in Table 9.6.3-1.

Table 9.6.3-1: Child resources of <CSEBase> resource

| Child Resource Name of <CSEBase> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <remoteCSE> | 0..n | See clause 9.6.4 |
| [variable] | <node> | 0..n | See clause 9.6.18 |
| [variable] | <AE> | 0..n | See clause 9.6.5 |
| [variable] | <container> | 0..n | See clause 9.6.6 |
| [variable] | <group> | 0..n | See clause 9.6.13 |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <mgmtCmd> | 0..n | See clause 9.6.16 |
| [variable] | <locationPolicy> | 0..n | See clause 9.6.10 |
| [variable] | <statsConfig> | 0..n | See clause 9.6.22 |
| [variable] | <statsCollect> | 0..n | See clause 9.6.24 |

The <CSEBase> resource shall contain the attributes described in Table 9.6.3-2.

Table 9.6.3-2: Attributes of <CSEBase> resource

| Attribute Name of <CSEBase> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |
| cseType | 1 | WO | Indicates the type of CSE node represented by the created resource. |
| CSE-ID | 1 | WO | The globally unique CSE identifier. |
| supportedResourceType | 1 | RO | List of the resource types which are supported in CSE. this attribute contains subset of resource types listed in clause 9.2 |
| pointOfAccess | 0..n | RW | Represents the list of physical addresses to be used by remote CSEs to connect to this CSE (e.g., IP address, FQDN). This attribute is used to announce its value to remote CSEs. |
| nodeLink | 0..1 | RO | A reference (URI) of a <node> resource that stores the node specific information. |
| Trigger-Recipient-ID | 0..1 | RW | See clause 7.1.10 where this attribute is described. |
| notificationCongestionPolicy | 0..1 | RO | This attribute applies to CSEs generating subscription notifications. It specifies the rule which is applied when the storage of notifications for each subscriber (an AE or CSE) reaches the maximum storage limit for notifications for that subscriber. E.g., Delete stored notifications of lower notificationStoragePriority to make space for new notifications of higher notificationStoragePriority, or delete stored notifications of older creationTime to make space for new notifications when all notifications are of the same notificationStoragePriority. |

### 9.6.4	Resource Type remoteCSE

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase>.

Conversely each registered CSE shall also be represented as a sub-set of <remoteCSE> resource in the registering CSE's <CSEBase>.

For example, when CSE1 registers with CSE2, there will be two <remoteCSE> resources created: one in CSE1 <CSEBase1>/<remoteCSE2> and one in CSE2 <CSEBase2>/<remoteCSE1>.

Note that the creation of the two resources does not imply mutual registration. The <CSEBase1>/<remoteCSE2> does not mean CSE2 registered with CSE1 in the example above.

Figure 9.6.4-1: Structure of <remoteCSE> resource

[9.6.4.a]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attribute.

[9.6.4.a1] Editor's Note; The picture above needs to be updated as per the new strucutre of remoteCSE resource.

The <remoteCSE> resource shall contain the child resources in Table 9.6.4-1.

- Table 9.6.4-1: Child resources of <remoteCSE> resource

| Child Resource Name of <remoteCSE> | Child Resource Type | Multiplicity | Description | <remoteCSEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <AE> | 0..n | See clause 9.6.5 | AE <AEAnnc> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> <accessControlPolicyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <pollingChannel> | 0..n | See clause 9.6.21. If requestReachability is FALSE, the CSE that created this <remoteCSE> should create a <pollingChannel> resource and perform long polling. | <pollingChannel> |
| [variable] | <schedule> | 0..1 | This resource defines the reachability schedule information of the node. See clause 9.6.9 for <schedule> | <scheduleAnnc> |

The <remoteCSE> resource shall contain the attributes described in Table 9.6.4-2.

- Table 9.6.4-2: Attributes of <remoteCSE> resource

| Attribute Name of <remoteCSE> | Multiplicity | RW/ RO/ WO | Description | <remoteCSEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. | MA |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| cseType | 1 | WO | Indicates the type of CSE node represented by the created resource. | OA |
| pointOfAccess | 0..n | RW | For request-reachable remote CSE it represents the list of physical addresses to be used to connect to it (e.g., IP address, FQDN). The attribute is absent if the remote CSE is not request-reachable. | OA |
| CSEBase | 1 | WO | The URI of the CSEBase of the original CSE represented by remote CSE. | OA |
| CSE-ID | 1 | WO | The globally unique CSE identifier. | OA |
| Trigger-Recipient-ID | 0..1 | RW | See clause 7.1.10 where this attribute is described. | OA |
| requestReachability | 1 | RW | If the CSE that created this <remoteCSE> can receive a request from other AE/CSE(s), this attribute is set to “TRUE” otherwise “FALSE”. NOTE: Even if this attribute is set to “FALSE”, it does not mean it AE/CSE is always unreachable by all entities. E.g., the requesting AE/CSE is behind the same NAT, so it can communicate within the same NAT. | OA |
| nodeLink | 0..1 | RO | Reference URI of a <node> resource that stores node specific information. | OA |

<remoteCSE> and announced <remoteCSE> shall have different resourceType coding.

### 9.6.5	Resource Type AE

The <AE> resource represents information about an Application Entity known to a given Common Services Entity.

Figure 9.6.5-1: Structure of <AE> resource

[9.6.5.a]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attribute.

[9.6.5a1] Editor's Note: The picture above needs to be updated as per the new strucutre of AE resource.

The <AE> resource shall contain the child resources in Table 9.6.5-1.

- Table 9.6.5-1: Child resources of <AE> resource

| Child Resource Name of <AE> | Child Resource Type | Multiplicity | Description | <AEAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> <accessControlPolicyAnnc> |
| [variable] | <pollingChannel> | 0..n | See clause 9.6.21. When the Registrar CSE of this AE is request-unreachable, the AE should create this <pollingChannel> resource and perform long polling. | <pollingChannel> |

The <AE> resource shall contain the attributes described in Table 9.6.5-2.

- Table 9.6.5-2: Attributes of <AE> resource

| Attribute Name of <AE> | Multiplicity | RW/ RO/ WO | Description | <AEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| accessControlPolicyIDs | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described. | MA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| name | 1 | RO | The (usually human readable) name of the application, as declared by the application developer (e.g., "HeatingMonitoring") | OA |
| App-ID | 1 | RO | Application Identifier | OA |
| AE-ID | 1 | RO | The identifier of the Application Entity (see clause 7.1.2). | OA |
| pointOfAccess | 0..n | RW | The list of addresses for communicating with the registered Application Entity over Mca reference point via the transport services provided by Underlying Network (e.g., IP address, FQDN, URI). This attribute shall be accessible only by the AE and the hosting CSE. | OA |
| ontologyRef | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is managed and understood by the AE; to be passed to the AE. | OA |
| nodeLink | 0..1 | RO | A reference (URI) of a <node> resource that stores the node specific information. | OA |

### 9.6.6	Resource Type container

This resource represents a container for data instances. It is used to share information among other entities and potentially to track the data. A <container> resource has no associated content, only attributes and child resources.

Figure 9.6.6-1: Structure of <container> resource

[9.6.6.a]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attributes.

The <container> resource shall contain the child resources in Table 9.6.6-1.

- Table 9.6.6-1: Child resources of <container> resource

| Child Resource Name of <container> | Child Resource Type | Multiplicity | Description | <containerAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <contentInstance> | 0..n | See clause 9.6.7 | <contentInstance> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |

The <container> resource shall contain the attributes in Table 9.6.6-2.

- Table 9.6.6-2: Attribute of <container> resource

| Attribute Name of <container> | Multiplicity | RW/ RO/ WO | Description | <containerAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. If no accessControlPolicyIDs are provided at the time of creation, the accessControlPolicyIDs of the parent resource is linked to this attribute | MA |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| creator | 1 | RW | The AE-ID or CSE-ID of the entity which created the resource. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| stateTag | 1 | RO | See clause 9.6.1 where this common attribute is described. | OA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| maxNrOfInstances | 0..1 | RW | Maximum number of instances of <contentInstance> child resources. | OA |
| maxByteSize | 0..1 | RW | Maximum number of bytes that are allocated for a <container> resource for all instances in the <container> resource. | OA |
| maxInstanceAge | 0..1 | RW | Maximum age of the instances of <contentInstance> resources within the <container>. The value is expressed in seconds. | OA |
| currentNrOfInstances | 1 | RO | Current number of instances in a <container> resource. It is limited by the maxNrOfInstances. | OA |
| currentByteSize | 1 | RO | Current size in bytes of data stored in a <container> resource. It is limited by the maxNrOfBytes. | OA |
| latest | 0 | RO | Reference to latest <contentInstance> resource, when present. | OA |
| locationID | 0..1 | RW | URI of the resource where the attributes/policies that define how location information are obtained and managed. This attribute is defined only when the <container> resource is used for containing location information. | OA |
| ontologyRef | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is stored in the instances of the container. NOTE: The access to this URI is out of scope of oneM2M. | OA |

### 9.6.7	Resource Type contentInstance

The <contentInstance> resource represents a data instance in the container. The content of the contentInstance can be encrypted.

Contrary to other resources, the <contentInstance> resource shall not  be modified once created. An AE shall be able to delete a contentInstance explicitly or it may be deleted by the platform based on policies. If the platform has policies for  contentInstance retention these shall be represented by the attributes maxByteSize, maxNrOfInstances and/or maxInstanceAge on the <container> resource. If multiple policies are in effect, the strictest policy shall apply.

The <contentInstance> resource inherits the same access control policies of the parent <container> resource, and does not have its own accessControlPolicyIDs attribute.

Figure 9.6.7-1: Structure of <contentInstance> resource

[9.6.7.a]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attributes.

[9.6.7b] Editor's Note; The picture above needs to be updated as per the new strucutre of AE resource.

This resource shall contain the child resource in Table 9.6.7-1.

- Table 9.6.7-1: Child resources of <contentInstance> resource

| Child Resource Name of <contentInstance> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |

This resource shall contain the attributes in Table 9.6.7-2.

- Table 9.6.7-2: Attributes of <contentInstance> resource

| Attribute Name of <contentInstance> | Multiplicity | RW/ RO/ WO | Description | <contentInstanceAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| labels | 0..1 | WO | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| stateTag | 1 | RO | See clause 9.6.1 where this common attribute is described. The stateTag attribute of the parent resource should be incremented first and copied into this stateTag attribute when a new instance is added to the parent resource. | OA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| typeOfContent | 0..1 | WO | Type of the content included in the content attribute. This is media-type as defined in [i-2]. | OA |
| contentSize | 1 | WO | Size in bytes of the content attribute. | OA |
| ontologyRef | 0..1 | WO | A reference (URI) of the ontology used to represent the information that is stored in the instances resources of the container. If this attribute is not present, the instance resource inherits the ontologyRef from the parent <container> resource if present NOTE: Access to this URI is out of scope of oneM2M. | OA |
| content | 1 | WO | Actual opaque content of an instance. This may for example be an image taken by a security camera, or a temperature measurement taken by a temperature sensor. | OA |

### 9.6.8	Resource Type subscription

The <subscription> resource contains subscription information for its subscribed-to resource. The subscribed-to resource is the resource that has the <subscription> resource as the child resource.

The <subscription> resource shall be represented as child resource of the subscribed-to resource. For example, <container> resource has <subscription> resource as a child resource (see clause 9.6.6). A <subscription> resource shall be deleted when a parent subscribed-to resource is deleted.

The <subscription> resource shall represent a subscription to a subscribed-to resource. An Originator shall be able to create a resource of <subscription> resource type when the originator has RETRIEVE permission to the subscribe-to resource. The Originator of a <subscription> resource becomes a resource subscriber.

When a modification to the subscribed-to resource occurs, that modification is compared to the filterCriteria attribute to determine whether a notification is to be sent to the resource subscriber.

Figure 9.6.8-1: Structure of <subscription> resource

[9.6.8.a]  Editor's Note: Verify if the picture needs to be re-drawn.

The <subscription> resource shall contain the child resources in Table 9.6.8-1.

- Table 9.6.8-1: Child resources of <subscription> resource

| Child Resource Name of <subscription> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| notificationSchedule | <schedule> | 0..1 | See clause 9.6.9 |

The <subscription> resource shall contain the attributes described in Table 9.6.8-2.

- Table 9.6.8-2: Attributes of <subscription> resource

| Attribute Name of <subscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | I | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. If no accessControlPolicyIDs is given at the time of creation, the accesControlPolicies of the parent resource is linked to this attribute. |
| filterCriteria | 0..1 | RW | Filtering conditions that when applied and provide a not empty matching set of resources, then a notification can be sent to notificationURI. |
| expirationCounter | 0..1 | RW | When the number of notifications becomes the same as this counter, the <subscription> resource shall be deleted. |
| notificationURI | 1..n | RW | URI where the resource subscriber will receive notifications. |
| aggregationURI | 0..1 | RW | URI to aggregate notifications from group members of a <group> resource. |
| batchNotify | 0..1 | RW | Indicates that notifications should be batched for delivery. When set, notification events are temporarily stored until either a specified number is ready to send or until a duration after the first notification event has expired. |
| rateLimit | 0..1 | RW | Indicates that notifications should be rate-limited. When set, notification events that exceed a specified number within a specified time are temporarily stored then sent when the number of events sent per specified time falls below the limit. |
| priorSubscriptionNotify | 0..1 | WO | Indicates that when this subscription is created, whether notification events prior to subscription should be sent, e.g., send prior "n" notifications, if available. |
| interimEventsNotify | 0..1 | RW | Indicates the notification action to be taken following a period of unreachability, when the resource subscriber becomes reachable (e.g., send "n" interim notifications, if available.) |
| notificationStoragePriority | 0..1 | RW | Indicates a priority for this subscription relative to other subscriptions belonging to this same subscriber for retention of notification events when storage is congested. The storage congestion policy which uses this attribute as input is specified in clause 9.6.3. |
| latestNotify | 0..1 | RW | Indicates if the subscriber wants only the latest notification or all the notification. If the subscriber only wants the latest notification, older notifications can be discarded. The attribute is mutual exclusive with batchNotify. |
| notificationStructure | 1 | RW | Type of information that shall be contained in notifications. E.g., modified attribute only of a subscribed-to resource, a whole subscribed-to resource, and/or URI of a corresponding <subscription> resource. |
| notificationDeliveryPriority | 0..1 | RW | Indicates a delivery priority for the notification. That is, this attribute defines the how to handle sending of notifications when notifications need to be sent. |
| notificationEventCat | 0..1 | RW | Defines the Event Categories for the notification message triggered by the <subscription> resource. |

The filterCriteria attribute shall be configured a priori in <subscription> resource and only if an event matches the filterCriteria conditions, then a notification shall be delivered.

The following Table describes the filterCriteria conditions.

Table 9.6.8-3: filterCriteria conditions

| Condition tag | Multiplicity | Matching condition |
| --- | --- | --- |
| createdBefore | 0..1 | The creationTime attribute of the resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically before the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the resource is chronologically after the specified value. |
| labels | 0..n | The labels attributes of the resource matches the specified value. |
| resourceType | 0..n | The resourceType attribute of the resource is the same as the specified value. It also allows discriminating between normal and announced resources. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> resource is smaller than the specified value. |
| contentType | 0..n | The typeOfContent attribute of the <contentInstance> resource matches the specified value |
| resourceStatus | 0..n | When the resource is changed by the operations or expiration, the resource status of the resource is the same as the specified value. Possible values are: created, updated, deleted. |
| operationMonitor | 0..n | The operations accessing the resource matches with the specified value. It allows monitoring which operation is attempted to the subscribed resource regardless of whether the operation is performed. This feature is useful when to find malicious AEs. Possible string arguments are: create, retrieve, update, delete. |
| attribute | 0..n | Attribute token name to be provided to reduce the scope of filtering to resource attributes. |

### 9.6.9	Resource Type schedule

The <schedule> resource contains scheduling information.

The <schedule> resource shall represent scheduling information in the context of its parent resource. An Originator shall have the same access control privileges to the <schedule> resource as it has to its parent resource.

Figure 9.6.9-1: Structure of <schedule> resource

[9.6.9.c]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attribute.

The <schedule> resource shall contain the child resource in Table 9.6.9-1.

- Table 9.6.9-1: Child resources of <schedule> resource

| Child Resource Name of <schedule> | Child Resource Type | Multiplicity | Description | <scheduleAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | None |

The <schedule> resource shall contain the attributes in Table 9.6.9-2.

- Table 9.6.9-2: Attributes of <schedule> resource

| Attribute Name of <schedule> | Multiplicity | RW/ RO/ WO | Description | <scheduleAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described. | MA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| scheduleElement | 1..n | RW | Expresses time periods defined by second, minute, hour day of month, month, and year. Supports repeating periods, and wildcards. | OA |

### 9.6.10	Resource Type locationPolicy

The <locationPolicy> resource represents the method for obtaining and managing geographical location information of an M2M Node.

The actual location information shall be stored in a <contentInstance> resource which is a child resource of the <container> resource.  The <container> resource includes the locationId attribute which holds the URI of this <locationPolicy> resource.. The CSE can obtain location information based on the attributes defined under <locationPolicy> resource, and store the location information in the target <container> resource.

Based on the locationSource attribute, the method for obtaining location information of a Node can be differentiated. The methods for obtaining location information shall be as follows:

- Network-based method: where the CSE on behalf of the AE obtains the target Node's location information from an Underlying Network,

- Device-based method: where the ASN is equipped with any location capable  modules or technologies (e.g., GPS) and is able to position itself

- Sharing-based method: where the ADN has no GPS nor Underlying Network connectivity. Its location information can be retrieved from either an ASN or a MN.

NOTE:	Geographical location information could include more than longitude and latitude.

Figure 9.6.10-1: Structure of <locationPolicy> resource

[9.6.10.a]  Editor's Note: To update the picture to include "announceTo" and "announcedAttribute" attribute.

[9.6.10.a-1]  Editor's Note: Verify if the picture needs to be re-drawn.

The <locationPolicy> resource shall contain the child resource in Table 9.6.10-1.

- Table 9.6.10-1: Child resources of <locationPolicy> resource

| Child Resource Name of <locationPolicy> | Child Resource Type | Multiplicity | Description | <locationPolicyAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | None |

The <locationPolicy> resource shall contain the attributes in Table 9.6.10-2.

- Table 9.6.10-2: Attributes of <locationPolicy> resource

| Attribute Name of <locationPolicy> | Multiplicity | RW/ RO/ WO | Description | <locationPolicyAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| accessControlPolicyIDs | 1..1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| locationSource | 1 | RW | Indicates the source of location information - Network Based - GPS Based - Sharing Based | OA |
| locationUpdatePeriod | 0..1 | RW | Indicates the period for updating location information. If the value is marked '0' or not defined, location information is updated only when a retrieval request is triggered. | OA |
| locationTargetId | 0..1 | RW | The identifier to be used for retrieving the location information of a remote Node and this attribute is only used in the case that location information is provided by a location server. | OA |
| locationServer | 0..1 | RW | Indicates the identity of the location server. This attribute is only used in that case location information is provided by a location server. | OA |
| locationContainerID | 0..1 | RO | A URI of the <container> resource where the actual location information of a M2M Node is stored. | OA |
| locationContainerName | 0..1 | RW | A Name of the <container> resource where the actual location information of a M2M Node is stored. If it is not assigned, the Hosting CSE automatically assigns a name of the resource. Note: The created <container> resource related to this policy shall be stored only in the Hosting CSE. | OA |

### 9.6.11	Resource Type delivery

When a CSE is requested to initiate an operation (CRUDN) targeting resources on another CSE, then it needs to do  scheduling and execution of delivery of data from the source CSE to the target CSE in line with the provisioned policies. It shall be in one of the following ways: Using delivery aggregation (da information set to ON) or by forwarding the original request as a separate request on the Mcc reference point without changes.

In order to be able to initiate and manage the execution of data delivery in a resource-based manner, resource type delivery is defined. This resource type shall be used for forwarding requests from one CSE to another CSE when the da parameter in the request is set to ON. If the da parameter is set to OFF, the original request shall be forwarded without change to the next CSE, i.e., without the use of <delivery> resource. If the da parameter is not present, the latter method shall be used.

Operations to Retrieve, Update or Delete a <delivery> resource shall allow authorized entities to inquire the status of a delivery, change delivery attributes or cancel a delivery.

As defined in clause 10.2.4, <delivery> resource can only be created by a CSE. A request for the creation of a <delivery> resource can only be issued to a Registrar CSE. <delivery> resource is deleted on successful delivery of  the data in aggregatedRequest attribute to the next hop CSE.

The parent of a <delivery> resource is the <CSEBase> resource of the CSE that accepted the Request for the creation of the <delivery> resource.

Figure 9.6.11-1: Structure of resource <delivery>

[9.6.11.a0]  Editor's Note: Verify if the picture needs to be re-drawn.

The <delivery> resource shall contain the child resource in Table 9.6.11-1.

- Table 9.6.11-1: Child resources of <delivery> resource

| Child Resource Name of <delivery> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |

The <delivery> resource shall contain the attributes shown in Table 9.6.11-2.

- Table 9.6.11-2: Attributes of <delivery> resource

| Attribute Name of <delivery> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described. |
| stateTag | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| source | 1 | WO | The CSE-ID of the CSE that initiated the delivery process represented by this <delivery> resource. |
| target | 1 | WO | CSE-ID that defines the Hosting CSE for delivering the data contained in the aggregatedRequest attribute |
| lifespan | 1 | RW | Defines the time limit when the delivery of the information in the aggregatedRequest attribute needs to complete. If the lifespan expires before successful delivery, no further attempts to deliver the information in the aggregatedRequest attribute need to be executed. If the delivery fails, a feedback may be expected by the source CSE depending on options reflected in the deliveryMetaData attribute. The lifespan attribute of a <delivery> resource shall be set consistent with the rqet parameters of the set of original requests contained in the aggregatedRequest attribute, i.e., lifespan shall not extend beyond the earliest expiring rqet parameter in the set of the original requests contained in the aggregatedRequest attribute. |
| eventCat | 1 | RW | Defines the category of the event that triggered the delivery request represented by this <delivery> resource |
| deliveryMetaData | 1 | RW | Contains meta information on the delivery process represented by this <delivery> resource, such as delivery status, delivery options, tracing information, etc |
| aggregatedRequest | 1 | WO | Attribute containing the request(s) to be delivered to the Hosting CSE. This represents one or more original requests that were targeting the same Hosting CSE. |

### 9.6.12	Resource Type request

The use of this resource type is optional depending on the configuration.

Creation of a <request> resource can only be done on a Registrar CSE implicitly when an AE issues a request for any other resource type to the Registrar CSE. Creation of a <request> resource instance  is only permitted by the Registrar CSE as a result of any request from the Originator which contains the rt parameter in the request message and where rt is set to 'nonBlockingReqeustSynch' or 'nonBlockingRequestAsynch'.

When a CSE is requested to initiate an operation for which the result should be available to the Originator by reference (rt information of the request set to 'nonBlockingReqeustSynch' or 'nonBlockingRequestAsynch'), the Registrar CSE which received the Request directly from the Originator shall provide a reference of the created <request> resource back to the Originator so that the Originator can access attributes of the Request at a later time - for instance in order to retrieve the result of an operation that was taking a longer time. If the Registrar CSE uses resources of type <request> to keep such context information, the reference that shall be given back to the Originator as part of the acknowledgment is the address of the <request> resource. The Originator (or any other authorized entity depending on access control) can access the request status and the requested operation result through it.

The <request> resource may be deleted by the CSE that is hosting it when the expiration time of the <request> resource is reached. So after the expiration time of a <request> resource is reached it cannot be assumed that that particular <request> resource is still accessible. Depending on implementation of the CSE that is hosting it, a <request> resource may also get deleted earlier than the expiration time, when the result of the requested operation (if any result was requested at all) has been sent back to the Originator.

For the purpose of providing a standardized structure for expressing and accessing the context of a previously issued Request, the resource type request is defined.

Figure 9.6.12-1: Structure of <request> resource

[9.6.12.a0]  Editor's Note: Verify if the picture needs to be re-drawn.

The <request> resource shall contain the child resource in Table 9.6.12-1.

- Table 9.6.12-1: Child resources of <request> resource

| Child Resource Name of <request> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |

The <request> resource shall contain the attributes shown in Table 9.6.12-2.

- Table 9.6.12-2: Attributes of <request> resource

| Attribute Name of <request> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. The value of the expirationTime is chosen by the CSE dependent on the rqet, rset, rp and oet parameters associated with the original request. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described. |
| stateTag | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| operation | 1 | RO | It contains the value of the parameter op in the original request message. |
| target | 1 | RO | It contains the value of the parameter to in the original request message. |
| originator | 1 | RO | It contains the value of the parameter fr in the original request message. |
| metaInformation | 1 | RO | Meta information about the request. The content of this attribute is equivalent to information in any other optional parameters described in clause 8.1. |
| content | 1 | RO | Contains the content that is carried in the cn parameter of the original request message. |
| requestStatus | 1 | RO | Contains information on the current status of the Request, e.g., "accepted and pending". |
| operationResult | 1 | RO | Contains the result of the originally requested operation in line with the rc parameter associated with the original request. |

All operations on <request> resources except for CREATE operations – which can only be triggered implicitly by a request for which a <request> resource shall capture the context – are controlled by the Access control policy.

### 9.6.13	Resource Type group

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the membersList attribute. The <group> resource contains an attribute that represents the members of the group and a virtual resource (the <fanOutPoint>) that allows operations to be applied to the resources represented by those members.

When used as one of the permission holders in an <accessControlPolicy> resource, the group can be used to grant a collection of AEs (represented by <AE> resources) or CSEs (represented by <remoteCSE> resources) permissions for accessing (e.g., creating child resources, retrieving, etc.) a resource.

Figure 9.6.13-1: Structure of <group> resource

[9.6.13.a]  Editor's Note: To update the picture to include "link", "announceTo" and "announcedAttribute" attribute.

[9.6.13.a-1]  Editor's Note: Verify if the picture needs to be re-drawn.

The <group> resource shall contain the child resources in Table 9.6.13-1.

- Table 9.6.13-1: Child resources of <group> resource

| Child Resource Name of <group> | Child Resource Type | Multiplicity | Description | <groupAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | none |
| fanOut | <fanOutPoint> | 1 | See clause 9.6.14 | none |

- The <group> resource shall contain the attributes in Table 9.6.13-2

- Table 9.6.13-2: Attributes of <group> resource

| Attribute Name of <group> | Multiplicity | RW/ RO/ WO | Description | <groupAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. | MA |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described. | MA |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| creator | 0..1 | RW | The AE-ID or CSE-ID of the entity which created the resource. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. | NA |
| announceTo | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| announcedAttribute | 1 | RW | See clause 9.6.1 where this common attribute is described. | NA |
| memberType | 1 | WO | It is the resource type of the members resources of the group if all members resources (including the members resources in any sub-groups) are of the same type. Otherwise, it's a type of 'mixed'. | OA |
| currentNrOfMembers | 1 | RO | Current number of members in a group. It is limited by the maxNrOfMembers. | OA |
| maxNrOfMembers | 1 | RW | Maximum number of members in the <group>. | OA |
| membersList | 1 | RW | List of zero or more member URIs referred to in the remaining of this specification as memberID. Each URI (memberID) should refer to a members resource or a (sub-) <group> resource of the <group>. | OA |
| membersAccessControlPolicyIDs | 0..1 | RW | URIs of the <accessControlPolicy> resources defining who is allowed to access the <fanOutPoint> resource. | OA |
| memberTypeValidated | 1 | RO | Denotes if memberType of all members resources of the group has been validated. | OA |
| consistencyStrategy | 0..1 | WO | This attribute determines how to deal with the <group> resource if the memberType validation fails. Which means delete the inconsistent member if the attribute is ABANDON_MEMBER; delete the group if the attribute is ABANDON_GROUP; set the memberType to "mixed" if the attribute is SET_MIXED. | OA |
| groupName | 0..1 | RW | Human readable name of the <group>. | OA |

### 9.6.14	Resource Type fanOutPoint

The <fanOutPoint> resource is a virtual resource because it does not have a representation. It is the child resource of a <group> resource. Whenever the request is sent to the <fanOutPoint> resource, the request is fanned out to each of the members of the <group> resource indicated by the membersList attribute of the <group> resource. The responses (to the request) from each member are then aggregated and returned to the Originator.

The procedure for fanning out requests to each member of the <group> and sub <groups> and how the results are returned to the hosting CSE are out of scope of this specification.  An example of how to achieve this is to use underlying transport protocols.

The aggregation of responses is simple; the hosting CSE gathers the individual responses together and forwards them as a batch to the Originator.

Rapporteur Note: The text above in turquoise added in V0.5.1. Need to verify the validity of the text.

The <fanOutPoint> resource does not have a resource representation by itself and consequently it does not have an accessControlPolicyIDs attribute. The <accessControlPolicy> resource used for access control policy validation is indicated by the membersAccessControlPolicyIDs attribute in the parent <group> resource.

### 9.6.15	Resource Type mgmtObj

The mgmtObj resource contains management data which represents individual M2M management functions. It represents a general structure to map to external management technology (e.g., OMA DM [i.5], BBF TR-069 [i.4] and LWM2M [i.6] data models.

Figure 9.6.15-1: Structure of <mgmtObj> resource

[9.6.15.b-1] Editor's Note; The picture above needs to be updated as per the new strucutre of mgmtObj resource.

The <mgmtObj> resource shall contain the child resource in Table 9.6.15-1.

- Table 9.6.15-1: Child resources of <mgmtObj> resource

| Child Resource Name of <mgmtObj> | Child Resource Type | Multiplicity | Description |  |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |  |

The <mgmtObj> resource shall contain the attributes described in Table 9.6.15-2.

- Table 9.6.15-2: Attributes of <mgmtObj> resource

| Attribute Name of <mgmtObj> | Multiplicity | RW/ RO/ WO | Description |  |
| --- | --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |  |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |  |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |  |
| accessControlPolicyIDs | 1 | RW | See clause 9.6.1 where this common attribute is described |  |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |  |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |  |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |  |
| mgmtDefinition | 1 | WO | Specifies the type of <mgmtObj> resource e.g., software, firmware, memory. The list of the value of the attribute can be seen in Annex D. |  |
| objectID | 0..1 | WO | Contains a URN that uniquely identifies the external management technology data model used for this <mgmtObj> resource as well as the managed function and version it represents. This attribute shall be provided during the creation of the <mgmtObj> resource and shall not be modifiable afterwards. This is mandatory for the <mgmtObj>, for which the data model is not specified by oneM2M but mapped from external management technology data model specified in other device management protocols. |  |
| objectPath | 0..1 | WO | Contains the local path of the external management object instance on the managed entity which is represented by the <mgmtObj> resource in the hosting CSE. This attribute shall be provided during the creation of the <mgmtObj>, so that the hosting CSE can correlate the created <mgmtObj> with the external management object instance on the managed entity for further management operations. It shall not be modifiable after creation. The format of this attribute shall be a local external management object path in the form as specified by existing management protocols. (e.g., "./anyPath/Fw1" in OMA DM [i.5], "Device.USBHosts.Host.3." in BBF TR-069 [i.4]). |  |
| mgmtLink | 0..n | RW | The attribute contains references to another <mgmtObj> resource in case a hierarchy of <mgmtObj> is needed |  |
| [objectAttribute] | 0..n | RW | Each [objectAttribute] is mapped from a leaf node of a hierarchical structured management object (including oneM2M data model and the existing management data models) based on the mapping rules below the table. |  |
| description | 0..1 | RW | Text format description of mgmtObj. |  |

When mapping resources from management technologies to a corresponding <mgmtObj> resource, the following rules shall apply:

- The root resource of external management objects maps to the <mgmtObj> resource

- For the child resource of the root resource of specific technology:

- Rule1: If the child external management object cannot have a child external management object, the external management object maps to the [objectAttribute] attribute of the <mgmtObj> resource with the same resource name.

- Rule2: If the child external management object can have another child external management object, the external management object maps to a new <mgmtObj> resource.  The URI of the new <mgmtObj> resource is stored as an mgmtLink attribute of the <mgmtObj> resource which is mapped from the parent external management object.

### 9.6.16	Resource Type mgmtCmd

The <mgmtCmd> resource represents a method to execute management procedures or to model commands and remote procedure calls (RPC) required by existing management protocols (e.g., BBF TR-069 [i.4]), and enables AEs to request management procedures to be executed on a remote entity. It also enables cancellation of cancellable and initiated yet unfinished management procedures or commands.

Figure 9.6.16-1: Structure of <mgmtCmd> resource

[9.6.16.b0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of mgmtCmd resource.

Each <mgmtCmd> corresponds to a specific type of management command, as defined by its attribute cmdType. For multiple requests of the same management command, <mgmtCmd> may use the child-resource (i.e., <execInstance>) to contain all execution instances. The execution of the management procedure represented by <mgmtCmd> shall be triggered using the UPDATE method to its attribute "execEnable".

This resource shall contain the child resources in Table 9.6.16-1.

- Table 9.6.16-1: Child resources of <mgmtCmd> resource

| Child Resource Name of <mgmtCmd> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <execInstance> | 1 | See clause 9.6.17 |

The <mgmtCmd> resource shall contain the attributes described in Table 9.6.16-2.

- Table 9.6.16-2: Attributes of <mgmtCmd> resource

| Attribute Name of <mgmtCmd> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| description | 0..1 | RW | The text-format description of this resource |
| cmdType | 1 | WO | The type to identify the management operation (e.g., download). |
| execReqArgs | 0..1 | RW | Structured attribute (e.g., abstract type) to contain any command-specific arguments of the request. |
| execEnable | 1 | RO | The attribute can be blank without any value or it can contain a URI that can be used to trigger execution of <mgmtCmd> using UPDATE method. |

[9.6.17.a]   Editor's Note: The types of management operations (e.g., cmdType) to be used by mgmtCmd are FFS.

### 9.6.17	Resource Type execInstance

The <execInstance> resource represents a successful instance of <mgmtCmd> execution request, which had been triggered by a M2M network application using the UPDATE method to the attribute "execEnable" of <mgmtCmd>.

Figure 9.6.17-1: Structure of <execInstance> resource

[9.6.17.a0] Editor's Note; Verify if the picture above needs to be updated as per the new strucutre of execInstance resource.

The <execInstance> resource shall contain the child resource in Table 9.6.17-1.

- Table 9.6.17-1: Child resources of <execInstance> resource

| Child Resource Name of <execInstance> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |

The <execInstance> resource shall contain the attributes described in Table 9.6.17-2.

- Table 9.6.17-2: Attributes of <execInstance> resource

| Attribute Name of <execInstance> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described |
| expirationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| execStatus | 1 | RO | The status of <execInstance>. It can be Initiated, Started, Finished, Cancelled, or Deleted. |
| execResult | 1 | RO | The execution result of <execInstance>. |
| execDisable | 0..1 | RW | The attribute is used to cancel <execInstance> using UPDATE method. |
| execTarget | 1 | RW | M2M-Node-ID of the target. |
| execMode | 0..1 | RW | Modes used to specify how the command will be executed (e.g., Immediate Once, Immediate and Repeatedly, Random Once, Random and Repeatedly). May be used together with execFrequency, execDelay and execNumber to provide the scheduling information. |
| execFrequency | 0..1 | RW | The minimum interval between two executions, to be used in conjunction with execMode. Modes involving random ececution can be used to add random valuse between individual executions. |
| execDelay | 0..1 | RW | The minimum delay before the instance should be executed. Modes involving random execution can be used to increase this delay randomly. |
| execNumber | 0..1 | RW | The number of times the instance should be executed, to be used when execMode indicates a repetition pattern. |
| execReqArgs | 0..n | RW | Structured attribute (e.g., abstract type) to contain any command-specific arguments used to trigger this <execInstance>. |

### 9.6.18	Resource Type node

The <node> resource represents specific information that provides properties of an oneM2M Node that can be utilized by other oneM2M operations.  The <node> resource has <mgmtObj> as its child resources.  The <mgmtObj> resources represent the Node's context information (e.g., memory and battery), network topology, device information, device capability etc. The <mgmtObj> resources are also resources to perform management of the Nodes.

This node specific information stored in this resource type such as memory and battery can be obtained either by the existing device management technologies (OMA DM [i.5], BBF TR-069 [i.4]) or any other way (e.g., JNI  [i.21])). Since <mgmtObj> resource type represents the management functions including both ways, the types of sub-resources are <mgmtObj> resource type.

For the case that the <node> resource belongs to an ADN, please see the description of nodeLink attribute in the <AE> resource (clause 9.6.5).

Figure 9.6.18-1: Structure of <node> resource

[9.6.18.c-1] Editor's Note: To update the picture to include "hostedCSEID attribute.

[9.6.18.c-2] Editor's Note: Verify if  the picture above needs to be updated as per the new strucutre of node resource.

The <node> resource shall contain the child resources in Table 9.6.18-1.

Table 9.6.18-1: Child resources of <node> resource

| Child Resource Name of <node> | Child Resource Type | Multiplicity | Description |  |
| --- | --- | --- | --- | --- |
| [variable] | <mgmtObj> with mgmtDefinition of value “memory” | 0..1 | This resource provides the memory (typically RAM) information of the node. (e.g., the amount of total volatile memory), See clause D.4. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “battery” | 0..n | The resource provides the power information of the node. (e.g., remaining battery charge). See clause D.7. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “areaNwkInfo” | 0..1 | This resource describes the list of Nodes attached behind the MN node and its physical or underlying relation among the nodes in the M2M Area Network. This attribute is defined in case the Node is MN. See clause D.5. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “areaNwkDeviceInfo” | 0..n | This resource describes the information about the Node in the M2M Area Network. See clause D.6. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “firmware” | 0..1 | This resource describes the information about the firmware of the Node include name, version etc, See clause D.2. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “software” | 0..n | This resource describes the information about the software of the Node. See clause D.3. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “deviceInfo” | 0..n | The resource contains information about the identity, manufacturer, model number of the device. See clause D.8. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “deviceCapability” | 0..n | The resource contains information about the capability supported by the Node. See clause D.9. |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “reboot” | 0..1 | The resource is the place to reboot or reset the Node. See clause D.11 |  |
| [variable] | <mgmtObj> with mgmtDefinition of value “eventLog” | 0..1 | The resource contains the information about the log of events of the Node. See clause D.12. |  |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |  |

The <node> resource shall contain the attributes described in Table 9.6.18-2.

- Table 9.6.18-2: Attributes of <node> resource

| Attribute Name of <node> | Multiplicity | RW/ RO/ WO | Description |  |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |  |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |  |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |  |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |  |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |  |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |  |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |  |
| nodeID | 1 | RW | The ID of the Node which is specified in the resource type. This attribute is identical to M2M-Node-ID. |  |
| hostedCSEID | 0..1 | RW | The ID of the CSEBase that is hosted on this node if type of node is ASN/MN/IN. |  |

### 9.6.19	Resource Type m2mServiceSubscription

This resource represents an M2M Service Subscription. It is used to represent all data pertaining to the M2M Service Subscription (the technical part of the contract between an M2M Application Service Provider and an M2M Service Provider).

Figure 9.6.19-1: Structure of <m2mServiceSubscription> resource

[9.6.19.a0] Editor's Note: Verify if  the picture above needs to be updated as per the new strucutre of the resource.

[9.6.19.b]  Editor's Note: the M2M-Sub-ID storage is FFS

The <m2mServiceSubscription> resource shall contain the child resources in Table 9.6.19-1.

Table 9.6.19-1: Child resources of <m2mServiceSubscription> resource

| Child Resource Name of <m2mServiceSubscription> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| nodeInfo | <nodeInfo> | 0..n | See clause 9.6.20 |

The <m2mServiceSubscription> resource shall contain the attributes in Table 9.6.19-2.

- Table 9.6.19-2: Attributes of <m2mServiceSubscription> resource

| Attribute Name of <m2mServiceSubscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. If no accessControlPolicyIDs is given at the time of creation, the accessControlPolicyIDs of the parent resource is linked to this attribute |
| creationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| subsSer&RoleList | 0..n | WO | This attribute indicates the list of subscribed services and roles. |
| App-ID | 0..n | RW | The value of this attribute shall be set to an application identifier as defined in clause 7.1.3 pertaining to applications in the infrastructure domain (IN-AE) of this M2M service subscription. |
| subsGroup | 0..n | RW | This attribute contains a group created by the subscriber for the purpose of helping easy setting of Access Control Privileges. An example of the subscriber defined group is all node applications within this service subscription with App-ID set to a specific value. |

### 9.6.20	Resource Type nodeInfo

This resource represents M2M node information that is needed as part of the M2M Service Subscription resource. It shall contain information about the node as well as application identifiers running on that node.

Figure 9.6.20-1: Structure of <nodeInfo> resource

[9.6.20.a0] Editor's Note: Verify if  the picture above needs to be updated as per the new strucutre of the resource.

The <nodeInfo> resource shall contain the child resource in Table 9.6.20-1.

Table 9.6.20-1: Child resources of <nodeInfo> resource

| Child Resource Name of <nodeInfo> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |

The <nodeInfo> resource shall contain the attributes described in Table 9.6.20-2.

Table 9.6.20-2: Attributes of <nodeInfo> resource

| Attribute Name of <nodeInfo> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. If no accessControlPolicyIDs is given at the time of creation, the accessControlPolicyIDs of the parent resource is linked to this attribute |
| creationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| nodeID | 1 | RW | Node identifier indicating where the M2M Service Subscription applies. A wildcard may be used to allow all the Nodes within a M2M Service Provider. |
| App-ID | 0..n | RW | See clause 7.1.3. Application ID pertaining to an application running on this node |
| CSE-ID | 0..1 | RW | CSE-ID pertaining to this node (for nodes that have a CSE). The CSE-ID becomes know once the pre CSE registration procedure is triggered. |

### 9.6.21	Resource Type pollingChannel

The <pollingChannel> resource shall represent a channel that can be used for a request-unreachable entity (i.e., an AE or a CSE which is behind NAT so it cannot receive a request from others). The request-unreachable entity creates a <pollingChannel> resource on a request-reachable CSE, and it polls any type of request(s) for itself from the <pollingChannel> hosting CSE. For example, an AE can retrieve notifications by long polling on the channel when it cannot receive notifications asynchronously from a subscription hosting CSE.

This <pollingChannel> resource shall have longPollingURI virtual attribute which is an input/output end point of the channel. After an AE or CSE creates this resource, it performs long polling on longPollingURI of this resource with RETRIEVE operation. The response to the long polling request is pending until there are any requests received on the channel.

Figure 9.5.22-1: Structure of <pollingChannel> resource

[9.6.21.a0] Editor's Note: Verify if  the picture above needs to be updated as per the new strucutre of the resource.

The <pollingChannel> resource shall contain the attributes in Table 9.6.21-1.

Table 9.6.21-1: Attributes of <pollingChannel>

| AttributeName of <pollingChannel> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |
| creator | 0..1 | WO | The AE-ID of the entity which created the resource. |
| longPollingURI | 1 | WO | A virtual attribute that represents a polling URI. This URI is used for long polling. Accessing this attribute is intended to store request(s) (clause 10.2.13.6) and retrieve the request(s) (clause 10.2.13.7). |

### 9.6.22	Resource Type statsConfig

The <statsConfig> resource shall be used to store configuration of statistics for AEs. The <statsConfig> may be established by the IN-CSEs or by AEs in IN-CSE.  The <statsConfig> resource shall be located directly under <CSEBase>.. <eventConfig> sub-resource shall be used to define events that trigger statistics collection. Below are some examples of events that can be generated:

- Collection based on a certain operation: collects any RETRIEVE operations on the data created by the collecting entity

- Collection based on storage size: collects the size of storage when a <container> resource created by the collecting entity exceeds a quota

- Combined configuration: collects all RETRIEVE operations on the data created by the collecting entity during a period of time

Figure 9.6.22-1: Structure of <statsConfig> resource

[9.6.22.a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <statsConfig> resource shall contain the child resources in Table 9.6.22-1.

Table 9.6.22-1: Child resources of <statsConfig> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <eventConfig> | 0..n | See clause 9.6.23. This resource configures an event for statistics collection. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <statsConfig> resource shall contain the attributes described in Table 9.6.22-2.

Table 9.6.22-2: Attributes of <statsConfig> resource

| Attribute Name of <statsConfig> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 1..n | RW | See clause 9.6.1 where this common attribute is described |
| creator | 1 | RW | The AE-ID or CSE-ID of the entity which created the resource. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described |

### 9.6.23	Resource Type eventConfig

Figure 9.6.23-1: Structure of <eventConfig> resource

[9.6.23.a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <eventConfig> resource shall contain the child resource in Table 9.6.23-1.

Table 9.6.23-1: Child resources of <eventConfig> resource

| Child Resource Name of <eventConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where this type of resource is described. |

The <eventConfig> resource shall contain the attributes described in Table 9.6.23-2.

Table 9.6.23-2: Attributes of <eventConfig> resource

| Attribute Name of <eventConfig> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 1..n | RW | See clause 9.6.1 where this common attribute is described |
| creator | 1 | RW | The AE-ID or CSE-ID of the entity which created the resource. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| eventID | 1 | RO | This attribute uniquely identifies the event to be collected for statistics for AEs. |
| eventType | 1 | RW | This attribute indicates the type of the event, such as timer based, data operation, storage based, etc. |
| eventStart | 0..1 | RW | This attribute indicates the start time of the event. |
| eventEnd | 0..1 | RW | This attribute indicates the end time of the event |
| transactionType | 0..1 | RW | This attribute defines the type of the operation to be collected by statistics, such as CREATE, RETRIEVE. |
| dataSize | 0..1 | RW | This attribute defines the data size if an event is triggered when the stored data exceeds a certain size. |

### 9.6.24	Resource Type statsCollect

The resource type <statsCollect> shall be used to collect information for AEs using the <eventConfig> resource as the triggers for the IN-CSE. The IN-CSE may setup multiple triggers. Each trigger may be activated or de-activated independently of others. The <statsCollect> resource shall be located directly under <CSEBase> of IN-CSE.

Figure 9.6.24-1: Structure of <statsCollect> resource

[9.6.24.a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <statsCollect> resource shall contain the child resource in Table 9.6.24-1.

Table 9.6.24-1: Child resources of <statsCollect> resource

| Child Resource Name of <statsCollect> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <statsCollect> resource shall contain the attributes described in Table 9.6.24-2.

Table 9.6.24-2: Attributes of <statsCollect> resource

| Attribute Name of <statsCollect> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 1..n | RW | See clause 9.6.1 where this common attribute is described |
| creator | 1 | RW | The AE-ID or CSE-ID of the entity which created the resource. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| statsCollectID | 1 | RO | This is the unique ID to identify a specific statistics collection scenario. It is created by the IN-CSE when the <statsCollect> resource is first created. |
| collectingEntityID | 1 | WO | This is the unique ID of the entity that requests the collection of statistics. For example, it can be an AE-ID or CSE-ID. |
| collectedEntityID | 1 | WO | This is the unique ID of the entity whose operations at IN-CSE will be collected. For example, it can be an AE-ID or CSE-ID. If no specific value is provided for this attribute, the IN-CSE interprets it as “any entity”. |
| status | 1 | RW | This attribute indicates whether the rule is “active” or “inactive”. |
| statModel | 1 | RW | This attribute indicates the collection model, such as “Subscriber based”, “event based”, etc. |
| M2M-Sub-ID | 1 | WO | This attribute indicates the service subscription Identifier (M2M-Sub-ID) being recorded. |
| collectPeriod | 0..1 | RW | This attribute defines the duration to collect service statistics information. |
| eventID | 0..1 | RW | This attribute refers to the <eventConfig> resource that defines the events that can be collected by the IN-CSE. It is mandatory if the statmodel attribute is set to “event based”. |

### 9.6.25	Resource Announcement

A resource can be announced to one or more remote CSEs to inform the remote CSEs of the existence of the original resource.  An announced resource can have a limited set of attributes and a limited set of child resources from the original resource.  The announced resource includes a link to the original resource hosted by the original resource-hosting CSE.

In case that the original resource is deleted, all announced resources for the original resource shall be deleted. If the announced resource is not deleted promptly (e.g., the announced resource is not reachable), the announced resource can be deleted later either by the original resource hosting CSE or by the expiration of the announced resource itself. The original resource shall store the list of links for the announced resources for those purposes.

Synchronization between the attributes announced by the original resource and the announced resource is the responsibility of the original resource hosting CSE. The access control policy for the announced resource shall synchronize with the one from the original resource. In case that the attribute accessControlPolicyIDs is not present in the original resource it is the responsibility of the original resource hosting CSE to choose the appropriate value depending on the policy for the original resource (e.g., take the parent accessControlPolicyIDs value).

The original resource shall have at least announceTo attribute present if the resource itself has been announced. If any of the Optional Announced (OA) attributes are also announced, then announcedAttribute attribute shall also be present. An AE or other CSE can request the original resource hosting CSE for announcing the original resource to the list of CSE-IDs or the URI(s) listed in the announceTo attribute in the announcing request. An Update to the announceTo attribute will trigger new resource announcement(s) or the de-announcement(s) of the announced resource. After a successful announcement procedure the attribute announceTo contains only the list of URI(s) of the announced resources.

In order to announce the attribute marked as OA (see clause 9.5.1.1), they shall be included in the announcedAttribute attribute at the original resource. The attributes included in the announcedAttribute attribute are announced to the announced resource. On successful announcement of the resource, such attributes shall be created at the announced resource; otherwise they shall not be present in the announced resource. Update to the announcedAttribute attribute in the original resource will trigger new attribute announcement or the de-announcement of the announced attribute(s). The announced attributes shall have the same value as the original resource, and synchronization between the value of the announced attributes at the original resource and the announced resource is the responsibility of the original resource hosting CSE.

An announced resource may have child resources. The child resources shall be of the same type of the original child resources or their associated Announce type.

Child resources of the original resource can be announced independently as needed. In this case, the child resources at the announced resource shall be of the child resource Announced Type. When a child resource at the announced resource is created locally at the remote CSE, the child resource shall be of normal child resource type.

#### 9.6.25.1	Common Attributes for Announced Resources

Table 9.6.25-1 lists the common attributes for the announced resources.

Table 9.6.25-1: Common Attributes for Announced Resources

| Common Attribute | Mandatory /Optional | Description |
| --- | --- | --- |
| resourceType | Mandatory | Resource Type. As specified in clause 9.2, a suffix of "Annc" to the name of the original resource type shall be used to indicate the name for the associated announced resource type. |
| parentID | Mandatory | Identifies the parent resource. |
| accessControlPolicyIDs | Mandatory | The list of identifiers (either an ID or a URI ) of an <accessControlPolicy> resource announced by the original resource See clause 9.6.1 for further information on this attribute. If this attribute was not present in the original resource, the original resource shall include this attribute by providing the accessControlPolicyIDs from the original resource’s parent resource or from the local policy according at the original resource. |
| creationTime | Mandatory | See clause 9.6.1 for information on this attribute. |
| expirationTime | Mandatory | See clause 9.6.1 for information on this attribute. This attribute is limited by the value received from the original resource but it can be overridden by the policies of the remote CSE hosting the announced resource. |
| lastModifiedTime | Mandatory | See clause 9.6.1 for information on this attribute. |
| labels | Conditionally Mandatory | Tokens used as keys for discovering resources as announced by the original resource. See clause 9.6.1 for further information on this attribute. The attribute is conditionally mandatory, which means that the attribute shall exist in the announced resource if it is present in the original resource. |
| link | Mandatory | Provides the URI to the original resource. |

# 10	Information Flows

[10.a] Editor's Note: It is FFS about the suitable clause for the proposed generic call flows and potential re-org of the clauses due to the correlation of current clauses 8 and 10.

10.1	Basic Procedures

As a pre-condition to the execution of the following procedures, M2M operational security procedures as specified in clauses 11.3.1 through 11.3.3 shall have been performed. In case of failure, the error shall be reported as specified in [TS0003].

The procedures in the following sub-clauses assume blocking requests as described in clause 8.2.2.

### 10.1.1	CREATE (C)

The CREATE procedure shall be used by an Originator CSE or AE to create a resource on a Receiver CSE (also called the Hosting CSE). The description of CREATE procedure has been divided in two separate clauses, since there is a need to distinguish between Registration related Create and Non-Registration related Create procedures.

The Registration related Create procedure is applicable for the following resource types only:

- <AE>, and

- <remoteCSE>

Whereas non-registration related Create procedure is applicable for all other resource types described in clause 9.6.

#### 10.1.1.1	 Non-registration related CREATE procedure

This procedure is valid for all resources which are not related to registration.

As a pre-condition for this procedure, the Originator has been already successfully authenticated, with the procedure described in [TS0003]. Therefore all exceptions that are related to authentication failure are not described in this clause.

Originator requests to create a resource by using the CREATE method. See clause 8.1.2 for the parameters to be included in the Request message.

Receiver, once the Originator has been successfully authenticated and that the request is allowed by the given privileges, shall validate request and shall create the resource.

- Figure 10.1.1-1: Procedure for CREATEing a Resource

Step 001: The Originator shall send the following parameters in the CREATE Request message:

op: 	C (Create);

to: 	URI of the target resource where the new resource should be created (parent resource)

- fr: 	ID of the Originator (either the AE or CSE)

[10.1.1.1.b]  Editor's Note: AE or CSE may not have the ID upon its registration request. Alternative information for fr is FFS.

nm:  optional name of the resource to be provided by the Originator, where permitted by the resource type as specified in clause 9.6.

NOTE: Some of the resources defined in clause 9.6 have a predefined name. For these resources the parameter nm is not applicable.

cn: 	attributes of the resource to be provided by the Originator. Of particular importance for the CREATE Request message is the common attribute resourceType (clause 9.6.1) which identifies the type of the resource to be Created.

[10.1.1.1.c]  Editor’s Note: which parameters are deducted to be mandatory/optional in the request is for FFS. Clause 8.1 should provide a generic mechanism to identify mandatory params for a request by looking at how the resource is defined. E.g., If the attribute in the resource definition is identified as mandatory and it is of type RW, then it is also intended as mandatory in the request.

[10.1.1.1.d]   Editor's Note: references to elements defined in sect 8 have been added here. They may be removed if we decide to move sect 8.1.2-4 to stage 3 doc. In this case, these references will be removed and sect 1 of this document would replace sect 8.1.2-4 in the TS-0001.

Step 002: The Receiver shall:

- Check if the Originator has the appropriate privileges for performing the request. Privileges are part of the attribute accessControlPolicyID of the targeted resource. In case that such an attribute does not exist, the Receiver shall check the accessControlPolicyID of the parent resource.

[10.1.1.1.e]   Editor’s Note: how far should the Receiver go back to a parent to look for an accessControlPolicyID is FFS

- Verify that the suggested nm, if provided by the Originator in the Create Request message, does not already exist in its resource structure.

- Assign an identifier to the resource to be created if nm is not provided by the Originator.

- Assign/modify default values for certain mandatory attributes of the resource, where allowed by the resource itself and if not provided by the Originator itself.

- the Receiver shall assign a value to the following common attributes specified in clause 9.6.1:

- parentID;

- creationTime;

- expirationTime: if not provided by the Originator, the Receiver shall assign the maximum value possible (within the restriction of the Receiver policies). If the value provided by the Originator cannot be supported, due to either policy or subscription restrictions, the Receiver will assign a new value.

- lastModifiedTime: which is equals to the creationTime;

- Any other RO (Read Only) attributes within the restriction of the Receiver policies.

- On successful validation of the Create Request, the Receiver shall create the requested resource.

Step 003: The Receiver shall respond with a Response message that shall contain the following parameters:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request , rd information can be used.

- fr: Optional. ID of the Receiver.

cn:	URI and optionally the content of the created resource. The Receiver shall provide the content if the created resource contains attributes which were modified by the Receiver in step 002.

See clauses 8.1.3 and 8.1.4 for the parameters to be included in the Response message.

- General Exceptions:

- The Originator does not have the privileges to create a resource on the Receiver. The Receiver responds with an error.

- The resource with the specified name (if provided) already exists at the Receiver. The Receiver responds with an error.

- The provided information in cn is not accepted by the Receiver (e.g., missing mandatory parameter). The Receiver responds with an error.

#### 10.1.1.2	Registration related CREATE procedure

This clause describes the CREATE procedure for <remoteCSE> and <AE> resource type.

##### 10.1.1.2.1	 CSE Registration procedure

The procedure for CSE Registration follows the procedure described in clause 10.1.1.1, but with some exceptions. Below is the detailed description on how to perform the CSE Registration and which part of the procedure deviates from the one described in clause 10.1.1.1.

The Registration procedure requires the creation of two resources (a <remoteCSE> on the Receiver CSE and a <remoteCSE> on the Originator CSE) rather than one resource. The Registration procedure is always initiated by a CSE in the field domain and follows the restrictions described in clause 6.2.9.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in [TS0003], the Receiver shall allow the creation of the <remoteCSE> resource since at registration time there are no appropriate privileges associated with the AE yet.

Figure 10.1.1.2.1-1: Procedure for CREATEing a <remoteCSE> Resource

All the parameters of the request and steps that are not indicated do not deviate from clause 10.1.1.1.

Step 001: The Originator shall send the following information in the CREATE Request message:

fr:	Credentials of the Originator, CSE-ID, as defined in [TS0003].

cn: 	Attributes of the resource to be provided by the Originator. Of particular importance for the CREATE Request message is the common attribute resourceType from clause 9.6.1, which identifies the type of resource; and the attributes CSE-ID and CSEBase. The CSEBase shall contain the absolute URI of the <CSEBase> resource at the Receiver.

Step 002:  The Receiver shall:

- This step from 10.1.1.1 cannot be performed for the creation of <remoteCSE> resource.

All the other steps: 2-6, from step 002 from clause 10.1.1.1 are still applicable

NOTE:     Optionally, if the M2M Service Provider supports inter-domain communication, the Receiver could perform this step if the attribute CSEBase (part of the cn parameter of the request) contains the public domain of the CSE. The Receiver could construct the domain as described in clause XXX. The Receiver could add an AAA or AAAA record in DNS with the public domain name of the Originator CSE and the IP address of the IN-CSE associated with the Originator

[10.1.1.2.1.a]   Editor’s Note: The clause that should refer to clause 6.5 somewhere where the public domains of CSEs are defined.

Step 003: See clause 10.1.1.1

Step 004: The Originator, upon receipt of the CREATE response message, shall create a <remoteCSE> resource locally under its <CSEBase> resource. This resource is representing the Receiver CSE. The Originator shall provide the appropriate values to all mandatory parameters as described in clause 9.6.4.

Step 005: The Originator may issue a RETRIEVE Request towards the Receiver (same to as for the CREATE request message) to obtain the optional parameters of the <remoteCSE> resource created at the Receiver as for step 004 (e.g., labels, accessControlPolicyID attributes). The RETRIEVE procedure is described in 10.1.2.

See clauses 8.1.2 for the information to be included in the Request message.

Step 006: The Receiver verifies that the Originator has the appropriate privileges to access the information.

Step 007: The Receiver sends a RETRIEVE response message, according to the procedure described in 10.1.2.

See clauses 8.1.3 and 8.1.4 for the information to be included in the Response message.

Step 008: The Originator shall update the created <remoteCSE> resource for the Receiver with the information obtained in step 007.

General Exceptions:

All exceptions from 10.1.1.1 are applicable; in addition the following exception may occur:

- The Originator does not have the privileges to retrieve the attributes of the Receiver CSE. The Receiver responds with an error.

##### 10.1.1.2.2	 Application Entity Registration procedure

The procedure for AE registration follows the procedure described in clause 10.1.1.1 with the following exception:

Receiver, once the Originator has been successfully authorized (as described in [TS0003]); the Receiver shall allow the creation of the <AE> resource according to the access control policy. The Receiver shall use, if present, the suggested name from the information parameter nm in the request. If the suggested name of the resource cannot be used (i.e., a resource with that name already exists) the Receiver shall reject the CREATE Request. If the request does not provide the optional nm, then the Receiver shall create the resource and assign a name to it.

[10.1.1.2.2.a]    Editor’s Note: which accessControlPolicy needs to be used is FFS.

Step 001: The Originator shall send the following information in the CREATE Request message:

fr: credentials of the Originator, as defined in [TS0003]

Step 002: for the <AE> resource the Receiver cannot perform the action described in the first step of clause 10.1.1.1:

All the other parameters of the request and the steps that follow do not deviate from clause 10.1.1.1.

### 10.1.2	RETRIEVE (R)

The RETRIEVE operation shall be used for retrieving the information stored for any of the attributes for a resource at the Receiver CSE. The Originator CSE or AE may request to retrieve a specific attribute by including the address (e.g., URI) of such attribute in the request message.

Originator requests retrieval of all attributes or a specific attribute of the target resource by using RETRIEVE Request. See clause 8.1.2 for the information to be included in the Request message. If only a specific attribute needs to be retrieved, the address of such attribute (e.g., the URI) shall be included in the cn information of the request message.

Receiver once the Originator has been successfully authenticated; the Receiver performs local processing to verify the existence of requested resource and checks privileges for retrieving the information related to the resource. After the successful verification, the Receiver shall return the requested information, otherwise an error indication shall be returned.

- Figure 10.1.2-1: Procedure for RETRIEVing a Resource

Step 001: The Originator shall request to RETRIEVE a resource or a specific attribute within a resource at the Receiver.

op:	R (Retrieve)

to: 	URI of the target resource or the target attribute

- fr:	 ID of the Originator (either the AE or CSE)

Step 002: The Receiver shall verify the existence of the target resource or the attribute and check if the Originator has appropriate privileges to retrieve information stored in the resource/attribute.

Step 003: The Receiver shall respond with a Response message that shall contain the following information:

to:	Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request, rd information can be used.

- fr:	Optional. ID of the Receiver.

cn:	content of resource/attribute retrieved

- General Exceptions:

- The targeted resource/attribute in to information does not exist. The Receiver responds with an error.

- The Originator does not have privileges to retrieve information stored in the resource on the Receiver. The Receiver responds with an error.

### 10.1.3	UPDATE (U)

The UPDATE operation shall be used for updating the information stored for any of the attributes at a target resource. Especially important is the expirationTime, since a failure in refreshing this attribute may result in the deletion of the resource. The Originator CSE or AE can request to update a specific attribute(s) at the target resource by including the name of such attribute(s) and its values in the request message.

Originator requests update any of the attributes at the target resource by using UPDATE Request message. The Originator shall send new (proposed) values for the attribute(s) that need to be updated. The UPDATE operation allows to modify the attributes (defined in clause 9.6) and that are indicated as “RW” (Read Write) for the specific resource type. See clause 8.1.2 for the information to be included in the Request message.

NOTE: Update operation can also be used for Execute operation. Such use of the Update operation does not use cn parameter.

Receiver once the Originator has been successfully authenticated, the Receiver verifies the existence of the addressed resource, the validity of the attributes provided and the privileges to modify them, shall update the attributes provided and shall return a Response message to the Originator with the operation results as specified in clause 8.1.3.

- Figure 10.1.3-1: Procedure for UPDATing a Resource

Step 001: The Originator shall send the following parameters in the UPDATE Request message:

op:	U (Update)

to: 	URI to the target resource

- fr: 	ID of the Originator (either the AE or CSE)

cn: 	Information related to the attribute(s) to be updated at the target resource. The name of such attribute(s) and associated updated values in the cn parameter. For the Update operation used for Execute operation, cn parameter does not exist.

Step 002: The Receiver shall validate if the Originator has appropriate privileges to perform the modification to the target resource. On successful validation, the Receiver shall update the resource as requested.

Step 003: The Receiver shall respond with a Response message that shall contain the following parameters:

to:	Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request, rd information can be used.

- fr:	Optional. ID of the Receiver.

rs:	Operation result

- General Exceptions:

- The targeted resource in to information does not exist. The Receiver responds with an error.

- The Originator does not have the privilege to modify the resource on the Receiver. The Receiver responds with error.

- The provided information in the cn is not accepted by the Receiver. The Receiver responds with error.

### 10.1.4	DELETE (D)

The DELETE operation shall be used by an Originator CSE or AE to delete a resource at a Receiver CSE. The DELETE procedure shall consist of the deletion of all related information of the target resource.

Originator requests deletion of a resource by using a DELETE Request message. See clause 8.1.2 for the information to be included in the Request message.

Receiver once the Originator has been successfully authenticated, the Receiver verifies the existence of the requested resource, and the privileges for deleting the resource.

- Figure 10.1.4-1: Procedure for DELETING a Resource

Step 001: The Originator shall send a DELETE Request message to the Receiver.

op:	D (Delete)

to: 	URI of the target resource

- fr: 	ID of the Originator (either the AE or CSE)

Step 002: The Receiver shall verify the existence of the requested resource and if the Originator has the appropriate privilege to delete the resource. On successful validation, the Receiver shall check for child resources and delete all child resources and the associated references in parent resources and it shall remove the resource itself.

Step 003: The Receiver shall respond with a Response message that shall contain the following information:

to:	Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request, rd information can be used.

- fr:	Optional. ID of the Receiver.

rs:	Operation result

- General Exceptions:

- The targeted resources in to information does not exist. The Receiver responds with an error.

- The Originator does not have the privileges to delete the resource on the Receiver. The Receiver responds with an error.

### 10.1.5	NOTIFY (N)

The NOTIFY operation shall be used for notifying information.

Originator: The Originator requests to notify an entity by using NOTIFY method. See clause 8.1.2 for the information to be included in a Request message.

Receiver: The Receiver responds to the Originator with the operation results as specified in clause 8.1.3.

- Figure 10.1.5-1: Procedure for NOTIFYing Information

[10.1.5.a]     Editor’s Note: not sure how to align this procedure with the other ones. The Receiver should also have a local processing, but if the address of the resource in the to is not one of the defined resource types what the receiver does is out of scope. However if the NOTIFY will corresponds to a create instance then the procedures described in the create should apply. This operation needs more refining.

Step 001: A notification to be sent to the Receiver is triggered in the Originator.

Step 002: The Originator shall send the following parameters in the NOTIFY Request message:

op:	N (Notify)

to: 	URI where the notification should be sent to

- fr:	ID of the Originator (only a CSE)

cn:	Notification data and/or Notification reference

Step 003: The Receiver responds with a Response message that shall contain the following information:

rs: 	operation result

- General Exceptions:

- None identified.

[10.1.5.b]     Editor's note: General exceptions are FFS in particular for notification failures.

## 10.2	Resource Type-Specific Procedures

The procedure for the corresponding operations as specified in clause 10.1 shall be performed with the modifications specific to the resource type procedures.

NOTE: Where the procedures in caluse 10.2 conflict with the procedures in clause 10.2. the procedures in clause 10.2 take precedence.

### 10.2.1	<AE> Resource

#### 10.2.1.1	Create <AE>, aka Application Registration

This flow is used for creating an <AE> resource. This operation is part of the registration procedure for AEs on the Registrar CSE (which is also the Hosting CSE), as described in clause 10.1.1.2.2

Originator: The Originator shall be an Application Entity. The Originator may suggest in the request the AE-ID.

Receiver: Once the Originator has been successfully authorized (as described in TS-0003 [TS0003]), the Receiver (which is also the Hosting CSE) shall grant the Request after successful validation of the Request.

The Create procedure shall be according to Table 10.2.1.1-1.

[10.2.1.1.a]  Editor’s Note: Currently there is no clause with specific text on the authentication/access control from SEC.

[10.2.1.1.b] Editor’s note: It is not clear if the case of non-authenticated AE can register to a local CSE, but with a very limited access. This case is FFS.

[10.2.1.1.c]  Editor’s note: the relation with the accessControlPolicy is for FFS.

Table 10.2.1.1-1 <AE> CREATE flow

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: fr: Credentials of the Originator, as defined in [TS0003] [10.2.1.1.d ] Editor’s Note: More investigation is needed in WG4 to clarify what will be the value of fr and in which clause it will be described. nm: The name of the <AE> resource is optional. cn: The resource content shall provide the information as defined in clause 9.6.5. The following attributes from clause 9.6.5 are mandatory for the request: - “resourceType” which shall be set to the appropriate tag that identify the AE resource as defined in 9.6.1 - “name” - “App-ID” The following attributes from clause 9.6.5 are optional for the request: - “expirationTime” - “labels” - “AE-ID” - “ontologyRef” [10.2.1.1.e] Editor’s Note: fix Table 7 for optional AE-ID |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1). The hosting CSE verifies that the Originator is an authenticated AE. |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the table with the specific details for: cn: URI of the created <AE> resource, according to clause 10.1.1 |
| Post-Conditions |  |
| Exceptions | All exception described in the basic procedure (clause 10.1.1) are applicable. |

#### 10.2.1.2	Retrieve <AE>

This flow is used for retrieving the representation of the <AE> resource with its attributes.

Originator: The Originator can be an Application Entity or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), the Receiver shall retrieve the <AE> resource.

Table 10.2.1.2-1 <AE> RETRIEVE flow

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the table with the specific details for: cn: void |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.2) |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the table with the specific details for: cn: attributes of the <AE> resource as defined in clause 9.6.5. |
| Post-Conditions |  |
| Exceptions | All exception described in the basic procedure (clause 10.1.2) are applicable. |

#### 10.2.1.3	Update <AE>

This flow is used for updating the attributes and the actual data of an <AE> resource.

Originator: The Originator shall be an Application Entity.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), , the Receiver shall update the <AE> resource.

Table 10.2.1.3-1 <AE> UPDATE flow

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the table with the specific details for: cn: attributes of the <AE> resource as defined in 9.6.5 which need be updated, with the exception of the following that cannot be modified: - “lastModifiedTime” - ”name”, “App-ID" - “AE-ID”. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.3) |
| Information on Response message | No change from the basic procedure (clause 10.1.3) |
| Post-Conditions |  |
| Exceptions | All exception described in the basic procedure (clause 10.1.3) are applicable. |

#### 10.2.1.4	Delete <AE>, aka De-Register

This flow is used for de-registering and deleting the <AE> resource with all related information.

Originator: The Originator can be an Application Entity or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), the Receiver shall delete the <AE>.

Table 10.2.1.4-1 <AE> DELETE flow

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.4). |
| Information on Response message | No change from the basic procedure (clause 10.1.4) |
| Post-Conditions |  |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable. |

### 10.2.2	<remoteCSE> Resource

#### 10.2.2.1	Create <remoteCSE>, aka CSE Registration

This flow is used for creating a <remoteCSE> resource. It is part of the registration procedure for remote CSEs on the Registrar CSE (which is also the Hosting CSE), as described in clause 10.1.1.2.1

Originator:  A CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), the Receiver shall grant the request after successful verification of the request.

If the IN-CSE is the receiver and if the M2M SP policies do allow access to the CSEs across multiple M2M domains, then the IN-CSE shall create the appropriate entry in the M2M SP's DNS for a successfully registered CSE.

Table 10.2.2.1-1 <remoteCSE> CREATE flow

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable plus specifically: fr: CSE-ID. nm: the name of the <remoteCSE> resource is optional. cn: the resource content shall provide the information as defined in clause 9.6.4. The following attributes from clause 9.6.4 are mandatory for the request: - “resourceType” which shall be set to the appropriate tag that identify the <remoteCSE> resource as defined in 9.6.1 - “CSE-ID”: which contains the CSE-ID - “CSEType” The following attributes from clause 9.6.4 are optional for the request: - “expirationTime” - “accessControlPolicyID” - “labels” - “pointOfAccess” |
| Local processing on Hosting CSE | No change from the basic CREATE procedure (clause 10.1.1) If the IN-CSE is the receiver and if the M2M SP policies do allow access to the CSEs across multiple domains, then the IN shall create the appropriate entry in the M2M SP's DNS for successfully registered CSE. |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the table with the specific details for: cn: URI of the created <remoteCSE> resource, according to clause 10.1.1 |
| Post-Conditions | Two new <remoteCSE> resources are created, one on the targeted CSE (Receiver) and one under the <CSEBase> of the Originator that correspond to the CSE of the Receiver. |
| Exceptions | All exception described in the basic CREATE procedure (clause 10.1.1) are applicable. |

#### 10.2.2.2	Retrieve <remoteCSE>

This flow is used for retrieving the representation of the <remoteCSE> resource with its attributes.

Originator: An AE or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), the Receiver shall retrieve the <remoteCSE> resource.

Table 10.2.2.2-1 <remoteCSE> RETRIEVE flow

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable plus specifically: cn: void |
| Local processing on Hosting CSE | No change from the basic RETRIEVE procedure (clause 10.1.2) |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable plus specifically: cn: attributes of the <remoteCSE> resource as defined in clause 9.6.4. |
| Post-Conditions |  |
| Exceptions | All exception described in the basic RETRIEVE procedure (clause 10.1.2) are applicable. |

#### 10.2.2.3	Update <remoteCSE>, aka Re-registration

This flow is used for updating the attributes and the actual data of an <remoteCSE> resource.

Originator: An AE or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), the Receiver shall update the <remoteCSE> resource.

Table 10.2.2.3-1 <remoteCSE> UPDATE flow

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable plus specifically: cn: attributes of the <remoteCSE> resource as defined in 9.6.4 which need be updated, with the exception of the parameters which are RO (Read Only) or WO (Write Once) in Table 9.6.1.19-2. |
| Local processing on Hosting CSE | No change from the basic UPDATE procedure (clause 10.1.3) |
| Information on Response message | No change from the basic UPDATE procedure (clause 10.1.3) |
| Post-Conditions |  |
| Exceptions | All exception described in the basic UPDATE procedure (clause 10.1.3) are applicable. |

#### 10.2.2.4	Delete <remoteCSE>, aka CSE De-Registration

This flow is used for de-registering and deleting the <remoteCSE> resource with all related information.

Originator: An AE or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause XXX), the Receiver shall delete the <remoteCSE>.

If the IN-CSE is the receiver and it has created an entry in the DNS to allow access to the CSE across multiple M2M domains, then it shall delete the entry from the DNS.

Table 10.2.2.4-1 <remoteCSE> DELETE flow

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable |
| Local processing on Hosting CSE | No change from the basic DELETE procedure (clause 10.1.4). For <remoteCSE>, if the resource is deleted then the Hosting CSE should also delete the correspondent DNS entry, if it exists. If the IN-CSE is the receiver and it has previously created an entry in the DNS, then it shall delete the entry from the DNS. |
| Information on Response message | No change from the basic DELETE procedure (clause 10.1.4) |
| Exceptions | All exception described in the basic DELETE procedure (clause 10.1.4) are applicable. |

### 10.2.3	<container> Resource

#### 10.2.3.1	Create <container>

This flow is used for creating a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause xxx), the Receiver shall create a <container> resource.

- Table 10.2.3.1-1: <container> CREATE

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for the following attributes from clause 9.6.5 are mandatory for the request: nm: Name of <container> resource is optional, cn: The resource content shall provide the information as defined in clause 9.6.3. The following attributes from clause 9.6.3 are mandatory for the Request: - “resourceType” which shall be set to the appropriate tag that identify the Container resource as defined in 9.6.1; - “creator” which it contains the information provided in the fr parameter of the request. The following attributes from clause 9.6.3 are optional for the request: - “expirationTime” - “accessControlPolicyID” - “labels” - “maxNrOfInstances” - “maxByteSize” - “maxInstanceAge” - “locationID”: which is used only if the instances of the container are for location information. - “ontologyRef” [ 10.2.3.1.a] Editor's Note: Clause x.y.z refers to the table which describes the attributes of <container> resource. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1) |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for: - cn: URI of the created <container> resource according to clause 10.1.1. |
| Post-Conditions |  |
| Exceptions | All exceptions described in the basic procedures (clause 10.1.1) are applicable. |

[ 10.2.3.1.b] Editor's Note: The type of the resource to be created (ty) needs to be specified by the contributor as part of "Information on Request message".

#### 10.2.3.2	Retrieve <container>

This flow is used for retrieving the attributes of a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause xxx), the Receiver shall retrieve the <container> resource.

- Table 10.2.3.2-1: <container> RETRIEVE

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for: - cn: void [ 10.2.3.2.a] Editor's Note: Clause 9.6.6 refers to the table which describes the attributes of <container>. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.2) |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for: - cn: attributes of the <container> resource as defined in clause 9.6.6 |
| Post-Conditions |  |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.2) are applicable. |

#### 10.2.3.3	Update <container>

This flow is used for updating the attributes and the actual data of a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause xxx), the Receiver shall update the <container> resource.

- Table 10.2.3.3-1: <container> UPDATE

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.3) |
| Information on Response message | No change from the basic procedure (clause 10.1.3) |
| Post-Conditions |  |
| Exceptions | All exceptions described in the basic procedure (clause10.1.2) are applicable. |

#### 10.2.3.4	Delete <container>

This flow is used for deleting a <container> resource residing under a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the Receiver has been granted (as described in clause xxx), the Receiver shall delete the <container>.

- Table 10.2.3.4-1: <container> DELETE

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions |  |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.4). |
| Information on Response message | No change from the basic procedure (clause 10.1.4) |
| Post-Conditions |  |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.4) are applicable. |

### 10.2.4	Access to Remotely Hosted Resources via <delivery>

#### 10.2.4.1	Introduction to usage of <delivery> resource type

In this introduction an example for delivering information from a source CSE to a target CSE via the use of the <delivery> resource is explained.

The information flow depicted in figure 10.2.4.1-1 defines the exchange of Requests/Responses for processing an original request targeting a resource that is not hosted on the Registrar CSE of the request Originator. The following assumptions hold:

- Originator is AE1;

- AE1 is registered with CSE1, i.e., CSE1 is the Registrar CSE for AE1;

- The original Request is an UPDATE to a remote resource hosted on CSE3, i.e., CSE3 is the Hosting CSE for the target resource;

- UPDATE options in the original Request are selected such that no feedback after completion of the update operation was requested, i.e., AE1 decided that it does not need to hear back from CSE3; this is expressed by setting the rc information to "nothing", see clause 8.1.2.

- Delivery related parameters  included in the original UPDATE request (may be set via CMDH policies):  rqet, ec, da and rp;

- rqet indicates how long the forwarding of the request can last at most

- ec  indicates the event category that should be used by CMDH to handle this request

- rp indicates how long after the request has expired, the local request context should still be available for retrieving status or result information.

- da would be set to ON indicating that <delivery> resource shall be used for forwarding the request

- CSE1 is the CSE of an Application Service Node.

- CSE1 is registered with CSE2 and interacts with CSE2 via the reference point Mcc(1).

- CSE2 is the CSE of a Middle Node

- CSE2 is registered with CSE3 and interacts with CSE3 via the reference point Mcc(2)

- CSE3 is the CSE of an Infrastructure Node.

The Originator AE1 shall get a confirmation from CSE1 when the original Request is accepted. The response informs AE1 that CSE1 has accepted the Request and has accepted responsibility to execute on the requested operation. Furthermore, AE1 has expressed by setting rc to "nothing" that no result of the requested operation is expected to come back from CSE3. With the provided reference (Req-Ref in figure 10.2.2.1-1), AE1 can retrieve the status of the issued request at a later time, for instance to find out if the request was already forwarded to CSE2 or if it is still waiting for being forwarded on CSE1. Before accepting the request from AE1, CSE1 has also verified if the delivery related parameters expressed by AE1 (settings of "rqet" and "ec") are in line with provisioned CMDH  policies. AE1 may not be authorized to use certain values for "rqet" or "ec".

In line with the delivery related parameters, CSE1 is generating a local <delivery> resource on CSE1 and attempts to forward the content of it in line with provisioned CMDH policies at a suitable time and via a suitable connection to CSE2 by requesting the creation of a <delivery> resource on CSE2. In this example case, the lifespan attribute of this delivery resource is set to the same value as the rqet parameter expressed by AE1. In general – i.e., also in cases where more than one original request is aggregated into a single create request for a <delivery> resource – the lifespan and eventCat attributes of the to be created <delivery> resource shall be set consistent with the rqet and ec parameters in the set of original requests. See the attribute definitions in clause 9.6.11.

CSE1 shall use a blocking request for requesting creation of a <delivery> resource on CSE2.

When CSE2 has accepted the incoming request from CSE1, CSE1 may delete the data attribute of the local <delivery> resource. Furthermore - if the expiration time of the local <delivery> resource is not exhausted - the Registrar CSE shall update the status of the local <delivery> resource to indicate that it has been forwarded to CSE2. CSE1 shall also update the status of the original request to indicate that it has been forwarded and it may delete the data attribute of the original request.

When CSE2 has accepted the request to create a local <delivery> resource, it shall attempt to forward it to CSE3. In line with the delivery related parameters, CSE2 shall create a local <delivery> resource on CSE2 and shall attempt to forward it in line with provisioned CMDH policies at a suitable time and via a suitable connection to CSE3 by requesting the creation of a <delivery> resource on CSE3.

CSE2 shall use a blocking request for requesting creation of a <delivery> resource on CSE3.

When CSE3 has accepted the incoming request from CSE2, CSE2 may delete the data attribute of the local <delivery> resource. Furthermore - if the expiration time of the local <delivery> resource is not exhausted - the Registrar CSE shall update the status of the local <delivery> resource to indicate that it has been forwarded to CSE3.

When CSE3 has accepted the request to create a local <delivery> resource, it shall determine that the target of the delivery was CSE3 itself. Therefore it shall forward internally the original request contained in the data attribute of the <delivery> resource.

Within CSE3, functions that are responsible for checking and executing local access to resources in CSE3 will execute the originally requested UPDATE operation. If successful, the targeted resource will be updated with the content provided by the Originator.

Since in the depicted case no result needed to be sent back to the Originator, the processing for the requested operation is then completed.

Figure 10.2.4.1-1: CMDH information flow for 2 hops - 
no result needs to be returned after operation completes

The following procedures shall be triggered by requesting the corresponding operations on a <delivery> resource:

- Initiate the delivery of one or more original request(s) stored for later forwarding from one CSE to another CSE:

- Request a CREATE operation for a <delivery> resource from an issuing CSE to a receiving CSE.

- The original request(s) need to be contained in the "data" attribute of the <delivery> resource.

- If successful, the receiving CSE takes the responsibility to further execute on the delivery process for the original Request.

- If not successful, the issuing CSE cannot assume that the receiving CSE will carry out the delivery of the original request.

- Get information about the status of a pending delivery process for an original request:

- Request a RETRIEVE operation of the content of a <delivery> resource representing a pending delivery or part of it.

- The status of the pending forwarding process is reflected the "deliveryMetaData" attribute defined in the <delivery> resource.

- Change parameters of pending delivery process:

- Request an UPDATE operation on applicable attributes of the <delivery> resource representing the pending delivery.

- For instance the time allowed for completion of a delivery process could be modified by updating the "lifespan" attribute of an existing <delivery> resource.

- Cancel a pending delivery request:

- Request a DELETE operation of a <delivery> resource that represents a pending delivery process.

#### 10.2.4.2	Create <delivery>

This procedure shall be used for requesting a CSE to take responsibility to deliver the provided data to a target CSE in line with CMDH parameters and provisioned CMDH policies in case <delivery> resource based CMDH processing is used. If indicated by the Originator, the Receiver shall confirm the acceptance of delivery responsibility by a successful Response.

Originator: The Originator of a Create request for a <delivery> resource can only be a CSE. The Originator needs to provide the content of a <delivery> resource type together with the Create request or can Update it after a successful creation of the <delivery> resource with empty data attribute. Otherwise the Receiver cannot accept the Create Request. The Originator shall use a blocking request for issuing the Create request to the Receiver.

Receiver: The receiver of a Create request for a <delivery> resource is a Registrar CSE or the Originator and it shall check whether the Originator is authorized to request a delivery procedure. The Receiver of the Create Request shall further check whether the provided attributes of the <delivery> resource that is requested to be created represents a valid request for forwarding data to a target CSE. If the Originator of the Create request is authorized and the Request is valid, the Receiver shall check whether it can actually satisfy the requested delivery in line with provisioned CMDH policies and requested eventCat and lifespan attributes of the <delivery> resource. If all these checks are positive, the Receiver shall create the requested <delivery> resource and assumes responsibility for delivering the requested data to the target CSE as soon as the content of the data attribute is available. In case an operation result is expected by the Originator, the Receiver shall confirm acceptance of the responsibility by indicating a successful creation of the <delivery> resource. If the Receiver CSE is the target CSE of the requested delivery, it shall forward the content of the delivered data - which represents one or more forwarded original request(s) - to the internal functions that handle incoming requests and continue processing of the forwarded request(s).

- Table 10.2.4.2-1: <delivery> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | Originator is a CSE and is responsible for delivery of data to a target CSE. Originator needs to forward the data and delivery responsibility to the Receiver. Receiver is a Registrar CSE of the Originator. |
| Information on Request message | rt: - Shall be set to "blockingRequest" which means a blocking request is issued. cn: - Content of a <delivery> resource in line with the definition in clause 9.6.11 representing a valid request for delivery of data to a target CSE All other information in the Request message are the same as in the generic procedure. |
| Local processing on Hosting CSE | - Check whether the originator is authorized to request a delivery procedure on the receiver CSE - Check whether the provided attributes of the <delivery> resource that is requested to be created represents a valid request for delivering data to a target CSE. - Check whether Receiver CSE can actually satisfy the requested delivery in line with provisioned policies and requested delivery parameters - If all checks are positive, the receiver shall create the requested <delivery> resource and assumes responsibility for delivering the provided data to the target CSE. - If the Receiver CSE is the target CSE of the requested delivery, it shall forward the content of the delivered data attribute to the internal CSFs that will interpret the delivered data as a forwarded request from a remote Originator. |
| Information on Response message | In case the Originator CSE has not asked for a Result of the requested Operation (rc set to "nothing"), the Response only contains an Acknowledgement indicator. This only indicates that the Receiver CSE received the Request. It does NOT indicate whether the Receiver CSE was able to take on responsibility for delivery of the data. In case the Originator CSE asked for the status of the requested Operation to be contained in the Result of the requested Operation (rc not set to "nothing"), the Receiver CSE shall respond with a Success or Failure indicator. In case the Originator CSE asked for the status of the requested Operation and the URI of the created Resource to be contained in the Result of the Request, the Receiver CSE shall respond with a Success indicator including the URI of the created <delivery> resource in case it has taken on responsibility to deliver the data to the target CSE or with Failure indicator including an error indication otherwise. |
| Post-Conditions | The Originator CSE shall update the local <delivery> resource to reflect the new status of the delivery process (e.g., '{Receiver-CSE-ID} accepted delivery responsibility'). In case the Originator CSE got a Success indicator as a Response, it shall stop any further delivery attempts. In that case or if there was no indication of a need to provide a result of the operation, the Originator CSE may delete the content of the 'data' attribute of the local <delivery> resource. In case the Originator CSE got a Failure indicator as a response, it may initiate further delivery attempts in line with CMDH policies and delivery parameters and depending on the reason for Failure. In case the Receiver CSE is the target CSE of the delivery, the Receiver CSE needs to execute on the forwarded request contained in the delivered data. |
| Exceptions | - The Originator CSE is not authorized to request a delivery procedure on the Receiver CSE - The provided content of the <delivery> resource is not in line with the specified structure. - The provided content of the <delivery> resource represents a request for delivery that is not consistent (e.g., lifespan attribute already expired) - The provided content of the <delivery> resource represents a request for delivery that cannot be met by the Receiver CSE within the limits of the provided delivery parameters and the provisioned CMDH policies on the Receiver CSE. |

#### 10.2.4.3	Retrieve <delivery>

This procedure shall be used for requesting a CSE to provide information on a previously created <delivery> resource which represents delivery of data to a target CSE.

Originator: Originator can be a CSE or an AE.

Receiver: The Receiver shall provide the content of the addressed <delivery> resource or the addressed attributes thereof.

- Table 10.2.4.3-1: <delivery> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to retrieve information about a previously issued delivery |
| Information on Request message | All information in the Request message are the same as in the generic procedure |
| Local processing on Hosting CSE | No change to generic procedure |
| Information on Response message | No change to generic procedure |
| Post-Conditions | None |
| Exceptions | - The Originator CSE is not authorized to retrieve the <delivery> resource or the addressed parts of it - The addressed <delivery> resource does not exist |

#### 10.2.4.4	Update <delivery>

This procedure shall be used for requesting a CSE to update information on a previously created <delivery> resource which represents a pending delivery of data to a target CSE. The update may have impact on further processing of the delivery.

Originator: Originator can be a CSE or an AE. Originator needs to be authorized to modify the addressed <delivery> resource, i.e., it needs to be authorized to change the represented delivery process.

Receiver: The Receiver shall change the content of the addressed <delivery> resource or the addressed attributes thereof and also modify the corresponding parameters of the delivery process

- Table 10.2.4.4-1: <delivery> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to modify information about a previously issued delivery that is still pending, i.e., it has not yet been forwarded to another CSE |
| Information on Request message | cn: - URI of the <deliver> resource - Content of a <delivery> resource in line with the definition in clause 9.6.11 representing a valid request for delivery of data to a target CSE All other information in the Request message are the same as in the generic procedure |
| Local processing on Hosting CSE | Receiver CSE checks if the requested changes to the delivery process can actually be accomplished If possible, the Receiver CSE modifies the previously established delivery process and changes the respective content of the <delivery> resource |
| Information on Response message | Same as in the generic procedure. Successful Response messages indicate that the delivery process was modified as requested |
| Post-Conditions | None |
| Exceptions | - The Originator CSE is not authorized to modify the <delivery> resource or the addressed parts of it - The addressed <delivery> resource does not exist - The responsibility for the further processing of the delivery process represented by the addressed <delivery> process was already forwarded to another CSE |

#### 10.2.4.5	Delete <delivery>

This procedure shall be used for requesting a CSE to cancel a pending delivery of data to a target CSE or to delete the <delivery> resource of an already executed delivery.

Originator: Originator can be a CSE or an AE. Originator needs to be authorized to modify the addressed <delivery> resource, i.e., it needs to be authorized to change the represented delivery process.

Receiver: The Receiver shall remove the addressed <delivery> resource and stop the corresponding delivery process if it is still pending

- Table 10.2.4.5-1: <delivery> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to cancel a previously issued delivery that is still pending, i.e., it has not yet been forwarded to another CSE or Originator needs to remove the <delivery> resource representing an already executed delivery |
| Information on Request message | No change to generic procedure |
| Local processing on Hosting CSE | - Receiver CSE checks if the corresponding delivery process is still pending. If so, it stops that delivery process - Receiver CSE removes the addressed <delivery> resource |
| Information on Response message | Same as in the generic procedure. Successful Response messages indicate that the delivery process was stopped as requested |
| Post-Conditions | None |
| Exceptions | - The Originator CSE is not authorized to delete the <delivery> - The addressed <delivery> resource does not exist |

### 10.2.5	Resource Discovery Procedure

#### 10.2.5.1	Introduction

The resource discovery procedures allow discovering of resources residing on a CSE. The use of the filterCriteria attribute allows limiting the scope of the results. Filtering shall be performed on a subset of the offered resource attributes using a query string. A match, that may include ranges, may be performed on the query string, and a successful response may be returned with a list for the resources addressable in any of the forms expressed in clause 9.3.1 that contains the matching attributes or a URI of a temporary resource holding the list.

[10.2.5.1.a]  Editor's Note: It is FFS how this temporary resource is operated such as the access control policy of this temporary resource, expiration time, the condition for creation.

Resource discovery shall be accomplished using the RETRIEVE method by an Originator which shall also include the root of where the discovery begins: <CSEBase>. Filter criteria may be provided as a parameter to the Retrieve method. The filter criteria describe the rules for resource discovery, e.g., searching scope, topic, semantic relationships, and matching string. The filter criteria can also contain the parameters for specifying the maximum size of the answer (upper limit), and/or sorting criteria for specifying in which order the searching result should be organized.

The resource discovery procedures shall identify all matching resources from the entire hierarchy under <CSEBase>. The filter criteria provided by an Originator or the discovery results may be modified by the Hosting CSE to restrict the scope of discoverable resources according to the Originator's access control policy or service subscription.

The Hosting CSE may also implement a configured upper limit on the size of the answer. In such a case when both the Originator and the Hosting CSE have the upper limits, the smaller of the upper limit in the Hosting CSE and the upper limit of the Originator shall apply.

#### 10.2.5.2	Discovery procedure via Retrieve Operation

This procedure shall be used for the discovery of resources under <CSEBase> that match the provided filterCriteria attribute. The discovery result shall be returned to the Originator using a successful message.

Originator: The Originator can be an AE or a CSE. The Originator shall provide the <CSEBase> URI in the Request message. Filter criteria may be provided as a parameter by the Originator.

Receiver or Hosting CSE: The Hosting CSE shall read the values of all attributes belonging to the addressed resource structure and the references of all sub-resources and it shall build a representation of these. The Hosting CSE shall use the appropriate addressing (see clause 9.3.1) for each element included in the list in accordance with the incoming request. If filterCriteria is provided in the request, the Hosting CSE uses it identifying the resources whose attributes match the filterCriteria. The Hosting CSE shall respond to the Originator with the appropriate list of discovered resources in the Hosting CSE. If sorting criteria has been provided by the Originator, the list of discovered resources shall be sorted in that order.

The Hosting CSE may modify the filterCriteria including upper limit provided by the Originator or the discovery results based on the local policies.

If the size of the result list is bigger than the upper limit or the scope of discoverable resources, according to the Originator's access control policy or service subscription has been reduced by the Hosting CSE, the full list is not returned. Instead, an incomplete list is returned and an indication is added in the response for warning the device.

The Hosting CSE may propagate the discovery request to other CSEs according to resource discovery policies.

[10.2.5.2.a]  Editor's Note: It is FFS about discovery propagation mechanism, i.e., multi-hop discovery propagation and its policies.

- Table 10.2.5.2-1: Discovery procedure via Retrieve Operation

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to discover resources in the Hosting CSE |
| Information on Request message | - URI of the discovery resource: <CSEBase> - Filter criteria for searching and expected returned result |
| Local processing on Hosting CSE | - Checks the validity of the Request (e.g., format of filterCriteria) - Checks if the request is in accordance with the M2M service subscription - May change the filter criteria according to local policies - Searches matched resources from the addressed resource hierarchy - Propagates the discovery request to other CSEs if it is permitted by discovery policies - Limits the discovery result according to access control privileges of the discovered resources - Limits the discovery result according to the upper limit on the size of the answer - Sorts the results according to the sorting criteria |
| Information on Response message | - Contains the URI list of discovered resources expressed in any of the methods depicted in clause 9.3.1. - Contains an incomplete list warning if the full list is not returned |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE: - The requesting M2M AE or CSE is not registered - The request contains invalid parameters |

### 10.2.6	Group Management Procedures

#### 10.2.6.1	Introduction

This clause describes different procedures for managing membership verification, creation, retrieval, update and deletion of the information associated with a <group> resource as well as the bulk management of all group member resources by invoking the corresponding operations upon the virtual resource <fanOutPoint> of a <group> resource.

#### 10.2.6.2	Create <group>

This procedure shall be used for creating a group resource.

Originator: The Originator shall request to Create a new group type resource to be named as <group> by using the CREATE operation. The request shall address <CSEBase> resource of a Hosting CSE. The Request shall also provide list of member URI and may provide expirationTime attributes. The list of member URI means a list of URIs of the member resources corresponding to the memberType attribute provided in the Request. The originator may be an AE or a CSE.

Receiver: For the CREATE procedure, the Receiver shall:

- Check if the Originator has CREATE permissions on the <CSEBase> resource.

- Check the validity of the provided attributes.

- Validate that the resource type of every member conforms to the memberType attribute of the <group> resource, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation.

- Upon successful validation of the provided attributes, create a new group resource including the <fanOutPoint> child-resource in the Hosting CSE.

- Conditionally, in the case that the group resource contains temporarily unreachable sub-group resources as member resource, set the memberTypeValidated attribute of the <group> resource to FALSE.

- Respond to the Originator with the appropriate generic Response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the URI of the created <group> resource if the CREATE was successful.

- As soon as any unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request. or by default if the attribute is not provided.

- Table 10.2.6.2-1: <group> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <CSEBase> where the <group> resource is intended to be Created. cn: The representation of the <group> resource for which the attributes are described in clause 9.6.13. |
| Local processing on Hosting CSE | Steps described for the Receiver of the CREATE Request as described above |
| Information on Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.6.3	Retrieve <group>

This procedure shall be used for retrieving <group> resource.

Originator: The Originator shall request to obtaining <group> resource information by using the RETRIEVE operation. The request shall address the specific <group> resource of a hosting CSE. The Originator may be an AE or a CSE.

Receiver: The Receiver shall check if the Originator has READ permission on the group resource. Upon successful validation, the hosting CSE shall respond to the Originator with the appropriate responses  and the resource representation.

- Table 10.2.6.3-1: <group> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource |
| Local processing on Hosting CSE | Same as the generic procedure |
| Information on Response message | Same as the generic procedure |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.6.4	Update <group>

This procedure shall be used for updating an existing <group> resource.

Originator: The Originator shall request to Update attributes of an existing <group> resource by using an UPDATE operation. The Request shall address the specific <group> resource of a CSE. The Originator may be an AE or a CSE.

Receiver: The UPDATE procedure shall be:

- Check if the Originator has WRITE permissions on the <group> resource.

- Check the validity of provided attributes.

- Validate that the resource type of every member conforms to the memberType attribute of the <group> resource, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation.

- Upon successful validation of the provided attributes, update the group resource  in the Hosting CSE.

- Conditionally, in the case that the group resource contains temporarily unreachable sub-group resources as members resource set the memberTypeValidated attribute of the <group> resource to FALSE.

- Respond to the Originator with the appropriate generic response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the URI of the created <group> resource if the UPDATE is successful.

- As soon as any unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request, or by default if the attribute is not provided.

- Table 10.2.6.4-1: <group> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource |
| Local processing on Hosting CSE | Steps described for the Receiver of the UPDATE Request as described above |
| Information on Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.6.5	Delete <group>

This procedure shall be used for deleting an existing <group> resource.

Originator: The Originator shall request to delete an existing <group> type resource by using the DELETE operation. The request shall address the specific <group> resource of a Hosting CSE. The Originator may be an AE or a CSE.

Receiver: The Receiver shall check if the Originator has DELETE permission on the <group> resource. Upon successful validation, the CSE shall remove the resource from its repository and shall respond to the Originator with the appropriate responses.

- Table 10.2.6.5-1: <group> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource |
| Local processing on Hosting CSE | Same as the generic procedure |
| Information on Response message | Same as the generic procedure |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.6.6	Create <fanOutPoint>

This procedure shall be used for creating the content of all members resources belonging to an existing <group> resource.

Originator: The Originator shall request to create the content in all members resources belonging to an existing <group> resource by using a CREATE operation. The Request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group hosting CSE to create the same content under all <members> resources. The request may also address the URI that results from appending a relative URI to the <fanOutPoint> URI in order to create the same content (e.g., attribute or child resource) under the corresponding attributes or child resources represented by the relative URI with respect to all members resources. The Originator may be an AE or CSE.

Group Hosting CSE: For the CREATE procedure, the Group Hosting CSE shall:

- Check if the Originator has WRITE permission in the accessControlPolicy resource referenced by the members AccessControlPolicyID in the <group> resource. In the case members membersAccessControlPolicyIDs is not provided the access control policy defined for the <group> resource shall be used.

- Upon successful validation, obtain the URIs of all members resources from the attribute membersList of the addressed <group> resource.

- Generate fan out requests addressing the obtained URIs (appended with the relative URI if any) to the member hosting CSEs as indicated in Figure 10.2.2.6-1.The fr parameter in the request is set to ID of the Originator from the request from the original Originator.

- In the case that the members resources contain a sub-group resource, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple members resources belong to one CSE according to the URIs of the members resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the members hosting CSE to collect all the members on that members hosting CSE.

- After receiving the responses from the members hosting CSEs, respond to the Originator with the aggregated results and the associated memberIDs.

Member Hosting CSEs: For the CREATE procedure, the Member Hosting CSE shall:

- Perform the corresponding CREATE procedure for the resource type in the request (clause 10.2 resource specific procedures).

- Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifiers stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier.

- Check if the original Originator has the CREATE permission on the addressed resource. Upon successful validation, perform the create procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2.

- Send the corresponding response to the Group Hosting CSE.

- Figure 10.2.6.6-1: Group content management procedures

The procedures illustrated in figure 10.2.6.6-1 apply to clauses 10.2.6.6 to 10.2.6.9.

- Table 10.2.6.6-1: <fanOutPoint> CREATE

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <fanOutPoint> virtual resource cn: The representation of the resource the Originator intends to create gid: The group request identifier |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE, addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups |
| Information on Response message | Converged responses from members hosting CSEs |
| Post-Conditions | None |
| Exceptions | Same request with identical group request identifier received Originator does not have the access control privileges to access the <fanOutPoint> resource |

#### 10.2.6.7	Retrieve <fanOutPoint>

This procedure shall be used for retrieving the content of all member resources belonging to an existing <group> resource.

Originator: The Originator shall request to obtain the content or specific information (e.g., attributes) of all member resources belonging an existing <group> resource by using a RETRIEVE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group hosting CSE for retrieving the content of all member resources. The request may also address the URI that results from appending a relative URI to the <fanOutPoint> URI in order to retrieve the corresponding attributes or child resources represented by the relative URI with respect to all members resources. The originator may be an AE or CSE.

Group Hosting CSE: For the RETRIEVE procedure, the Group Hosting CSE shall:

- Check if the Originator has READ permission in the accessControlPolicy resource referenced by the membersAccessControlPolicyIDsin the addressed <group> resource. In the case membersAccessControlPolicyIDsis not provided, the access control policy defined for the group resource shall be used.

- Upon successful validation, obtain the URIs of all members resources from the membersList attribute of the addressed <group> resource.

- Generate fan out requests addressing the obtained URIs (appended with the relative URI if any) to the members hosting CSEs as indicated in Figure 10.2.2.6-1.The fr parameter in the request is set to ID of the Originator from the request from the original Originator.

- In the case that the members resources contain a sub-group resource, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple members resources belong to one CSE according to the URIs of the members resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the members hosting CSE to collect all the members on that members hosting CSE.

- After receiving the responses from the members hosting CSEs, respond to the originator with the aggregated results and the associated memberIDs.

Member Hosting CSEs: For the RETRIEVE procedure, the Member Hosting CSE shall:

- Perform the corresponding RETRIEVE procedure for the resource type in the request (clause 10.2 resource specific procedures).

- Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the request identifier.

- Check if the original originator has the READ permission on the addressed resource. Upon successful validation, perform the retrieve procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2.

- Send the corresponding response to the group hosting CSE.

- Table 10.2.6.7-1: <fanOutPoint> RETRIEVE

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <fanOutPoint> resource gid: The group request identifier |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE, addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups |
| Information on Response message | Converged responses from member hosting CSEs |
| Post-Conditions | None |
| Exceptions | Same request with identical request identifier received Originator does not have the access control privileges to access the <fanOutPoint> resource |

#### 10.2.6.8	Update <fanOutPoint>

This procedure shall be used for updating the content of all member resources belonging to an existing <group> resource.

Originator: The Originator shall request to update the content of all member resources belonging to an existing <group> resource with the same new data by using a UPDATE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group hosting CSE to update all <members> resources. The request may also address the URI that results from appending a relative URI to the <fanOutPoint> in order to update only the corresponding attributes or child resources represented by the relative URI with respect to all <members> resources. The originator may be an AE or CSE.

Group Hosting CSE: For the UPDATE procedure, the Group Hosting CSE shall:

- Check if the originator has WRITE permission in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDsin the group resource. In the case members membersAccessControlPolicyIDsis not provided the access control policy defined for the group resource shall be used.

- Upon successful validation, obtain the URIs of all member resources from the attribute membersList of the addressed <group> resource.

- Generate fan out requests addressing the obtained URIs (appended with the relative URI if any) to the members hosting CSEs as indicated in Figure 10.2.2.6-1.The fr parameter in the request is set to ID of the Originator from the request from the original Originator.

- In the case that the members resources contain a sub-group resource, generate a unique group request identifier, include it in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple members resources belong to one CSE according to the URIs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the member hosting CSE to collect all the members on that members hosting CSE.

- After receiving the responses from the member hosting CSEs, respond to the originator with the aggregated results and the associated memberIDs.

Member Hosting CSEs: For the UPDATE procedure, the Member Hosting CSE shall:

- Perform the corresponding UPDATE procedure for the resource type in the request (clause 10.2 resource specific procedures).

- Check if the request has a group request identifier. Check if the request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the request identifier.

- Check if the original originator has the UPDATE permission on the addressed resource. Upon successful validation, perform the update procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2.

- Send the corresponding response to the group hosting CSE.

- Table 10.2.6.8-1: <fanOutPoint> UPDATE

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource cn: The representation of the resource the Originator intend to Update gid: The group request identifier |
| Local processing on Hosting CSE | Fan out requests to each members hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups |
| Information on Response message | Converged responses from member hosting CSEs |
| Post-Conditions | None |
| Exceptions | Same request with identical request identifier received Originator does not have the access control privileges to access the <fanOutPoint> resource |

#### 10.2.6.9	Delete <fanOutPoint>

This procedure shall be used for deleting the content of all members resources belonging to an existing <group> resource.

Originator: The Originator shall request to delete the content of all members resources belonging to an existing <group> resource by using a DELETE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group hosting CSE to delete all members resources. The request may also address the URI that results from appending a relative URI to the <fanOutPoint> in order to delete only the corresponding attributes or child resources represented by the relative URI with respect to all member resources. The originator may be an AE or a CSE.

<Group> Hosting CSE: For the DELETE procedure, the <group> Hosting CSE shall:

- Check if the Originator has WRITE permission in the accessControlPolicy resource referenced by the membersAccessControlPoliciIDs in the <group> resource. In the case membersAccessControlPolicyIDs is not provided the access control policy defined for the group resource shall be used.

- Upon successful validation, obtain the URIs of all member resources from the attribute membersList of the addressed <group> resource.

- Generate fan out requests addressing the obtained URIs (appended with the relative URI if any) to the member hosting CSEs as indicated in Figure 10.2.2.6-1. fr parameter in the request is set to ID of the Originator from the request from the original Originator.

- In the case that the members resources contain a sub-group resource, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier.

- If the <group> hosting CSE determines that multiple members resources belong to one CSE according to the URIs of the members resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the member hosting CSE to collect all the members on that member hosting CSE.

- After receiving the responses from the members hosting CSEs, respond to the originator with the aggregated results and the associated memberIDs.

Members Hosting CSEs: For the DELETE procedure, the Members Hosting CSE shall:

- Perform the corresponding DELETE procedure for the resource type in the request (clause 10.2 resource specific procedures).

- Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier.

- Check if the original originator has the DELETE permission on the addressed resource. Upon successful validation, perform the delete procedures for the corresponding type of addressed resource as described in other sub-clauses of clause 10.2.

- Send the corresponding response to the Group Hosting CSE.

- Table 10.2.6.9-1: <fanOutPoint> DELETE

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource gid: The group request identifier |
| Local processing on Hosting CSE | Fan out requests to each members hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups |
| Information on Response message | Converged responses from member hosting CSEs. |
| Post-Conditions | None |
| Exceptions | Same request with identical request identifier received Originator does not have the access control privilege to access the <fanOutPoint> resource |

#### 10.2.6.10	Subscribe and Un-Subscribe <fanOutPoint> of a group

This procedure shall be used for receiving information about modifications of all member resources belonging to an existing <group> resource.

Originator: shall request to create a subscription resource under all member resources belonging to an existing <group> resource by using a CREATE operation. The request shall address the child resource <fanOutPoint> of the specific <group> resource of a group hosting CSE appended with the ID of the <subscription> resource to be created to subscribe to the modifications of all member resources. The request shall include an empty aggregationURI attribute if the originator wants the group hosting CSE to aggregate the notifications. The request shall include the required information and may include the optional information as described in subscription management clause 10.2.11. The originator may be an AE or a CSE.

Group Hosting CSE: For the subscribe/un-subscribe procedure, the <Group> Hosting CSE shall:

Check if the originator has WRITE permission in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDsin the group resource. In the case membersAccessControlPolicyIDs is not provided the access control policy defined for the group resource shall be used.

If the subscription resource in the request contains an aggregationURI attribute, assign an URI to the aggregationURI of the subscription resource. In case the aggregationURI attribute has a value, maintain the mapping of the generated aggregationURI and the former aggregationURI.

Upon successful validation, obtain the URIs of all member resources from the attribute membersList of the addressed <group> resource and fan out requests to the members hosting CSEs addressing the obtained URIs appended with the ID of the <subscription> resource to be created.

If the group hosting CSE determines that multiple members resources belong to one CSE according to the URIs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the <group> hosting CSE creating a <group> resource on the members hosting CSE to collect all the members on that members hosting CSE.

After receiving the responses from the members hosting CSEs, respond to the originator with the aggregated results and the associated memberIDs.

Members Hosting CSEs: For the subscribe/un-subscribe procedure, the Members Hosting CSE shall treat the request received from the group hosting CSE as a normal SUBSCRIBE request on the addressed member resource as if it comes from the original originator. Therefore the members hosting CSE shall:

Check if the original originator has the READ permission on the members resource.

Upon successful validation, perform the subscribe procedures for the corresponding type of member resource as described in clause 10.2.11

Send the corresponding response to the group hosting CSE.

Table 10.2.6.10-1: <fanOutPoint> Subscribe/Un-subscribe

| Description |  |
| --- | --- |
| Call flow type | CREATE |
| Pre-conditions | none |
| Information on Request message | op: C fr: Identifier of the AE or CSE that initiates the request. to: The URI of the <fanOutPoint> resource appended with the ID of the <subscription> resource to be created. gid: The group request identifier |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with the ID of the <subscription> resource to be created. Generate request identifier and include the identifier in the fanned out requests in the case of sub groups. |
| Information on Response message | Converged responses from member hosting CSEs. |
| Post-condition | None |
| Exceptions | Same request with identical request identifier received. Originator does not have the access control privilege to access the <fanOutPoint> resource. |

#### 10.2.6.11	Aggregate the notifications by group

This procedure is used for the group hosting CSE to aggregate the notifications from member hosting CSEs and forward the aggregated notification to the subscriber.

Members Hosting CSEs: Whenever the resource that is subscribed-to is modified in a way that matches the policies as is specified in clause 9.6.8, notification needs to be sent to the subscribe, the Members Hosting CSE shall:

If the subscription resource contains aggregationURI, notify the subscriber at the aggregationURI and include the notificationURI in the notification. Otherwise, notify at the URI represented by the attribute notificationURI.

Group Hosting CSE: For the notification procedure, the Group Hosting CSE shall:

On receiving the notifications from the member hosting CSEs at the aggregationURI, validate if the notification is sent from its member resource and contain a notificationURI attribute.

Upon successful validation, aggregate the notifications which have the same notificationURI address of a single subscriber. Send the aggregated notification to the subscriber according to the notificationURI in the notification. In the case the addressed group is the member of another group through which the subscription is created the notification shall be sent according to the mapping of the aggregationURI of the two <group> hosting CSEs.

Wait for the response. After receiving the response, split the response and respond to the members hosting CSEs separately.

The group hosting CSE may stop aggregating the notifications when the expirationTime of the corresponding subscription expires.

Subscriber: shall treat every notification extracted from the aggregated notification as a separate notification received from the subscribed resource and generate corresponding responses. The subscriber shall aggregate the responses to these notifications and send the aggregated response to the group hosting CSE.

### 10.2.7	mgmtObj Management Procedures

#### 10.2.7.1	Introduction

This clause describes the management procedures over Mca and Mcc reference points. Different RESTful requests addressing a <mgmtObj> resource (or its attributes or child resources) shall be translated into existing management commands and procedures performed on the mapped external management object on the managed entity. The Receiver in the following procedures is IN-CSE.

#### 10.2.7.2	Create <mgmtObj>

This procedure is used to create a specific <mgmtObj> resource in the hosting CSE to expose the corresponding management function of a managed entity (i.e., M2M Device/Gateway) over the Mca reference point. Depending on the data model being used, the created <mgmtObj> resource may be a partial or complete mapping from the external management object onto the managed entity. If such an external management object is missing from the managed entity, it shall be added to the managed entity. Further RESTful operations performed on the created <mgmtObj> resource shall be converted by the hosting CSE into a corresponding device management action performed on the mapped external management object on the managed entity using existing device management protocols (e.g., OMA-DM [i.5] or BBF TR-069 [i.4]).

Originator: The Originator shall request to create a new <mgmtObj> resource by using a CREATE operation. The request shall address the <CSEBase> resource of the hosting CSE.

The Originator may be:

- The CSE on the managed entity: In this case, the CSE first collects the original external management object (the management tree structure or also the value of the tree nodes if needed) of the local device and transforms the data into the <mgmtObj> resource representation, then requests the hosting CSE to create the corresponding <mgmtObj> resource.

- An AE: In this case, the AE requests the hosting CSE to add the corresponding external management object to the managed entity by creating an <mgmtObj> resource in the hosting CSE.

NOTE 1:	The IN-CSE may also create the <mgmtObj> resource locally by itself. The details are out of scope. In this case, the hosting CSE first collects the original external management object on the managed entity via existing device management protocols (e.g., OMA DM [i.5], BBF TR-069 [i.4] or LWM2M [i.6]), then transforms the object into the <mgmtObj> resource representation and create the <mgmtObj> resource locally in the IN-CSE.

NOTE 2:	The <mgmtObj> resource could also be created in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: For the CREATE procedure, the Receiver shall:

- Check if the Originator has the CREATE permission on the addressed <CSEBase> resource. Then check the validity of the provided attributes.

- Upon successful validation, create a new <mgmtObj> resource in the hosting CSE with the provided attributes.

- If the originator is an AE: Check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to the managed entity or to the management server to add the corresponding external management object to the managed entity based on existing management protocol.

- Maintain the mapping relationship between the created <mgmtObj> resource and the external management object on the managed entity.

- Respond to the originator with the appropriate generic responses. It shall also provide in the response the URI of the created new resource.

- Table 10.2.7.2-1: <mgmtObj> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <CSEBase> where the <mgmtObj> resource is intended to be Created cn: The representation of the <mgmtObj> resource for which the attributes are described in clause 9.13 |
| Local processing on Hosting CSE | New external management object created on the managed entity using external management technologies in case the originator is IN-AE |
| Information on Response message | Proper error code if the new external management object is not created |
| Post-Conditions | None |
| Exceptions | - The creation of the external management object is not allowed - The created external management object already exists - Corresponding external management object cannot be added to the managed entity for some reason (e.g., not reachable, memory shortage) |

#### 10.2.7.3	Retrieve <mgmtObj>

This procedure is used to retrieve information from an existing <mgmtObj> resource or its <parameters> child resources.

Originator: The Originator shall request to retrieve an existing <mgmtObj> resource by using a RETRIEVE operation. The request shall address a specific "mgmtObj" resource of a CSE or a specific <parameters> child resource of the <mgmtObj> resource to retrieve all attributes of the <mgmtObj> resource or the <parameters> child resource.

The originator may be:

- An IN-AE.

- An ASN/MN-CSE on the managed entity.

Receiver: For the RETRIEVE procedure, the Receiver shall:

- Check if the originator has the READ permission on the addressed <mgmtObj> resource.

- Upon successful validation, retrieve the corresponding <mgmtObj> resource or the <parameters> child resource including all attributes and references to all its child resources from its repository.

- Respond to the originator with the appropriate generic responses.

- If the Originator is an IN-AE and if the requested information of the <mgmtObj> resource is not available, identify the corresponding external management object on the managed entity according to the mapping relationship it maintains. Check if there is an existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to get the corresponding external management object from the managed entity based on the existing device management protocol (e.g., OMA-DM [i.5], BBF TR-069 [i.4]), then return the result to the originator.

- Table 10.2.7.3-1: <mgmtObj> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource |
| Local processing on Hosting CSE | Get corresponding external management object from managed entity by existing management procedures in case the addressed <mgmtObj> or <parameters> is not available |
| Information on Response message | Proper error code if the external management object can not be retrieved |
| Post-Conditions | None |
| Exceptions | Corresponding external management object data cannot be retrieved from managed entity due to some reason (e.g., external management object not found) |

#### 10.2.7.4	Update <mgmtObj>

This procedure is used to update information of an existing <mgmtObj> resource or its <parameters>child resource.

Originator: The Originator shall request to update information of an existing <mgmtObj> resource by using an UPDATE operation. The Request shall address a specific <mgmtObj> resource on the hosting CSE to update the all attributes of the <mgmtObj> resource with new values when using UPDATE operation.

The Originator may be:

- IN-AE.

- CSE on the managed entity.

Receiver: For the UPDATE procedure, the Receiver shall:

- Check if the Originator has the WRITE permission on the address <mgmtObj> resource. Check the validity of provided attributes if any.

- Upon successful validation, update the corresponding attribute(s) of the <mgmtObj> resource accordingly.

- If the originator is an IN-AE, identify the corresponding external management object on the managed entity according to the mapping relationship it maintains. Check if there is an existing management session between the management server and the managed entity. If not, request the management server to establish a management session towards the managed entity. Send the management request to update the corresponding external management object in the managed entity accordingly based on existing device management protocol (e.g., OMA DM [i.5], BBF TR-069 [i.4] or LWM2M [i.6]).

- Respond to the Originator with the appropriate generic responses.

- Table 10.2.7.4-1: <mgmtObj> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource cn: The representation of the <mgmtObj> resource for which the attributes are described in clause 9.13 |
| Local processing on Hosting CSE | Override corresponding external management object in managed entity by existing management procedures |
| Information on Response message | Proper error code if the external management object can not be updated |
| Post-Conditions | None |
| Exceptions | Corresponding external management object cannot be updated to managed entity due to some reason (e.g., not reachable, external management object not found) |

#### 10.2.7.5	Delete <mgmtObj>

This procedure is used to delete an existing <mgmtObj> resource so as to hide the management function of a managed entity (i.e., M2M Device/Gateway) from being exposed via Mca reference point. An IN-AE uses this procedure to remove the corresponding external management object (e.g., an obsolete software package) from the managed entity.

Originator: The Originator shall request to delete an existing <mgmtObj> resource by using a DELETE operation. The request shall address the specific <mgmtObj> resource on the hosting CSE.

The originator may be:

- The CSE on the managed entity: In this case, the CSE issues the request to the hosting CSE to hide the corresponding management function from being exposed by the <mgmtObj> resource.

- An IN-AE: In this case, the IN-AE requests the hosting CSE to delete the <mgmtObj> resource from the hosting CSE and to remove the corresponding external management object from the managed entity.

NOTE 1:	The hosting CSE in the network domain could also delete the <mgmtObj> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <mgmtObj> resource could also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: For the DELETE procedure, the Receiver shall:

- Check if the Originator has the DELETE permission on the addressed <mgmtObj> resource.

- Upon successful validation, remove the addressed resource from its repository.

- If the Originator is an IN-AE, identify the corresponding external management object on the managed entity according to the mapping relationship it maintains. Check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to remove the corresponding external management object from the managed entity based on existing management protocol.

- Respond to the Originator with the appropriate generic responses.

- Table 10.2.7.5-1: <mgmtObj> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource. |
| Local processing on Hosting CSE | Trigger existing (OMA-DM [i.5], BBF TR-069 [i.4] or LWM2M [i.6]) management procedures to delete the corresponding external management object from the managed entity. |
| Information on Response message | Proper error code if the external management object can not be deleted. |
| Post-Conditions | None |
| Exceptions | Corresponding external management object cannot be deleted from managed entity due to some reason (e.g., not reachable, external management object not found). |

#### 10.2.7.6	Execute <mgmtObj>

This procedure is used for executing a specific management command on a managed entity through an existing <mgmtObj> resource on the hosting CSE.

Originator: The Originator shall request to execute a specific management command which is represented by an existing <mgmtObj> resource or its attribute by using an UPDATE operation. The request shall address the specific executable <mgmtObj> resource or its attribute and shall contain an empty body. In the case that the management command is represented by an attribute of a <mgmtObj> resource, such attribute shall contain a URI that the request may also address to trigger the command execution alternatively.

After the execution request, the originator may request to retrieve the execution result or status from the executable <mgmtObj> resource or its attribute/child resource by using a RETRIEVE method as described in clause 10.2.7.3.

The Originator shall be an IN-AE.

Receiver: For the execute procedure, the Receiver shall:

- Check if the Originator has the WRITE permission on the addressed <mgmtObj> resource or its attribute.

- Upon successful validation, check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to execute the corresponding management command (e.g., "Exec" in OMA DM [i.5) on the managed entity based on existing device management protocol.

- Respond to the Originator with the appropriate generic responses. The Response may also contain execution results.

- Upon receiving a RETRIEVE request to retrieve the execution result or status from the executable <mgmtObj> resource or its attribute, perform the procedures as described in clause 10.2.7.3.

- Upon receiving from managed entity a management notification (e.g., OMA-DM [i.5] "Generic Alert" message or BBF TR-069 [i.4] "Inform" message) regarding the execution result or status, the receiver may actively send the management request to retrieve the execution result or status external management object information from the managed entity and update the corresponding <mgmtObj> resource or its attribute locally.

[10.2.7.6.b]  Editor's note: The definition of Execute is to be for FFS in clause 10.1.

- Table 10.2.7.6-1: <mgmtObj> EXECUTE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource cn: None |
| Local processing on Hosting CSE | Trigger existing (OMA DM [i.5], BBF TR-069 [i.4], LWM2M [i.6]) device management procedures to execute the corresponding management command on the managed entity |
| Information on Response message | Proper error code if the external management object can not be executed |
| Post-Conditions | None |
| Exceptions | Corresponding external management object operation cannot be executed in managed entity due to some reason (e.g., not reachable, external management object not found) |

### 10.2.8	External Management Operations through <mgmtCmd>

[10.2.8.a]  Editor's Note: Add external references (BBF etc)

#### 10.2.8.1	Introduction

This clause describes how RESTful management operations may be performed using <mgmtCmd> resources over the Mca and Mcc reference points.  The <mgmtCmd> resource, together with its attributes or sub-resources, may be used in the process of translating between RESTful operations and management commands and procedures from existing management technologies (e.g., BBF TR-069 [i.4]). These procedures can then be performed on the remote entity, using the Management Adapter and the procedures described in the following clauses.

#### 10.2.8.2	Create <mgmtCmd>

A CREATE request is used by an Originator to create a specific <mgmtCmd> resource in a hosting CSE.

The created <mgmtCmd> resource will be mapping a RESTful method to management commands and/or procedures which may be translated from existing management protocols (e.g., BBF TR-069 [i.4]). At run-time the hosting CSE can expose the translated commands, over the Mcc reference point, to the remote entities (i.e., ASN/MN-CSE).

Originator: The Originator shall request to create a new <mgmtCmd > resource to be named as "mgmtCmd" by using a CREATE operation. The request shall address <CSEBase> resource of the hosting CSE. The request may also provide the attributes of the <mgmtCmd> resource to be created as described in clause 9.6.15 such as "cmdType".

The Originator may be:

- An AE registered to the IN-CSE.

- The CSE on the managed entity: In this case, the CSE transforms supported management command into the <mgmtCmd> resource representation, then requests the hosting CSE to create the corresponding <mgmtCmd> resource.

NOTE 1:	The hosting IN-CSE in the network domain may also create the <mgmtCmd> resource locally by itself. The details are out of scope. Then an AE can discover the created <mgmtCmd> and manipulate it.

NOTE 2:	The <mgmtCmd> resource could also be created in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: The Receiver shall check if the Originator has the CREATE permission on the addressed <CSEBase> resource (or the parent <mgmtCmd> resource in the case of child resource creation). The hosting CSE shall also check the validity of provided attributes. Upon successful validation, a new resource with name "mgmtCmd" shall be created in the hosting CSE with the provided attributes. The hosting CSE shall maintain the mapping between the created <mgmtCmd> resource and the corresponding nonRESTful commands represented by the "cmdType" attribute of <mgmtCmd> resource. The hosting CSE shall respond to the Originator with the appropriate generic response code. It shall also provide in the Response the URL of the created new resource.

The hosting CSE shall be an IN-CSE.

- Table 10.2.8.2-1: <mgmtCmd> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | The CSE on the originating node shall first collect local management command. |
| Information on Request message | op: C fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | No change from the basic procedures |
| Information on Response message | fr: Receiver CSE-ID to: Originator AE-ID or CSE-ID cn: URI of created <mgmtCmd> resource |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the creation is not allowed, the <mgmtCmd> resource shall not be created, and a proper error code shall be returned to the originator in the Response message |

#### 10.2.8.3	Retrieve <mgmtCmd>

A RETRIEVE Request is used by an Originator to retrieve all or part information from an existing <mgmtCmd> resource on a hosting CSE. Alternatively, the Originator can request to retrieve only a specific attribute or part of an attribute.

Originator: The Originator shall request to retrieve all or part of the information from an existing <mgmtCmd> resource by using a RETRIEVE operation. The request shall address a specific "<mgmtCmd>" resource to retrieve all attributes and the references to the sub-resources of the <mgmtCmd> resource.  Alternatively, the request shall address the individual attributes of the specific "<mgmtCmd>" resource to retrieve the corresponding attribute value.

[10.3.8.3.a]  Editor's Note: We need to agree on a terminology on how to refer to a specific instance of a resource type.

The Originator may be:

- An AE.

- A CSE.

Receiver: The Receiver shall check if the Originator has the READ permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall retrieve the corresponding attributes the <mgmtCmd> resource and the references to the sub-resources from its repository and shall respond to the Originator with the appropriate generic responses.

The hosting CSE shall be an IN-CSE.

- Table 10.2.8.3-1: <mgmtCmd> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to create a resource |
| Information on Request message | op: R fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | No change from the basic procedures |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the retrieval is not allowed, or the specific <mgmtCmd> resource does not exist in the IN-CSE, a proper error code shall be returned to the originator |

#### 10.2.8.4	Update <mgmtCmd>

An UPDATE request is used by an Originator to update all or part information of an existing <mgmtCmd> resource on a hosting CSE with new attributes. Alternatively, the Originator can request to update only a specific attribute or part of an attribute.

Originator: The Originator shall request to update all or partial information of an existing <mgmtCmd> resource by using a UPDATE operation. The request shall address a specific "<mgmtCmd>" resource of a CSE to update all attributes of the <mgmtCmd> resource with the provided new value.  Alternatively, the request shall address the individual attribute of the specific "<mgmtCmd>" resource to update the corresponding attribute with the provided new value.

The Originator may be:

- An AE.

- A CSE.

Receiver: The Receiver shall check if the Originator has the WRITE permission on the addressed <mgmtCmd> resource. The hosting CSE shall also check the validity of provided attributes if any. Upon successful validation, the hosting CSE shall overwrite the corresponding attributes of the <mgmtCmd> resource with the provided new data. The CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be an IN-CSE.

- Table 10.2.8.4-1: <mgmtCmd> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to retrieve a resource |
| Information on Request message | op: U fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | No change from the basic procedures |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the update is not allowed or the specific <mgmtCmd> resource (or its sub-resource) does not exist in the IN-CSE, a proper error code shall be returned to the Originator |

#### 10.2.8.5	Delete <mgmtCmd>

A DELETE request is used by the Originator to delete an existing <mgmtCmd> resource on a hosting CSE. An AE may also use this procedure to cancel all initiated <execInstance> of an <mgmtCmd> if applicable.

Originator: The Originator shall request to delete an existing <mgmtCmd> resource by using a DELETE operation. The request shall address the specific "<mgmtCmd>" resource on the hosting CSE.

The Originator may be:

- The CSE on the manageable entity: In this case, the CSE issues the request to the hosting CSE to hide the corresponding management command from being exposed by the <mgmtCmd> resource.

- An AE: In this case, the AE requests the hosting CSE to delete the <mgmtCmd> resource from the hosting CSE and cancel all initiated <execInstance> of an <mgmtCmd> if applicable.

NOTE 1:	The hosting CSE in the network domain could also delete an <mgmtCmd> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <mgmtCmd> resource could also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: The Receiver shall check if the Originator has the DELETE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall remove the resource from its repository. If the Originator is an AE and there is any initiated <execInstance> under the <mgmtCmd> that can be cancelled by a corresponding management command. The hosting CSE shall also issue the management command to the remote entity to cancel those initiated <execInstance> based on existing management protocol (i.e., BBF TR-069 [i.4]). Then the CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be an IN-CSE.

- Table 10.2.8.5-1: <mgmtCmd> DELETE by ASN-CSE or MN-CSE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Before issuing a DELETE request to the IN-CSE, the originating CSE may perform cancelling of the corresponding management command locally. |
| Information on Request message | op: D fr: Originator AE-ID or CSE-ID. to: Receiver CSE-ID. cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | The <mgmtCmd> resource shall be deleted from the repository of the IN-CSE |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the deletion is not allowed or the specific <mgmtCmd> resource does not exist in the IN-CSE, the deletion shall be failed and a proper error code shall be returned to the Originator |

- Table 10.2.8.5-2: <mgmtCmd> DELETE by an AE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to delete the resource. |
| Information on Request message | op: D fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | If there is any initiated <execInstance> under <mgmtCmd> and it is cancellable, IN-CSE shall cancel those initiated <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e., CancelTransfer RPC in BBF TR-069 [i.4 ]) The <mgmtCmd> resource shall be deleted from the repository of the IN-CSE |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the deletion is not allowed or the specific <mgmtCmd> resource does not exist, there is no local processing in the IN-CSE and a proper error code shall be returned to AE If the corresponding initiated commands cannot be deleted from remote entity due to some reason (e.g., not found) a response with the proper indication shall be returned to AE |

#### 10.2.8.6	Execute <mgmtCmd>

The Execute procedure is used by an originator for execution of a specific management command on a remote entity, through an existing <mgmtCmd> resource on the hosting CSE.

Originator: The Originator shall request to execute a specific management command which is represented by an existing <mgmtCmd> resource by using an UPDATE operation. The UPDATE request shall address the execEnable attribute of the <mgmtCmd> resource without any payload. Alternatively, the UPDATE request shall address the URI provided as the value of the execEnable attribute of the <mgmtCmd> resource.

After issuing the execution request, the Originator may request to retrieve the execution result or status from an <execInstance> sub-resource of the <mgmtCmd>by using a RETRIEVE method as described in clause 10.2.6.2.

The Originator shall be an AE.

Receiver: The Receiver shall check if the Originator has the WRITE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall perform command conversion and mapping, and send the converted management command over mId reference point to execute the corresponding management command with the provided arguments on the remote entity based on existing device management protocol (i.e., BBF TR-069 [i.4]). Then the hosting CSE shall create a corresponding <execInstance> resource under <mgmtCmd> for this command execution. And the hosting CSE shall respond to the originator with the appropriate generic responses. It shall also provide in the response the URL of the created <execInstance> resource.

Upon receiving from the remote entity a management notification (i.e., BBF TR-069 [i.4] "Inform" message) regarding the execution result or status, the hosting CSE may update the corresponding <execInstance> sub-resource locally.

The hosting CSE shall be an IN-CSE.

[10.2.8.6.a]  Editor's note: The definition of Execute is to be for FFS in clause 10.1.

- Table 10.2.8.6-1: <mgmtCmd> EXECUTE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to retrieve a resource. |
| Information on Request message | op: U. fr: Originator AE-ID or CSE-ID. to: Receiver CSE-ID. cn: <mgmtCmd>/execEnbale The UPDATE request shall address to <mgmtCmd>/execEnable without any payload. The mandatory and/or optional attributes defined in clause 9.6.15, as needed. |
| Local processing on Hosting CSE | (a) If the execution is allowed, Hosting CSE will do command conversion and mapping. (b) The Hosting CSE shall trigger existing device management procedures (i.e., BBF TR-069 [i.4]) to execute the corresponding management command on the remote entity. (c) The Hosting CSE shall perform local processing: if Step b is successful, the IN-CSE shall create an <execInstance> resource under the triggered <mgmtCmd> to maintain status and results for this execution. |
| Information on Response message | No change from the basic procedures. |
| Post-Conditions | The following processing on the Hosting CSE is dependent on the type of the command and execution status and may occur after the Response message has been sent. (a) After the command execution is finished, the remote entity sends response including execution results to the Hosting CSE, who will store the execution results in corresponding <execInstance> resource. This step may occur any time after the Response message has been sent and might occur after step b below. (b) The AE may use normal RETRIEVE procedure to retrieve the execution results or status of an <execInstance>. (c) After receiving the RETRIEVE request from the AE, the Hosting CSE can retrieve the execution status or results on the remote entity using existing management protocol. If step d. occurs before step e, step f. may not be needed. (d) A response shall be returned to the AE. |
| Exceptions | If the execution is not allowed or the specified <mgmtCmd> resource or its attribute/sub-resource does not exist in the IN-CSE, no further processing is required in the Hosting CSE and a proper error code shall be returned to AE in the Message response. If the corresponding management command cannot be executed in remote entity due to some reason, step 004 <??> shall be skipped and a proper error code shall be returned to AE in step 005 <??>. |

[10.2.8.6.b]  Editor's Note: Reference to setps 004 and 005 needed.

#### 10.2.8.7	Cancel <execInstance>

The Cancel procedure is used by an originating AE to disable/stop/cancel an initiated management command execution on the remote entity, through an existing <execInstance> resource on the hosting CSE.

Originator: The Originator shall request to disable/stop/cancel an initiated management command execution which is represented by an existing <execInstance> resource, by using an UPDATE operation. The UPDATE request shall address the execDisable attribute without any payload.

The Originator shall be an AE.

Receiver: The Receiver shall check if the Originator has the WRITE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall perform command conversion and mapping, then use existing management protocol (i.e., BBF TR-069 [i.4]) to cancel the corresponding management command execution initiated on the remote entity, and the hosting CSE shall respond to the originator with the appropriate responses.

The hosting CSE shall be an IN-CSE.

- Table 10.2.8.7-1: <execInstance> CANCEL

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to disable/stop/cancel an initiated management command execution on the remote entity |
| Information on Request message | op: U fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: <mgmtCmd>/execInstance. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | If the cancellation is allowed, IN-CSE will do command conversion and mapping The IN-CSE shall cancel the <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e., CancelTransfer RPC in BBF TR-069 [i.4]) The IN-CSE shall delete the corresponding <execInstance> resource |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the execution is not allowed or the specified <execInstance> resource does not exist in the IN-CSE or the <execInstance> is finished, the local processing on Hosting CSE shall be skipped and a proper error code shall be returned to AE in the Response message If the corresponding management command cannot be executed in remote entity due to some reason, the IN-CSE shall still delete the corresponding <execInstance> resource and return a Response message |

[10.2.8.7.a]  Editor's Note: The Cancel flow shall be updated based on the definition of EXEC procedure.

#### 10.2.8.8	Retrieve <execInstance>

A RETRIEVE request is used by an Originator to retrieve an existing <execInstance> resource, including individual attributes, from a hosting CSE. Alternatively, the Originator can request to retrieve only a specific attribute or part of an attribute.

Originator: The Originator shall request to retrieve all or part of information from an existing <execInstance> resource by using a RETRIEVE operation. The Request shall address a specific "<execInstance>" resource of a hosting CSE to retrieve all attributes of the <execInstance> resource. Or the request shall address the individual attribute of the specific "<execInstance>" resource to retrieve the corresponding attribute value.

The Originator shall be an AE.

Receiver: The Receiver shall check if the Originator has the READ permission on the addressed <mgmtCmd> resource. Upon successful validation, the CSE shall retrieve the corresponding attributes of the <execInstance> resource and the references to the sub-resources from its repository and shall respond to the originator with the appropriate generic responses.

The hosting CSE may be an IN-CSE.

- Table 10.2.8.8-1: <execInstance> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to create a resource. |
| Information on Request message | op: R fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | If the retrieval is allowed, the IN-CSE can retrieve the execution status or results on the remote entity using existing management protocol (i.e., BBF TR-069 [i.4]) If the retrieval is allowed, the addressed attributes of the <execInstance> resource shall be retrieved from the repository of the IN-CSE |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the retrieval is not allowed or the specific <execInstance> resource does not exist in the IN-CSE, there is no local processing on the hosting CSE and a proper error code shall be returned to AE in the Response Message |

#### 10.2.8.9	Delete <execInstance>

The DELETE request is used by an originating AE to delete an existing <execInstance> resource on a hosting CSE.

Originator: The Originator shall request to delete an existing <execInstance> resource by using a DELETE operation. The request shall address the specific "<execInstance>" resource of an <mgmtCmd> on the hosting CSE.

The originator shall be an AE.

NOTE 1:	The hosting CSE in the network domain could also delete an <execInstance> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <execInstance> resource could also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: The Receiver shall check if the Originator has the DELETE permission on the addressed <execInstance> resource. Upon successful validation, the hosting CSE shall remove the resource from its repository. If the <execInstance> is not finished on the remote entity and it is cancellable, the hosting CSE shall also use existing management protocols (i.e., BBF TR-069 [i.4] CancelTransfer RPC) to cancel the corresponding management currently initiated at the remote entity. Then the CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be an IN-CSE.

- Table 10.2.8.9-1: <execInstance> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to delete the resource |
| Information on Request message | op: D fr: Originator AE-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in clause 9.6.15, as needed |
| Local processing on Hosting CSE | If the <execInstance> is not finished yet and it is cancellable, the IN-CSE shall cancel the <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e., CancelTransfer RPC in BBF TR-069 [i.4]) The <execInstance> resource shall be deleted from the repository of the IN-CSE. If the corresponding initiated commands cannot be deleted from remote entity due to some reason (e.g., not found), the <execInstance> resource shall be still deleted be performed |
| Information on Response message | No change from the basic procedures |
| Post-Conditions | No change from the basic procedures |
| Exceptions | If the deletion is not allowed or the specific <execInstance> resource does not exist in the IN-CSE, there is no processing on the Hosting CSE and a proper error code shall be returned to AE in the Response message If the <execInstance> is already complete or it is not cancellable, there is no processing on the Hosting CSE |

### 10.2.9	Supported Attribute Discovery Procedure

#### 10.2.9.1	Introduction

The supported attribute discovery procedure allows discovering of supported attributes of a certain resource type in a CSE. This procedure shall be applied when the Originator is either an AE or a CSE and the Receiver is a CSE. This specification defines multiple optional attributes for each resource type, which means that Originator can suffer multiple times of trial and error for accessing (e.g., creating or updating) attributes if the Originator doesn't know which attributes of each resource type are supported by the Receiver. This supported attribute discovery procedure helps reduce unnecessary trial and error by discovering which attributes of the resource type are supported in the target Receiver.

In order to reduce communicational overhead between Originator and Receiver, this procedure discovers supported attributes from only optional attributes of a certain resource type. After this procedure, the Originator should send requests to the Receiver only including the supported optional attributes confirmed by the Receiver with this procedure.

Supported attributes discovery shall be performed using the RETRIEVE request with the parameter indicating supported attributes discovery by the Originator to distinguish from discovery of resource and retrieval of resource. This discovery supports retrieving list of all the attributes and child resource types of a certain resource type, or informing whether the attribute is supported or not.

#### 10.2.9.2	Supported Attributes Discovery procedure via Retrieve Operation

This procedure shall be used for discovering supported optional attributes of a CSE. The discovery result shall be returned to the Originator using a successful message.

Originator: The Originator shall be either an AE or a CSE. The Originator shall provide the <CSEBase> of the Receiver and the parameter indicating supported attributes discovery in the request. Resource type name shall be included in the request and attribute name may be included in the request.

Receiver or Hosting CSE: The Receiver shall be a CSE. If the Originator sends Retrieve operation with supported attributes discovery indicator, the Receiver shall perform the following procedures based on parameters of Originator's Request.

Receiver CSE will be able to maintain information about supported resource types to be exposed to other CSEs.

[10.2.9.2.a]  Editor's Note: How to maintain/expose supported resource types of CSEs is FFS.

- If the request addresses <CSEBase> with a resource type, the Receiver shall respond to the Originator with the list of all the supported optional attributes and supported optional child resource types the Receiver supports for that resource type if the Receiver supports the resource type.

- If the Receiver doesn't support the resource type the Originator indicates in the Request, the Receiver informs the Originator the resource type is not supported.

- If the Request addresses <CSEBase> with resource type and attribute, the Receiver shall respond to the Originator whether the attribute of the resource type is supported or not by the Receiver if the Receiver supports the resource type.

- If the Request addresses <CSEBase> with attribute only, the Receiver shall respond to the Originator with format error.

- Table 10.2.9.2-1: Supported Attribute Discovery Procedure via Retrieve Operation

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to discover supported resource types and attributes in the Hosting CSE |
| Information on Request message | to: <CSEBase> of Receiver cn: - Supported attributes discovery indicator to determine this request is supported attribute discovery - Resource type and/or attribute the Originator needs to discover |
| Local processing on Hosting CSE | Procedures described for the Receiver of the Retrieve Request as described above |
| Information on Response message | Contains the list of name of attributes Contains indicator whether attribute is supported or not |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE The Request contains invalid parameters (e.g., only attribute parameter is specified) |

### 10.2.10	Location Management Procedures

#### 10.2.10.1	Procedure related to <locationPolicy> resource

This clause introduces the procedures for obtaining and managing a target M2M Node's location information, which are associated with the <locationPolicy> resource that contains the method for obtaining and managing location information.

##### 10.2.10.1.1	Create <locationPolicy>

This procedure shall be used for creating a <locationPolicy> resource.

Originator: The Originator shall request to CREATE a <locationPolicy> resource including the relevant attributes and the address <CSEBase> resource of a Hosting CSE. Minimally, the Request shall provide the mandatory attributes defined in the Table 9.6.10-1. The Originator in this case is an AE or CSE.

[10.2.10.1.1.a]  Editor's Note: Handling of various levels of the <locationPolicy> resources needs to be defined.

Receiver: For the CREATE procedure, the Receiver (Hosting CSE) shall:

- Check whether the Originator is authorized to request the procedure.

- Check whether the provided attributes of the <locationPolicy> resource represent a valid Request.

- Upon successful validation of the above procedures, the Hosting CSE creates the <locationPolicy> resource and automatically creates <container> resource where the actual location information is/are stored and the resources shall contain cross-reference between the both resources: locationContainerID attribute for <locationPolicy> resource and locationID attribute for <container> resource.

- Check the defined locationSource attribute to determine which method is used.. The locationSource attribute shall be set based on the capabilities of a target M2M Node, the required location accuracy of the Originator and the Underlying Network in which a target M2M Node resides:

- For the Network-based case, the Hosting CSE shall transform the Request from the Originator into Location Server request following the attributes (e.g., locationTargetID, locationServer) defined in the <locationPolicy> resource. Additionally, the hosting CSE shall also provide default values for other parameters (e.g., required quality of position) in the Location Server request [i.1] (wrong reference) according to local policies. The request towards the Location Server crosses over the Mcn reference point. Then the Location Server in the Underlying Network performs positioning procedures, and returns the results over the Mcn reference point.

- The specific mechanism used to communicate with the network Location Server depends on the capabilities of the Underlying Network and other factors. For example, it could be either the OMA Mobile Location Protocol [i.7] or OMA RESTful NetAPI for Terminal Location [i.8].

NOTE:	The details of the mechanisms are addressed in the oneM2M Core Protocol Specification [i.2].

[10.2.10.1.1.b]  Editor's Note:  Align and/or provide references.

- For the Device-based case, this case is applicable if the Originator is ASN-AE and the ASN has location determination capabilities (e.g., GPS). The Hosting CSE is capable of performing positioning procedure using the module or technologies. For example, if the ASN has a GPS module itself, the ASN-CSE obtains the location information of Node from the GPS module through internal interfaces (e.g., System call or JNI). The detail procedure is out-of-scope.

- For the Sharing-based case, this case shall be applicable if the Originator is an AE and the Hosting CSE is MN-CSE and the ADN is a resource constrained node, no location determination capabilities (e.g., GPS) and Network-based positioning capabilities. Also according to the required location accuracy of the AE, the Originator may choose this case.

- When the Hosting CSE receives the CREATE request and if the Hosting CSE can find the closest Node that is registered with the Hosting CSE and has location information from the Originator in the M2M Area Network, the location information of the closest Node shall be stored as the location information of the Originator, or if the Hosting CSE cannot find any closest Node or has no topology information, the location information of the Node of the Hosting CSE (MN) shall be stored as the location information of the Originator. The closest Node can be determined by the minimum hop based on the topology information stored in the <node> resource.

- Table 10.2.10.1.1-1: <locationPolicy> CREATE

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: the URI of the <CSEBase> resource cn: The representation of the <locationPolicy> resource described in clause 9.6.10 |
| Local processing on Hosting CSE | Detail procedure for the Hosting CSE of the CREATE request described above |
| Information on Response message | The representation of the created <locationPolicy> resource |
| Post-Conditions | Detail steps for the Hosting CSE after the CREATE request are described above |
| Exceptions | No change from the generic procedure |

##### 10.2.10.1.2	Retrieve <locationPolicy>

This procedure shall be used for retrieving an existing <locationPolicy> resource.

Originator: The Originator shall request to obtain <locationPolicy> resource information by using RETRIEVE operation. The Originator is either an AE or a CSE.

Receiver: The Receiver shall check if the Originator has RETRIEVE permission on the <locationPolicy> resource. Upon successful validation, the Hosting CSE shall respond to the Originator with the appropriate responses.

- Table 10.2.10.1.2-1: <locationPolicy> RETRIEVE

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: the URI of the target <locationPolicy> resource |
| Local processing on Hosting CSE | No change from the generic procedure |
| Information on Response message | No change from the generic procedure |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

##### 10.2.10.1.3	Update <locationPolicy>

This procedure shall be used for updating an existing <locationPolicy> resource.

Originator: The Originator shall request to update attributes of an existing <locationPolicy> resource by using an UPDATE operation. The request shall address the specific <locationPolicy> resource of a CSE. The Originator may be either an AE or a CSE.

Receiver: The Receiver of an UPDATE request shall check whether the Originator is authorized to request the operation. The receiver shall further check whether the provided attributes of the <locationPolicy> resource represent a valid request for updating <locationPolicy> resource. The updatable attributes are (excluding common attributes):

- locationUpdatePeriod: This value is updated to change the period for updating location information.

- Table 10.2.10.1.3-1: <locationPolicy> UPDATE

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: the URI of the target <locationPolicy resource cn: The attributes which are to be updated. |
| Local processing on Hosting CSE | No change from the generic procedure |
| Information on Response message | No change from the generic procedure |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

##### 10.2.10.1.4	Delete <locationPolicy>

This procedure shall be used for deleting an existing <locationPolicy> resource.

Originator: The Originator shall request to delete an existing <locationPolicy> resource by using the DELETE operation. The Originator may be either an AE or a CSE.

[10.2.10.1.4.a]  Editor's Note: If deleted by CSE is the AE that created it notified.

Receiver: The Receiver shall check if the Originator has DELETE permission on the <locationPolicy> resource. Upon successful validation, the CSE shall remove the resource from its repository and shall respond to the Originator with appropriate responses.

Once the <locationPolicy> resource is deleted, the Receiver shall delete the associated resources (e.g., <container>, <contentInstance> resources). If the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource has been set with appropriate value, the Receiver shall tear down the session. The specific mechanism used to tear down the session depends on the support of the Underlying Network and other factors.

- Table 10.2.10.1.4-1: <locationPolicy> DELETE

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions | When the locationSource of the created <locationPolicy> resource is "sharing-based" and the AE disconnects from the registered MN-CSE |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: the URI of the target <locationPolicy resource |
| Local processing on Hosting CSE | No change from the generic procedure |
| Information on Response message | No change from the generic procedure |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.10.2	Procedure when the <container> and <contentInstance> resource contain location information

Since the actual location information of a target M2M Node shall be stored in the <contentInstance> resource as per the configuration described in the associated <locationPolicy> resource, this clause introduces the procedures related to the <contentInstance> and <container> resource.

##### 10.2.10.2.1	Procedure for <container> resource that stores the location information

This procedure is mainly triggered by the creation of <locationPolicy> resource. Based on the defined attributes related to the <container> resource such as ‘locationContainerID’ and ‘locationContainerName’, the Hosting CSE will create <container> resource to store the location information in its child resource, <contentInstance> resource  after the CSE obtains the actual location information of a target M2M Node. Since the <container> is unnamed resource type, the name of the created <container> resource shall be determined by the ‘locationContainerID if it is available. After the creation of the <container> resource, the actual location of the resource shall be stored in the ‘locationContainerID’.

##### 10.2.10.2.2	Procedure for <contentInstance> resource that stores location information

After the <container> resource that stores the location information is created, each instance of location information will be stored in the different <contentInstance> resources. In order to store the location information in the <contentInstance> resource, the Hosting CSE firstly checks the defined ‘locationUpdatePeriod’ attribute. If a valid period value is set for this attribute, the Hosting CSE performs the positioning procedures as defined period value, locationUpdatePeriod, in the associated <locationPolicy> resource and stores the results (e.g., position fix and uncertainty) in the <instanace> resource under the created location resource. However, if no value (e.g., null) is set, the positioning procedure is performed when the created <container> is retrieved and the result will be stored in the <contentInstance> resource.

### 10.2.11	<subscription> Resource Procedures

#### 10.2.11.1	Introduction

An Originator can create a <subscription> resource on a subscribed-to resource Hosting CSE to be notified when the resource is modified. After successful <subscription> resource creation, the Hosting CSE shall notify the Originator of a subscribed-to resource modification that meets conditions configured in the <subscription> resource.

A subscription shall be represented by a <subscription> resource (see clause 9.6.8). This allows manipulation of the subscription in a resource oriented manner, e.g., the conditions of a subscription may be modified by modifying a <subscription> resource, or a resource subscriber may unsubscribe by deleting the <subscription> resource.

The following clauses describe procedures for Creation, Retrieval, Update and Deletion of a <subscription> resource.

#### 10.2.11.2	Create <subscription>

This procedure shall be used to request a new subscription to be notified for the modifications of a subscribed-to resource. Generic create procedure is described in clause 10.1.1.1.

Originator: The Originator can be an AE or a CSE. The Originator shall request to create a <subscription> resource. The Request shall address a subscribable resource.

The Request shall include notificationURI(s).

Receiver: Which is also the Hosting CSE shall validate the followings:

- if the subscribed-to resource, addressed in the to parameter in the Request, is a subscribable resource;

- if the Originator has  privileges for retrieving the subscribed-to resource;

- if the Originator has privileges to send notifications to the entity(ies) hosting notificationURI(s) when the Originator is not a hosting notificationURI;

- if it has access control privileges to send notifications to the entity(ies) that hosting notificationURI(s).

[10.2.11.2.b]   Editor's Note: The validation process described in the last two bullet points above needs to be investigated further, This is FFS.

If all those checks are successfully done, the Hosting CSE shall send positive Response to the Originator.

- Table 10.2.11.2-1: <subscription> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | The Originator configures proper permissions and access control privileges described above |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the table with the specific details for: cn: The representation of the <subscription> resource for which the attributes are described in clause 9.6.8 The following attributes from clause 9.6.8 are mandatory for the request: - “resourceType” which shall be set to the appropriate tag that identify the subscription resource as defined in 9.6.1 - “notificationURI” - “notificationStructure” All attributes indicated as optional in clause 9.6.8 are optional for the request. |
| Local processing on Hosting CSE | Check access privileges regarding creation of the <subscription> resource and sending notification to the notificationURI. |
| Information on Response message | No change from the generic procedure in clause 10.1.1.1 |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.1.1) are applicable. |

#### 10.2.11.3	Retrieve <subscription>

This procedure shall be used to retrieve information of a subscription such as notificationURI, filterCriteria, expirationTime, etc. The generic retrieve procedure is described in clause 10.1.2.

Originator: The Originator can be an AE or a CSE. The Request shall address a <subscription> resource or one of its attributes.

Receiver: Once the Originator has been successfully authenticated and access to resource has been granted by the Receiver (which is also the Hosting CSE), the Receiver shall create the <subscription> resource.

- Table 10.2.11.3-1: <subscription> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | The Originator has RETRIEVE permission to the <subscription> resource |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table. |
| Local processing on Hosting CSE | No change from the generic procedure. |
| Information on Response message | No change from the generic procedure |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.2) are applicable. |

#### 10.2.11.4	Update <subscription>

This procedure shall be used to update an existing subscription, e.g., extension of its lifetime or the modification of the notificationURI. Generic update procedure is described in clause 10.1.3.

Originator: The Originator can be an AE or a CSE. The Originator may send new value(s) for read-write attribute(s) of a <subscription> resource.

Receiver: Once the Originator has been successfully authenticated and access to the resource is granted by the Receiver (which is also the Hosting CSE), the Receiver shall update <subscription> resource.

- Table 10.2.11.4-1: <subscription> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | The Originator has UPDATE permission to the <subscription> resource |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table. |
| Local processing on Hosting CSE | No change from the generic procedure |
| Information on Response message | No change from the generic procedure |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.1) are applicable. |

#### 10.2.11.5	Delete <subscription>

This procedure shall be used to unsubscribe an existing subscription. Generic delete procedure is described in clause 10.1.4.

Originator: The Originator can be an AE or a CSE.

Receiver: Once the Originator has been successfully authenticated and access to the resource has been granted by the Receiver (which is also the Hosting CSE), the Receiver shall delete the <subscription> resource.

- Table 10.2.11.5-1: <subscription> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | The Originator has DELETE permission to the <subscription> resource |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table. |
| Local processing on Hosting CSE | No change from the generic procedure |
| Information on Response message | No change from the generic procedure |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.4) are applicable. |

### 10.2.12	Notification Procedures for Resource Subscription

This procedure shall be used to notify Receiver(s) of modifications of a resource for an associated <subscription> resource.

Originator: The Originator can be a CSE which is hosting a <subscription> resource. The Originator shall consider sending a notification request containing the subscribed information following the detection of a modification of the subscribed-to resource that matches the specified filterCriteria as specified in the <subscription> resource.

The Originator shall contain a notification structure and send this to Receiver(s).

In case the notification is forwarded or aggregated by transit CSEs, the originator shall contain attributes related to notification policies such as latestNotify or batchNotify in the notification so that transit CSE is able to enforce the policy defined by the subscriber.

[10.2.12.a]  Editor’s Note: The notification policy enforcement between subscription resource hosting CSE and the notification target CSE need to be FFS.

Receiver: The Receiver can be AE(s) or a CSE(s) which shall be presented in the notificationURI attribute of the <subscription> resource. The Receiver responds positively to the Request. The Receiver can also be a transit CSE in case the notification is forwarded or aggregated.

A considered Notification Request shall be sent when notification policies are satisfied.

The expirationCounter shall be decreased by one when the Originator successfully sends the notification request to Receiver(s). If the counter meets zero, the corresponding subscription resource is deleted.

The batchNotify policy is based on a minimum number of notification events and a maximum time to wait for that number of events. Notification events shall be temporarily stored for some duration (e.g., 10 minutes) or until a specified number are stored (e.g., 20 notifications) before sending - then sent when the first of these two conditions are satisfied. The sending order is first-in first out (FIFO). notificationEventCat is checked at the time of batch transmission and applied to each notification in the batch. Stored notification events may be dropped according to the notificationStoragePriority and the notificationCongestionPolicy (see clause 9.6.3).

The rateLimit policy is based upon a maximum specified number of events (e.g., 10, 000) that can be sent within some specified rateLimitWindow duration (e.g., 60 seconds). The ratelimitWindows are sequential (not rolling). A considered Notification Request may only be sent whenever the current total number of events sent is less than the maximum number of events within the current rateLimitWindow duration. Notification events that do not meet this policy are temporarily stored or dropped according to the notificationStoragePriority and the notificationCongestionPolicy (see clause 9.6.3). In the event that transmission of a batch triggers rate-limiting, the transmission of the batch shall complete, and the rate limiting mechanism may hold off sending subsequent notifications within the rateLimit unit time in which transmissions have exceeded the allowed notifications.

The interimEventsNotify policy indicates the notification action to be taken following a period of connectionless (e.g., reachability schedule, network malfunction), when Receiver(s) again become reachable. When a notification is ready to be sent, the Originator shall verify with CMDH CSF whether connectivity to the Receiver is currently allowed for the notification request. If that is the case, the notification shall be passed on to CMDH CSF for delivery. However, during the period of loss of connectivity the interimEventsNotify policy is applied if the notification message needs not to be immediately sent.

The interimEventsNotify attribute specifies how many of the most recent eligible notifications during the connectionless (interim) period shall be sent to Receiver(s) relative to Receiver(s) reconnect time. When a notification becomes eligible in the connectionless period, it is temporarily stored in the Originator. After the connection recovery, the Originator shall determine which notifications to send according to the interimEventsNotify attribute and send notifications (if available) to Receiver(s). The interimEventsNotify attribute shall be classified into "sendNo" (i.e., ignore prior notifications), "sendLatest" (i.e., send the last pending notification), "sendAll" (i.e., send all pending notifications).

- Figure 10.2.12-1: Notification Mechanism when interimEventsNotify (sendNo) is used

- Figure 10.2.12-2: Notification Mechanism when interimEventsNotify (sendLatest) is used

Figure 10.2.12-3:  Notification Mechanism when interimEventsNotify (sendAll) is used

The priorSubscriptionNotify policy indicates the notification action to be taken following a new subscription. It enables sending of event notifications, which happened before the new subscription was created. When a new subscription is created, the most recent number "n" subscription events prior to the new subscription shall be sent to Receiver(s), if available.

The latestNotify indicates if the subscriber is only interested in the latest state of the subscribed-to resource. In the case the Receiver is a transit CSE which forwards or aggregates the notifications before sending to the subscriber or the other transit CSEs, upon receiving the notification with the latestNotify attribute, the Receiver shall identify the latest notification with the same subscription reference while storing the notifications locally. When the receiver as an transit CSE needs to send the pending notifications, it shall send the latest notification.

[10.2.12.b] Editor’s Note: How the latestNotify is carried in the notification is FFS.

The notificationDeliveryPriority policy indicates how to prioritise sending of notifications when notifications need to be sent. A notification with the higher priority indicated by the notificationDeliveryPriority attribute shall be sent before notifications with the lower priority. In order not to constantly defer notifications with the lower priority; expiration time of the notification needs to be taken into account. In case the connectivity is lost and the notification priority exceeds a specific threshold, then if the loss of connectivity is due to the Originator’s schedule then the schedule will be over-ridden and that notification shall be sent immediately.

[10.2.12.c]  Editor’s Note: The specific threshold mentioned above needs to be added as an attribute in the <CSEBase> resource type.

The notificationEventCat policy indicates an Event Category of the subscription that will be included in the notification request to be able for the Receiver to correctly handle the notification. When the notificationEventCat policy is not configured by the subscriber, it shall be determined as a default value by the CMDH policy.

- Table 10.2.12-1:  Notification Procedure

| Description |  |
| --- | --- |
| Call Flow Type | NOTIFY |
| Pre-Conditions | Notification is triggered regarding subscription information in a <subscription> resource |
| Information on Request message | fr: ID of the Originator to: notificaitonURI specified in <subscription> resource cn: - notification data that represents the modified content of subscribed-to resource may be included - subscription reference (i.e., URI of the corresponding <subscription> resource) that generates this notification shall be included - changed resource status shall be included when resourceStatus filter criteria condition is configured - monitored operation shall be included when operationMonitor filter criteria condition is configured |
| Information on Response message | No change from the basic procedure |
| Post-Conditions | None |
| Exceptions | None |

### 10.2.13	Polling Channel Management Procedures

#### 10.2.13.1	Introduction

An AE or a CSE that is request unreachable cannot receive a request from other entities directly. Instead this AE/CSE can retrieve requests that others sent to this AE/CSE once it created <pollingChannel> resource on a request reachable CSE.

This clause consist of manipulation procedures of <pollingChannel> resource, re-targeting request to <pollingChannel> and the Long Polling procedure to retrieve requests from <pollingChannel>.

#### 10.2.13.2	Create <pollingChannel>

This procedure is used to create a <pollingChannel> resource and an AE/CSE can be an Originator. After the creation of the <pollingChannel> resource, the AE/CSE can perform Long Polling on it.

Originator: Can be an AE/CSE. If an AE is the Originator, it addresses the <AE> resource that it already created. Otherwise, if a CSE is the Originator, it addresses the <remoteCSE> resource that it already created. With to parameter addressing the <AE> or <remoteCSE> resource, the Originator sends the request to the Receiver.

Receiver: shall perform the following.

- Check if the Originator has a CREATE privilege to create <pollingChannel> resource under <AE> resource or <remoteCSE> resource. If the Originator is the AE who created the <AE> resource that the request is targeting, the request shall be granted. If the Originator is the CSE who created the <remoteCSE> resource that the request is targeting, the Request shall be granted. Otherwise, the Request will be rejected.

- Send the successful/unsuccessful Response to the Originator depending on the check above.

Table 10.2.13.2-1: <pollingChannel> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | The Originator is request-unreachable. |
| Information on Request message | op: C fr: ID of the Originator to: URI of <AE> or <remoteCSE> resource cn: Mandatory and/or optional attributes defined in clause 9.6.9. |
| Local processing on Hosting CSE | Check access control policies regarding creation of the <pollingChannel> resource. |
| Information on Response message | No change from the basic procedure in clause 10.1.1 |
| Post-Conditions | None |
| Exceptions | No change from the basic procedure in clause 10.1.1 |

#### 10.2.13.3	Retrieve <pollingChannel>

This procedure is used to retrieve a <pollingChannel> resource and an AE/CSE can be an Originator.

Originator: Sends a Request to the Receiver addressing a <pollingChannel> resource.

Receiver: Shall perform the following.

- Check if the Originator has a RETRIEVE privilege to create <pollingChannel> resource.

- Send the successful/unsuccessful response to the Originator depending on the check above.

Table 10.2.13.3-1: <pollingChannel> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: ID of the Originator to: URI of the <pollingChannel> resource cn: Mandatory and/or optional attributes defined in clause 9.6.9. |
| Local processing on Hosting CSE | Check access control policies regarding retrieval of the <pollingChannel> resource |
| Information on Response message | NOTE: Virtual attribute longPollingURI is not included. No change from the basic procedure in clause 10.1.2 |
| Post-Conditions | None |
| Exceptions | No change from the basic procedure in clause 10.1.2. |

#### 10.2.13.4	Update <pollingChannel>

This procedure is used to update a <pollingChannel> resource and an AE/CSE can be an Originator.

Originator: Sends a Request to the Receiver addressing a <pollingChannel> resource.

Receiver: Shall perform the following.

- Check if the Originator has a UPDATE privilege to create <pollingChannel>.

- Send the successful/unsuccessful response to the Originator depending on the check above.

Table 10.2.13.4-1: <pollingChannel> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: ID of the Originator to: URI of the <pollingChannel> resource cn: Mandatory and/or optional attributes defined in clause 9.6.21. |
| Local processing on Hosting CSE | Check access control policies regarding creation of the subscription resource and sending notification to the notificationURI. |
| Information on Response message | No change from the basic procedure in clause 10.1.3 |
| Post-Conditions | None |
| Exceptions | No change from the basic procedure in clause 10.1.3 |

#### 10.2.13.5	Delete <pollingChannel>

This procedure is used to delete a <pollingChannel> resource and an AE/CSE can be an Originator.

Originator: Sends a Request to the Receiver addressing a <pollingChannel> resource.

Receiver: Shall perform the following.

- Check if the Originator has a UPDATE privilege to create <pollingChannel>.

- Send the successful/unsuccessful Response to the Originator depending on the check above.

Table 10.2.13.5-1: <pollingChannel> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | op: D fr: ID of the Originator to: URI of the <pollingChannel> resource |
| Local processing on Hosting CSE | Check access control policies regarding deletion of the <pollingChannel> resource |
| Information on Response message | No change from the basic procedure in clause 10.1.4 |
| Post-Conditions | None |
| Exceptions | No change from the basic procedure in clause 10.1.4 |

#### 10.2.13.6	Re-targeting Requests to Polling Channel

This procedure is used to re-target a request targeting a request-unreachable AE/CSE. When a <pollingChannel> hosting CSE receives a request to the request-unreachable AE/CSE, it internally re-targets the request to the <pollingChannel> of the AE/CSE.

It is assumed that the request-unreachable AE/CSE already set its requestReachability attribute as FALSE and created the <pollingChannel> resource. If there is no <pollingChannel> for the entity and requestReachability is FALSE, re-targeting is not performed. Re-targeted requests shall be transmitted to its target entity when it performs Long Polling on its <pollingChannel> resource.

#### 10.2.13.7	Long Polling on Polling Channel

This procedure is originated by a request-unreachable entity to poll requests from a polling channel. Once the Originator starts Long Polling on a <pollingChannel> by sending a RETRIEVE request, the Receiver who is the <pollingChannel> hosting CSE holds the request until it has any responses to return to the Originator. If the request expires and there’s no available request return, the Receiver shall send the response to inform the Originator that a new polling request should be generated again.

Originator: Sends a RETRIEVE Request to the Receiver addressing longPollingURI attribute  of a <pollingChannel> resource.

Receiver: Shall perform the following.

- Check if the Originator is the creator of the <pollingChannel> resource.

- If creator is not present in the resource, check if the Originator ID is the same as the CSE-ID or AE-ID of the parent resource.

- If both checks are failed, the Receiver shall reject the request.

- Check if there is any request to be returned to the Originator. If there is any, the Receiver shall generate the response containing the request(s) for the Originator. If none, the Receiver shall wait for any request for the Originator to be reached at the polling channel until the request expiration time.

- Send the successful/unsuccessful response to the Originator depending on the checks above.

Table 10.2.13.7-1: Long Polling RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | The Originator created a <pollingChannel> |
| Information on Request message | op: R fr: ID of the Originator to: longPollingURI of the <pollingChannel> resource |
| Local processing on Hosting CSE | Check ownership of the <pollingChannel> resource as described above. If it is confirmed and there’s available request(s) to return, the Receiver generate the response message containing the request(s). |
| Information on Response message | cn: request message(s) targeting the Originator |
| Post-Conditions | If the Originator receives the response from the Receiver that the Long Polling request is expired, the Originator can send a new Long Polling request. |
| Exceptions | If the Long Polling request is expired at the Receiver, the Receiver shall send an unsuccessful response to the Originator with the request expiry information. |

### 10.2.14	<node> Resource Procedure

#### 10.2.14.1	Create <node>

This procedure shall be used for creating a <node> resource.

Originator: The Originator shall request to create a new <node> resource. The request shall address different resources depending on the type of node that is issuing the request, as described in 9.6.18. Following that, the Originator can create the child resources (defined in the 9.6.18) to acquire node specific information as occasion demands of the Originator. The Originator shall be an AE or a CSE.

Receiver: The Receiver or Hosting CSE shall:

- Check whether the Originator is authorized to request the procedure.

- Check whether the provided attributes of the <node> resource represent a valid Request.

- Upon successful validation of the above procedures, create a new <node> resource with given name in the Hosting CSE with the provided attributes.

Table 10.2.14.1-1: <node> CREATE

| Description |  |
| --- | --- |
| Pre-Conditions | Originator has already registered. |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: The representation of the <node> resource described in clause 9.6.18. The following attributes from clause 9.6.18 are mandatory for the request: - “resourceType” which shall be set to the appropriate tag that identify the <node> resource as defined in 9.6.1 The following attributes from clause 9.6.18 are optional for the request: - “accessControlPolicyID” - “expirationTime” - “labels” NOTE: if the Originator is a CSE, it could take the information that is stored in the <node> resource under its own <CSEBase> resource and provide the information in the cn. |
| Local processing on Hosting CSE | No change from the basic procedure described in clause 10.1.1 |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: URI of the created <node> resource , according to clause 10.1.1 |
| Post-Conditions | If the Originator is either ASN or MN-CSE registered with a CSE: The registered CSE checks the announceTo attributes whether the created <node> resource under the <CSEBase> is announced or not. If so, the created <node> resource is automatically announced by the registered CSE. |
| Exceptions | All exception described in the basic procedure (clause 10.1.1) are applicable. |

#### 10.2.14.2	Retrieve <node>

This flow is used for retrieving the attributes of a <node> resource.

Originator: The Originator shall request to obtain <node> resource information by using RETRIEVE operation. The Originator can be either an Application Entity or a CSE.

Receiver: The Receiver shall:

- Check if the Originator has RETRIEVE privileges on the <node> resource and the request is valid.

- Upon successful validation, the Hosting CSE shall respond to the Originator with the appropriate responses.

Table 10.2.14.2-1: <node> RETRIEVE

| Description |  |
| --- | --- |
| Pre-Conditions | The targeted <node> has been already created |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: void |
| Local processing on Hosting CSE | No change from the basic procedure described in clause 10.1.2 |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: attributes of the <node> resource as defined in clause 9.6.18. |
| Post-Conditions | None |
| Exceptions | All exception described in the basic procedure (clause 10.1.2) are applicable. |

#### 10.2.14.3	Update <node>

This flow is used for updating the attributes and the actual data of a <node> resource and its child resources.

Originator: The Originator shall request to update attributes of an existing <node> resource by using an UPDATE operation. The request shall address the specific <node> resource reside in a CSE. The Originator may be either an AE or a CSE.

Receiver: The Receiver of an UPDATE request shall check whether the Originator is authorized to request operation. The Receiver shall check whether the provided attributes of the <node> resource represent a valid request for updating <node> resource.

Table 10.2.14.3-1: <node> UPDATE

| Description |  |
| --- | --- |
| Pre-Conditions | The targeted <node> has been already created |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: attributes of the <node> resource as defined in 9.6.18 which need be updated, with the exception of the Read Only (RO) attributes cannot be modified |
| Local processing on Hosting CSE | No change from the generic procedure from clause 10.1.3. |
| Information on Response message | No change from the generic procedure from clause 10.1.3. |
| Post-Conditions | None |
| Exceptions | All exception described in the basic procedure (clause 10.1.3) are applicable. |

#### 10.2.14.4	Delete <node>

This procedure shall be used for deleting an existing <node> resource.

Originator: The Originator shall request to delete an existing <node> resource by using DELETE operation. The Originator can be either an AE or a CSE.

Receiver or Hosting CSE: The Receiver shall check if the Originator has DELETE permission on the <node> resource and the request is valid. Upon successful validation, the CSE shall remove the resource from its repository and shall respond to the Originator with appropriate responses.

Table 10.2.14.4-1: <node> DELETE

| Description |  |
| --- | --- |
| Pre-Conditions | The targeted <node> has been already created |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table. |
| Local processing on Hosting CSE | No change from the generic procedure from clause 10.1.4. |
| Information on Response message | No change from the generic procedure from clause 10.1.4. |
| Post-Conditions | The targeted resource and all the children resources shall not exist anymore. Also all the related credential shall be deleted. [10.2.14.4.a] Editor’s Note: if and how credentials are cleaned is FFS, in scope of WG4. |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable. |

### 10.2.15	Service Charging and Accounting Procedures

#### 10.2.15.1	Introduction

This clause is informative and provides a use case example to explain how the Infrastructure Node provides  statistics for AEs using the <statsConfig> and <statsCollect> resources as defined in clause 9.6.22, 9.6.23.and 9.6.24.

##### 10.2.15.1.1	Service Event-based Statistics Collection for Applications

Figure 10.2.15.1-1 shows an example of service layer event-based charging based on the Infrastructure Node.

Step 1-2:   A statistics collection resource called <statsConfigSCA1> was created at the IN-CSE by a billing application. Note that the <statsConfig> can also be provisioned. In this use case, the <statsConfigSCA1> has the <eventConfig> sub-resource. For this specific use case, the <eventConfig> can be set as following: The “eventID” is set with a unique ID to differentiate from other chargeable events. The “eventType” defines what event will trigger the generation of service statistics collection record and is set to “Data Operation” for this case. “eventStart” and “eventEnd” apply to timer based event so they will not be included in this event. “transactionType” will be “RETRIEVE”. “dataSize” does not apply so it is not included.

Step 3-5:  In this example, AE1 already registered to IN-CSE. IN-CSE can make the statistics collection configuration accessible by AE. Based on the <statsConfigSCA1>, AE1 creates a statistics collection trigger for itself, stored in <statsCollectAE1>. AE1 will fill in the information for the collection rule. For example, it fills the “collectingEntityID” with the App-ID of AE1, and the “collectedEntityID” empty, which means to collect for any entities. “status” is set to “Active”. The “statsModel” is “event-based”. The “subscriberID” is set to the M2M-Sub-ID for AE1. The “eventID” is set with the same ID value as the “eventID” in the  <statsConfigSCA1>.  This event collection trigger can be saved at the IN-CSE and IN-CSE will assign a unique ID in attribute “statsCollectID”.

Step 6-8:  When the configured event happens, i.e., when AE2 performed a RETRIEVE operation to the data owned by AE1 at IN-CSE, the event is recorded by IN-CSE. The SCA CSF at IN-CSE generates a service statistics collection record that sends it to AE1. AE1 can choose to use such information for its own billing.

Step 9:      The AE of billing application can update or retrieve the charging policies and collection scenarios that it has the access control privilege.

Figure 10.2.15.1-1: Event-based Statistics Collection for Applications

#### 10.2.15.2 	CREATE <statsConfig>

This procedure shall be used for the Originator to establish a set of configurations for statistics collection at the Receiver.

The configurations shall be stored at the <statsConfig> resource and each instance of the <statsConfig> resource shall represent a specific configuration.

Originator: The Originator shall be an AE or the IN-CSE that wants to set up the statistics collection configurations. According to the definition of the <statsConfig>, the Originator shall:

- provide the sub-resource <eventConfig> for Event based statistics collection.

This sub-resource is used to define different events to be collected, each identified by a unique “eventID”.

eventID: The “eventID” shall uniquely identify an event. The Originator may provide a value in the Request. The Receiver shall verify whether the ID is unique or not, and if not, provides a new value.

eventType: The “eventType” is a mandatory attribute. The Originator shall provide an eventType. An example of an eventType is “Data Operation”, which means that chargeable events information recording is triggered when the charged entity conducts a data operation. “Data Operation” can be further refined by the “transactionType” attribute.

eventStart: The “eventStart” is an optional attribute. The Originator shall provide a start time for the event only if the “eventType” is set to “Timer based”.

eventEnd: The “eventEnd” is an optional attribute. The Originator shall provide an end time for the event only if the “eventType” is set to “Timer based”.

transactionType: The “transactionType” is an optional attribute. Examples of “transactionType” can be “CREATE”, “RETRIEVE”.

dataSize: The “dataSize” is an optional attribute. The Originator shall provide a value for it only if the “eventType” is set to “Storage based”.

Receiver: The Receiver shall be an IN-CSE containing the SCA CSF. The Receiver shall validate whether the Originator has proper permissions for creating a <statsConfig> resource, and whether the values provides in the Request message are valid. Upon successful validation, the Receive shall create a new <statsConfig> resource with the provided values.

Table 10.2.15.2-1: CREATE Call Flow

| Description |  |
| --- | --- |
| Call flow type | CREATE |
| Pre-conditions | Originator needs to create a <statsConfig> resource at the Receiver |
| Information on Request message | op: Create fr: ID of the Originator to: URI of the parent of the target resource to be created cn: contain the resource representation of <statsConfig> The Originator can provide the <eventConfig> sub-resource to enable event-based configuration for statistics collection The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.3	RETRIEVE <statsConfig>

The RETRIEVE call flow shall be used for the Originator to retrieve the existing <statsConfig> resource from the Receiver.

Originator: The Originator shall be an AE that is allowed to retrieve configuration information available for AE from the SCA CSF within a CSE.

Receiver: The Receiver shall be the IN- CSE containing the SCA CSF. The Receiver shall verify if the Originator has the READ permission on the <statsConfig> resource targeted. Upon successful validation, the Receiver CSE shall respond to the Originator with the resource representation.

Table 10.2.15.3-1: RETRIEVE Call Flow

| Description |  |
| --- | --- |
| Call flow type | RETRIEVE |
| Pre-conditions | Originator needs to retrieve the <statsConfig> resource at the Receiver |
| Information on Request message | op: Retrieve fr: ID of the Originator to: URI of the <statsConfig> resource or its attribute or sub-resource to be retrieved cn: void The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.4 	UPDATE <statsConfig>

An UPDATE procedure on the <statsConfig> resource is used for the Originator to update charging related policies at the Receiver.

Originator: The Originator shall be the AE that created the <statsConfig> resource. The same AE shall be able to update the resource.

Receiver: The Receiver shall be a CSE containing the SCA CSF. The Receiver shall check if the Originator has the WRITE permission to update the addressed resource.

Table 10.2.15.4-1: UPDATE Call Flow

| Description |  |
| --- | --- |
| Call flow type | UPDATE |
| Pre-conditions | The Originator needs to update the charging policies that it established at the Receiver by using the <statsConfig> resource |
| Information on Request message | op: Update fr: ID of the Originator to: URI of the <statsConfig> resource or its attribute or sub-resource to be updated cn: the Originator provides the sub-resource or attributes to be updated The Originator can update attributes under <eventConfig> to update event-based configuration for statistics collection The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.5 	DELETE <statsConfig>

Originator: The Originator shall be the AE that created the <statsConfig> resource.

Receiver: The Receiver shall be a CSE containing the SCA CSF.

Table 10.2.15.5-1: DELETE Call Flow

| Description |  |
| --- | --- |
| Call flow type | DELETE |
| Pre-conditions | Originator needs to DELETE the <statsConfig> resource at the Receiver |
| Information on Request message | op: Delete fr: ID of the Originator to: URI of the <statsConfig> resource to be deleted cn: void The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.6 	CREATE <statsCollect>

This procedure shall be used for the Originator to establish collection scenarios at the Receiver.

The collection scenarios are stored at the  <statsCollect> resource. Multiple collection scenarios can be created based on one <statsConfig>.

Originator: The Originator shall be an AE that wants to set up the collection scenarios to the SCA CSF in a CSE. According to the definition of the <statsCollect>  resource, the Originator shall populate the attributes defined for the <statsCollect> resource except for “statsCollectID”. (see clause 9.6.24). The “statsCollectID” shall be unique in the same service provider domain. To ensure the uniqueness, the IN-CSE shall create the statsCollectID.

Here are other attributes in the <statsCollect> resources:

collectingEntityID: The “collectingEntityID” is a mandatory attribute. The Originator shall provide the ID as defined in  clause 7.1 to identify the entity that charges.

collectedEntityID: The “collectedEntityID” is a mandatory attribute. The Originator shall provide the ID as defined in  clause 7.1 to identify the entity that being charged.

status: The “status” is a mandatory attribute. The Originator shall set the value for this attribute to specify the status of the rule. Examples of values can be “ACTIVE”, “INACTIVE”.

statsModel: The “statsModel” is a mandatory attribute. The Originator shall set the value to indicate a collection model. Examples of values can be “subscriber-based”, “event-based”.

subscriberID: The “subscriberID” is a mandatory attribute. It is the service subscriber ID for the collected entity. The Originator shall provide the “M2M-Sub-ID” as defined in clause 7.1.

collectPeriod: The “collectPeriod” is an optional attribute. It applies to the subscriber-based charging model. The Originator shall provide a value of the information recording collection period. An example of the value can be “monthly”.

eventID: The “eventID” is an optional attribute. The Originator shall provide a value only if the “statsModel” is “event-based”. The Originator shall provide the eventID that is defined in the resource <statsConfig>. Since there can be many different chargeable events defined by different charging policies, the eventID uniquely identifies the specific chargeable event for this charging rule.

Receiver: The Receiver shall be an IN-CSE containing the SCA CSF. The Receiver shall validate whether the Originator has proper permissions for creating a <statsCollect> resource. Upon successful validation, create a new <statsCollect> resource with the provided attributes. The IN-CSE shall also create a unique “statsCollectID”.

Once a <statsCollect> is created and the “status” is “ACTIVE, the SCA CSF in the IN-CSE shall generate service statistics collection record when the conditions defined by the rules are met.

Table 10.2.15.6-1: CREATE Call Flow

| Description |  |
| --- | --- |
| Call flow type | CREATE |
| Pre-conditions | Originator needs to create a <statsCollect> resource at the Receiver |
| Information on Request message | op: Create fr: ID of the Originator to: URI of the parent of the target resource to be created cn: contain the resource representation of <statsCollect> The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.7	RETRIEVE <statsCollect>

The RETRIEVE call flow shall be used for the Originator to retrieve the existing <statsCollect> resource from the Receiver.

Originator: The Originator shall be an AE that is allowed to retrieve the charging rule information from the SCA CSF within a CSE.

Receiver: The Receiver shall be the IN- CSE containing the SCA CSF. The Receiver shall verify if the Originator has the READ permission on the <statsCollect> resource targeted. Upon successful validation, the Receiver CSE shall respond to the Originator with the resource representation.

Table 10.2.15.7-1: RETRIEVE Call Flow

| Description |  |
| --- | --- |
| Call flow type | RETRIEVE |
| Pre-conditions | Originator needs to retrieve the <statsCollect> resource at the Receiver |
| Information on Request message | op: Retrieve fr: ID of the Originator to: URI of the <statsCollect> resource or its attribute to be retrieved cn: void The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.8	UPDATE <statsCollect>

An UPDATE procedure on the <statsCollect> resource is used for the Originator to update charging related policies at the Receiver.

Originator: The Originator shall be the AE that created the <statsCollect> resource. The same AE shall be able to update the resource.

Receiver: The Receiver shall be a CSE containing the SCA CSF. The Receiver shall check if the Originator has the WRITE permission to update the addressed resource.

Table 10.2.15.8-1: UPDATE Call Flow

| Description |  |
| --- | --- |
| Call flow type | UPDATE |
| Pre-conditions | The Originator needs to update the collection scenarios that it established at the Receiver by updating the <statsCollect> resource |
| Information on Request message | op: Update fr: ID of the Originator to: URI of the <statsCollect> resource or its attribute to be updated cn: the Originator provides the attributes to be updated The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.9	DELETE <statsCollect>

Originator: The Originator shall be the AE that created the <statsCollect> resource.

Receiver: The Receiver shall be a CSE containing the SCA CSF.

Table10. 2.15.9-1: DELETE Call Flow

| Description |  |
| --- | --- |
| Call flow type | DELETE |
| Pre-conditions | Originator needs to DELETE the <statsCollect> resource at the Receiver |
| Information on Request message | op: Delete fr: ID of the Originator to: URI of the <statsCollect> resource to be deleted cn: void The Originator may specify other Optional parameters as discussed in clause 8.1.2 Request |
| Local processing on Receiver CSE | No change from the generic procedure |
| Generic Information on Response message | No change from the generic procedure |
| Post-conditions | None |
| Exceptions | No change from the generic procedure |

#### 10.2.15.10	Service Statistics Collection Record

When the Service Statistics Collection is supported, the Information Elements shall be generated according to Table 10.2.15.10-1.

The contents of each Service statistics collection record are decided by the specific collection scenario that triggered the information recording.

Table 10.2.15.10-1: Information Elements for Service Statistics Collection Record

| Information Element | Mandatory / optional | Description |
| --- | --- | --- |
| statsCollectID | M | It is the unique ID that identifies a specific statistics collection scenario, which triggers information recording for a specific event. |
| collectingEntityID | M | This is the unique ID of the entity that collects the statistics. It can be an AE-ID or CSE-ID |
| collectedEntityID | M | This is the unique ID of the entity whose service layer operation statistics are being collected. It can be an AE-ID or CSE-ID. |
| subscriberID | M | The M2M service subscription ID M2M-Sub-ID if the collected entity |
| event | O | This indicates a specific event type in each record, such as timer based, data operation, storage triggering. It is only present if the statModel is “event based” |
| eventStart | O | The start time for the recording the M2M event record |
| eventEnd | O | The end time for the recording the M2M event record |
| transactionType | O | Specifies the detailed type of a transaction, such as CREATE, RETRIEVE, etc. |
| dataSize | O | Storage Memory in Kbytes, where applicable, to store data associated events with container related operations |
| Vendor Specific Information | O | Defines Vendor specific information |

### 10.2.16	<m2mServiceSubscription> Resource

#### 10.2.16.1	Create <m2mServiceSubscription>

This flow is used for creating a <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity (e.g., administrator AE of M2M Service Provider) or a Hosting CSE.

Receiver: The Receiver or Hosting CSE shall validate whether the Originator has CREATE privilege for a <m2mServiceSubscription> resource. The Receiver or Hosting CSE shall be IN-CSE.

Table 10.2.16.1-1: <m2mServiceSubscription> CREATE

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions | The access to the this Resource at Receiver has been granted |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: fr: Originator AE-ID or CSE-ID nm: the name of the <m2mServiceSubscription> resource is optional cn: the resource content shall provide the information as defined in clause 9.6.19. The following attributes from clause 9.6.19 are mandatory for the request: - “resourceType” which shall be set to the appropriate tag that identify the m2mServiceSubscription resource as defined in 9.6.1 - “subsSer&RoleList” The following attributes from clause 9.6.19 are optional for the request: - “expirationTime” - “appID” - “subsGroup” |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1) |
| Information on Response message | No change from the basic procedure (clause 10.1.1) |
| Post-Conditions | A new <m2mServiceSubscription> resource is created. |
| Exceptions | All exception described in the basic procedure (clause 10.1.1) are applicable. |

#### 10.2.16.2	Retrieve <m2mServiceSubscription>

This flow is used for retrieving the attributes of a <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver: The Receiver or Hosting CSE shall validate whether the Originator has RETRIEVE privilege for a <m2mServiceSubscription> resource. The Receiver or Hosting CSE is IN-CSE.

Table 10.2.16.2-1: <m2mServiceSubscription> RETRIEVE

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions | The Targeted <m2mServiceSubscription> resource exists |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: void |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.2) |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: attributes of the <m2mServiceSubscription> resource as defined in clause 9.6.19. |
| Post-Conditions | No changes on the resource as consequence of the RETRIEVE operation |
| Exceptions | All exception described in the basic procedure (clause 10.1.2) are applicable. |

#### 10.2.16.3	Update <m2mServiceSubscription>

This flow is used for updating the attributes of a <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver: The Receiver or Hosting CSE shall validate whether the Originator has UPDATE privilege for a <m2mServiceSubscription> resource. The Receiver or Hosting CSE is IN-CSE.

Table 10.2.16.3-1: <m2mServiceSubscription> UPDATE

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions | The Targeted <m2mServiceSubscription> resource exists |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: attributes of the <m2mServiceSubscription> resource as defined in 9.6.19 which need be updated, with the exception of the following that cannot be modified: - “lastModifiedTime”, - ” subsSer&RoleList”. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.3) |
| Information on Response message | No change from the basic procedure (clause 10.1.3) |
| Post-Conditions | The <m2mServiceSubscription> resource should contain the new information |
| Exceptions | All exception described in the basic procedure (clause 10.1.3) are applicable. |

#### 10.2.16.4	Delete <m2mServiceSubscription>

This flow is used for deleting a <m2mServiceSubscription> resource residing under a <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: The Receiver or Hosting CSE shall validate whether the Originator has DELETE privilege for a <m2mServiceSubscription> resource. The Receiver or Hosting CSE is IN-CSE.

Table 10.2.16.4-1: <m2mServiceSubscription> DELETE

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions | The Targeted <m2mServiceSubscription> resource exists |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.4). |
| Information on Response message | No change from the basic procedure (clause 10.1.4) |
| Post-Conditions | The targeted resource and all the children resources shall not exist anymore. |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable |

### 10.2.17	<nodeInfo> Resource

#### 10.2.17.1	Create <nodeInfo>

This flow is used for creating a <nodeInfo> resource which is sub-resource of <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: The Receiver or Hosting CSE shall validate whether the Originator has CREATE privilege for a <nodeInfo> resource. The Receiver or Hosting CSE is IN-CSE.

Table 10.2.17.1-1: <nodeInfo> CREATE

| CREATE: Description |  |
| --- | --- |
| Pre-Conditions | The access to the this Resource at Receiver has been granted |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: fr: Originator AE-ID or CSE-ID nm: the name of the <nodeInfo> resource is optional cn: the resource content shall provide the information as defined in clause 9.6.20. The following attributes from clause 9.6.20 are mandatory for the request: - “resourceType” which shall be set to the appropriate tag that identify the nodeInfo resource as defined in 9.6.1 - “nodeID” The following attributes from clause 9.6.16 are optional for the request: - “appID” - “CSEID” |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1) |
| Information on Response message | No change from the basic procedure (clause 10.1.1) |
| Post-Conditions | A new <nodeInfo> resource is created. |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable |

#### 10.2.17.2	Retrieve <nodeInfo>

This flow is used for retrieving the attributes of a <nodeInfo> resource which is sub-resource of <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: The Receiver or Hosting CSE shall validate whether the Originator has RETRIEVE privilege for a <nodeInfo> resource. The Receiver or Hosting CSE is an IN-CSE.

Table 10.2.17.2-1: <nodeInfo> RETRIEVE

| RETRIEVE: Description |  |
| --- | --- |
| Pre-Conditions | The Targeted <nodeInfo> resource exists |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: void |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.2) |
| Information on Response message | No change from the basic procedure (clause 10.1.2) |
| Post-Conditions | No changes on the resource as consequence of the RETRIEVE operation |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable |

#### 10.2.17.3	Update <nodeInfo>

This flow is used for updating the attributes of a <nodeInfo> resource which is sub-resource of <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: The Receiver or Hosting CSE shall validate whether the Originator has UPDATE privilege for a <nodeInfo> resource. The Receiver or Hosting CSE is IN-CSE.

Table 10.2.17.3-1: <nodeInfo> UPDATE

| UPDATE: Description |  |
| --- | --- |
| Pre-Conditions | The Targeted <nodeInfo> resource exists |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table with the specific details for: cn: attributes of the <nodeInfo> resource as defined in 9.6.16 which need be updated, with the exception of the following that cannot be modified: “lastModifiedTime” |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.3) |
| Information on Response message | No change from the basic procedure (clause 10.1.3) |
| Post-Conditions | The <nodeInfo> resource should contain the new information |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable |

#### 10.2.17.4	Delete <nodeInfo>

This flow is used for deleting a <nodeInfo> resource residing under a <m2mServiceSubscription> resource.

Originator: The Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: The Receiver or Hosting CSE shall validate whether the Originator has the DELETE privilege for a <nodeInfo> resource. The Receiver or Hosting CSE is an IN-CSE.

Table 10.2.17.4-1: <nodeInfo> DELETE

| DELETE: Description |  |
| --- | --- |
| Pre-Conditions | The Targeted <nodeInfo> resource exists |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the table. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.4). |
| Information on Response message | No change from the basic procedure (clause 10.1.4) |
| Post-Conditions | The targeted resource and all the children resources shall not exist anymore. |
| Exceptions | All exception described in the basic procedure (clause 10.1.4) are applicable. |

### 10.2.18	Resource Announcement Procedure

#### 10.2.18.1 	Procedure for AE and CSE to initiate Creation of an Announced Resource

This clause describes the procedure for an AE or a CSE to initiate the creation of an announced resource.

Originator: The Originator of a Request for initiating resource announcement can be either an AE or a CSE. The Originator can initiate the creation of an announced resource during the creation of the original resource by providing the announceTo attribute in the CREATE Request. The Originator can initiate the creation of an announced resource by using the UPDATE Request to the announceTo attribute in the original resource as follows:

The Originator shall perform the following for  the creation of an announced resource:

- The Originator can provide either the exact URI(s) for the announced resource or the list of CSE-IDs of the remote CSEs where the original resource needs to be announced by including such information within the attributeTo of the UPDATE or CREATE Request. In case of the CSE-IDs, the Receiver (original resource hosting CSE) shall decide the exact location for the announced resource at the remote CSE identified by the CSE-ID.

Receiver: Once the Originator has been successfully authorized, the Receiver (which shall be the original resource hosting CSE) shall grant the Request after successful validation of the Request.

- For UPDATE Request, the Originator shall add the received exact URI or the CSE-ID to the announceTo attribute at the original resource, if such exact URI or CSE-ID is not already stored in the announceTo attribute.

- For CREATE Request, the Originator shall include the received exact URI or the CSE-ID in the announceTo attribute created at the original resource.

- If the Request provides an exact URI(s) that are not already stored in the announceTo attribute or for newly created announceTo attribute, the Receiver shall announce the resource to the location identified by the URI(s) as per procedures in clause10.2.18.4.

- If the Request provides a list of CSE-IDs of the remote CSE that are not already stored in the announceTo attribute of for newly created announceTo attribute, the Receiver shall decide the location at the remote CSE(s) identified by CSE-ID(s) for announcing the resource and shall perform the procedures in clause 10.2.18.4.

On successful completion of resource announcement as in clause 10.2.18.4, the Receiver shall performs the following:

- The Receiver shall provide the exact URI(s) of the announced resource to the Originator by updating the content of the announceTo attribute in the original resource and by providing it in the UPDATE or CREATE Response message depending on the Request.

Table 10.2.18.1-1: Resource Announcement: UPDATE or CREATE

| UPDATE or CREATE Description |  |
| --- | --- |
| Pre-Conditions | The Originator suggests the URI(s) or the CSE-ID(s) to which the resource will be announced. |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table. In addition in case of CREATE the procedure for the specific resource is described in clause 10.2. The specific information relevant for announcing is: cn: contains address where the resource needs to be announced (within announcedTo attribute) |
| Local processing on Hosting CSE | Steps described for the Receiver of the Request as described above |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for cn as described above for the Receiver of the Response |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedures (section 10.1.1) are applicable. |

#### 10.2.18.2 	Procedure at AE or CSE to Retrieve information from an Announced Resource

This clause describes the procedures for an AE or a CSE to retrieve information about an announced resource or its attributes.

Originator: The Originator of a Request for initiating retrieval of information about a resource can be either an AE or a CSE. The Originator initiates this procedure by using RETRIEVE Request. Clause 8.1.2 specifies the information to be included in the Request message. Specifically, the to parameter is set to the address (e.g., URI) of the resource to be retrieved. If a specific attribute is to be retrieved, the address of such attribute is included in the to parameter. The Originator can request retrieval of information from an announced resource at the Hosting CSE by using the Request message parameter rc. If rc is  set to "announced-resource" it means that the actual announced resource representation is requested. If rc is set to ”links”, only the content of the link attribute is requested. If rc is not present, by default the originator request the content of the original resource that is linked by the announced resource.

The Originator shall perform as follows for retrieving information from an announced resource.

- The Originator shall set the to parameter in the RETRIEVE Request to the address (e.g., URI) of the resource or attribute to be retrieved.

- The Originator can request retrieval of information from an announced resource at the Hosting CSE. Optionally the Originator can specify one of the values for the optional rc parameter.

Receiver: Once the Originator has been successfully authorized, the Receiver (Hosting CSE) shall grant the Request after successful validation of the Request.

- If rc request message parameter set to "announced-resource" is included in the Request message, the Receiver shall provide in the RETRIEVE Response the announced resource representation, as described in clause 9.6.25.

- If rc request message parameter is set to "links" is included in the Request message, the Receiver shall provide in the RETRIEVE Response the content of the link attribute at the announced resource.

- If no rc request message parameter is provided in the Request message, the Receiver shall provide in the RETRIEVE Response the resource representation for the original resource that is linked by the link attribute.

- For a successful match, information from the identified announced resource (at hosting CSE) shall be returned to Originator via RETRIEVE Response.

Table 10.2.18.2-1: Resource Retrieval at local CSE: RETRIEVE

| RETRIEVE Description |  |
| --- | --- |
| Pre-Conditions | For RETRIEVE operation, the Originator knows the exact URI of the resource or the attribute that needs to be retrieved. |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for: rc: either: - not present - set to "announced-resource" - set to “links” cn: void |
| Local processing on Hosting CSE | Steps as described by the basic procedure (clause 10.1.2) apply. In addition the steps described above for the Receiver of the Request apply. |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for: cn: as described above for the Receiver of the Response |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedure (section 10.1.2) are applicable. |

#### 10.2.18.3 	Procedure for AE and CSE to initiate Deletion of an Announced Resource

This clause describes the procedure for an AE or a CSE (not the original resource hosting CSE) to initiate the deletion of an announced resource.

Originator: The Originator of a Request for initiating resource de-announcement can be either an AE or a CSE The Originator can request to initiate the deletion of an announced resource by using UPDATE Request to the announceTo attribute at the original resource hosting CSE. Resource de-announcement (deletion) shall also be performed when the Originator deletes the original resource at the original resource hosting CSE by using DELETE Request.

The Originator shall perform as follows for the deletion of an announced resource.

- The Originator shall update the announceTo attribute at the original resource hosting CSE by providing new content of the announceTo which does not include the URI of the announced resource that needs to be de-announced (deleted) in the UPDATE operation.

- The Originator shall update the original resource in the hosting CSE by providing new representation of the resource that does not contain the announceTo attribute for all the previously announced resources by using an UPDATE operation

- For DELETE Request, the Originator shall include the URI of the original resource hosting CSE that needs to be deleted, in the DELETE Request.

Receiver: Once the Originator has been successfully authorized, the Receiver (which shall be the original resource hosting CSE) shall grant the Request after successful validation of the Request. The Receiver shall be the resource hosting CSE. On receiving the UPDATE or DELETE Request, the Receiver shall perform as follows:

- For UPDATE Request, the Receiver shall delete the announced resource whose URI is deleted in the announceTo attribute as per procedures in clause 10.2.18.5 and update the announceTo attribute so as to not include the URI for the deleted announced resource. The UPDATE operation results in the removal of some URI(s) that is currently stored in the announceTo attribute or the removal of the announceTo attribute itself

- For DELETE Request, the Receiver shall delete all announced resources in the announcedTo attribute as per procedures in clause 10.2.18.5.

Receiver after completion of the resource de-announcement procedure in clause 10.2.18.5:

On successful completion of resource de-announcement procedure in clause 10.2.18.5, the Receiver knows that the announced resource has been deleted.

- The Receiver shall provide confirmation of resource de-announcement to the Originator, and the content of the updated announceTo attribute shall be provided to the Originator to indicate the successfully deleted announced resource if the announceTo attribute is not deleted by the Originator.

Table 10.2.18.3-1: Resource De-Announcement: UPDATE and DELETE

| UPDATE and DELETE Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for. cn: contains - Either the modified announceTo attribute - Or a complete representation of the original resource without the announceTo attribute In case of DELETE, the generic procedure is described in clause 10.1.4 cn: void |
| Local processing on Hosting CSE | The basic procedure (clause 10.1.3 for UPDATE and 10.1.14 for DELETE) still applies. In addition the steps described above for the Receiver of the Request also apply |
| Information on Response message | The basic procedure (clause 10.1.3 for UPDATE and 10.1.4 for DELETE) still applies. In addition the steps described above for the Receiver of the Request also apply |
| Post-Conditions | None |
| Exceptions | All exceptions described in the basic procedure (section10.1.2) are applicable for UPDATE operation. All exceptions described in the basic procedure (section10.1.4) are applicable for DELETE operation |

#### 10.2.18.4	Procedure for original resource Hosting CSE to Create an Announced Resource

This clause explains the resource announcement procedure that shall be used by the original resource hosting CSE to announce the original resource to the remote CSE.

Originator: The Originator of this Request shall be the original resource hosting CSE. The Originator shall request to create the announced resource by using CREATE Request. The Request shall provide the information as follows:

- Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute at the original resource shall be provided in the CREATE Request. Such attributes shall have the same value as for the original resource.

- Attributes marked with NA for the original resource shall not be provided in the CREATE Request.

- The link attribute of the announced resource shall have the URI for the original resource

- The labels attribute of the announced resource shall have the same value with the original resource

- The accessControlPolicyIDs attribute shall always be provided in the CREATE Request even if it is not present in the original resource. In this case the original resource shall include accessControlPolicyIDs from its parent resource or from the local policy at the original resource, as needed.

accessControlPolicyIDs and labels attributes, if present at the original resource, shall be marked MA and shall be provided by the original resource hosting CSE in the CREATE Request. Such attributes shall have the same value at the original resource and at the announced resource(s).

Receiver: Once the Originator has been successfully authorized, the Receiver shall grant the Request after successful validation of the Request. The Receiver shall perform as follows:

- The created announced resource shall include the common attributes specified in clause 9.6.25.1. The created announced resource shall contain the additional attributes that are provided by the Originator; i.e., attributes marked with MA and the attributes marked with OA that are included in the announcedAttribute attribute.

- The created announced resource shall set the accessControlPolicyIDs attribute to the value received in the Request message, and shall set the labels attribute (if present) and the link attribute to the value received in the Request message.

- Respond to the Originator with the CREATE Response. In this Response, the URI of the successfully announced resource shall be provided.

The Originator after receiving the Response from the Receiver shall perform the following steps:

- If the announced resource has been successfully created, the announceTo attribute of the original resource shall be updated  to include the URI(s) for the successfully announced resource at the Receiver. The announcedAttribute attribute shall be updated as well to represent the successfully announced attributes as received in the Response

- For the attributes marked as MA and for the attributes marked as OA that are included in the announcedAttribute attribute, the Originator shall further take the responsibility to keep their values synchronized at the announced resource by using UPDATE operation (clause 10.1.3).

Table 10.2.18.4-1: Resource Hosting CSE to Announce Resource: CREATE

| CREATE Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for the following attributes from clause 9.6.25 are mandatory for the request: cn: contains MA attributes and OA attributes that are included in announcedAttribute attribute The following attributes from clause 9.6.25 are mandatory for the Request: - resourceType which shall be set to the appropriate tag that identify the <Annc> resource; - accessControlPolicyIDs which it contains the value provided by the Originator, as described above. - expirationTime provided by the originator equal to the one of the original resource - labels provided by the originator equal to the one of the original resource, if present All attributes from the original resource that are present in the announcedAttribute shall be created in the announced resource and the value of these attribute shall be equal to the one from the original resource. |
| Local processing on Hosting CSE | The basic procedure (clause 10.1.1) and the steps described above for the Receiver of the CREATE Request apply |
| Information on Response message | All parameters defined in Table 8.1.2.1-1 are applicable as indicated in the Table with the specific details for: cn: URI where the announced resource is created according to clause 10.1.1 |
| Post-Conditions | Steps described for the 'Originator on receiving the Response from the Receiver' as described above |
| Exceptions | All exception described in the basic procedure (clause 10.1.1) are applicable. |

#### 10.2.18.5	Procedure for original resource Hosting CSE to Delete an Announced Resource

This clause explains the procedure for deleting an announced resource (i.e., the resource de-announcement). This procedure shall be used by the original resource hosting CSE for deleting the announced resource that resides at the remote CSE.

Originator: The Originator of this Request shall be the original resource hosting CSE. The Originator shall request to delete an announced resource by using the DELETE Request in which the to parameter provides a URI that identifies the announced resource to be deleted.

Receiver: Once the Originator has been successfully authorized, the Receiver shall grant the Request after successful validation of the Request.

- Delete the announced resource identified by the to parameter in the Request.

- Respond to the Originator with the appropriate DELETE Response.

Originator after receiving the Response from the Receiver:

- If the announced resource is successfully deleted, the announceTo attribute in the original resource shall be updated to delete the URI for the deleted announced resource.

Table 10.2.18.5-1: Resource Hosting CSE to Announce Resource: DELETE

| DELETE Description |  |
| --- | --- |
| Pre-Conditions | None |
| Information on Request message | All parameters defined in Table 8.1.2.1-1 are applicable as indicate in the Table. op: D fr: Identifier of the CSE that initiates the Request to: the URI where announced resource needs to be deleted |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.4). Steps for the Receiver as described above |
| Information on Response message | No change from the basic procedure (clause 10.1.4) |
| Post-Conditions | Steps for the 'Originator on receiving the Response from the Receiver' as described above |
| Exceptions | All exception described in the basic procedure (section 10.1.4) are applicable. |

#### 10.2.18.6	Procedure for AE and CSE to initiate creation or deletion of Announced Attribute

In this clause, the procedure for the AE and CSE (not the original resource hosting CSE) to initiate the creation or deletion of the announced attribute is explained using the relevant common attributes in the clause 9.6.1.

Originator: The Originator of a Request for initiating the attribute announcement can be either AE or CSE (not the original resource hosting CSE). The Originator shall request to initiate the creation or deletion of the announced attribute by using the UPDATE Request to the announcedAttribute attribute in the original resource. The Originator shall perform as follows depending on whether it is a creation or a deletion of the announced attribute:

- To create an announced attribute: The Originator shall add the attribute name of the original resource into the announcedAttribute attribute in the original resource using the UPDATE Request. Note that only the attribute marked with OP can be announced to the remote announced resource.

- To delete an announced attribute: The Originator shall remove the attribute name from the announcedAttribute of the original resource by using the UPDATE Request.

Receiver: The Receiver shall be only the original resource hosting CSE. On receiving the Request, the Receiver shall perform as follows:

- Check whether the Originator is authorized to the Request, and further check whether the Request is valid. The Request, that adds or deletes an attribute name that are not marked with OP, is invalid

- If the Request provides a new attribute name that is not stored in the announcedAttribute attribute and is marked with OP as well, then the Receiver will announce the attribute to all announced resources listed in the announceTo attribute. The Receiver can use the CREATE Request to create the attribute in the announced resource. The initial value for the announced attribute shall use the same value with the original resource

- If the Request removes the attribute name from the announcedAttribute attribute, then the Receiver will delete the attribute from all announced resources listed in the announceTo attribute. The Receiver can use the DELETE Request to delete the attribute in the announced resource

Table 10.2.18.6-1: Creating or Deletion of Announced Attribute

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | Steps described for the Originator of the Request as described above |
| Local processing on Hosting CSE | Steps described for the Receiver of the Request as described above |
| Information on Response message | Steps described for the Receiver of the Request as described above |
| Post-Conditions | None |
| Exceptions | Local processing on the Receiver: - The Request is invalid as described above |

#### 10.2.18.7	Procedure for original resource Hosting CSE to Announce Attribute

In this clause, the attribute announcement procedure is explained using the relevant common attributes in the clause 9.6.1. This procedure shall be used for the original resource hosting CSE to create the announced attribute to the announced resource in the remote CSE (i.e., the attribute announcement).

Originator: The Originator of this Request shall only be the original resource hosting CSE. Only attributes marked with OP can be announced to the announced resource in the remote CSE. Originator shall request to create the announced attribute by using the CREATE Request as the same value with the original resource.

Receiver: On receiving the Request, the Receiver shall perform as follows:

- Check whether the Originator is authorized for the Request, and further check whether the Request is valid. If the Request is valid, then the Receiver shall create the announced attribute according to the Request.

- Respond to the Originator with the appropriate generic Response.

Originator after receiving the Response from the Receiver: On receiving the Response, the Originator shall perform as follows:

- If the announced attribute is successfully created, then the announcedAttribute attribute shall be updated accordingly to include the attribute name for the successfully announced attribute in the Receiver.

- For the newly announced attributes in the announcedAttribute attribute, the Originator shall further take the responsibility to synchronize attributes between the original resource and the announced resource

Table 10.2.18.7-1: Original Resource Hosting CSE to Announce Attribute: CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | Only the attribute marked with OP can be announced. |
| Information on Request message | Information described for the Originator of the CREATE Request as described above |
| Local processing on Hosting CSE | Steps described for the Receiver of the CREATE Request as described above |
| Information on Response message | Steps described for the Receiver of the CREATE Request as described above |
| Post-Conditions | Steps described for the 'Originator on receiving the Response from the Receiver' as described above |
| Exceptions | Local processing on the Receiver: - The Request is invalid as described above |

#### 10.2.18.8	Procedure for original resource Hosting CSE to delete Announced Attribute

In this clause, the procedure for deleting the announced attribute (i.e., the attribute de-announcement) is explained. This procedure shall be used for the original resource hosting CSE to delete the announced attribute that resides in the announced resource in the remote CSE.

Originator: The Originator of this Request shall only be the original resource hosting CSE. The Originator shall request deletion of the announced attribute by using the DELETE Request in which the to parameter provides a URI that identifies the announced attribute to be deleted

Receiver: On receiving the Request, the Receiver shall perform as follows:

- Check whether the Originator is authorized for the Request, and further check whether the Request is valid

- If the Originator is authorized for the Request and the Request is valid, then delete the announced attribute identified by the to parameter in the Request

- Respond to the Originator with the appropriate generic Response

Originator after receiving the Response from the Receiver: On receiving the Response, the Originator shall perform as follows:

- If the announced attribute is successfully deleted, then the announcedAttribute attribute in the original resource shall be updated accordingly to delete the attribute name that was de-announced.

Table 10.2.18.8-1: Original Resource Hosting CSE to Announce Attribute: DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | No change from the generic procedure |
| Local processing on Hosting CSE | Steps described for the Receiver as described above |
| Information on Response message | No change to the generic procedure |
| Post-Conditions | Steps described for the 'Originator on receiving the Response from the Receiver' as described above |
| Exceptions | Local processing on the Receiver: - The Request is invalid as described above |

# 11	Trust Enabling Architecture

The Trust Enabling Architecture serves the purpose of establishing security and trust between all parties involved in the M2M ecosystem. It comprises the following infrastructure functions which may be external to the CSEs:

- M2M Enrolment functions, which manage the enrolment of M2M Nodes and M2M applications for access to M2M Services provided by an M2M Service Provider.

- M2M Authentication functions, in charge of identification and authentication of CSEs and AEs.

- M2M Authorization functions, which handle authorization requests to access resources.

The above functionalities are assumed to be operated by trusted parties (generally M2M Service Providers but possibly trusted third parties). These functions are detailed in TS-0003 [TS0003].

## 11.1	Enrolling M2M Nodes and M2M Applications for oneM2M Services

Though M2M nodes in the field domain are assumed to communicate without human involvement, individuals or organizations remain responsible for setting the access control policies used to authorize their M2M nodes to access M2M services. In the following text, M2M Nodes refers to M2M field nodes.

In particular, individuals or organizations acquiring M2M nodes can subscribe to a contract with an M2M Service provider (M2M Service Subscription) under which they enrol their M2M nodes (e.g., using identifiers pre-provisioned on the nodes, such as Node-ID). This in turn may require an M2M Service provisioning step (including Security provisioning) that takes place on the target M2M nodes themselves, for which interoperable procedures are specified by oneM2M (see clause A.1). Following M2M service provisioning, the nodes can be identified and authenticated by an M2M Authentication Function for association with an M2M Service Subscription, whose properties reflect the contractual agreement established between their owner and the M2M Service Provider.

Similarly, it shall be possible for an M2M Service Provider to mandate that application accessing M2M services be provisioned with security credentials used to authorize specific operations to instantiated applications (see clause A.2). This step facilitates the deployment and management of applications that are instantiated in great numbers, as it enables all instances of an application to be managed through common security policies that are set once for all. It also enables keeping control over applications issued by untrusted sources.

The above steps may be delegated to an M2M trust enabler, when this role is not assumed by the M2M Service Provider.

## 11.2 	M2M Initial Provisioning Procedures

### 11.2.1	M2M Node Enrolment and Service Provisioning

M2M service provisioning is the process by which M2M nodes are loaded with the specific information needed to seamlessly access the M2M Services offered by an M2M Service Provider. This is an initial step performed only when an M2M node is enroled for using the M2M services of an M2M Service Provider. Though this process can be performed during device manufacturing, there is a need to enable this process to take place during field deployment in an interoperable way. M2M service provisioning assumes the existence of an M2M service subscription contracted with the target M2M Service Provider for the target M2M node. Remote provisioning scenarios require the M2M node to be mutually authenticated using pre-existing credentials (e.g., Node-ID and associated credential) with an M2M enrolment function, to securely exchange the provisioning information with the contracted M2M Service Provider. The M2M Service Provisioning takes place between an M2M node (without provisioned CSE) and an M2M Service Provider via an M2M enrolment function. As a result of provisioning, M2M Nodes are provided with necessary credentials and possibly other M2M service related parameters (e.g., CSE-ID, M2M-Sub-ID).

The first step of M2M service provisioning is the security provisioning procedure, by which M2M service provider specific credentials are shared between the M2M node in the field domain and an M2M authentication function in the infrastructure. Authenticated M2M nodes can then be associated with an M2M Service Subscription used to determine their specific authorizations.

The following security provisioning scenarios are supported by the oneM2M architecture:

- Pre-provisioning

Pre-provisioning includes all forms of out-of band provisioning, e.g., provisioning M2M nodes with M2M subscription information during the manufacturing stage.

- Remote provisioning:

Remote provisioning relies on pre-existing credentials in M2M Nodes (e.g., digital certificates or network access credentials) to provision subscription related parameters through a secure session with an M2M Enrolment Function. This form of provisioning enables M2M nodes already in the field (e.g., operational M2M Nodes) to be provisioned with M2M Service subscription.

When supported, remote provisioning procedure shall be implemented as described in the oneM2M security specification [TS0003].

Following M2M service provisioning, a CSE associated with the target M2M Service provider in ASN/MN securely stores credentials used for authentication in association with M2M Authentication Function, with an associated lifetime (e.g., corresponding to the duration of the contractual agreement embodied by the M2M service subscription).

### 11.2.1	M2M Application Enrolment

This procedure is an optional step that enables the M2M SP and/or M2M application provider to control which applications are allowed to use the M2M services. It assumes that M2M applications obtains or registers credentials to be used for controlling authorization. Each application will then be provisioned with a security credential (M2M Application key) which can be used to grant specific authorization to access an approved list of M2M services. Such authorization takes place between a CSE and an AE.

When supported, the application enrolment procedure shall be implemented as specified in the oneM2M security specification [TS0003].

## 11.3	M2M Operational Security Procedures

This clause introduces high level procedures that shall be performed before any other procedure on Mcc and Mca can take place.

Figure 11.3-1. High Level Procedures on Mcc or Mca

### 11.3.1	Identification of CSE and AE

Identification is the process of identifying CSEs and AEs with the associated M2M service subscription to an M2M Authentication Function.

The Identification procedure shall be implemented as specified in the oneM2M security specification [TS0003].

### 11.3.2	Authentication of CSE and AE

Prior to granting access to M2M services, the credentials resulting from the M2M node and M2M application enrolment procedures shall be used, together with the identities supplied in the identification step, to perform mutual authentication of the entities (AEs or CSEs) with an M2M Authentication Function. Upon mutual authentication, the corresponding entities receive authorization to access the M2M services defined in the M2M Service Subscription.

The Authentication procedure shall be implemented as specified in the oneM2M security specification [TS0003].

### 11.3.3	M2M Security Association Establishment

The M2M Security Association Establishment procedure is performed to generate a security credential (M2M Connection key) shared between communicating AEs/CSEs, when an AE/CSE on one node initiates communication with an AE/CSE on another node. This procedure is performed after successful identification and mutual authentication of the corresponding M2M entities and derives resulting keys that may be used to provide desired security services to the communicating entities, such as confidentiality and/or integrity of information exchange (these security services may be provided through establishment of a secure channel between the communicating entities or through object based security where only relevant information is encrypted prior to being shared). The lifetime of a security association shall be shorter than the lifetime of the credential used for authentication from which it is derived: It may be valid for the duration of a communication session, or be determined according to the validity period of the protected data. In case of a security association between two AEs, the lifetime of the security association can result from a contractual agreement between the subscribers of the communicating AEs.

The security association establishment procedure shall be implemented as specified in the oneM2M security specification [TS0003].

### 11.3.4	M2M Authorization Procedure

The M2M authorization procedure controls access to resources and services by CSEs and AEs. This procedure requires that the originator has been identified to an M2M Authentication Function and mutually authenticated and associated with an M2M Service Subscription. Authorization depends on:

- The privileges set by the M2M Service Subscription associated with the originator (e.g., service/role assigned to the originator),

- These privileges are set-up based on the access control policies associated with the accessed resource or service. They condition the allowed operations (e.g., CREATE) based on the originator’s privileges and other access control attributes (e.g., contextual attributes such as time or geographic location).

The authorization/access grant involves an Access Decision step to determine what the authenticated CSE or AE can actually access, by evaluating applicable access control policies based on the CSE or AE privileges. Access Decision is described in [TS0003].

The following set of access control policy attributes shall be available for an Access Decision.

- Access control attributes of Originator (e.g., Role, CSE_IDs, AE-IDs, …)

- Access control attributes of Environment/Context (e.g., time, day, IP address,…)

- Access control attributes of Operations (e.g.,  create, execute, …)

The M2M Service Provider/administrator and owner of resources are responsible to establish access control policies that determine by whom, in what context and what operations may be performed upon those resources. If the requesting entity satisfies the owner’s access control policy, then the access to the resource is granted.

The authorization procedure involves rerouting of access requests to an M2M authorization function and delivering access tokens valid for specific authorization.

The authorization procedure shall be implemented as specified in the oneM2M security specification [TS0003].

# 12	Information Recording

## 12.1	M2M Infrastructure Node (IN) Information Recording

Various informational elements have to be recorded by the M2M infrastructure nodes for a variety of reasons including but not limited to statistics, charging, maintenance, diagnostics, etc.

This clause describes a framework for recording the necessary information by infrastructure nodes.

### 12.1.1	Information Recording Triggers

Triggers have to be configured in the IN node by the M2M service provider to initiate information recording.

The M2M infrastructure nodes shall be able to initiate recording based on any of the following triggers:-

- A request received by the M2M IN over the Mcc reference point.

- A request received by the M2M IN over the Mca reference point

- A request initiated by the M2M IN over any reference point

- Timer- based triggers for non- request based information recording. This trigger is used only when the memory size of a container over a period of time is required.

More than one trigger can be simultaneously configured.

The recording triggers may also be configurable, for example, as follows:

- On a per CSE basis, or a group of CSEs for requests originating/arriving from/at the M2M IN.

- On a per AE basis or a group of AEs

- The default behavior is that no  CSEs/AEs are configured.

### 12.1.2	M2M Recorded Information Elements

#### 12.1.2.1	Unit of Recording

A unit of recording refers to a number of informational elements recorded by the IN and that can be used as a basis for additional post-processing for a specific purpose such as generating Charging Data Records (CDRs), statistics, etc. In that respect, each unit of recording can be thought of as an M2M information record. The actual informational elements that make up a recording unit shall be described later.

For request-based triggers, as defined in clause 12.1.1, the unit of recording shall include a request and its response.

A unit of recording shall be referred to as an M2M Event Record. This shall apply to all recording triggers as defined in 12.1.1.

#### 12.1.2.2	Information Elements within an M2M Event Record

The information elements within an M2M event record are defined in Table 12.1.2.2-1.

Every M2M event record shall be tagged to depict its content according to the following classification:

- Data related procedures: represent procedures associated with data storage or retrieval from the M2M IN (e.g., Container related procedures)

- Control related procedures: represent all procedures that are not associated with data storage/retrieval from the M2M IN with the exclusion of group and device management related procedures (e.g., subscription procedures, registration)

- Group related procedures: represent procedures that handle groups.  The group name may be derived from the target resource in these cases.

- Device Management Procedures

- Occupancy based trigger for recording the occupancy as described in Clause 12.1.1

Table 12.1.2.2-1: Information Elements within an M2M Event Record

| Information Element | For request based triggers Mandatory / optional | For timer based triggers Mandatory / optional | Description |
| --- | --- | --- | --- |
| M2M Subscription Identifier | M | M | The M2M subscription ID associated with the request. This is inserted by the IN (see 12.1.3) |
| Application Entity ID | CM (when applicable) | NA | The M2M Application Entity ID if applicable |
| External ID | CM (when Applicable) | NA | The external ID to communicate over Mcn where applicable |
| Receiver | M | NA | Receiver of an M2M request (can be any M2M Node) |
| Issuer | M | NA | Issuer of the M2M request (can be any M2M node) |
| Hosting CSE-ID | O | NA | The hosting CSE-ID for the request in case the receiver is not the host, where applicable |
| Target ID | M | NA | The target URL for the M2M request if available. Alternatively can be the target resource identifier |
| Protocol Type | O | NA | E.g., HTTP, CoAP |
| Request Operation | O | NA | Request Operation as defined in clause 8.1.2 |
| Request Headers size | O | NA | Number of bytes for the headers in the Request, or number bytes of control information |
| Request Body size | O | NA | Number of bytes of the body transported in the Request if applicable |
| Response Headers size | O | NA | Number of bytes for the headers in the Response or number bytes of control information |
| Response Body size | O | NA | Number of bytes of the body transported in the Response if applicable |
| Response Code | O | NA |  |
| Time Stamp | M | M | The time for the recording the M2M event record |
| M2M-Event-Record-Tag | M | M | A Tag for the M2M event record for classification purposes. This tag is inserted by the IN and is M2M SP specific |
| Control Memory Size | O | NA | Storage Memory (in bytes), where applicable, to store control related information associated with the M2M event record(excludes data storage associated with container related operations) |
| Data Memory Size | O | NA | Storage Memory in Kbytes, where applicable, to store data associated with container related operations |
| Access Network Identifier | O | O | Identifier of the access network associated with the M2M event record. |
| Additional Information | O |  | Vendor specific information |
| Occupancy | NA | M | Overall size (in Bytes) of the containers generated by a set of AEs identified by the M2M Subscription Identifier |
| Group Name | CM | NA | Shall be included by the IN in the following cases: - Fanning operation initiated by the M2M IN - A request received by the M2M IN as a result of a fanning operation initiated elsewhere. The group tag shall also be set in this case |
| maxNrOfMembers | O | NA | Maximum number of members of the group for Create and Update operation. |
| currentNrOfMembers | O | NA | Current number of members in a group. The request shall be logged and information elements shall be recorded from the request before processing it or sending it out. After obtaining corresponding response, currentNrOfMembers shall be updated with the values from the response. |
| Subgroup Name | CM | NA | Includes the subgroup member name of a group. It shall be included by the M2M IN in the case when - Fanning operation initiated by the M2M IN and one of the members of the group is a subgroup - A request received by the M2M IN as a result of a fanning operation initiated elsewhere. The group tag shall also be set in this case |

The choice for the mandatory elements is motivated by the need to include all M2M identifiers within an M2M event record so that it is possible to support multiple charging scenarios.

For all non-mandatory elements, the M2M IN shall be configurable by the M2M service provider to select any additional desired information to be recorded in addition to the mandatory elements.

### 12.1.3	Identities Associations in Support of Recorded Information

To enable the M2M IN to record the necessary information, as described above, the following associations shall be maintained by the M2M service provider:-

- The CSE-ID  (for all M2M nodes in the M2M framework)  and the allocated M2M subscription ID

- The  AE-ID   and the allocated M2M subscription ID

For established associations, as described above, the M2M IN shall derive the appropriate M2M subscription ID for insertion in the M2M record event.

## 12.2	Offline Charging

### 12.2.1	Architecture

Figure 12.2.1-1 depicts the charging architecture. Charging information, in the form of charging data records (CDRs), shall be derived from recorded information, and transferred to a Charging Server. As such, it is essential that all information required for charging shall be first selected for recording. There shall be a 1 to 1 mapping between a M2M Event Record and a CDR.

The Charging Function (CHF included within the SCA CSF) embedded within the M2M IN is responsible for interaction with the Charging Server using the Mch reference point.

Billing aspects are out of scope.

Figure 12.2.1-1: Offline Charging Architecture

[12.2.1.a]  Editors Note:  Update Figure 5.2.1-1 to include this new interface.

[12.2.1.b]   Editor's Note: Once this picture is moved to clause 5.5; need to remove the picture from here.

### 12.2.2	Filtering of Recorded Information for Offline Charging

Recorded information is the basis for offline charging. To fulfil the needs of different billing systems not all recorded information is required in all cases. Hence, the M2M Charging Function shall be configurable to only select the desired information from the recorded information for transfer to the Charging Server. This configuration shall support selecting the desired information based on the following capabilities:

- On a per CSE basis, or a group of CSEs, for requests originating/arriving from/at the IN. This applies to all M2M nodes within the M2M framework

- On a per AE basis or a group of AEs.

- The default behavior is that no CSEs/AEs are configured.

The charging function shall ensure that information selected for transfer to the charging server has also been selected for recording before a configuration is deemed acceptable for execution.

### 12.2.3	Examples of Charging Scenarios

Charging scenarios refer to scenarios for which an M2M entity can be billed if the scenario is deemed billable by the M2M service provider. Some charging scenarios may require single CDR. Other scenarios may require multiple CDRs, and suitable correlation information shall have to be identified to select the CDRs for the charging scenario in this case.

The following clause lists some potential charging scenarios as examples only. Each scenario shall require the appropriate configuration of the CHF, and for that matter the M2M recording function, to ensure that all pertinent data is available.

#### 12.2.3.1	Example Charging Scenario 1 – Data Storage Resource Consumption

In this scenario, the M2M entity that stores application data, using container procedures for that purpose, will be billed, for storage resources within the M2M IN, until such time as the resources are deleted. This scenario will require correlation between multiple CDRs to identify the entity that stored the data, the entity that deleted the same data, and the duration and amount of storage.

#### 12.2.3.2	Example Charging Scenario 2 – Data transfer

In this scenario, the M2M entity that retrieves/stores container data will be billed for the amount of transferred data.

#### 12.2.3.3	Example Charging Scenario 3 – Connectivity

This scenario is relevant for an M2M entity that contacts the M2M IN frequently to transfer small amounts of data for storage. In this scenario, the M2M entity will be charged for the connectivity as opposed to the stored amount of data. The same applies to an M2M entity that also contacts frequently the M2M IN to retrieve stored data.

### 12.2.4	 Correlation between charging information of M2M IN and charging in access network

[12.2.4.a]  Editor's Note:  <<FFS>>

### 12.2.5	  Definition of Charging Information

Charging information in the form of CDR is essentially a subset of the information elements within the M2M event records recorded by the M2M IN for transmission over the Mch reference point.

#### 12.2.5.1	Triggers for Charging Information

The charging function within the M2M IN shall initiate transmission of CDRs if configured for that purpose in accordance with clause 12.2.2

#### 12.2.5.2	Charging Messages over Mch Reference Point

The Mch shall be used in case the CDRs are to be transferred to an external Charging Server. It is assumed that the Mch is equivalent to the Rf reference point as defined in [i.18], and [i.19].

Hence, every CDR shall be transferred in a single message, namely Accounting-Request and that elicits a response, namely Accounting-Answer.

The following table describes the use of these messages for offline charging.

Table 12.2.5.2-1: Offline Charging Messages Reference Table

| Request-Name | Source | Destination | Abbreviation |
| --- | --- | --- | --- |
| Accounting-Request | M2M IN | Charging Server | ACR |
| Accounting-Answer | Charging Server | M2M IN | ACA |

#### 12.2.5.3	Structure of the Accounting Message Formats

##### 12.2.5.3.1	Accounting-Request Message

Table 12.2.5.3.1-1 illustrates the basic structure of an ACR message generated from the M2M IN for offline charging in accordance with [i.18], [i.19], [i.10] and [i.13].

Table 12.2.5.3.1-1: Accounting-Request (ACR) Message Contents

| Informational Element | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. The usage of this field is left to the M2M SP. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Destination-Realm | M | This field contains the realm of the operator domain. The realm will be addressed with the domain address of the corresponding public URI. |
| Accounting-Record-Type | M | This field defines the transfer type: This field shall always set to event based charging |
| Accounting-Record-Number | M | This field contains the sequence number of the transferred messages. |
| Acct-Application-Id | OC | Advertises support for accounting for M2M |
| Event-Timestamp | O | Defines the time when the event occurred |
| Proxy-Info | OC | Includes host information about a proxy that added information during routing of the message |
| Route-Record | Oc | This field contains an identifier inserted by a relaying or proxying charging node to identify the node it received the message from. |
| Service-Context-Id | M | This field identifies the M2M domain |
| Service-Information | M | This parameter is set to “M2M service-information” |
| M2M Subscription-Id | M | Identifies the M2M subscription ID |
| M2M Information | M | This parameter holds the M2M informational element specified in Table 11.1 with the exception of the M2M subscription ID. |
| Proprietaryinformation | O | This is for proprietary information |

OC	This is a parameter that, if provisioned by the service provider to be present, shall be included in the CDRs when the required conditions are met. In other words, an OC parameter that is configured to be present is a conditional parameter.

[12.2.5.3.1.b]   Editors Note: The inclusion of additional information is FFS

##### 12.2.5.3.2	Accounting-Answer Message

The following table illustrates the basic structure of an ACA message generated by the charging server as a response to an ACR message

Table 12.2.5.3.2-1: Accounting-Answer (ACA) Message Contents

| Information element | Category | Description |
| --- | --- | --- |
| Session-Id | M | Same as Table 12.2.5.3.1-1 |
| Origin-Host | M | Same as Table 12.2.5.3.1-1 |
| Origin-Realm | M | Same as Table 12.2.5.3.1-1 |
| Destination-Realm | M | Same as Table 12.2.5.3.1-1 |
| Accounting-Record-Type | M | Same as Table 12.2.5.3.1-1 |
| Accounting-Record-Number | M | Same as Table 12.2.5.3.1-1 |
| Acct-Application-Id | OC | Same as Table 12.2.5.3.1-1 |
| Error-Reporting-Host | OC | Included only if the host that inserted the error is different from the Origin-Host |
| Event-Timestamp | O | Same as Table 12.2.5.3.1-1 |
| Proxy-Info | OC | Same as Table 12.2.5.3.1-1 |
| Proprietary Information | O | Same as Table 12.3.5.3.1-1 |

OC	This is a parameter that, if provisioned by the operator to be present, shall be included in the CDRs when the required conditions are met. In other words, an OC parameter that is configured to be present is a conditional parameter.

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e., clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

# Annex A			(Informative): Mapping of Requirements with CSFs

The following table illustrates the mapping of the Requirements specified in TS-0002 [i.1] with the CSFs specified in this document.

- Table A-1: Mapping of Requirements to CSFs

| CSF Name | Supported Sub-Functions | Associated Requirements | Notes |
| --- | --- | --- | --- |
| Addressing and Identification (AID) | - Management of identifiers | - OSR-026 - OSR-023 - OSR-024 - OSR-025 | Overlap w/: DSC for OSR-023, OSR-024, and OSR-025 |
| Communication Management/Delivery Handling (CMDH) | - Providing communications with other CSE's, AE's, and NSE's - Communications management: best effort - Communications policy management - Underlying Network connectivity management - Communications management: data store and forward - Ability to trigger off-line device | - OSR-001 - OSR-002 - OSR-005 - OSR-006 - OSR-008 - OSR-009 - OSR-012 - OSR-013 - OSR-014 - OSR-015 - OSR-018 - OSR-019 - OSR-021 - OSR-027 - OSR-032 - OSR-035 - OSR-038 - OSR-039 - OSR-040 - OSR-048 - OSR-049 - OSR-050 - OSR-053 - OSR-062 - OSR-063 - OSR-064 - OSR-065 - OSR-066 - OSR-067 - OSR-068 - CRPR-001 - CRPR-002 - CRPR-003 - MGR-016 | Overlap w/: DMR for OSR-001, OSR-009, OSR-021, OSR-032 SSM for OSR-009 LOC for OSR-006 GMG for OSR-006 NSSE for OSR-006, OSR-027 SSM for OSR-009 |
| Data Management and Repository (DMR) | - Data storage and management - Semantic support - Data aggregation - Data analytics - Device data backup and recovery | - OSR-001 - OSR-007 - OSR-009 - OSR-016 - OSR-020 - OSR-021 - OSR-032 - OSR-034 - OSR-036 - OSR-058 - SMR-006 - SER-015 | Overlap w/: CMDH for OSR-001, OSR-009, OSR-021, OSR-032 SUB for OSR-016 GMG for OSR-020 . |
| Device Management (DMG) | - Configuration Management - Diagnostics and Monitoring - Firmware management - Software management - Device Area Network topology management | - OSR-017 - OSR-069 - OSR-070 - OSR-071 - OPR-001 - OPR-002 - OPR-003 - MGR-001 - MGR-003 - MGR-004 - MGR-006 - MGR-007 - MGR-008 - MGR-009 - MGR-011 - MGR-012 - MGR-013 - MGR-014 - MGR-015 - MGR-019 - MGR-020 - MGR-021 - SER-013 - SER-014 | Overlaps w/: GMG for OSR-017 SEC for SER-013 |
| Discovery (DIS) | - Discover resource - Local discovery (within CSE) - Directed remote discovery | - OSR-023 - OSR-024 - OSR-025 - OSR-059 - OSR-060 - OSR-061 - MGR-002 - SMR-004 | Overlaps w/: AID for OSR-023, OSR-024, OSR-025 |
| Group Management (GMG) | - Management of a group and its membership - CRUD - Use Underlying Network group capabilities - Bulk operations - Access control | - OSR-006 - OSR-017 - OSR-020 - OSR-029 - OSR-030 - OSR-031 - OSR-037 - OSR-047 - MGR-005 | Overlaps w/: CMDH for OSR-006 LOC for OSR-006 GMG for OSR-006 NSSE for OSR-006, OSR-037 DMR for OSR-020 DMG for OSR-017 |
| Location (LOC) | - Location management - Network-provided - GPS-provided - Confidentiality enforcement as it relates to location | - OSR-006 - OSR-051 - OSR-052 - SER-026 |  |
| Network Service Exposure /Service execution and triggering (NSSE) | - Access Underlying Network service - Location - Device triggering - Small data - Policy and charging - Support multiple Underlying Network functions | - OSR-006 - OSR-011 - OSR-027 - OSR-037 - OSR-054 - OSR-055 - OSR-056 - MGR-017 - MGR-018 - OPR-004 - OPR-005 - OPR-006 | Overlaps w/: CMDH for OSR-027 GMG for OSR-006, OSR-037 LOC for OSR-006 |
| Registration (REG) | - CSE registration - Application registration - Device registration - ID correlation | - MGR-010 | Overlaps w/: SEC |
| Security (SEC) | - Sensitive Data Handling - Secure storage - Secure execution - Independent environments - Security administration - Pre-provisioning - Dynamic bootstrap - Network bootstrap - Security association - Link level - Object level - Authorization and access - Identity protection | - SER-001 - SER-002 - SER-003 - SER-004 - SER-005 - SER-006 - SER-007 - SER-008 - SER-009 - SER-010 - SER-011 - SER-012 - SER-013 - SER-016 - SER-017 - SER-018 - SER-019 - SER-020 - SER-021 - SER-022 - SER-023 - SER-024 - SER-025 - MGR-010 | Overlap w/: DMG for SER-013 REG for MGR-010 SSM for SER-007 |
| Service Charging and Accounting (SCA) | - Charging enablers - Sending charging information to charging server - Subscription-based charging - Event-based charging - Session-based charging - Service-based charging - Correlation with Underlying Network - Charging management - Offline charging - Online charging | - CHG-001, - CHG-002a, - CHG-002b, - CHG-003, - CHG-004, - CHG-005 |  |
| Service Session Management (SSM) | - Service Session Management (CSE to CSE, AE to CSE, and AE to AE) - Session persistence over link outage - Session context handling - Assignment of session ID - Session routing - Multi-hop session management - Session policy management | - OSR-003 - OSR-004 - OSR-009 - OSR-045 - SER-007 | Overlap w/: CMDH and DMR for OSR-009 SEC for SER-007 |
| Subscription/Notification Support (SUB) | - Subscribe (CSE, AE) - Local - Remote - Subscription to a group - Notification - Synchronous - Asynchronous | - OSR-010 - OSR-016 - OSR-033 | Overlaps w/: DMR for OSR-016 |

# Annex B			(Informative): oneM2M System and 3GPP MTC Release-11 Underlying Network Interworking

[B.a]  Editor's Note: Many terms and abbreviations are missing from clause 3.

## B.1	3GPP MTC Release-11 Underlying Network Introduction

In order to provide M2M services, interworking between oneM2M System and the 3GPP Underlying Network is required. This entails the following aspects:

- Connectivity establishment between the oneM2M System and the 3GPP Underlying Network from IN-CSE initiated scheme and device (i.e., ASN-CSE) initiated scheme.

- Mapping of oneM2M and 3GPP IDs to establish connectivity between specific entities.

This clause provides system level information on the above aspects specifically related to the interworking i.e., connectivity between the oneM2M System and the 3GPP Underlying MTC network.

## B.2	3GPP Release-11 MTC Functionality

Interworking with oneM2M Release-1 is based on 3GPP Release-11 specifications. The relevant 3GPP Release-11 specification references are as follows:

- TS 23.682 : MTC System Architecture.

- TS 23.401 and 23.402: LTE/SAE System Architecture.

- TS 23.060: General Packet Radio Service (GPRS).

- TS 23.228: IP Multimedia Subsystem.

In annex A of 3GPP TS 23.682 [i.17] the following MTC deployment scenarios are depicted.

- Figure B.2-1: MTC deployment scenarios for Direct and Indirect model (3GPP TS 23.682 [i.17])

The focus of this annex is on deployment scenario B (Indirect Model), where the M2M Services Capability Server (IN-CSE) is outside the operator domain. However, the indirect model C in figure B.2-1 is not ruled out.

Hybrid model which is a combination of above scenario B and C are also mentioned in TS 23.682 which may also be supported.

Taking 3GPP Release-11 MTC network as an Underlying Network, oneM2M IN-CSE is considered as equivalent or part of the Services Capability Server (SCS) function, and the oneM2M ASN is considered equivalent to a UE, as captured in 3GPP MTC architecture (3GPP TS 23.682 [i.17]) shown below.

[B.2.c]  Editor's Note: Many acronyms and abbreviations in the diagram need to be added to clause 3.

- Figure B.2-2: 3GPP Architecture for Machine-Type Communication (3GPP TS 23.682 [i.17])

In the above diagram, the SCS is shown located in the HPLMN and within the operator domain. However, it is assumed that the SCS is equivalent to the oneM2M Common Services Entity (CSE) or the two are integrated, and is outside the operator domain. When the CSE is outside the operator domain, it has to interact with the 3GPP Underlying Network via MTC-IWF and/or GGSN/P-GW. This requires mapping of oneM2M reference points (Mcn, Mcc) and 3GPP reference points (Tsp, Gi/SGi) respectively, along with the usage of the identifiers in the two systems.

[B.2.d]  Editor's Note: The mapping of identifiers between the oneM2M System and the 3GPP MTC network are FFS.

## B.3	ASN-CSE initiated connectivity establishment

It is assumed that there is no connectivity previously established, i.e., no association between the ASN-CSE (device node) and the serving IN-CSE exists. When the ASN-CSE has to send data to the serving IN-CSE it has to first discover the serving IN-CSE, which is located in a packet data network, and establish connection. Two methods can be used, as follows:

### B.3.1	Use of DHCP and DNS

The ASN-CSE requests the DNS server address from the DHCP server followed by requesting the serving IN-CSE IP address from the DNS server.

[B.3.1.a]  Editor's Note: How a non-CSE resident device node (i.e., Application Dedicated Node) interworks with this system is FFS.

### B.3.2	Pre-configuration

The ASN-CSE is preconfigured with the fully qualified domain name (FQDN) of the serving IN-CSE or the IP address of the serving IN-CSE. If the FQDN is known, DNS resolution is used to obtain the IP address.

[B.3.2.a]  Editor's Note: There might be other ASN-CSE initiated IN-CSE discovery schemes and this clause does not intend to exclude them. Further description of these schemes is FFS.

## B.4	Serving IN-CSE initiated connectivity establishment

It is assumed that there is no connectivity previously established between the ASN-CSE and the serving IN-CSE. When the serving IN-CSE has to contact the ASN-CSE to send data or request data, connectivity between them has to be established. This connectivity has to be triggered by the IN-CSE.

[B.4.a]  Editor's Note: How a non-CSE resident ADN interworks with this system is FFS.

## B.5	Idle mode Management

[B.5.a]  Editor's Note: This clause should describe oneM2M System dormant device management capability in conjunction with 3GPP related Idle Mode management.

## B.6	Procedure

ASN-CSE communicates with the serving IN-CSE after completion of the Underlying Network bearer establishment and discovery of the serving IN-CSE. Data can then traverse between CSEs over the IP connection in the Underling Network over 3GPP Gi/SGi interface. In addition, the signalling connectivity between the two CSEs is also realized. The following figure depicts the connectivity between the ASN-CSE and the IN-CSE.

- Figure B.6-1: Initial Connectivity Establishment between Device and Infrastructure CSE

### B.6.1	General

When data has to be exchanged between the ASN-CSE and the IN-CSE, connectivity between them has to be established. The need for this connectivity can arise for two reasons:

- ASN-CSE initiated: When the ASN-CSE has to send/receive data to/from the IN-CSE; or

- IN-CSE initiated: When the IN-CSE has to send/receive data to/from the ASN-CSE.

[B.6.1.a]  Editor's Note: The procedure indicated below is one of the realization examples. This does not exclude any other realizations.

#### B.6.1.1	ASN/MN-CSE Initiated Connectivity Establishment Procedure

- Figure B.6.1.1-1: ASN/MN-CSE initiated connectivity establishment

Step-0: Trigger

Subsequent procedures are triggered by either UE power on or Device Triggering mentioned in B.6.1.2.

Step-1: Bearer Setup Procedure

Establish an Underlying Network bearer if not already available by using the procedures available in the Underlying Network.

Step-2: DHCP Query & Response

The ASN/MN-CSE sends a query to a DHCP server to find a particular DNS server IP address. Then the DHCP server responds with an IP address of a corresponding DNS server. Additionally, it is also possible to include one or a list of domain names, i.e., FQDN of target IN-CSEs.

Step-3: DNS Query & Response

The ASN/MN-CSE performs a DNS query to retrieve the IN-CSE(s) IP addresses from which one is selected. If the response does not contain the IP addresses, an additional DNS query is needed to resolve a Fully Qualified Domain Name (FQDN) of the serving IN-CSE to an IP address.

Step-4: Connection Establishment

After reception of domain name and IP address of an IN-CSE, the ASN/MN-CSE can initiate communication towards the IN-CSE via IP connection from this procedure. The IN-CSE at this time will be informed which Trigger Recipient ID of the ASN/MN-CSE will be used for communication.

Step-5: CSE-PoA Update

Once the M2M Service Connection (Mcc) is established, in the IN-CSE the CSE-PoA of the ASN/MN-CSE shall be updated with the new established IP address.

The IN-CSE which holds the state information, it needs to be informed when the connection is closed.

[B.6.1.1.a]  Editor's Note: It is FFS how oneM2M and 3GPP IDs map to each other for a particular device node to be able to deliver information to a particular serving IN-CSE.

#### B.6.1.2	IN-CSE initiated connectivity establishment procedure over Tsp

Connection Establishment between IN-CSE and ASN/MN-CSE

Whenever the IN-CSE requires to establish a connection towards another entity (i.e., ASN-CSE), the Device Triggering procedure over the Tsp interface as described in [TS 23.682] shall be used.

- Figure B.6.1.2-1: IN-CSE initiated connectivity establishment

Pre-condition

The CSE which is the target of the device triggering has to be registered with the IN-CSE. The IN-CSE checks the state information of the target device, which some of this information is the result of previous connection establishment or triggering requests, such as the case of power-off, dormant and/or connected state. The IN-CSE decides its next action, i.e., if it needs to start Triggering or report to IN-AE about the unavailability to perform the request.

The CSE-PoA for the ASN/MN-CSE either already contain an IP address which is not valid anymore or no address at all, or FQDN does not resolve to a valid IP address. This is a pre-requisite for performing the device triggering procedure.

[B.6.1.2.a]  Editor’s Note:  How the M2M-Ext-ID is obtained is FFS. The ID should at least be available somewhere in the ASN/MN-CSE resource or the IN-CSE should keep a map between M2M-Ext-ID and CSE-ID, so that the IN-CSE can access the information when needed.

[optional] Step 1: Request targeted to ASN/MN-CSE

The IN-AE requests to perform one of the CRUD operation on a resource residing on the ASN/MN-CSE, the request is sent via the Mca reference point to the IN-CSE.

[B.6.1.2.b]   Editor’s Note: In case of any request as described in clause 8.1 for resource manipulation over Mca reference point, the request indicates by the address of the target resource URI. In order to reach the specified resource a Device Triggering may be required.

Step 2: DNS Query / Response

The IN-CSE determines the need to trigger the ASN-CSE.

If the IN-CSE has no contact details for a contact MTC-IWF, it may determine the IP address(es)/port(s) of the MTC-IWF by performing a DNS query using the M2M-Ext-ID (M2M External Identifier) or using a locally configured MTC-IWF identifier.

[B.6.1.2.c]   Editor's Note: Mechanisms other than DNS query could be used for determining the IP address(es)/port(s) of the MTC-IWF. These are FFS.

Step-3: Device Triggering Request

The IN-CSE buffers the original request information and sends the Trigger Request message that contains information as specified in [3GPP TS23.682]:

- M2M-Ext-ID or MSISDN,

- SCS-Identifier, (is set to the IN-CSE ID)

- Trigger reference number, (used to correlate the request with the response)

- Validity period, (which indicated how long the request is valid)

- Priority, (this field only allows to set the priority on or off)

- Application Port ID, (is set to the ASN-CSE Trigger-Recipient-ID since it is the triggering application addressed in the device from 3GPP point of view)

- Trigger payload, (optional information can be set to the payload)

NOTE:     In case that the Device Triggering request is for M2M Service Connection setup request as the present flow, it is assumed that the target CSE (i.e., ASN-CSE) when it is woken up, received the trigger it could connect automatically to the IN-CSE where it is registered. The information of the IN-CSE may be pre-stored in the target CSE (i.e., ASN-CSE). Therefore it is assumed that the trigger payload does not include optional information and the target CSE is registered to only one IN-CSE. How to use the optional part of the trigger payload is described as below.

Acknowledge

Once, 3GPP-MTC-IWF receives the Trigger Request, it will ask the HSS to determine if the IN-CSE is authorized to perform the triggering to the target CSE (i.e., ASN-CSE) and the HSS will resolve the M2M-Ext-ID to IMSI (or MSISDN). Then the 3GPP-MTC-IWF acknowledges to the IN-CSE with the confirmation of the received Device Triggering Request.

Step-4: Device Triggering delivery procedure

The MTC-IWF will initiate the T4 trigger delivery procedure according to the 3GPP TS 23.682 [i.17], based on the information received from HSS and local policy.

NOTE:     3GPP Network Entities (e.g., SMS-SC) can select appropriate device triggering mechanism (e.g., SMS based or SIP based via IP-SM-GW) according to the device capabilities.

Step 5: ASN-CSE receives the trigger

As a result of the device triggering procedure the addressed ASN/MN-CSE is initiated, in this case of the flow the ASN-CSE will be started according to the received Application Port ID by the UE.

NOTE:     In case that the UE received optional part of the trigger payload, it is assumed that the information would be transmitted to the started application inside the UE.

Step 6: Device Triggering report

Request:

The MTC-IWF sends the Device Trigger Report message (containing the M2M-Ext-ID or MSISDN and trigger reference number) to the IN-CSE with a cause value indicating whether the trigger delivery succeeded or failed and the reason for the failure.

Acknowledge:

IN-CSE acknowledges to the MTC-IWF with the conformation of the received Device Triggering Report

[optional] Step 7: Connection establishment procedure

The ASN-CSE will perform the Connection establishment procedure as described in B.6.1.1 and oneM2M TS-0003 [i.3] for Secure Connection establishment.

As a result of this procedure now the initial request over the reference point Mcc can be executed.

[optional] Step 8: CSE-PoA/Reachability state updated

Once the connection over Mcc is established, in the IN-CSE the PoA of the ASN-CSE shall be updated with the new established IP address and the IN-CSE holds the reachability state of the ASN/MN-CSE.

[optional] Step 9: Re-sending of original request

As a result of step 7, the communication is established and now the initial request with the information stored in the buffer of the IN-CSE at Step 3 can be re-issued over the reference point Mcc.

In the flow presented above not all parameters allowed in the Device Triggering Request message from 3GPP Tsp interface are used. Optionally the following cases are allowed:

By providing a payload which may contain:

Either actual content information (as permitted by the limitation of the payload parameter). For example;

- It could contain a resource (or attribute) identifier (as expressed inside the ASN-CSE) and an actual content for the resource (or attribute) of any of the resources stored in the ASN-CSE.

- Or any other instructions, like to initiate a specific procedure. For example, execute a command.

- Or it could contain of the URI of an entity outside the oneM2M domain where the target ASN-CSE should connect to. If a URI is provided, in this case the steps 7, 8 and 9 of the previous flow cannot be performed since the connection establishment is not performed between two oneM2M entities, therefore how the actual setup is performed it is outside the scope of this specification.

[B.6.1.2.d]  Editor’s Note: What information could be transported in the payload is FFS.

# Annex C			(Informative): Interworking between oneM2M System and 3GPP2 Underlying Networks

## C.1	General Concepts

Interworking between oneM2M System and 3GPP2 Underlying Networks is based on 3GPP2 X.P0068 specification [i-13].

In order to provide M2M services, interworking between oneM2M System and the 3GPP2 Underlying Network is required. M2M Applications (AEs) in the M2M UEs (M2M Nodes such as the ASNs and MNs) and the M2M Applications in the external network (Infrastructure Domain) use services provided by the 3GPP2 Underlying Network, and optionally the services provided by an M2M Server (IN-CSE). The 3GPP2 Underlying Network provides transport and communication services, including 3GPP2 bearer services, IMS and SMS.

- 3GPP2 Underlying Network supports several interworking models, such as the following:

- Direct Model – Direct Communication provided by the 3GPP2 Network Operator: 
The M2M Applications in the external network connect directly to the M2M Applications in the UEs used for M2M via the 3GPP2 Underlying Network without the use of any M2M Server;

- Indirect Model – M2M Service Provider controlled communication: 
Uses an M2M Server that is an entity outside the 3GPP2 Underlying Network operator domain for enabling communications between the Applications in the external network and at the UEs used for M2M. Tsp interface is an external interface that the third party M2M Server supports with the entities that are within the 3GPP2 Underlying Network domain;

- Indirect Model – 3GPP2 Operator controlled communication: 
Uses an M2M Server that is an entity inside the 3GPP2 Underlying Network operator domain for enabling communications between the Applications in the external network and at the UEs used for M2M. Tsp interface is an internal interface that the 3GPP2 Underlying Network operator controlled M2M Server supports with other entities within the 3GPP2 Underlying Network domain;

- Hybrid Model: 
Direct and Indirect models are used simultaneously in the hybrid model i.e., performing Control Plane signaling using the Indirect Model and connecting the M2M Applications in the external network and at the UEs used for M2M over User Plane using the Direct Model.

## C.2	M2M Communication Models

In the indirect and hybrid models, the deployment of an M2M Server (IN-CSE) may be inside or outside the 3GPP2 Underlying Network operator domain as illustrated in Figures C.2-1 and C.2-2. When the M2M Server is part of the 3GPP2 Underlying Network operator domain (Figure C.2-1(C) and Figure C.2-2), the M2M Server is considered a 3GPP2 Underlying Network operator internal network function, is operator controlled, and may provide operator value-added services. In this case, security and privacy protection for communication between the M2M-IWF and the M2M Server (IN-CSE) is optional. When the M2M Server is deployed outside the 3GPP2 Underlying Network operator domain (Figure C.2-1(B) and C.2-2), the M2M Server is M2M Service Provider controlled. In this case, security and privacy protection for communication between the M2M-IWF and the M2M Server (IN-CSE) is needed. In the direct model (Figure C.2-1(A)), there is no external or internal M2M Server in the communication path.

Figure C.2-1: M2M Communication Models

Figure C.2-2: Multiple M2M Applications Using Diverse Communication Models

A 3GPP2 network operator may deploy the hybrid model with a combination of no internal and external M2M Server (as in the Direct Model) and internal and/or external M2M Server (as in the Indirect Model). As shown in Figure C.2-2, a UE (an M2M Node such as ASN/MN) may be in communication with multiple M2M Servers which can be made up of a combination of 3GPP2 Underlying Network operator controlled and M2M Service Provider controlled M2M Servers. In that scenario, the M2M Service Provider controlled M2M Server, and the 3GPP2 Underlying Network operator controlled M2M Server may offer different capabilities to the M2M Applications.

Though not illustrated, it is also possible that in the Indirect Service Model with 3GPP2 network operator controlled M2M Server; the M2M Application may be inside the 3GPP2 network operator domain and under 3GPP network operator control.

### C.3	3GPP2 Architectural Reference Model for M2M

Figure C.3-1 shows the architecture for a UE used for M2M connecting to the 3GPP2 Underlying Network. The architecture supports various architectural models described in clause C.2.

Figure C.3-1: Enhanced 3GPP2 Network Architecture for Supporting M2M

The M2M Server (IN-CSE) is the entity which connects to the 3GPP2 Underlying Network for providing communication with the UEs used for M2M. The M2M Server offers capabilities for use by one or multiple M2M Applications (AEs) hosted by the UE (ASN/MN). The corresponding M2M Applications in the external network (Infrastructure Domain) are hosted by one or multiple M2M Application platform(s).

The M2M Server interfaces with the 3GPP2 Underlying Network entities located in the home domain of the UE used for M2M via the Tsp and IP interfaces. M2M Server encompasses the IN-CSE entity specified by oneM2M. M2M Server interfaces with the M2M-IWF via Tsp Interface for Control Plane communications. User plane interactions between the M2M Server and 3GPP2 Underlying Network entities such as the PDSN and/or HA/LMA is via native-IP. With this configuration, oneM2M reference points Mcn and Mcc map to 3GPP2 reference points Tsp and IP respectively.

## C.4	Communication between oneM2M Service Layer and 3GPP2 Underlying Network

Communication between the M2M Server (IN-CSE) and the entities in the 3GPP2 Underlying Network make use of the User Plane and the Control Plane communication paths, as needed for different 3GPP2 M2M communication models. User Plane communication path uses IP transport between the M2M Server (IN-CSE) and the CSE in the UE used for M2M (ASN/MN-CSE). The User Plane maps to oneM2M Mcc reference point. Control Plane communication path is over Tsp interface and maps to oneM2M Mcn reference point.

Figure C.4-1: User Plane and Control Plane Communication Paths

## C.5		Information Flows

3GPP2 X.S0068 specifies several system optimizations that can be used for M2M applications. Such optimizations include the following:

- Interaction of M2M Server with M2M-IWF for device triggering

- Device trigger using SMS

- Device trigger using broadcast SMS

- Device trigger using IP transport

### C.5.1	Discovery of M2M Devices

[C.5.1.a]  Editor's Note:  <tbd>

### C.5.2	Tsp Interface Call Flow

The following is the high level call flow illustrating device triggering using Tsp interface.

Figure C.5-1: Tsp Interface Call Flow

- M2M Server (IN-CSE) receives a request from an M2M Application Server (AE in Infrastructure Domain) to deliver data to a UE used for M2M (ASN/MN-CSE) located in the 3GPP2 Underlying Network. Knowing the CSE-ID of the destination M2M Node, IN-CSE deduces its 3GPP2 External Identifier.

- M2M Server (IN-CSE) may perform DNS query to obtain the IP address of the M2M-IWF for reaching the destination M2M Node.

- M2M Server sends Device Trigger Request message to the M2M-IWF that includes destination M2M Node External ID and other information.

- M2M-IWF checks that the M2M Server is authorized to send trigger requests and performs other tasks such as verifying that the M2M Server has not exceeded its quota or rate of trigger submission over Tsp. If such checks fail, the MTC-IWF sends a Device Trigger Confirm message with a cause value indicating the reason for the failure condition and the call flow stops at this step.

- Otherwise, the MTC-IWF continues to interact with HAAA/HLR for obtaining 3GPP2 Internal ID for the M2M Node and other information for reaching the M2M Node in the 3GPP2 Underlying Network. M2M-IWF also determines the device trigger mechanisms (e.g., Mechanism 1, Mechanism 2 etc.) supported by the M2M Node. The flow continues with Step 5.

- M2M-IWF decides to deliver device trigger using e.g., Mechanism 1 and performs appropriate 3GPP2 Underlying Network specific procedures.

- M2M-IWF may try alternative device trigger delivery mechanism (e.g., Mechanism 2) if Mechanism 1 fails. Or both Mechanism 1 and 2 can be performed in parallel.

- M2M-IWF performs appropriate 3GPP2 Underlying Network specific procedures for delivering device trigger using Mechanism 2.

- M2M-IWF sends Device Trigger Report to the M2M Server upon receiving the acknowledgment from the M2M Node that it has received M2M device trigger.

- The M2M Node and the M2M Server/AS take actions in response to the device trigger as needed.

### C.5.2	Point to Point Device Triggering

3GPP2 Underlying Network supports the following point-to-point device triggering mechanisms:

- SMS on common channel

- SMS on 1xCS traffic channel

- Device trigger using IP interface

Device trigger using IP interface assumes that PPP sessions has been established and maintained between the M2M Node and the PDSN. An IP address has been assigned to the M2M Node by the IP anchor (PDSN/HA/LMA) and is maintained by the M2M Node and by other entities (e.g., HAAA) in the 3GPP2 Underlying Network. Upon receiving device trigger from the M2M Server, the M2M-IWF obtains the IP address assigned to the M2M Node from the M2M-AAA/HAAA. After that, the M2M-IWF sends device trigger to the M2M Node through IP routing via IP interface to the HA/LMA for MIP and PMIP operation, or to the PDSN for Simple IP operation.

### C.5.3	Broadcast Device Triggering

3GPP2 Underlying Network supports the following broadcast device triggering mechanisms:

- SMS broadcast

# Annex D			(Normative) <mgmtObj> Resource Instances Description

## D.1	oneM2M Management Functions

This clause describes the management functions supported by oneM2M. These functions are fulfilled by defining instances of <mgmtObj> resources. The instances are resources of <mgmtObj> type with specific designing to support different management capabilities through operations defined by oneM2M. The instances are service layer information models for the purpose of management. The instances can then be used within the M2M service layer or they can be further mapped to existing management technologies such as OMA DM, OMA LWM2M and BBF TR-069 to enable the management of devices with OMA or BBF compliant management clients.

NOTE: The resource instances defined in this clause are all of type <mgmtObj>. The instances are distinguished by attribute mgmtDefinition. The names of the instances are not fixed.

[D.1.a]  Editor’s Note: The notation for addressing the resources in this clause still need to be further studied.

## D.2	Resource firmware

Figure D.2-1: Structure of <firmware> resource

[D.2-a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <firmware> resource shall contain the child resource in Table D.2-1.

Table D.2-1: Child resources of <firmware> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <firmware> resource shall contain the attributes described in Table D.2-2.

Table D.2-2: Attributes of <firmware> resource

| Attribute Name of <firmware> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “Firmware” to indicate the resource is for firmware management. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| version | 1 | RW | The version of the firmware. |
| name | 1 | RW | The name of the firmware to be used on the device. |
| URL | 1 | RW | The URL from which the firmware image can be downloaded. |
| update | 1 | RW | The action that downloads and installs a new firmware in a single operation. |
| updateStatus | 1 | RO | Indicates the status of the update. |

## D.3	Resource software

Figure D.3-1: Structure of <software> resource

[D.3.a] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <software> resource shall contain the child resource in Table D.3-1.

Table D.3-1: Child resources of <software> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <software> resource shall contain the attributes described in Table D.3-2.

Table D.3-2: Attributes of <software> resource

| Attribute Name of <software> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “Software” to indicate the resource is for software management. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| version | 1 | RW | The version of the software. |
| name | 1 | RW | The name of the software to be used on the device. |
| URL | 1 | RW | The URL from which the software package can be downloaded. |
| install | 1 | RW | The action that downloads and installs new software in a single operation. |
| uninstall | 1 | RW | The action that un-installs the software. |
| installStatus | 1 | RO | Indicates the status of the install. |
| activate | 0..1 | RW | The action that activates software previously installed. |
| deactivate | 0..1 | RW | The action that deactivates software. |
| activeStatus | 0..1 | RW | The status of active or deactivate action. |

The state machine for managing the software in oneM2M is shown in the following figure.

Uninstalled

Execute: ./[software]/Install

Execute: ./[software]/Uninstall

Removed

Installed

Delete: ./[software]

Figure D.3-2: State machine for software management

The state machine after install starts from the deactivated state.

Figure D.3-3: State machine for software management after install

## D.4	Resource memory

Figure D.4-1: Structure of <memory> resource

[D.4-a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <memory> resource shall contain the child resource according to their multiplicity in Table D.4-1.

Table D.4-1: Child resources of <memory> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <memory> resource shall contain the attributes described in Table D.4-2.

Table D.4-2: Attributes of <memory> resource

| Attribute Name of <memory> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “Memory” to indicate the resource is for memory management. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| memAvailable | 1 | RW | The current available amount of memory. |
| memTotal | 1 | RW | The total amount of memory. |

## D.5	Resource areaNwkInfo

Figure D.5-1: Structure of <areaNwkInfo> resource

[D.5-a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <areaNwkInfo> resource shall contain the child resource in Table D.5-1.

Table D.5-1: Child resources of <areaNwInfo> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <areaNwkInfo> resource shall contain the attributes described in Table D.5-2.

Table D.5-2: Attributes of <areaNwkInfo> resource

| Attribute Name of <areaNwInfo> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “AreaNwkInfo” to indicate the resource is for area network information. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| areaNwkType | 1 | RW | The areaNwkType is an implementation-chosen string that indicates the type of M2M Area Network. |
| listOfDevices | 1 | RW | Indicates the list of devices in the M2M Area Network. The attribute contains references to AreaNwkDeviceInfo resource. From listOfDevices, the topology of the area network can be discovered and retrieved. |

## D.6	Resource areaNwkDeviceInfo

Figure D.6-1: Structure of <areaNwkDeviceInfo> resource

[D.6-a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <areaNwkDeviceInfo> resource shall contain the child resource in Table D.6-1.

Table D.6-1: Child resources of <areaNwInfo> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <areaNwkDeviceInfo> resource shall contain the attributes described in Table D.6-2.

Table D.6-2: Attributes of <areaNwkDeviceInfo> resource

| Attribute Name of <areaNwkDeviceInfo> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “AreaNwkDeviceInfo” to indicate the resource is for area network device information. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| devId | 1 | RW | Indicates the id of the device. It could be the id of the hardware or nodeId. |
| devType | 1 | RW | Indicates the type of the device. The attribute also indicates the functions or services that are provided by the device. Examples include temperature sensor, actuator, Zigbee coordinator or Zigbee router. |
| areaNwkId | 1 | RW | The reference to an AreaNwkInfo resource which this device associates with. |
| sleepInterval | 0..1 | RW | The interval between two sleeps. |
| sleepDuration | 0..1 | RW | The time duration of each sleep. |
| status | 0..1 | RW | The status of the device (sleeping or waked up). |
| listOfNeighbors | 1 | RW | Indicates the neighbour devices of the same area network. When modified, the connection relationship of the devices shall be modified accordingly. |

## D.7	Resource Type battery

This resource is used to share information regarding the battery. The resource type of the Resource battery is <mgmtObj>.

[D.7a,]  Editor's Note: Using [battery] notation needs more discussion with WG2.

[D.7.b]  Editor's Note: The notation used for describing [battery] needs to be discussed and to be added to the Guidelines clause 9.1

Figure D.7-1: Structure of <battery> resource

[D.7-b0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <battery> resource shall contain the child resource in Table D.7-1.

Table D.7-1: Child resources of <battery> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <battery> resource shall contain the attributes in Table D.7-2.

Table D.7-2: Attributes of <battery> resource

| Attribute Name of <battery> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| Labels | 0..1 |  | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | See clause 9.6.10. This attribute shall have the fixed value “battery”. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| batteryLevel | 1 | RO | The current battery level. |
| batteryStatus | 1 | RO | Indicates the status of the battery. |

## D.8	Resource Type deviceInfo

This resource is used to share information regarding the device. The resource type of the Resource deviceInfo is <mgmtObj>.

[D.8.a]  Editor's Note: The notation used for describing [deviceInfo] needs to be discussed and to be added to the Guidelines clause 9.1

Figure D.8-1: Structure of <deviceInfo> resource

[D.8-a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <deviceInfo> resource shall contain the child resource in Table D.8-1.

Table D.8-1: Child resources of <deviceInfo> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <deviceInfo> resource shall contain the attributes in Table D.8-2.

Table D.8-2: Attributes of <deviceInfo> resource

| Attribute Name of <deviceInfo> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| Labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | See clause 9.6.10. This attribute shall have the fixed value “deviceInfo”. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.10 |
| deviceLabel | 1 | RO | Unique device label assigned by the manufacturer. The uniqueness may be global or only valid within a certain domain (e.g., vendor-wise or for a certain deviceType). |
| Manufacturer | 1 | RO | The name/identifier of the device manufacturer. |
| Model | 1 | RO | The name/identifier of the device mode assigned by the manufacturer. |
| deviceType | 1 | RO | The type (e.g., cell phone, photo frame, smart meter) or product class (e.g., X-series) of the device. |
| fwVersion | 1 | RO | The firmware version of the device. NOTE: If the device only supports one kind of Software this is identical to softwareVersion. |
| swVersion | 1 | RO | The software version of the device. |
| hwVersion | 1 | RO | The hardware version of the device. |

[D.9.a]  Editor's Note: The notation used for describing [deviceCapability] needs to be discussed.

## D.9	Resource Type deviceCapability

This resource is the instance for representing each device capability. The resource type of the resource deviceCapability is <mgmtObj>.

Figure D.9-1: Structure of <deviceCapability> resource

[D.9.a] Editor's Note; The picture above needs to be updated as per the new strucutre of deviceCapability resource.

The <deviceCapability> resource shall contain the child resource in Table D.9-1.

Table D.9-1: Child resources of <deviceCapability> resource

| Child Resource Name of <deviceCapability> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <deviceCapability> resource shall contain the attributes in Table D.9-2

Table D.9-2: Attributes of <deviceCapability> resource

| Attribute Name of <deviceCapability> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| capabilityName | 1 | WO | The name of the capability. |
| attached | 1 | RO | Indicates whether the capability is attached to the device or not. |
| capabilityActionStatus | 1 | RO | Indicates the status of the Action (including a progress indicator, a final state and a reminder of the requested action). |
| enable | 0..1 | WO | The action that allows enabling the device capability. |
| disable | 0..1 | WO | The action that allows disabling the device capability. |

## D.10	Resource Type reboot

This resource is used to reboot the device. The resource type of the resource reboot is <mgmtObj>.

[D.10.a]  Editor's Note: The notation used for describing [reboot] needs to be discussed.

Figure D.10-1: Structure of <reboot> resource

[D.10.a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <reboot> resource shall contain the child resource in Table D.10-1.

Table D.10-1: Child resources of <reboot> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <reboot> resource shall contain the attributes in Table D.10-2.

Table D.10-2: Attributes of <reboot> resource

| Attribute Name of <reboot> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| Labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | See clause 9.6.10. This attribute shall have the fixed value “reboot”. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| reboot | 1 | RW | The action that allows rebooting the device. |
| factoryReset | 1 | RW | The action that allows making the device returning to the factory settings. |

## D.11	Resource Type eventLog

This resource is used to record the event log for the device. The resource type of the resource eventLog is <mgmtObj>.

[D.11.a]  Editor's Note: The notation used for describing [eventLog] needs to be discussed.

Figure D.11-1: Structure of <eventLog> resource

[D.11.a0] Editor's Note: Verify if the picture above needs to be updated as per the new strucutre of the resource.

The <eventLog> resource shall contain the child resource in Table D.11-1.

Table D.11-1: Child resources of <eventLog> resource

| Child Resource Name of <statsConfig> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The <eventLog> resource shall contain the attributes in Table D.11-2.

Table D.11-2: Attributes of <eventLog> resource

| Attribute Name of <eventLog> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | WO | See clause 9.6.1 where this common attribute is described |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | See clause 9.6.10. This attribute shall have the fixed value “eventLog”. |
| objectID | 0..1 | WO | See clause 9.6.13 |
| objectPath | 0..1 | WO | See clause 9.6.13 |
| description | 0..1 | RW | See clause 9.6.13 |
| logTypeId | 1 | RW | Identifies the types of log to be recorded. E.g., security log, system log. |
| logData | 1 | R | Diagnostic data logged upon event of interests defined by this diagnostic function. |
| logActionStatus | 1 | RO | Indicates the status of the Action. E.g., Started, Stopped. |
| logStart | 1 | RW | The action that allows starting the log corresponding to the mentioned logTypeId. |
| logStop | 1 | RW | The action that allows stopping the log corresponding to the mentioned logTypeId. |

## D.12	Resource cmdhPolicy

A [cmdhPolicy] resource is defined as an instance of the <mgmtObj> resource type as specified in clause 9.6.15. It includes a number of child resources which are referenced by means of mgmtLink attributes. Each of these linked child resources represents itself an instance of the <mgmtObj> resource type. These child resources and their child resources are defined in the following sub-clauses D.12.1 to D.12.7.

The [cmdhPolicy] resource represents a set of rules defining which CMDH parameters will be used by default when a request issued by a local originator (e.g., Registree AE or another CSF inside the Registrar CSE) contains the “ec” parameter but not all other CMDH parameters.

One or several [cmdhPolicy] resources may be linked as child resources of the local <CSEBase> resource, thus defining a set of CMDH policies applicable whenever this Hosting CSE receives requests.

In addition, one or several [cmdhPolicy] resources may be linked as child resources of other resources of type <AE> or <remoteCSE>, thus overriding the default set of policies for the relevant locally registered Application Entities or remote CSEs.

[9.6.23.a] Editor’s Note: The parent resource for this [cmdhPolicy] resource is TBD.

Figure D-12-1: Structure of [cmdhPolicy] resource

[D.12.b] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc

[D.12.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteur thinking is NO. Need to verify with the WG before removing the child resource table below.

The [cmdhPolicy] resource shall contain the child resource in Table D.12-1

Table D.12-1: Child resources of [cmdPolicy] resource

| Child Resource Name of [cmdhPolicy] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The [cmdhPolicy] resource shall contain attributes shown in Table D.12-2.

Table D.12-2: Attributes of [cmdhPolicy] resource

| Attribute Name of [cmdhPolicy] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhPolicy” to indicate the resource is for CMDH policy management. |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| name | 1 | RW | A name under which the CMDH policy will be referred. |
| status | 1 | RW | Either “active” or “inactive”, this attribute allows to provision policies without activating them immediately, thus sparing the need to completely erase policies and then re-provision them remotely when one only needs to deactivate them temporarily. Only one policy can be “active” at any given time. |
| mgmtLink | 4..n | RW | 1 link to [cmdhDefaults] resource, 1..n link(s) to [cmdhLimits] resource(s), 1..n link(s) to [cmdhNetworkAccessRules] resource(s), 1..n link(s) to [cmdhBuffer] resource(s) |

NOTE: In order to avoid race conditions, it may be better that the CSE refers to the currently active cmdhPolicy instead of using the “status” attribute.

[9.6.23.b] Editor’s Note: When the <CSEBase> resource is accepted, an attribute or link in the <CSEBase> resource could be used to point to the active <cmdhPolicy> resource. Now reference should be to link to <mgmtObj> resource.

### D.12.1	Resource cmdhDefaults

The [cmdhDefaults] resource is used to define default values that shall be used for CMDH-related parameters when requests issued by Originators (registered AEs or functions inside the CSE itself) do not contain a value for the parameters ec (event category), rqet (request expiration timestamp), rset (result expiration timestamp), oet (operational execution time), rp (response persistence), and/or da (delivery aggregation).

Upon receiving a request, the CSE will first look if the ec (event category) parameter is set. If not, it will use the [cmdhDefEcValue] resources (see below) to determine a value that should be used for ec.

Then, if any of the parameters rqet, rset, oet, rp or da is not set, the CSE will use the [cmdhEcDefParamValues] resources (see below) to populate the missing parameters (and only the missing ones).

Figure D.12.1-1: Structure of [cmdhDefaults] resource

[D.12.1.1] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc

[D.12.1.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with the WG before removing the child resource table below.

The [cmdhDefaults] resource shall contain the child resource in Table D.12.1-1

Table D.12.1-1: Child resources of [cmdhDefaults] resource

| Child Resource Name of [cmdhDefaults] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The [cmdhDefaults] resource shall contain attributes shown in Table D.12-1-2.

Table D.12.1-2: Attributes of [cmdhDefaults] resource

| Attribute Name of [cmdhDefaults] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..1 | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhDefaults” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| mgmtLink | 2..n | RW | 1…n link(s) to [cmdhDefEcValue] resource(s), 1..n link(s) to [cmdhEcDefParamValues] resource(s) |

### D.12.2	Resource cmdhDefEcValue

The [cmdhDefEcValue] resource is used to define a value for the ec (event category) parameter of an incoming request when it is not defined.

Upon receiving a request, the CSE will go through all the [cmdhDefEcValue] resources (in the order of their "order" attribute), check the requestOrigin and any present requestContext and requestCharacteristics attributesto see if they match (see description of matching), and if they all do, assign the value stored in the defEcValue attribute to the ec parameter.

Figure D.12.2-1: Structure of [cmdhDefEcValue] resource

[D.12.2.1] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc

[D.12.2.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with WG before removing the child resource table below.

The [cmdhDefEcValue] resource shall contain the child resource in Table D.12.2-1

Table D.12.2-1: Child resources of [cmdhDefEcValue] resource

| Child Resource Name of [cmdhDefEcValue] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The [cmdhEcDefEcValue] resource shall contain attributes shown in Table D.12.2-2.

Table D.12.2-2: Attributes of [cmdhDefEcValue] resource

| Attribute Name of [cmdhDefEcValue] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0.1 | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhDefEcValue” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| order | 1 | RW | The index indicating in which order the cmdhDefEcValue resource will be treated by the CSE to determine a value for the ec parameter |
| defEcValue | 1 | RW | The actual value to use for the ec parameter if the conditions expressed in the requestOrigin, requestContext and requestCharacteristics all match. If none of these attributes are defined, then the defEcValue shall be applied. |
| requestOrigin | 1 | RW | The requestOrigin attribute is a list of zero or more local AE-IDs, App-IDs, or the strings ‘localAE’ or ‘thisCSE’. When an AE-ID appears in the requestOrigin attribute, the default ec value defined inside the defEcValue attribute is applicable for the ec if the request was issued by that specific Application Entity. When an App-ID appears in the requestOrigin attribute, the default ec value defined inside the defEcValue attribute is applicable for the ec if the request was issued by the AE with that App-ID unless covered by another <cmdhDefEcValue> resource with a requestOrigin attribute containing its specific AE-ID.. When the string ‘localAE’ appears in the requestOrigin attribute, the default ec value defined inside the defEcValue attribute is applicable for the ec for requests issued by all local AEs unless covered by another <cmdhDefEcValue> resource with a requestOrigin attribute containing the specific AE-ID or App-ID of the originator of the request. When the string ‘thisCSE’ appears in the requestOrigin attribute, the default ec value defined inside the defEcValue attribute is applicable for the ec for requests that are originating from within the Registrar CSE. The hosting CSE shall contain at least one <cmdhDefEcValue> resource that contains ‘localAE’ in the ‘requestOrigin’ attribute and has no ‘requestContext’ and no ‘requestCharacteristics’ attribute. The hosting CSE shall contain at least one <cmdhDefEcValue> resource that contains ‘thisCSE’ in the ‘requestOrigin’ attribute and has no ‘contextCondtion’ and no ‘requestCharacteristics’ attribute. |
| requestContext | 0..1 | RW | The requestContext attribute represents the Dynamic Context condition under which the default ec value defined inside the defEcValue attribute is applicable for the ec. This may refer to conditions such as current battery status, or current network signal strength. |
| requestContextNotification | 0..1 | RW | True or false. If set to true, then this CSE will establish a subscription to the dynamic context information defined in the requestContext attribute as well as a subscription to this <cmdhDefEcValue> resource for all AEs corresponding to the AE-ID or an App-ID appearing in the requestOrigin attribute. Both, changes in the context information and changes to the <cmdhDefEcValue> resource will be notified to the respective AEs. The subscription(s) is/are established when the <cmdhDefEcValue> is provisioned or updated. |
| requestCharacteristics | 0..1 | RW | The requestCharacteristics attribute represents conditions pertaining to the request itself, such as the requested response type (rt parameter) or other parameters of the request. |

### D.12.3	Resource cmdhEcDefParamValues

The [cmdhEcDefParamValues] resource is used to represent a specific set of default values for the CMDH related parameters rqet (request expiration timestamp), rset (result expiration timestamp), oet (operational execution time), rp (response persistence) and da (delivery aggregation) that are applicable for a given ec (event category) if these parameters are not specified in the request.

Figure D.12.3-1: Structure of [cmdhEcDefParamValues] resource

[D.12.3.1] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc

[D.12.3.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with the WG before removing the child resource table below.

The [cmdhEcDefParamValues] resource shall contain the child resource in Table D.12.3-1

Table D.12.3-1: Child resources of [cmdhEcDefParamValues] resource

| Child Resource Name of [cmdhEcDefParamValues] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The [cmdhEcDefParamValues] resource shall contain attributes shown in Table D.12.3-2.

Table D.12.3-2: Attributes of [cmdhEcDefParamValues] resource

| Attribute Name of [cmdhEcDefParamValues] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhEcDefParamValues” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| applicableEventCategory | 1 | RW | This attribute defines the event categories for which this set of default parameters defined in this <cmdhEcDefParamValues> resource are applicable.This attribute is a list of zero or more Event Category values (‘ec’ parameter of a request), or the string ‘default’. When an Event Category value appears in the ‘applicableEventCategory’ attribute, the set of default parameters defined in this <cmdhEcDefParamValues> resource are applicable for requests associated with that specific Event Category (‘ec’) value. When the string ‘default’ appears in the ‘applicableEventCategory’ attribute, the set of default parameters defined in this <cmdhEcDefParamValues> resource are applicable for all requests whose associated Event Category value (‘ec’) is not listed in the ‘applicableEventCategory’ attribute of any other provisioned <cmdhEcDefParamValues> resource on the hosting CSE. A specific Event Category value (‘ec’) shall appear at most once in any of the ‘applicableEventCategory’ attributes of any of the provisioned <cmdhEcDefParamValues> resources on the hosting CSE. The string ‘default’ shall appear exactly once in any of the ‘applicableEventCategory’ attributes of any of the provisioned <cmdhEcDefParamValues> resources on the hosting CSE. |
| defaultRequestExpTime | 1 | RW | Default value for the request expiration time parameter (‘rqet’) in a request when the ‘rqet’ parameter of the request is not set. |
| defaultResultExpTime | 1 | RW | Default value for the result expiration time parameter (‘rset’) in a request when the ‘rset’ parameter of the request is not set. |
| defaultOpExecTime | 1 | RW | Default value for the operational execution time parameter (‘oet’) in a request when the ‘oet’ parameter of the request is not set. |
| defaultRespPersistence | 1 | RW | Default value for the response persistence parameter (‘rp’) in a request when the ‘rp’ parameter of the request is not set. |
| defaultDelAggregation | 1 | RW | Default value for the delivery aggregation parameter (‘da’) in a request when the ‘da’ parameter of the request is not set. |

### D.12.4	Resource cmdhLimits

The [cmdhLimits] resource is used to define limits for CMDH related parameter values used in requests issued by Originators (registered AEs or functions inside the CSE itself). When an incoming request is processed that does not comply with the limits defined by the corresponding [cmdhLimits] resource, the request shall be rejected by the CSE.

Figure D.12.4-1: Structure of [cmdhLimits] resource

[D.12.4.a] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc.

[D.12.4.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with the WG before removing the child resource table below.

The [cmdhLimits] resource shall contain the child resource in Table D.12.4-1

Table D.12.4-1: Child resources of [cmdhLimits] resource

| Child Resource Name of [cmdhLimits] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The [cmdhLimits] resource shall contain attributes Table D.12.4-2.

Table D.12.4-2: Attributes of [cmdhLimits] resource

| Attribute Name of [cmdhLimits] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhLimits” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| order | 1 | RW | The index indicating in which order the [cmdhLimits] resource will be treated by the CSE to determine a value for the limit parameters. |
| requestOrigin | 1 | RW | The requestOrigin attribute is a list of zero or more local AE-IDs, App-IDs, or the strings ‘localAE’ or ‘thisCSE’. When an AE-ID appears in the requestOrigin attribute, the CMDH parameter limits defined inside [cmdhLimits] resources are applicable for requests issued by that specific Application Entity. When an App-ID appears in the requestOrigin attribute, the CMDH parameter limits defined inside [cmdhLimits] resources are applicable for requests issued by the AE with that App-ID unless already covered by another [cmdhLimits] resource with a requestOrigin attribute containing its specific AE-ID. When the string ‘localAE’ appears in the requestOrigin attribute, CMDH parameter limits defined inside [cmdhLimits] resources are applicable for all local AEs unless covered by another [cmdhLimits] resource with a requestOrigin attribute containing the specific AE-ID or App-ID of the originator of the request. When the string ‘thisCSE’ appears in the requestOrigin attribute, CMDH parameter limits defined inside [cmdhLimits] resources are applicable for all requests that are originating from within the hosting CSE. The hosting CSE shall contain at least one [cmdhLimits] resource that contains ‘localAE’ in the ‘requestOrigin’ attribute and has no ‘contextCondition’ and no ‘requestCharacteristics’ attribute. The hosting CSE shall contain at least one [cmdhLimits] resource that contains ‘thisCSE’ in the ‘requestOrigin’ attribute and has no ‘requestContext’ and no ‘requestCharacteristics’ attribute. |
| requestContext | 0..1 | RW | The requestContext attribute represents the Dynamic Context condition under which CMDH parameter limits defined inside the [cmdhLimits] resource is applicable. This may refer to conditions such as current battery status, or current network signal strength. |
| requestContextNotification | 0..1 | RW | True or false. If set to true, then this CSE will establish a subscription to the dynamic context information defined in the requestContext attribute as well as a subscription to this [cmdhLimits] resource for all AEs corresponding to the AE-ID or an App-ID appearing in the requestOrigin attribute. Both, changes in the context information and changes to the [cmdhLimits] resource will be notified to the respective AEs. The subscription(s) is/are established when the [cmdhLimits] is provisioned or updated |
| requestCharacteristics | 0..1 | RW | The requestCharacteristics attribute represents conditions pertaining to the request itself, such as the requested response type (rt attribute) or other attributes of the request. |
| limitsEventCategory | 1 | RW | Allowed values for the event category parameter (‘ec’) in a request of any of the originators indicated in the requestOrigin attribute. |
| limitsRequestExpTime | 1 | RW | Range of allowed values for the request expiration time parameter (‘rqet’) in a request of any of the originators indicated in the requestOrigin attribute. |
| limitsResultExpTime | 1 | RW | Range of allowed values for the result expiration time parameter (‘rset’) in a request of any of the originators indicated in the requestOrigin attribute. |
| limitsOpExecTime | 1 | RW | Range of allowed values for the operational execution time parameter(‘oet’) in a request of any of the originators indicated in the requestOrigin attribute. |
| limitsRespPersistence | 1 | RW | Range of allowed values for the response persistence parameter (‘rp’) in a request of any of the originators indicated in the requestOrigin attribute. |
| limitsDelAggregation | 1 | RW | List of allowed values for the delivery aggregation parameter (‘da’) in a request of any of the originators indicated in the requestOrigin attribute. |

### D.12.5	Resource cmdhNetworkAccessRules

The [cmdhNetworkAccessRules] resource is used to define the usage of underlying networks for forwarding information to other CSEs during processing of CMDH-related requests in a CSE. When an incoming request is processed by a CSE, it can only use underlying networks for forwarding any information to other CSEs in compliance with the rules defined by the corresponding [cmdhNetworkAccessRules] resource.

If a request cannot be successfully completed in compliance with the rules defined in the corresponding [cmdhNetworkAccessRules] resource, that request shall either be rejected in case it has not already been accepted by the CSE or it has to be purged. Error reporting on failed CMDH processing depends on error reporting parameters (TBD).

Figure D.12.5-1: Structure of [cmdhNetworkAccessRules] resource 
(only child resources resource specific attributes are shown)

[D.12.5.a] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc.

[D.12.5.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with th eWG before removing the child resource table below.

The [cmdhNetworkAccessRules] resource shall contain the child resource shown in Table D.12.5-1.

Table D.12.5-1: Child resources of <cmdhNetworkAccessRules> resource

| Child Resource Name of [cmdhNetworkAccessRules] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

If a [cmdhNetworkAccessRules] resource has no mgmtLink attribute to [cmdhNwAccessRules] resources (i.e., multiplicity of 0), requests that match with the applicableEventCategorie attribute (see description of attributes in Table D.12.5-2) will not be allowed to use any underlying network for forwarding information, i.e., such requests need to be rejected.

The [cmdhNetworkAccessRules] resource shall contain attributes shown in Table D.12.5-2.

Table D.12.5-2: Attributes of [cmdhNetworkAccessRules] resource

| Attribute Name of [cmdhNetworkAccessRules] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhNetworkAccessRules” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| applicableEventCategories | 1 | RW | This attribute defines for which requests the rules contained in [cmdhNwAccessRule] resources linked from this [cmdhNetworkAccessRules] resource shall be applied. This attribute is a list of zero or more Event Category values (‘ec’ parameter of a request), or the string ‘default'. When an Event Category value appears in the applicableEventCategories attribute, the network usage rules defined inside [cmdhNwAccessRule] child resources are applicable for requests associated with that specific Event Category (‘ec’) value. When the string ‘default’ appears in the applicableEventCategories attribute, the network usage rules defined inside [cmdhNwAccessRule] child resources are applicable for all requests whose associated Event Category value (‘ec’) is not listed in the applicableEventCategories attribute of any other provisioned [cmdhNetworkAccessRules] resource on the hosting CSE. A specific Event Category value (‘ec’) shall appear at most once in any of the applicableEventCategories attributes of any of the provisioned [cmdhNetworkAccessRules] resources on the hosting CSE. The string ‘default’ shall appear exactly once in any of the applicableEventCategories attributes of any of the provisioned [cmdhNetworkAccessRules] resources on the hosting CSE. |
| mgmtLink | 0..n | RW | Link(s) to [cmdhNwAccessRule] resource(s) |

### D.12.6	Resource cmdhNwAccessRule

The [cmdhNwAccessRule] resource is used define limits in usage of specific underlying networks for forwarding information to other CSEs during processing of CMDH-related requests.

Figure D.12.6-1: Structure of [cmdhNwAccessRule] resource

[D.12.6.a] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc.

[D.12.6.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with the WG before removing the child resource table below.

The [cmdhNwAccessRule] resource shall contain the child resource shown in Table D.12.6-1.

Table D.12.6-1: Child resources of [cmdhNwAccessRule] resource

| Child Resource Name of [cmdhNwAccessRule] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The allowedSchedule child resource defines the periods of time during which it is allowed to use the underlying networks that match with the targetNetwork attribute of this [cmdhNwAccessRule] resource (see description of attributes in Table D.12.6-2) when forwarding information to other CSEs due to requests that match the applicableEventCategories attribute of the parent [cmdhNetworkAccessRules] resource of this [cmdhNwAccessRule] resource.

The [cmdhNwAccessRule] resource shall contain attributes shown in Table D.12.6-2.

Table D.12.6-2: Attributes of [cmdhNwAccessRule] resource

| Attribute Name of [cmdhNwAccessRule] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhNwAccessRules” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| targetNetwork | 1 | RW | The targetNetwork attribute defines for which underlying networks the usage limits contained in this [cmdhNwAccessRule] resource shall be applied. The targetNetwork attribute is a list of one or more strings identifying names of underlying networks or the string ‘default’. Note: An agreed naming convention for Underlying Network names are TBD. When a name of an Underlying Network appears in the targetNetwork attribute, the usage limits contained in this [cmdhNwAccessRule] resource shall be applied for usage of that specific Underlying Network when processing requests matching with the parent [cmdhNetworkAccessRules] resource’s applicableEventCategories attribute. When the string ‘default’ appears in the targetNetwork attribute, the usage limits contained in this [cmdhNwAccessRule] resource shall be applied for usage of all Underlying Networks that are not listed with their specific name in the targetNetwork attribute of any other [cmdhNwAccessRule] child resource under the same parent [cmdhNetworkAccessRules] resource when processing requests matching with the parent [cmdhNetworkAccessRules] resource’s targetNetwork. Each Underlying Network name or the string ‘default’ shall appear at most once in any of the targetNetwork attributes of any of the provisioned [cmdhNwAccessRule] child resources under the same parent [cmdhNetworkAccessRules] resource. |
| minReqVolume | 1 | RW | Minimum amount of data that needs to be aggregated before any of the Underlying Networks matching with the targetNetwork attribute of this [cmdhNwAccessRule] resource can be used for forwarding information to other CSEs. |
| backOffParameters | 1 | RW | Parameters that define how usage of any of the Underlying Networks matching with the targetNetwork attribute of this [cmdhNwAccessRule] resource shall be handled when attempts to use such networks have failed. The backOffParameters attribute consists of 3 values: A back-off time that defines how long a CSE needs to wait before attempting to use a specific Underlying Network again after a first failed attempt A back-off time increment that defines by how much the back-off time shall be increased after each additional consecutive failed attempt to use the same Underlying Network without success A maximum back-off time that defines the maximum wait time before attempting to use an Underlying Network again after previous failures. |
| otherConditions | 0..n | RW | Additional conditions that need to be fulfilled before any of the Underlying Networks matching with the targetNetwork attribute of this [cmdhNwAccessRule] resource can be used for forwarding information to other CSEs. |
| mgmtLink | 1 | RW | Link to an instance “allowedSchedule” of a <schedule> resource as defined in clause 9.6.9 |

### D.12.7	Resource cmdhBuffer

The [cmdhBuffer] resource is used to define limits in usage of buffers for temporarily storing information that needs to be forwarded to other CSEs during processing of CMDH-related requests in a CSE. When an incoming request is processed by a CSE, it can only use buffers for temporary storage in compliance with the rules defined by the corresponding [cmdhBuffer] resource.

If a request cannot be processed in compliance with the rules defined in the corresponding [cmdhBuffer] resource, that request shall either be rejected in case it has not already been accepted by the CSE or it has to be purged. Error reporting on failed CMDH processing depends on error reporting parameters (TBD).

Figure D.12.7-1: Structure of [cmdhBuffer] resource

[D.12.7.a] Editor's Note; The picture above may need to be updated as per the new resource structure. Need to verify during AdHoc.

[D.12.7.b] Editor's Note; Verify if <subscripion> child resource needs to be added as proposed in AEC-2014-1247R01. Rapporteru thinking is NO. Need to verify with the WG before removing the child resource table below.

The [cmdhBuffer] resource shall contain the child resource shown in Table D.12.7-1.

Table D.12.7-1: Child resources of [cmdhBuffer] resource

| Child Resource Name of [cmdhBuffer] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 where the type of this resource is described. |

The [cmdhBuffer] resource shall contain attributes shown in Table D.12.7-2.

Table D.12.7-2: Attributes of [cmdhBuffer] resource

| Attribute Name of [cmdhBuffer] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| parentID | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| expirationTime | 1 | RW | See clause 9.6.1 where this common attribute is described. |
| accessControlPolicyIDs | 0..n | RW | See clause 9.6.1 where this common attribute is described. |
| creationTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| lastModifiedTime | 1 | RO | See clause 9.6.1 where this common attribute is described. |
| labels | 0..1 | RO | See clause 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Has fixed value “cmdhBuffer” |
| objectID | 0..1 | WO | See section 9.6.15 |
| objectPath | 0..1 | WO | See section 9.6.15 |
| description | 0..1 | RW | See section 9.6.15 |
| applicableEventCategory | 1 | RW | The applicableEventCategory attribute defines for which requests the limits contained in this [cmdhBuffer] resource shall be applied. The applicableEventCategory attribute is a list of zero or more Event Category values (‘ec’ parameter of a request), or the string ‘default’. When an Event Category value appears in the applicableEventCategory attribute, the buffer usage limits defined inside this [cmdhBuffer] resource are applicable for requests associated with that specific Event Category (‘ec’) value. When the string ‘default’ appears in the applicableEventCategory attribute, the buffer usage limits defined inside this [cmdhBuffer] resource are applicable for all requests whose associated Event Category value (‘ec’) is not listed in the applicableEventCategory attribute of any other provisioned <cmdhBuffer> resource on the hosting CSE. A specific Event Category value (‘ec’) shall appear at most once in any of the applicableEventCategory attributes of any of the provisioned [cmdhBuffer] resources on the hosting CSE. The string ‘default’ shall appear exactly once in any of the applicableEventCategory attributes of any of the provisioned [cmdhBuffer] resources on the hosting CSE. |
| maxBufferSize | 1 | RW | Maximum amount of memory that can be used for buffering requests matching with the applicableEventCategory attribute of this [cmdhBuffer] resource. |
| storagePriority | 1 | RW | Storage priority for data that is stored for buffering requests matching with the attribute of this [cmdhBuffer] resource. The storage priority defines the how to handle purging of buffered data when buffer memory is exhausted and buffered requests need to be purged. Buffered requests associated with a lower storage priority shall be purged before buffered requests with a higher storage priority. The range of storage priority is from 1 to 10. |

# Annex E			(Informative) CSE Minimum Provisioning

The present clause defines the minimum set of resources instantiated in a CSE node with the scope to make it ready to provide services to entities that will register to.

For the purpose of the initial configuration two roles are identified:

- superuser: this role allows the full CSE control according to infrastructure provider policies. Only one superuser role is allowed per CSE;

- user: is the role associated to an AE that will register itself to Registrar CSE. More than one user roles are allowed per CSE. More than one applications can access to CSE with the same role.

Superuser role may be created with the following associated resources:

- Definition or assignment of CSE-ID name that may be unique in the node hosting the CSE to be instantiated.

- Creation of <CSEBase> resource with name equal to CSE-ID.

- Creation of following child resources belonging to a tree with <CSEBase> as root:

- <accessControlPolicy> child resource enabling full access control for superuser’s invoked operations to the tree resources. Subsequent created resources may have accessControlPolicyID attribute addressing this <accessControlPolicy> resource.

- <AE> child resource to be used as registered AE dedicated to superuser related activities.

Each user role may be created with the following associated resources:

- Definition or assignment of an AE name that may be unique in the CSE.

- Creation of <AE> child resource of <CSEBase> resource named as described in step 1, to be used as registered application dedicated to user related activities.

- Creation of following child resources belonging to a tree with <AE> as root:

- <accessControlPolicy> resource enabling partial access control (e.g., these resources cannot be deleted be the user, superuser’s resources can only be read by user) for user’s invoked operations to the tree resources. <AE> resource can be updated with accessControlPolicyID attribute addressing <accessControlPolicy> resource.

The above described operations may be executed in the node in order provide the elements and the access control privileges required to provide the initial access to resource operations.

Same user can create more than one <AE> resources and other child resource types.

Once user role resource trees have been created the registered AE associated to <AE> resource (defined for a user role in step 2) is able to create its own <container> resource to store business logic application data that can be shared to other registered AEs in a controlled way acting on its own <accessControlPolicy> resource.

# Annex F			(Informative) Interworking / Integration of non-oneM2M solutions and protocols

## F.1	Introduction

Non-oneM2M solutions are currently installed and will continue to evolve and to be adopted in future for specific deployments. Some of these solution are the evolution of M2M that have a long history and significant mass installations (e.g., the PLC-related protocols commonly used in building and industrial automation), and are also significantly represented by proprietary solutions, especially in terms of semantic of the data model. The non-oneM2M solutions are potentially used for:

- Legacy deployment:  such solutions can make use of both, proprietary or standard protocols; often  proprietary data models and functionality are combined with the use of standard protocol.

- New system deployment that privilege the vertical optimization rather the horizontal aspects

- Area network deployment for which native IP based oneM2M is perceived as not optimized respect to the used technology

For those non-oneM2M solutions oneM2M needs to provide a means to enable:

- Mixed deployment that are partially oneM2M compliant and partially not, where the oneM2M System provides the solution to integrate multiple technologies (e.g., to add new technologies on top of old installations).

- Hybrid deployment that are still using non-oneM2M protocol (proprietary/standard) and want to use at the same time some of the oneM2M functionalities. A typical case is the exchange of heavy data traffic outside the CSE (e.g., for video surveillance), together with the use of CSE services for control and light traffic exchange.

## F.2	Interworking with non-oneM2M solutions through specialized interworking applications

The solution is based on the use of specialized interworking Application Entities that are interfaced to the CSE via standard Mca reference points.

Such specialized applications are named Inter-working Proxy and are described in the following picture:

Non oneM2M

interface

AE:

Interworking

Proxy

Mca

Non oneM2M

interface

Interworking

Proxy

Mca

Figure F.2-1: Interworking Proxy

The Inter-working Proxy Application Entity (IPE) is characterized by the support of a non-oneM2M reference point, and by the capability of remapping the related data model to the oneM2M resources exposed via the Mca reference point.

This is typically supported via a full semantic inter-working of the data model used by the non oneM2M and a related protocol inter-working logic, and, depending on the complexity of the non oneM2M data model, can imply the definition of a complex set of resources built via the basic oneM2M ones, or a simple direct mapping of the communication via the containers.

The approach enable a unique solution for enabling communications among different protocols, catering for different level of inter-working including  protocol inter-working, semantic information exchange,  data sharing among the different solution and deployments.

And enables the offering additional values respect to what is today available via existing protocols and proprietary service exposures.

The following picture shows the typical scenarios supported by the oneM2M architecture in the context of inter-working.  The combination of the different scenarios allows mixed deployments.

Hybrid Application

CSE(s)

Non oneM2M interface

Mca

Inter-working Proxy

Mca

(note 1)

Hybrid Application

Non oneM2M interface

Mca

Non OneM2M Application

Non oneM2M interface

Inter-working Proxy

Mca

OneM2M native Application

Mca

Note 1 :  The additional option of an inter-working proxy  embedded in the CSE as a module with an internal specified interface is underconsideration

Figure F.2-2: Scenarios Supported by oneM2M Architecture

These scenarios are applicable to the CSE with the AE as application dedicated node, in the application Service Node, in the Middle Node and in the infrastructure Node.

The following picture provides an example of the use of such capabilities an area network adopting specific protocols, e.g., Zigbee Telco Profile and Mbus using COSEM Data model.

Sensor/Meter

CSE

Mbus/COSEM

AE

Inter-working Proxy

Mca

Utily Application

Mca

Sensor/Meter

Zigbee telco Profile

AE

Inter-working Proxy

Mca

Application Service  Node

CSE

Mcc

Infrastructure Node

specific

Data model

Awareness

Common

Data model

Awareness

Figure F.2-3: TBD

[F.2.a]  Editor's Note: Figure title is needed.

There exist three variants of how interworking through an Inter-working Proxy Application Entity over Mca can be supported:

- Interworking with full mapping of the semantic of the non-oneM2M data model to Mca

- This is typically supported via a full semantic inter-working of the data model used by the non-oneM2M solution and the generic data model used in oneM2M (based on usage of containers) for exchanging application data. The IPE includes the related protocol inter-working logic.

- Depending on the complexity of the non-oneM2M data model, this can imply that the Inter-working Proxy Application Entity constructs a complex set of resources (built from the basic oneM2M resources) in the CSE. These resources are oneM2M representations of the non-oneM2M data model and are exposed by the IPE on Mca. They enable CSEs and AEs to access the entities in the non-oneM2M via the IPE.

- The benefit of this level of interworking is that it offers a unique solution for enabling communications among different protocols. The data model of the non-oneM2M solution determines its representation (the names, data types and structure of the containers) in the M2M System. It caters for different levels of inter-working including protocol inter-working, semantic information exchange, data sharing among the different solution and deployments. It enables offering additional values with respect to what is today available via existing protocols and proprietary service exposures.

- Note: With this level of interworking an M2M Application can access non-oneM2M solutions without the need to know the specific protocol encoding for these solutions. A drawback is that the IPE also potentially needs to interwork between a non-oneM2M security solution and oneM2M security. E.g., it needs to be the termination point of any non-oneM2M specific encryption.

- Interworking using containers for transparent transport of encoded non-oneM2M data and commands via Mca

- In this variant non-oneM2M data and commands are transparently packed by the Inter-working Proxy Application Entity into containers for usage by the CSEs and AEs.

- In this case the CSE or AE needs to know the specific protocol encoding rules of the non-oneM2M Solution to be able to en/de-code the content of the containers.

- Interworking using a retargeting mechanism

[F.2.b]  Editor's Note: Retargeting mechanisms may also be mentioned, if and when defined.

## F.3	Interworking versus integration of non-oneM2M solutions

Interworking:

With the approach given above – where specialized interworking applications (IPEs) allow to interact with any non-oneM2M system via the Mca interface – proprietary non-oneM2M solutions as well as non-oneM2M solutions that follow open standards can be interworked with the oneM2M System.

Integration:

When it is desired to make a certain type of non-oneM2M solution (e.g., some type of non-IP based Area Network) a permanent part of the deployed oneM2M Solution then the functionality of the Inter-working Proxy Application Entity can be integrated into the CSE of an Application Node. This is called “Integration” non-oneM2M solutions.

## F.4	Entity-relation representation of non-IP based M2M Area Network

The following figure provides an entity-relation model that represents a non-IP based M2M area network as well as its relationship to an Interworking Proxy Application Entity (IPE).

IPE

M2M Area Network

1

n

device

Application

Interface

Data Field

Method

1

1

1

1

1

n

n

n

n

n

Figure F.4-1: Generic entity-relation diagram for an IPE and 
an M2M Area Network running legacy devices

This entity-relation diagram is e.g., applicable to the following M2M Area Networks:

- ZigBee

- DLMS/COSEM

- Zwave

- BACnet

- ANSI C12

- mBus

### F.4.1	Responsibilities of Interworking Proxy Application Entity (IPE)

More specifically, the IPE is responsible to:

- create oneM2M resources representing the M2M Area Network structure (devices, their applications and interfaces) in the oneM2M Service Capability Layer, accessible via Mca,

- manage the oneM2M resources in case the M2M Area Network structure changes.

- discover the M2M Area Network structure and its changes automatically if this is supported by the technology of the M2M Area Network

[F.4.1.a]  Editor’s Note: Mapping principles of the none-oneM2M information model into oneM2M resources is FFS.

# Annex G			(Informative) List of M2M Services

This annex provides the list of M2M Services supported in this document as well as the list of associated roles mapped to each M2M Service.

Figure G-1: M2M Services

| M2M Service (name) | M2M-Serv-ID | Roles |
| --- | --- | --- |
| Application and service management | 01 | 1-Configuration 2-Diagnostic and monitoring 3-Software management |
| Device management | 02 | 1-Device Configuration 2-Device Diagnostics and Monitoring 3-Device Firmware Management 4-Device Topology |
| Location | 03 | 1-Location |
| Data exchange | 04 | 1-Basic data |
| Device onboarding | 05 | 1-onboarding |
| Service Provider specific | 80 to 99 range |  |

[G.a]  Editor’s Note: Each role must be mapped to a set of resource types and related primitives.

# Annex H			(Informative) Object Identifier Based M2M Device Identifier

## H.1.	Overview of Object Identifier

In M2M systems, it is required for devices to be distinguishable from one another through some kind of ID system. In other words, the ID which is allocated to the device must be globally unique to ensure the proper operation of M2M systems, such as finding and connecting devices.

In relation to this requirement, the use of Object Identifiers may provide a convenient method to ensure the global uniqueness of M2M devices. The Object Identifier (OID) is an identification mechanism jointly developed by ITU-T and ISO/IEC which can be applied to objects, concepts, and all kinds of tangible or intangible things.

OID uses a hierarchical tree structure and is represented as a sequence of integer values, as shown in figure X. OID consists of several segments called arcs which provide placeholders for identification and description in the hierarchal tree. In the OID tree, the Root arc is unnamed and is represented by the forward slash (/) sign. The first arc represents the organization code and is used to manage and allocate its corresponding lower arc. The first arc can take the following values: itu-t (0), iso (1), and joint-iso-itu-t (2).

OID is hierarchically allocated, and the organization or the nation has the authority to define its lower arcs. For example, ITU-T can manage and allocate the lower arc below itu-t (0), and ISO can allocate the lower arc below iso (1). The general procedure regarding the use of OID is described in ITU-T X.660 | ISO/IEC 9834-1 “Information technology – Open Systems Interconnection - Procedures for the operation of OSI registration authorities: General procedures and top arcs of the international object identifier tree”.

Figure H.1-1: International OID Tree

## H.2.	OID Based M2M Device Identifier

An M2M device shall be identified individually through a globally unique ID system. This clause explains how to allocate a globally unique ID to each M2M device by using the OID scheme. M2M device ID is an example which shows that OID can be applied to any M2M identifiers which need globally unique IDs.

The M2M device ID consists of a higher arc and a sequence of four arcs. It takes the form of {(higher arc) (x) (y) (z) (a)} as illustrated in figure Y. The higher arc is defined and managed according to the OID procedure. Each arc in the remaining sequence of four arcs represents the manufacturer ID, product model ID, serial number ID, and expanded ID, respectively.

Figure H.2-1: M2M Device ID

### H.2.1	M2M Device Indication ID – (higher arc)

The M2M Device Indication ID (higher arc) represents a globally unique identifier for the M2M device. The composition of the higher arc is variable and may be composed of several sub-arcs. The higher arc is assigned and managed by ITU-T/ISO.

### H.2.2.	Manufacturer ID – (x)

The 1st arc (x) among the sequential 4 arcs is used to identify the manufacturer which produces the M2M device. The first arc (x) is managed and allocated by the authority related with (higher arc).

### H.2.3.	Model ID – (y)

The 2nd arc (y) among the sequential 4 arcs identifies the device model produced by the manufacturer x. The second arc is managed and allocated by the manufacturer represented by the (x) arc.

### H.2.4.	Serial Number ID – (z)

The 3rd arc (z) among the sequential 4 arcs is for identifying the serial number of the device model y. The third arc is managed and allocated by the manufacturer represented by the (x) arc.

### H.2.5.	Expanded ID – (a)

The 4th arc (a) among the sequential 4 arcs is for identifying the legacy device which operates under the M2M device. The 4th arc for Expanded ID is allocated by the M2M device by adding a 4th arc to its device ID {(higher arc) (x) (y) (z)}. Therefore, the ID of legacy device which operates under the M2M device takes the form of {(higher arc) (x) (y) (z) (a)}. The fourth arc is managed and allocated by the M2M device.

## H.3.	Example of M2M device ID based on OID

Let us assume an M2M Device ID of {0 2 481 1 100 3030 10011}. The M2M device ID can be interpreted as follows:

- (0 2 481 1) in {0 2 481 1 100 3030 10011} – represents the M2M Device Indication ID (higher arc)

- (0) in {0 2 481 1 100 3030 10011} – identifies the managing organization ITU-T

- (2) in {0 2 481 1 100 3030 10011} – identifies “Administration”

- (481) in {0 2 481 1 100 3030 10011} – identifies the data country code for Korea

- (1) in {0 2 481 1 100 3030 10011} – identifies an M2M device

- (100) in {0 2 481 1 100 3030 10011} – identifies the device Manufacturer

- (3030) in {0 2 481 1 100 3030 10011} – identifies the device Model

- (10011) in {0 2 481 1 100 3030 10011} – identifies the device Serial number

# Annex I			(Informative) Functionalities for the Subsequent Architecture TS Release

## I.1	Service Session Management Text for Removal

### 6.2.12	Service Session Management

#### 6.2.12.1	General Concepts

An M2M service session is an end-to-end Service Layer connection managed by the Service Session Management (SSM) CSF.  The SSM CSF manages M2M service sessions between AEs, between an AE and a CSE, or between CSEs.

Editor's Note: Provide information as to when an M2M service session is needed.

The management of a M2M service session includes capabilities such as the management of session state, session authentication and establishment, management of Underlying Network connections and services related to the session, coordination of sessions spanning multiple hops of CSEs, exchange of information between session endpoints, and session termination.

The SSM CSF uses the CMDH CSF within the same CSE for sending/receiving messages to/from the next-hop CSE or to/from an AE for a given M2M service session. The SSM CSF also uses the SEC CSF for the management of session related security credentials and authentication of session participants. The SSM CSF generates session specific charging events also that it communicates to the SCA CSF within the same CSE.

#### 6.2.12.2	Detailed Descriptions

An M2M service session is an end-to-end Service Layer connection managed by the SSM CSF. The SSM CSF shall support the management of the following M2M service session related information.

- Session Identifier: The Service Session Identifier is used by the SSM CSF and session endpoints to uniquely identify M2M service layer session.

- Session Credentials: Security credentials associated with M2M service session. For example, E2E security certificates, public keys, etc. An M2M service session can support an independent set of credentials or it can optionally leverage security credentials from Underlying Network sessions or network connections.

- Session Descriptor: Information describing the M2M service session that can be used by the existing session endpoints or prospective session participants in order to discover an existing session.  For example, a description for each session participant (e.g., device identifiers, type of participant, services that participant supports, interface requirements of participant, type of compression used, etc.).

- Session Routing Information: Information describing how to route M2M service session messages. For example, list of CSEs in the routing path, or next-hop CSE in the routing path.

- Session Context/History: Information related to M2M service session activity such as session related events that have occurred or transactions that have been processed. For example, keeping track of the type, number, rate, size, etc. of the resources targeted by the session endpoints. Or keeping track of different service sessions that a specific application establishes (e.g., rate, type, etc).

- Session Policies: Policies that define rules for how SSM CSF manages sessions. For example, session routing policies, session store-and-forward policies, session access control policies, session data management policies, etc. The SSM CSF can use these policies by itself or provide such policies to other CSFs (e.g., CMDH CSF, DMR CSF, SEC CSF, etc).

Some M2M service sessions may require security. In order to secure an M2M service session, proper security credentials shall be used by session endpoints (e.g., AEs and/or CSEs).  If M2M service session credentials are not pre-provisioned, the SSM CSF shall support securely bootstrapping of the session credentials to the session endpoints. The SSM CSF shall use the SEC CSF for supporting such bootstrapping. The SSM CSF may also leverage security credentials and trust relationships from Underlying Networks.

The SSM CSF shall support requests to establish an M2M service session between AEs, between an AE and a CSE, or between CSEs. Before a request to establish an M2M service session is granted, the SSM CSF shall first authenticate the requester using session credentials. The SSM CSF shall use the SEC CSF to support session authentication. Once authenticated, the SSM CSF shall establish the M2M service session between the requesting and targeted session endpoints. This shall involve the SSM CSF coordinating with the targeted session endpoint on an agreed upon session ID that can be used to identify the session messages. The SSM CSF shall return this session ID to the requester. The SSM CSF shall also maintain additional session information for the management of the session such as session policies, session routing information, session descriptor, etc.

The SSM CSF shall support layering of a M2M service session over the top of Underlying Network connections. The SSM CSF shall support persistency of the M2M service sessions with respect to the Underlying Network connections. The SSM CSF shall maintain an active M2M service session independent of the state of the Underlying Network connections and shall be robust to network connections that are dynamically torn-down and re-established. The SSM CSF shall support initiating or providing input to other CSFs and/or the Underlying Network on whether the network connections should be torn-down/re-established based on M2M service session activity or state.

The SSM CSF shall support requests to terminate an M2M service session between AEs, between an AE and a CSE, or between CSEs. Before a request to terminate an M2M service session is granted, the SSM CSF shall first authenticate the requester using session credentials. The SSM CSF shall use the SEC CSF to support session authentication. Once authenticated, the SSM CSF shall terminate the M2M service session between the requesting and targeted session endpoints. This shall involve removal of session information on the session endpoints as well as the SSM CSF.

The SSM CSF shall support M2M service sessions that span multiple transit CSE hops. The SSM CSF shall leverage the SEC CSF on the same CSE as well the SSM CSF and the SEC CSF on transit CSEs to support multi-hop M2M service sessions. In doing so, SSM CSFs on different CSEs shall support coordinated M2M service session management. This includes bootstrapping of session credentials, session authentication and establishment, management of underlying network connections and services related to the session, management of session routing information, and session termination across multiple SSM CSFs.

Editor' Note: Security related descriptions for this CSF need to be updated.

Editor' Note: The session involves Mcn reference point or does it involve Mca and Mcc reference points as well. This is FFS.

Editor’s Note: This Annex shall be removed at the time of approval of TS-0001 Release-1.

.

## 6.3	Functional Entities Comprising Enablers

Figure 6.3-1 illustrates the architecture of the Common Services Entity (CSE). The CSE comprises of a set of Common Services Functions (CSFs) and a set of Enabler Functions (EFs).

- Figure 6.3-1: Common Services Entity Architecture

Figure 6.3-2 illustrates an example of CSFs and EFs within the Common Services Entity (CSE), showing newly added CSFs and associated CSF capabilities.

- Figure 6.3-2: Example of CSFs and EFs within Common Services Entity Architecture

The Services Extension Enabler enables the CSE to offer M2M Services over the Mca and Mcc reference points. The Services Extension Enabler provides the following functions:

- Check module authentication.

- Check node resources.

- Check interoperability with existing modules.

- Check policy and rights to determine how to handle conflicts e.g., do not register new module or deregister existing module, etc.

- Register new module.

- Add new service(s) due to new module to list of services.

- Modify API support to reflect new service capabilities.

- Modify inter-module communications to incorporate new module.

[6.3.a] Editor's Note: The above list is preliminary and is for FFS.

[6.3.b]  Editor's Note: How the Service Extension Enabler is associated with the APIs is FFS.

# Annex Z			(Informative) Bibliography

[Z.a]  Editor's Note: Need decision on Annex Z Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

- OR

<Publication>: "<Title>".

# History

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 27-FEB-2013 | Rapporteur Input - Draft Skeleton |
| V0.0.2 | 20-APR-2013 | Rapporteur Input -Draft; Including the following contributions: 1. oneM2M-ARC-2013-0255R03-Arcitecture_Description 2. oneM2M-ARC-2013-0256R01-Definitions_for_Arc_TS 3. oneM2M-ARC-2013-0253R03-Definition_to_SCX_and_SFM |
| V0.0.3 | 08-JUNE-2013 | 1. oneM2M-ARC-2013-0262R02-Update_to_Figure_5-1 |
| V0.0.4 | 28-JUNE-2013 | Includes the following contributions agreed at TP#05: 1. oneM2M-ARC-2013-0275R03-Revised_functional_architecture 2. oneM2M-ARC-2013-0280R03-Definitions_of_Reference_Points 3. oneM2M-ARC-2013-0281R02-M2M_Identifiers 4. oneM2M-ARC-2013-0285R05-Delivery_Handling_in_CSE 5. oneM2M-ARC-2013-0287R04-High_level_Application_Programming_ Interface |
| V0.0.5 | 26-JULY-2013 | Includes the following contributions agreed during ARC#28 and ARC#29 1. oneM2M-ARC-2013-0307R03-Refinement_proposal_for_reference _architecture 2. oneM2M-ARC-2013-0311R01-Definition_of_oneM2M_Node 3. oneM2M-ARC-2013-0312R01-Missing_definitions_for_WG1_work_progress |
| V0.0.6 | 23-AUGUST-2013 | Includes the following contributions agreed during ARC#30 (TP#06 F2F) meeting: 1. oneM2M-ARC-2013-0315R04-Proposal_for_definition_of_specific_Nodes 2. oneM2M-ARC-2013-0316R03-flow_message_extensions 3. oneM2M-ARC-2013-0317R02-Service_Enabler 4. oneM2M-ARC-2013-0340R04-General_mechanism_for_X_and_Y_reference_points 5. oneM2M-ARC-2013-0343R02-Multiple_domains 6. oneM2M-ARC-2013-0348R02-Session-ID 7. oneM2M-ARC-2013-0354R03-Definition_of_Common_Service_Entities_Collection 8. oneM2M-ARC-2013-0357R03-CSF_Descriptions 9. oneM2M-ARC-2013-0359R02-Introducing_Resource_Structure The following documents are included in "oneM2M-ARC-2013-0357R03-CSF_Descriptions": 1. oneM2M-ARC-2013-0302R02-Location_CSF 2. oneM2M-ARC-2013-0318R01-oneM2M-ARC-2013-0318-Data_Management_and_Repository_CSF 3. oneM2M-ARC-2013-0319R02-Network_Service_Exposure_Service_execution_and_triggering_CSF 4. oneM2M-ARC-2013-0322R01-Group_Management_CSF 5. oneM2M-ARC-2013-0325R01-A_CSF_description_for_registration 6. oneM2M-ARC-2013-0328R02-A_CSF_description_for_Discovery 7. oneM2M-ARC-2013-0329R02-Normative_Description_for_Subscription_Notification_Support_CSF 8. oneM2M-ARC-2013-0330R02-Device_management_CSF 9. oneM2M-ARC-2013-0331R01-descriptions_of_charging_CSF 10. oneM2M-ARC-2013-0334R02-description_of_session_management_CSF 11. oneM2M-ARC-2013-0335R02-CSF-ID 12. oneM2M-ARC-2013-0339R01-Service_charging_and_accounting_CSF 13. oneM2M-ARC-2013-0345R02-Security_related_CSFs |
| V0.0.7 | 26-AUGUST-2013 | Includes the following contributions agreed during ARC#31 meeting: 1. 1It was brought to attention that contribution # oneM2M-ARC-2013-0348R02-Session-ID had not been incorporated in v0.0.6 correctly. Corrected in clause 7.1.7 2. Review comments provided by Phil Jacobs via his email dated August 21, 2013 |
| V0.1.0 | 19-SEPT-2013 | Includes the following contributions agreed during ARC#6.2 conf call: 1. oneM2M-ARC-2013-0361R04-generic_call_flow_template 2. oneM2M-ARC-2013-0347R03-External-Identifier Includes the following contributions agreed during ARC#6.4 conf call: 1. oneM2M-ARC-2013-0364R08-Resource_template_and_example |
| V0.1.1 | 19-SEPT-2013 | Added Rapporteur comments over TS-0001 v0.1.0, with all changes accepted. |
| V0.1.2 | 01-OCT-2013 | Includes v0.1.0 plus the following comments: 1. Rapporteur comments over TS-001 v0.1.0 with all changes accepted. 2. Comments provided in "oneM2M-TS-0001-oneM2M-Functional-Architecture-V0 1 1_Sec9-5-Comments_JBv2". 3. Comments provided in "oneM2M-TS-0001-oneM2M-Functional-Architecture-V0.1.1pj". |
| V0.2.1 | 28-OCT-2013 | Includes the following contributions agreed during ARC#6.6 conf call: 1. oneM2M-ARC-2013-0366R04-Functional_Architecture_Clarifications 2. oneM2M-ARC-2013-0374R01-Management_CSF_Descriptions Includes the following contributions agreed during ARC#6.7 conf call: 1. oneM2M-ARC-2013-0320R02-Mapping_between_requirements_and_CSF 2. oneM2M-ARC-2013-0388R01-Renaming_reference_points Includes the following contributions agreed during ARC#7.0 F2F Meeting: 1. oneM2M-ARC-2013-0378R03-Reference_Points 2. oneM2M-ARC-2013-0391R02-CR_on_ARC_TS_clause_8_1 3. oneM2M-ARC-2013-0401R03-operation_effectivity_time 4. oneM2M-ARC-2013-0446R01-Update_to_M2M_information_related_to_M2M_Request-ID 5. oneM2M-ARC-2013-0433-Editorial_CR_on_clause_6_2_2_on_CMDH 6. oneM2M-ARC-2013-0435R04-CR_for_clause_9_6_addition_of_ resource_type_request 7. oneM2M-ARC-2013-0436R03-CR_for_clause_8_1_refinement_of_response_type 8. oneM2M-ARC-2013-0437R05-CR_for_clause_8_2_non_blocking_ procedures_to_access_resources 9. oneM2M-ARC-2013-0390R02-3gpp_interworking 10. oneM2M-ARC-2013-0456R03-default_information_to_unsuccessful_operation 11. oneM2M-ARC-2013-0467R03-CR_on_SUB_CSF_Description 12. oneM2M-ARC-2013-0408R01-Removing_AID_CSF 13. oneM2M-ARC-2013-0399-Device_Management_Function_Descriptions 14. oneM2M-ARC-2013-0400R02-Introduction_to_security_in_TS-0001 15. oneM2M-ARC-2013-0480R01-CR_for_clause_3_1_Definitions_and_6_2_3_DMR 16. oneM2M-MAS-2013-0096R04-DM_Server_Interaction 17. oneM2M-MAS-2013-0097R02-DM_Management_Resource_Lifecycle 18. oneM2M-ARC-2013-0448-CR_for_Revision_of_LOC_CSF 19. oneM2M-ARC-2013-0427R03-definition_proposal 20. oneM2M-ARC-2013-0434R03-CR_for_clause_9_6_addition_of_ resource_type_delivery 21. oneM2M-ARC-2013-0438R03-CR_for_clause_10_2_addition_of_CMDH_flows 22. oneM2M-ARC-2013-0392R04-CR_on_ARC_TS_clause_10_1 23. oneM2M-ARC-2013-0485-resource_type_parameter_in_clause_8_1 24. oneM2M-ARC-2013-0449R03-Resource_for_LOC_CSF 25. oneM2M-ARC-2013-0482-CR_for_Revision_of_LOC_CSF 26. oneM2M-ARC-2013-0406R03-M2M_AE_Addressing 27. oneM2M-ARC-2013-0484R01-Updates_to_the_Session_Management_CSF_texts 28. oneM2M-MAS-2013-0098R02-Updates_to_Management_CSF_Descriptions 29. oneM2M-ARC-2013-0487R03-Containers,_Content_instances_and_ Access_Rights_resources_and_flows 30. oneM2M-ARC-2013-0418R05-Section_for_Z_reference_point 31. oneM2M-ARC-2013-0428R02-Discovery_Call_Flow 32. oneM2M-ARC-2013-0419R02-group_resource 33. oneM2M-ARC-2013-0421R04-group_resource_message_flow 34. oneM2M-ARC-2013-0459R02-Virtual_Resource_Description 35. oneM2M-ARC-2013-0444R04-Update-to-Request-ID |
| V0.2.2 | 08-NOV-2013 | Includes Rapporteur Input and editorial corrections such as: 1. Updates Section 7.1 related to M2M Identifiers. 2. Highlights inconsistent use of the notation of pointy-brackets <..> in section 9.2. See the proposed Editor's Notes in section 9.2. 3. Editorial fixes and new editable pictures in section 9.6.x(s). 4. Restructure the "common attributes" Table in Section 9.6.1. Contributions agreed during ARC#7.2 conf call: 1. oneM2M-ARC-2013-0489-ASM_CSF_Architecture_TS_merge_corrections 2. oneM2M-ARC-2013-0490R01-DMG_CSF_Architecture_TS_merge_corrections Contributions agreed during ARC#7.3 conf call: 1. oneM2M-ARC-2013-0393R05-subscription_resource 2. oneM2M-ARC-2013-0504-ASM_CSF_Release_Package_Definition |
| V0.3.1 | 18-DEC-2013 | Includes Rapporteur Input and editorial corrections such as: 1. Clarification of text in section 6.2.9, 7.3, 8.3, 9.5.2.2 2. Update of some pictures and some text in Annex B (3GPP Interworking) Contributions agreed during ARC#7.4 conf call: 1. oneM2M-ARC-2013-0503R03-DMG_CSF_DM_Architecture_Diagram_Updates Contributions agreed during ARC#7.5 conf call: 1. oneM2M-ARC-2013-0505R01-'To'_parameter_clarifications Contributions agreed during ARC#8.0 F2F Meeting: 1. oneM2M-ARC-2013-0584R01-Registration_Clarification 2. oneM2M-ARC-2013-0579R02-Modification_of_abbreviation_NSE 3. oneM2M-ARC-2013-0540-CR_to_ARC_TS_adding_device_triggering_mechanism_in_section_7_3 4. oneM2M-ARC-2013-0549R04-mgmtObj_resource 5. oneM2M-ARC-2013-0550R01-mgmtObj-message-flow 6. oneM2M-MAS-2013-0135R04-mgmtCmd_resource 7. oneM2M-MAS-2013-0136R02-mgmtCmd_flows 8. oneM2M-ARC-2013-0452R04-NOTIFY_call_flow_template 9. oneM2M-ARC-2013-0539R01-Enhancement_on_container 10. oneM2M-ARC-2013-0537-Correction_to_figure_9_5_1_1-1 11. oneM2M-ARC-2013-0565R01-resources_to_establish_charging_policies 12. oneM2M-ARC-2013-0595-Renaming_the_searchStrings_common_attribute 13. oneM2M-ARC-2013-0538-Correction_to_figure_9_6_7 14. oneM2M-ARC-2013-0545-attribute-consistencyStrategy 15. oneM2M-ARC-2013-0546-members-as-virtual-resource 16. oneM2M-ARC-2013-0521R02-Reachability_schedule 17.oneM2M-ARC-2013-0535R01-Expiration_times 18. oneM2M-ARC-2013-0508R01-10_1_Figures_Update 19. oneM2M-ARC-2013-0506R02-Identifiers_Life_Cycle 20.oneM2M-ARC-2013-0548R01-members-message-flow 21. oneM2M-ARC-2013-0598R01-summary_tables 22. oneM2M-ARC-2013-0520R01-CR_AE_Addressing 23. oneM2M-ARC-2013-0555R03-General_Principles_of_Identification 24. oneM2M-ARC-2013-0519R04-Management_Adapter_Enhancements 25. oneM2M-ARC-2013-0574R05-nodeSpecific_Resource_Type 26. oneM2M--ARC-2013-0594R05-Application_Resource_Type 27. oneM2M-ARC-2013-0518-DMG_CSF_DM_Lifecycle_Updates 28. oneM2M-ARC-2013-0568R06-CSE_Resource_Description 29. oneM2M-ARC-2013-0509R04-Revision_of_figure_6-1 30. oneM2M-ARC-2013-0542R02-Inter-M2M_SP_Communication_references 31. oneM2M-ARC-2013-0599R03-M2M_Service_Subscription_Management 32. oneM2M-ARC-2013-0610R03-Naming_Convention 33. oneM2M-ARC-2013-0522R02-Supported_Resource_Type_Discovery_Procedure 34. oneM2M-ARC-2013-0536R01-Multi-protocol_Information_Elements 35. oneM2M-ARC-2013-0547R03-members-resource 36. oneM2M-ARC-2013-0511R03-CR_for_locationPolicy_Resource 37. oneM2M-ARC-2013-0570R01-Location_Management_Procedure 38. oneM2M-ARC-2013-0552R03-Big_Data_Integration 39. oneM2M-ARC-2013-0576R02-mod_resource_tables 40. oneM2M-ARC-2013-0556R03-Response_message_information |
|  |  | 41. oneM2M-ARC-2013-0593R04-Use_of_Links_in_to_relate_resources 42. oneM2M-ARC-2013-0577R02-Resource_Announcement 43. oneM2M-ARC-2013-0566R02-CR_subscription_resource_type 44. oneM2M-ARC-2013-0567R02-Subscription_call_flows 45. oneM2M-ARC-2013-0572R03-Notification_Call_Flows 46. oneM2M-ARC-2013-0607R01-Information_Flow 47. oneM2M-ARC-2013-0606R03-Inter-M2M_SP_Communication 48. oneM2M-ARC-2013-0541R01-Resources_Extensibility 49. oneM2M-ARC-2013-0608R03-Functional_Arch_Update 50. oneM2M-ARC-2013-0611R03-Integrated_structured-flat_URI 51. oneM2M-ARC-2013-0558R03-Additional_attributes_for_group_resource |
| V0.3.2 | xx-JAN-2014 | Review comments from WG2 Members: 1. Addition of parentID attribute to several resources. 2. Changed picture in section 10.1.1-1 3. Removed short names for attributes 4. Editorial fixes, such as section re-numbering, cross reference fixes etc. 5. Change of Figure 9.6.12-1, section 7.1.7, |
| V0.4.0 | February 2014 | Clean-up done by editHelp! e-mail: mailto:edithelp@etsi.org |
| V0.4.1 & V0.4.2 | 04-March 2014 | Contributions agreed during ARC#8.2 conf call: 1. ARC-2014-0620R01-Addition_of_NOTIFY_to_clause_8 2. ARC-2014-0623R02-Clarification_to_base_URI Contributions agreed during ARC#8.3 conf call and assigned for email approval: 1. ARC-2014-0621R02-Deletion_of_lifespan_parameter 2. oneM2M-ARC-2013-0617R02-oneM2M_and_3GPP2_ Underlying_Network_Interworking Contributions agreed during ARC#8.4 conf call: 1. ARC-2014-0031R04-Addition_of_Registration_Table Contributions agreed during ARC#8.5 conf call: 1. ARC-2014-0038R01-Correction_of_section_8_1_2_1_and_8_1_5 Contributions agreed during ARC#9.0 F2F Meeting: 1. ARC-2014-0101R02-Addressing_a_Resource 2. ARC-2014-0136R01-CR_for_node_resource_type 3. ARC-2014-0123R01-pointOfAccess_attribute_for_non-CSE_Capable_AEs 4. ARC-2014-0062R01-CR_remove_variable_part_on_attributes 5. ARC-2014-0049R02-App-Registration 6. ARC-2014-0091R02-CSE-Registration 7. ARC-2014-0092R03-Update-CREATE-CSE_Registration 8. ARC-2014-0046-Device_Management_Editorial_Updates 9. MAS-2014-0013R04-Management_Server_Interaction_Updates 10. ARC-2014-0163R04-Annex_for_management_resources 11. ARC-2014-0164R01-Change_to_moAttribute 12. ARC-2014-0165R02-Definition_of_mgmtDefinition 13. ARC-2014-0177R02-deviceInfo,_batteryMgmt,_deviceCapability,_ capabilityInstance_Resource 14. ARC-2014-0096-Naming_and_Rules_for_Announcing_Resources 15. ARC-2014-0145-Child_Resource_Announcement 16. ARC-2014-0170R02-group_procedure_clarification 17. ARC-2014-0162-Aggregation_of_notification 18. ARC-2014-1170R01-New_Annex_Integration_of_nononeM2M_ native_system_and_protocols 19. ARC-2014-0083R02-pollingChannel_resource_type 20. ARC-2014-0158R03-call_flows_for_pollingChannel_resource_type 21. ARC-2014-0061R03-CR_for_Location_Managment_Procedure 22. ARC-2014-0133R01-CR_for_locationPolicy_Resource 23. ARC-2014-1183-Relationship_between_M2M-Subscription_and_CMDH_Policies 24. ARC-2014-0174R02-Resource_Type_for_CMDH_Policy 25. ARC-2014-0100R05-Resource_Type_For_CMDH_Default_Parameters 26. ARC-2014-0103R04-Resource_Type_For_CMDH_Parameter_Limits 27. ARC-2014-0112R02-Resource_Type_For_CMDH_Network_Usage 28. ARC-2014-0116R01-Resource_Type_For_CMDH_Buffer_Usage 29. ARC-2014-0124R01-Underlying_Network_Identifier 30. ARC-2014-0141R04-charging_CSF_updates 31. ARC-2014-0143R02-service_charging_definitions 32. ARC-2014-0134-Manaing_the_description_that_dose_not_belong_to_ the_1st_release 33. ARC-2014-0186R03-Common_Attribute_Modification_Tag |
|  |  | 34. ARC-2014-0624R04-Resource_Specification_Refinements 35. ARC-2014-0128R02-Addition_of_resource_summary_table 36. ARC-2014-0135R01-Procedure_for_node_resource_type 37. ARC-2014-0073R06-Clarifications_to_Registration_CSF_clause 38. ARC-2014-0130R02-M2M_Identifiers_lifecycle_and_characteristics 39. ARC-2014-0189R01-Access_Right_Link_Type 40. ARC-2014-0078R06-General_Procedure_for_Device_Triggering 41. ARC-2014-0077R03-3GPP_Interworking_Procedure 42. ARC-2014-1186-Trigger_Recipient_ID_Definition_CR 43. ARC-2014-1180R03-Security_alignments_on_M2M_identifiers 44. ARC-2014-1174R08-Description_of_security_procedures 45. ARC-2014-0099R03-Charing_Architecture 46. ARC-2014-1177R02-updates_of_SCA_resources 47. ARC-2014-0142R03-SCA_CSF_call_flows 48. ARC-2014-0021R01-AE_re-targeting 49. ARC-2014-0032R04-baseRoot_Resource_Type 50. ARC-2014-0087R03-filterCriteria_description 51. ARC-2014-0167R03-M2M_Service_Subscription_Resource 52. ARC-2014-0089R03-Access_right_privileges 53. ARC-2014-0169R01-M2M_Service_Subscription_Resource_Type_Procedure 54. ARC-2014-1179R03-Application_Port_ID 55. ARC-2014-0146R03-Resource_Announcement_Procedures 56. ARC-2014-0160R02-CSE_minimum_provisioning 57. ARC-2014-0152R01-Notification_Indicator 58. ARC-2014-0151R02-Addition_of_Notification_Action_in_Unreachability 59. ARC-2014-0180R01-Device_Identifier 60. ARC-2014-0166R03-Improvement_to_subscription 61. ARC-2014-0147R01-Annouce-able_Resource 62. ARC-2014-0108R03-Inter-M2M_Communication_Procedures 63. ARC-2014-0192R01-Hosting_Node_Link_Type 64. ARC-2014-0193R01-Hosted_CSE_Link_Type 65. ARC-2014-0048R03-Update-CREATE-Registration 66. ARC-2014-1182R01-Supporting_ADN_case_for_node_resource_type 67. ARC-2014-0168R02-URI_types_and_their_relationships 68. ARC-2014-0127R02-Some_Enhancements_for_Supporting_Device_Triggering |
| V0.4.3 | 27-MARCH-2014 | Contributions agreed during ARC#9.4 conf call: 1. ARC-2014-1246R01-TS-0001_EditorsNoteReview |
| V0.5.0 | 22-APRIL-2014 | Contributions agreed during ARC#9.2 conf call: 1. ARC-2014-1205R01-Added-parentID Contributions for Email approval at ARC#9.2 conf call: 1. ARC-2014-0047R08-Section-8_1-Refinement 2. ARC-2014-1206R02-Parent-child-issue-resolution Contributions agreed during ARC#9.3 conf call: 1. ARC-2014-1210R02-Cleanup_of_TS-0001_section_8_1 2. Change all occurences of Section/section with Clause/clause Contributions agreed during ARC#9.4 conf call: 1. ARC-2014-1203R01-Removal_attribute_acronyms Contributions agreed during TP#10 meeting: 1. ARC-2014-0027R02-Definition_cleanup 2. ARC-2014-0028R07-NodeLess-AE_renamed 3. ARC-2014-0178R05-Attribute_consistency 4. ARC-2014-1198R03-Update_table_9_6-1 5. ARC-2014-1221R01-expirationTime_alignment 6. ARC-2014-1225R01-application_terminology_cleanup 7. ARC-2014-1226-m2m_application_service_cleanup 8. ARC-2014-1227R01-application_resource_type_rename_cleanup 9. ARC-2014-1228R01-AE_CSE_definition 10. ARC-2014-1229R02-instance_notion_cleanup 11. ARC-2014-1230R01-app-inst-id_cleanup 12. ARC-2014-1231R04-Location_Request 13. ARC-2014-1233R04-Correction_of_node_resource 14. ARC-2014-1234R05-Clarification_on_M2M_Identifiers_used_for_device_triggering 15. ARC-2014-1235R02-Rename_the_attribute_of_PoA 16. ARC-2014-1239R01-Cleanup_of_Subscription_Procedure 17. ARC-2014-1247R01-Resource_Refinements_for_Subscription 18. ARC-2014-1249R01-Local_CSE_cleanup 19. ARC-2014-1250R01-Hosting_CSE_definition_modification 20. ARC-2014-1253R01-virtual_attribute_description 21. ARC-2014-1256R01-Clarify_Optional_Resources 22. ARC-2014-1262R01-VersionTag_cleanup 23. ARC-2014-1263R01-cn-optional 24. ARC-2014-1264R01-modification_to_mgmtObj_procedure 25. ARC-2014-1265R01-modify-deviceCapability 26. ARC-2014-1266R01-remove-parameters-resource 27. ARC-2014-1267R01-solve-editor-note-for-subscription 28. ARC-2014-1268R02-managements_to_nodes 29. ARC-2014-1269R02-state-machine-for-sw-mgmt 30. ARC-2014-1270R01-notificationCongestionPolicy 31. ARC-2014-1272R02-Resource_Announcement_Cleanup_Part_1 32. ARC-2014-1273R01-Identifiers_cleanup 33. ARC-2014-1274R02-Resource_Announcement_Cleanup_Part_2 34. ARC-2014-1276R03-Resource_Announcement_Cleanup_Part_4 35. ARC-2014-1277R02-mgmtCmd_CR |
|  |  | 36. ARC-2014-1278R03-Conversion_of_CMDH_Policies_to_mgmtObj 37. ARC-2014-1282R01-SCA_resource_description_clean_up 38. ARC-2014-1283R01-SCA_Call_Flow_Clean_Up 39. ARC-2014-1284R01-update_CSEbase_with_SCA_resources 40. ARC-2014-1292R02-Request_Parameter_Review 41. ARC-2014-1293R03-Routing_Clarifications 42. ARC-2014-1295R03-DNS-Provisioning_for_Inter_M2M_SP_Communication 43. ARC-2014-1296R03-Guidelines_for_Resource_Addressing 44. ARC-2014-1302R01-Security_Consideration_of_Updating_accessControlPolicyID 45. ARC-2014-1309R02-Cleanup_clause_9_6_12 46. ARC-2014-1310-Correction_of_CMDH_procedures 47. ARC-2014-1316R02-Unifying_Successful,_Unsuccessful_and_Ack_Responses 48. MAS-2014-0365R04-App-ID_Additional_Clarification_Text |
| V0.5.1 | 28-APRIL-2014 | 1. Includes gramatical and consistency clean-up provided in ARC-2014-1337R01. 2. Rapporteur roll-back of change of "appData" to "instance". 3. Rapportuer change of "access rights" to <access control policy". |
| V0.6.0 | 30-APRIL-2014 | Contributions agreed during ARC#10.1 conf call: 1. ARC-2014-1211R03-Cleanup_of_section_10_1 2. ARC-2014-1275R02-Resource_Announcement_Cleanup_Part_3 Contributions agreed during ARC#10.bis (AdHoC) on April 28, 2014: 1. ARC-2014-1334R01-Cleanup_of_intro_to_CSF_clause 2. ARC-2014-1335R01-DMR_CSF_cleanup 3. ARC-2014-1298R06-CR_for_Discovery_CSF 4. ARC-2014-1358R01-GMG_CSF_cleanup 5. ARC-2014-1336R01-NSSE_CSF_cleanup 6. ARC-2014-1349R01-Resolution_of_ed_note_3_1_g 7. ARC-2014-1350R01-Resolution_of_editors_notes_in_6-1 Contributions agreed during ARC#10.bis (AdHoC) on April 29, 2014: 1. ARC-2014-1326R02-CMDH_CSF_cleanup 2. ARC-2014-1362R01-GMG_Clean-up_proposal 3. ARC-2014-1356R02-SEC_CSF_cleanup 4. ARC-2014-1348R01-SCA_CSF_clean_up 5. ARC-2014-1343R01-SUB_CSF_cleanup 6. ARC-2014-1342R01-Removal_of_clause_6_3 7. ARC-2014-1339R02-DMG_CSF_cleanup 8. ARC-2014-1338R02-ASM_CSF_cleanup |
| V0.6.1 | 05-MAY-2014 | Contributions agreed during ARC#10 conf call and then email approval: 1. ARC-2014-1202-Removal_section_9_5_2 Contributions accepted during ARC#10.bis (AdHoC) on April 30, 2014: 1. ARC-2014-1351R01-Resolution_of_Editorâ€™s_notes_in_clause_9_1 2. ARC-2014-0030R01-Section_5_2-cleanup3. ARC-2014-1244R02-Resource_Template_Refinements 4. ARC-2014-1341R01-Editors_notes_instance_resource 5. Rename <instance> resource type to <contentInstance> resource type 6. Rename accessControlPolicies attribute to accessControlPolicyIDs attribute Contributions accepted during ARC#10.bis (AdHoC) on May 01, 2014: 1. ARC-2014-1363R01-LOC_CSF_cleanup 2. ARC-2014-1367R02-RC_Parameter_cleanup 3. ARC-2014-1353R01-Removal_EN_on_delivery_resource 4. ARC-2014-1354R01-Resolving_deletion_of_request_resource 5. ARC-2014-1359R03-group_procedure_cleanup 6. ARC-2014-1352-Missing_agreed_changes_and_adjustment_of_terms 7. ARC-2014-1299R07-CR_on_Registration_CSF 8. Restore changes to clause 6.4.1 and remove clause 6.4.4 9. Rename channelHandle attribute to longPollingURI attribute Contributions accepted during ARC#10.bis (AdHoC) on May 02, 2014 1. ARC-2014-1370R01-Management_of_security_capabilities |