| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0001 - V-0.3.2 |
| Document Name: | oneM2M Functional Architecture |
| Date: | 2014-JANUARY-07 |
| Abstract: | This document specifies the functional architecture for oneM2M Services Platform. This version of the document provides corrections based on review comments for v-0.3.1. Includes some editorial corrections and Rapporteur Input as well over v-0.3.1. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	8

2	References	8

2.1	Normative references	8

2.2	Informative references	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	10

3.3	Abbreviations	10

4	Conventions	11

5	Architecture Model	11

5.1	General Concepts	11

5.2	Architecture Reference Model	12

5.2.1	Functional Architecture	12

5.2.2	Reference Points	13

5.2.2.1	Mca Reference Point	13

5.2.2.2	Mcc Reference Point	13

5.2.2.3	Mcn Reference Point	13

5.2.2.4	Mcc' Reference Point	14

6	Configurations supported by oneM2M Architecture	14

6.2	Common Service Functions	16

6.2.1	 Application and Service Layer Management	17

6.2.1.1	General Concepts	17

6.2.1.2	Detailed Descriptions	17

6.2.1.2.1	Configuration Function	18

6.2.1.2.2	Diagnostic and Monitoring Function	18

6.2.1.2.2.1	CSE Diagnostics	18

6.2.1.2.2.2	CSE Monitoring	18

6.2.1.2.3	Software Management Function	18

6.2.2	Communication Management and Delivery Handling	19

6.2.2.1 	General Concepts	19

6.2.2.2	Detailed Descriptions	19

6.2.3	Data Management and Repository	20

6.2.3.1	General Concepts	20

6.2.3.2	Detailed Descriptions	20

6.2.4	Device Management	20

6.2.4.1	General Concepts	20

6.2.4.1.1	Device Management Architecture	21

6.2.4.1.2	Device Management Server Interaction	22

6.2.4.1.2.1	Overview	22

6.2.4.1.2.2	Device Management Server – Access Permissions	23

6.2.4.1.3	Device Management Client Interaction	23

6.2.4.1.3.1	Overview	23

6.2.4.1.4	Device Management Resource Lifecycle	24

6.2.4.1.4.1	Overview	24

6.2.4.1.4.2	Overview	24

6.2.4.1.4.3	Procedures for Creation, Update and Deletion of Device Management Resources	24

6.2.4.2	Detailed Descriptions	25

6.2.4.2.1	Device Configuration Function	25

6.2.4.2.2	Device Diagnostics and Monitoring Function	26

6.2.4.2.3	Device Firmware Management Function	26

6.2.4.2.4	Device Topology Management Function	26

6.2.5	Discovery	26

6.2.5.1	General Concepts	26

6.2.5.2	Detailed Descriptions	27

6.2.6	Group Management	27

6.2.6.1	General Concepts	27

6.2.6.2	Detailed Descriptions	27

6.2.7	Location	28

6.2.7.1	General Concepts	28

6.2.7.2	Detailed Descriptions	28

6.2.8	Network Service Exposure, Service Execution and Triggering	28

6.2.8.1	General Concepts	28

6.2.8.2	Detailed Descriptions	29

6.2.9	Registration	29

6.2.9.1	General Concepts	29

6.2.9.2	Detailed Descriptions	29

6.2.10	Security	30

6.2.10.1	General Concepts	30

6.2.10.2	Detailed Descriptions	30

6.2.11	Service Charging and Accounting	31

6.2.11.1	General Concepts	31

6.2.11.2	Detailed Descriptions	32

6.2.12	Service Session Management	32

6.2.12.1	General Concepts	32

6.2.12.2	Detailed Descriptions	33

6.2.13	Subscription and Notification	34

6.2.13.1	General Concepts	34

6.2.13.2	Detailed Descriptions	34

6.3	Functional Entities Comprising Enablers	34

6.4	Security Aspects	36

6.5	Other Aspects	37

6.5.1	Inter M2M SP communication for oneM2M compliant nodes	37

6.5.1.1	Public Domain Names and CSEs	37

7	M2M Identification and Addressing	38

7.1	M2M Identifiers	38

7.1.1	M2M Service Provider Identifier (M2M-SP-ID)	38

7.1.2	Application Instance Identifier (App-Inst-ID)	38

7.1.3	Application Identifier (App-ID)	39

7.1.4	CSE Identifier (CSE-ID)	39

7.1.5	M2M Node Identifier (M2M-Node-ID)	39

7.1.6	M2M Service Subscription Identifier (M2M-Sub-ID)	39

7.1.7	M2M Request Identifier (M2M-Request-ID)	39

7.1.8	M2M External Identifier (M2M-Ext-ID)	40

7.2	M2M Identifiers lifecycle and characteristics	40

7.3	Addressing an Application Entity	41

7.3.1	Application Entity Addressing	41

7.3.2	Application Entity Reachability	42

7.3.2.1	CSE Point of Access (CSE-PoA)	42

7.3.2.2	 Locating Application Entities	42

7.3.2.3	Usage of CSE-PoA by the M2M System	42

7.3.2.3.1	CSE-PoA related to CSEs associated with a Fixed Network	42

7.3.2.3.2	CSE-PoA related to CSEs associated with Mobile Networks	42

7.3.2.3.3	CSE-PoA to CSEs associated with multiple Underlying Networks	43

8	Description and Flows of Reference Points	43

8.1	General Communication Flow Scheme on Mca and Mcc Reference Points	43

8.1.1	Description	44

8.1.2	Request	44

8.1.2.1	Summary of Request Message Parameters	47

8.1.3	Successful Operation	48

8.1.4	Unsuccessful Operation	49

8.1.5	Summary of Response Message Parameters	50

8.2	Procedures for Accessing Resources	50

8.2.1	Generating Responses	51

8.2.2	Accessing Resources in CSEs – Blocking Requests	51

8.2.3	Accessing Resources in CSEs – Non-Blocking Requests	56

8.2.3.1	Response with Reference to Result of Requested Operation	56

8.2.3.2	Synchronous Case	56

8.2.3.3	Asynchronous Case	59

8.3.	Description and Flows on Mcn Reference Point	60

8.3.1	General Description	60

9	Resource Management	60

9.1	General Principles	60

9.2	Resource Types	61

9.2.1	Types of Virtual Resources	63

9.3	Resource Addressing	63

9.4	Resource Structure	64

9.4.1	Links between resources	64

9.5	Resource Specification Guidelines	64

9.5.1	Resource Template	66

9.5.1.1	Resource Type resourceType	66

9.5.2	Examples for Resource Specification	67

9.5.2.1	Example of a Resource with attributes only	67

9.5.2.1.1	Resource Type accessRight	67

9.5.2.2	Example of a Resource with attributes and child resources	68

9.5.2.2.1	Resource Type container	68

9.5.3	Handling of Unsupported and non compliant Resources	70

9.6	Specified Resource Types	70

9.6.1	Common Attributes	70

9.6.2	Resource Type accessRight	72

9.6.3	Resource Type container	73

9.6.4	Resource Type delivery	75

9.6.5	Resource Type group	77

9.6.6	Resource Type instance	79

9.6.7	 Resource Type locationPolicy	80

9.6.8	Resource Type request	82

9.6.9	Resource Type subscription	84

9.6.10	Resource Type mgmtObj	86

9.6.11	Resource Type parameters	89

9.6.12	Resource Type mgmtCmd	90

9.6.13	Resource Type execInstances	91

9.6.14	Resource Type execInstance	92

9.6.15	Resources for Service Charging and Accounting	93

9.6.15.1	Resource Type chargingPolicy	93

9.6.15.2	Resource Type chargeableEvent	95

9.6.15.3	Resource Type chargingRule	96

9.6.16	Resource Type schedule	97

9.6.17	Resource Type node	98

9.6.18	Resource Type application	100

9.6.19	Resource Type remoteCSE	101

9.6.20	Resource Type m2mServiceSubscription	103

9.6.21	Resource Type Members	104

9.6.22	Resource Announcement	105

9.6.23	Resource Type xxxx	105

10	Information Flows	106

10.1	Basic Procedures	106

10.1.1	CREATE (C)	106

10.1.2	RETRIEVE (R)	107

10.1.3	UPDATE (U)	108

10.1.4	DELETE (D)	109

10.1.5	NOTIFY (N)	110

10.2	Resource Specific Procedures	111

10.2.1	<container> Resource	111

10.2.1.1	Create <container>	111

10.2.1.2	Retrieve <container>	112

10.2.1.3	Update <container>	112

10.2.1.4	Delete <container>	113

10.2.2	Access to Remotely Hosted Resources via CMDH CSF	114

10.2.2.1	Introduction to use of <delivery> related flows	114

10.2.2.2	Create <delivery>	116

10.2.2.3	Retrieve <delivery>	117

10.2.2.4	Update <delivery>	118

10.2.2.5	Delete <delivery>	119

10.2.3	Resource Discovery Procedure	119

10.2.3.1	Introduction	119

10.2.3.2	 Discovery procedure via Retrieve Operation	120

10.2.4	Group Management Procedures	121

10.2.4.1	Introduction	121

10.2.4.2	Create <group>	121

10.2.4.3	Retrieve <group>	122

10.2.4.4	Update <group>	122

10.2.4.5	Delete <group>	123

10.2.4.6	Create <Members>	124

10.2.4.7	Retrieve <Members>	126

10.2.4.8	Update <Members>	127

10.2.4.9	Delete <Members>	128

10.2.5	mgmtObj Management Procedures	129

10.2.5.1	Introduction	129

10.2.5.2	Create <mgmtObj>	129

10.2.5.3	Retrieve <mgmtObj>	131

10.2.5.4	Update <mgmtObj>	132

10.2.5.5	Delete <mgmtObj>	133

10.2.5.6	Execute <mgmtObj>	133

10.2.6	External Management Operations through <mgmtCmd>	134

10.2.6.1	Introduction	134

10.2.6.2	Create <mgmtCmd>	135

10.2.6.3	Retrieve <mgmtCmd>	136

10.2.6.4	Update <mgmtCmd>	136

10.2.6.5	Delete <mgmtCmd>	137

10.2.6.6	Execute <mgmtCmd>	139

10.2.6.7	Cancel <execInstance>	140

10.2.6.8	Retrieve <execInstance>	141

10.2.6.9	Delete <execInstance>	142

10.2.7	Supported Attribute Discovery Procedure	143

10.2.7.1	Introduction	143

10.2.7.2	Supported Attributes Discovery procedure via Retrieve Operation	143

10.2.8	Location Management Procedures	144

10.2.8.1	Introduction	144

10.2.8.2	Create <locationPolicy>	144

10.2.8.3	Retrieve <locationPolicy>	145

10.2.8.4	Update <locationPolicy>	146

10.2.8.5	Delete <locationPolicy>	146

10.2.9	<subscription> Resource Procedures	147

10.2.9.1	 Introduction	147

10.2.9.2	 Create <subscription>	147

10.2.9.3	Retrieve <subscription>	148

10.2.9.4	Update <subscription>	149

10.2.9.5	Delete <subscription>	149

10.2.10	Notification Procedures for Resource Subscription	150

11	<tbd>	151

Proforma copyright release text block	152

Annex <A>: (Informative): Deployment Scenarios	153

ANNEX B: (Informative) oneM2M System and 3GPP MTC Release-11 Underlying Network Interworking	153

B.1	3GPP MTC Release-11 Underlying Network Introduction	153

B.2	3GPP Release-11 MTC functionality	153

B.3	ASN-CSE initiated connectivity establishment	155

B.3.1	Use of DHCP and DNS	155

B.3.2	Pre-configuration	155

B.4	Serving IN-CSE initiated connectivity establishment	156

B.5	Idle mode Management	156

B.6	Procedure	156

B.6.1	General	156

B.6.1.1	 ASN-CSE initiated connectivity establishment procedure	156

B.6.1.2	Serving IN-CSE initiated connectivity establishment procedure	158

Annex <X>: (Informative):	Mapping of Requirements with CFSs	159

Annex <Y>:	(Informative/Normative): Remove Informative or Normative as appropriateTitle of annex (style H9)	163

Y.1	First clause of the annex (style H1)	163

Y.1.1	First subdivided clause of the annex (style H2)	163

Annex <Z>:	 Bibliography	164

History	165

# 1	Scope

This specification describes the end-to-end oneM2M functional architecture, including the description of the functional entities and associated reference points.

oneM2M functional architecture has focus on the Service Layer aspects and takes Underlying Network-independent view of the end-to-end services. The Underlying Network is used for the transport of data and potentially for other services.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1		Normative references

The following referenced documents are necessary for the application of the present document.

[1]	 ETSI TR 102 473: "<Title>".

## 2.2		Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	OMA-ER-DMClientAPIfw-V1_0, “DM Client Side API Framework (DMClientAPIfw)”

[i.3]	oneM2M TR-0008, "Security Analysis: ..."

[i.4]	IETF RFC 3986, "Uniform Resource Identifier (URI): General Syntax"

[i.5] 	IETF RFC 1035, “Domain names – Implementation and specification”

[i.6]	IETF RFC 3596, “DNS Extensions to Support IP Version 6”

[i.7]	GSMA-IR.67, "DNS/ENU Guidelines for Service Providers & GRX/IPX Providers"

[i.8]	IETF RFC 2929, "DNS IANA Considerations"

[i.9]	IETF RFC 6874, “Representing IPV6 Zone Identifiers in Address Literals and Uniform Resources Identifiers"

[i.10]	IETF RFC 6895, "DNS IANA Considerations"

[i.11]	3GPP TS 23.682, "Architecture Enhancements to facilitate communications with packet data networks and applications"

[i.12]	TR-0069, " ..."

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Application Layer:  This layer comprises oneM2M Applications and related business and operational logic.

Common Services Layer:  This layer consists of oneM2M service functions that enable oneM2M Applications (via management, discovery and policy enforcement to name a few).

Common Services Entity:  The Common Services Entity is an instantiation of Common Services Functions (CSFs). Common Services Entity provides a subset of CSFs that could be used and shared by M2M Applications. Common Services Entity can utilize Underlying Network capabilities and can interact with each other to fulfil the service.

Editor’s Note: This is a provisional definition.

Common Services Function: A Common Services Function (CSF) is a set of service functions that are common to the M2M environment and specified by oneM2M. A CSE contains one or more CSFs.

Editor's Note: This definition needs some more work so that it is more clear (about the M2M environment and the relationship between CSE/CSF and level of service functions.

Execution Environment:  An Execution Environment is a logical entity that represents an environment capable of running some software modules

Hosting CSE: The CSE where the addressed master/original resource resides.

Local CSE: The CSE where an Application or a CSE has registered. It is the first CSE that receives Request from an Originator. For example:

- If an Application on an Infrastructure Node is the Originator, the Local CSE is the CSE on the Infrastructure Node;

- If an Application on a Middle Node is the Originator, the Local CSE is the CSE on the Middle Node;

- If an Application on an End Node is the Originator, the Local CSE is the CSE on the End Node.

Editor's Note: Need to provide definition of an End Note. This is FFS.

Editor's Note: The concept of a CSE registering at a Local CSE needs to be understood. This is FFS.

Editor's Note: Review/clarify "or a CSE" in the definition of Local CSE.  Should "or a CSE" be replaced with "or a Node"?  This is FFS.

Editor's Note: Need definition for "Application Entity". Need to define relationship between an Application Entity and an Application.

M2M Service Provider Domain: It is the part of the M2M System that is associated with a given M2M Service Provider.

Managed Entity: M2M Device, M2M Gateway, devices in the M2M Area Network as well as M2M Applications and M2M service layer software components.

Network Services Layer:  Provides transport, connectivity and service functions.

Node:  A functional entity containing one of the following:

- one or more M2M Applications,

- one CSE and zero or more M2M Applications.

Originator: The actor that initiates a Request. An Originator can either be an Application or a CSE.

Receiver: The actor that receives the Request. A Receiver can be a CSE or an Application.

Resource: A uniquely addressable entity in oneM2M System such as by the use of a Universal Resource Identifier (URI). A resource can be accessed and manipulated by using the specified procedures.

Editor’s Note: The above stated definition of Resource needs to be revisited. This is FFS.

Editor’s Note: It is expected that more definitions are needed. This is FFS.

Software Package:  An entity that can be deployed on the Execution Environment.  It can consist of entities such as software modules, configuration files, or other entities.

Structured Data: Data that either have a structure according to a specified Information Model or are otherwise organized in a defined manner.

## 3.2	Symbols

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

ADN	Application Dedicated Node

ADN-AE	AE which resides in the Application Dedicated Node

AE	Application Entity

ASN	Application Service Node

ASE-AE	Application Entity that is registered with the CSE in the Application Service Node

ASN-CSE	CSE which resides in the Application Service Node

CSE	Common Service Entity

CSF	Common Service Function

EF	Enabler Function

IN	Infrastructure Node

IN-AE	Application Entity that is registered with the CSE in the Infrastructure Node

IN-CSE	CSE which resides in the Infrastructure Node

JNI	Java Native Interface

MN	Middle Node

MN-CSE	CSE which resides in the Middle Node

NSE	Network Service Entity

The following Common Service Functions (CSFs) have been defined:

AID CSF	Addressing and Identification CSF

ASM	Application and Service Layer Management CSF

CMDH CSF	Communication Management and Delivery Handling CSF

DMR CSF	Data Management and Repository CSF

DMG CSF	Device Management CSF

DIS CSG	Discovery CSF

GMG CSF	Group Management CSF

LOC CSF	Location CSF

NSSE CSF	Network Service Exposure, Service Execution and Triggering CSF

REG CSF	Registration CSF

SEC CSF	Security CSF

SCA CSF	Service Charging and Accounting CSF

SSM CSF	Service Session Management CSF

SUB CSF	Subscription and Notification CSF

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

Editor’s Note: The meaning of URI (Uniform Resource Indicator) as used in this stage 2 Arch TS should be stated here since it may be constrained, extended or otherwise diverge from the very well known Generic Syntax of RFC3986. For example considering the following:

- What is the difference between Request(op=”CREATE”,to=” HYPERLINK "http://thebaseofmycse/?view=default&update=10s#NEW", http://theBaseOfmyCSE?view=default&update=10s#NEW ”, ty=”container”…) and Request(op=”CREATE”, to=”//theBaseOfmyCSE”, ty=”container”,…) ? 
In both cases the ‘to’ parameter uses an URI. We have some constraints to define here.

- What if some resource is hosted by a CSE that is reachable in public internet through ‘normal’ IP technology (FQDN may be resolved into a public IP address), but another resource may reside on a CSE that has no IP connectivity and in fact needs to be reached through some specific gateway that offers a conversion into anther transport and node addressing scheme (e.g. KNX or just some non-IP transport). In that case we may decide to use the {scheme} part of a URI to indicate that. The first resource may use “to=oneM2MpublicIP://{FQDN of the target CSE}/xxx” and the second one may use “to=oneM2MprivateKNX://{FQDN of KNX Gateway/{KNX-address-of-target-CSE}/xxx” 
These examples also using URIs but the “scheme” part now refers to how to interpret the segments of the path. One could also think of allowing some hierarchy in the “scheme” field.

- Even in IP-only scenarios, one could have a private network A with a number of CSEs and a private network B with a number of CSE. If you need to address a resource of Network B within a request made in Network A, how would that look? Can we do that with a URI? Probably we can, but we would have to define some special interpretation of a URI. Or we would define some extension like a AE in network A makes a request with to=networkB::oneM2MprivateIP://{IP-address-of-target-CSE-in-Network-B}/container. In that case the receiver of the request only needs to know how to reach a gateway to network B (a CSE that is connected to Network A and Network B).

# 5	Architecture Model

Editor's Note: Definitions are FFS. As definitions get refined and agreed to, the following text to be adjusted to eflect the agreements

## 5.1	General Concepts

Figure 5.1-1 depicts the oneM2M Layered Model for supporting end-to-end (E2E) M2M Services. This layered model comprises three layers: Application Layer, Common Services Layer and the Underlying Network Services Layer.

Figure 5.1-1: oneM2M Layered Model

Figure 5.1-1: oneM2M Layered Model

5.2	Architecture Reference Model

### 5.2.1	Functional Architecture

Figure 5.2.1-1illustrates oneM2M functional architecture.

Figure 5.2.1-1: oneM2M Functional Architecture

Figure 5.2.1-1: oneM2M Functional Architecture

The oneM2M functional architecture in Figure 5.2.1-1 comprises of the following functions:

1.	Application Entity (AE): Application Entity provides Application logic for the end-to-end M2M solutions. Examples of the Application Entities can be fleet tracking application, remote blood sugar monitoring application, or remote power metering and controlling application.

2.	Common Services Entity (CSE):  A Common Services Entity comprises the set of "service functions" that are common to the M2M environments and specified by oneM2M. Such service functions are exposed to other entities through Reference Points Mca and Mcc. Reference point Mcn is used for accessing Underlying Network Service Entities.

Examples of service functions offered by CSE are: Data Management, Device Management, M2M Subscription Management, Location Services etc. Such "subfunctions" offered by a CSE may be logically apprehended as Common Services Functions (CSFs). Inside a Common Services Entity (CSE), some of the CSFs can be mandatory and others can be optional. Also, inside a CSF, some subfunctions can be mandatory or optional (e.g., inside a "Device Management" CSF, some of the subfunctions like "Application Software Installation", "Firmware Updates", "Logging", "Monitoring", etc. can be mandatory or optional).

3.	Underlying Network Services Entity (NSE):  An Underlying Network Services Entity provides services to the CSEs. Examples of such services include device management, location services and device triggering. No particular organization of the NSEs is assumed.

NOTE: Underlying Networks provide data transport services between entities in the oneM2M System. Such data transport services are not included in the NSE.

### 5.2.2	Reference Points

The following reference points are supported by the Common Service Entity (CSE). The "Mc(-) nomenclature is based on the mnemonic "M2M communications".

#### 5.2.2.1	Mca Reference Point

The Mca reference point designates communication flows between an Application Entity (AE) and a Common Services Entity (CSE). The Mca reference point shall allow an AE to use the services provided by the CSE, and for the CSE to communicate with the AE.

The services offered via the Mca reference point are dependent on the functionality supported by the CSE.

NOTE:  The AE and the CSE it invokes may or may not be co-located within the same physical entity.

#### 5.2.2.2	Mcc Reference Point

The Mcc reference point designates communication flows between two Common Service Entities (CSEs). The Mcc reference point shall allow a CSE to use the services of another CSE in order to provide needed functionality. The services offered via the Mcc reference point are dependent on the functionality supported by the CSEs.

NOTE: The Mcc reference point between two CSEs may be supported between different M2M nodes.

Editor’s Note: In the event oneM2M develops specifications for more than one CSE implementation, the use of the Mcc reference point between them (for both intra-domain and inter-domain instances) would need elaboration, and is FFS. This requires coordination between the ARC and PRO working groups.

#### 5.2.2.3	Mcn Reference Point

The Mcn reference point designates communication flows between a Common Service Entity (CSE) and the Underlying Network Services Entity (NSE). The Mcn reference point shall allow a CSE to use the services (other than transport and connectivity services) provided by the Underlying NSE in order to provide the needed functionality.

The services offered via the Mcn reference point are dependent on the services provided by the Underlying NSE.

Editor's Note: The following NOTE should be moved into section 6.x.

NOTE: Information exchange between two M2M nodes assumes the usage of the transport and connectivity services of the Underlying Network Services Entity, which are considered to be the basic services.

Editor's Note: While allowing for different instantiations of the CS Function into different functional Entities, this specification follows the following guidelines/principles while specifying the interfaces on Mca, Mcc and Mcn reference points:

•	Specification of the interfaces on reference points Mca and Mcc are independent of the functionality provided by the CSE.

•	Specification of the interfaces on reference point Mcn may take into account the functionality provided by the CSE.

#### 5.2.2.4	Mcc' Reference Point

The Mcc’ reference point designates communication flows between two Common Service Entities (CSEs) in infrastructure nodes that are oneM2M compliant and that resides in different M2M SP domains. Hence, it allows a CSE of an infrastructure node residing in the Network Domain of an M2M Service Provider to communicate with a CSE of another infrastructure node residing in the Network Domain of another M2M Service Provider to use its services,  and vice versa.

The actual trigger for this communication could be initiated elsewhere in the oneM2M network. Hence, Mcc’ extends the reachability of services offered over the Mcc reference point, or a subset thereof.

Note that the services offered via the Mcc reference point are dependent on the functionality supported by the CSEs.

# 6	Configurations supported by oneM2M Architecture

Figure 6-1 illustrates some configurations supported by oneM2M architecture, illustrating the relations amongst the functional entities in relation to the reference points identified in Figure 5.2.1-1.

Figure 6-1: Configurations supported by oneM2M Architecture

Figure 6-1: Configurations supported by oneM2M Architecture

Node-less AEs: This entity indicates that IN-AEs maybe hosted by an Application Service Provider. Any relationship, binding or management of these IN-AEs to other co-located functions is out-of-scope.

Node:	A oneM2M Node is a functional entity that shall contain at least one oneM2M Common Services Entity and/or one oneM2M Application Entity. A oneM2M Node may be contained in a physical object e.g., M2M Device, Gateway or Server Infrastructure.

NOTES:

1.	CSEs resident in different Nodes are not identical and are dependent on the services supported by the CSE in that Node.

2.	Mcc' reference point aims to be as similar as possible to the Mcc reference point. But due to the nature of inter-M2M Service Provider communications, some differences are anticipated.

Editor's Notes:

•	The illustration above does not provide an exhaustive list of possible configurations and is subject to qualifications. Other possible functions/configurations are FFS.

•	Mcc' reference point aims to be as similar as possible to the Mcc reference point. But due to the nature of inter-M2M Service Provider communications, some differences are anticipated.

Description of Nodes:

oneM2M architecture foresees the following types of Nodes. It is important that as functional objects, such Nodes are not necessarily mapped to actual physical objects, although they may be mapped to actual physical objects.

Application Service Node (ASN):

An Application Service Node is a Node that contains one Common Services Entity and contains at least one Application Entity.

An Application Service Node may communicate over a Mcc reference point with either:

- exactly one Middle Node;

- or exactly one Infrastructure Node.

Example of physical mapping: an Application Service Node could reside in an M2M Device.

Application Dedicated Node (ADN):

An Application Dedicated Node is a Node that contains at least one Application Entity and does not contain a Common Services Entity.

An Application Dedicated Node communicates with a Middle Node or an Infrastructure Node over an Mca reference point.

Example of physical mapping: an Application Dedicated Node could reside in a constrained M2M Device.

Middle Node (MN):

A Middle Node is a Node that contains one Common Services Entity and contains zero or more Application Entities.

A Middle Node communicates with:

- either an IN or another MN over Mcc, plus at least

- an IN / MN / ASN  over Mcc, or

- an ADN over Mca.

Example of physical mapping: a Middle Node could reside in an M2M Gateway.

Infrastructure Node (IN):

An Infrastructure Node is a Node that contains one Common Services Entity and contains zero or more Application Entities.

An Infrastructure Node communicates over respective Mcc reference points with:

- one or more Middle Node(s); and/or

- one or more Application Service Node(s).

In addition, an Infrastructure Node communicates with one or more Application Dedicated Nodes over respective Mca reference points.

Example of physical mapping: an Infrastructure Node could reside in an M2M Server Infrastructure.

Editor’s Note: It is not clear what differentiates an Infrastructure Node from an Application Service Node or from a Middle Node. It could be a minimum feature scope of its CSE that would be bigger, or a particular requirement on its Mcn reference point. It could also be the cardinality of this Node in a deployment.  Typically there would be just one Infrastructure Node in a deployment.

## 6.2	Common Service Functions

This clause describes the "service functions" provided by the Common Services Layer that are common to the oneM2M environment. Such service functions are referred to as Common Service Functions (CSFs). The CSFs provide services to the Applications via the Mca reference point and to other CSEs via the Mcc reference point. Interaction with the Underlying Network Service Functions is via the Mcn reference point. An instantiation of a CSE in a Node comprises a subset of the CSFs from the CSFs described herein.

Figure 6.2-1: Common Services Functions

### 6.2.1		Application and Service Layer Management

#### 6.2.1.1	General Concepts

The Application and Service Layer Management (ASM) CSF is responsible for providing management of AEs and CSEs on the Application Dedicated Nodes, Application Service Nodes, Middle Nodes and Infrastructure Nodes. This includes functions to configure, troubleshoot and upgrade the functions of the CSE as well as upgrade the AEs.

#### 6.2.1.2	Detailed Descriptions

The ASM CSF provides management capabilities for functions of a CSE. These management capabilities are aligned with the management functions provided by the CSE.

Figure 6.2.1.2-1: Management Layers and Functions

The ASM CSF capabilities on Application Dedicated Nodes, Application Service Nodes and Middle Nodes should utilize the functions provided by the DMG CSF for interaction with the Device Management Server.

Editor's Note:  Specific management capabilities for functions are FFS.

Editor's Note:  Once more work has been completed on the DMG CSF and this CSF – these CSFs should be reviewed to determine if they should be merged or kept separate.

The management functions include:

- Configuration Function (CF): This function includes the configuration of the capabilities and features of the CSE.

- Diagnostics and Monitoring Function (DMF): This function includes the troubleshooting of the CSE through the use of diagnostic tests, alert generation and retrieval of operational status (e.g., local context) and statistics of associated with the CSE capabilities and features.

- Software Management Function (SMF): This function provides the lifecycle management for software components and associated artifacts for different components such as CSE and AE.

The ASM CSF shall provide capabilities to manage the CSE’s configuration capabilities on Infrastructure Nodes, Middle Nodes and Application Service Nodes.

The ASM CSF shall provide capabilities to manage the CSE’s troubleshooting capabilities on Infrastructure Nodes, Middle Nodes and Application Service Nodes.

The ASM CSF shall provide capabilities to manage the lifecycle for software components and associated artifacts that comprise the CSE on Infrastructure Nodes, Middle Nodes and Application Service Nodes.

The ASM CSF should provide capabilities to manage the lifecycle for software components and associated artifacts that comprise an AE on Application Dedicated Nodes, Application Service Nodes and Middle Nodes.

NOTE: Management functions for the AE and CSE are controlled through Role based permissions.

##### 6.2.1.2.1	Configuration Function

The ASM CSF’s configuration function provides the interface(s) to configure the capabilities CSE provided by the following CSF’s:

- TBD

- TBD

##### 6.2.1.2.2	Diagnostic and Monitoring Function

The ASM CSF’s Diagnostic and Monitoring function provides the interface(s) to:

- Execute requested diagnostics for specified CSE capabilities

- Retrieve logs, statistics and status of resources associated with CSE capabilities.

###### 6.2.1.2.2.1	CSE Diagnostics

CSE Diagnostics are tests or measurements that are performed to troubleshoot and monitor capabilities within the CSE. Depending on the test or measurement, a test shall be performed using a synchronous and/or asynchronous communication scheme.

In addition to the type of communication scheme, CSE test and measurements shall define a recurrence pattern or schedule (e.g., daily, once, every 4 hours). CSE tests shall define a test repetition to be executed within the schedule.

CSE tests and measurements are provided by the following CSF’s:

- TBD

- TBD

###### 6.2.1.2.2.2	CSE Monitoring

CSE Monitoring provides operating characteristics of the CSE capabilities. These operating characteristics include attributes for operational status, configuration and statistics as well as management of alerts.

CSE Monitoring is provided by the following CSF’s:

- ASM Software Management Function:

- Retrieval of  log files associated with the software module

- Retrieval of the lifecycle state and associated information related to the artifacts and software modules of the Software Package

- Notifications for faults associated with software lifecycle state transitions

- Others ... TBD

A CSE’s operational characteristics information exchange shall be provided to an Application by the CSE as a result of a retrieval request, alert generation or bulk transfer operations.

##### 6.2.1.2.3	Software Management Function

The Software Management Function (SMF) provides the capability to manage software components (e.g. Software Package, Software Module) for AEs and CSEs.

The ASM CSF provides the capability to manage the lifecycle of the Software Packages within a CSE.   Likewise the ASM CSF provides the capability to manage the lifecycle of Software Packages for AEs. These Software Packages may be deployed on any node that supports the AE; including those on the Middle Nodes, Application Dedicated Nodes and Application Service Nodes.

The lifecycle of a Software Package consists of states (e.g., Installing, Installed, Updating, Uninstalling, Uninstalled) that transition when an action (e.g., Download, Install, Update, Remove) is applied to the Software Package.

When a Software Package is installed into an execution environment the software component that is capable of executing in the Execution Environment is called a Software Module. The lifecycle of a Software Module consists of states (e.g., Idle, Starting, Active, Stopping) that transition when an action (e.g., Start, Stop) is applied to the Software Module.

### 6.2.2		Communication Management and Delivery Handling

#### 6.2.2.1 	General Concepts

The Communication Management and Delivery Handling (CMDH) CSF is responsible for providing communications with other CSEs, AEs and NSEs .

The CMDH CSF is responsible to decide at what time which communication connection to use for delivering communication (e.g. CSE-to-CSE communication) and, when needed and allowed, store communication requests so that they can be forwarded at a later time. This processing in the CMDH CSF has to be carried out in line with the provisioned policies and delivery handling parameters that can be specific to each request for communication.

For communication using the Underlying Network data transport services, the Underlying Network can support the equivalent delivery handling functionality. In such case the CMDH CSF is able to use the Underlying Network, and it may act as a front end to access the Underlying Network equivalent delivery handling functionality.

#### 6.2.2.2	Detailed Descriptions

The services that other CSFs, AEs or NSEs can request from the CMDH CSF is to transport some given data to a specific target (CSE, AE or NSE), according to specific delivery instructions while staying within the constraints of provisioned communication management and delivery handling Policies.

The content of the data provided by the requestor shall be opaque to the CMDH CSF and shall not influence the behaviour of the CMDH CSF. In particular, the CMDH CSF shall not be aware of the specific destination function (CSF) within the target entity including the parameters passed to such a function. For instance if the destination of the data to be delivered is a DMR CSF within a target entity (CSE, NSE, AE), the CMDH CSF shall not be aware of the DMR CSF including the specific storage location or other parameters pertaining to that DMR CSF. That means all attributes that are intended to be shared with the destination function (e.g., which CSF is the destination on the target entity, what that CSF should do with the data etc.) should be hidden from the CMDH CSF.

The target entity may be reached either directly or via the CSE of a Middle Node.

Editor’s Note:  For Rel-1 it would probably make sense to limit to one Middle Node along the path since we need to define how to handle routing which could become more complex and should be able to be added at a later stage without or with little impact on the message exchanges.

The delivery instructions that are given to the CMDH CSF shall set the limits in the delivery process that the requestor is capable of accepting. For instance those instructions can contain acceptable delay tolerance etc.

In line with provisioned policies and delivery instructions that can be specific to each communication request, the CMDH CSF shall decide at what time which communication path to use for delivering the communication (e.g. CSE-to-CSE communication) and, when needed and allowed, store communication requests so that they can be forwarded at a later time.

As a consequence the CMDH CSF shall need to be aware of the already established connections to other Nodes via the Underlying Networks, request establishment of new connections and tear down existing ones.

In order to accomplish this function, the CMDH CSF shall need to have access to provisioned delivery handling policies and shall need to parse them so that a proper use of buffers for store-and-forward processing can be managed.

Editor’s Note: Clarification of complexity impacts due to use of multiple Underlying Networks are FFS.

### 6.2.3	Data Management and Repository

#### 6.2.3.1	General Concepts

One of the goals of oneM2M CSEs is to enable M2M Applications to exchange data with each other.

Data Management and Repository (DMR) CSF is responsible for providing data storage and mediation functions. It includes the capability of collecting and aggregating large amounts of data, converting this data into a specified format, and storing it for analytics and semantic processing. The "data" can be either raw data transparently retrieved from an M2M Device, or processed data which is calculated and/or aggregated by M2M entities. This collection of large amounts of data constitutes what is known as the Big Data Repository functionality.

#### 6.2.3.2	Detailed Descriptions

DMR CSF shall provide the capability to store data such as Application data, subscriber information, location information, device information, semantic information, communication status, access permission, etc. The "data" stored by the DMR CSF enables management of the data and provides the foundation of Big Data. Further, the DMR CSF shall be able to support the physical replacement of M2M Devices and/or Gateways in terms of the stored data.

The DMR CSF shall support transfer of data to/from the AEs, other CSFs and remote CSEs. The DMR CSF shall support transfer of data regardless of the peer entity being on-line or not. The DMR CSF shall support operations such as Create, Read, Update and Delete. The DMR CSF may associate event categorization (e.g., normal, urgent) with the data for supporting differentiated services. External entities such as AEs, other CSFs or remote CSEs shall be able to be granted access to the data in the DMR CSF based on defined policies.

Editor's Note:  Needs to clarify what is meant by "peer entity" being on-line or not. Is the reference to peer-CSE or something more than that?

The organization of the data helps describe how they relate with each other and how they are addressed. DMR CSF shall allow removal, addition, search of the data. The DMR CSF shall support structured data. Some structures may be specified by oneM2M, others may not. Elements of data stored in the DMR CSF shall be uniquely addressable.

The DMR CSF shall support aggregating the data from the same device or from different devices.

The DMR CSF shall enable the sharing of the data amongst local CSFs, remote CSEs and AEs.

Contextual information such as data types, semantic information, time stamps, location etc. shall complement the data stored by the DMR CSF. This will allow the DMR CSF to access and search the data based on a rich set of parameters. The DMR CSF shall be able to trigger data processing based on access to its data.

The DMR CSF shall provide a means to perform analytics on large amount of data, so that service providers can provide value-added services to Applications. The DMR CSF shall be responsible for enabling Applications to discover, execute and use analytics based on large amount of data from different sources.

Semantic information needs to be available on the data that are transferred within the oneM2M System. The DMR CSF shall provide functions annotating such semantic information to M2M data and exposing M2M resources using the semantic information so that the M2M resources can be discovered by applications that do not have prior knowledge of them. The DMR CSF shall be responsible for enabling applications to discover, interpret and use M2M data from different sources.

Editor's Note: Semantics related capabilities are FFS subject to discussions in MAS (WG5) WG.

### 6.2.4	Device Management

#### 6.2.4.1	General Concepts

The Device Management (DMG) CSF is responsible for providing management of device capabilities on Middle Nodes (M2M Gateways), Application Service Nodes and Application Dedicated Nodes (M2M Devices) as well as devices that reside within an M2M Area network. Application Entities (AE) can manage the device capabilities on those Nodes by using the services provided by DMG CSF, and AE does not need to be aware of the management technology specific knowledge. However, management technology specific knowledge might be exposed to the AE for administrative purposes (diagnostics and troubleshooting etc.).

Editor's Note: In section 6.2.4, it is difficult to keep track of "M2M Devices" vs "devices". To verify all occurrences of "devices" for consistency in the use of terminology.

##### 6.2.4.1.1	Device Management Architecture

In order to manage the CSE and device capabilities of the Middle Nodes (MNs), Application Service Nodes (ASNs) and Application Dedicated Nodes (ADNs), the DMG may utilize existing device management technologies (e.g., TR-069, OMA-DM, and LWM2M) in addition to management of Management Resources across the Mcc reference point. When the device management technology is used to manage the MN, ASN or ADN, the DMG of the IN translates or adapts the management related requests from other CSEs or from AEs to the device management commands corresponding of the device management technology.

In order to perform the translation and adaptation functions, the DMG has a functional component termed the Management Adapter (Figure 6.2.4.1.1-1). The Management Adapter in the DMG of the IN (IN-DMG-MA) performs the adaptation between the DMG and Management Servers; while the Management Adapter in the DMG of the MN  (MN-DMG-MA) and ASN (ASN-DMG-MA) performs translation and adaptation between the DMG and the Management Client. Only one Management Adapter is shown in the DMG although it can interact with Management Server using different management technologies.

The interface between Management Server and Management Client is the "mc" interface which is subject to the device management technology that is used (e.g. OMA DM, BBF TR069 or OMA LWM2M). The "mc" interface is technology dependent and is outside the scope of this specification.

The DMG in the CSE of the Middle Node has the same functionality as the DMG in the CSE of the  Application Service Node. In addition, the DMG in the Middle Node can be used to manage devices in the M2M Area Network. In this case, the DMG is deployed with Proxy functionality that interacts with the Proxy Management Client using the "mp" interface. The "mp" interface  is technology dependent and is outside the scope of this specification.

The Device Management Server and Device Management Client may be implemented as an entity external to the Node or they may be implemented as an entity embedded within the Node (Figure 6.2.4.1.1-1). The Management Server and the Management Client are located in the boundary of the Node to indicate this situation. In case that the Management Server is located outside of the IN, the Management Server might belong to the different service provider with one that operates the IN.

Figure 6.2.4.1.1-1: Device Management Architecture

6.2.4.1.2	Device Management Server Interaction

###### 6.2.4.1.2.1	Overview

The DMG CSF in IN has the capability to utilize Device Management Servers from existing device management technologies (e.g. OMA-DM, TR-069) to implement the Device Management Functions. The IN-DMG-MA communicates with the Management Server using the "ms" interface that is provided by the Management Server. Note that "ms" interface is outside the scope of this specification. The IN-DMG-MA takes the following roles:

- Protocol Translation between DMG and the Management Server
After the DMG receives the requests from AE via the Mca reference point, the IN-DMG-MA translates the requests from the AEs to requests that can be understood by the Management Server.

- Interaction with the Management Server
By using "ms" interface, the IN-DMG-MA can communicate with the Management Server. This is for delivering the requests from the AE to the Management Server, or receiving information from the Management Server that will be notified to AE.

- Management Server Selection
This is applicable when the IN-DMG-MA can communicate with the multiple Management Servers that have the different level of access rights to resources from the management technologies. The IN-DMG-MA selects the proper Management Server that has the access rights to perform the management requests. In order to obtain the knowledge of which resources are managed by a Management Server, the access right information for resources from Management Servers may be queried using the "ms" interface.

A Device Management Server may be located in the Underlying Network using the Mcn reference point as depicted in Figure 6.2.4.1.2.1-1 or the Device Management Server may be located in the M2M Service Layer as depicted in Figure 6.2.4.1.2.1-2.

Figure 6.2.4.1.2.1-1: Device Management Server in Underlying Network

Figure 6.2.4.1.2.1-2: Device Management Server in M2M Service Layer

The "ms" interface is functionally the same interface regardless if the Device Management Server resides in the Underlying Network or the Service Layer. However, the access rights that the Management Server has for resources from the management technology can be different depending whether the Management Server resides in the Underlying Network or in the Services Layer. For example in Figure 6.2.4.2.1.1-1, the Device Management Server in the Underlying Network controls access of the exposed resources from the management technology while, in the Figure 6.2.4.2.1.1-2, the Device Management Server in the M2M Service Layer controls access to the resources.

###### 6.2.4.1.2.2	Device Management Server – Access Permissions

When an operation on a M2M Service Layer Resource is performed, the access permissions are checked by the security functions described in the Security CSF. The Security CSF uses the information in the Access Rights associated with the Resource to determine if the requested access to the Resource is granted.

If the access to the Resource is granted and the operation for the Resource utilizes Device Management Server external to the service Layer, the DMG CSF selects one or more among the authenticated Device Management Servers necessary to access the requested resources.

Editor's Note:  The procedure for selecting the Device Management Server is FFS.

##### 6.2.4.1.3	Device Management Client Interaction

###### 6.2.4.1.3.1	Overview

The DMG CSF in MN or ASN has the capability to utilize Device Management Client from existing management technologies (e.g. OMA-DM, TR-069) to implement the Device Management Functions. The MN-DMG-MA or ASN-DMG-MA communicates with the Management Client using the "la" interface (e.g. DM-7,8,9 ClientAPI in OMA DM[i.2]) that is provided by the Management Client. Note that the "la" interface is outside the scope of this specification. The MN-DMG-MA or ASN-DMG-MA takes the following roles:

- Interaction with the Management Client 
By using "la" interface, the Management Adapter may communicate with the Management Client to find out the device capabilities supported by the Management Client.

- Mapping between the DMG and Management Client 
After the Management Adapter collects the device capabilities supported by the Management Client; the Management Adapter performs the mapping between the device capabilities to oneM2M Management Resources. The DMG in MN or ASN may create those Management Resources in the IN-CSE, and the Management Resources may be used by the IN-AE to manage the device capabilities pertaining to the MN or ASN.

Figure 6.2.4.1.3.1-1: Device Management Client Interaction using "Ia" interface

##### 6.2.4.1.4	Device Management Resource Lifecycle

###### 6.2.4.1.4.1	Overview

The lifecycle of a Device Management Resource is implemented using the resource management information defined in Sections 9.1 through 9.5 and corresponding procedures to Create, Retrieve, Update and Delete the resources are defined in Section 10. This section describes additional resource management and procedures for Device Management Resources.

###### 6.2.4.1.4.2	Overview

Section 9.1 through Section 9.5 define resource management information that is applicable to any type of resource, including Device Management Resources. In addition a Device Management Resource also maintains information and relationships that are specific Device Management Resources. This information is used to:

- Manage the technology specific objects via a Device Management Server which requires the information necessary to identify and access the Device Management Server.

- Invoke the security mechanism of the Device Management Server in order to authorize access to the technology specific objects.

###### 6.2.4.1.4.3	Procedures for Creation, Update and Deletion of Device Management Resources

Section 10 defines the procedures to Create, Update and Delete resources. These procedures are also applicable to Device Management Resources, In addition to the procedures Device Management Resources are Created, Updated or Deleted:

- By administrative means using the Mca reference point

- Directly by a CSE based on a discovery or another event within the CSE

- Indirectly by the Device Management Server or Device Management Client when an event (firmware update, fault notification) occurs within the Device Management Server or Client

Regardless of the Create, Update or Delete operation, the Originator of the operation shall be authorized to perform the operation. In addition, at most one Device Management Server shall be able to Create, Delete or Update addressable elements of a Management Resource.

#### 6.2.4.2	Detailed Descriptions

The DMG CSF shall provide capabilities for the purpose of managing M2M Devices/Gateways as well as devices in M2M Area Networks.

Figure 6.2.4.2-1: Device Management Entities and Functions

Such capabilities shall include:

- Device Configuration Function (DCF): This function includes the configuration of the capabilities and features of the M2M Device, M2M Gateway or device in the M2M Area Network.

- Device Diagnostics and Monitoring Function (DDMF): This function includes the troubleshooting through the use of diagnostic tests, alert generation and retrieval of operational status and statistics associated with the M2M Device, M2M Gateway or device in the M2M Area Network.

- Device Firmware Management Function (DFMF): This function provides the software lifecycle management for firmware components and associated artifacts for the M2M Device, M2M Gateway or device in the M2M Area Network.

- Device Topology Management Function (DTMF): This function provides the management of the topology of the M2M Area Network.  A M2M Area Network is comprised of Application Dedicated Nodes and other devices in the M2M Area Network

NOTE: Such device management capabilities are assigned to one or more Execution Environments and access to such Execution Environments is controlled through Role based permissions.

The DMG CSF shall be able to interact with a Device Management Server for managing M2M Devices/Gateways as well as with devices in the M2M Area Networks that are provisioned with Management Clients.

Editor's Note:  Need to provide definition of Device Management Server in order to describe the interaction with the Device Management Server.

Editor's Note:  Need to provide definition of Execution Environment, Device Management Server, Management Clients, Role based permissions.

Editor's Note:  The CSF must document the management functions that cause impacts to the underlying network and document they must utilize access rights that are delegated from the network operator.

##### 6.2.4.2.1	Device Configuration Function

The Device Configuration Function (DCF) provides the configuration of device capabilities and features that are necessary to support M2M Services and AEs in M2M Devices, M2M Gateways or devices in a M2M Area Network.

These device configuration capabilities and features include:

- Discovery of a device’s management objects and attributes

- Ability to enable or disable a device capability

- Provisioning configuration parameters of a device

Editor's Note:  Handling of Security configuration parameters and the interaction with SEC CSF is for FFS.

##### 6.2.4.2.2	Device Diagnostics and Monitoring Function

The Device Diagnostics and Monitoring Function (DDMF) allows the troubleshooting of device features that are necessary to support M2M Services and AEs in M2M Devices, M2M Gateways or devices in a M2M Area Network.

These device diagnostic and monitoring features include:

- Configuration of diagnostics and monitoring parameters on the device

- Retrieval of device information that identifies a device and its model and manufacturer

- Retrieval of device information for the software and firmware installed on the device

- Retrieval of information related to a battery within the device

- Retrieval of information associated with the memory in use by a device

- Retrieval of the event logs from a device

- Device reboot diagnostic operation

- Device factory reset diagnostic operation

##### 6.2.4.2.3	Device Firmware Management Function

The Device Firmware Management Function (DFMF) provides lifecycle management for firmware associated with a Device.

Device firmware is comprised of firmware modules and artifacts (e.g., configuration files) that are maintained on a Device. A Device may maintain more than one firmware image and the capability to manage individual firmware images shall be provided. The firmware lifecycle includes actions to download, update or remove a firmware image. In addition firmware could be downloaded and updated within the same action.

##### 6.2.4.2.4	Device Topology Management Function

The Device Topology Management Function (DTMF) is a function that is specific to M2M Gateways where a M2M Gateway maintains zero or more M2M Area Networks.

These device topology management features include:

- Configuration of the topology of the M2M Area Network

- Retrieval of information related to the devices attached to the M2M Area Network

- Retrieval of information that describes the transport protocol associated with the M2M Area Network

- Retrieval of information that describes the characteristics associated with online/offline status of devices in the M2M Area Network

### 6.2.5	Discovery

#### 6.2.5.1	General Concepts

Discovery (DIS) CSF is responsible for searching information e.g., supported attributes and resources according to a Request from an Originator within a given scope and subject to permissions, including those allowed by M2M service subscription. An Originator could be an Application or another CSE. The scope of the search could be in one CSE, or in more than one CSE. The discovery results are returned back to the Originator.

#### 6.2.5.2	Detailed Descriptions

DIS CSF shall provide discovery of information/resources residing in its own CSE subject to permissions including the M2M service subscription. The DIS CSF shall be able to support various discovery methods through the use of "filter criteria" that can limit the scope of information/resources that are returned based on various filters (e.g. keyword, identifiers, location, semantic information) or a combination of such filters.

The DIS CSF shall be triggered by sending a "request" to the address (e.g., URI) of the discovery resource. Upon receiving such request, the DIS CSF shall identify and return the matching information/resources. A successful response may include the actual discovered information or address(es) (e.g., URI(s)) pertaining to the discovered resources. In addition, based on the policies or Originator request, the CSE which received the Discovery request may forward the request to other CSEs on its registered Middle/Infrastructure Node.

### 6.2.6	Group Management

#### 6.2.6.1	General Concepts

Group Management (GMG) CSF is responsible for handling Group related requests. The request is sent for the management of a Group and its membership as well as for the bulk operations supported by the Group. When adding or removing members to/from a Group, it is necessary to validate if the member complies with the purpose of the Group. Bulk operations includes read, write, subscribe, notify, device management, etc. Whenever a request or a subscription is made via the Group, the Group is responsible for aggregating its responses and notifications. The members of a Group can have the same role with regards to access rights control towards a resource. In this case, access control is facilitated by grouping. When the Underlying Network provides broadcasting and multicasting capability, the GMG CSF is able to utilize such capability.

#### 6.2.6.2	Detailed Descriptions

GMG CSF shall be responsible for the management of a Group and its membership. The GMG CSF shall support bulk operations to multiple resources of interest and aggregate the results. The GMG CSF shall be able to facilitate access control based on grouping. When needed and available, the GMG CSF may leverage the existing capabilities of the Underlying Network including broadcasting/multicasting.

The GMG CSF handles requests from Applications and/or from other CSEs.

Grouping enables the oneM2M System to perform bulk operations to multiple devices, applications, or resources. The GMG CSF shall manage the resources and operations associated with grouping.

The GMG CSF shall handle the requests to create, query, update, and delete a Group. An Application or a CSE may request the creation/retrieve/update/deletion of a Group as well as the addition and deletion of members of the Group. The GMG CSF shall be able to create one or more Groups in CSEs in any of the Nodes in oneM2M System for a particular purpose (i.e. facilitation of access control, device management, fan-out common operations to a group of devices, etc.). The GMG CSF shall handle the requests to retrieve the information (e.g. URI, metadata, etc.) of a Group and its associated members.

The GMG CSF shall manage Group membership. The GMG CSF shall handle requests to add or remove members to and from a Group’s member list. A member may belong to one or more Groups. A Group may be a member of another Group. When new members are added to a Group, the GMG CSF should validate if the member complies with the purpose of the Group.

In order to fulfil the functionalities of the GMG CSF, functionalities from other CSFs may be needed. Examples include Network Service Exposure CSF for leveraging broadcasting/multicasting capability from the Underlying Network, Security CSF for authentication and authorization etc.

Editor’s Note: How the Underlying Network supports broadcasting/multicasting is FFS.

When facilitating access control using a Group, only members with the same access rights towards a resource shall be included in the same Group. Also, only M2M Applications or CSEs which have a common role with regards to access rights shall be included in the same Group. This shall be used as a representation of the "role" when facilitating role based access control.

Editor's Note:  Suggest adding description on what is meant by Group member, what entities can be members of a Group?  Can a Group consist of heterogeneous members?

The GMG CSF shall forward requests to all members in the Group. In case the Group contains another Group as a member, the forwarding process shall be done recursively, i.e. the nested Group shall forward the request to its members. After forwarding the request to all members in the Group, the GMG CSF shall generate an aggregated response by aggregating the corresponding responses from the Group members.

Editor’s Note: The structure of the resource in oneM2M System is FFS.

The GMG CSF shall support subscriptions to individual Groups. Subscriptions to a Group shall be made only if the subscriber is interested in all members of the Group. If subscription to a Group is made, the GMG CSF shall aggregate the notifications from the Group members, and it shall notify the subscriber with the aggregated notification. Responses and event notifications relevant to a subscription may be selectively filtered by filtering criteria. The filtering criteria shall be managed by the GMG CSF. In order to subscribe to or query a Group, methods for group discovery shall be provided.

Editor's Note:  Methods used for subscribing to and for querying a Group are FFS.

### 6.2.7	Location

#### 6.2.7.1	General Concepts

The Location (LOC) CSF allows M2M AEs to obtain geographical location information of M2M Nodes (e.g., ASN, MN) for location-based services. Such location information requests may be from an M2M AE residing on either a local Node or a remote Node.

NOTE: Geographical location information can be more than simply the longitude and the latitude information.

#### 6.2.7.2	Detailed Descriptions

LOC CSF shall obtain and manage geographical location information based on the requests from M2M AEs residing on either a local Node or a remote Node. The LOC CSF shall interact with any of the following: a location server in the Underlying Network, a GPS module in the M2M Node, or, information for inferring location stored in other M2M Nodes.

In order to Update the location information, an M2M AE shall be able to configure an attribute (e.g., update period). Based on the such defined attribute, the LOC CSF shall Update the location information using one of interactions defined above.

NOTE: The location technology (e.g., Cell-ID, Assistant-GPS, and Fingerprint) used by the Underlying Network depends on the capabilities of the Underlying Network.

The LOC CSF shall be able to request M2M Nodes to share and report their own or other M2M Node's geographical location information with the requesting M2M Applications.

The LOC CSF shall be able to provide means for protecting the confidentiality of geographical location information.

### 6.2.8	Network Service Exposure, Service Execution and Triggering

#### 6.2.8.1	General Concepts

Network Service Exposure, Service Execution and Triggering (NSSE) CSF manages communications with the Underlying Networks for accessing network service functions over the Mcn reference point. The NSSE CSF uses the available/supported methods for service "requests" on behalf of M2M Applications. The NSSE CSF shields other CSFs and AEs from the specific technologies and mechanisms supported by the Underlying Networks.

NOTE: The NSSE CSF provides adaptation for different set of network service functions supported by various Underlying Networks.

The network service functions provided by the Underlying Network include service functions such as, but not limited to, device triggering, small data transmission, location notification, policy rules setting, location queries, IMS services, device management. Such services do not include the general transport services.

#### 6.2.8.2	Detailed Descriptions

NSSE CSF shall manage communication with the Underlying Networks for obtaining network service functions on behalf of other CSFs, remote CSEs or M2M Applications. The NSSE CSF shall use the Mcn reference point for communicating with the Underlying Networks.

NOTE: The network services provided by the Underlying Networks include services such as, but not limited to, device triggering, small data transmission, location notification, policy rules setting, location queries, IMS services, device management. Such services do not include general transport services.

The M2M System shall allow the Underlying Networks to control network service procedures and information exchange over the Underlying Networks while providing such network services. For example, for the 3GPP networks, the Underlying Network may chose to provide the network services based on control plane signalling mechanisms.

Other CSFs in a CSE that need to use the services offered by the Underlying Network shall use the NSSE CSF.

The NSSE CSF shall shield other CSFs and AEs from the specific technology and mechanisms supported by the Underlying Networks.

NOTE: The NSSE CSF provides adaptation for different set of network service functions supported by various Underlying Networks.

The NSSE CSF shall maintain over the Mcn reference point, the necessary connections and/or sessions between the CSE and the Underlying Network when local CSFs are in need of a network service.

The NSSE CSF shall provide to the CMDH CSF information related to the Underlying Network so the CMDH CSF can include them to determine proper communication handling.

### 6.2.9	Registration

#### 6.2.9.1	General Concepts

Registration (REG) CSF is responsible for handling an Application or another CSE to register with a CSE in order to allow the registered entities to use the services offered by the registered-with CSE. The REG CSF handles registration of a Device also, so as to allow registration of Device's properties/attributes with the CSE.

#### 6.2.9.2	Detailed Descriptions

Registration is the process of delivering AE or CSE information to another CSE for being able to use the M2M Services.

The CSE on an Application Service Node or on a Middle Node shall perform registration with the CSE in the Infrastructure Node in order to be able to use M2M services offered by the CSE in the Infrastructure Node. An Application on an Application Service Node or on a Middle Node shall perform registration locally with the corresponding CSE in order to use M2M services offered by that CSE. The same holds for the registration of an Application on the Infrastructure Node. An Application on an Infrastructure Node shall perform registration locally with the associated CSE on Infrastructure Node. A registered Application shall be able to have interactions with its local CSE without the need to have the local CSE register with other CSEs. Such registrations are applicable to a single M2M Service Provider Domain.

A CSE on an Application Service Node or on a Middle Node shall perform registration with the CSE on an Infrastructure Node and vice versa. As a result of successful CSE registration, the CSEs on ASN/Middle Node and Infrastructure Node establish a peering relationship and shall be able to exchange context information.

A (physical) Device shall be able to register with its local CSE for registering its properties/attributes. Such registration enables correlation of Devices Identities such as M2M-Node-ID with the CSE-ID.

Editor’s Note: The need for Device Registration and the relationship with CSE Registration are FFS.

Editor’s Note: The use of External ID in Registration is FFS.

Registration information shall be able to include:

    - Identifier

    - Reachability schedules; which are elements of a node’s policy, and specify when messaging may occur between nodes. Reachability schedules may be used in conjunction with other policy elements. When reachability schedules are not present in a node then that node is expected to be always reachable.

Editor’s note: Further Registration information is FFS.

### 6.2.10	Security

#### 6.2.10.1	General Concepts

Security (SEC) CSF comprises the following functionalities:

- Sensitive Data Handling functionality;

- Security Administration functionality;

- Security Association Establishment functionality;

- Authorization and Access Control functionality;

- Identity Protection Functionality.

Sensitive Data Handling functionality in the SEC CSF protects the local credentials on which security relies during storage and manipulation. Sensitive Data Handling functionality performs other sensitive functions as well such as security algorithms. This functionality is able to support several cryptographically separated security environments.

Security Administration functionality enables services such as the following:

- Creation and administration of dedicated security environment supported by Sensitive Data Handling functionality;

- Post-provisioning of a root credential protected by the security environment;

- Provisioning and administration of subscriptions related to M2M services and M2M application services.

Security Association Establishment functionality is responsible for establishing security association between corresponding M2M nodes, in order to provide services such as confidentiality, integrity, authentication, authorization, etc.

Authorization and Access Control functionality is responsible for authorizing services and data access to authenticated entities, according to provisioned security policies and assigned roles.

While unique identifier of an entity are used for authentication, the Identity Protection functionality provides pseudonyms which serve as temporary identifiers which cannot be linked to the true identity of either the associated entity or its user.

#### 6.2.10.2	Detailed Descriptions

SEC CSF shall comprise the following functionalities:

- Sensitive Data Handling functionality;

- Security Administration functionality;

- Security Association Establishment functionality;

- Authorization and Access Control functionality;

- Identity Protection Functionality.

Sensitive Data Handling Functionality in the SEC CSF shall have the capability to protect the local credentials on which security relies during storage and manipulation. The SEC CSF shall be able to extend sensitive data handling functionality to other sensitive data used in the M2M Systems such as subscription related information, security policies and personal data pertaining to individuals. The Sensitive Data Handling functionality shall perform other sensitive functions as well, such as security algorithms. The Sensitive Data Handling functionality shall be able to support several cryptographically separated security environments.

Security Administration functionality in the SEC CSF shall enable the following services:

- Creation and administration of a dedicated security environment supported by Sensitive Data Handling functionality;

- Post-provisioning of a root credential protected by the security environment;

NOTE: The security environment can also be pre-provisioned with a root credential prior to deployment; therefore this capability is not always required. Post-provisioning is required when the security bootstrapping needs to be performed or re-initiated after deployment.

- Provisioning and administration of subscriptions related to M2M services and M2M application services. Besides the root secret, a subscription includes other information classified as sensitive data such as associated authorization and security policies.

Security Association Establishment functionality in the SEC CSF shall be able to establish security associations between corresponding M2M nodes, in order to provide specific security services (e.g. confidentiality, integrity, authentication, authorisation, or support for application level signature generation and verification) involving specified security algorithms and sensitive data. This involves key derivation based on provisioned root secrets. This functionality of the SEC CSF is mandatory when security is supported.

Editor's Node: The definition of Security Association will be provided by WG4. A definition may be: "Security associations (SAs) are logical relationships between 2 entities that may be associated with a communications link, but SAs are not communications links. Security associations may take a number of forms but in each case they identify the nature of the security service (confidentiality, integrity, authentication or authorisation), the required algorithm and key. Security associations may be established for single transactions (and thus their establishment may form part of the transaction itself) or for session based associations (in such instances the association is generally established independently of the individual transactions that are to be secured)".

The Authorization and Access Control functionality in the SEC CSF shall be able to authorize services and data access to authenticated entities, according to provisioned security policies and assigned roles. This functionality is mandatory when any services relying on authorization and access control are present. Among other usages, the services of this functionality may be applied to personal information as a means to preserve privacy.

Although an M2M System User is generally considered to be an application or functional agent that represents a human, there are links between a device and its User that can be either directly derived or indirectly deduced. Consequently, identifiers used for communication in the M2M System shall not be directly related to the real identity of either the device or its User, except where this is a requirement for operation of a specific M2M application.

While the unique identifier of an entity shall be used for authentication, the identity protection functionality provides pseudonyms which serve as temporary identifiers which shall not be able to be linked to the true identity of either the associated entity or its user.

### 6.2.11	Service Charging and Accounting

#### 6.2.11.1	General Concepts

Service Charging and Accounting (SCA) CSF provides charging functions for the Service Layer. It supports different charging models which include online charging and offline charging. The SCA CSF is responsible for capturing chargeable events, recording of information, generating charging records and charging information. The SCA CSF can interact with the charging System in the Underlying Network also. But the SCA CSF is responsible for generating and recording of the final service level charging information. It is the responsibility of the SCA CSF in the Infrastructure Node or the Service Layer Charging Server to handle the charging information for the purpose of charging.

Editor's Node: To provide more information Service Layer Charging Server. What is this entity?

#### 6.2.11.2	Detailed Descriptions

SCA CSF shall perform information recording corresponding to a chargeable event. The SCA CSF shall be able to handle the charging information for the purpose of charging. The SCA CSF shall be able to send the charging information transformed from the specific recorded information to the charging server by the use of a standard or proprietary interface for charging purposes.

The SCA CSF shall support "independent service layer charging" and "correlated charging with the Underlying Network" charging system.  For independent service layer charging, only charging functions in the Service Layer shall be involved. For correlated charging, charging functions in both the Service Layer and the Underlying Network shall be involved.

The SCA CSF shall support one or multiple charging models, such as the follows:

- Subscription Based Charging: A service subscriber is charged based on Service Layer subscriptions.

- Event Based Charging: Charging is based on Service Layer chargeable events. A chargeable event refers to the discrete transactions. For example, an operation on data (Create, Update, Retrieve) can be an "event". Chargeable events shall be configurable.

- Session Based Charging: Charging is based on an M2M service session, such as based on secure service connections.

- Service Based Charging: Service Layer can support charging based on the services it provides. A service may require multiple transactions and the requestor is charged based on the services it received.

Two types of service layer charging shall be able to be supported: Online and Offline charging. Offline charging does not affect services provided in real time. Charging triggering information is generated at the CSFs where the chargeable transaction happens. The charging report is passed via different nodes to the network to generate CDRs. Online charging affects the services granted in real time. A service request is first generated and the requestor’s credit is checked before the service is granted.

Editor's Note: Online charging items is FFS.

The Service Layer charging system shall consist of the following logical functions:

- Charging Triggering Function: This function shall reside in the Service Layer and shall be able to capture the chargeable event and generate recorded information for charging. Recorded information may contain mandatory and optional elements.

Editor's Note: The system may record information for other purposes also such as for Event Logging. Some of such information may be applicable for charging purposes.

- Offline Charging Function: This function shall handle offline charging related operations. It shall generate charging information, such as Service Layer Charging Data Record (CDR)s. CDRs are formatted collection of information about a chargeable event (e.g. amount of data transferred) for use in billing and accounting.

- Online Charging Function: This function shall handle online charging related operations.

- Charging Management Function: This function shall handle charging related policies, configurations, inter-node charging function communications, and interacting with the charging system in the Underlying Network.

### 6.2.12	Service Session Management

#### 6.2.12.1	General Concepts

An M2M service session is an end-to-end Service Layer connection managed by the Service Session Management (SSM) CSF.  The SSM CSF manages M2M service sessions between M2M Applications, between an M2M Application and a CSE, or between CSEs.

Editor's Note: Provide information as to when an M2M service session is needed.

The management of a M2M service session includes capabilities such as the management of session state, session authentication and establishment, management of Underlying Network connections and services related to the session, coordination of sessions spanning multiple hops of CSEs, exchange of information between session endpoints, and session termination.

The SSM CSF uses the CMDH CSF within its local CSE for sending/receiving messages to/from the next-hop CSE or to/from an Application for a given M2M service session. The SSM CSF also uses the SEC CSF for the management of session related security credentials and authentication of session participants. The SSM CSF generates session specific charging events also that it communicates to the SCA CSF within its local CSE.

#### 6.2.12.2	Detailed Descriptions

An M2M service session is an end-to-end Service Layer connection managed by the SSM CSF. The SSM CSF shall support the management of the following M2M service session related information.

- Session Identifier: The Service Session Identifier is used by the SSM CSF and session endpoints to uniquely identify M2M service layer session.

- Session Credentials: Security credentials associated with M2M service session.  For example, E2E security certificates, public keys, etc. An M2M service session can support an independent set of credentials or it can optionally leverage security credentials from Underlying Network sessions or network connections.

- Session Descriptor: Information describing the M2M service session that can be used by the existing session endpoints or prospective session participants in order to discover an existing session.  For example, a description for each session participant (e.g. device identifiers, type of participant, services that participant supports, interface requirements of participant, type of compression used, etc.).

- Session Routing Information: Information describing how to route M2M service session messages. For example, list of CSEs in the routing path, or next-hop CSE in the routing path.

- Session Context/History: Information related to M2M service session activity such as session related events that have occurred or transactions that have been processed.  For example, keeping track of the type, number, rate, size, etc. of the resources targeted by the session endpoints. Or keeping track of different service sessions that a specific application establishes (e.g. rate, type, etc).

- Session Policies: Policies that define rules for how SSM CSF manages sessions. For example, session routing policies, session store-and-forward policies, session access control policies, session data management policies, etc.  The SSM CSF can use these policies by itself or provide such policies to other CSFs (e.g. CMDH CSF, DMR CSF, SEC CSF, etc).

Some M2M service sessions may require security. In order to secure an M2M service session, proper security credentials shall be used by session endpoints (e.g., M2M Applications and/or CSEs).  If M2M service session credentials are not pre-provisioned, the SSM CSF shall support securely bootstrapping of the session credentials to the session endpoints. The SSM CSF shall use the SEC CSF for supporting such bootstrapping. The SSM CSF may also leverage security credentials and trust relationships from Underlying Networks.

The SSM CSF shall support requests to establish an M2M service session between M2M Applications, between an M2M Application and a CSE, or between CSEs.  Before a request to establish an M2M service session is granted, the SSM CSF shall first authenticate the requester using session credentials. The SSM CSF shall use the SEC CSF to support session authentication. Once authenticated, the SSM CSF shall establish the M2M service session between the requesting and targeted session endpoints. This shall involve the SSM CSF coordinating with the targeted session endpoint on an agreed upon session ID that can be used to identify the session messages. The SSM CSF shall return this session ID to the requester. The SSM CSF shall also maintain additional session information for the management of the session such as session policies, session routing information, session descriptor, etc.

The SSM CSF shall support layering of a M2M service session over the top of Underlying Network connections. The SSM CSF shall support persistency of the M2M service sessions with respect to the Underlying Network connections.  The SSM CSF shall maintain an active M2M service session independent of the state of the Underlying Network connections and shall be robust to network connections that are dynamically torn-down and re-established. The SSM CSF shall support initiating or providing input to other CSFs and/or the Underlying Network on whether the network connections should be torn-down/re-established based on M2M service session activity or state.

The SSM CSF shall support requests to terminate an M2M service session between M2M Applications, between an M2M application and a CSE, or between CSEs. Before a request to terminate an M2M service session is granted, the SSM CSF shall first authenticate the requester using session credentials. The SSM CSF shall use the SEC CSF to support session authentication. Once authenticated, the SSM CSF shall terminate the M2M service session between the requesting and targeted session endpoints. This shall involve removal of session information on the session endpoints as well as the SSM CSF.

The SSM CSF shall support M2M service sessions that span multiple intermediate CSE hops. The SSM CSF shall leverage the SEC CSF on its local CSE as well the SSM CSF and the SEC CSF on intermediate CSEs to support multi-hop M2M service sessions. In doing so, SSM CSFs on different CSEs shall support coordinated M2M service session management. This includes bootstrapping of session credentials, session authentication and establishment, management of underlying network connections and services related to the session, management of session routing information, and session termination across multiple SSM CSFs.

Editor' Note:  Security related descriptions for this CSF need to be updated.

Editor' Note: The session involves Mcn reference point or does it involve Mca and Mcc reference points as well. This is FFS.

### 6.2.13	Subscription and Notification

#### 6.2.13.1	General Concepts

The Subscription and Notification (SUB) CSF is responsible for providing notifications pertaining to a subscription that tracks changes on a resource (e.g., deletion of a resource). A subscription to a resource is initiated by an AE or a CSE, and is granted by the Hosting CSE subject to access rights. During an active subscription, the Hosting CSE sends a notification per modification of the resource to the address(es) where the resource subscriber wants to receive it.

Editor’s Note: Determining subscribe-able resources is FFS.

#### 6.2.13.2	Detailed Descriptions

The SUB CSF shall manage subscriptions to resources, subject to access rights, and send corresponding notifications to the address(es) where the resource subscribers want to receive them. An AE or a CSE shall be the subscription resource subscriber. An  AE shall be able to subscribe to a resource on a local CSE or on a remote CSE. A CSE shall be able to subscribe to resources on multiple CSEs. A subscription Hosting CSE shall send a notification to the address where the resource subscriber specified per modification on a resource.

A subscription request shall include the resource subscriber ID, the hosting CSE-ID and subscribed-to resource address(es). It may also include criteria (e.g., resource modifications of interest and subscription policy) and the address where to send the notifications.

A single subscription can subscribe to a single resource. Multiple resources shall be able to be subscribed via a single subscription when they are grouped and represented as a single group resource. However, there may be resources that cannot be subscribed to.

A subscription is represented as "subscription resource" in CSE resource structure.

6.3	Functional Entities Comprising Enablers

Figure 6.3-1 illustrates the architecture of the Common Services Entity (CSE). The CSE comprises of a set of Common Service Functions (CSFs) and a set of Enabler Functions (EFs).

Figure 6.3-1: Common Service Entity Architecture

Figure 6.3-2 illustrates an example of CSFs and EFs within the Common Services Entity (CSE), showing newly added CSFs and associated CSF capabilities.

Figure 6.3-2: Example of CSFs and EFs within Common Services Entity Architecture

The Services Extension Enabler enables the CSE to offer M2M Services over the Mca and Mcc reference points. The Services Extension Enabler provides the following functions:

- Check module authentication

- Check node resources

- Check interoperability with existing modules

- Check policy and rights to determine how to handle conflicts e.g. Do not register new module or deregister existing module, etc.

- Register new module

- Add new service(s) due to new module to list of services

- Modify API support to reflect new service capabilities

- Modify inter-module communications to incorporate new module

Editor’s Note: The above list is preliminary and is for FFS.

Editor’s Note: How the Service Extension Enabler is associated with the APIs is FFS.

## 6.4	Security Aspects

The oneM2M Security Analysis Technical Report [i.3] differentiates security domains related to the transport layer (Underlying Network), service layer (M2M common services) and Application Layer. It also considers possible trust scenarios involving these different security domains, and investigates countermeasures to threats that potentially affect the security of the M2M System.

Each of the security domains may provide their own set of security capabilities. The oneM2M security solution shall provide configurable security services through an API for upper security domains to leverage on, or enable the use of the exposed security features of other security domains when appropriate.

As a result, beyond providing security solutions that protect the integrity of the M2M Service Layer, the oneM2M architecture exposes, through its APIs, further security services that are made available to M2M Applications. This enables M2M Applications to benefit from security solutions deployed in the M2M Service Architecture, without adding redundant and/or proprietary security solutions on application nodes and devices.

Editor's Note: Clarify the difference between "application nodes and devices" in the para above.

NOTE: It remains the responsibility of M2M Application Providers to perform their own risk assessment process to identifying the specific threats affecting them and derive their actual security needs.

## 6.5	Other Aspects

### 6.5.1	Inter M2M SP communication for oneM2M compliant nodes

To enable M2M entities (eg. CSE, AE) in different M2M Service Provider domains to communicate, there is a need to configure within the M2M domain if such a communication is allowed. If allowed, the M2M System shall support routing the traffic across the M2M Service Provider domain and within the target domain.

Communication between different M2M SPs which occurs over the reference point Mcc’, is subject to business agreements. The offered functionality is typically a subset of the functionality offered over the Mcc reference point.

Any interM2M SP communication in support of a request originating from one M2M SP domain shall be processed and forwarded through the Infrastructure Node of the originating M2M domain towards the Infrastructure Node of the target M2M SP domain and finally forwarded to its target CSE, if different from the Infrastructure Node. Hence the Infrastructure Node in both M2M domains shall be the exit and entry points, respectively, for all inter M2P SP communication traffic.

In this configuration approach, public DNS shall be used to support traffic routing for inter M2M SP communication in accordance with [i.7]. This relies on public domain names being allocated to communicating CSE entities within the oneM2M architecture, and to whom access across domains is permitted through policies.  To that effect, an M2M SP supporting inter- M2M SP communication shall ensure that the public domain names for the CSEs whose functionality is available across domains are held in its public DNS and shall always point to the IP address associated with the Infrastructure Node  for the domain  (being the entry point) for accessibility purposes.

The M2M SP could optionally also have additional policies (example: black list or white list) that governs accessibility from other domains to CSE functionality located within its own domain. These policies are however out of scope.

The public domain names of CSEs to whom access from other domains is allowed by policies, shall be created in the DNS of the M2M SP by the Infrastructure Node at registration time of these CSEs, and shall be removed at de-registration. DNS entries for CSEs can also be created/removed for registered CSEs at any time by the M2M SP through administrative means to handle dynamic policies.

Editors Note: The CSE registration procedures will be updated to include normative text for the creation of the needed DNS entries

#### 6.5.1.1		Public Domain Names and CSEs

To enable the usage of public DNSs as described above, there is a need for a naming convention for public names for CSEs.  This naming convention facilitates the creation of the necessary entries of the public domain names of CSEs in the DNS by the infrastructure node.

CSEs public domain names shall be a sub-domain of the Infrastructure Node public domain name. This naming convention allows the Infrastructure Node to include the needed DNS entry corresponding to the CSE to whom access from other domains is allowed. This would typically occur when the CSE registers with the Infrastructure Node, subject to policies, or administratively. The Infrastructure Node shall also have a public domain name.

Accordingly, the structure of the public domain of the CSEs in any node (Middle Node/Application Service Node) shall follow the following naming convention, which relies on the CSE identifier (CSE-ID) as part of the naming convention to facilitate the entry creation:

    - Infrastructure Node public domain name: <Infrastructure Node CSE Identifier>.<M2M Service Provider domain name>

    - CSE Middle Node public domain name: <Middle Node CSE Identifier>.<Infrastructure Node public domain name>

    - CSE Application  Service Node  public domain name: <Application Service Node CSE Identifier>.<Infrastructure Node public domain name>

Both the CSE Middle Node and the CSE Application Service Node are sub-domains of the Infrastructure Node.

The A/AAAA records in the DNS, as per [i.5], [i.6], and [i.10] shall consist of the public domain name of the CSE and the IP address of the M2M Infrastructure Node, since the M2M Infrastructure Node is the entry point of the M2M Service Provider domain name where it belongs to.

Editors Note: The population of the A/AAAA records are FFS and to be elaborated

# 7	M2M Identification and Addressing

This clause provides a list of identifiers required for the purpose of interworking within the oneM2M architectural model.

An Identifier provides unique information or name, tag label which has an identical meaning to the same type of Identifier in the M2M System. For example, a CSE-ID for MN is used for the purpose of CSE identification at MN. A CSE-ID for ASN is used for CSE identification at ASN.

One Identifier shall not be assigned to two or more different entities. An Identifier is used to reference M2M entities during their lifetime. Such Identifiers provide information for association with other Identifiers.

M2M Identification and Addressing provides for the provisioning of different types of M2M Identifiers and the association of such Identifiers with M2M Application Entities, CSEs, M2M devices etc.

M2M Identification and Addressing provides the capability to support relevant CSFs e.g., Registration CSF and Security CSF, and the capability to associate with M2M service and management information as well.

It is assumed that the Application Identifier (App-ID) and the CSE Identifier (CSE-ID) have been assigned initially and are known before M2M System boots up.

Editor’s Note: There is need to clarify the usage of the term CSE-ID/App-ID as opposed to CSE-ID instance/App-ID instance.

## 7.1	M2M Identifiers

### 7.1.1	M2M Service Provider Identifier (M2M-SP-ID)

An M2M Service Provider shall be uniquely identified by the M2M Service Provider Identifier (M2M-SP-ID). This is a static value assigned to the Service Provider.

### 7.1.2	Application Instance Identifier (App-Inst-ID)

An Application Instance Identifier (App-Inst-ID) uniquely identifies an M2M Application instance resident on an M2M node, or an M2M Application instance that requests to interact with an M2M node. An App-Inst-ID shall identify an Application for the purpose of all interactions from/to the Application within the M2M System.

It is the responsibility of the M2M Service Provider to ensure that the App-Inst-ID is globally unique. The App-Inst-ID shall include the Application ID (see section 7.1.3.)

### 7.1.3	Application Identifier (App-ID)

This is equivalent to the application name and is not guaranteed to be globally unique on its own.

Editor's Note:  Who assigns the App-ID is FFS.

### 7.1.4	CSE Identifier (CSE-ID)

A CSE shall be identified by a globally unique identifier, the CSE-ID, when instantiated within an M2M node in the M2M System.

The CSE-ID shall identify the CSE for the purpose of all interactions from/to the CSE within the M2M System.

### 7.1.5	M2M Node Identifier (M2M-Node-ID)

An M2M node, hosting a CSE and/or Application(s) shall be identified by a globally unique identifier, the M2M-Node-ID.

The M2M System shall allow the M2M Service Provider to set the CSE-ID and the M2M-Node-ID to the same value.

The M2M-Node-ID enables the M2M Service Provider to bind a CSE-ID to a specific M2M node.

### 7.1.6	M2M Service Subscription Identifier (M2M-Sub-ID)

The M2M-Sub-ID enables the M2M Service Provider to bind application(s), M2M nodes, CSEs and services identified by service identifiers to a particular M2M service subscription.

Editor's Note: The specification needs to include a section on services, service identifiers.

The M2M Service Subscription Identifier has the following characteristics:

•	belongs to the M2M Service Provider,

•	identifies the subscription to an M2M Service Provider,

•	enables communication with the M2M Service Provider,

•	can differ from the M2M Underlying Network Subscription Identifier,

There can be multiple M2M Service Subscription Identifiers per M2M Underlying Network subscription.

### 7.1.7	M2M Request Identifier (M2M-Request-ID)

This is an identifier that tracks a Request initiated by an application over the Mca reference point, and by a CSE over the Mcc and Mcn reference points, if applicable, end to end. It is also included in the Response to the Request over all the above reference points.

To enable applications to track Requests and corresponding Responses over the Mca reference point, applications shall include a distinct Application Request Identifier per request over the Mca Reference point to the CSE for any initiated request.

If the Application Request Identifier used by the application over the Mca reference point is guaranteed to be globally unique, then the CSE can reuse the Application Request Identifier as M2M Request Identifier over the Mcc and Mca reference points where applicable. If the Application Request Identifier used by the application cannot be guaranteed to be globally unique, then the CSE shall allocate a new globally unique M2M Request Identifier in its interactions with peer CSEs to service the application request. Note that the Request initiated by the CSE could be the result of an application request, or a request initiated autonomously by the CSE to fulfil a service.

Note that the M2M Request Identifier can be made globally unique by including the CSE-ID in combination with any random number.

If the CSE creates an M2M Request Identifier, then the CSE shall maintain a binding between the Application Request Identifier and the CSE M2M Request Identifier it created in its interactions towards other peer CSEs. The CSE shall include the Application Request Identifier in its Response to the application. This binding shall be maintained by the CSE until the Request is deposed off.

A CSE receiving a Request from a peer CSE shall include the received M2M Request Identifier in all additional Requests unspanned (i.e.1:1) it has to generate (including propagation of the incoming Request) and that are associated with the incoming Request, where applicable.

The CSE shall include the same received M2M Request Identifier in its interactions with the Underlying Network Service Entity, over the Mcn reference point where applicable

Editor’s Note:  Bring in a contribution to indicate where this information will carried

Editor’s Note:  In order to allow interworking, how the platform understands whether the Request Identifier is globally unique or not needs to be specified.

### 7.1.8	M2M External Identifier (M2M-Ext-ID)

This is an identifier used by an M2M Service Provider (M2M SP) when services targeted to a CSE, identified by a CSE-ID, are requested from the Underlying Network.

The M2M External Identifier allows the Underlying Network to identify the M2M Device associated with the CSE-ID for the service request. To that effect, the Underlying Network maps the M2M-Ext-ID to the Underlying Network Identifier it allocated to the target M2M Device. In addition, the M2M SP shall maintain the association between the CSE-ID, the M2M-Ext-ID and the identity of the Underlying Network.

Both pre-provisioned and dynamic plug and play association between the CSE-ID with the M2M-Ext-ID should to be supported.

NOTE: There shall be 1:1:1 mapping between M2M-Ext-ID, the identity of the Underlying Network and the CSE-ID. Hence an M2M SP interworking with multiple Underlying Networks shall have a different M2M-Ext-IDs associated with the same CSE-ID, one per Underlying Network and  shall select the appropriate M2M-Ext-ID for any service request it initiates towards an Underlying Network.

NOTE: The mapping by the Underlying Network of the M2M-Ext-ID to the M2M Device is access network specific.

Editor's Note:  The method for dynamic plug-and-play association of M2M-Ext-ID and the 3GPP Underlying Network Identifier is FFS.

Editor's Note: Define "identity of the Underlying Network".

## 7.2	M2M Identifiers lifecycle and characteristics

Table 7.2-1: M2M Identifiers lifecycle and characteristics

| Identifier | Assigned by | Assigned to | Assigned during | Lifetime | Uniqueness | Used during | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M2M Service Provider Identifier | Out of scope | AE, CSE | Out of scope | Out of scope | Global | FFS |  |
| Application Instance Identifier | FFS | AE | Note that the Application instance identifier is created when the executing AE registers to a local CSE | Application Instance Registration | Global | - Application instance Registration - Other operations FFS |  |
| CSE Identifier | M2M SP | CSE | During the first M2M service connection unless pre-provisioned | Life of the CSE | Global | Information flows (section 10) |  |
| M2M Node Identifier | M2M SP | M2M Node hosting CSE | Pre-provisioned | Life of the M2M Node | Global | Out of scope |  |
| M2M Subscription Identifier | M2M SP | Application Entities, and one or more CSEs belonging to the same M2M subscriber | At service signup | Life of the Subscription with the MM SP | Global | - Charging and Information Recorded - Other operations are FFS | Multiple CSEs can be allocated the same M2M Subscription Identifier |
| M2M-Request-ID | Mcc: CSE Mca: Application Instance | A request initiated by an AE or CSE | Mcc: When a request is initiated by a CSE, or handling of a request received by a CSE. Mca: When a request is initiated by an AE | Equal to the lifetime of the Request and its corresponding Response | Mcc: Global Mca: Local or global | Requests and corresponding responses |  |
| External Identifier | Manual mode: Jointly between the Underlying Network provider and M2M SP, where applicable. Plug-And- Play: FFS | M2M Node belonging to a CSE that wants to utilize services of the Underlying Network. | Manual Mode: Administrative Agreement. Plug-And- Play: FFS | Life time of the CSE. | FFS | Requests initiated by a CSE over the Interface, where applicable. |  |

## 7.3	Addressing an Application Entity

### 7.3.1	Application Entity Addressing

In M2M communication, the goal of M2M addressing is to reach the CSE with which the target AE is registered, and ultimately the correct AE on the M2M Node on which the target AE is resident. This principle applies to all Application Entities.

Reachability and routing from/to AEs on M2M Nodes is  associated with the CSEs with which these AEs are registered, and the connectivity of such CSEs to the Underlying Networks. Reaching an AE shall be performed through reaching the CSE the AE is registered with. A CSE-PoA (CSE Point of Access) shall provide the set of information needed to reach a CSE from an Underlying Network perspective. Typically a CSE-PoA contains information that is resolved into a network address.

### 7.3.2		Application Entity Reachability

#### 7.3.2.1		CSE Point of Access (CSE-PoA)

The CSE-PoA shall be used by the M2M System to communicate with a CSE on an M2M Node. Once communication with a CSE is achieved, an AE registered with that CSE can be reached as long as the AE can be uniquely identified.

The information included in the CSE-PoA as well as the refresh of the CSE-PoA, depends on the characteristics of the Underlying Network and the M2M Node transport  capabilities.

#### 7.3.2.2		Locating Application Entities

Locating an AE is a two-step process as follows:

- Step 1: There is a need to locate the CSE where the AE is registered. Locating the CSE shall be accomplished as follows:

- For AEs associated with ASNs/MNs/INs, the CSE-PoA of the ASN-CSE/MN-CSE/IN-CSE where the AE is registered shall be used.

- For AEs associated with ADNs, the CSE-PoA of the MN-CSE/IN-CSE where the ADN is registered  shall be used.

- Step 2:  the CSE shall locate the appropriate AE using its Application Instance Identifier (APP-Inst-ID).

#### 7.3.2.3		Usage of CSE-PoA by the M2M System

The CSE-PoA holds the information used by the M2M System to locate routing information for a CSE. This information shall be provided by the CSE at registration time. However, the routing information related to a CSE (and ultimately to the target AE) in an M2M System depends on the characteristic of the Underlying Network. This impacts the criteria for updating the CSE-PoA by the registered CSE, in addition to the regular CSE registration refresh. The information to be conveyed as CSE-PoA needs to support Underlying Network specifics as well.

Editor's Note: CSE registration refresh procedure is FFS.

In general the addressing and routing information related to a CSE can be achieved when a static public IP address is assigned to and M2M Node and direct DNS address translation or dynamic DNS address translation is used.

In those circumstances, the CSE-PoA for n registered CSE shall have a URI conforming to RFC 3986 [i-4] as follows:

URI = scheme:/fullyqualifieddomainname/path/; or

URI = scheme://ip-address/path/

The following clauses specify the information to be conveyed in the CSE-PoA by a registered CSE for various types of Underlying Networks, as well as the criteria for updating the CSE-PoA for the registered CSEs, in addition to the normal CSE registration refresh.

##### 7.3.2.3.1	CSE-PoA related to CSEs associated with a Fixed Network

In this case the CSE-PoA for a registered CSE shall have a URI as described above. If the IP address is private, then the address is usually built based on the address of the related PPP protocol which is a public IP address. This in turn is mapped to the corresponding private address.

##### 7.3.2.3.2	CSE-PoA related to CSEs associated with Mobile Networks

If the IP address for the registered CSE cannot be reliably used, and cannot be included in the CSE-PoA, then the CSE-PoA for the registered CSE shall include appropriate information as required by the respective Underlying Networks and supported by oneM2M.

Each Underlying Network shall need to specify the means for allowing an M2M SP to fetch the IP address associated with a CSE attaching to that Underlying Network and consequently the information to be included in the CSE-PoA for the registered CSE.

In the event that the M2M SP has connections to multiple  Underlying Networks, there is a need to establish a binding between the registered CSE and the associated Underlying Network. That binding may be established through CSEs explicitly identifying the Underlying Network at registration/update time. Otherwise the M2M SP may derive the identity of the Underlying Network, e.g., by using the link over which the registration arrived, store it and bind it to the registration information.

In the scenarios an M2M Node in mobile networks is not reachable by the previously known IP address and it supports SMS, the originating CSE can make use of SMS for device triggering mechanism to wake up the M2M Node to renew the IP addresses or perform specific functionalities.

To support this option, the CSE-PoA shall, on Mcn interface to the Underlying Networks supporting such an SMS for device triggering mechanism, include information about identification information of the CSE (such as the external identifier as defined by 3GPP TS23.682 in the case of Tsp-based triggering, or MSISDN or any identifier used by triggering network APIs), and send the request to the Underlying Network via the mechanisms supported, such as Tsp, Tsms, Network APIs.

The Annex B shows the 3GPP defined interfaces for machine type communication interfaces and example device triggering flows.

##### 7.3.2.3.3	CSE-PoA to CSEs associated with multiple Underlying Networks

When an M2M Node attaches to a fixed network, the CSE-PoA for a registered CSE shall conform to the procedures associated with the fixed network.

When an M2M Node attaches to a mobile network, the CSE-PoA for a registered CSE shall conform to the procedures associated that mobile network.

If an M2M Node is already attached to an Underlying Network and the attaches to another Underlying Network, the CSE shall update its reachability  information.

Editor's Note: Multiple sets of reachibility information for M2M Nodes accessible via multiple Underlying Networks needs to be supported. This is FFS.

# 8	Description and Flows of Reference Points

## 8.1	General Communication Flow Scheme on Mca and Mcc Reference Points

Procedures involving a CSE and Applications are executed by the exchange of data according to message flow described in this clause.

The exchange of data consists of the information transferred across the reference points, and stored in a standardized resource structure as described in section 9.

The use of the addressable resources also enables the “store-and-share” paradigm of application information and Big Data.  Access and manipulation of the resources is subject to their associated permissions.

Some of the procedures on the Mca and Mcc reference points may adopt a different approach than the ones described in this clause. Such procedures are described in <reference to be included when available>.

Editor's Note: It is suggested to delete the above para if such alternative procedures cannot be identified.

Editors Note: Which of the procedures do not adopt the approach described in this clause and how they are performed is FFS.

### 8.1.1	Description

Figure 8.1.1-1 shows the general flow that governs the information exchange within a procedure, which is based on the use of Request and Response scheme. The scheme applies to communications such as:

- between an Application and a CSE (Mca reference point), and

- among CSEs (Mcc reference point).

The communications can be initiated either by the Applications or by the CSEs.

Figure 8.1.1-1: Resource Manipulation by an Application within an Entity

8.1.2	Request

Request from an Originator to a Receiver includes the following information:

op:  operation to be executed: Create (C), Retrieve (R), Update (U), Delete (D),

to:   URI of the target resource or the parent of the target resoruce, e.g. /m2m.provider1.com/netBase/temp1,

Editor's Note: This is made under the assumption that the CSE-ID is incorporated in the  URI of the target resource or the parent of the target resoruce.

fr:  ID of the Originator,

NOTE:  fr is used for access control of the target resource.

mi:  meta-information about the Request,

Editor's Note: Defaults for meta-information are FFS.

cn:  resource content to be transferred.

NOTE: The to target resource needs to be known by the Originator. It can be known either by pre-provisioning or by discovery.

Editor’s Note: How pre-provisioning and discovery are performed is FFS.

The op information shall indicate the operation to be executed at the Receiver:

Create (C):  a new resource addressable with to parameter is created,

Retrieve (R):  an existing to addressable resource is read and provided back to the Originator,

Update (U):  the content of an existing to addressable resource is replaced with cn new content,

Delete (D):  an existing to addressable resource and all its sub-resources are deleted from the Resource Storage.

Editor’s Note: This is an initial list of the verbs. The need for more verbs is anticipated to cover all use cases, configuration and functions.

The to information shall address the target resource in the Receiver.

The fr information shall be used by the Receiver to check the Originator identity for access permission verification.

The ty information shall be present in Request for the following operations:

Create:  ty is the type of the resource to be created.

The cn information shall be present in Request for the following operations:

Create:  cn is the content of the new resource with the resource type tag as in Table 9.2-1,

Update:  cn is the content to be replaced in an existing resource,

Retrieve:  cn is the filter to be applied for discovery purposes.

Note that the cn information can also be empty.

The mi information shall be as follows:

ot:  optional originating timestamp of when the message was built,

Example usage of the originating timestamp includes: to measure and enable operation (e.g. message logging, correlation, message prioritisation/scheduling, accept performance requests, charging, etc.) and to measure performance (distribution and processing latency, closed loop latency, SLAs, analytics, etc.)

rqet:  optional request expiration timestamp,

Example usage of the request expiration timestamp includes to indicate when request messages (including delay-tolerant) should expire due to their staleness being no longer of value, and to inform message scheduling/prioritisation.

rset:  optional result expiration timestamp,

Example usage of the result expiration timestamp includes to indicate when result messages (including delay-tolerant) should expire due to expected staleness of the result, being no longer of value, and to inform message scheduling/prioritisation. It can be used to set the maximum allowed total request/result message sequence round trip deadline.

rt:  optional response type: indicates  what the response to the issued request shall contain and when the response is sent to the Originator:

- Acknowledgement: In case the request is accepted by the Local CSE, the Local CSE responds after acceptance with an Acknowledgement confirming that the Local CSE will further process the request.

- Result: In case the request is accepted by the Local CSE, the Local CSE responds with the result of the requested operation  after completion of the requested operation.

Example usage of the response type set to Acknowledgment: An Originator that is optimized to minimize communication time and energy consumption wants to express a Request to the local CSE and get an acknowledgement on whether the Request got accepted. After that the Originator may switch into a less power consuming mode and retrieve a Result of the requested Operation at a later time.

rd:  optional result destination:

- Local CSE: The Local CSE includes in the response to an accepted request a reference that can be used to access the status of the request and the result of the requested operation at a later time.

- Originator: The result of the requested operation needs to be sent as a notification to the Originator.

Example usage of the result destination set to Local CSE includes when the result content is extremely large, or when the result consists of multiple content parts from a target group which are to be aggregated asynchronously over time.

rc:  optional result content: Indicates what are the expected components of the result of the requested operation. The Originator of a request may not need to get back a result of an operation at all. This shall be indicated in the rc information. Which exact settings of rc are possible depends on the requested operation specified in op. Possible values of rc are:

- "resource": Only a representation of the requested resources will be returned as content, without any mention of the links of the requested resources to other resources. This is the default value.

- "resource;links-of-types:A,B,C": A representation of the requested resources, along with links of types A, B and C to other resources (possibly limited by a maximum number of retrieved links), will be returned as content.

Editor's Note: Link types A, B, C are examples and are FFS. Or should it be converted to a NOTE.

- "resource;links": A representation of the requested resources, along with all links to other resources (possibly limited by a maximum number of retrieved links), will be returned as content.

- "links": Only the links of the requested resources to other resources (possibly limited by a maximum number of retrieved links), without any representation of the actual resources requested, will be returned as content.

- "nothing": Nothing will be returned as content of the Response.

- "result-notification": A notification will be returned when the operation is completed.

Editor’s Note: Details on what settings for rc are possible can be added at a later when the specific operations that can be requested are defined (e.g. register an AE, modify content of a container, etc.)

rp:  optional response persistence: indicates the duration for which the address containing the responses is to persist.

Example usage of response persistence includes requesting sufficient persistence for analytics to process the response content aggregated asynchronously over time. If an expiration timestamp is specified then the response persistence should last beyond the Request expiry time.

ri:  Request Identifier mandatory over the Mca, Mcc, and Mcn reference points.

Example usage of request identifier includes enabling the correlation between a Request and one of the many received Responses.

oet:  optional operational execution time: indicates the time when the specified operation op is to be executed by the target CSE. A target CSE shall execute the specified operation of a Request having its operational execution time indicator set, starting at the operational execution time. If the execution time has already passed or if the indicator is not set, then the specified operation shall be immediately executed, unless the expiration time, if set, has been reached.

Example usage of operational execution time includes asynchronous distribution of flows, which are to be executed synchronously at the operational execution time.

NOTE: Time-based flows may not be able to be supported depending upon time services available at CSEs.

ls:  optional lifespan: Indicates time limit until when the delivery of a Request to a Hosting CSE that is different from the CSE that has received the request.

Example usage of  lifespan: When a request has set lifespan to a specific time and the Request demands an operation on Hosting CSE that is not the CSE currently processing the request, then the current CSE shall keep on trying to deliver the request to the Hosting CSE in line with provisioned policies,

ec:  optional event category: Indicates the event category that should be used to handle this request. Event categories are impacting how Requests to access remotely hosted resources are processed in the CMDH CSF. Selection and scheduling of connections via CMDH are driven by policies that can differentiate event categories.

Editor’s Note: Details on how to limit use of ec by AEs and relationship to provisioned policies are FFS.

Example usage of "event category" set to specific value X: When the request is demanding an operation to be executed on a Hosting CSE that is different from the Local CSE, the request may be stored in the CSE that is currently processing the request on the way to the Hosting CSE until it is allowed by provisioned policies for that event category X to use a communication link to reach the next CSE on a path to the Hosting CSE or until the lifespan of the request is expired.

da:  optional delivery aggregation on/off: Use CRUD operations of <delivery> resources to express forwarding of one or more original requests to the same target CSE(s).

NOTE: Since da is optional, there could be a default value to be used when not present in the Request. This 	parameter 	may not be exposed to AEs via Mca.

Example usage of delivery aggregation set on: The CSE processing a request shall use aggregation of requests to the same target CSE by requesting CREATE of a <delivery> resource on the next CSE on the path to the target CSE.

gid:  optional group request identifier: Identifier added to the group request that is to be fanned out to each member of the group.

Once the Request is delivered, the Receiver shall analyze the Request to determine the target resource.

If the target resource is addressing another M2M node, the Receiver shall route the request appropriately.

If the target resource is addressing the Receiver, it shall:

- Check the existence of to addressed resource,

- Identify the resource type with the tag value as in Table 9.2-1,

- Check the permission for fr Originator to perform the requested operation,

- Perform the requested operation (using cn content when provided)

- Perform the requested operation (using cn content when provided) according to the operational execution time

- Depending on the request result content, respond to the Originator with indication of successful or unsuccessful operation results. In some specific cases (e.g. limitation in the binding protocol or based on application indications), the Response could be avoided.

The message flow procedure started with an Originator Request shall be considered closed when either:

- A Request is received with an expired rqet (request expiration timestamp),

- A Response is delivered to the Originator,

- The requested operation at the Receiver is successfully completed and no Response is needed.

#### 8.1.2.1	Summary of Request Message Parameters

Table 8.1.2.1-1summarises the parameters specified in section 8.1.2 for the Request message, showing any differences as applied to C, R, U or D operations. “M” indicates mandatory, “O” indicates optional, “N/A” indicates “not applicable”.

Table 8.1.2.1-1: Summary of Request Message Parameters

| Request Message Parameter \ Operation | Create: a new resource is created | Retrieve: an existing resource is read and the result is provided | Update: the content of an existing resource is replaced with new content | Delete: an existing resource and all its sub-resources are deleted |
| --- | --- | --- | --- | --- |
| To (to) – the address of the target resource on the target CSE | M | M | M | M |
| From (fr) – the identifier of the message Originator | M | M | M | M |
| Request Identifier (ri) – uniquely identifies a Request message | M | M | M | M |
| Resource Type (rt) - of resource to be created | M | N/A | N/A | N/A |
| Content (cn) - to be transferred | O | O | M | N/A |
| Originating Timestamp (ot) - when the message was built | O | O | O | O |
| Request Expiration Timestamp (rqet) – when the request message expires | O | O | O | O |
| Result Expiration Timestamp (rset) -when the result message expires | O | O | O | O |
| Operational Execution Time (oet) - the time when the specified operation is to be executed by the target CSE. | O | O | O | O |
| Response Type (rt) – local CSE response shall either indicate that the Request was accepted, or include the operation result | O | O | O | O |
| Result Destination (rd) – either a reference to where the result is stored, or the result is sent to the Originator | O | O | O | O |
| Result Persistence (rp) - the duration for which the reference containing the responses is to persist | O | O | O | O |
| Result Content (rc) – the expected components of the result | O | O | O | O |
| Lifespan (ls) - time after which the system should not deliver the message | O | O | O | O |
| Event Category (ec) – indicates how and when the system should deliver the message | O | O | O | O |
| Delivery Aggregation (da) - aggregation of requests to the same target CSE is to be used | O | O | O | O |

### 8.1.3	Successful Operation

The Response from the Receiver of a Request to the Originator of that Request, in the case of a successful completion of the requested operation includes the following information:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request,  “rd” information can be used.

fr: Optional. ID of the Receiver.

mi:  meta-information about the Request,

Editor's Note: Defaults for meta-information are FFS.

rs:  operation result: e.g. Okay, Okay and Done; Okay and Scheduled; Okay and In Progress, etc.,

ai:  optional additional result information,

cn:  resource content to be transferred (optional).

The mi information includes:

ot:  optional originating timestamp of when the message was built,

rset:  optional result expiration timestamp. The Receiver shall echo the result expiration timestamp if set in the Request message, or may set the result expiration timestamp itself.

Example usage of the Receiver setting the result expiration timestamp is when the value of the delivery time is dependent upon some changing Receiver context eg. Result message deadline for aircraft position based upon velocity.

ri:  Request Identifier mandatory in all Responses over the Mca, Mcc, and Mcn reference points.

Note that ri in Response shall match the ri in the corresponding Request.

The ai information includes:

cs:  optional, status codes (e.g., authorization timeout, etc.)

ra:  optional, address for the temporary storage of end node Responses.

The cn information may be present in a Response in the following cases:

Create:  cn is the address and/or the content of the created resource,

Update:  cn is the content replaced in an existing resource,

Delete:  cn is the content actually deleted.

The cn information shall be present in a Response in the following cases:

Retrieve:  cn is the retrieved resource content or aggregated contents of discovered resources.

### 8.1.4	Unsuccessful Operation

The Response from the Receiver of a Request to the Originator of the Request, in the case of unsuccessful completion of the requested operation includes the following information:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request , “rd” information can be used.

fr: Optional. ID of the Receiver.

mi:  meta-information about the Request,

Editor's Note: Default for meta-information are FFS.

rs:  operation result: e.g. Not okay.

ai:  optional additional result information, e.g. status codes.

The mi information includes:

ot:  optional originating timestamp of when the message was built,

rset:  optional result expiration timestamp.

ri:  Request Identifier mandatory in all Responses over the Mca, Mcc, and Mcn reference points.

Note that ri in a Response shall match the ri in the corresponding Request

The ai information includes:

cs:  optional, status codes (e.g., authorization timeout, etc.)

### 8.1.5	Summary of Response Message Parameters

Table 8.1.5-1 summarises the parameters specified in sections 8.1.1 and 8.1.3 for the Response messages, showing any differences as applied to successful C, R, U or D operations, and unsuccessful operations. “M” indicates mandatory, “O” indicates optional, “N/A” indicates “not applicable”.

Table 8.1.5-1: Summary of Request Message Parameters

| Response Message Parameter \ Success or not | Successful Operation: Create: a new resource is created | Successful Operation: Retrieve: an existing resource is read and the result is provided | Successful Operation: Update: the content of an existing resource is replaced with new content | Successful Operation: Delete: an existing resource and all its sub-resources are deleted | Unsuccessful Operation |
| --- | --- | --- | --- | --- | --- |
| Request Identifier (ri) – uniquely identifies a Request message | M | M | M | M | M |
| Result (rs) - operation result: e.g. Okay, Okay and Done; Okay and Scheduled; Okay and In Progress, etc., | M | M | M | M | M |
| Status codes (cs) - (e.g., authorization timeout, etc.) | O | O | O | O | O |
| Response Address (ra) - address for the temporary storage of end node Responses. | O | O | O | O | N/A |
| Content (cn) - to be transferred | O (The address and/or the content of the created resource) | M (the retrieved resource content or aggregated contents of discovered resources) | O (The content replaced in an existing resource) | O (The content actually deleted) | N/A |
| Originating Timestamp (ot) - when the message was built | O | O | O | O | O |
| Result Expiration Timestamp (rset) – when the message expires | O | O | O | O | O |

## 8.2	Procedures for Accessing Resources

This clause describes the procedures for accessing the resources. The procedures include:

- Generating Responses to the Requests for accessing resources,

- Accessing the resources that may be located in different CSEs.

Editor’s Note: Other procedures related to accessing the resources are FFS.

### 8.2.1	Generating Responses

The following types of Responses are supported for accessing the resources over the Mca and Mcc reference points:

- Success: indicates to the Originator that the Request has been executed successfully by the Hosting CSE.

- Failure: indicates to the Originator that the Request has not been executed successfully by the Hosting CSE.

Both, the Success and the Failure Response types may convey other information – e.g., the result of the requested operation to the Originator of the Request. More specific details on what other information shall be conveyed with the responses may be defined by using the rc information as part of the original request, see section 8.1.2.

Detailed Success and Failure codes are provided in the Protocol specifications.

Editor’s Note: References to Protocol specifications will be provided when available.

- In addition to the above mentioned Success and Failure Response types, there is another Response type that indicates an Acknowledgement of the Request. This indicates to the  entity that issued the Request that the Request has been received and accepted by the  attached CSE, i.e., by the CSE that received the Request from the issuing entity directly, but it has not executed the Request yet. The Success or the Failure of the execution of the Request is to be conveyed later.

Editor's Note: Details of Acknowledgement Response type is FFS.

### 8.2.2	Accessing Resources in CSEs – Blocking Requests

For the procedures described herein, the addressed resource can be stored in different CSEs. Table 8.2.2-1 describes the possible scenarios, where the addressed resource may be on the local-CSE or on a CSE located elsewhere in the oneM2M System.

In this sub-clause – for simplicity – it is assumed that the Originator of a Request can always wait long enough to get a Response to the Request after the requested operation has finished. This implies potentially long or unknown blocking times (time for which a pending Request has not been responded to) for the Originator of a Request.

For scenarios that avoid such possibly long blocking times, section 8.2.3 specifies mechanisms to handle synchronous and asynchronous resource access procedures via returning appropriate references.

Editor’s Note: How the target of a Request (i.e, an Application or local/hosting CSE) is identified and addressed, is FFS.

Editor’s Note: A message (Request, Response) can be forwarded immediately or forwarded at a later time (e.g in the case of congestion).The procedures related to the forwarding of the Request at a later time are FFS.

Table 8.2.2-1: Accessing Resources in different CSEs

| Traversals across Mca/Mcc Reference Points | Description | Reference |
| --- | --- | --- |
| No Hops | - The Originator of the Request accesses a resource. - The Originator of the Request can be an Application or a CSE. - Local CSE and Hosting CSE are the same entity. - The CSE shall check the Access Rights for accessing the resource. - Depending on the expected result content, the CSE shall respond to the Originator of the Request, either with a Success or Failure Response, or not at all. | Figure 8.2.2-1 |
| 1 Hop | - The Originator of the Request accesses a resource. - The Originator of the Request may only be an Application. - Local CSE and Hosting CSEs different entities. - Local CSE shall forward the Request to the Hosting CSE, after an optional checking of the Access Rights for accessing the resource and the syntax of the Request message. - Hosting CSE shall check the Access Rights for accessing the resource and depending on the expected result content respond with a Success or Failure Response, or not at all. | Figure 8.2.2-2 (Editor's Note-1) |
| Multi Hops | - The Originator of the Request accesses a resource. - The Originator of the Request may be an Application or a CSE. - Local CSE, Intermediate CSE(s) and the Hosting CSE are different entities. - Local CSE shall forward the Request to an Intermediate CSE (e.g. MN-CSE) that the Local CSE is registered with, if it cannot communicate with the Hosting CSE directly. An optional checking of the Access Rights for accessing the resource and the syntax of the Request message may be performed prior to forwarding the Request. - Intermediate CSE may forward the Request to another Intermediate CSE (e.g. another MN-CSE) that the Intermediate CSE is registered with, if it cannot communicate with the Hosting CSE directly. An optional checking of the Access Rights for accessing the resource and the syntax of the Request message can be performed prior to forwarding the Request. - The Intermediate CSE shall forward the request to the Hosting CSE. An optional checking of the Access Rights and the syntax of the Request message can be performed prior to forwarding the Request. - Hosting CSE shall check the Access Rights for accessing the resource and depending on the expected result content respond with a Success or Failure Response, or not at all. | Figure 8.2.2-3 (Editor's Note-2) |
| Editor's Note-1: One-Hop case could potentially include the CSE-to-CSE communication also. The need for such procedures is FFS. Editor's Note-2: The multi-hop procedures address the scenario when the target is on a specific remote CSE. How a flow will work when the target is distributed on multiple remote CSEs is FFS. |  |  |

Figure 8.2.2-1: Originator accesses a resource on the Local CSE (No Hops)

Figure 8.2.2-2: Application accesses a resource at the Hosting CSE (One Hop)

Figure 8.2.2-3: Originator accesses a resource at the Hosting CSE (Multi Hops)

### 8.2.3	Accessing Resources in CSEs – Non-Blocking Requests

#### 8.2.3.1	Response with Reference to Result of Requested Operation

In case the Originator of a Request has asked for only responding with an Acknowledgement of the Request and a reference to the result of the requested operation – see rt information in section 8.1.2 – it is necessary to provide a prompt response to the Originator with a reference to an internal resource on the Local CSE or another specified reference, so that the Originator can retrieve the outcome of the requested operation at a later time. The details of such an internal resource are defined in section 9.6.X. The reference is provided in the response to the Request. The abbreviation "Req-Ref" is used for simplicity in the figures of the following clauses.

Two different cases are defined in the following two clauses for allowing the Originator of a request to retrieve the result of a requested operation.

#### 8.2.3.2	Synchronous Case

In the synchronous case, it is assumed that the Originator of a Request is not able to receive asynchronous messages, i.e. all exchange of information between Originator and Local CSE need to be initiated by the Originator.

In that case the information flow depicted in Figure 8.2.3.2-1 is applicable. For the flow depicted in Figure 8.2.3.2-1 it is assumed that completion of the requested operation happens before the Originator is trying to retrieve the result of the requested operation with a second request referring to the "Req-Ref" provided in the Response to the original Request.

Another variation of the information flow for the synchronous case is depicted in Figure 8.2.3.2-2. In this variation it is assumed that the requested operation completes after the second request but before the third request sent by the Originator.

Equivalent information flows are valid also for cases where the target resource of the requested operation is not hosted on the Local CSE. From an Originator’s perspective there is no difference as the later retrieval of the result of a requested operation would always be an exchange of Request/Response messages between the Originator and the Local CSE using the reference to the original request.

Figure 8.2.3.2-1:  Non-blocking access to resource in synchronous mode 
(Hosting CSE = Local CSE), requested operation completed before second request.

Figure 8.2.3.2-2: Non-blocking access to resource in synchronous mode 
(Hosting CSE = Local CSE), requested operation completed after the second but before the third request.

#### 8.2.3.3	Asynchronous Case

In the asynchronous case, it is assumed that the Originator of a Request is actually able to receive notification messages, i.e. the Local CSE could send an unsolicited message to the Originator at an arbitrary time to send the result to a notification target. The possible mechanisms how the notification then reaches the Originator are the same as in the case of a notification after a subscription.

In that case the information flow depicted in Figure 8.2.3.3-1 is applicable. In this case it is assumed that the Originator of the Request provided a reference – notificationReference – for notification when the result of the requested operation is available.

Equivalent information flows are valid also for cases where the target resource of the requested operation is hosted on the Local CSE. From an Originator’s perspective there is no difference as the later notification of the result of a requested operation would always be an exchange of request/response messages between the Originator and the Local CSE using reference to the original Request.

Figure 8.2.3.3-2: Non-blocking access to resource in notification mode 
(Hosting CSE not equal to Local CSE), Originator provided reference for notification.

8.3.	Description and Flows on Mcn Reference Point

### 8.3.1	General Description

Communications between the CSEs and the NSEs (Network Service Entities) use the Mcn reference point, which includes the CSE(s) accessing network service functions provided by Underlying Networks, and optimizing network service processing for Underlying Networks. Such services normally are more than just the general transport services.

Communications which pass over the Mcn reference point to Underlying Networks include multiple mechanisms such as:

- Messaging services that are widely deployed by Applications and network operators using a number of existing mechanisms,

- Network APIs defined by other SDOs (e.g., OMA and GSMA) are used by network operators for their services,

- Interworking for services and security aspects for MTC (Machine Type Communications) has been defined by 3GPP.

Examples of service requests that an IN-CSE can communicate to the Underlying Networks are:

- Connection requests with/without QoS requirements,

- Payments, messages, location, bearer information, call control and other network capabilities, (currently supported by GSMA oneAPI now),

- Device triggering,

- Device management,

- Management information exchange such as charging/accounting records, monitoring and management data exchange.

Editor’s Note:  We need a definition for device triggering.

Editor's Note:  The above is example of services requests from IN-CSE to Underlying Networks. To provide similar examples of service requests from ASN-CSEs and MN-CSEs to the Underlying Networks.

# 9	Resource Management

All entities in the oneM2M System, such as Applications, CSEs, "data", etc., are represented as "resources". A "resource structure" is specified as a representation of the "resources". Such "resources" are uniquely addressable. Procedures for accessing such resources are also specified.

## 9.1	General Principles

The following are the general principles for the design of the resource model.

- The "type" of all resource shall be specified. New resource types shall be supported as the need for them is identified;

- The root of the resource structure in a CSE shall be assigned an absolute address, such as <baseURI>;

- The resource structure shall be kept reasonably simple. In particular:

- <Application> Resources belonging to the local CSE shall be located directly under the baseURI resource,

- <Access Right> Resources belonging to the local CSE shall be located either directly under the baseURI resource, or under an <Application> resource.

- <Group> resources belonging to the local CSE shall be located either directly under the baseURI resource, or under an <Application> resource.

- <remoteCSE> resources representing a remote CSE shall be located directly under the baseURI resource,

- <Container> Resources belonging to the local CSE shall be located either under the baseURI resource or, or under an <Application> resource, or under another <container> resource;

Editor’s Note: These above stated resource types needs to be validated as for section 9.6.

- The attributes for all resource type shall be specified;

- Each resource type can have multiple instances;

- All resource and associated attributes shall be uniquely addressable via their associated Universal Resource Identifiers (URI), consisting of:

- a structured URI based on the chain of child-parent relations

- a flat URI made of a unique identifier addressable via the base root

Editor’s Note: Contribution #0505R1 already address the fact that URI is not used in a proper manner in the TS. However the term structured and flat URI still use the term URI. Detailed definition on how to do addressing in oneM2M still needs to be drafted. Alignment of the proper terminology in the TS is needed.

Editor’s Note: how the flat URI can be distinguished from the structured URI is for FFS.

Editor’s Note: if there is a need to provide both structured URI or flat URI of a resource if for FFS.

- Examples:

- Root example “myCSE” (well known, common to structured and  flat)

- Example 1: the following two examples address the same resource

- //myCSE/123(structured)

- //myCSE/123 (flat)

- Example 2: the following two examples address the same resource

- //myCSE/myApplication/987 (structured)

- //myCSE/234 (flat)

- Example 3: the following two examples address the same resource

- //myCSE//myContainer (structured)

- //myCSE/mC (flat)

- The flat URI is always stored in the parent attribute, that is a mandatory attribute

- In case of structured URI, the relationship parent-child and vice-versa is determined in the structured URI. When created via the flat URI, the parent child-relation is determined by the parentID attribute.

- This, however, does not imply any specific storage structure for the <container> resource. The structure of a <container> resource is subject to implementations.

## 9.2	Resource Types

This clause introduces the types of resources used in a CSE. A resource scheme is used for modelling the resource structure and associated relationships.

Table 9.2-1: Resource Types and Tagging

| Resource type Editor's Note-2. Editor's Note-3. | Tag value | Description |
| --- | --- | --- |
| Base URI Resource: <baseURI> | B | Base URI resource shall be the root of the resource structure. It shall store information about the local CSE. All resources belonging to the local CSE shall be child resources of the Base URI resource. |
| CSE Resource: <entity> | E | Entity resource shall store information related to CSEs. Editor's Note-1. |
| Application Resource: <application> | A | Application resource shall store information about the Applications. Application resource is created as a result of successful registration of an Application with the local CSE. Applications shall register with their local CSE only. |
| Access Right Resource: <accessRight> | R | AccessRight resource shall store a representation of the permissions. An accessRight resource is associated with resources that shall be accessible to entities external to the hosting CSE. Basically, accessRight resource controls "who" (permissionHolder) is allowed to do "what" (permissionFlag). It can be used for privacy protection as well. |
| Container Resource: <container> | C | Container resource is a generic resource that shall be used to exchange "data" between Applications and/or CSEs. "Container" is used as a mediator that takes care of buffering the data. The exchange of data between Applications (e.g. an Application on an end-device and the peer-Application on the network side) is abstracted from the need to set up direct connections and allows for the scenarios where both parties in the exchange are not online at the same time. |
| Group Resource: <group> | G | Group resource shall store information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group. |
| Editor's Note-1": For the <Entity> resource storing information related to CSEs: Is the reference to local CSEs or to both the local and remote CSEs. Need to be clarified. Editor's Note-2: The type of resources included in this table is not complete yet. This is FFS. Editor’s Note-3: As the work progresses, the resource types may change. This is FFS. |  |  |

Editor'Note: Resource Type Tag in the table above (Table 9.2-1) seems to have the same function as the common attribute "resourceType" which is defined in section 9.6.1. There is inconsistency amongst the two: single character in Table 9.2.1 and two capital-letters in section 9.6.1 definition of "resourceType" attribute. It is proposed that a single naming convention be used - and all resource types be defined in Table 9.2.1 - possibly using two capital-letters.

Editor's Note:  The use of the notation of pointy brackets <...> is for indicating "resource type". For the case of <baseURI>,  they (pointy brackets) have been used in another way. Instead of being used to indicate a "resource type", they are being used as a placeholder for an address {which could be URI}. Since the URI has nothing to do with the "resource type", it is suggested to call this Base Resource as either the "Base Resource <base>", or the "Base-CSE Resource <baseCSE>".  If such change is agreed, it needs to be reflected in the "General Principles" section 9.1 as well. Need discussions and resolution with the WG.  If agreed, it will have impact on Figure 9.4.1 as well.

The resources are indicated in the following notation:

| <name>: | Indicates the resource name assigned during the creation of the resource. The name may be provided by the Originator of the Request that created the resource or by the CSE that hosts the resource (if name is not provided by the Originator). |
| --- | --- |
| Attributes | FFS |
| Editor’s Note: Attributes for the Resource Types are FFS. |  |

Editor's Note: The term “name” does not seem appropriate in <name> in the table above. It is probably meant to be the Resource Type. Whenever we have <xxxxx>, the string within < >, should be a resource type identifier. Needs discussions and resolution within the WG. If agreed, the descriptive section in the table above also needs to be adjusted.

### 9.2.1	Types of Virtual Resources

A virtual resource is a resource that is not necessarily present in the resource structure. A virtual resource can be addressed to trigger actions and to retrieve some processing results that shall be represented in an agreed format.

Editor’s Note: The list of virtual resources is FFS. E.g., <Members> is one of the virtual resource type specified in this document.

## 9.3	Resource Addressing

Resources shall be uniquely addressable via URI. The following are examples of addressable resources.

/baseRoot/Entity Instance 1

/baseRoot/Entity Instance n

/baseRoot/Application Instance 1

/baseRoot/Application Instance n

/baseRoot/Container1/Container2

Resources may be associated by using a link with reference to another resource.

The baseRoot shall contain all resources of the hosting node. A baseRoot is the root of all other resources it contains. The baseRoot shall represent the first element of the path representing a resource in an absolute URI, as per [i-4]. All other resources hosted in the CSE shall also be identified by an absolute URI.

The authority part of the absolute URI, as per [i.9] and [i.4] shall be either an Infrastructure Node public domain name, a Middle Node public domain name or an Application Service Node public domain name and shall conform to the naming convention described in section 6.5.1.1.

Example of a URI identifier of a container resource hosted by a CSE could be "http://IntermediatenodeCSEID.infrastructurenodeCSEID.m2m.myoperator.org/baseRoot/container1/myExampleContainer2"

Editor's Note: With this change of /baseURI to /baseRoot:  we need to examine the whole specification for the appropriate use of the /baseRoot vs. /baseURI  terminology. It is suggested to identify a task group to review the document and suggest appropriate changes.

## 9.4	Resource Structure

### 9.4.1	Links between resources

Figure 9.4.1-1: Topological Resource Structure at a CSE

Editor's Note:  The resource structure picture needs to be updated as the resource structure evolves.

As represented in the figure above, resources in the oneM2M System may be linked with each other via unidirectional or bidirectional links.

NOTE: A resource can also be linked using an attribute containing the linked resource ID.

A link shall contain the following information:

- linkedResourceURI: The target linked resource, usually given by using the URI of that resource

- linkType: The type of the link, describing the relationship that the current resource has with the other linked resource (only in one direction, i.e. from this resource to the other linked resource).

A link may also contain the following information:

- linkedResourceType: The type of the target linked resource

Editor’s Note: Links management (creation, deletion, ,..) is FFS.

9.4.2 Link Types

The following link types are defined:

- TBD.

Editor's Note: The types of links is FFS.

## 9.5	Resource Specification Guidelines

The following guidelines shall be used for the specification of resources.

Resources shall be specified by a tabular notation and the associated graphical representation as follows:

- The resources shall be specified in association with a CSE. The resources are the representation in the CSE of the components and elements within the M2M System. Other CSEs, Applications, application data representing sensors, commands, etc. are known to the CSE by means of their resource representation.

Resource: is a uniquely addressable entity in the RESTful architecture. A resource shall be transferred and manipulated using the verbs, such as those specified in section 8.1.

- A resource may contain child resource(s) and attribute(s).

Child Resource: It is a resource that has a containment relationship with the addressed (parent) resource. The parent resource representation contains references to the child resources(s). The lifetime of a child resource is limited by the parent's resource lifetime.

Attributes: store information about the resource itself. The set of attributes, which are common to all resources, are not detailed in the graphical representation of a resource.

- When an instance of a resource is created, a URI is assigned. URI is used to uniquely address the resource.

- Resources and attributes are addressed in the same manner, e.g., by the use of the URI.

- Resource names and attribute names are strings in lower case. In case of a composed name, the subsequent word(s) start with a capital letter; e.g., accessRights, creationTime, expirationTime.

Editor's Note: Clarification is needed on how acronyms should be handled.  For example we could choose nodeID or nodeId. It is proposed that acronyms/abbreviated-names are always CAPITALIZED.

- A string delimited with ‘<’ and ‘>’ e.g., ‘<resourceType>’ is a placeholder for the type of a resource.

- A collection is a parent resource that contains attributes and child resources. All child resources of a collection are of the same type. A collection is represented by a collection name.

- Read/Write access modes for attributes can assume the following values:

- Read/Write (RW), it can be set by a Create or an Update operation,

- Read Only (RO), it is set by the CSE when the resource is Created and/or Updated. Such an attribute can only be read,

- Write Once (WO), it is set by the Create operation. It can then only be read.

- The graphical representation of a resource shows the multiplicity of the attributes and child resources. The following graphical representations are used for representing the attributes, child resources and collection resources:

- Square boxes are used for resources and child resources

- Square boxes with round corners are used for attributes

- Parallelograms are used for collection resources.

- If a child resource of a given resource type with a specific name or an attribute can occur more than once within the containing (parent) resource and if the child resource/attribute is created at runtime then the name of the child resource or the attribute shall consist of a fixed part and a variable part separated with ‘_’ (underscore) between the two parts. The fixed part is a character string and the variable part is indicated by [x] whereby the distinguishable value of [x] (numbers and/or strings) can be assigned by the Originator, or assigned by the CSE if not provided by the Originator. For example:

- For the child resource case, childResourceName_[x] will be instantiated as childResourceName_1, childResourceName_2 etc.

- For the attribute case, attributeName_[x] will be instantiated as attributeName_abc, attributeName_def, etc.

### 9.5.1	Resource Template

#### 9.5.1.1	Resource Type resourceType

<Fill a short description of the resourceType>

Figure 9.5.1.1-1:	Structure of resource <resourceType>
(only child resources and resource specific attributes are shown)

The resource <resourceType> shall contain child resources according to their multiplicity shown in Table 9.5.1.1-1. A value of "0" for multiplicity indicates the child resource is optional.

Table 9.5.1.1-1: Child Resources of <resourceType>

| Child Resource Name of <resourceType> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| <Fill in the name of Child Resource1 if a fixed name is required or [variable] if no fixed name is required> | <Fill in the type of Child Resource1> | <Fill in Multiplicity> | See section <XRef <section> where the type of this child resource is described. |
| <Fill in the name of Child ResourceN if a fixed name is required or [variable] if no fixed name is required> | <Fill in the type of Child ResourceN> | <Fill in Multiplicity> | See section <XRef <section> where the type of this child resource is described. |
| <Fill in the name of Child Collection1 if a fixed name is required or [variable] if no fixed name is required> | Collection of <Fill in the type of resources allowed in this collection> | <Fill in Multiplicity> | See section <XRef <section> where the resource type that is allowed in this child collection resource is described. |
| <Fill in the name of Child CollectionN if a fixed name is required or [variable] if no fixed name is required> | Collection of <Fill in the type of resources allowed in this collection> | <Fill in Multiplicity> | See section <XRef <section> where the resource type that is allowed in this child collection resource is described. |

<Fill Additional information about child resources (as needed)>.

The resource <resourceType> shall contain attributes according to their multiplicity shown in Table 9.5.1.1-2.  A value of "0" for multiplicity indicates the attribute is optional.

Table 9.5.1.1-2: Attributes of <resourceType>

| Attribute Name of <resourceType> | Multiplicity | RW/ RO/ WO | Description | <announcedResourceType> (M/O/NA) |
| --- | --- | --- | --- | --- |
| <Fill in name of Common Attribute1> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute…to be moved later to a common attribute section. | <Fill as in specified in section 9.6.22> |
| <Fill in name of Common AttributeN> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute…to be moved later to a common attribute section. | <Fill as in specified in section 9.6.22> |
| <Fill in name of Resource Specific Attribute1> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute .. to be moved later to a central attribute table that also defines the type of the attribute, allowed ranges etc. | <Fill as in specified in section 9.6.22> |
| <Fill in name of Resource-Specific AttributeN> | <Fill in Multiplicity> | <Fill in RW or RO or WO> | Provide description of this attribute .. .. to be moved later to a central attribute table that also defines the type of the attribute, allowed ranges etc. | <Fill as in specified in section 9.6.22> |

<Fill additional information about attributes (as needed)>.

### 9.5.2	Examples for Resource Specification

#### 9.5.2.1	Example of a Resource with attributes only

##### 9.5.2.1.1	Resource Type accessRight

Access rights are defined as "white lists" or permissions, i.e. each permission defines "allowed" entities (defined in the permissionHolders) for certain access modes (permissionFlags). Sets of permissions are handled such that the resulting permissions for a group of permissions are the sum of the individual permissions. I.e., an action is permitted if it is permitted by some / any permission in the set.

By setting an accessRightID attribute on a resource, the permissions for accessing that resource are then defined by the permissions defined in the accessRight resource.

Figure 9.5.2.1.1-1: Structure of resource <accessRight>
(only resource specific attributes are shown)

The resource <accessRight> shall contain the attributes according to their multiplicity shown in Table 9.5.2.1-1.

Table 9.5.2.1.1-1: Attributes of <accessRight>

| Attribute Name of <accessRight> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime | 1 | RW | See section <XRef <section> where attribute expirationTime is described. |
| creationTime | 1 | RO | See section <XRef <section> where attribute creationTime is described. |
| lastModifiedTime | 1 | RO | See section <XRef <section> where the attribute lastModifiedTime is described. |
| Permissions | 1 | RW | The list of permissions defined by accessRight resource. These permissions are applied to resources referencing the accessRight resource using the accessRightID attribute. |
| selfPermissions | 1 | RW | Defines the list of permissions for the accessRight resource itself. |

The permissionFlags and the permissionHolders could then be generalised to actions (which might be granting access, but might also be more specific, like granting access to a subset, i.e. filtering part of the data). The permissionHolders could be generalised to conditions, which may include things like the identity of the requestor, everybody except specified identities, but it might also include time based conditions etc.

#### 9.5.2.2	Example of a Resource with attributes and child resources

##### 9.5.2.2.1	Resource Type container

This resource represents a container for instances. It is used to share information among other entities and potentially for tracking the data.

Figure 9.5.2.2.1-1: Structure of resource <container>
(only child resources and resource specific attributes are shown)

The resource <container> shall contain child resources according to their multiplicity shown in Table 9.5.2.2-1. A value of "0" for multiplicity indicates the child resource is optional.

Table 9.5.2.2.1-1: Child resources of <container>

| Child Resource Name of <container> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <container> | 0..n | XRef <section> where the container resource is described. |

<Additional information about child resources (as needed)>

The resource container shall contain the attributes according to their multiplicity as shown in Table 9.5.2.2-2. A value of  "0" for multiplicity indicates that the attribute is optional.

Table 9.5.2.2.1-2: Attributes of <container>

| Attribute Name of <container> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime | 1 | RW | XRef <section> where this attribute is described. |
| accessRightID | 1..n | RW | XRef <section> where this attribute is described. |
| creationTime | 1 | RO | XRef <section> where this attribute is described. |
| lastModifiedTime | 1 | RO | XRef <section> where this attribute is described. |
| maxNrOfInstances | 0..1 | RW | Maximum number of instances. |
| maxByteSize | 0..1 | RW | Maximum number of bytes that are allocated. |
| maxInstanceAge | 0..1 | RW | Maximum age of the instances. |

<Additional information about child resources (as needed)>

### 9.5.3	Handling of Unsupported and non compliant Resources

Any AE or CSE shall ignore a resource and resource attribute received in Request or a Response, in case such resource or attribute is non compliant to the version of oneM2M specification supported.

Any AE or CSE shall ignore an unsupported optional resource and unsupported optional resource attribute received in a Request or Response.

## 9.6	Specified Resource Types

### 9.6.1	Common Attributes

Many of the attributes of the resources described in the present document are common. Such attributes are described here once in order to avoid duplicating the description for every resource that contains it.

Attributes that are only used in one or two resource types are described only in the section for that resource type.

In all tables representing resource attributes, each attribute name is followed by its “short name” in brackets that shall be used in resource representations to be transferred across reference points.

Table 9.6.1-1: Common Attributes

| Common Attribute | Description |
| --- | --- |
| resourceType (rT) | Resource Type. This WO resourceType attribute stores <in a two capital letters token> the information useful for procedure processing, e.g. CO for <container>, CI for <instance>. |
| parentID (pID) | The flat URI used to identify the parent resource without using the full structured URI. Editor's Note: The adoption of a URI format is still to be confirmed. |
| accessRightID (aRI) | The URI of an <access rights> resource. The permissions defined in the <accessRight> resource that are referenced determine who is allowed to access the resource containing this attribute for a specific purpose (e.g., Retrieve, Update, Delete etc.). If a resource type does not have an accessRightID attribute definition, then the accessRights for resources of that type are governed in a different way, for example, the accessRight associated with the parent may apply to a child resource that does not have an accessRightID attribute definition, or the permissions for access are fixed. Refer to the corresponding procedures to see how permissions are handled in such cases. If a resource type does have an accessRightID attribute definition, but the (optional) accessRightID attribute is not set, or it is set to a value that does not correspond to an valid, existing <accessRight> resource, or it refers to an <accessRight> resource that is not reachable (e.g. because it is located on a remote CSE that is offline or not reachable), then the system default access permissions shall apply. The system default access permissions grant all permissions (i.e. the full set of permissionsFlags) to the following permission holders depending on the prefix of URI of the resource.: << Editor's Note: what is the list of permission holders>> |
| creationTime (cT) | Time/date of creation of the resource. |
| expirationTime (eT) | Time/date after which the resource will be deleted by the hosting CSE. This attribute can be provided by the Originator, and in such a case it will be regarded as a hint to the hosting CSE on the lifetime of the resource. The hosting CSE can however decide on the real expirationTime. If the hosting CSE decides to change the expirationTime attribute value, this is communicated back to the Originator. The lifetime of the resource can be extended by providing a new value for this attribute in an UPDATE verb. Or by deleting the attribute value, e.g. by not providing the attribute when doing a full UPDATE, in which case the hosting CSE can decide on a new value. |
| filterCriteria (fC) | Criteria to be used to filter the results. They can either be used in a GET (as query parameters) or in a subscription. Editor's Note: Is GET a supported operation. Editor's Note: The applicability of this attribute is FFS. |
| lastModifiedTime (lMT) | Last modification time/date of the resource. |
| labels(lBs) | Tokens used as keys for discovering resources. |
| creator (cr) | The App-inst-ID or CSE-ID of the entity which created the resource |
| link (ln) | This attribute shall be present only for the announced resource. This attribute shall provide the URI to the original resource. |
| announceTo (aT) | This attributes shall be present only for the announceable original resource. This attribute maintains the list of URIs to the announced resources. Updates on this attribute will trigger the new resource announcement or de-announcement. In case of new resource announcement, the exact URI to the announced resource that will be created can be provided, or the CSE-ID can be provided, which means the resource-hosting CSE will decide the exact location in the CSE identified by the CSE-ID. |
| announcedAttribute (aA) | This attributes shall be present only for the announceable original resource. This attribute maintains the list of the announced attributes in the original resource. Updates on this attribute will trigger new attribute announcement if a new attribute is added or de-announcement if the existing attribute is removed. |

Editor’s Note:  The usage of creator (cr) attribute is FFS.

Editor's Note: The following attributes are FFS depending on resource structure discussions:

- child: resources with child relation with the considered resource

- parent: resources with parent relation with the considered resource

### 9.6.2	Resource Type accessRight

Access rights are defined as "white lists" or permissions, i.e. each permission defines "allowed" entities (defined  as permissionHolders) for certain access modes (defined as permissionFlags). Sets of permissions are handled such that the resulting permissions for a group of permissions are the sum of the individual permissions; i.e., an action is permitted if it is permitted by some / any permission in the set. The selfPermission attribute lists those that have the rights to Read/Update/Delete for the resource <accessRights>.

By setting an accessRightID attribute on a resource, the permissions for accessing that resource are then defined by the permissions defined in the <accessRight> resource.

Figure 9.6.2-1: Structure of <accessRight> resource 
(only resource specific attributes are shown)

Table 9.6.2-1: Child resources of <accessRight> resource

| Child Resource Name of <accessRight> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See section 9.6.9 |

This resource shall contain the attributes according to their multiplicitly in Table 9.6.2-2 (0 indicates the optionality of the attribute).

Table 9.6.2-2: Attributes of <accessRight> resource

| Attribute Name of <accessRight> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| permissions (ps) | 1 | RW | The list of permissions defined by this <accessRight> resource. These permissions are applied to resources referencing this <accessRight> resource using the accessRightID attribute. |
| selfPermissions (sP) | 1 | RW | Defines the list of permissions for the <accessRight> resource itself. |

Editor’s Note:  Mechanisms for announcing are FFS.

The permissions could then be generalised to actions (which might be granting access, but might also be more specific, like granting access to a subset, i.e. filtering part of the data). The permissions could be generalised to conditions, which may include things like the identity of the requestor, everybody except specified identities, but it might also include time based conditions etc.

### 9.6.3	Resource Type container

This resource represents a container for data instances. It is used to share information among other entities and potentially to track the data. A <container> resource has no associated content, only attributes and child resources.

Figure 9.6.3-1: Structure of <container> resource
(only child resources and resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicitly in Table 9.6.3-1 (0 indicates the optionality of the child resource).

Editor’s Note: Susbscription relation with this resource is for FFS.

Editor’s Note: location resource relation with containers is for FFS.

Table 9.6.3-1: Child resources of <container> resource

| Child Resource Name of <container> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <instance> | 0..n | See section 9.6.6 |
| [variable] | <subscription> | 0..n | See section 9.6.9 |
| [variable] | <container> | 0..n | See section 9.6.3 |

This resource shall contain the attributes according to their multiplicitly in Table 9.6.3-2 (0 indicates the optionality of the attribute).

Table 9.6.3-2: Attribute of <container> resource

| Attribute Name of <container> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 0..1 | RW | See section 9.6.1 where this common attribute is described. If no accessRightID is given at the time of creation, the accessRightID of the parent resource is linked to this attribute |
| labels (lBs) | 0..1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (st) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| creator (cr) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| maxNrOfInstances (mi) | 0..1 | RW | Maximum number of instances of <instance> child resources. |
| maxByteSize (mb) | 0..1 | RW | Maximum number of bytes that are allocated for a <container> resource for all instances in the <container> resource. |
| maxInstanceAge (ma) | 0..1 | RW | Maximum age of the instances of <instance> resources within the <container>. The value is expressed in seconds. |
| currentNrOfInstances (ni) | 1 | RO | Current number of instances in a <container> resource. It is limited by the maxNrOfInstances. |
| currentByteSize (nb) | 1 | RO | Current size in bytes of data stored in a <container> resource. It is limited by the maxNrOfBytes. |
| latest (lt) | 0..1 | RO | Reference to latest instance, when present. |
| locationID (LID ) | 0..1 | RW | URI of the resource where the attributes/policies that define how location information are obtained and managed. This attribute is defined only when the <container> resource is used for containing location information. |
| ontologyRef (oR) | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is stored in the instances of the container. NOTE: the access to this URI is out of scope of oneM2M |

Editor’s Note: Mechanisms for announcing are FFS.

Editor’s Note: The container type is FFS.

Editor’s Note: The following attributes are FFS depending on resource structure discussions:

- child: resources with child relation with the considered resource

- parent: resources with paren relation with the considered resource

### 9.6.4	Resource Type delivery

When a CSE is requested to initiate an operation (CRUD) targeting resources on another CSE, then it needs to involve the CMDH CSF in order to do proper scheduling and execution of delivery of data from the source CSE to the target CSE in line with provisioned policies. It is possible to do that in two different ways: Using delivery aggregation ("da" information set to on) or by forwarding the original request as a separate request on the Mcc reference point without changes.

In order to be able to initiate and manage the execution of data delivery in a resource-based manner, the resource type delivery is defined. This resource type needs to be used for forwarding requests from one CSE to another CSE when "da" information in the request is set to ON. If the "da" information is set to OFF, the original request shall be forwarded without change  to the next CSE.

Operations to Retrieve, Update or Delete a <delivery> resource will allow authorized entities to inquire the status of a delivery, change delivery attributes or cancel a delivery.

Editor’s Note: Further details on mechanisms how to do delivery handling are FFS.

Editor’s Note: The particular position in the addressable space of resources id FFS.

Figure 9.6.4-1: Structure of resource <delivery>
(only resource specific attributes are shown)

The resource <delivery> shall contain the attributes according to their multiplicity shown in Table 9.6.4-1.

Table 9.6.4-1: Attributes of <delivery> resource

| Attribute Name of <delivery> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| source | 1 | WO | The CSE-ID of the CSE that initiated the delivery process represented by this <delivery> resource. |
| target | 1 | WO | A list of CSE-IDs that defines one or more target CSEs for delivering the data contained in the data attribute |
| lifespan | 1 | RW | Defines the time limit until when the delivery of the information in the data attribute needs to complete. If the lifespan is expired before successful delivery, no further attempts to deliver the information in the data attribute need to be executed. If the delivery fails, a feedback may be expected by the source CSE depending on options reflected in the deliveryMetaData attribute. |
| eventCat | 1 | RW | Defines the category of the event that triggered the delivery request represented by this <delivery> resource |
| deliveryMetaData | 1 | RW | Contains meta information on the delivery process represented by this <delivery> resource, such as delivery status, delivery options, tracing information, etc |
| data | 1 | WO | Attribute containing the data to be delivered to the target CSE(s). This data represents one or more original requests that were targeting the same CSE(s). |

### 9.6.5	Resource Type group

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the members. The <group> resource contains an attribute that represents the members of the group and a virtual resource (the members) that allows verbs to be applied to the resources represented by those members.

When used as one of the permission holders in an <accessRight> resource, the group can be used to grant a collection of applications (represented by <Application> resources) or CSEs (represented by <Entity> resources) permissions for accessing (Creating children, Retrieving, etc.) a resource.

Figure 9.6.5-1:  Structure of <group> resource
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicitly in Table 9.6.5-1.

Table 9.6.5-1: Child resources of <group> resource

| Child Resource Name of <group> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| members | <Members> | 1 | See section 9.6.21 |

The <group> resource shall contain the attributes with the indicated multiplicity in Table 9.6.5-2

Table 9.6.5-2: Attributes of <group> resource

| Attribute Name of <group> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| creator (cr) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| memberType (mT) | 1 | WO | It is the resource type of the member resources of the group if all member resources (including the member resources in any sub-groups) are of the same type. Otherwise, it’s a type of ‘mixed’. |
| currentNrOfMembers (nM) | 1 | RO | Current number of members in a group. It is limited by the maxNrOfMembers. |
| maxNrOfMembers (mM) | 1 | RW | Maximum number of members in the group. |
| membersList (mL) | 1 | RW | List of zero or more member URIs referred to in the remaining of this specification as memberID. Each URI (memberID) should refer to a member resource or a (sub-) group resource of the group. |
| membersAccessRightID(mAR) | 0..1 | RW | URI of the accessRight resource defining who is allowed to access the members attribute. |
| memberTypeValidated(mTV) | 1 | RO | Denotes if memberType of all member resources of the group has been validated. |
| consistencyStrategy (cS) | 0..1 | WO | This attribute determines how to deal with the <group> resource if the memberType validation fails.Which means delete the inconsistent member if the attribute is ABANDON_MEMBER; delete the group if the attribute is ABANDON_GROUP; set the memberType to “mixed” if the attribute is SET_MIXED. |
| groupName (gN) | 0..1 | RW | Human readable name of the group. |

### 9.6.6	Resource Type instance

The <instance> resource represents a data instance in the container. The content of the instance is opaque to the M2M platform and it might even be encrypted. However, there is meta-data associated with an instance which shall be accessible.

Contrary to other resources, the <instance> resource cannot be modified once created, regardless of the accessRightID associated with the parent resource. An instance may be deleted explicitly or it may be deleted by the platform based on policies. If the platform has policies for the instance retention these shall be represented by the attributes maxByteSize, maxNrOfInstances and/or maxInstanceAge on the <container> resource. If multiple policies are in effect, the strictest policy shall apply.

The <instance> resource inherits the same access rights of the parent <container> resource, and does not have its own <accessRights> attribute.

Figure 9.6.6-1: Structure of <instance> resource
(only child resources and resource specific attributes are shown)

This resource shall contain the attributes according to their multiplicity in Table 9.6.6-1 (0 indicates the optionality of the attribute).

Table 9.6.6-1: Attributes of <instance> resource

| Attribute Name of <instance> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| typeOfContent (tOC) | 0..1 | WO | Optional type of the content included in the content attribute. This is media-type as defined in <reference to stage 3 document>. |
| contentSize (cS) | 1 | WO | Size in bytes of the content attribute. |
| ontologyRef | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is stored in the instances of the container. If this attribute is not present, the instance inherits the ontologyRef from the parent <container> resource if present NOTE: Access to this URI is out of scope of oneM2M |
| content (c) | 1 | WO | Actual opaque content of an instance. This may for example be an image taken by a security camera, or a temperature measurement taken by a temperature sensor. |

### 9.6.7		Resource Type locationPolicy

The <locationPolicy> resource in LOC CSF represents the method for obtaining and managing geographical location information of an M2M Nide.

The actual location information shall be stored in the <instance> resource, a child resource of <container> resource.  The <container> resource has the locationId attribute which holds the URI of this <locationPolicy> resource as a linkage. The LOC CSF can obtain location information based on the attributes defined under <locationPolicy> resource, and store the location information in the target <container> resource.

Based on the locationSource attribute, the way of obtaining location information of an M2M Node can be differentiated: Network-based where the CSE on behalf of the AE obtains the target M2M Node’s location information from Underlying Network, Device-based where the ASN is equipped with any location capable  modules or technologies (e.g., GPS) and is able to position itself, and Sharing-based where the ADN has no GPS and Underlying Network connection capability, but its location information can be retrieved from either ASN or MN.

NOTE: Geographical location information is more than longitude and latitude.

Editor’s Note: The specific information where this resource is created is FFS.

Figure 9.6.7-1:  Structure of <locationPolicy> resource
(only resource specific attributes are shown)

This resource shall contain the attributes according to their multiplicity in Table 9.6.7-1 (0 indicates the optionality of the attribute).

Table 9.6.7-1: Attributes of <locationPolicy> resource

| Attribute Name of <locationPolicy> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 1..n | RW | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| locationSource | 1 | RW | Indicates the source of location information - Network Based - GPS Based - Sharing Based |
| locationUpdatePeriod | 0..1 | RW | Indicates the period for updating location information. If the value is marked ‘0’ or not defined, location information is updated only when a retrieval request is triggered. |
| locationTargetId | 0..1 | RW | The identifier to be used for retrieving the location information of a remote Node and this attribute is only used in the case that location information is provided by a location server. |
| locationServer | 0..1 | RW | Indicates the identity of the location server. This attribute is only used in that case location information is provided by a location server. |
| locationContainerID_[i] | 0..n | RO | URI of the <container> resource where the actual location information of a M2M Node is stored. |
| locationContainerName_[i] | 0..n | RW | Name of the <container> resource where the actual location information of a M2M Node is stored. If it is not assigned, the Hosting CSE automatically assigns a name of the resource. Editor’s Note: The association between multiple container resources and a location policy resource is FFS. |

Editor’s Note: The naming convention of locationContainerID and locationContainerName that have multiplicty  more than 1 is just following what WG2 discussed in the drafting session. If the convention or the rule is changed in the template section, the notation will be changed.

### 9.6.8	Resource Type request

The use of this resource type is optional depending on the configuration.

When a CSE is requested to initiate an operation for which the result should be available to the Originator by reference (‘rt’ information of the request set to ‘Acknowledgement'), the local CSE which received the Request directly from the Originator may need to provide a reference back to the Originator so that the Originator can access attributes of the Request at a later time – for instance in order to retrieve the result of an operation that was taking a longer time. The Originator (or any other authorized entity depending on access rights) can access the request status and the requested operation result through it.

The <request> resource may get deleted by the CSE that is hosting it:

- when the result of the requested operation (if any result was requested at all) has been sent back to the Originator (either by notification to the Originator or by retrieval initiated by the Originator);

- when the expiration time of the <request> resource is reached;

- at any other time -  depending on local settings in the CSE that hosts the <request> resource.

Editor’s Note: The exact way how to define that setting (e.g. via DM) is FFS.

For the purpose of providing a standardized structure for expressing and accessing the context of a previously issued Request, the resource type request is defined.

Figure 9.6.8-1: Structure of <request> resource
(only resource specific attributes are shown)

The resource <request> shall contain the attributes according to their multiplicity shown in Table 9.6.8-1.

Table 9.6.8-1: Attributes of <request> resource

| Attribute Name of <request> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime (et) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| operation | 1 | RO | The operation requested by the Originator, e.g. modifying the content of a container, registering an Application. |
| target | 1 | RO | The address or the target resource for the requested operation. |
| originator | 1 | RO | The originating entity for the request. |
| metaInformation | 1 | RO | Meta information about the request. The content of this attribute is equivalent to the “mi” information in section 8.1. |
| content | 1 | RO | Contains the content that is supposed to be sent to the target resource. |
| requestStatus | 1 | RO | Contains information on the current status of the Request, e.g. “accepted and pending”. |
| operationResult | 1 | RO | Contains information on the result of the requested action, e.g. “Resource created with URI: XXX”. |

Creation of a <request> resource can only be done on a local CSE implicitly by an AE issuing a generic CRUD request for any other resource type to the Local CSE. A Local CSE cannot accept requests to create a <request> resource directly.

Retrieving of <request> resources or attributes of a <request> resource is allowed for authorized entities.

Update of a <request> resource can only be done by the CSE that hosts the <request> resource or by CSEs that provide operation results pertaining to the request (for instance when the result of an operation requested for a remote CSE becomes available).

Deletion of a <request> resource is possible by authorized entities. If the execution of the request was still pending at the time of deletion, the hosting CSE should to stop any further attempts to execute the request.

### 9.6.9	Resource Type subscription

The <subscription> resource contains subscription information for its subscribed-to resource.

The list of subscribable resources is as follows.

- <entity> resource (see section TBD)

- <Application> resource (see section 9.6.18)

- <accessRight> resource (see section 9.6.2)

- <container> resource (see section 9.6.3)

- <group> resource (see section 9.6.5)

Editor’s Note: It is not yet clear how or where to best to explain the applicability of the resource <subscription>. Subscribable resources can be added to the list above, Non-subscribable resources can also be added to the list above, and/or we could ensure that the applicability of the resource <subscription> is clearly described with each resource, removing or keeping the list here.

The relationship between a subscribed-to resource and a <subscription> resource shall be represented as parent-child resource in each subscribable resource. For example, <container> resource has <subscription> resource as a child resource (see section 9.6.3). A <subscription> resource shall be deleted when a parent subscribed-to resource is deleted.

The <subscription> resource shall represent a subscription to a subscribed-to resource. An Originator shall be able to create a resource of <subscription> resource type when the originator has RETRIEVE permission to the subscribable resource. The Originator of a <subscription> resource becomes a resource subscriber.

When a modification to the subscribed-to resource occurs, that modification is compared to the criteria attribute to determine whether a notification is to be sent to the resource subscriber.

Figure 9.6.9-1: Structure of <subscription> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicitly in Table 9.6.9-1.

Table 9.6.9-1: Child resources of <subscription> resource

| Child Resource Name of <subscription> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| notificationSchedule | <schedule> | 0..1 | See section 9.6.16 |

Editor’s Note: criteria is shown as an attribute, However, whether we need a separate attribute or child resource for notification policy is FFS.

The <subscription> resource shall contain the attributes described in Table 9.6.9-2.

Table 9.6.9-2: Attributes of <subscription> resource

| Attribute Name of <subscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | WO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| filterCriteria (fC) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | I | RW | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 0..1 | RW | See section 9.6.1 where this common attribute is described. If no accessRightID is given at the time of creation, the accessRightID of the parent resource is linked to this attribute. |
| expirationCounter | 0..1 | RW | When the number of notifications becomes the same as this counter, the <subscription> resource shall be deleted. |
| notificationURI | 1..n | RW | URI where the resource subscriber will receive notifications. |
| aggregationURI | 0..1 | RW | URI to aggregate notifications from group members of a <group> resource. |
| batchNotify | 0..1 | RW | Indicates that notifications should be batched for delivery. When set, notification events are temporarily stored until either a specified number is ready to send or until a duration after the first noticifaction event has expired. |
| rateLimit | 0..1 | RW | Indicates that notifications should be rate-limited. When set, notification events that exceed a specified number within a specified time are temporarily stored then sent when the number of events sent per specified time falls below the limit. |
| priorSubscriptionNotify | 0..1 | WO | Indicates that when this subscription is created, whether notification events prior to subscription should be sent, e.g. send prior “n” notifications, if available. |
| interimEventsNotify | 0..1 | RW | Indicates the notification action to be taken following a period of unreachability, when the resource subscriber becomes reachable (e.g. send “n” iterim notificiations, if available.) |
| notificationStoragePriority | 0..1 | RW | Indicates a priority for this subscription relative to other subscriptions belonging to this same subscriber for retention of notification events when storage is congested. The storage congestion policy which uses this attribute as input is specified in section TBD. |
| notificationStructure | 1 | RW | Type of information that shall be contained in notifications. E.g., modified attribute only of a subscribed-to resource, a whole subscribed-to resource, and/or URI of a corresponding <subscription> resource. |

Editor’s Note: In the case when there are Intermediate Nodes between the Originator and a Receiver, the Intermediate Nodes may not act in accordance with the subscription notification policy. Examples include: a policy of batching subscribed events for up to 30 seconds then deliver immediately, but an intermediate node may decide to delay or further aggregate the subscription notifications. The solution to this problem is FFS, but may include sending a representation of policy with the subscription notifications, which is to be considered by Intermediate Nodes (e.g. Request Expiration Time).

### 9.6.10	Resource Type mgmtObj

The mgmtObj resource contains management data which represents individual M2M management functions. It represents a general structure to map to external management technology (e.g. OMA DM, BBF TR-069 and LWM2M) data models.

Figure 9.6.10-1: Structure of <mgmtObj> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicitly in Table 9.6.10-1 (0 indicates the optionality of the child resource).

Table 9.6.10-1: Child resources of <mgmtObj> resource

| Child Resource Name of <mgmtObj> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <parameters> | 0..n | See section 9.6.11. Collects multiple parameters or attributes for management purpose. A <parameters> resource can have zero or more other <parameters> sub0resources so as to form hierarchical structure, i.e. the management data for this specific <mgmtObj>. Such structure helps to import existing management objects as defined in OMA DM, BBF TR-069, LWM2M, or any other technology. |

The <mgmtObj> resource shall contain the attributes described in Table 9.6.10-2.

Table 9.6.10-2: Attributes of <mgmtObj> resource

| Attribute Name of <mgmtObj> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | WO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described |
| accessRightID (aRI) | 1 | RW | See section 9.6.1 where this common attribute is described |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| mgmtDefinition | 1 | WO | Indicates specific device management functionalities. Editor’s Note: Description needed. |
| objectID | 0..1 | WO | Contains a URN that uniquely identifies the external management technology data model used for this <mgmtObj> resource as well as the managed function and version it represents. This attribute shall be provided during the creation of the <mgmtObj> resource and shall not be modifiable afterwards. This is mandatory for the <mgmtObj>, for which the data model is not specified by oneM2M but mapped from external management technology data model specified in other device management protocols (e.g. OMA-DM, BBF TR-069 and LWM2M). |
| objectPath | 0..1 | WO | Contains the local path of the external management object instance on the managed entity which is represented by the <mgmtObj> resource in the hosting CSE. This attribute shall be provided during the creation of the <mgmtObj>, so that the hosting CSE can correlate the created <mgmtObj> with the external management object instance on the managed entity for further management operations. It shall not be modifiable after creation. The format of this attribute shall be a local external management object path in the form as specified by existing management protocols. (e.g. "./anyPath/Fw1" in OMA DM, "Device.USBHosts.Host.3." in BBF TR-069). |
| moAttribute | 0..n | RW | Each moAttribute is mapped from a leaf node of a hierarchical structured management object (including oneM2M data model and the existing management data models) based on the mapping rules below the table. |
| description | 0..1 | RW | Text format description of mgmtObj. |

When mapping resources from management technologies to a corresponding <mgmtObj> resource, the following rules shall apply:

- The root resource of external management objects maps to the <mgmtObj> resource

- For child resource of the root resource of specific technology:

- Rule1:  If the child external management object cannot have child external management object, the external management object maps to the "moAttribute"  attribute of the <mgmtObj> resource with the same resource name;

- Rule2:  If the child external management object can have another child external management object, the external management object maps to the <parameters> resource with the same resource name.

- For the external management object that is mapped to the <parameters> resource, the child external management object of the external management object are mapped to the "moAttribute" attribute or to the <parameter> resource according to the Rule1 and Rule2.

### 9.6.11	Resource Type parameters

Figure 9.6.11-1: Structure of <parameters> resource 
(only resource specific attributes are shown)

Figure 9.6.11-1: Structure of <parameters> resource 
(only resource specific attributes are shown)

(0 indicates the optionality of the child resource).

Table Child resources of <parameters> resource

| Child Resource Name of <parameters> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <parameters> | 0..n | See section 9.6.11 |

The <parameters> resource shall contain the attributes described in Table 9.6.11-2.

Table 9.6.11-2: Attributes of <parameters> resource

| Attribute Name of <parameters> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| accessRightID (aRI) | 1 | RW | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| objectID | 0..1 | WO | See Table 9.6.10.1 |
| objectPath | 0..1 | WO | See Table 9.6.10.1 |
| <moAttribute> | 0..n | RW | See Table 9.6.10.1 |
| description | 0..1 | RW | Text format description of mgmtObj. |

### 9.6.12	Resource Type mgmtCmd

The <mgmtCmd> resource represents a method to execute management procedures or to model commands and remote procedure calls (RPC) required by existing management protocols (e.g. BBF TR-069), and enables AEs to request management procedures to be executed on a remote entity. It also enables cancellation of cancellable and initiated yet unfinished management procedures or commands.

Figure 9.6.12-1: Structure of <mgmtCmd> resource 
(only resource specific attributes are shown)

Each <mgmtCmd> corresponds to a specific type of management command, as defined by its attribute cmdType. For multiple requests of the same management command, <mgmtCmd> may use the sub-resource (i.e. execInstances) to contain all execution instances. The execution of the management procedure represented by <mgmtCmd> shall be triggered using the UPDATE method to its attribute "execEnable".

This resource shall contain the child resources according to their multiplicity in Table 9.6.12-1 (0 indicates the optionality of the child resource).

Table 9.6.12-1: Child resources of <mgmtCmd> resource

| Child Resource Name of <mgmtCmd> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <execInstances> | 1 | See section 9.6.13. A collection resource as the placeholder to contain all execution instances of the same <mgmtCmd>. |

The <mgmtCmd> resource shall contain the attributes described in Table 9.6.12-2.

Table 9.6.12-2: Attributes of <mgmtCmd> resource

Table 9.6.12-2: Attributes of <mgmtCmd> resource

| Attribute Name of <mgmtCmd> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | WO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described |
| accessRightID (aRI) | 1 | RW | See section 9.6.1 where this common attribute is described |
| labels (lBs) | 1 | RW | See section 9.6.1 where this common attribute is described |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| description | 0..1 | RW | The text-format description of this resource |
| cmdType | 1 | WO | The type to identify the management operation (e.g. download). |
| execReqArgs | 0..1 | RW | Structured attribute (e.g. abstract type) to contain any command-specific arguments of the request. |
| execEnable | 1 | RO | The attribute can be blank without any value or it can contain a URI that can be used to trigger execution of <mgmtCmd> using UPDATE method. |

Editor’s Note: The types of management operations (e.g. cmdType) to be used by mgmtCmd are FFS.

### 9.6.13	Resource Type execInstances

The <execInstances> resource contains all execution instances of the same management command <mgmtCmd>. Each <execInstance> is a sub-resource of <execInstances>. When <mgmtCmd> is successfully triggered by an M2M network application using UPDATE method, a corresponding <execInstance> sub-resource shall be generated.

Figure 9.6.13-1: Structure of <execInstances> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicitly in Table 9.6.13-1 (0 indicates the optionality of the child resource).

Table 9.6.13-1: Child resources of <execInstances> resource

| Child Resource Name of <execInstances> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <execInstance> | 0..n | See section 9.6.14. An execution instance of <mgmtCmd>. |

The <execInstances> resource shall contain the attributes described in Table 9.6.13-2.

Table 9.6.13-2: Attributes of <execInstances> resource

| Attribute Name of <execInstances> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |

### 9.6.14	Resource Type execInstance

The <execInstance> resource represents a successful instance of <mgmtCmd> execution request, which had been triggered by a M2M network application using the UPDATE method to the attribute "execEnable" of <mgmtCmd>.

Figure 9.6.14-1: Structure of <execInstance> resource 
(only resource specific attributes are shown)

The <execInstance> resource shall contain the attributes described in Table 9.6.14-1.

Table 9.6.14-1: Attributes of <execInstances> resource

| Attribute Name of <execInstance> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime (eT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 1 | RW | See section 9.6.1 where this common attribute is described |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| execStatus | 1 | RO | The status of <execInstance>. It can be Initiated, Started, Finished, Cancelled, or Deleted. |
| execResult | 1 | RO | The execution result of <execInstance>. |
| execDisable | 0..1 | RW | The attribute is used to cancel <execInstance> using UPDATE method. |
| execTarget | 1 | RW | M2M-Node-ID of the target. |
| execMode | 0..1 | RW | Modes used to specify how the command will be executed (e.g. Immediate Once, Immediate and Repeatedly, Random Once, Random and Repeatedly). May be used together with execFrequency, execDelay and execNumber to provide the scheduling information. |
| execFrequency | 0..1 | RW | The interval between two executions, to be used in conjunction with execMode. |
| execDelay | 0..1 | RW | The delay before the instance should be executed |
| execNumber | 0..1 | RW | The number of times the instance should be executed, to be used in conjunction with execMode. |
| execReqArgs | 0..n | RW | Structured attribute (e.g. abstract type) to contain any command-specific arguments used to trigger this <execInstance>. |

Editor’s note: the relationships between execMode, execFrequncy and execNumber are FFS.

### 9.6.15	Resources for Service Charging and Accounting

The <chargingPolicy> and <chargingRule> resources enable the SCA CSF to generate accounting information for particular services based on policies provided by the Service Provider. The SCA CSF and its resources residing on one CSE can be used by other CSEs and applications that subscribed to this CSE.

#### 9.6.15.1	Resource Type chargingPolicy

The <chargingPolicy> resource shall be used to store charging related policies. The policy may be established by the oneM2M service provider. It may be provisioned using device management.  The <chargingPolicy> resource may be used by the charging management function in one node to configure charging policies at another node.  For example, a Service Provider may only keep the charging policy at the Infrastructure Node, or it may decide to pass the policy to the Middle Nodes. The <chargingPolicy> shall be applied to the Infrastructure Node. Whether or not it is applied to the Middle Node or Application Service Node shall depend on the configuration of the Service Provider.

Editor’s Note: Only chargeable events and subscriber based charging policy are shown. More charging policies, such as chargeable services can be added using the same resource model.

Figure 9.6.15.1-1: Structure of <chargingPolicy> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicity in Table 9.6.15.1-1 (0 indicates the optionality of the child resource).

Table 9.6.15.1-1: Child resources of <chargingPolicy> resource

| Child Resource Name of <chargingPolicy> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <chargeableEvent> | 0..n | See section 9.6.15.2. This resource defines the charging policy for a chargeable event. It is conditionally mandatory when the charging model is ‘Event Based’. |

The <chargingPolicy> resource shall contain the attributes described in Table 9.6.15.1-2.

Table 9.6.15.1-2: Attributes of <chargingPolicy> resource

| Attribute Name of <chargingPolicy> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 1..n | RW | See section 9.6.1 where this common attribute is described |
| creator (cr) | 1 | RW | See section 9.6.1 where this common attribute is described |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described |
| subscriberBased | 0..1 | RW | If this attribute is set to “ON”, it enables the subscriber based charging policy. Charging is done by service subscription contract, not by service layer operations. |

#### 9.6.15.2	Resource Type chargeableEvent

Figure 9.6.15.2-1: Structure of <chargeableEvent> resource 
(only resource specific attributes are shown)

The <chargeableEvent> resource shall contain the attributes described in Table 9.6.15.2-1.

Table 9.6.15.2-1: Attributes of <chargeableEvent> resource

| Attribute Name of <chargeableEvent> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 1..n | RW | See section 9.6.1 where this common attribute is described |
| creator (cr) | 1 | RW | See section 9.6.1 where this common attribute is described |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described |
| chargeableEventID | 1 | RO | This attribute uniquely identifies the chargeable event |
| eventType | 1 | RW | This attribute indicates the type of the event, such as timer based, data operation, storage based charging, etc. |
| eventStart | 0..1 | RW | This attribute indicates the start time of the event. |
| eventEnd | 0..1 | RW | This attribute indicates the end time of the event |
| transactionType | 0..1 | RW | This attribute defines the type of the chargeable operation, such as CREATE, RETRIEVE. |
| dataSize | 0..1 | RW | This attribute defines the data size if an event is triggered when the stored data exceeded a certain size. |

#### 9.6.15.3	Resource Type chargingRule

The resource type <chargingRule> shall be used to define different rules for charging purpose. One node may setup multiple rules. One rule may be activated or de-activated.

Figure 9.6.15.3-1: Structure of <chargingRule> resource 
(only resource specific attributes are shown)

The <chargingRule> resource shall contain the attributes described in Table 9.6.15.3-1.

Table 9.6.15.3-1: Attributes of <chargingRule> resource

| Attribute Name of <chargingRule> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 1..n | RW | See section 9.6.1 where this common attribute is described |
| creator (cr) | 1 | RW | See section 9.6.1 where this common attribute is described |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described |
| chargingRuleID | 1 | RO | This is the unique ID of the charging rule. It is created by the CSE when the charging rule resource is first created. |
| chargingEntityID | 1 | WO | This is the unique ID of the entity that charges another entity. For example, it can be an application ID (App-ID) or a CSE ID (CSE-ID). |
| chargedEntityID | 1 | WO | This is the unique ID of the entity that is being charged. For example, it can be an application ID (App-ID) or a CSE ID (CSE-ID) or a resource ID. |
| ruleStatus | 1 | RW | This attribute indicates whether the rule is “active” or “inactive”, or “information recording only” which means no charging is performed. |
| chargingType | 1 | RW | This attribute indicates whether the charging rule applies for ONLINE or OFFLINE charging. |
| chargingModel | 1 | RW | This attribute indicates the charging model, such as “Subscriber based charging”, “event based charging”, “service based charging”, “session based charging”, etc. |
| subscriberID | 1 | WO | This attribute indicates the service subscriber (M2M-Sub-ID) being charged. |
| collectPeriod | 0..1 | RW | This attribute defines the duration for the SCA to collect service charging information. |
| sessionID | 0..1 | RW | This attribute indicates the unique service session ID, if the chargingType is “session based charging”. It is mandatory of the chargingModel is session based charging. |
| serviceID | 0..1 | RW | This attribute indicates the unique ID of the service being charged. A service provider can define its own services. It is mandatory if the charging model is service based charging. |
| chargeableEventID | 0..1 | RW | This attribute refers to the resource that defines the chargeable event that should be applied to this charging rule. It is mandatory if the charging model is event based charging. |

Editor’s Note: The table above includes the basic parameters needed to configure charging rules. More parameters may be added when the feature is further developed. Online charging is FFS.

### 9.6.16	Resource Type schedule

The <schedule> resource contains scheduling information for delivery of messages.

Editor’s Note: It is expected that the reachability period will be extended by the duration of a transaction which is currently occurring, but the details need further study.

The list of resources, which use <schedule> resource, is as follows.

- TBD

Editor’s Note: This list will be revised as resources are further progressed

The <schedule> resource shall represent scheduling information in the context of its parent resource. An Originator shall have the same access rights to the <schedule> resource as it has to its parent resource.

Figure 9.6.16-1: Structure of <schedule> resource 
(only resource specific attributes are shown)

The <schedule> resource shall contain the attributes described in Table 9.6.16-1.

Table 9.6.16-1: Attributes of <schedule> resource

| Attribute Name of <schedule> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| scheduleElement | 1..n | RW | Expresses time periods defined by seconds, minutes, day of month, month, day of month, and year. Supporting repeating periods, and wildcards. |

Editor’s Note: The multiplicity of attribute must be aligned to the new understanding.

### 9.6.17	Resource Type node

The <node> resource represents specific information of a M2M Node that can be utilized by other oneM2M operations. For example, the CMDH CSF can determine when the CSE sends a request message to the target CSE based on the target CSE’s reachability schedule. This resource type mainly contains the properties of an M2M node such as reachability schedule, context information (e.g., memory and storage) and network topology.

This node specific information such as memory and storage can be obtained either by the existing device management technologies (OMA DM, BBF TR-069) or any other way (e.g., through JNI (Java Native Interface)). Since <mgmtObj> resource type represents the management functions including both ways, the type of sub-resources are <mgmtObj> resource type.

The location of <node> resource can be differentiated according to the type of node.

- ASN: under <baseURI> of the local CSE

- ADN: under <Application> resource of the hosting CSE

- MN: under <baseURI> of the local CSE

Editor’s Note: More specific information regarding a node can be added.

Figure 9.6.17-1: Structure of <node> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicity in Table 9.6.17-1 (0 indicates the optionality of the child resource).

Table 9.6.17-1: Child resources of <node> resource

| Child Resource Name of <node> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| memory | <mgmtObj> | 0..1 | This resource provides the memory (typically RAM) information of the node. (e.g. the amount of total volatile memory), See section 9.6.10 for <mgmtObj>. |
| storage_[i] | <mgmtObj> | 0..n | This resource provides the storage (typically flash or disk) information of the node. (e.g. the amount of total writable persistent storage). See section 9.6.10 for <mgmtObj>. |
| power | <mgmtObj> | 0..1 | The resource provides the power information of the node. (e.g. remaining battery charge). See section 9.6.10 for <mgmtObj>. |
| networkTopology | <mgmtObj> | 0..1 | This resource describes the list of Nodes attached behind the MN node and its relation among the nodes. This attribute is defined in case the Node is MN. See section 9.6.10 for <mgmtObj>. |
| schedule | <schedule> | 0..1 | This resource defines the reachability schedule information of the node. See section 9.6.16 for <schedule>. |

The <node> resource shall contain the attributes described in Table 9.6.17-2.

Table 9.6.17-2: Attributes of <node> resource

| Attribute Name of <node> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| accessRightID_[i] (aRI_[i]) | 1..n | RW | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| nodeID | 1 | RW | The ID of the Node which is specified in the resource type. |

### 9.6.18	Resource Type application

The <application> resource represents information about an Application Entity known to a given Common Services Entity.

Figure 9.6.18-1: Structure of <application> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicity in Table 9.6.18-1 (0 indicates the optionality of the child resource).

Table 9.6.18-1: Child resources of <application> resource

| Child Resource Name of <application> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See section 9.6.9. |
| [variable] | <container> | 0..n | See section 9.6.3. |
| [variable] | <group> | 0..n | See section 9.6.5. |
| [variable] | <accessRight> | 0..n | See section 9.6.2. |
| software_[i] | <mgmtObj> | 1..n | See section 9.6.10 for <mgmtObj>. This resource represents the software component (or components, if the multiplicity is greater than 1) that compose the application. This mgmtObj shall have at least the following <parameters> child resources of its “parameters” subresource: - name (multiplicity 1) - version (multiplicity 1) - state (multiplicity 1) |
| [variable] | <commCapabilities> | 1..n | Resource <commCapabilities> needs to be defined. This resource describes the communication capabilities (essentially the protocols) supported and used by the corresponding Application Entity over the Mca reference point to this CSE. |

The <application> resource shall contain the attributes described in Table 9.6.18-2.

Table 9.6.18-2: Attributes of <application> resource

| Attribute Name of <application> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RO | See section 9.6.1 where this common attribute is described. |
| name | 1 | RO | The (usually human readable) name of the application, as declared by the application developer (eg. “HeatingMonitoring”) |
| App-ID | 1 | RO | Application Identifier |
| App-Inst-ID | 1 | RO | When the application resource refers to an actual application instance in the executing environment. |
| ontologyRef | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is managed and understood by the application; to be passed to the application |

### 9.6.19	Resource Type remoteCSE

A <remoteCse> resource shall represent a remote CSE that is registered to the local CSE. CSE resources shall be located directly under the <baseRoot>.

Conversely each registered CSE shall also be represented as a sub-set of <remoteCse> resource in the registering CSE's <baseRoot>.

For example, when CSE1 registers with CSE2, there will be two <remoteCse> resources created: one in CSE1 <baseRoot1>/<remoteCse2> and one in CSE2 <baseRoot2>/<remoteCse1>.

Note that the creation of the two resources does not imply mutual registration. The <baseRoot1>/<remoteCse2> is only local to CSE1 and is for its internal use.

Figure 9.6.19-1: Structure of <remoteCSE> resource 
(only resource specific attributes are shown)

This resource shall contain the child resources according to their multiplicitly in Table 9.6.19-1 (0 indicates the optionality of the child resource).

Table 9.6.19-1: Child resources of <remoteCSE> resource

| Child Resource Name of <remoteCSE> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <Application> | 0..n | See section 9.6.18. |
| [variable] | <container> | 0..n | See section 9.6.3. |
| [variable] | <group> | 0..n | See section 9.6.5. |
| [variable] | <accessRight> | 0..n | See section 9.6.2. |
| [variable] | <subscription> | 0..n | See section 9.6.9. |
| [variable] | <mgmtObj> | 0..n | See section 9.6.10. |

The <remoteCSE> resource shall contain the attributes described in Table 9.6.19-2.

Table 9.6.19-2: Attributes of <remoteCSE> resource

| Attribute Name of <remoteCSE> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| creationTime (cT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| accessRightID_[i] (aRI_[i]) | 0..n | RW | See section 9.6.1 where this common attribute is described. |
| labels (lBs) | 0..1 | RW | See section 9.6.1 where this common attribute is described. |
| cseType (csT) | 1 | WO | Indicates the type of CSE node represented by the created resource. |
| pointOfAccess_[i] (poA_[i]) | 0..n | RW | For server capable remote CSE it represents the list of physical addresses to be used to connect to it (e.g. IP address, FQDN). The attribute is absent if the remote CSE is not server capable. |
| baseRoot (bRt) | 1 | WO | The URI of the baseRoot. |
| CSE-ID | 1 | WO | The globally unique CSE identifier. |

<remoteCse> and announced <remoteCse> shall have different resourceType coding.

Editor’s Note on CSE-ID: an alignment between CSE-ID and baseURI is needed.

Editor’s Note on baseURI: the alignment with naming convention proposed for inter-M2M service provider is needed.

Editor’s Note on child resources: child resources of <remoteCse> resource can accommodate listed resources and their announced versions.

### 9.6.20	Resource Type m2mServiceSubscription

This resource represents an M2M Service Subscription. It is used to represent all data pertaining to the M2M Service Subscription (the technical part of the contract between an M2M Application provider and an M2M Service Provider).

Editor’s Note: The link between the M2M Service Subscription resource and RBAC will be added once RBAC is included in the specification.

Figure 9.6.20-1: Structure of <m2mServiceSubscription> resource 
(only resource specific attributes are shown)

Editor’s Note: The actual node resource shall include information such as, extID, profile, device specific ID if applicable and link to CSE and Applications running on the node.

Editor’s Note: the M2M-Sub-ID storage is FFS

The <m2mServiceSubscription> resource shall contain the attributes described in Table 9.6.20-1.

Table 9.6.20-1: Attributes of <m2mServiceSubscription> resource

| Attribute Name of <m2mServiceSubscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| parentID (pID) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| expirationTime (eT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| accessRightID (aRI) | 0..1 | RW | See section 9.6.1 where this common attribute is described. If no accessRightID is given at the time of creation, the accessRightID of the parent resource is linked to this attribute |
| creationTime (cT) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| creator (cr) | 1 | RW | See section 9.6.1 where this common attribute is described. |
| lastModifiedTime (lMT) | 1 | RO | See section 9.6.1 where this common attribute is described. |
| serviceId_[i] | 0..n | RW | This attribute indicates the unique ID of the service being subscribed to. A service provider can define its own services. |
| nodeId_[i] | 0..n | RW | Identifier of a node pertaining to the M2M Service Subscription. The node identifier is specified in section 7.1.5 . The nodeId attributes link to the actual node resource that provides information pertaining to the node such as the external identifier, etc. |
| App-Inst-ID_<i> | 0..n | RW | Identifier of an application instance identifier as defined in section 7.1.2. It shall link to an application resource. |

### 9.6.21	Resource Type Members

The <Members> resource is a virtual resource because it does not have a representation. It is the child resource of a <group> resource. Whenever the request is sent to the <Members> resource, the request is fanned out to each of the members of the <group> resource indicated by the membersList attribute of the <group> resource. The responses (to the request) from each member are then aggregated and returned to the Originator.

The <Members> resource does not have a resource representation by itself and consequently it does not have an accessRightID attribute. The <accessRight> resource used for access right validation is indicated by the membersAccessRightID attribute in the parent <group> resource.

### 9.6.22	Resource Announcement

A resource can be announced to one or more remote CSEs to inform the remote CSEs of the existence of the original resource. With the resource announcement, the original resource can be discovered more easily since the remote CSEs with the interests on the specific resource do not need to contact huge range of CSEs. Basically, the announced resource has only limited set of attributes including a link to the original resource hosted by the resource-hosting CSE.

In case that the original resource is deleted, all announced resources for the original resource shall be deleted as well. If the announced resource is not deleted promptly (e.g., the announced resource is not reachable), the announced resource can be deleted later either by the original resource hosting CSE or by the expiration of the announced resource itself. The original resource shall store the list of link for the announced resources for those purposes.

The labels attribute in the announced resource shall have the same value with the original resource in order to facilitate the discovery of the original resource. The synchronization between the original resource and the announced resource is the responsibility of the original resource hosting CSE, and the access right for the announced resource shall be properly configured to allow the updates from the original resource hosting CSE.

Some resources are announceable, while others are not. The resource specification for the announceable resources shall specify an additional column (i.e., <announcedResource>) in the attribute table where the <announcedResource> specifies the announced resource type. In the announced resource, some attributes may be present and some attributes may not be present. The <announcedResource> column shall specify for each attribute one of the below:

- M: Mandatory attributes in the original resource shall be present in the announced resources.

- O: Optional attributes in the original resource may or may not be present in the announced resources depending on the contents of the announcedAttribute attribute of the original resource.

- N/A: This attribute shall not be present in the announced resource

The announceable original resource shall have announceTo and announcedAttribute attributes. The AE or other CSE can request the resource hosting CSE for announcing the original resource to the URI listed in the announceTo attribute. Update on the announceTo attribute will trigger the new resource announcement or the de-announcement of the announced resource. To announce the attribute marked as O, the announcedAttribute attribute can be used. If the attribute marked as O in the original resource is announced to the announced resource, the attribute shall be present in the announced resource; otherwise it shall not be present in the announced resource. Update on the announcedAttribute attribute will trigger the new attribute announcement or the de-announcement of the announced attribute. The announced attribute shall have the same value with the original resource, and the synchronization between the original resource and the announced resource is the responsibility of the original resource hosting CSE.

In the announced resource, the child resource of the original resource shall not be present unless the resource hosting CSE requests to create them. The child resource of the original resource can be announced as well by using the same resource announcement mechanism.

Editor's Note: For the announced resource type, some postfix shall be used for the resource type. The naming convention is FFS.

### 9.6.23	Resource Type xxxx

# 10	Information Flows

Editor’s Note: It is FFS about the suitable section for the proposed generic call flows and potential re-org of the sections due to the correlation of current section 8 and 10.

## 10.1	Basic Procedures

Editor's Note: The procedures in the following sub-clauses assume blocking requests as described in section 8.2.2.

### 10.1.1		CREATE (C)

The CREATE procedure shall be used by an Originator CSE or AE to create a resource on a Receiver CSE (also called the hosting CSE).

Originator requests to create a resource by using the CREATE method. See section 8.1.2 for the information to be included in the Request message.

Receiver, after ensuring that the Originator has been authenticated, creates the resource. The creation of a resource shall always be allowed if the Originator has been authenticated. This is due to the fact that the requested resource is not known prior to the creation by the Originator, therefore the accessRights resource in the Receiver does not contain a priori the permission for the resource that is being created.

Editor' s Note: The statement above assumes that accessRights resources has been agreed to in the resource structure. This is a presumptive statement. The statement above to be reviewed once the definition of resource structure has stabilized.

Figure 10.1.1-1: Procedure for CREATEing a Resource

Step 001: The Originator shall send the following information in the CREATE Request message:

op: 	C (Create);

to: 	URI of parent of the target resource

fr: 	ID of the Originator

Editor's Note: AE or CSE may not have the ID upon its registration request. Alternative information for fr is FFS.

ty:	type the resource to be created

cn: 	mandatory attributes of the resource to be provided by the Originator:

NOTE: Who (Originator or Receiver) provides which mandatory attributes to be specified in the Protocol Technical Specification.

Editor’s Note: references to elements defined in sect 8 have been added here. They may be removed if we decide to move sect 8.1.2-4 to stage 3 doc. In this case, these references will be removed and sect 1 of this document would replace sect 8.1.2-4 in the TS-0001.

NOTE: The identifier of the <target resource> (i.e., the name of the resource) can be optionally provided in the cn. If cn does not contain the identifier of the <target resource>, the hosting CSE shall provide an identifier for that <target resource>.

Step 002:  The Receiver shall:

1.	Verify that the identifier, if provided by the Originator in the Create Request message, does not already exist in its resource structure.

2.	Assign an identifier to the resource to be created if a valid identifier is not provided by the Originator.

3.  Assign default values for mandatory attributes of the resource if not provided by the Originator.

NOTE: How the uniqueness of the identifier is achieved is FFS. <??>.

On successful validation of the Create Request, the Receiver shall create the requested resource.

Step 003: The Receiver shall respond with a Response message that  may contain the following information:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request , rd information can be used.

fr: Optional. ID of the Receiver.

cn:	URI and/or content of the created resource.

See sections 8.1.3 and 8.1.4 for the information to be included in the Response message.

General Exceptions:

1.	The Originator is not registered with the Receiver CSE. The Receiver responds with an Error.

2.	The identifier provided by the Originator already exists at the Receiver. The Receiver responds with an Error.

3.	The Originator is not authorised to Create a resource on the Receiver. The Receiver responds with an Error.

### 10.1.2	RETRIEVE (R)

The RETRIEVE operation shall be used for retrieving the information stored for any of the attributes for a resource at the Receiver CSE. The Originator CSE or AE may also request to retrieve only a specific attribute or part of an attribute by including the identity of such attribute in the Request message.

Editor's Note: The statement in the para above is not clear. Is this operation targeted for retrieving ALL attributes for a resource together (as a Group) or for specific attributes only? Depending on the answer, the text above needs to be updated. The text in the para that follows assumes that this operation is for retrieving all attributes for a resource. If the Request is for retrieving only a specific attribute, then the identity of such attribute is included in the Request message.

Originator requests to retrieve all attributes of the target resource by using RETRIEVE Request.  See section 8.1.2 for the information to be included in the Request message. If only specific attribute(s) or part of an attribute needs to be retrieved, the address of such attribute (e.g., the URI) shall also be included in the Request message.

Receiver performs local processing to verify the existence of requested resource and checks permissions for retrieving the information related to the resource. After the successful verification, the Receiver shall return the requested information, else an error indication shall be returned.

Figure 10.1.2-1:	Procedure for RETRIEVing a Resource

Step 001:	The Originator shall request to RETRIEVE a resource from the Receiver.

op:	R (Retrieve)

to: 	URI of the target resource

fr:	 ID of the Originator

NOTE: If a specific attribute(s) or part of an attribute needs to be retrieved, the address of such attribute(s) (e.g., URI) shall also be included in the cn information. Details are FFS, possibly for WG3 to specify (WG2 to confirm this text).

Step 002:	The Receiver shall verify the existence of the resource and check if the Originator is authorised to retrieve the resource.

Step 003:	The Receiver shall respond with a Response message that shall contain the representation of the resource, and the following information:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request , rd information can be used.

fr: Optional. ID of the Receiver.

cn: 	content of resource retrieved

General Exceptions:

1.	The Originator is not authorized to retrieve the resource. The Receiver responds with an Error.

2.	The resource does not exist. The Receiver responds with an Error.

### 10.1.3	UPDATE (U)

The UPDATE operation shall be used for updating the information stored for any of the attributes at a target resource. Especially important is the "expirationTime", since a failure in refreshing this attribute may result in the deletion of the resource. Alternatively, the Originator CSE or AE can request to update only a specific attribute or part of an attribute at the target resource.

Originator requests to update any of the attributes at the target resource by using UPDATE Request message. The Originator shall send new (proposed) values for the attribute(s) that need to be updated.  See section 8.1.2 for the information to be included in the Request message.

Receiver after verifying the existence of the addressed resource, the validity of the attributes provided and the permissions to modify them, shall update the attributes provided and shall return a Response message to the Originator with the operation results as specified in section 8.1.3.

Figure 10.1.3-1: Procedure for UPDATing a Resource

Step 001:	The Originator shall send the following information in the UPDATE Request message:

op:	U (Update)

to: 	URI to the target resource

fr: 	ID of the Originator

cn: 	Information to be updated at the target resource

Step 002:	The Receiver shall validate if the Originator is authorised to perform the modification to the target resource. On successful validation, the Receiver shall update the resource as requested.

Step 003:	The Receiver shall respond with a Response message that  shall contain the following information:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request , rd information can be used.

fr: Optional. ID of the Receiver.

rs: 	Operation result

General Exceptions:

1.	The Originator is not authorized to update the resource. The Receiver responds with an Error.

2.	The resource does not exist. The Receiver responds with an Error.

### 10.1.4	DELETE (D)

The DELETE operation shall be used by an Originator CSE or AE to delete a resource at a Receiver CSE. The delete procedure shall consist of the deletion of all related information of the target resource.

Originator requests to delete a resource by using a DELETE Request message. See section 8.1.2 for the information to be included in the Request message.

Receiver shall verify the existence of the requested resource, and the permissions for deleting the resource. The Receiver shall remove the target resource and shall return a Response message to the Originator with the appropriate operation results as specified in section 8.1.3.

Figure 10.1.4-1: Procedure for DELETING a Resource

Step 001:	The Originator shall send a DELETE Request message to the Receiver.

op:	D (Delete)

to: 	URI of the target resource

fr: 	ID of the Originator

Step 002:	The Receiver shall verify the existence of the requested resource and if the Originator is authorised to delete the resource. On successful validation, the Receiver shall remove the resource.

Step 003:	The Receiver shall respond with a Response message that shall contain the following information:

to: Optional. ID of the Originator. In case this is a response carrying the result of an operation triggered by a non-blocking request , rd information can be used.

fr: Optional. ID of the Receiver.

rs: 	Operation result

General Exceptions:

1.	The Originator is not authorized to update the resource. The Receiver responds with an Error.

2.	The resource does not exist. The Receiver responds with an Error.

### 10.1.5	NOTIFY (N)

The NOTIFY operation shall be used for notifying information.

Originator: The Originator requests to notify information by using NOTIFY method. See section 8.1.2 for the information to be included in a Request message.

Receiver: The Receiver responds to the Originator with the operation results as specified in section 8.1.3.

Figure 10.1.5-1: Procedure for NOFIFYing Information

Step 001: A notification to be sent to the Receiver is triggered in the Originator.

Step 002: The Originator shall send the following information in the NOTIFY Request message:

op:	N (Notify)

to: 	Address where the notification should be sent to

fr:	ID of the Originator

cn:	Notification data and/or Notification reference

Editor’s Note: cn needs to be updated in section 8.1 to include references to information.

Step 003: The Receiver responds with a Response message that shall contain the following information:

rs: 	operation result

General Exceptions:

1.	If the notification cannot be sent to the URI then error response will be provided by the CSE Receiver.

Editor’s note: General exceptions are FFS in particular for notification failures.

## 10.2	Resource Specific Procedures

### 10.2.1	<container> Resource

#### 10.2.1.1	Create <container>

This flow is used for creating a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: the Receiver or Hosting CSE shall validate whether the Originator has proper permissions for creating a <container> resource.

Table 10.2.1.1-1: <container> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | N/A |
| Information on Request message | op: C fr: Originator App-Inst-ID or CSE-ID to: URI of the parent of the <container> resource cn: Name of <container> resource (optional), Some mandatory and/or optional attributes defined in section 9.6.3 are needed. |
| Local processing on Hosting CSE | No change from the basic procedure (section 10.1.1) |
| Information on Response message | fr: Receiver CSE-ID to: Originator App-Inst-ID or CSE-ID cn: URI of the created <container> if the URI sent by the originator has been changed by the CSE or was not provided in the Request |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE: - The provided attributes are not acceptable to the Hosting CSE or mandatory attributes are not provided by the Hosting CSE. - The CSE does not have storage capacity to create the <container> |

10.2.1.2	Retrieve <container>

This flow is used for retrieving the attributes of a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: the Receiver or Hosting CSE shall validate whether the Originator has proper permissions for retrieving a <container> resource.

Table 10.2.1.2-1: <container> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to retrieve a resource |
| Information on Request message | op: R fr: Originator App-Inst-ID or CSE-ID to: URI of the <container> resource cn: void |
| Local processing on Hosting CSE | No change from the basic procedure (section 10.1.2) |
| Information on Response message | fr: Receiver CSE-ID to: Originator App-Inst-ID or CSE-ID. In case this is a Response carrying the result of an operation triggered by a non-blocking Request, rd information can be used. cn: attributes of the <container> resource Editor’s Note: The procedures for the retrieval of subresources is FFS. |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE: - The provided attributes are not acceptable to the Hosting CSE or mandatory attributes are not provided by the Hosting CSE. |

#### 10.2.1.3	Update <container>

This flow is used for updating the attributes and the actual data of a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: the Receiver or Hosting CSE shall validate whether the originator has proper permissions for updating a <container> resource.

Table 10.2.1.3-1: <container> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to update a resource |
| Information on Request message | op: U fr: Originator App-Inst-ID or CSE-ID to: URI of the <container> resource cn: the attributes which to be updated. |
| Local processing on Hosting CSE | No change from the basic procedure (section 10.1.3) |
| Information on Response message | No change from the basic procedure (section 10.1.3) |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE: - The provided attributes are not acceptable to the Hosting CSE or mandatory attributes are not provided by the Hosting CSE. |

#### 10.2.1.4	Delete <container>

This flow is used for deleting a <container> resource residing under a <container> resource.

Originator: the Originator can be an Application Entity or a CSE.

Receiver or Hosting CSE: the Receiver can be a CSE.

Table 10.2.1.4-1: <container> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to delete a resource |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID to: URI of the <container> resource cn: void |
| Local processing on Hosting CSE | No change from the basic procedure (section 10.1.4). Shall check for child resources and delete all child resources and shall delete child references in parent resources. |
| Information on Response message | No change from the basic procedure (section 10.1.4) fr: Receiver CSE-ID to: Originator App-Inst-ID or CSE-ID. In case this is a response carrying the result of an operation triggered by a non-blocking Request, rd information can be used. cn: void |
| Post-Conditions | None |
| Exceptions | None |

## 10.2.2	Access to Remotely Hosted Resources via CMDH CSF

#### 10.2.2.1	Introduction to use of <delivery> related flows

In this introduction an example for delivering information via the use of the <delivery> resource is explained.

The information flow depicted in Figure 10.2.2.1-1 defines the exchange of Requests/Responses for processing an original request targeting a resource that is not hosted on the Local CSE of the request Originator. The following assumptions hold:

- Originator is AE1;

- AE1 is registered with CSE1, i.e. CSE1 is the local CSE for AE1;

- The original Request is an "UPDATE" to a remote resource hosted on CSE3, i.e. CSE3 is the Hosing CSE for the target resource;

- "UPDATE" options in the original Request are selected such that no feedback after completion of the update operation was requested, i.e. AE1 decided that it does not need to hear back from CSE3; this is expressed by setting the "rc" information in "mi" to "None", see section 8.1.2.

- Delivery related parameters included in "mi" (may be set via a default): "et", "ls" , "ec", "da" and "rp";

- "ls" would reflect how long the forwarding of the request can last at most

- "et" would reflect how long the request shall be processed on the local CSE at most

- "ec"  indicates the event category that should be used by CMDH to handle this request

- "rp" would reflect how long after the request has expired, the local request resources should still be available for retrieving status or result information.

- "da" would be set to on indicating that <delivery> resource shall be used for forwarding the request

- CSE1 is the CSE of an Application Service Node.

- CSE1 is registered with CSE2 and interacts with CSE2 via the reference point Mcc(1).

- CSE2 is the CSE of a Middle Node

- CSE2 is registered with CSE3 and interacts with CSE3 via the reference point Mcc(2)

- CSE3 is the CSE of an Infrastructure Node.

Under these assumptions, the Originator AE1 is getting a confirmation from CSE1 when the original Request is accepted. The response indicates only that CSE1 has accepted the Request and will execute on the requested operation. Furthermore, AE1 has expressed by setting "rc" to "None" that no result of the requested operation is expected to come back from CSE3. With the provided reference (Req-Ref in Figure 10.2.2.1-1), AE1 can retrieve the status of the issued request at a later time, for instance to find out if the request was already forwarded to CSE2 or if it is still waiting for being forwarded on CSE1. Before accepting the request from AE1, CSE1 has also verified if the delivery related parameters expressed by AE1 (settings of "ls", "et" and "ec") are in line with provisioned policies. AE1 may not be authorized to use certain values for "ls" "et" or "ec".

In line with the delivery related parameters, CSE1 is generating a local <delivery> resource on CSE1 and attempts to forward the content of it in line with provisioned policies at a suitable time and via a suitable connection to CSE2 by requesting the creation of a <delivery> resource on CSE2.

CSE2 may confirm the acceptance of the Request for creation of a <delivery> resource to CSE1. Confirmation of acceptance of a Request to create a <delivery> resource between two CSEs – here the acceptance by CSE2 indicated to CSE1 – is subject to details in protocol specifications and may not always occur in CMDH processing.

When CSE2 has accepted the incoming request from CSE1, CSE1 may delete the data attribute of the local <delivery> resource. Furthermore – if the expiration time of the local <delivery> resource is not exhausted – the local CSE shall update the status of the local <delivery> resource to indicate that it has been forwarded to CSE2. CSE1 shall also update the status of the original request to indicate that it has been forwarded and it may delete the data attribute of the original request.

When CSE2 has received (and accepted) the Request to create a local <delivery> resource, it will attempt to forward it to CSE3. In line with the delivery related parameters, CSE2 is generating a local <delivery> resource on CSE3 and attempts to forward it in line with provisioned policies at a suitable time and via a suitable connection to CSE3 by requesting the creation of a <delivery> resource on CSE3.

CSE3 may confirm the acceptance of the Request for creation of a <delivery> resource to CSE2. Confirmation of acceptance of a Request to create a <delivery> resource between two CSEs – here the acceptance by CSE3 indicated to CSE2 – is subject to details in protocol specifications and may not always occur in CMDH processing.

When CSE3 has accepted the incoming Request from CSE2, CSE2 may delete the data attribute of the local <delivery> resource. Furthermore – if the expiration time of the local <delivery> resource is not exhausted – the local CSE shall update the status of the local <delivery> resource to indicate that it has been forwarded to CSE3.

When CSE3 has received (and accepted) the request to create a local <delivery> resource, it will determine that the target of the delivery was CSE3 itself. Therefore it will forward internally the original request contained in the data attribute of the <delivery> resource.

Within CSE3, CSFs that are responsible for checking and executing local access to resources in DMR CSF will execute the originally requested UPDATE operation. If successful, the targeted resource will be updated with the content provided by the Originator.

Since in the depicted case no result needed to be sent back to the Originator, the processing for the requested operation is then completed.

Figure 10.2.2.1-1: CMDH information flow for 2 hops – 
no result needs to be returned after operation completes

The following procedures are triggered by requesting the corresponding operations on a <delivery> resource:

- Initiate the delivery of one or more original request stored for later forwarding from one CSE to another CSE

- Request a CREATE operation for a <delivery> resource from an issuing CSE to a receiving CSE.

- The original request(s) need to be contained in the "data" attribute of the <delivery> resource.

- If successful, the receiving CSE takes the responsibility to further execute on the delivery process for the original Request.

- If not successful, the issuing CSE cannot assume that the receiving CSE will carry out the delivery of the original request.

- Get information about the status of a pending delivery process for an original request

- Request a RETRIEVE operation of the content of a <delivery> resource representing a pending delivery or part of it

- The status of the pending forwarding process is reflected the "deliveryMetaData" attribute defined in the <delivery> resource

- Change parameters of the delivery process

- Request an UPDATE operation on applicable attributes of the <delivery> resource representing the pending delivery

- For instance the lifespan of a delivery request could be modified by updating the "lifespan" attribute

- Cancel a pending delivery request

- Request a DELETE operation of a <delivery> resource that represents a pending delivery process.

#### 10.2.2.2	Create <delivery>

This procedure shall be used for requesting a CSE to take responsibility to deliver the provided data to a target CSE in line with CMDH parameters and provisioned policies in case <delivery> resource based CMDH processing is used. If indicated by the Originator, the Receiver shall confirm the acceptance of delivery responsibility by a successful Response.

Originator: The Originator of a Create request for a <delivery> resource can only be a CSE. The Originator needs to provide the content of a <delivery> resource type together with the Create request or can Update it after a successful creation of the <delivery> resource with empty data attribute. Otherwise the Receiver cannot accept the Create Request.

Receiver: The receiver of a Create request for a <delivery> resource will check whether the Originator is authorized to request a delivery procedure. The Receiver of the Create Request will further check whether the provided attributes of the <delivery> resource that is requested to be created represents a valid request for forwarding data to a target CSE. If the Originator of the Create request is authorized and the Request is valid, the Receiver will check whether it can actually satisfy the requested delivery in line with provisioned policies and requested parameters for CMDH processing. If all these checks are positive, the Receiver will create the requested <delivery> resource and assumes responsibility for delivering the requested data to the target CSE as soon as the content of the data attribute is available. In case an operation result is expected by the Originator, the Receiver will confirm acceptance of the responsibility by indicating a successful creation of the <delivery> resource. If the Receiver CSE is the target CSE of the requested delivery, it shall forward the content of the delivered data – which represents one or more forwarded original request(s) -  to the internal CSFs that handle incoming requests and continue processing of the forwarded request(s).

Editor’s Note: This call flow template will be evolving as we handle more examples.

Table 10.2.2.2-1: <delivery> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | Originator is a CSE and is responsible for delivery of data to a target CSE. Originator needs to forward the data and delivery responsibility to the Receiver. |
| Information on Request message | cn: - Resource Type identifier for <delivery> resource - Content of a <delivery> resource in line with the definition in Section 9.6.4 representing a valid request for delivery of data to a target CSE - The data attribute may be provided by an Update at a later time. - (optional) Unique Identifier within the Receiver CSE for the <delivery> resource All other information in the Request message are the same as in the generic procedure |
| Local processing on Hosting CSE | - Check whether the originator is authorized to request a delivery procedure on the receiver CSE - Check whether the provided attributes of the <delivery> resource that is requested to be created represents a valid request for delivering data to a target CSE. - Check whether Receiver CSE can actually satisfy the requested delivery in line with provisioned policies and requested delivery parameters - If all checks are positive, the receiver will create the requested <delivery> resource and assumes responsibility for delivering the provided data to the target CSE. - If the Receiver CSE is the target CSE of the requested delivery, it shall forward the content of the delivered data to the internal CSFs that will interpret the delivered data as a forwarded request from a remote Originator. |
| Information on Response message | In case the Originator CSE has not asked for a Result of the requested Operation, the Response only contains an Acknowledgement indicator. This only indicates that the Receiver CSE received the Request. It does NOT indicate whether the Receiver CSE was able to take on responsibility for delivery of the data. In case the Originator CSE asked for the status of the requested Operation to be contained in the Result of the requested Operation, the Receiver CSE shall respond with a Success or Failure indicator. In case the Originator CSE asked for the status of the requested Operation and the URI of the created Resource to be contained in the Result of the Request, the Receiver CSE shall respond with a Success indicator including the URI of the created <delivery> resource in case it has taken on responsibility to deliver the data to the target CSE or with Failure indicator including an error indication otherwise. |
| Post-Conditions | The Originator CSE shall update the local <delivery> resource to reflect the new status of the delivery process (e.g. ‘{Receiver-CSE-ID} accepted delivery responsibility’). In case the Originator CSE got a Success indicator as a Response, it shall stop any further delivery attempts. In that case or if there was no indication of a need to provide a result of the operation, the Originator CSE may delete the content of the ‘data’ attribute of the local <delivery> resource. In case the Originator CSE got a Failure indicator as a response, it may initiate further delivery attempts in line with policies and delivery parameters and depending on the reason for Failure. In case the Receiver CSE is the target CSE of the delivery, the Receiver CSE needs to execute on the forwarded request contained in the delivered data. |
| Exceptions | - The Originator CSE is not authorized to request a delivery procedure on the Receiver CSE - The provided content of the <delivery> resource is not in line with the specified structure. - The provided content of the <delivery> resource represents a request for delivery that is not consistent (e.g. lifespan already expired) - The provided content of the <delivery> resource represents a request for delivery that cannot be met by the Receiver CSE within the limits of the provided delivery parameters and the provisioned policies on the Receiver CSE. |

#### 10.2.2.3	Retrieve <delivery>

This procedure shall be used for requesting a CSE to provide information on a previously created <delivery> resource which represents delivery of data to a target CSE.

Originator: Originator can be a CSE or an Application.

Receiver: The Receiver shall provide the content of the addressed <delivery> resource or the addressed attributes thereof.

Editor’s Note: This call flow template will be evolving as we handle more examples.

Table 10.2.2.3-1: <delivery> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to retrieve information about a previously issued delivery. |
| Information on Request message | cn: - URI of the <deliver> resource - (optional) filter to retrieve only certain attributes. All other information in the Request message are the same as in the generic procedure |
| Local processing on Hosting CSE | No change to generic procedure. |
| Information on Response message | No change to generic procedure. |
| Post-Conditions | None |
| Exceptions | - The Originator CSE is not authorized to retrieve the <delivery> resource or the addressed parts of it. - The addressed <delivery> resource does not exist. |

#### 10.2.2.4	Update <delivery>

This procedure shall be used for requesting a CSE to update information on a previously created <delivery> resource which represents a pending delivery of data to a target CSE. The update may have impact on further processing of the delivery.

Originator:  Originator can be a CSE or an application. Originator needs to be authorized to modify the addressed <delivery> resource, i.e. it needs to be authorized to change the represented delivery process.

Receiver:  The Receiver shall change the content of the addressed <delivery> resource or the addressed attributes thereof and also modify the corresponding parameters of the delivery process

Editor’s Note: This call flow template will be evolving as we handle more examples.

Table 10.2.2.4-1: <delivery> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to modify information about a previously issued delivery that is still pending, i.e. it has not yet been forwarded to another CSE. |
| Information on Request message | cn: - URI of the <deliver> resource - Content of a <delivery> resource in line with the definition in Section 9.6.4 representing a valid request for delivery of data to a target CSE All other information in the Request message are the same as in the generic procedure. |
| Local processing on Hosting CSE | - Receiver CSE checks if the requested changes to the delivery process can actually be accomplished. - If possible, the Receiver CSE modifies the previously established delivery process and changes the respective content of the <delivery> resource. |
| Information on Response message | Same as in the generic procedure. Successful Response messages indicate that the delivery process was modified as requested. |
| Post-Conditions | None |
| Exceptions | - The Originator CSE is not authorized to modify the <delivery> resource or the addressed parts of it. - The addressed <delivery> resource does not exist. - The responsibility for the further processing of the delivery process represented by the addressed <delivery> process was already forwarded to another CSE. |

#### 10.2.2.5	Delete <delivery>

This procedure shall be used for requesting a CSE to cancel a pending delivery of data to a target CSE or to delete the <delivery> resource of an already executed delivery.

Originator:  Originator can be a CSE or an Application. Originator needs to be authorized to modify the addressed <delivery> resource, i.e. it needs to be authorized to change the represented delivery process.

Receiver:  The Receiver shall remove the addressed <delivery> resource and stop the corresponding delivery process if it is still pending

Editor’s Note: This call flow template will be evolving as we handle more examples.

Table 10.2.2.5-1: <delivery> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to cancel a previously issued delivery that is still pending, i.e. it has not yet been forwarded to another CSE or Originator needs to remove the <delivery> resource representing an already executed delivery. |
| Information on Request message | No change to generic procedure. |
| Local processing on Hosting CSE | - Receiver CSE checks if the corresponding delivery process is still pending. If so, it stops that delivery process. - Receiver CSE removes the addressed <delivery> resource. |
| Information on Response message | Same as in the generic procedure. Successful Response messages indicate that the delivery process was stopped as requested. |
| Post-Conditions | None |
| Exceptions | - The Originator CSE is not authorized to delete the <delivery>. - The addressed <delivery> resource does not exist. |

### 10.2.3	Resource Discovery Procedure

#### 10.2.3.1		Introduction

The resource discovery procedures allow discovering of resources residing on a CSE. The use of filter criteria allows limiting the scope of the results. Filtering shall be performed on a subset of the offered resource attributes using a query string. A match, that may include ranges, may be performed on the query string, and a successful response may be returned with a URI(s) list for the resources that contains the matching attributes or a URI of a temporary resource holding the URI(s) list.

Editor’s Note: It is FFS how this temporary resource is operated such as the access right of this temporary resource, expiration time, the condition for creation.

Resource discovery shall be done using the RETRIEVE method by an Originator which shall also include the root of where the discovery begins: <startURI>. Filter criteria may be provided as a parameter to the Retrieve method. The filter criteria describe the rules for resource discovery, e.g. searching scope, topic, semantic relationships, and matching string. The filter criteria can also contain the parameters for specifying the maximum size of the answer (upper limit), and/or sorting criteria for specifying in which order the searching result should be organized.

The resource discovery procedures shall identify all matching resources from the entire hierarchy under <startURI>. The filter criteria provided by an Originator or the discovery results may be modified by the Hosting CSE to restrict the scope of discoverable resources according to the Originator’s access right or service subscription.

The Hosting CSE may also implement a configured upper limit on the size of the answer. In such a case when both the Originator and the Hosting CSE have the upper limits, the minimum of the upper limit in the Hosting CSE and the upper limit of the Originator shall apply.

#### 10.2.3.2		Discovery procedure via Retrieve Operation

This procedure shall be used for the discovery of resources under <startURI> that match the provided filter criteria. The discovery result shall be returned to the Originator using a successful message.

Originator:  The Originator can be an AE or a CSE. The Originator shall provide the <startURI> URI in the Request message. Filter criteria may be provided as a parameter by the Originator.

Receiver or Hosting CSE:  The Hosting CSE shall read the values of all attributes belonging to the addressed resource structure and the references of all sub-resources and it shall build a representation of these. If filter criteria is provided in the request, the Hosting CSE uses it identifying the resources whose attributes mach the filter criteria. The Hosting CSE shall respond to the Originator with the appropriate list of URIs of discovered resources in the Hosting CSE. If sorting criteria has been provided by the Originator, the list of URIs of discovered resources shall be sorted in that order.

The Hosting CSE may modify the filter criteria including upper limit provided by the Originator or the discovery results based on the local policies.

If the size of the result URI list is bigger than the upper limit, or the scope of discoverable resources according to the Originator’s access right or service subscription has been reduced by the Hosting CSE the full list is not returned. Instead, an incomplete list is returned and an indication is added in the response for warning the device.

The Hosting CSE may propagate the discovery request to other CSEs according to resource discovery policies.

Editor's Note: It is FFS about discovery propagation mechanism, i.e., multi-hop discovery propagation and its policies.

Table 10.2.3.2-1: Discovery procedure via Retrieve Operation

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to discover resources in the Hosting CSE. |
| Information on Request message | - URI of the discovery resource: <startURI>. - Filter criteria for searching and expected returned result. |
| Local processing on Hosting CSE | - Checks the validity of the Request (e.g., format of filter criteria). - Checks if the request is in accordance with the M2M service subscription. - May change the filter criteria according to local policies. - Searches matched resources from the addressed resource hierarchy. - Propagates the discovery request to other CSEs if it is permitted by discovery policies. - Limits the discovery result according to access rights of the discovered resources. - Limits the discovery result according to the upper limit on the size of the answer. - Sorts the results according to the sorting criteria. |
| Information on Response message | - Contains the URI list of discovered resources. - Contains an incomplete list warning if the full list is not returned. |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE: - The requesting M2M AE or CSE is not registered. - The request contains invalid parameters. |

### 10.2.4	Group Management Procedures

#### 10.2.4.1	Introduction

This clause describes different procedures for managing membership verification, creation, retrieval, update and deletion of the information associated with a <group> resource as well as the bulk management of all group member resources by invoking the corresponding verbs upon the virtual resource members of a <group> resource.

#### 10.2.4.2	Create <group>

This procedure shall be used for creating a group resource.

Originator: The Originator shall request to Create a new group type resource to be named as <group> by using the CREATE verb. The request shall address baseURI resource of a Hosting CSE. The Request shall also provide list of member URI and may provide expirationTime attributes. The list of member URI means a list of URIs of the member resources corresponding to the memberType attribute provided in the Request. The originator may be an AE or a CSE.

Receiver:  For the CREATE procedure, the Receiver shall:

- Check if the Originator has CREATE permissions on the baseURI resource.

- Check the validity of the provided attributes.

- Validate that the resource type of every member conforms to the memberType attribute of the <group> resource, if the memberType attribute of the <group> resource is not ‘mixed’. Set the memberTypeValidated attribute to TRUE upon successful validation.

- Upon successful validation of the provided attributes, Create a new group resource with name <group> including the members attribute in the Hosting CSE.

- Conditionally, in the case that the group resource contains temporarily unreachable sub-group resources as member resource, set the memberTypeValidated attribute of the <group> resource to FALSE.

- Respond to the Originator with the appropriate generic Response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the URI of the created <group> resource if the CREATE was successful.

- As soon as any unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request. Or by default if the attribute is not provided.

Table 10.2.4.2-1: <group> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the baseURI where the <group> resource is intended to be Created. cn: The representation of the <group> resource for which the attributes are described in section 9.5 |
| Local processing on Hosting CSE | Steps described for the Receiver of the CREATE Request as described above. |
| Information on Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.4.3	Retrieve <group>

This procedure shall be used for retrieving <group> resource.

Originator: The Originator shall request to obtaining <group> resource information by using the RETRIEVE verb. The request shall address the specific <group> resource of a hosting CSE. The Originator may be an AE or a CSE.

Receiver:  The Receiver shall check if the Originator has READ permission on the group resource. Upon successful validation, the hosting CSE shall respond to the Originator with the appropriate responses described in section xxx and the resource representation.

Table 10.2.4.3-1: <group> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource. |
| Local processing on Hosting CSE | Same as the generic procedure. |
| Information on Response message | Same as the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.4.4	Update <group>

This procedure shall be used for updating an existing <group> resource.

Originator: The Originator shall request to Update attributes of an existing <group> resource by using an UPDATE verb. The Request shall address the specific <group> resource of a CSE. The Originator may be an AE or a CSE.

Receiver:  The UPDATE procedure shall be:

- Check if the Originator has WRITE permissions on the <group> resource.

- Check the validity of provided attributes.

- Validate that the resource type of every member conforms to the memberType attribute of the <group> resource, if the memberType attribute of the <group> resource is not ‘mixed’. Set the memberTypeValidated attribute to TRUE upon successful validation.

- Upon successful validation of the provided attributes, create a new group resource with name <group> including the members attribute in the hosting CSE.

- Conditionally, in the case that the group resource contains temporarily unreachable sub-group resources as member resource set the memberTypeValidated attribute of the <group> resource to FALSE.

- Respond to the Originator with the appropriate generic response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the URI of the created <group> resource if the UPDATE is successful.

- As soon as any unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request. Or by default if the attribute is not provided

Table 10.2.4.4-1: <group> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource. |
| Local processing on Hosting CSE | Steps described for the Receiver of the UPDATE Request as described above. |
| Information on Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.4.5	Delete <group>

This procedure shall be used for deleting an existing <group> resource.

Originator: The Originator shall request to delete an existing <group> type resource by using the DELETE verb. The request shall address the specific <group> resource of a Hosting CSE. The Originator may be an application or a CSE.

Receiver: The Receiver shall check if the Originator has DELETE permission on the <group> resource. Upon successful validation, the CSE shall remove the resource from its repository and shall respond to the Originator with the appropriate responses.

Table 10.2.4.5-1: <group> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource. |
| Local processing on Hosting CSE | Same as the generic procedure. |
| Information on Response message | Same as the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

Editor's Note:  The original contributor has been reqeusted to provide update/fixes for sections 10.2.4.6 to 10.2.4.9 that are in line with the ARC TS notation rules. The contributor is requested to verify the cross references provided by the Rapporteur and help provide missing cross-references as well.

#### 10.2.4.6	Create <Members>

This procedure shall be used for creating the content of all member resources belonging to an existing <group> resource.

Originator: The Originator shall request to create the content in all member resources belonging to an existing <group> resource by using a CREATE verb. The Request may address the virtual child resource members of the specific <group> resource of a group hosting CSE to create the same content under all member resources. The request may also address the URI that results from appending a relative URI to the members URI in order to create the same content (e.g. attribute or child resource) under the corresponding attributes or child resources represented by the relative URI with respect to all member resources. The Originator may be an AE or CSE.

Group Hosting CSE: For the CREATE procedure, the Group Hosting CSE shall:

- Check if the Originator has WRITE permission in the accessRight resource referenced by the membersAccessRightID in the <group> resource. In the case membersAccessRightID is not provided the access right defined for the <group> resource shall be used.

- Upon successful validation, obtain the URIs of all member resources from the attribute membersList of the addressed <group> resource.

- Fan out requests addressing the obtained URIs (appended with the relative URI if any) to the member hosting CSEs as indicated in Figure 10.2.4.6-1.

- In the case that the member resources contain a sub-group resource, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple member resources belong to one CSE according to the URIs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the member hosting CSE to collect all the members on that member hosting CSE.

- After receiving the responses from the member hosting CSEs, respond to the Originator with the aggregated results and the associated <memberId>s.

Member Hosting CSEs: For the CREATE procedure, the Member Hosting CSE shall:

- Treat the request received from the group hosting CSE as a normal CREATE request on the addressed resource as if it comes from the original Originator.

- Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifiers stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier.

- Check if the original Originator has the CREATE permission on the addressed resource. Upon successful validation, perform the create procedures for the corresponding type of addressed resource as described in other subclauses of section 10.2.

- Send the corresponding response to the Group Hosting CSE.

Editor’s Note: How to aggregate synchronous and asynchronous responses is to be considered in stage 3.

Editor’s Note: How to generate and maintain the gid is to be FFS is stage 3.

Figure 10.2.4.6-1: Group content management procedures

The procedures illustrated in Figure 10.2.4.6-1 apply to sections 10.2.4.6 to 10.2.4.9:

Table 10.2.4.6-1: <Members> CRETAE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the members virtual resource. cn: The representation of the resource the Originator intends to create gid: The group request identifier |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any. Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups. |
| Information on Response message | Converged responses from member hosting CSEs. |
| Post-Conditions | None |
| Exceptions | Same request with identical group request identifier received. Originator does not have the access right to access the members resource. |

#### 10.2.4.7	Retrieve <Members>

This procedure shall be used for retrieving the content of all member resources belonging to an existing <group> resource.

Originator: The Originator shall request to obtain the content or specific information (e.g. attributes) of all member resources belonging an existing <group> resource by using a RETRIEVE verb. The request may address the virtual child resource members of the specific <group> resource of a group hosting CSE for retrieving the content of all member resources. The request may also address the URI that results from appending a relative URI to the members URI in order to retrieve the corresponding attributes or child resources represented by the relative URI with respect to all member resources. The originator may be an AE or CSE.

Group Hosting CSE: For the RETRIEVE procedure, the Group Hosting CSE shall:

- Check if the Originator has READ permission in the accessRight resource referenced by the membersAccessRightID in the addressed <group> resource. In the case membersAccessRightID is not provided the access right defined for the group resource shall be used.

- Upon successful validation, obtain the URIs of all member resources from the membersList attribute of the addressed <group> resource.

- Fan out requests addressing the obtained URIs (appended with the relative URI if any) to the member hosting CSEs as indicated in Figure 10.2.4.6-1.

- In the case that the member resources contain a sub-group resource, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple member resources belong to one CSE according to the URIs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the member hosting CSE to collect all the members on that member hosting CSE.

- After receiving the responses from the member hosting CSEs, respond to the originator with the aggregated results and the associated <memberId>s.

Member Hosting CSEs: For the RETRIEVE procedure, the Member Hosting CSE shall:

- Treat the Request received from the group hosting CSE as a normal RETRIEVE request on the addressed resource as if it comes from the original originator.

- Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the request identifier.

- Check if the original originator has the READ permission on the addressed resource. Upon successful validation, perform the retrieve procedures for the corresponding type of addressed resource as described in other subclauses of section 10.2.<??>

- Send the corresponding response to the group hosting CSE.

Table 10.2.4.7-1: <Members> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the members resource. gid: The group request identifier. |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any. Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups. |
| Information on Response message | Converged responses from member hosting CSEs. |
| Post-Conditions | None |
| Exceptions | Same request with identical request identifier received. Originator does not have the access right to access the members resource. |

#### 10.2.4.8	Update <Members>

This procedure shall be used for updating the content of all member resources belonging to an existing <group> resource.

Originator: The Originator shall request to update the content of all member resources belonging to an existing <group> resource with the same new data by using a UPDATE verb. The request may address the virtual child resource members of the specific <group> resource of a group hosting CSE to update all member resources. The request may also address the URI that results from appending a relative URI to the members in order to update only the corresponding attributes or child resources represented by the relative URI with respect to all member resources. The originator may be an AE or CSE.

Group Hosting CSE: For the UPDATE procedure, the Group Hosting CSE shall:

- Check if the originator has WRITE permission in the accessRight resource referenced by the membersAccessRightID in the group resource. In the case membersAccessRightID is not provided the access right defined for the group resource shall be used.

- Upon successful validation, obtain the URIs of all member resources from the attribute membersList of the addressed <group> resource.

- Fan out requests addressing the obtained URIs (appended with the relative URI if any) to the member hosting CSEs as indicated in Figure 10.2.4.6-1.

- In the case that the member resources contain a sub-group resource, generate a unique group request identifier, include it in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple member resources belong to one CSE according to the URIs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the member hosting CSE to collect all the members on that member hosting CSE.

- After receiving the responses from the member hosting CSEs, respond to the originator with the aggregated results and the associated <memberId>s.

Member Hosting CSEs: For the UPDATE procedure, the Member Hosting CSE shall:

- Treat the request received from the group hosting CSE as a normal UPDATE request on the addressed resource as if it comes from the original originator.

- Check if the request has a group request identifier.Check if the request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the request identifier.

- Check if the original originator has the UPDATE permission on the addressed resource. Upon successful validation, perform the update procedures for the corresponding type of addressed resource as described in other subclauses of section 10.2 <??>

- Send the corresponding response to the group hosting CSE.

Table 10.2.4.8-1: <Members> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource. cn: The representation of the resource the Originator intend to Udpate. gid: The group request identifier. |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any. Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups. |
| Information on Response message | Converged responses from member hosting CSEs. |
| Post-Conditions | None |
| Exceptions | Same request with identical request identifier received. Originator does not have the access right to access the members resource. |

#### 10.2.4.9	Delete <Members>

This procedure shall be used for deleting the content of all member resources belonging to an existing <group> resource.

Originator: The Originator shall request to delete the content of all member resources belonging to an existing <group> resource by using a DELETE verb. The request may address the virtual child resource members of the specific <group> resource of a group hosting CSE to delete all member resources. The request may also address the URI that results from appending a relative URI to the members in order to delete only the corresponding attributes or child resources represented by the relative URI with respect to all member resources. The originator may be an AE or a CSE.

Group Hosting CSE: For the DELETE procedure, the Group Hosting CSE shall:

- Check if the Originator has WRITE permission in the accessRight resource referenced by the membersAccessRightID in the <group> resource. In the case membersAccessRightID is not provided the access right defined for the group resource shall be used.

- Upon successful validation, obtain the URIs of all member resources from the attribute membersList of the addressed <group> resource.

- Fan out requests addressing the obtained URIs (appended with the relative URI if any) to the member hosting CSEs as indicated in Figure 10.2.4.6-1.

- In the case that the member resources contain a sub-group resource, generate a unique group request identifier, include the group request identifier in all the requests to be fanned out and locally store the group request identifier.

- If the group hosting CSE determines that multiple member resources belong to one CSE according to the URIs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the group hosting CSE creating a <group> resource on the member hosting CSE to collect all the members on that member hosting CSE.

- After receiving the responses from the member hosting CSEs, respond to the originator with the aggregated results and the associated <memberId>s.

Member Hosting CSEs: For the DELETE procedure, the Member Hosting CSE shall:

- Treat the request received from the group hosting CSE as a normal DELETE request on the addressed resource as if it comes from the original originator.

- Check if the request has a group request identifier. Check if the group request identifier is contained in the requested identifier stored locally. If match is found, ignore the current request and respond an error. If no match is found, locally store the group request identifier.

- Check if the original originator has the DELETE permission on the addressed resource. Upon successful validation, perform the delete procedures for the corresponding type of addressed resource as described in other subclauses of clause 10.2<??>

- Send the corresponding response to the Group Hosting CSE.

Table 10.2.4.9-1: <Members> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <group> resource. gid: The group request identifier. |
| Local processing on Hosting CSE | Fan out requests to each member hosting CSE addressing the obtained URI from the attribute membersList of the group resource appended with relative URI if any. Generate group request identifier and include the identifier in the fanned out requests in the case of sub groups. |
| Information on Response message | Converged responses from member hosting CSEs. |
| Post-Conditions | None |
| Exceptions | Same request with identical request identifier received. Originator does not have the access right to access the members resource. |

### 10.2.5	mgmtObj Management Procedures

#### 10.2.5.1	Introduction

This clause describes the management procedures over Mca and Mcc reference points. Different RESTful requests addressing a <mgmtObj> resource (or its attributes or child resources) shall be translated into existing management commands and procedures performed on the mapped external management object on the managed entity. The Receiver in the following procedures is IN-CSE.

#### 10.2.5.2	Create <mgmtObj>

This procedure is used to create a specific <mgmtObj> resource in the hosting CSE to expose the corresponding management function of a managed entity (i.e. M2M Device/Gateway) over Mca reference point. Depending on the data model being used, the created <mgmtObj> resource may be a partial or complete mapping from the external management object on the managed entity. If such external management object is missing from the managed entity, it shall be added to the managed entity. Further RESTful operations performed on the created <mgmtObj> resource shall be converted by the hosting CSE into a corresponding device management action performed on the mapped external management object on the managed entity using existing device management protocols (e.g. OMA-DM or BBF TR-069).

Originator: The Originator shall request to create a new <mgmtObj> resource to be named as "mgmtObj" by using a CREATE verb. The request shall address baseURI resource of the hosting CSE. If the <mgmtObj> resource contains one or more <parameters> child resources which may also contains <parameters> child resources recursively, the Originator may create each of the <parameters> child resource step by step as needed by sending the CREATE Request, addressing the parent resource of the <parameters> child resource.

The Originator may be:

- The CSE on the managed entity: In this case, the CSE first collects the original external management object (the management tree structure or also the value of the tree nodes if needed) of the local device and transforms the data into the <mgmtObj> resource representation, then requests the hosting CSE to create the corresponding <mgmtObj> resource.

- An AE: In this case, the AE requests the hosting CSE to add the corresponding external management object to the managed entity by creating an <mgmtObj> resource in the hosting CSE.

NOTE 1:	The IN-CSE may also create the <mgmtObj> resource locally by itself. The details are out of scope. In this case, the hosting CSE first collects the original external management object on the managed entity via existing device management protocols (e.g. OMA DM, BBF TR-069 or LWM2M), then transforms the object into the <mgmtObj> resource representation and create the <mgmtObj> resource locally in the IN-CSE.

NOTE 2:	The <mgmtObj> resource may also be created in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: For the CREATE procedure, the Receiver shall:

- Check if the Originator has the CREATE permission on the addressed baseURI resource (or the parent <mgmtObj> or <parameters> resource in the case of child resource creation). Check the validity of provided attributes.

- Upon successful validation, create a new <mgmtObj> resource with name "mgmtObj" in the hosting CSE with the provided attributes.

- If the originator is an AE: Check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to the managed entity or to the management server to add the corresponding external management object to the managed entity based on existing management protocol.

- Maintain the mapping relationship between the created <mgmtObj> resource and the external management object on the managed entity.

- Respond to the originator with the appropriate generic responses. It shall also provide in the response the URI of the created new resource.

Table 10.2.5.2-1: <mgmtObj> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of the AE or the CSE that initiates the Request to: Th eURI of the baseURI where the <mgmtObj> resource is intended to be Created. cn: The representation of the <mgmtObj> resource for which the attributes are described in section 9.10. |
| Local processing on Hosting CSE | New external management object created on the managed entity using external management technologies in case the originator is IN-AE. |
| Information on Response message | Proper error code if the new external management object is not created. |
| Post-Conditions | None |
| Exceptions | - The creation of the external management object is not allowed. - The created external management object already exists. - Corresponding external management object cannot be added to the managed entity due to some reason (e.g. not reachable, memory shortage) |

#### 10.2.5.3	Retrieve <mgmtObj>

This procedure is used to retrieve information from an existing <mgmtObj> resource or its <parameters> child resources.

Originator: The Originator shall request to retrieve an existing <mgmtObj> resource by using a RETRIEVE verb. The request shall address a specific "mgmtObj" resource of a CSE or a specific <parameters> child resource of the <mgmtObj> resource to retrieve all attributes of the <mgmtObj> resource or the <parameters> child resource.

The originator may be:

- An IN-AE

- An ASN/MN-CSE on the managed entity

Receiver: For the RETRIEVE procedure, the Receiver shall:

- Check if the originator has the READ permission on the addressed <mgmtObj> resource (or the <parameters> child resource in the case of child resource retrieval)

- Upon successful validation, retrieve the corresponding <mgmtObj> resource or the <parameters> child resource including all attributes and references to all its child resources from its repository

- Respond to the originator with the appropriate generic responses

- If the Originator is an IN-AE and if the requested information of the <mgmtObj> or <parameters> resource is not available, identify the corresponding external management object on the managed entity according to the mapping relationship it maintains. Check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to get the corresponding external management object from the managed entity based on existing device management protocol (e.g. OMA-DM, BBF TR-069), then return the result to the originator.

Table 10.2.5.3-1: <mgmtObj> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource. |
| Local processing on Hosting CSE | Get corresponding external management object from managed entity by existing management procedures in case the addressed <mgmtObj> or <parameters> is not available. |
| Information on Response message | Proper error code if the external management object can not be retrieved. |
| Post-Conditions | None |
| Exceptions | Corresponding external management object data cannot be retrieved from managed entity due to some reason (e.g. external management object not found) |

#### 10.2.5.4	Update <mgmtObj>

This procedure is used to update information of an existing <mgmtObj> resource or its <parameters>child resource.

Originator: The Originator shall request to update information of an existing <mgmtObj> resource by using an UPDATE verb. The Request shall address a specific <mgmtObj> resource or a specific <parameters> child resource of the <mgmtObj> resource on the hosting CSE to update the all attributes of the <mgmtObj> resource or the <parameters> child resource with new values when using UPDATE verb; or

The Originator may be:

- IN-AE.

- CSE on the managed entity.

Receiver: For the UPDATE procedure, the Receiver shall:

- Check if the Originator has the WRITE permission on the address <mgmtObj> resource (or the <parameters> child resource in the case of child resource update). Check the validity of provided attributes if any.

- Upon successful validation, update the corresponding attribute(s) of the <mgmtObj> resource or the <parameters> child resource accordingly.

- If the originator is an IN-AE, identify the corresponding external management object on the managed entity according to the mapping relationship it maintains. Check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to update the corresponding external management object in the managed entity accordingly based on existing device management protocol (e.g. OMA DM, BBF TR-069 or LWM2M).

- Respond to the Originator with the appropriate generic responses.

Table 10.2.5.4-1: <mgmtObj> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource. cn: The representation of the <mgmtObj> resource for which the attributes are described in section 9.10. |
| Local processing on Hosting CSE | Override corresponding external management object in managed entity by existing management procedures. |
| Information on Response message | Proper error code if the external management object can not be updated. |
| Post-Conditions | None |
| Exceptions | Corresponding external management object cannot be updated to managed entity due to some reason (e.g. not reachable, external management object not found). |

#### 10.2.5.5	Delete <mgmtObj>

This procedure is used to delete an existing <mgmtObj> resource or its <parameters> child resource so as to hide the management function of a managed entity (i.e. M2M Device/Gateway) from being exposed via Mca reference point. An IN-AE uses this procedure to remove the corresponding external management object (e.g. an obsolete software package) from the managed entity.

Originator: The Originator shall request to delete an existing <mgmtObj> resource or its <parameters> child resource by using a DELETE verb. The request shall address the specific <mgmtObj> resource or a specific <parameters> child resource of the <mgmtObj> resource on the hosting CSE.

The originator may be:

- The CSE on the managed entity: In this case, the CSE issues the request to the hosting CSE to hide the corresponding management function from being exposed by the <mgmtObj> resource.

- An IN-AE: In this case, the IN-AE requests the hosting CSE to delete the <mgmtObj> resource from the hosting CSE and to remove the corresponding external management object from the managed entity.

NOTE 1:	The hosting CSE in the network domain may also delete the <mgmtObj> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <mgmtObj> resource may also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: For the DELETE procedure, the Receiver shall:

Check if the Originator has the DELETE permission on the addressed <mgmtObj> resource (or the <parameters> child resource in the case of child resource deletion).

Upon successful validation, remove the addressed resource from its repository.

If the Originator is an IN-AE, identify the corresponding external management object on the managed entity according to the mapping relationship it maintains. Check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to remove the corresponding external management object from the managed entity based on existing management protocol.

Respond to the Originator with the appropriate generic responses.

Table 10.2.5.5-1: <mgmtObj> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | None |
| Information on Request message | op: D fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource. |
| Local processing on Hosting CSE | Trigger existing (OMA-DM, BBF TR-069 or LWM2M) management procedures to delete the corresponding external management object from the managed entity. |
| Information on Response message | Proper error code if the external management object can not be deleted. |
| Post-Conditions | None |
| Exceptions | Corresponding external management object cannot be deleted from managed entity due to some reason (e.g. not reachable, external management object not found). |

#### 10.2.5.6	Execute <mgmtObj>

This procedure is used for executing a specific management command on a managed entity through an existing <mgmtObj> resource on the hosting CSE.

Originator: The Originator shall request to execute a specific management command which is represented by an existing <mgmtObj> resource or its attribute/child resource by using an UPDATE verb. The request shall address the specific executable <mgmtObj> resource or its attribute/child resource and shall contain an empty body. In the case that the management command is represented by an attribute of a <mgmtObj> resource, such attribute shall contain a URI that the request may also address to trigger the command execution alternatively.

After the execution request, the originator may request to retrieve the execution result or status from the executable <mgmtObj> resource or its attribute/child resource by using a RETRIEVE method as described in section 10.3.4.3.

The Originator shall be:

- IN-AE.

Receiver: For the EXECUTE procedure, the Receiver shall:

Check if the Originator has the WRITE permission on the addressed <mgmtObj> resource or its attribute/child resource.

Upon successful validation, check if there is existing management session between the management server and the managed entity. If not, request the management server to establish management session towards the managed entity. Send the management request to execute the corresponding management command (e.g. "Exec" in OMA DM) on the managed entity based on existing device management protocol.

Respond to the Originator with the appropriate generic responses. The Response may also contain execution results.

Upon receiving a RETRIEVE request to retrieve the execution result or status from the executable <mgmtObj> resource or its attribute/child resource, perform the procedures as described in section 10.2.5.3.

Upon receiving from managed entity a management notification (e.g. OMA-DM "Generic Alert" message or BBF TR-069 "Inform" message) regarding the execution result or status, the receiver may actively send the management request to retrieve the execution result or status external management object information from the managed entity and update the corresponding <mgmtObj> resource or its attribute/child resource locally.

Editor’s note: The definition of Execute is to be for FFS in 10.1.

Table 10.2.5.6-1: <mgmtObj> EXECUTE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of the AE or the CSE that initiates the Request to: The URI of the <mgmtObj> resource. cn: None |
| Local processing on Hosting CSE | Trigger existing (OMA DM, BBF TR-069, LWM2M) device management procedures to execute the corresponding management command on the managed entity. |
| Information on Response message | Proper error code if the external management object can not be executed. |
| Post-Conditions | None |
| Exceptions | Corresponding external management object operation cannot be executed in managed entity due to some reason (e.g. not reachable, external management object not found) |

### 10.2.6	External Management Operations through <mgmtCmd>

#### 10.2.6.1	Introduction

This clause describes how RESTful management operations may be performed using <mgmtCmd> resources over the Mca and Mcc reference points.  The <mgmtCmd> resource, together with its attributes or sub-resources, may be used in the process of translating between RESTful operations and management commands and procedures from existing management technologies (e.g. BBF TR-069). These procedures can then be performed on the remote entity, using the Management Adapter and the procedures described in the following clauses.

#### 10.2.6.2	Create <mgmtCmd>

A CREATE request is used by an Originator to create a specific <mgmtCmd> resource in a hosting CSE.

The created <mgmtCmd> resource will be mapping a RESTful method to management commands and/or procedures which may be translated from existing management protocols (e.g. BBF TR-069). At run-time the hosting CSE can expose the translated commands, over the Mcc reference point, to the remote entities (i.e. ASN/MN-CSE).

Originator: The Originator shall request to create a new <mgmtCmd > resource to be named as "mgmtCmd" by using a CREATE verb. The request shall address baseURI resource of the hosting CSE. The request may also provide the attributes of the <mgmtCmd> resource to be created as described in section 9.6.12 such as "cmdType".

The Originator may be:

- An AE registered to the IN-CSE.

- The local CSE on the managed entity: In this case, the local CSE transforms supported management command into the <mgmtCmd> resource representation, then requests the hosting CSE to create the corresponding <mgmtCmd> resource.

NOTE 1:	The hosting IN- CSE in the network domain may also create the <mgmtCmd> resource locally by itself. The details are out of scope. Then an AE can discover the created <mgmtCmd> and manipulate it.

NOTE 2:	The <mgmtCmd> resource may also be created in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: The Receiver shall check if the Originator has the CREATE permission on the addressed baseURI resource (or the parent <mgmtCmd> resource in the case of child resource creation). The hosting CSE shall also check the validity of provided attributes. Upon successful validation, a new resource with name "mgmtCmd" shall be created in the hosting CSE with the provided attributes. The hosting CSE shall maintain the mapping between the created <mgmtCmd> resource and the corresponding nonRESTful commands represented by the "cmdType" attribute of <mgmtCmd> resource. The hosting CSE shall respond to the Originator with the appropriate generic response code. It shall also provide in the Response the URL of the created new resource.

The hosting CSE shall be:

- An  IN-CSE.

Table 10.2.6.2-1: <mgmtCmd> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | The CSE on the originating node shall first collect local management command. |
| Information on Request message | op: C fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in 9.6.12, as needed |
| Local processing on Hosting CSE | No change from the basic procedures |
| Information on Response message | fr: Receiver CSE-ID to: Originator APP-Inst-ID or CSE-ID cn: URI of created <mgmtCmd> resource. |
| Post-Conditions | No change from the basic procedures. |
| Exceptions | If the creation is not allowed, the <mgmtCmd> resource shall not be created, and a proper error code shall be returned to the originator in the Response message |

#### 10.2.6.3	Retrieve <mgmtCmd>

A RETRIEVE Request is used by an Originator to retrieve all or part information from an existing <mgmtCmd> resource on a hosting CSE. Alternatively, the Originator can request to retrieve only a specific attribute or part of an attribute.

Originator: The Originator shall request to retrieve all or part of the information from an existing <mgmtCmd> resource by using a RETRIEVE verb. The request shall address a specific "<mgmtCmd>" resource to retrieve all attributes and the references to the sub-resources of the <mgmtCmd> resource.  Alternatively, the request shall address the individual attributes of the specific "<mgmtCmd>" resource to retrieve the corresponding attribute value.

Editor's Note: We need to agree on a terminology on how to refer to a specific instance of a resource type.

The Originator may be:

- An AE.

- A CSE.

Receiver: The Receiver shall check if the Originator has the READ permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall retrieve the corresponding attributes the <mgmtCmd> resource and the references to the sub-resources from its repository and shall respond to the Originator with the appropriate generic responses.

The hosting CSE shall be:

- An  IN-CSE.

Table 10.2.6.3-1: <mgmtCmd> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to create a resource. |
| Information on Request message | op: R fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in 9.6.12, as needed |
| Local processing on Hosting CSE | No change from the basic procedures, |
| Information on Response message | No change from the basic procedures. |
| Post-Conditions | No change from the basic procedures. |
| Exceptions | If the retrieval is not allowed, or the specific <mgmtCmd> resource does not exist in the IN-CSE, a proper error code shall be returned to the originator |

#### 10.2.6.4	Update <mgmtCmd>

An UPDATE request is used by an Originator to update all or part information of an existing <mgmtCmd> resource on a hosting CSE with new attributes. Alternatively, the Originator can request to update only a specific attribute or part of an attribute.

Originator: The Originator shall request to update all or part information of an existing <mgmtCmd> resource by using a UPDATE verb. The request shall address a specific "<mgmtCmd>" resource of a CSE to update all attributes of the <mgmtCmd> resource with the provided new value.  Alternatively, the request shall address the individual attribute of the specific "<mgmtCmd>" resource to update the corresponding attribute with the provided new value.

The Originator may be:

- An AE.

- A CSE.

Receiver: The Receiver shall check if the Originator has the WRITE permission on the addressed <mgmtCmd> resource. The hosting CSE shall also check the validity of provided attributes if any. Upon successful validation, the hosting CSE shall overwrite the corresponding attributes of the <mgmtCmd> resource with the provided new data. The CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An IN-CSE.

Table 10.2.6.4-1: <mgmtCmd> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to retrieve a resource |
| Information on Request message | op: U fr: Originator APP-Inst-ID or CSE-ID to: Receiver CSE-ID. cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in 9.6.12, as needed |
| Local processing on Hosting CSE | No change from the basic procedures, |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | No change from the basic procedures, |
| Exceptions | If the update is not allowed or the specific <mgmtCmd> resource (or its sub-resource) does not exist in the IN-CSE, a proper error code shall be returned to the Originator |

#### 10.2.6.5	Delete <mgmtCmd>

A DELETE request is used by the Originator to delete an existing <mgmtCmd> resource on a hosting CSE. An AE may also use this procedure to cancel all initiated <execInstance> of an <mgmtCmd> if applicable.

Originator: The Originator shall request to delete an existing <mgmtCmd> resource by using a DELETE verb. The request shall address the specific "<mgmtCmd>" resource on the hosting CSE.

The Originator may be:

- The CSE on the manageable entity: In this case, the local -CSE issues the request to the hosting CSE to hide the corresponding management command from being exposed by the <mgmtCmd> resource.

- An AE: In this case, the AE requests the hosting CSE to delete the <mgmtCmd> resource from the hosting CSE and cancel all initiated <execInstance> of an <mgmtCmd> if applicable.

NOTE 1:	The hosting CSE in the network domain may also delete an <mgmtCmd> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <mgmtCmd> resource may also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: The Receiver shall check if the Originator has the DELETE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall remove the resource from its repository. If the Originator is an AE and there is any initiated <execInstance> under the <mgmtCmd> that can be cancelled by a corresponding management command. The hosting CSE shall also issue the management command to the remote entity to cancel those initiated <execInstance> based on existing management protocol (i.e. BBF TR-069). Then the CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An IN-CSE.

Table 10.2.6.5-1: <mgmtCmd> DELETE by ASN-CSE or MN-CSE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Before issuing a DELETE request to the IN-CSE, the originating CSE may perform cancelling of the corresponding management command locally. |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in 9.6.12, as needed |
| Local processing on Hosting CSE | The <mgmtCmd> resource shall be deleted from the repository of the IN-CSE. |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | No change from the basic procedures, |
| Exceptions | If the deletion is not allowed or the specific <mgmtCmd> resource does not exist in the IN-CSE, the deletion shall be failed and a proper error code shall be returned to the Originator |

Table 10.2.6.5-2: <mgmtCmd> DELETE by an AE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to delete the resource. |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in 9.6.12, as needed |
| Local processing on Hosting CSE | If there is any initiated <execInstance> under <mgmtCmd> and it is cancellable, IN-CSE shall cancel those initiated <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e. CancelTransfer RPC in BBF TR-069). The <mgmtCmd> resource shall be deleted from the repository of the IN-CSE. |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | No change from the basic procedures, |
| Exceptions | If the deletion is not allowed or the specific <mgmtCmd> resource does not exist, there is no local processing in the IN-CSE and a proper error code shall be returned to AE If the corresponding initiated commands cannot be deleted from remote entity due to some reason (e.g. not found) al response with the proper indication shall be returned to AE |

#### 10.2.6.6	Execute <mgmtCmd>

The Execute procedure is used by an originator for execution of a specific management command on a remote entity, through an existing <mgmtCmd> resource on the hosting CSE.

Originator: The Originator shall request to execute a specific management command which is represented by an existing <mgmtCmd> resource by using an UPDATE verb. The UPDATE request shall address the execEnable attribute of the <mgmtCmd> resource without any payload. Alternatively, the UPDATE request shall address the URI provided as the value of the execEnable attribute of the <mgmtCmd> resource.

After issuing the execution request, the Originator may request to retrieve the execution result or status from an <execInstance> sub-resource of the <mgmtCmd>by using a RETRIEVE method as described in section 10.2.6.2.

The Originator shall be:

- An AE.

Receiver: The Receiver shall check if the Originator has the WRITE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall perform command conversion and mapping, and send the converted management command over mId reference point to execute the corresponding management command with the provided arguments on the remote entity based on existing device management protocol (i.e. BBF TR-069). Then the hosting CSE shall create a corresponding <execInstance> resource under <mgmtCmd> for this command execution. And the hosting CSE shall respond to the originator with the appropriate generic responses. It shall also provide in the response the URL of the created <execInstance> resource.

Upon receiving from the remote entity a management notification (i.e. BBF TR-069 "Inform" message) regarding the execution result or status, the hosting CSE may update the corresponding <execInstance> sub-resource locally.

The hosting CSE shall be:

- An  IN-CSE.

Editor’s note: The definition of Execute is to be for FFS in 10.1.

Table 10.2.6.6-1: <mgmtCmd> EXECUTE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to retrieve a retrieve a resource. |
| Information on Request message | op: U fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: <mgmtCmd> / execEnbale The UPDATE request shall address to <mgmtCmd>/execEnable without any payload. The mandatory and/or optional attributes defined in 9.6.12, as needed. |
| Local processing on Hosting CSE | (a). If the execution is allowed, Hosting CSE will do command conversion and mapping. (b) The Hosting CSE shall trigger existing device management procedures (i.e. BBF TR-069) to execute the corresponding management command on the remote entity. (c) The Hosting CSE shall perform local processing: if Step b is successful, the IN-CSE shall create an <execInstance> resource under the triggered <mgmtCmd> to maintain status and results for this execution. |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | The following processing on the Hosting CSE is dependent on the type of the command and execution status and may occur after the Response message has been sent. (a) After the command execution is finished, the remote entity sends response including execution results to the Hosting CSE, who will store the execution results in corresponding <execInstance> resource. This step may occur any time after the Response message has been sent and might occur after step b below. (b) The AE may use normal RETRIEVE procedure to retrieve the execution results or status of an <execInstance>. (c) After receiving the RETRIEVE request from the AE, the Hosting CSE can retrieve the execution status or results on the remote entity using existing management protocol. If step d. occurs before step e, step f. may not be needed. (d) A response shall be returned to the AE. |
| Exceptions | If the execution is not allowed or the specified <mgmtCmd> resource or its attribute/sub-resource does not exist in the IN-CSE, no further processing is required in the Hosting CSE and a proper error code shall be returned to AE in the Message response If the corresponding management command cannot be executed in remote entity due to some reason, step 004 <??> shall be skipped and a proper error code shall be returned to AE in step 005 <??>. |

#### 10.2.6.7	Cancel <execInstance>

The Cancel procedure is used by an originating AE to disable/stop/cancel an initiated management command execution on the remote entity, through an existing <execInstance> resource on the hosting CSE.

Originator: The Originator shall request to disable/stop/cancel an initiated management command execution which is represented by an existing <execInstance> resource, by using an UPDATE verb. The UPDATE request shall address the execDisable attribute without any payload.

The Originator shall be:

- An AE.

Receiver: The Receiver shall check if the Originator has the WRITE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall perform command conversion and mapping, then use existing management protocol (i.e. BBF TR-069) to cancel the corresponding management command execution initiated on the remote entity, and the hosting CSE shall respond to the originator with the appropriate responses.

The hosting CSE shall be:

- An IN-CSE.

Table 10.2.6.7-1: <execInstance> CANCEL

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | Originator needs to disable/stop/cancel an initiated management command execution on the remote entity |
| Information on Request message | op: U fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: <mgmtCmd> / execInstance. The mandatory and/or optional attributes defined in 9.6.12, as needed. |
| Local processing on Hosting CSE | If the cancellation is allowed, IN-CSE will do command conversion and mapping. The IN-CSE shall cancel the <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e. CancelTransfer RPC in BBF TR-069). The IN-CSE shall delete the corresponding <execInstance> resource. |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | No change from the basic procedures, |
| Exceptions | If the execution is not allowed or the specified <execInstance> resource does not exist in the IN-CSE or the <execInstance> is finished, the local processing on Hosting CSE shall be skipped and a proper error code shall be returned to AE in the Response message. If the corresponding management command cannot be executed in remote entity due to some reason, the IN-CSE shall still delete the corresponding <execInstance> resource and return a Response message. |

Editor’s Note: The Cancel flow shall be updated based on the definition of EXEC procedure.

#### 10.2.6.8	Retrieve <execInstance>

A RETRIEVE request is used by an Originator to retrieve an existing <execInstance> resource, including individual attributes, from a hosting CSE. Alternatively, the Originator can request to retrieve only a specific attribute or part of an attribute.

Originator: The Originator shall request to retrieve all or part of information from an existing <execInstance> resource by using a RETRIEVE verb. The Request shall address a specific "<execInstance>" resource of a hosting CSE to retrieve all attributes of the <execInstance> resource. Or the request shall address the individual attribute of the specific "<execInstance>" resource to retrieve the corresponding attribute value.

The Originator shall be:

- An AE.

Receiver: The Receiver shall check if the Originator has the READ permission on the addressed <mgmtCmd> resource. Upon successful validation, the CSE shall retrieve the corresponding attributes of the <execInstance> resource and the references to the sub-resources from its repository and shall respond to the originator with the appropriate generic responses.

The hosting CSE may be:

- An IN-CSE.

Table 10.2.6.8-1: <execInstance> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to create a resource. |
| Information on Request message | op: R fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: <mgmtCmd> resoruce. The mandatory and/or optional attributes defined in 9.6.12, as needed. |
| Local processing on Hosting CSE | If the retrieval is allowed, the IN-CSE can retrieve the execution status or results on the remote entity using existing management protocol (i.e. BBF TR-069). If the retrieval is allowed, the addressed attributes of the <execInstance> resource shall be retrieved from the repository of the IN-CSE. |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | No change from the basic procedures, |
| Exceptions | If the retrieval is not allowed or the specific <execInstance> resource does not exist in the IN-CSE, there is no local processing on the hosting CSE and a proper error code shall be returned to AE in the Response Message. |

#### 10.2.6.9	Delete <execInstance>

The DELETE request is used by an originating AE  to delete an existing <execInstance> resource on a hosting CSE.

Originator: The Originator shall request to delete an existing <execInstance> resource by using a DELETE verb. The request shall address the specific "<execInstance>" resource of an <mgmtCmd> on the hosting CSE.

The originator shall be:

- An AE.

NOTE 1:	The hosting CSE in the network domain may also delete an <execInstance> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <execInstance> resource may also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: The Receiver shall check if the Originator has the DELETE permission on the addressed <execInstance> resource. Upon successful validation, the hosting CSE shall remove the resource from its repository. If the <execInstance> is not finished on the remote entity and it is cancellable, the hosting CSE shall also use existing management protocols (i.e. BBF TR-069 CancelTransfer RPC) to cancel the corresponding management currently initiated at the remote entity. Then the CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An IN-CSE.

Table 10.2.6.9-1: <execInstance> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | Originator needs to delete the resource. |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID. to: Receiver CSE-ID. cn: Name of <mgmtCmd> resource. The mandatory and/or optional attributes defined in 9.6.12, as needed |
| Local processing on Hosting CSE | If the <execInstance> is not finished yet and it is cancellable, the IN-CSE shall cancel the <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e. CancelTransfer RPC in BBF TR-069 [xx]). The <execInstance> resource shall be deleted from the repository of the IN-CSE. If the corresponding initiated commands cannot be deleted from remote entity due to some reason (e.g. not found), the <execInstance> resource shall be still deleted be performed |
| Information on Response message | No change from the basic procedures, |
| Post-Conditions | No change from the basic procedures, |
| Exceptions | If the deletion is not allowed or the specific <execInstance> resource does not exist in the IN-CSE, there is no processing on the Hosting CSE and a proper error code shall be returned to AE in the Response message If the <execInstance> is already complete or it is not cancellable, there is no processing on the Hosting CSE |

### 10.2.7	Supported Attribute Discovery Procedure

#### 10.2.7.1	Introduction

The supported attribute discovery procedure allows discovering of supported attributes of a certain resource type in a CSE. This procedure shall be applied when the Originator is either an AE or a CSE and the Receiver is a CSE. This specification defines multiple optional attributes for each resource type, which means that Originator can suffer multiple times of try and error for accessing (e.g., creating or updating) attributes if the Originator doesn’t know which attributes of each resource type are supported by the Receiver. This supported attribute discovery procedure helps reduce unnecessary try and error by discovering which attributes of the resource type are supported in the target Receiver.

In order to reduce communicational overhead between Originator and Receiver, this procedure discovers supported attributes from only optional attributes of a certain resource type. After this procedure, the Originator should send requests to the Receiver only including the supported optional attributes confirmed by the Receiver with this procedure.

Supported attributes discovery shall be done using the RETRIEVE request with the parameter indicating supported attributes discovery by the Originator to distinguish from discovery of resource and retrieval of resource. This discovery supports retrieving list of all the attributes and child resource types of a certain resource type, or informing whether the attribute is supported or not.

#### 10.2.7.2	Supported Attributes Discovery procedure via Retrieve Operation

This procedure shall be used for discovering supported optional attributes of a CSE. The discovery result shall be returned to the Originator using a successful message.

Originator:  The Originator shall be either an AE or a CSE. The Originator shall provide the baseURI of the Receiver and the parameter indicating supported attributes discovery in the request. Resource type name shall be included in the request and attribute name may be included in the request.

Receiver or Hosting CSE:  The Receiver shall be a CSE. If the Originator sends Retrieve operation with supported attributes discovery indicator, the Receiver shall perform the following procedures based on parameters of Originator’s Request.

Receiver CSE will be able to maintain information about supported resource types to be exposed to other CSEs.

Editor’s Note: How to maintain/expose supported resource types of CSEs is FFS.

- If the request addresses baseURI with a resource type, the Receiver shall respond to the Originator with the list of all the supported optional attributes and supported optional child resource types the Receiver supports for that resource type if the Receiver supports the resource type.

- If the Receiver doesn’t support the resource type the Originator indicates in the Request, the Receiver informs the Originator the resource type is not supported.

- If the Request addresses baseURI with resource type and attribute, the Receiver shall respond to the Originator whether the attribute of the resource type is supported or not by the Receiver if the Receiver supports the resource type.

- If the Request addresses baseURI with attribute only, the Receiver shall respond to the Originator with format error.

Table 10.2.7.2-1: Supported Attribute Discovery Procedure via Retrieve Operation

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | Originator needs to discover supported resource types and attributes in the Hosting CSE. |
| Information on Request message | to: baseURI of Receiver cn: - Supported attributes discovery indicator to determine this request is supported attribute discovery. - Resource type and/or attribute the Originator needs to discover. |
| Local processing on Hosting CSE | Procedures described for the Receiver of the Retrieve Request as described above. |
| Information on Response message | Contains the list of name of attributes. Contains indicator whether attribute is supported or not. |
| Post-Conditions | None |
| Exceptions | Local processing on Hosting CSE: The Request contains invalid parameters (e.g., only attribute parameter is specified.). |

### 10.2.8	Location Management Procedures

#### 10.2.8.1	Introduction

This section introduces the procedures for obtaining and managing a target M2M Node’s location information, which are associated with the <locationPolicy> resource that contains the method for obtaining and managing location information.

Since the actual location information of a target M2M Node shall be stored in the <instance> resource, a child of the <container> resource, as per the configuration described in the <locationPolicy> resource; the related procedures regarding both, the <container> resource and the <instance> resource after the CRUD operations of <locationPolicy> are described in this section.

Editor's Node: Detailed procedures regarding both <container> and <instance> resources when the <container> resource contains the location information of a target M2M Node are described separately in <container> and <instance> resource and are FFS.

#### 10.2.8.2	Create <locationPolicy>

This procedure shall be used for creating a <locationPolicy> resource.

Originator: The Originator shall request to CREATE a <locationPolicy> resource including the relevant attributes and the address <baseURI> resource of a Hosting CSE. Minimally, the Request shall provide the mandatory attributes defined in the Table 9.6.7-1. The Originator in this case is an AE.

Editor’s Note: Handling of various levels of the <locationPolicy> resources needs to be defined.

Receiver: For the CREATE procedure, the Receiver (Hosting CSE) shall:

- Check whether the Originator is authorized to request the procedure.

- Check whether the provided attributes of the <locationPolicy> resource represent a valid Request.

- Upon successful validation of the above procedures, the Hosting CSE creates the <locationPolicy> resource and automatically creates <container> resource where the actual location information is/are stored and the resources shall contain cross-reference between the both resources: locationContainerID attribute for <locationPolicy> resource and locationID attribute for <container> resource.

- Check the defined locationUpdatePeriod attribute. If a valid period value is set for this attribute, the Hosting CSE performs the positioning procedures as per the configurations in the <locationPolicy> resource and stores the results (e.g., position fix, uncertainty) in the created <container> resource. However, if no value (e.g., null) is set, the positioning procedure is performed when the created <container> for location information is retrieved.

- Check the defined locationSource attribute, the positioning procedures mentioned above are different. The locationSource attribute is set based on the capabilities of a target M2M Node and the Underlying Network in which the Node resides.

- For Network-based case, the Hosting CSE will transform the Request from the Originator into Location Server request following the attributes (e.g., locationTargetID, locationServer) defined in the <locationPolicy> resource. Additionally, the hosting CSE shall also provide default values for other parameters (e.g. required quality of position) in the Location Server request [i.1] according to local policies. Then the Location Server in the Underlying Network performs positioning procedures, and returns the results.

The specific mechanism used to communicate with the network Location Server depends on the capabilities of the Underlying Network and other factors. For example, it could be either the OMA Mobile Location Protocol [i.2] or OMA RESTful NetAPI for Terminal Location [i.3].

NOTE: The details of the mechanisms will be addressed in the Protocol Specification.

- For Device-based case, this case is applicable if the Originator is ASN-AE and the ASN has location determination capabilities (e.g., GPS). The Hosting CSE is capable of performing positioning procedure using the module or technologies.

- For Sharing-based case, this case is applicable if the Originator is ADN-AE and the Hosting CSE is MN-CSE and the ADN is resource constrained node, no GPS module and Network-based positioning capabilities. When the Hosting CSE receives the CREATE request and if the Hosting CSE can find the closest Node from the Originator in the M2M area network (e.g., based on the topology information defined in the <node> resource), the location information of the closest Node shall be stored as the location information of the Originator, or if the Hosting CSE cannot find any closest Node, the location information of the Node of the Hosting CSE (MN) shall be stored as the location information of the Originator.

Table 10.2.8.2-1: <locationPolicy> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | None |
| Information on Request message | op: C fr: Identifier of an AE that initiates the Request to: the URI of the <baseURI> resource cn: The representation of the <locationPolicy> resource described in section 9.6.7 |
| Local processing on Hosting CSE | Detail procedure for the Hosting CSE of the CREATE request described above. |
| Information on Response message | The representation of the created <locationPolicy> resource. |
| Post-Conditions | Detail steps for the Hosting CSE after the CREATE request are described above. |
| Exceptions | No change from the generic procedure. |

#### 10.2.8.3	Retrieve <locationPolicy>

This procedure shall be used for retrieving an existing <locationPolicy> resource.

Originator: The Originator shall request to obtain <locationPolicy> resource information by using RETRIEVE operation. The Originator is either an AE or a CSE.

Receiver: The Receiver shall check if the Originator has RETRIEVE permission on the <locationPolicy> resource. Upon successful validation, the Hosting CSE shall respond to the Originator with the appropriate responses.

Table 10.2.8.3-1: <locationPolicy> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | None |
| Information on Request message | op: R fr: Identifier of an AE that initiates the Request to: the URI of the target <locationPolicy> resource |
| Local processing on Hosting CSE | No change from the generic procedure. |
| Information on Response message | No change from the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.8.4	Update <locationPolicy>

This procedure shall be used for updating an existing <locationPolicy> resource.

Originator: The Originator shall request to update attributes of an existing <locationPolicy> resource by using an UPDATE operation. The request shall address the specific <locationPolicy> resource of a CSE. The Originator may be either an AE or a CSE.

Receiver: The Receiver of an UPDATE request will check whether the Originator is authorized to request the operation. The receiver will further check whether the provided attributes of the <locationPolicy> resource represent a valid request for updating <locationPolicy> information. The updatable attributes are (excluding common attributes):

- locationUpdatePeriod: This value is updated to change the period for updating location information.

Table 10.2.8.4-1: <locationPolicy> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | None |
| Information on Request message | op: U fr: Identifier of an AE or a CSE that initiates the Request to: the URI of the target <locationPolicy resource |
| Local processing on Hosting CSE | No change from the generic procedure. |
| Information on Response message | No change from the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.8.5	Delete <locationPolicy>

This procedure shall be used for deleting an existing <locationPolicy> resource.

Originator: The Originator shall request to delete an existing <locationPolicy> resource by using the DELETE operation. The Originator may be either an AE or a CSE.

Editor’s Note: If deleted by CSE is the AE that created it notified.

Receiver: The Receiver shall check if the Originator has DELETE permission on the <locationPolicy> resource. Upon successful validation, the CSE shall remove the resource from its repository and shall respond to the Originator with appropriate responses.

Once the <locationPolicy> resource is deleted, the Receiver shall delete the associated resources (e.g., <container>, <instance> resources). If the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource has been set with appropriate value, the Receiver shall tear down the session. The specific mechanism used to tear down the session depends on the support of the Underlying Network and other factors.

Table 10.2.8.5-1:  <locationPolicy> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | When the locationSource of the created <locationPolicy> resource is "sharing-based" and the AE disconnects from the registered MN-CSE. |
| Information on Request message | op: D fr: Identifier of an AE or a CSE that initiates the Request to: the URI of the target <locationPolicy resource |
| Local processing on Hosting CSE | No change from the generic procedure. |
| Information on Response message | No change from the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

### 10.2.9	<subscription> Resource Procedures

#### 10.2.9.1		Introduction

An Originator can create a <subscription> resource on a subscribed-to resource Hosting CSE to be notified when the resource is modified. After successful <subscription> resource creation, the Hosting CSE shall notify the Originator of a subscribed-to resource modification that meets conditions configured in the subscription resource.

A subscription shall be represented by a <subscription> resource (see clause 9.6.9). This allows manipulation of the subscription in a resource oriented manner, e.g. the conditions of a subscription may be modified by modifying a <subscription> resource, or a resource subscriber may unsubscribe by deleting the <subscription> resource.

The following clauses describe procedures for Creation, Retrieval, Update and Deletion of a <subscription> resource.

#### 10.2.9.2		Create <subscription>

This procedure shall be used to request a new subscription to be notified for the modifications of a subscribable resource.

Originator: The Originator can be an AE or a CSE. The Originator shall request to create a <subscription> resource. The Request shall address a subscribable resource.

The Request shall include notificationURI(s).

Hosting CSE: The Hosting CSE shall validate the followings:

- if the subscribed-to resource, addressed in the to parameter in the Request, is a subscribable resource

- if the Originator has RETRIEVE permission to the subscribed-to resource

- if the Originator has access rights to send notifications to the entity(ies) hosting notificationURI(s) when the Originator is not a hosting notificationURI

- if it has access rights to send notifications to the entity(ies) that hosting notificationURI(s).

Editor's Note: Access right to send notification to an entity can be represented as NOTIFY permission.

Editor’s Note: The validation process described in the last two bullet points above needs to be investigated further, This is FFS.

If all those checks are successfully done, the Hosting CSE shall send positive Response to the Originator.

Table 10.2.9.2-1: <subscription> CREATE

| Description |  |
| --- | --- |
| Call Flow Type | CREATE |
| Pre-Conditions | The Originator configures proper permissions and access rights described above. |
| Information on Request message | op: C fr: ?? to: ?? cn: Mandatory and/or optional attributes defined in section 9.6.9. |
| Local processing on Hosting CSE | Check access rights regarding creation of the subscription resource and sending notification to the notificationURI. |
| Information on Response message | No change from the basic procedure in section 10.1.1 |
| Post-Conditions | None |
| Exceptions | One of validations on the Hosting CSE above fails. |

#### 10.2.9.3		Retrieve <subscription>

This procedure shall be used to retrieve information of a subscription such as notificationURI, filterCriteria, expirationTime etc.

Originator: The Originator can be an AE or a CSE. The Request shall address a <subscription> resource or one of its attributes.

Hosting CSE: If the Hosting SCE validates the Originator has RETRIEVE permission referring accessRightID , it shall return the complete representation of the <subscription> resource or the value of a specific attribute of the <subscription> resource..

Table 10.2.9.3-1: <subscription> RETRIEVE

| Description |  |
| --- | --- |
| Call Flow Type | RETRIEVE |
| Pre-Conditions | The Originator has RETRIEVE permission to the subscription resource. |
| Information on Request message | fr: ?? to: ?? No change from the basic procedure. |
| Local processing on Hosting CSE | fr: ?? to: ?? No change from the basic procedure. |
| Information on Response message | No change from the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.9.4		Update <subscription>

This procedure shall be used to update an existing subscription, e.g. extension of its lifetime or the modification of the notificationURI.

Originator: The Originator can be an AE or a CSE. The Originator may send new value(s) for read-write attribute(s) of a <subscription> resource.

Hosting CSE: After the Hosting CSE verifying the existence of the addressed resource, the validity of the attributes provided and the permissions to modify them referring accessRightID, shall update the attributes provided and shall send positive response to the Originator.

Table 10.2.9.4-1: <subscription> UPDATE

| Description |  |
| --- | --- |
| Call Flow Type | UPDATE |
| Pre-Conditions | The Originator has UPDATE permission to the subscription resource. |
| Information on Request message | No change from the basic procedure. |
| Local processing on Hosting CSE | No change from the basic procedure. |
| Information on Response message | No change from the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

#### 10.2.9.5		Delete <subscription>

This procedure shall be used to unsubscribe an existing subscription.

Originator: The Originator can be an AE or a CSE. The request shall address a subscription resource.

Hosting CSE: If the Hosting CSE validates the Originator has the DELETE permission to delete a subscription resource, it shall delete the resource and shall send a positive response to the Originator.

Table 10.2.9.5-1: <subscription> DELETE

| Description |  |
| --- | --- |
| Call Flow Type | DELETE |
| Pre-Conditions | The Originator has DELETE permission to the subscription resource. |
| Information on Request message | No change from the basic procedure. |
| Local processing on Hosting CSE | No change from the basic procedure. |
| Information on Response message | No change from the generic procedure. |
| Post-Conditions | None |
| Exceptions | No change from the generic procedure. |

### 10.2.10	Notification Procedures for Resource Subscription

This procedure shall be used to notify Receiver(s) of modifications of a resource for an associated <subscription> resource.

Originator: The Originator can be a CSE which is hosting a <subscription> resource. The Originator shall consider sending a notification request containing the subscribed information following the detection of a modification of the subscribed-to resource that matches the specified filterCriteria as specified in the <subscription> resource.

The Originator shall contain a notification structure and send this to Receiver(s).

Receiver: The Receiver can be AE(s) or a CSE(s) which shall be presented in the notificationURI attribute of the <subscription> resource. The Receiver responds positively to the Request.

A considered Notification Request shall be sent when notification policies (i.e., expirationCounter, batchNotify, rateLimit, interimEventsNotify and  priorSubscriptionNotify) are satisfied.

The expirationCounter shall be decreased by one when the Originator successfully sends the notification request to Receiver(s). If the counter meets zero, the corresponding subscription resource is deleted.

The batchNotify policy is based on a minimum number of notification events and a maximum time to wait for that number of events. Notification events shall be temporarily stored for some duration (e.g. 10 minutes) or until a specified number are stored (e.g. 20 notifications) before sending – then sent when the first of these two conditions are satisfied. Stored notification events may be dropped according to the notificationStoragePriority and the notificationCongestionPolicy (see section 9.6.9).

The rateLimit policy is based upon a maximum specified number of events (e.g. 10, 000) that can be sent within some specified rateLimitWindow duration (e.g. 60 seconds). Note that ratelimitWindows are sequential not rolling. A considered Notification Request may only be sent whenever the current total number of events sent is less than the maximum number of events within the current rateLimitWindow duration. Notification events that do not meet this policy are temporarily stored or dropped according to the notificationStoragePriority and the notificationCongestionPolicy (see section 9.6.9).

The interimEventsNotify policy indicates the notification action to be taken following a period of connectionless (e.g., reachability schedule, network malfunction), when Receiver(s) again become reachable. The interimEventsNotify attribute specifies how many of the most recent eligible notifications during the connectionless (interim) period shall be sent to Receiver(s) relative to Receiver(s) reconnect time. When a notification becomes eligible in the connectionless period, it is temporarily stored in the Originator. After the connection recovery, the Originator shall determine which notifications to send according to the interimEventsNotify attribute and send notifications (if available) to Receiver(s). The interimEventsNotify attribute shall be classified into "sendNo" (i.e., ignore prior notifications), "sendLatest" (i.e., send the last pending notification), "sendAll" (i.e., send all pending notifications).

Figure 10.2.10-1:  Notification Mechanism when interimEventsNotify (sendNo) is used

Figure 10.2.10-2:  Notification Mechanism when interimEventsNotify (sendLatest) is used

The priorSubscriptionNotify policy indicates the notification action to be taken following a new subscription. It enables sending of event notifications, which happened before the new subscription was created. When a new subscription is created, the most recent number “n” subscription events prior to the new subscription shall be sent to Receiver(s), if available.

Table 10.2.10-1:  Notification Procedure

| Description |  |
| --- | --- |
| Call Flow Type | NOTIFY |
| Pre-Conditions | Notification is triggered regarding subscription information in a <subscription> resource. |
| Information on Request message | fr: ID of the Originator to: notificaitonURI specified in <subscription> resource cn: - notification data that represents the modified content of subscribed-to resource may be included. - subscription reference (i.e., URI of the corresponding <subscription> resource) that generates this notification shall be included. |
| Information on Response message | No change from the basic procedure |
| Post-Conditions | None |
| Exceptions | None |

# 11	<tbd>

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

# Annex <A>: (Informative): Deployment Scenarios

<Text>

# ANNEX B: (Informative) oneM2M System and 3GPP MTC Release-11 Underlying Network Interworking

## B.1	3GPP MTC Release-11 Underlying Network Introduction

In order to provide M2M services, interworking between oneM2M System and the 3GPP Underlying Network is required. This entails the following aspects:

- Connectivity establishment between the oneM2M System and the 3GPP Underlying Network from IN-CSE initiated scheme and device (i.e., ASN-CSE) initiated scheme.

- Mapping of oneM2M and 3GPP IDs to establish connectivity between specific entities

This section provides system level information on the above aspects specifically related to the interworking i.e., connectivity between the oneM2M System and the 3GPP Underlying MTC network.

## B.2	3GPP Release-11 MTC functionality

Interworking with oneM2M Release-1 is based on 3GPP Release-11 specifications. The relevant 3GPP Release-11 specification references are as follows:

- TS 23.682 : MTC System Architecture

- TS 23.401 and 23.402: LTE/SAE System Architecture

- TS 23.060: General Packet Radio Service (GPRS)

- TS 23.228: IP Multimedia Subsystem

Editor's Note:   Further release upgrade of 3GPP MTC functionality will be incorporated into this Annex, depending on the progress.

In Annex A of 3GPP TS 23.682 [i.11] the following MTC deployment scenarios are depicted:

Figure B.2-1: MTC deployment scenarios for Direct and Indirect model [Ref 3GPP TS 23.682]

The focus of this Annex is on deployment scenario B (Indirect Model), where the M2M Services Capability Server (IN-CSE) is outside the operator domain. However, the indirect model C in Figure B.2-1 is not ruled out.

Hybrid model which is a combination of above scenario B and C are also mentioned in TS 23.682 which may also be supported.

Taking 3GPP Release-11 MTC network as an Underlying Network, oneM2M IN-CSE is considered as equivalent or part of the Services Capability Server (SCS) function, and the oneM2M ASN is considered equivalent to a UE, as captured in 3GPP MTC architecture (Ref: TS 23.682) shown below:

Figure B.2-2: 3GPP Architecture for Machine-Type Communication [Ref 3GPP TS 23.682]

In the above diagram, the SCS is shown located in the HPLMN and within the operator domain. However, it is assumed that the SCS is equivalent to the oneM2M Common Services Entity (CSE) or the two are integrated, and is outside the operator domain. When the CSE is outside the operator domain, it has to interact with the 3GPP Underlying Network via MTC-IWF and/or GGSN/P-GW. This requires mapping of oneM2M reference points (Mcn, Mcc) and 3GPP reference points (Tsp, Gi/SGi) respectively, along with the usage of the identifiers in the two systems.

Editor’s Note: The mapping of identifiers between the oneM2M System and the 3GPP MTC network are FFS.

## B.3	ASN-CSE initiated connectivity establishment

It is assumed that there is no connectivity previously established, i.e., no association between the ASN-CSE (device node) and the serving IN-CSE exists. When the ASN-CSE has to send data to the serving IN-CSE it has to first discover the serving IN-CSE, which is located in a packet data network, and establish connection. Two methods can be used, as follows:

### B.3.1	Use of DHCP and DNS

The ASN-CSE requests the DNS server address from the DHCP server followed by requesting the serving IN-CSE IP address from the DNS server.

Editor’s Note: How a non-CSE resident device node (i.e., Application Dedicated Node) interworks with this system is FFS.

### B.3.2	Pre-configuration

The ASN-CSE is preconfigured with the fully qualified domain name (FQDN) of the serving IN-CSE or the IP address of the serving IN-CSE. If the FQDN is known, DNS resolution is used to obtain the IP address.

Editor’s Note: There might be other ASN-CSE initiated IN-CSE discovery schemes and this section does not intend to exclude them. Further description of these schemes is FFS.

## B.4	Serving IN-CSE initiated connectivity establishment

It is assumed that there is no connectivity previously established between the ASN-CSE and the serving IN-CSE. When the serving IN-CSE has to contact the ASN-CSE to send data or request data, connectivity between them has to be established. This connectivity has to be triggered by the IN-CSE.

Editor’s Note: How a non-CSE resident ADN interworks with this system is FFS.

## B.5	Idle mode Management

Editor’s Note: This section should describe oneM2M System dormant device management capability in conjunction with 3GPP related Idle Mode management.

## B.6	Procedure

ASN-CSE communicates with the serving IN-CSE after completion of the Underlying Network bearer establishment and discovery of the serving IN-CSE. Data can then traverse between CSEs over the IP connection in the Underling Network over 3GPP Gi/SGi interface. In addition, the signalling connectivity between the two CSEs is also realized. The following figure depicts the connectivity between the ASN-CSE and the IN-CSE.

Figure B.6-1: Initial Connectivity Establishment between Device and Infrastructure CSE

B.6.1	General

When data has to be exchanged between the ASN-CSE and the IN-CSE, connectivity between them has to be established. The need for this connectivity can arise for two reasons:

- ASN-CSE initiated: When the ASN-CSE has to send/receive data to/from the IN-CSE, or

- IN-CSE initiated: When the IN-CSE has to send/receive data to/from the ASN-CSE.

Editor’s Note: The procedure indicated below is one of the realization examples. This does not exclude any other realizations.

#### B.6.1.1	 ASN-CSE initiated connectivity establishment procedure

Figure B.6.1.1-1: ASN-CSE initiated connectivity establishment

- Establish an Underlying Network bearer if not already available by using the procedures available in the Underlying Network.

- The ASN-CSE sends a query to a DHCP server to find a particular DNS server IP address. Then the DHCP server responds with an IP address of a corresponding DNS server. Additionally, it is also possible to include one or a list of domain names, i.e, FQDN of target IN-CSEs.

- The ASN-CSE performs a DNS query to retrieve the IN-CSE(s) IP addresses from which one is selected. If the response does not contain the IP addresses, an additional DNS query is needed to resolve a Fully Qualified Domain Name (FQDN) of the serving IN-CSE to an IP address.

- After reception of domain name and IP address of an IN-CSE, the ASN-CSE can initiate communication towards the IN-CSE via IP connection from this procedure. The IN-CSE at this time will be informed which port of the ASN-CSE will be used for communication.

Editor’s Note: It is FFS how oneM2M and 3GPP IDs map to each other for a particular device node to be able to deliver information to a particular serving IN-CSE.

#### B.6.1.2	Serving IN-CSE initiated connectivity establishment procedure

Figure B.6.1.2-1: IN-CSE initiated connectivity establishment

- The Serving IN-CSE receives a Request from Application Server with Terminated-CSE-ID which indicates the destination ASN-CSE.

- The Serving IN-CSE determines the need to trigger a particular ASN-CSE. Once Serving IN-CSE receives the Terminated-CSE ID, serving IN-CSE deduces 3GPP-External-ID which enables it to deliver data to the destination ASN-CSE in 3GPP Underlying Network via MTC functions.

If the serving IN-CSE has no connectivity details for an 3GPP MTC-IWF, it may determine the IP address(es)/port(s) of the MTC-IWF by performing a DNS query using the 3GPP External Identifier or using a locally configured MTC-IWF identifier.

Editor’s Note: Mechanisms other than DNS query could be used for determining the IP address(es)/port(s) of the MTC-IWF. These are FFS.

- The Serving IN-CSE sends the Trigger Request (includes 3GPP External Identifier or MSISDN, ASN-CSE Identifier, trigger reference number, validity period, priority, Application Port ID and trigger payload) message to the MTC-IWF. The IN-CSE includes a trigger payload that contains the information destined for the ASN-CSE (i.e., target device).

Once, 3GPP-MTC-IWF receives the Trigger Request, it deduces particular MSISDN from given 3GPP-External-ID which one-to-one mapping is obtained from a query to 3GPP-HSS.

- The MTC-IWF selects T4 trigger delivery procedure according to the 3GPP TS23.682, based on the information received from HSS/HLR and local policy.

- The MTC-IWF sends the Device Trigger Report (External Identifier or MSISDN and trigger reference number) message to the Serving IN-CSE with a cause value indicating whether the trigger delivery succeeded or failed and the reason for the failure.

- ASN-CSE establishes IP connection with Underlying Network.

After reception of domain name and IP address of the ASN-CSE the IN-CSE can initiate communication with the target device via the IP connection.

# Annex <X>: (Informative):	Mapping of Requirements with CFSs

The following table illustrates the mapping of the Requirements specified in TS-0002 with the CSFs specified in this document.

Table X-1: Mapping of Requirements to CSFs

| CSF Name | Supported Sub-Functions | Associated Requirements | Notes |
| --- | --- | --- | --- |
| Addressing and Identification (AID) | - Management of identifiers | - OSR-026 - OSR-023 - OSR-024 - OSR-025 | Overlap w/: DSC for OSR-023, OSR-024, and OSR-025 |
| Communication Management / Delivery Handling (CMDH) | - Providing communications with other CSE’s, AE’s, and NSE’s - Communications management: best effort - Communications policy management - Underlying Network connectivity management - Communications management: data store and forward - Ability to trigger off-line device | - OSR-001 - OSR-002 - OSR-005 - OSR-006 - OSR-008 - OSR-009 - OSR-012 - OSR-013 - OSR-014 - OSR-015 - OSR-018 - OSR-019 - OSR-021 - OSR-027 - OSR-032 - OSR-035 - OSR-038 - OSR-039 - OSR-040 - OSR-048 - OSR-049 - OSR-050 - OSR-053 - OSR-062 - OSR-063 - OSR-064 - OSR-065 - OSR-066 - OSR-067 - OSR-068 - CRPR-001 - CRPR-002 - CRPR-003 - MGR-016 | Overlap w/: DMR for OSR-001, OSR-009, OSR-021, OSR-032 SSM for OSR-009 LOC for OSR-006 GMG for OSR-006 NSSE for OSR-006, OSR-027 SSM for OSR-009 |
| Data Management and Repository (DMR) | - Data storage and management - Semantic support - Data aggregation - Data analytics - Device data backup and recovery | - OSR-001 - OSR-007 - OSR-009 - OSR-016 - OSR-020 - OSR-021 - OSR-032 - OSR-034 - OSR-036 - OSR-058 - SMR-006 - SER-015 | Overlap w/: CMDH for OSR-001, OSR-009, OSR-021, OSR-032 SUB for OSR-016 GMG for OSR-020 . |
| Device Management (DMG) | - Configuration Management - Diagnostics and Monitoring - Firmware management - Software management - Device Area Network topology management | - OSR-017 - OSR-069 - OSR-070 - OSR-071 - OPR-001 - OPR-002 - OPR-003 - MGR-001 - MGR-003 - MGR-004 - MGR-006 - MGR-007 - MGR-008 - MGR-009 - MGR-011 - MGR-012 - MGR-013 - MGR-014 - MGR-015 - MGR-019 - MGR-020 - MGR-021 - SER-013 - SER-014 | Overlaps w/: GMG for OSR-017 SEC for SER-013 |
| Discovery (DIS) | - Discover resource - Local discovery (within CSE) - Directed remote discovery | - OSR-023 - OSR-024 - OSR-025 - OSR-059 - OSR-060 - OSR-061 - MGR-002 - SMR-004 | Overlaps w/: AID for OSR-023, OSR-024, OSR-025 |
| Group Management (GMG) | - Management of a group and its membership - CRUD - Use Underlying Network group capabilities - Bulk operations - Access control | - OSR-006 - OSR-017 - OSR-020 - OSR-029 - OSR-030 - OSR-031 - OSR-037 - OSR-047 - MGR-005 | Overlaps w/: CMDH for OSR-006 LOC for OSR-006 GMG for OSR-006 NSSE for OSR-006, OSR-037 DMR for OSR-020 DMG for OSR-017 |
| Location (LOC) | - Location management - Network-provided - GPS-provided - Confidentiality enforcement as it relates to location | - OSR-006 - OSR-051 - OSR-052 - SER-026 |  |
| Network Service Exposure / Service execution and triggering (NSSE) | - Access Underlying Network service - Location - Device triggering - Small data - Policy and charging - Support multiple Underlying Network functions | - OSR-006 - OSR-011 - OSR-027 - OSR-037 - OSR-054 - OSR-055 - OSR-056 - MGR-017 - MGR-018 - OPR-004 - OPR-005 - OPR-006 | Overlaps w/: CMDH for OSR-027 GMG for OSR-006, OSR-037 LOC for OSR-006 |
| Registration (REG) | - CSE registration - Application registration - Device registration - ID correlation | - MGR-010 | Overlaps w/: SEC |
| Security (SEC) | - Sensitive Data Handling - Secure storage - Secure execution - Independent environments - Security administration - Pre-provisioning - Dynamic bootstrap - Network bootstrap - Security association - Link level - Object level - Authorization and access - Identity protection | - SER-001 - SER-002 - SER-003 - SER-004 - SER-005 - SER-006 - SER-007 - SER-008 - SER-009 - SER-010 - SER-011 - SER-012 - SER-013 - SER-016 - SER-017 - SER-018 - SER-019 - SER-020 - SER-021 - SER-022 - SER-023 - SER-024 - SER-025 - MGR-010 | Overlap w/: DMG for SER-013 REG for MGR-010 SSM for SER-007 |
| Service Charging and Accounting (SCA) | - Charging enablers - Sending charging information to charging server - Subscription-based charging - Event-based charging - Session-based charging - Service-based charging - Correlation with Underlying Network - Charging management - Offline charging - Online charging | - CHG-001, - CHG-002a, - CHG-002b, - CHG-003, - CHG-004, - CHG-005 |  |
| Service Session Management (SSM) | - Service Session Management (CSE to CSE, Application to CSE, and Application to Application) - Session persistence over link outage - Session context handling - Assignment of session ID - Session routing - Multi-hop session management - Session policy management | - OSR-003 - OSR-004 - OSR-009 - OSR-045 - SER-007 | Overlap w/: CMDH and DMR for OSR-009 SEC for SER-007 |
| Subscription/Notification Support (SUB) | - Subscribe (CS, Application) - Local - Remote - Subscription to a group - Notification - Synchronous - Asynchronous | - OSR-010 - OSR-016 - OSR-033 | Overlaps w/: DMR for OSR-016 |

# Annex <Y>:	(Informative/Normative): Remove Informative or Normative as appropriateTitle of annex (style H9)

<Text>

## Y.1	First clause of the annex (style H1)

<Text>

### Y.1.1	First subdivided clause of the annex (style H2)

<Text>

# Annex <Z>:		
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

# History

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 27-FEB-2013 | Rapporteur Input - Draft Skeleton |
| V0.0.2 | 20-APR-2013 | Rapporteur Input –Draft; Including the following contributions: 1. oneM2M-ARC-2013-0255R03-Arcitecture_Description 2. oneM2M-ARC-2013-0256R01-Definitions_for_Arc_TS 3. oneM2M-ARC-2013-0253R03-Definition_to_SCX_and_SFM |
| V0.0.3 | 08-JUNE-2013 | 1. oneM2M-ARC-2013-0262R02-Update_to_Figure_5-1 |
| V0.0.4 | 28-JUNE-2013 | Includes the following contributions agreed at TP#05: 1. oneM2M-ARC-2013-0275R03-Revised_functional_architecture 2. oneM2M-ARC-2013-0280R03-Definitions_of_Reference_Points 3. oneM2M-ARC-2013-0281R02-M2M_Identifiers 4. oneM2M-ARC-2013-0285R05-Delivery_Handling_in_CSE 5. oneM2M-ARC-2013-0287R04-High_level_Application_Programming_ Interface |
| V0.0.5 | 26-JULY-2013 | Includes the following contributions agreed during ARC#28 and ARC#29 1. oneM2M-ARC-2013-0307R03-Refinement_proposal_for_reference _architecture 2. oneM2M-ARC-2013-0311R01-Definition_of_oneM2M_Node 3. oneM2M-ARC-2013-0312R01-Missing_definitions_for_WG1_work_progress |
| V0.0.6 | 23-AUGUST-2013 | Includes the following contributions agreed during ARC#30 (TP#06 F2F) meeting: 1. oneM2M-ARC-2013-0315R04-Proposal_for_definition_of_specific_Nodes 2. oneM2M-ARC-2013-0316R03-flow_message_extensions 3. oneM2M-ARC-2013-0317R02-Service_Enabler 4. oneM2M-ARC-2013-0340R04-General_mechanism_for_X_and_Y_reference_points 5. oneM2M-ARC-2013-0343R02-Multiple_domains 6. oneM2M-ARC-2013-0348R02-Session-ID 7. oneM2M-ARC-2013-0354R03-Definition_of_Common_Service_Entities_Collection 8. oneM2M-ARC-2013-0357R03-CSF_Descriptions 9. oneM2M-ARC-2013-0359R02-Introducing_Resource_Structure The following documents are included in "oneM2M-ARC-2013-0357R03-CSF_Descriptions": 1. oneM2M-ARC-2013-0302R02-Location_CSF 2. oneM2M-ARC-2013-0318R01-oneM2M-ARC-2013-0318-Data_Management_and_Repository_CSF 3. oneM2M-ARC-2013-0319R02-Network_Service_Exposure_Service_execution_and_triggering_CSF 4. oneM2M-ARC-2013-0322R01-Group_Management_CSF 5. oneM2M-ARC-2013-0325R01-A_CSF_description_for_registration 6. oneM2M-ARC-2013-0328R02-A_CSF_description_for_Discovery 7. oneM2M-ARC-2013-0329R02-Normative_Description_for_Subscription_Notification_Support_CSF 8. oneM2M-ARC-2013-0330R02-Device_management_CSF 9. oneM2M-ARC-2013-0331R01-descriptions_of_charging_CSF 10. oneM2M-ARC-2013-0334R02-description_of_session_management_CSF 11. oneM2M-ARC-2013-0335R02-CSF-ID 12. oneM2M-ARC-2013-0339R01-Service_charging_and_accounting_CSF 13. oneM2M-ARC-2013-0345R02-Security_related_CSFs |
| V0.0.7 | 26-AUGUST-2013 | Includes the following contributions agreed during ARC#31 meeting: 1. 1It was brought to attention that contribution # oneM2M-ARC-2013-0348R02-Session-ID had not been incorporated in v0.0.6 correctly. Corrected in section 7.1.7 2. Review comments provided by Phil Jacobs via his email dated August 21, 2013 |
| V0.1.0 | 19-SEPT-2013 | Includes the following contributions agreed during ARC#6.2 conf call: 1. oneM2M-ARC-2013-0361R04-generic_call_flow_template 2. oneM2M-ARC-2013-0347R03-External-Identifier Includes the following contributions agreed during ARC#6.4 conf call: 1. oneM2M-ARC-2013-0364R08-Resource_template_and_example |
| V0.1.1 | 19-SEPT-2013 | Added Rapporteur comments over TS-0001 v0.1.0, with all changes accepted. |
| V0.1.2 | 01-OCT-2013 | Includes v0.1.0 plus the following comments: 1. Rapporteur comments over TS-001 v0.1.0 with all changes accepted. 2. Comments provided in "oneM2M-TS-0001-oneM2M-Functional-Architecture-V0 1 1_Sec9-5-Comments_JBv2". 3. Comments provided in "oneM2M-TS-0001-oneM2M-Functional-Architecture-V0.1.1pj". |
| V0.2.1 | 28-OCT-2013 | Includes the following contributions agreed during ARC#6.6 conf call: 1. oneM2M-ARC-2013-0366R04-Functional_Architecture_Clarifications 2. oneM2M-ARC-2013-0374R01-Management_CSF_Descriptions Includes the following contributions agreed during ARC#6.7 conf call: 1. oneM2M-ARC-2013-0320R02-Mapping_between_requirements_and_CSF 2. oneM2M-ARC-2013-0388R01-Renaming_reference_points Includes the following contributions agreed during ARC#7.0 F2F Meeting: 1. oneM2M-ARC-2013-0378R03-Reference_Points 2. oneM2M-ARC-2013-0391R02-CR_on_ARC_TS_clause_8_1 3. oneM2M-ARC-2013-0401R03-operation_effectivity_time 4. oneM2M-ARC-2013-0446R01-Update_to_M2M_information_related_to_M2M_Request-ID 5. oneM2M-ARC-2013-0433-Editorial_CR_on_clause_6_2_2_on_CMDH 6. oneM2M-ARC-2013-0435R04-CR_for_clause_9_6_addition_of_ resource_type_request 7. oneM2M-ARC-2013-0436R03-CR_for_clause_8_1_refinement_of_response_type 8. oneM2M-ARC-2013-0437R05-CR_for_clause_8_2_non_blocking_ procedures_to_access_resources 9. oneM2M-ARC-2013-0390R02-3gpp_interworking 10. oneM2M-ARC-2013-0456R03-default_information_to_unsuccessful_operation 11. oneM2M-ARC-2013-0467R03-CR_on_SUB_CSF_Description 12. oneM2M-ARC-2013-0408R01-Removing_AID_CSF 13. oneM2M-ARC-2013-0399-Device_Management_Function_Descriptions 14. oneM2M-ARC-2013-0400R02-Introduction_to_security_in_TS-0001 15. oneM2M-ARC-2013-0480R01-CR_for_clause_3_1_Definitions_and_6_2_3_DMR 16. oneM2M-MAS-2013-0096R04-DM_Server_Interaction 17. oneM2M-MAS-2013-0097R02-DM_Management_Resource_Lifecycle 18. oneM2M-ARC-2013-0448-CR_for_Revision_of_LOC_CSF 19. oneM2M-ARC-2013-0427R03-definition_proposal 20. oneM2M-ARC-2013-0434R03-CR_for_clause_9_6_addition_of_ resource_type_delivery 21. oneM2M-ARC-2013-0438R03-CR_for_clause_10_2_addition_of_CMDH_flows 22. oneM2M-ARC-2013-0392R04-CR_on_ARC_TS_clause_10_1 23. oneM2M-ARC-2013-0485-resource_type_parameter_in_clause_8_1 24. oneM2M-ARC-2013-0449R03-Resource_for_LOC_CSF 25. oneM2M-ARC-2013-0482-CR_for_Revision_of_LOC_CSF 26. oneM2M-ARC-2013-0406R03-M2M_AE_Addressing 27. oneM2M-ARC-2013-0484R01-Updates_to_the_Session_Management_CSF_texts 28. oneM2M-MAS-2013-0098R02-Updates_to_Management_CSF_Descriptions 29. oneM2M-ARC-2013-0487R03-Containers,_Content_instances_and_ Access_Rights_resources_and_flows 30. oneM2M-ARC-2013-0418R05-Section_for_Z_reference_point 31. oneM2M-ARC-2013-0428R02-Discovery_Call_Flow 32. oneM2M-ARC-2013-0419R02-group_resource 33. oneM2M-ARC-2013-0421R04-group_resource_message_flow 34. oneM2M-ARC-2013-0459R02-Virtual_Resource_Description 35. oneM2M-ARC-2013-0444R04-Update-to-Request-ID |
| V0.2.2 | 08-NOV-2013 | Includes Rapporteur Input and editorial corrections such as: 1. Updates Section 7.1 related to M2M Identifiers. 2. Highlights inconsistent use of the notation of pointy-brackets <..> in section 9.2. See the proposed Editor's Notes in section 9.2. 3. Editorial fixes and new editable pictures in section 9.6.x(s). 4. Restructure the "common attributes" Table in Section 9.6.1. Contributions agreed during ARC#7.2 conf call: 1. oneM2M-ARC-2013-0489-ASM_CSF_Architecture_TS_merge_corrections 2. oneM2M-ARC-2013-0490R01-DMG_CSF_Architecture_TS_merge_corrections Contributions agreed during ARC#7.3 conf call: 1. oneM2M-ARC-2013-0393R05-subscription_resource 2. oneM2M-ARC-2013-0504-ASM_CSF_Release_Package_Definition |
| V0.3.1 | 18-DEC-2013 | Includes Rapporteur Input and editorial corrections such as: 1. Clarification of text in section 6.2.9, 7.3, 8.3, 9.5.2.2 2. Update of some pictures and some text in Annex B (3GPP Interworking) Contributions agreed during ARC#7.4 conf call: 1. oneM2M-ARC-2013-0503R03-DMG_CSF_DM_Architecture_Diagram_Updates Contributions agreed during ARC#7.5 conf call: 1. oneM2M-ARC-2013-0505R01-'To'_parameter_clarifications Contributions agreed during ARC#8.0 F2F Meeting: 1. oneM2M-ARC-2013-0584R01-Registration_Clarification 2. oneM2M-ARC-2013-0579R02-Modification_of_abbreviation_NSE 3. oneM2M-ARC-2013-0540-CR_to_ARC_TS_adding_device_triggering_mechanism_in_section_7_3 4. oneM2M-ARC-2013-0549R04-mgmtObj_resource 5. oneM2M-ARC-2013-0550R01-mgmtObj-message-flow 6. oneM2M-MAS-2013-0135R04-mgmtCmd_resource 7. oneM2M-MAS-2013-0136R02-mgmtCmd_flows 8. oneM2M-ARC-2013-0452R04-NOTIFY_call_flow_template 9. oneM2M-ARC-2013-0539R01-Enhancement_on_container 10. oneM2M-ARC-2013-0537-Correction_to_figure_9_5_1_1-1 11. oneM2M-ARC-2013-0565R01-resources_to_establish_charging_policies 12. oneM2M-ARC-2013-0595-Renaming_the_searchStrings_common_attribute 13. oneM2M-ARC-2013-0538-Correction_to_figure_9_6_7 14. oneM2M-ARC-2013-0545-attribute-consistencyStrategy 15. oneM2M-ARC-2013-0546-members-as-virtual-resource 16. oneM2M-ARC-2013-0521R02-Reachability_schedule 17.oneM2M-ARC-2013-0535R01-Expiration_times 18. oneM2M-ARC-2013-0508R01-10_1_Figures_Update 19. oneM2M-ARC-2013-0506R02-Identifiers_Life_Cycle 20.oneM2M-ARC-2013-0548R01-members-message-flow 21. oneM2M-ARC-2013-0598R01-summary_tables 22. oneM2M-ARC-2013-0520R01-CR_AE_Addressing 23. oneM2M-ARC-2013-0555R03-General_Principles_of_Identification 24. oneM2M-ARC-2013-0519R04-Management_Adapter_Enhancements 25. oneM2M-ARC-2013-0574R05-nodeSpecific_Resource_Type 26. oneM2M--ARC-2013-0594R05-Application_Resource_Type 27. oneM2M-ARC-2013-0518-DMG_CSF_DM_Lifecycle_Updates 28. oneM2M-ARC-2013-0568R06-CSE_Resource_Description 29. oneM2M-ARC-2013-0509R04-Revision_of_figure_6-1 30. oneM2M-ARC-2013-0542R02-Inter-M2M_SP_Communication_references 31. oneM2M-ARC-2013-0599R03-M2M_Service_Subscription_Management 32. oneM2M-ARC-2013-0610R03-Naming_Convention 33. oneM2M-ARC-2013-0522R02-Supported_Resource_Type_Discovery_Procedure 34. oneM2M-ARC-2013-0536R01-Multi-protocol_Information_Elements 35. oneM2M-ARC-2013-0547R03-members-resource 36. oneM2M-ARC-2013-0511R03-CR_for_locationPolicy_Resource 37. oneM2M-ARC-2013-0570R01-Location_Management_Procedure 38. oneM2M-ARC-2013-0552R03-Big_Data_Integration 39. oneM2M-ARC-2013-0576R02-mod_resource_tables 40. oneM2M-ARC-2013-0556R03-Response_message_information |
|  |  | 41. oneM2M-ARC-2013-0593R04-Use_of_Links_in_to_relate_resources 42. oneM2M-ARC-2013-0577R02-Resource_Announcement 43. oneM2M-ARC-2013-0566R02-CR_subscription_resource_type 44. oneM2M-ARC-2013-0567R02-Subscription_call_flows 45. oneM2M-ARC-2013-0572R03-Notification_Call_Flows 46. oneM2M-ARC-2013-0607R01-Information_Flow 47. oneM2M-ARC-2013-0606R03-Inter-M2M_SP_Communication 48. oneM2M-ARC-2013-0541R01-Resources_Extensibility 49. oneM2M-ARC-2013-0608R03-Functional_Arch_Update 50. oneM2M-ARC-2013-0611R03-Integrated_structured-flat_URI 51. oneM2M-ARC-2013-0558R03-Additional_attributes_for_group_resource |
| V0.3.2 | xx-JAN-2014 | Review comments from WG2 Members: 1. Addition of parentID attribute to several resources. 2. Changed picture in section 10.1.1-1 3. Removed short names for attributes 4. Editorial fixes, such as section re-numbering, cross reference fixes etc. 5. Change of Figure 9.6.12-1, section 7.1.7, |