| OneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0003-Architecture Part_2 V0.4.0 |
| Document Name: | Architecture_TR_Part_2-Architecture_Merging Part 2: Study for the merging of architectures proposed for consideration by oneM2M |
| Date: | 2013 June 25 |
| Abstract: | Technical analysis of merging existing M2M-related Architecture work undertaken by the oneM2M founding partners (including: ARIB, ATIS, CCSA, ETSI, TIA, TTA, and TTC). V0.4.takes the version agreed by the Working Group and applies the Secretariat pre-approval processing. |

oneM2M IPR and copyright statements

{Editors Note:  pro-forma text required}

# Contents

Contents	2

1	Scope	3

2	References	3

2.1	Partner organization references	3

2.2	Other references	3

3	Definitions, abbreviations and acronyms	3

3.1	Definitions	3

3.2	Abbreviations	3

3.3	Acronyms	4

4	Conventions	4

5	Analysis of Functional Entities	5

5.1	Existing Functional Entities: ATIS, ETSI, TIA	5

6	Analysis of existing Reference Points	7

6.1	Reference Point analysis	7

6.2	Reference Point vs Functions	11

7	Analysis of architecture styles	12

7.1	REST	12

7.2	Web services	12

8	Conclusions	14

History	17

# 1	Scope

This informative Technical Report (TR) provides an evaluation of existing M2M-related Architecture work undertaken by the founding partners of oneM2M, including: the Association of Radio Industries and Businesses (ARIB) and the Telecommunication Technology Committee (TTC) of Japan; the Alliance for Telecommunications Industry Solutions (ATIS) and the Telecommunications Industry Association (TIA) of the USA; the China Communications Standards Association (CCSA); the European Telecommunications Standards Institute (ETSI); and the Telecommunications Technology Association (TTA) of Korea.  Common Functional Entities and Reference Points are identified, as well as critical differences.  New functionality will not be considered as part of this study.

This document is intended to ensure a common understanding of existing M2M Architectural approaches, in order to facilitate future normative work resulting in oneM2M Technical Specifications (TS).

This document has been prepared under the auspices of the oneM2M Technical Plenary, by the oneM2M Architecture Working Group.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative References

The following reference document from oneM2M is used as the basis for the analysis provided within the present document.

Not applicable.

## 2.2	Informative References

The following referenced documents from non-Partners of oneM2M provide additional information for the analysis provided within the present document.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc

[i.2]	oneM2M TR-0002 Architecture Analysis Part 1

[i.3]	X.P0068 (3GPP2) “Network Enhancements for Machine to Machine) that relate to the architectural enhancements and deployment models for supporting Machine to Machine services in 3GPP2 networks”

[i.4]	ATIS M2M

[i.5]	ETSI M2M

[i.5]	IETF CoRE CoAP

[i.6]	OMA Lightweight M2M

[i.7]	OMA-DM (OMA) “OMA Device Management”

[i.8]	REST Dr. Fieldings Disertation 7.1

[i.9]	TIA TR50

3	Definitions, abbreviations and acronyms

## 3.1	Definitions

For this document, the following definitions apply:

Pachube	Pachube is a web service available at pachube.com that enables you to store, share & discover realtime sensor, energy and environment data from objects, devices & buildings around the world.

3.2	Abbreviations

For this document, the following abbreviations apply:

ASPIF	Application Service Provider Interface

CoAP 	Constrained Application Protocol

CoRE	Constrained Restful Environments

MSPIF	Machine to Machine Service Provider Interface

NWIF	Network Interface

REST	Representational State Transfer is a style of API interface

TIA	Telecommunications Industry Association

SOAP

XML	Extensible Markup Language

## 3.3	Acronyms

For this document, the following acronyms apply:

3GPP

AAA	Authentication, Authorization and Accounting

ATIS		Alliance for Telecommunications Industry Solutions

ARIB		Association of Radio Industries and Businesses

CCSA	China Communications Standards Association

ETSI	European Telecommunications Standards Institute

HTTP 	HyperText Transfer Protocol

DA	Device Application

DSCL	Device Service Capability Layer

GA	Gateway Applications

GSCL	Gateway Service Capability Layer

HATEOAS 	Hypermedia As The Engine Of Application State

IETF 	Internet Engineering Task Force

M2M	Machine to Machine (communications)

MAS	M2M Authentication Server

MSBF	M2M Service Bootstrap Function

MTC	Machine Type Communication

NA	Network Applications

NSCL	Network Service Capability Layer

OMA	Open Mobile Alliance

OMA DM 2.0	Open Mobile Alliance Device Management V 2.0

OMA LWM2M	Open Mobile Alliance Light Weight M2M

PoA	Point of Attachment

RPC	Remote Procedure Call

TTA	Telecommunications Technology Association

TTC	Telecommunication Technology Committee

WSDL	Web Service Description Language

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Analysis of Functional Entities

# 5.1	Existing Functional Entities: ATIS, ETSI, TIA

This section provides a comparative analysis of existing TIA TR-50[i.9], ETSI M2M[i.5], and ATIS M2M [i.4] functional architectural entities.

Table 5.1 provides a comparative matrix of the functional architectural entities supported by each architecture.

Table. 5.1 : TIA/ETSI/ATIS M2M Functional Entity Comparative Matrix

| Functional Entity | TIA TR-50 | ATIS | ETSI M2M |
| --- | --- | --- | --- |
| M2M Service Capability hosted in the network domain | Yes Server Container | Yes Service Capabilities | Yes Network Service Capability Layer (NSCL) |
| M2M Service Capability hosted on an intermediary node | Yes PoA Container | No | Yes Gateway Service Capability Layer (GSCL) |
| M2M Service Capability hosted on an M2M Device | Yes PoA Container | No | Yes Device Service Capability Layer (DSCL) |
| Applications in the network domain | Yes home applications | Yes Applications | Yes Network Applications (NA) |
| Applications in the intermediary node | Yes Node Application | No | Yes Gateway Application (GA) |
| Applications in the M2M Device | Yes PoA Application | No | Yes Device Application (DA) |
| M2M Network | Yes Server | Yes Network Service Functions | Yes Network Domain |
| M2M intermediary node | Yes PoA Device | No | Yes M2M Gateway |
| M2M Device | Yes PoA Device | Yes Device | Yes Device with Service Capabilities (D) Device without Service Capabilities (D’) Legacy non-ETSI Device (d) |
| AAA Server | Yes AAA-SD | No | Yes M2M Authentication Server (MAS), M2M Service Bootstrap function (MSBF) |

Note: Functional Elements depicted in this figure may be placed anywhere, not necessarily outside the Access/Core network. The role/placement of AAA Server is FFS.

Figure. 5.1: TIA/ETSI/ATIS Functional Architectural Entities

Figure 5.1 provides a high level overview of the common components when examining TIA TR-50, ETSI M2M, and ATIS M2M functional architectural entities.

# 6	Analysis of existing Reference Points

## 6.1	Reference Point analysis

This section provides a comparative analysis of existing TIA TR-50, ETSI M2M, ATIS, and 3GPP M2M [i.9] architectural reference points.   Figure 6.1 shows the reference points of each architecture consolidated into a single functional architecture.  Table 6.1 provides a comparative matrix of the reference points supported by each architecture.

Figure. 6.1 : TIA/ETSI/ATIS/3GPP M2M Reference Point Analysis

Figure 6.1 provides a high level architectural overview with reference points of the common components when examining TIA TR-50, ETSI M2M, and ATIS M2M functional architectural entities.

Details of the reference points are provided in Table 6.1 and Table 6.2.  Table 6.1 also provides details of 3GPP reference points pertaining to Machine Type Communication (MTC).

Table. 6.1: TIA/ETSI/ATIS/3GPP M2M Reference Point Analysis

| Reference Point / Interface Description | Corresponding Reference Point |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  | TIA | ETSI | ATIS | 3GPP |  |
| M2M Device Application ↔ M2M Device Application | B5 |  |  |  |  |
| M2M Gateway Application ↔ M2M Gateway Application | B4 |  |  |  |  |
| M2M Device Application ↔ M2M Gateway Application | B3 |  |  |  |  |
| M2M Device Application ↔ M2M Network Application | B2 |  |  |  |  |
| M2M Gateway Application ↔ M2M Network Application | B1 |  |  |  |  |
| M2M Device Application ↔ M2M Device | B9 |  |  |  |  |
| M2M Gateway Application ↔ M2M Device | B3’ |  |  |  |  |
| M2M Network Application ↔ M2M Device | B2’ |  | * |  | * ATIS Shows an interface ‘Traffic Flow (Option 2)’ |
| M2M Network Application ↔ Access/Core Network Provider |  |  |  | Gi/Sgi |  |
| M2M Device Application ↔ M2M Device Service Layer |  | dIa |  |  |  |
| M2M Device Application ↔ M2M Gateway Service Layer |  | dIa |  |  |  |
| M2M Gateway Application ↔ M2M Device Service Layer | B8 |  |  |  |  |
| M2M Gateway Application ↔ M2M Gateway Service Layer |  | dIa |  |  |  |
| M2M Network Application ↔ M2M Device Service Layer | B7 |  |  |  |  |
| M2M Network Application ↔ M2M Gateway Service Layer | B6 |  |  |  |  |
| M2M Network Application ↔ M2M Network Service Layer |  | mIa | ASP IF |  |  |
| M2M Device Service Layer ↔ M2M Network Service Layer |  | mId |  |  |  |
| M2M Gateway Service Layer ↔ M2M Network Service Layer |  | mId |  |  |  |
| M2M Network Service Layer ↔ M2M Network Service Layer |  | mIm |  |  |  |
| M2M Network Service Layer ↔ M2M Device |  |  | * |  | * ATIS Shows an interfaces ‘Traffic Flow (Option 1)’ and ‘Device Mangement’ |
| M2M Network Service Layer ↔ Access/Core Network Provider |  |  | MSP IF | TSP |  |
| M2M Device ↔ M2M Device | B5’ |  |  |  |  |
| M2M Device ↔ Access/Core Network Provider |  |  | NW IF |  |  |
| M2M Network Application ↔ AAA Server | A1 |  |  |  |  |
| M2M Gateway Application ↔ AAA Server | A2 |  |  |  |  |
| M2M Device Application ↔ AAA Server | A3 |  |  |  |  |
| M2M Device ↔ AAA Server | A3’ |  |  |  |  |
| M2M Network Service Layer ↔ AAA Server |  | MAS/ MSBF |  |  |  |

## 6.2	Reference Point vs Functions

Table 6.2 provides a comparative analysis of the reference points defined by TIA TR-50, ATIS, and ETSI M2M.

Table. 6.2: TIA/ETSI/ATIS Comparative Reference Point Feature Matrix

| Feature | TIA TR-50 | ATIS | ETSI M2M |
| --- | --- | --- | --- |
| Reference points to support M2M Service Capabilities hosted on a M2M device | Yes (B7, B8) | No | Yes (dIa,mId) |
| Reference points to support an intermediary M2M gateway/node | Yes (B1,B4,B6,B3,B3’,B8, A2) | No | Yes (dIa,mId) |
| Reference points to support communication with access/core network entities | No | Yes (NW IF, MSP IF) | No |
| Reference points to support M2M Service Capabilities hosted in the network | No | Yes (MSP IF, ASP IF) | Yes (mIa,mId,mIm) |
| Reference points to support hierarchical M2M Service Capabilities on Device, Gateway and Network | No | No | Yes (mIa,mId,dIa) |
| Reference points to support direct application-to-application communication | Yes (B1, B2, B3, B4, B5) | No | No |
| Reference points to support direct device-to-device communication | Yes (B5’) | No | No |
| Reference points to support communication between two instances of M2M Service Capabilities hosted in the network | No | No | Yes (mIm) |
| Reference points to support direct communication between network applications and devices and gateways (i.e. bypass M2M Network Service Capabilities) | Yes (B2’) | Yes (Optional Traffic Flow option #2) | No |
| Reference points to support direct communication between network applications and M2M Service Capabilities hosted on devices and gateways (i.e. bypass M2M Network Service Capabilities) | Yes (B6, B7) | No | No |

# 7	Analysis of architecture styles

## 7.1	REST

REST stands for Representational State Transfer. It is a style of API interface. When the API of a system qualifies REST’s features, we say the system is RESTful.

REST is first raised by Doctor Fielding from his doctor’s dissertation [i.8]. The basic notion of REST is resource. Any information in the oneM2M system that can be named and addressed can be a resource: a document or image, a temporal service, a collection of other resources, a non-virtual object, a fragment of data, and so on. REST can be summarised to several basic constraints.

- Client to Server. Client is separated from the Server by interfaces. As long as the interface stays the same, Client and Server can evolve separately.

- The interface between client and server is Stateless. The request on the interface contains all the information needed for the server to handle the request.

- Cache. Cache is used to improve the scalability and performance.

- Uniform Interface. The resources could be addressed by the same methods. There are four constraints about the Uniform Interface:

- identification of resources;

- manipulation of resources through representations;

- self descriptive messages;

- hypermedia as the engine of application state (HATEOAS).

- Layered system. The system is divided by several layers. Each layer provides functions to the upper layer by utilise the functions provided by the lower layer. Each layer can evolve separately.

RESTful guarantees that the client needs no prior knowledge of the server. Every client can access to the resources using uniform interfaces. The method to parse the resource is along with the resource. A REST Client can interact with the server entirely using hypermedia provided by the server, which is the concept of HATEOAS. Resources are connected with each other using links. The REST Client can navigate from resources to resources to obtain the information desired. The HATEOAS constraint serves to decouple client and server in a way that allows the server to evolve functionality independently.

More and more architecture designers went to RESTful architecture in M2M area now, ETSI M2M, OMA DM2.0, OMA LWM2M, IETF CoRE CoAP, pachube etc.

## 7.2	SOAP

Web services provide a layer of abstraction above existing software, such as application servers, messaging, and packaged applications.

Applications expose interfaces that are described in a machine process-able format, the Web Service Description Language (WSDL). It is also possible for applications to interact through SOAP interfaces which provide a means to describe message format. These messages are often transported over HTTP and encoded using XML.

SOAP is method for exchanging XML based message over the Internet for providing and consuming web services. SOAP message are transferred forming the SOAP-Envelope.

RPC (remote procedure call) is another way of providing and consuming web services. It uses XML to encode and decode the remote procedure call along with its parameters.

8	Conclusions

This report offers an overview and summary of the most current standards activity related to M2M, with a goal of providing a common understanding of existing M2M Architectural approaches.

This report may be used to facilitate future normative work resulting in oneM2M Technical Specifications (TS).

# History

| Publication history |  |  |
| --- | --- | --- |
| V.1.0.0 | <dd Mmm 2013> | <Milestone: Scheduled for TP#3 – tbc> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 07 Feb 2013 | Rapporteur Input – Skeleton Draft |
| V0.0.2 | 13 Feb 2013 | Rapporteur Input – Skeleton Draft (editorial revisions) |
| V0.0.3 | 25 Feb 2013 | Rapporteur Input – Skeleton Draft (input to ARC F2F meeting) |
| V0.0.4 | 27 Feb 2013 | Rapporteur Input – Skeleton Draft (agreed during ARC F2F meeting) |
| V0.0.5 | 28 Feb 2013 | Rapporteur Input – Skeleton Draft (revised document title) |
| V0.1.0 | 11 Mar 2013 | Rapporteur Output Draft from TP#003 including following agreed documents: oneM2M-ARC-2012-0168R03 (Initial text for clause 5 and clause 6) |
| V0.1.1 | 18-Mar-2013 | Rapporteur Output Draft from ARC call 14-Mar-2013, with following revisions: 1) Added Table 6.1 and Table 6.2 from agreed document 0168R03 (these tables were inadvertently omitted from V0.1.0) 2) Added Note above Figure 5.1 re: placement of Functional Elements and AAA 3) Changed label on “M2M Constrained Device” in Figure 5.1 to “M2M Device” |
| V0.2.0 | 17-Apr-2013 | Rapporteur Output Draft from TP#004 including following agreed documents: oneM2M-ARC-2013-0250 (Introduction of REST) |
| V0.3.0 | 09-June-2013 | Rapporteur Input Draft for TP#005 based on output draft V0.2.0 from TP#004: Updated sections 2.1, 5.1, 6.1, 6.2, & 8. Removed sections 6.3, 8.1 through 8.5, and Annex |
| V0.3.1 | 18-June-2013 | Replaced section 7.2 title from Web Service to SOAP. Removed editor notes related to further input from TIA. |
| V0.4.0 | 25 June-2013 | Pre-approval processing by the Secretariat |