| OneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0001 oneM2M Functional Architecture-V-0.0.5 |
| Document Name: | oneM2M Functional Architecture |
| Date: | 2013-JULY-26 |
| Abstract: | This document illustrates end-to-end M2M Services Architecture and specifes the functional architecture of the oneM2M Services Platform. |

oneM2M IPR and copyright statements

Text required

# Contents

Contents	2

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols and abbreviations	4

3.1	Definitions	4

3.2	Symbols	5

3.3	Abbreviations	5

4	Conventions	5

5	Architecture Model	5

5.1	General Concepts	5

5.2	Architecture Reference Model	6

5.2.1	High Level Functional View	6

5.2.2	Reference Points	6

5.2.2.1	X Reference Point	6

5.2.2.2	Y Reference Point	7

5.2.2.3	Z Reference Point	7

6.	Functional Architecture	7

6.1	Deployment Scenarios	9

6.1.1 	Single Intermediate Node case	9

6.1.2	Multiple Intermediate Nodes	9

6.1.3	Application located outside of Device Node	9

6.1.x2	TBD	9

6.2	Functional Entities	9

6.2.1	Functional Entities comprising the set of CSFs	9

6.2.1.1	Communication Management and Delivery Handling	10

6.3	Security Aspects	10

6.4	Other Aspects	10

7.	M2M Identification and addressing	11

7.1	Introduction	11

7.2	M2M Identifiers	11

7.2.1	M2M Service Provider Identifier (M2M-SP-ID)	11

7.2.2	Application Instance Identifier  (App-Inst-ID)	11

7.2.3	Application Identifier (App-ID)	11

7.2.4	CSE Identifier (CSE-ID)	11

7.2.5	M2M Node Identifier/Device Identifier (M2M-Node-ID)	11

7.2.6	M2M Service Subscription Identifier (M2M-Sub-ID)	11

7.3	M2M Identifiers lifecycle and characteristics	12

8.	Functional Description	12

8.1	General Concepts	12

8.7.1	Communication Management and Delivery Handling	12

8.7.2	Other CSFs?	13

9.	X and Y Reference Points	13

9.1	Introduction	13

9.2	General communication  Flow scheme	13

9.2.1	Description	13

9.2.2	Send	14

9.2.3	Successful Response	15

9.2.4	Unsuccessful Response	15

9.3	Resource Structure	15

9.3.1	Introduction	15

9.4	Procedure Description	16

10.	Information Flows	16

10.1	General Concepts	16

Proforma copyright release text block	16

Annex <y>: Bibliography	19

History	20

# 1	Scope

This specification describes the end-to-end oneM2M functional architecture, including the description of the functional entities and associated reference points.

oneM2M functional architecture has focus on the Service Layer aspects and takes Underlying Network-independent view of the end-to-end services. The Underlying Network is used for the transport of data and potentially for other services.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	 ETSI TR 102 473: "<Title>".

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Application Layer:  This layer comprises oneM2M Applications and related business and operational logic.

Common Services Layer:  This layer consists of oneM2M service functions that enable oneM2M Applications (via management, discovery and policy enforcement to name a few).

Common Services Entity:  The Common Services Entity is an instantiation of Common Services Functions (CSFs). Common Services Entity provides a subset of CSFs that could be used and shared by M2M Applications. Common Services Entity can utilize Underlying Network capabilities and can interact with each other to fulfil the service.

Editor’s Note: This is a provisional definition.

Common Services Function: A Common Services Function (CSF) is a set of service functions that are common to the M2M environment and specified by oneM2M. A CSE contains one or more CSFs.

Editor's Note: This definition needs some more work so that it is more clear (about the M2M environment and the relationship between CSE/CSF and level of service functions.

Network Services Layer:  Provides transport, connectivity and service functions.

Node:  A functional entity containing one of the following:

•	one or more M2M Applications,

•	one CSE and zero or more M2M Applications.

## 3.2	Symbols

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Architecture Model

Editor's Note: Definitions are FFS. As definitions get refined and agreed to, the following text to be adjusted to eflect the agreements

## 5.1	General Concepts

Figure 5.1-1 depicts the oneM2M Layered Model  for supporting end-to-end (E2E) M2M Services. This layered model comprises three layers: Application Layer, Common Services Layer and the Underlying Network Services Layer.

Figure 5.1-1: oneM2M Layered Model

## 5.2	Architecture Reference Model

### 5.2.1	High Level Functional View

Figure 5.2.1-1illustrates high level functional view of the oneM2M architecture.

Figure 5.2.1-1: High Level Functional View

Editor's Note: Applicability of this functional view to network infrastructure and /or to M2M devices is FFS.

The high level functional view in Figure 5.2.1-1 comprises of the following functions:

1.	Application Entity (AE): Application Entity provides Application logic for the end-to-end M2M solutions. Examples of the Application Entities can be fleet tracking application, remote blood sugar monitoring application, or remote power metering and controlling application.

2.	Common Services Entity (CSE):  A Common Services Entity comprises the set of "service functions" that are common to the M2M environments and specified by oneM2M. Such service functions are exposed to other entities through Reference Points X and Y. Reference point Z is used for accessing Underlying Network Service Entities.

Examples of service functions offered by CSE are: Data Management, Device Management, M2M Subscription Management, Location Services etc. Such "subfunctions" offered by a CSE may be logically apprehended as Common Services Functions (CSFs). Inside a Common Services Entity (CSE), some of the CSFs can be mandatory and others can be optional. Also, inside a CSF, some subfunctions can be mandatory or optional (e.g., inside a "Device Management" CSF, some of the subfunctions like "Application Software Installation", "Firmware Updates", "Logging", "Monitoring", etc. can be mandatory or optional).

3.	Underlying Network Services Entity (NSE):  An Underlying Network Services Entity provides services to the CSEs. Examples of such services include device management, location services and device triggering. No particular organization of the NSEs is assumed.

Note: Underlying Networks provide data transport services between entities in the oneM2M system. Such data transport services are not included in the NSE.

### 5.2.2	Reference Points

The following reference points are supported by the CSE.

#### 5.2.2.1	X Reference Point

This is the reference point between an Application Entity and a CSE. The X reference point shall allow an  Application Entity to use the services provided by the CSE, and for the CSE to communicate with the  Application Entity.

The services offered via the X reference point are thus dependent on the functionality supported by the CSE. The  Application Entity and the CSE it invokes may or may not be co-located within the same physical entity.

#### 5.2.2.2	Y Reference Point

This is the reference point between two CSEs. The Y reference point shall allow a CSE to use the services of  another CSE in order to fulfill needed functionality. Accordingly, the Y reference point between  two CSEs shall be supported over different M2M physical entities. The services offered via the Y reference point are dependent on the functionality supported by the CSEs

#### 5.2.2.3	Z Reference Point

This is the reference point between a CSE and the Underlying Network Services Entity. The Z reference point shall allow a CSE to use the services (other than transport and connectivity services) provided by the Underlying Network Services Entity in order to fulfill the needed functionality.

The instantiation of the Z reference point is dependent on the services provided by the Underlying Network Services Entity.

Note: Information exchange between two  M2M nodes assumes the usage of the transport and connectivity services of the Underlying Network Services Entity, which are considered to be the basic services.

Editor's Note: While allowing for different instantiations of the CS Function into different functional Entities, this specification follows the following guidelines/principles while specifying the interfaces on X, Y and Z reference points:

•	Specification of the interfaces on reference points X and Y are independent of the functionality provided by the CSE.

•	Specification of the interfaces on reference point Z may take into account the functionality provided by the CSE.

# 6.	Functional Architecture

Figure 6-1 illustrates the Functional Architecture for providing  oneM2M services.

Figure 6-1: Functional Architecture

Node:	A oneM2M Node is a functional entity that shall contain at least one oneM2M Common Services Entity or one oneM2M Application Entity. A oneM2M Node may be contained in a physical object e.g., M2M device, gateway or server/service infrastructure.

NOTES:

1.	CSEs resident in different nodes are not identical and are dependent on the services supported by the CSE in that node.

Editor's Notes:

•	The illustration above does not provide an exhaustive list of possible configurations and is subject to qualification. Other possible functions/configurations are FFS.

•	Illustration of Multiple domain cases are FFS.

Editor's Note:

- In Figure 6-1, should we explain how Application Function (AF) and Application (A) relate?

- should Case 1 and 2 Application Nodes have Z interfaces else should it be explained why not?

- Should the placement of the text "Application node" and "Device Node" in Figure 6-1 be placed to indicate which applies to cases 2 and 3 ?

## 6.1	Deployment Scenarios

In this clause various deployment scenarios are introduced based on the functional architecture in Figure 6-1.

### 6.1.1 	Single Intermediate Node case

A single Intermediate Node is placed between a Device Node and an Infrastructure Node.

### 6.1.2	Multiple Intermediate Nodes

An Intermediate Node can be connected to one or more other Intermediate Nodes. In this deployment scenario, the CSE in an Intermediate Node can communicate with the CSE in another Intermediate Node over the Y reference point.

### 6.1.3	Application located outside of Device Node

For a node such as that illustrated in Case 3, Applications can be located outside of the Device Node.

### 6.1.x2	TBD

<Text>

## 6.2	Functional Entities

### 6.2.1	Functional Entities comprising the set of CSFs

This clause defines the functional entities that are part of the oneM2M-specified CSFs. A specific instantiation of a set of functions picked from these CSFs, i.e. a specific CSE may not contain all of the listed functions.

The functions in the Common Servics Layer provide common services functions to the Applications via the X reference point and to other CSEs via the Y reference point, and interaction with the Underlying Network Service Functions via  the Z reference point. In order to communicate with the Applications, the CSEs and Underlying Networks, the CSFs issues requests to the Communication Management and Delivery Handling function as defined below.

Figure 6.2.1-1 Common Services Functions

Editor's Note: The illustration above will be replaced based on the agreements in contribution # ARC-0293.

#### 6.2.1.1	Communication Management and Delivery Handling

The Communication Management and Delivery Handling CSF is responsible for providing communications with other CSEs, AFs and NSFs .

The Communication Management and Delivery Handling function is responsible to decide at what time which communication connection to use for delivering communication (e.g. CSE-to-CSE)  and, when needed and allowed,  store communication requests so that they can be forwarded at a later time. This processing in the Communication Management and Delivery Handling function has to be carried out in line with the provisioned policies and delivery handling parameters that can be specific to each request for communication.

For communication using the Underlying Network data transport services, the deployed Underlying Network can support the equivalent delivery handling functionality. In such case the Communication Management and Delivery Handling function shall be able to use the Underlying Network, and it may act as a simple front end to access the Underlying Network equivalent functionality.

## 6.3	Security Aspects

<Text>

## 6.4	Other Aspects

<Text>

# 7.	M2M Identification and addressing

## 7.1	Introduction

This clause provides a list of identifiers required for the purpose of interworking within the oneM2M architectural model.

## 7.2	M2M Identifiers

### 7.2.1	M2M Service Provider Identifier (M2M-SP-ID)

An M2M Service Provider shall be uniquely identified by the M2M Service Provider Identifier (M2M-SP-ID). This is a static value assigned to the Service Provider.

### 7.2.2	Application Instance Identifier  (App-Inst-ID)

An Application Instance Identifier (App-Inst-ID) uniquely identifies an M2M Application instance resident on an M2M node, or an M2M Application instance that requests to interact with an M2M node. An App-Inst-ID shall identify an Application for the purpose of all interactions from/to the Application within the M2M framework.

It is the responsibility of the M2M Service Provider to ensure that the App-Inst-ID is globally unique. The App-Inst-ID shall include the Application ID (see clause 7.2.3.)

### 7.2.3	Application Identifier (App-ID)

This is equivalent to the application name and is not guaranteed to be globally unique on its own.

Editor's Note:  Who assigns the App-ID is FFS.

### 7.2.4	CSE Identifier (CSE-ID)

A CSE shall be identified by a globally unique identifier, the CSE-ID, when instantiated within an M2M node in the M2M architecture.

The CSE-ID shall identify the CSE for the purpose of all interactions from/to the CSE within the M2M framework.

### 7.2.5	M2M Node Identifier/Device Identifier (M2M-Node-ID)

An M2M node, hosting a CSE and/or Application(s) shall be identified by a globally unique identifier, the M2M-Node-ID.

The M2M system shall allow the M2M Service Provider to set the CSE-ID and the M2M-Node-ID to the same value.

The M2M-Node-ID enables the M2M Service Provider to bind a CSE-ID to a specific M2M node.

Editor's Note: Several  terms used in this clause: "M2M framework", "M2M architecture", "M2M system". Need to agree on a single terminology.

### 7.2.6	M2M Service Subscription Identifier (M2M-Sub-ID)

The M2M-Sub-ID enables the M2M Service Provider to bind application(s), M2M nodes, CSEs to a particular M2M service subscription.

The M2M Service Subscription Identifier has the following characteristics:

•	belongs to the M2M Service Provider,

•	identifies the subscription to an M2M Service Provider,

•	enables communication with the M2M Service Provider,

•	can differ from the M2M Underlying Network Subscription Identifier,

•	changes with the change of the M2M Service Provider.

There can be multiple M2M Service Subscription Identifiers per M2M Underlying Network subscription.

## 7.3	M2M Identifiers lifecycle and characteristics

FFS.

# 8.	Functional Description

<Text>

## 8.1	General Concepts

### 8.7.1	Communication Management and Delivery Handling

The basic services that other CSFs, AFs or NSFs can request from the Communication Management and Delivery Handling CSF is to transport some given data to a specific target (CSE, AF or NSF),  according to specific delivery instructions while staying within the constraints of provisioned Communication Management and Delivery Handling Policies.

The content of the data provided by the requestor is opaque to the delivery handling and does not influence the behaviour of the Communication Management and Delivery Handling CSF. In particular, the Communication Management and Delivery Handling CSF is not aware of the specific destination function (CSF) within the target entity including the parameters of such a function. For instance if the destination of the data to be delivered is a data management CSF within a target entity (CSE, NSF, AF), the Communication Management and Delivery Handling CSF shall not be aware of the data management CSF including the specific storage location or other parameters pertaining to that data management CSF. That means all attributes that are intended to be shared with the destination function (e.g., which CSF is the destination on the target entity, what that CSF should do with the data etc.) should be hidden from the Communication Management and Delivery Handling CSF.

The target entity may be reached either directly or via the CSE of an Intermediate Node.

Editor’s Note:  For Rel-1 it would probably make sense to limit to one Intermediate Node along the path since we need to define how to handle routing which could become more complex and should be able to be added at a later stage without or with little impact on the message exchanges.

The delivery instructions that are given to the Communication Management and Delivery Handling function set the limits in the delivery process that the requestor is willing to accept. For instance those instructions can contain acceptable delay tolerance etc.

In line with provisioned policies and delivery instructions that can be specific to each communication request  the Communication Management and Delivery Handling CSF shall decide at what time which communication path to use for delivering the communication (e.g. CSE-to-CSE) and, when needed and allowed, store communication requests so that they can be forwarded at a later time.

As a consequence the Communication Management and Delivery Handling CSF will have to be aware of the already established connections to other nodes via the Underlying Networks, request establishment of new connections and tear down existing ones.

In order to accomplish this function, the Communication Management and Delivery Handling CSF will need to have access to provisioned delivery handling policies and needs to parse them so that a proper use of buffers for Store-and-Forward processing can be managed.

Editor’s Note: Clarification of complexity impacts due to use of multiple Underlying Networks are FFS.

### 8.7.2	Other CSFs?

<Text>

# 9.	X and Y Reference Points

## 9.1	Introduction

Procedures involving a CSE and Applications are executed by data exchange according to message flow described in clause 9.4.

The data exchange consists of information transferred across the reference points and stored in a standardized resource structure as described in clause 9.3.

The use of the addressable resources is also enabling the “store-and-share” paradigm of application information and big data.  Access and manipulation of the resources is subject to their correlated permissions.

Some of the procedures on the X andY reference points may adopt a different approach with respect to the ones described in this clause. Such procedures are described in <reference to be included when available>.

Editors Note: Which of the procedures do not adopt the approach described in this clause and how they are performed is FFS.

## 9.2	General communication  Flow scheme

### 9.2.1	Description

Figure 9.2.1-1 shows the general flow that governs the information exchange within a procedure, which is based on the use of Send and Response scheme. The scheme applies to communications such as:

- between an Application and a CSE (X reference point), and

- among CSEs (Y reference point).

The communications can be initiated either by the Applications or by the CSEs.

Figure 9.2.1-1: Resource Manipulation by an Application within an Entity

### 9.2.2	Send

Send request from an Issuer to a Receiver includes the following information:

op:  operation to be executed: Create (C), Retrieve (R), Update (U), Delete (D),

to:  address of target resource, e.g. /m2m.provider1.com/netBase/temp1,

fr:  address of the resource representing the Issuer. Used for access control, e.g., /m2m.provider2.com/remBase/app1,

hd:  headers containing meta-information about the Send request,

cn:  resource content to be transferred.

Note: The to target resource needs to be known by the Issuer. It can be known either by pre-provisioning or by discovery.

Editor’s Note: How pre-provisioning and discovery are performed is FFS.

The op information shall indicate the operation to be executed in the Receiver:

Create (C):  a new resource addressable with to parameter is created,

Retrieve (R):  an existing to addressable resource is read and provided back to the Issuer,

Update (U):  the content of an existing to addressable resource is replaced with cn new content,

Delete (D):  an existing to addressable resource and all its sub-resources are deleted from the Resource Storage.

Editor’s Note: This is an initial list of the verbs. The need for more verbs is anticipated to cover all use cases, configuration and functions.

The to information shall address the target resource in the Receiver.

The fr information shall be used by the Receiver to check the Issuer identity for access permission verification.

The cn information shall be present in Send request for the following operations:

Create:  cn is the content of the new resource,

Update:  cn is the content to be replaced in an existing resource,

Retrieve:  cn is the filter to be applied for discovery purposes.

Once the Send request is delivered, the Receiver shall analyze the Send to determine the target resource.

If the target resource is addressing another M2M node, the Receiver shall route the request appropriately.

If the target resource is addressing the Receiver, it shall:

•	Check the existence of to addressed resource,

•	Identifying the resource type,

•	Check the permission for fr Issuer to perform the requested operation,

•	Perform the requested operation (using cn content when provided),

•	Respond to the  Receiver for successful or unsuccessful operation results. In some specific cases (e.g. limitation in the binding protocol or based on application indications), the Response could be avoided.

The message flow procedure started with an Issuer request shall be considered closed when a Response is delivered to the Issuer.

### 9.2.3	Successful Response

Response from a Receiver to an Issuer includes the following information:

op:  operation being executed: Create (C), Retrieve (R), Update (U), Delete (D) (optional),

hd:  headers containing meta-information about the request,

rs:  operation result: e.g. Okay, Okay and Done; Okay and Scheduled; Okay and In Progress, etc.,

cs:  optional additional result information, e.g. status codes,

to:  address of target resource, e.g. /m2m.provider1/netBase/temp1,

cn:  resource content to be transferred (optional).

The cn information may be present in a Response in the following cases:

Update:  cn is the content replaced in an existing resource,

Delete:  cn is the content actually deleted.

The cn information shall be present in a Response in the following cases:

Retrieve:  cn is the retrieved resource content or aggregated contents of discovered resources.

### 9.2.4	Unsuccessful Response

Response from a Receiver to an Issuer includes the following information:

op:  operation being executed: Create (C), Retrieve (R), Update (U), Delete (D) (optional),

hd:  headers containing meta-information about the request,

rs:  operation result: e.g. Not okay.

cs:  optional additional result information, e.g. status codes.

## 9.3	Resource Structure

### 9.3.1	Introduction

This clause introduces the types of resources used in a CSE. A resource scheme is used for modelling the resource structure and associated relationships.

Table X.1: Resource types and tagging

| Resource type | Tag value | Description |
| --- | --- | --- |
| Base | b |  |
| CSE | e |  |
| Application | a |  |
| Access Right | r |  |
| <tbd> |  |  |

Editor's Note:  The contents of this table is only an initial draft proposal.

## 9.4	Procedure Description

Editor's Note: This sclause describes the procedures and associated Stage 2 information flows.

# 10.	Information Flows

<Text>

## 10.1	General Concepts

<Text>

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

Annex <A> (Informative): Deployment Scenarios

<Text>

Annex <B>(Informative/Normative): Remove Informative or Normative as appropriateTitle of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

# History

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 27-FEB-2013 | Rapporteur Input - Draft Skeleton |
| V0.0.2 | 20-APR-2013 | Rapporteur Input –Draft; Including the following contributions: 1. oneM2M-ARC-2013-0255R03-Arcitecture_Description 2. oneM2M-ARC-2013-0256R01-Definitions_for_Arc_TS 3. oneM2M-ARC-2013-0253R03-Definition_to_SCX_and_SFM |
| V0.0.3 | 08-JUNE-2013 | 1. oneM2M-ARC-2013-0262R02-Update_to_Figure_5-1 |
| V0.0.4 | 28-JUNE-2013 | Includes the following contributions agreed at TP#05: 1. oneM2M-ARC-2013-0275R03-Revised_functional_architecture 2. oneM2M-ARC-2013-0280R03-Definitions_of_Reference_Points 3. oneM2M-ARC-2013-0281R02-M2M_Identifiers 4. oneM2M-ARC-2013-0285R05-Delivery_Handling_in_CSE 5. oneM2M-ARC-2013-0287R04-High_level_Application_Programming_ Interface |
| V0.0.5 | 26-JULY-2013 | Incluses the following contributions agreed during ARC#28 and ARC#29 1. oneM2M-ARC-2013-0307R03-Refinement_proposal_for_reference _architecture 2. oneM2M-ARC-2013-0311R01-Definition_of_oneM2M_Node 3. oneM2M-ARC-2013-0312R01-Missing_definitions_for_WG1_work_progress |