| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0006-Study_of_Management_Capability_Enablement-V0_4_1 |
| Document Name: | Study of Management Capability Enablement Technologies for Consideration by oneM2M |
| Date: | 2013-Oct-09 |
| Abstract: | Collect and describe the state-of-the-art technologies (e.g. OMA DM and BBF TR069) that are relevant to oneM2M management capabilities. Analyze the collected technologies to match with the oneM2M requirements on management aspects; Evaluate the possibility of leveraging all or part of those technologies by oneM2M to enable its management capability. |

oneM2M IPR and copyright statements

“Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of an agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

# Contents

Contents	2

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions, symbols, abbreviations  and acronyms	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

3.4	Acronyms	8

4	Conventions	10

5	Introduction of existing technologies	10

5.1	 Introduction to OMA DM	10

5.1.1	Description	10

5.1.2	Architecture	11

5.1.3	Reference points	12

5.1.3.1	Introduction	12

5.1.3.2	DM-1 DM Client-Server Notification	12

5.1.3.3	DM-2 DM Client-Server Protocol	12

5.1.3.4	DM-3 DM Bootstrap Profile via Smart Card	12

5.1.3.5	DM-4 DM Bootstrap Profile OTA	12

5.1.3.6	CP-1 CP Bootstrap Profile	12

5.1.3.7	DM-6 DM Server-Server Interface	12

5.1.3.8	DM-7,8,9 Client API	12

5.1.3.9	Procedures	13

5.1.4	Protocols	13

5.1.4.1	Protocol Stack	13

5.1.4.2	Application MO	14

5.1.4.3	DM Protocol	14

5.1.4.4	DM Representation	14

5.1.4.5	Binding to transports and Transports	14

5.1.5	Functions	14

5.1.5.1	Introduction	14

5.1.5.2	The MO tree	15

5.1.5.2.1	Standard Objects	15

5.1.5.2.2	Other Management Objects	15

5.2	TR-069 Family of Specifications	16

5.2.1	Description	16

5.2.2	Architecture	16

5.2.2.1	TR-069 Proxy Management	17

5.2.2.1.1	Proxied Device Deployment Archirecture	17

5.2.3	Reference points	18

5.2.4	Protocols	19

5.2.4.1	ACS to CPE Protocol	19

5.2.4.2	CPE to BSS	19

5.2.4.2.1	IPDR Reference Points	20

5.2.4.3	CPE to Device Protocol	21

5.2.4.3.1	UPnP DM Proxy	21

5.2.4.3.2	ZigBee Proxy	21

5.2.5	Functions	23

5.3	Introduction to OMA LightweightM2M (LWM2M)	23

5.3.1	Description	23

5.3.2	Architecture	24

5.3.3	Reference Points	24

5.3.3.1	Functional Components	24

5.3.3.1.1	LWM2M Server	24

5.3.3.1.2	LWM2M Client	25

5.3.3.2	Interfaces	25

5.3.4	Protocols	25

5.3.4.1	Protocol Stack	25

5.3.4.2	Resource Model	26

5.3.4.3	Interface Descriptions	26

5.3.4.3.1	Device Discovery and Registration	26

5.3.4.3.2	Bootstrap	27

5.3.4.3.3	Device Management and Service Enablement	27

5.3.4.3.4	Information Reporting	28

5.3.5	Functions	28

5.4	Introduction to OMA Device Management 2.0	29

5.4.1	Description	29

5.4.2 	Architecture	29

5.4.3 	Reference Points	30

5.4.3.1 	Functional Components	30

5.4.3.1.1 	DM Client	30

5.4.3.1.2 	DM Server	30

5.4.3.1.3 	Web Server Component	30

5.4.3.1.4 	Web Browser Component	30

5.4.3.1.5 	Data Repository	30

5.4.3.2 	Interfaces	31

5.4.4	Protocol	31

5.4.4.1 	DM Packages	31

5.4.4.2 	DM Commands	32

5.4.5	Functions	32

5.4.5.1	Introduction	32

5.4.5.2	Management Objects supported by OMA DM 2.0	33

5.4.5.3	Detailed Comparisons with OMA DM 1.x	33

5.4.5.4	Protocol Examples	34

6	Gap analysis of existing relevant technolgoies	35

6.1	Management related requirements gap analysis reference	35

6.2	MGR-001	36

6.2.1	Requirement Description	36

6.2.2	OMA DM 1.3	36

6.2.3	BBF TR-069	36

6.2.4	OMA DM LWM2M	37

6.2.5	OMA DM 2.0	37

6.3	MGR-002	37

6.3.1	Requirement Description	37

6.3.2	OMA DM 1.3 and OMA DM 2.0	37

6.3.3	BBF TR-069	38

6.3.4	OMA DM LWM2M	38

6.4	MGR-003	39

6.4.1	Requirement Description	39

6.4.2	OMA DM 1.3 and OMA DM 2.0	39

6.4.3	BBF TR-069	39

6.4.4	OMA DM LWM2M	39

6.5	MGR-004	39

6.5.1	Requirement Description	39

6.5.2	OMA DM 1.3 and OMA DM 2.0	39

6.5.3	BBF TR-069	40

6.5.4	OMA DM LWM2M	40

6.6	MGR-005	40

6.6.1	Requirement Description	40

6.6.2	OMA DM 1.3 and OMA DM 2.0	40

6.6.3	BBF TR-069	40

6.6.4	OMA DM LWM2M	40

6.7	MGR-006	41

6.7.1	Requirement Description	41

6.7.2	OMA DM 1.3 and OMA DM 2.0	41

6.7.3	BBF TR-069	41

6.7.4	OMA DM LWM2M	41

6.8	MGR-007	41

6.8.1	Requirement Description	41

6.8.2	OMA DM 1.3 and OMA DM 2.0	41

6.8.3	BBF TR-069	41

6.8.4	OMA DM LWM2M	42

6.9	MGR-008	42

6.9.1	Requirement Description	42

6.9.2	OMA DM 1.3 and OMA DM 2.0	42

6.9.3	BBF TR-069	42

6.9.4	OMA DM LWM2M	42

6.10	MGR-009	43

6.10.1	Requirement Description	43

6.10.2	OMA DM 1.3 and OMA DM 2.0	43

6.10.3	BBF TR-069	43

6.10.4	OMA DM LWM2M	43

6.11	MGR-010	43

6.11.1	Requirement Description	43

6.11.2	OMA DM 1.3 and OMA DM 2.0	43

6.11.3	BBF TR-069	43

6.11.4	OMA DM LWM2M	44

6.12	MGR-011	44

6.12.1	Requirement Description	44

6.12.2	OMA DM 1.3 and OMA DM 2.0	44

6.12.3	BBF TR-069	44

6.12.4	OMA DM LWM2M	44

6.13	MGR-012	44

6.13.1	Requirement Description	44

6.13.2	OMA DM 1.3 and OMA DM 2.0	44

6.13.3	BBF TR-069	44

6.13.4	OMA DM LWM2M	45

6.14	MGR-013	45

6.14.1	Requirement Description	45

6.14.2	OMA DM 1.3 and OMA DM 2.0	45

6.14.3	BBF TR-069	45

6.14.4	OMA DM LWM2M	45

6.15	MGR-014	46

6.15.1	Requirement Description	46

6.15.2	OMA DM 1.3 and OMA DM 2.0	46

6.15.3	BBF TR-069	46

6.15.4	OMA DM LWM2M	46

6.16	MGR-015	46

6.16.1	Requirement Description	46

6.16.2	OMA DM 1.3 and OMA DM 2.0	46

6.16.3	BBF TR-069	46

6.16.4	OMA DM LWM2M	47

6.17	MGR-016	47

6.17.1	Requirement Description	47

6.17.2	OMA DM 1.3 and OMA DM 2.0	47

6.17.3	BBF TR-069	47

6.17.4	OMA DM LWM2M	47

6.18	MGR-017	47

6.18.1	Requirement Description	47

6.18.3	TR-069	48

7	oneM2M Architectural Considerations	48

7.1	Introduction	48

7.2	Current Management Deployment Scenarios	48

7.2.1	Managed Device Using Network Operator Management	48

7.2.2	Managed Device Using Service Provider Management	49

7.3	Possible Future Management Deployment Scenarios	49

7.3.1	Shared Managed Device Using Network Operator Management	49

7.3.2	Shared Managed Device Using Service Provider Management	50

7.3.3	Shared Managed Device Using Separate Management	50

7.3.4	Federated Managed Device Using Separate Management	51

7.3.5	Conclusions To Guide the Device Management Architecture	52

7.4	Management Generalized Architectural Framework	52

7.4.1	Generalized Architecture	52

7.4.2	Architectural Framework Across the X Reference Point	53

7.4.3	Architectural Framework Across the Y Reference Point	54

Annex A: Guidance for Managing Resource Constrained Devices	56

A.1	Classification of Resource Constrained Devices	56

A.2	Device Classes and Management Technologies	57

Annex <B>: Title of annex	58

Annex <y>: Bibliography	58

History	58

# 1	Scope

The present document describes and collects the state-of-art of the existing technologies on management capability, evaluates if the technologies can match the requirements defined in oneM2M, analyzes how the technologies can leverage the design of the architecture of oneM2M.

Editor’s note: the title of this TR needs further study to e.g. separate the management of application and device.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Not applicable.

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references, (see example).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	OMA-AD-DM-V1_3 “Device Management Architecture”

[i.3]	OMA-TS-DM_Protocol-V1_3 “OMA Device Management Protocol”

[i.4]	OMA-TS-DM_RepPro-V1_3 “OMA Device Management Representation Protocol”

[i.5]	OMA-TS-DM_StdObj-V1_3 “OMA Device Management Standardized Objects”

[i.6]	OMA-TS-DCMO-V1-0: “Device Capability Management Object”

[i.7]	OMA-TS-LAWMO-V1-0: “Lock and Wipe Management Object”

[i.8]	OMA-TS-DM-FUMO-V1-0: “Firmware Update Management Object”,.

[i.9]	OMA-TS-DM-SCOMO-V1-0: “Software Component Management Object”, Version 1.0.

[i.10]	OMA-TS-GwMO-V1-0: “Gateway Management Object Technical Specification”, Version 1.0.

[i.11]	OMA-TS-DiagMonFunctions-1-0: “DiagMon Functions Supplemental Specification”, Version 1.0.

[i.12]	OMA-AD-GwMO-V1_1-20130214-D “Gateway Management Object Architecture”

[i.13]	BBF TR-069 CPE WAN Management Protocol Issue: 1 Amendment 4, July 2011

[i.14]	BBF MR-239 Broadband Forum Value Proposition for Connected Home  Issue: 1, April 2011

[i.15]	BBF TR-232 Bulk Data Collection  Issue: 1, May 2012

[i.16]	TMForum IPDR Service Specification Design Guide, Version 3.8, Release 1.0, 2009

[i.17]	ZigBee Alliance ZigBee Specification: ZigBee Documents 053474r17, 2008

[i.18]	OMA-RD-LightweightM2M-V1_0”OMA Lightweight Machine to Machine Requirement”

[i.19]	OMA-AD-LightweightM2M-V1_0 “OMA Lightweight Machine to Machine Architecture”

[i.20]	OMA-TS-LightweightM2M-V1_0 “OMA Lightweight Machine to Machine Protocol” (work on progress)

[i.21]	Shelby, Z., Hartke, K., Bormann, C., and B. Frank, "Constrained Application Protocol (CoAP)", draft-ietf-core-coap-14 (work in progress), Sept 2012.

[i.22]	Rescorla, E. and N. Modadugu, "Datagram Transport Layer Security Version 1.2", RFC 6347, January 2012.

[i.23]	oneM2M-TS-0002-Requirements-V0_5_2

[i.24]	ETSI M2M TS 103 092 OMA DM compatible Management Objects for ETSI M2M v2.1.1

[i.25]	“Device Management Requirements”, Version 2.0, Open Mobile Alliance™, OMA-RD-DM-V2_0, http://www.openmobilealliance.org/

[i.26]	“Device Management Architecture”, Version 2.0, Open Mobile Alliance™, OMA-AD-DM-V2_0, http://www.openmobilealliance.org/

[i.27]	“OMA Device Management Protocol”, Version 2.0, Open Mobile Alliance™, OMA-TS-DM-V2_0, http://www.openmobilealliance.org/

[i.28]	“Enabler Release Definition for Firmware Update Management Object”, Version 1.0, Open Mobile Alliance™, OMA-ERELD-FUMO-V1_0, http://www.openmobilealliance.org/

[i.29]	“DM Client Side API Framework (DMClientAPIfw)” OMA-ER-DMClientAPIfw-V1_0

[i.30]	C. Bormann and M. Ersue, “Terminology for Constrained Node Networks”, draft-ietf-lwig-terminology-05, July 09 2012.

[i.31]	A. Sehgal, V. Perelman, S Kuryla and J Schonwalder, “Management of Resource Constrained Devices in the Internet of Things”, IEEE Communication Magazine (Vol.50, Issue.12), Dec 2012.

[i.32]	BBF TR-131 ACS Northbound Interface Requirements, Issue:1, November 2009

[i.33]	BBF TR-143 Enabling Network Throughput Performance Tests and Statistical Monitoring, Corrigendum 1, December 2008.

[i.34]	BBF TR-181 Device Data Model for TR-069, Issue 2 Amendment 6, November 2012.

[i.35]	BBF TR-135 Device Data Model for TR-069 Enabled STB, Amendment 3, November 2012.

[i.36]	BBF TR-104 Provisioning Parameters for VoIP CPE, September 2005.

[i.37]	BBF TR-196 Femto Access Point Service Data Model, Issue 2, November 2012.

[i.38]	BBF TR-140 TR-069 Data Model for Storage Service Enabled Devices, Issue 1.1, December 2007.

# 3	Definitions, symbols, abbreviations  and acronyms

## 3.1	Definitions

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

mc: The interface between the management server and the management client. This interface can be realized by the existing device management technologies such as BBF TR-069, OMA DM, etc.

ms: The interface between the management adapter and the management server in the underlying network domain or in the M2M service domain for use by other systems. Using this interface, systems can perform management operations on devices through the management server.

mp: The interface that is exposed by the proxy management client in the area network for devices that connect to a Proxy. This interface is realized by existing LAN based protocols (e.g., ZigBee, UPnP) as well as existing device management technologies (e.g., OMA-DM).

<defined term>: <definition>

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

## 3.4	Acronyms

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

ACS	Auto-Configuration Server

BBF	Broadband Forum

BSS	Business Support System

CoAP	Constrained Application Protocol

CPE	Customer Premises Equipment

CPU	Centralized Processing Unit

CWMP	CPE WAN Management Protocol

DM	Device Management

DTLS	Datagram Transport Layer Security

FTP	File Transfer Protocol

GW	Gateway

HTTP	Hypertext Transfer Protocol

IP	Internet Protocol

IPDR	Internet Protocol Detail Record

IrDA	Infrared Data Association

MO	Management Object

OBEX	OBject EXchange

OMA	Open Mobile Alliance

OSS 	Operation Support System

OTA	Over The Air

PAN	Personal Area Network

RPC 	Remote Procedure Call

SCTP	Stream Control Transmission Protocol

SE	Service Element

SIP	Session Initiation Protocol

SOAP	Simple Object Access Protocol

SMS	Short Message Service

SSL	Secure Session Layer

TCP	Transmission Control Protocol

TMForum	Telemanagement Forum

TR	Technical Report

TLS	Transport Layer Security

UDP	User Datagram Protocol

UPnP DM	Universial Plug and Play Device Management

UI	User Interaction

WAP	Wireless Application Protocol

WSP	Wireless Session Protocol

XDR	External Data Representation

XML	Extensible Markup Language

ZC	ZigBee Coordinator

ZDO	ZigBee Device Object

ZED	ZigBee End Device

ZR	ZigBee Router

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# 5	Introduction of existing technologies

## 5.1	 Introduction to OMA DM

Editor’s Note: The chapter is to introduce certain kind of device management technology. All the description in the chapter is the introduction to the currently existing technology. The relation between the technology and oneM2M is not covered in this section.

### 5.1.1	Description

Editor’s Note: The chapter is to give description to the technology

OMA DM is a protocol for device management designed by Open Mobile Alliance. It’s widely used in the remote management of mobile devices. It is composed of a number of specifications including protocol, architecture, underlying network binding etc. In the most common scenario, by implementing OMA DM specifications, the DM Server is able to do remote management on devices with DM Clients which are usually mobile phones. The devices could also include sensors, actuators, and gateways as well. With implementing the Management Object and the DM Client, the DM Server can perform remote management on devices such as provisioning, diagnostics, firmware upgrade, and remove, install, activate software components.

Figure 5.1.1: OMA DM Use Case

As is shown from Figure 5.1.1, the user of a mobile phone doesn’t know what to do when his mobile is unable to send out MMS. After calling to the Call Center, the operator of the Call Center can remotely upgrade the MMS configuration file via OMA DM Server.

Figure 5.1.2: Device Management Protocol

OMA DM protocol deploys management between a DM Client and a DM Server as Figure 5.1.2. DM Server can send DM commands to DM Client to manage the device. The DM Client can also send command to DM Server to indicate the result corresponding to the commands from DM Server. A group of tree structured Management Objects are used to manage the device.

### 5.1.2	Architecture

Editor’s Note: The chapter is to give the high level architecture of the technology and the corresponding description.

DM Server2

DM Server1

DM-6

DM Client

DM-1

DM-2

Smart Card

OTA Provisioning Server

CP Enabler

DM-3

DM-4

CP-1

DM Gateway

DM-1

DM Enabler

DM-1

Local Application

DM-7,8,9

Figure 5.1.3: Architecture and Reference Points

The architecture of OMA Device Management Enabler is shown in Figure 5.1.3 [i.2] . Functional components which are DM Server and DM Client compose the DM Enabler. Components Smart Card, OTA Provisioning Server and CP Enabler are outside of the DM Enabler. They are used to bootstrap the DM Client.

DM Server can also manage a device with DM Client through a DM Gateway. DM Gateway can be deployed in DM-1 interface in Transparent Mode, Proxy Mode or Adaption Mode [i.12].

### 5.1.3	Reference points

Editor’s Note: The chapter is to introduce the reference point.

#### 5.1.3.1	Introduction

This clause introduces the interfaces carried over the reference points between DM Server, DM Client, Smart Card, OTA Provision Server and CP Enabler. Also the procedures of packages exchanged via these interfaces are also briefly introduced.

#### 5.1.3.2	DM-1 DM Client-Server Notification

The DM-1 interface provides the ability for the DM Servers to send device management notifications to the DM Clients. Because devices with DM Clients may not be able to continuously listen for connection all the time, DM Server may send notifications to DM Client to start a DM session. More details can be referred to [i.2]

#### 5.1.3.3	DM-2 DM Client-Server Protocol

The interface provides the ability for the DM Servers and DM Clients to exchange DM commands and corresponding responses. The interface can be bound to different underlying protocols including HTTP and HTTPS. More details can be referred to [i.2].

#### 5.1.3.4	DM-3 DM Bootstrap Profile via Smart Card

Bootstrap via Smart Card is one way to provision a DM Client. The DM Client gets all the related configuration settings from the Smart Card. More details can be referred to [i.2].

#### 5.1.3.5	DM-4 DM Bootstrap Profile OTA

Bootstrap via push protocol over the air can provision necessary configuration setting file to DM Client. The file contains a series of DM Commands. More details can be referred to [i.2].

#### 5.1.3.6	CP-1 CP Bootstrap Profile

Bootstrap via CP enabler can provision necessary configuration setting file to DM Client. The file contains a series of DM Commands. More details can be referred to [i.2].

#### 5.1.3.7	DM-6 DM Server-Server Interface

DM Server-Server Interface enables one DM Server delegate the management of a device to another DM Server. More details can be referred to [i.2].

#### 5.1.3.8	DM-7,8,9 Client API

DM-7 is the interface that enables the local application of a device to register or deregister Management Object to the DM Client. [i.29]

DM-8 is the interface that enables the DM Client to send Management Object update notifications to local application. [i.29]

DM-9 is the interface that enables the local application to send Management Object manipulation and retrieve commands to the DM Client. [i.29]

Local application resides in the same execution environment with the DM Client.

#### 5.1.3.9	Procedures

Server

Client

Package 0: alert from the DM Server

Package 1: client initialization with client credentials and device information

Setup phase

Package 2: server initialization with server credentials, initial management operations or user interaction commands from the server

Server

Client

Management phase

Package 3: client response to server management operations

Package 4: more user interaction and management operations if the session is continued.

Figure 5.1.4: DM Phases

The interaction between Client and Server is achieved by Packages. OMA DM Protocol consists of two parts: setup phase (authentication and device information exchange) and management phase. Management phase can be repeated as many times as the DM Server wishes. The setup phase is composed of Pachage#0, Package#1 and Package#2. The management phase is composed of Package#3 and Package#4 as shown in Figure 5.1.4[i.3].

### 5.1.4	Protocols

Editor’s Note: The chapter is to give the protocol stack used between the reference points.

#### 5.1.4.1	Protocol Stack

Application MO

DM Protocol

DM Representation

Binding to transports

Transports

Figure 5.1.5: Protocol Stack

As shown in Figure 5.1.5, the protocol stack of OMA DM is composed of five layers which are Application MO, DM Protocol, DM Representation, Binding to transports and Transports.

#### 5.1.4.2	Application MO

The Management Object is built on top of the DM Protocol to be transferred to fulfil the management of devices. MO is implemented in the device with DM Client and DM Server to carry on the management. DM Server manages the device by operation to the MO through DM Client. The introduction to the MOs will be shown in chapter 5.1.5.

#### 5.1.4.3	DM Protocol

DM Protocol is the Packages exchanged between the entities of OMA DM. As is described in the reference point part, OMA DM uses these Packages to exchange the MO between DM Client and DM Server.

#### 5.1.4.4	DM Representation

OMA DM uses DM representation syntax and semantics for device management. The DM representation is carried in the XML formatted DM Messages between OMA DM entities. The DM representation protocol also can be identified as a MIME content type.

The DM representation protocol is performed in a request/response way using the concept of DM Package. The concept of DM Package is shown in the Procedure chapter. It’s used to carry the device management operations.

A DM Message is a well-formed XML document and adheres to the DTD.

OMA DM uses SyncML as the container for the DM Message. SyncML was first designed and used by OMA CP, and was reused by OMA DM. SyncML provides a set of tags and syntaxes to mark up the language to be understandable to both DM Clients and DM Servers.

Details can be referred to [i.4].

#### 5.1.4.5	Binding to transports and Transports

TCP/IP

IrDA

WAP

OBEX

SIP

Push OTA

DM Message

Push OTA

HTTP

WSP

Figure 5.1.6: OMA DM Transports

OMA DM provides the following ways for transporting DM Messages which are HTTP, OBEX, WSP, SIP and Push OTA as shown in Figure 5.1.6 OMA DM Transports.

### 5.1.5	Functions

Editor’s Note: The chapter makes a list of all the functionalities supported by the technology and describes how the function is achieved.

#### 5.1.5.1	Introduction

The device management functionalities are achieved by the Management Objects defined by OMA DM and some other third party organizations.

#### 5.1.5.2	The MO tree

Figure 5.1.7: MO tree

OMA DM uses Management Object to management the device. The MOs forms a tree structure and the tree is stored with the DM Client. Each MO in the tree is a node. If the MO has child Nodes, the MO is an Interior Node. Otherwise, the MO is a Leaf Node. Nodes in the Management Tree can be either permanent or dynamic.

Permanent Nodes are typically built in at device manufacture. Permanent Nodes can also be temporarily added to a device by, for instance, connecting new accessory hardware. A DM Server cannot modify permanent Nodes at run-time.

Dynamic Nodes can be created and deleted at run-time by DM Servers. DM Server use Add and Delete command to create or delete Dynamic Nodes. If the deleted Dynamic Nodes is a Interior Node, all the related Nodes which are the children of the Interior Node shall also be deleted.

##### 5.1.5.2.1	Standard Objects

The MOs that shall be supported by DM Client and DM Server are standard objects. Standard objects expose basic information of the DM Client for the DM Server to perform managements.

| Management Object | Reference | Description |
| --- | --- | --- |
| DMAcc | [i.5] | Settings for the DM client in a managed device. |
| DevInfo | [i.5] | Device information for the OMA DM server. Sent from the client to the server. Needed by the DM Server for problem free operation of the DM protocol. |
| DevDetail | [i.5] | General device information that benefits from standardization. DevDetail contains parameters that are manipulated by the server for the operation purposes. |
| Inbox | [i.5] | Reserved URI where the device SHOULD use the management object identifier to identify the absolute URI. |

Table 5.1.1: Standard Objects

##### 5.1.5.2.2	Other Management Objects

Besides the Standard Objects, there may be other Management Objects to carry on further management functionalities as well. MOs that are considered as relevant to the management of M2M Devices or Gateways are listed in Table 5.1.2.

| Management Object | Reference | Description |
| --- | --- | --- |
| SCOMO | [i.9] | Device information collection, remote configuration, software management |
| DIAGMON | [i.11] | Diagnostics and monitoring |
| GwMO | [i.10] | Managements to devices through gateway. |
| FUMO | [i.8] | Firmware update |
| DCMO | [i.6] | Specify the mechanisms required for the remote management of device capabilities. |
| LAWMO | [i.7] | The MO is designed to protect user and enterprise-related data by means including Lock/Unlock Device, Wipe Device’s Data and Factory Reset |
| <TBD> | <TBD> | <TBD> |

Table 5.1.2: Other MOs

Editor’s note: Table 5.1.2 is to be further populated.

## 5.2	TR-069 Family of Specifications

### 5.2.1	Description

The Broadband Forum has developed a series of specifications that have been termed the TR-069 Family of Specifications. These specifications provides the capability to manage CPEs within the connected home. MR-239 Broadband Forum Value Proposition for Connected Home [i.14] provides an overview of the value proposition for utilizing the TR-069 family of specifications for the connected home.

### 5.2.2	Architecture

The TR-069 family of specifications is anchored by the TR-069 [i.13] specification for the CPE WAN Management Protocol (CWMP) protocol. TR-143 [i.33] for diagnostic tests. TR-131 [i.32] for requirements related to the ACS North Bound Interface.

In addition Service Providers are increasingly interested in retrieving large quantities of data from their installed CPE base at regular intervals. The amount of data being requested represents a significant portion of the CPE’s data model and is thus a large amount of data. In response to this, the Broadband Forum has documented a data collection solution in TR-232 Bulk Data Collection [i.15]. This specification is based on the IPDR protocol from the TMForum.

Devices within the Connected Home are managed via a set of data models for CWMP Enabled Devices. These data models are anchored by TR-181 [i.34] which defines the objects and attributes for management for most capabilities offered by a device (e.g., physical interfaces, bridging and routing, firewalls, NAT, software modules). Likewise services and capabilities specific to a type of device are included in a separate set of specifications. For example, CWMP enabled STB are managed using TR-135 [i.35]; Femto cell devices are managed using TR-196 [i.38]; VoIP capable devices are managed using TR-104 [i.36]. Not all devices within the Connected Home are CWMP enabled; in this situation TR-069 provides the capability for a CWMP enabled device to act a proxy for the device.

Figure 5.2.1: TR-069 Family of Specifications

#### 5.2.2.1	TR-069 Proxy Management

CWMP can be extended to devices that do not have a native CWMP Endpoint of their own, but instead support management of devices with another management protocol or “Proxy Protocol”.  A CPE Proxier is a CPE that supports a CWMP Endpoint(s) and also supports one or more Proxy Protocols (example services include UPnP DM, Z-Wave etc.). A CPE Proxier uses these Proxy Protocols to manage the devices connected to it, i.e. the Proxied Devices. This approach is designed to support Proxy Protocols of all types that can exist in the CPE network now or in the future. Annex J of the TR-069 [i.13] provides an overview of CWMP Proxy Management.

Figure 5.2.2: Proxy management terminology

##### 5.2.2.1.1	Proxied Device Deployment Archirecture

Figure 5.2.4: TR-069 UPnP DM Proxied Device depicts an example scenario where a proxied device that supports the UPnP DM protocol is managed using CWMP.

Figure 5.2.3: TR-069 UPnP DM Proxied Device

The entities include the Service Provider OSS/BSS systems that interface with the ACS (1); the CWMP and IPDR protocols between the ACS and the TR-069 enabled CPE (2) and the Home Area Network protocol UPnP (3).

### 5.2.3	Reference points

The CWMP enabled devices in the Connected Home typically communicates with three (3) entities, the ACS, OSS/BSS and devices within the Connected Home via standardized reference points.

These references points are defined as:

- ACS to CPE

- CPE to BSS

- CPE to Device

Figure 5.2.4: TR-069 Reference Points

### 5.2.4	Protocols

#### 5.2.4.1	ACS to CPE Protocol

The protocol that is supported on the ACS to CPE reference point is CWMP as defined in  BBF TR-069 [i.13]. CWMP takes a layered approach to the protocol based on several standard protocols for transport and exchange of messages. The protocol stack defined byCWMP is shown in Figure 5.2.5. A brief description of each layer is provided in Table 5.2.1.

Figure 5.2.5: Protocol stack

| Layer | Description |
| --- | --- |
| CPE/ACS Application | The application uses the CPE WAN Management Protocol on the CPE and ACS, respectively. The application is locally defined and not specified as part of the CPE WAN Management Protocol. |
| RPC Methods | The specific RPC methods that are defined by the CPE WAN Management Protocol. These methods are specified in CPE WAN Management Protocol. |
| SOAP | A standard XML-based syntax used here to encode remote procedure calls. Specifically SOAP 1.1, as specified in Simple Object Access Protocol (SOAP) 1.1. |
| HTTP | HTTP 1.1, as specified in RFC 2616, Hypertext Transfer Protocol -- HTTP/1. |
| TLS | The standard Internet transport layer security protocol. Specifically, TLS 1.2 (Transport Layer Security) as defined in RFC 5246, The Transport Layer Security (TLS) Protocol, Version 1.2 (or a later version). Note that previous versions of this specification referenced SSL 3.0 and TLS 1.0. |
| TCP/IP | Standard TCP/IP. |

Table 5.2.1: Protocol layer summary

#### 5.2.4.2	CPE to BSS

The protocol that is supported on the CPE to BSS reference point is the IPDR protocol. The IPDR reference architecture is presented in Figure 5.2.6 is defined in TMForum IPDR Service Specification Design Guide [i.16]. The figure depicts a Service Element communicating to an IPDR Recorder that sends messages to the IPDR Transmitter and optionally to an IPDR Store. TR-232[i.15] utilizes the A and D interfaces of this specification where the Service Element is a device within the Connected Home.

Figure 5.2.6: IPDR Reference Architecture

From the perspective of the Broadband Forum, the CPE or device is the Service Element and IPDR Exporter. The IPDR Data Collector is the BSS.  As described in Annex A IPDR Theory of Operaton of TR-232[i.15], the IPDR documentation clarifies that the following scenario, where the Service Element directly communicates to the BSS, is valid and simply means that the IPDR Recorder and IPDR Transmitter (collectively the IPDR Exporter in this use case) are all incorporated into the Service Element. The Service Element is permitted to directly interface with the BSS if it supports the “D” interface specifications including backing stores and retransmission of IPDR documents.

Figure 5.2.7: Simplified IPDR Architecture

##### 5.2.4.2.1	IPDR Reference Points

TR-232[i.15] defines 6 interfaces and 4 defintions for the IPDR Reference Model:

| Interface | Description |
| --- | --- |
| A | Vendor proprietary. High-volume with high granularity void of context. This interface is not part of the IPDR Protocol. |
| B | IPDR Data Interface. From IPDR Recorders to IPDR Stores or IPDR Transmitters. |
| C | IPDR Store Export Interface. |
| D | BSS Interface. XML or XDR data from IPDR Exporter to IPDR Collector |
| E | Settlement Interface. Connects Service Delivery Business Management Systems. |
| F | Financial System Interface. This interface is not part of the IPDR Protocol. |

Table 5.2.2: IPDR Interfaces

The IPDR File Transfer Protocol uses FTP or HTTP to transfer files that contain IPDR records from the SE to the BSS.  The IPDR Streaming Protocol uses SCTP or TCP to transfer IPDR records from the SE to the BSS using highly efficient XDR encoding as described in the IPDR/XDR Encoding Format document or an XML encoding as described in the IPDR/XML File Encoding Format document.

#### 5.2.4.3	CPE to Device Protocol

The TR-069 proxy mechanism is designed to incorporate any protocol for area networks within the customer premises. The following protocols have been standardized or are currently in development:

- UPnP DM

- ZigBee

##### 5.2.4.3.1	UPnP DM Proxy

The CPE Proxier consists of three logical modules: CWMP client, TR-069/UPnP DM Proxy Module and UPnP DM Control Point. CWMP requests received by the CWMP client from the ACS are translated by the TR-069/UPnP DM Proxy Module to the UPnP DM actions, and then passed to the UPnP DM Control Point to be sent to the UPnP DM devices. When an UPnP action response or event is received by the UPnP DM Control Point, the action response and event is passed to the TR-069/UPnP DM Proxy Module to be converted to a CWMP response or sent to the ACS using the CWMP event notification mechanism.

Figure 5.2.8: TR-069/UPnP DM Proxy Management Architecture

##### 5.2.4.3.2	ZigBee Proxy

Figure 5.2.9 and Figure 5.2.10 present the principle and an example basic sequence for the management of ZigBee devices by using TR-069 with the ZigBee data model.

The ZigBee devices reside behind a GW and communicate with the ACS via this GW. The GW resides normally in a CPE such as a broadband router (home gateway or business gateway). The GW has a proxy function to change a CWMP message to a ZDO function invocation based on the ZigBee data model object. The proxy function changes messages by referring to a mapping of ZigBee data model objects and CWMP methods to ZDO functions and their parameters. A management example is shown in Figure 5.2.10.

Figure 5.2.9: Usage of the data model to manage ZigBee devices with TR-069

Figure 5.2.10: Example sequence diagram of ZigBee management with TR-069

This example shows how the ACS gets a ZigBee device’s network address by using TR-069 communication based on the ZigBee data model. The ACS sends a CWMP message which includes the “GetParameterValues” as a method and the part of the ZigBee data model “Device.ZigBee.ZDO.{i}.NetworkAddress”, which refers to the network address, as a parameter name. The proxy function in the GW changes the received message to a ZDO handling message to call some ZDO function on the ZC. The ZC manages the ZigBee devices according to the called ZDO function and sends the result (the searched network address, in this case) to the proxy. The proxy function changes the ZDO management result to a CWMP message which is denoted in Figure 5.2.10 as “GetParameterValuesResponse”. The name of the parameter list is “Device.ZigBee.ZDO.{i}.NetworkAddress” and the value of the parameter list is “0x0fE3” (network address instance).

### 5.2.5	Functions

The TR-069 family of specifications is intended to support a variety of functionalities to manage a collection of devices, including the following primary capabilities:

- Auto-configuration and dynamic service provisioning

- Software/firmware image management

- Software module management

- Status and performance monitoring

- Diagnostics

- Proxy Management

- Bulk data collection

## 5.3	Introduction to OMA LightweightM2M (LWM2M)

### 5.3.1	Description

OMA Lightweight M2M is a protocol for device and service management for M2M. The main purpose of this technology is to address service and management needs for constrained M2M devices, over a number of transports and bearers. The crucial aspects in this work are:

- The target, that is a variety of devices “constrained” at very different levels

- Data collection and remote controlling without complex computing and UI operations

- The optimization of network resources, as a very large numbers of devices may be connected to the communication network simultaneously

- The fusion of device functionalities management and service manipulation in a single protocol

From the implementation view, the LWM2M has the following features:

- Suitable for really resource constraint devices

- Compact binary packet

- Support Binary Data Format and JSON format

- Easy to be implemented (reusing implementation of IETF technologies)

One of typical use cases of using LWM2M technology is a firmware upgrade of streetlight [i.18].

- A Streetlights supervisor is responsible to manage streetlights system. (There are thousands of streetlights in the city and low-cost LWM2M Devices embedded in the streetlights.)

- The supervisor needs to remotely upgrade of the firmware of a specific streetlight or a group of streetlights.

Figure 5.3.1: Firmware Upgrade of Streetlight of Use Case using LWM2M

### 5.3.2	Architecture

Figure 5.3.2: LWM2M Architecture

As shown in the Figure 5.3.2, the layout is the architecture of LWM2M [i.19]. The Components specified by OMA LWM2M compose the LWM2M enabler which specifies the LWM2M Server / LWM2M Client interface. LWM2M Server and LWM2M Client are typically instantiated in an M2M Server and an M2M Device.

Based on the deployment scenario, the LWM2M Server has the bootstrapping capability itself, or the LWM2M Bootstrap Server exists separately for security reasons.

### 5.3.3	Reference Points

This section introduces the interfaces carried over the reference point consisting of two main components LWM2M Server and LWM2M Client.

#### 5.3.3.1	Functional Components

##### 5.3.3.1.1	LWM2M Server

The LWM2M Server is a logical component which serves as an endpoint of the LWM2M protocol.

##### 5.3.3.1.2	LWM2M Client

The LWM2M Client is a logical component. This LWM2M Client serves as an endpoint of the LWM2M protocol, and communicates with the LWM2M Server to execute the operations from the LWM2M Server and report results of the operations for the device management and the service enablement.

#### 5.3.3.2	Interfaces

There are four interfaces supported by the reference point between LWM2M server and LWM2M Client. The logical operation of each interface is defined as follow.

- Device Discovery and Registration

This interface allows the LWM2M Client do the Device Registration. This procedure lets the Server know the existence and information (e.g., capability) of the LWM2M Device so that LWM2M Server can perform M2M services and device management on the LWM2M Client.

- Bootstrap

This interface is used to initial parameters and configurations on the LWM2M Client so that the Client can register to the Server after bootstrap procedure has completed.

- Device Management and Service Enablement

This interface allows the LWM2M Server to perform the device management and M2M service enablement. Over this interface, the LWM2M Server can send operations to the Client and gets response of the operations from the Client.

- Information Reporting

This interface allows the LWM2M Client to report resource information to the LWM2M Server. This Information Reporting can be triggered periodically or by events.

### 5.3.4	Protocols

#### 5.3.4.1	Protocol Stack

The LWM2M has the protocol stack defined as below.

Figure 5.3.3: LWM2M Protocol Stack

- LWM2M Objects: LWM2M Objects are designed for each functionality the LWM2M enabler provides. The LWM2M specification [i.20] provides Standard Objects. Other Objects may be added by OMA, external SDOs or vendors to enable certain M2M Services. (E.g. the IPSO alliance has already started developing LWM2M Objects.)

- LWM2M Protocol: LWM2M protocol defines logical operations and mechanisms per each interface.

- CoAP: The LWM2M utilizes the IETF Constrained Application Protocol [i.21] as an underlying transfer protocol across UDP and SMS bearers. This protocol defines the message header, request/response codes, message options, and retransmission mechanisms. The LWM2M only uses the subset of features defined CoAP.

- DTLS: DTLS [i.22] is used to provide security channel between the LWM2M Server and the LWM2M Client for all the messages interchanged.

- UDP Binding with CoAP (Mandatory): NoSec mode and reliability over the UDP transport is provided by the built-in retransmission mechanism of CoAP.

- SMS Binding with CoAP (Optional): CoAP is used over SMS by placing a CoAP message in the SMS payload using 8-bit encoding.

#### 5.3.4.2	Resource Model

In the LWM2M Enabler technical specification [i.20], a simple resource model is described. Basically, a resource made available by resource model of the LWM2M Client is a Resource, and Resources are logically organized into Objects. Figure 5.3.4 illustrates this structure, and the relationship between Resources, Objects, and the LWM2M Client. The LWM2M Client may have any number of Resources, each of which belongs to an Object.

Figure 5.3.4: LWM2M Resource Model [i.20]

Resources are defined per Object, and each resource is given a unique identifier within that Object. Each Resource is designed to have one or more Operations that it supports. A Resource may contain multiple instances dependent on the Resource definition in Object specification.

An Object defines a grouping of Resources, for example the Firmware Object contains all the Resources used for firmware update purposes. The LWM2M enabler defines standard Objects and Resources and other Objects may be added to enable a certain M2M Services.

Object shall be instantiated either by the LWM2M Server or the LWM2M Client, which is called Object Instance before using the functionality of an Object. After Object Instance is created, the LWM2M Server can access that Object Instance and Resources in the Object Instance.

Editor’s Note: Support of linking a resource to another object is currently under discussion in OMA.

#### 5.3.4.3	Interface Descriptions

##### 5.3.4.3.1	Device Discovery and Registration

The Device Discovery and Registration Interface are used by a LWM2M Client to register with one or more LWM2M Servers, maintain each registration, and de-register from the Server(s). When registering, the LWM2M Client indicates its Endpoint Name, Client registration lifetime, the list of Objects the Client supports and available Object Instances. The registration is soft state, with a lifetime indicated by the registration lifetime. The LWM2M Client periodically performs an update of its registration information to the registered Server(s). If the lifetime of a registration expires without receiving an update from the Client, the Server removes the registration. Finally, when shutting down or discontinuing use of a Server, the Client performs de-registration.

Figure 5.3.5: Example of Registration Procedure

##### 5.3.4.3.2	Bootstrap

Bootstrap process is used to provision essential information into the LWM2M Client to make the LWM2M Client be able to register to a certain LWM2M Server. There are four modes for bootstrapping.

- Manufacturer pre-configuration: the LWM2M Client is already provisioned at manufacturer stage. The pre-configuration data can be stored in a storage of the LWM2M Client.

- SmartCard provisioning: When the Device supports a SmartCard and retrieval of bootstrap message from SmartCard is successful, the LWM2M Client processes the bootstrap message from the SmartCard and apply it to the Device configuration

- Client initiated Bootstrap: the LWM2M Client retrieves the bootstrap message from a LWM2M Bootstrap Server in this case the LWM2M Client needs to be pre-provisioned at the manufacturer stage with a bootstrap Server URI.

- Server initiated Bootstrap: the LWM2M Server provision the bootstrap message to the LWM2M Client after recognizing the existence of the LWM2M Device.

Figure 5.3.6: Four Modes for bootstrapping

##### 5.3.4.3.3	Device Management and Service Enablement

This interface is used by the LWM2M Server to access Resources available from a LWM2M Client using Create, Read, Write, Delete, or Execute operations. The operations that a Resource supports are defined in the definition of its Object.

Figure 5.3.7: Example of Device Management and Service Enablement Interface

##### 5.3.4.3.4	Information Reporting

This interface is used by a LWM2M Server to observe any changes in a Resource on a LWM2M Client, receiving notifications when new values are available. (Note: this procedure is similar to the operation of notification procedure in M2M service) This observation relationship is initiated by sending an Observe operation to the L2M2M Client for an Object Instance or Resource. An observation ends when a Cancel Observation operation is performed.

Figure 5.3.8: Example of Information Reporting Interface

### 5.3.5	Functions

A first set of standard Objects for the LWM2M 1.0 enabler have been developed. The Standard Objects are intended to support a variety of functionalities to manage LWM2M Devices. OMA might create further objects in future. Furthermore, other organizations and companies may define additional LWM2M Objects for their own M2M services using according to LWM2M Object Template and Guideline Annex in [i.20].

- Server: data, configuration and security data related to a LWM2M server

- Access Control: to check whether the LWM2M server has access right for performing an operation

- Device: provision of a range of device related information, device reboot and factory reset function

- Connectivity: provision of management of parameters related to underlying network connectivity

- Firmware: provision of firmware management, installing and updating new firmware

- Location: provision of location information of the LWM2M devices

## 5.4	Introduction to OMA Device Management 2.0

### 5.4.1	Description

OMA Device Management 2.0 is the new evolution of the OMA Device Management 1.x Protocols, and defines the interface between the DM Server and the DM Client to manage devices. Compared to OMA DM 1.x Protocols, the unique features of OMA DM 2.0 are as follows:

- Protocol Simplification and Optimization – Transaction model, DM Packages, addressing schemes and security model are simplified and optimized as well for the fast market penetrations. Simplifications and optimization is the fundamental spirits that goes throughout OMA DM 2.0.

- Extended DM Command Set – OMA DM 2.0 supports 10 DM commands, that is much extended compared to OMA DM 1.x. For example, the HGET/HPUT/HPOST commands are specified to utilize the RESTful interface, and the SHOW command is specified for the web-based user interaction.

- Management Data Delivery using HTTP (RESTful interface) – The DM Client can retrieve or send the management data from or to the Data Repository using the HTTP protocol. This enables the effective management data delivery.

- Web-based User Interaction – The DM Server can utilize web pages to interact with the user. The Web Browser Component and the Web Server Component are newly introduced, and these two components can perform the user interaction session, which is independent with the DM session. This feature brings the rich user interactions to OMA DM 2.0.

- JSON Representation for DM Packages – In OMA DM 2.0, JSON replaces XML. The new JSON format for DM Packages lightens the parsing overheads and shortens the DM Package length keeping the same degree of the extensibility.

- New Addressing Scheme – OMA DM 2.0 introduces the new addressing scheme that uniquely addresses each node based on the Management Object Instance. In OMA DM 2.0, Management Objects do not need to be organized as a tree, and the DM Server does not need to have the knowledge of the DM Tree that can be different for each type of devices.

OMA DM 2.0 is not backward compatible with OMA DM 1.x. This is because OMA DM 2.0 uses the completely different representation based on JSON, and eliminates complex functionalities that are not required by the market. Although OMA DM 2.0 is not backward compatible with OMA DM 1.x, Management Objects (e.g., FUMO, SCOMO, DiagMon, LAWMO) designed for OMA DM 1.x can be still used for OMA DM 2.0. This is possible since OMA DM 2.0 uses the same Management Object definition and provides the necessary functionalities such as Generic Alerts. The complete separation between Management Objects and the OMA DM Protocol is one of the success factors as well.

Note that the OMA DM 2.0 is in the draft version, and the Candidate Approval is expected at the 15th Oct 2013.

### 5.4.2 	Architecture

The following figure shows the OMA DM 2.0 Architecture and the related components and the interfaces:

Figure 5.4.1: OMA DM 2.0 Architecture

The Web Server Component and the Web Browser Component can be utilized for the user interactions. For the user interaction, OMA DM 2.0 specifies that the DM Server can send the SHOW command; therefore, the DM Client initiates the Web Browser Component with the specified web page. The actual user interaction can be performed using the HTTP/HTML, which is out-of-scope of the OMA DM 2.0. Please note that how to initiate the Web Browser Component, and how the transfer the user interaction results are out-of-scope of OMA DM 2.0 as well.

Using the Data Repository, the DM Client can retrieve and upload the management data from or to the Data Repository using the HTTP protocol. The DM Server can send the HPUT/HPOST and HGET command for those purposes.

### 5.4.3 	Reference Points

#### 5.4.3.1 	Functional Components

##### 5.4.3.1.1 	DM Client

The DM Client is the logical software component that conforms to the requirements for DM Clients specified in OMA DM 2.0.

##### 5.4.3.1.2 	DM Server

The DM Server is the logical software component that conforms to the requirements for DM Servers specified in OMA DM 2.0. The DM Server is also responsible for providing the Bootstrap Message using the DM-3 Interface.

##### 5.4.3.1.3 	Web Server Component

Web Server Component is the logical software component responsible to host web pages for the Web Browser Component in the device. The web pages are used for the user interactions.

##### 5.4.3.1.4 	Web Browser Component

Web Browser Component is the logical software component responsible for retrieving web pages from the Web Server Component and presenting them to the user.

##### 5.4.3.1.5 	Data Repository

Data Repository is the logical software component that the DM Client can retrieve or send management data to or from this component by using HTTP.

#### 5.4.3.2 	Interfaces

The brief explanations for the defined interfaces are as follows:

- DM-1 Notification – the interface over which the DM Server sends DM Notification to the DM Client to initiate the DM session.

- DM-2 Device Management – the interface over which the DM Server sends device management commands to the DM Client and the DM Client returns status code, results and Alerts.

- DM-3/4 Bootstrap – the interface over the Bootstrap Message is delivered. OMA DM 2.0 supports factory bootstrap, client-initiated bootstrap, server-initiated bootstrap and SmartCard bootstrap.

### 5.4.4	Protocol

#### 5.4.4.1 	DM Packages

The below figure describes the DM Package exchanges between the DM Client and the DM Server defined in OMA DM 2.0:

Figure 5.4.2: OMA DM 2.0 Package Flow

- Step 0 (DM Notification): The DM Server requests the DM session by sending the DM notification to the DM Client.

- Step 1 (DM Session Initiation): This DM package initiates the DM session and might contain information for the supported Management Object and Generic Alerts generated by the DM Client.

- Step 2 (Request): The DM Server sends the DM commands to the DM Client.

- Step 3 (Command Processing): The DM Client processes the DM commands received. To process the DM command, the DM Client might interact with components such as the Web Browser Component, the Data Repository.

- Step 4 (Response): Unless the END command is received, the DM Client responses to the DM Server. It contains the results for the DM command and Generic Alerts generated by the DM Client.

#### 5.4.4.2 	DM Commands

The below table shows the DM commands specified in the OMA DM 2.0:

| Logical Op. | Name | Description | DM 1.x Relation |
| --- | --- | --- | --- |
| Read | GET | To retrieve data from the device. The data is included in Package#3. | Get |
|  | HPUT | To request the device to send data to the Data Repository using HTTP PUT. | MO individually implements this |
|  | HPOST | To request the device to send data to the Data Repository using HTTP POST. |  |
| Write | DELETE | To delete data in the device. | Delete |
|  | HGET | To requests the DM Client to retrieve data from the Data Repository using HTTP GET, and add or replace the received data into the device. | Add, Replace |
| Exec | EXEC | To execute an executable node. | Exec |
| Not related to MO | SHOW | To initiate a UI session between the Web Browser Component and the Web Server Component. | Alert for UI. Only 5 UI types exist |
|  | CONT | To continue the DM session with the specified DM Server URI. | <RespURI> element |
|  | END | To terminate the DM session. | <Final> element |
|  | DEFAULT | To use a specific address to capture configuration if that is missing in the device for a specific MOID. | None |

Table 5.4.1: OMA DM 2.0 Commands

### 5.4.5	Functions

#### 5.4.5.1	Introduction

Each device that deploys OMA DM 2.0 supports Management Objects. Management Object is the set of related nodes for a specific function, and OMA DM 2.0 achieves management functions by using the Management Objects. The type of the Management Object is defined by the MOID.

For example, for the firmware management functions, the Firmware Update Management Object as specified in [i.4] can be used as shown below:

Figure 5.4.3: Firmware Update Management Object

OMA DM 2.0 does not require that Management Objects in the device are organized as a hierarchical tree structure since the nodes are addressed based on the Management Object instance. In the device, Management Object is realized as a Management Object instance. Once the Management Object instance is created in the device, the DM Client assigns the MO Instance Identifier (MIID) to it, and the DM Server can use the ClientURI to uniquely identify each node in the Management Object Instance. Note that ClientURI is built based on the MOID and MIID.

Suppose that the above FUMO is realized as an instance in the device. Then the DM Server can use below Client URIs:

- urn:oma:mo:oma-fumo:1.0/fumo1/PkgName  – to identify the <x>/PkgName node

- urn:oma:mo:oma-fumo:1.0/fumo1/DownloadAndUpdate/PkgURL – to identify the <x>/DownloadAndUpdate/PkgURL node

The “fumo1” in the above ClientURI is the MIID assigned to the FUMO instance, and can be omitted if the MOID is enough to uniquely identify a Management Object Instance (i.e., there is only one Management Object Instance for the MOID). Hence, the ClientURI can be simplified further to “urn:oma:mo:oma-fumo:1.0/ /PkgName” if MIID is not needed.

#### 5.4.5.2	Management Objects supported by OMA DM 2.0

OMA DM 2.0 shares the same Management Object definitions, which allows that Management Objects designed for OMA DM 1.x can be reused for OMA DM 2.0. Hence, every MO identified in the section 5.1.5.2.2 that is considered as relevant to the management of M2M Device/Gateway are supported by the OMA DM 2.0 (e.g., FUMO, SCOMO, Gateway MO, etc.). In addition, OMA DM 2.0 supports below standard MOs:

| Standard MO | Description |
| --- | --- |
| DevInfo MO V1.2 | - This MO provides the basic device information such as the device identifier, manufactures, etc. |
| DM Account MO V2.0 | - This MO provides the account information for the DM Servers. |
| Delegation Access Control MO V1.0 | - This MO provides the information for the access control. |
| Session Info MO V1.0 | - This MO provides the information for the DM session. |

Table 5.4.2: Standard Management Object for OMA DM 2.0

#### 5.4.5.3	Detailed Comparisons with OMA DM 1.x

Below table explains the differences between OMA DM 2.0 and OMA DM 1.x from the overall perspective:

| Label | DM 2.0 | DM 1.x |
| --- | --- | --- |
| Package Representation | - JSON | - XML |
| DM Command | - 10 DM commands. - Sequential processing only | - 6 DM commands. - Random, Sequential, Atomic processing |
| MO Addressing | - ClientURI | - Absolute, Relative, Virtual |
| User Interaction | - Web-based User Interaction using SHOW command | - User Interaction Alert supports 5 types (Display, Confirmation, User Input, Single Choice, Multi Choice) |
| Security | - Authorization – ACL for MO instance granularity (can be extended to node granularity) - Confidentiality/Authentication – transport layer security only | - Authorization – ACL for node granularity - Confidentiality/Authentication – DM Protocol security or transport layer security |
| DM Notification | - TLV (2 Headers, 5 Options) - Transport: SMS, Google Cloud Messaging | - TLV (6 Headers, 8 Options, Digest) + Binary Format (backward compatible with DM 1.2) - Transport: SMS, OBEX, SIP, HTTP, Cell Broadcast. |

Table 5.4.3: Comparison between OMA DM 2.0 and OMA DM 1.x

#### 5.4.5.4	Protocol Examples

In this section, an example is presented in which the DM Server updates the firmware of the device. The Package#1 (DM session initialization) sent from the DM Client to the DM Server is as follows:

POST /dmclient/dm20 HTTP/1.1

Content-Type: application/vnd.oma.dm.initiation+json

Accept: application/vnd.oma.dm.request+json

OMADM-DevID: IMEI:493005100592800

Host: www.dms.com

{

"MOS": [

{

"DDF": "http://www.vendor.com/DDF/devinfo1.0.ddf",

"MOID": "urn:oma:mo:oma-dm-devinfo:1.0",

"MIID": ["miid1"]

},

{

"DDF": "http://www.vendor.com/DDF/oma-fumo1.0.ddf",

"MOID": "urn:oma:mo:oma-fumo:1.0",

"MIID": ["miid1"]

},

{

"DDF": "http://www.vendor.com/DDF/oma-dm-dmacc2.0.ddf",

"MOID": "urn:oma:mo:oma-dm-dmacc:2.0",

"MIID": ["miid1"]

}

]

}

In the above Package#1, the JSON object “MOS” carries the Management Object information that is supported by the device. According to the “MOS”, the DM Server can know that the DM Client supports DevInfo MO, FUMO, DM Account MO.

After receiving the Package#1, the DM Server sends the Package#2 to update the firmware of the device. The Package#2 is as follows:

HTTP/1.1 200 OK

Content-Type: application/vnd.oma.dm.request+json

{

"CMD": [

["EXEC", "oma:mo:oma-fumo:1.0//Update"]

]

}

After completing the firmware updating, the DM Client sends the Package#3 for the response as follows:

POST /dmserver/dm20 HTTP/1.1

Content-Type: application/vnd.oma.dm.response+json

Accept: application/vnd.oma.dm.request+json

OMADM-DevID: IMEI:493005100592800

Host: www.dms.com

{

"Status": [

{"sc": 200}

]

}

After receiving the Packge#3 from the DM Client, the DM Server terminates the DM session by sending the END command. The Package#2 for this is as follows:

HTTP/1.1 200 OK

Content-Type: application/vnd.oma.dm.request+json

{

"CMD": [

["END"]

]

}

On receiving the Package#2, the DM Client does not send the Package#3. The DM session is terminated.

# 6	Gap analysis of existing relevant technolgoies

Editor’s Note: The chapter lists all the requirements related to management in oneM2M and analyzes how the technologies can fulfil the requirements and identifies potential gaps. The chapter also gives reference to the specification of the analyzed technologies.

## 6.1	Management related requirements gap analysis reference

Editor’s Note: The table below gives cross reference to the requirements and the technologies. The matrix represents whether or not the technology can fulfil the requirement. At the same time, in the following chapters, detailed description of the depth of how well the technology can fulfil the requirement should be shown.

Some MGR requirements (e.g., MGR-004) target functionality outside the scope of the existing technology (e.g., TR-069, OMA-DM). In those cases the analysis of the technology against the requirement determines if the the functionality that is outside the scope of the technology is allowed.

The definitions for the values in below table are:

- FULL: the requirement can be fulfilled by the technology alone

- PARTIAL: the requirement can be partially fulfilled by the technology

- ALLOWED: Adopting this technology will allow this requirement to be implemented

- NOT SUPPORTED: This technology does not fulfil the requirement AND adopting this technology would not allow the requirement to be implemented

| Requirement Support |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | OMA DM 1.3 | BBF TR-069 | OMA DM LWM2M | OMA DM 2.0 |
| MGR-001 | Partial | Partial | Full | Partial |
| MGR-002 | Full | Full | Full | Full |
| MGR-003 | Full | Full | Full | Full |
| MGR-004 | Allowed | Allowed | Allowed | Allowed |
| MGR-005 | Partial | Partial | Partial | Partial |
| MGR-006 | Full | Full | Full | Full |
| MGR-007 | Full | Full | Full | Full |
| MGR-008 | Full | Full | Partial | Full |
| MGR-009 | Full | Full | Full | Full |
| MGR-010 | Partial | Partial | Partial | Partial |
| MGR-011 | Full | Full | Full | Full |
| MGR-012 | Full | Full | Full | Full |
| MGR-013 | Allowed | Allowed | Allowed | Allowed |
| MGR-014 | Full | Full | Full | Full |
| MGR-015 | Full | Full | Full | Full |
| MGR-016 | Full | Full | Full | Full |
| MGR-017 |  | Not Supported |  |  |

Table 6.1.1: Requirements fulfilment reference

## 6.2	MGR-001

Editor’s Note: This chapter gives descriptions in depth how the technologies can fulfil each management related requirements defined in oneM2M as well as the gaps related to the requirements.

### 6.2.1	Requirement Description

Editor’s note: This section is a copy paste of the requirement text in the requirement TS. Revision number of the requirement TS should be explicitly referenced.

The M2M System shall support management and configuration of M2M Gateways/ Devices including resource constrained M2M Devices.[i.23]

Note: See the Annex A as a guidance about the definition of resource constrained and what kinds of the existing management technologies are suitable to apply the constrained devices.

### 6.2.2	OMA DM 1.3

OMA DM 1.3 provides PARTIAL support for this requirement.

OMA DM 1.3 requires an OMA DM compliance device shall have at least one of the protocol stacks among TCP/IP, IrDA or WSP. And the devices shall also have a capability to parse the xml file. Because the DM Representation OMA DM uses to deliver the DM Message is in the format of XML. The OMA DM devices shall also be capable of store a certain amount of information which is the MO trees to carry the management functions. For constrained devices that serve very simple functions and have the basic capability of parsing short XML and small amount of storage to store the MO, OMA DM 1.3 can be used for device management. As a result, OMA DM can be applied to some resource constrained devices but not those very limited in resources (no memory, cannot parse the XML, no communication module).

OMA DM 1.3 can also configure devices with DM Client using the ClientAPI between DM Client and the local application. With the local application defined by oneM2M, devices can be configured using service layer protocols.

### 6.2.3	BBF TR-069

TR-069  provides PARTIAL support for this requirement.

The TR-069 provides support for resource constrained devices that are CWMP enabled through the use of its standard CWMP protocols. For resource constrained devices that are not CWMP enabled (e.g., ZigBee devices, IP devices without CWMP stack), TR-069 provides mechanisms to access the constrained devices through a CWMP enabled device called a CWMP Proxy. Section 5.2.1.1 TR-069 Proxy Management describes this architecture. A technology constraint exists in that the CWMP Proxy must have connectivity, typically LAN, with the non-CWMP enabled device. As such, the TR-69 Proxy Management functions generally reside on a M2M Gateway within the customer premises.

Resource constrained devices that are CWMP enabled requires, at a minimum, the support for the:

- Protocol stack as defined in Section 5.1.4.1 ACS to CPE Protocol

- Implementation of the TR-181i2 Baseline:3 profile [i.34]

Resources required to implement a CWMP stack have been advertised as low as 150 Kilobytes storage and 30 Kilobytes DRAM (heap and stack) on an Android operating system.

Many resource constrained devices require monitoring of the device’s environment (e.g., processor, memory, battery, temperature), the TR-181i2 data model provides support for many of these objects (processor, memory, temperature) where these objects may be monitored and alarmed using the FaultMgmt objects of the data model or using the Active/Passive notification mechanism described in Section 3.7.1.5 of TR-069 [i.13]. While TR-181i2 provides support for many objects within a resource constrained device, the current data model does not provide support for a Battery resource. This type of resource may be implemented using Vendor specific extensions or submitted to the Broadband Forum for inclusion in a revision of the TR-181i2 data model.

### 6.2.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Since the main focusing M2M device of LWM2M is resource constraint device, LWM2M is specialized in managing and configuring resource constraint devices.

For resource constraint device, LWM2M has the several features which are listed below:

- CoAP with minimum set of required features for header and options

- Either UDP or SMS as transport layer binding

- Binary TLV format (Tag, Length, Value) for conveying values of multiple Resources

- JSON format is optionally supported

Editor’s Note: According to the definition of resource constrained, this analysis can be reformulated.

### 6.2.5	OMA DM 2.0

OMA DM 2.0 provides PARTIAL support for this requirement.

The device that conforms to OMA DM 2.0 shall support TCP/IP, HTTP protocol stack, which might be not applicable for severely resource-constrained devices (e.g., 8-bit microcontrollers with small accounts of memory).

OMA DM 2.0 also requires the HTTP client in the device that can be used to retrieve management data from the Data Repository. Note that this is a mandatory feature of OMA DM 2.0. Optionally, OMA DM 2.0 might require the Web Browser Component in the device to support the web-based user interaction.

If the resource constrained device can support TCP/IP and HTTP protocol, OMA DM 2.0 can be used to manage those devices with the simple DM package representations based on the JSON format.

Compared to OMA DM 1.3, OMA DM 2.0 has different factors to support resource constrained devices as follows:

- Supporting only HTTP transport-binding,

- Providing the simple JSON-based DM package representations,

- Requiring the HTTP Client to interact with the Data Repository,

Optionally requiring the Web Browser Component for the user interaction.

## 6.3	MGR-002

### 6.3.1	Requirement Description

The M2M System shall provide the capability to discover the M2M Area Networks including information about devices on those networks and the parameters (e.g. topology, protocol) of those networks.[i.23]

### 6.3.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

In the setup phase of the OMA DM protocols. The MO DevInfo is transported from DM Client to DM Server. And the MO DevDetail can be requested by DM Server if necessary. In the MO DevInfo and DevDetail, information about how the device can be reached, the protocol, the address, port number, required security parameters is transferred from device to DM server.

For devices in the local area network that are attached to the DM Gateway, GwMO can be used to get the address information.

Also some work has been done in ETSI M2M to define MANMO and MANDMO [i.24] to enable the DM Server to get the information about the topology and protocol of the local area network.

In this way, the DM Server can get to know the connection related parameters (protocol) from the device.GwMO defines how DM Server can manage device in a local area network through DM Gateway. DM Gateway can work in three modes which are transparent mode, proxy mode and adaptor mode. OMA DM devices and non-OMA DM devices can all be managed using DM Gateway. Combined with other MOs defined by OMA DM, devices in the local area network can be managed by DM Server.

### 6.3.3	BBF TR-069

TR-069  provides FULL support for this requirement.

The TR-069 provides support for discovery of  devices in the associated Local Area Networks for CWMP enabled devices.

TR-069 proxy management has mechanisms where a CPE Proxier discovers devices using device discovery mechanisms as described in Appendix I of TR-069 [i.13]. These mechanisms rely on the discovery of the device using the device’s native protocol (e.g., UPnP DM, ZigBee, Z-wave).

The discovery of the topology of the area network in which the device exists is constrained by the device’s native protocol support for topology discovery. For example – UPnP DM doesn’t provide any support for discovery of topologies while ZigBee topologies can be inferred by evaluating the routing tables of the ZigBee nodes. The TR-181 data model exposes these elements (e.g., ZigBee routing tables) but rely on the management systems to develop the topologies.

The TR-181i2 data model [reference TR-181 Device Data Model for TR-069 Issue: 02 Amendment 6 November 2012] provides support for the following LAN topologies:

- Ethernet

- WiFi

- USB

- HPNA

- MoCA

- G.hn

- HomePlug

- Universal Powerline Association (UPA)

- UPnP

In addition the ZigBee Pro topology support is expected to be included in the next release of TR-181i2.

### 6.3.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Connectivity Monitoring Object contains which networks are available and which network is currently used.

Also, it has parent router IP address of each M2M Device so the M2M System can discover entire topology of M2M Local Area Network.

Protocol (e.g., WLAN, Bluetooth) used by M2M Device is specified in Connectivity Object also.

## 6.4	MGR-003

### 6.4.1	Requirement Description

The M2M System shall provide the capability to maintain and describe the management information model of devices and parameters (e.g. topology, protocol) of M2M Area Networks. [i.23]

### 6.4.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

For devices in the local area network, some work has been done in ETSI M2M to define MANMO and MANDMO [i.24] to maintain and describe the information model about the topology and protocol of the local area network. The MANMO and MANDMO have been submitted for registration in OMA.

### 6.4.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides capabilities to describe and maintain the management information model of CWMP and non-CWMP enabled devices as through the Supported Data Model and  Software/Firmware Management features of the protocol. Within TR-069 all devices and services that are of interest to the problem space are modelled using the TR-069 XML meta-model. These models are a description of the device and services that are under management. These models can be either configured within the device or CWMP Proxy (if the device is not CWMP enabled) or the device can report its Supported Data Model to the ACS.

### 6.4.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Connectivity Monitoring Object contains lots of Resources to maintain and describe the information model about the topology and protocol of the local area network.

LWM2M has several features to be able to manage LWM2M Clients in M2M Local Area Network. The main barrier is how to reach LWM2M Clients from LWM2M Server.

- Let a LWM2M Server know when IP Address is changed at LWM2M Client by sending Registration Update logical operation.

- Support Queue mode which makes LWM2M Server queue the request until LWM2M Client is online. If LWM2M Client with Queue mode is within M2M Local Area Network, LWM2M Client sends Update message triggered by time or event. After LWM2M Server receives Update message, the LWM2M Server reaches LWM2M Client so the LWM2M Server sends queued message.

## 6.5	MGR-004

### 6.5.1	Requirement Description

The M2M System shall support common means to manage devices enabled by different management technologies (e.g. OMA DM, BBF TR-069).[i.23]

### 6.5.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 can ALLOW the fulfilment of the requirement.

OMA DM does not provide features that translate between OMA DM and other management protocols (e.g., BBF TR-069, oneM2M). As such there is an expectation that the M2M System would translate between management protocols.

The management of devices defined by OMA DM is fulfilled by sending DM Messages from DM Server to DM Client. The management related information is carried by Management Objects. For the oneM2M system to support common means to manage devices through OMA DM, the oneM2M system could include the DM Server, DM Client and DM Gateway in the oneM2M system and could have some abstraction to include the MO trees in the oneM2M system to enable the device management in spite of the detailed technologies.

### 6.5.3	BBF TR-069

TR-069  ALLOWS  fulfilment of the requirement by Service Layer mechanisms.

The TR-069 family of specifications do not provide features that translate between TR-069 and other management protocols (e.g., OMA-DM, oneM2M). As such there is an expectation that the M2M System would translate between management protocols.

TR-069 provides access to manage devices through its Auto-configuration Server. The Auto-configuration Server has interfaces with the NMS/OSS and BSS systems of the Service provider. As such the ACS would have expected to interface with the M2M System.

### 6.5.4	OMA DM LWM2M

OMA DM LWM2M ALLOWS fulfilment of the requirement when common means between the oneM2M Server and LWM2M Server are defined. OMA DM LWM2M only defines the protocol between the LWM2M Server and LWM2M Client.

LWM2M does not provide features that translate between LWM2M and other management protocols (e.g., OMA-DM, oneM2M, TR-069).

## 6.6	MGR-005

### 6.6.1	Requirement Description

The M2M System shall provide the capability to manage multiple devices in a grouped manner. [i.23]

### 6.6.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides PARTIAL support for this requirement.

The requirement can be fulfilled both in service layer and by OMA DM technology.

For service layer, one DM Server could manage multiple DM Clients. The oneM2M system is able to manage multiple devices in a grouped manner by utilize one DM Server. The M2M System could have multiple devices in a group. When the M2M System needs to send identical DM Message to each device in the group, the M2M System could pass the command to DM Server to send DM Messages one by one or simultaneously.

For manage group of devices in OMA DM. GwMO is defined how to fan out DM Command to a group devices.

### 6.6.3	BBF TR-069

TR-069  provides PARTIAL support for this requirement.

The TR-069 family of specifications defines management actions that are destined for a single CWMP enabled device. The concept of groups within the TR-069 family of specifications does not exist. Grouping, while not specified within the CWMP protocol is implemented within ACS or the NMS/OSS/BSS systems. As such, the M2M System would be required to implement the grouping feature.

### 6.6.4	OMA DM LWM2M

OMA DM LWM2M provides PARTIAL support for this requirement.

The requirement can be fulfilled at service layer.

A LWM2M Server could manage multiple LWM2M Clients. The oneM2M system is able to manage multiple devices in a grouped manner by utilizing one LWM2M Server. The M2M System could have multiple devices in a group. When the oneM2M System needs to send identical message to each device in the group, the oneM2M System could pass the command to the LWM2M Server to send the messages one by one.

## 6.7	MGR-006

### 6.7.1	Requirement Description

The M2M System shall provide the capability for provisioning and configuration of devices in M2M Area Networks. [i.23]

### 6.7.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

For devices in the local area network, GwMO can be used to provision and configure End Devices that are attached to the DM Gateway. Gateway Config MO contains the information related to each attached End Device.\

### 6.7.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for provisioning and configuration of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. TR-069 has the capability to manage a device through the device’s life-cycle (bootstrap through decommissioning).

For CWMP enabled devices that reside behind a Gateway with Firewalls and Network Address Translation features enabled, the CWMP protocol provides a mechanism that allows the ACS to communicate those devices. This mechanism is described in as described in Annex G of TR-069 [i.13].

### 6.7.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

In LWM2M, bootstrap mechanism is used to provision and configure the M2M Device. Four approaches are introduced in TS: Manufacturer Pre-configuration, SmartCard Provisioning, Client Initiated Bootstrap, and Server Initiated Bootstrap. And detail bootstrap steps are described.

## 6.8	MGR-007

### 6.8.1	Requirement Description

The M2M System shall provide the capability for monitoring and diagnostics of M2M Gateways/Devices in M2M Area Networks. [i.23]

### 6.8.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

The capability of monitoring and diagnostics of OMA DM is mainly achieved by MO DiagMon. DiagMon supports diagnostics policies management, fault reporting, performance monitoring, device interrogation, remote diagnostics procedure invocation and remote device repairing. The monitoring and diagnostics of the devices in the local area network can be fulfilled by DiagMon plus GwMO.

### 6.8.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for monitoring and diagnostics of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. Monitoring can include notification support for devices that need immediate attention as well as passive monitoring of device information and statistics that may be collected in a periodic manner. Likewise, TR-069 provides the capability to execute diagnostics in a synchronous or asynchronous fashion; allowing for long lived diagnostics to be executed on a device,

For non-CWMP enabled devices, the device is implemented as a non-CWMP enabled Virtual Device where the procedure is documented in Appendix I of TR-069 [i.13]

CWMP and non-CWMP enabled Virtual and Embedded devices support the following diagnostic operations:

- Reboot

- Factory Reset

In addition CWMP enabled and non-CWMP enabled Virtual Devices also support the following standard diagnostics:

- IP Diagnostics (Ping, Trace Route, HTTP or FTP Download or Upload, UDP Echo)

- DNS – NS Lookup

- HPNA Diagnostics

- UPA Diagnostics

- Device Self Tests

- DSL Line

- ATM Interface

These tests are documented within the TR-181i2 data model [reference TR-181 Device Data Model for TR-069 Issue: 02 Amendment 6 November 2012].

### 6.8.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

In LWM2M, Connectivity Monitoring Object and Connectivity Statistics Object are used to monitor current connection and collect connection information to configure parameters based on the collected result respectively.

LWM2M enabler also has concept to send diagnostic information such as GPS module failure, IP connectivity failure, and Peripheral malfunction.

## 6.9	MGR-008

### 6.9.1	Requirement Description

The M2M System shall provide the capability for software management of devices in M2M Area Networks. [i.23]

### 6.9.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

Software management capability is fulfilled by the MO SCOMO (Software Component Management Object). SCOMO can be used to remotely manage a software component within a device. The functionalities provided by SCOMO includes delivery, download, installation, update, removal, activation, and de-activation of software. The software management of devices in local area network can be fulfilled by SCOMO plus GwMO.

### 6.9.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for software and firmware management of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. Software and firmware management of non-CWMP enabled devices may be performed using Software modules within the CWMP enabled M2M Gateway or may be downloaded directly to device by implementing the non-CWMP enabled device as a Non-CWMP enabled Virtual Device.

### 6.9.4	OMA DM LWM2M

OMA DM LWM2M provides PARTIAL support for this requirement.

Since LWM2M talks about resource constraint M2M devices, LWM2M enabler provides the capability for management of a full package, firmware. So firmware contains all the software which is necessary for all the services provided by M2M Service Platform.

## 6.10	MGR-009

### 6.10.1	Requirement Description

The M2M System shall provide the capability for rebooting and/or resetting of M2M Gateways/Devices and other devices in M2M Area Networks. [i.23]

### 6.10.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

The functionality for reset the device is provided by the MO LAWMO. LAWMO is for remotely locking and wiping the device. The functionality of rebooting a device is enabled by DiagMon. In this way, OMA DM can reset the device to its original state and reboot the device as well. The reset and reboot of a device in local area network can be enabled by DCMO or DiagMon plus GwMO.

### 6.10.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for resetting (boot, factory) of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. For non-CWMP enabled devices, the device is implemented as a Non-CWMP enabled Virtual Device.

In the scenario where the device in the M2M Local Area network is a CWMP enabled device, TR-069 provides a mechanism where the device can communicate through a Gateway (which might be NAT or Firewall enabled) to the ACS. Annex G of TR-069 [i.13] describes this mechanism.

### 6.10.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Device Object has a Reset Resource which is used for resetting M2M Devices. LWM2M also provides factory reset function which makes M2M Device initial state of deployment.

## 6.11	MGR-010

### 6.11.1	Requirement Description

The M2M System shall provide the capability for authorizing devices to access M2M Area Networks. [i.23]

### 6.11.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides PARTIAL support for this requirement.

Configuration methods of OMA DM can be used to detach certain device from the network. Coordinator and router can also be configured to block the new access request to deny devices to be attached to the local area network. In this way, devices can be authorized to access M2M Local Area Network.

### 6.11.3	BBF TR-069

TR-069  provides PARTIAL support for this requirement.

TR-069 does not provide direct support for authorizing devices for access to Local Area Networks but does allow for configuration of credentials and other properties that these networks utilize. TR-069 does this for CWMP and non-CWMP enabled devices.

### 6.11.4	OMA DM LWM2M

OMA DM LWM2M provides PARTIAL support for this requirement.

If M2M devices have configuration for accessing M2M Local Area Networks prior to deployment, the M2M devices can be authorized based on the configured information.

## 6.12	MGR-011

### 6.12.1	Requirement Description

The M2M System shall provide the capability for modifying the topology of devices in M2M Area Networks. [i.23]

### 6.12.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

OMA DM can be used to modify the topology of devices in M2M Local Area Network by activate and de-activate devices that serve as coordinator or router in the area network. For example, if a coordinator or router is de-activated, devices attached to the coordinator or router will automatically find other coordinators or routers to access the local area network.

Configuration methods can also be used to configure the router or coordinator to accept or deny access request of new devices. The topology is modified.

### 6.12.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 does allow for configuration  of properties for Local Area Networks that can determine which devices are to be included within a Local Area Network. TR-069 does this for CWMP and non-CWMP enabled devices.

### 6.12.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

OMA DM LWM2M can be used to modify the topology of devices in M2M Local Area Network by disabling M2M devices which serve as coordinator in the area network. For example, if a coordinator is disabled, devices in M2M Local Area Network attached to the coordinator will automatically find other coordinators to access the local area network.

## 6.13	MGR-012

### 6.13.1	Requirement Description

Upon detection of a new device the M2M Gateway shall be able to be provisioned by the M2M Service Infrastructure with an appropriate configuration which is required to handle the detected device. [i.23]

### 6.13.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

In a DM Server assisted bootstrap procedure defined in GwMO. Whenever a new device is detected, DM Server will install the End Device credentials to the gateway through Gateway Config MO.

In OMA DM 1.3, DM Gateways with DM Client can be also provisioned using the ClientAPI between DM Client and the local application. With the local application defined by oneM2M, devices can be configured using service layer protocols.

### 6.13.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for detection of new devices in the Local Area Networks. TR-069 has the capability to detect new devices via active and passive notification mechanisms described in Section 3.7.1.5 of TR-069 [i.13]  as well as using the CWMP protocol’s inform event (e.g., Bootstrap) mechanisms for CWMP enabled devices.

In the most common M2M deployment scenarios, the M2M Gateway would include a CWMP Agent making the M2M Gateway a CWMP-enabled device with a CWMP Proxier as defined in Annex J of TR-069 [i.13].

In this scenario, the M2M Gateway’s CWMP agent could detect and report the new device to the ACS via the M2M Gateway’s CWMP agent. Once a device is reported to the ACS, the ACS can inform the M2M System about the device addition for further configuration and software/firmware management activities. Possible configuration activities could include:

- Software management (Annex H of TR-069[i.13])

- Device configuration via Proxy management (Appendix I of TR-069[i.13])

### 6.13.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

LWM2M doesn’t have gateway concept so to be able to handle the detected devices, the M2M Gateway needs to have LWM2M Server feature which means that device thinks gateway as LWM2M Server. In this case, M2M Gateway can manage the device detected.

## 6.14	MGR-013

### 6.14.1	Requirement Description

The M2M System shall be able to identify and manage M2M Service status of M2M Devices. [i.23]

### 6.14.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 ALLOWS fulfilment of the requirement by development of a oneM2M Service Layer data model for management purposes.

DCMO defined in OMA DM can be used to enable or disable device capabilities, such as hardware, IO and connectivity. OMA DM 1.3 and 2.0, as a device management protocol, is able to manage the M2M Service layer using the DM protocol to configuration and retrieval of M2M Services that execute within M2M Devices. The only thing necessary is that a data model of the M2M Service be defined for management purposes.

### 6.14.3	BBF TR-069

TR-069 ALLOWS fulfilment of the requirement by development of a oneM2M Service Layer data model for management purposes.

The TR-069 family of specifications, as a device management protocol, is able to manage the M2M Service layer using the CWMP protocol to configuration and retrieval of M2M Services that execute within M2M Devices. The only thing necessary is that a data model of the M2M Service be defined for management purposes.

### 6.14.4	OMA DM LWM2M

OMA DM LWM2M ALLOWS fulfilment of requirement by development of a M2M Service Layer data model for management purposes.

The LWM2M specification, as a device management protocol, is able to manage the M2M Service layer using the LWM2M protocol to configuration and retrieval of M2M Services that execute within M2M Devices. The only thing necessary is that a data model of the M2M Service be defined for management purposes.

## 6.15	MGR-014

### 6.15.1	Requirement Description

The M2M System shall be able to retrieve events and information logged by M2M Gateways/ Devices and other devices in M2M Area Networks. [i.23]

### 6.15.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 provides FULL support for this requirement..

Related technologies are defined in DiagMon:Trap which describes how the DM Client monitors the performance of the device. The DiagMon Client can send notification to DM Server or collect trap event together to response the retrieve request. As a result, OMA DM 1.3 and 2.0 can fully fulfill the requirement.

With regard to devices in the M2M Area Network, DiagMon is combined with GwMO to fulfill the requirement.

### 6.15.3	BBF TR-069

TR-069 provides FULL support for this requirement.

The TR-069 family of specifications, as a device management protocol, is able to retrieve events and logs for M2M Gateway/Devices and devices in M2M Area Networks through the use of the CWMP protocol using the standard data model and proxy mechanisms. Information can be retrieved using the CWMP get RPC as well as file transfer mechanisms (e.g., FTP, HTTP). In addition, the IPDR protocol can be used to retrieve information that would be considered bulk transfer.

### 6.15.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

The LWM2M Client has capability to collect error or event information such as GPS module failure, out of memory, SMS failure, and low battery power. The LWM2M has functionality to collect data to be notified due to subscription when the LWM2M Client is offline or the LWM2M Server is temporarily disabled.

## 6.16	MGR-015

### 6.16.1	Requirement Description

The M2M System shall be able to support firmware management (e.g. update) of M2M Gateways/ Devices and other devices in M2M Area Networks.. [i.23]

### 6.16.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 provides FULL support for this requirement.

It is defined in FUMO about how to support firmware management. FUMO defined in OMA DM can be used to initiate firmware update, exchange device information, download update package, install update package, notify firmware update.

With regard to devices in the M2M Area Network, FUMO is combined with GwMO to fulfill the requirement.

### 6.16.3	BBF TR-069

TR-069 provides FULL support for this requirement.

The TR-069 family of specifications, as a device management protocol, is able to download firmware for M2M Gateway/Devices and devices in M2M Area Networks through the use of the CWMP protocol using the standard data model and proxy mechanisms. Firmware can be retrieved using the CWMP download RPC file transfer mechanisms (e.g., FTP, HTTP). One constraint exists in the proxy mechanism for downloading firmware to device in a M2M Area Network. A device in a M2M Area Network must be a Virtual Device to have allow for the download RPC to be utilized.

### 6.16.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

LWM2M has Firmware Object to update firmware of the LWM2M Client. LWM2M supports two download mechanisms: directly write firmware package, and give URI and let the LWM2M Client download firmware package. After updating firmware, LWM2M has capability to inform the connected LWM2M Servers of what functionalities are added in the LWM2M Client.

## 6.17	MGR-016

### 6.17.1	Requirement Description

The M2M System shall be able to retrieve information related to the Static and Dynamic Device/Gateway Context for M2M Gateways/Devices as well as Device Context for other devices in M2M Area Networks. [i.23]

### 6.17.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 provides FULL support for this requirement.

DiagMon defined by OMA DM 1.3 and 2.0 enable the DM Server to acquire information related to the devices local context including battery level, available memory as well as some network capabilities.

DiagMon also defines Trap method which can be used to collect events related to the change of dynamic context in the device. The collected events can be retrieved by the DM Server.

ClientAPI defined in OMA DM 1.3 and 2.0 also provides interfaces that can be used to retrieve MO information from DM Client which can be static device context.

### 6.17.3	BBF TR-069

TR-069 provides FULL support for this requirement.

The TR-069 family of specifications, as a device management protocol, is able to retrieve device specific information for M2M Gateway/Devices and devices in M2M Area Networks through the use of the CWMP protocol using the standard data model and proxy mechanisms.  As the TR-069 model utilizes data models for representing information that is easily extended either through standardization activities or vendor specific attributes.

### 6.17.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

LWM2M has Static Context such as manufacturer, model number, and serial number, and Dynamic Context such as battery level, memory free, location, time, IP address, current network (e.g., WCDMA, GSM, LTE, Bluetooth, WiFi), and serving cell id. LWM2M has capability to expose parent IP address, which can make topology of M2M Area Network.

## 6.18	MGR-017

### 6.18.1	Requirement Description

The M2M system shall support the capability to map M2M service subscription role(s) to roles used within technology specific Device Management protocols. [i.23]

### 6.18.3	TR-069

This requirement is NOT SUPPORTED by the TR-069 family of specifications.

The TR-069 family of specifications does provide security capabilities (ACLs) within the CPE to ensure authorized parties are permitted access control of an Object. However this ACL mechanism doesn’t discern different management roles within the ACS. The ACL mechanism in the CPE treats the ACS as the one authorized party. In addition, a CPE can only report to one ACS at a time. As such, the TR-069 ACS mechanism does not provide security authorization of resources to various roles within the ACS as expectation is, since a CPE can only be managed by 1 ACS, that this function is the responsibility of the ACS.

Likewise the ACL capability, while specified, isn’t widely supported in CPEs. In fact the BBF certification program does not include ACLs in its current certifications suite of tests.

Also the TR-069 family of specifications does not specify an interface northbound of the ACS except that TR-131 [i.33] does provide guidance to northbound systems such as M2M systems. If an interface with associated Service Layer to Device Management Layer security mechanisms are required, the expectation would be that oneM2M would either specify an interface between the ACS or work collaboratively with the Broadband Forum to specify a northbound interface from an ACS for the purposes of M2M Service Layer interaction.

# 7	oneM2M Architectural Considerations

## 7.1	Introduction

This chapter evaluates whether utilisation of the analysed management technologies, to fulfil the oneM2M management related requirements, results in any architectural recommendations for, or potential constraints to, the oneM2M architectural design. This chapter also highlights any restrictions that the oneM2M arhctecture potentially places on utilisation of the analysed management technologies within oneM2M.

## 7.2	Current Management Deployment Scenarios

This section describes the common deployment scenarios that exist today for deployments of the Device Management technologies listed in this TR.

### 7.2.1	Managed Device Using Network Operator Management

When discussing M2M Device Management deployment scenarios we must review the Device Management deployment scenarios that exist today in the underlying network operator’s communication network. In the scenario depicted below the underlying network operator has, other than the end user, exclusive control of the resources within the device. Also in this deployment scenario, the device management technologies described in the TR utilize a management client in the device that connects to the underlying network operator’s management server. Typically there is one instance of the management client within the device that connects to one management server controlled by the underlying network operator. In several of the device management technologies the management client in the device offers a proxier capabability that provides the underlying network operator with the capability to manage devices that do not have their own management client. The underlying network operator’s Operational Support System(s) (OSS) manage devices through their interaction with the underlying network operator’s management server.

The underlying network operator ensures exclusive control of the resources within the device by providing device firmware and software that have been approved for use by the underlying network operator.

Figure 7.2.1: Device Management in the Communication Network

### 7.2.2	Managed Device Using Service Provider Management

With the introduction of the M2M System a new stakeholder, the M2M Service Provider, also requires control of resources of the device. In this scenario depicted below, the M2M Service Provider controls all or selected resources of the device via its own management server which may be part of the M2M Service Platform. In this scenario the underlying network operator ensures that the M2M Service Provider has control of underlying network operator restricted resources using an out-of-band responsibility delegation mechanism. The delegation mechanism utilized is usually a certification process by the underlying network operator of the firmware and software that is downloaded on the device by the M2M Service Provider. The process of certifying the software and firmware of the device is outside the scope of this TR.

Figure 7.2.2: Service Provider Controlled Devices

## 7.3	Possible Future Management Deployment Scenarios

This section describes common deployment scenarios that are expected to exist in the future for deployments of the Device Management technologies listed in this TR in addition to the ones described above.

### 7.3.1	Shared Managed Device Using Network Operator Management

In some scenarios, the underlying network operator and the M2M Service Provider share control of the device by utilizing the underlying network operator’s management server. The underlying network operator restricts which resources the M2M Service Provider can control by exposing the capabilities from the management server to the M2M Service Platform. Typically this is performed by providing the M2M Service Provider an account, with appropriate access control to the resources, within the underlying network operators management server.

Figure 7.3.1: Shared Control via Network Operator Management Server

### 7.3.2	Shared Managed Device Using Service Provider Management

In some scenarios, the underlying network operator and the M2M Service Provider share control of the device by utilizing the service provider’s management server. The service provider’s management server is the primary DM server, and any requests that the underlying network operator wants to initiate on the M2M device using occurs over the X interface to the service provider’s Common Services Entity, which in turn sends the request to the service provider’s DM server who will execute the DM operation. This scenario is typically interesting when the M2M Service Provider uses different underlying networks that all require some “unified” DM access to the M2M devices (example: the use case “Oil and Gas Pipeline Cellular/Satellite Gateway from TR-0001 Use Cases).

Figure 7.3.2: Shared Control via Network Operator Management Server

### 7.3.3	Shared Managed Device Using Separate Management

In this scenario the underlying network operator and the M2M Service Provider share control of the device by utilizing separate management servers as depicted by the below figure. It should be noted that some technologies (e.g., TR-069) cannot implement the scenario as a management client can only connect to one management server. In this scenario, the authorization enforcement point can be implemented within the device using the delegation and access control list features of the device management technologies or in the M2M Service Platform. Regardless of the enforcement point, the network operator restricts control to the network operator controlled resources by certifying the firmware and software that is placed on the device.

Figure 7.3.3: Shared Control via Separate Management Servers

### 7.3.4	Federated Managed Device Using Separate Management

In some M2M Devices, the control of resources in a device is federated within separate operating environments of the device as depicted by the figure below. This scenario is typical of devices with multiple CPU complexes where one complex is dedicated to the access network termination functions (e.g., machine termination) while another CPU complex is dedicated to application functions. One important point of this type of deployment scenario is fact that there are multiple management clients where a management client is assigned to the M2M Service Provider’s management server and another management client is assigned to the underlying network operator’s management server.

Figure 7.3.4: Federated Managed Device

### 7.3.5	Conclusions To Guide the Device Management Architecture

The deployment scenarios described in the Section 7.2 allows several conclusions can be described to guide the development of the device management architecture in M2M Systems. These conclusions are:

The M2M Service Provider manages resources in a device by:

Utilizing the Network Operator's management server to access resources in the device

Operating a separately owned management server that allows access to resources in the device. In this case, a separate management client might exist in the device to which the M2M Service Provider’s management server connects.

The M2M Service Platform reaches the Network Operator or M2M Service Provider management server through the Z reference point

Resources within a device are owned by either the Network Operator or the M2M Service Provider. Access to the resource is controlled by:

Certifying the firmware or software that is used on the device

Utilizing the access control mechanisms of  the device management technologies (e.g., accounts, delegation, access control lists)

## 7.4	Management Generalized Architectural Framework

### 7.4.1	Generalized Architecture

Infrastructure Node

CSE

Application

X

Y(mc)

Device in M2M Area Networks

Management Server

Z(ms)

Service Layer

Underlying Network

DMF

Intermediate/End Node

DMF

CSE

Management Client

Proxy

Z(mc)

Proxy Management Client

Management Server

mp

Management Adapter

Management Adapter

Figure 7.4.1: DM Architecture and reference points of oneM2M

Figure 7.4.1 is the generalized architecture framework for device management.

The Device Management Function (DMF) is the Common Service Function (CSF) functional architectural component that is responsible for management of Intermediate and End Nodes. The DMF is responsible for exposing the device management capabilities supported by the CSE to other CSEs or M2M Applications. In addition the DMF is responsible for management of the CSE within for Intermediate and End Nodes. In order to manage the CSE and device capabilities of these nodes, the DMF utilizes existing device management technologies (e.g., TR-069, OMA-DM, and LWM2M). As such, the DMF translates or adapts the management related requests from other CSEs or M2M Applications to device management commands corresponding of the device management technology.

In the oneM2M functional architecture, the DMF in the CSE of the Infrastructure Node is used to manage Intermediate and End Nodes that are representative of M2M Gateways/Devices as well as devices in the M2M Area Network. The DMF of the Infrastructure Node manages the Intermediate and End Nodes using the device management technologies Management Server. These Management Servers could reside within the CSE (embedded Management Server), outside the CSE as part of the Infrastructure Node or as separate entity in the Underlying Network.

The DMF has a functional component termed the Management Adapter that performs the adaptation between:

- Device Management Function and Management Servers 
After the DMF receives the requests from M2M Applications via the X reference points, the Management Adapter in the DMF translates the requests from the M2M Applications to requests that can be understood by the Management Server. The Management Server then communicates with the Management Client or Proxy.

- DMF and Management Clients
The Management Adapter in the DMF of Intermediate Node or End Node interacts with the Management Client to exchange information with the Management Client using the interface provided by the Management Client (e.g. DM-7,8,9 in OMA DM). The Management Adapter can then interact with DMF in the Infrastructure Node over the Y reference point using interface defined between CSEs.

While the Management Adapter provides a translation function between the DMF and Management Servers and Clients, the communication between the Management Client and Management Server uses the technology specific protocol (e.g., TR-069, OMA-DM, LWM2M).

The interface between Management Server and Management Client is the mc interface which is subject to the device management technology that is used (e.g. OMA DM, BBF TR069 or OMA LWM2M). The interface between Management Server and Management Adapter is the ms interface which is provided by the Management Server.

In the case of the Intermediate and End Nodes, the DMF in the CSE of the Intermediate Node has the same functionality as the DMF in the CSE of the End Node. In addition, the DMF in the Intermediate Node can be used to manage devices in the M2M Area Network. In this case, the DMF is deployed with Proxy functionality that interacts with the Proxy Management Client using mp interface. Depending on the technology of the Management Client (e.g., TR-069, OMA-DM, LWM2M), the Proxy functionality (mode) can differ (e.g., transparent, adaptation, proxy) as well as the mp interface. For example TR-069 only supports an adaptation mode, OMA-DM supports transparent, adaptation and proxy modes [i.10] and LWM2M does not currently support a Proxy function. Depending on the mode, the Proxy Management Client may be a Technology Specific (OMA-DM) or Native Protocol (ZigBee) Management Client.

### 7.4.2	Architectural Framework Across the X Reference Point

Infrastructure Node

CSE

Application

X

DMF

Management Adapter

Figure 7.4.2: Device Management Across X Reference Point

The X reference point describes how M2M Applications communicate with CSFs within a CSE using messaging capabilities defined for the X reference point. In the case of the DMF, M2M Applications communicate with the DMF using a representation exposed by the DMF that is common between the Device Management technologies utilized by the M2M System. For example, the capability to reboot a device is exposed to the M2M Application using a representation that is independent of the device management technology (e.g., TR-069, OMA-DM). By using a common representation toward the M2M Application across the X reference point, M2M Applications interact with the M2M System independent of the a specific Device Management Technology. In addition, the M2M capabilities (e.g., Security) exposed by the M2M System across the X reference point are integrated within the DMF as with other CSFs. Of particular note to the DMF is the Authorization capability exposed by the X reference point. The DMF utilizes the Security CSF to authorize management requests by M2M Application across the X reference point. Because the X reference point utilizes a common representation and the Security CSFs authorization capabilities, the DMF must adapt the mechanisms of the M2M System into the protocol of the Device Management technology.This adaptation is exposed across the Z reference point and is described in greater detail in Section 7.3.4.

### 7.4.3	Architectural Framework Across the Y Reference Point

Infrastructure Node

CSE

Application

X

Y(mc)

Device in M2M Area Networks

DMF

Intermediate/End Node

DMF

CSE

Management Client

Proxy

Proxy Management Client

Management Server

mp

Management Adapter

Management Adapter

Figure 7.4.3: Device Management across Y Reference Point

In case that the Management Server is embedded within the CSE in the Infrastructure Node, the Y reference point can be used for the device management purposes. The Management Server embedded in the CSE in the Infrastructure Node communicates with the Management Client in the Intermediate or End Node via the Y(Mc) reference point. Y(Mc) means that Y reference point is realized by the Mc interface pertaining to the device management functionalities. Note that the Mc interface is specific to the device manage technologies (e.g., TR-069, OMA-DM, LWM2M).

Depending on the relationship with the Management Server, the Management Adapter in the Infrastructure Node may need authentication information that is used to access the Management Server. The typical role for the Management Adapter in the Infrastructure Node is to adapt the oneM2M service layer to the device management layer.

Once the Application delivers the management request via the X reference point the DMF within the receiving CSE takes the management request for further processing which includes the Management Adapter functionality within the DMF performing the adaptation that is needed to interact with the Management Server. Note that this adaptation is not specific to the device management technology but this is a general adaptation from the oneM2M service layer to the device management layer. After the adapted management request is delivered to the Management Server, the Management Server performs the relevant device management actions that are specific to the device management technology.

The Management Adapter in the Intermediate/End Node can be used to expose the device management information to the CSE in the Infrastructure Node using the Y reference point. To collect the device management information, the Management Adapter can interact with the Management Client using the interface defined by the device management technologies. The device management information is specific to the device management technology, and the Management Adapter can perform the adaptation to report them to the CSE in the Infrastructure Node. When the CSE in the Infrastructure Node receives this device management information, the CSE can configure the Management Client by interacting with the Management Adapter using the Y reference point.

- 7.4.4	Architectural Framework Across the Z Reference Point

Infrastructure Node

CSE

Application

X

Device in M2M Area Networks

Management Server

Z(ms)

Service Layer

Underlying Network

DMF

Intermediate/End Node

DMF

CSE

Management Client

Proxy

Z(mc)

Proxy Management Client

mp

Management Adapter

Figure 7.4.4: Device Management across Z Reference Point

The Z reference point allows the CSE to use the services provided by the Underlying Network. In case of the device management, the Underlying Network exposes the device management functionalities to the service layer using the Z reference point. Below figure shows the generalized architecture that shows only relevant entities from the Figure 7.4.1

In the Infrastructure Node, the DMF uses the Management Adapter functions to translate requests to the Management Server from the DMF by using the Z(Ms) reference point. Z(Ms) means that Z reference point is realized by the Ms interface for a specific Device Management technology (e.g., TR-069, OMA-DM, LWM2M). The Management Server implementation can be based on various device management technologies (e.g., OMA DM Server, ACS, OMA Lightweight M2M Server), but the Ms interface provides the common means to access the Management Server regardless of the specific device management technology.

Since the Management Server resides in the Underlying Networks, the Management Adapter needs to be provisioned with the authentication information used to access the Management Server and the Management Server should properly configure the access control for the Management Adapters use of the devices resources.

Once the M2M Application delivers the management request via the X reference point, then the DMF processes the command using the Management Adapter function for the adaptation that is needed to interact with the Management Server in the Underlying Networks. Note that this adaptation is not specific to the device management technologies, but is general adaptation from the oneM2M service layer to the device management layer. The adapted management request is sent to the Management Server in the Underlying Networks via the Ms interface across the Z reference point. When the Management Server receives the adapted management request, the Management Server performs the relevant device management logic that is specific to the device management technology.

The Management Server in the Underlying Networks needs to be properly configured with the access rights to manage the resources requested by the DMF in the Infrastructure Node.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

# Annex A: Guidance for Managing Resource Constrained Devices

The oneM2M specification is expected to support devices that are resource constrained. As seen clearly in the section 6 of this report, different types of constrained devices, according to memory and processing capabilities, can be managed by using different management technology. In this annex, the proper definition what resource constrained devices are and what kinds of management technologies are suitable to apply the constrained devices are discussed as a guideline.

## A.1	Classification of Resource Constrained Devices

One of controversial issues is that there is no the clear definition of ‘resource constrained devices’ and ‘lightweight or heavy devices’. However, there is a useful reference [i.30] to the definition of constrained devices. It can be used for enabling the classification of devices according to the RAM and storage usage for implementing protocol stacks to apply the management technologies since OMA DM 1.3/2.0, OMA LWM2M, and BBF TR-069 utilize the different binding protocols stacks.

[i.30] defines some succinct terminology for different classes of constrained devices. The table below represents the criteria of each classes based on the memory constraints.

| Name | Data Size (e.g. , RAM) | Code Size (e.g., Flash) |
| --- | --- | --- |
| Class 0 | << 10 kilobytes | << 100 kilobytes |
| Class 1 | ~ 10 Kilobytes | ~ 100 kilobytes |
| Class 2 | ~ 50 kilobytes | ~ 250 kilobytes |
| Class 3 | >> 50 kilobytes | >> 250 kilobytes |

- Class 0 (C0)

  - Devices are very constrained (i.e., CPU, RAM, Flash) sensor-like nodes

  - No possibility to have a direct communications with the Internet in a secure manner

  - Deployed with a larger device acting as a management proxier and/or gateway

- Class 1 (C1)

  - Has the capability to connect with nodes across the Internet in a secure manner using a constrained protocol stack (e.g., DTLS, UDP, CoAP) and various encoding protocols (e.g., TLV, JSON, Javascript)

  - Messages between nodes are typically transmitted within 1 packet due to the cost of packet fragmentation and reassembly within the Device

  - Devices typically support 1 M2M Application

  - Devices have simple mechanisms in place to communicate behind network firewalls and NATs.

- Class 2 (C2)

  - Has the capability to connect with nodes across the Internet in a secure manner using a full featured and reliable protocol stack that typically consists of TCP, HTTP, TLS (security) and various encoding protocols (e.g., XML, SOAP).

  - Devices have mechanisms in place to communicate behind network firewalls and NATs.

- Class 3 (C3)

  - Has the capability to be deployed as a management proxier – connecting C0 devices to nodes within the Internet.

  - Provides additional gateway features (e.g., multiple M2M applications)

Note: [i.30] defines only 3 classes from C0 to C2. In this annex, C3 is defined to designate some management proxier and/or gateway as a new class, higher than C2.

Note: The class of a device doesn’t mean a device cannot be deployed with lightweight and energy-efficient aspects of the transport protocols which can consume less bandwidth across the network (e.g., HTTP compression).

## A.2	Device Classes and Management Technologies

The existing management technologies utilize the different protocol stacks and the different protocol stacks consumes different amount of memory. The figure below demonstrates what kinds of protocol stacks are used by the management technologies and the appropriate device class of each technology based on the analysis of the previous section A.1.

Figure A.2.1: Protocol Stacks and Device Class used by the management technologies

It means that, as an instance, the BBF TR-069 enabled devices might be consisted of Embedded OS (2 KB) + TCP (4 KB) + SSL/TLS (36 KB) + HTTP (4 KB) + REST Engine (0.7 KB) + BBF TR-069 Client (less than 150 KB) < 250 KB (C2 Requirement) to fulfil the BBF TR-069’s protocol and resource requirement for devices that do not support the TR-069 proxy features. For the OMA LWM2M enabled devices can be operated on Embedded OS (2KB) + UDP (1.3 KB) + DTLS (36 KB) + CoAP (4 KB) + REST Engine (0.7 KB) + LWM2M Client (less than 20 KB) < 100 KB (C1 Requirement).

Note: [i.31] is referred to indicate the code size of each protocol.

Note: The size of Embedded OS is based on Contiki OS which is an OS for tiny networked sensors. The summation of size of all modules such as kernel, program loader, multi-threading and timer library is 2280 bytes.

To sum up, this guideline identifies the minimum resource required on prospective oneM2M Device and Gateway by implementing different existing management technologies on resource constrained devices.

# Annex <B>: Title of annex

<Text>

B.1	First clause of the annex

<Text>

B.1.1	First subdivided clause of the annex

<Text>

# Annex <y>: Bibliography

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

# History

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V 0.0.1 | 08/04/2013 | Skeleton draft |
| V 0.1.0 | 26/04/2013 | Incoporate contributions: oneM2M-MAS-2013-0016R02-Introduction_to_OMA_DM oneM2M-MAS-2013-0024R05-Introduction_to_OMA_DM_LWM2M oneM2M-MAS-2013-0014R04-Study_of_TR-069_Management |
| V 0.1.1 | 08/05/2013 | Editorial changes: Move the acronyms from clause 3.3 to 3.4 Added IETF document as informative reference in Convention section The titles of 5.1.3, 5.1.4, 5.2.3, 5.2.4, 5.3.3 and 5.3.4 changed to plural |
| V 0.1.2 | 14/06/2013 | Incoporate contributions: oneM2M-MAS-2013-0030R01-requirements_list_for_gap_analysis oneM2M-MAS-2013-0032-OMA_DM_DM6_interface |
| V 0.2.0 | 28/06/2013 | Incoporate contributions: oneM2M-MAS-2013-0031R04-BBF_TR-069_Technology_–_Management_TR_Section_6_Gap_Analysis oneM2M-MAS-2013-0042R02-OMA_DM_requirements_gap_analysis oneM2M-MAS-2013-0043R02-Introduction_to_OMA_DM_2_0 oneM2M-MAS-2013-0048R01-LWM2M_Gap_Analysis oneM2M-MAS-2013-0053R01-OMA_DM_2_0_Gap_Analysis Template updated according to: oneM2M-Template-TR-20130606 Add new MGR requirements from: oneM2M-TS-0002-Requirements-V0_4_0 |
| V 0.3.0 | 31/07/2013 | Incoporate contributions: oneM2M-MAS-2013-0062R02-Section_7_to_Management_Capability_Enablement_TR-006 oneM2M-MAS-2013-0058-oneM2M_TR006_Section_5_TR-069_Editorials oneM2M-MAS-2013-0061R04-device_management_architecture |
| V 0.4.0 | 15/08/2013 | Incoporate contributions: oneM2M-MAS-2013-0065R02-YZ_Reference_Point_for_Management_Purposes oneM2M-MAS-2013-0068R01-Guidance_for_Managing_Resource_Constrained_Devices oneM2M-MAS-2013-0072R02-Symbols_Section_7_TR006 oneM2M-MAS-2013-0073-Section_7_4_X_Reference_TR00 oneM2M-MAS-2013-0076R01-OMA_DM_client_API oneM2M-MAS-2013-0077R03-OMA_DM_new_requirement_gap_analyze oneM2M-MAS-2013-0080R02-LWM2M_Gap_Analysis_for_new_REQ oneM2M-MAS-2013-0082R02-TR-069_Gap_Analysis_for_Additional_Requirements Some editorial changes |
| V 0.4.1 | 09/10/2013 | Incoporate contributions: oneM2M-MAS-2013-0088R01-TR-069_Gap_Analysis_for_Additional_Requirements_from_TP6 oneM2M-MAS-2013-0094-Comments_for_sections_related_to_TR-069 Editorial changes: Inconsistency in Table 6.1.1 compared with section 6.13 changed Update the footer to be consistent with the new TR template |