| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M TR-0009 v0.3.0 |
| Document Name: | Technical Report - Protocol Analysis |
| Date: | 03 Nov 2013 |
| Abstract: | This document identifies protocols deployed in Industry segments, describes their use, then analyses protocols, their security aspects and their data models, with a view towards interoperability with oneM2M protocols. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

# Contents

Contents	2

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions, symbols, abbreviations  and acronyms	8

3.1	Definitions	8

3.2	Abbreviations	8

3.3	Acronyms	8

4	Conventions,	9

5	M2M Related Protocols Overview	9

5.1	Analysis of Design Styles	9

5.1.1	RESTful Style protocols	9

5.1.1.1	REST Style	10

5.1.1.2	RESTful Protocols	11

5.1.2	SOAP Style Protocols	11

5.1.2.1	SOAP Style	11

5.1.2.2	SOAP Protocols	11

5.1.3	RPC Style Protocols	11

5.1.3.1	RPC Style	11

5.1.3.2	RPC Style Protocols	11

5.1.4	Comparison of protocol design styles	11

5.2	M2M-specific Protocols	11

5.3	General Protocols used for M2M	11

5.4	Legacy Protocols	11

5.5	Security-specific Protocols	12

5.6	Management-specific Protocols	12

5.7	Other	12

6	Industry Utilization of Protocols	12

6.1	Agriculture	12

6.2	Energy	12

6.3	Enterprise	12

6.4	Finance	12

6.5	Healthcare	12

6.6	Industrial	12

6.7	Public Services	12

6.8	Residential	13

6.9	Retail	13

6.10	Transportation	13

7	Analysis of Protocols	13

7.1	CoAP - Constrained Application Protocol	13

7.1.1	Background	13

7.1.2	Status	13

7.1.2.1	Current Status	13

7.1.2.1	Ongoing IETF Activity	14

7.1.3	Category and Architectural Style	14

7.1.4	Intended use	15

7.1.5	Deployment Trend	15

7.1.6	Key features	15

7.1.7	Protocol Stack	16

7.1.8	Data Model	17

7.1.9	Security	17

7.1.10	Dependencies	18

7.1.11	Benefits and Constraints	18

7.1.11.1	Benefits	18

7.2.11.2	Constraints	18

7.1.12	Support of oneM2M requirements	18

7.1.12.1	Fully Supported Requirements	19

7.1.12.2	Partially Supported Requirements	19

7.1.12.3	Disallowed Requirements	19

7.2	MQTT - Message Queuing Telemetry Transport	19

7.2.1	Background	19

7.2.2	Status	19

7.2.3	Category and Architectural Style	20

7.2.4	Intended use	20

7.2.5	Deployment Trend	20

7.2.6	Key features	20

7.2.6.1	Publish/Subscribe	21

7.2.6.2	Topics/Subscriptions	21

7.2.6.3	Quality of Service	21

7.2.6.4	Retained Messages	21

7.2.6.5	Clean session / Durable connection	22

7.2.6.6	Wills	22

7.2.7	Protocol Stack	22

7.2.8	Data Model	22

7.2.9	Security	22

7.2.10	Dependencies	22

7.2.11	Benefits and Constraints	22

7.2.11.1	Benefits	22

7.2.11.2	Constraints	22

7.2.12	Support of oneM2M requirements	23

7.2.12.1	Fully Supported Requirements	23

7.2.12.2	Partially Supported Requirements	23

7.2.12.3	Disallowed Requirements	23

7.3 	TIA TR-50 Protocol	23

7.3.1	Background	23

7.3.2	Status	23

7.3.3	Category and Architectural Style	23

7.3.4	Intended use	23

7.3.5	Deployment Trend	23

7.3.6	Key features	24

7.3.7	Protocol Stack	24

7.3.7.1	Frame Details	24

7.3.7.2	Request Frame Details	24

7.3.7.3	Response Frame Details	25

7.3.8	Data Model	26

7.3.9	Security	26

7.3.10	Dependencies	26

7.3.11	Benefits and Constraints	26

7.3.11.1	Benefits	26

7.3.11.2	Constraints	26

7.3.12	Support of oneM2M requirements	26

7.3.12.1	Fully Supported Requirements	26

7.3.12.2	Partially Supported Requirements	26

7.3.12.3	Disallowed Requirements	27

7.4 	HTTP as RESTful API	27

7.4.1	Description	27

7.4.2 	HTTP Status	27

7.4.2.1	HTTP/1.x Status	27

7.4.2.2	HTTP/2.0 (httpbis) Status	27

7.4.3 	Intended Use	28

7.4.4 	Deployment Trend	28

7.4.5 	Key Features	28

7.4.5.1	Relevant Instance of RESTful Design	28

7.4.5.2	Using XML and JSON	28

7.4.6 	Security	28

7.4.7 	Dependencies	28

7.4.8 	Benefits and Constrains	29

7.5 	XMPP: eXtensible Messaging and Presence Protocol	29

7.5.1	Background	29

7.5.2	Status	29

7.5.3	Category and Architectural Style	29

7.5.4	Intended use	30

7.5.5	Deployment Trend	30

7.5.6	Key features	31

7.5.7	Protocol Stack	32

7.5.7.1 	XEP-0323 Sensor data	33

7.5.7.2	XEP-0324 IoT Provisioning	33

7.5.7.3	XEP-0325 Internet of Things - Control	34

7.5.7.4	XEP-0326 Internet of Things - Concentrators	34

7.5.8	Data Model	35

7.5.9	Security	35

7.5.10	Dependencies	35

7.5.11	Benefits and Constraints	35

7.5.11.1	Benefits	35

7.5.11.2	Constraints	36

7.5.12	Support of oneM2M requirements	36

7.5.12.1	Fully Supported Requirements	36

7.5.12.2	Partially Supported Requirements	36

7.5.12.3	Disallowed Requirements	36

7.6 	WebSocket Protocol	36

7.6.1	Background	36

7.6.2	Status	37

7.6.3	Category and Architectural Style	37

7.6.4	Intended use	37

7.6.5	Deployment Trend	37

7.6.5.1	Server-Side Implementations	37

7.6.5.2	Client-Side Implementations	37

7.6.6	Key features	37

7.6.7	Protocol Stack	38

7.6.8	Data Model	38

7.6.9	Security	38

7.6.10	Dependencies	38

7.6.11	Benefits and Constraints	38

7.6.11.1	Benefits	38

7.6.11.2	Constraints	38

7.6.12	Support of oneM2M requirements	39

7.6.12.1	Fully Supported Requirements	39

7.6.12.2	Partially Supported Requirements	39

7.6.12.3	Disallowed Requirements	39

7.7 	Bluetooth® Wireless Technology	39

7.7.1	Background	39

7.7.2	Status	40

7.7.3	Category and Architectural Style	40

7.7.4	Intended use - Personal Area Network protocols	40

7.7.5	Deployment Trend - Bluetooth and Bluetooth Smart (low energy)	40

7.7.5.1	Bluetooth Smart (low energy) Technology	41

7.7.5.2	Bluetooth High Speed Wireless Technology	41

7.7.6	Key features	41

7.7.7	Protocol Stack	42

7.7.7.1	Single mode and dual mode	45

7.7.8	Data Model	47

7.7.9	Security	47

7.7.10	Dependencies	47

7.7.11	Benefits and Constraints	47

7.7.11.1	Benefits	48

7.7.11.2	Constraints	48

7.7.12	Support of oneM2M requirements	48

7.7.12.1	Fully Supported Requirements	48

7.7.12.2	Partially Supported Requirements	48

7.7.12.3	Disallowed Requirements	48

7.x 	Protocol x {template}	48

7.x.1	Background	48

7.x.2	Status	48

7.x.3	Category and Architectural Style	48

7.x.4	Intended use	48

7.x.5	Deployment Trend	49

7.x.6	Key features	49

7.x.7	Protocol Stack	49

7.x.8	Data Model	49

7.x.9	Security	49

7.x.10	Dependencies	49

7.x.11	Benefits and Constraints	49

7.x.11.1	Benefits	49

7.x.11.2	Constraints	49

7.x.12	Support of oneM2M requirements	49

7.x.12.1	Fully Supported Requirements	49

7.x.12.2	Partially Supported Requirements	49

7.x.12.3	Disallowed Requirements	50

8	Summary	50

Proforma copyright release text block	51

Annex A List of M2M-related Protocols (Informative)	52

Annex B Definitions of Radio metrics for Technologies used for M2M releated Protocols (Informative)	60

B.1 	Bluetooth® Wireless Technology	60

B.2	ZigBee (IEEE 802.15.4)	61

B.3	Ultra-Wideband (UWB)	61

B.4	Certified Wireless USB	61

B.5	Wi-Fi (IEEE 802.11)	61

B.6	Radio Frequency Identification (RFID)	62

B.7	Near Field Communication (NFC)	62

Annex Z Bibliography	63

History	64

# 1	Scope

Editor’s Note:  From oneM2M-WI-0008-Protocol-TR-V1_0.DOC (2013-06-20)

The present document will:

- Analyze the protocols, with consideration of security aspects (in cooperation with WG4 - Security) and data models (in cooperation with WG5 - Management, Abstraction & Semantics) widely considered for use within oneM2M's target industry segments

- Create a list of those protocols with which oneM2M could encapsulate and/or interoperate

Noting that: Widely used protocol mappings may be candidates for oneM2M work; 
Industry or application-specific protocol mappings to oneM2M may be done by external organizations

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Not applicable.

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	oneM2M TS-0002	oneM2M Requirements

[i.3]	IBM MQ Telemetry Ttranport (MQTT)  v3.1 Protocol Specification

[i.4]	IETF draft-ietf-core-coap-18 Constrained Application Protocol

[i.5]	TIA-4940-020, Smart Device Communications Protocol Aspects 	TIA TR-50

[i.6]	IETF RFC6120 XMPP: Core

[i.7]	IETF RFC2616 Hypertext Transfer Protocol -- HTTP/1.1

[i.8]	IETF RFC6690 Constrained RESTful Environments (CoRE) Link Format

[i.9]	IETF RFC4944 Transmission of IPv6 Packets over IEEE 802.15.4 Networks

[i.10]	IETF RFC0768  User Datagram Protocol

[i.11]	IETF RFC6347 Datagram Transport Layer Security Version 1.2

[i.12]	W3C Extensible Makup Language (XML) 1.0 (Fifth Edition)

[i.13]	IETF RFC4627 The application/json Media Type for JavaScript Object Notation (JSON)

[i.14]	IETF RFC6121 Extensible Messaging and Presence Protocol (XMPP): Instant Messaging and Presence, March 2011.

[i.15]	IETF RFC6122 Extensible Messaging and Presence Protocol (XMPP): Address Format, March 2011

[i.16]	IETF RFC4492 Elliptic Curve Cryptography (ECC) Cipher Suites for Transport Layer Security (TLS), May 2006

[i.17]	IETF RFC4422 Simple Authentication and Security Layer (SASL).

[i.18]	XMPP Standards Foundation XEP-0016 Privacy Lists

[i.19]	XMPP Standards Foundation XEP-0030 Service Discovery

[i.20]	XMPP Standards Foundation XEP-0045 Multi-user conferencing service

[i.21]	XMPP Standards Foundation XEP-0060 Publish-Subscribe service

[i.22]	XMPP Standards Foundation XEP-0079 Advanced-Message Processing

[i.23]	XMPP Standards Foundation XEP-0080 User Location

[i.24]	XMPP Standards Foundation XEP-0136 Message Archiving

[i.25]	XMPP Standards Foundation XEP-0138 Stream Compression

[i.26]	XMPP Standards Foundation XEP-0149 Time Periods

[i.27]	XMPP Standards Foundation XEP-0166 Jingle

[i.28]	XMPP Standards Foundation XEP-0167 Jingle RTP Sessions

[i.29]	XMPP Standards Foundation XEP-0177 Jingle Raw UDP Transport Method

[i.30]	XMPP Standards Foundation XEP-0198 Stream Management

[i.31]	XMPP Standards Foundation XEP-0199 XMPP Ping

[i.32]	XMPP Standards Foundation XEP-0124 Bidirectional-streams Over Synchronous HTTP (BOSH)

[i.33]	XMPP Standards Foundation XEP-0206 XMPP Over BOSH

[i.34]	XMPP Standards Foundation XEP-0203 Delayed Delivery

[i.35]	XMPP Standards Foundation XEP-0322 Efficient XML Interchange (EXI) Format for XMPP

[i.36]	XMPP Standards Foundation XEP-0323 Internet of Things – Sensor Data

[i.37]	XMPP Standards Foundation XEP-0324 Internet of Things – Provisioning

[i.38]	XMPP Standards Foundation XEP-0325 Internet of Things – Control

[i.39]	XMPP Standards Foundation XEP-0326 Internet of Things – Concentrators

[i.40]	IETF RFC6455 The Websocket Protocol, 2011

[i.n]

# 3	Definitions, symbols, abbreviations  and acronyms

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

<defined term>: <definition>

<defined term>[N]: <definition>

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations  apply:

CHG	Charging Requirement

OPR	Operational Requirement

PHY	Physical layer of the OSI model

<ABBREVIATION>	<Explanation>

## 3.3	Acronyms

For the purposes of the present document, the following acronyms apply:

6LOWPAN	IPv6 over Low power Wireless Personal Area Networks

ACT	Availability for Concurrent Transactions

AMI	Advanced Metering Infrastructure

API	Application Programming Interface

BLE	Bluetooth Low Energy

BOSH	Bidirectional-streams Over Synchronous HTTP

CoAP	Constrained Application Protocol

CoRE	Constrained RESTful Environments

CRPR	Communications Request Processing Requirement

CRUD	Create Read Update Delete

DI	Distributed Intelligence

DTLS	Datagram Transport Layer Security

EDR	Enhanced Data Rate

EXI	Efficient XML Interchange

GAP	Generic Access Profile

GATT	Generic ATTribute Profile

HATEOAS	Hypermedia As The Engine Of Application State

HIDS	Human Interface Devices

HTML	HyperText Markup Language

HTTP	Hyper Text Transfer Protocol

IESG	Internet Engineering Steering Group

IETF	Internet Engineering Task Force

IBM	International Business Machines

IP	Internet Protocol

IPSO	Internet Protocol for Smart Objects

IPR	Intellectual Property Rights

IoT	Internet of Things

JSON	JavaScript Object Notation

MAC	Media Access Control

MIME	Multipurpose Internet Mail Extensions

MQTT	Message Queuing Telemetry Transport

M2M	Machine to Machine

OASIS	Organization for the Advancement of Structured Information Standards

OSR	Overall System Requirement

P2M	Person to Machine

PC	Personal Computer

QoS	Quality Of Service

RAM	Random Access Memory

REST	REpresentational State Transfer

RF	Radio Frequency

RFC	Request For Comments

ROM	Read Only Memory

RPC	Remote Procedure Call

SASL	Simple Authentication and Security Layer

SCADA	Supervisory Control And Data Acquisition

SDO	Stanadrds Development Organisation

SER	Security Requirement

SIG	Special Interest Group

SIP	Session Initiation Protocol

SMR	Semantics Requirement

SMS	Short Message Service

SOAP	Simple Object Access Protocol

TAG	Technical Architecture Group

TIA	Telecommunications Industry Association

TCP	Transmission Control Protocol

TLS	Transport Layer Security

UCD	Unicast Connectionless Data

UDP	User Datagram Protocol

URI	Uniform Resource Identifier

WADL	Web Application Description Language

WC3	World Wide Web Consortium

WG	Working Group

WSN	Wireless Sensory Nodes

XEP	XMPP Extension Protocol

XML	eXtensible Markup Language

XMPP	Extensible Messaging and Presence Protocol

<ACRONYM>	Explanation>

# 4	Conventions,

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	M2M Related Protocols Overview

Editor’s Note: For convenience, protocols are categorized into the groups below, these will include application, service layer and other middleware, and transport layer protocols.This clause provides a brief overview of protocol groups and will be completed as a result of the progress on the later clauses.

## 5.1	Analysis of Design Styles

### 5.1.1	RESTful Style protocols

The REST architectural style was developed by W3C Technical Architecture Group (TAG) in parallel with HTTP/1.1, based on the existing design of HTTP/1.0.

REST-style architectures conventionally consist of clients and servers. Clients initiate requests to servers; servers process requests and return appropriate responses. Requests and responses are built around the transfer of representations of resources. A resource can be essentially any coherent and meaningful concept that may be addressed. A representation of a resource is typically a document that captures the current or intended state of a resource.

The client begins sending requests when it is ready to make the transition to a new state. While one or more requests are outstanding, the client is considered to be in transition. The representation of each application state contains links that may be used the next time the client chooses to initiate a new state-transition

#### 5.1.1.1	REST Style

The REST architectural style describes the following six constraints applied to the architecture, while leaving the implementation of the individual components free to design:

Client–server: A uniform interface separates clients from servers. This separation of concerns means that, for example, clients are not concerned with data storage, which remains internal to each server, so that the portability of client code is improved. Servers are not concerned with the user interface or user state, so that servers can be simpler and more scalable. Servers and clients may also be replaced and developed independently, as long as the interface between them is not altered.

Stateless: The client–server communication is further constrained by no client context being stored on the server between requests. Each request from any client contains all of the information necessary to service the request, and any session state is held in the client.

Cacheable: As on the World Wide Web, clients can cache responses. Responses must therefore, implicitly or explicitly, define themselves as cacheable, or not, to prevent clients reusing stale or inappropriate data in response to further requests. Well-managed caching partially or completely eliminates some client–server interactions, further improving scalability and performance.

Layered system: A client cannot ordinarily tell whether it is connected directly to the end server, or to an intermediary along the way. Intermediary servers may improve system scalability by enabling load-balancing and by providing shared caches. They may also enforce security policies.

Code on demand (optional): Servers can temporarily extend or customize the functionality of a client by the transfer of executable code. Examples of this may include compiled components such as Java applets and client-side scripts such as JavaScript.

Uniform interface: The uniform interface implies that the interactions between the components in a RESTful architectural style depend on the uniformity of its interface.  If any of the components do not follow these constraints, then a RESTful architectural system can result in faults.

The components in a RESTful architectural style interoperate consistently in accordance with the uniform interface’s four constraints as follow:

- Identification of resources: a resource is addressed by a unique identifier, such as URI. (e.g., http://onem2m.com/cse1/application)

- Manipulation of resources through representations: Clients manipulate representation of resources. The same exact resource can be represented to different clients in different ways. For example, a resource can be represented as HTML or as JSON.

- Self-descriptive message: A resource’s desired state can be represented within a request message. A resource’s current state may be represented within a response message.

- Hypermedia as the engine of application state (HATEOAS): a resource’s state representation includes links to related resources.

The only optional constraint of REST architecture is "code on demand". One can characterise applications conforming to the REST constraints described in this section as "RESTful". If a service violates any of the required constraints, it cannot be considered RESTful.

Editor’s Note:  The text in Clause 5.1.1 should either be written as original text, and/or the original text source should be delared as a Reference. (per WG3 PRO discussion 09 Sep 2013)

#### 5.1.1.2	RESTful Protocols

The following protocols adhere to the principles of RESTful design:

- HTTP as RESTful API

- CoAP

### 5.1.2	SOAP Style Protocols

Editor’s Note:  Input is needed for this clause

#### 5.1.2.1	SOAP Style

<text>

#### 5.1.2.2	SOAP Protocols

The following protocols adhere to the principles of SOAP style design:

- <protocol name>

- <protocol name>

### 5.1.3	RPC Style Protocols

Editor’s Note:  Input is needed for this clause

#### 5.1.3.1	RPC Style

<text>

#### 5.1.3.2	RPC Style Protocols

The following protocols adhere to the principles of RPC style design:

- <protocol name>

- <protocol name>

### 5.1.4	Comparison of protocol design styles

Editor’s Note Input is needed for this clause - <Table> should be inserted with analysis

## 5.2	M2M-specific Protocols

<Text>

## 5.3	General Protocols used for M2M

<Text>

## 5.4	Legacy Protocols

Editor’s Note: e.g. industrial control protocols

<Text>

## 5.5	Security-specific Protocols

<Text>

## 5.6	Management-specific Protocols

<Text>

## 5.7	Other

Editor’s Note: Middleware, Syntaxes, Libraries, and Operating Systems

<Text>

# 6	Industry Utilization of Protocols

Editor’s Note:  Summary of protocols used by Key Industry Segments, including where (within protocol stack and geographically) and how (profile) they are used.

<Text>

## 6.1	Agriculture

<Text>

## 6.2	Energy

<Text>

## 6.3	Enterprise

<Text>

## 6.4	Finance

<Text>

## 6.5	Healthcare

<Text>

## 6.6	Industrial

<Text>

## 6.7	Public Services

<Text>

## 6.8	Residential

<Text>

## 6.9	Retail

<Text>

## 6.10	Transportation

<Text>

# 7	Analysis of Protocols

Editor’s Note:  Analysis of Protocols relevant to oneM2M including: category, status (current release, under development), trending (increasing, flat, decreasing), support of oneM2M requirements, architectural style, intended purpose, security aspects, and data model

## 7.1	CoAP - Constrained Application Protocol

The following clauses describe the Constrained Application Protocol CoAP. [i.4]

### 7.1.1	Background

The Constrained Application Protocol (CoAP) is a specialized web transfer protocol for use with constrained nodes and constrained (e.g., low-power, lossy) networks.  The nodes often have 8-bit microcontrollers with small amounts of ROM and RAM, while constrained networks such as 6LoWPAN often have high packet error rates and a typical throughput of 10s of kbit/s.  The protocol is designed for machine-to-machine (M2M) applications such as smart energy and building automation.  CoAP provides a request/response interaction model between application endpoints, supports built-in discovery of services and resources, and includes key concepts of the Web such as URIs and Internet media types.  CoAP is designed to easily interface with HTTP for integration with the Web while meeting specialized requirements such as multicast support, very low overhead and simplicity for constrained environments.

One of the main goals of CoAP is to design a generic web protocol for the special requirements of this constrained environment, especially considering energy, building automation and other machine-to-machine (M2M) applications.  The goal of CoAP is not to blindly compress HTTP [i.7], but rather to realize a subset of REST common with HTTP but optimized for M2M applications.  Although CoAP could be used for refashioning simple HTTP interfaces into a more compact protocol, it more importantly also offers features for M2M such as built-in discovery, multicast support and asynchronous message exchanges.

CoAP is designed to easily translate to HTTP for simplified integration with the web, while also meeting specialized requirements such as multicast support, very low overhead, and simplicity.  Multicast, low overhead, and simplicity are extremely important for M2M devices, which tend to be deeply embedded and have much less memory and power supply than traditional internet devices have.

CoAP can run on most devices that support UDP or a UDP analogue, and is intended to be used for M2M / IoT segments such as home building automation and smart metering.

### 7.1.2	Status

#### 7.1.2.1	Current Status

The IETF Constrained RESTful environments (CORE) Working Group has done the major standardization work for this protocol. In order to make the protocol suitable to IoT and M2M applications, various new functionalities have been added. The protocol has completed IETF last call and is in the final stages of processing for Internet Standards documents.

CoAP is particularly targeted for small low power sensors, switches, valves and similar components that need to be controlled or supervised remotely, through standard Internet networks. CoAP is an application layer protocol that is intended for use in resource-constrained internet devices, such as wireless sensory network (NSN) nodes.

#### 7.1.2.1	Ongoing IETF Activity

CoAP is being standardized with in the IETF CORE working group.  Key IETF CoRE WG documents are as follows:

•	CoRE Link Format – RFC6690 [i8]

•	CoAP Protocol [i.4] –  With IESG for publication as an RFC

•	Blockwise transfer in CoAP: IETF draft. WG document.

•	Observing resources in CoAP –  IETF draft. WG document.

•	CoRE Resource Directory – IETF draft. WG document

•	Group communication for CoAP – IETF draft. WG document.

•	Best practices for HTTP to CoAP Mapping Implementation –  IETF draft. WG document.

There are several options proposed for use with CoAP. Some of these are as follows:

•	Conditional observe in CoAP: IETF draft

•	CoAP Patience option:  IETF draft

•	Enhanced sleep mode support of  IoT / M2M devices: IETF draft

•	Stateful observation in CoAP (to optimize re-registration traffic in the network): IETF draft

•	Minimum request interval for successive CoAP requests – IETF draft

•	Transport of CoAP over SMS – IETF draft

•	TCP transport for CoAP – IETF draft

•	CoAP option to indicate payload length – IETF draft

•	CoAP over SMS – IETF draft

Editor’s Note:  IETF CORE WG Status and drafts should be reviewed and updated prior to publication.

### 7.1.3	Category and Architectural Style

The Constrained Application Protocol (CoAP) is a specialized web transfer protocol for use with constrained nodes and constrained (e.g., low-power, lossy) networks. The nodes often have 8-bit microcontrollers with small amounts of ROM and RAM, while constrained networks such as 6LoWPAN often have high packet error rates and a typical throughput of 10s of kbit/s. The protocol is designed for machine-to-machine (M2M) applications such as smart energy and building automation. CoAP provides a request/response interaction model between application endpoints, supports built-in discovery of services and resources, and includes key concepts of the Web such as URIs and Internet media types. CoAP is designed to easily interface with HTTP for integration with the Web while meeting specialized requirements such as multicast support, very low overhead and simplicity for constrained environments.

The use of web services (web APIs) on the Internet has become ubiquitous in most applications, and depends on the fundamental Representational State Transfer [REST] architecture of the web. The Constrained RESTful Environments (CoRE) work aims at realizing the REST architecture in a suitable form for the most constrained nodes (e.g. 8-bit microcontrollers with limited RAM and ROM) and networks (e.g. 6LoWPAN [i.9]). Constrained networks such as 6LoWPAN support the fragmentation of IPv6 packets into small link- layer frames, however incurring significant reduction in packet delivery probability. One design goal of CoAP has been to keep message overhead small, thus limiting the need for fragmentation.

One of the main goals of CoAP is to design a generic web protocol for the special requirements of this constrained environment, especially considering energy, building automation and other machine-to-machine (M2M) applications. The goal of CoAP is not to blindly compress HTTP [i.7], but rather to realize a subset of REST common with HTTP but optimized for M2M applications. Although CoAP could be used for refashioning simple HTTP interfaces into a more compact protocol, it more importantly also offers features for M2M such as built-in discovery, multicast support and asynchronous message exchanges.

The protocol supports the caching of responses in order to efficiently fulfil requests. Simple caching is enabled using freshness and validity information carried with CoAP responses. A cache could be located in an endpoint or an intermediary.

Proxying is useful in constrained networks for several reasons, including network traffic limiting, to improve performance, to access resources of sleeping devices or for security reasons. The proxying of requests on behalf of another CoAP endpoint is supported in the protocol. When using a proxy, the URI of the resource to request is included in the request, while the destination IP address is set to the address of the proxy.

As CoAP was designed according to the REST architecture [REST] and thus exhibits functionality similar to that of the HTTP protocol, it is quite straightforward to map from CoAP to HTTP and from HTTP to CoAP. Such a mapping may be used to realize an HTTP REST interface using CoAP, or for converting between HTTP and CoAP. This conversion can be carried out by a cross-protocol proxy ("cross-proxy"), which converts the method or response code, media type, and options to the corresponding HTTP feature.

### 7.1.4	Intended use

CoAP (Constrained Application Protocol) over UDP is used for resource constrained, low-power sensors and devices connected via lossy networks, especially when there is a high number of sensors and devices within the network. Soon to be released as a suite of IETF RFCs, CoAP has already found success as a key enabling technology for electric utility AMI (advanced metering infrastructure) and DI (distributed intelligence) applications

CoAP makes use of two message types, requests and responses, using a simple binary base header format. The base header may be followed by options in an optimized Type-Length-Value format. CoAP is by default bound to UDP and optionally to DTLS, providing a high level of communications security.

### 7.1.5	Deployment Trend

A First IoT CoAP Plugtest interoperability event  organized by ETSI, the IPSO Alliance. and the Probe-IT project was held in March 2012. This interoperability event tested features that included the base CoAP specification, CoAP Block Transfer, CoAP Observation and the the CoRE Link Format.  As described in draft-bormann-core-roadmap-03, it was attended by 18 companies and more than 3000 tests were performed during this event. The 2nd CoAP plugtest event was held in November 2012.

### 7.1.6	Key features

The key features of CoAP are:

- CoAP is a RESTful protocol.

- Four methods similar to HTTP: Get, Put, Post and Delete.

- Three types of response code: 2.xx (success), 4.xx (client error) and 5.xx (server error).

- Four different message types: Confirmable, Non-Confirmable, Acknowledgement and Reset (Nack).

- Synchronous message exchange

- Asynchronous message exchange via Observe / Notifications  Client uses Observe option with Get request to indicate interest in getting further updates from server. Client receives an asynchronous notification each time state of resource changes at server.

- Conditional Observe allows CoAP clients to be informed only when certain conditions on observed resources are met (such as inform periodically or only inform when observed value changes by a pre-specified step size)

- East to proxy to and from HTTP.

- Constrained web protocol fulfilling M2M requirements.

- UDP [i.10] binding with optional reliability supporting unicast and multicast requests. Confirmable and Acknowledgement / Reset messages to provide optional reliability when required. Asynchronous message exchanges.

- Low header overhead and parsing complexity.

- URI and Content-type support.

- Simple proxy and caching capabilities.

- A stateless HTTP mapping, allowing proxies to be built providing access to CoAP resources via HTTP in a uniform way or for HTTP simple interfaces to be realized alternatively over CoAP.

- Security binding to Datagram Transport Layer Security (DTLS) (RFC6347 [i.11]) A wide variety of key management mechanisms may be used for this purpose.

- CoRE link format that defines use of Web Linking using link formats for use by constrained devices to describe hosted resources, their attributes and relationships between links (RFC6690 [i.8]) A well known URI “./well-known/core” is used as a default entry point for requesting list of links about the resources hosted by a server

- A Resource Directory mechanism where IoT / M2M devices (i.e. CoAP servers) can register / update their list of resources. It stores the URIs (called links) to resources stored on servers. If a device is in sleep mode and not able to communicate with the network, it can be discovered via this resource directory. It could also be used if network doesn’t support multicast traffic efficiently.

- Mechanism to transfer multiple blocks of information from a resource representation in multiple request-response pairs at CoAP message level itself (i.e. without relying on IP fragmentation). Large file transfers (such as firware updates) can be done using this mechanism.

- Group based communication using (unreliable) IP multicast by source sending non-confirmable CoAP message to a multicast IP address  (and via serial unicast for links that do not support multicast).  Some other group based communication mechanisms being explored include the following: overlay multicast that uses proxies to deliver IP packets to end devices, and support of multicast at CoAP level without any explicit multicast support from lower layers.

- CoAP Patience option that informs a recipient of the preferred time frame for a response or request depending on usage context. Useful for time (or delay) tolerant exchanges

- Proposal for a mechanism where device can register its sleep state and related parameters (such as sleep duration, sleep / active state etc.) with the Resource Directory

- Stateful Observation intends to reduce overhead in the network due to multiple re-registration requests from CoAP client to CoAP server when server (i.e. IoT / M2M device) is not in a position to accept additional clients

- Identification of Proxies between a CoAP client and CoAP server.

- Provides mechanism where a client and server can negotiate the minimum time between two subsequent requests. Helps to reduce excessive load at the CoAP server

- An IETF draft “CoAP Payload Length Option Extension” defines a way to indicate length of the payload when underlying transport layer (such as for RS 232, RS 422 or RS 485) doesn’t indicate payload length.

- Mechanism to transport CoAP over SMS for cellular networks

- Representation of links in JSON format in unconstrained environment

- Editor’s Note:  IETF draft reference and status should be reviewed and updated prior to publication.

### 7.1.7	Protocol Stack

The interaction model of CoAP is similar to the client/server model of HTTP. However, machine-to-machine interactions typically result in a CoAP implementation acting in both client and server roles. A CoAP request is equivalent to that of HTTP, and is sent by a client to request an action (using a method code) on a resource (identified by a URI) on a server. The server then sends a response with a response code; this response may include a resource representation.

Unlike HTTP, CoAP deals with these interchanges asynchronously over a datagram-oriented transport such as UDP. This is done logically using a layer of messages that supports optional reliability (with exponential back-off). CoAP defines four types of messages: Confirmable, Non-confirmable, Acknowledgement, Reset; method codes and response codes included in some of these messages make them carry requests or responses. The basic exchanges of the four types of messages are somewhat orthogonal to the request/response interactions; requests can be carried in Confirmable and Non- confirmable messages, and responses can be carried in these as well as piggy-backed in Acknowledgement messages. One could think of CoAP logically as using a two-layer approach, a CoAP messaging layer used to deal with UDP and the asynchronous nature of the interactions, and the request/response interactions using Method and Response codes (see Figure 7.1.7 below). CoAP is however a single protocol, with messaging and request/response just features of the CoAP header.

+----------------------+

|      Application     |

+----------------------+

+----------------------+ \

| Requests/Responses | |

|----------------------| | CoAP

|       Messages       | |

+----------------------+ /

+----------------------+

|          UDP         |

+----------------------+

Fig. 7.1.7 Abstract layering of CoAP

### 7.1.8	Data Model

CoAP allows to explicitly indicate payload of the content type in its header.  CoAP Content Format Registry provides following initial entries: plain text, XML, JSON, EXI, octet stream, link-format.  New Internet media types may be used depending on the target IoT segment

### 7.1.9	Security

As CoAP realizes a subset of the features in HTTP/1.1, the security considerations of RFC2616 [i.7] are also pertinent to CoAP. This section analyzes the possible threats to the protocol.  There are a number of security limitations with CoAP, and this section will describe those in detail.  These will include:

- Protocol Parsing, Processing URIs

- Proxying and Caching

- Risk of amplification

- IP Address Spoofing Attacks

- Cross-Protocol Attacks

- Constrained node considerations

COAP uses DTLS1.2 and security keys generated by DTLS are used to protect CoAP level messages. Some constraints associated with DTLS are as follows:

- It may be challenging to support DTLS in constrained M2M devices that have limited memory (such as RAM ~ 10 KB) and processing power. This is the reason for the current IETF initiative “DTLS In Constrained Environments” (DICE) initiative (http://www.ietf.org/proceedings/87/slides/slides-87-dice-0)

- Use of DTLS (handshake protocol) results in high overhead in the network and that may not be desirable.

- No clear standardized definition of a constrained DTLS profile

No efficient support of multicast with IP DTLS. The multicast suitability of CoAP are lost when using DTLS (point-to-point). On this aspect, there are also initiatives attempting  to find solutions, e.g. “DTLS-based multicast security for Low power Lossy Networks” (http://tools.ietf.org/id/draft-keoh-tls-multicast-security-00.txt)

- No standardized approaches for (dynamic) key management for group based communication

Editor’s Note:  Reference to IETF Drafts need to be checked prior to publication.

### 7.1.10	Dependencies

CoAP is designed to run over datagram transport protocol such as UDP. In this case, it uses DTLS to provide application layer security.

An IETF draft “A TCP transport for CoAP” is exploring changes needed to run CoAP over TCP.  Use of CoAP over RS 232 / 422 / 485 is also being explored.

Editor’s Note:  IETF draft reference and status should be reviewed and updated prior to publication.

### 7.1.11	Benefits and Constraints

Editor’s Note: To be completed

#### 7.1.11.1	Benefits

CoAP is a lightweight application layer protocol designed for constrained devices (such as devices with 8-bit microcontroller and limited memory) and constrained  networks (such as low power, low data rate, lossy networks that use IEEE802.15.4)

•	It runs over UDP and avoids overhead of TCP

•	It is easy to do HTTP – CoAP translation

#### 7.2.11.2	Constraints

CoAP has:

•	Constraints associated with DTLS (as listed in the Security subsection)

•	No standardized framework for authorization and access control for CoAP exists as of now. The IETF draft “Access Control Framework for constrained environments” (http://datatracker.ietf.org/doc/draft-selander-core-access-control/?include_text=1)  attempts to resolve this issue.

Editor’s Note:  Reference to IETF Draft needs to be checked prior to publication.

•	No explicit support for real-time IoT application at present.

### 7.1.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by CoAP is shown in the following clauses:

#### 7.1.12.1	Fully Supported Requirements

OSR-001, OSR-002, OSR-008, OSR-009, OSR-010, OSR-014, OSR-21, OSR-24, OSR-25, OSR-28, OSR-30, OSR-37…

Editor’s Note: To be reviewed and completed in alignment with Approved oneM2M Requirements

#### 7.1.12.2	Partially Supported Requirements

OSR-00, OSR-013, OSR-015, OSR-020, OSR-022, OSR-029, OSR-030, OSR-033,OSR-035, OSR-036, OSR-040…

Editor’s Note: To be completed

#### 7.1.12.3	Disallowed Requirements

Editor’s Note: To be completed

## 7.2	MQTT - Message Queuing Telemetry Transport

<Text> Editor’s Note: To be completed

### 7.2.1	Background

MQTT was invented by IBM and Arcom (now Eurotech), in 1999. It was designed for low-bandwidth, high latency networks. As a result, the designers made a number of key choices which influenced the way it “looks and feels”.

- Simplicity, simplicity, simplicity! Don't add too many “bells and whistles” but provide a solid building block which can easily be integrated into other solutions. Be simple to implement.

- Publish/subscribe messaging. Useful for most sensor applications, and enables devices to come online and publish “stuff” that hasn't been previously known about or predefined.

- Zero administration (or as close as possible). Behave sensibly in response to unexpected actions and enable applications to “just work” e.g. dynamically create topics when needed.

- Minimise the on-the-wire footprint. Add an absolute minimum of data overhead to any message. Be lightweight and bandwidth efficient.

- Expect and cater for frequent network disruption (for low bandwidth, high latency, unreliable, high cost-to-run networks)… → Last Will and Testament

- Continuous session awareness → Last Will and Testament

- Expect that client applications may have very limited processing resources available.

- Provide traditional messaging qualities of service where the environment allows. Provide “quality of service”

- Data agnostic. Don't mandate content formats, remain flexible.

MQTT v3.1 was published by IBM in Aug 2010 under a royalty free license. Further pre-OASIS information is available at MQTT.org.

### 7.2.2	Status

Based on the pre-standard MQTT v3.1 specifications [i.3] there is an OASIS standardization process which started in March 2013 to make MQTT an open, simple and lightweight standard protocol for M2M telemetry data communication. The target for completion is March 2014.

The OASIS MQTT TC is producing a standard for the Message Queuing Telemetry Transport Protocol compatible with MQTT V3.1, together with requirements for enhancements, documented usage examples, best practices, and guidance for use of MQTT topics with commonly available registry and discovery mechanisms. It operates under the Non-Assertion Mode of the OASIS IPR Policy. Changes to the input document, other than editorial changes and other points of clarification, will be limited to the Connect command, and should be backward compatible with implementations of previous versions of the specification such that a client coded to speak an older version of the protocol will be able to connect to, and successfully use, a server that implements a newer version of the protocol. Candidates for enhancements include message priority and expiry, message payload typing, request/reply, and subscription expiry.

The Eclipse foundation through their M2M working group, is providing open source MQTT client code via their Paho Project.

### 7.2.3	Category and Architectural Style

MQTT is an M2M/Internet of Things (IoT) connectivity protocol. It is connection session reliant. It supports 14 command messages; the message format includes a fixed and variable header plus the payload.

The grouped commands are:

•	Client requests a connection to a server, Acknowledge connection request & Disconnect notification

•	Publish message & Publish acknowledgment

•	Assured publish received (part 1), Assured publish release (part 2) & Assured publish complete (part 3)

•	Subscribe to named topics & Subscription acknowledgement

•	Unsubscribe from named topics & Unsubscribe acknowledgment

•	Ping request & Ping response

### 7.2.4	Intended use

MQTT is designed to support messaging transport from remote locations/devices involving small code footprints (e.g., 8-bit, 256KB ram controllers), low power, low bandwidth, high-cost connections, high latency, variable availability, and negotiated delivery guarantees. For example, MQTT is being used in sensors communicating to a server / broker via satellite links, SCADA, over occasional dial-up connections with healthcare providers (medical devices), and in a range of home automation and small device scenarios. MQTT is also a fit for mobile applications because of its small size, minimized data packets, and efficient distribution of information to one or many receivers (subscribers).

### 7.2.5	Deployment Trend

MQTT is estimated to be running on 250k devices. It is deployed in the Healthcare Industry Segment  (hospitals use the protocol to communicate with pacemakers and other medical devices) and in the Energy Industry Segment (oil and gas companies use MQTT to monitor thousands of miles of oil pipelines). It is also used in Facebook’s Messenger application.

MQTT is not deployed in the largest message queue-based telemetry projects.

### 7.2.6	Key features

The key features of MQTT are:

•	Publish/Subscribe - to provide one-to-many message distribution and decoupling of applications

•	Topics/Subscriptions – to categorise messages into channels for delivery to subscribers

•	Quality of Service – to provide different assurances of message delivery

•	Retained messages – to provide past published messages to new subscribers

•	Clean session / Durable connections – to choose whether a client’s state is to be stored between connection sessions

•	Wills – to send messages after a client disconnects unexpectedly

#### 7.2.6.1	Publish/Subscribe

The MQTT protocol is based on the principle of publishing messages and subscribing to topics, or "pub/sub". Multiple clients connect to a server / broker and subscribe to topics that they are interested in. Clients also connect to the broker and publish messages to topics. Many clients may subscribe to the same topics. The server / broker and MQTT act as a simple, common interface for clients to connect to. A publisher may publish a message once and be received by multiple subscribers.

Figure 7.2.6.1 MQTT publish-subscribe messaging

#### 7.2.6.2	Topics/Subscriptions

Messages in MQTT are published on topics. Topics are structured into topic trees, which are treated as hierarchies, using a forward slash (/) as a separator. This allows arrangement of common themes to be created. Topic trees and topics are created by using publish messages.

A subscription may contain special characters, which allow clients to subscribe to multiple topics at once, within a single level or within multiple levels in a topic tree.

#### 7.2.6.3	Quality of Service

MQTT defines three levels of Quality of Service (QoS). The QoS defines how hard the broker & client will try to ensure that a message is received. Messages may be sent at any QoS level, and clients may attempt to subscribe to topics at any QoS level. This means that the client chooses the maximum QoS it will receive. For example, if a message is published at QoS 2 and a client is subscribed with QoS 0, the message will be delivered to that client with QoS 0. If a second client is also subscribed to the same topic, but with QoS 2, then it will receive the same message but with QoS 2. For a second example, if a client is subscribed with QoS 2 and a message is published on QoS 0, the client will receive it on QoS 0.

Higher levels of QoS are more reliable, but involve higher latency and have higher bandwidth requirements.

0.	The server / broker & client will deliver the message once, according to the best efforts of the underlying TCP/IP network, with no confirmation. The message arrives at the server either once or not at all.

1.	The server / broker & client will deliver the message at least once, with confirmation required. If there is an identified failure of either the communications link or the sending device, or the acknowledgement message is not received after a specified period of time, the sender resends the message

2.	The server / broker & client will deliver the message exactly once by using additional protocol flows.

#### 7.2.6.4	Retained Messages

Publish messages may be set to be retained. This means that the server / broker will keep the message even after sending it to all current subscribers. If a new subscription is made that matches the topic of the retained message, then the message will be sent to the client. This is useful as a "last known good" mechanism. If a topic is only updated infrequently (such as for “report by exception”), then without a retained message, a newly subscribed client may have to wait a long time to receive an update. With a retained message, the client will receive an instant update.

#### 7.2.6.5	Clean session / Durable connection

MQTT clients choose whether to use clean or durable connection session. If a clean connection is used then the server / broker discards any previously maintained information about the client, the client needs to re-subscribe to topics of interest, and the server / broker discards any state when the client disconnects.

If a durable connection is used, then when the client disconnects, any subscriptions it has will remain and any subsequent QoS 1 or QoS 2 messages will be stored until it connects again.

#### 7.2.6.6	Wills

When a client connects to a broker, it may inform the broker that it has a Will. This is a message that it wishes the broker to send to interested parties when the client disconnects abnormally. The Will message has a topic, QoS and retain status just the same as any other message. Abnormal disconnection occurs when either an I/O error is encountered by the server / broker during communication with the client, or the client fails to communicate within the Keep Alive timer schedule.

### 7.2.7	Protocol Stack

Editor’s Note: To be completed

### 7.2.8	Data Model

No formal data model has been published. The data elements included in the version 3.1 specification include: topic trees, user name and password, connection state, subscriptions, retained messages, and message headers (fixed and variable).

### 7.2.9	Security

MQTT supports user names and passwords in connection requests. Connections can be refused due to a bad user name or password.

### 7.2.10	Dependencies

MQTT uses the TCP/IP layer to provide basic network connectivity.

### 7.2.11	Benefits and Constraints

#### 7.2.11.1	Benefits

- Protocol compressed into bit-wise headers and variable length fields. Typical message header size is 6 bytes.

- MQTT has been implemented in devices with less than 64kb of RAM

- In comparison to HTTPS, MQTT tested faster throughput, required less battery, and less network overhead.

#### 7.2.11.2	Constraints

- The simple user name and password scheme is insufficient to support comprehensive access control

- MQTT does not permit fragmentation of messages, making it difficult to transmit large messages to constrained memory devices

- MQTT does not support transactions; it does support basic acknowledgments.

- MQTT does not address connection security

- MQTT does not support discovery of clients or servers

- MQTT is not extensible, requiring a new protocol revision to evolve capabilities

### 7.2.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by MQTT is shown in the following clauses:

#### 7.2.12.1	Fully Supported Requirements

Editor’s Note: To be reviewed and completed in alignment with Approved oneM2M Requirements

Editor’s Note:  Some of the following requirements may only be “Partially Supported”; and some of the requirements listed are not fully agreed within REQ WG1.  The present list needs further review and input.

OSR-001, OSR-002, OSR-003, OSR-008, OSR-009, OSR-012, OSR-20, OSR-21, OSR-24, OSR-25, OSR-29, OSR-30, OSR-32, OSR-39, SMR-001, SMR-002, SMR-004, SER-002, SER-008, SER-011, and SER-025.

#### 7.2.12.2	Partially Supported Requirements

Editor’s Note: To be reviwed:

#### 7.2.12.3	Disallowed Requirements

Editor’s Note: To be completed

## 7.3 	TIA TR-50 Protocol

The following clauses describe an implementation of the oneM2M “x” Interface based on the current TIA TIA-4940-020 [i.5].

### 7.3.1	Background

Editor’s Note: To be completed

### 7.3.2	Status

Editor’s Note: To be completed

### 7.3.3	Category and Architectural Style

Editor’s Note: To be completed

### 7.3.4	Intended use

Editor’s Note: To be completed

### 7.3.5	Deployment Trend

Editor’s Note: To be completed

### 7.3.6	Key features

Editor’s Note: To be completed

### 7.3.7	Protocol Stack

The TIA TR-50 Protocol is based on a frame that is flexible to provide extensions for further enhancements and/or improvements. Communication on the onem2M “x” Interface assumes a reliable network connection.

#### 7.3.7.1	Frame Details

The communication is based on:

- Requests

- Responses

#### 7.3.7.2	Request Frame Details

The M2M request frames are based on a JSON structure and consist of two major sections:

- Authentication – this is the place where the entire authentication items are placed.

- Command(s) – this is the place where the commands items are placed

Below is the structure of the OneM2M frame:

{

"auth": {

"applicationToken": "<application token>",

"sessionId": "<session token>"

},

"ref": {

"command": "<command keyword>",

"params": []

}

}

In order to minimize the traffic, it is possible to have M2M frames with multiple commands:

"ref1": {

"command": "<first command>",

"params": []

},

"ref2": {

"command": "<second command>",

"params": []

}

The description of the fields is presented in Table 7.3.7.2 below

Table 7.3.7.2: Request Frame Field Descriptions

| Name | Description | Type | Mandatory |
| --- | --- | --- | --- |
| auth | Keyword. Identifies the authentication stanza in the M2M request frame | String | Yes |
| applicationToken | Keyword. Identifies the application making the request | String | Yes |
| sessionId | Keyword. Unique ID received after the use of the authentication services. Identifies the current session between the two entities | String | Yes |
| ref1, ref2, ref3 | Identifies the commands that are in the request. Can be simple identifiers. They are not keyword. It is expected that the response will contain them. | String | Yes |
| command | Keyword. Identifies a known command that can be executed | String | Yes |
| params | Keyword. Identifies the parameters required by the command. The field must exist, but it can be empty. | String | Yes |

#### 7.3.7.3	Response Frame Details

The response frame is based on the JSON format and it has the following structure:

{

"ref1": {

"success": true,

"params": []

},

"ref2": {

"success": false,

"errorCodes":

[

<errorCode1>,

<errorCode2>

]

"errorMessages":

[

<errorMessage>,

<errorMessage>

]

},

}

- “ref1” response is an example for a positive return of a request

- “ref2” response is an example for an negative or error return of a request

The description of the fields is presented Table 7.3.7.3 below

Table 7.3.7.3: Response Frame Field Descriptions

| Name | Description | Type | Mandatory |
| --- | --- | --- | --- |
| ref1,ref2, ref3 | Identifies the request commands. | String | Yes |
| success | Keyword. Identifies the response. Can be true or false | String | Yes |
| errorCodes | Keyword. Identifies the error codes . |  |  |
| errorMessages | Keyword. Identifies the error message. | String | No |
| params | Keyword. Identifies the response parameters. Can be empty. | String | Yes |

### 7.3.8	Data Model

Editor’s Note: To be completed

### 7.3.9	Security

Editor’s Note: To be completed

### 7.3.10	Dependencies

Editor’s Note: To be completed

### 7.3.11	Benefits and Constraints

Editor’s Note: To be completed

#### 7.3.11.1	Benefits

Editor’s Note: To be completed

#### 7.3.11.2	Constraints

Editor’s Note: To be completed

### 7.3.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by <protocol x> is shown in the following clauses:

Editor’s Note: The requirments listed herein will need review and alignment with the final approved oneM2M Requirements TS-0002 [i.2]

#### 7.3.12.1	Fully Supported Requirements

Editor’s Note: Per discussion in PRO WG3 - TP#6

#### 7.3.12.2	Partially Supported Requirements

Editor’s Note: Added per discussion in PRO WG3 - TP#6

#### 7.3.12.3	Disallowed Requirements

Editor’s Note: Added per discussion in PRO WG3 - TP#6

## 7.4 	HTTP as RESTful API

### 7.4.1	Description

HTTP protocol is widely used for Web Services in different ways . One of those usages is using HTTP as RESTful API (HTTP-REST-API).

Editor’s Note: Need to add additional examples other than RESTful API.

HTTP-REST-API, will provides CRUD (Create/Read/Update/Delete) operational primitives naturally with its method (e.g. POST/GET/PUT/DELETE) and well-defined status codes.

HTTP-REST-API is subset of HTTP with RESTful  design style.

Editor’s Note:  further text may added to improve wording

HTTP-REST-API usually supports XML or JSON (JavaScript Object Notation) for passing API parameters. HTTP-REST-API can handle various data formats using Content-Negotiation feature which is part of HTTP specification.

### 7.4.2 	HTTP Status

#### 7.4.2.1	HTTP/1.x Status

- HTTP version 1.1 was published as RFC 2616 [i.7] (Draft Standard) in June 1999.

- Extensible Markup Language (XML) 1.0 (Fifth Edition) [i.12] was published as W3C Recommendation on 26 November 2008.

- “The application/json Media Type for JavaScript Object Notation (JSON)” [i.13] was published as RFC4627 on July 2006.

#### 7.4.2.2	HTTP/2.0 (httpbis) Status

The Hypertext Transfer Protocol Bis (httpbis) Working Group of the IETF is working on HTTP/2.0, which is intended to supersede HTTP/1.1.  It is expected that HTTP/2.0 will be submitted to IESG for consideration as a Proposed Standard in Nov 2014. HTTP/2.0 is intended to retain the semantics of HTTP without the legacy of HTTP/1.x message framing and syntax, which have been identified as hampering performance and encouraging misuse of the underlying transport.  As part of the HTTP/2.0 work, the following issues are being considered:

•	A negotiation mechanism that is capable of not only choosing between HTTP/1.x and HTTP/2.x, but also for bindings of HTTP URLs to other transports (for example).

•	Header compression (which may encompass header encoding or tokenisation)

•	Server push (which may encompass pull or other techniques)

It is expected that HTTP/2.0 will:

•	Substantially improve end-user perceived latency in most cases, over HTTP/1.1 using TCP.

•	Address the "head of line blocking" problem in HTTP/1.1 created by pipelining

•	Not require multiple connections to a server to enable parallelism, thus improving its use of TCP, especially regarding congestion control

•	Retain the semantics of HTTP/1.1, including HTTP methods, status codes, URIs, and where appropriate, header fields.

•	Define how HTTP/2.0 interacts with HTTP/1.x, (both 2->1 and 1->2).

•	Identify any new extensibility points and policy for their appropriate use.

The resulting specification(s) are expected to meet these goals for common existing deployments of HTTP; in particular, Web browsing (desktop and mobile), non-browsers ("HTTP APIs"), Web serving (at a variety of scales), and intermediation (by proxies, corporate firewalls, "reverse" proxies and Content Delivery Networks). Likewise, current and future semantic extensions to HTTP/1.x (e.g., headers, methods, status codes, cache directives) should be supported in HTTP/2.0.

### 7.4.3 	Intended Use

HTTP-REST-API provides easy to understand, scalable, secure APIs for Web service in distributed computing environments, such as the Internet.

### 7.4.4 	Deployment Trend

According to the ‘API Directory’ provided by independent web site ‘programableweb.com’, over 6000 RESTful APIs are published (as of Aug 27th, 2013).

Since API’s value can be enhanced by combined use of other APIs, called ‘mush up’, choosing API to be ‘RESTful’ potentially  increase its value as twice or more.

W3C published WADL specification to describe HTTP-REST-API. Several tool can generate skelton codes for web application from WADL definition.

### 7.4.5 	Key Features

#### 7.4.5.1	Relevant Instance of RESTful Design

Since HTTP specification are designed to implement RESTful architecture, you can develop robust, secure, scalable system with HTTP-REST-API.

HTTP-REST-API also can be secured by applying TLS. Unlike other solutions, TLS will not imply the complexity. Additionally, there are many hardware-based solutions to accelerate crypt graphical processing like load balancer, embedded co-processor.

#### 7.4.5.2	Using XML and JSON

Both XML and JSON can carry extensible data structures easily.

Even JSON format can transfer same information in smaller size than XML, XML can provide strong message-level security, like partial encryption and/or digital signature which cannot be provided by JSON.

REST isn't just about JSON or XML though, but any of the media types that the browser or platform can natively handle with content negotiation mechanism which is part of HTTP specification.

### 7.4.6 	Security

HTTP-REST-API is based Web services are prone to the same vulnerabilities as standard web applications, including broken authentication, injection attacks, cross-site scripting and cross-site request forgery.

Fortunately, many HTTP security practices can be successfully applied for securing HTTP-REST-API.

The Web Service with HTTP-REST-API can be secured by those rules with configuring a policy, ensuring that access to the service requires usage of TLS and authorizing service access based on group membership.

### 7.4.7 	Dependencies

HTTP-REST-API depends on HTTP, XML, JSON, and MIME specifications.

### 7.4.8 	Benefits and Constrains

The greatest benefit of HTTP-REST-API is the use of HTTP, but the largest pitfall can also be the use of HTTP.

## 7.5 	XMPP: eXtensible Messaging and Presence Protocol

The following clauses describe the eXtensible Messaging and Presence Protocol (XMPP). [i.6]

### 7.5.1	Background

XMPP was first proposed by Jabber open source community and later formalized by IETF in RFC3920.  It is an open XML-based protocol for near real-time messaging, presence and request-response services.  Several extensions have been added to achieve other capabilities.

### 7.5.2	Status

IETF RFCs and drafts:

•	RFC6120: XMPP: Core (Standards Track RFC. Obsoleted RFC3920) [i.6]. It defines the base XMPP protocol along with RFC6121.

•	RFC6121: XMPP: Instant Messaging and Presence [i.14]. Standards track RFC (obsoletes RFC 3921)

•	RFC6122: XMPP Address Format (Standards Track) [i.15].  Updates RFC3920

XEP (XMPP Extension Protocol) documents specify extensions to XMPP and are standardized by XSF (XMPP Standards Foundation, http://xmpp.org/extensions/).

### 7.5.3	Category and Architectural Style

XMPP uses a federated client-server model with multiple interconnected servers as shown in Figure 7.5a.  Each server is responsible for managing its own domain and works cooperatively with servers of other domains as peers.

XMPP supports Availability for Concurrent Transactions (ACT) style where asynchronous end-to-end exchange of structured data is carried out using direct and persistent XML streams among a distributed network of globally addressable, presence aware clients and servers (RFC6120 [i.6]).

Figure 7.5a: XMPP Federated Client – Server Model

Editor’s Note: Need editable source for Fig 7.5a

### 7.5.4	Intended use

XMPP is intended to be used for P2P, P2M and M2M / IoT purposes.

### 7.5.5	Deployment Trend

Millions of users world-wide use XMPP for instant messaging and presence based applications.

List of some servers that support XMPP is given at http://xmpp.org/xmpp-software/servers/.  These servers provide basic messaging, presence and XML routing features.  These servers are available for different platforms such as Linux, Solaris, Windows and Mac OS X.

A list of XMPP clients is available at http://xmpp.org/xmpp-software/clients/. Clients are available for different platforms such as Linux, Windows, Android, Blackberry, iOS, Mac OS X, J2ME, Palm OS and Browser.

A list of XMPP software libraries is available at http://xmpp.org/xmpp-software/libraries/. These libraries are implemented in various languages such as C, C++, Java, Perl, Ruby, PHP, Python, JavaScript, Tcl, Objective C, Flash / Action Script and C # /.Net/Mono.

XMPP is used by the Jabber messaging client. The first instant messaging service based on  XMPP was Jabber.org.

- Jabber is used for text conferencing of IETF meetings.

- Cisco / WebEx uses XMPP.

- Google Talk uses XMPP protocol for instant messaging and presence.   It uses extensions of XMPP for VoIP, video and peer-to-peer communication.

- Microsoft provides XMPP interface to its Microsoft Messenger Service and have XMPP gateways integrated in their messaging systems.

- Facebook presents an XMPP interface to its clients for its chart feature

### 7.5.6	Key features

XMPP supports Availability for Concurrent Transactions (ACT) style (RFC6120 [i.6]).

XMPP supports a distributed client server architecture where a client needs to connect with a server to gain access to network. Only after that, it is allowed to exchange XML stanzas with other entities in the network (e.g. in a different domain). End-to-end communication is logically peer-to-peer but physically client-to-server, server-to-server and server-to-client.

TLS (RFC 4492 [i.16]) is supported for encryption purposes (between client – server and server – server).

SASL (Simple Authentication and Security Layer, RFC 4422 [i.17]) is used for authentication of initiating entity (e.g. an XMPP client) with the receiving entity (e.g. XMPP server) before the initiating entity is allowed to send XML stanzas to receiving entity.

Server to server model allows authenticated and secure inter-domain communication.

Each domain is controlled by a server in a decentralized architecture. Each domain owner can define level of security needed, QoS and policies that are needed for that domain.

Relevant XMPP Extension Protocols (XEPs) include:

- XEP-0016 Privacy Lists [i.18]: It can be used to block communication from some XMPP users. It can be potentially used for M2M applications as well. In some sense, it allows to implement simple policies to allow or block access to an M2M device from another M2M device.

- XEP-0030 Service Discovery [i.19]: Allows to discover XMPP entities and features supported by these entities.

- XEP-0045 Multi-user conferencing service. [i.20]

- XEP-0060 Publish-Subscribe service [i.21]. It enables a service to generate notifications and deliver those to multiple subscribers. It is more generalized than the special form of publish-subscribe model supported by presence service. Personal Event Profile (XEP-0163) specifies a stripped down profile of pubSub. Publish-Subscribe feature helps to support asynchronous communication for IoT / M2M applications.

- XEP-0079 Advanced-Message Processing [i.22]. Allows including message expiration feature. It can be useful to indicate expiration time for M2M data that is cached.

- XEP-0080: Allows publishing location information [i.23]; Useful to associate location information with M2M devices.

- XEP-0136 Message Archiving [i.24]: In addition to P2P applications, this can be potentially used for caching M2M data at an XMPP server.

- XEP-0138 – Supports application layer compression [i.25]; Useful in constrained IoT environment.

- XEP-0149 – Allows XMPP entities to specify time period for state, event or activity [i.26].

- Jingle specifications (XEP-0166 [i.27], XEP-0167 [i.28], XEP-0177 [i.29], ….) extend XMPP for initiating and managing peer-to-peer media sessions between two XMPP entities.  It uses XMPP for signaling purposes and uses different transport methods (such as TCP, UDP, ….) for data plane packets.

- XEP-0198 Reliability, Stream Management Protocol [i.30]

- XEP-0199 : Provides support for application level pings [i.31]

- XEP-0124 [i.32] and XEP-0206 [i.33] : Allows use of HTTP as transport for XML streams.

- XEP-0203 [i.34]:  In addition to P2P applications, this can potentially be also used for M2M applications. It allows an XMPP server to store a message if corresponding XMPP client (e.g. an M2M device) is in offline (e.g. sleep) state and send message as soon as it gets to know that the client / device is available. It gets to know the status via presence notification as the client / device moves from offline (sleep) state to available state.

- XEP-0322 Efficient XML Interchange (EXI) Format for XMPP [i.35]. Useful for M2M applications.

- XEP-0323 Sensor data [i.36]:  It provides architecture, data structures and basic operations for sensor (M2M) data communication over XMPP networks. This is designed for implementation in sensors that may have limited amount of memory or processing power.

- XEP-0324 IoT Provisioning [i.37]

- XEP-0325 Internet of Things – Control [i.38]: It provides mechanism to control actuators in XMPP based sensor networks.

- XEP-0326 Internet of Things – Concentrators [i.39]

### 7.5.7	Protocol Stack

As described in RFC6120 [i.6], XMPP is an application profile of XML that enables near real-time exchange of structured yet extensible data between any two or more network aware entities.  An XMPP address (called a Jabber Identifier or JID) is represented as localpart@domainpart/resourcepart. An example of JID is tom@jabber.org/Laptop.

For client-to-server communication, client resolves FQDN of the receiving entity to an IP address and opens a TCP connection to the advertised port at receiver’s IP address. Channel encryption is optionally supported using TLS and authentication is done using SASL. After a client authenticates with a server, it binds a specific resource to the stream so that server can properly address the client. Address for use over that stream is a full Jabber ID of the form <localpart@domainpart/resource>. Client opens an XML stream over TCP with a server (of its domain) and exchanges XML stanzas.

Here, an XML stream is a container for carrying XML elements between two elements in a network. XML elements in an XML stream carry XML stanzas (i.e. actual payload message in XML format) or the elements that are used to negotiate the stream (e.g. for TLS and SASL purposes).  Each stream is unidirectional and thus two streams are needed between an initiating entity and a receiving entity for bidirectional transfer of XML stanzas. An XML stanza is a basic unit of meaning in XMPP.  An XML stream begins with an open tag <stream> and ends with a close tag </stream>. Each direction of conversation is represented as a streaming XML document that ends when that connection is terminated. Root node of that streaming document is the <stream/> element.

For server-to-server communication, a server opens an XML stream over TCP with a server of different domain for inter-server (or inter-domain) communication. Server-to-server streams are typically negotiated in the initialization phase.

Figure 7.5b: XMPP Protocol Stack

Editor’s Note: Need editable source for Fig 7.5b

There are three core stanza types, <presence/>, <message/> and <iq/>,  each with its own semantics. These three core stanzas are briefly described below:

- presence stanza: A basic publish-subscribe mechanism that allows several entities to receive information (about presence or availability) of a specific entity to which they have subscribed. In an IM application, presence information of a user’s (or client’s) contacts is displayed in user’s contact list or roster. When a user gets online, XMPP software announces user’s current status to server of user’s domain and that server informs user’s contacts about its online status. It is a simple broadcast mechanism in that sense. The server also informs the presence status of user’s contacts to user. An M2M device if not available for processing some action can use “do not disturb” to indicate that it is not available.

- message stanza: This supports a push mechanism where one entity pushes information to another entity. It supports real-time as well as delayed (i.e. store and push) delivery of messages. In an IM scenario, the message typically encapsulates chat data.  Messages are also used for group chat, event delivery and notifications. Message type includes the following: normal, chat, groupchat, headline and error. Message type “headline” is used to send alert and notifications.  Type “chat” is used for Instant Messaging.

For IM, XMPP servers are optimized to handle large number of small messages. As an XMPP server knows about the availability (or presence) status of XMPP clients, it can quickly take an appropriate decision. It can either send message to that client quickly if that client is available or can store it in buffer and send as soon as it gets to know that the client is available. This feature can also be used for M2M purposes.

- IQ (Info/Query) stanza: A request-response mechanism that allows structured exchange of data between an initiating entity and a requesting entity in a somewhat reliable way.  It allows operations such as get (reading), set (a variable), result and error. Values of type attribute used with IQ stanza are get, result, set and error.

#### 7.5.7.1 	XEP-0323 Sensor data

XEP-0323 [i.36] describes framework for sensor data exchange in an XMPP based sensor network (SN).  Specific support of XMPP SN feature is indicated by including “urn”xmpp:sn” in the service discovery procedure.  A sensor device, an actuator or a gateway is an example of a node in a sensor network. A Concentrator is a device that handles multiple nodes (e.g. a node handling multiple sensors) behind it. Field Name is name of a field of sensor data  (e.g. pressure or vibration level).  Possible values of Field Type include the following: momentary value, calculated value, peak value, status value, historical value etc.

#### 7.5.7.2	XEP-0324 IoT Provisioning

XEP-0324 [i.37], IoT provisioning, deals with access rights, user privileges and provisioning of services in a sensor network. This architecture uses distributed third parties that provide the following services:

- Control who can communicate with whom (i.e. control friendship)

- Control read access

- Control configure / write access

- Provide a user interface to set / update these policies

- Provide interoperability services (such as unit conversion)

A trust relationship is created between a device and a provisioning server using some mechanisms.   A device, client or user can get a token from a provisioning server that it can use to validate access rights with the server. <iq> stanza with xmlns ‘urn:xmpp:sn:provisioning’ is used for this purpose.  If a device supports provisioning feature, it advertises this feature in service discovery.

If there are multiple provisioning servers, device / client / user has one token from each of the provisioning server. While sending request to another entity (e.g. read or write some data), it includes all these tokens.  As an IoT device gets a request to read or write some data, it contacts a provisioning server with the tokens provided in the request and validates access rights of the requesting entity (Figure 7.5c).  As an IoT device gets friendship request from a third party, it contacts provisioning server and checks whether or not to accept that request. A provisioning server can also delegate a secondary trust to a device by which that device can add its own friends.

Figure 7.5c: Validation of access rights during a read or write operation

Editor’s Note: Need editable source for Fig 7.5c

#### 7.5.7.3	XEP-0325 Internet of Things - Control

XEP-0325 [i.38] specifies mechanisms to control actuators in an XMPP based sensor network.  <message> or <iq> stanzas can be used for this purpose.  Response from device is suppressed when using <message> stanza but <iq> stanza can be used if an acknowledgement is needed from the actuator.  Operation “Set” and the xmlns “urn:xmpp:sn:control” are used for this purpose in the message stanza. A control form can also be used to set values for various fields at the actuator. Actuators behind a concentrator can be controlled by specifying node elements for those actuators.

#### 7.5.7.4	XEP-0326 Internet of Things - Concentrators

XEP-0326 [i.39] deals with IoT Concentrators. A concentrator is defined to be a device that concentrates management of a subset of devices (of a sensor network) at a point.  A concentrator can be small (e.g. a PLC managing a set of sensors and actuators), medium (e.g. a branch of a network using a different communication protocol), large (e.g. a sub-system managed by a partner organization) or massive.  A concentrator works with multiple data sources where a data source is defined to contain a collection of nodes.  There are three types of data sources: Singular, Flat and Tree.  There is only one node object in a singular data source while a flat data source contains list of node objects.  Nodes are represented in a tree structure in a tree data source.  Asynchronous events are sent from a concentrator to each client that has subscribed to these using message stanzas. A concentrator can also store data from sensors locally (or at a remote server but controlled locally).

A client that needs to communicate with a concentrator can get type of commands supported by getting capabilities of the concentrator. The xmlns ‘urn:xmpp:sn:concentrators’ is used for this purpose. Some such commands are given below:

- Get all data sources managed by the concentrator,

- Get root data sources,

- Get child data sources (of a root data source),

- Given node id, check to see if a node is supported by a (given) concentrator,

- Get basic information about a node supported by a concentrator (e.g. is it readable? Is it configurable? what are the parameters supported by a node e.g. location of a meter? etc.)

- Change order of nodes in a tree (by moving nodes up or down among siblings),

- Get and set parameters of a node,

- Create or destroy a node,

- Get commands that are supported by a node,

- Subscribe to changes in data source by allowing devices to register for asynchronous events,

- Allow retrieval of historic events

### 7.5.8	Data Model

XMPP is based on XML. EXI (Efficient XML Interchange) is also supported for M2M applications.

### 7.5.9	Security

TLS is supported for encryption of client-to-server and server-to-server communication; Its use is optional. A receiving entity (such as a client or a peer server) can mandate that the initiating entity use TLS for data encryption.

An initiating entity needs to authenticate with the receiving entity before sending XML stanzas. If TLS is used, it is used before negotiating for SASL (Simple Authentication and Security Layer Protocol). It helps protect the authentication information exchanged during SASL negotiation.

### 7.5.10	Dependencies

- XMPP streams as defined in RFC6120 [i.6] use TCP as transport

- Use of HTTP as transport is allowed as per XEP-0124 [i.32] and XEP-0206 [i.33]

- Uses TLS and SASL for security.

- Jingle extensions use XMPP for signalling but data plane packets are sent over other transport mechanisms such as TCP, UDP, ….

- XML

### 7.5.11	Benefits and Constraints

XMPP is an open standard. IETF has approved XMPP RFCs for core methods (RFC6121), instant messaging and presence technology (RFC6121 [i.14]) and address format (RFC6122 [i.15]). XMPP standard foundation and IETF continue to extend this.

#### 7.5.11.1	Benefits

- XMPP is easily extensible. It provides basic set of features that can be expanded by protocol extensions (XEPs) to provide new set of features.

- Resource location is specified in the address itself and that makes it easy to identify different resources of an XMPP user.

- XMPP is already used by some devices for IM applications.  In that sense, it improves Person-to-Machine  (P2M) communication as user is able to directly interact with smart object running XMPP. It does not necessarily require use of protocol gateways as may be needed with CoAP (e.g. for HTTP to CoAP conversion) and MQTT.

- It supports a federated client-server architecture where no (global) centralized server is needed. Anyone with a domain name can run an XMPP server on its own domain. Public XMPP servers are available for everyone.

- Support of message, presence and IQ stanza types helps meet needs of several IoT applications.

- As it uses “store and push” mechanism to transfer data, it can store contents if the receiving entity is offline (e.g. if IoT device is in sleep mode).

- <xml:lang> common attribute enables internationalization.

- An XMPP address (or JID) can include any Unicode character and is not restricted to ASCII characters.

- Some of the existing mechanisms (such as publish-subscribe, caching, delayed delivery, support for EXI, etc.) are applicable for IoT use cases as well.

#### 7.5.11.2	Constraints

- Use of TCP may not be desirable for some IoT segments.

- Overhead may be high if XML data is used but EXI extensions available for IoT applications.

### 7.5.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by <protocol x> is shown in the following clauses:

Editor’s Note: The requirments listed herein will need review and alignment with the final approved oneM2M Requirements TS-0002 [i.2]

#### 7.5.12.1	Fully Supported Requirements

Editor’s Note: To be completed

#### 7.5.12.2	Partially Supported Requirements

Editor’s Note: To be completed

#### 7.5.12.3	Disallowed Requirements

Editor’s Note: To be completed

## 7.6 	WebSocket Protocol

The following clauses describe the WebSocket Protocol.

### 7.6.1	Background

The HTTP protocol is widely used for Web Services in different ways . One of those usages is standardized as WebSocket Protocol.

WebSocket Protocol provides an alternative way to ‘HTTP polling’  for two-way communication, which transmits data with upgrading established HTTP connection with remote host.

WebSocket Protocol was originally introduced as part of HTML5 specification, but later the transport protocol specification was standardized as the extension for HTTP in IETF.

### 7.6.2	Status

WebSocket Protocol specification is published as RFC6455 [i.40] (PROPOSED STANDARD) by the IETF in 2011.

SIP over WebSocket and XMPP over WebSocket are under development.

### 7.6.3	Category and Architectural Style

Editor’s Note: To be completed

### 7.6.4	Intended use

WebSocket Protocol provides relatively simple transport for two-way communication with remote host, which can coexist with HTTP and deployed HTTP infrastructure.

Unlike HTTP 2.0, which also possible to two-way communication over HTTP connection, WebSocket is designed for use in Ajax applications.

### 7.6.5	Deployment Trend

Following are some of implementations which include WebSocket support:

#### 7.6.5.1	Server-Side Implementations

- Apache Tomcat 7 (or later)

- Jetty 8 (or later)

- Tornado

- em-websocket (Ruby)

- gevent-websocket (Python)

- Node.js

#### 7.6.5.2	Client-Side Implementations

- Google Chrome 14.0 (or later)

- Firefox 10.0 (or later)

- Safari 6.0 (or later)

- Internet Explorer 10.0 (or later)

- Cordova (AKA PhoneGap)

### 7.6.6	Key features

NOTE: The following text is exerpted from Section “1.5 Design Philosophy” of RFC6455 [i.40]

The WebSocket Protocol is designed on the principle that there should  be minimal framing (the only framing that exists is to make the protocol frame-based instead of stream-based and to support a distinction between Unicode text and binary frames).  It is expected  that metadata would be layered on top of WebSocket by the application layer, in the same way that metadata is layered on top of TCP by the application layer (e.g., HTTP).

Conceptually, WebSocket is really just a layer on top of TCP that does the following:

- adds a web origin-based security model for browsers

- adds an addressing and protocol naming mechanism to support  multiple services on one port and multiple host names on one IP  address

- layers a framing mechanism on top of TCP to get back to the IP packet mechanism that TCP is built on, but without length limits

- includes an additional closing handshake in-band that is designed  to work in the presence of proxies and other intermediaries

Other than that, WebSocket adds nothing.  Basically it is intended to be as close to just exposing raw TCP to script as possible given the constraints of the Web.  It's also designed in such a way that its servers can share a port with HTTP servers, by having its handshake  be a valid HTTP Upgrade request.  One could conceptually use other protocols to establish client-server messaging, but the intent of WebSockets is to provide a relatively simple protocol that can coexist with HTTP and deployed HTTP infrastructure (such as proxies)   and that is as close to TCP as is safe for use with such infrastructure given security considerations, with targeted additions  to simplify usage and keep simple things simple (such as the addition  of message semantics).

The protocol is intended to be extensible; future versions will  likely introduce additional concepts such as multiplexing.

### 7.6.7	Protocol Stack

Editor’s Note: To be completed

### 7.6.8	Data Model

Editor’s Note: To be completed

### 7.6.9	Security

Security feature on WebSocket can be independent, but HTTP can provide necessary protections in  transport layer.

### 7.6.10	Dependencies

- WebSocket Protocol  depends on HTTP (1.1 or later) protocol.

- TCP stack should be provided for HTTP communication.

### 7.6.11	Benefits and Constraints

Editor’s Note: To be completed

#### 7.6.11.1	Benefits

- Allowing co-existance with existing Web infrastructures

- Reuse security solutions which is applied on HTTP Servers

- HTML5 browser can supports WebSocket natively (seamless integration on HTTP based application)

#### 7.6.11.2	Constraints

- Leaving  WebSocket connection opened for long time periods may cause DoS attack risk

- Server-side WebSocket  implementation can host limited number of connections

### 7.6.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by <protocol x> is shown in the following clauses:

Editor’s Note: The requirments listed herein will need review and alignment with the final approved oneM2M Requirements TS-0002 [i.2]

#### 7.6.12.1	Fully Supported Requirements

Editor’s Note: To be completed

#### 7.6.12.2	Partially Supported Requirements

Editor’s Note: To be completed

#### 7.6.12.3	Disallowed Requirements

Editor’s Note: To be completed

## 7.7 	Bluetooth® Wireless Technology

This section outline the background and practical use and charatersitics of Bluetooth® with particular focus on the Bluetooth Low Energy (BLE) version that is well-suited to M2M type of applications. There is also a description of the application layer and attribute protocol based on the GATT (Generic Attribute Profile) interactions and data structure (Services and Characterstics), that is included in the Bluetooth Smart version 4.0. Finally the GATT although a general purpose solution and well specified to be interoperable is not immediately available of plain HTTP. For this reason there is a host of developer resources provided by the Bluetooth SIG developer portal that outline how to develop for Bluetooth Low Energy and GATT for various popular OS and platforms. A more recent activity (Whitepaper, publication in process) also present a practical method to transfer and interact with GATT devices using HTTP and JSON data over Internet and Web.

### 7.7.1	Background

Bluetooth® technology is a wireless communications system intended to replace the cables connecting electronic devices. Bluetooth technology is powerful, low energy and lower in cost to make your development faster and easier.

The Bluetooth Core System consists of an RF transceiver, baseband, and protocol stack. The system offers services enabling the connection of devices and the exchange of a variety of classes of data between these devices. Many features of the core specification are optional, allowing product differentiation.

The most recent enhancement, Bluetooth v4.0, is like three specifications in one—Classic Bluetooth technology, Bluetooth low energy technology, and Bluetooth high speed technology—all of which can be combined or used separately in different devices according to their functionality.

To use Bluetooth wireless technology, a device must be able to interpret certain Bluetooth profiles. Bluetooth profiles are definitions of possible applications and specify general behaviors that Bluetooth enabled devices use to communicate with other Bluetooth devices. There is a wide range of Bluetooth profiles describing many different types of applications or use cases for devices. By following the guidance provided by the Bluetooth specification, developers can create applications to work with other Bluetooth devices.

### 7.7.2	Status

When the Bluetooth® SIG announced the formal adoption of Bluetooth Core Specification version 4.0, it included the Bluetooth Smart (low energy) feature. This final step in the adoption process opened the door for qualification of all Bluetooth product types to version 4.0.

Bluetooth v.1.2 is endorsed and ratified as IEEE 802.15.1; this is based on the standard issued in 2005).  Recent versions of Bluetooth now known as 4.0 – or “Bluetooth Smart” are openly & publically available thought the Bluetooth SIG. The specificatons may be endorsed ratified & published. Publication is an option, if preferred by an SDO or partnership project. The Bluetooth SIG is encourages direct liaison with oneM2M.

The Bluetooth SIG consideres that Version 4.0 Bluetooth Smart is frozen and stable. It is being tested agaist for devices and conformance, new profiles are being added. However the core specification is being used and updated toward.

### 7.7.3	Category and Architectural Style

The Bluetooth® core system covers the four lowest layers and associated protocols defined by the Bluetooth specification as well as one common service layer protocol, the service discovery protocol (SDP) and the overall profile requirements specified in the generic access profile (GAP). A complete Bluetooth application requires a number of additional services and higher layer protocols defined in the Bluetooth specification.

To use Bluetooth wireless technology, a device must be able to interpret certain Bluetooth profiles. Bluetooth profiles are definitions of possible applications and specify general behaviors that Bluetooth enabled devices use to communicate with other Bluetooth devices. There is a wide range of Bluetooth profiles describing many different types of applications or use cases for devices. By following the guidance provided by the Bluetooth specification, developers can create applications to work with other Bluetooth devices.

At a minimum, each Bluetooth profile contains information on the following topics:

- Dependencies on other profiles

- Suggested user interface formats

- Specific parts of the Bluetooth protocol stack used by the profile. To perform its task, each profile uses particular options and parameters at each layer of the stack and this may include, if appropriate, an outline of the required service record

### 7.7.4	Intended use - Personal Area Network protocols

Bluetooth Smart (low energy) technology allows enhancement of devices like watches, toothbrushes or toys with Bluetooth wireless technology. It also provides the ability for developers to incorporate new functionalities into devices already enabled by Bluetooth technology such as sports & fitness, health care, human interface (HIDs) and entertainment devices. For example, sensors in pedometers and glucose monitors will only run low energy technology. These single mode devices benefit from the power savings provided by v4.0 as well as the low cost implementation. Watches take advantage of both low energy technology while collecting data from body-worn fitness sensors and Classic Bluetooth technology when sending that information to a PC, or displaying caller ID information when wirelessly connected to a smartphone. Smartphones and PCs, which support the widest range of use cases for the specification, utilizing the full dual-mode package with Classic, low energy and high speed versions of the technology running side by side.

### 7.7.5	Deployment Trend - Bluetooth and Bluetooth Smart (low energy)

Originally intended to be a wireless replacement for cables on phones, headsets, keyboards and mice, Bluetooth technology now goes way beyond that. Bluetooth technology is bringing everyday devices into a digital and connected world. In the health and fitness market, the use cases vary widely — from sensors that monitor activity levels to medical and wellness devices that monitor healthcare, like a glucometer, inhaler or toothbrush. The top-selling Smartphones, PCs and tablets all support Bluetooth technology. In-vehicle systems give the ability to make phone calls, send texts, and even make dinner reservations. The Bluetooth SIG is also seeing developments where drivers will monitor important information like vehicle diagnostics, traffic, even driver health — all in real time. Bluetooth technology is creating opportunities for companies to develop solutions that make a consumer's life better.

Bluetooth Smart and Bluetooth Smart Ready are extensions of the original Bluetooth brand introduced in 2011. The Smart and Smart Ready designations indicate compatibility of products using the low energy feature of the Bluetooth v4.0 specification. A Bluetooth Smart Ready product connects to both classic Bluetooth and Bluetooth Smart low energy products. By contrast, a Bluetooth Smart product collects data and runs for months or years on a tiny battery. Think of a Smart product as a sensor that works for a long time without changing the battery (like a fitness heart rate monitor) and a Smart Ready product as a collector (like a smart phone or tablet receiving the information and displaying it in an application).

#### 7.7.5.1	Bluetooth Smart (low energy) Technology

When the Bluetooth® SIG announced the formal adoption of Bluetooth Core Specification version 4.0, it included the hallmark Bluetooth Smart (low energy) feature. This final step in the adoption process opened the door for qualification of all Bluetooth product types to version 4.0.

Bluetooth Smart (low energy) technology allows it to be included in devices like watches, toothbrushes or toys to enable the connectivity using Bluetooth wireless technology. It also provides the ability for developers to incorporate new functionalities into devices already enabled by Bluetooth technology such as sports & fitness, health care, human interface (HIDs) and entertainment devices. For example, sensors in pedometers and glucose monitors will only run low energy technology. These single mode devices benefit from the power savings provided by v4.0 as well as the low cost implementation. Watches take advantage of both low energy technology while collecting data from body-worn fitness sensors and Classic Bluetooth technology when sending that information to a PC, or displaying caller ID information when wirelessly connected to a smartphone. Smartphones and PCs, which support the widest range of use cases for the specification, utilizing the full dual-mode package with Classic, low energy and high speed versions of the technology running side by side.

#### 7.7.5.2	Bluetooth High Speed Wireless Technology

Bluetooth high speed wireless technology delivers new opportunities in the home entertainment and consumer electronics markets. By enabling wireless users to quickly send video, music and other large files between devices, Bluetooth high speed wireless technology provides a richer experience while maintaining the same familiar user interface.

Key features of Bluetooth high speed wireless technology include:

- Power Optimization. The new Bluetooth technology reduces power consumption. The high speed radio is used only when necessary, which means longer battery life for your devices

- Improved Security. The Generic Alternate MAC/PHY in Bluetooth high speed enables the radio to discover other high speed devices only when they are needed in the transfer of music, video and other large data files. This decreases power consumption and increases radio security

- Enhanced Power Control. Drop-out reduction is now a reality: enhanced Bluetooth technology makes power control faster and reduces the impact of a power or signal loss. Users are now less likely to experience a dropped headset connection – even when a phone is deep inside a coat pocket or tote

- Lower Latency Rates. Unicast Connectionless Data (UCD) improves the user’s speed experience by moving small amounts of data faster, which lowers latency rates

### 7.7.6	Key features

- Bluetooth wireless technology is geared towards voice and data applications

- Bluetooth wireless technology operates in the unlicensed 2.4 GHz spectrum

- The range of Bluetooth wireless technology is application specific. The Bluetooth Specification mandates operation over a minimum distance of 10 meters or 100 meters depending on the Bluetooth device class, but there is not a range limit for the technology. Manufacturers may tune their implementations to support the distance required by the use case they are enabling.

- The peak data rate with EDR is 3 Mbps

NOTE:	The term Enhanced Data Rate (EDR) is used to describe π/4-DPSK and 8DPSK schemes, each giving 2 and 3 Mbit/s respectively

- EDR Profiles in steps 10 to 100 metres.

- Bluetooth wireless technology is able to penetrate solid objects

- Bluetooth technology is omni-directional and does not require line-of-sight positioning of connected devices

- Security has always been and continues to be a priority in the development of the Bluetooth specification. The Bluetooth specification allows for three modes of security, see below for security.

Bluetooth Smart (low energy) wireless technology features:

- Ultra-low peak, average and idle mode power very low consumption

- Ability to run for years on standard coin-cell batteries

- Multi-vendor interoperability

- Enhanced range (EDR Profiles in steps 10 to 100+ metres).

| Technology | Bluetooth BR/EDR/HS Technology | Bluetooth Low Energy Technology |
| --- | --- | --- |
| Radio Frequency | 2.4 GHz ISM | 2.4 GHz ISM |
| Range | 10 to 100 meters | 10 to 100+ meters |
| Data Rate | 1-3 Mbps (Classic) >400 Mbps (AMP, 802.11n) | 1 Mbps |
| Nodes/Active Slaves | 7 / 16777184 | Unlimited |
| Security | 56b E0 (classic)/128b AES (AMP) and applications layer user defined | 128b AES and application layer user defined |
| Robustness | Adaptive frequency hopping, | FEC Adaptive frequency hopping |
| Latency (from non-connected state) | 100ms | <3ms |
| Regulatory Acceptance | Worldwide | Worldwide |
| Voice Capable | Yes | No |
| Network Topology | Scatternet | Star-bus |
| Power Consumption | 1 as the reference, x10 for AMP | 0.01 to 0.5 (use case dependent) |
| Service Discovery | Yes | Yes |

Table 7.7a: Table of Blutooth capabilities

### 7.7.7	Protocol Stack

The Bluetooth 4.0 specification uses a service-based architecture based on the attribute protocol (ATT). All communication in low energy takes place over the Generic Attribute Profile (GATT). An application or another profile uses the GATT profile so a client and server can interact in a structured way.

The server contains a number of attributes, and the GATT Profile defines how to use the Attribute Protocol to discover, read, write and obtain indications. These features support a service-based architecture. The services are used as defined in the profile specifications. GATT enables to expose service and characteristics defined in the profile specification.

The GATT profile is also part of the core and defined in the core specification.

Profiles: The first specification of Bluetooth 4.0 low energy wireless technology included profiles to optimize its functionality for a specific group of products.

Adopted GATT based Bluetooth Profiles and Services: The GATT architecture makes it easy to both create and implement new profiles. Many new profiles are under development so this continues to grow. The simplicity of implementing the profiles contributes to a rapid growth of applications and embedded devices supporting these.

Generic Attribute Profile (GATT) is built on top of the Attribute Protocol (ATT) and establishes common operations and a framework for the data transported and stored by the Attribute Protocol. GATT defines two roles: Server and Client. The GATT roles are not necessarily tied to specific GAP roles and but may be specified by higher layer profiles. GATT and ATT are not transport specific and can be used in both BR/EDR and LE. However, GATT and ATT are mandatory to implement in LE since it is used for discovering services

The GATT server stores the data transported over the Attribute Protocol and accepts Attribute Protocol requests, commands and confirmations from the GATT client. The GATT server sends responses to requests and when configured, sends indication and notifications asynchronously to the GATT client when specified events occur on the GATT server. GATT also specifies the format of data contained on the GATT server.

Attributes, as transported by the Attribute Protocol, are formatted as services and characteristics. Services may contain a collection of characteristics. Characteristics contain a single value and any number of descriptors describing the characteristic value.

With the defined structure of services, characteristics and characteristic descriptors a GATT client that is not specific to a profile can still traverse the GATT server and display characteristic values to the user. The characteristic descriptors can be used to display descriptions of the characteristic values that may make the value understandable by the user.

Figure 7.7: Bluetooth Protocol Architecture

Editor’s Note: Need editable source for Fig 7.7

Below are links to the specifications for the current list of profiles supported by GATT in the above protocol architecture.

| GATT-Based Specifications (Qualifiable) |  | Adopted Versions |
| --- | --- | --- |
| ANP | Alert Notification Profile | 1.0 |
| ANS | Alert Notification Service | 1.0 |
| CTS | Current Time Service | 1.0 |
| DIS | Device Information Service | 1.0 |
| FMP | Find Me Profile | 1.0 |
| HTP | Health Thermometer Profile | 1.0 |
| HTS | Health Thermometer Service | 1.0 |
| HRP | Heart Rate Profile | 1.0 |
| HRS | Heart Rate Service | 1.0 |
| IAS | Immediate Alert Service | 1.0 |
| LLS | Link Loss Service | 1.0 |
| NDCS | Next DST Change Service | 1.0 |
| PASP | Phone Alert Status Profile | 1.0 |
| PASS | Phone Alert Status Service | 1.0 |
| PXP | Proximity Profile | 1.0 |
| RTUS | Reference Time Update Service | 1.0 |
| TIP | Time Profile | 1.0 |
| TPS | Tx Power Service | 1.0 |

Table 7.7b: List of Profiles

The link layer provides low power idle mode operation, simple device discovery and reliable point-to-multipoint data transfer with advanced power-save and encryption functionalities.

#### 7.7.7.1	Single mode and dual mode

Bluetooth Smart (low energy) wireless technology contains two equally important implementation alternatives: single mode and dual mode:

- Small devices like watches and sports sensors use the single mode Bluetooth Smart (low energy) implementation.

- Dual mode implementations use parts of the Bluetooth hardware, sharing one physical radio and antenna.

| BR/EDR Profiles |  | Description |
| --- | --- | --- |
| A2DP | Advanced Audio Distribution Profile | describes how stereo quality audio can be streamed from a media source to a sink. |
| AVRCP | Audio/Video Remote Control Profile | is designed to provide a standard interface to control TVs, stereo audio equipment, or other A/V devices. This profile allows a single remote control (or other device) to control all A/V equipment to which a user has access. |
| BIP | Basic Imaging Profile | defines how an imaging device can be remotely controlled, how an imaging device may print, and how an imaging device can transfer images to a storage device. |
| BPP | Basic Printing Profile | allows devices to send text, e-mails, v-cards, images or other information to printers based on print jobs. |
| DI | Device ID Profile | provides additional information above and beyond the Bluetooth Class of Device and to incorporate the information into both the Service Discovery Profile (SDP) record and the EIR response. |
| DUN | Dial-Up Network Profile | provides a standard to access the Internet and other dial-up services via Bluetooth technology. |
| FTP | File Transfer Profile | defines how folders and files on a server device can be browsed by a client device. |
| GAVDP | Generic Audio/Video Distribution Profile | provides the basis for A2DP and VDP, which are the basis of the systems designed for distributing video and audio streams using Bluetooth technology. |
| GOEP | Generic Object Profile | is used to transfer an object from one device to another. |
| HFP | Hands-Free Profile | HFP describes how a gateway device can be used to place and receive calls for a hand-free device. |
| HCRP | Hard Copy Cable Replacement Profile | defines how driver-based printing is accomplished over a Bluetooth wireless link. |
| HDP | Health Device Profile | enables Healthcare and Fitness device usage models. |
| HSP | Headset Profile | describes how a Bluetooth enabled headset should communicate with a Bluetooth enabled device. |
| HID | Human Interface Device Profile | defines the protocols, procedures and features to be used by Bluetooth keyboards, mice, pointing and gaming devices and remote monitoring devices. |
| MAP | Message Access Profile | defines a set of features and procedures to exchange messages between devices. |
| MPS | Multi Profile | defines a set of features and procedures between Multiple Profiles Single Device and Multiple Profiles Multiple Devices |
| OPP | Object Push Profile | defines the roles of push server and push client. |
| PBAP | Phone Book Access Profile | defines the procedures and protocols to exchange Phone Book objects between devices. |
| PAN | Personal Area Networking Profile | describes how two or more Bluetooth enabled devices can form an ad-hoc network and how the same mechanism can be used to access a remote network through a network access point. |
| SAP | SIM Access Profile | defines the protocols and procedures that shall be used to access a GSM SIM card, a UICC card or an R-UIM card via a Bluetooth link. |
| SDAP | Service Discovery Application Profile | describes how an application should use SDP to discover services on a remote device. |
| SPP | Service Port Profile | defines how to set-up virtual serial ports and connect two Bluetooth enabled devices. |
| SYNC | Synchronization Profile | used in conjunction with GOEP to enable synchronization of calendar and address information (personal information manager (PIM) items) between Bluetooth enabled devices. |
| VDP | Video Distribution Profile | defines how a Bluetooth enabled device streams video over Bluetooth wireless technology. |

Table 7.7c: List of BR/EDR Profiles

The following table is a list of the Core Bluetooth spaecication version 4.0 that may be found at https://www.bluetooth.org/en-us/specification/adopted-specifications.

| Specification | Adopted Date | Notes |
| --- | --- | --- |
| Core Specification Addendum (CSA) 4 | 12 February 2013 | Refer to the Mixing of Specification Versions Part for applicability |
| Core Specification Supplement (CSS) v3 | 12 February 2013 |  |
| Core Specification Addendum (CSA) 3 | 24 July 2012 | Refer to the Mixing of Specification Versions Part for applicability |
| Core Specification Addendum (CSA) 2 | 27 December 2011 | Refer to the Mixing of Specification Versions Part for applicability |
| Core Version 4.0 | 30 June 2010 |  |

Table 7.7d: Bluetooth Core Specifications

### 7.7.8	Data Model

The GATT Profile specifies the structure in which profile data is exchanged. This structure defines basic elements such as services and characteristics, used in a profile. The top level of the hierarchy is a profile. A profile is composed of one or more services necessary to fulfill a use case. A service is composed of characteristics or references to other services. Each characteristic contains a value and may contain optional information about the value. The service and characteristic and the components of the characteristic (i.e., value and descriptors) contain the profile data and are all stored in attributes on the server.

A service is a collection of data and associated behaviors to accomplish a particular function or feature of a device or portions of a device. A service may reference other primary or secondary services and/or a set of characteristics that make up the service.

There are two types of services: primary and secondary. A primary service provides the primary functionality of a device. A secondary service provides auxiliary functionality of a device and is referenced from at least one primary service on the device.

To maintain backward compatibility with earlier clients, later revisions of a service definition can only add new referenced services or optional characteristics. Later revisions of a service definition are also forbidden from changing behaviors from previous revision of the service definition. Services may be used in one or more profiles to fulfill a particular use case.

A referenced service is a method incorporating another service definition on the server as part of the service referencing it. When a service references another service, the entire referenced service becomes part of the new service including any nested referenced services and characteristics. The referenced service still exists as an independent service. There are no limits to the depth of nested references.

A characteristic is a value used in a service along with properties and configuration information about how the value is accessed and information about how the value is displayed or represented. A characteristic definition contains a characteristic declaration, characteristic properties, and a value. It may also contain descriptors that describe the value or permit configuration of the server with respect to the characteristic value.

### 7.7.9	Security

Bluetooth® Smart (low energy) technology has some security differences with respect to BR/EDR security features such as Secure Simple Pairing. The association models are similar to Secure Simple Pairing from the user perspective and have the same names but differences in the quality of the protection provided.

The overall goal of keeping the cost of the controller and the complexity of a slave device to a minimum was used in making compromises on security capabilities in Bluetooth Smart (low energy) technology.

Bluetooth Smart (low energy) technology uses three association models referred to as Just Works, Out of Band and Passkey Entry. Bluetooth low energy technology does not have an equivalent of Numeric Comparison. Each of these association models is similar to Secure Simple Pairing with the following exception; Just Works and Passkey Entry do not provide any passive eavesdropping protection. This is because Secure Simple Pairing uses Elliptic Curve Diffie-Hellman and Bluetooth Smart (low energy) does not. The use of each association model is based on the I/O capabilities of the devices in a similar manner as Secure Simple Pairing.

Bluetooth Smart (low energy) technology supports a feature that reduces the ability to track a Bluetooth device over a period of time by changing the address on a frequent basis. The privacy feature is not used in the GAP discovery mode and procedures but it is used when supported during connection mode and connection procedures.

### 7.7.10	Dependencies

### 7.7.11	Benefits and Constraints

Editor’s Note: To be completed

#### 7.7.11.1	Benefits

Editor’s Note: To be completed

#### 7.7.11.2	Constraints

Editor’s Note: To be completed

### 7.7.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by <protocol x> is shown in the following clauses:

Editor’s Note: The requirments listed herein will need review and alignment with the final approved oneM2M Requirements TS-0002 [i.2]

#### 7.7.12.1	Fully Supported Requirements

Editor’s Note: To be completed

#### 7.7.12.2	Partially Supported Requirements

Editor’s Note: To be completed

#### 7.7.12.3	Disallowed Requirements

Editor’s Note: To be completed

## 7.x 	Protocol x {template}

### 7.x.1	Background

### 7.x.2	Status

### 7.x.3	Category and Architectural Style

### 7.x.4	Intended use

### 7.x.5	Deployment Trend

### 7.x.6	Key features

### 7.x.7	Protocol Stack

Editor’s Note: Added per discussion in PRO WG3 - TP#6

### 7.x.8	Data Model

### 7.x.9	Security

### 7.x.10	Dependencies

### 7.x.11	Benefits and Constraints

#### 7.x.11.1	Benefits

#### 7.x.11.2	Constraints

### 7.x.12	Support of oneM2M requirements

Support of oneM2M Requirements [i.2] by <protocol x> is shown in the following clauses:

Editor’s Note: The requirments listed herein will need review and alignment with the final approved oneM2M Requirements TS-0002 [i.2]

#### 7.x.12.1	Fully Supported Requirements

Editor’s Note: Per discussion in PRO WG3 - TP#6

#### 7.x.12.2	Partially Supported Requirements

Editor’s Note: Added per discussion in PRO WG3 - TP#6

#### 7.x.12.3	Disallowed Requirements

Editor’s Note: Added per discussion in PRO WG3 - TP#6

# 8	Summary

Editor’s Note:  Summary and “list of those protocols with which oneM2M could encapsulate and/or interoperate” to be provided

The following tables summarise how selected traits are addressed by the analysed protocols. The selected traits include:

•	Traits explicitly titled in clauses: 7.x.3 (architectural style), 7.x.5 (deployment), 7.x.8 (data model), 7.x.9 (security) and 7.x.10 (dependencies)

•	Traits incorporated into clauses 7.x.x , e.g. QoS, Stability

•	Traits which are highly relevant to M2M e.g. Multicast, Pub/sub, efficiency, time services

| Protocol / Traits | Archi-tecture Style | Primary deployment & size | QoS | Messaging (Req/Res, Pub/Sub) | Multicast (app level, IP level) | Compute Resources Req’d | Domain scale |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CoAP |  |  |  |  |  |  |  |
| MQTT |  |  |  |  |  |  |  |
| RESTful HTTP |  |  |  |  |  |  |  |
| XMPP |  |  |  |  |  |  |  |
| TR-50 |  |  |  |  |  |  |  |
| Websockets |  |  |  |  |  |  |  |
| BT |  |  |  |  |  |  |  |

Table 8a: Summary of Protocols' Traits

| Protocol / Traits | Data Model | Dependencies -Underlying protocols (UDP/TCP, L2, serial interfaces) -Infrastructure (server/broker) | Time services (time-stamp) | Efficiency | Data conditional actions | Security | Stability (last update, next update) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CoAP |  |  |  |  |  |  |  |
| MQTT |  |  |  |  |  |  |  |
| RESTful HTTP |  |  |  |  |  |  |  |
| XMPP |  |  |  |  |  |  |  |
| TR-50 |  |  |  |  |  |  |  |
| Websockets |  |  |  |  |  |  |  |
| BT |  |  |  |  |  |  |  |

Table 8b: Summary of Protocols' Traits [continued]

Editor’s Note:  Additional traits to consider include: message size, session management, encryption, integrity, authentication, protocol addressability limitations, and involved SDO.

Editor’s Note: We may wish to format the tables by categories of the traits and/or the protocols.

Editor’s Note: Some traits (e.g. efficiency and scale) may best be considered in context of a few use case scenarios since they may be dependent upon number of data elements, frequency of exchange, etc.

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

# Annex A
List of M2M-related Protocols (Informative)

NOTE:  The following list table has been created for reference from publicly-available sources, and no representation is made regarding the accuracy or timeliness of the information it contains. In addition, the appearance or omission of any M2M-related information in this list does not imply either the intention, or lack of intention, to undertake any normative or other work within oneM2M.

Table A.1: M2M-related Protocols

Editor’s Note: “Context” Column Added per discussion in PRO WG3 - TP#6

| Short Name (docs link) | Full Name / Description | Originating org (source link) | Context | Notes |
| --- | --- | --- | --- | --- |
|  |  |  | tbc |  |
| 1-Wire® | 1-Wire® | [Dallas-Maxim] |  |  |
| 6LoWPAN | IPv6 over Low-Power Wireless Personal Area Networks | IETF 6LoWPAN |  | RFC 4944 |
| AllJoyn™ | AllJoyn™ | AllJoyn Alliance [Qualcomm] |  | (open source) |
| ANSI | ~~ | ANSI |  |  |
| ANSI C12.18 | Protocol Specification for ANSI Type 2 Optical Port | ANSI / NEMA |  |  |
| ANSI C12-21 | Protocol Specification for Telephone Modem Communication | ANSI / NEMA |  |  |
| ANSI C12.22 | Interfacing to Data Communication Networks | ANSI / NEMA |  | Advanced Metering Infrastructure (AMI) |
| ANT+ | ANT+ | ThisIsAnt [Dynastream Inc] |  |  |
| BACnet™ | Building Automation & Control Network | ASHRAE SSPC 135 BACnet International |  |  |
| BâtiBUS, | Bâtiment-Bus -superceded- |  |  | see KNX |
| BitXML | BitXchange Markup Language | BitXML [Your Voice S.P.A.] |  |  |
| Bluetooth | ~~ | Bluetooth SIG |  | IEEE 802.15.1 |
| Bluetooth HDP | Bluetooth Health Device Profile | Continua Health Alliance Bluetooth SIG |  | Partner Type 2 |
| Bluetooth LE / SMART | Bluetooth Low Energy / Smart Devices | Bluetooth SIG |  |  |
| C-Bus | C-Bus | [Clipsal / Schneider Electric] |  |  |
| CANOpen | Controller Area Network - Open | CANOpen Forum [CiA. e.V.] |  | EN 50325-4 2002 Part 4 |
| CC-Link | CC-Link | CC-Link Partner Association [Mitsubishi] |  | SEMI E54.12-0701E |
| CEBus | Consumer Electronics Bus | CEA (was EIA) |  | EIA 600 |
| CIP | Common Industrial Protocol | Open DeviceNet Vendors Association [Rockwell Automation] |  |  |
| CoAP | Constrained Application Protocol | IETF CORE WG |  | See Clause 7.1 |
| CompoNet | CompoNet (CIP) on TDMA Technology | Open DeviceNet Vendors Association |  |  |
| Contiki | Contiki Operating System | Contiki Project |  | (open source) |
| ControlNet | ControlNet (CIP) on CTDMA Technology | Open DeviceNet Vendors Association |  |  |
| DALI | Digital Addressable Lighting Interface | DALI |  | IEC 62386 |
| DLMS | Device Language Message Specification |  |  | see IEC 62056 |
| DASH7 | (ISO 18000) - Dash 7 | DASH7 Alliance |  | ISO/IEC 18000-7 |
| DDS | Data Distribution Service for Real-Time Systems | OMG |  |  |
| DDS-RTPS | DDS Real-Time Publish-Subscribe | OMG |  |  |
| DECT™ ULE | Digital Enhanced Cordless Telecommunications - Ultra Low Energy | ETSI TC DECT |  | TS 102 939-1 |
| DeviceNet | DeviceNet (CIP) on CAN Technology | Open DeviceNet Vendors Association [Allen-Bradley / Rockwell] |  |  |
| DNP | Distributed Network Protocol | IEEE / DNP |  | IEEE Std 1815™ |
| Dynet 1 / 2 | Dynalite Network | [Philips Dynalite] |  | RS-485 |
| E5 | (Ease, Energy, Efficiency, Environment and Earth) Smart Thermostat | [EarthNetworks] |  |  |
| Eclipse | ~~ | Eclipse Foundation |  |  |
| Concierge (proposed) | Lightweight, embeddable OSGi framework | Eclipse Foundation |  | (open source) |
| Kura (proposed) | Java M2M framwork | Eclipse Foundation |  | (open source) |
| Lua API | Scripting API | Eclipse Foundation |  | (open source) |
| Mihini / M3DA | Mihini/M3DA Specification | Eclipse Foundation |  | (open source) |
| Paho | Implementations of Open and Standard Messaging Protocols | Eclipse Foundation |  | (open source) |
| Ponte (proposed) | M2M to REST bridge | Eclipse Foundation |  | (open source) |
| SCADA | open Supervisory Control and Data Acquisition | Eclipse Foundation (was openSCADA) |  | (open source) |
| E-DCP | Ericsson Device Connection Platform | [Ericsson] |  |  |
| EHS | European Home Systems -superceded- |  |  | see KNX |
| EIB | European Installation Bus -superceded- |  |  | see KNX |
| Energyhub | Mercury™ smart thermostat platform | [Energyhub] |  |  |
| EnOcean | EnOcean Weqipment Profiles (EEP) | EnOcean Alliance [EnOcean / Siemens] |  | EN 50090, ISO/IEC 14543 |
| ETSI M2M TS 102 921 | Machine-to-Machine communications (M2M); mIa, dIa and mId interfaces | ETSI M2M |  | Partner Type 1 oneM2M Pool Document |
| EXALTED | EXpAnding LTE for Devices | Exalted consortium [Eurescom] |  | EC FP7, 2010-2013 |
| FieldBus | FieldBus | Fieldbus Foundation |  | IEC 61158 |
| FlatMesh | Remote Condition Monitoring | [Senceive] |  | IEEE 802.15.4 |
| flexWARE | Flexible Wireless Automation in Real-Time Environments | flexWARE Interest Group (FIG) |  | EU FP7 |
| HBS | Honeywell Building Solutions | [Honeywell] |  |  |
| IEC | ~~ | IEC |  |  |
| IEC 60870-5-xxx | Telecontrol (Supervisory Control and Data Acquisition) | IEC TC57 WG03 |  | IEC 101 IEC 103 IEC 104 |
| IEC 61107 | Smart Meter Communications Protocol | IEC TC57 |  |  |
| IEC 61850 | Electrical Substation Automation. | IEC TC57 |  |  |
| IEC 62056 DLMS/ COSEM | Device Language Message Specification/Companion Specification for Energy Metering | IEC TC13 WG 14 DLMS User Association |  |  |
| IEC 62351 | Security | IEC TC57 WG15 |  |  |
| INSTEON | Dual-mesh (RF/PL) Home Management Metwork | Insteon [SmartLabs, Inc.] |  |  |
| IEEE | ~~ | IEEE |  |  |
| 1451.x | Smart Transducer Interface for Sensors and Actuators | IEEE |  |  |
| P1451.1.4 | Smart Transducer Interface for Sensors, Actuators, and Devices - XMPP | IEEE IM/ST - TC9 |  | ISO/IEC/IEEE 21451-1-4 |
| 802.11a 802.11b 802.11g 802.11n | Wi-Fi | IEEE Wi-Fi Alliance |  |  |
| 802.11p | WAVE - Wireless Access in Vehicular Environments | IEEE |  | IEEE 1609 |
| 802.11ag | WiGig | IEEE Wi-Fi Alliance (was WiGig) |  |  |
| 802.11ah (in progress) | Sub 1 GHz (S1G) Wireless Sensor Network for Smart Metering | IEEE |  |  |
| 802.15 | Wireless Personal Area Network (WPAN) | IEEE |  |  |
| 802.16 | WiMAX Wireless Metropolitan Area Networks | WiMAX Forum IEEE |  |  |
| IETF | ~~ | IETF |  |  |
| IP (v4) | Internet Protocol | IETF |  | RFC791 Updated by: RFC1349, RFC2474, RFC6864 |
| IPv6 | Internet Protocol version 6 | IETF |  | RFC2460 Updated by: RFC5095, RFC5722, RFC5871, RFC6437, RFC6564, RFC6935, RFC6946 |
| TCP | Transmission Control Protocol | IETF |  | RFC793 Updated by: RFC1122, RFC3168, RFC6093, RFC6528 |
| UDP | User Datagram Protocol | IETF |  | RFC768 |
| Instabus | -superceded- |  |  | see KNX |
| IPDR | IP Data Record | TM Forum |  |  |
| IrDA | ~ | Infrared Data Association |  |  |
| FIR | Fast IrDA | Infrared Data Association |  |  |
| SIR | Serial Infrared | Infrared Data Association |  |  |
| IRsimple™ | IrDA Simple (high-speed wireless) | Infrared Data Association |  |  |
| ISA100.11a | ISA100.11a | ISA ISA100 Wireless Compliance Institute (WCI) |  |  |
| ISO 21215 CALM M5 | Communication Access for Land Mobile | ISO TC 204/WG 16 |  |  |
| KNX | KNX (Konnex) | KNX Association |  | ISO/IEC 14543-3 CENELEC EN 50090 CEN EN 13321-1 [CN] GB/Z 20965 |
| HGI (in progress) | ~~ | Home Gateway Initiative (HGI) |  | Partner Type 2 |
| RWD036 | Smart Home Architecture and System Requirements | Home Gateway Initiative (HGI) |  |  |
| RWD043 | Requirements for RP1 on the Smart Home Platform | Home Gateway Initiative (HGI) |  |  |
| GWD042 | Smart Home Appliance (Device) Model Template | Home Gateway Initiative (HGI) |  |  |
| HL7 | Health Level Seven | Health Level Seven International |  |  |
| Jabber | - |  |  | see XMPP |
| LonWorks® | Control Network Protocol Specification | LonMark [Echelon] |  | ANSI/CEA-709.1-B SO/IEC 14908-1) |
| M2MXML | Machine-To-Machine XML-based Protocol | M2MXML Project |  | (open source) |
| Mango | Mango Automation | [Serotonin Software] |  | (open source) |
| M-Bus | Meter Bus | M-Bus Usergroup |  | EN 13757-2, -3 |
| MiWi | Microchip P2P Wireless Protocol | [Microchip Tech] |  | IEEE 802.15.4 |
| ModBus | Modicon Bus | Modbus Organization [Schneider Automation] (was Modicon) |  |  |
| ModBus TCP (in progress) | Modicon Bus TCP/IP | Modbus Organization [Schneider Automation] |  |  |
| MyriaNed® | Self organizing Wireless Sensor Network | [DevLab] [Chess] |  |  |
| OASIS | ~~ | OASIS |  |  |
| AMQP | Advanced Message Queuing Protocol | OASIS AMQP TC [JPMorgan-Chase] |  |  |
| MQTT | Message Queuing Telemetry Transport | OASIS MQTT TC MQTT.org [IBM/Eurotec (Arcom)] |  | See Clause 7.2 |
| oBIX | Open Building Information Exchange | OASIS oBIX TC oBix |  |  |
| OMA M2M Enablers | ~~ | Open Mobile Alliance |  | Partner Type 2 |
| CPNS | Converged Personal Network Services | Open Mobile Alliance |  |  |
| DM 1.3 | Device Management | Open Mobile Alliance |  |  |
| DM 2.0 | Device Management | Open Mobile Alliance |  |  |
| GwMO | Gateway Management Object | Open Mobile Alliance |  |  |
| LWM2M | Lightweight M2M protocol | Open Mobile Alliance |  |  |
| M2M DC | M2M Device Classification | Open Mobile Alliance |  |  |
| OCMAPI | Open Connection Manager API | Open Mobile Alliance |  |  |
| oneNet | Low Power Wireless Protocol | ONE-NET |  | (open source) |
| OpenSCADA | -superceded- | - |  | see Eclipse SCADA |
| OpenTag | -superceded- | - |  | see DASH7 |
| OpenWSN | Open Wireless Sensor Networks | OpenWSN Project (was UC Berkeley) |  | (open source) |
| OSGi™ R5 | OSGi R5 for embedded devices | OSGi™ Alliance |  |  |
| OSGP | Open Smart Grid Protocol | ETSI (was ISG OSG) |  | GS OSG 001 Ver. 1.1.1 with ISO/IEC 14908 |
| OSIAN | Open Source IPv6 Automation Network | OSIAN Project |  | (open source) |
| Profibus | Process Field Bus | Profibus & Profinet International (PI) |  |  |
| RFID | Radio-Frequency IDentification |  |  |  |
| EN 300 220 | ERM Short Range Devices | ETSI |  |  |
| EN 302 208 | ERM Radio Frequency Identification Equipment | ETSI |  |  |
| EPC Gen2 | EPCglobal UHF Class 1 Generation 2 | EPCglobal |  |  |
| ISO/IEC 14443 | HighFID Proximity Card | ISO/IEC |  |  |
| ISO/IEC 15693 | HighFID non-contact Smart Tags | ISO/IEC |  |  |
| ISO/IEC 18000 | Radio frequency identification for item management | ISO/IEC |  |  |
| ISO/IEC 18092 | Near Field Communication NFCP-1 | ISO/IEC |  |  |
| ISO/IEC 21481 | Near Field Communication NFCP-2 | ISO/IEC |  |  |
| RS-232 | -superceded- | EIA |  | see TIA-232-F |
| RS-422 | -superceded- | EIA |  | see TIA -422 |
| RS-485 | -superceded- | EIA |  | see TIA-485 |
| RuBee | High security Wireless Asset Visibility Network | RuBee [Visible Assets] |  | IEEE 1902.1, 1901.2 |
| Sinec H1 | Siemens Ethernet Control - H1 -legacy- | [Siemens] |  |  |
| SOAP | Simple Object Access Protocol | W3C |  |  |
| SMART | -superceded- |  |  | see Bluetooth SMART |
| SmartBus | SmartBus Home Automation Control (HAC) | Smart Home Group [Digitcom Technology] |  |  |
| SMS | Short Message Service | 3GPP |  | TS 23.040 |
| SNMP | ~~ | IETF OPSA WG (was SNMP WG) |  |  |
| SNMP v3 | Simple Network Management Protocol v3 | IETF OPSA WG |  | RFC3411 |
| SNMP MIB | Management Information Block | IETF OPSA WG |  | RFC3418 |
| TIA | ~~ | TIA |  | Partner Type 1 |
| TIA-232-F | Interface Between Data Terminal Equipment and Data Circuit-Terminating Equipment Employing Serial Binary Data Interchange | TIA TR-30 (was EIA RS-232) |  |  |
| TIA-422 | Electrical Characteristics of the Balanced Voltage Digital Interface Circuit | TIA TR-30 (was EIA RS-422) |  |  |
| TIA-485 | Electrical Characteristics of Generators and Receivers for Use in Balanced Digital Multipoint Systems | TIA TR-30 (was EIA RS-485) |  |  |
| TIA-4940.020 | Smart Device Communications Protocol Aspects | TIA TR-50 |  | Partner Type 1 oneM2M Pool Document |
| UPnP | Universal Plug and Play | UPnP Forum |  |  |
| TR-069 | CPE WAN Management Protocol | Broadband Forum (BBF) |  | Partner Type 2 |
| VSCP | Very Simple Control Protocol | VSCP Project [Grodans Paradis AB] |  | (open source) |
| WAP | Wireless Application Protocol | Open Mobile Alliance (was: WAP Forum) |  |  |
| WAVE2M | Open Low-Power Wireless | WAVE2M |  |  |
| Wavenis | -superceded- |  |  | see WAVE2M |
| Weightless 1.0 | Low-power White-space Wireless Network | Weightless SIG [Neul] |  |  |
| Wibree | -superceded- |  |  | see Bluetooth SMART |
| WirelessHART | Wireless Highway Addressable Remote Transducer | HART Communication Foundation |  | IEEE 802.15.4 / IEC 62591 |
| Wireless HD | Wireless High-Definition Digital Interface | WirelessHD Consortium |  |  |
| Wireless USB | Wireless Universal Serial Bus | USB Implementers Forum |  |  |
| WorldFIP | World Factory Instrumentation Protocol | WorldFIP |  |  |
| X-10 | X-10 | [X-10 (USA)] |  |  |
| xAP | XAP Home Automation Protocol | XAP Forum |  | (open source) |
| xPL | xPL Home Automation Project | xPL Project |  |  |
| XMPP | eXtensible Messaging and Presence Protocol | IETF XMPP WG |  | RFC6120 |
| XMPP XEP | eXtensible Messaging and Presence Protocol Extensions | XMPP Standards Foundation |  |  |
| ZigBee® | ZigBee 2012 | ZigBee Alliance |  | IEEE 802.15.4 |
| ZigBee IP | ZigBee IPv6 for Smart Energy | ZigBee Alliance |  |  |
| ZigBee RF4CE | ZigBee for Consumer Electronics | ZigBee Alliance |  |  |
| SEP 2 | Smart Energy Profile 2.0 | CSEP ZigBee Alliance |  |  |
| Z-Wave | Wireless RF-based Communications Technology | Z-Wave Alliance Z-Wave |  | ITU G.9959 |

# Annex B
Definitions of Radio metrics for Technologies used for M2M releated Protocols (Informative)

NOTE:  The following definitions are quoted for reference from publicly-available sources, and no representation is made regarding the accuracy or timeliness of the information it contains. In addition, the appearance or omission of any M2M-related information in this list does not imply either the intention, or lack of intention, to undertake any normative or other work within oneM2M.

|  |  | ZigBee | Bluetooth | 802.11b | 802.11g | 802.11a | 802.11n | UWB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput | Mbps | 0.03 | 1-3 | 11 | 54 | 54 | 200 | 200 |
| Max range | ft | 75 | 30 | 200 | 200 | 150 | 150 | 30 |
| Sweet spot | Mbps-ft | .03@75 | 1-3@10 | 2@200 | 2@200 | 36@100 | 100@100 | 200@10 |
| Service | bps-ft2 | 530 | 314M | 251G | 251G | 1.13T | 3.14T | 62G |
| Power | mW | 30 | 100 | 750 | 1000 | 1500 | 2000 | 400 |
| Bandwidth | MHz | 0.6 | 1 | 22 | 20 | 20 | 40 | 500 |
| Spectral efficiency | b/Hz | 0.05 | 1 | 0.5 | 2.7 | 2.7 | 5 | 0.4 |
| Power efficiency1 | mW/Mbps | 1000 | 100 | 68 | 19 | 27 | 10 | 2 |
| Power efficiency2 | mAh/GB | 2211 | 67 | 46 | 12 | 18 | 7 | 1.3 |
| TTGB | Time | 3.1 day | 2.2 hr | 12 min | 2.5 min | 2.5 min | 40 sec | 40 sec |
| Price | US$ | $2 | $3 | $5 | $9 | $12 | $20 | $7 |
| Note: TTGB: Time To Generate (the time to live for a security session key). |  |  |  |  |  |  |  |  |

Table B-1: Wireless access technologies in the M2M landscape

### B.1 	Bluetooth® Wireless Technology

- Bluetooth wireless technology is geared towards voice and data applications

- Bluetooth wireless technology operates in the unlicensed 2.4 GHz spectrum

- The range of Bluetooth wireless technology is application specific. The Bluetooth Specification mandates operation over a minimum distance of 10 meters or 100 meters depending on the Bluetooth device class, but there is not a range limit for the technology. Manufacturers may tune their implementations to support the distance required by the use case they are enabling.

- The peak data rate with EDR is 3 Mbps

- Bluetooth wireless technology is able to penetrate solid objects

- Bluetooth technology is omni-directional and does not require line-of-sight positioning of connected devices

- Security has always been and continues to be a priority in the development of the Bluetooth specification. The Bluetooth specification allows for three modes of security

### B.2	ZigBee (IEEE 802.15.4)

The promoter companies of the ZigBee Alliance include: Philips, Honeywell, Mitsubishi Electric, Motorola, Samsung, BM Group, Chipcon, Freescale and Ember; more than 70 members

- Capacity of 250 Kbits at 2.4 GHz, 40 Kpbs at 915 Mhz, and 20 Kpbs at 868 Mhz with a range of 10-100 M

- Its purpose is to become a wireless standard for remote control in the industrial field

- The ZigBee technology is targeting the control applications industry, which does not require high data rates, but must have low power, low cost and ease of use (remote controls, home automation, etc.)

- The specification was formally adopted in December 2004

- Security was not considered in the initial development of the specification. Currently there are three levels of security

### B.3	Ultra-Wideband (UWB)

- UWB technology for Personal Area Networks offers a unique combination of low power consumption (~1mW/Mbps) and high data throughput (up to 480 Mbps).

- WiMedia UWB is an internationally recognized standard (ECMA-368, ISO/IEC 26970 and ECMA-369, ISO/IEC 26908) and has regulatory approval in major markets worldwide, including US, EU, Korea and Japan. Additional regions, e.g. China and Canada are expecting regulatory approval in the near future.

- Ideally, it will have low power consumption, low price, high speed, use a wide swath of radio spectrum, carry signals through obstacles (doors, etc.) and apply to a wide range of applications (defense, industry, home, etc.)

- WiMedia UWB takes a "Common Radio Platform" approach allowing the same radio to be used for a variety of applications.

- WiMedia UWB allows for data rates up to 480Mbps at ranges of several meters and a data rate of approximately 110 Mbps at a range of up to 10 meters

- While Wireless USB has initially utilized UWB technology, it is expected that the Bluetooth high speed solution will not suffer the same performance and interoperability issues due to the specification development and qualification process employed by the Bluetooth SIG.

- The Bluetooth SIG announced in May 2005 its intentions to work with UWB to develop a high rate Bluetooth specification on the UWB radio

### B.4	Certified Wireless USB

- Speed: Wireless USB is projected to be 480 Mbps up to 2 meters and 110 Mbps for up to 10 meters. Wireless USB hub can host up to 127 wireless USB devices

- Wireless USB will be based on and run over the UWB radio promoted by the WiMedia Alliance.

- Allows point-to-point connectivity between devices and the Wireless USB hub

- Intel established the Wireless USB Promoter Group in February 2004

- The USB Implementers Forum, Inc. (USB-IF) tests and certifies the "certified Wireless USB" based wireless equipment

### B.5	Wi-Fi (IEEE 802.11)

- Bluetooth technology uses a fifth of the power of Wi-Fi

- The Wi-Fi Alliance tests and certifies 802.11 based wireless equipment

- 802.11a: This uses OFDM, operates in the 5 GHz range, and has a maximum data rate of 54 Mbps

- 802.11b: Operates in the 2.4 GHz range, has a maximum data rate of 11 Mbps and uses DSSS. 802.11b is the original Wi-Fi standard

- 802.11g: Operates in the 2.4 GHz range, uses OFDM and has a maximum data rate of 54 Mbps. This is backwards compatible with 802.11b

- 802.11e: This standard will improve quality of service

- 802.11h: This standard is a supplement to 802.11a in Europe and will provide spectrum and power control management. Under this standard, dynamic frequency selection (FS) and transmit power control (TPC) are added to the 802.11a specification

- 802.11i: This standard is for enhanced security. It includes the advanced encryption standard (AES). This standard is not completely backwards compatible and some users will have to upgrade their hardware. The full 802.11i support is also referred to as WPA2

- 802.11k: Under development, this amendment to the standard should allow for increased radio resource management on 802.11 networks

- 802.11n: This standard is expected to operate in the 5 GHz range and offer a maximum data rate of over 100 Mbps (though some proposals are seeking upwards of 500 Mbps). 802.11n will handle wireless multimedia applications better than the other 802.11 standards

- 802.11p: This standard will operate in the automotive-allocated 5.9 GHz spectrum. It will be the basis for the dedicated short range communications (DSRC) in North America. The DSRC will allow vehicle to vehicle and vehicle to roadside infrastructure communication

- 802.11r: This amendment to the standard will improve users' ability to roam between access points or base stations. The task group developing this form in spring/summer 2004

- 802.11s: Under development, this amendment to the standard will allow for mesh networking on 802.11 networks. The task group developing this formed in spring/summer 2004.

Editor’s Note: verify Dates/status

### B.6	Radio Frequency Identification (RFID)

There are over 140 different ISO standards for RFID for a broad range of applications

- With RFID, a passive or unpowered tag can be powered at a distance by a reader device. The receiver, which must be within a few feet, pulls information off the 'tag,' and then looks up more information from a database. Alternatively, some tags are self-powered, 'active' tags that can be read from a greater distance

- RFID can operate in low frequency (less than 100 MHz), high frequency (more than 100 MHz), and UHF (868 to 954 MHz)

- Uses include tracking inventory both in shipment and on retail shelves

### B.7	Near Field Communication (NFC)

The NFC Forum is involved in the development and promotion of NFC. The 12 sponsor members of the NFC Forum include MasterCard International, Microsoft, Motorola, NEC, Nokia, Panasonic, Philips, Renesas, Samsung Electronics, Sony, Texas Instruments and Visa

- Capacity: 212 kbps over a distance from 0 to 20 centimeters over the 13.56 Mhz frequency range

- The NFC standard is based on RFID technology

- Applications suggested for NFC include ticketing, payment and gaming.

- Support for a passive mode of communication leads to savings on battery power

# Annex Z
Bibliography

Note: It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

<Publication>: "<Title>".

<Publication>: "<Title>".

<Publication>: "<Title>".

# History

| Approval history |  |  |
| --- | --- | --- |
| V.1.x.x | xx-mmm-2013 | <Approved Version> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | dd Mmm 2013 | Skeleton Draft |
| V.0.1.1 | 08 Aug 2013 | Output Draft - PRO WG3 at TP#6 - Toronto - Including Contributions: oneM2M-PRO-2013-023, -024R01, -029R02 |
| V0.1.2 | 08 Aug 2013 | Revised Output Draft - PRO WG3 at TP#6 - Toronto: adding input from oneM2M-PRO-2013-025R03, |
| V0.2.0 | 02 Sep 2013 | PRO WG3 Agreed output from TP#6 |
| V0.2.1 | 25 Sep 2013 | Output from PRO WG3 meeting 11 Sep 2013, including oneM2M-PRP-2013-0034R02, oneM2M-PRP-2013-0041R01 |
| V0.2.2 | 30 Sep 2013 | Output from PRO WG3 meeting 25 Sep 2013, including V0.2.1 and oneM2M-PRO-2013-0044R01, oneM2M-PRO-2013-0045, and oneM2M-PRO-2013-0046R01. |
| V0.2.3 | 23 Oct 2013 | Output Draft - PRO WG3 at TP#7 - Sophia Antipolis; Including Contributions: oneM2M-PRO-2013-0050R02, 0051R01 (w/text from 0058), -0053R02, -0054R03, -0056R02, and -0062. Added references and acronyms. |
| V0.3.0 | 03 Nov 2013 | PRO WG3 Agreed output from TP#7 |