| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0004-Protocols-V-0.4.0 |
| Document Name: | oneM2M Service Layer Protocol Core Specification |
| Date: | 2014-April-22 |
| Abstract: | < An abstract of the document that may be used to help reader decide whether to read the entire document, and provide information that may be used in subsequent electronic searches> |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	8

3.3	Abbreviations and Acronyms	8

4	Conventions	8

5	Protocol Design Principles and Requirements	8

5.1	Introduction	8

5.1.1	Reference Points	9

5.1.2	Common Service Functions	9

5.2	API Design Guidelines	9

5.3	Primitives	10

5.3.1	Introduction	10

5.3.2	Primitives modelling	10

5.3.3	Primitive principles	11

5.4	Design Principles	11

5.4.1	Scalability	11

5.4.2	Extensibility	11

5.4.3	Robustness	11

5.4.4	Efficiency	11

5.4.5	Security	11

6	oneM2M Protocols / API Overview	11

6.1	Introduction	11

6.2	M2M Identifiers	12

6.3	Common Data Types	12

6.3.1	Simple Data Types incorporated from XML Schema	12

6.3.2	Additional Data Types used in oneM2M	15

6.3.2.1	oneM2M Data Types	15

6.3.2.2	ExternalID	15

6.3.2.3	Id	15

6.3.3	Resource common attributes	16

6.4	Structured Data Types	16

6.4.1	regularResourceType	17

6.4.2	announceableResourceType	18

6.4.3	subordinateResourceType	18

6.5	oneM2M Device Management  API	18

6.5.1	Introduction	18

6.5.2	Management Data Types	18

6.5.3	<mgmtObj> Resources and Procedures	19

6.5.3.1	<mgmtObj> Resources	19

6.5.3.2	<mgmtObj> Primitives and Procedures	19

6.5.4	<mgmtCmd> Resources and Procedures	19

6.5.4.1	<mgmtCmd> Resource	19

6.5.4.2	<mgmtCmd> Primitive and Procedures	19

6.6	oneM2M Connectivity Management API	19

6.7	oneM2M Data Management API	19

7	19

8	oneM2M Resource Types	20

8.1	Introduction	20

7.2	Prerequisites	20

7.2.1	Primitive templates and procedure outlines	20

7.2.2	Common operations	20

7.2.2.1	Originator Actions	20

7.2.2.1.1	Compose Request primitive	20

7.2.2.1.2	Send a Request to the Receiver CSE	20

7.2.2.1.3	Wait for Response primitive	20

7.2.2.2	Hosting CSE actions	21

7.2.2.2.1	Check existence of the addressed resource	21

7.2.2.2.2	Check the syntax of received message	21

7.2.2.2.3	Check validity of resource representation for CREATE	21

7.2.2.2.4	Check validity of resource representation for UPDATE	22

7.2.2.2.5	Create the resource	22

7.2.2.2.6	Create a successful Response	23

7.2.2.2.7	Create an unsuccessful Response	23

7.2.2.2.8	Read the addressed resource	23

7.2.2.2.9	Update the addressed resource	23

7.2.2.2.10	Delete the addressed resource	24

7.2.2.2.11	Send Response primitive	24

7.2.2.3	Receiver CSE actions	24

7.2.3	Resource Type definition	24

7.3	Resource Type-specific procedures and definitions	24

7.3.1	Resource Type CSEBase	24

7.3.2	Resource Type remoteCSE	24

7.3.3	Resource Type application	24

7.3.4	Resource Type container	24

7.3.5	Resource Type instance	25

7.3.6	Resource Type subscription	25

7.3.7	Resource Type schedule	25

7.3.8	Resource Type locationPolicy	25

7.3.9	Resource Type delivery	25

7.3.10	Resource Type request	25

7.3.11	Resource Type group	25

7.3.12	Resource Type Members	25

7.3.13	Resource Type mgmtObj	25

7.3.14	Resource Type parameters	25

7.3.15	Resource Type mgmtCmd	25

7.3.16	Resource Type execInstance	25

7.3.17	Resource Type node	25

7.3.18	Resource Type m2mServiceSubscription	25

7.3.19	Resource Type nodeInfo	26

7.3.20	Resource Type pollingChannel	26

7.3.21	Resource Type cmdhPolicy	26

7.3.22	Resource Type cmdhDefaults	26

7.3.23	Resource Type cmdhDefEcValue	26

7.3.24	Resource Type cmdhEcDefParamValues	26

7.3.25	Resource Type cmdhLimits	26

7.3.26	Resource Type cmdhNetworkAccessRules	26

7.3.27	Resource Type cmdhNwAccessRule	26

7.3.28	Resource Type cmdhBuffer	26

7.3.29	Resource Type statsConfig	26

7.3.30	Resource Type eventConfig	26

7.3.31	Resource Type statsCollect	26

Annex A (Normative): Resource attributes	27

Annex B (Normative): Device Triggering	37

B.1	Providing Device Triggering service by means of 3GPP networks	37

B.1.1	Introduction	37

B.1.2	Device Action Request command	37

B.1.3	Device Action Answer command	39

B.1.4	Device Notification Request command	39

B.1.5	Device Notification Answer command	40

Annex Z.	Template for oneM2M resource type	40

7.3.x	Resource Type <<resource name>>	40

7.3.x.1	Introduction	40

7.3.x.2	Operations	41

7.3.x.2.1	Create	41

7.3.x.2	Retrieve	41

7.3.x.3	Update	41

7.3.x.4	Delete	41

History	42

# 1	Scope

The present document specifies the communication protocol(s)  for oneM2M compliant Systems,  M2M Applications, and/or other M2MSystems.

This document also specifies the  common data formats, interfaces and message sequences to support reference points(s) defined by oneM2M.

The Scope shall not contain requirements.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Clause 2.1 only shall contain normative (essential) references which are cited in the document itself. These references have to be publicly available and in English.

The following referenced documents are necessary for the application of the present document.

- Use the EX style, enclose the number in square brackets and separate it from the title with a tab (you may use sequence fields for automatically numbering references, see clause A.4: "Sequence numbering") (see example).

[1]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[2]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[3]	W3C XMLSchemaP2: "W3C Recommendation (2004), XML Schema Part 2:Datatypes Second Edition.".

[4]	ISO 19136 (2007): "Geographic information -- Geography Markup Language (GML).".

[5]	W3C XMLPath: "W3C Recommendation (2010), XML Path Language (XPath) 2.0 (Second Edition).".

[6]	oneM2M TS-0001 “Functional Architecture”. TBD.

[7]

[8]	IEEE 754-2008: IEEE. IEEE Standard for Floating-Point Arithmetic. 29 August 2008.
http://ieeexplore.ieee.org/servlet/opac?punumber=4610933

[9]	IETF RFC 3548: “The Base16, Base32, and Base64 Data Encodings”. 2003.

[10]	IETF RFC 2045: “Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies”. 1996.

[11]	IETF RFC 3987:“ Internationalized Resource Identifiers (IRIs)” . January 2005.

[12]	IETF BCP 47: “Best Current Practices 47”. Concatenation of RFC 4646:“ Tags for Identifying Languages”(2006) and RFC 4647: “Matching of Language Tags”( 2006).

[13]	IETF RFC 3588: “Diameter Base Protocol”. September 2003.

[14]	IETF RFC 6733: “Diameter Base Protocol”. October 2012.

[15]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”.

[16]	3GPP TS 29.368: “Tsp interface protocol between the MTC Interworking Function (MTC-IWF) and Service Capability Server (SCS)”.

Editor:s Note: Further documents to add TBD

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	Fielding, Roy Thomas (2000): "Architectural Styles and the Design of Network-based Software Architectures", Doctoral dissertation, University of California, Irvine.

# 3	Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations and Acronyms

Abbreviations and acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION/ACRONYM1>	<Explanation>

<ABREVIATION/ACRONYM2>	<Explanation>

<ABREVIATION/ACRONYM3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Protocol Design Principles and Requirements

The following sub-clauses contain the design principles and requirements for the oneM2M protocol.

Editor’s Note: 
The following sub-clauses are intended to provide design principle and specify aspects of protocol requirements, including but not limited to scalability, performance, common message format, reliability, security, extensibility, robustness, resilience, efficiency, message minimisation, etc. Requirements derived from the Requirements TS and inferred from the evolving Architecture TS should be captured here and expressed in terms of Protocol Requirements. Beyond these, additional Protocol requirements are expected. More contributions are requested

## 5.1	Introduction

The oneM2M architecture is resource-based, (see oneM2M TS-0001 [6]). The functionality of the system is exposed by means of APIs over the reference points specified in [6]. Operations upon resources hosted by a CSE are carried over an established channel that constitutes the communication on the reference points Mca and Mcc.

Each resource operation comprises a pair of primitives: Request and Response.

In order to provide a well-defined interface for the reference points in the architecture [6], the following aspects need to be provided:

- The collection of primitives carried over a specific reference point, and

- The description and applicability of security methods in relation to the underlying protocols and reference points involved.

The current document provides:

- Data type definitions,

- Primitive definitions, and

- XML definitions and schema.

NOTE: The actual binding of the interface to a specific protocol is not part of this document, but is specified in a separate TS.

In accordance with the oneM2M architecture, each reference point is applicable to a wide range of underlying network technologies and transport protocols. oneM2M will only define a set of bindings for specific underlying network technologies and transport protocols, these bindings are not limiting the applicability of the interfaces when used in other underlying networks and transport protocols. However the behaviour of the interface needs to be respected in accordance to this document and the architecture (see [6]).

### 5.1.1	Reference Points

### 5.1.2	Common Service Functions

## 5.2	API Design Guidelines

The following are the guidelines for designing APIs:

- API shall follow the principle of RESTful architecture, as described in [i.2]

- APIs shall define how to address resources and how to manipulate resources, in accordance with oneM2M TS-0001 [6]; the resource is identified by a Universal Resource Identifier (URI), [2].

- Resource has a representation (see [i.2]) that shall be transferred and manipulated with the verbs. These verbs are identified as operations in [6]: CREATE. RETRIEVE, UPDATE, DELETE and NOTIFY

- All primitives shall be defined as well as the way that those primitives are sent. The functionality of the primitives shall be compliant to the resource type specific procedure as specified in [6], clause 10.2

- API shall provide the format and syntax of the operation primitives for all resources defined in [6]. In case that for a particular protocol binding an operation cannot be supported it has to be clearly stated in the specific protocol binding technical specification.

- Primitives shall include attributes in accordance with [6] for a specific resource

- Primitive shall be self-descriptive and contain all the information needed for the receiver of the primitives to handle the primitives.

- Primitive should be idempotent operations which means no matter how many times the primitive is sent, the result doesn’t change, in accordance to [i.2]. .

- API shall indicate which features are supported and not supported over the reference points specified in [6].

- Primitives shall be mapped on the transport layer protocols.

## 5.3	Primitives

### 5.3.1	Introduction

Primitives are service layer messages transmitted over the Mca and Mcc reference points. The Create, Update, Retrieve, Delete and Notify operation is mapped to one or more primitives. The primitive is then further mapped to transport layer protocols such as HTTP, CoAP or MQTT for the transmission. The primitive is independent from the transport protocols.

Figure 5.3.1-1: Primitive overview

A single primitive in the application/service layer may be mapped to zero or more transport messages in the transport layer.

The Originators shall send requests to Receivers through primitives. The Originator and Receiver may be AE or CSE. The CRUD request primitive addresses a resource residing in a CSE. The Notify request primitive may address an AE or CSE.

Each CRUD+N operation shall consist of one request and one response primitive.

Communication between an originator and receiver in the Application/Service layer shall be performed with the above primitives.

Communication between an originator and receiver in the Transport layer shall be performed using underlying network.

### 5.3.2	Primitives modelling

Primitives shall be modelled as follows.

A primitive shall be a data structure that describes with appropriate attributes a specific procedure request or answer in both originator and receiver entities.

A primitive shall consist of:

- Control part: contains attributes required for the processing of the request or response, and

- Optional content part: user data. This is the representation of the resource or the value of the attribute in partial addressing case.

Figure 5.3.2-1: Primitives modeling

### 5.3.3	Primitive principles

Primitives shall be atomic. When two concurrent request primitives affect the same resource, the execution of one primitive shall finish completely before the second one starts.

When creating or updating the resource, the representation of the resource shall be contained in the Content part of the primitive. Based on the representation of the resource, the Hosting CSE of the addressed resource can create or update the entire resource without the need for further information.

The handling of primitives shall be idempotent. This means no matter how many times the same primitive is targeted to the same resource, the resource does not change after the first handling of the primitive.

## 5.4	Design Principles

### 5.4.1	Scalability

### 5.4.2	Extensibility

### 5.4.3	Robustness

### 5.4.4	Efficiency

Energy Efficiency:

- As energy consumption directly affects the overall system performance, oneM2M protocols should consider energy efficiency, especially in resource constrained environments with battery-powered oneM2M devices.

- Energy efficient oneM2M protocols shall aim at reducing the overall energy consumption while maintaining the performance required by the oneM2M Applications.

### 5.4.5	Security

# 6	oneM2M Protocols / API Overview

## 6.1	Introduction

This technical specification describes message formats and procedures to communicate with oneM2M compliant M2M Platform System.

This specification describes:

- Data representation for communication protocol messages

- Normal and exceptional procedure

- Status codes

- Guidelines for drafting APIs

## 6.2	M2M Identifiers

This clause describes all identifiers from [6] and the data types used to represent them.

Table 6.2-1: M2M Identifiers

| Identifier | Data Type | Description |
| --- | --- | --- |
| M2M-SP-ID | m2m:id (see clause X.X) | A globally unique ID as specified in [6] |
| App-ID | m2m:id (see clause X.X) | The identifier is specified in [6] |
| AE-ID | m2m:id (see clause X.X) | A globally unique ID as specified in [6] |
| CSE-ID | m2m:id (see clause X.X) | A globally unique ID as specified in [6] |
| M2M-Node-ID | TBD (see clause X.X) | A globally unique ID as specified in [6] |
| M2M-Sub-ID | m2m:id (see clause X.X) | A globally unique ID as specified in [6]. |
| M2M-Request-ID | TBD (see clause X.X) | A unique ID as specified in [6] |
| M2M-Ext-ID | ExternalID (see clause 6.3.2.2) | The identifier is specified in [6] |
| UNetwork-ID | m2m:id (see clause X.X) | A unique ID as specified in [6] |
| Trigger-Recipient-ID | TBD (see clause X.X) | The identifier is specified in [6] |
| M2M-Serv-ID | TBD | The identifier is specified in [6] |

## 6.3	Common Data Types

For wide acceptance by industrial markets, this specification describes structured and non-structured data for oneM2M Protocol using XML Schema Language (aka XSD)[3].

Note that the actual data format is depends on chosen Protocol Bindings. Each protocol binding specification will specify the mapping rule between XSD version of data definitions and its native data format.

Any Data Types and XML elements defined for use in oneM2M protocols shall use the namespace 
http://www.onem2m.org/xml/protocols.

This document, and any XML or XML Schema Documents produced by oneM2M shall use the prefix m2m: to refer to that namespace.

### 6.3.1	Simple Data Types incorporated from XML Schema

Following ‘built-in data types’ are incorporated from XML Schema definition [3].

Note that name space identifier for ‘http://www.w3.org/2001/XMLSchema' shall be described as ‘xs:’ in this specification.

Table 6.3.1-1: Data Types from XML Schema

| Data Type | Description | Notes |
| --- | --- | --- |
| xs:string | The string datatype represents character strings in XML |  |
| xs:boolean | boolean represents the values of two-valued logic. |  |
| xs:decimal | decimal represents a subset of the real numbers, which can be represented by decimal numerals. The ·value space· of decimal is the set of numbers that can be obtained by dividing an integer by a non-negative power of ten, i.e., expressible as i / 10n where i and n are integers and n ≥ 0. Precision is not reflected in this value space; the number 2.0 is not distinct from the number 2.00. The order relation on decimal is the order relation on real numbers, restricted to this subset. |  |
| xs:float | The float datatype is patterned after the IEEE single-precision 32-bit floating point datatype IEEE 754-2008 ([8]). Its value space is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:double | The double datatype is patterned after the IEEE double-precision 64-bit floating point datatype IEEE 754-2008 ([8]). Each floating point datatype has a value space that is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:duration | duration is a datatype that represents durations of time. |  |
| xs:dateTime | dateTime represents instants of time, optionally marked with a particular time zone offset. Values representing the same instant but having different time zone offsets are equal but not identical. |  |
| xs:time | time represents instants of time that recur at the same point in each calendar day, or that occur in some arbitrary calendar day. |  |
| xs:date | date represents top-open intervals of exactly one day in length on the timelines of dateTime, beginning on the beginning moment of each day, up to but not including the beginning moment of the next day). For non-timezoned values, the top-open intervals disjointly cover the non-timezoned timeline, one per day. For timezoned values, the intervals begin at every minute and therefore overlap. |  |
| xs:hexBinary | hexBinary represents arbitrary hex-encoded binary data. |  |
| xs:base64Binary | base64Binary represents arbitrary Base64-encoded binary data. For base64Binary data the entire binary stream is encoded using the Base64 Encoding defined in RFC 3548([9]), which is derived from the encoding described in RFC 2045([10]). |  |
| xs:anyURI | anyURI represents an Internationalized Resource Identifier Reference (IRI). An anyURI value can be absolute or relative, and may have an optional fragment identifier (i.e., it may be an IRI Reference). This type should be used when the value fulfills the role of an IRI, as defined in RFC 3987([11]) or its successor(s) in the IETF Standards Track. |  |
| xs:normalizedString | normalizedString represents white space normalized strings. The ·value space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The ·lexical space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The ·base type· of normalizedString is string. |  |
| xs:token | token represents tokenized strings. The ·value space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The ·lexical space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The ·base type· of token is normalizedString. |  |
| xs:language | language represents formal natural language identifiers, as defined by BCP 47([12]). |  |
| xs:integer | integer is ·derived· from decimal by fixing the value of ·fractionDigits· to be 0 and disallowing the trailing decimal point. This results in the standard mathematical concept of the integer numbers. The ·value space· of integer is the infinite set {...,-2,-1,0,1,2,...}. The ·base type· of integer is decimal. |  |
| xs:nonNegativeInteger | nonNegativeInteger has a lexical representation consisting of an optional sign followed by a non-empty finite-length sequence of decimal digits (#x30-#x39). If the sign is omitted, the positive sign ('+') is assumed. If the sign is present, it must be "+" except for lexical forms denoting zero, which may be preceded by a positive ('+') or a negative ('-') sign. For example: 1, 0, 12678967543233, +100000. |  |
| xs:positiveInteger | positiveInteger is ·derived· from nonNegativeInteger by setting the value of ·minInclusive· to be 1. This results in the standard mathematical concept of the positive integer numbers. The ·value space· of positiveInteger is the infinite set {1,2,...}. The ·base type· of positiveInteger is nonNegativeInteger. |  |
| xs:unsignedLong | unsignedLong is derived· from nonNegativeInteger by setting the value of ·maxInclusive· to be 18446744073709551615. The ·base type· of unsignedLong is nonNegativeInteger. |  |
| xs:unsignedInt | unsignedInt is ·derived· from unsignedLong by setting the value of ·maxInclusive· to be 4294967295. The ·base type· of unsignedInt is unsignedLong. |  |
| xs:unsignedShort | unsignedShort is ·derived· from unsignedInt by setting the value of ·maxInclusive· to be 65535. The ·base type· of unsignedShort is unsignedInt. |  |
| xs:dateTimeStamp | The dateTimeStamp datatype is derived· from dateTime by giving the value required to its explicitTimezone facet. The result is that all values of dateTimeStamp are required to have explicit time zone offsets and the datatype is totally ordered. |  |

Editor's Note:  How to choose the local time zone at timestamp is FFS.

### 6.3.2	Additional Data Types used in oneM2M

This clause defines the complex data types that are used by oneM2M API.

#### 6.3.2.1	oneM2M Data Types

Table 6.3.2.1-1 describes oneM2M-specific Data Type definitions.

Table 6.3.2.1-1: oneM2M-specific Data Type

| Data Type | Type Name | Description |
| --- | --- | --- |
| m2m:id | Generic ID | Used to represent generic IDs generated and used within oneM2M |
| m2m:nodeId | Node ID | Used for Node IDs. The constraints on this type are different from those on Generic IDs |
| m2m:requestId | Request ID | Used for Request IDs. This type includes the ID of the target CSE as well as a part that varies for each ID |
| m2m:externalId | External ID | Used for External IDs. This type includes the ID of the underlying network provider as well as a part that varies for each ID |
| m2m:acpType | ACP Type | Used to represent an AccessControlPolicy identifier. This can be either a URI or an opaque token |
| m2m:nhURI | Non-hierarchical URI | This is used to indicate that the URI cannot be interpreted in an hierarchical manner |
| m2m:resourceCode | Resource Code | A 2 capital letter code as defined in [7]. It is defined as an xs:string enumeration. |

#### 6.3.2.2	ExternalID

This type represents the M2M-Ext-ID.

Table 6.3.2.2-1: ExternalID Data Type

| Name | Type | Description |
| --- | --- | --- |
| UNetwork-ID | m2m:id | The identifier of the underlying network provider for which the external Identifier can be used. |
| Id | AnyURI | The identifier of the node for the underlying network provider |

#### 6.3.2.3	Id

TBD.

### 6.3.3	Resource common attributes

Resource common attributes are specified in oneM2M TS-0001 [6]. The type and values shall be supported according to the description given in Table 6.3.3-1.

Table 6.3.3-1: Resource common attributes

| Attribute Name | Data Type | Default | Value restrictions | Notes |
| --- | --- | --- | --- | --- |
| accessControlPolicyID | m2m:acpType | Policy is inherited from the parent resource |  |  |
| parentID | m2m:nhURI | Not applicable | Must be a non-hierarchical URI |  |
| creationTime | xs:dateTime | Not applicable |  |  |
| expirationTime | xs:dateTime | Absence of the attribute means that the resource has no explicit expirationTime |  |  |
| lastModifiedTime | xs:dateTime | Not applicable |  |  |
| resourceType | m2m:resourceCode | In the absence of this attribute, the resource type can be inferred from the root element |  |  |
| labels | list of xs:token | Absence of this attribute means there are no labels |  |  |
| stateTag | xs:nonNegativeInteger | Not applicable |  |  |
| link | xs:anyURI | Not applicable |  |  |
| announceTo | list of xs:anyURI | Not applicable |  |  |
| announcedAttribute | list of xs:token | Not applicable |  |  |

## 6.4	Structured Data Types

This specification defines common set of structured data as ‘Common Data Types’.

oneM2M Resource Data Types are defined in XSD, and they are supplied as separate documents. The source of XSD file can be Resource definition in accordance with oneM2M TS-0001 [6].

Although XSD files are managed independent from [6], they shall conform to the following rules:

- Each Resource Type will be defined in a separate XSD file. The name of that file should be prefixed with ‘CDT-‘ and  followed by the resource type name, version of the Core Protocol TS, and date of update. Each Resource Type file will contain a single Global Element Declaration whose name is the name of the Resource Type in accordance with [6]. This means that the root element of a Resource (when represented in XML) contains an m2m: (or equivalent) namespace prefix.

- The root element of each resource shall have an attribute called “name” which gives an identifier for that particular resource instance. A URI to the resource instance can be constructed by taking the URI of its parent and appending /<name> where <name> is the value of the name attribute.

- Each resource  attribute is represented as a child element of the top level element, and shall not have any child elements. It shall be declared as an element that is local to the resource that contains it, and so does not have a namespace prefix in any XML representation of the resource.

- Each child resource may be represented as a child element of the top level element (in which case the resource schema  should refer to the child’s resource type definition using ‘ref’ attribute) or it may be referenced via a URI. The designer of the resource schema should decide, on a case by case basis, whether it is appropriate to include the child element inline (via the first approach) or to reference it via a URI (the second approach) or whether to permit either.

Editor’s Note: Decisions must be made in the WG, not by the designer. Resolution is FFS.

- Each attribute should be declared to use either one of the simple data types listed in clause 6.3.1 or 6.3.2, or a data type derived by restriction from one of the types listed in clause 6.3.1 or 6.3.2.

- Common ‘Simple types’ are collected as dedicated XSD file, which is named 
‘CDT-simple_types-<version>-<date>.xsd’

- With the exception of CSEBase, all Resource types will extend one of the XML complex types defined in the file CDT-commonTypes-v1_0<date>.xsd.

- The resource-specific attributes and child resources shall appear as an XSD sequence, with their order being detemined by the order shown  in the tables in clause 9.6 of [6].

Table 6.4-1: List of oneM2M Resource Data Types

| Data Type ID | File Name | Where defined | Notes |
| --- | --- | --- | --- |
| Simple Types | CDT-simpleType-v1_0-<date>.xsd | 6.3.1 |  |
| CommonTypes | CDT-commonTypes-v1_0_0-<date>.xsd |  |  |

#### 6.4.1	regularResourceType

Description

This type definition includes the common attributes used by the majority of M2M resources. It is included  in CDT-commonTypes-v1_0_0-<date>.xsd .

Reference

See Clause 9.6.1 of [6].

Usage Example

<?xml version="1.0" encoding="UTF-8"?>
<xs:schema xmlns="http://www.w3.org/2001/XMLSchema"	targetNamespace=http://www.onem2m.org/xml/protocols
   xmlns:m2m=http://www.onem2m.org/xml/protocols
	elementFormDefault="unqualified" xmlns:xs="http://www.w3.org/2001/XMLSchema">
	<xs:include schemaLocation="common_types-v1_0_0-20140409.xsd" />
	<xs:include schemaLocation="CDT-instance-v1_0_0-20140409.xsd" />
	<xs:include schemaLocation="CDT-subscription-v1_0_0-20140409.xsd" />
	<xs:element name="container">
		<xs:complexType>
			<xs:complexContent>
           <!-- 	Inherit Common Attributes from regularResourceType     -->
				<xs:extension base="m2m:regularResourceType">		    
					<xs:sequence>
					    <!-- 			Resource Specific Attributes           -->
						<xs:element name="maxNrOfInstances" type="xs:nonNegativeInteger" minOccurs="0" />
						<xs:element name="maxByteSize" type="xs:nonNegativeInteger" minOccurs="0" />
						<xs:element name="maxInstanceAge" type="xs:nonNegativeInteger" 	minOccurs="0" />
						<xs:element name="currentNrOfInstances" type="xs:nonNegativeInteger" />
						<xs:element name="currentByteSize" type="xs:nonNegativeInteger" />
						<xs:element name="latest" type="xs:anyURI"  minOccurs="0" />
						<xs:element name="locationID" type="xs:anyURI" minOccurs="0" />
						<xs:element name="ontologyRef" type="xs:anyURI"  minOccurs="0" />	
						<!-- 			Child Resources                          -->
						<!--  Instances are included inline in the parent        -->			
						<xs:element ref="m2m:instance" minOccurs = "0" maxOccurs="unbounded"/>
						<!--  Subscriptions can either be included inline or referenced by URI -->
						<xs:choice>
						  <xs:element ref="m2m:subscription" minOccurs="0" maxOccurs="unbounded"/>
						  <xs:element name="subscription" type="xs:anyURI"  minOccurs="0" 
										  maxOccurs="unbounded"/>
						</xs:choice>
						<!--  Containers are always referenced by URI             -->		
					    <xs:element name="container" type="xs:anyURI"  minOccurs="0" 
											maxOccurs="unbounded"/>	
					</xs:sequence>
				</xs:extension>
			</xs:complexContent>
		</xs:complexType>
	</xs:element>
</xs:schema>

#### 6.4.2	announceableResourceType

Description

This type definition includes the common attributes used by M2M resource types that are capable of being announced.  In addition to the attributes of a regularResource, It includes (as optional)  the common attributes that are used by the announcement mechanism.

Reference

See Clause 9.6.1 of [6].

Usage Example

#### 6.4.3	subordinateResourceType

Description

This type definition includes the common attributes used by M2M resource types that are subordinate children over other resource types (for example the Instance child of a container). It excludes attributes like expirationTime, which aren’t defined for such resources.

Reference

See Clause 9.6.1 of [6].

Usage Example

Editor’s Note: This is example of description for each Common Data Type, and remove this  sub clause when actual entries are added.

## 6.5	oneM2M Device Management  API

Editor’s Note: This sub-clause should be considered to move as part of “oneM2M Resource Types”.

<Text>

The following text is to be used when appropriate:

### 6.5.1	Introduction

This section describes the management specific primitives and procedures to communicate across the Mca and Mcc reference points.

### 6.5.2	Management Data Types

This section describes the data types that are extensions of the data types defined in clause 6.2.1 and are specific to the management resource and subresources.

### 6.5.3	<mgmtObj> Resources and Procedures

This section describes the primitives and data types necessary to communicate the <mgmtObj> resource and subresources across the Mca and Mcc reference points.

#### 6.5.3.1	<mgmtObj> Resources

This section describes the definition of the <mgmtObj> resource and subresources. In addition, the policy (e.g., mandatory, optional) related to the presence of a resource’s attributes with respect to the primitives is documented in this section.

#### 6.5.3.2	<mgmtObj> Primitives and Procedures

This section describes the primitive and associated procedures for the <mgmtObj> resource and subresources.

### 6.5.4	<mgmtCmd> Resources and Procedures

This section describes the primitives and procedures to communicate the <mgmtCmd> resource and subresources across the Mca and Mcc reference points.

#### 6.5.4.1	<mgmtCmd> Resource

This section describes the definition of the <mgmtCmd> resource and subresources. In addition, the policy (e.g., mandatory, optional) related to the presence of a resource’s attributes with respect to the primitives is documented in this section.

#### 6.5.4.2	<mgmtCmd> Primitive and Procedures

This section describes the primitive and associated procedures for the <mgmtObj> resource and subresources.

## 6.6	oneM2M Connectivity Management API

Editor’s Note: This sub-clause should be considered to move as part of “oneM2M Resource Types”.

<Text>

The following text is to be used when appropriate:

## 6.7	oneM2M Data Management API

Editor’s Note: This sub-clause should be considered to move as part of “oneM2M Resource Types”.

<Text>

The following text is to be used when appropriate:

# 7

# 8	oneM2M Resource Types

## 8.1	Introduction

## 7.2	Prerequisites

### 7.2.1	Primitive templates and procedure outlines

### 7.2.2	Common operations

Editor’s Note: Actual Error handling is FFS.

Editor’s Note: the options will need to be filled in at some point

#### 7.2.2.1	Originator Actions

##### 7.2.2.1.1	Compose Request primitive

The originator shall compose a Request message that shall be mapped to a specific protocol.

The Request shall include the “fr” and “to” to indicate the identifier of the originator of the request and the targeted receiver of the request.

The Request shall include the other attributes in case needed depend on the resource the request is addressing.

When including a resource representation in the request indication for create and update, the originator shall take into account the validation rules as specified in "Check validity for resource representation for create" and "Check validity for resource representation for update" respectively. For example, any attributes marked with NP shall not be present in the resource representation for the corresponding request indication.

##### 7.2.2.1.2	Send a Request to the Receiver CSE

The originator shall determine the receiver CSE.

The receiver of the Request shall be the registrar CSE of the originator in case the originator is not IN-CSE.

If the originator is the IN-CSE, the receiver of the Request shall be the CSE whose identifier is the prefix of the “to” attribute of the Request.

If this results in no matching CSE, then the request is rejected with a STATUS_NOT_FOUND.
If results in multiple CSEs, the request is rejected with a status STATUS_INTERNAL_SERVER_ERROR, since this shall never happen.

##### 7.2.2.1.3	Wait for Response primitive

The originator shall wait for the Response primitive from the receiver that corresponds to the Request primitive that was sent by the originator. Correlation between the Request and the corresponding Response is handled by the transport layer.

If no Response primitive is received within a certain time, specified by server policy and/or by the underlying transport technology, this shall be handled as if a Response primitive with a statusCode STATUS_REQUEST_TIMEOUT was received.

#### 7.2.2.2	Hosting CSE actions

##### 7.2.2.2.1	Check existence of the addressed resource

The hosting CSE shall check if the resource addressed by the “to” attribute exists in the repository. If the resource does not exist, the hosting CSE shall reject the request with a "STATUS_NOT_FOUND".

##### 7.2.2.2.2	Check the syntax of received message

Message validity is specified by the protocol mapping TSes (HTTP binding, CoAP binding and MQTT binding). The received resource representation (e.g. in plain XML, binary XML or JSON) shall be validated against the provided schema definitions.

If the message is not valid, the request shall be rejected with a "STATUS_BAD_REQUEST".

##### 7.2.2.2.3	Check validity of resource representation for CREATE

The handling below shall apply t to each attribute in the resource for CREATE request primitives and the handling depends on the "presence in CREATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

If no resource representation is present in the CREATE request, then the request is rejected with a STATUS_BAD_REQUEST statusCode.

The id attribute has special handling. If the id-attribute is present in the CREATE request, the hosting CSE shall check if a resource with the same id already exists in the addressed collection. If such a resource exists and the response column is marked as M, then the hosting CSE shall reject the request with a "STATUS_CONFLICT".

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a STATUS_BAD_REQUEST.

N/A attribute

Indicates that the attribute is not supported in the CREATE request. This common procedure is never referenced from a CREATE request.

M attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M then the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

If the attribute is not present in the resource representation in the CREATE request the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

O attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M or O then the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

NP attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

##### 7.2.2.2.4	Check validity of resource representation for UPDATE

The handling below shall apply to each attribute in the resource for UPDATE request primitives and the handling depends on the "presence in UPDATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a STATUS_BAD_REQUEST.

N/A attribute

Indicates that the attribute is not supported in the UPDATE request. This common procedure is never referenced from a UPDATE request.

M attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

If the attribute is not present in the resource representation in the UPDATE request, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

O attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M or O then the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST" statusCode.

NP attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST" unless the value provided for the attribute exactly matches the value in the current resource representation stored in the hosting CSE. In addition, the lastModifiedTime attribute shall always be accepted (but ignored) by the hosting CSE, no matter what value was provided in the request.

##### 7.2.2.2.5	Create the resource

A new resource shall be created and correlated to the addressed and existing parent resource. This shall modify the resource representation of the addressed parent resource, specifically, if the parent resource has a lastModifiedTime attribute this shall be set to same value as the creationTime attribute of the created resource. The following rules shall be applied:

If the created resource type has an id attribute, then the URI of the created resource shall be the URI of its parent resource with the URI-encoded id appended. (e.g. http://CSEbase.operator.org/myAppID, for an application resource with id "myAppID" created in the parent resource http://CSEbase.operator.org).
If a resource with the same ID already exists in the addressed collection, the hosting CSE shall provide a new id that is unique within the collection.

If expirationTime attribute is present in the resource representation of the to be created resource and the expirationTime is set to a non-negative time, then an expiration timer shall be started by the hosting CSE. At timer expiration the related resource is deleted by "Delete the addressed resource".

For setting the attributes in the resource representation the following rules shall apply in CREATE request primitives:

M attribute

If the provided value is acceptable, the server shall use the provided value in the resource representation of the created resource.

O attribute

If a value is provided and accepted, then the server shall use the provided value in the resource representation of the created resource.

If the attribute is not provided or accepted, but the attribute is marked M in the response, the hosting CSE shall assign default value or assign value based on local policy.

If the attribute is not present in the resource representation in the CREATE request and the response column is marked O then the hosting CSE shall create the resource without the attribute.

NP attribute

If the attribute is not present in the resource representation in the CREATE request, and the response column is marked M, then the hosting CSE shall create the resource with the default value.

##### 7.2.2.2.6	Create a successful Response

The receiver shall create a successful Response primitive with a statusCode indicating "STATUS_OK". The response shall include the representation of the addressed resource, if the hosting CSE modified any of the provided attributes as provided in the Request. The Response shall also include the URI of a created resource.

##### 7.2.2.2.7	Create an unsuccessful Response

The receiver shall create an unsuccessful Response primitive with a statusCode indicating the detected error condition.

##### 7.2.2.2.8	Read the addressed resource

An existing and accessible resource is addressed. The content of its attributes and references to its child resources shall be read in form of resource representation.

When the resource is read to provide a response to RETRIEVE request primitives, then the following rules shall be applied:

M attribute

The attribute is always present in the resource representation.

O attribute

The attribute is present in the resource representation if some conditions occur.

NP attribute

The attribute is never present in the resource representation.

##### 7.2.2.2.9	Update the addressed resource

An existing and accessible resource is addressed. The content of its attributes shall be updated with the following rules in UPDATE request primitives:

M attribute

If the provided value is accepted, the server shall use the provided value in the resource representation of the updated resource.

O attribute

If a value is provided and the value is accepted, the server shall use the provided value in the resource representation of the updated resource.

If the attribute is not provided or accepted, but the attribute is marked M in the response, the hosting CSE shall assign default value or assign value based on local policy.

If the attribute is not present in the resource representation in the UPDATE request and the response column is marked O then the hosting CSE shall remove the attribute in the updated resource.

NP attribute

If the attribute is not present in the resource representation in the UPDATE request and the response column is marked M then the hosting CSE shall not update the attribute value. There is only one exception to this rule and that is the lastModifiedTime attribute. The hosting CSE shall set the lastModifiedTime to the current time whenever an update primitive is received.

If the attribute is present in the resource representation in the UPDATE request the presented value shall be ignored, i.e. the hosting CSE shall never update its resource representation based on the presence of an NP attribute value in an update.

If the expirationTime attribute is present and modified by the procedure and it is set to a non-negative time, then an expiration timer shall be re-started by the hosting CSE. At timer expiration the related resource is deleted by "Delete the addressed resource".

##### 7.2.2.2.10	Delete the addressed resource

An existing and accessible resource is addressed. The resource with all its attributes shall be deleted. Any expiration timer shall be stopped. This same procedure shall then be invoked (recursively) for each child resource of the deleted resource in case the child resource is only linked to the deleted resource.

The parent resource of the addressed resource shall be updated to remove the reference to the deleted resource. If the parent resource has a lastModificationTime attribute then this attribute shall be set to the time of the deletion.

If the resource is announced, the CSE shall try to de-announce the resource correspondingly.

##### 7.2.2.2.11	Send Response primitive

A Response primitive shall be sent back to the originator.

#### 7.2.2.3	Receiver CSE actions

##### 7.2.2.3.1	Re-targeting

If the “to” attribute in the request does not start with the CSEBase URI of the receiver, the receiver CSE shall forward the request or shall serve the request locally (see below).

If the “to” attribute in the request starts with the CSEBase URI of the receiver, then the receiver CSE shall handle the request locally.

Acting as an originator the CSE shall perform the following procedures:

- "Send a Request to the receiver CSE".

- "wait for Response primitive".

When the Response is received the receiver CSE shall:

- Primitive specific procedure: Forward the Response to the original CSE.

### 7.2.3	Resource Type definition

## 7.3	Resource Type-specific procedures and definitions

### 7.3.1	Resource Type CSEBase

### 7.3.2	Resource Type remoteCSE

### 7.3.3	Resource Type application

### 7.3.4	Resource Type container

### 7.3.5	Resource Type instance

### 7.3.6	Resource Type subscription

### 7.3.7	Resource Type schedule

### 7.3.8	Resource Type locationPolicy

### 7.3.9	Resource Type delivery

### 7.3.10	Resource Type request

### 7.3.11	Resource Type group

### 7.3.12	Resource Type Members

### 7.3.13	Resource Type mgmtObj

### 7.3.14	Resource Type parameters

### 7.3.15	Resource Type mgmtCmd

### 7.3.16	Resource Type execInstance

### 7.3.17	Resource Type node

### 7.3.18	Resource Type m2mServiceSubscription

### 7.3.19	Resource Type nodeInfo

### 7.3.20	Resource Type pollingChannel

### 7.3.21	Resource Type cmdhPolicy

### 7.3.22	Resource Type cmdhDefaults

### 7.3.23	Resource Type cmdhDefEcValue

### 7.3.24	Resource Type cmdhEcDefParamValues

### 7.3.25	Resource Type cmdhLimits

### 7.3.26	Resource Type cmdhNetworkAccessRules

### 7.3.27	Resource Type cmdhNwAccessRule

### 7.3.28	Resource Type cmdhBuffer

### 7.3.29	Resource Type statsConfig

### 7.3.30	Resource Type eventConfig

### 7.3.31	Resource Type statsCollect

Annex A (Normative): Resource attributes

Resource Attributes are specified in oneM2M TS-0001 [6]. The type and values shall be supported according to the description given in table A-1.

The attributes are specified by trhe following information in the table:

- ResourceType: indicates the resource where the attribute is used, in case that the attribute is present in all resources the tag “ALL” is used.

- Attribute Name: indicates the name of the Attribute from [6]

- Short Name: indicates the acronym for the correspondent Attribute

- Data type: indicate the used type for the attribute, simple data types are defined in clause 6.3.1 and complex data types are defined in clause 6.3.2.

- Default: specifies the default value of the attribute as set by hosting CSE if no specific value was provided in the operation CREATE or UPDATE request, or if the provided value was unacceptable for the hosting CSE and is not specified to respond with an error then the hosting CSE is allowed to set a value.

- Value restrictions: indicates if the value of the attributes has a specific limitation.

Table A-1: Resource attributes

| ResourceType | Attribute Name | Short Name | Data Type | Default | Value restrictions | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ALL except <accessControlPolicy> | accessControlPolicyID | aRI | xs:string | NONE | See TBD | If the attribute is absent, all the entities that correspond to ancestor resources shall have the full set of permissions |
| firmware | activate |  |  |  |  |  |
| firmware | activateStatus |  |  |  |  |  |
| <subscription> | aggregationURI |  | xs:anyURI |  |  |  |
| TBD | announceAttribute | aA | xs:string | NONE |  |  |
| TBD | announceTo | aT | xs:anyURI | NONE |  |  |
| <application>, <m2mServiceSubscription>, <nodeInfo> | App-ID |  | xs:string |  |  |  |
| <application> | App-Inst-ID |  | xs:string |  |  |  |
| <cmdhEcDefParamValues>, <cmdhNetworkAccessRules>, <cmdhBuffer> | applicableEventCategory | aEC |  |  |  |  |
| <areaNwkDeviceInfo> | areaNwkId |  |  |  |  |  |
| <areaNwInfo> | areaNwkType |  |  |  |  |  |
| <capabilityInstance> | attached |  |  |  |  |  |
| <cmdhNwAccessRule> | backOffParameters | bOP |  |  |  |  |
| <subscription> | batchNotify |  |  |  |  |  |
| <battery> | batteryLevel |  |  |  |  |  |
| <battery> | batteryStatus |  |  |  |  |  |
| <capabilityInstance> | capabilityActionStatus |  |  |  |  |  |
| <capabilityInstance> | capabilityName |  |  |  |  |  |
| <pollingChannel> | channelHandle |  |  |  |  |  |
| <mgmtCmd> | cmdType |  |  |  |  |  |
| <statsCollect> | collectedEntityID |  |  |  |  |  |
| <statsCollect> | collectingEntityID |  |  |  |  |  |
| <statsCollect> | collectPeriod |  |  |  |  |  |
| <group> | consistencyStrategy | cS |  |  |  |  |
| <instance> | content | c | Content | NONE |  |  |
| <request> | content | ? | ? |  |  |  |
| <instance> | contentSize | cS | xs:long |  |  | Set by the CSE to the actual size of the received content of the instance |
| ALL | creationTime | cT | xs:dateTime |  |  | Generated by the hosting SCE. The value is set to the actual time of creation of the resource. |
| TBD | creator | cr | xs:anyURI |  |  | Generated by the hosting |
| <remoteCSE> | cseBase | bRt | xs:anyURI |  |  |  |
| <CSEBase>, <remoteCSE>, <nodeInfo> | CSE-ID |  | xs:string |  |  |  |
| <CSEBase> and <remoteCSE> | cseType | csT | TBD |  |  |  |
| <container> | currentByteSize | nb | xs:long |  | Limited by maxByteSize attribute of the same container resource | Set to the actual number of bytes of data stored in the Container resource |
| <container> | currentNrOfInstances | ni | xs:long |  | Limited by maxNrOfInstances attribute of the same container resource | Set to the actual number of instances resource in the Container |
| <group> | currentNrOfMembers | nM | xs:long |  |  |  |
| <delivery> | data |  |  |  |  |  |
| <eventConfig> | dataSize |  |  |  |  |  |
| firmware | deactivate |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultDelAggregation |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultOpExecTime |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultRequestExpTime | dRqET |  |  |  |  |
| <cmdhEcDefParamValues> | defaultRespPersistence |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultResultExpTime | dRsET |  |  |  |  |
| <cmdhDefEcValue> | defEcValue | dev |  |  |  |  |
| <delivery> | deliveryMetaData |  |  |  |  |  |
| <mgmtObj>, <parameters>, <mgmtCmd>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | description |  | xs:string |  |  |  |
| <deviceInfo> | deviceLabel |  |  |  |  |  |
| <deviceInfo> | deviceType |  |  |  |  |  |
| <areaNwkDeviceInfo> | devId |  |  |  |  |  |
| <areaNwkDeviceInfo> | devType |  |  |  |  |  |
| <capabilityInstance> | disable |  |  |  |  |  |
| <capabilityInstance> | enable |  |  |  |  |  |
| <delivery> | eventCat |  |  |  |  |  |
| <eventConfig> | eventEnd |  | xs:dateTime |  |  |  |
| <eventConfig>, <statsCollect> | eventID |  |  |  |  |  |
| <eventConfig> | eventStart |  | xs:dateTime |  |  |  |
| <eventConfig> | eventType |  |  |  |  |  |
| <execInstance> | execDelay |  |  |  |  |  |
| <execInstance> | execDisable |  |  |  |  |  |
| <mgmtCmd> | execEnable |  |  |  |  |  |
| <execInstance> | execFrequency |  |  |  |  |  |
| <execInstance> | execMode |  |  |  |  |  |
| <execInstance> | execNumber |  |  |  |  |  |
| <mgmtCmd>, <execInstance> | execReqArgs |  |  |  |  |  |
| <execInstance> | execResult |  |  |  |  |  |
| <execInstance> | execStatus |  |  |  |  |  |
| <execInstance> | execTarget |  |  |  |  |  |
| <subscription> | expirationCounter |  |  |  |  |  |
| ALL, except <CSEBase>, <instance> and <parameters> | expirationTime | eT | xs:dateTime |  |  | The value may be determined by CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <reboot> | factoryReset |  |  |  |  |  |
| <subscription> | filterCriteria |  | FilterCriteria |  |  |  |
| <deviceInfo> | fwVersion |  |  |  |  |  |
| <group> | groupName | gN | xs:string | NONE |  |  |
| <node> | hostedCSEID |  | Link |  |  |  |
| <deviceInfo> | hwVersion |  |  |  |  |  |
| firmware | install |  |  |  |  |  |
| firmware | installStatus |  |  |  |  |  |
| <subscription> | interimEventNotify |  |  |  |  |  |
| ALL | labels | lBs | xs:string | NONE |  |  |
| ALL | lastModifiedTime | lMT | xs:dateTime | NONE |  |  |
| <container> | latest | lt | xs:anyURI |  | Limited to the URI of the instances resources in the container | Set by the CSE to the URI of the latest added instance resource to the container |
| <subscription> | latestNotify |  |  |  |  |  |
| <delivery> | lifespan |  |  |  |  |  |
| <cmdhLimits> | limitsDelAggregation | lDA |  |  |  |  |
| <cmdhLimits> | limitsOpExecTime | lOET |  |  |  |  |
| <cmdhLimits> | limitsRequestExpTime | lRqET |  |  |  |  |
| <cmdhLimits> | limitsRespPersistence | lRP |  |  |  |  |
| <cmdhLimits> | limitsResultExpTime | lRsET |  |  |  |  |
| TBD | link | ln | Link | NONE |  |  |
| <areaNwInfo> | listOfDevices |  |  |  |  |  |
| <areaNwkDeviceInfo> | listOfNeighbors |  |  |  |  |  |
| <locationPolicy> | locationContainerID |  | xs:anyURI |  |  |  |
| <locationPolicy> | locationContainerName |  | xs:string |  |  |  |
| <container> | locationID | LID | xs:anyURI | NONE |  |  |
| <locationPolicy> | locationServer |  |  |  |  |  |
| <locationPolicy> | locationSource |  |  |  |  |  |
| <locationPolicy> | locationTargetId |  |  |  |  |  |
| <locationPolicy> | locationUpdatePeriod |  |  |  |  |  |
| <eventLog> | logActionStatus |  |  |  |  |  |
| <eventLog> | logData |  |  |  |  |  |
| <eventLog> | logStart |  |  |  |  |  |
| <eventLog> | logStop |  |  |  |  |  |
| <eventLog> | logTypeId |  |  |  |  |  |
| <deviceInfo> | manufacturer |  |  |  |  |  |
| <cmdhBuffer> | maxBufferSize | mBS |  |  |  |  |
| <container> | maxByteSize | mb | xs:long |  |  | Determined by the CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <container> | maxInstanceAge | ma | xs:duration |  |  | Determined by the CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <container> | maxNrOfInstances | mi | xs:long |  |  | Determined by CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <group> | maxNrOfMembers | nM | xs:long |  |  | Determined by the CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| memory | memAvailable |  |  |  |  |  |
| <group> | memberAccessRightID | mAR | xs:string |  |  |  |
| <group> | memberList |  | AnyURIList |  |  |  |
| <group> | memberType | mT | MemberType | NONE |  |  |
| <group> | memberTypeValidated | mTV | xs:boolean | NONE |  |  |
| memory | memTotal |  |  |  |  |  |
| <request> | metaInformation |  |  |  |  |  |
| <mgmtObj>, <parameters>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | mgmtDefinition |  | xs:string |  |  |  |
| <cmdhNwAccessRule> | minReqVolume | mRV |  |  |  |  |
| <deviceInfo> | model |  |  |  |  |  |
| <application>,<cmdhPolicy>, software, firmware | name |  | xs:string |  |  |  |
| <node>, <nodeInfo> | nodeID |  | Link |  |  |  |
| <CSEBase> and <application> | nodeLink |  | Link |  |  |  |
| <subscription> | notificationDeliveryPriority |  |  |  |  |  |
| <subscription> | notificationEventCat |  |  |  |  |  |
| <subscription> | notificationStoragePriority |  |  |  |  |  |
| <subscription> | notificationStructure |  |  |  |  |  |
| <subscription> | notificationURI |  | xs:anyURI |  |  |  |
| <mgmtObj>, <parameters>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | objectID |  | xs:anyURI |  |  |  |
| <mgmtObj>, <parameters>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | objectPath |  |  |  |  |  |
| <application>, <container>, <instance> | ontologyRef |  | xs:anyURI |  |  |  |
| <request> | operation |  |  |  |  |  |
| <request> | operationResult |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | order | or |  |  |  |  |
| <request> | originator |  |  |  |  |  |
| <cmdhNwAccessRule> | otherConditions |  |  |  |  |  |
| ALL, except <CSEBase> | parentID | pID | xs:anyURI | NONE | See TBD |  |
| <CSEBase>, <remoteCSE> and <application> | pointOfAccess | pOA | AnyURIList |  |  |  |
| <subscription> | priorSubscriptionNotify |  |  |  |  |  |
| <accessControlPolicy> | privileges | ps | Privileges |  |  |  |
| <subscription> | rateLimit |  |  |  |  |  |
| <reboot> | reboot |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestCharacteristics |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestContextNotification |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestContext | rctxt |  |  |  |  |
|  | requestEventCategory | IEC |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestOrigin | roig |  |  |  |  |
| <remoteCSE> | requestReachability |  | xs:boolean |  |  |  |
| <request> | requestStatus |  |  |  |  |  |
| ALL | resourceType | rT | ResourceType | NONE | See TBD |  |
| <schedule> | scheduleElement |  | Schedule |  |  |  |
| <accessControlPolicy> | selfPrivileges | sP | Privileges |  |  |  |
| <areaNwkDeviceInfo> | sleepDuration |  |  |  |  |  |
| <areaNwkDeviceInfo> | sleepInterval |  |  |  |  |  |
| delivery | source |  |  |  |  |  |
| <statsCollect> | statModel |  |  |  |  |  |
| <statsCollect> | statsCollectID |  |  |  |  |  |
| <cmdhPolicy>, <areaNwkDeviceInfo> | status |  | xs:boolean | TRUE = active, FALSE=inactive |  |  |
| <statsCollect> | status | ?? | ?? | Editor’s note: this is a different status of active/inactive |  |  |
| <cmdhBuffer> | storagePriority | sP |  |  |  |  |
| <statsCollect> | subscriberID |  |  |  |  |  |
| <m2mServiceSubscription> | subsGroup |  |  |  |  |  |
| <m2mServiceSubscription> | subsSer&RoleList |  |  |  |  |  |
| <CSEBase> | supportedResourceType | sRT | SupporteResourceType |  |  |  |
| <deviceInfo> | swVersion |  |  |  |  |  |
| <delivery>, <request> | target |  |  |  |  |  |
| <cmdhNwAccessRule> | targetNetwork | tNet |  |  |  |  |
| <eventConfig> | transactionType |  |  |  |  |  |
| <CSEBase>, <remoteCSE> | Trigger-Recipient-ID |  | TBD |  |  |  |
| <instance> | typeOfContent | tOC | ContentType | NONE |  |  |
| software | update |  |  |  |  |  |
| software | updateStatus |  |  |  |  |  |
| software, firmware | URL |  | xs:anyURI |  |  |  |
| software, firmware | version |  |  |  |  |  |
| TBD | versionTag | vT | xs:nonNegativeInteger | 0 |  |  |

Annex B (Normative): Device Triggering

# B.1	Providing Device Triggering service by means of 3GPP networks

## B.1.1	Introduction

3GPP Underlying Network has defined a dedicated interface for requesting device triggering. The normative references for applicable interfaces are as follows: 3GPP TS 23.682 [15]. The specification for the interface Tsp is described in 3GPP TS 29.368 [16]. Tsp interface uses Diameter Base Protocol as specified in IETF RFC 3588 [13], in order to use such an interface the CSE shall act as a Diameter client as described in IETF RFC 6733 [14].

## B.1.2	Device Action Request command

When a CSE needs to issue a device triggering request from MTC-IWF, the CSE shall send a Device-Action-Request (DAR) command (see [16]). The following tables provide an overview of the parameters and the parameter values needed for the DAR command.

Note: the values of the parameters indicated in the “Value” column need to be mapped to the specific 3GPP data type.

Table B.1.2-1: AVPs in Device Action Request (DAR) command

| Attribute Value Pair (AVP) name | AVP description | 3GPP Data Type | AVP Code | Value |
| --- | --- | --- | --- | --- |
| Session-Id | It is used to identify a specific session (see [13]) | UTF8String | 263 | CSE implementation specific |
| Auth-Application-Id | This parameter transport the Tsp application identifier as specified in [16], clause 6.1.3 | Enumerated | 274 | 16777309 |
| Auth-Session-State | The Tsp session is terminated after each request/answer pair and in accordance with [16] the parameter shall indicate that the session state shall not be maintained | Enumerated | 277 | NO_STATE_MAINTAINED (see [16]) |
| Origin-Host | It identifies the endpoint that originated the Diameter message (see [13]) | DiamIdent | 264 | CSE implementation specific |
| Origin-Realm | It identifies the Realm of the originator of theDiameter message (see [13]) | DiamIdent | 296 | CSE implementation specific |
| Destination-Realm | It identifies the realm the message is to be routed to. (see [13]) | DiamIdent | 283 | Domain Identifier part from M2M-Ext-ID |
| Destination-Host | The absence of the Destination-Host causes the message to be sent to any Diameter server supporting the application within the specified realm. (see [13]) | DiamIdent | 293 | NULL |
| Origin-State-Id | It is an increasing value that is advanced whenever a Diameter entity restarts with loss of previous state, for example upon reboot (see [13]) | Unsigned32 | 278 | CSE implementation specific |
| Device-Action | It contains the specific action for the device (see [16]) | Grouped | 3001 | Specific format described in Table B.1.2-2 |
| Proxy-Info | It’s a grouped AVP which provides the proxy state and host (see [13]) | Grouped | 284 | CSE implementation specific |
| Route-Record | The identity added in this AVP is be the same as the one received in the Origin-Host (see [13]) | DiamIdent | 282 | Not applicable |

Table B.1.2-2: AVPs in Device Action AVP (Grouped AVP)

| Attribute Value Pair (AVP) name | AVP description | 3GPP Data Type | AVP Code | Value |
| --- | --- | --- | --- | --- |
| External-Id | It is the identifier of the device as specified in [TS 29.336] | UTF8String | 3111 | M2M-Ext-ID |
| MSISDN | It is the identifier of the device as specified in [TS 29.329] | OctetString | 701 | M2M-Ext-ID |
| SCS-Identifier | It identifies the CSE issuing the request (see [16]) | OctetString | 3104 | CSE-ID |
| Reference-number | It uniquely identifies a transaction (see [16]) | Unsigned32 | 3007 | CSE implementation specific Editor’s note: possibility to maybe use M2M-Request-ID is FFS |
| Action-Type | It informs the MTC-IWF of the required action | Enumerated | 3005 | 1 (Device Trigger Request) |
| Trigger-Data |  | Grouped | 3003 | See Table B.1.2-3 |
| Validity-Time | The validity time in seconds for the specific action requested. (see [16]) | Unsigned32 | 448 | CSE implementation specific |
| NOTE: either external-Id or MSISDN shall be used, not both. |  |  |  |  |

Table B.1.2-3: AVPs in Trigger Data AVP (Grouped AVP)

| Attribute Value Pair (AVP) name | AVP description | 3GPP Data Type | AVP Code | Value |
| --- | --- | --- | --- | --- |
| Payload | It contains the payload to be transferred to the addressed device. (see [16]) | OctetString | 3004 | NULL Furhter values can be defined (FFS) Editor’s Note: in the current TS-0001 does not specify how to use payload. |
| Priority-Indication | It identifies priority of the device trigger. (see [TS 29.368]) | Enumerated | 3006 | CSE implementation specific (possible values are 0 or 1) |
| Application-Port-Identifier | It uniquely identifies the triggering entity addressed in the device. (see [TS 29.368]) | Unsigned32 | 3010 | Trigger-Recipient-ID |

## B.1.3	Device Action Answer command

As a result of device triggering request to MTC-IWF, the CSE receives a Device-Action-Answer (DAA) command (see [16]). The relevant parameters for a CSE are the following ones:

- Result-Code AVP: see [13] for detail;

- Session-Id AVP: see [13] for detail;

- Other AVPs for Diameter Base Protocol: see [13] for detail.

Following AVPs are set in Device-Notification AVP (Grouped AVP):

- Either MSISDN or External-Id: the CSE shall check that the identifier of the target CSE is correct;

- SCS-Identifier: the CSE shall check that the value corresponds to its CSE-ID;

- Reference-Number: the CSE can use this value for correlating the DAA with the corresponding Device-Action-Request command issued previously by the CSE;

- Action-Type: the CSE shall check that the value is se to 1 (Trigger Request) as same value of DAR command;

- Request-Status: see Table B.1.3-1 for the list of possible value.

Table B.1.3-1: Result Status

| Result-Status value | Name | Parameter description |
| --- | --- | --- |
| 0 | SUCCESS | This value indicates that device action requested is confirmed. |
| 201 | TEMPORARYERROR | See [16] for detail. |
| 101 | INVALIDPAYLOAD | See [16] for detail. |
| 102 | INVEXTID | See [16] for detail. |
| 103 | INVSCSID | See [16] for detail. |
| 104 | INVPERIOD | See [16] for detail. |
| 105 | NOTAUTHORIZED | See [16] for detail. |
| 106 | SERVICEUNAVAILABLE | See [16] for detail. |
| 107 | PERMANENTERROR | See [16] for detail. |
| 108 | QUOTAEXCEEDED | See [16] for detail. |
| 109 | RATEEXCEEDED | See [16] for detail. |

## B.1.4	Device Notification Request command

As a report of the result for device triggering delivery by 3GPP network, the CSE receives a Device-Notification-Request (DNR) command (see [16]). The relevant parameters for a CSE are the following ones:

- Result-Code AVP: see [13] for detail;

- Session-Id AVP: see [13] for detail;

- Other AVPs for Diameter Base Protocol: see [13] for detail.

Following AVPs are set in Device-Notification AVP (Grouped AVP):

- Either MSISDN or External-Id: the CSE shall check that the identifier of the target CSE is correct;

- SCS-Identifier: the CSE shall check that the value corresponds to its CSE-ID;

- Reference-Number: the CSE can use this value for correlating the DAA with the corresponding Device-Action-Request command issued previously by the CSE;

- Action-Type: the CSE shall check that the value is set to 2 (Delivery Report);

- Delivery-Outcome: see TableB.1.4-1 for the list of possible value

Table B.1.4-1: Delivery-Outcome

| Delivery-Outcome value | Name | Parameter description |
| --- | --- | --- |
| 0 | SUCCESS | This value indicates that the device action request was successfully completed. |
| 1 | EXPIRED | See [16] for detail. |
| 2 | TEMPORARYERROR | See [16] for detail. |
| 3 | UNDELIVERABLE | See [16] for detail. |
| 4 | UNCONFIRMED | See [16] for detail. |

## B.1.5	Device Notification Answer command

As a result of device notification request to MTC-IWF, the CSE sends a Device-Notification-Answer (DNA) command (see [16]). The relevant parameters for a CSE are the following ones:

- Auth-Application-ID: set to 16777309 as indicated in in [16]);

- Auth-Session-State: set to NO_STATE_MAINTAINED as indicated in [16];

- Other AVPs for Diameter Base Protocol: see [13] for detail, the value of such AVP is CSE implementation specific.

- Annex Z.	Template for oneM2M resource type

Editor’s Note: this is template text for proposal to Clause 8 “oneM2M Resource Types Definitions”.

Editor’s Note: this template will be kept updated, but should be removed before publication.

### 7.3.x	Resource Type <<resource name>>

#### 7.3.x.1	Introduction

The few (2 statemanets are max.) will be copied from Architecture TS.

The detailed description can be found in clause x.x.x in Architecture TS.

Table 7.3.x.1.1 Data Type Definition of <<resource name>>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.x.1.2 Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | one of M/O | one of M/O | one of M/O |
| Retrieve | one of M/O | one of M/O | one of M/O |
| Update | one of M/O | one of M/O | one of M/O |
| Delete | one of M/O | one of M/O | one of M/O |

Table 7.3.x.1.3 Data Types for resource specific attributes

| Attribute Name | Data Type |
| --- | --- |
| <<attribute name1>> | Data Type ex. xs:string |
| <<attribute name 2>> | another Data Type |

Table 7.3.x.1.4 Reference of child resources

| Child Resource Type Name | Data Type ID | Ref. in ArchTS |
| --- | --- | --- |
| <<resource name1>> | Array of <<data type ID2>> |  |
| <<resource name 2>> | <<data type ID2>> |  |

#### 7.3.x.2	Operations

This sub-clause describes <<resource name>> resource specific behaviour for CRUD operations.

#### 7.3.x.2.1	Create

some text

#### 7.3.x.2	Retrieve

#### 7.3.x.3	Update

#### 7.3.x.4	Delete

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.1.0 | 20 Jun 2013 | Initial Version of TS |
| V.0.1.1 | 15 Aug 2013 | Incorporate agreed contributions at TP#5 Ref: 2013-0020R01,2013-0026R02,2013-0028R02 |
| V.0.1.2 | 12 Nov 2013 | Incorporate agreed contrintuin at TP#7.1 Ref: 2013-0067R01 |
| V.0.2.0 | 20 Dec 2013 | Incorporate agreed contribution at TP#8 Ref: 2013-0092R01 |
| V0.2.1 | 19 Feb 2014 | Incorporate agreed contribution at PRO CC 8.x Ref: 2014-0106R01, 2014-0011R01 |
| v0.3.0 | 5 May 2014 | Incorporate agreed contribution at PRO #9 Ref: 2014-0031R01, 2014-0033R02 |
| v0.3.1 | 25 May 2014 | Incorporate agreed contributions at PRO CC 9.2 and 9.3 Ref: 2014-0117R02 |
| v0.3.2 | 28 May 2014 | Incorporate agreed contribution at PRO CC 9.4 Ref: 2014-0118R02 |
| V0.3.3 | 11 Apr 2014 | Incorporate agreed contributions at PRO#10 Ref: PRO-2014-0142R02 (XML Schema) Ref: PRO-2014-0144R01 (Device triggering for 3GPP) Ref: PRO-2014-0145R02 (TS-0004 restructuring) Ref: PRO-2014-0150R01 (Primitives) Ref: PRO-2014-0153R03 (Identifiers) Ref: PRO-2014-0157R01 (Energy efficiency principles) |
| V0.4.0 | 22 Apr 2014 | Incorporate agreed contributions at PRO #10 after 11 Apr 2014: Ref: PRO-2014-172R02(Template for oneM2M Resource) Ref:PRO-2014-151R01(common-operations) |