| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0004-Protocols-V-0.5.1 |
| Document Name: | oneM2M Service Layer Protocol Core Specification |
| Date: | 2014-Jun-17 |
| Abstract: | < An abstract of the document that may be used to help reader decide whether to read the entire document, and provide information that may be used in subsequent electronic searches> |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	8

2	References	8

2.1.	Normative references	8

2.2.	Informative references	9

3	Definitions, symbols, abbreviations and acronyms	9

4	Conventions	10

5	Protocol Design Principles and Requirements	10

5.1.1.	Reference Points	11

5.1.2.	Common Service Functions	11

5.2.	API Design Guidelines	11

5.3.	Primitives	12

5.3.1.	Introduction	12

5.3.2.	Primitives modelling	12

5.3.3.	Primitive principles	13

5.4.	Design Principles	13

5.4.1.	Scalability	13

5.4.2.	Extensibility	13

5.4.3.	Robustness	13

5.4.4.	Efficiency	13

5.4.5.	Security	14

6	oneM2M Protocols / API Overview	14

6.1.	Introduction	14

6.2.	M2M Identifiers	14

6.3.	Common Data Types	14

6.3.1.	Simple Data Types incorporated from XML Schema	15

6.3.2.	Additional Data Types used in oneM2M	18

6.3.2.1.	oneM2M Data Types	18

6.3.2.2.	ExternalID	18

6.3.2.3	Id	18

6.3.3.	Resource common attributes	19

6.4.	Structured Data Types	19

6.4.1.	regularResourceType	20

6.4.1.1.	Description	20

6.4.1.2.	Reference	20

6.4.1.3.	Usage Example	20

6.4.2.	announceableResourceType	21

6.4.2.1.	Description	21

6.4.2.2.	Reference	21

6.4.2.3.	Usage Example	21

6.4.3.	subordinateResourceType	21

6.4.3.1.	Description	21

6.4.3.2.	Reference	21

6.4.3.3.	Usage Example	21

6.5.	oneM2M Device Management  API	21

6.5.1.	Introduction	21

6.5.2.	Management Data Types	22

6.5.3.	Management Common Operations	22

6.5.3.1.	Identify the managed entity and the management protocol	22

6.5.3.2.	Locate the external management objects to be managed on the managed entity	22

6.5.3.3.	Establish a management session with the managed entity	22

6.5.3.4.	Send the management request(s) to the managed entity corresponding to the received Request primitive	22

6.5.4.	<mgmtObj> Resources and Procedures	23

6.5.4.1.	<mgmtObj> Resources	23

6.5.4.1.1.	[cmdhPolicy] Resource	23

6.5.4.1.2.	[activeCmdhPolicy] Resource	24

6.5.4.1.3.	[cmdhDefaults] Resource	24

6.5.4.1.4.	[cmdhDefEcValue] Resource	25

6.5.4.1.5.	[cmdhEcDefParamValues] Resource	26

6.5.4.1.6.	[cmdhLimits] Resource	27

6.5.4.1.7.	[cmdhNetworkAccessRules] Resource	28

6.5.4.1.8.	[cmdhNwAccessRule] Resource	28

6.5.4.1.9.	[cmdhBuffer] Resource	29

6.5.4.2.	<mgmtObj> Primitives and Procedures	30

6.5.5.	<mgmtCmd> Resources and Procedures	30

6.5.5.1.	<mgmtCmd> Resource	30

6.5.5.2.	<mgmtCmd> Primitive and Procedures	30

6.6.	oneM2M Connectivity Management API	30

6.7.	oneM2M Data Management API	30

6.8.	Response Status Codes	30

6.8.1.	Introduction	30

6.8.2.	RSC Data Type	31

6.8.3.	RSC Framework Overview	31

6.8.4.	Current Response Status Codes	33

6.8.4.1.	Successful Responses	33

6.8.4.2.	Un-successful Responses	33

6.8.4.3.	Acknowledgement Responses	36

7	oneM2M Resource Types	37

7.1.	Introduction	37

7.2.	Prerequisites	37

7.2.1.	Primitive format and procedure outlines	37

7.2.1.1.	Primitive format	37

7.2.1.1.1.	Request primitive format	37

7.2.1.1.2.	Response primitive format	37

7.2.1.2.	Description of Generic Procedures	38

7.2.1.2.1.	Generic Resource Create Request Procedure	38

7.2.1.2.2.	Generic Resource Retrieve Request Procedure	40

7.2.1.2.3.	Generic Resource Update Request Procedure	42

7.2.1.2.4.	Generic Resource Delete Request Procedure	43

7.2.1.2.5.	Generic Notify Procedure	44

7.2.1.2.6.	Resource Discovery Request Procedure	44

7.2.2.	Common operations	45

7.2.2.1.	Originator Actions	45

7.2.2.1.1.	Compose Request primitive	45

7.2.2.1.2.	Send a Request to the Receiver CSE	45

7.2.2.1.3.	Wait for Response primitive	45

7.2.2.2.	Hosting CSE actions	45

7.2.2.2.1.	Check existence of the addressed resource	45

7.2.2.2.2.	Check the syntax of received message	46

7.2.2.2.3.	Check validity of resource representation for CREATE	46

7.2.2.2.4.	Check validity of resource representation for UPDATE	46

7.2.2.2.5.	Create the resource	47

7.2.2.2.6.	Create a successful Response	48

7.2.2.2.7.	Create an unsuccessful Response	48

7.2.2.2.8.	Read the addressed resource	48

7.2.2.2.9.	Update the addressed resource	48

7.2.2.2.10.	Delete the addressed resource	49

7.2.2.2.11.	Send Response primitive	49

7.2.2.2.12.	Announce the resource or attribute	49

7.2.2.2.13.	De-announce the resource or attribute	50

7.2.2.3.	Receiver CSE actions	51

7.2.2.3.1.	Re-targeting	51

7.3.	Resource Type-specific procedures and definitions	51

7.3.1.	Resource Type CSEBase	51

7.3.1.1.	Introduction	51

7.3.1.2.	Operations	52

7.3.1.2.1.	Create	52

7.3.1.2.2.	Retrieve	52

7.3.1.2.3.	Update	52

7.3.1.2.4.	Delete	52

7.3.2.	Resource Type remoteCSE	52

7.3.2.1.	Introduction	52

7.3.2.2.1.	Create	53

7.3.2.2.2.	Retrieve	53

7.3.2.2.3.	Update	54

7.3.2.2.4.	Delete	54

7.3.3.	Resource Type application	54

7.3.4.	Resource Type container	54

7.3.5.	Resource Type instance	54

7.3.6.	Resource Type subscription	54

7.3.7.	Resource Type schedule	54

7.3.8.	Resource Type locationPolicy	54

7.3.9.	Resource Type delivery	57

7.3.10.	Resource Type request	57

7.3.10.1.	Introduction	57

7.3.10.2.	Resource Specific Procedure on CRUD Operations	58

7.3.10.2.1.	Create	58

7.3.10.2.2.	Retrieve	59

7.3.10.2.3.	Update	59

7.3.10.2.4.	Delete	59

7.3.11.	Resource Type group	59

7.3.11.1.	Introduction	59

7.3.11.2.	Resource Specific Procedure on CRUD Operations	60

7.3.11.2.1.	Create	60

7.3.11.2.2.	Retrieve	61

7.3.11.2.3.	Update	61

7.3.11.2.4.	Delete	61

7.3.12.	Resource Type Members	61

7.3.13.	Resource Type mgmtObj	61

7.3.13.1.	Introduction	61

7.3.13.2.	Resource Specific Procedure on CRUD Operations	62

7.3.13.2.1.	Create	62

7.3.13.2.2.	Retrieve	62

7.3.13.2.3.	Update	63

7.3.13.2.4.	Delete	63

7.3.14.	Resource Type parameters	63

7.3.15.	Resource Type mgmtCmd	63

7.3.15.1.	Introduction	63

7.3.15.2.	Resource Specific Procedures	64

7.3.15.2.1.	Create	65

7.3.15.2.2.	Retrieve	65

7.3.15.2.3.	Update	65

7.3.15.2.3.1.	Update (Normal)	65

7.3.15.2.3.2.	Update (Execute)	65

7.3.15.2.4.	Delete	66

7.3.16.	Resource Type execInstance	66

7.3.16.1.	Introduction	66

7.3.16.2.	Resource Specific Procedures	67

7.3.16.2.1.	Update (Cancel)	67

7.3.16.2.2.	Retrieve	68

7.3.16.2.3.	Delete	68

7.3.17.	Resource Type node	68

7.3.17.1.	Introduction	68

7.3.17.2.	Resource Specific Procedure on CRUD Operations	69

7.3.17.2.1.	Create	69

7.3.17.2.2.	Retrieve	69

7.3.17.2.3.	Update	69

7.3.17.2.4.	Delete	69

7.3.18.	Resource Type m2mServiceSubscription	69

7.3.19.	Resource Type nodeInfo	69

7.3.20.	Resource Type pollingChannel	69

7.3.20.1.	Introduction	70

7.3.20.2.	Operations	70

7.3.20.2.1.	Create	70

7.3.20.2.2.	Retrieve	70

7.3.20.2.3.	Update	70

7.3.20.2.4.	Delete	71

7.3.21.	Resource Type pollingChannelURI	71

7.3.21.1.	Introduction	71

7.3.21.2.	Operations	71

7.3.21.2.1.	Create	71

7.3.21.2.2.	Retrieve	71

7.3.21.2.3.	Update	72

7.3.21.2.4.	Delete	72

7.3.22.	Resource Type cmdhPolicy	72

7.3.23.	Resource Type cmdhDefaults	72

7.3.24.	Resource Type cmdhDefEcValue	72

7.3.25.	Resource Type cmdhEcDefParamValues	72

7.3.26.	Resource Type cmdhLimits	72

7.3.27.	Resource Type cmdhNetworkAccessRules	72

7.3.28.	Resource Type cmdhNwAccessRule	73

7.3.29.	Resource Type cmdhBuffer	73

7.3.30.	Resource Type statsConfig	73

7.3.30.1.	Introduction	73

7.3.30.2.	<statsConfig> resource-specific procedure on CRUD operations	73

7.3.30.2.1.	Create	73

7.3.30.2.2.	Retrieve	74

7.3.30.2.3.	Update	74

7.3.30.2.4.	Delete	74

7.3.31.	Resource Type eventConfig	74

7.3.31.1.	Introduction	74

7.3.31.2.	<eventConfig> resource-specific procedure on CRUD operations	75

7.3.31.2.1.	Create	75

7.3.31.2.2.	Retrieve	75

7.3.31.2.3.	Update	75

7.3.32.	Resource Type statsCollect	75

7.3.32.1.	Introduction	75

7.3.32.2.	<statsCollect> resource-specific procedure on CRUD operations	76

7.3.32.2.1.	Create	76

7.3.32.2.2.	Retrieve	76

7.3.32.2.3.	Update	76

7.3.32.2.4.	Delete	77

Proforma copyright release text block	78

Annexes	78

Annex A (Normative): Resource attributes	78

Annex B (Normative): Device Triggering	88

B.1	Providing Device Triggering service by means of 3GPP networks	88

B.1.1	Introduction	88

B.1.2	Device Action Request command	88

B.1.3	Device Action Answer command	90

B.1.4	Device Notification Request command	91

B.1.5	Device Notification Answer command	92

Annex C	(Informative): XML Examples	92

C.1	XML Schema for container resource type	92

C.2	Container resource that conforms to the Schema given above (see C.1)	93

Annex Z (Infomative): Template for one M2M resource type	94

7.3.x	Resource Type <<resource name>>	94

7.3.x.1	Introduction	94

7.3.x.2	Resource Specific Procedure on CRUD Operations	95

7.3.x.2.1 	Create	95

7.3.x.2.2 	Retrieve	95

7.3.x.2.3 	Update	95

7.3.x.2.4 	Delete	95

List of Tables and Figures	96

History	98

# Scope

The present document specifies the communication protocol(s)  for oneM2M compliant Systems,  M2M Applications, and/or other M2MSystems.

This document also specifies the  common data formats, interfaces and message sequences to support reference points(s) defined by oneM2M.

# References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## Normative references

The following referenced documents are necessary for the application of the present document.

[1]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[2]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[3]	W3C XMLSchemaP2: "W3C Recommendation (2004), XML Schema Part 2:Datatypes Second Edition.".

[4]	ISO 19136 (2007): "Geographic information -- Geography Markup Language (GML).".

[5]	W3C XMLPath: "W3C Recommendation (2010), XML Path Language (XPath) 2.0 (Second Edition).".

[6]	oneM2M TS-0001 “Functional Architecture”. TBD.

[7]

[8]	IEEE 754-2008: IEEE. IEEE Standard for Floating-Point Arithmetic. 29 August 2008. http://ieeexplore.ieee.org/servlet/opac?punumber=4610933

[9]	IETF RFC 3548: “The Base16, Base32, and Base64 Data Encodings”. 2003.

[10]	IETF RFC 2045: “Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies”. 1996.

[11]	IETF RFC 3987:“ Internationalized Resource Identifiers (IRIs)” . January 2005.

[12]	IETF BCP 47: “Best Current Practices 47”. Concatenation of RFC 4646:“ Tags for Identifying Languages”(2006) and RFC 4647: “Matching of Language Tags”( 2006).

[13]	IETF RFC 3588: “Diameter Base Protocol”. September 2003.

[14]	IETF RFC 6733: “Diameter Base Protocol”. October 2012.

[15]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”.

[16]	3GPP TS 29.368: “Tsp interface protocol between the MTC Interworking Function (MTC-IWF) and Service Capability Server (SCS)”.

## Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	Fielding, Roy Thomas (2000): "Architectural Styles and the Design of Network-based Software Architectures", Doctoral dissertation, University of California, Irvine.

[i.3]	OMA-TS-REST-NetAPI_TerminalLocation-V1_0-20130924-A: “RESTful Network API for Terminal Location”, Version 1.0

[i.4]	“RESTful Network API for Notification Channel”, Open Mobile Alliance™, OMA-TS-REST_NetAPI_NotificationChannel-V1_0

[i.5]		OMA-TS-MLP-V3_4-20130226-C: “Mobile Location Protocol”, Version 3.4

# Definitions, symbols, abbreviations and acronyms

Delete from the above heading the word(s) which is/are not applicable.

  - Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

  - Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

  - Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

  - Acronyms

Acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Acronym format

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# Protocol Design Principles and Requirements

The following sub-clauses contain the design principles and requirements for the oneM2M protocol.

Editor’s Note: The following sub-clauses are intended to provide design principle and specify aspects of protocol requirements, including but not limited to scalability, performance, common message format, reliability, security, extensibility, robustness, resilience, efficiency, message minimisation, etc. Requirements derived from the Requirements TS and inferred from the evolving Architecture TS should be captured here and expressed in terms of Protocol Requirements. Beyond these, additional Protocol requirements are expected. More contributions are requested

  - Introduction

The oneM2M architecture is resource-based, (see oneM2M TS-0001 [6]). The functionality of the system is exposed by means of APIs over the reference points specified in [6]. Operations upon resources hosted by a CSE are carried over an established channel that constitutes the communication on the reference points Mca and Mcc.

Each resource operation comprises a pair of primitives: Request and Response.

In order to provide a well-defined interface for the reference points in the architecture [6], the following aspects need to be provided:

- The collection of primitives carried over a specific reference point, and

- The description and applicability of security methods in relation to the underlying protocols and reference points involved.

The current document provides:

- Data type definitions,

- Primitive definitions, and

- XML definitions and schema.

NOTE: The actual binding of the interface to a specific protocol is not part of this document, but is specified in a separate TS.

In accordance with the oneM2M architecture, each reference point is applicable to a wide range of underlying network technologies and transport protocols. oneM2M will only define a set of bindings for specific underlying network technologies and transport protocols, these bindings are not limiting the applicability of the interfaces when used in other underlying networks and transport protocols. However the behaviour of the interface needs to be respected in accordance to this document and the architecture (see [6]).

### Reference Points

### Common Service Functions

## API Design Guidelines

The following are the guidelines for designing APIs:

- API shall follow the principle of RESTful architecture, as described in [i.2]

- APIs shall define how to address resources and how to manipulate resources, in accordance with oneM2M TS 0001 [6]; the resource is identified by a Universal Resource Identifier (URI), [2].

- Resource has a representation (see [i.2]) that shall be transferred and manipulated with the verbs. These verbs are identified as operations in [6]: CREATE. RETRIEVE, UPDATE, DELETE and NOTIFY

- All primitives shall be defined as well as the way that those primitives are sent. The functionality of the primitives shall be compliant to the resource type specific procedure as specified in [6], clause 10.2

- API shall provide the format and syntax of the operation primitives for all resources defined in [6]. In case that for a particular protocol binding an operation cannot be supported it has to be clearly stated in the specific protocol binding technical specification.

- Primitives shall include attributes in accordance with [6] for a specific resource

- Primitive shall be self-descriptive and contain all the information needed for the receiver of the primitives to handle the primitives.

- Primitive should be idempotent operations which means no matter how many times the primitive is sent, the result doesn’t change, in accordance to [i.2]. .

- API shall indicate which features are supported and not supported over the reference points specified in [6].

- Primitives shall be mapped on the transport layer protocols.

## Primitives

### Introduction

Primitives are service layer messages transmitted over the Mca and Mcc reference points. The Create, Update, Retrieve, Delete and Notify operation is mapped to one or more primitives. The primitive is then further mapped to transport layer protocols such as HTTP, CoAP or MQTT for the transmission. The primitive is independent from the transport protocols.

Figure 5.3.1-1: Primitive overview

A single primitive in the application/service layer may be mapped to zero or more transport messages in the transport layer.

The Originators shall send requests to Receivers through primitives. The Originator and Receiver may be AE or CSE. The CRUD request primitive addresses a resource residing in a CSE. The Notify request primitive may address an AE or CSE.

Each CRUD+N operation shall consist of one request and one response primitive.

Communication between an originator and receiver in the Application/Service layer shall be performed with the above primitives.

Communication between an originator and receiver in the Transport layer shall be performed using underlying network.

### Primitives modelling

Primitives shall be modelled as follows.

A primitive shall be a data structure that describes with appropriate attributes a specific procedure request or answer in both originator and receiver entities.

A primitive shall consist of:

- Control part: contains attributes required for the processing of the request or response, and

- Optional content part: user data. This is the representation of the resource or the value of the attribute in partial addressing case.

Figure 5.3.2-2: Primitives modeling

### Primitive principles

Primitives shall be atomic. When two concurrent request primitives affect the same resource, the execution of one primitive shall finish completely before the second one starts.

When creating or updating the resource, the representation of the resource shall be contained in the Content part of the primitive. Based on the representation of the resource, the Hosting CSE of the addressed resource can create or update the entire resource without the need for further information.

The handling of primitives shall be idempotent. This means no matter how many times the same primitive is targeted to the same resource, the resource does not change after the first handling of the primitive.

## Design Principles

### Scalability

When considering scalability as a requirement in the design of oneM2M protocols, one or several of the following mechanisms may be used:

- Wherever possible, ensure direct addressability to the CSEs hosting target resources, to minimize network hops

- Asynchrony in terms of data processing, with the ultimate objective of minimizing the number of discarded packets.

- Caching mechanisms that allow all the received packets to be processed.

- Efficient load distribution to avoid bottlenecks and data loss.

- Data compression and/or aggregation, in order to reduce the amount of data sent through the network.

### Extensibility

### Robustness

### Efficiency

Energy Efficiency:

- As energy consumption directly affects the overall system performance, oneM2M protocols should consider energy efficiency, especially in resource constrained environments with battery-powered oneM2M devices.

- Energy efficient oneM2M protocols shall aim at reducing the overall energy consumption while maintaining the performance required by the oneM2M Applications.

### Security

# oneM2M Protocols / API Overview

## Introduction

This technical specification describes message formats and procedures to communicate with oneM2M compliant M2M Platform System.

This specification describes:

- Data representation for communication protocol messages

- Normal and exceptional procedure

- Status codes

- Guidelines for drafting APIs

## M2M Identifiers

This clause describes all identifiers from [6] and the data types used to represent them.

Table 6.2-1: M2M Identifiers

| Identifier | Data Type | Description |
| --- | --- | --- |
| M2M-SP-ID | m2m:id (see clause 6.3.2.1) | A globally unique ID as specified in [6] |
| App-ID | m2m:id (see clause 6.3.2.1) | The identifier is specified in [6] |
| AE-ID | m2m:id (see clause 6.3.2.1) | A globally unique ID as specified in [6] |
| CSE-ID | m2m:id (see clause 6.3.2.1) | A globally unique ID as specified in [6] |
| M2M-Node-ID | TBD (see clause 6.3.2.1) | A globally unique ID as specified in [6] |
| M2M-Sub-ID | m2m:id (see clause 6.3.2.1) | A globally unique ID as specified in [6]. |
| M2M-Request-ID | TBD (see clause X.X) | A unique ID as specified in [6] |
| M2M-Ext-ID | ExternalID (see clause 6.3.2.2) | The identifier is specified in [6] |
| UNetwork-ID | m2m:id (see clause 6.3.2.1) | A unique ID as specified in [6] |
| Trigger-Recipient-ID | TBD (see clause X.X) | The identifier is specified in [6] |
| M2M-Serv-ID | TBD | The identifier is specified in [6] |

## Common Data Types

For wide acceptance by industrial markets, this specification describes structured and non-structured data for oneM2M Protocol using XML Schema Language (aka XSD)[3].

Note that the actual data format is depends on chosen Protocol Bindings. Each protocol binding specification will specify the mapping rule between XSD version of data definitions and its native data format.

Any Data Types and XML elements defined for use in oneM2M protocols shall use the namespace

http://www.onem2m.org/xml/protocols.

This document, and any XML or XML Schema Documents produced by oneM2M shall use the prefix m2m: to refer to that namespace.

### Simple Data Types incorporated from XML Schema

Following ‘built-in data types’ are incorporated from XML Schema definition [3].

Note that name space identifier for ‘http://www.w3.org/2001/XMLSchema' shall be described as ‘xs:’ in this specification.

Table 6.3.2.1-2: Simple Data Types incorporated from XML Schema

| Data Type | Description | Notes |
| --- | --- | --- |
| xs:string | The string datatype represents character strings in XML |  |
| xs:boolean | boolean represents the values of two-valued logic. |  |
| xs:decimal | decimal represents a subset of the real numbers, which can be represented by decimal numerals. The ·value space· of decimal is the set of numbers that can be obtained by dividing an integer by a non-negative power of ten, i.e., expressible as i / 10n where i and n are integers and n ≥ 0. Precision is not reflected in this value space; the number 2.0 is not distinct from the number 2.00. The order relation on decimal is the order relation on real numbers, restricted to this subset. |  |
| xs:float | The float datatype is patterned after the IEEE single-precision 32-bit floating point datatype IEEE 754-2008 ([8]). Its value space is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:double | The double datatype is patterned after the IEEE double-precision 64-bit floating point datatype IEEE 754-2008 ([8]). Each floating point datatype has a value space that is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:duration | duration is a datatype that represents durations of time. |  |
| xs:dateTime | dateTime represents instants of time, optionally marked with a particular time zone offset. Values representing the same instant but having different time zone offsets are equal but not identical. |  |
| xs:time | time represents instants of time that recur at the same point in each calendar day, or that occur in some arbitrary calendar day. |  |
| xs:date | date represents top-open intervals of exactly one day in length on the timelines of dateTime, beginning on the beginning moment of each day, up to but not including the beginning moment of the next day). For non-timezoned values, the top-open intervals disjointly cover the non-timezoned timeline, one per day. For timezoned values, the intervals begin at every minute and therefore overlap. |  |
| xs:hexBinary | hexBinary represents arbitrary hex-encoded binary data. |  |
| xs:base64Binary | base64Binary represents arbitrary Base64-encoded binary data. For base64Binary data the entire binary stream is encoded using the Base64 Encoding defined in RFC 3548([9]), which is derived from the encoding described in RFC 2045([10]). |  |
| xs:anyURI | anyURI represents an Internationalized Resource Identifier Reference (IRI). An anyURI value can be absolute or relative, and may have an optional fragment identifier (i.e., it may be an IRI Reference). This type should be used when the value fulfills the role of an IRI, as defined in RFC 3987([11]) or its successor(s) in the IETF Standards Track. |  |
| xs:normalizedString | normalizedString represents white space normalized strings. The ·value space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The ·lexical space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The ·base type· of normalizedString is string. |  |
| xs:token | token represents tokenized strings. The ·value space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The ·lexical space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The ·base type· of token is normalizedString. |  |
| xs:language | language represents formal natural language identifiers, as defined by BCP 47([12]). |  |
| xs:integer | integer is ·derived· from decimal by fixing the value of ·fractionDigits· to be 0 and disallowing the trailing decimal point. This results in the standard mathematical concept of the integer numbers. The ·value space· of integer is the infinite set {...,-2,-1,0,1,2,...}. The ·base type· of integer is decimal. |  |
| xs:nonNegativeInteger | nonNegativeInteger has a lexical representation consisting of an optional sign followed by a non-empty finite-length sequence of decimal digits (#x30-#x39). If the sign is omitted, the positive sign ('+') is assumed. If the sign is present, it must be "+" except for lexical forms denoting zero, which may be preceded by a positive ('+') or a negative ('-') sign. For example: 1, 0, 12678967543233, +100000. |  |
| xs:positiveInteger | positiveInteger is ·derived· from nonNegativeInteger by setting the value of ·minInclusive· to be 1. This results in the standard mathematical concept of the positive integer numbers. The ·value space· of positiveInteger is the infinite set {1,2,...}. The ·base type· of positiveInteger is nonNegativeInteger. |  |
| xs:unsignedLong | unsignedLong is derived· from nonNegativeInteger by setting the value of ·maxInclusive· to be 18446744073709551615. The ·base type· of unsignedLong is nonNegativeInteger. |  |
| xs:unsignedInt | unsignedInt is ·derived· from unsignedLong by setting the value of ·maxInclusive· to be 4294967295. The ·base type· of unsignedInt is unsignedLong. |  |
| xs:unsignedShort | unsignedShort is ·derived· from unsignedInt by setting the value of ·maxInclusive· to be 65535. The ·base type· of unsignedShort is unsignedInt. |  |
| xs:dateTimeStamp | The dateTimeStamp datatype is derived· from dateTime by giving the value required to its explicitTimezone facet. The result is that all values of dateTimeStamp are required to have explicit time zone offsets and the datatype is totally ordered. |  |

Editor's Note:  How to choose the local time zone at timestamp is FFS.

### Additional Data Types used in oneM2M

This clause defines the complex data types that are used by oneM2M API.

#### oneM2M Data Types

Table 6.3.2.1-3:describes oneM2M-specific Data Type definitions.

Table 6.3.2.1-3: oneM2M Simple Data Types

| Data Type | Type Name | Description |
| --- | --- | --- |
| m2m:id | Generic ID | Used to represent generic IDs generated and used within oneM2M |
| m2m:nodeId | Node ID | Used for Node IDs. The constraints on this type are different from those on Generic IDs |
| m2m:requestId | Request ID | Used for Request IDs. This type includes the ID of the target CSE as well as a part that varies for each ID |
| m2m:externalId | External ID | Used for External IDs. This type includes the ID of the underlying network provider as well as a part that varies for each ID |
| m2m:acpType | ACP Type | Used to represent an AccessControlPolicy identifier. This can be either a URI or an opaque token |
| m2m:nhURI | Non-hierarchical URI | This is used to indicate that the URI cannot be interpreted in an hierarchical manner |
| m2m:resourceCode | Resource Code | A 2 capital letter code as defined in [7]. It is defined as an xs:string enumeration. |

#### ExternalID

This type represents the M2M-Ext-ID.

Table 6.3.2.2-4: oneM2M External IDs

| Name | Type | Description |
| --- | --- | --- |
| UNetwork-ID | m2m:id | The identifier of the underlying network provider for which the external Identifier can be used. |
| Id | AnyURI | The identifier of the node for the underlying network provider |

#### 6.3.2.3	Id

Editor’s Note: TBD.

### Resource common attributes

Resource common attributes are specified in oneM2M TS-0001 [6]. The type and values shall be supported according to the description given in Table 6.3.3-5.

Table 6.3.3-5: Resource Common Attributes

| Attribute Name | Data Type | Default | Value restrictions | Notes |
| --- | --- | --- | --- | --- |
| accessControlPolicyID | m2m:acpType | Policy is inherited from the parent resource |  |  |
| parentID | m2m:nhURI | Not applicable | Must be a non-hierarchical URI |  |
| creationTime | xs:dateTime | Not applicable |  |  |
| expirationTime | xs:dateTime | Absence of the attribute means that the resource has no explicit expirationTime |  |  |
| lastModifiedTime | xs:dateTime | Not applicable |  |  |
| resourceType | m2m:resourceCode | In the absence of this attribute, the resource type can be inferred from the root element |  |  |
| labels | list of xs:token | Absence of this attribute means there are no labels |  |  |
| stateTag | xs:nonNegativeInteger | Not applicable |  |  |
| link | xs:anyURI | Not applicable |  |  |
| announceTo | list of xs:anyURI | Not applicable |  |  |
| announcedAttribute | list of xs:token | Not applicable |  |  |

## Structured Data Types

This specification defines common set of structured data as ‘Common Data Types’.

oneM2M Resource Data Types are defined in XSD, and they are supplied as separate documents. The source of XSD file can be Resource definition in accordance with oneM2M TS-0001 [6].

Although XSD files are managed independent from [6], they shall conform to the following rules:

- Each Resource Type will be defined in a separate XSD file. The name of that file should be prefixed with ‘CDT-‘ and  followed by the resource type name, version of the Core Protocol TS, and date of update. Each Resource Type file will contain a single Global Element Declaration whose name is the name of the Resource Type in accordance with [6]. This means that the root element of a Resource (when represented in XML) contains an m2m: (or equivalent) namespace prefix.

- The root element of each resource shall have an attribute called “name” which gives an identifier for that particular resource instance. A URI to the resource instance can be constructed by taking the URI of its parent and appending /<name> where <name> is the value of the name attribute.

- Each resource  attribute is represented as a child element of the top level element, and shall not have any child elements. It shall be declared as an element that is local to the resource that contains it, and so does not have a namespace prefix in any XML representation of the resource.

- Each child resource shall be represented as a child element of the top level element which named as ‘childResource’ which shall be used to describe a non-hierarchical URI for the associated child resource. This element shall have two attributes( in XSD) : a) type; Data type ID of instances, b) name; the name of a child resource instance.

- Each attribute should be declared to use either one of the simple data types listed in clause 6.3.1 or 6.3.2, or a data type derived by restriction from one of the types listed in clause 6.3.1 or 6.3.2.

- Common ‘Simple types’ are collected as dedicated XSD file, which is named 
‘CDT-simple_types-<version>-<date>.xsd’

- With the exception of CSEBase, all Resource types will extend one of the XML complex types defined in the file CDT-commonTypes-v1_0<date>.xsd.

- The resource-specific attributes and child resources shall appear as an XSD sequence, with their order being detemined by the order shown  in the tables in clause 9.6 of [6].

Table 6.4-6: oneM2M Structured Data Types

| Data Type ID | File Name | Where defined | Notes |
| --- | --- | --- | --- |
| Simple Types | CDT-simpleType-v1_0-<date>.xsd | 6.3.1 |  |
| CommonTypes | CDT-commonTypes-v1_0_0-<date>.xsd |  |  |

### regularResourceType

#### Description

This type definition includes the common attributes used by the majority of M2M resources. It is included  in CDT-commonTypes-v1_0_0-<date>.xsd .

#### Reference

See Clause 9.6.1 of [6].

#### Usage Example

<?xml version="1.0" encoding="UTF-8"?>
<xs:schema xmlns="http://www.w3.org/2001/XMLSchema"	targetNamespace=http://www.onem2m.org/xml/protocols
   xmlns:m2m=http://www.onem2m.org/xml/protocols
	elementFormDefault="unqualified" xmlns:xs="http://www.w3.org/2001/XMLSchema">
	<xs:include schemaLocation="common_types-v1_0_0-20140409.xsd" />
	<xs:include schemaLocation="CDT-instance-v1_0_0-20140409.xsd" />
	<xs:include schemaLocation="CDT-subscription-v1_0_0-20140409.xsd" />
	<xs:element name="container">
		<xs:complexType>
			<xs:complexContent>
           <!-- 	Inherit Common Attributes from regularResourceType     -->
				<xs:extension base="m2m:regularResourceType">		    
					<xs:sequence>
					    <!-- 			Resource Specific Attributes           -->
						<xs:element name="maxNrOfInstances" type="xs:nonNegativeInteger" minOccurs="0" />
						<xs:element name="maxByteSize" type="xs:nonNegativeInteger" minOccurs="0" />
						<xs:element name="maxInstanceAge" type="xs:nonNegativeInteger" 	minOccurs="0" />
						<xs:element name="currentNrOfInstances" type="xs:nonNegativeInteger" />
						<xs:element name="currentByteSize" type="xs:nonNegativeInteger" />
						<xs:element name="latest" type="xs:anyURI"  minOccurs="0" />
						<xs:element name="locationID" type="xs:anyURI" minOccurs="0" />
						<xs:element name="ontologyRef" type="xs:anyURI"  minOccurs="0" />	
						<!-- 			Child Resources                          -->
						<!--  Instances are included inline in the parent        -->			
						<xs:element ref="m2m:instance" minOccurs = "0" maxOccurs="unbounded"/>
						<!--  Subscriptions can either be included inline or referenced by URI -->
						<xs:choice>
						  <xs:element ref="m2m:subscription" minOccurs="0" maxOccurs="unbounded"/>
						  <xs:element name="subscription" type="xs:anyURI"  minOccurs="0" 
										  maxOccurs="unbounded"/>
						</xs:choice>
						<!--  Containers are always referenced by URI             -->		
					    <xs:element name="container" type="xs:anyURI"  minOccurs="0" 
											maxOccurs="unbounded"/>	
					</xs:sequence>
				</xs:extension>
			</xs:complexContent>
		</xs:complexType>
	</xs:element>
</xs:schema>

### announceableResourceType

#### Description

This type definition includes the common attributes used by M2M resource types that are capable of being announced.  In addition to the attributes of a regularResource, It includes (as optional)  the common attributes that are used by the announcement mechanism.

#### Reference

See Clause 9.6.1 of [6].

#### Usage Example

### subordinateResourceType

#### Description

This type definition includes the common attributes used by M2M resource types that are subordinate children over other resource types (for example the Instance child of a container). It excludes attributes like expirationTime, which aren’t defined for such resources.

#### Reference

See Clause 9.6.1 of [6].

#### Usage Example

Editor’s Note: This is example of description for each Common Data Type, and remove this  sub clause when actual entries are added.

## oneM2M Device Management  API

Editor’s Note: This sub-clause should be considered to move as part of “oneM2M Resource Types”.

<Text>

The following text is to be used when appropriate:

### Introduction

This section describes the management specific primitives and procedures to communicate across the Mca and Mcc reference points.

### Management Data Types

This section describes the data types that are extensions of the data types defined in clause 6.2.1  and are specific to the management resource and subresources.

### Management Common Operations

#### Identify the managed entity and the management protocol

The Hosting CSE shall identify the managed entity to be managed via the <node> resource which is the parent resource of the addressed <mgmtObj> resource. Through the <node> resource, the identifier of the device can be retrieved. Then the Hosting CSE shall determine the management protocol to be used for communicating with the managed entity based on the objectID of the addressed <mgmtObj> resource. If the managed entity cannot be identified, the Hosting CSE shall reject the request with the statusCode set to "STATUS_NOT_FOUND" in the Response primitive.

Editor’s Note: The other statusCode when the managed entity cannot be identified is FFS.

#### Locate the external management objects to be managed on the managed entity

The Hosting CSE shall locate the external management object information to be managed on the managed entity by the objectPath attribute of the <mgmtObj> resource addressed by the URI provided in the to primitive attribute. In the case that the to addresses an [objectAttribute] without objectPath attribute, the Hosting CSE shall locate the external management object information on the managed entity through the objectPath attribute of the <mgmtObj> resource of the addressed [objectAttribute], combining with their relative position in the external management object tree. If the external management object information cannot be located, the Hosting CSE shall reject the request with the statusCode set to "STATUS_NOT_FOUND" in the Response primitive.

#### Establish a management session with the managed entity

In the case that the management server is embedded with the CSE, if there is no existing management session between the Hosting CSE and the managed entity, the Hosting CSE shall also trigger the managed entity to establish a management session with the Hosting CSE by sending triggering message to the managed entity using the determined management protocol in case such triggering mechanism is supported by the external management technology. If the triggering mechanism is not supported by the external management technology, the Hosting CSE shall reject the request with the statusCode “STATUS_NOT_SUPPORTED”.  If the management session cannot be established within a limited time span as per local policy, the Hosting CSE shall reject the request with the statusCode set to "STATUS_GATEWAY_TIMEOUT" in the Response primitive.

Editor’s Note: The procedure to establish management session when management server is separate from the CSE is FFS.

#### Send the management request(s) to the managed entity corresponding to the received Request primitive

The Hosting CSE shall send the management request(s) to the managed entity in the established management session in order to perform the management operation as requested by the received Request primitive. The management request shall address the external management object information on the managed entity as determined in clause 6.5.3.1 or in the primitive specific clauses. The management request being used is specific to the external management technology according to a pre-defined mapping relationship with the Request primitive. The internal data structure of the external management object addressed by the management request shall be determined based on the mapping relationship of the <mgmtObj> resource and the external management objects or based on the generic mapping rule as specified in TS0001 clause 9.6.15, for any other <mgmtObj> resource instances. The Hosting CSE shall extract the management results received from the managed entity in order to prepare a Response primitive to be sent to the originator later. Unless explicitly stated, if the management request cannot be performed successfully, the Hosting CSE shall reject the Request primitive with the proper statusCode in the Response primitive according to the mapping relationship withthe external management technology.

Editor’s Note: The common operation for handling notification of external management technology from managed entity is FFS.

### <mgmtObj> Resources and Procedures

This section describes the primitives and data types necessary to communicate the <mgmtObj> resource and subresources across the Mca and Mcc reference points.

#### <mgmtObj> Resources

This section describes the definition of the <mgmtObj> resource and subresources. In addition, the policy (e.g., mandatory, optional) related to the presence of a resource’s attributes with respect to the primitives is documented in this section.

        - [cmdhPolicy] Resource

The Resource [cmdhPolicy] represents a set of rules defining which CMDH parameters will be used by default when a request issued by a local originator contains the ec (event category) parameter but not all other CMDH parameters.

The detailed description can be found in clause D.12 of Architecture TS-0001 [1].

Table 6.5.4.1.2-7: Data Type Definition of [cmdhPolicy]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhPolicy-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.2-8: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.2-9: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhPolicy” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| Name | xs:string |  |  |
| cmdhDefaults | m2m:mgmtLink |  |  |
| cmdhLimits | m2m:mgmtLink |  |  |
| cmdhNetworkAccessRules | m2m:mgmtLink |  |  |
| cmdhBuffer | m2m:mgmtLink |  |  |

Editor’s note: How to represent multiplicity need to clarify.

Editor’s Note: Optionality of Attributes should be referred in Arch-TS or self-contained ?

Editor’s Note: defining default value sound in scope of stage 3, but may be subject to remove in future.

Editor’s Note: Possible restrictions on length and ranges of the data types as well as representation of these lengths and ranges are FFS. This applies to the above table and the tables following in clauses 6.5.3.1.b to 6.5.3.1.i.

The resource [cmdhPolicy] does not have any child resources other than those referenced by m2m:mgmtLink type of attribute.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [activeCmdhPolicy] Resource

The resource [activeCmdhPolicy] provides a link to the currently active set of CMDH policies.

The detailed description can be found in clause D.12.1 of Architecture TS-0001 [1].

Table 6.5.4.1.2-10: Data Type Definition of [activeCmdhPolicy]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-activeCmdhPolicy-v1_0_0-<<date of published>>.xsd | At most one Policy instance shall be enabled at a time. |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.2-11: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.2-12: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “ActiveCmdhPolicy” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| cmdhPolicy | m2m:mgmtLink |  |  |

The resource [activeCmdhPolicy] does not have any child resources other than the one referenced by m2m:mgmtLink type of attribute.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhDefaults] Resource

The resource [cmdhDefaults] defines default CMDH policy values. The detailed description can be found in clause D.12.2 of Architecture TS-0001 [1].

Table 6.5.4.1.3-13: Data Type Definition of [cmdhDefaults]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhDefaults-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.3-14: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.3-15: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhDefaults” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| cmdhDefEcValue | m2m:mgmtLink |  |  |
| cmdhEcDefParamValues | m2m:mgmtLink |  |  |

The resource [cmdhDefaults] does not have any child resources other than the one referenced by m2m:mgmtLink type of attribute.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhDefEcValue] Resource

The resource [cmdhDefEcValue] represents a value for the ec (event category) parameter of an incoming request. The detailed description can be found in clause D.12.3 of Architecture TS-0001 [1].

Table 6.5.4.1.4-16: Data Type Definition of [cmdhDefEcValue]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhDefEcValue-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.4-17: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.4-18:  Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhDefEcValue” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| Order | xs:positiveInteger |  |  |
| defEcValue | xs:string |  |  |
| requestOrigin | xs:string |  |  |
| requestContext | xs:string |  |  |
| requestContextNotification | xs:Boolean |  |  |
| requestCharacteristics | xs:string |  |  |

The resource [cmdhDefEcValue] does not have any child resources.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhEcDefParamValues] Resource

The resource [cmdhEcDefParamValues] represents a specific set of default values for the CMDH related parameters rqet (request expiration timestamp), rset (result expiration timestamp), oet (operational execution time), rp (response persistence) and da (delivery aggregation) that are applicable for a given ec (event category) if these parameters are not specified in the request. The detailed description can be found in clause D.12.4 of Architecture TS-0001 [1].

Table 6.5.4.1.5-19: Data Type Definition of [cmdhEcDefParamValues]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhEcDefParamValues-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.5-20: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.5-21: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhEcDefParamValues” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| applicableEventCategory | xs:string |  |  |
| defaultRequestExpTime | xs:nonNegativeInteger |  |  |
| defaultResultExpTime | xs:nonNegativeInteger |  |  |
| defaultOpExecTime | xs:nonNegativeInteger |  |  |
| defaultRespPersistence | xs:nonNegativeInteger |  |  |
| defaultDelAggregation | xs:string |  |  |

The resource [cmdhEcDefParamValues] does not have any child resources.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhLimits] Resource

The Resource [cmdhLimits] represents limits for CMDH related parameter values. The detailed description can be found in clause D.12.5 of Architecture TS-0001 [1].

Table 6.5.4.1.6-22: Data Type Definition of [cmdhLimits]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhLimits-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.6-23: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.6-24: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhEcDefParamValues” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| Order | xs:positiveInteger |  |  |
| requestOrigin | xs:string |  |  |
| requestContext | xs:string |  |  |
| requestContextNotification | xs:boolean |  |  |
| requestCharacteristics | xs:string |  |  |
| limitsEventCategory | xs:string |  |  |
| limitsRequestExpTime | xs:string |  |  |
| limitsResultExpTime | xs:string |  |  |
| limitsOpExecTime | xs:string |  |  |
| limitsRespPersistence | xs:string |  |  |
| limitsDelAggregation | xs:string |  |  |

The resource [cmdhLimits] does not have any child resources.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhNetworkAccessRules] Resource

The resource [cmdhNetworkAccessRules] defines the usage of underlying networks for forwarding information to other CSEs during processing of CMDH-related requests in a CSE. The detailed description can be found in clause D.12.6 of Architecture TS-0001 [1].

Tabla 6.5.4.1.7-25:Type Definition of [cmdhNetworkAccessRules]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhNetworkAccessRules-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.7-26:  Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.7-27:  Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhNetworkAccessRules” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| description | See Table 7.3.x.1.2 |  |  |
| applicableEventCategories | xs:string |  |  |
| cmdhNwAccessRule | m2m:mgmtLink |  |  |

The resource [cmdhNetworkAccessRules] does not have any child resources other than those referenced by m2m:mgmtLink type of attribute.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhNwAccessRule] Resource

The resource [cmdhNwAccessRule] defines limits in usage of specific underlying networks for forwarding information to other CSEs during processing of CMDH-related requests. The detailed description can be found in clause D.12.7 of Architecture TS-0001 [1].

Table 6.5.4.1.8-28: Data Type Definition of [cmdhNwAccessRule]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhNwAccessRule-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.8-29: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.8-30:  Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhNwAccessRule” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| targetNetwork | xs:string |  |  |
| minReqVolume | xs:nonNegativeInteger |  |  |
| backOffParameters | xs:complexType |  |  |
| otherConditions | TBD |  |  |
| allowedSchedule | m2m:mgmtLink |  |  |

Editor’s Note: The backOffParameters attribute needs to be specified as a specific complexType data type consisting of three elements (BackoffTime, BackoffTimeIncrement, MaximumBackoffTime). It needs to be clarified how to document such attributes in the present Data Type Definition tables.

Editor’s Note: The otherConditions attribute needs to be clarified  in TS-0001 before we can determine the data type.

The resource [cmdhNwAccessRule] does not have any child resources other than the one referenced by m2m:mgmtLink type of attribute.

The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

##### [cmdhBuffer] Resource

The resource [cmdhBuffer] represents limits in usage of buffers for temporarily storing information that needs to be forwarded to other CSEs during processing of CMDH-related requests in a CSE. The detailed description can be found in clause D.12.8 of Architecture TS-0001 [1].

Table 6.5.4.1.9-31: Data Type Definition of [cmdhBuffer]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-cmdhBuffer-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Table entries will be filled after general agreement on technical issues is achieved in PRO WG

Table 6.5.4.1.9-32: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | N/A |
| Retrieve | M | M | N/A |
| Update | M | M | N/A |
| Delete | M | M | N/A |

Table 6.5.4.1.9-33:  Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows Operation |
| --- | --- | --- | --- |
| mgmtDefinition | Fixed string “cmdhBuffer” |  |  |
| objectID | See Table 7.3.x.1.2 |  |  |
| objectPath | See Table 7.3.x.1.2 |  |  |
| Description | See Table 7.3.x.1.2 |  |  |
| applicableEventCategory | xs:string |  |  |
| maxBufferSize | xs:nonNegativeInteger |  |  |
| storagePriority | xs:nonNegativeInteger |  |  |

The resource [cmdhBuffer] does not have any child resources. The Resource Specific Procedure on CRUD Operations as specified in clause 7.3.x.2 for the generic <mgmtObj> resource type apply.

#### <mgmtObj> Primitives and Procedures

This section describes the primitive and associated procedures for the <mgmtObj> resource and subresources.

### <mgmtCmd> Resources and Procedures

This section describes the primitives and procedures to communicate the <mgmtCmd> resource and subresources across the Mca and Mcc reference points.

#### <mgmtCmd> Resource

This section describes the definition of the <mgmtCmd> resource and subresources. In addition, the policy (e.g., mandatory, optional) related to the presence of a resource’s attributes with respect to the primitives is documented in this section.

#### <mgmtCmd> Primitive and Procedures

This section describes the primitive and associated procedures for the <mgmtObj> resource and subresources.

## oneM2M Connectivity Management API

Editor’s Note: This sub-clause should be considered to move as part of “oneM2M Resource Types”.

<Text>

The following text is to be used when appropriate:

## oneM2M Data Management API

Editor’s Note: This sub-clause should be considered to move as part of “oneM2M Resource Types”.

<Text>

The following text is to be used when appropriate:

## Response Status Codes

### Introduction

This clause specifies the allocated and reserved Response Status Codes (RSC) for a oneM2M System. This clause defines the framework for RSCs to support current and future oneM2M messages and transactions, current and future security procedures, external device management protocols, underlying transport protocols, interworking protocols, and vendor extensions.. The detailed mappings between the oneM2M message RSCs and security procedures and other protocols are specified in the oneM2M TS of each other protocol. These include:

- RSCs for security procedures in [TS0003]

- RSCs for OMA device management [TS0005]

- RSCs for BBF device management [TS0006]

- RSCs for CoAP underlying transport protocol [TS0008]

- RSCs for HTTP underlying transport protocol [TS0009]

- RSCs for MQTT underlying transport protocol [TS0010]

### RSC Data Type

An RSC data type consists of a constrained integer and a list of strings. Each instance of an RSC is defined to include the constrained integer (numeric code) and mandatory list of zero or more strings (arguments) dependent upon the constrained integer. A single RSC instance is supported in each Response message. An error is a condition, which prevents the requested operation from being completed. The first error detected during validation procedures during an unsuccessful operation shall be reported via this single RSC instance. A warning is a condition that is associated with, but does not prevent the requested operation from being completed. The first warning detected during validation procedures during a successful operation shall be reported via this single RSC instance.

### RSC Framework Overview

All RSCs are categorised as one of three classes: successful, unsuccessful or as an acknowledgement. For each response class as indicated in Table X.3-1, the framework includes two RSC Source partitions: “current” and “reserved for future”. RSCs are permanently and un-mutably allocated in the “current” partition. The order of allocation of RSCs from various sources within a partition is not specified. New RSCs shall be allocated by adding them to the “current” partition while reducing the “reserved for future” partition.  RSCs within the “current” partition are associated with a oneM2M release. For each subsequent oneM2M release, a new set of RSCs shall be allocated and added to the “current” partition.

The term “native” is used to indicate that the RSC source originated as a result of a resource operation validation procedure before protocol mapping is applied.

Table 6.8.3-34 RSC Framework Overview

| Response Class | RSC Source | Numeric Code Range |
| --- | --- | --- |
| Successful | Current - native oneM2M codes - mapped security codes - mapped device management codes - mapped transport codes - mapped interworking codes - mapped vendor extensions | 0000 - aaaa |
|  | Reserved for future - native oneM2M codes - mapped security codes - mapped device management codes - mapped transport codes - mapped interworking codes - mapped vendor extensions | (aaaa+1) - 1999 |
| - Un-successful | Current - native oneM2M codes - mapped security codes - mapped device management codes - mapped transport codes - mapped interworking codes - mapped vendor extensions | 2000 - bbbb |
|  | Reserved for future - native oneM2M codes - mapped security codes - mapped device management codes - mapped transport codes - mapped Interworking codes - mapped vendor extensions | bbbb+1 - 7999 |
| - Acknowledgements | Current - native oneM2M codes - mapped security codes - mapped device management codes - mapped transport codes - mapped interworking codes - mapped vendor extensions | 8000 - cccc |
|  | Reserved for future - native oneM2M codes - mapped security codes - mapped device management codes - mapped transport codes - mapped interworking codes - mapped vendor extensions | cccc+1 - 9999 |

Editor’s Note: Other terms rather than “current” and “reserved for future” may be preferred e.g. first release and subsequent release.

In Table X.3-1, 0<= aaaa <= 1999, 2000 <= bbbb <= 7999, 8000 <= cccc <= 9999

### Current Response Status Codes

The tables in the following clauses specify the RSCs for oneM2M releases. Each RSC includes: a response status description, a numeric code, arguments, and applicable APIs (indicated by reference points). The response status description shall be unique within the set of response status descriptions for a release. Response status descriptions may be re-used among RSCs associated with different releases.

#### Successful Responses

The following tables specify the RSCs for successful responses for each release.

Table 6.8.4.1-35 RSCs for Successful Responses for Release 1

| Response Status Description | Numeric Code | Argument1 | Argument2 | Argument 3 | Applicability |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mca | Mcc | Mcn | Mcc' |
|  | 0000 |  |  |  |  |  |  |  |
|  | 0001 |  |  |  |  |  |  |  |
|  | 0002 |  |  |  |  |  |  |  |
|  | 0003 |  |  |  |  |  |  |  |
|  | 0004 |  |  |  |  |  |  |  |
|  | 0005 |  |  |  |  |  |  |  |
|  | 0006 |  |  |  |  |  |  |  |
|  | 0007 |  |  |  |  |  |  |  |
|  | 0008 |  |  |  |  |  |  |  |
|  | 0009 |  |  |  |  |  |  |  |

Editor’s Note: Since more RSCs may be added until the first release, the Numeric Codes will be finalized just before the first release.

#### Un-successful Responses

The following tables specify the RSCs for un-successful responses for each release.

Table 6.8.4.2-36: RSCs for Un-successful Responses for Release 1

| Description | Numeric Code | Argument1 | Argument2 | Argument 3 | Applicability |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mca | Mcc | Mcn | Mcc' |
| Location info not authorized | 2000 |  |  |  |  |  | X |  |
| Unsupported resource | 2001 | Resource URI |  |  | X | X |  |  |
| Unsupported attribute | 2002 | Attribute URI |  |  | X | X |  |  |
| Cannot forward, target not reachable | 2003 | Resource URI |  |  | X | X |  |  |
| Cannot forward, other reason TBD | 2004 | Resource URI |  |  | X | X |  |  |
| Create error - no privilege | 2005 | Resource URI |  |  | X | X |  |  |
| Create error – already exists | 2006 | Resource URI |  |  | X | X |  |  |
| Create error – missing mandatory parameter | 2007 | Missing parameter |  |  | X | X |  |  |
| Retrieve error - no privilege | 2008 | Resource URI |  |  | X | X |  |  |
| Retrieve error – does not exist | 2009 | Target URI |  |  | X | X |  |  |
| Update error - no privilege | 2010 | Resource URI |  |  | X | X |  |  |
| Update error – does not exist | 2011 | Resource URI |  |  | X | X |  |  |
| Update error – unacceptable contents | 2012 | Attribute URI |  |  | X | X |  |  |
| Delete error – does not exist | 2013 | Target URI |  |  | X | X |  |  |
| Delete error – no privilege | 2014 | Resource URI |  |  | X | X |  |  |
| Create delivery – not able to take on responsibility | 2015 |  |  |  | X | X |  |  |
| Create fanoutpoint – group request identifier exists | 2016 |  |  |  | X | X |  |  |
| Retrieve fanoutpoint – group request identifier exists | 2017 |  |  |  | X | X |  |  |
| Update fanoutpoint – group request identifier exists | 2018 |  |  |  | X | X |  |  |
| Delete fanoutpoint – group request identifier exists | 2019 |  |  |  | X | X |  |  |
| Create mgmtObj – not allowed | 2020 | CSEBase URI |  |  | X | X |  |  |
| Create mgmtObj – already exists | 2021 | CSEBase URI |  |  | X | X |  |  |
| Create mgmtObj – not reachable | 2022 | CSEBase URI |  |  | X | X |  |  |
| Create mgmtObj – memory shortage | 2023 | CSEBase URI |  |  | X | X |  |  |
| Retrieve mgmtObj – not found | 2024 | MgmtObj URI |  |  | X | X |  |  |
| Update mgmtObj – not reachable | 2025 | MgmtObj URI |  |  | X | X |  |  |
| Update mgmtObj – not found | 2026 | MgmtObj URI |  |  | X | X |  |  |
| Delete mgmtObj – not reachable | 2027 | MgmtObj URI |  |  | X | X |  |  |
| Delete mgmtObj – not found | 2028 | MgmtObj URI |  |  | X | X |  |  |
| Execute mgmtObj – not reachable | 2029 | MgmtObj URI |  |  | X | X |  |  |
| Execute mgmtObj – not found | 2030 | MgmtObj URI |  |  | X | X |  |  |
| Cancel execInstance – not cancellable | 2031 | Name of mgmtCmd resource |  |  | X | X |  |  |
| Cancel execInstance – already complete | 2032 | Name of mgmtCmd resource |  |  | X | X |  |  |
| Delete execInstance – not cancellable | 2033 | Name of mgmtCmd resource |  |  | X | X |  |  |
| Delete execInstance – already complete | 2034 | Name of mgmtCmd resource |  |  | X | X |  |  |
| Retrieve CSEBase – format error | 2035 | CSEBase URI |  |  | X | X |  |  |
| CMDH rules –non compliant | 2036 |  |  |  |  |  |  |  |
|  | 2037 |  |  |  |  |  |  |  |
|  | 2038 |  |  |  |  |  |  |  |
|  | 2039 |  |  |  |  |  |  |  |

Editor’s Note: The contents of this table are exemplary and are expected to be revised. In particular ARC discussions regarding UPDATE and attributes may impact argument1. Some of the exemplary RSCs may be equivalent and can be collapsed to fewer RSCs.

Editor’s Note: Since more RSCs may be added until the first release, the Numeric Codes will be finalized just before the first release.

#### Acknowledgement Responses

The following tables specify the RSCs for acknowledgement responses for each release.

Table 6.8.4.3-37: RSCs for Acknowledgement Responses for Release 1

| Description | Numeric Code | Argument1 | Argument2 | Argument 3 | Applicability |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mca | Mcc | Mcn | Mcc' |
|  | 8000 |  |  |  |  |  |  |  |
|  | 8001 |  |  |  |  |  |  |  |
|  | 8002 |  |  |  |  |  |  |  |
|  | 8003 |  |  |  |  |  |  |  |
|  | 8004 |  |  |  |  |  |  |  |
|  | 8005 |  |  |  |  |  |  |  |
|  | 8006 |  |  |  |  |  |  |  |

- Editor’s Note: Since more RSCs may be added until the first release, the Numeric Codes will be finalized just before the first release.

# oneM2M Resource Types

## Introduction

## Prerequisites

### Primitive format and procedure outlines

#### Primitive format

##### Request primitive format

Table 7.2.1.1-38summarizes the primitive parametersattributes for Request primitive, showing any differences as applied to C, R, U, D or N operations. “M” indicates mandatory, “O” indicates optional, “NP” indicates not present, “C” indicates conditional.

Table 7.2.1.1-38 : Request Primitive Parameters

| Primitive Parameter | CREATE |  | RETRIEVE |  | UPDATE |  | DELETE |  | NOTIFY |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | M/O |  | M/O |  | M/O |  | M/O |  | M/O |  |
|  | Mca | Mcc | Mca | Mcc | Mca | Mcc | Mca | Mcc | Mca | Mcc |
| primitiveType (=REQUEST) | M | M | M | M | M | M | M | M | M | M |
| Operation | M | M | M | M | M | M | M | M | M | M |
| To | M | M | M | M | M | M | M | M | M | M |
| From | M | M | M | M | M | M | M | M | M | M |
| Request Identifier | M | M | M | M | M | M | M | M | M | M |
| Resource Type | M | M | NP | NP | NP | NP | NP | NP | NP | NP |
| Name | C | C | NP | NP | NP | NP | NP | NP | NP | NP |
| Content | C | C | C | C | C | C | NP | NP | M | M |
| Original Timestamp | O | O | O | O | O | O | O | O | O | O |
| Request Expiration Timestamp | C | C | C | C | C | C | C | C | C | C |
| Result Expiration Time | C | C | C | C | C | C | C | C | C | C |
| Operation Execution Time | C | C | C | C | C | C | C | C | C | C |
| Response Type | C | C | C | C | C | C | C | C | C | C |
| Result Persistence | C | C | C | C | C | C | C | C | NP | NP |
| Result Content | C | C | C | C | C | C | C | C | NP | NP |
| Event Category | C | C | C | C | C | C | C | C | C | C |
| Delivery Aggregation | C | C | C | C | C | C | C | C | C | C |
| Group Request Identifier | C | C | C | C | C | C | C | C | C | C |
| Filter Criteria | NP | NP | C | C | NP | NP | NP | NP | NP | NP |
| Discovery Result Type | NP | NP | C | C | NP | NP | NP | NP | NP | NP |

Editor’s Note: This is an initial list of primitive parameters. Parameter name, mandatory/optional or description for each parameter is FFS.

##### Response primitive format

Table 7.2.1.1-39 summarizes the primitive parameters for Response primitive, showing any differences as applied to C, R, U, D or N operations and unsuccessful operations. “M” indicates mandatory, “O” indicates optional, “NP” indicates not present, “C” indicates conditional.

Table 7.2.1.1-39 : Response Primitive Parameters (1/2)

| Primitive parameter | Ack |  | Successful CREATE |  | Successful RETRIEVE |  | Successful UPDATE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | M/O |  | M/O |  | M/O |  | M/O |  |
|  | Mca | Mcc | Mca | Mcc | Mca | Mcc | Mca | Mcc |
| primitiveType (=RESPONSE) | M | M | M | M | M | M | M | M |
| Operation | M | M | M | M | M | M | M | M |
| Response Code | M | M | M | M | M | M | M | M |
| Request Identifier | M | M | M | M | M | M | M | M |
| Content | NP | NP | C | C | M | M | C | C |
| To | C | C | C | C | C | C | C | C |
| From | C | C | C | C | C | C | C | C |
| Originating Timestamp | O | O | O | O | O | O | O | O |
| Result Expiration Timestamp | C | C | C | C | C | C | C | C |
| Status Code | C | C | C | C | C | C | C | C |
| Response Address | C | C | C | C | C | C | C | C |
| Pending Requests | NP | NP | NP | NP | C | C | NP | NP |

Table 7.2.1.1-40 : Response Primitive Parameters (2/2)

| Primitive parameter | Successful DELETE |  | Successful NOTIFY |  | Unsuccessful |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | M/O |  | M/O |  | M/O |  |
|  | Mca | Mcc | Mca | Mcc | Mca | Mcc |
| primitiveType (=RESPONSE) | M | M | M | M | M | M |
| Operation | M | M | M | M | M | M |
| Response Code | M | M | M | M | M | M |
| Request Identifier | M | M | M | M | M | M |
| Content | C | C | NP | NP | C | C |
| To | C | C | C | C | C | C |
| From | C | C | C | C | C | C |
| Originating Timestamp | O | O | O | O | O | O |
| Result Expiration Timestamp | C | C | NP | NP | C | C |
| Status Code | C | C | C | C | C | C |
| Response Address | C | C | NP | NP | NP | NP |

Editor’s Note: This is an initial list of primitive parameters. Parameter name, mandatory/optional or description for each parameter is FFS.

#### Description of Generic Procedures

##### Generic Resource Create Request Procedure

A Generic Resource Create Request Procedure shall be comprised of the following actions. Additional actions specific to individual procedures may be listed in the respective sections by referencing these actions and providing additional steps.The Originator shall execute the following steps in order:

BlockingSync:

- “Compose Request primitive”

- “Send a Request to the receiver CSE”.

- “Wait for Response primitive”.

Non-BlockingSync:

- “Copose Request primitive"

- "Send a Request to the receiver CSE".

- "Wait for Response primitive". The Response contains information about the <request> resource.

- “Compose Request primitive”, the Request shall be addressing the <request> resource responded from the Response in the former step 3.

- “Send a Request to the receiver CSE”. The Request is addressing the <request> resource.

- "Wait for Response primitive".

Non-BlockingAsync

- "Compose Request primitive"

- "Send a Request to the receiver CSE".

- "Wait for Response primitive".

- “Receive the Notify of the result”

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an unsuccessful Response" and then "Send Response primitive". The corresponding Response Code shall be included in the Response primitive.

Hosting CSE of the targeted resource for blocking requests:

- “Check the syntax of received message”.

- “Check existence of the addressed resource”

- "Check authorization of the requesting entity based on accessControlPolicyID".

- "Check validity of resource representation for the given resource type

- "Create the resource".

- “Announce the resource”.

- "Create a successful Response".

- "Send Response primitive".

Transit CSE of the targeted resource for blocking requests:

- “Check the syntax of received message”.

- “Re-targeting”

Hosting CSE for non-blocking synchronous requests:

- “Check the syntax of received message”.

- “Check existence of the addressed resource”

- "Check authorization of the requesting entity based on accessControlPolicyID".

- " Response primitive (ack) with the information of <request> resource”

- "Check validity of resource representation for the given resource type

- "Create the resource".

- “Announce the resource”.

- “Update the result to the <request> resource”

Hosting CSE for non-blocking asynchronous requests:

- “Check the syntax of received message”.

- “Check existence of the addressed resource”

- "Check authorization of the requesting entity based on accessControlPolicyID".

- "Response primitive (ack) with the information of <request> resource”

- "Check validity of resource representation for the given resource type

- "Create the resource".

- “Announce the resource”.

- “Notify the originator with the result of the operation”

Hosting CSE for non-blocking asynchronous requests:

- “Check the syntax of received message”.

- “Check existence of the addressed resource”

- "Check authorization of the requesting entity based on accessControlPolicyID".

- "Response primitive (ack) with the information of <request> resource”

- "Check validity of resource representation for the given resource type

- "Create the resource".

- “Announce the resource”.

- “Notify the originator with the result of the operation”

Transit CSE for non-blocking synchronous requests:

- “Check the syntax of received message”.

- “Generate <request> resource based on the Request”

- " Response primitive (ack) with the information of <request> resource”

- “Re-targeting”

Transit CSE for non-blocking asynchronous requests:

- “Check the syntax of received message”.

- " Response primitive (ack)”

- “Re-targeting”

- “Notify the originator with the result of the Request”

Editor’s Note: These procedures and steps are FFS. Steps are pointer to the common operation section in TS-0004.

Editor’s Note: BlockingSync, Non-blockingSync, Non-blockingAsync need to be further clarified.

##### Generic Resource Retrieve Request Procedure

A Generic Resource Retrieve Request Procedure shall be comprised of the following actions. Additional actions specific to individual procedures may be listed in the respective sections by referencing this procedure and providing additional steps.

The Originator shall execute the following steps in order:

Orig-R-1.0	"Compose Request primitive".

Orig-R-2.0	"Send a Request to the receiver CSE".

Orig-R-3.0	"Wait for Response primitive".

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an unsuccessful Response" and then "Send Response primitive". The corresponding Response Code shall be included in the Response primitive.

Rcv-R-1.0	“Check validity of the primitive parameter”.

NOTE: “validity” in this sentence means that the primitive parameters conform to the specification and also that it is possible for the Receiver to process the request within the constraints of the primitive parameters. For example, if there are expiration times (e.g. Request Expiration Timestamp, Result Expiration Time and Operation Execution Time) in the primitive parameters, then this step will check that it is possible to meet those expiration times in line with the configured policies of the Receiver, if any.

If the Response Type primitive parameter is set to either ‘nonBlockingRequestSynch’ or ‘nonBlockingRequestAsynch’, then the Receiver shall:

Rcv-R-2.0	"Create a Response primitive (ack)”.

Rcv-R-3.0	"Send Response primitive".

If the To primitive parameter addresses a CSE that is not the receiver CSE, then:

The Receiver shall

Rcv-R-4.1 	“Forward Request primitive”

If the Result Content primitive parameter is set to ‘nothing’, then

If the Response Type primitive parameter is set to ‘blockingRequest’ then the Receiver shall

Rcv-R-4.2 	“Create a Response primitive (ack)”.

Rcv-R-4.3 	"Send Response primitive".

The Receiver shall then

Rcv-R-4.4		“Complete the procedure”.

else the Receiver shall

Rcv-R-4.5 	“Wait for operational result”.

Rcv-R-4.6 	“Provide the received operational result to the Originator”.

Rcv-R-4.7 	"Complete the procedure".

else (if the To primitive parameter addresses the receiver CSE) then do the following steps.

Rcv-R-5.0		"Check existence of the addressed resource".

Rcv-R-6.0		"Check authorization of the requesting entity based on accessControlPolicyID".

Rcv-R-7.0		"Read the addressed resource".

Rcv-R-8.0		"Create a successful Response".

Rcv-R-9.0		"Send Response primitive".

Editor’s Note: These procedures and steps are FFS. Steps are pointer to the common operation section in TS-0004.

##### Generic Resource Update Request Procedure

A Generic Resource Update Procedure shall be comprised of the following actions. Additional actions specific to individual procedures may be listed in the respective sections by referencing this procedure and providing additional steps.

The Originator shall execute the following steps in order:

Orig-U-1.0	"Compose Request primitive".

Orig-U-2.0	"Send a Request to the receiver CSE".

Orig-U-3.0	"Wait for Response primitive".

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an unsuccessful Response" and then "Send Response primitive". The corresponding Response Code shall be included in the Response primitive.

Rcv-U-1.0 	“Check validity of the primitive parameters”.

NOTE: “validity” in this sentence means that the primitive parameters conform to the specification and also that it is possible for the Receiver to process the request within the constraints of the primitive parameters. For example, if there are expiration times (e.g. Request Expiration Timestamp, Result Expiration Time and Operation Execution Time) in the primitive parameters, then this step will check that it is possible to meet those expiration times in line with the configured policies of the Receiver, if any.

If the Response Type primitive parameter is set to either ‘nonBlockingRequestSynch’ or ‘nonBlockingRequestAsynch’, then the Receiver shall:

Rcv-U-2.0		"Create a Response primitive (ack)”.

Rcv-U-3.0		"Send Response primitive".

else do the following steps.

If the To primitive parameter addresses a CSE that is not the receiver CSE, then:

The Receiver shall

Rcv-U-4.1 	“Forward Request primitive”

If the Result Content primitive parameter is set to ‘nothing’, then

If the Response Type primitive parameter is set to ‘blockingRequest’ then the Receiver shall

Rcv-U-4.2 	“Create a Response primitive (ack)”.

Rcv-U-4.3 	"Send Response primitive".

The Receiver shall then

Rcv-U-4.4		“Complete the procedure”.

else the Receiver shall

Rcv-U-4.5 	“Wait for operational result”.

Rcv-U-4.6 	“Provide the received operational result to the Originator”.

Rcv-U-4.7 	"Complete the procedure".

else (if the To primitive parameter addresses the receiver CSE) then do the following steps.

Rcv-U-5.0		"Check existence of the addressed resource".

Rcv-U-6.0		"Check authorization of the requesting entity based on accessControlPolicyID".

Rcv-U-7.0		"Check validity of resource representation.

Rcv-U-8.0		"Updated the addressed resource".

Rcv-U-9.0		"Announce the resource"

Rcv-U-10.0	"Create a successful Response".

Rcv-U-11.0	"Send Response primitive".

Editor’s Note: These procedures and steps are FFS. Steps are pointer to the common operation section in TS-0004.

##### Generic Resource Delete Request Procedure

A Generic Resource Delete Procedure shall be comprised of the following actions. Additional actions specific to individual procedures may be listed in the respective sections by referencing this procedure and providing additional steps.

The Originator shall execute the following steps in order:

Orig-D-1.0	"Compose Request primitive".

Orig-D-2.0	"Send a Request to the receiver CSE".

Orig-D-3.0	"Wait for Response primitive".

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an unsuccessful Response" and then "Send Response primitive". The corresponding Response Code shall be included in the Response primitive.

Rcv-D-1.0 	“Check validity of the primitive parameters”.

NOTE: “validity” in this sentence means that the primitive parameters conform to the specification and also that it is possible for the Receiver to process the request within the constraints of the primitive parameters. For example, if there are expiration times (e.g. Request Expiration Timestamp, Result Expiration Time and Operation Execution Time) in the primitive parameters, then this step will check that it is possible to meet those expiration times in line with the configured policies of the Receiver, if any.

If the Response Type primitive parameter is set to either ‘nonBlockingRequestSynch’ or ‘nonBlockingRequestAsynch’, then the Receiver shall:

Rcv-D-2.0		"Create a Response primitive (ack)”.

Rcv-D-3.0		"Send Response primitive".

If the To primitive parameter addresses a CSE that is not the receiver CSE, then:

The Receiver shall

Rcv-D-4.1 	“Forward Request primitive”

If the Result Content primitive parameter is set to ‘nothing’, then

If the Response Type primitive parameter is set to ‘blockingRequest’ then the Receiver shall

Rcv-D-4.2 	“Create a Response primitive (ack)”.

Rcv-D-4.3 	"Send Response primitive".

The Receiver shall then

Rcv-D-4.4		“Complete the procedure”.

else the Receiver shall

Rcv-D-4.5 	“Wait for operational result”.

Rcv-D-4.6 	“Provide the received operational result to the Originator”.

Rcv-D-4.7 	"Complete the procedure".

else (if the To primitive parameter addresses the receiver CSE) then do the following steps.

Rcv-D-5.0		"Check existence of the addressed resource".

Rcv-D-6.0		"Check authorization of the requesting entity based on accessControlPolicyID".

Rcv-D-7.0		If the resource is announced, then “De-Announce the resource”.

Rcv-D-8.0		"Delete the addressed resource".

Rcv-D-9.0		"Create a successful Response".

Rcv-D-10.0	"Send Response primitive".

Editor’s Note: These procedures and steps are FFS. Steps are pointer to the common operation section in TS-0004.

##### Generic Notify Procedure

A Generic Notify Procedure shall be comprised of the following actions. Additional actions specific to individual procedures may be listed in the respective sections by referencing these actions and providing additional steps.

The Notification Originator shall execute the following steps in order:

Orig-N-1.0	“Compose Notify Request Primitive”

Orig-N-2.0	“Send a Notify Request to the Receiver”

Orig-N-3.0	“Wait for Response Primitive”

The Receiver shall execute the following steps in order.

Editor’s Note: Error cases for this Notify Procedure is FFS.

Rcv-N-1.0	“Check Validity of the Primitive parameters”

Editor’s Note: The Representation of Notify Request/Response is FFS.

Editor’s Note: These procedures and steps are FFS. Steps are pointer to the common operation section in TS-0004.

##### Resource Discovery Request Procedure

A resource discovery is used to discover resources in a CSE. A Resource discovery request is done by sending RETRIEVE request with filterUsage, one of the filterCriteria parameters, configured as “discovery” and the request may include other filterCriteria prameters as well. A resource discovery request procedure shall be comprised of the following actions.

Originator: The Originator shall execute the following steps in order:

Orig-R-1.0	"Compose Request primitive".

The to parameter in the RETRIEVE Request indicates the root of where the discovery begins.

The RETRIEVE Request shall include filterUsage parameter in fc.

The RETRIEVE Request may include other parameters of filterCriteria in fc.

Orig-R-2.0	"Send a Request to the receiver CSE".

Orig-R-3.0	"Wait for Response primitive".

Receiver: the procedure of the Receiver written in the clause 7.2.1.2.2 (from Rcv-R-1.0 to Rcv-R-5.0) shall be the same as initial steps. The following steps are for the resource discovery specific procedure.

Rcv-R-6.0		"Find resources, which match all the configured filterCriteria in cn and which the Originator has “Discover” access right, under the addressed resource".

Rcv-R-7.0		"Create a successful Response".

Rcv-R-8.0		"Send Response primitive".

### Common operations

Editor’s Note: Actual Error handling is FFS.

Editor’s Note: the options will need to be filled in at some point

#### Originator Actions

##### Compose Request primitive

The originator shall compose a Request message that shall be mapped to a specific protocol.

The Request shall include the “fr” and “to” to indicate the identifier of the originator of the request and the targeted receiver of the request.

The Request shall include the other attributes in case needed depend on the resource the request is addressing.

When including a resource representation in the request indication for create and update, the originator shall take into account the validation rules as specified in "Check validity for resource representation for create" and "Check validity for resource representation for update" respectively. For example, any attributes marked with NP shall not be present in the resource representation for the corresponding request indication.

##### Send a Request to the Receiver CSE

The originator shall determine the receiver CSE.

The receiver of the Request shall be the registrar CSE of the originator in case the originator is not IN-CSE.

If the originator is the IN-CSE, the receiver of the Request shall be the CSE whose identifier is the prefix of the “to” attribute of the Request.

If this results in no matching CSE, then the request is rejected with a STATUS_NOT_FOUND.

If results in multiple CSEs, the request is rejected with a status STATUS_INTERNAL_SERVER_ERROR, since this shall never happen.

##### Wait for Response primitive

The originator shall wait for the Response primitive from the receiver that corresponds to the Request primitive that was sent by the originator. Correlation between the Request and the corresponding Response is handled by the transport layer.

If no Response primitive is received within a certain time, specified by server policy and/or by the underlying transport technology, this shall be handled as if a Response primitive with a statusCode STATUS_REQUEST_TIMEOUT was received.

#### Hosting CSE actions

##### Check existence of the addressed resource

The hosting CSE shall check if the resource addressed by the “to” attribute exists in the repository. If the resource does not exist, the hosting CSE shall reject the request with a "STATUS_NOT_FOUND".

##### Check the syntax of received message

Message validity is specified by the protocol mapping TSes (HTTP binding, CoAP binding and MQTT binding). The received resource representation (e.g. in plain XML, binary XML or JSON) shall be validated against the provided schema definitions.

If the message is not valid, the request shall be rejected with a "STATUS_BAD_REQUEST".

##### Check validity of resource representation for CREATE

The handling below shall apply t to each attribute in the resource for CREATE request primitives and the handling depends on the "presence in CREATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

If no resource representation is present in the CREATE request, then the request is rejected with a STATUS_BAD_REQUEST statusCode.

The id attribute has special handling. If the id-attribute is present in the CREATE request, the hosting CSE shall check if a resource with the same id already exists in the addressed collection. If such a resource exists and the response column is marked as M, then the hosting CSE shall reject the request with a "STATUS_CONFLICT".

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a STATUS_BAD_REQUEST.

N/A attribute

Indicates that the attribute is not supported in the CREATE request.

The resource indicated as N/A shall be created when the parent resource was created, or the resource shall be permanently exist in oneM2M System.

M attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M then the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

If the attribute is not present in the resource representation in the CREATE request the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

O attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M or O then the hosting CSE shall reject the request with a "STATUS_NOT_IMPLEMENTED".

NP attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

##### Check validity of resource representation for UPDATE

The handling below shall apply to each attribute in the resource for UPDATE request primitives and the handling depends on the "presence in UPDATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a STATUS_BAD_REQUEST.

N/A attribute

Indicates that the attribute is not supported in the UPDATE requestThe resource indicated as N/A shall not allow any modification on the resource. If Originator requested to perform UPDATE operation on the resource, the request shall be rejected with a STATUS_METHOD_NOT_ALLOWED.

M attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

If the attribute is not present in the resource representation in the UPDATE request, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST".

O attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted and the response column is marked M or O then the hosting CSE shall reject the request with a "STATUS_NOT_IMPLEMENTED" statusCode.

NP attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall reject the request with a "STATUS_BAD_REQUEST" unless the value provided for the attribute exactly matches the value in the current resource representation stored in the hosting CSE. In addition, the lastModifiedTime attribute shall always be accepted (but ignored) by the hosting CSE, no matter what value was provided in the request.

##### Create the resource

A new resource shall be created and correlated to the addressed and existing parent resource. This shall modify the resource representation of the addressed parent resource, specifically, if the parent resource has a lastModifiedTime attribute this shall be set to same value as the creationTime attribute of the created resource. The following rules shall be applied:

If the created resource type has an id attribute, then the URI of the created resource shall be the URI of its parent resource with the URI-encoded id appended. (e.g. http://CSEbase.operator.org/myAppID, for an application resource with id "myAppID" created in the parent resource http://CSEbase.operator.org).

If a resource with the same ID already exists in the addressed collection, the hosting CSE shall provide a new id that is unique within the collection.

If expirationTime attribute is present in the resource representation of the to be created resource and the expirationTime is set to a non-negative time, then an expiration timer shall be started by the hosting CSE. At timer expiration the related resource is deleted by "Delete the addressed resource".

For setting the attributes in the resource representation the following rules shall apply in CREATE request primitives:

M attribute

If the provided value is acceptable, the server shall use the provided value in the resource representation of the created resource.

O attribute

If a value is provided and accepted, then the server shall use the provided value in the resource representation of the created resource.

If the attribute is not provided or accepted, but the attribute is marked M in the response, the hosting CSE shall assign default value or assign value based on local policy.

If the attribute is not present in the resource representation in the CREATE request and the response column is marked O then the hosting CSE shall create the resource without the attribute.

NP attribute

If the attribute is not present in the resource representation in the CREATE request, and the response column is marked M, then the hosting CSE shall create the resource with the default value.

##### Create a successful Response

The receiver shall create a successful Response primitive with a statusCode indicating "STATUS_CREATED". The response shall include the representation of the addressed resource, if the hosting CSE modified any of the provided attributes as provided in the Request. The Response shall also include the URI of a created resource.

##### Create an unsuccessful Response

The receiver shall create an unsuccessful Response primitive with a statusCode indicating the detected error condition.

Note: possible error codes and its error handling is described in resource specific procedure.

##### Read the addressed resource

An existing and accessible resource is addressed. The content of its attributes and references to its child resources shall be read in form of resource representation.

When the resource is read to provide a response to RETRIEVE request primitives, then the following rules shall be applied:

M attribute

The attribute is always present in the resource representation.

O attribute

The attribute is present in the resource representation if some conditions occur.

NP attribute

The attribute is never present in the resource representation.

##### Update the addressed resource

An existing and accessible resource is addressed. For attributes that are not included in the content parameter, the hosting CSE shall not change such attributes. For attributes provided in the content parameter, The content of its attributes shall be updated with the following rules shall be applied:

If the announceTo attribute of the resource is updated, the hosting CSE  shall perform the resource announcement or de-announcement as specified in the clause 7.2.2.4.

M attribute

If the provided value is accepted, the server shall use the provided value in the resource representation of the updated resource.

O attribute

If a value is provided and the value is accepted, the server shall use the provided value in the resource representation of the updated resource.

If the attribute is not provided or accepted, but the attribute is marked M in the response, the hosting CSE shall assign default value or assign value based on local policy.

If this attribute is provided in the content parameter and does not exist in the target resource, the hosting CSE shall create such attribute with the provided value.

If this attribute is set to NULL in the content parameter and exists in the target resource, the hosting CSE shall delete such attribute if the deletion of the attribute is allowed by the local policy.

NP attribute

If the attribute is not present in the resource representation in the UPDATE request and the response column is marked M then the hosting CSE shall not update the attribute value. There is only one exception to this rule and that is the lastModifiedTime attribute. The hosting CSE shall set the lastModifiedTime to the current time whenever an update primitive is received.

If the attribute is present in the resource representation in the UPDATE request the presented value shall be ignored, i.e. the hosting CSE shall never update its resource representation based on the presence of an NP attribute value in an update.

If the expirationTime attribute is present and modified by the procedure and it is set to a non-negative time, then an expiration timer shall be re-started by the hosting CSE. At timer expiration the related resource is deleted by "Delete the addressed resource".

##### Delete the addressed resource

An existing and accessible resource is addressed. The resource with all its attributes shall be deleted. Any expiration timer shall be stopped. This same procedure shall then be invoked (recursively) for each child resource of the deleted resource in case the child resource is only linked to the deleted resource.

The parent resource of the addressed resource shall be updated to remove the reference to the deleted resource. If the parent resource has a lastModificationTime attribute then this attribute shall be set to the time of the deletion.

If the resource is announced, the CSE shall try to de-announce the resource correspondingly.

##### Send Response primitive

A Response primitive shall be sent back to the originator.

##### Announce the resource or attribute

If CREATE request that contains an announceTo attribute is received,

- Compose the CREATE Request primitive as follows:

- link is set to the URI of the original resource.

- If accessControlPolicyIDs of the original resource is not present, accessControlPolicyIDs is set to the same value with the parent resource or from the local policy of the original resource.

- Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute. Such attributes shall be present in the original resource and set to same value as the original resource.

- Send a CREATE Request to the the CSE(s) represented by exact URI(s) or CSE-ID(s) in the announceTo of the request.

- Wait for Response primitive

- Add the URI of successfully announced resource to the announceTo attribute of the resource

- Include updated announceTo attribute in the content parameter in the Response to the received CREATE Request.

If UPDATE request that adds the URI or CSE-ID into the announceTo attribute is received,

- Compose the CREATE Request primitive as follows:

- link is set to the URI of the original resource.

- If accessControlPolicyIDs of the original resource is not present, accessControlPolicyIDs is set to the same value with the parent resource or from the local policy of the original resource.

- Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute. Such attributes shall be present in the original resource and set to same value as the original resource.

- Send a CREATE Request to the CSE(s) represented by exact URI(s) or CSE-ID(s) in the announceTo of the request, which is not included in the announceTo attribute of the original resource.

- Wait for Response primitive

- Add the URI of successfully announced resource to the announceTo attribute of the resource

- Include updated announceTo attribute in the content parameter in the Response to the received UPDATE Request.

If UPDATE request that adds the attribute name into the announcedAttribute attribute is received,

- Compose the UPDATE Request. The UPDATE Request shall provide the attribute name for the attribute to be announced, and the initial value for the attribute in the content parameter. The initial value shall be the same with the value from the original resource. The attribute that will be announced shall be marked as OA

- Send UPDATE Requests to all announced resources listed in the announceTo attribute.

- Wait for Response primitive

- Add the attribute name of the successfully announced attribute to the announcedAttribute attribute

- Include updated announcedAttribute attribute in the content parameter in the Response to the received UPDATE Request.

##### De-announce the resource or attribute

If UPDATE Request that deletes the URI from the announceTo attribute is received:

- Compose the DELETE Request primitive

- Send a DELETE Request to the CSE(s) represented by URI(s) in the announceTo attribute of the resource, which is not included in the announceTo of the request. The to parameter in the DELETE Request shall be set to the URI for the announced resource that will be deleted.

- Wait for Response primitive

- Remove the URI of  successfully de-announced resource from the announceTo attribute of the resource

- Include updated announceTo attribute in the Content parameter in the Response to the UPDATE Request of the original resource.

If DELETE Request is received:

- Compose the DELETE Request primitive

- Send DELETE Requests to all announced resources addressed by the URI(s) in the announceTo attribute of the resource.

- Wait for Response primitive

If UPDATE request that deletes the attribute name from the announcedAttribute attribute is received,

- Compose the UPDATE Request primitive. The to parameter in the UPDATE Request shall be set to the URI for the announced resource. The UPDATE Request shall set the attribute to NULL that will be de-announced (i.e., to be deleted) in the content parameter. The attribute that will be de-announced shall be marked as OA.

- Send UPDATE Requests to all announced resources listed in the announceTo attribute of the original resource.

- Wait for Response primitive

- Delete the attribute name of the successfully de-announced attribute from the announcedAttribute attribute

- Include updated announcedAttribute attribute in the content parameter in the Response to the received UPDATE Request.

#### Receiver CSE actions

##### Re-targeting

If the “to” attribute in the request does not start with the CSEBase URI of the receiver, the receiver CSE shall forward the request or shall serve the request locally (see below).

If the “to” attribute in the request starts with the CSEBase URI of the receiver, then the receiver CSE shall handle the request locally.

Acting as an originator the CSE shall perform the following procedures:

- "Send a Request to the receiver CSE".

- "wait for Response primitive".

When the Response is received the receiver CSE shall:

- Primitive specific procedure: Forward the Response to the original CSE.

## Resource Type-specific procedures and definitions

### Resource Type CSEBase

#### Introduction

A <CSEBase> resource shall represent a CSE. This <CSEBase> resource shall be the root for all the resources that are residing on the CSE. The detailed description can be found in clause 9.6.3 in Architecture TS([ref-Arch-TS]).

Table 7.3.1.1-41: Data Type Definition of cseBase

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Data Type definition should be posted after general agreement on technical issues

Table 7.3.1.1-42: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | N/A | N/A | N/A |
| Retrieve | M | M | M |
| Update | N/A | N/A | N/A |
| Delete | N/A | N/A | N/A |

Table 7.3.1.1-43: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows operation |
| --- | --- | --- | --- |
| supportedResourceType | m2m:commaList | (TBD) | Read-Only |

Table 7.3.1.1-44: Reference of child resources

| Child Resource Type Name | Multiplicity | Ref . to in Resource Type Definition |
| --- | --- | --- |
| remoteCse(variable) | 0..n | 7.3.1 |
| node(variable) | 0..n |  |
| AE (variable) | 0..n | 7.3.3 |
| container(variable) | 0..n | 7.3.4 |
| group(variable) | 0..n | 7.3.11 |
| accessControlPolicy | 0..n |  |
| subscription(variable) | 0..n | 7.3.6 |
| mgmtCmd(variable) | 0..n | 7.3.13 |
| locationPolicy(variable) | 0..n | 7.3.8 |
| statsConfig(variable) | 0..n | 7.3.29 |
| statsCollect(variable) | 0..n | 7.3.31 |

#### Operations

This sub-clause describes cseBase  resource’s specific behaviour for CRUD operations.

##### Create

The <cseBase > resource shall not be created via API.

Receiver:  The Receiver shall execute the following steps in order.

Rcv-C-Rq-1.0		“Create an unsuccessful Response” with responseCode TBD (Method not allowed).

Rcv-C-Rq-2.0		“Send Response primitive”.

##### Retrieve

This procedure follows the Generic Resource RetrieveRequest Procedure specified in clause 7.2.1.2.2.

##### Update

The <cseBase > resource shall not be UPDATEed via API.

Receiver:  The Receiver shall execute the following steps in order.

Rcv-U-Rq-1.0		“Create an unsuccessful Response” with responseCode TBD (Method not allowed).

Rcv-U-Rq-2.0		“Send Response primitive”.

##### Delete

The <cseBase > resource shall not be DELETEed via API.

Receiver:  The Receiver shall execute the following steps in order.

Rcv-D-Rq-1.0		“Create an unsuccessful Response” with responseCode TBD (Method not allowed).

Rcv-D-Rq-2.0		“Send Response primitive”.

### Resource Type remoteCSE

#### Introduction

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase>.

Conversely each registered CSE shall also be represented as a sub-set of <remoteCSE> resource in the registering CSE's <CSEBase>.

The detailed description can be found in clause 9.6.4 in Architecture TS.

Table 7.3.2.1-45: Data Type Definition of <<resource name>>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| remoteCSEType | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd |  |

Table 7.3.2.1-46: Applicability of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | M | M |
| Retrieve | M | M | M |
| Update | M | M | M |
| Delete | M | M | M |

Table 7.3.2.1-47: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows operation |
| --- | --- | --- | --- |
| cseType | m2m:cseTypeID | TBD |  |
| pointOfAccess | xs:anyURI | None |  |
| CSEBase | xs:anyURI | None |  |
| requestReachability | xs:boolean | TRUE |  |

Table 7.3.2.1-48: Reference of child resources

| Child Resource Type and Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- |
| <AE>(variable) | 0..n | Clause 9.6.5 |
| <container>(variable) | 0..n | Clause 7.3.4 |
| <group>(variable) | 0..n | Clause 7.3.11 |
| <accessControlPolicy>(variable) | 0..n | Clause TBD |
| <subscription>variable) | 0..n | Clause 7.3.6 |
| <pollingChannel>(variable) | 0..n | Clause 7.3.20 |
| <schedule>(variable) | 0..1 | Clause 7.3.7 |

      - Resource Specific Procedure on CRUD Operations

This sub-clause describes <remoteCSE> resource specific behaviour for CRUD operations.

##### Create

This procedure follows the Generic Resource Create Request Procedure specified in clause 7.2.1.2.1.

Editor’s Note: Place to describe internal procedures (ex. Collate CSE-ID with PoA) is FFS.

##### Retrieve

This procedure follows the Generic Resource Retrieve Request Procedure specified in clause 7.2.1.2.2.

##### Update

This procedure follows the Generic Resource Update Request Procedure specified in clause 7.2.1.2.3.

##### Delete

This procedure follows the Generic Resource Delete Request Procedure specified in clause 7.2.1.2.4.

### Resource Type application

Editor’s Note:WG5 will take a lead on this topic.

### Resource Type container

Editor’s Note:WG5 will take a lead on this topic. ???

Editor’s Note: See PRO-2014-0177

### Resource Type instance

Editor’s Note:WG5 will take a lead on this topic. ???

### Resource Type subscription

Editor’s Note:Jason(Huawei) will take a lead on this topic.

### Resource Type schedule

Editor’s Note:Phil(Cisco) will take a lead on this topic.

### Resource Type locationPolicy

      - Introduction

The <locationPolicy> resource represents the method for obtaining and managing geographical location information of an M2M Node. The detailed description can be found in the clause 9.6.10 in [6].

The resource specific attributes information is defined in the Annex A.

Table 7.3.2.1-49: Data Type Definition of locationPolicy

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Data Type definition should be posted after general agreement on technical issues

Table 7.3.2.1-50: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | O | O | N/A |
| Retrieve | O | O | N/A |
| Update | O | O | N/A |
| Delete | O | O | N/A |
| Notify | O | O | N/A |

Table 7.3.2.1-51: Data Types for resource specific attributes

| Attribute Name | Data Type |
| --- | --- |
| locationSource | m2m:locationSource |
| locationUpdatePeriod | xs:duration |
| locationTargetId | m2m:nodeId |
| locationServer | xs:anyURI |
| locationContainerID | xs:anyURI |
| locationContainerName | xs:string |

Table 7.3.2.1-52: Reference of child resources

| Child Resource Type Name | Data Type ID | Ref. in ArchTS |
| --- | --- | --- |
| subscription (variable) | TBD | 9.6.8 |

      - Operations

This sub-clause describes <locationPolicy> resource specific primitive behaviour for CRUD operations.

        - Create

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.1.

Receiver: the procedure of the Receiver written in the clause 7.2.1.2.1  (from Rcv-C-1.0 to Rcv-C-11.0) shall be the same as initial steps. The following steps are the <locationPolicy> resource type specific procedure for CREATE operation.

Rcv-C-LP-1.0	After the successful creation of <locationPolicy> resource, the Hosting CSE shall create <container> resource where the actual location information will be stored and the resource shall contain cross-references for the both resources, locationContainerID attribute for the <locationPolicy> resource and locationID attribute for the <container> resource. The name of the created <container> resource shall be determined by the locationContainerID attribute if it is applicable.

Rcv-C-LP-2.0	Check the locationSource and locationUpdatePeriod attributes:

- If the locationSource attribute is set by ‘Network Based’ and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the next step (Rcv-C-LP-3.0)

- If the locationSource attribute is set by ‘Device Based’ and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the next step (Rcv-C-LP-4.0)

- If the locationSource attribute is set by ‘Sharing Based’ and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the next step (Rcv-C-LP-5.0)

Rcv-C-LP-3.0	The Hosting CSE shall retrieve the locationTargetID and locationServer attributes from the stored <location Policy> resource.

In case either the locationTargetID or locationServer attribute cannot be obtained, the hosting CSE shall reject the request with the error code.

Editor’s Note: The ‘Error code’ should be posted after general agreement.

Then, the Hosting CSE shall transform the location-acquisition request into Location Server request [i.X], using the attributes stored in <locationPolicy> resource. The Hosting CSE shall also provide default values for other required parameters (e.g., quality of position) in the Location Server request according to local policies.

The Hosting CSE shall send this Location Server request to the location server using, for example, OMA Mobile Location Protocol [i.Y] and OMA RESTful NetAPI for Terminal Location [i.X]. The location server performs positioning procedure based upon the Location Server request. Then continue with the step Rcv-C-LP-6.0.

Based on the period information, locationUpdatePeriod attribute, this step can be periodically repeated or the location server can only notify the Hosting CSE of location information that performs periodically.

Note: the location server performs the privacy control and only responds successfully if the positioning procedure is permitted.

Note: the detail information on how the Location Server request message is converted into OMA RESTful NetAPI for Terminal Location message is described in Annex X.

Rcv-C-LP-4.0	The Hosting CSE shall perform positioning procedure using location determination modules and technologies (e.g., GPS). Then continue with the step Rcv-C-LP-6.0.

Based on the period information, locationUpdatePeriod attribute, this step can be periodically repeated.

Note: the Hosting CSE can utilize the internal interface (e.g., System Call) to communicate with the modules and technologies. The detail procedure is out-of-scope.

Rcv-C-LP-5.0	The Hosting CSE shall collect information of topology of M2M Area Network using <node> resource and find the closest Node from the Originator that has registered with the Hosting CSE and has location information. The closest Node is determined by the minimum hop based on the collected topology information.

- If the Hosting CSE can find the closest Node from the Originator, the location information of the closest Node shall be stored as the location information of the Originator into a <contentInstance> resource under the created <container> resource.

- If the Hosting CSE cannot find the closest Node from the Originator, the location information of the Hosting CSE shall be stored as the location information of the Originator into a <contentInstance> resource under the created <container> resource.

Rcv-C-LP-6.0	The Hosting CSE shall receive the corresponding response and transform it into a Response primitive.

- If the positioning procedure is failed, the Hosting CSE shall store a statusCode based on the error code in the locationStatus attribute in the created <locationPolicy> resource.

- If the positioning procedure is successfully complete which means that the Hosting CSE acquires the location information, The Hosting CSE shall store the acquired location information into a <contentInstance> resource under the created <container> resource.

Editor’s Note: The ‘Error code’ should be posted after general agreement.

        - Retrieve

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.2.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.2.

        - Update

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.3.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.3.

        - Delete

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.4.

Receiver: the procedure of the Receiver written in the clause 7.2.1.2.4 (from Rcv-D-1.0 to Rcv-D-10.0) shall be the same as initial steps. A following step is the <locationPolicy> resource type specific procedure for DELETE operation.

Rcv-D-LP-1.0	Once the <locationPolicy> resource is deleted, the Receiver shall delete the associated resources (e.g., <container>, <contentInstance> resources). If the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource has been set with appropriate value, the Receiver shall tear down the session. The specific mechanism used to tear down the session depends on the support of the Underlying Network and other factors.

### Resource Type delivery

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

### Resource Type request

#### Introduction

The <request> resource is used to represent information on locally issued requests (i.e. issued by an AE or CSE internal). This allows for robust synchronous and asynchronous request processing coping with various constraints on maximum blocking time. When an AE or CSE issues a request for targeting any other resource type or requesting a notification in non-blocking mode , i.e. the rt parameter of the request is set to either ‘nonBlockingRequestSynch’ or ‘nonBlockingRequestAsynch’, and if the Registrar CSE of the Originator supports the <request> resource type as indicated by the ‘supportedResourceType’ attribute of the <CSEBase> resource representing the Registrar CSE of the Originator, the Registrar CSE of the Originator shall create an instance of <request> to capture and expose the context of the associated non-blocking request. The detailed description can be found in clause 9.6.12 in Architecture TS.

Table 7.3.10.1-53: Data Type Definition of request

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-request-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.10.1-54 : Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | N/A | N/A | N/A |
| Retrieve | O | O | O |
| Update | N/A | N/A | N/A |
| Delete | O | O | O |

Table 7.3.10.1-55 : Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows operation |
| --- | --- | --- | --- |
| operation | m2m:operation | None |  |
| target | xs:anyURI | None |  |
| originator | xs:anyURI | None |  |
| requestIdentifier | m2m:requestId | None |  |
| metaInformation | m2m:metaInformation | None |  |
| content | m2m:content | None |  |
| requestStatus | m2m:requestStatus | None |  |
| operationResult | m2m:operationResult | None |  |

Table 7.3.10.1-56 : Reference of child resources

| Child Resource Type Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- |
| subscription (variable) | 0..n | Clause 7.3.6 |

#### Resource Specific Procedure on CRUD Operations

This sub-clause describes request resource specific procedure on Resource Hosting CSE  for CRUD operations.

##### Create

The <request> resource shall not be created via API.

The Receiver CSE of a non-blocking Request that was issued by either a Registrar AE of the Receiver CSE or a Registree/Registrar CSE of the Receiver CSE is the Hosing CSE for the <request> resource that shall be associated with the non-blocking request.

Hosting CSE:  The Hosting CSE shall execute the following steps in order.

Hst-C-Rq-1.0		Assign an identifier to the <request> resource to be created.

Hst-C-Rq-2.0		Assign a value to the following common attributes:

- parentID;

- creationTime;

- expirationTime: The Receiver shall assign a value that is consistent with the rqet, rc, rset and rp parameters effective for the associated non-blocking request that implied the creation of this <request> resource (within the restriction of the Receiver policies). If a value consistent with the rqet, rc, rset and rp parameters effective for the associated non-blocking request that implied the creation of this <request> resource cannot be supported, due to either policy or subscription restrictions, the Receiver will assign a new value.

- lastModifiedTime: which is equals to the creationTime;

- stateTag;

- accessControlPolicyIDs: Populate with one ID of an <accessControlPolicy> that  contains  the following:

  - In the ‘privileges’ child resource

    - Allow RUD operations to  <request> resource being created to the Hosting CSE

    - Allow RD operations to this <request> resource being created to the Originator of the associated non-blocking request, i.e. the value of the parameter fr in the associated non-blocking request that implied the creation of this <request> resource

  - In the 'selfPrivileges' child resource

    - Allow U operations the parent <accessControlPolicy> resource to the Originator of the associated non-blocking request, i.e. the value of the parameter fr in the associated non-blocking request that implied the creation of this <request> resource

Hst-C-Rq-3.0		Assign any other RO (Read Only) attributes of <request> resource type within the restriction of the Receiver policies:

- operation: Value of the parameter op in the associated non-blocking request that implied the creation of this <request> resource;

- target: Value of the parameter to in the associated non-blocking request that implied the creation of this <request> resource;

- originator: Value of the parameter fr in the associated non-blocking request that implied the creation of this <request> resource;

- requestIdentifier: Value of the parameter ri in the associated non-blocking request that implied the creation of this <request> resource;

- metaInformation: The content of this attribute is set to information in any other optional parameters described in clause 8.1. given in the associated non-blocking request that implied the creation of this <request> resource;

- content: Value of the parameter cn – if any – in the associated non-blocking request that implied the creation of this <request> resource;

- requestStatus: Information on the initial status of the associated non-blocking request that implied the creation of this <request> resource. The initial value of this attribute shall be identical to the status that is contained in the Acknowledgement response message of the associated non-blocking request. Possible values for status information contained in this attribute are specified in TS-0004. The value of this attribue is subject to changes according to the progress in processing of the non-blocking request that implied the creation of this <request> resource;

- operationResult: Initially Empty. This attribute will be used for respresenting the result of the originally requested operation – if any – in line with the rc parameter in the associated non-blocking request that implied the creation of this <request> resource.

Hst-C-Rq-4.0		The Hosting CSE shall create the <request> resource.

Receiver:  The Receiver shall execute the following steps in order.

Rcv-C-Rq-1.0		“Create an unsuccessful Response” with responseCode TBD (Method not allowed).

Rcv-C-Rq-2.0		“Send Response primitive”.

##### Retrieve

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.2.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.2.

##### Update

The <request> resource shall not be updated via API.

For a <request> resource explicit update requests shall not be supported. Changes in the attributes of a <request> resources can only be done by the Hosting CSE due to changes of the status of the associated non-blocking request that implied the creation of this <request> resource or due to reception of an operation result in response to the associated non-blocking request that implied the creation of this <request> resource.

Receiver: the Receiver shall execute the ollowing steps in order.

Rcv-U-Rq-1.0		“Create an unsuccessful Response” with responseCode TBD (Method not allowed).

Rcv-U-Rq-2.0		“Send Response primitive”.

##### Delete

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.4.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.4.

Editor’s Note: Need to define how to cancel request resource.

### Resource Type group

#### Introduction

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the membersList attribute. The <group> resource contains an attribute that represents the members of the group and a virtual resource (the <fanOutPoint>) that allows operations to be applied to the resources represented by those members. The detailed description can be found in clause 9.6.13 in Architecture TS.

Table 7.3.11.1-57: Data Type Definition of <group>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-group-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.11.1-58: Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows operation |
| --- | --- | --- | --- |
| memberType | m2m:memberType |  |  |
| currentNrOfMembers | xs:integer |  |  |
| maxNrOfMembers | xs:integer |  |  |
| membersList | m2m:anyURIList |  |  |
| membersAccessControlPolicyIDs | xs:anyURI |  |  |
| memberTypeValidated | xs:boolean |  |  |
| consistencyStrategy | m2m:consistencyStrategy |  |  |
| groupName | xs:string |  |  |

Editor’s Note: Default Value and Constraints need to be defined.

Table 7.3.11.1-59: Reference of child resources

| Child Resource Type Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- |
| <subscription> | 0..n | Error: Reference source not found |
| <fanOutPoint> | 1 | 7.3.x |

Editor’s Note: Ref. to <fanOutPoint> in TS-0004 needs to be specified.

#### Resource Specific Procedure on CRUD Operations

This sub-clause describes <group> resource specific procedure on Resource Hosting CSE for CRUD operations.

##### Create

Primitive specific operation after Rcv-C-7.0 "Check validity of resource representation for the given resource type” and before Rcv-C-8.0 "Create the resource". See clause 7.2.1.2.1.

- Primitive specific operation: Validate the provided attributes. It shall also check whether the number of URIs present in the membersList attribute of the group resource representation does not exceed the maximum as specified by the attribute maxNrOfMembers. If the maximum is exceeded, the request shall be rejected with a STATUS_NOT_ALLOWED.
If the memberType attribute of the <group> resource is not "MIXED", the hosting CSE shall also verify that all the member URIs including sub-groups in the attribute membersList of the group resource representation provided in the request shall conform to the memberType of the group resource.

- In the case that the <group> resource contains sub-group member resources, the receiver shall retrieve the memberType of the sub-group member resources to validate the memberType. If the memberType cannot be retrieved due to lack of privilege, the request shall be rejected with a STATUS_FORBIDDEN. If the sub-group member resources are temporarily unreachable, the receiver shall set the memberTypeValidated attribute of the <group> resource to FALSE and return the result to the originator in the response of the request. As soon as any unreachable sub-group resource becomes reachable, the receiver shall perform the memberType validation procedure. The originator may get to know the validation result by subscribe to the created resource if the memberTypeValidated attribute is FALSE. Upon unsuccessful validation, the receiver shall delete the <group> resource if the consistencyStrategy of the <group> resource is ABANDON_GROUP, or remove the inconsistent members from the <group> resource if the consistencyStrategy attribute is ABANDON_MEMBER, or set the memberType attribute of the <group> resource to "MIXED" if the consistencyStrategy attribute is MODIFY_TYPE.
The memberTypeValidated attribute shall be set to TRUE if all the members have been validated successfully.

##### Retrieve

No primitive specific operations

##### Update

Primitive specific operation after Rcv-U-7.0 "Check validity of resource representation” and before Rcv-U-8.0 ”Update the addressed resource”. See clause 7.2.1.2.3.

- Primitive specific operation: If the memberType attribute of the <group> resource is not "MIXED", the hosting CSE shall verify that all the member URIs including sub-groups in the attribute membersList of the group resource representation provided in the request shall conform to the memberType of the group resource.

- In the case that the <group> resource contains sub-group member resources, the receiver shall retrieve the memberType of the sub-group member resource to validate the memberType. If the memberType cannot be retrieved due to lack of privilege, the request shall be rejected with a STATUS_FORBIDDEN. If the sub-group member resources are temporarily unreachable, the receiver shall set the memberTypeValidated attribute of the <group> resource to FALSE and return the result to the originator in the response of the request. As soon as any unreachable sub-group resource becomes reachable, the receiver shall perform the memberType validation procedure. The originator may get to know the validation result by subscribe to the created resource if the memberTypeValidated attribute is FALSE. Upon unsuccessful validation, the receiver shall delete the <group> resource if the consistencyStrategy of the <group> resource is ABANDON_GROUP, or remove the inconsistent members from the <group> resource if the consistencyStrategy attribute is ABANDON_MEMBER, or set the memberType attribute of the <group> resource to "MIXED" if the consistencyStrategy attribute is MODIFY_TYPE.
The memberTypeValidated attribute shall be set to TRUE if all the members have been validated successfully.

- Primitive specific operation: The hosting CSE shall check whether the number of provided membersList in the attribute members exceeds the limitation of maxNrOfMembers. If it exceeds, the hosting CSE shall reject the request with STATUS_NOT_ALLOWED.

##### Delete

No primitive specific operations.

### Resource Type Members

Editor’s Note:Jason(Huawei) will take a lead on this topic.

### Resource Type mgmtObj

#### Introduction

The mgmtObj resource contains management data which represents individual M2M management functions. It represents a general structure to map to external management technology data models.

Table 7.3.13.1-60: Data Type Definition of <mgmtObj>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.13.1-61: Data Types for resource specific attributes

| Attribute Name | Data Type |
| --- | --- |
| mgmtDefinition | m2m:mgmtDefinition |
| objectID | xs:string |
| objectPath | xs:string |
| mgmtLink | xs:anyURI |
| [objectAttribute] | xs:string |
| Description | xs:string |

Table 7.3.13.1-62: Reference of child resources

| Child Resource Type Name | Data Type ID | Ref. in ArchTS |
| --- | --- | --- |
| <subscription> | xs:anyURI | 9.6.8 |

#### Resource Specific Procedure on CRUD Operations

- This sub-clause describes <mgmtObj> resource specific procedure on resource Hosting CSE for CRUD operations.

##### Create

Primitive specific operation before Orig-C-1.0:

- Primitive specific operation: If the originator is the managed entity, it shall generate the <mgmtObj> resource representation based on the external management object information of the managed entity to be exposed. The objectID and objectPath attribute may be set with the Request.

Primitive specific operation after Rcv-C-7.0 and before Rcv-C-8.0 if the originator is an IN-AE:

- "Identify the managed entity and the management protocol ".

Primitive specific operation: the receiver shall generate the external management object to be added to the managed entity based on the <mgmtObj> resource representation provided in the Request primitive. The receiver may determine the target location on the managed entity where the generated external management object shall be added based on the "objectID" and “objectPath” provided in the request primitive and the protocol specific data model being used. The receiver may also choose to let the managed entity decide the target location where the generated external management object shall be added using protocol specific mechanism.

- "Establish a management session with the managed entity".

- "Send the management request(s) to the managed entity corresponding to the received Request primitive ". If the receiver receives an error response from the managed entity because the external management object to be added already exists on the managed entity, the receiver shall check (by using e.g. OMA-DM Get command or TR069 GetParameterValues/GetParameterAttributes command) if the existing external management object is the same as the one to be added, then it shall consider the requested primitive as successfully performed instead of sending an unsuccessful Response primitive; otherwise, it shall reject the request with the StatusCode set to "STATUS_CONFLICT" in the Response primitive. The receiver shall also record the location where the external management object is added to the managed entity in the successful case. The objectID and objectPath attribute may be set with the Request.

- The receiver may repeat Step 4 in order to add to the managed entity the external management objects that are mapped from the mandatory sub-resources (including any descendents) that are required to be created automatically with the default attribute values.

##### Retrieve

Primitive specific operation after Rcv-R-7.0 and before Rcv-R-8.0 if the originator is an IN-AE:

- "Identify the managed entity and the management protocol ".

- "Locate the external management objects to be managed on the managed entity ".

- "Establish a management session with the managed entity".

- "Send the management request(s) to the managed entity corresponding to the received Request primitive". The receiver may also update the <mgmtObj> resource representation with the retrieved external management object information if required according to the local policy.

##### Update

The Update primitive is used for the update of the resource as well as the execution of a management procedure. The execution is performed using an Update primitive which without any content as the payload part of the primitive by addressing specific attribute to start the management procedure.

Primitive specific operation after Rcv-U-7.0 and before Rcv-U-8.0 if the originator is IN-AE.

- "Identify the managed entity and the management protocol ".

- "Locate the external management objects to be managed on the managed entity ".

- "Establish a management session with the managed entity".

- "Send the management request(s) to the managed entity corresponding to the received Request primitive". The receiver may also update the <mgmtObj> resource representation with the retrieved external management object information if required according to the local policy.

##### Delete

Primitive specific operation after Rcv-D-7.0 and before Rcv-D-8.0 if the originator is IN-AE.

- "Identify the managed entity and the management protocol ".

- "Locate the external management objects to be managed on the managed entity ".

- "Establish a management session with the managed entity".

- "Send the management request(s) to the managed entity corresponding to the received Request primitive". The receiver may also update the <mgmtObj> resource representation with the retrieved external management object information if required according to the local policy.

### Resource Type parameters

Editor’s Note:Jason(Huawei) will take a lead on this topic.

### Resource Type mgmtCmd

#### Introduction

The <mgmtCmd> resource shall contain the following attributes and child resource as illustrated in Table 7.3.15.1-64 and Table 7.3.15.1-65. The data type and default value of these attributes and child resources are included in the Tables.

Table 7.3.15.1-63: Data Type Definition of <mgmtCmd>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.15.1-64: Data Types for resource specific attributes

| Attribute Name | Data Type | Default | Value Restrictions | Notes |
| --- | --- | --- | --- | --- |
| description | xs:string | Not applicable | size: 256 |  |
| cmdType | m2m:cmdType | Not applicable | RESET, REBOOT, UPLOAD, DOWNLOAD, SOFTWAREINSTALL, SOFTWAREUNINSTALL |  |
| execReqArgs | m2m: execReqArgsListType | Not applicable | A list of entries which are dependent on execReqArgsListType. If cmdType=Reset, execReqArgsListType=ResetArgsType. If cmdType=REBOOT, execReqArgsListType=RebootArgsType. If cmdType=UPLOAD, execReqArgsListType=UploadArgsType. If cmdType=DOWNLOAD, execReqArgsListType=DownloadArgsType. If cmdType=SOFTWAREINSTALL, execReqArgsListType=SoftwareInstallArgsType. If cmdType= SOFTWAREUNINSTALL, execReqArgsListType=SoftwareUninstallArgsType. For different execReqArgsListType, the list of entries is different which are described in corresponding .xsd file. |  |
| execEnable | xs: anyURI | Not applicable |  |  |
| execTarget | m2m:nodeID | Not applicable |  |  |
| execMode | m2m:execModeType | IMMEDIATEONCE | IMMEDIATEONCE, IMMEDIATEREPEAT, RANDOMONCE, RANDOMREPEAT |  |
| execFrequency | xs:duration | Not applicable |  |  |
| execDelay | xs:duration | 0 |  |  |
| execNumber | xs: nonNegativeInteger | 1 |  |  |

Editor’s Note: It is FFS for how to use execTarget for group management.

Table 7.3.15.1-65: Reference of child resources

| Child Resource Type Name | Data Type ID | Ref. in ArchTS |
| --- | --- | --- |
| <execInstance> | xs:anyURI | 9.6.17 |

The <mgmtCmd> shall be executed for the following modes:

- If execMode is IMMEDIATEONCE, <mgmtCmd> shall be executed immediately and only once. In this mode, execFrequency, execDelay, and execNumber shall not be used.

- If execMode is IMMEDATEREPEAT, <mgmtCmd> shall be executed immediately and repeated multiple times as determined by execNumber and the time interval between each execution is specified by execFrequency. In this mode, execDelay shall not be used.

- If execMode is RANDONONCE, <mgmtCmd> shall be executed only once at a delayed time which is specified by execDelay. In this mode, execFrequency and execNumber shall not be used.

- If execMode is RANDOMREPEAT, <mgmtCmd> shall be executed multiple times as specified by execNumber but the first execution shall be executed at a delayed time. execDelay specifies the delayed time. The time interval between each execution is specified by execFrequency.

#### Resource Specific Procedures

This sub-clause describes <mgmtCmd> resource specific procedures for CRUD operations.

##### Create

This procedure shall use the Create common operations detailed in section 7.2.1.2.1 without primitive specific actions. The Originator shall use the steps Orig-C-1.0, Orig-C-2.0, and Orig-C-3.0 as described in Section 7.2.1.2.1. The Receiver shall use the steps Rcv-C-1.0 to Rcv-C-11.0 as described in Section 7.2.1.2.1.

The Originator shall provide the <mgmtCmd> resource representation to the Receiver (e.g. IN-CSE). The Receiver may generate one of the following status codes and send it to the Originator.

If the Originator provides an invalid cmdType value in the Create primitive, the Receiver shall generate the status code STATUS_INVALID_CMDTYPE.

If the name/value entry in execReqArgs does not match the value of cmdType in the Create primitive, the Receiver shall generate the status code STATUS_ INCORRECT _ARGUMENTS.

If name/value entries in execReqArgs do not contain mandatory arguments as required by cmdType, the Receiver shall generate the status code STATUS_INSUFFICIENT_ARGUMENTS

##### Retrieve

This procedure shall use the Retrieve common operations detailed in section 7.2.1.2.2 without primitive specific actions. The Originator shall use the steps Orig-R-1.0, Orig-R-2.0, and Orig-R-3.0 as described in Section 7.2.1.2.2. The Receiver shall use the steps Rcv-R-1.0 to Rcv-R-9.0 as described in Section 7.2.1.2.2.

##### Update

###### Update (Normal)

If the Update primitive does not address the execEnable attribute of the <mgmtCmd> or the URI provided as the value of the execEnable , it results in update of all or part of the information of an existing <mgmtCmd> resource with the new attribute values. The procedure uses the common Update operations detailed in section 7.2.1.2.3, without primitive specific actions. The Originator shall use the steps Orig-U-1.0, Orig-U-2.0, and Orig-U-3.0 as described in Section 7.2.1.2.3. The Receiver shall use the steps Rcv-U-1.0 to Rcv-U-11.0 as described in Section 7.2.1.2.3. This normal Update operation shall only be applied to attributes execReqArgs, execTarget, execMode, execFrequency, execDelay, and execNumber, if <mgmtCmd> has no any child resource <execInstance> created yet.

If the Originator attempts to update any attribute other than execReqArgs, execTarget, execMode, execFrequency, execDelay, and execNumber, the Receiver shall generate the status code STATUS_REQUEST_DENIED.

If the Originator attempts to update attributes execReqArgs, execTarget, execMode, execFrequency, execDelay, and/or execNumber, but the <mgmtCmd> has child resource <execInstance> already created, the Receiver shall generate the status code STATUS_REQUEST_DENIED.

If the Originator attempts to update attributes execReqArgs, execTarget, execMode, execFrequency, execDelay, and/or execNumber, and the <mgmtCmd> does not have any child resource <execInstance>  but gives a not allowed value, the Receiver shall generate the status code STATUS_WRONG_VALUE.

If the Update primitive for <mgmtCmd> does address the execEnable attribute of the <mgmtCmd>, it effectively triggers an Execute <mgmtCmd> procedure, see section 7.3.15.2.3.2.

###### Update (Execute)

The execute operation is triggered by an Update primitive, if the primitive addresses the execEnable attribute of the <mgmtCmd> or the URI provided as the value of the execEnable. The procedure uses the Update common operations detailed in section 7.2.1.2.3 with the following primitive specific operation after Rcv-U-4.0 and before Rcv-U-5.0:

- The Receiver shall identify the managed entity and the management protocol. The parentID or execTarget attribute of <mgmtCmd> indicates the managed entity.

The Receiver shall automatically create an <execInstance> based on the <mgmtCmd> resource. The Receiver shall copy the following attributes from <mgmtCmd> to <execInstance>: execTarget, execMode, execFrequency, execDelay, execNumber, and execReqArgs. The execStatus of <execInstance> is set as INITIATED.

The Receiver shall determine if the <mgmtCmd> shall be executed immediately or postponed according to the combination of execMode, execFrequency, execDelay, and execNumber. If the <mgmtCmd> shall be executed immediately (e.g. execMode is IMMEDIATEONCE), the following steps shall be performed; otherwise the following steps shall be postponed and skipped until the delay is expired (e.g. as indicated by execDelay).

The Receiver shall establish a management session with the identified managed entity.

The Receiver shall perform management command conversion and execution and set the execStatus attribute of <execInstance> to STARTED. If the Receiver cannot perform the command conversion successfully (e.g. execReqArgs does not have sufficient name/value pairs), the Receiver shall generate status code STATUS_CONVERSION_FAILURE.

After receiving completion response from the managed entity, the Receiver shall set execStatus attribute of corresponding <execInstance> to FINISHED.

If the Update primitive for <mgmtCmd> does not address the execEnable attribute of the <mgmtCmd>, it effectively triggers an Update <mgmtCmd> procedure, see section 7.3.15.2.3.1.

##### Delete

This procedure uses the Delete common operations detailed in section 7.2.1.2.4 with the following primitive specific operation after Rcv-D-3.0 and before Rcv-D-4.0:

If the Originator is IN-AE and the requested <mgmtCmd> resource includes pending management commands previously initiated on the managed entity (i.e. <mgmtCmd> resource has instantiated <execInstance> child resources and their execStatus is INITIATED or STARTED) and the pending command is cancellable, the Receiver shall perform the following steps, otherwise it shall continue to step Rcv-D-4.0.

- It shall identify the managed entity associated with each <execInstance> child resource which execStatus is INITIATED or STARTED and the management protocol. The execTarget attribute of the pending management command resource (i.e. <execInstance>) indicates the managed entity.

- It shall establish a management session with the identified managed entity.

It shall perform management command conversion and execution resulting in cancellation of the pending management commands which were previously initiated on the managed entity. The Receiver shall convert the received Delete primitive to corresponding management operation for cancellation and execute it on the managed entity.

The managed entity may report the following status codes to the Receiver: STATUS_REQUEST_UNSUPPORTED (cancellation operation is not supported at the managed entity), STATUS_REQUEST_DENIED (cancellation request as a general request is denied at the managed entity), STATUS_CANCEL_DENIED (cancellation request is not permitted at the managed entity in its current transfer state)._

- The Receiver shall delete the addressed resource and send the received status codes in the response to the Originator.

### Resource Type execInstance

#### Introduction

The <execInstance> resource shall contain the following child resource and attributes.

Table 7.3.16.1-66: Data Type Definition of <execInstance>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.16.1-67: Data Types for resource specific attributes

| Attribute Name | Data Type | Default | Value Restrictions | Notes |
| --- | --- | --- | --- | --- |
| execStatus | m2m:execStatusType | INITIATED | INITATED, STARTED, FINISHED, CANCELLING, CANCELLED |  |
| execResult | xs:execResultType | Not applicable |  |  |
| execDisable | xs:anyURI | Not applicable |  |  |
| execTarget | m2m:nodeID | Not applicable |  |  |
| execMode | m2m:execModeType | IMMEDIATEONCE | IMMEDIATEONCE, IMMEDIATEREPEAT, RANDOMONCE, RANDOMREPEAT |  |
| execFrequency | xs:duration | Not applicable |  |  |
| execDelay | xs:duration | 0 |  |  |
| execNumber | xs:nonNegativeInteger | 1 |  |  |
| execReqArgs | m2m:execReqArgsListType | Not applicable |  |  |

#### Resource Specific Procedures

This sub-clause describes <execInstance> resource specific procedures for CRUD operations.

##### Update (Cancel)

The <execInstance> Cancel operation is triggered by an Update primitive, if the primitive addresses the execDisable attribute or the URI provided as the value of the execDisable. The procedure uses the Update common operations detailed in section 7.2.1.2.3, with the following primitive specific operation after Rcv-U-4.0 and before Rcv-U-5.0:

- The Receiver of this primitive shall identify the managed entity and the management protocol. The <execTarget> attribute indicates the managed entity.

- The Receiver shall check the cmdType attribute of <execInstance>’s parent resource (i.e. <mgmtCmd>) and the execStatus attribute of <execInstance> to determine if the management operation associated with the <execInstance> are pending (i.e. only if its execStatus attribute is INITIATED or STARTED) and/or cancellable. If the management operation is pending or not cancellable, the following steps shall be skipped.

  - If the management operation associated with the <execInstance> is pending, the Receiver shall return the status code STATUS_CANCEL_DENIED.

  - If the management operation associated with the <execInstance> is not cancellable, the Receiver shall return the status code STATUS_NON_CANCELLABLE.

- If the management operation is cancellable, the Receiver shall determine the management operations resulting in cancellation of the pending management commands which were previously initiated on the managed entity. The Receiver shall convert the received Update primitive to the corresponding command for cancellation and execute it on the managed entity. The execStatus attribute of <execInstance> is set as CANCELLING.

- The Receiver shall establish a management session with the managed  entity.

- The Receiver shall perform the corresponding command for cancelling the pending management commands as denoted by the cmdType attribute on the managed entity.

The managed entity may report the following status codes to the Receiver: STATUS_REQUEST_UNSUPPORTED (cancellation request is not supported at the managed entity), STATUS_REQUEST_DENIED (cancellation request as a general request is denied at the managed entity), STATUS_CANCEL_DENIED (cancellation request is not permitted at the managed entity in its current transfer state).

- If the cancelling is successfully executed on the managed entity, the Receiver shall set execStatus of <execInstance> as CANCELLED.

##### Retrieve

This procedure shall use the Retrieve common operations detailed in section 7.2.1.2.2, without primitive specific actions. The Originator shall use the steps Orig-R-1.0, Orig-R-2.0, and Orig-R-3.0 as described in Section 7.2.1.2. The Receiver shall use the steps Rcv-R-1.0 to Rcv-R-9.0 as described in Section 7.2.1.2.2.

##### Delete

This procedure uses the Delete common operations detailed in section 7.2.1.2.4 with the following primitive specific operation after Rcv-D-4.0 and before Rcv-D-5.0:

If the Originator is IN-AE and the addressed<execInstance> resource initiated management commands is still pending on the remote entity and the pending command is cancellable, the Receiver shall perform the following steps, otherwise it shall continue to step Rcv-D-5.

- The Receiver shall identify the managed entity and the management protocol. The execTarget attribute indicates the managed entity.

- The Receiver shall establish a management session with the managed entity.

The Receiver shall perform management command conversion and execution resulting in cancellation of the pending and cancellable management commands which were previously initiated on the managed entity.

The managed entity may report the following status codes to the Receiver: STATUS_REQUEST_UNSUPPORTED (cancellation REQUEST is not supported at the managed entity), STATUS_REQUEST_DENIED (cancellation request as a general request is denied at the managed entity), STATUS_CANCEL_DENIED (cancellation request is not permitted at the managed entity in its current transfer state).

- It shall delete the addressed <execInstance> resource and send the received status codes in the response to the Originator.

### Resource Type node

#### Introduction

The <node> resource represents specific information that provides properties of an oneM2M Node that can be utilized by other oneM2M operations. The <node> resource has <mgmtObj> as its child resources.

Table 7.3.17.1-68: Data Type Definition of <node>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.17.1-69: Applicability of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create |  |  |  |
| Retrieve |  |  |  |
| Update |  |  |  |
| Delete |  |  |  |

Editor’s Note: Place actual information in this table

Table 7.3.17.1-70: Data Types for resource specific attributes

| Attribute Name | Data Type |
| --- | --- |
| nodeID | xs:string |
| hostedCSEID | xs:anyURI |

Table 7.3.17.1-71: Reference of child resources

| Child Resource Type Name | Data Type ID | Ref. in ArchTS |
| --- | --- | --- |
| <mgmtObj> | xs:anyURI | 9.6.15 |
| <subscription> | xs:anyURI | 9.6.8 |

#### Resource Specific Procedure on CRUD Operations

This sub-clause describes <node> resource specific procedure on Resource Hosting CSE for CRUD operations.

##### Create

No primitive specific operations

##### Retrieve

No primitive specific operations

##### Update

No primitive specific operations

##### Delete

No primitive specific operations.

### Resource Type m2mServiceSubscription

Editor’s Note:Seongyoon(LGE) will take a lead on this topic.

### Resource Type nodeInfo

### Resource Type pollingChannel

Editor’s Note: The structure change of pollingChannel resource type needs to be incorporated in the TS-0001 to reflect the decision on the ARC WG, however the corresponding contribution (ARC-2014-1440) needs time to be agreed for the time being. This contribution assumes that agreement in advance. Therefore, incorporating clause 7.3.20 and 7.3.21 needs to be confirmed once the ARC WG agrees ARC-2014-1440.

#### Introduction

The <pollingChannel> resource is used to perform service layer long polling when an AE/CSE cannot receive a request from other entities, however it can get a request as a response to a long polling request. Actual long polling can be performed on the <pollingChannelURI> resource which is the child resource of the <pollingChannel> resource.

The detailed description can be found in clause 9.6.21 in TS-0001[6].

Table 7.3.20.1-72: Data Type Definition of pollingChannel

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Data Type definition should be posted after general agreement on technical issues.

Table 7.3.20.1-73: Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M/O/C | M/O/C | M/O/C |
| Retrieve | M/O/C | M/O/C | M/O/C |
| Update | M/O/C | M/O/C | M/O/C |
| Delete | M/O/C | M/O/C | M/O/C |
| Notify | N/A | N/A | N/A |

Editor’s Note: Yellow highlight is TBD

Table 7.3.20.1-74: Reference of child resources

| Child Resource Type Name | Data Type ID | Ref. in ArchTS |
| --- | --- | --- |
| longPollingURI | TBD | 9.6.21 |

#### Operations

This sub-clause describes <pollingChannel> resource specific behaviour for CRUD operations.

##### Create

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.1.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.1.

##### Retrieve

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.2.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.2.

##### Update

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.3.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.3.

##### Delete

Originator: the procedure of the Originator is the same as the clause 7.2.1.2.4.

Receiver: the procedure of the Receiver is the same as the clause 7.2.1.2.4.

### Resource Type pollingChannelURI

#### Introduction

The <pollingChannelURI> resource is the virtual child resource which is automatically generated during the parent <pollingChannel> resource creation.

The detailed description can be found in clause 9.6.xx in TS-0001[6].

Table 7.3.21.1-75: Data Type Definition of pollingChannel

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Editor’s Note: Data Type definition should be posted after general agreement on technical issues

Table 7.3.21.1-76:Optionality of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | N/A | N/A | N/A |
| Retrieve | M/O/C | M/O/C | M/O/C |
| Update | N/A | N/A | N/A |
| Delete | N/A | N/A | N/A |
| Notify | N/A | N/A | N/A |

Editor’s Note: Yellow highlight is TBD

#### Operations

This sub-clause describes <pollingChannelURI> resource specific behaviour for the Retrieve operation as a service layer long polling request. CUDN requests to the <pollingChannelURI> resource shall be rejected.

##### Create

This specification does not define Create operation on a <pollingChannelURI> resource. A Create request for the resource shall be rejected.

A <pollingChannelURI> virtual resource shall only be created during its parent <pollingChannel> resource creation procedure.

##### Retrieve

Originator: shall execute Originator actions in clause 7.2.1.2.2 as a service layer long polling request. It’s the Originator’s responsibility to initiate this procedure after it gets long polling response either successful or unsuccessful. The Originator shall send this Retrieve request as blocking request (clause 8.2.1 in [6]).

Receiver: shall execute the following steps in order:

Rcv-PCU-1.0	Check if the request Originator is the creator of the parent <pollingChannel> resource.

If the Originator is the creator,

If there is a pending request(s) to be sent to the Originator

Rcv-PCU-2.0	Create a Response primitive including Pending Requests primitive parameter.

Else

Rcv-PCU-2.1	Wait for a request for the Originator until the rqet of the Originator’s request. If a request is available before the rqet timeout, then execute Rcv-PCU-2.0. Otherwise, create an unsuccessful Response. Error information is request timeout.

Else

Rcv-PCU-2.3	Create an unsuccessful Response. Error information is access denied.

Rcv-PCU-3.0	Send the Response primitive.

##### Update

This specification does not define Update operation on a <pollingChannelURI> resource. An Update request for the resource shall be rejected.

##### Delete

This specification does not define Delete operation on a <pollingChannelURI> resource. A Delete request for the resource shall be rejected.

### Resource Type cmdhPolicy

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhDefaults

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhDefEcValue

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhEcDefParamValues

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhLimits

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhNetworkAccessRules

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhNwAccessRule

Editor’s Note:Nobu(Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type cmdhBuffer

Editor’s Note:Nobu (Qualcomm) will take a lead on this topic.

Editor’s Note: See PRO-2014-0227

### Resource Type statsConfig

#### Introduction

The <statsConfig> resource is used to store configuration data for collecting statistics for AEs. The <eventConfig> child resource is a mechanism for defining events that trigger statistics collection activity. Additional description of the <statsConfig> resource is contained in clauses 9.6.22 and 10.2.15 of oneM2M TS-0001 [6].

Table  7.3.30.1-77: Data Type definition of <statsConfig>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-statsConfig-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table  7.3.30.1-78: Optionality of operations by reference point for <statsConfig>

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | N/A | N/A |
| Retrieve | M | N/A | N/A |
| Update | M | N/A | N/A |
| Delete | M | N/A | N/A |

Table  7.3.30.1-79: Data Types for resource specific attributes of <statsConfig>

| Attribute Name | Data Type | Default Value and Constraints | Entity which allows operation |
| --- | --- | --- | --- |
| None |  |  |  |

Table  7.3.30.1-80: Reference of child resources of <statsConfig>

| Child Resource Type Name | Data Type ID | Ref. to in Resource Type Definition |
| --- | --- | --- |
| <eventConfig> | list of xs:anyURI | 7.3.30 |
| <subscription> | list of xs:anyURI | 7.3.6 |

Editor’s note: How to represent multiplicity need to clarify.

#### <statsConfig> resource-specific procedure on CRUD operations

##### Create

This procedure follows the Generic Resource Create Request Procedure specified in clause 7.2.1.2.1 with the following <statsConfig> resource-specific updates.

Resource-specific operation before Rcv-C-5.0:

- If the To primitive parameter addresses a receiver CSE that is not an IN-CSE, then the request shall be rejected with a “STATUS_BAD_REQUEST”.

##### Retrieve

This procedure follows the Generic Resource Retrieve Request Procedure specified in clause 7.2.1.2.2.

##### Update

This procedure follows the Generic Resource Update Request Procedure specified in clause 7.2.1.2.3.

##### Delete

This procedure follows the Generic Resource Delete Request Procedure specified in clause 7.2.1.2.4.

### Resource Type eventConfig

#### Introduction

The <eventConfig> resource defines events that trigger statistics collection activity on an IN-CSE. Additional description of the <eventConfig> resource is contained in clauses 9.6.23 and 10.2.15 of oneM2M TS-0001 [6].

Table 7.3.31.1-81: Data Type definition of <eventConfig>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-eventConfig-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table  7.3.31.1-82: Optionality of operations by reference point for <eventConfig>

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | N/A | N/A |
| Retrieve | M | N/A | N/A |
| Update | M | N/A | N/A |
| Delete | M | N/A | N/A |

Table  7.3.31.1-83: Resource-specific attributes of <eventConfig>

| Attribute Name | Presence in CREATE request | Presence in UPDATE request | Presence in Response | Description |
| --- | --- | --- | --- | --- |
| eventID | O | NP | M | see Table A-1 |
| eventType | M | M | M | see Table A-1 |
| eventStart | O | O | O | see Table A-1 |
| eventEnd | O | O | O | see Table A-1 |
| transactionType | O | O | O | see Table A-1 |
| dataSize | O | O | O | see Table A-1 |

Editor’s Note: Optionality of Attributes should be referred in Arch-TS or self-contained?

Table  7.3.31.1-84: Reference of child resources of <eventConfig>

| Child Resource Type Name | Data Type ID | Ref. to in Resource Type Definition |
| --- | --- | --- |
| <subscription> | list of xs:anyURI | 7.3.6 |

Editor’s note: How to represent multiplicity need to clarify.

#### <eventConfig> resource-specific procedure on CRUD operations

##### Create

This procedure follows the Generic Resource Create Request Procedure specified in clause 7.2.1.2.1, with the following <eventConfig> resource-specific updates.

Resource-specific operation before Orig-C-1.0 “Compose Request primitive”:

- If event-based statistics collection will be used, the Originator shall generate the representation of the <eventConfig> child resource instance to produce the desired trigger condition for the intended event. For example, one representation of <eventConfig> could have eventType set to “DATA OPERATION” and transactionType set to “RETRIEVE”. In another example, a representation could have eventType set to “TIMER-BASED”, eventStart set to midnight tomorrow and eventEnd set to midnight of the day after tomorrow. See Table A-1 for value restrictions and default settings pertaining to the attributes of <eventConfig>.

##### Retrieve

This procedure follows the Generic Resource Retrieve Request Procedure specified in clause 7.2.1.2.2.

##### Update

This procedure follows the Generic Resource Update Request Procedure specified in clause 7.2.1.2.3.

        - Delete

This procedure follows the Generic Resource Delete Request Procedure specified in clause 7.2.1.2.4.

### Resource Type statsCollect

#### Introduction

The <statsCollect> resource controls the collection of statistics information on an IN-CSE. Information in an associated <eventConfig> resource shall be used by the IN-CSE or IN-AE to define specific event-related triggers. Additional description of the <statsCollect> resource is contained in clauses 9.6.24 and 10.2.15 of oneM2M TS-0001 [6].

Table 7.3.32.1-85: Data Type definition of <statsCollect>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-statsCollect-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.32.1-86: Optionality of operations by reference point for <statsCollect>

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | M | N/A | N/A |
| Retrieve | M | N/A | N/A |
| Update | M | N/A | N/A |
| Delete | M | N/A | N/A |

Table 7.3.32.1-87: Resource-specific attributes of <statsCollect>

| Attribute Name | Presence in CREATE request | Presence in UPDATE request | Presence in Response | Description |
| --- | --- | --- | --- | --- |
| statsCollectID | NP | NP | M | see Table A-1 |
| collectingEntityID | M | NP | M | see Table A-1 |
| collectedEntityID | M | NP | M | see Table A-1 |
| status | M | O | M | see Table A-1 |
| statModel | M | O | M | see Table A-1 |
| subscriberID | M | NP | M | see Table A-1 |
| collectPeriod | O | O | O | see Table A-1 |
| eventID | O | O | O | see Table A-1 |

Table 7.3.32.1-88: Reference of child resources of <statsCollect>

| Child Resource Type Name | Data Type ID | Ref. to in Resource Type Definition |
| --- | --- | --- |
| <subscription> | list of xs:anyURI | 7.3.6 |

Editor’s note: How to represent multiplicity need to clarify.

#### <statsCollect> resource-specific procedure on CRUD operations

##### Create

This procedure follows the Generic Resource Create Request Procedure specified in clause 7.2.1.2.1, with the following <statsCollect> resource-specific updates.

Resource-specific operation before Orig-C-1.0:

- The Originator shall generate and populate a representation of the <statsCollect> resource to produce the desired collection scenario, with the exception of statsCollectID (which is populated by the IN-CSE). If statModel is set to “EVENT-BASED” then the Originator shall provide a value for eventID that corresponds to an eventID value stored in a <eventConfig> resource (which defines the event triggers to be used). See Table A-1 for value restrictions and default settings pertaining to the attributes of <statsCollect>.

Resource-specific operation before Rcv-C-5.0:

- If the To primitive parameter addresses a receiver CSE that is not an IN-CSE, then the request shall be rejected with a “STATUS_BAD_REQUEST”.

Resource-specific operation before Rcv-C-9.0 and after Rcv-C-8.0:

- The receiver IN-CSE shall generate and store a unique (within the Service Provider domain) value for statsCollectID.

- If the status attribute is set to “ACTIVE”, the IN-CSE shall begin monitoring the conditions defined by the <statsCollect> resource and generating Service Statistics Collection Records as the conditions are met.

##### Retrieve

This procedure follows the Generic Resource Retrieve Request Procedure specified in clause 7.2.1.2.2.

##### Update

This procedure follows the Generic Resource Update Request Procedure specified in clause 7.2.1.2.3.

Resource-specific operation before Rcv-U-9.0 and after Rcv-U-8.0:

- If the status attribute is set to “ACTIVE”, the IN-CSE shall begin monitoring the conditions defined by the <statsCollect> resource and generating Service Statistics Collection Records as the conditions are met.

- If the status attribute is set to “INACTIVE”, the IN-CSE shall stop monitoring the conditions defined by the <statsCollect> resource.

##### Delete

This procedure follows the Generic Resource Delete Request Procedure specified in clause 7.2.1.2.4.

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex A (Normative): Resource attributes

Resource Attributes are specified in oneM2M TS-0001 [6]. The type and values shall be supported according to the description given in table A-1.

The attributes are specified by trhe following information in the table:

- ResourceType: indicates the resource where the attribute is used, in case that the attribute is present in all resources the tag “ALL” is used.

- Attribute Name: indicates the name of the Attribute from [6]

- Short Name: indicates the acronym for the correspondent Attribute

- Data type: indicate the used type for the attribute, simple data types are defined in clause 6.3.1 and complex data types are defined in clause 6.3.2.

- Default: specifies the default value of the attribute as set by hosting CSE if no specific value was provided in the operation CREATE or UPDATE request, or if the provided value was unacceptable for the hosting CSE and is not specified to respond with an error then the hosting CSE is allowed to set a value.

- Value restrictions: indicates if the value of the attributes has a specific limitation.

Table A-1: Resource attributes

| ResourceType | Attribute Name | Short Name | Data Type | Default | Value restrictions | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ALL except <accessControlPolicy> | accessControlPolicyID | aRI | xs:string | NONE | See TBD | If the attribute is absent, all the entities that correspond to ancestor resources shall have the full set of permissions |
| firmware | activate |  |  |  |  |  |
| firmware | activateStatus |  |  |  |  |  |
| <subscription> | aggregationURI |  | xs:anyURI |  |  |  |
| TBD | announceAttribute | aA | xs:string | NONE |  |  |
| TBD | announceTo | aT | xs:anyURI | NONE |  |  |
| <application>, <m2mServiceSubscription>, <nodeInfo> | App-ID |  | xs:string |  |  |  |
| <application> | App-Inst-ID |  | xs:string |  |  |  |
| <cmdhEcDefParamValues>, <cmdhNetworkAccessRules>, <cmdhBuffer> | applicableEventCategory | aEC |  |  |  |  |
| <areaNwkDeviceInfo> | areaNwkId |  |  |  |  |  |
| <areaNwInfo> | areaNwkType |  |  |  |  |  |
| <capabilityInstance> | attached |  |  |  |  |  |
| <cmdhNwAccessRule> | backOffParameters | bOP |  |  |  |  |
| <subscription> | batchNotify |  |  |  |  |  |
| <battery> | batteryLevel |  |  |  |  |  |
| <battery> | batteryStatus |  |  |  |  |  |
| <capabilityInstance> | capabilityActionStatus |  |  |  |  |  |
| <capabilityInstance> | capabilityName |  |  |  |  |  |
| <pollingChannel> | channelHandle |  |  |  |  |  |
| <mgmtCmd> | cmdType |  |  |  |  |  |
| <statsCollect> | collectedEntityID |  |  |  |  |  |
| <statsCollect> | collectingEntityID |  |  |  |  |  |
| <statsCollect> | collectPeriod |  |  |  |  |  |
| <group> | consistencyStrategy | cS |  |  |  |  |
| <instance> | content | c | Content | NONE |  |  |
| <request> | content | ? | ? |  |  |  |
| <instance> | contentSize | cS | xs:long |  |  | Set by the CSE to the actual size of the received content of the instance |
| ALL | creationTime | cT | xs:dateTime |  |  | Generated by the hosting SCE. The value is set to the actual time of creation of the resource. |
| TBD | creator | cr | xs:anyURI |  |  | Generated by the hosting |
| <remoteCSE> | cseBase | bRt | xs:anyURI |  |  |  |
| <CSEBase>, <remoteCSE>, <nodeInfo> | CSE-ID |  | xs:string |  |  |  |
| <CSEBase> and <remoteCSE> | cseType | csT | TBD |  |  |  |
| <container> | currentByteSize | nb | xs:long |  | Limited by maxByteSize attribute of the same container resource | Set to the actual number of bytes of data stored in the Container resource |
| <container> | currentNrOfInstances | ni | xs:long |  | Limited by maxNrOfInstances attribute of the same container resource | Set to the actual number of instances resource in the Container |
| <group> | currentNrOfMembers | nM | xs:long |  |  |  |
| <delivery> | data |  |  |  |  |  |
| <eventConfig> | dataSize |  |  |  |  |  |
| firmware | deactivate |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultDelAggregation |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultOpExecTime |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultRequestExpTime | dRqET |  |  |  |  |
| <cmdhEcDefParamValues> | defaultRespPersistence |  |  |  |  |  |
| <cmdhEcDefParamValues> | defaultResultExpTime | dRsET |  |  |  |  |
| <cmdhDefEcValue> | defEcValue | dev |  |  |  |  |
| <delivery> | deliveryMetaData |  |  |  |  |  |
| <mgmtObj>, <parameters>, <mgmtCmd>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | description |  | xs:string |  |  |  |
| <deviceInfo> | deviceLabel |  |  |  |  |  |
| <deviceInfo> | deviceType |  |  |  |  |  |
| <areaNwkDeviceInfo> | devId |  |  |  |  |  |
| <areaNwkDeviceInfo> | devType |  |  |  |  |  |
| <capabilityInstance> | disable |  |  |  |  |  |
| <capabilityInstance> | enable |  |  |  |  |  |
| <delivery> | eventCat |  |  |  |  |  |
| <eventConfig> | eventEnd |  | xs:dateTime |  |  |  |
| <eventConfig>, <statsCollect> | eventID |  |  |  |  |  |
| <eventConfig> | eventStart |  | xs:dateTime |  |  |  |
| <eventConfig> | eventType |  |  |  |  |  |
| <execInstance> | execDelay |  |  |  |  |  |
| <execInstance> | execDisable |  |  |  |  |  |
| <mgmtCmd> | execEnable |  |  |  |  |  |
| <execInstance> | execFrequency |  |  |  |  |  |
| <execInstance> | execMode |  |  |  |  |  |
| <execInstance> | execNumber |  |  |  |  |  |
| <mgmtCmd>, <execInstance> | execReqArgs |  |  |  |  |  |
| <execInstance> | execResult |  |  |  |  |  |
| <execInstance> | execStatus |  |  |  |  |  |
| <execInstance> | execTarget |  |  |  |  |  |
| <subscription> | expirationCounter |  |  |  |  |  |
| ALL, except <CSEBase>, <instance> and <parameters> | expirationTime | eT | xs:dateTime |  |  | The value may be determined by CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <reboot> | factoryReset |  |  |  |  |  |
| <subscription> | filterCriteria |  | FilterCriteria |  |  |  |
| <deviceInfo> | fwVersion |  |  |  |  |  |
| <group> | groupName | gN | xs:string | NONE |  |  |
| <node> | hostedCSEID |  | Link |  |  |  |
| <deviceInfo> | hwVersion |  |  |  |  |  |
| firmware | install |  |  |  |  |  |
| firmware | installStatus |  |  |  |  |  |
| <subscription> | interimEventNotify |  |  |  |  |  |
| ALL | labels | lBs | xs:string | NONE |  |  |
| ALL | lastModifiedTime | lMT | xs:dateTime | NONE |  |  |
| <container> | latest | lt | xs:anyURI |  | Limited to the URI of the instances resources in the container | Set by the CSE to the URI of the latest added instance resource to the container |
| <subscription> | latestNotify |  |  |  |  |  |
| <delivery> | lifespan |  |  |  |  |  |
| <cmdhLimits> | limitsDelAggregation | lDA |  |  |  |  |
| <cmdhLimits> | limitsOpExecTime | lOET |  |  |  |  |
| <cmdhLimits> | limitsRequestExpTime | lRqET |  |  |  |  |
| <cmdhLimits> | limitsRespPersistence | lRP |  |  |  |  |
| <cmdhLimits> | limitsResultExpTime | lRsET |  |  |  |  |
| TBD | link | ln | Link | NONE |  |  |
| <areaNwInfo> | listOfDevices |  |  |  |  |  |
| <areaNwkDeviceInfo> | listOfNeighbors |  |  |  |  |  |
| <locationPolicy> | locationContainerID |  | xs:anyURI |  |  |  |
| <locationPolicy> | locationContainerName |  | xs:string |  |  |  |
| <container> | locationID | LID | xs:anyURI | NONE |  |  |
| <locationPolicy> | locationServer |  |  |  |  |  |
| <locationPolicy> | locationSource |  |  |  |  |  |
| <locationPolicy> | locationTargetId |  |  |  |  |  |
| <locationPolicy> | locationUpdatePeriod |  |  |  |  |  |
| <eventLog> | logActionStatus |  |  |  |  |  |
| <eventLog> | logData |  |  |  |  |  |
| <eventLog> | logStart |  |  |  |  |  |
| <eventLog> | logStop |  |  |  |  |  |
| <eventLog> | logTypeId |  |  |  |  |  |
| <deviceInfo> | manufacturer |  |  |  |  |  |
| <cmdhBuffer> | maxBufferSize | mBS |  |  |  |  |
| <container> | maxByteSize | mb | xs:long |  |  | Determined by the CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <container> | maxInstanceAge | ma | xs:duration |  |  | Determined by the CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <container> | maxNrOfInstances | mi | xs:long |  |  | Determined by CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| <group> | maxNrOfMembers | nM | xs:long |  |  | Determined by the CSE policy. If a value is provided, the CSE shall try to find an acceptable value that is as close as possible to the requested value. |
| memory | memAvailable |  |  |  |  |  |
| <group> | memberAccessRightID | mAR | xs:string |  |  |  |
| <group> | memberList |  | AnyURIList |  |  |  |
| <group> | memberType | mT | MemberType | NONE |  |  |
| <group> | memberTypeValidated | mTV | xs:boolean | NONE |  |  |
| memory | memTotal |  |  |  |  |  |
| <request> | metaInformation |  |  |  |  |  |
| <mgmtObj>, <parameters>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | mgmtDefinition |  | xs:string |  |  |  |
| <cmdhNwAccessRule> | minReqVolume | mRV |  |  |  |  |
| <deviceInfo> | model |  |  |  |  |  |
| <application>,<cmdhPolicy>, software, firmware | name |  | xs:string |  |  |  |
| <node>, <nodeInfo> | nodeID |  | Link |  |  |  |
| <CSEBase> and <application> | nodeLink |  | Link |  |  |  |
| <subscription> | notificationDeliveryPriority |  |  |  |  |  |
| <subscription> | notificationEventCat |  |  |  |  |  |
| <subscription> | notificationStoragePriority |  |  |  |  |  |
| <subscription> | notificationStructure |  |  |  |  |  |
| <subscription> | notificationURI |  | xs:anyURI |  |  |  |
| <mgmtObj>, <parameters>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | objectID |  | xs:anyURI |  |  |  |
| <mgmtObj>, <parameters>, memory, software, firmware,<areaNwInfo>, <areaNwkDeviceInfo>, battery, <deviceInfo>, <deviceCapability>, <capabilityInstance>, reboot, <eventLog> | objectPath |  |  |  |  |  |
| <application>, <container>, <instance> | ontologyRef |  | xs:anyURI |  |  |  |
| <request> | operation |  |  |  |  |  |
| <request> | operationResult |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | order | or |  |  |  |  |
| <request> | originator |  |  |  |  |  |
| <cmdhNwAccessRule> | otherConditions |  |  |  |  |  |
| ALL, except <CSEBase> | parentID | pID | xs:anyURI | NONE | See TBD |  |
| <CSEBase>, <remoteCSE> and <application> | pointOfAccess | pOA | AnyURIList |  |  |  |
| <subscription> | priorSubscriptionNotify |  |  |  |  |  |
| <accessControlPolicy> | privileges | ps | Privileges |  |  |  |
| <subscription> | rateLimit |  |  |  |  |  |
| <reboot> | reboot |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestCharacteristics |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestContextNotification |  |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestContext | rctxt |  |  |  |  |
|  | requestEventCategory | IEC |  |  |  |  |
| <cmdhDefEcValue>, <cmdhLimits> | requestOrigin | roig |  |  |  |  |
| <remoteCSE> | requestReachability |  | xs:boolean |  |  |  |
| <request> | requestStatus |  |  |  |  |  |
| ALL | resourceType | rT | ResourceType | NONE | See TBD |  |
| <schedule> | scheduleElement |  | Schedule |  |  |  |
| <accessControlPolicy> | selfPrivileges | sP | Privileges |  |  |  |
| <areaNwkDeviceInfo> | sleepDuration |  |  |  |  |  |
| <areaNwkDeviceInfo> | sleepInterval |  |  |  |  |  |
| delivery | source |  |  |  |  |  |
| <statsCollect> | statModel |  |  |  |  |  |
| <statsCollect> | statsCollectID |  |  |  |  |  |
| <cmdhPolicy>, <areaNwkDeviceInfo> | status |  | xs:boolean | TRUE = active, FALSE=inactive |  |  |
| <statsCollect> | status | ?? | ?? | Editor’s note: this is a different status of active/inactive |  |  |
| <cmdhBuffer> | storagePriority | sP |  |  |  |  |
| <statsCollect> | subscriberID |  |  |  |  |  |
| <m2mServiceSubscription> | subsGroup |  |  |  |  |  |
| <m2mServiceSubscription> | subsSer&RoleList |  |  |  |  |  |
| <CSEBase> | supportedResourceType | sRT | SupporteResourceType |  |  |  |
| <deviceInfo> | swVersion |  |  |  |  |  |
| <delivery>, <request> | target |  |  |  |  |  |
| <cmdhNwAccessRule> | targetNetwork | tNet |  |  |  |  |
| <eventConfig> | transactionType |  |  |  |  |  |
| <CSEBase>, <remoteCSE> | Trigger-Recipient-ID |  | TBD |  |  |  |
| <instance> | typeOfContent | tOC | ContentType | NONE |  |  |
| software | update |  |  |  |  |  |
| software | updateStatus |  |  |  |  |  |
| software, firmware | URL |  | xs:anyURI |  |  |  |
| software, firmware | version |  |  |  |  |  |
| TBD | versionTag | vT | xs:nonNegativeInteger | 0 |  |  |

Annex B (Normative): Device Triggering

## B.1	Providing Device Triggering service by means of 3GPP networks

### B.1.1	Introduction

3GPP Underlying Network has defined a dedicated interface for requesting device triggering. The normative references for applicable interfaces are as follows: 3GPP TS 23.682 [15]. The specification for the interface Tsp is described in 3GPP TS 29.368 [16]. Tsp interface uses Diameter Base Protocol as specified in IETF RFC 3588 [13], in order to use such an interface the CSE shall act as a Diameter client as described in IETF RFC 6733 [14].

### B.1.2	Device Action Request command

When a CSE needs to issue a device triggering request from MTC-IWF, the CSE shall send a Device-Action-Request (DAR) command (see [16]). The following tables provide an overview of the parameters and the parameter values needed for the DAR command.

Note: the values of the parameters indicated in the “Value” column need to be mapped to the specific 3GPP data type.

Table B.1.2-1: AVPs in Device Action Request (DAR) command

| Attribute Value Pair (AVP) name | AVP description | 3GPP Data Type | AVP Code | Value |
| --- | --- | --- | --- | --- |
| Session-Id | It is used to identify a specific session (see [13]) | UTF8String | 263 | CSE implementation specific |
| Auth-Application-Id | This parameter transport the Tsp application identifier as specified in [16], clause 6.1.3 | Enumerated | 274 | 16777309 |
| Auth-Session-State | The Tsp session is terminated after each request/answer pair and in accordance with [16] the parameter shall indicate that the session state shall not be maintained | Enumerated | 277 | NO_STATE_MAINTAINED (see [16]) |
| Origin-Host | It identifies the endpoint that originated the Diameter message (see [13]) | DiamIdent | 264 | CSE implementation specific |
| Origin-Realm | It identifies the Realm of the originator of theDiameter message (see [13]) | DiamIdent | 296 | CSE implementation specific |
| Destination-Realm | It identifies the realm the message is to be routed to. (see [13]) | DiamIdent | 283 | Domain Identifier part from M2M-Ext-ID |
| Destination-Host | The absence of the Destination-Host causes the message to be sent to any Diameter server supporting the application within the specified realm. (see [13]) | DiamIdent | 293 | NULL |
| Origin-State-Id | It is an increasing value that is advanced whenever a Diameter entity restarts with loss of previous state, for example upon reboot (see [13]) | Unsigned32 | 278 | CSE implementation specific |
| Device-Action | It contains the specific action for the device (see [16]) | Grouped | 3001 | Specific format described in Table B.1.2-2 |
| Proxy-Info | It’s a grouped AVP which provides the proxy state and host (see [13]) | Grouped | 284 | CSE implementation specific |
| Route-Record | The identity added in this AVP is be the same as the one received in the Origin-Host (see [13]) | DiamIdent | 282 | Not applicable |

Table B.1.2-2: AVPs in Device Action AVP (Grouped AVP)

| Attribute Value Pair (AVP) name | AVP description | 3GPP Data Type | AVP Code | Value |
| --- | --- | --- | --- | --- |
| External-Id | It is the identifier of the device as specified in [TS 29.336] | UTF8String | 3111 | M2M-Ext-ID |
| MSISDN | It is the identifier of the device as specified in [TS 29.329] | OctetString | 701 | M2M-Ext-ID |
| SCS-Identifier | It identifies the CSE issuing the request (see [16]) | OctetString | 3104 | CSE-ID |
| Reference-number | It uniquely identifies a transaction (see [16]) | Unsigned32 | 3007 | CSE implementation specific Editor’s note: possibility to maybe use M2M-Request-ID is FFS |
| Action-Type | It informs the MTC-IWF of the required action | Enumerated | 3005 | 1 (Device Trigger Request) |
| Trigger-Data |  | Grouped | 3003 | See Table B.1.2-3 |
| Validity-Time | The validity time in seconds for the specific action requested. (see [16]) | Unsigned32 | 448 | CSE implementation specific |
| NOTE: either external-Id or MSISDN shall be used, not both. |  |  |  |  |

Table B.1.2-3: AVPs in Trigger Data AVP (Grouped AVP)

| Attribute Value Pair (AVP) name | AVP description | 3GPP Data Type | AVP Code | Value |
| --- | --- | --- | --- | --- |
| Payload | It contains the payload to be transferred to the addressed device. (see [16]) | OctetString | 3004 | NULL Furhter values can be defined (FFS) Editor’s Note: in the current TS-0001 does not specify how to use payload. |
| Priority-Indication | It identifies priority of the device trigger. (see [TS 29.368]) | Enumerated | 3006 | CSE implementation specific (possible values are 0 or 1) |
| Application-Port-Identifier | It uniquely identifies the triggering entity addressed in the device. (see [TS 29.368]) | Unsigned32 | 3010 | Trigger-Recipient-ID |

Editor’s Note: Removal of Editor’s Note in the Value file for Payload should be considered.

### B.1.3	Device Action Answer command

As a result of device triggering request to MTC-IWF, the CSE receives a Device-Action-Answer (DAA) command (see [16]). The relevant parameters for a CSE are the following ones:

- Result-Code AVP: see [13] for detail;

- Session-Id AVP: see [13] for detail;

- Other AVPs for Diameter Base Protocol: see [13] for detail.

- Following AVPs are set in Device-Notification AVP (Grouped AVP):

- Either MSISDN or External-Id: the CSE shall check that the identifier of the target CSE is correct;

- SCS-Identifier: the CSE shall check that the value corresponds to its CSE-ID;

- Reference-Number: the CSE can use this value for correlating the DAA with the corresponding Device-Action-Request command issued previously by the CSE;

- Action-Type: the CSE shall check that the value is se to 1 (Trigger Request) as same value of DAR command;

- Request-Status: see Table B.1.3-1 for the list of possible value.

Table B.1.3-1: Result Status

| Result-Status value | Name | Parameter description |
| --- | --- | --- |
| 0 | SUCCESS | This value indicates that device action requested is confirmed. |
| 201 | TEMPORARYERROR | See [16] for detail. |
| 101 | INVALIDPAYLOAD | See [16] for detail. |
| 102 | INVEXTID | See [16] for detail. |
| 103 | INVSCSID | See [16] for detail. |
| 104 | INVPERIOD | See [16] for detail. |
| 105 | NOTAUTHORIZED | See [16] for detail. |
| 106 | SERVICEUNAVAILABLE | See [16] for detail. |
| 107 | PERMANENTERROR | See [16] for detail. |
| 108 | QUOTAEXCEEDED | See [16] for detail. |
| 109 | RATEEXCEEDED | See [16] for detail. |

### B.1.4	Device Notification Request command

As a report of the result for device triggering delivery by 3GPP network, the CSE receives a Device-Notification-Request (DNR) command (see [16]). The relevant parameters for a CSE are the following ones:

- Result-Code AVP: see [13] for detail;

- Session-Id AVP: see [13] for detail;

- Other AVPs for Diameter Base Protocol: see [13] for detail.

- Following AVPs are set in Device-Notification AVP (Grouped AVP):

- Either MSISDN or External-Id: the CSE shall check that the identifier of the target CSE is correct;

- SCS-Identifier: the CSE shall check that the value corresponds to its CSE-ID;

- Reference-Number: the CSE can use this value for correlating the DAA with the corresponding Device-Action-Request command issued previously by the CSE;

- Action-Type: the CSE shall check that the value is set to 2 (Delivery Report);

- Delivery-Outcome: see TableB.1.4-1 for the list of possible value

Table B.1.4-1: Delivery-Outcome

| Delivery-Outcome value | Name | Parameter description |
| --- | --- | --- |
| 0 | SUCCESS | This value indicates that the device action request was successfully completed. |
| 1 | EXPIRED | See [16] for detail. |
| 2 | TEMPORARYERROR | See [16] for detail. |
| 3 | UNDELIVERABLE | See [16] for detail. |
| 4 | UNCONFIRMED | See [16] for detail. |

### B.1.5	Device Notification Answer command

As a result of device notification request to MTC-IWF, the CSE sends a Device-Notification-Answer (DNA) command (see [16]). The relevant parameters for a CSE are the following ones:

- Auth-Application-ID: set to 16777309 as indicated in in [16]);

- Auth-Session-State: set to NO_STATE_MAINTAINED as indicated in [16];

- Other AVPs for Diameter Base Protocol: see [13] for detail, the value of such AVP is CSE implementation specific.

Annex C	(Informative): XML Examples

## C.1	XML Schema for container resource type

<?xml version="1.0" encoding="UTF-8"?>

<!--

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC). All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand

and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations.

No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE,

GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY

PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS.

NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO

ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES.

oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

-->

<xs:schema xmlns="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://www.onem2m.org/xml/protocols"  xmlns:m2m="http://www.onem2m.org/xml/protocols"

elementFormDefault="unqualified" xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:include schemaLocation="common_types-v1_0_0.xsd" />

<xs:element name="container">

<xs:complexType>

<xs:complexContent>

<!-- Inherit Common Attributes from regularResourceType -->

<xs:extension base="m2m:regularResourceType">

<!-- Resource Specific Attributes -->

<xs:sequence>

<xs:element name="maxNrOfInstances" type="xs:nonNegativeInteger"

minOccurs="0" />

<xs:element name="maxByteSize" type="xs:nonNegativeInteger"

minOccurs="0" />

<xs:element name="maxInstanceAge" type="xs:nonNegativeInteger"

minOccurs="0" />

<xs:element name="currentNrOfInstances" type="xs:nonNegativeInteger" />

<xs:element name="currentByteSize" type="xs:nonNegativeInteger" />

<xs:element name="latest" type="xs:anyURI" minOccurs="0" />

<xs:element name="locationID" type="xs:anyURI"

minOccurs="0" />

<xs:element name="ontologyRef" type="xs:anyURI"

minOccurs="0" />

<!-- Child Resources -->

<xs:element name="childResource" type="m2m:childResourceType"

minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

NOTE : The XML Schema documents  need to contain some legal infor-mation at the top. Please use the statements shown in this example for now – thought it is likely that they will need to be replaced prior to external shipment.

## C.2	Container resource that conforms to the Schema given above (see C.1)

<?xml version="1.0" encoding="UTF-8"?>

<m2m:container xmlns:m2m="http://www.onem2m.org/xml/protocols"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xsi:schemaLocation=

"http://www.onem2m.org/xml/protocols CDT-container-v1_0_0-20140609.xsd "

name="12xx">

<parentID>//IN-CSEID.m2m.myoperator.org/96734</parentID>

<accessControlPolicyIDs>//IN-CSEID.m2m.myoperator.org/93405</accessControlPolicyIDs>

<creationTime>2013-12-31T12:00:00</creationTime>

<expirationTime>2013-12-31T12:30:00</expirationTime>

<lastModifiedTime>2013-12-31T12:00:00</lastModifiedTime>

<stateTag>0</stateTag>

<labels>label1 label2</labels>

<maxNrOfInstances>5</maxNrOfInstances>

<maxByteSize>104857600</maxByteSize>

<maxInstanceAge>3600</maxInstanceAge>

<currentNrOfInstances>2</currentNrOfInstances>

<currentByteSize>6</currentByteSize>

<latest>//IN-CSEID.m2m.myoperator.org/96739</latest>

<locationID>//IN-CSEID.m2m.myoperator.org/1112</locationID>

<ontologyRef>http://tempuri.org/ontologies/xyz</ontologyRef>

<childResource name="instance1234" type="instance">//IN-CSEID/1722</childResource>

<childResource name="instance1235" type="instance">//IN-CSEID/34722</childResource>

<childResource name="1923" type="subscription">//IN-CSEID/2323</childResource>

</m2m:container>

Annex Z (Infomative): Template for one M2M resource type

Editor’s Note: this is template text for proposal to Clause 8 “oneM2M Resource Types Definitions”.

Editor’s Note: this template will be kept updated, but should be removed before publication.

### 7.3.x	Resource Type <<resource name>>

#### 7.3.x.1	Introduction

The few (2 statemanets are max.) will be copied from Architecture TS.

The detailed description can be found in clause x.x.x in Architecture TS.

Table 7.3.x.1.1 Data Type Definition of <<resource name>>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-<<resource name>>-v1_0_0-<<date of published>>.xsd | some note texts can be added here |

Table 7.3.x.1.2 Applicability of operations by reference point

| Operations | Mca | Mcc | Mcc’ |
| --- | --- | --- | --- |
| Create | Applicabble or N/A | Applicable or N/A | Applicable or N/A |
| Retrieve | Applicable or N/A | Applicable or N/A | Applicable or N/A |
| Update | Applicable or N/A | Applicable or N/A | Applicable or N/A |
| Delete | Applicable or N/A | Applicable or N/A | Applicable or N/A |

Table 7.3.x.1.3 Data Types for resource specific attributes

| Attribute Name | Data Type | Default Value and Constraints |
| --- | --- | --- |
| <<attribute name1>> | Data Type ex. xs:string | Texts describes default value when the actual value is not given, or some constraints regarding implementation matter. |
| <<attribute name 2>> | another Data Type |  |

Editor’s note; How to represent multiplicity need to clarify.

Editor’s Note: Optionality of Attributes should be referred in Arch-TS or self-contained ?

Editor’s Note: defining default value sound in scope of stage 3, but may be subject to remove in future.

Table 7.3.x.1-4: Reference of child resources

| Child Resource Type and Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- |
| <<resource type1>>(name ) | Place ‘multiplicity’ at resource <<resource type>> | Place the reference to the Resource Type definition in Core Protocol TS |
| Ex. <AE>(variable) | Ex. 0..n | Ex. Clause 7.3.4 |

#### 7.3.x.2	Resource Specific Procedure on CRUD Operations

This sub-clause describes <<resource name>> resource specific behaviour for CRUD operations.

##### 7.3.x.2.1 	Create

The resource specific procedures shall be described here. Those can be mandatory attributes, expecting returning data types, and possible errors

##### 7.3.x.2.2 	Retrieve

The resource specific procedures shall be described here. Those can be mandatory attributes, expecting returning data types, and possible errors

##### 7.3.x.2.3 	Update

The resource specific procedures shall be described here. Those can be mandatory attributes, expecting returning data types, and possible errors

##### 7.3.x.2.4 	Delete

The resource specific procedures shall be described here. Those can be mandatory attributes, expecting returning data types, and possible errors

# List of Tables and Figures

Figure 5.3.1-1: Primitive overview	12

Figure 5.3.2-1: Primitives modeling	13

Table 6.2-1: M2M Identifiers	15

Table 6.3.2.1-1: Simple Data Types incorporated from XML Schema	16

Table 6.3.2.1-1: oneM2M Simple Data Types	18

Table 6.3.2.2-1: oneM2M External IDs	18

Table 6.3.3-1: Resource Common Attributes	19

Table 6.4-1: oneM2M Structured Data Types	20

Table 6.5.4.1.2-1: Data Type Definition of [cmdhPolicy]	23

Table 6.5.4.1.2-2: Optionality of operations by reference point	23

Table 6.5.4.1.2-3: Data Types for resource specific attributes	23

Table 6.5.4.1.2-1: Data Type Definition of [activeCmdhPolicy]	24

Table 6.5.4.1.2-2: Optionality of operations by reference point	24

Table 6.5.4.1.2-3: Data Types for resource specific attributes	24

Table 6.5.4.1.3-1: Data Type Definition of [cmdhDefaults]	24

Table 6.5.4.1.3-3: Data Types for resource specific attributes	25

Table 6.5.4.1.4-1: Data Type Definition of [cmdhDefEcValue]	25

Table 6.5.4.1.4-2: Optionality of operations by reference point	25

Table 6.5.4.1.4-3:  Data Types for resource specific attributes	26

Table 6.5.4.1.5-1: Data Type Definition of [cmdhEcDefParamValues]	26

Table 6.5.4.1.5-2: Optionality of operations by reference point	26

Table 6.5.4.1.5-3: Data Types for resource specific attributes	26

Table 6.5.4.1.6-1: Data Type Definition of [cmdhLimits]	27

Table 6.5.4.1.6-2: Optionality of operations by reference point	27

Table 6.5.4.1.6-3: Data Types for resource specific attributes	27

Tabla 6.5.4.1.7-1:Type Definition of [cmdhNetworkAccessRules]	28

Table 6.5.4.1.7-2:  Optionality of operations by reference point	28

Table 6.5.4.1.8-1: Data Type Definition of [cmdhNwAccessRule]	28

Table 6.5.4.1.8-2: Optionality of operations by reference point	29

Table 6.5.4.1.9-1: Data Type Definition of [cmdhBuffer]	29

Table 6.5.4.1.9-2: Optionality of operations by reference point	29

Table 6.5.4.1.9-3:  Data Types for resource specific attributes	30

Table 6.8.3-1 RSC Framework Overview	32

Table 6.8.4.1-1 RSCs for Successful Responses for Release 1	33

Table 6.8.4.2-1: RSCs for Un-successful Responses for Release 1	34

Table 6.8.4.3-1: RSCs for Acknowledgement Responses for Release 1	36

Table 7.2.1.1-1 : Request Primitive Parameters	37

Table 7.2.1.1-2 : Response Primitive Parameters (1/2)	38

Table 7.2.1.1-3 : Response Primitive Parameters (2/2)	38

Table 7.3.1.1-1: Data Type Definition of cseBase	50

Table 7.3.1.1-2: Optionality of operations by reference point	50

Table 7.3.1.1-3: Data Types for resource specific attributes	50

Table 7.3.1.1-4: Reference of child resources	51

Table 7.3.2.1-1: Data Type Definition of <<resource name>>	52

Table 7.3.2.1-4: Reference of child resources	52

Table 7.3.10.1-1: Data Type Definition of request	56

Table 7.3.10.1-2 : Optionality of operations by reference point	56

Table 7.3.10.1-3 : Data Types for resource specific attributes	56

Table 7.3.10.1-4 : Reference of child resources	57

Table 7.3.11.1-1: Data Type Definition of <group>	59

Table 7.3.11.1-2: Data Types for resource specific attributes	59

Table 7.3.11.1-3: Reference of child resources	59

Table 7.3.13.1-1: Data Type Definition of <mgmtObj>	61

Table 7.3.13.1-2: Data Types for resource specific attributes	61

Table 7.3.13.1-3: Reference of child resources	61

Table 7.3.15.1-1: Data Type Definition of <mgmtCmd>	63

Table 7.3.15.1-2: Data Types for resource specific attributes	63

Table 7.3.15.1-3: Reference of child resources	63

Table 7.3.16.1-1: Data Type Definition of <execInstance>	66

Table 7.3.16.1-2: Data Types for resource specific attributes	66

Table 7.3.17.1-1: Data Type Definition of <node>	67

Table 7.3.17.1-2: Applicability of operations by reference point	68

Table 7.3.17.1-3: Data Types for resource specific attributes	68

Table 7.3.17.1-4: Reference of child resources	68

Table 7.3.20.1-1: Data Type Definition of pollingChannel	69

Table 7.3.20.1-2: Optionality of operations by reference point	69

Table 7.3.20.1-3: Reference of child resources	69

Table 7.3.21.1-1: Data Type Definition of pollingChannel	70

Table 7.3.21.1-2:Optionality of operations by reference point	70

Table  7.3.30.1-1: Data Type definition of <statsConfig>	72

Table  7.3.30.1-2: Optionality of operations by reference point for <statsConfig>	72

Table  7.3.30.1-3: Data Types for resource specific attributes of <statsConfig>	72

Table  7.3.30.1-4: Reference of child resources of <statsConfig>	72

Table 7.3.31.1-1: Data Type definition of <eventConfig>	73

Table  7.3.31.1-2: Optionality of operations by reference point for <eventConfig>	73

Table  7.3.31.1-3: Resource-specific attributes of <eventConfig>	73

Table  7.3.31.1-4: Reference of child resources of <eventConfig>	73

Table 7.3.32.1-1: Data Type definition of <statsCollect>	74

Table 7.3.32.1-2: Optionality of operations by reference point for <statsCollect>	74

Table 7.3.32.1-3: Resource-specific attributes of <statsCollect>	75

Table 7.3.32.1-4: Reference of child resources of <statsCollect>	75

Table A-1: Resource attributes	78

Table B.1.2-1: AVPs in Device Action Request (DAR) command	88

Table B.1.2-2: AVPs in Device Action AVP (Grouped AVP)	89

Table B.1.2-3: AVPs in Trigger Data AVP (Grouped AVP)	89

Table B.1.3-1: Result Status	90

Table B.1.4-1: Delivery-Outcome	90

Table 7.3.x.1.1 Data Type Definition of <<resource name>>	93

Table 7.3.x.1.2 Applicability of operations by reference point	94

Table 7.3.x.1.3 Data Types for resource specific attributes	94

Table 7.3.x.1-4: Reference of child resources	94

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.1.0 | 20 Jun 2013 | Initial Version of TS |
| V.0.1.1 | 15 Aug 2013 | Incorporate agreed contributions at TP#5 Ref: 2013-0020R01,2013-0026R02,2013-0028R02 |
| V.0.1.2 | 12 Nov 2013 | Incorporate agreed contrintuin at TP#7.1 Ref: 2013-0067R01 |
| V.0.2.0 | 20 Dec 2013 | Incorporate agreed contribution at TP#8 Ref: 2013-0092R01 |
| V0.2.1 | 19 Feb 2014 | Incorporate agreed contribution at PRO CC 8.x Ref: 2014-0106R01, 2014-0011R01 |
| v0.3.0 | 5 May 2014 | Incorporate agreed contribution at PRO #9 Ref: 2014-0031R01, 2014-0033R02 |
| v0.3.1 | 25 May 2014 | Incorporate agreed contributions at PRO CC 9.2 and 9.3 Ref: 2014-0117R02 |
| v0.3.2 | 28 May 2014 | Incorporate agreed contribution at PRO CC 9.4 Ref: 2014-0118R02 |
| V0.4.0 | 22 Apr 2014 | Incorporate agreed contributions at PRO #10 after 11 Apr 2014: Ref: PRO-2014-172R02(Template for oneM2M Resource) Ref:PRO-2014-151R01(common-operations) |
| V0.4.1 | 26 May 2014 | Edit to use new template with line numbers |
| v0.4.2 | 30 May 2014 | Incorporated agreed contributions at PRO #10.5 Ref:PRO-2014-202R01-CRUD_common_operations Ref:PRO-2014-205R01-Resource_Type_Definition_Template Ref:PRO-2014-199R02- Primitive_templates_and_procedure_outlines |
| V0.4.3 | 4 Jun 2014 | Incorporated agreed contribuition at PRO #10.7 Ref: PRO-2014-195R03-Child_Resource_Conversion_Rules Added Editor’s Notes to indicate name of leaders |
| V0.5.0 | 14 Jun 2014 | Incorporated agreed contributions at PRO #11.0: Ref: PRO-2014-195R03- Child Resource Conversion Rules Ref: PRO-2014-216R02- Response Status Clause Ref: PRO-2014-217R03- Attribute Creation and Deletion Ref: PRO-2014-220R02- management-common-operations Ref: PRO-2014-229R03- Location Request on Mcn Reference Point Ref: PRO-2014-242- Correction to Table 7.2.1.1-2 in TS-0004 Ref: PRO-2014-219R02- Resource Discovery Procedure Ref: PRO-2014-224R01- design_principles_scalability Ref: PRO-2014-218R05- Announcement Procedures Ref: PRO-2014-249R03- Resource Type Definition Template Update Ref: PRO-2014-251R01- remoteCSE Resource Type Definition Ref: PRO-2014-236R01-Stage 3 text for Resource Type <statsConfig> Ref: PRO-2014-237R02-Stage 3 text for Resource Type <eventConfig> Ref: PRO-2014-238R02-Stage 3 text for Resource Type <statsCollect> Editor’s Note:PRO-2014-222R02 is pending to apply Incorporated agreed contribuition at PRO #11 (by Nobu U) Ref: PRO-2014-0152R02-group-management-procedures Ref: PRO-2014-0194R04-CSEBase_Resource_Type_Definition Ref: PRO-2014-0221R03-mgmtObj-management-procedures Ref: PRO-2014-0228R01- LocationPolicy_Resource_Primitive Ref: PRO-2014-0230R03- mgmtCmd_input_TS0004 Ref: PRO-2014-0231R04-Request_resource_type_definition Ref: PRO-2014-0235R02- pollingChannel_procedures Incorporated PRO-2014-0239R03-Resource_Type_XSD_example in Annex with Editor’s changes Incorporated agreed contributions (missing in R01) Ref: PRO-2014-223R01-node-resource-type-procedures Ref: PRO-2014-227R02-cmdh_policies_TS0004 |
| v.0.5.1 | 17 Jun 2014 | Incorporated pended Agreed contribution: Ref: PRO-2014-222R02-Primitive_templates_and_procedure_outlines_modification Removed resolved Editor’s Notes and unused sub-clauseses. |