| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0004-Protocols-V-0.3.1 |
| Document Name: | oneM2M Core Protocol Specification Version 1.0 |
| Date: | 2014-March 25 |
| Abstract: | < An abstract of the document that may be used to help reader decide whether to read the entire document, and provide information that may be used in subsequent electronic searches> |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	5

3	Definitions, symbols, abbreviations and acronyms	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

3.4	Acronyms	6

4	Conventions	6

5.	Protocol Design Principles and Requirements	6

5.1.	Introduction	6

5.2.	API Design Guidelines	7

5.3.	Design Principles	8

5.3.1.	Scalability	8

5.3.2.	Extensibility	8

5.3.3.	Robustness	8

5.3.4.	Efficiency	8

5.3.5.	Security	8

6.	oneM2M Protocols / API Overview	8

6.1.	Introduction	8

6.2.	Data Types and Definitions	8

6.2.1.	Simple Data Types incorporated from XML Schema	8

6.2.2.	Data Types use in oneM2M	11

6.2.3.	Resource Attributes specificied in oneM2M	12

6.2.3.1.	Resource-common attributes	12

6.2.3.2.	Resource-specific attributes	13

6.2.4.	oneM2M Resource Data Types	13

6.2.4.1.	fooDataType	14

6.3.	oneM2M Device Management  API	14

6.3.1.	Introduction	14

6.3.2.	Management Data Types	14

6.3.3.	<mgmtObj> Resources and Procedures	14

6.3.3.1.	<mgmtObj> Resources	15

6.3.3.2.	<mgmtObj> Primitives and Procedures	15

6.3.4.	<mgmtCmd> Resources and Procedures	15

6.3.4.1.	<mgmtCmd> Resource	15

6.3.4.2.	<mgmtCmd> Primitive and Procedures	15

6.4.	oneM2M Connectivity Management API	15

6.5.	oneM2M Data Management API	15

7.	oneM2M Communication Protocol	15

8	User defined clause(s) from here onwards	16

8.1	User defined subdivisions of clause(s) from here onwards	16

Proforma copyright release text block	16

Annex <y>: Bibliography	17

History	17

# 1	Scope

The present document specifies the communication protocol(s)  for oneM2M compliant Systems,  M2M Applications, and/or other M2MSystems.

This document also specifies the  common data formats, interfaces and message sequences to support reference points(s) defined by oneM2M.

The Scope shall not contain requirements.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Clause 2.1 only shall contain normative (essential) references which are cited in the document itself. These references have to be publicly available and in English.

The following referenced documents are necessary for the application of the present document.

- Use the EX style, enclose the number in square brackets and separate it from the title with a tab (you may use sequence fields for automatically numbering references, see clause A.4: "Sequence numbering") (see example).

[1]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[2]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[3]	W3C XMLSchemaP2: "W3C Recommendation (2004), XML Schema Part 2:Datatypes Second Edition.".

[4]	ISO 19136 (2007): "Geographic information -- Geography Markup Language (GML).".

[5]	W3C XMLPath: "W3C Recommendation (2010), XML Path Language (XPath) 2.0 (Second Edition).".

[6]	oneM2M Architecture TS Table 9.2

[7]	oneM2M TS-0003: Architecture TS (on going work)

[8]	IEEE 754-2008: IEEE. IEEE Standard for Floating-Point Arithmetic. 29 August 2008. http://ieeexplore.ieee.org/servlet/opac?punumber=4610933

[9]	IETF RFC 3548: “The Base16, Base32, and Base64 Data Encodings”. 2003.

[10]	IETF RFC 2045: “Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies”. 1996.

[11]	IETF RFC 3987:“ Internationalized Resource Identifiers (IRIs)” . January 2005.

[12]	IETF BCP 47: “Best Current Practices 47”. Concatenation of RFC 4646:“ Tags for Identifying Languages”(2006) and RFC 4647: “Matching of Language Tags”( 2006).

Editor:s Note: Further documents to add TBD

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	Fielding, Roy Thomas (2000): "Architectural Styles and the Design of Network-based Software Architectures", Doctoral dissertation, University of California, Irvine.

# 3	Definitions, symbols, abbreviations and acronyms

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

## 3.4	Acronyms

Acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Acronym format

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# Protocol Design Principles and Requirements

The following sub-clauses contain the design principles and requirements for the oneM2M protocol.

Editor’s Note: 
The following sub-clauses are intended to provide design principle and specify aspects of protocol requirements, including but not limited to scalability, performance, common message format, reliability, security, extensibility, robustness, resilience, efficiency, message minimisation, etc. Requirements derived from the Requirements TS and inferred from the evolving Architecture TS should be captured here and expressed in terms of Protocol Requirements. Beyond these, additional Protocol requirements are expected. More contributions are requested

## Introduction

The oneM2M architecture is resource-based, (see [6]). The functionality of the system is exposed by means of APIs over the reference points specified in [6]. Operations upon resources hosted by a CSE are carried over an established channel that constitutes the communication on the reference points Mca and Mcc.

Each resource operation comprises a pair of primitives: Request and Response.

In order to provide a well-defined interface for the reference points in the architecture [6], the following aspects need to be provided:

- The collection of primitives carried over a specific reference point, and

- The description and applicability of security methods in relation to the underlying protocols and reference points involved.

The current document provides:

- Data type definitions,

- Primitive definitions, and

- XML definitions and schema.

NOTE: The actual binding of the interface to a specific protocol is not part of this document, but is specified in a separate TS.

In accordance with the oneM2M architecture, each reference point is applicable to a wide range of underlying network technologies and transport protocols. oneM2M will only define a set of bindings for specific underlying network technologies and transport protocols, these bindings are not limiting the applicability of the interfaces when used in other underlying networks and transport protocols. However the behaviour of the interface needs to be respected in accordance to this document and the architecture (see [6]).

## API Design Guidelines

The following are the guidelines for designing APIs:

- API shall follow the principle of RESTful architecture, as described in [i.2]

- APIs shall define how to address resources and how to manipulate resources, in accordance with [6] the resource is identified by a Universal Resource Identifier (URI), [2].

- Resource has a representation (see [i.2]) that shall be transferred and manipulated with the verbs. These verbs are identified as operations in [6]: CREATE. RETRIEVE, UPDATE, DELETE and NOTIFY

- All primitives shall be defined as well as the way that those primitives are sent. The functionality of the primitives shall be compliant to the resource type specific procedure as described in [6] in section 10.2

- API shall provide the format and syntax of the operation primitives for all resources defined in [6]. In case that for a particular protocol binding an operation cannot be supported it has to be clearly stated in the specific protocol binding technical specification.

- Primitives shall include attributes in accordance with [6] for a specific resource

- Primitive shall be self-descriptive and contain all the information needed for the receiver of the primitives to handle the primitives.

- Primitive should be idempotent operations which means no matter how many times the primitive is sent, the result doesn’t change, in accordance to [i.2]. .

- API shall indicate which features are supported and not supported over the reference points defined in [6].

- Primitives shall be mapped on the transport layer protocols.

## Design Principles

## Scalability

## Extensibility

## Robustness

## Efficiency

## Security

# oneM2M Protocols / API Overview

## Introduction

This technical specification describes message formats and procedures to communicate with oneM2M compliant M2M Platform System.

This specification describes:

- Data representation for communication protocol messages

- Normal and exceptional procedure

- Status codes

- Guidelines for drafting APIs

## Data Types and Definitions

For wide acceptance by industrial markets, this specification describes structured and non-structured data for oneM2M Protocol using XML Schema Language (aka XSD)[3].

Note that the actual data format is depends on chosen Protocol Bindings. Each protocol binding specification will specify the mapping rule between XSD version of data definitions and its native data format.

## Simple Data Types incorporated from XML Schema

Following ‘built-in data types’ are incorporated from XML Schema definition [3].

Note that name space identifier for ‘http://www.w3.org/2001/XMLSchema' MUST be described as ‘xs:’ in this specification.

| Data Type | Description | Notes |
| --- | --- | --- |
| xs:string | The string datatype represents character strings in XML |  |
| xs:boolean | boolean represents the values of two-valued logic. |  |
| xs:decimal | decimal represents a subset of the real numbers, which can be represented by decimal numerals. The ·value space· of decimal is the set of numbers that can be obtained by dividing an integer by a non-negative power of ten, i.e., expressible as i / 10n where i and n are integers and n ≥ 0. Precision is not reflected in this value space; the number 2.0 is not distinct from the number 2.00. The order relation on decimal is the order relation on real numbers, restricted to this subset. |  |
| xs:float | The float datatype is patterned after the IEEE single-precision 32-bit floating point datatype IEEE 754-2008 ([8]). Its value space is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:double | The double datatype is patterned after the IEEE double-precision 64-bit floating point datatype IEEE 754-2008 ([8]). Each floating point datatype has a value space that is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:duration | duration is a datatype that represents durations of time. |  |
| xs:dateTime | dateTime represents instants of time, optionally marked with a particular time zone offset. Values representing the same instant but having different time zone offsets are equal but not identical. |  |
| xs:time | time represents instants of time that recur at the same point in each calendar day, or that occur in some arbitrary calendar day. |  |
| xs:date | date represents top-open intervals of exactly one day in length on the timelines of dateTime, beginning on the beginning moment of each day, up to but not including the beginning moment of the next day). For non-timezoned values, the top-open intervals disjointly cover the non-timezoned timeline, one per day. For timezoned values, the intervals begin at every minute and therefore overlap. |  |
| xs:hexBinary | hexBinary represents arbitrary hex-encoded binary data. |  |
| xs:base64Binary | base64Binary represents arbitrary Base64-encoded binary data. For base64Binary data the entire binary stream is encoded using the Base64 Encoding defined in RFC 3548([9]), which is derived from the encoding described in RFC 2045([10]). |  |
| xs:anyURI | anyURI represents an Internationalized Resource Identifier Reference (IRI). An anyURI value can be absolute or relative, and may have an optional fragment identifier (i.e., it may be an IRI Reference). This type should be used when the value fulfills the role of an IRI, as defined in RFC 3987([11]) or its successor(s) in the IETF Standards Track. |  |
| xs:normalizedString | normalizedString represents white space normalized strings. The ·value space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The ·lexical space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The ·base type· of normalizedString is string. |  |
| xs:token | token represents tokenized strings. The ·value space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The ·lexical space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The ·base type· of token is normalizedString. |  |
| xs:language | language represents formal natural language identifiers, as defined by BCP 47([12]). |  |
| xs:integer | integer is ·derived· from decimal by fixing the value of ·fractionDigits· to be 0 and disallowing the trailing decimal point. This results in the standard mathematical concept of the integer numbers. The ·value space· of integer is the infinite set {...,-2,-1,0,1,2,...}. The ·base type· of integer is decimal. |  |
| xs:nonNegativeInteger | nonNegativeInteger has a lexical representation consisting of an optional sign followed by a non-empty finite-length sequence of decimal digits (#x30-#x39). If the sign is omitted, the positive sign ('+') is assumed. If the sign is present, it must be "+" except for lexical forms denoting zero, which may be preceded by a positive ('+') or a negative ('-') sign. For example: 1, 0, 12678967543233, +100000. |  |
| xs:positiveInteger | positiveInteger is ·derived· from nonNegativeInteger by setting the value of ·minInclusive· to be 1. This results in the standard mathematical concept of the positive integer numbers. The ·value space· of positiveInteger is the infinite set {1,2,...}. The ·base type· of positiveInteger is nonNegativeInteger. |  |
| xs:unsignedLong | unsignedLong is derived· from nonNegativeInteger by setting the value of ·maxInclusive· to be 18446744073709551615. The ·base type· of unsignedLong is nonNegativeInteger. |  |
| xs:unsignedInt | unsignedInt is ·derived· from unsignedLong by setting the value of ·maxInclusive· to be 4294967295. The ·base type· of unsignedInt is unsignedLong. |  |
| xs:unsignedShort | unsignedShort is ·derived· from unsignedInt by setting the value of ·maxInclusive· to be 65535. The ·base type· of unsignedShort is unsignedInt. |  |
| xs:dateTimeStamp | The dateTimeStamp datatype is derived· from dateTime by giving the value required to its explicitTimezone facet. The result is that all values of dateTimeStamp are required to have explicit time zone offsets and the datatype is totally ordered. |  |

## Data Types use in oneM2M

This specification define standard set of information as data types used are shown in Table 1

Editor’s Note: We should consider how best to add references to the namespaces

Table 1	Data types used

| Data Type | Type Name | Where defined | Notes |
| --- | --- | --- | --- |
| ca:civicAddr | Civic Address | [1] |  |
| gml:Point | Point | [4] | A point consists of a <Point> element with a child <coords> element. Within <coords> the latitude and longitude values are separated by a space. |
| nonEmptyString | Non-empty string | oneM2M | A string having at least one character. Defined as xs:string [3] with restriction of minLength = 1 |
| xpath | Xpath | [5] | Path to and including the target element or attribute. |
| xs:integer enumeration | Integer with enumeration restriction | [3] | Restricted integer values. |
| xs:string enumeration | String with enumeration restriction | [3] | Restricted string values. |

Editor’s Note: The data types listed in the table are examples and FFS.

## Resource Attributes specificied in oneM2M

All resource attributes used are listed within the following sub-clauses.

## Resource-common attributes

Resource attributes, which are widely used in resources, are specified in Error: Reference source not found.

Table 2 Resource common attributes

| Attribute Name | Data Type | Default | Value restrictions | Notes |
| --- | --- | --- | --- | --- |
| accessRightID | xs:string | All permisisons |  |  |
| parentID | xs:string | NONE |  |  |
| creator | xs:string | NONE |  |  |
| creationTime | xs:dateTime | NONE |  |  |
| expirationTime | xs:dateTime | NONE |  |  |
| filterCriteria | xs:string | "" |  |  |
| lastModifiedTime | xs:dateTime | NONE |  |  |
| resourceType | xs:string | NONE |  |  |
| labels | xs:string | NONE |  |  |
| link | xs:anyURI | NONE |  |  |
| announceTo | xs:anyURI | NONE |  |  |
| announceAttribute | xs:string | NONE |  |  |

Note: The common attributes above are described in [6]

  - Editor’s Note: The attributes listed in the table are examples and FFS.

Editor's Note:  How to choose the local time zone at timestamp is FFS.

## Resource-specific attributes

Resource attributes, which are specific to certain resources, are specified in Table 3.

Table 3 Resource-specific attributes

| Attribute Name | Data Type | Default | Value restrictions | Notes |
| --- | --- | --- | --- | --- |
| dayOfWeek | xs:nonNegativeInteger | 0 | 0 = "everyday", 1 = "Monday", 2 = "Tuesday", 3 = "Wednesday", 4 = Thursday", 5 = "Friday", 6 = "Saturday", 7 = "Sunday", 8 = “weekday”, 9 = “weekend” |  |

Editor’s Note: The attributes listed in the table are examples and FFS.

## oneM2M Resource Data Types

This specification defines common set of structured data as ‘Common Data Types’.

oneM2M Resource Data Types are defined in XSD, and they are supplied as separate documents. The source of XSD file can be Resource definition in Architecture TS [7].

Even XSD files are managed independent from Architecture TS, the skelton XSD file can be generated following conversion rules:

- Each Resource Types will be defined in a separate XSD file. The name of that file should be prefixed with ‘CDT-‘ and  followed by the resource type name, version of the Core Protocol TS, and date of update. Each Resource Type file will contain a single Global Element Definition whose name is the name of the Resource Type as defined in Architecture TS[7].

- Each attributes are mapped as child element of the top level element, and should not have any child elements

- Each child resources are mapped as child element of the top level element, and  should refer another resource type definition using ‘ref’ attribute

- Each attributes should be declared to use either one of the simple data types listed in section 6.3, or a data type derived by restriction from one of the types listed in 6.3.

- Common ‘Simple types’ are collected as dedicated XSD file, which is named ‘ simple_types-v1_0-<date>.xsd’

Table 4 List of oneM2M Resource Data Types

| Data Type ID | File Name | Where defined | Notes |
| --- | --- | --- | --- |
| simpleType | CDT-simpleType-v1_0-<date>.xsd | 6.2.1.1.1 |  |

Editor’s Note: Actual data entries should be filled in

## fooDataType

Description

<Short descritpion about the Data Type>

Reference

<Reference to the Architecture TS>

Usage Example

<Example data for illustration purpose>

Editor’s Note: This is example of description for each Common Data Type, and remove this  sub clause when actual entries are added.

## oneM2M Device Management  API

<Text>

The following text is to be used when appropriate:

## Introduction

This section describes the management specific primitives and procedures to communicate across the Mca and Mcc reference points.

## Management Data Types

This section describes the data types that are extensions of the data types defined in section 6.2.1 and are specific to the management resource and subresources.

## <mgmtObj> Resources and Procedures

This section describes the primitives and data types necessary to communicate the <mgmtObj> resource and subresources across the Mca and Mcc reference points.

## <mgmtObj> Resources

This section describes the definition of the <mgmtObj> resource and subresources. In addition, the policy (e.g., mandatory, optional) related to the presence of a resource’s attributes with respect to the primitives is documented in this section.

## <mgmtObj> Primitives and Procedures

This section describes the primitive and associated procedures for the <mgmtObj> resource and subresources.

## <mgmtCmd> Resources and Procedures

This section describes the primitives and procedures to communicate the <mgmtCmd> resource and subresources across the Mca and Mcc reference points.

## <mgmtCmd> Resource

This section describes the definition of the <mgmtCmd> resource and subresources. In addition, the policy (e.g., mandatory, optional) related to the presence of a resource’s attributes with respect to the primitives is documented in this section.

## <mgmtCmd> Primitive and Procedures

This section describes the primitive and associated procedures for the <mgmtObj> resource and subresources.

## oneM2M Connectivity Management API

<Text>

The following text is to be used when appropriate:

## oneM2M Data Management API

<Text>

The following text is to be used when appropriate:

# oneM2M Communication Protocol

# 8	User defined clause(s) from here onwards

## 8.1	User defined subdivisions of clause(s) from here onwards

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A> (Informative/Normative):Remove Informative or Normative as appropriatTitle of annex (style H9)

<Text>

<PAGE BREAK>

Annex <B>(Informative/Normative): Remove Informative or Normative as appropriateTitle of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

The following text is to be used when appropriate:

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.1.0 | 20 Jun 2013 | Initial Version of TS |
| V.0.1.1 | 15 Aug 2013 | Incorporate agreed contributions at TP#5 Ref: 2013-0020R01,2013-0026R02,2013-0028R02 |
| V.0.1.2 | 12 Nov 2013 | Incorporate agreed contrintuin at TP#7.1 Ref: 2013-0067R01 |
| V.0.2.0 | 20 Dec 2013 | Incorporate agreed contribution at TP#8 Ref: 2013-0092R01 |
| V0.2.1 | 19 Feb 2014 | Incorporate agreed contribution at PRO CC 8.x Ref: 2014-0106R01, 2014-0011R01 |
| v0.3.0 | 5 May 2014 | Incorporate agreed contribution at PRO #9 Ref: 2014-0031R01, 2014-0033R02 |
| v0.3.1 | 25 May 2014 | Incorporate agreed contributions at PRO CC 9.2 and 9.3 Ref: 2014-0117R02 |