| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0007-Service_Component_Architecture-V0_4_0 |
| Document Name: | Service Component Architecture |
| Date: | 2014-August-04 |
| Abstract: | This document specifies the M2M Services provided by the M2M Services Platform, the integration and interworking of the M2M Services functional architecture of the oneM2M Services Platform and informatively illustrates the use of the M2M Services within the context of complex business services. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions, symbols, abbreviations and acronyms	6

3.1	Definitions	6

3.2	Symbols	7

3.3	Abbreviations	7

3.4	Acronyms	7

4	Conventions	7

5	M2M Services Architecture	8

5.1	Introduction	8

5.2	Reference Points	9

5.2.1	Mca Reference Point	9

5.2.2	Msc Reference Point	9

5.2.2.1	 Message Exchange Patterns	9

5.2.2.2	 M2M Identifiers	11

5.2.2.2.1	 M2M Service Identifier (M2M-Srv-ID)	11

5.2.3	Mcc’ Reference Point	11

5.2.4	Mcn Reference Point	11

5.3	Configurations support by M2M Service Architecture	11

6	M2M Services	12

6.1	Introduction	12

6.2	Service Subscription	13

6.2.1	Service Capabilities	13

6.2.1.1	validateSupportingService	13

6.2.1.2	retrieveAuthorization	14

6.2.1.3	getBroker	15

6.2.1.4	getManagementAdapter	16

6.3	Authorization	17

6.3.1	Service Capabilities	17

6.3.1.1	authorizeServiceCapability	17

6.4	Data Exchange	19

6.4.1	Overview	19

6.4.1.1	Subscribe-Publish-Notify Data Exchange	19

6.4.2	Service Capabilities	20

6.4.2.1	subscribe	20

6.4.2.2	publish	22

6.4.2.3	notify	24

6.5	Broker	25

6.5.1	Overview	25

6.5.2	Service Capabilities	25

6.5.2.1	subscribe	25

6.5.2.2	publish	26

6.5.2.3	notify	27

6.6	Device Management	30

6.6.1	Overview	30

6.6.1.1	Device Management	30

6.6.2	Service Capabilities	34

6.6.2.1	downloadFirmware	34

6.6.2.2	installFirmware	37

6.6.2.3 	getFirmwareInformation	40

6.6.2.4 	getFirmwareExecStatus	42

6.6.2.5	reportFirmwareStatus	44

6.6.2.6	upgradeFirmware	45

6.6.2.7	getDeviceInformation	47

6.6.2.8	getDeviceCapabilities	49

6.6.2.9 	enableDeviceCapability	51

6.6.2.10	 disableDeviceCapability	53

6.6.2.11	getAreaNetworks	55

6.6.2.12	updateDeviceForAreaNetwork	57

6.7	Management Adapter	59

6.7.1	Overview	59

6.7.2	Service Capabilities	59

6.7.2.1	downloadFirmware	59

6.7.2.2	installFirmware	60

6.7.2.3	getFirmwareInformation	61

6.7.2.4	getFirmwareExecStatus	62

6.7.2.5	reportFirmwareStatus	63

6.7.2.6	getDeviceInformation	65

6.7.2.7	getDeviceCapabilities	66

6.7.2.8	enableDeviceCapability	67

6.7.2.9	disableDeviceCapability	68

6.7.2.10	getAreaNetworks	69

6.7.2.12	updateDeviceForAreaNetwork	70

6.8	Supporting Service Administration	71

6.8.1	Overview	71

6.8.1.1	Supporting Service Entity	71

6.8.2	Service Capabilities	71

6.8.2.1	createSupportingService	71

6.8.3.1	deleteSupportingService	73

6.8.4.1	updateSupportingService	75

6.8.5.1	addRoleToSupportingService	77

6.8.6.1	deleteRoleFromSupportingService	79

6.8.7.1	getSupportingService	81

6.9	Service Subscription Administration	83

6.9.1	Overview	83

6.9.1.1	Service Subscription Entity	83

6.9.1.2	Device Entity for Service Subscription	84

6.9.1.2.1	Device Filter Criteria	84

6.9.2	Service Capabilities	84

6.9.2.1	createServiceSubscription	84

6.9.3.1	deleteServiceSubscription	86

6.9.4.1	updateServiceSubscription	88

6.9.5.1	addRoleToServiceSubscription	90

6.9.6.1	deleteRoleFromServiceSubscription	92

6.9.7.1	getServiceSubscription	94

6.9.8.1	addDeviceToServiceSubscription	96

6.9.9.1	deleteDeviceFromServiceSubscription	98

6.9.10.1	getDevicesForServiceSubscription	100

6.9.11.1	addApplicationsToServiceSubscription	102

6.9.12.1	deleteApplicationsFromServiceSubscription	104

6.9.13.1	getApplicationsForServiceSubscription	106

6.9.14.1	updateApplicationForDevice	108

6.10	Event Collection	110

7	M2M Service Components	111

7.1	Introduction	111

7.1.1	Service Component Interaction Cross Reference	111

7.3	Infrastructure Component (INF)	112

7.3.1	INF to Service Cross Reference	112

7.4	Service Subscription Component (SSUB)	112

7.4.1	SSUB to Service Cross Reference	112

7.5	Transport Adapter (TRA)	112

7.5.1	TRA to Service Cross Reference	112

7.6	Accounting (ACC)	112

7.6.1	Accounting to Service Cross Reference	112

7.7	Service Exposure (SE)	112

7.7.1	Service Exposure to Service Cross Reference	112

7.8	Management Adapter (MA)	113

7.8.1	Management Adapter to Service Cross Reference	113

Proforma copyright release text block	114

Annex A (Informative): Common Services: Mca Received Requests	115

A.1	Overview	115

A.2	Supporting Services	115

A.2.1	Authentication and Authorization of Requests	115

A.2.1.1	receiveRequest	115

A.2.1.2	getServiceCapability	117

A.2.1.3	getSupportingService	118

Annex B (Informative): Data Exchange Services	119

B.1	Overview	119

B.2	Supporting Services	119

B.2.1	Subscribe-Publish-Notify Message Exchange	119

B.2.1.2	Service Capabilities	119

Annex C (Informative): Service Subscription Administration Services	128

C.1	Overview	128

C.2	Supporting Services	128

C.2.1	Service Capabilities	128

Annex D (Informative): Device Management Services	144

D.1	Overview	144

D.2	Supporting Services	144

D.2.1	Message Exchange	144

D.2.1.1	Service Capabilities	144

Annex X(Informative): oneM2M Service Capability Template	168

X.1	<serviceCapabilityName>	168

X.1.1	Signature	168

X.1.2	Pre-conditions	169

X.1.3	Service Interactions	169

X.1.4	Post-Conditions	169

X.1.5	Exceptions	169

X.1.6	Policies for Use	169

X.1.7	oneM2M Resource Interworking	169

Annex Y(Informative): oneM2M Service Requirements	170

History	171

# 1	Scope

This specification describes the M2M Services provided by the oneM2M Services Platform, the integration and interworking of the M2M Services functional architecture of the oneM2M Services Platform and informatively illustrates the use of the M2M Services within the context of complex business services.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0002: “oneM2M Requirements; oneM2M-TS-0002-V-0.6.2”

[2]	oneM2M TS-0001: “oneM2M Functional Architecture”

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	W3C WD-wsdl20-patterns-20040326:”Web Services Description Language (WSDL) Version 2.0 Part 2: Message Exchange Patterns.”

[i.3]	 OASIS soa-rm: "Reference Model for Service Oriented Architecture 1.0"

# 3	Definitions, symbols, abbreviations and acronyms

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions apply:

Business Service: A Service that creates added-value to an organization and is relevant to the business process of the organization that consumes Services from the M2M Services Platform. Business Services are typically complex, orchestrated Services which consume M2M Services among other Services. A Tiered Data Service is an example of a Business Service.

M2M Service: A Service that creates added-value to the providing M2M Service Platform and is consumed by one or more Business Services. The M2M Subscription Service is an example of a M2M Service.

M2M Service Registration: When a Service is deployed into a SOA environment; the SOA environment’s Service Registry is updated so that the Service can be accessed by other Services.

Service[i.3]: A mechanism to enable access to one or more Service Capabilities, where the access is provided using a prescribed interface and is exercised consistent with constraints and policies as specified by the service description.

Service Capability[i.3]: A real-world effect that a service provider is able to provide to a service consumer.

Service Cluster: A collection of distributed and related Services that are gathered to solve a business problem.

Service Component: A entity of the M2M Services Architecture which may contain one or more M2M Services.

Service Execution Environment: An Execution Environment is a logical entity that represents an environment capable of running Services.

Service Exposure Component: A Service Component that provides an enforcement point for specific oneM2M reference points (e.g. Mca, Mcn, Mcc’).

Service Registry: A component of an underlying SOA environment that maintains information about Services and provides facilities to publish and discover Services to entities that would utilize the Service.

## 3.2	Symbols

For the purposes of the present document, the symbols apply:

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the following apply:

AE	Application Entity

BSS	Business Support System

CSE	Common Service Entity

ESB	Enterprise Service Bus

MEP	Message Exchange Pattern

M2M	Machine-to-Machine

M2M-Srv-ID	M2M Service Identifier

NMS	Network Management System

OSS	Operational Support System

SEE	Service Execution Environment

## 3.4	Acronyms

For the purposes of the present document, the following acronyms apply:

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	M2M Services Architecture

This clause describes the architecture the M2M Services within the context of the M2M Services Platform.

## 5.1	Introduction

The architecture of the M2M Service Platform as defined in the oneM2M Functional Architecture [2] includes an entity described as the Common Service Entity (CSE). The CSE comprises a set of service functions that common to the M2M environment and are exposed through the Mca and Mcc’ Reference Points. These Reference Points are described in the oneM2M Functional Architecture [2]. The M2M Service Architecture described in this specification is primarily suitable for the Infrastructure Domain where the CSE is viewed as a set of Service Components.

The M2M Service Architecture augments the oneM2M Functional Architecture by specifying M2M Services provided to M2M Application and M2M Service Providers.

These M2M Services are consumed by:

- AEs across the Mca reference point via the Service Exposure Component

- Other Infrastructure CSEs across the Mcc’ reference point via the Remote Service Exposure Component

- Other Service Components across the Msc reference point

These M2M Services utilize services of the:

- Underlying Network across the Mcn reference point via the Network Service Utilization Component

Figure 5.1-2: oneM2M Services Architecture

The oneM2M Services architecture in Figure 5.1-2 comprises of the following entities:

- Application Entity (AE): Defined by the oneM2M Functional Architecture [2], the Application Entity provides Application logic for the end-to-end M2M solutions.

- Common Services Entity (CSE): Defined by the oneM2M Functional Architecture [2], a Common Services Entity comprises the set of "service functions" that are common to the M2M environments and specified by oneM2M. For oneM2M Services, this definition of a CSE is a logical representation where “service functions” that are exposed through the Mca and Mcc’ reference points via the corresponding Service Exposure and Remote Service Exposure Components and the Network Service Utilization Component utilizes services of the Underlying Network through the Mcn referent point. In addition Service Components consume and provide M2M Services with other Service Components.

As a logical representation of loosely coupled Service Components, the CSE is entity that in itself is identifiable but not directly addressable. Instead the addressable entities are the corresponding Service Exposure Components of the reference points.

Editors note: Need to clarify that the Services are the addressable entities within a components; components are not directly addressable.

  - Service Exposure Component: The Service Exposure Component exposes services to AEs.

  - Network Service Utilization Component: The Network Service Utilization Component consumes services from the NSE.

  - Remote Service Exposure Component: The Remote Service Exposure connects Services from different M2M environments.

The Service Exposure, Network Service Utilization and Remote Service Exposure Components follow the CSE Public Domain Names convention described in section 6.5.1.2 of the Functional Architecture [2] but are extended as a sub-domain of the Infrastructure Node public domain name.

## 5.2	Reference Points

The M2M Service Architecture exposes M2M Services across the Mca, Mcn, Mcc’ and Msc reference points. The enhancement of the Mcc reference point to expose M2M Services is FFS.

### 5.2.1	Mca Reference Point

The Mca reference point is defined within the oneM2M Functional Architecture [2]. Additional protocol support is defined in this specification for other application protocols. In the M2M Service Architecture, when an AE uses a protocol across the Mca reference point that is not defined by oneM2M (e.g., XMPP, DDS, MQTT), the CSE passes through the payload and offers additional M2M Services.

Editor’s note: This description needs further clarification; specifically what does it mean to pass through the payload.

### 5.2.2	Msc Reference Point

The Msc reference point specifies set of interactions between the Service Capabilities of different Service Components. The realization of the interaction between the Services Capabilities is implementation specific.

Editor’s Note: There is a need to define a term for an interface which specifies a service definition but does not specify the communication bindings of the stage 3 work.

### 5.2.2.1	 Message Exchange Patterns

Communication between M2M Service Components which pass over the Msc reference point utilizes a web services approach, e.g., Web Services Message Exchange Patterns (MEP) defined by WSDL[i.2]:

- In-Only

- Robust In-Only

- In-Out

- In-Optional-Out

- Out-Only

- Robust Out-Only

- Out-In

- Out-Optional-In

Figure 5.2.2.1-1: Message Exchange Patterns

The MEP(s) utilized by a Service Capability is documented within the Service Capabilities Policy clause.

Note: A subset of the MEPs provided by a Service Capability would be exposed by the Service Exposure Component or the Remote Service Exposure Component.

### 5.2.2.2	 M2M Identifiers

### 5.2.2.2.1	 M2M Service Identifier (M2M-Srv-ID)

| Identifier | Assigned by | Assigned to | Assigned during | Lifetime | Uniqueness | Used during | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M2M Service Identifier | M2M SP | M2M Service Note: Service Components contain multiple M2M Services; the M2M Service Identifier is assigned to each M2M Service in the Service Component. | M2M Service Registration. This is the identifier from the Services Registry. | Life of the M2M Service Registration. | Global | Message exchange between Service Components across the Msc and Mcc’. Message exchange across the Mca. |  |

Table 5.2.2.2.1-1 M2M Service Identifier

### 5.2.3	Mcc’ Reference Point

The Mcc’ reference point is defined within the oneM2M Functional Architecture [2]. The oneM2M Service Architecture utilizes communications over the Mcc’ reference point to construct a Service Cluster for service sharing and capacity expansion. A messaging exchange protocol (e.g., AMQP, XMPP, JMS) between two service execution environments (SEE) (e.g., ESB, Apache Service Mix) accommodates the SEEs with same and different ownership.

Note: The protocol used for communication between SEEs with different ownership may require additional security capabilities and is FFS.

### 5.2.4	Mcn Reference Point

The Mcn reference point is defined within the oneM2M Functional Architecture [2].

## 5.3	Configurations support by M2M Service Architecture

M2M Services exposed across the Mca, Mcn, Mcc’ and Msc reference are supported for Infrastructure Nodes as defined in the oneM2M Functional Architecture [2].

# 6	M2M Services

This clause describes the M2M Services provided by the M2M Services Platform.

## 6.1	Introduction

The oneM2M Services defined in this clause are utilized by the Supporting Services described in the Annexes of this specification.

Figure 6.1-1: oneM2M Services

## 6.2	Service Subscription

The Service Subscription service provides the ability to:

- Validate the service capabilities (e.g., requests, notifications) that go across the Mca reference point by ensuring Supporting Service requested by the Originator is permitted within the context of the M2M Service Subscription

- Assist in authorization of a service capability by returning a role for the requested service capability

- Assist in returning the Broker instance needed to exchange data for publish and subscribe service capabilities

- Assist in returning the Management Adapter instance needed to manage the device

### 6.2.1	Service Capabilities

#### 6.2.1.1	validateSupportingService

This service capability ensures that a valid M2M Service Subscription exists for a service capability that is either received by the M2M Service Platform from an AE or is transmitted to the AE from the M2M Service Platform.

##### 6.2.1.1.1	Pre-conditions

A correlation between the serviceId that defines the Supporting Service and the subscriptionId has been defined in order to obtain the serviceId.

##### 6.2.1.1.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Sub-ID) |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) |
| responseType | OUT | NO | Response types: - Supporting Service does not exist for requested Service Subscription - Supporting Service does exist for the requested Service Subscription |

Table 6.2.1.1.2-1 validateSupportingService Signature

##### 6.2.1.1.3	Post-Conditions

Not Applicable

##### 6.2.1.1.4	Exceptions

Not Applicable

##### 6.2.1.1.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.1.1.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

#### 6.2.1.2	retrieveAuthorization

This service capabilities determines the Role associated with the Service Capability and the M2M Subscription.

##### 6.2.1.2.1	Pre-conditions

A correlation between the subscriptionId, serviceId and serviceCapId to a Role.

##### 6.2.1.2.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Sub-ID) |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Srv-ID) |
| rolePolicies | OUT | Yes | The Attribute Based Control Role policies that are valid for this subscription, service and service capability. |
| responseType | OUT | NO | Response types - Roles exist - Roles do not exist |

Table 6.2.1.1.2-1 retrieveAuthorization Signature

##### 6.2.1.2.3	Post-Conditions

Not Applicable

##### 6.2.1.2.4	Exceptions

Not Applicable

##### 6.2.1.2.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.1.2.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified from the Security TS.

#### 6.2.1.3	getBroker

This service capability obtains the Broker instance that will be used to subscribe and publish requests based on the requested service capability (e.g., publishRequest, subscribeRequest), AE and Resource.

For the publishRequest service capability the AE is the from AE-ID and the resource is the toResource representing a leaf node from the publication resource.

For the subscribeRequest service capability the AE is the to AE-ID and the resource is the publication resource.

##### 6.2.1.3.1	Pre-conditions

A correlation between a Service Subscription, from AE-ID and Broker exist.

##### 6.2.1.3.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Sub-ID) |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) of the service capability |
| to | IN | NO | The identifier of the AE that is to target of the request (AE-ID) |
| broker | OUT | NO | The returned instance of the Broker service associated with the subscription. |
| responseType | OUT | NO | Response types: - Broker does not exist for requested service capability, Service Subscription and target |

Table 6.2.1.3.2-1 getBroker Signature

##### 6.2.1.3.3	Post-Conditions

Not Applicable

##### 6.2.1.3.4	Exceptions

Not Applicable

##### 6.2.1.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.1.3.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

#### 6.2.1.4	getManagementAdapter

This service capability obtains the Management Adapter instance that will be used to manage device based on the requested service capability (e.g., downloadFirmware, installFirmware) and device.

##### 6.2.1.4.1	Pre-conditions

A correlation between a Management Adapter, the service capability and device exist.

##### 6.2.1.4.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Sub-ID) |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) of the service capability |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| managementAdapter | OUT | NO | The returned instance of the Management Adapter service associated with the subscription. |
| responseType | OUT | NO | Response types: - Management Adapter does not exist |

Table 6.2.4.1.2-1 getManagementAdapter Signature

##### 6.2.1.4.3	Post-Conditions

Not Applicable

##### 6.2.1.4.4	Exceptions

Not Applicable

##### 6.2.1.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.1.4.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

## 6.3	Authorization

The Authorization service provides the ability to:

- Authorize the Originator for the Service Capability

### 6.3.1	Service Capabilities

#### 6.3.1.1	authorizeServiceCapability

This service capability authenticates an originator is valid Originator of the request and then authorizes the service capability within the context of a M2M Service Subscription between the from AE-ID and to AE-ID.

##### 6.3.1.1.1	Pre-conditions

The M2M Identifiers for the request originator, subscriptionId, from AE-ID and to AE-ID are assigned to the Service Subscription.

A correlation between the service capability and the M2M authorization event has been defined in order to authorize the service capability.

##### 6.3.1.1.2	Common Service Capability Parameters for Request Authentication and Authorization

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | The identifier of the Originator that is issuing the request. |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-SUB-ID) |
| from | IN | NO | The identifier of the Originator of the request (AE-ID) |
| to | IN | NO | The identifier of the AE that is to target of the request (AE-ID) |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca reference point. - Originator is not authenticated - Originator does not have a Service Subscription - Originator not authorized for the Service Capability |

Table 6.3.1.1.2-1 Common Service Capability Parameters for Request Authentication and Authorization

##### 6.3.1.1.3	Service Interactions

This service capability will consume the ServiceSubscription:retrieveAuthorization service capability:

##### 6.3.1.1.4	Post-Conditions

Not Applicable

##### 6.3.1.1.5	Exceptions

Not Applicable

##### 6.3.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.3.1.1.6	oneM2M Resource/CSF Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource/Service Interworking will be identified once the Security TS-0003 is frozen.

## 6.4	Data Exchange

### 6.4.1	Overview

The Data Exchange service provides the ability to exchange payloads between AEs across the Mca reference point using supported data exchange patterns. This service utilizes an underlying transport adapter to provide the transportation functions.

The supported data exchange patterns are:

- Subscribe-Publish-Notify

#### 6.4.1.1	Subscribe-Publish-Notify Data Exchange

This data exchange pattern permits AEs to subscribe to publication resources. When an AE publishes data to a specific publication resource, the AEs that have subscribe to the publication resource are notified of the publication by receiving the published data.

##### 6.4.1.1.1	Supporting Rules

A publication resource is an entity that represents the mechanism used by underlying publish and subscribe protocols which represent one or more queues in the underlying protocol. A publication resource in the data exchange service provides capabilities to reference one or more queues in the underlying protocol using a tree mechanism where wildcards are placed to represent all elements of the tree branch or leaf.

###### 6.4.1.1.1.1	Publication Resources

When a AE publishes a payload to a publication resource, the publication resource is evaluated to obtain the leaf node of the tree. The AEs that have subscribed to leaf nodes of the tree receive the message via the notify service capability.

###### 6.4.1.1.1.2	Delivery Policy

As part of the notify service capability the Service Layer attempts to honor the delivery policy of the subscriber and publisher. The delivery policy is the attempt by the Service Layer to provide a reliable delivery of the payload that was published using the mechanisms of the underlying Broker. The reliable delivery of a payload is defined in the deliveryPolicy parameter.

| Attribute | Description |
| --- | --- |
| retryLimit | The number of retries by the Broker to connected subscribers before assuming failure of the notification. |

Table 6.4.1.1.1..2-1 Parameter: deliveryPolicy

###### 6.4.1.1.1.3	Message Retainment Policy

When a message is published, the M2M Service Layer, using the resources of the underlying Broker, attempts to retain messages according to a retainment policy. Retainment of messages permits AEs that are subscribed to the publication resource but not connected to the underlying Broker to receive messages, up to the retainLimit once the AE connects to the Broker. The message retainment policy is defined in the retainmentPolicy parameter.

| Attribute | Description |
| --- | --- |
| retainLimit | The maximum number of messages retained by the Broker for a subscriber that is not connected to the underlying Broker. |

Table 6.4.1.1.1..3-1 Parameter: retainmentPolicy

###### 6.4.1.1.1.4	Service Subscription Integration

In order for the Data Exchange Service to take advantage of the underlying transport, there shall be an association between the AE and the identifier of the underlying transport within the context of the Service Subscription.

### 6.4.2	Service Capabilities

#### 6.4.2.1	subscribe

This service  capability provides the ability for AEs to subscribe to receive payloads based on a publication resource.

As part of the subscription, the subscribing AE can provide a deliveryPolicy to enhance the robustness of the notification to AEs that have subscribed to the publication resource identified in the request.

##### 6.4.2.1.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Service Subscription, from Originator and Broker exist.

##### 6.4.2.1.2	Signature - subscribe

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.1.2-1 Data Exchange Service – subscribe capability

##### 6.4.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the subscribe request to the Supporting Service to validate the request and obtain the Broker instance for the transfer of the payload.

- Issue the request to the broker to subscribe the from AE-ID for the publicationResource

- Issue the subscribe complete subscription request to the Supporting Service

Table 6.4.2.1.2-1 Data Exchange Service – subscribe capability

##### 6.4.2.1.4	Post-Conditions

Not Applicable

##### 6.4.2.1.5	Exceptions

Not Applicable

##### 6.4.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.2.1.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

#### 6.4.2.2	publish

This service  capability provides the ability for AEs to publish payloads to a leaf node of a publication resource.

As part of the publication, the publishing AE can provide delivery and retainment policies to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.4.2.2.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Service Subscription, from AE-ID and Broker exist.

##### 6.4.2.2.2	Signature – publish

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| payload | IN | NO | The payload to published. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.2.2-1 Data Exchange Service – publish capability

##### 6.4.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the publish request to the Supporting Service to validate the request

- Issue the request to the broker to publish the payload from the from AE-ID to the toResource

- Issue the complete publication request to the Supporting Service

Table 6.4.2.2.2-1 Data Exchange Service –publish (In-Out) capability

##### 6.4.2.2.4	Post-Conditions

Not Applicable

##### 6.4.2.2.5	Exceptions

Not Applicable

##### 6.4.2.2.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying transport layer protocol. Response types or Exceptions are not returned in this message exchange pattern.

Transaction Pattern: Participation allowed

##### 6.4.2.2.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

#### 6.4.2.3	notify

This service capability provides the ability to notify AEs that have subscribed to the publication resource identified in the request.

##### 6.4.2.3.1	Pre-conditions

A correlation between a Service Subscription, to AE-ID and Broker exist.

##### 6.4.2.3.2	Signature – notify

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| to | IN | NO | See Table A.2.1.1.2-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| payload | IN | NO | The Payload of the message to notify to the AE |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.3.2-1 Data Exchange Service – notify capability

##### 6.4.2.3.3	Post-Conditions

Not Applicable

##### 6.4.2.3.4	Exceptions

Not Applicable

##### 6.4.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.2.3.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

## 6.5	Broker

### 6.5.1	Overview

The Broker service provides the ability to adapt the Publish-Subscribe-Notify data exchange patterns to the underlying transport protocol and deployment configuration.

The Broker maintains the subscriptions to the publication resources

### 6.5.2	Service Capabilities

#### 6.5.2.1	subscribe

This service  capability provides the ability for AEs to subscribe to receive payloads based on a publication resource.

As part of the publication, the publishing AE can provide a deliveryPolicy to enhance the robustness of the notification to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.1.1	Pre-conditions

A correlation between a Service Subscription, from AE-ID and Broker exist.

##### 6.5.2.1.2	Signature – subscribe

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.5.2.2.2-1 Broker – subscribe capability

##### 6.5.2.1.3	Post-Conditions

Not Applicable

##### 6.5.2.1.4	Exceptions

Not Applicable

##### 6.5.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying Transaction Pattern: Participation allowed

#### 6.5.2.2	publish

This service  capability provides the ability for AEs to publish payloads to a leaf node of a publication resource.

As part of the publication, the publishing AE can provide delivery and retainment policies to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.2.1	Pre-conditions

A correlation between a Service Subscription, from AE-ID and Broker exist.

##### 6.5.2.2.2	Signature – publish

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| payload | IN | NO | Payload to be published. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.5.2.2.2-1 Broker – publish capability

##### 6.5.2.3.3	Post-Conditions

Not Applicable

##### 6.5.2.3.4	Exceptions

Not Applicable

##### 6.5.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying Transaction Pattern: Participation allowed

##### 6.5.2.3.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

#### 6.5.2.3	notify

This service  capability provides the ability for the Broker to notify AEs that have subscribed to the publication resource identified in the request.

##### 6.5.2.3.1	Pre-conditions

The Broker maintains the AEs (to AE-ID, clientId) that have subscribed to publicationResources.

The Broker maintains the Service Subscription (subscriptionId) for AE subscriptions to publicationResources.

A correlation between a Service Subscription, to AE-ID and Broker exist.

##### 6.5.2.3.2	Signature – notify

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| payload | IN | NO | See Table A.2.1.1.2-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested toResource - Originator has requested an invalid deliveryPolicy - Exception: Request may not have been completed |

Table 6.5.2.3.2-1 Broker – notify capability

##### 6.5.2.3.3	Service Interactions

For each AE subscribed to the resource perform the interactions of service capabilities required for this service capability:

- Issue the notify request to the Supporting Service to validate the request

- Issue the request to the SE: Data Exchange

- Issue the complete publication request to the Supporting Service

Table 6.5.2.3.2-1 Broker –notify (In-Out) capability

##### 6.5.2.3.4	Post-Conditions

Not Applicable

##### 6.5.2.3.5	Exceptions

Not Applicable

##### 6.5.2.3.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying transport layer protocol. Response types or Exceptions are not returned in this message exchange pattern.

Transaction Pattern: Participation allowed

##### 6.5.2.3.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- TBD

Editors note: The oneM2M Resource Interworking will be identified if a M2M Service Subscription operation is identified

## 6.6	Device Management

### 6.6.1	Overview

The Device Management service provides for the capability for an AE to manage devices across the Mca reference point using supported device management message exchange patterns. This service utilizes management adapters to provide the management functions.

The supported device management message exchanges patterns are:

- Request-Response

- Request- Asynchronous Notification

#### 6.6.1.1	Device Management

The Device Management service permits AEs to request management operations (e.g., device configuration operations, device troubleshooting operations, device firmware management operations, and application software management operations) and provide the result or report/retrieve the status of the operations to the originating AE.

AEs have the option to request to manage a targeted set of devices that include:

- Individual device

- Multiple devices through the use of lists of devices

- A group that designates as list of devices using the groupID.

AEs have option to request management operations to be performed based on a schedule.

AEs have the option to request that responses to requests be delivered synchronous (the request is held until all responses are received) or asynchronously (Reports of the operation are transmitted to the AE when an operation has completed) to the request submission. For asynchronous requests, reports are transmitted based on a report policy configured as part of the original request.

AEs also have the capability to request to manage devices by complex operations (e.g. upgradefirmware), where the M2M service platform can orchestrate management operations (e.g. installfirmware, downloadware) according to the business process.

##### 6.6.1.1.1	Supporting Rules

###### 6.6.1.1.1.1	Report Period Parameter

The reportPolicy parameter is used to determine when a report is sent by the M2M System to the AE. The report period attribute is defined in the reportPolicy parameter.

| Parameter | Description |
| --- | --- |
| reportPeriod | Describes the period which reports for a operation are sent to the AE. |

Table 6.6.1.1.1.1-1 Parameter:reportPolicy

###### 6.6.1.1.1.2	Aggregation Policy Parameter

The aggregationPolicy parameter is used to determine the time period for aggregating the result of the management operation prior to the reports being sent to the AE. The aggregation period attribute is defined in the aggregationPolicy parameter.

| Parameter | Description |
| --- | --- |
| aggregationPeriod | The time period of aggregating reports prior to transmission to the AE. |

Table 6.6.1.1.1.2-1 Parameter: aggregationPolicy

###### 6.6.1.1.1.3	Firmware Report Type

The FirmwareReport is complex type that defines the current state of a firmware management operation for a specific device.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The firmware management operation.(e.g. installFirmware, downloadFirmware) |
| operationResult | YES | The firmware management operation execution result. |
| operationStatus | YES | The firmware management operation execution status. It can be Initiated, Started, Finished, Cancelled, or Deleted. |

Table 6.6.1.1.1.3-1 Type: FirmwareReport

###### 6.6.1.1.1.4	Orchestration Rule Type

The OrchestrationRule is a complex type that describes the target and associated schedule of the request for the execution of the management operation.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | NO | The orchestrated operation. (e.g. installFirmware, downloadFirmware) |
| schedule | YES | The time when to execute firmware operation. If it is not provided, the operation shall be execute immediately. |

Table 6.6.1.1.1.4-1 Type: OrchestrationRule

###### 6.6.1.1.1.5	Firmware Info Type

The FirmwareInfo is a complex type that describes the version, name, URL of the device firmware.

| Parameter | Optional | Description |
| --- | --- | --- |
| version | YES | The version of the firmware. |
| name | YES | The name of the firmware to be used on the device. |
| url | YES | The URL from which the firmware image can be downloaded. |

Table 6.6.1.1.1.5-1 Type: FirmwareInfo

###### 6.6.1.1.1.6	Service Subscription Integration

In order for the Device Management Service to take advantage of the Management Adapter, there shall be association between the device associated with a Service Subscription and the service capability to the Management Adapter.

###### 6.6.1.1.1.7	Device Info Type

The DeviceInfo is a complex type that describes the device label, manufacturer, model, device type, firmware version, software version, hardware version.

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceLabel | YES | Unique device label assigned by the manufacturer. The uniqueness may be global or only valid within a certain domain (e.g. vendor-wise or for a certain deviceType). |
| manufacturer | YES | The name/identifier of the device manufacturer. |
| model | YES | The name/identifier of the device mode assigned by the manufacturer. |
| deviceType | YES | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. |
| fwVersion | YES | The firmware version of the device. NOTE: If the device only supports one kind of Software this is identical to softwareVersion. |
| swVersion | YES | The software version of the device. |
| hwVersion | YES | The hardware version of the device. |

Table 6.6.1.1.1.7-1 Type: DeviceInfo

###### 6.6.1.1.1.8	Memory Type

The Memory is a complex type that describes the current available amount of memory and the total amount of memory.

| Parameter | Optional | Description |
| --- | --- | --- |
| memAvailable | YES | The current available amount of memory in bytes. |
| memTotal | YES | The total amount of memory in bytes. |

Table 6.6.1.1.1.8-1 Type: Memory

###### 6.6.1.1.1.9	Battery Type

The Battery is a complex type that describes the current battery level and the status of the battery.

| Parameter | Optional | Description |
| --- | --- | --- |
| level | YES | The current battery level as a percent of the battery capacity |
| status | YES | Indicates the status of the battery. Enum BatteryStatus, see 6.6.1.1.1.11. |

Table 6.6.1.1.1.9-1 Type: Battery

###### 6.6.1.1.1.10	DeviceCapability Type

The DeviceCapability is a complex type that describes the name of the capability, attach label, capability action status.

| Parameter | Optional | Description |
| --- | --- | --- |
| name | YES | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| attached | YES | Indicates whether the capability is attached to the device or not. |
| state | NO | Indicates if the capability is enabled or disabled |

Table 6.6.1.1.1.10-1 Type: DeviceCapability

###### 6.6.1.1.1.11	BatteryStatus Enum

| Battery Status | Description |
| --- | --- |
| Normal | The battery is operating normally and not on power. |
| Charging | The battery is currently charging. |
| Charge Complete | The battery is fully charged and still on power. |
| Damaged | The battery has some problem. |
| Low Battery | The battery is low on charge. |
| Not Installed | The battery is not installed. |
| Unknown | The battery information is not available. |

Table 6.6.1.1.1.11-1 Enum: BatteryStatus

###### 6.6.1.1.1.12	DeviceCapabilityName Enum

| Device Capability Name | Description |
| --- | --- |
| Hardware: ExternalMemory | Used to enable/disable the removable card memory or storage disk. |
| Hardware: Display | Used to enable/disable the display screen. |
| Hardware: Camera | Used to enable/disable the camera. |
| Hardware: Speaker | Used to enable/disable the speaker. |
| I/O: Tethering | Used to enable/disable the device to be attached to other devices. |
| I/O: AudioInputEncoder | Used to enable/disable the audio input encoder. |
| I/O: AttachedDevice | Used to enable/disable the capability to allow other devices to be attached to this device. |
| I/O: Keyboard | Used to enable/disable the keyboard |
| I/O: InputPeripheral | Used to enable/disable the input peripheral. |
| I/O: OutputPeripheral | Used to enable/disable the output peripheral. |
| I/O: USB | Used to enable/disable the USB port. |
| I/O: SerialPort | Used to enable/disable the serial port. |
| I/O: ParallelPort | Used to enable/disable the Parallel port. |
| I/O: GPS | Used to enable/disable GPS capability |
| I/O: GNSS | Used to enable/disable GNSS capability |
| Connectivity: Bluetooth | Used to enable/disable the Bluetooth connectivity. |
| Connectivity: WLAN | Used to enable/disable the WLAN connectivity. |
| Connectivity: Infrared | Used to enable/disable the Infrared connectivity. |
| Connectivity: WCDMA | Used to enable/disable the WCDMA connectivity |
| Connectivity: GPRS | Used to enable/disable the GPRS connectivity |
| Connectivity: EDGE | Used to enable/disable the EDGE connectivity |
| Connectivity: CDMA | Used to enable/disable the CDMA connectivity |
| Connectivity: WiMAX | Used to enable/disable the WiMAX connectivity |
| Connectivity: LTE | Used to enable/disable the LTE connectivity |
| Connectivity: NFC | Used to enable/disable the NFC connectivity |
| Connectivity: TD-SCDMA | Used to enable/disable the TD-SCDMA connectivity |
| LAN Interfaces: USB | Used to enable/disable the USB Interface |
| LAN Interfaces: Wi-Fi | Used to enable/disable the Wi-Fi Radio |
| LAN Interfaces: HomePlug | Used to enable/disable the HomePlug Interface |
| LAN Interfaces: MoCA | Used to enable/disable the MoCA Interface |
| LAN Interfaces: UPA | Used to enable/disable the UPA Interface |
| Hardware: SmartCardReader | Used to enable/disable the SmartCardReader |

Table 6.6.1.1.1.11-1 Enum: DeviceCapabilityName

###### 6.6.1.1.1.13	LockStatus Enum

| Lock Status | Description |
| --- | --- |
| locked | The device is locked. |
| unlocked | The device is unlocked. |

Table 6.6.1.1.1.11-1 Enum: LockedStatus

###### 6.6.1.1.1.14	Area Network Info Type

The AreaNwkInfo is a complex type that describes the information about a M2M area network. Information includes the area network identifier, area network name, area network type, and the list of devices in the M2M Area Network.

| Parameter | Optional | Description |
| --- | --- | --- |
| areaNwkId | NO | The unique area network identifier. |
| areaNwkName | YES | The Area Network name. |
| areaNwkType | YES | The areaNwkType is an implementation-chosen string that indicates the type of M2M Area Network. |
| listOfDevices | YES | Indicates the list of devices in the M2M Area Network. Type AreaNwkDevice, see 6.6.1.1.1.15. From listOfDevices, the topology of the area network can be discovered and retrieved. |

Table 6.6.1.1.1.14-1 Type: AreaNwkInfo

###### 6.6.1.1.1.15	Area Network Device Type

The AreaNwkDevice is a complex type that describes the device information for a device in the context of a M2M Area Network. Information includes the device identifier, area network identifier and the area network device specific information.

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | YES | The area network device information. Type AreaNwkDeviceInfo, see 6.6.1.1.1.16. |

Table 6.6.1.1.1.15-1 Type: AreaNwkDevice

###### 6.6.1.1.1.16	Area Network Device Info Type

The AreaNwkDeviceInfo is a complex type that describes information about the device in the context of the M2M Area Network. Information includes the type of device, the interval of time between two sleep cycles, the time duration of each sleep cycle, the status of the device and any neighbor devices.

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceType | YES | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. |
| sleepInterval | YES | The interval between two sleep cycles. |
| sleepDuration | YES | The time duration of each sleep cycle. |
| status | YES | The status of the device (sleeping or waked up). |
| listOfNeighbors | YES | Indicates the neighbour devices of the same area network. When modified, the connection relationship of the devices shall be modified accordingly. |

Table 6.6.1.1.1.16-1 Type: AreaNwkDeviceInfo

### 6.6.2	Service Capabilities

#### 6.6.2.1	downloadFirmware

This service capability provides the ability for AEs to download device firmware.

##### 6.6.2.1.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.1.2	Signature - downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter - Operation exception: Firmware miss dependency - Operation exception: The new firmware is too large for available program or data memory |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.1.2-1 Device Management Service –downloadFirmware capability

##### 6.6.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.1.3-1 downloadFirmware Diagram

##### 6.6.2.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

##### 6.6.2.1.5	Exceptions

- Missing dependency within the device for the firmware

- New firmware is too large for available program or data memory

When invoking the service capability meet these exceptions, the response feedback and no subsequence notifications. AE can continue issue the request after M2M Area network meet the requirement or invoke other service capabilities to troubleshoot related devices.

Editors note: The highlighted text is unclear; we need to clarify the exception handling

##### 6.6.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.1.7	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.2	installFirmware

This service capability provides the ability for AEs to install firmware on the device.

##### 6.6.2.2.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

- The firmware to be installed exists on the device.

##### 6.6.2.2.2	Signature –installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter - Operation exception: Firmware is missing dependency on the device - Operation exception: The new firmware is too large for available program or data memory |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.2.2-1 Device Management Service –installFirmware capability

##### 6.6.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.2.3-1 installFirmware Diagram

##### 6.6.2.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

##### 6.6.2.2.5	Exceptions

- Missing dependency of the firmware on the device

- New firmware is too large for available program or data memory

When invoking the service capability meet these exceptions, the response feedback and no subsequence notifications. AE can continue issue the request after M2M Area network meet the requirement or invoke other service capabilities to troubleshoot related devices.

Editors note: The highlighted text is unclear; we need to clarify the exception handling

##### 6.6.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.2.7	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.3 	getFirmwareInformation

This service capability provides the ability for AEs to get device firmware information.

##### 6.6.2.3.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.3.2	Signature –getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |

Table 6.6.2.3.2-1 Device Management Service –getFirmwareInformation capability

##### 6.6.2.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.3.3-1 getFirmwareInformation Diagram

##### 6.6.2.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

##### 6.6.2.3.5	Exceptions

Not Applicable

##### 6.6.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.3.7	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

#### 6.6.2.4 	getFirmwareExecStatus

This service capability provides the ability for AEs to get state of firmware management operation for a specific device.

##### 6.6.2.4.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and previously submitted firmware request exist.

##### 6.6.2.4.2	Signature –getFirmwareExecInstance

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy is not supported by the Management Adapter - The previously submitted firmware request does not exist |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.4.2-1 Device Management Service –getFirmwareExecInstance capability

##### 6.6.2.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.3.3-1 getFirmwareExecStatus Diagram

##### 6.6.2.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware state of management operation.

##### 6.6.2.4.5	Exceptions

Not Applicable

##### 6.6.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.4.7	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

#### 6.6.2.5	reportFirmwareStatus

This service capability provides the ability to report the firmware management operation execution result or status information to the AE.

##### 6.6.2.5.1	Pre-conditions

A correlation between a Management Adapter, the service capability and previously submitted firmware request exist.

##### 6.6.2.5.2	Signature –reportFirmwareStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The target AE does not exist. - Aggregation policy does not supported |

Table 6.3.2.5.2-1 Device Management Service –reportFirmwareStatus capability

##### 6.6.2.5.3	Post-Conditions

The AE has received a report of firmware operation execution status or result.

Editor’s note; There are post conditions that requires the M2M System to maintain the “state” of the report (i.e., sequence number.

##### 6.6.2.5.4	Exceptions

Not Applicable

##### 6.6.2.5.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.5.6	oneM2M Resource Interworking

Not Applicable

#### 6.6.2.6	upgradeFirmware

This service capability is a complex operation for AEs to upgrade firmware on one or more devices. The service capability utilizes orchestration rules for the related management operations such as downloadFirmware, installFirmware, getFirmwareInformation, getFirmwareExecInstance and reportFirmwareStatus according to the business process.

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations.

##### 6.6.2.6.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between Management Adapter, the service capability and devices or device group exist.

##### 6.6.2.6.2	Signature - upgradeFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.1.1.1.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter - Aggregation policy is not supported by the Management Adapter - The Orchestration Rules are not supported by the Management Adapter - Operation exception: The device’s reachability schedule is inconsistent with the schedule of the Orchestration rule. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReportList | OUT | YES | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.6.2-1 Device Management Service –upgradeFirmware capability

Editors Note: Need to clarify the device’s reachability schedule.

##### 6.6.2.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.1.6-1 upgradeFirmware Diagram

##### 6.6.2.6.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to manage firmware.

##### 6.6.2.6.5	Exceptions

- The device’s reachability schedule is inconsistent with the schedule of the Orchestration rule.

##### 6.6.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.6.7	oneM2M Resource Interworking

Not Applicable

#### 6.6.2.7	getDeviceInformation

This service capability provides the ability for AEs to get device information.

##### 6.6.2.7.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.7.2	Signature - getDeviceInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |
| deviceInfo | OUT | NO | The device information. Type DeviceInfo, see 6.6.1.1.1.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.1.1.1.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.1.1.1.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.1.1.1.13. |

Table 6.6.2.7.2-1 Device Management Service –getDeviceInformation capability

##### 6.6.2.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.7.3-1 getDeviceInformation Diagram

##### 6.6.2.7.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device information.

Based on the capabilities supported by the device, the memory, battery and lock status information may or may not be returned.

##### 6.6.2.7.5	Exceptions

Not Applicable

##### 6.6.2.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.7.7	oneM2M Resource Interworking

This service capability is used to get device information. The service capability aligns with the <deviceInfo> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <memory> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <battery> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.8	getDeviceCapabilities

This service capability provides the ability for AEs to get the device capabilities.

##### 6.6.2.8.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.8.2	Signature –getDeviceCapabilities

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |
| deviceCapabilities | OUT | YES | List of deviceCapability. Type DeviceCapability, see 6.6.1.1.1.10. |

Table 6.6.2.8.2-1 Device Management Service –getDeviceCapabilities capability

##### 6.6.2.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.8.3-1 getDeviceCapabilities Diagram

##### 6.6.2.8.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

##### 6.6.2.8.5	Exceptions

Not Applicable

##### 6.6.2.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.8.7	oneM2M Resource Interworking

This service capability is used to get device capabilities. The service capability aligns with the<deviceCapability> resource and maps to the RETRIEVE procedure for the resource.

#### 6.6.2.9 	enableDeviceCapability

This service capability provides the ability for AEs to enable device capability.

##### 6.6.2.9.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.9.2	Signature –enableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.9.2-1 Device Management Service –enableDeviceCapability capability

##### 6.6.2.9.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.9.3-1 enableDeviceCapability Diagram

##### 6.6.2.9.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

##### 6.6.2.9.5	Exceptions

Not Applicable

##### 6.6.2.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.9.7	oneM2M Resource Interworking

This service capability is used to enable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute enable of the resource.

#### 6.6.2.10	disableDeviceCapability

This service capability provides the ability for AEs to disable device capability.

##### 6.6.2.10.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.10.2	Signature –disableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.10.2-1 Device Management Service –disableDeviceCapability capability

##### 6.6.2.10.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.10.3-1 disableDeviceCapability Diagram

##### 6.6.2.10.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

##### 6.6.2.10.5	Exceptions

Not Applicable

##### 6.6.2.10.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.10.7	oneM2M Resource Interworking

This service capability is used to disable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute disable of the resource.

#### 6.6.2.11	getAreaNetworks

This service capability provides the ability for AEs to get area networks information.

##### 6.6.2.11.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and area network exist.

##### 6.6.2.11.2	Signature - getAreaNetworks

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |
| AreaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.1.1.1.14. |

Table 6.6.2.11.2-1 Device Management Service –getAreaNetworks capability

##### 6.6.2.11.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.11.3-1 getAreaNetworks Diagram

##### 6.6.2.11.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

##### 6.6.2.11.5	Exceptions

Not Applicable

##### 6.6.2.11.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.11.7	oneM2M Resource Interworking

This service capability is used to get area networks information. The service capability aligns with the <areaNwkInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.12	updateDeviceForAreaNetwork

This service capability provides the ability for AEs to update device information for area network.

##### 6.6.2.12.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability, the device and area network exist.

##### 6.6.2.12.2	Signature –updateDeviceForAreaNetwork

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.1.1.1.16. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |
| lastModifiedTime | OUT | NO | The modified time. |

Table 6.6.2.12.2-1 Device Management Service –updateDeviceForAreaNetwork capability

##### 6.6.2.12.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.12.3-1 updateDeviceForAreaNetwork Diagram

##### 6.6.2.12.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

##### 6.6.2.12.5	Exceptions

Not Applicable

##### 6.6.2.12.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.12.7	oneM2M Resource Interworking

The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Update procedure for attribute devType , sleepInterval , sleepDuration , status , listOfNeighbors of the resource.

## 6.7	Management Adapter

### 6.7.1	Overview

The Management Adapter service provides the ability to adapt the M2M Service Layer operations to the technology specific operations of the Management Server.

### 6.7.2	Service Capabilities

#### 6.7.2.1	downloadFirmware

This service capability provides the ability to download the specific device firmware.

##### 6.7.2.1.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.1.2	Signature – downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.7.2.1.2-1 Management Adapter –downloadFirmware capability

##### 6.7.2.1.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download the firmware.

##### 6.7.2.1.4	Exceptions

- New firmware is too large for available program or data memory

When these exceptions are thrown, the requested operation is completed and any corresponding reports for the device are not sent.

Editors Note: These exceptions should be considered as whether they are exceptions or should be error response code. Exceptions stop processing immediately; errors response complete the request-response interaction. Both will be reported to the target AE.

##### 6.7.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.1.6	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.2	installFirmware

This service capability provides the ability to install firmware on a device.

##### 6.7.2.2.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.2.2	Signature –installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.7.2.2.2-1 Management Adapter –installFirmware capability

##### 6.7.2.2.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

##### 6.7.2.2.4	Exceptions

- Missing dependency in the device for the firmware and requested operation

- New firmware is too large for available program or data memory

When these exceptions are thrown, the requested operation is completed and any corresponding reports for the device are not sent.

##### 6.7.2.2.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.2.6	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.3	getFirmwareInformation

This service capability provides the ability to retrieve the information related to the firmware on a device..

##### 6.7.2.3.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.3.2	Signature –getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |

Table 6.7.2.3.2-1 Management Adapter –getFirmwareInformation capability

##### 6.7.2.3.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware information.

##### 6.7.2.3.4	Exceptions

Not Applicable

##### 6.7.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.3.6	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

#### 6.7.2.4	getFirmwareExecStatus

This service capability provides the ability to get state of firmware management operation for a specific device.

##### 6.7.2.4.1	Pre-conditions

A correlation between a Management Server, the service capability and previously submitted firmware request exist.

##### 6.7.2.4.2	Signature –getFirmwareExecInstance

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. When provided the FirmwareReport type is generated for only the specified device. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The previously submitted firmware request does not exist |
| firmwareReports | OUT | YES | The list of firmware management operation execution result or status for each device in the operation request. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.7.2.4.2-1 Management Adapter –getFirmwareExecStatus capability

##### 6.7.2.4.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware operation execution status or result for each device in the operation request.

##### 6.7.2.4.4	Exceptions

Not Applicable

##### 6.7.2.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.4.6	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

#### 6.7.2.5	reportFirmwareStatus

This service capability provides the ability for the Management Adapter to report the firmware management operation execution result or status information. The Management Adapter can send one or multiple firmware reports according to the aggregation policy.

##### 6.7.2.5.1	Pre-conditions

A correlation between a Management Server, the service capability and previously submitted firmware requests exist.

##### 6.7.2.5.2	Signature –reportFirmwareStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| to | IN | NO | The identifier of the AE that is to receive the data(M2M-AppInst-ID) |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The target AE does not exist. - Aggregation policy does not supported |

Table 6.7.2.5.2-1 Management Adapter –reportFirmwareStatus capability

##### 6.7.2.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.7.2.5.3-1 firmwareReport Diagram

##### 6.7.2.5.4	Post-Conditions

Not Applicable

Editors note: We need to clarify the post conditions around the sequence number of the report.

##### 6.7.2.5.5	Exceptions

Not Applicable

##### 6.7.2.5.6	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

##### 6.7.2.5.7	oneM2M Resource Interworking

Not Applicable

#### 6.7.2.6	getDeviceInformation

This service capability provides the ability to get the specific device information.

##### 6.7.2.6.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.6.2	Signature –getDeviceInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |
| deviceInfo | OUT | YES | The device information. Type DeviceInfo, see 6.6.1.1.1.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.1.1.1.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.1.1.1.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.1.1.1.13. |

Table 6.7.2.6.2-1 Management Adapter –getDeviceInformation capability

##### 6.7.2.6.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the device information.

##### 6.7.2.6.4	Exceptions

Not Applicable

##### 6.7.2.6.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.6.6	oneM2M Resource Interworking

This service capability is used to get device information. The service capability aligns with the <deviceInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <memory> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <battery> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.7	getDeviceCapabilities

This service capability provides the ability to get device capabilities.

##### 6.7.2.7.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.7.2	Signature –getDeviceCapabilities

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |
| deviceCapabilities | OUT | YES | Array of deviceCapability. Type DeviceCapability, see 6.6.1.1.1.10. |

Table 6.7.2.7.2-1 Management Adapter –getDeviceCapabilities capability

##### 6.7.2.7.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

##### 6.7.2.7.4	Exceptions

Not Applicable

##### 6.7.2.7.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.7.6	oneM2M Resource Interworking

This service capability is used to get device capabilities. The service capability aligns with the<deviceCapability> resource and maps to the RETRIEVE procedure for the resource.

#### 6.7.2.8	enableDeviceCapability

This service capability provides the ability to enable device capability.

##### 6.7.2.8.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.8.2	Signature –enableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.8.2-1 Management Adapter –enableDeviceCapability capability

##### 6.7.2.8.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

##### 6.7.2.8.4	Exceptions

Not Applicable

##### 6.7.2.8.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.8.6	oneM2M Resource Interworking

This service capability is used to enable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute enable of the resource.

#### 6.7.2.9	disableDeviceCapability

This service capability provides the ability to disable device capability.

##### 6.7.2.9.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.9.2	Signature –disableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.9.2-1 Management Adapter –disableDeviceCapability capability

##### 6.7.2.9.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

##### 6.7.2.9.4	Exceptions

Not Applicable

##### 6.7.2.9.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.9.6	oneM2M Resource Interworking

This service capability is used to disable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute disable of the resource.

#### 6.7.2.10	getAreaNetworks

This service capability provides the ability to get area networks information.

##### 6.7.2.10.1	Pre-conditions

A correlation between a Management Server, the service capability and area network exist.

##### 6.7.2.10.2	Signature –getAreaNetworks

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |
| AreaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.1.1.1.13. |

Table 6.7.2.10.2-1 Management Adapter –getAreaNetworks capability

##### 6.7.2.10.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

##### 6.7.2.10.4	Exceptions

Not Applicable

##### 6.7.2.10.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.10.6	oneM2M Resource Interworking

This service capability is used to get area networks information. The service capability aligns with the <areaNwkInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.12	updateDeviceForAreaNetwork

This service capability provides the ability for the Management Adapter to update device information for area network.

##### 6.7.2.12.1	Pre-conditions

A correlation between a Management Server, the service capability, the device and area network exists.

##### 6.7.2.12.2	Signature –updateDeviceForAreaNetwork

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.1.1.1.15. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |
| lastModifiedTime | OUT | NO | The modified time. |

Table 6.7.2.16.2-1 Management Adapter –updateDeviceForAreaNetwork capability

##### 6.7.2.12.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

##### 6.7.2.12.4	Exceptions

Not Applicable

##### 6.7.2.12.5	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

##### 6.7.2.12.6	oneM2M Resource Interworking

The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Update procedure for attribute devType , sleepInterval , sleepDuration , status , listOfNeighbors of the resource.

## 6.8	Supporting Service Administration

### 6.8.1	Overview

The Supporting Service Administration service provides the administrate the following across the Msc reference points:

- Maintenance of Supporting Services

- Association of Service Roles

#### 6.8.1.1	Supporting Service Entity

The Supporting Service is a primary entity that provides the attributes to identify the Supporting Service and associated Service Roles within the oneM2M System. These Supporting Services and associated Service Roles shall be maintained by the M2M Service Provider.

| Attribute name | Description |
| --- | --- |
| serviceId | The Supporting Service Identifier (M2M-Serv-ID). |
| labels | The List of labels used as filter criteria for the Service Subscription |
| serviceRoleIds | List of Service Role Identifiers (Role-ID) associated with the Service Subscription. |
| lastModifiedTime | See Table A.2.1.1.2-1 |
| creationTime | See Table A.2.1.1.2-1 |

Table 6.8.1.1-1 Supporting Service Entity

##### 6.8.1.1.1	M2M Service Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceId | The Supporting Service Identifier (M2M-Serv-ID). |
| labels | One or more labels assigned to the Supporting Service entity. |
| serviceRoleIds | One or more Service Role Identifiers (Role-ID) associated with the Supporting Service entity. |
| lastModifiedTime | See Table A.2.1.1.2-1 |
| creationTime | See Table A.2.1.1.2-1 |

Table 6.8.1.1.1-1 Supporting Service Filter Criteria

### 6.8.2	Service Capabilities

#### 6.8.2.1	createSupportingService

This service capability provides the ability to create a Supporting Service.

##### 6.8.2.1.1	Pre-conditions

Not Applicable

##### 6.8.2.1.2	Signature - createSupportingService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| name | IN | NO | The unique name of the Supporting Service. |
| description | IN | YES | The description of the Supporting Service. |
| serviceId | OUT | NO | The identifier of the created M2M Service (M2M-Serv-ID) |
| responseType | OUT | YES | Unique response types for this service. - Supporting Service exists for the name parameter - Supporting Service Role exists for serviceRole name parameter |

Table 6.4.2.1.2-1 Supporting Service Administration – createSupportingService capability

##### 6.4.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the createSupportingService capability

Table 6.8.2.1.3-1 Supporting Service Administration – createSupportingService

##### 6.4.2.1.4	Post-Conditions

Not Applicable

##### 6.4.2.1.5	Exceptions

Not Applicable

##### 6.4.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.2.1.7	oneM2M Resource Interworking

This service capability creates the Supporting Service (M2M Service). The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2]. The M2M-Serv-ID created using this capability shall map to the M2M-Serv-ID of the oneM2M Functional Architecture [2].

#### 6.8.3.1	deleteSupportingService

This service capability provides the ability to delete a Supporting Service. This service capability shall be restricted to the Msc reference point.

##### 6.8.3.1.1	Pre-conditions

The Supporting Service’s Service Roles are not associated with any Service Subscriptions.

##### 6.8.3.1.2	Signature – deleteSupportingService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| responseType | OUT | YES | Unique response types for this service. - Service does not exist - Service has Service Roles associated with Service Subscriptions |

Table 6.8.3.1.2-1 Supporting Service Administration – deleteSupportingService capability

##### 6.8.3.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteSupportingService capability

Table 6.8.3.1.3-1 Supporting Service Administration – deleteSupportingService

##### 6.8.3.1.4	Post-Conditions

The Supporting Service is deleted along with any associations of the has to the Service Roles. The Node information associated with the Supporting Service is also deleted.

##### 6.8.3.1.5	Exceptions

Not Applicable

##### 6.8.3.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.1.7	oneM2M Resource Interworking

This service capability deletes the Supporting Service (M2M Service). The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.4.1	updateSupportingService

This service capability provides the ability to update a Supporting Service. This service capability shall be restricted to the Msc reference point.

##### 6.8.4.1.1	Pre-conditions

Not Applicable

##### 6.8.4.1.2	Signature – updateSupportingService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| labels | IN | YES | See Table A.2.1.1.2-1 The existing labels are replaced with the labels in this parameter. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist |

Table 6.8.4.1.2-1 Supporting Service Administration – updateSupportingService capability

##### 6.8.4.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateSupportingService capability

Table 6.8.4.1.3-1 M2M Service Administration – updateSupportingService

##### 6.8.4.1.4	Post-Conditions

Not Applicable

##### 6.8.4.1.5	Exceptions

Not Applicable

##### 6.8.4.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.4.1.7	oneM2M Resource Interworking

This service capability updates the Supporting Service (M2M Service). The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.5.1	addRoleToSupportingService

This service capability provides the ability to add new Service Roles to a Supporting Service. This service capability shall be restricted to the Msc reference point.

##### 6.8.5.1.1	Pre-conditions

M2M Service exists.

##### 6.8.5.1.2	Signature – addRoleToSupportingService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (Role-ID) to be associated with the Supporting Service. If a Role-ID in the parameter is already associated with the Supporting Service, nothing is done for the Role-ID as it already is associated. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service does not exist |

Table 6.8.5.1.2-1 Supporting Service Administration – addRoleToSupportingService capability

##### 6.8.5.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addRoleToSupportingService capability

Table 6.8.5.1.3-1 Supporting Service Administration – addRoleToSupportingService

##### 6.8.5.1.4	Post-Conditions

The Supporting Service is updated with any associations of the Supporting Service has to the Service Roles.

##### 6.8.5.1.5	Exceptions

Not Applicable

##### 6.8.5.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.5.1.7	oneM2M Resource Interworking

This service capability updates the Supporting Service (M2M Service) with the respective M2M Service Roles. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.6.1	deleteRoleFromSupportingService

This service capability provides the ability to delete existing Service Roles from a Supporting Service. This service capability shall be restricted to the Msc reference point.

##### 6.8.6.1.1	Pre-conditions

The Service Role is not associated with any Service Subscriptions.

##### 6.8.6.1.2	Signature – deleteRoleFromSupportingService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The Supporting Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (Role-ID) to be deleted from the association with the Supporting Service. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service does not exist - Service Role does not exist - Service Role is associated with a Service Subscription |

Table 6.8.6.1.2-1 Supporting Service Administration – deleteRoleFromSupportingService capability

##### 6.8.6.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteRoleFromSupportingService capability

Table 6.8.6.1.3-1 Supporting Service Administration – deleteRoleFromSupportingService

##### 6.8.6.1.4	Post-Conditions

The Supporting Service is updated with associations of the identified Service Roles deleted.

##### 6.8.6.1.5	Exceptions

Not Applicable

##### 6.8.6.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.6.1.7	oneM2M Resource Interworking

This service capability updates the Supporting Service (M2M Service) with the respective Service Roles removed. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.7.1	getSupportingService

This service capability provides the ability to retrieve the existing Supporting Service. This service capability shall be restricted to the Msc reference points.

##### 6.8.7.1.1	Pre-conditions

Not Applicable

##### 6.8.7.1.2	Signature – getSupportingService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.8.1.1.1-1 |
| services | OUT | NO | The resulting Supporting Service entities in Table 6.8.1.1-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.8.7.1.2-1 Supporting Service Administration – getSupportingService capability

##### 6.8.7.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getSupportingService capability

Table 6.8.7.1.3-1 Supporting Service Administration – getSupportingService capability

##### 6.8.7.1.4	Post-Conditions

Not Applicable

##### 6.8.7.1.5	Exceptions

Not Applicable

##### 6.8.7.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.7.1.7	oneM2M Resource Interworking

This service capability retrieves the Supporting Service (M2M Service). The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

## 6.9	Service Subscription Administration

Editors Note: Need to develop a Section that delineates the request attributes needed by the Mca vs. the Msc.

Editors Note: Need to determine how the entity instance based access management is configured in order to ensure no cross ASP access to other ASPs data.

Editors Note: Need a clause on the language that is permitted for evaluation of Filter Criteria

Editors Note: Need to consider which service capabilities are exposed across the Mcc’ reference point and what the request looks like.

### 6.9.1	Overview

The Service Subscription Administration service provides the capability to administrate the following across the Mca and Msc reference points:

- Maintenance of Services Subscriptions

- Association of information for devices and Applications to the Service Subscription

The Service Subscription defines the technical part of the contract between a M2M Subscriber (typically an M2M Application Service Provider) and a M2M Service Provider. Each Service Subscription has a unique identifier (M2M-Sub-ID).

A Service Subscription establishes an association between one or more M2M Applications and one or more devices (M2M Nodes). In addition, the Service Subscription is associated with one or more Service Roles.

A Service Subscription shall be used for the following purposes:

- Serve as a basis for authorization for service capabilities

- Serve as the basis for charging

- Identify which devices and M2M Applications are part of this Service Subscription

#### 6.9.1.1	Service Subscription Entity

| Attribute name | Description |
| --- | --- |
| serviceSubscriptionId | The Service Subscription Identifier (M2M-Sub-ID). |
| labels | List of labels used as filter criteria for the Service Subscription |
| serviceRoleIds | List of Service Role Identifiers (Role-ID) associated with the Service Subscription. |
| lastModifiedTime | See Table A.2.1.1.2-1 |
| creationTime | See Table A.2.1.1.2-1 |

Table 6.9.1.1-1 Service Subscription Entity

##### 6.9.1.1.1	Service Subscription Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceSubscriptionId | The Service Subscription Identifier (M2M-Sub-ID). |
| labels | List of labels assigned to the Service Subscription entity. |
| serviceRoleIds | List of Service Role Identifiers (Role-ID) associated with the Service Subscription entity. |
| lastModifiedTime | See Table A.2.1.1.2-1 |
| creationTime | See Table A.2.1.1.2-1 |

Table 6.9.1.1.1-1 Service Subscription Filter Criteria

#### 6.9.1.2	Device Entity for Service Subscription

| Attribute name | Description |
| --- | --- |
| serviceSubscriptionId | The Service Subscription Identifier (M2M-Sub-ID) for the Device. |
| deviceId | The unique device identifier in the context of the Service Subscription. |
| externalIds | List of URNs that represent the external identifiers associated with this device. |
| applicationIds | List of M2M Application Identifiers (App-ID) associated with the device. |

Table 6.9.1.2-1 Device Entity for Service Subscription

#### 6.9.1.2.1	Device Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceSubscriptionId | The Service Subscription Identifier (M2M-Sub-ID) for the Device. |
| deviceId | The unique device identifier in the context of the Service Subscription. |
| externalIds | List of URNs that represent the external identifiers associated with this device. |
| applicationIds | List of M2M Application Identifiers (App-ID) associated with the device. |

Table 6.9.1.2.1-1 Device Filter Criteria

### 6.9.2	Service Capabilities

#### 6.9.2.1	createServiceSubscription

This service capability provides the ability to create a Service Subscription. This service capability shall be restricted to the Msc reference point.

##### 6.9.2.1.1	Pre-conditions

Not Applicable

##### 6.9.2.1.2	Signature – createServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN-OUT | YES | The Service Subscription Identifier (M2M-Sub-ID). If the attribute is not provided on input, the oneM2M System shall assign the M2M-Sub-ID. The M2M-Sub-ID is unique for the oneM2M System. |
| labels | IN | YES | See Table A.2.1.1.2-1 |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (Role-ID) to be associated with the Service Subscription. |
| creationTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription exists - Service Role does not exist |

Table 6.9.2.1.2-1 Service Subscription Administration – createServiceSubscription capability

##### 6.9.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the createServiceSubscription capability

Table 6.9.2.1.3-1 Service Subscription Administration – createServiceSubscription

##### 6.9.2.1.4	Post-Conditions

The Service Subscription is created and any Service Roles are associated with the Service Subscription.

##### 6.9.2.1.5	Exceptions

Not Applicable

##### 6.9.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.1.7	oneM2M Resource Interworking

This service capability creates the Service Subscription with the identifier (M2M-Sub-ID) and maps to the CREATE procedure for the <m2mServiceSubscription> resource.

When the Service Subscription is created the creationTime, M2M-Sub-ID and subsServ&RoleList attributes are present.

This service capability utilizes the access management services with the privileges associated with the associated Service Role. As such the accessControlPolicyIDs and subsGroup attributes of the <m2mServiceSubscription> resource is not used within this service capability.

#### 6.9.3.1	deleteServiceSubscription

This service capability provides the ability to delete a Service Subscription. This service capability shall be restricted to the Msc reference point.

##### 6.9.3.1.1	Pre-conditions

Not Applicable

##### 6.9.3.1.2	Signature – deleteServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription Identifier (M2M-Sub-ID). |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist |

Table 6.9.3.1.2-1 Service Subscription Administration – deleteServiceSubscription capability

##### 6.9.3.1.3	Service Interactions

- The interactions of service capabilities required for this service capability:

- Issue the deleteServiceSubscription capability

Table 6.9.3.1.3-1 Service Subscription Administration – deleteServiceSubscription

##### 6.9.3.1.4	Post-Conditions

The Service Subscription is deleted along with any associations of the Service Subscription has to the Service Roles. The Node information (e.g., devices, M2M Applications) associated with the Service Subscription is also deleted.

##### 6.9.3.1.5	Exceptions

Not Applicable

##### 6.9.3.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.1.7	oneM2M Resource Interworking

This service capability deletes the Service Subscription with the identifier (M2M-Sub-ID) and maps to the DELETE procedure for the <m2mServiceSubscription> resource.

#### 6.9.4.1	updateServiceSubscription

This service capability provides the ability to update a Service Subscription. This service capability shall be restricted to the Msc reference point.

##### 6.9.4.1.1	Pre-conditions

Not Applicable

##### 6.9.4.1.2	Signature – updateServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription Identifier (M2M-Sub-ID). |
| labels | IN | YES | See Table A.2.1.1.2-1 The existing labels are replaced with the labels in this parameter. |
| serviceRoleIds | IN | YES | List of Service Role Identifiers (Role-ID) to be associated with the Service Subscription. If supplied, Role-IDs in the list that are not already associated with the Service Subscription are added. Role-IDs that are associated with the Service Subscription but not in the list are deleted. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| roleIdResult | OUT | YES | If the parameter serviceRoleIds is present, this parameter provides the results (added, deleted) for each affected Service Roles. |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist - Service Role does not exist for a Supporting Service |

Table 6.9.4.1.2-1 Service Subscription Administration – updateServiceSubscription capability

##### 6.9.4.1.3	Service Interactions

- The interactions of service capabilities required for this service capability:

- Issue the updateServiceSubscription capability

Table 6.9.4.1.3-1 Service Subscription Administration – updateServiceSubscription

##### 6.9.4.1.4	Post-Conditions

The Service Subscription is updated along with any associations of the Service Subscription has to the Service Roles.

##### 6.9.4.1.5	Exceptions

Not Applicable

##### 6.9.4.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.4.1.7	oneM2M Resource Interworking

This service capability updates the Service Subscription with the identifier (M2M-Sub-ID) and maps to the UPDATE procedure for the <m2mServiceSubscription> resource.

#### 6.9.5.1	addRoleToServiceSubscription

This service capability provides the ability to add new Service Roles to a Service Subscription. This service capability shall be restricted to the Msc reference point.

##### 6.9.5.1.1	Pre-conditions

Not Applicable

##### 6.9.5.1.2	Signature – addRoleToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription Identifier (M2M-Sub-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (Role-ID) to be associated with the Service Subscription. If a Role-ID in the parameter is already associated with the Service Subscription, nothing is done for the Role-ID as it already is associated. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist - Service Role does not exist for a Supporting Service |

Table 6.9.5.1.2-1 Service Subscription Administration – addRoleToServiceSubscription capability

##### 6.9.5.1.3	Service Interactions

- The interactions of service capabilities required for this service capability:

- Issue the addRoleToServiceSubscription capability

Table 6.9.5.1.3-1 Service Subscription Administration – addRoleToServiceSubscription

##### 6.9.5.1.4	Post-Conditions

The Service Subscription is updated with any associations of the Service Subscription has to the Service Roles.

##### 6.9.5.1.5	Exceptions

Not Applicable

##### 6.9.5.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.5.1.7	oneM2M Resource Interworking

Not Applicable

#### 6.9.6.1	deleteRoleFromServiceSubscription

This service capability provides the ability to delete existing Service Roles from a Service Subscription. This service capability shall be restricted to the Msc reference point.

##### 6.9.6.1.1	Pre-conditions

Not Applicable

##### 6.9.6.1.2	Signature – deleteRoleFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription Identifier (M2M-Sub-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (Role-ID) to be deleted from the association with the Service Subscription. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist - Service Role does not exist for a Supporting Service |

Table 6.9.6.1.2-1 Service Subscription Administration – deleteRoleFromServiceSubscription capability

##### 6.9.6.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteRoleFromServiceSubscription capability

Table 6.9.6.1.3-1 Service Subscription Administration – deleteRoleFromServiceSubscription

##### 6.9.6.1.4	Post-Conditions

The Service Subscription is updated with associations of the identified Service Roles deleted.

##### 6.9.6.1.5	Exceptions

Not Applicable

##### 6.9.6.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.6.1.7	oneM2M Resource Interworking

Not Applicable

#### 6.9.7.1	getServiceSubscription

This service capability provides the ability to retrieve the existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.7.1.1	Pre-conditions

Not Applicable

##### 6.9.7.1.2	Signature – getServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.1.1.1-1 |
| serviceSubscriptions | OUT | NO | The resulting Service Subscription entities in Table 6.9.1.1-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.7.1.2-1 Service Subscription Administration – getServiceSubscription capability

##### 6.9.7.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getServiceSubscription capability

Table 6.9.7.1.3-1 Service Subscription Administration – getServiceSubscription

##### 6.9.7.1.4	Post-Conditions

Not Applicable

##### 6.9.7.1.5	Exceptions

Not Applicable

##### 6.9.7.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.7.1.7	oneM2M Resource Interworking

This service capability retrieves the Service Subscription for the specified filter criteria and maps to the RETRIEVE procedure for the <m2mServiceSubscription> resource.

#### 6.9.8.1	addDeviceToServiceSubscription

This service capability provides the ability to add a device to the existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.8.1.1	Pre-conditions

Not Applicable

##### 6.9.8.1.2	Signature – addDeviceToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) to add the Device |
| externalIds | IN | YES | A List of URNs that represent the external identifiers associated with this device. |
| applicationIds | IN | YES | A list of Application identifiers (App-ID) associated with this device. |
| deviceId | OUT | NO | The unique device identifier in the context of the Service Subscription. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Device exists for this Service Subscription |

Table 6.9.8.1.2-1 Service Subscription Administration – addDeviceToServiceSubscription capability

##### 6.9.8.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addDeviceToServiceSubscription capability

Table 6.9.8.1.3-1 Service Subscription Administration – addDeviceToServiceSubscription

##### 6.9.8.1.4	Post-Conditions

Not Applicable

##### 6.9.8.1.5	Exceptions

Not Applicable

##### 6.9.8.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.8.1.7	oneM2M Resource Interworking

This service capability adds the device information for the Service Subscription. This capability maps to the CREATE procedure of the <nodeInfo> resource.

#### 6.9.9.1	deleteDeviceFromServiceSubscription

This service capability provides the ability to delete a device from the existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.9.1.1	Pre-conditions

Not Applicable

##### 6.9.9.1.2	Signature – deleteDeviceFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the Devices |
| deviceId | IN | NO | The unique device identifier in the context of the Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - Device does not exist for this Service Subscription |

Table 6.9.9.1.2-1 Service Subscription Administration – deleteDeviceFromServiceSubscription capability

##### 6.9.9.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteDeviceFromServiceSubscription capability

Table 6.9.9.1.3-1 Service Subscription Administration – deleteDeviceFromServiceSubscription

##### 6.9.9.1.4	Post-Conditions

Not Applicable

##### 6.9.9.1.5	Exceptions

Not Applicable

##### 6.9.9.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.9.1.7	oneM2M Resource Interworking

This service capability removes the device information for the Service Subscription. This capability maps to the DELETE procedure of the <nodeInfo> resource.

#### 6.9.10.1	getDevicesForServiceSubscription

This service capability provides the ability to retrieve the devices for an existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.10.1.1	Pre-conditions

Not Applicable

##### 6.9.10.1.2	Signature – getDevicesForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.1.2.1-1 |
| devices | OUT | NO | The resulting Device entities in Table 6.9.1.2-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.10.1.2-1 Service Subscription Administration – getDevicesForServiceSubscription capability

##### 6.9.10.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getDevicesForServiceSubscription capability

Table 6.9.10.1.3-1 Service Subscription Administration – getDevicesForServiceSubscription

##### 6.9.10.1.4	Post-Conditions

Not Applicable

##### 6.9.10.1.5	Exceptions

Not Applicable

##### 6.9.10.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.10.1.7	oneM2M Resource Interworking

This service capability retrieves the devices for a Service Subscription for the specified filter criteria and maps to the RETRIEVE procedure for the <nodeInfo> resource.

#### 6.9.11.1	addApplicationsToServiceSubscription

This service capability provides the ability to add M2M Applications (App-ID) to an existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.11.1.1	Pre-conditions

Not Applicable

##### 6.9.11.1.2	Signature – addApplicationsToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the M2M Applications. |
| applicationIds | IN | NO | A list of Application identifiers (App-ID) |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - M2M Application exists for this Service Subscription |

Table 6.9.11.1.2-1 Service Subscription Administration – addApplicationsToServiceSubscription capability

##### 6.9.11.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addApplicationsToServiceSubscription capability

Table 6.9.11.1.3-1 Service Subscription Administration – addApplicationsToServiceSubscription

##### 6.9.11.1.4	Post-Conditions

Not Applicable

##### 6.9.11.1.5	Exceptions

Not Applicable

##### 6.9.11.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.11.1.7	oneM2M Resource Interworking

This service capability adds M2M Applications to a Service Subscription. The service capability aligns with the <m2mServiceSubscription> resource. However there isn’t a procedure specified that allows an AE to update just the M2M Applications associated with the <m2mServiceSubscription> resource.

#### 6.9.12.1	deleteApplicationsFromServiceSubscription

This service capability provides the ability to delete M2M Applications (App-ID) from an existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.12.1.1	Pre-conditions

Not Applicable

##### 6.9.12.1.2	Signature – deleteApplicationsFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the Applications |
| applicationIds | IN | NO | A list of M2M Application identifiers (App-ID) |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - M2M Application exists for the Service Subscription |

Table 6.9.12.1.2-1 Service Subscription Administration – deleteApplicationsFromServiceSubscription capability

##### 6.9.12.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteApplicationsFromServiceSubscription capability

Table 6.9.12.1.3-1 Service Subscription Administration – deleteApplicationsFromServiceSubscription

##### 6.9.12.1.4	Post-Conditions

Not Applicable

##### 6.9.12.1.5	Exceptions

Not Applicable

##### 6.9.12.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.12.1.7	oneM2M Resource Interworking

This service capability delete M2M Applications from a Service Subscription. The service capability aligns with the <m2mServiceSubscription> resource. However there isn’t a procedure specified that allows an AE to update just the M2M Applications associated with the <m2mServiceSubscription> resource.

#### 6.9.13.1	getApplicationsForServiceSubscription

This service capability provides the ability to retrieve the M2M Applications for an existing Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.13.1.1	Pre-conditions

Not Applicable

##### 6.9.13.1.2	Signature – getApplicationsForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the Applications |
| applicationIds | OUT | NO | A list of Application identifiers (App-ID) |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.13.1.2-1 Service Subscription Administration – getApplicationsForServiceSubscription capability

##### 6.9.13.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getApplicationsForServiceSubscription capability

Table 6.9.13.1.3-1 Service Subscription Administration – getApplicationsForServiceSubscription

##### 6.9.13.1.4	Post-Conditions

Not Applicable

##### 6.9.13.1.5	Exceptions

Not Applicable

##### 6.9.13.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.13.1.7	oneM2M Resource Interworking

This service capability retrieves M2M Application Identifiers for a Service Subscription. The service capability aligns with the <m2mServiceSubscription> resource and maps to the RETRIEVE procedure for the Resource.

#### 6.9.14.1	updateApplicationForDevice

This service capability provides the ability to update the M2M Applications to be associated to a device within the context of a Service Subscription. This service capability shall be restricted to the Msc and Mca reference points.

##### 6.9.14.1.1	Pre-conditions

Not Applicable

##### 6.9.14.1.2	updateApplicationForDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the device |
| deviceId | IN | NO | The unique device identifier in the context of the Service Subscription. |
| applicationIds | IN | NO | A list of Application identifiers (App-ID) |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist - Device does not exist for the Service Subscription |

Table 6.9.14.1.2-1 Service Subscription Administration – updateApplicationForDevice capability

##### 6.9.14.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateApplicationForDevice capability

Table 6.9.14.1.3-1 Service Subscription Administration – updateApplicationForDevice

##### 6.9.14.1.4	Post-Conditions

The M2M Applications for the device is replaced.

##### 6.9.14.1.5	Exceptions

Not Applicable

##### 6.9.14.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.14.1.7	oneM2M Resource Interworking

This service capability updates the M2M Application list for the device information in Service Subscription. This capability maps to the UPDATE procedure of the <nodeInfo> resource.

## 6.10	Event Collection

The Event Collection service provides the capability to record events for accounting purposes.

Editors Note: Event Collection for accounting purposes is FFS.

# 7	M2M Service Components

This clause describes the M2M Service Components provided by the M2M Services Platform.

## 7.1	Introduction

An M2M Service Component defines the set of M2M Services that are related in the sense that the M2M Services that comprise the M2M Service Component are typically deployed together in order to support one or more Supporting Services.

The M2M Service Architecture augments the oneM2M Functional Architecture by specifying M2M Services provided to M2M Service Providers. These M2M Services are exposed to the AE across the Mca reference point as well as other Service Components across the Msc reference point.

## 7.1.1	Service Component Interaction Cross Reference

Table 7.1-1 Service Component Cross Reference

## 7.3	Infrastructure Component (INF)

The INF Component provides Services that are exposed to other M2M Service Components across the Msc reference point. The Services within the INF Component are common and utility type Services needed to enable other M2M Services.

## 7.3.1	INF to Service Cross Reference

Authorization: All Service Capabilities

ServiceSubscription: retrieveAuthorization

## 7.4	Service Subscription Component (SSUB)

The SSUB Component provides services that are necessary to maintain Supporting Service, the associations (e.g., Services Roles) to the Supporting Service, M2M Service Subscriptions, the associations (e.g., AE, devices) to the M2M Service Subscription and any External Identifiers associated with AE and devices. In addition the services used to manage devices and AEs are part of this component.

## 7.4.1	SSUB to Service Cross Reference

ServiceSubscription: All Service Capabilities

Supporting Service Administration: All Service Capabilities

Service Subscription Administration: All Service Capabilities

## 7.5	Transport Adapter (TRA)

The TRA component provides Services are used to integrate or adapt Data Exchange requests to the underlying Data Exchange transport (e.g., XMPP or MQTT Broker).

## 7.5.1	TRA to Service Cross Reference

Broker: All Service Capabilities

SE:Data Exchange: notify

## 7.6	Accounting (ACC)

The ACC Component provides Services that are record events necessary to charge for the Services provided to AEs.

## 7.6.1	Accounting to Service Cross Reference

Event Collection: All Service Capabilities

## 7.7	Service Exposure (SE)

The SE Component acts as the primary interface for AEs to interact with the M2M Service Layer across the Mca reference point.

## 7.7.1	Service Exposure to Service Cross Reference

SE Data Exchange: All Service Capabilities

Service Subscription: getBroker

Broker: publish, subscribe

Service Subscription Administration: getServiceSubscription, addDeviceToServiceSubscription, deleteDeviceFromServiceSubscription, getDevicesForServiceSubscription, addApplicationsToServiceSubscription, deleteApplicationsFromServiceSubscription, getApplicationsForServiceSubscription

Management Adapter: All Service Capabilities

## 7.8	Management Adapter (MA)

The Management Adapter component provides Services are used to integrate or adapt Device Management requests to the Management Servers of existing device management technologies(e.g., TR-069, OMA-DM, and LWM2M) .

## 7.8.1	Management Adapter to Service Cross Reference

Management Adapter: All Service Capabilities

SE: Device Management: reportFirmwareStatus

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

# Annex A (Informative): Common Services: Mca Received Requests

## A.1	Overview

This annex illustrates the usage of the common services described in this technical specification for implementation requests received from AEs across the Mca Reference Point.

### A.2	Supporting Services

### A.2.1	Authentication and Authorization of Requests

Upon reception of requests across the Mca reference, the request originator is authenticated and the request service capability is authorized when the:

- Request originator has a service subscription for the request service capability.

- Request originator is authorized for the request service capability.

#### A.2.1.1	receiveRequest

##### A.2.1.1.1	Pre-conditions

The external identifiers for the request Originator, subscriptionId, from Orginator and to AE-ID are assigned to the Service Subscription.

A correlation between the service capability and the M2M authorization event has been defined in order to obtain the authorize the service capability.

##### A.2.1.1.2	Common Service Capability Parameters for Request Authentication and Authorization

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | The identifier of the originator that is issuing the request. |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-SUB-ID) |
| from | IN | NO | The identifier of the Originator of the request (AE-ID) |
| to | IN | NO | The identifier of the AE that is to target of the request (AE-ID) |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca reference point. - Originator is not authenticated - Originator does not have a Service Subscription - Originator not authorized for the Service Capability |

Table A.2.1.1.2-1 Common Service Capability Parameters for Request Authentication and Authorization

##### A.2.1.1.3	Service Interactions

The interactions of service capabilities required to authenticate and authorize a request:

- Validate the Service Subscription using the Originator, to AE-ID and from AE-ID parameters received from the Originator.

- Authorized the use of the service capability within the Service Subscription

Figure A.2.1.1.3-1 receiveRequest Interaction Diagram

##### A.2.1.1.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and an optional response type is transmitted back the request Originator.

##### A.2.1.1.5	Exceptions

Not Applicable

##### A.2.1.1.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only, Robust In-Only, In-Optional-Out

Transaction Pattern: Participation allowed

#### A.2.1.2	getServiceCapability

The getServiceCapability capability inspects the request to determine the M2M Service Capability associated with the request. Each request that is received has a service capability defined in the request header.

##### A.2.1.2.1	Pre-conditions

Not Applicable

##### A.2.1.2.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operation | IN | NO | Request operation – Common to all requests |
| serviceCapId | OUT | NO | The M2M Service Capability Identifier (M2M-Srv-ID) |
| responseType | OUT | YES | Response types: - Unable to determine Service Capability from Operation |

Table A.2.1.2.2-1 getServiceCapability Signature

##### A.2.1.1.3	Post-Conditions

Success case: The M2M-Srv-ID is returned

Failure case: An error code is returned

##### A.2.1.1.4	Exceptions

Not Applicable

##### A.2.1.1.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

#### A.2.1.3	getSupportingService

The getSupportingService capability inspects the request to determine the Supporting Service for the Service Capability. This service capability is assigned to M2M Service.

##### A.2.1.3.1	Pre-conditions

A correlation between the M2M-Srv-ID that defines the request operation and the M2M-Serv-ID that defines the Supporting Service has been defined in order to obtain the serviceId.

##### A.2.1.3.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Srv-ID) |
| serviceId | OUT | NO | The M2M Service Identifier (M2M-Serv-ID) of the Supporting Service |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca reference point. - M2M Services doesn’t exist for this Service Capability |

Table A.2.1.3.2-1 getSupportingService Signature

##### A.2.1.3.3	Post-Conditions

Success case: The M2M-Serv-ID is returned

Failure case: An error code is returned

##### A.2.1.3.4	Exceptions

Not Applicable

##### A.2.1.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

# Annex B (Informative): Data Exchange Services

## B.1	Overview

This annex illustrates the usage of the services for requests to exchange data from AEs across the Mca Reference Point.

### B.2	Supporting Services

### B.2.1	Subscribe-Publish-Notify Message Exchange

This service provides the capability to exchange data using the publish and subscribe message patterns where AEs:

- Request to receive payloads from a publication resource by subscribing to the publication resource.

- Request to publish a payload to a resource.

#### B.2.1.2	Service Capabilities

##### B.2.1.2.1	subscribeRequest

This service capability provides the ability to validate a subscription request from an AE.

###### B.2.1.2.1.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Service Subscription, from AE-ID and Broker exist.

###### B.2.1.2.1.2	Signature - subscribeRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested Resource - Delivery policy not supported by the underlying Broker |

Table B.2.1.2.1.2-1 Data Exchange Service – subscribeRequest capability

###### B.2.1.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the Broker to be used for the from AE-ID and publicationResource

- Perform the Common Request Services for requests across the Mca reference point

- Validate the deliveryPolicy

Figure B.2.1.2.1.3-2 subscribeRequest and subscribeComplete Interaction Diagram

###### B.2.1.2.1.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back the from AE-ID.

###### B.2.1.2.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250ms

##### B.2.1.2.2	subscribeComplete

This service  capability provides the ability to complete the actions required once the subscription request has been completed in the Broker.

###### B.2.1.2.2.1	Pre-conditions

No unique pre-conditions for this service capability.

###### B.2.1.2.2.2	Signature - subscribeComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.1.2.2.2-1 Data Exchange Service – subscribeComplete capability

###### B.2.1.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.2.1.3-2:

- Upon a successful subscribe by the Broker, record the event for accounting purposes

###### B.2.1.2.2.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back the from AE-ID.

###### B.2.1.2.2.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

##### B.2.1.2.3	publishRequest

This service capability provides the ability to validate a publication request from an AE..

As part of the publication, the publishing AE can provide delivery and retainment policies to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

###### B.2.1.2.3.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Service Subscription, from AE-ID and Broker exist.

###### B.2.1.2.3.2	Signature - publishRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested toResource - Originator has requested an invalid deliveryPolicy - Originator has requested an invalid retainmentPolicy - Delivery policy not supported by the underlying Broker - Retainment policy not supported by the underlying Broker - Exception: Request may not have been completed |

Table B.2.1.2.3.2-1 Data Exchange Service – publishRequest capability

###### B.2.1.2.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the Broker to be used for the from AE-ID and toResource

- Perform the Common Request Services for requests across the Mca reference point

- Validate the deliveryPolicy and retainmentPolicy for the Broker

- Upon a successful publication to the Broker, record the event for accounting purposes

Figure B.2.1.2.3.3-3 publishRequest and publishComplete Interaction Diagram

###### B.2.1.2.3.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back the from AE-ID.

###### B.2.1.2.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### B.2.1.2.4	publishComplete

This service  capability provides the ability to complete the actions required once the publication request has been completed in the Broker.

###### B.2.1.2.4.1	Pre-conditions

No unique pre-conditions for this service capability.

###### B.2.1.2.4.2	Signature - publishComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.1.2.3.2-1 Data Exchange Service – pubishComplete capability

###### B.2.1.2.4.3	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.2.1.3-2:

- Upon a successful subscribe by the Broker, record the event for accounting purposes

###### B.2.1.2.4.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back the from AE-ID.

###### B.2.1.2.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

##### B.2.1.2.5	notifyRequest

This service capability provides the ability to validate a notification request to an AE.

As part of the notification, delivery policy that is requested to be used is validated and adjusted if necessary.

###### B.2.1.2.5.1	Pre-conditions

A correlation between a Service Subscription, to AE-ID and Broker exist.

The pre-conditions for MCA Received Requests are met.

###### B.2.1.2.5.2	Signature - notifyRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| to | IN | NO | See Table A.2.1.1.2-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN-OUT | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested toResource - Originator has requested an invalid deliveryPolicy - Exception: Request may not have been completed |

Table B.2.1.2.5.2-1 Data Exchange Service – notifyRequest capability

###### B.2.1.2.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Validate that deliveryPolicy requested by the Broker is allowed for the AE-ID and subscriptionId

Figure B.2.1.2.5.3-4 notifyRequest and notifyComplete Interaction Diagram

###### B.2.1.2.5.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back the from AE-ID.

###### B.2.1.2.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### B.2.1.2.6	notifyComplete

This service capability provides the ability to complete the actions required once the notification request has been completed in the Data Exchange service.

###### B.2.1.2.6.1	Pre-conditions

No unique pre-conditions for this service capability.

###### B.2.1.2.6.2	Signature - notifyComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| to | IN | NO | See Table A.2.1.1.2-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.1.2.63.2-1 Data Exchange Service – pubishComplete capability

###### B.2.1.2.6.3	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.2.5.3-4:

- Upon a successful notification by the Data Exchange, record the event for accounting purposes

###### B.2.1.2.6.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the Originator.

###### B.2.1.2.6.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

# Annex C (Informative): Service Subscription Administration Services

## C.1	Overview

This annex illustrates the usage of the services for requests to administer Service Subscriptions from AEs across the Mca Reference Point.

### C.2	Supporting Services

This service provides the capability for AEs to administer the Service Subscriptions that have been previously created by the M2M Service Provider. The following capabilities are provided.

- Retrieval of Service Subscriptions based on a set of filter criteria.

- Administration of the devices associated with a Service Subscriptions as well as the M2M Application associated with the device.

- Administration of the M2M Applications associated with a Service Subscription.

Editor’s note: Need to consider if we should define Service Capabilities to Add/Delete Devices and Applications in Bulk.

#### C.2.1	Service Capabilities

##### C.2.1.1	getServiceSubscription

This service  capability provides the ability for an AE to retrieve the Service Subscription across the Mca reference point.

###### C.2.1.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.1.2	Signature – getServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca common request attributes | - | - | See Table A.2.1.1.2-1 |
| filterCriteria | IN | NO | See Table 6.9.1.1.1-1 |
| serviceSubscriptions | OUT | NO | The resulting Service Subscription entities in Table 6.9.1.1-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table C.2.2.1.1.2-1 Service Subscription Administration – getServiceSubscription capability

###### C.2.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the Service Subscriptions for the supplied criteria

- Account for the event

Figure C.2.1.1.3-1 getServiceSubscription Interaction Diagram

###### C.2.1.1.4	Post-Conditions

Not Applicable

###### C.2.1.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.2	addDeviceToServiceSubscription

This service  capability adds a device to a Service Subscription.

###### C.2.1.2.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.2.2	Signature – addDeviceToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca common request attributes | - | - | See Table A.2.1.1.2-1 |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) to add the device |
| externalIds | IN | YES | A List of URNs that represent the external identifiers associated with this Device. |
| applicationIds | IN | YES | A list of Application identifiers (App-ID) |
| deviceId | OUT | NO | The unique device identifier in the context of the Service Subscription. |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Device exists for the Service Subscription |

Table C.2.1.2.2-1 Service Subscription Administration – addDeviceToServiceSubscription capability

###### C.2.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Add the device to the Service Subscription

- Account for the event

Figure C.2.1.2.3-1 addDeviceToServiceSubscription Interaction Diagram

###### C.2.1.2.4	Post-Conditions

The device is added to the Service Subscription returning the oneM2M System defined deviceId.

###### C.2.1.2.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.3	deleteDeviceFromServiceSubscription

This service  capability deletes a device from a Service Subscription.

###### C.2.1.3.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.3.2	Signature – deleteDeviceFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| issuer | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the Devices |
| deviceId | IN | NO | The unique device identifier in the context of the Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - Device does not exist for Service Subscription |

Table C.2.1.3.2-1 Service Subscription Administration – deleteDeviceFromServiceSubscription capability

###### C.2.1.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Delete the device from the Service Subscription

- Account for the event

Figure C.2.1.3.3-1 deleteDeviceFromServiceSubscription Interaction Diagram

###### C.2.1.3.4	Post-Conditions

The device is deleted from the Service Subscription.

###### C.2.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.4	getDevicesForServiceSubscription

This service  capability retrieves the devices for a Service Subscription.

###### C.2.1.4.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.4.2	Signature – getDevicesForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| issuer | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| filterCriteria | IN | NO | See Table 6.9.1.2.1-1 |
| devices | OUT | NO | The resulting Device entities in Table 6.9.1.2-1. |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist |

Table C.2.1.4.2-1 Service Subscription Administration – getDevicesForServiceSubscription capability

###### C.2.1.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the devices of the Service Subscription with a filter condition

- Account for the event

Figure C.2.1.4.3-1 getDevicesForServiceSubscription Interaction Diagram

###### C.2.1.4.4	Post-Conditions

Not Applicable

###### C.2.1.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.5	addApplicationsToServiceSubscription

This service  capability adds M2M Applications to a Service Subscription.

###### C.2.1.5.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.5.2	Signature – addApplicationsToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| issuer | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the M2M Applications |
| applicationIds | IN | NO | List of M2M Application identifiers (App-ID) |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - M2M Application exists for the Service Subscription |

Table C.2.1.5.2-1 Service Subscription Administration – addApplicationsToServiceSubscription capability

###### C.2.1.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Add the M2M Applications to the Service Subscription

- Account for the event

Figure C.2.1.5.3-5 addApplicationsToServiceSubscription Interaction Diagram

###### C.2.1.5.4	Post-Conditions

The M2M Applications are added to the Service Subscription.

###### C.2.1.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.6	deleteApplicationsFromServiceSubscription

This service  capability deletes the M2M Applications from a Service Subscription.

###### C.2.1.6.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.6.2	Signature – deleteApplicationsFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| issuer | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the Applications |
| applicationIds | IN | NO | A list of Application identifiers (App-ID). |
| responseType | OUT | YES | Unique response types for this service. - M2M Application does not exist for the Service Subscription |

Table C.2.1.6.2-1 Service Subscription Administration – deleteApplicationsFromServiceSubscription capability

###### C.2.1.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Delete the M2M Application from the Service Subscription

- Account for the event

Figure C.2.1.6.3-1 deleteApplicationFromServiceSubscription Interaction Diagram

###### C.2.1.6.4	Post-Conditions

M2M Applications are deleted from the Service Subscription.

###### C.2.1.6.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.7	getApplicationsForServiceSubscription

This service  capability retrieves the M2M Applications for a Service Subscription.

###### C.2.1.7.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.7.2	Signature – getApplicationsForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| issuer | IN | NO | See Table A.2.1.1.2-1 |
| subscriptionId | IN | NO | See Table A.2.1.1.2-1 |
| from | IN | NO | See Table A.2.1.1.2-1 |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the M2M Applications |
| applicationsIds | OUT | NO | The resulting M2M Application Ids (App-ID) |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist. |

Table C.2.1.7.2-1 Service Subscription Administration – getApplicationsForServiceSubscription capability

###### C.2.1.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Applications of the Service Subscription

- Account for the event

Figure C.2.1.7.3-1 getApplicationsForServiceSubscription Interaction Diagram

###### C.2.1.7.4	Post-Conditions

Not Applicable

###### C.2.1.7.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.8	updateApplicationForDevice

This service  capability to update the M2M Applications for a device associated with a Service Subscription.

###### C.2.1.8.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.8.2	Signature – addDeviceToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca common request attributes | - | - | See Table A.2.1.1.2-1 |
| serviceSubscriptionId | IN | NO | The Service Subscription (M2M-Sub-ID) for the device |
| deviceId | IN | NO | The unique device identifier in the context of the Service Subscription. |
| applicationIds | IN | NO | A list of Application identifiers (App-ID) |
| lastModifiedTime | OUT | NO | See Table A.2.1.1.2-1 |
| responseType | OUT | YES | Unique response types for this service. - Service Subscription does not exist - Device does not exist for the Service Subscription |

Table C.2.1.8.2-1 Service Subscription Administration – updateApplicationForDevice capability

###### C.2.1.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Update the M2M Applications for the device by replacing the existing M2M Application list.

- Account for the event

Figure C.2.1.8.3-1 updateApplicationForDevice Interaction Diagram

###### C.2.1.8.4	Post-Conditions

The M2M Applications for the device are replaced with the M2M Applications in the request.

###### C.2.1.8.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

# Annex D (Informative): Device Management Services

## D.1	Overview

This annex illustrates the usage of the services for requests to manage device from AEs across the Mca Reference Point.

### D.2	Supporting Services

### D.2.1	Message Exchange

This service provides the capability to manage device using the request-response or request-asynchronous notification message patterns where AEs:

- Request to manage device using device specific service capabilities or more complex service capabilities that the Support Service will orchestrate into the individual service capabilities according to their business process.

- Request to obtain management operation execution results or status using the reporting capability. Reports can be generated immediately or periodically based on the report policy. Reports can also be aggregated prior to reporting.

#### D.2.1.1	Service Capabilities

##### D.2.2.1.1	downloadFirmware

This service capability provides the ability to execute a downloadFirmware request from an AE.

###### D.2.2.1.1.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.2.1.1.2	Signature - downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table D.2.2.1.1.2-1 Device Management Service –downloadFirmware capability

###### D.2.2.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to download the firmware

- Records the event

Figure D.2.2.1.1.3-1 downloadFirmware Interaction Diagram

###### D.2.2.1.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

- The event has been recorded.

###### D.2.2.2.1.5	Exceptions

- Not Applicable

###### D.2.2.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.2.1.2	installFirmware

This service capability provides the ability to execute a installFirmware request from an AE.

###### D.2.2.1.2.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.2.1.2.2	Signature - installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table D.2.2.1.2.2-1 Device Management Service –installFirmware capability

###### D.2.2.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to install the firmware

- Records the event

Figure D.2.2.1.2.3-1 installFirmware Interaction Diagram

###### D.2.2.1.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

- The event has been recorded.

###### D.2.2.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.2.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.2.1.3	getFirmwareInformation

This service capability provides the ability to execute a getFirmwareInformation request from an AE.

###### D.2.2.1.3.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.2.1.3.2	Signature - getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |

Table D.2.2.1.3.2-1 Device Management Service –getFirmwareInformation capability

###### D.2.2.1.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to get the firmware information

- Records the event

- Figure D.2.2.1.3.3-1 getFirmwareInformation Interaction Diagram

###### D.2.2.1.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

- The event is recorded

###### D.2.2.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.2.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.2.1.4	getFirmwareExecStatus

This service capability provides the ability to retrieve the execution status or result of a requested firmware operation.

###### D.2.2.1.4.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and previously submitted firmware request exist.

###### D.2.2.1.4.2	Signature - getFirmwareExecStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The previously submitted firmware request does not exist |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table D.2.2.1.4.2-1 Device Management Service –getFirmwareExecStatus capability

###### D.2.2.1.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to retrieve the firmware operations execution status or result

- Records the event

Figure D.2.2.1.4.3-1 getFirmwareExecStatus Interaction Diagram

###### D.2.2.1.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware operation execution status or result.

- Record the event.

###### D.2.2.1.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.2.1.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.2.1.5	reportFirmwareStatus

This service capability provides the ability to notify an AE about the status or result of a firmware operation for a previously submitted operation request.

###### D.2.2.1.5.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and previously submitted firmware requests exist.

###### D.2.2.1.5.2	Signature - reportFirmwareStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.2.1.5.2-1 Device Management Service –reportFirmwareStatus capability

###### D.2.2.1.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Record the event.

Figure D.2.2.1.5.3-6 reportFirmwareStatus Interaction Diagram

###### D.2.2.1.5.4	Post-Conditions

The AE has received a report of firmware operation execution status or result.

- The event is recorded.

###### D.2.2.1.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.2.1.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.2.1.6	upgradeFirmware

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations

###### D.2.2.1.6.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between Management Adapter, the service capability and devices or device group exist.

###### D.2.2.1.6.2	Signature - upgradeFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.1.1.1.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareReportList | OUT | YES | List of firmware reports. Type FirmwareReport, see 6.6.1.1.1.3. |

Table D.2.2.1.6.2-1 Device Management Service –upgradeFirmware capability

###### D.2.2.1.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues a series of operations to the Management Adapter to upgrade the firmware on a the devices in the orchestration rule list. Each entry in the orchestration rule list contains the operation, the devices and schedule for the operation.

- Records the event

- Aggregate the report status or results

- Figure D.2.2.1.6.3-7 upgradeFirmware Interaction Diagram

Editors Note: We need a common service capability for Aggregate reports. Flow needs to account for the Report Policy.Aggregate reports need further study.

###### D.2.2.1.6.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to upgrade the firmware.

- The event is recorded.

###### D.2.2.2.6.5	Exceptions

The reachability schedule of the device doesn’t not match the schedule of the orchestration rule.

###### D.2.2.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.7	getDeviceInformation

This service capability provides the ability for an AE to retrieve the information about a device across the Mca reference point.

###### D.2.1.1.7.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.1.1.7.2	Signature – getDeviceInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| deviceInfo | OUT | NO | The device information. Type DeviceInfo, see 6.6.1.1.1.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.1.1.1.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.1.1.1.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.1.1.1.13. |

Table D.2.1.1.7.2-1 Device Management Service –getDeviceInformation capability

###### D.2.1.1.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to get device information

- Records the event

Figure D.2.1.1.7.3-1 getDeviceInformation Interaction Diagram

###### D.2.1.1.7.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device information.

Based on the capabilities supported by the device, the memory, battery and lock status information may or may not be returned.

The event has been recorded.

###### D.2.1.1.7.5	Exceptions

Not Applicable

###### D.2.1.1.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.8	getDeviceCapabilities

This service capability provides the ability to execute a getDeviceCapabilities request from an AE.

###### D.2.1.1.8.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.1.1.8.2	Signature – getDeviceCapabilities

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| deviceCapabilities | OUT | YES | Array of deviceCapability. Type DeviceCapability, see 6.6.1.1.1.10. |

Table D.2.1.1.8.2-1 Device Management Service –getDeviceCapabilities capability

###### D.2.1.1.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to get device capabilities

- Records the event

Figure D.2.1.1.8.3-1 getDeviceCapabilities Interaction Diagram

###### D.2.1.1.8.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

The event has been recorded.

###### D.2.1.1.8.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.9	enableDeviceCapability

This service capability provides the ability to execute a enableDeviceCapability request from an AE.

###### D.2.1.1.9.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.1.1.9.2	Signature – enableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.9.2-1 Device Management Service –enableDeviceCapability capability

###### D.2.1.1.9.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to enable device capability

- Records the event

Figure D.2.1.1.9.3-1 enableDeviceCapability Interaction Diagram

###### D.2.1.1.9.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

The event is recorded

###### D.2.1.1.9.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.10	disableDeviceCapability

This service capability provides the ability to execute a disableDeviceCapability request from an AE.

###### D.2.1.1.10.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

###### D.2.1.1.10.2	Signature – disableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.10.2-1 Device Management Service –disableDeviceCapability capability

###### D.2.1.1.10.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to disable device capability

- Records the event

Figure D.2.1.1.10.3-1 disableDeviceCapability Interaction Diagram

###### D.2.1.1.10.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

Record the event.

###### D.2.1.1.10.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.10.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.11	getAreaNetworks

This service capability provides the ability to execute a getAreaNetworks request from an AE.

###### D.2.1.1.11.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability and area network exist.

###### D.2.1.1.11.2	Signature –getAreaNetworks

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| AreaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.1.1.1.13. |

Table D.2.1.1.11.2-1 Device Management Service –getAreaNetworks capability

###### D.2.1.1.11.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to get area networks information

- Records the event

Figure D.2.1.1.11.3-1 getAreaNetworks Interaction Diagram

###### D.2.1.1.11.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

The event has been recorded.

###### D.2.1.1.11.5	Exceptions

Not Applicable

###### D.2.1.1.11.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.12	updateDeviceForAreaNetwork

This service capability provides the ability to execute a updateDeviceForAreaNetwork request from an AE.

###### D.2.1.1.12.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the service capability, the device and area network exist.

###### D.2.1.1.12.2	Signature –updateDeviceForAreaNetwork

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.1.1.1.15. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| lastModifiedTime | OUT | NO | The modified time. |

Table D.2.1.1.12.2-1 Device Management Service –updateDeviceForAreaNetwork capability

###### D.2.1.1.12.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to update device information for area network

- Records the event

Figure D.2.1.1.12.3-8 updateDeviceForAreaNetwork Interaction Diagram

###### D.2.1.1.12.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

The event is recorded.

###### D.2.1.1.12.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.12.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

# Annex X(Informative): oneM2M Service Capability Template

The Service Capability template is used as a specification that defines the contract between the consumer of the service capability and the producer of the service capability. The specification contract includes documenting the provided Signature, constraints (pre-conditions, post-conditions, exceptions), policies and runtime expectations.

### X.1	<serviceCapabilityName>

<Document introductory description of the service capability>

The <serviceCapabilityName> reflects the operational functionality provided by the Service Capability. oneM2M Service Capabilities are named using a construct [verb][noun][context] where the:

verb: Used to indicate what will happen to the noun

noun: Is the target of the noun

context: Is optional and further defines the service capability

### X.1.1	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Name of the parameter. | IN, OUT, IN-OUT | YES, NO | Description of the parameter in the context of the service capability. |

Table X.1.1-1 Service Capability Signature

Direction: The direction is relative to the entity that provides (implements) the Service Capability. The value “IN” means that the entity expects to receive a value for the parameter from the consumer (sender) of the Service Capability request. The value “Out” means that the entity will send a value for the parameter to the consumer (sender) of the Service Capability request. A value of “IN-OUT” means that the entity will receive the value for the parameter from the consumer and then send a value (not necessarily the same value) for the parameter back to the consumer.

Optional: Indicates if the parameter is required in requests (Direction: IN, IN-OUT) or responses (Direction: IN-OUT, OUT). A value of YES means that the parameter is optional. A value of NO means that the parameter is mandatory.

### X.1.2	Pre-conditions

<Document the pre-conditions that may only be valid to run provided some testable prior business rule check or other value to be changed is actually present and available for the service capability to act upon. Pre-conditions are documented that are in addition to that provided by the internal resource operations and procedures>

### X.1.3	Service Interactions

<Document the detail procedure for <serviceCapabilityName>. The detail procedure for the service capability is written as an interaction diagram in a time order and step-by-step. The originator and what the originator carries out (a task, function, accessing to resources or other services) are defined in each step.

### X.1.4	Post-Conditions

<Document the post-conditions of the <serviceCapabilityName>. If the service capability has no defined output that can be validated except a notification of success (or failure) to set a value in the target of the service capability, a testable post condition of the service capability is documented. Post-conditions are documented that are in addition to that provided by the internal resource operations and procedures.>

### X.1.5	Exceptions

< Document the exception or error conditions that may be returned by the <serviceCapabilityName>. Exceptions are documented that are in addition to that provided by the internal resource operations and procedures.>

### X.1.6	Policies for Use

Message Exchange Patterns : See section 5.2.2.1Message Exchange Patterns for allowable patterns

Transaction Pattern:

- Creates Transaction: This service capability starts and completes a transaction

- Participation allowed: This service capability is able to participate in a transaction without violating the ongoing transaction’s properties (i.e. Atomicity, Consistency, Isolation, Durability)

- Participation not allowed: This service capability is unable to participate in a transaction

Maximum Response: The maximum time in milliseconds permitted to complete this service capability.

### X.1.7	oneM2M Resource Interworking

<Document the related oneM2M Resources operations to <serviceCapabilityName> here. Interworking rules are defined for each Resource operation. If there is no related internal resource, this resource list can be empty (e.g., the service capability is ‘sendTrigger’).>.

# Annex Y(Informative): oneM2M Service Requirements

The primary motivation of this technical specification is to integrate oneM2M Service capabilities with existing M2M deployments without affecting that existing M2M solution’s underlying deployment framework and protocols. For example if an existing M2M solution uses a XMPP based framework to communicate between applications in the network and devices, then M2M Service capabilities are efficiently exposed to the XMPP based framework without requiring modifications to the existing M2M solution’s deployment framework or protocol. A secondary motivation for this technical specification is to offer M2M Service capabilities to M2M service provider’s who utilize service oriented architectural frameworks.

This annex defines requirements that have not been captured in the oneM2M Requirements [1].

| Requirement ID | Description |
| --- | --- |
| SCA-001 | The M2M System shall be capable of integrating oneM2M service capabilities with existing M2M deployments without affecting the existing M2M solution’s underlying deployment framework and protocols. |
| SCA-002 | The M2M System shall be capable of offering oneM2M service capabilities to SOA architectural frameworks. |

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 19-Feb-2014 | Rapporteur Input - Draft Skeleton |
| V.0.1.0 | 22-Feb-2014 | Added agreed contributions from TP9 ARC-2014-0081R02 |
| V.0.2.0 | 15-May-2014 | Added agreed contributions from TP10 and interim conference calls ARC-2014-1259-TS-0007-Reference_Update ARC-2014-1260R02-TS-0007-Mca-Request_Services ARC-2014-1324-TS-0007_Attribute_Alignment ARC-2014-1261R02-TS-0007-Mca-Sub-Pub-Notify |
| V.0.3.0 | 17-June-2014 | Added agreed contributions from TP11 ARC-2014-1390-TS-0007_IN-OUT_DeSignation ARC-2014-1391-TS-0007_Alignment_with_TS-0001_terms ARC-2014-1398-TS-0007_Clause-Role-Interworking-Removal ARC-2014-1401R02-TS-0007_Response_Code_Cleanup ARC-2014-1404-TS-0007-Service_Adminstration ARC-2014-1405-TS-0007-Service_Subscription_Adminstration |
| V.0.4.0 | 04-August-2014 | Added agreed contributions from TP12 ARC-2014-1411R02-TS-0007-Mca_Device_Firmware_management ARC-2014-1473R01-TS-0007-Mca_Device_Configuration_service ARC-2014-1474R01-TS-0007-Mca_Device_Topology_Management_service |