| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0008-CoAP Protocol Binding-V-0.4.0 |
| Document Name: | CoAP Protocol Binding Technical Specification |
| Date: | 2014-Jul-27 |
| Abstract: | The specification will cover the protocol specific part of communication protocol used by oneM2M compliant systems as ‘CoAP binding’ |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols, abbreviations and acronyms	4

3.1	Definitions	4

3.2	Symbols	5

3.3	Abbreviations	5

3.4	Acronyms	5

4	Conventions	5

5	Overview	6

5.1	Required Features	6

5.2	Message Format	6

5.3	Caching	6

5.3.1	Freshness	6

5.3.2	Validity	7

5.4	Blockwise Transfers	7

6	oneM2M Protocol Mapping	7

6.1	Primitive Types Mapping	7

6.1.1	Request primitive to CoAP Request	7

6.1.2	CoAP Request to Request Primitive	8

6.2	Meta Information Mapping	8

6.2.1	Common Meta Information Mapping	8

6.2.2	Specific Meta Information Mapping	8

6.3	Response Codes Mapping	8

6.3.1	Response Primitive to CoAP Response	8

6.3.2	CoAP Response Code to ResponseConfirm Primitive	9

6.3.3	Additional Information	9

7	Security Consideration	10

History	10

# 1	Scope

The present document …

EXAMPLE:	The present document provides the necessary adaptions to the endorsed document.

The Scope shall not contain requirements.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)"

[2]	oneM2M TS-0004: Protocol TS

[3]	IETF draft: “Blockwise transfers in CoAP”, draft-ietf-core-block-15

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definitions, symbols, abbreviations and acronyms

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

## 3.4	Acronyms

Acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Acronym format

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Overview

This specification defines how to map oneM2M APIs into CoAP messages and vice versa.

## 5.1	Required Features

This section specifies required features from CoAP[1] to be properly mapped into oneM2M APIs:

- CoAP message types for message correlation, including CON, NON, ACK, and RST, shall be supported.

- GET, PUT, POST and DELETE methods shall be supported.

- Related response code shall be supported.

- CoAP defines a single set of options that are used in both requests and responses.  Related options shall be supported.

- Other features of CoAP shall be supported, such as blockwise transfers.(TBD)

Editor’s note: which other features are needed is FFS. Identifying related response codes/options is FFS.

## 5.2	Message Format

This section specifies details about the CoAP [1] message format:

- CoAP message occupies the data section of one UDP datagram.

- CoAP message format supports a 4-byte fixed-size header.

- Fixed-size header is followed by a Token value of length 0 or 8 bytes.

- The Token value is followed by a sequence of zero or more CoAP Options in TLV format.

- CoAP Options are followed by the payload part.

For more details on the CoAP Message Format and the supported Header Fields, refer [1].

## 5.3	Caching

CoAP [1] supports caching of responses to fulfill future equivalent requests to the same resource. Caching is supported using freshness and validity information carried with CoAP [1] responses.

## 5.3.1	Freshness

- CoAP server shall use Max-Age CoAP Option to specify the explicit expiration time for the CoAP Response’s resource representation. This indicates that the response is not fresh after its age is greater than the specified number of seconds.

- Max-Age Option defaults to a value of 60 (seconds). In case, Max-Age Option is not present in the cacheable response, the response shall not be considered fresh after its age is greater than 60 seconds.

- The CoAP server shall set the Max-Age Option value to 0 (zero) to prevent or disable caching.

- The CoAP client, having a fresh stored response, can make new request matching the request for that stored response. In this case, the new response shall invalidate the old response.

## 5.3.2	Validity

- A CoAP endpoint with stored responses but not able to satisfy subsequent requests (for example, the response is not fresh), shall use the ETag Option to perform a conditional request to the CoAP server where the resource is hosted.

- If the cached response with the CoAP client is still valid, the server shall include the Max-Age Option in the response along with a code of 2.03 - Valid. This shall update the freshness of the cached response at the CoAP client.

- If the cached response with the CoAP client is not valid, the server shall respond with an updated representation of the resource with response code 2.05 – Content. The CoAP client shall use the updated response to satisfy request and may also replace/update the stored or cached response.

## 5.4	Blockwise Transfers

CoAP Block [3] Option shall be used for handling cases where oneM2M resource representations will need to transfer large payloads e.g. firmware, software updates. Instead of relying on IP fragmentation, CoAP Block Option shall be used for transferring multiple blocks of information in multiple request-response pairs.

Using Block Options, larger resource representations can be fragmented and reassembled by CoAP independently of the lower layers as well as the above application. The CoAP Block1 Option shall be used to define the size of the blocks used for oneM2M requests and the CoAP Block2 Option shall be used to define the size of the blocks used for oneM2M responses. Refer [3] for further details.

# 6	oneM2M Protocol Mapping

## 6.1	Primitive Types Mapping

The CoAP request method shall be mapped into oneM2M primitive types as following:

Editor’s Note: in this context, primitive types depend on operation.

Candidates of primitive types are <resource type name>CreateRequest/Response, <resource type name>RetrieveRequest/Response, <resource type name>UpdateRequest/Response, and <resource type name>DeleteRequest/Response. This shall be updated based on Protocol Core TS.

| CoAP Methods | oneM2M Primitive Types |
| --- | --- |
| POST |  |
| GET |  |
| PUT |  |
| DELETE |  |

Editor’s note: primitive types are for FFS, and detailed parts will be added when related TS finished.

## 6.1.1	Request primitive to CoAP Request

The oneM2M request primitive type shall be mapped to a CoAP Method according to the table x.x.

The CoAP request shall be constructed using the selected CoAP method, selected options as described in options.

## 6.1.2	CoAP Request to Request Primitive

The CoAP request shall be mapped to a oneM2M request primitive according to the table x.x..

## 6.2	Meta Information Mapping

Some meta information may map to CoAP options. This section describes which meta information is mapped to which CoAP options.

## 6.2.1	Common Meta Information Mapping

This section specifies a binding from meta information which is commonly used in all the APIs to CoAP options.

Editor’s note: which is common meta information will be discovered at Protocol TS.

## 6.2.2	Specific Meta Information Mapping

This section specifies a binding from meta information which is used in specific APIs to CoAP options.

## 6.3	Response Codes Mapping

## 6.3.1	Response Primitive to CoAP Response

The response primitive shall be correlated to the corresponding request primitive and the following rules shall be followed:

- The CoAP response shall be correlated to the CoAP request corresponding to the request primitive.

- If the response primitive has any resource representation, this shall be transported in the payload of the CoAP response.

- The status code of the response for successful and unsuccessful response shall be set according to the table below.

- Table 6.3.2-1 Successful Cases

| Status Code | Status Code of CoAP |
| --- | --- |
| STATUS_CREATED | 2.01 Created |

- Table 6.3.2-2 Unsuccessful Cases

| Status Code | Status Code of CoAP |
| --- | --- |
| STATUS_BAD_REQUEST | 4.00 Bad Request |
| "STATUS_NOT_FOUND" | 4.04 Not Found |
| STATUS_METHOD_NOT_ALLOWED | 4.05 Method Not Allowed |
| STATUS_INTERNAL_SERVER_ERROR | 5.00 Internal Server Error |
| STATUS_NOT_IMPLEMENTED | 5.01 Not Implemented |
| STATUS_REQUEST_TIMEOUT | 4.00 Bad Request |
| STATUS_CONFLICT | 4.00 Bad Request |

Editor’s note:  more status code will be added when oneM2M statue code defined.

## 6.3.2	CoAP Response Code to ResponseConfirm Primitive

If the CoAP response code is in the range 2.01 to 2.05, then the response shall be considered as a successful case.

| CoAP Response Code |  | Description | Note |
| --- | --- | --- | --- |
| Success 2.xx | 2.01 Created |  |  |
|  | 2.02 Deleted |  |  |
|  | 2.03 Valid |  |  |
|  | - Changed |  |  |
|  | 2.05 Content |  |  |

If the CoAP response code is in the range of 4.00 to 4.15 or 5.00 to 5.05, then the response shall be considered as unsuccessful. Additional information about the error should be included.

| CoAP Response Code | Description | Note |
| --- | --- | --- |
| Client Error 4.xx |  | Additional information about the error |
| Server Error 5.xx |  | Additional information about the error |

## 6.3.3	Additional Information

CoAP message includes the 8-bit Code.  In case of a request, the Code field shall indicate the Request Method; in case of a response, the Code field shall indicate a Response Code.

The Code field is limited to indicate all oneM2M response code, so additional information shall be carried via the CoAP payload field.

# 7	Security Consideration

This section specifies how to use DTLS and which cipher suites shall be supported.

# History

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.1.0 | 2014-02-05 | Initial Draft Version |
| V.0.2.0 | 2014-04-10 | Incorporates: PRO-2014-0023R04 |
| V.0.3.0 | 2014-06-18 | Incorporates: PRO-2014-0255R01, PRO-2014-0254 |
| V.0.4.0 | 2014-07-27 | Incorporates: PRO-2014-0319, PRO-2014-0318, PRO-2014-0317, 2014-0297R01, 2014-0264R01 |