| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0009-V0.5.0 |
| Document Name: | HTTP Protocol Binding |
| Date: | 2014-September-26 |
| Abstract: | HTTP Protocol binding TS |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols, abbreviations and acronyms	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations & Acronyms	5

4	Conventions	5

5	Overview of HTTP Binding	5

5.1	Introduction	5

5.2	Request-Line	5

5.3	Status-Line	5

6	HTTP Message Mapping	6

6.1	Introduction	6

6.2	Parameter Mappings on Request-Line	6

6.3	Status-Line	7

6.4	Header Fields	9

6.5	Message-body	10

7	Security Consideration	10

Proforma copyright release text block	13

Annex <y>: Bibliography	13

History	13

# 1	Scope

The specification will cover the protocol specific part of communication protocol used by oneM2M compliant systems as RESTful HTTP binding.

The scope of this specification is (not limited to as shown below):

-	Binding oneM2M Protocol primitive types to HTTP method

-	Binding oneM2M response status codes (successful/unsuccessful) to HTTP response codes

-	Binding oneM2M RESTful resources to HTTP resources

This specification is depending on Core Protocol specification (TS-0004) for data types.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

[1]	RFC7230: “Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing”, IETF, June 2014.

[2]	oneM2M TS-0001: Architecture TS

[3]	oneM2M TS-0004: Core Protocol TS

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	RFC2617: “HTTP Authentication: Basic and Digest Access Authentication”, IETF, June 1999

[i.3]	RFC6750: “The OAuth 2.0 Authorization Framework: Bearer Token Usage”, IETF, October 2012.

[i.4]	RFC6455:“The WebSocket Protocol”, IETF, December 2011.

# 3	Definitions, symbols, abbreviations and acronyms

## 3.1	Definitions

## 3.2	Symbols

## 3.3	Abbreviations & Acronyms

For the purposes of the present document, the following abbreviations and acronyms apply:

HTTP		Hyper Text Transfer Protocol

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Overview of HTTP Binding

This clause describes what oneM2M primitive parameters can be mapped to HTTP request/response messages.

## 5.1		Introduction

The present specification assumes AE has the capability of HTTP Client, and CSE has the capability of both HTTP Client and Server.

Single request primitive will be mapped to single HTTP request message, and single response primitive will be mapped to single HTTP response message.

An HTTP request message consists of Request-Line, headers and message-body. An HTTP response message consists of Status-Line, headers and message-body [4]. This clause describes how oneM2M request/response primitives are mapped to HTTP messages at a high level. Corresponding details of each sub-clause are specified in clause 6.

The Registrar CSE shall behave as ‘proxy server’(see[1]).

## 5.2	Request-Line

Method is mapped to the oneM2M operation parameter.

Request-URI is derived from the oneM2M to parameter, including a query string which carries specific primitive parameters.

HTTP-Version is specified in clause 6.

## 5.3	Status-Line

HTTP Version is specified in clause 6.

Status-Code and Reason-Phrase are derived from the oneM2M responseStatusCode parameter of the response primitive.

# 6	HTTP Message Mapping

## 6.1	Introduction

Mapping between HTTP message and oneM2M primitive shall be applied in the following cases:

- when the Originator sends a request primitive,

- when the Receiver receives a request primitive,

- when the Receiver sends a response primitive,

- when the Originator receives a response primitive.

The following sub-clauses specify how to map each oneM2M primitive parameter to a corresponding HTTP message field to compose a HTTP request/response message..

## 6.2	Parameter Mappings on Request-Line

- 6.2.1	Method

The HTTP ‘Method’ shall be specified according to the oneM2M operation parameter of the request primitive.

Table 6.2.1-1: HTTP Method Mapping

| oneM2M Operation | HTTP Method |
| --- | --- |
| Create | POST |
| Retrieve | GET |
| Update | PUT or POST (for partial update) |
| Delete | DELETE |
| Notify | POST |

Editor’s Note: Update operation mapping is TBD until the attribute level manipulation scheme is resolved in TS-0001 and TS-0004.

At the Receiver, an HTTP request message with POST method shall be mapped to a oneM2M Create or Notify request primitive in accordance with the value of the operation parameter.

- 6.2.2	Request-Target

The HTTP Request-Target shall be prepared as absolute-form (see [1]).

The to parameter shall be ampped to ‘absolute-path’ in Request-Target.

Other request parameter shall be specified as pair of field-name and value in query-string as shown in Table 6.2.2-1.

Table 6.2.2-1: Request parameters mapped as query-string field

| Request Parameter | Field Name | Note |
| --- | --- | --- |
| Original Timestamp | ot filed |  |
| Request Expiration Timestamp | ret field |  |
| Result Expiration Timestamp | rst field |  |
| Operation Execution Time | oet field |  |
| Response Type | rt field |  |
| Result Persistence | rp field |  |
| Result Content | rc field |  |
| Event Category | ec field |  |
| Delivery Aggregation | da field |  |
| Group Request Identifier | gid field |  |
| Filter Criteria | fc field |  |
| Discovery Result Type | drt field |  |

- Editor’s Note: encoding structured information such as ec and fc is FFS.

- 6.2.3	HTTP-Version

This specification supports binding to HTTP 1.1[1], so the version field shall be set to “HTTP/1.1”.

- 6.2.4	Mapping on Request Headers

Following request parameters are mapped to HTTP Headers in present specification.

| Request parameter | Header name | Note |
| --- | --- | --- |
| From | From |  |
| Request Identifier | X-M2M-RI |  |
| Resource Type | Content-Type | ty parameter of application/vnd.onem2m-resource-data+xml |

## 6.3	Status-Line

- 6.3.1	HTTP-Version

This specification supports binding to HTTP 1.1[1], so the version field shall be set to “HTTP/1.1”.

- 6.3.2	Status-Code

The oneM2M responseStatusCode shall be mapped to HTTP Status-Code. Since the responseStatusCode have been defined more specifically, one or more responseStatusCodes may be mapped to Status-Code. The original responseStatusCode parameter shall be carried in message-body (see clause 6.5).

N:1 status code mapping from the oneM2M request primitive to HTTP request message shall be:

Table 6.3.2-1: Status Code Mapping

| oneM2M Response Status Codes | HTTP Status Codes |
| --- | --- |
| Success | 200 (OK) |
| Accepted | 202 (Accepted) |
| Location info not authorized | 403 (Forbidden) |
| Unsupported resource | 404 (Not Found) |
| Unsupported attribute | 404 (Not Found) |
| Cannot forward, target not reachable | 404 (Not Found) |
| Cannot forward, other reason TBD | 404 (Not Found) |
| Create error - no privilege | 403 (Forbidden) |
| Create error - already exists | 403 (Forbidden) |
| Create error - missing mandatory parameter | 400 (Bad Request) |
| Retrieve error - no privilege | 403 (Forbidden) |
| Retrieve error - does not exist | 404 (Not Found) |
| Update error - no privilege | 403 (Forbidden) |
| Update error - does not exist | 404 (Not Found) |
| Update error - unacceptable contents | 415 (Unsupported Media Type) |
| Delete error - does not exist | 404 (Not Found) |
| Delete error - no privilege | 403 (Forbidden) |
| Create delivery - not able to take on responsibility | 403 (Forbidden) |
| Create fanoutpoint - group request identifier exists | 409 (Conflicts) |
| Retrieve fanoutpoint - group request identifier exists | 409 (Conflicts) |
| Update fanoutpoint - group request identifier exists | 409 (Conflicts) |
| Delete fanoutpoint - group request identifier exists | 409 (Conflicts) |
| Create mgmtObj - memory shortage | 403 (Forbidden) |
| External object not found | 404 (Not Found) |
| Cancel execInstance - not cancellable | 403 (Forbidden) |
| Cancel execInstance - already complete | 403 (Forbidden) |
| Delete execInstance - not cancellable | 403 (Forbidden) |
| Delete execInstance - already complete | 403 (Forbidden) |
| Retrieve CSEBase - format error | 400 (Bad Request) |
| CMDH rules -non compliant | 403 (Forbidden) |
| Target is not subscribable | 403 (Forbidden) |
| Cannot initiate subscription verification | 403 (Forbidden) |
| Subscription verification failed | 403 (Forbidden) |

Editor’s Note: This table needs to be updated aligning with TS-0004.

- 6.3.3	Reason-Phrase

Reason-Phrase shall be mapped to the description of the corresponding responseStatusCode of the response primitive (see clause 6.5.4 [3]).

## 6.4	Header Fields

- 6.4.1	Host

A Host header shall be included in an HTTP request message. Other headers may be included in an HTTP request/response message.

Host shall be derived from the to parameter of the request primitive. This shall consist of the host and optionally port number.

- 6.4.2	Accept

An Originator may use the Accept header to indicate which content-type is supported by the Originator.  The Accept header shall be set to “application/onem2m-resource+xml” or “application/onem2m-resource+json”.

Editor’s Note: Supported types are TBD.

- 6.4.3	Content-type

Any request or response containing message-body shall include the Content-type header set to “application/onem2m-resource+xml” or “application/onem2m-resource+json”.

Editor’s Note: Supported types are TBD.

- 6.4.4	Content-Location

The Content-Location header shall be set to the URI of:

- the created resource, when responding to a Create request primitive;

- the retrieved resource, when responding to a Retrieve request primitive if the retrieved resource location is different from the requested resource location;

- the updated resource, when responding to a Update request primitive

Editor’s Note: Usage of this header needs more investigation.

Editor’s Note: How to indicate the oneM2M URI scheme is TBD

- 6.4.5	Content-Length

If message-body is included, the Content-Length header shall be included indicating the length of the message-body in octets (8-bit bytes).

- 6.4.6	ETag

A retrieve response primitive corresponding to a resource retrieval request primitive should include an ETag header together with the resource representation [1].

ETag facilitates the use of conditional requests (i.e. using the if-match and if-none-match HTTP headers).

If a CSE supports the ETag header, then the CSE shall support conditional requests.

- 6.4.7	From

The From header shall be mapped to the from parameter of the request/response primitive, and shall contain the oneM2M specified ID of the Originator (e.g. CSE-ID or AE-ID).

- 6.4.8	Location

The Location header shall be mapped to the URI of the created resource. This header shall be present in the response primitive corresponding to a create request and shall not be present in any other request or response.

Editor’s Note: if Originator’s nm parameter can be modified and accepted by the Hosting CSE is TBD.

- 6.4.9	X-M2M-RI

All requests and responses shall include an oneM2M defined header field called X-M2M-RI that contains the requestID.

Editor’s Note: mapping request ID can be query or this extension header, this is TBD.

- 6.4.10	Other Header Fields

Other HTTP header fields shall be mapped with oneM2M primitive parameters.

Editor’s Note: The list of the fields and details are FFS.

## 6.5	Message-body

Depends on the operation type and the reference point, the combination of the primitive parameters may be different (see clause 7.2.1.1 [3]).

In HTTP request message, among the request parameters (see Table 7.2.1.1-1 [3]) Message-body shall include mandatory parameters except primitive type, to, from, request identifier parameter and may include conditional/optional parameters.

In HTTP response message, among the response parameters (see Table 7.2.1.1-2 and 7.2.1.1-3 [3]) Message-body shall include mandatory parameters except primitive type parameter and may include conditional/optional parameters.

# 7	Security Consideration

- 7.1	Authentication on HTTP Request Message

When sending the credential to be checked by Registrar CSE, Proxy-Authorization header should be used as specified in HTTP/1.1 (see RFC2617).

When sending the credential to be checked by Hosting CSE, Authorization header should be used as specified in HTTP/1.1.

When the credential to be checked by Hosting CSE is an Access Token which is compatible with OAuth 2.0 framework, the Bearer authentication scheme shall be used as specified in RFC6750.

Note: TS-0003 [oneM2M Security Solutions] does not provide any details on usage or provisioning the token.

- 7.2	Transport Layer Security

oneM2M primitive parameters contained in HTTP messages may be protected by TLS as hop-by-hop manner, not end-to-end. For the details, see clause 6.1 [TS-0003]

Annex A (Informative): Example Procedures

- A.1	AE Registration

The following diagram is HTTP mapping of procedure described in clause 7.3.4.2.1.

Figure A.1-1 oneM2M HTTP Binding Example – AE Registration

Annex B (Informative)	WebSocket

- B.1	Notification using WebSocket

WebSocket [3] can be used for transporting notifications to an AE/CSE. This can be useful for an AE/CSE which is not server-capable or cannot be reachable for delivery of unsolicited requests.

For example, when an AE needs to receive a notification message from the CSE, the AE establishes a WebSocket connection to a CSE. When a new notification message is generated, the notification will be sent to the AE as the data frame of the WebSocket.

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| v.0.1.0 | 2014-Jan-10 | Initial version of the TS |
| v.0.1.1 | 2014-Mar-04 | The first baseline TS with table of contents agreed with: 1. PRO-2014-0107R01-HTTP Binding TS TOC |
| v.0.2.0 | 2014-Apr-22 | Includes an agreed contribution at PRO#9.3 meeting: 1. PRO-2014-0125R01-HTTP_Basic_Flows Includes agreed contributions at PRO#10 F2F meeting: 1. PRO-2014-0131R03-HTTP-REST Overview 2. PRO-2014-0147-HTTP_binding_TS_clause2_clause3 3. PRO-2014-0148R01-HTTP_binding_TS_clause5 4. PRO-2014-0149R02-HTTP_binding_TS_clause6_clause7 5. PRO-2014-0159R03-HTTP Authentication 6. PRO-2014-0160R03-WebSocket based Notification Includes Rapporteur’s input: 1. Re-numbering clause numbers, figure numbers in clause 5 and a table number in clause 6.1 |
| v0.3.2 | 2014-Jun-18 | Includes agreed contribution at PRO#10.6 and PRO#10.7 meetings: 1. PRO-2014-0201R02-CRUD_mapping_on_HTTP 2. PRO-2014-0207-more_mapping_of_HTTP_status_codes Includes agreed contributions at PRO#11 meeting: 1. PRO-2014-0226R01-Send_Request_in_HTTP 2. PRO-2014-0240-TS-0009 cleanup |
| v0.4.0 | 2014-Jul-30 | Includes agreed contributions at PRO#12 meeting: 1. PRO-2014-0352R01-TS-0009_overview_of_HTTP_binding 2. PRO-2014-0353R01-TS-0009_header_mapping 3. PRO-2014-0354R03-TS-0009_cleanup 4. PRO-2014-0358R03-TS-0009_body_mapping 5. PRO-2014-0407-TS-0009_response_code |
| v0.4.3 | 2014-Aug-01 | Correcting clause number in 7.2 |
| V0.5.0 | 2014-Sep-26 | Includes agreed contributions at PRO#13 meeting: 1. PRO-2014-0474R01-HTTP_Binding_Overview 2. PRO-2014-0475R03-HTTP_Binding_CRUD_operations |