| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0018-V-0.4.0 |
| Document Name: | Industrial Domain Enablement |
| Date: | 2015-Oct-05 |
| Abstract: | This oneM2M Technical Report collects the use cases of the industrial domain and the requirements needed to support the use cases collectivly. Furthermore, the Technical Report also identifies necessary technical works need to be addressed while enhancing the oneM2M specifications in the future. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definitions, symbols, abbreviations  and acronyms	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

3.4	Acronyms	6

4	Conventions,	6

5	Introduction to Industrial Domain	7

5.1	Industrial Domain Overview	7

5.2	Technology Trends in Industrial Domain	8

6	Use Cases	10

6.1	An industrial use case for on-demand data collection for factories	10

6.1.1	Description	10

6.1.2	Source	10

6.1.3	Actors	10

6.1.4	Pre-conditions	11

6.1.5	Triggers	11

6.1.6	Normal Flow	11

6.1.7	Alternative Flow	11

6.1.8	Post-conditions	11

6.1.9	High Level Illustration	11

6.1.10	Potential requirements	12

6.2	Integrity of Data Collection Monitoring	12

6.2.1      Description	12

6.2.2    Source	13

6.2.3     Actors	13

6.2.4     Pre-conditions	13

6.2.5     Triggers	13

6.2.6      Normal Flow	14

6.2.7     Alternative flow	14

6.2.8     Post-conditions	14

6.2.9     High Level Illustration	15

6.2.10     Potential requirements	15

6.3	Data Process for Inter-factory manufacturing	16

6.3.1   	Description	16

6.3.2        Actors	16

6.3.3	Pre-conditions	16

6.3.4	Triggers	16

6.3.5	Normal Flow	16

6.3.6	Post-conditions	17

6.3.7	High Level Illustration	17

6.3.8	Potential requirements	17

6.4	Aircraft construction and maintenance	17

6.4.1      Description	17

6.4.2      Source	18

6.4.3      Actors	18

6.4.4      Pre-conditions	18

6.4.5      Triggers	18

6.4.6      Normal Flow	19

6.4.7      Alternative flow	19

6.4.8     Post-conditions	19

6.4.9     High Level Illustration	20

6.4.10     Potential requirements	20

6.5	Real Time Data Collection	21

6.5.1	Description	21

6.5.2	Source	21

6.5.3	Actors	21

6.5.4	Pre-conditions	22

6.5.5	Triggers	22

6.5.6	Normal Flow	22

6.5.7	Alternative flow	22

6.5.8	Post-conditions	22

6.5.9	High Level Illustration	22

6.5.10	Potential requirements	23

6.6	Data Encryption in Industrial Domain	23

6.6.1	Description	23

6.6.2	Actors	24

6.6.3	Pre-conditions	24

6.6.4	Triggers	24

6.6.5	Normal Flow	24

6.6.6	Post-conditions	25

6.6.7	High Level Illustration	25

6.6.8	Potential requirements	26

6.7	QoS/QoI monitoring in industrial domain	26

6.7.1	Description	26

6.7.2	Source	26

6.7.3	Actors	26

6.7.4	Pre-conditions	27

6.7.5	Triggers	27

6.7.6	Normal Flow	27

6.7.7	Alternative flow	27

6.7.8	Post-conditions	27

6.7.9	High Level Illustration	28

6.7.10	Potential requirements	28

7	Overview of Potential Requirements	28

8	High Level Architecture	30

8.1 Deployment Mapping Using IPE	30

8.2 Deployment Mapping Using Peer-to-Peer Communication	31

8.2 Conclusion	32

9	Security Analysis	32

10	Conclusion	32

Proforma copyright release text block	32

Annexes	32

Annex <y>: Bibliography	33

History	33

# 1	Scope

The present document collects the use cases of the industrial domain and the requirements needed to support the use cases collectively. In addition it also identifies the necessary technical work needed to be addressed while enhancing the oneM2M specifications in the future.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Not applicable.

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules

[i.2]	oneM2M TS-0011, “Common Terminology”

[i.3]	IEC TC News, http://www.iec.ch/tcnews/2014/tcnews_0214.htm.

[i.4]	http://www.is-inotek.or.jp/archive/05_Ishikuma_Smart_Manufacturing.pdf, Dec 2014.

[i.5]	IIC website, http://www.industrialinternetconsortium.org/.

[i.6]	IIC document ‘Engineering: The First Steps’, Sep 2014.

[i.7]	IIC report ‘Engineering Update: November 2014’, Nov 2014.

[i.8]	IEEE P2413 website, http://grouper.ieee.org/groups/2413/.

[i.9]	IEEE P2413 presentation ‘Standard for an Architectural Framework for the Internet of Things (IoT)’, Sep 2014.

[i.10]	IEEE P2413 report ‘oneM2M Specification Comment Collection’, Oct 2014.

[i.11]	SMLC website, https://smartmanufacturingcoalition.org/.

[i.12]	SMLC presentation, https://smartmanufacturingcoalition.org/sites/default/files/savannah_rivers_03-10-2014.pdf, March 2014.

[i.13]	oneM2M TS-0002: "Requirements".

[i,14]	oneM2M TS-0001 "Functional Architecture"

# 3	Definitions, symbols, abbreviations  and acronyms

- For the purposes of the present document, the terms and definitions given in oneM2M TS-0011, “Common Terminology” [i.2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [i.2].

## 3.1	Definitions

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

## 3.4	Acronyms

Acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Acronym format

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# 4	Conventions,

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Introduction to Industrial Domain

## 5.1	Industrial Domain Overview

In previous industrial domains, the information exchange from factory-to-factory or centre-to-factory needed support from humans. Normally the exchange is non-synchronous, discrete, inefficient and unable to achieve the capacity to respond rapidly to market changes.

Currently M2M technologies are considered to achieve the communication and interaction from machine-to-machine without human support. It brings opportunities to achieve synchronous, continuous and effective information exchange in manufacturing. Based on M2M, new manufacturing methods can be suitable for increasing complex requirements of future market.

Many industrial companies have been aware of the potential power to update traditional manufacturing systems by introducing M2M technologies, not only because of the technical requirements, such as improving the performance of productivity, quality, delivery, cost reduction and security, but also new opportunities to cooperate with other domains for mass production, and furthermore, to build the new architecture for next generation industry. Figure 5.1-1 is an example architecture.

Figure 5.1-1 Industrial Domain Architecture

In Figure 5.1-1, factories will be connected with manufacturing services via the M2M system(s). Generally, the gateway in the factory will collect data from the factory and send it to manufacturing services in a management centre. The service will be initiated by different management modules and sent to factories.

In addition, with the M2M system(s), the complex service can be sent to several factories synchronously, to enable effective collaboration between factories. Every factory is expected be able to make accurate decisions and to operate effectively, because it can work based on the results of analysis from data from all the factories rather than from itself alone. The management centre with manufacturing services is also expected to be able to make accurate decisions by utilizing field data from all factories, and also via other support systems, such as cloud computing, to improve efficiency of local or global services.

In the future, if more and more industry related domains, such as logistics and power management systems, can be connected into the M2M system, resources (warehouses, trucks, ships, power, etc.) can be integrated efficiently. Therefore more flexible services will be created to face this complex situation.

As oneM2M architecture provides general Application Layer, Common Services Layer and the Underlying Network Services Layer, and will be connected with other vertical systems, it is important to consider the integration of industrial domain systems with the oneM2M architecture.

Editor’s note: paragraph is expected to show the role of oneM2M.

## 5.2	Technology Trends in Industrial Domain

To accelerate the update of manufacturing systems, many worldwide organizations have been established and started making efforts.

In June 2014, the IEC (International Electrotechnical Commission), Standardization Management Board (SMB) set up a Strategy Group, SG8, to deal with a number of tasks related to Smart Manufacturing. [i.3]

Table 5.2-1 Industrial Domain Research in IEC SMB SG8 [i.4]

| Mission & Scope | • Develp a function model/reference architecture that helps to identify gaps in standardization based on to-be-collected use cases • Develop a common strategy for the implementation of Industry 4.0 • Extend standards towards: environmental conditions, security, properties, energy efficiency, product and functional safety |
| --- | --- |
| Technical Keywords | • Industrial process measurement, control and automation • Application: semantics relationships descriptive technologies • Services: web services /SOA repositories /cloud dependable connections • Communication: data access real-time communications |

IIC (Industrial Internet Consortium) was founded in March 2014 to bring together the organizations and technologies necessary to accelerate growth of the Industrial Internet by identifying, assembling and promoting best practices.  [i.5]

Table 5.2-2 Industrial Domain Research in IIC [i.6] [i.7]

| Mission & Scope | • Productivity and efficiencies can be improved by production process governing themselves with intelligent machines and devices • Real time data report from handheld digital device • Wearable sensors track location of employees in case of emergency • Future scenarios: new steering instruments will interlink things to ensure the entire value chain and trigger adjustments on the factory floor in case of chain changing; raw materials will be programmed to record standard process and their customer to realize automatic customization. |
| --- | --- |
| Technical Keywords | • Representative use case areas include connectivity, logistics, transportation, and healthcare • Key capabilities system characteristics including resilience, safety and security. (such as key system characteristic, intelligent and resilient control, operations support, connectivity, integration and orchestration, security, trust and privacy, and business viewpoint) • Data management and analytics • Security: endpoint security, secure communications and security management and monitoring (currently focused on general security use case) |

IEEE P2413 defines an architectural framework for the Internet of Things (IoT), which includs descriptions of various IoT domains including industrial domain and is sponsored by the IEEE-SA. [i.8]

Table 5.2-3 Industrial Domain Research in IEEE P2413 [i.9] [i.10]

| Mission & Scope | • Ranges from the connected consumer to smart home & buildings, e-health, smart grids, next generation manufacturing and smart cities • Promote cross-domain interaction instead of being confined to specific domains |
| --- | --- |
| Technical Keywords | • Energy efficiency during data transmission • Areas of interest: industrial Internet, cross sector common areas, common architecture, security safety privacy |

SMLC (Smart Manufacturing Leadership Coalition) is a non-profit organization committed to the development and deployment of Smart Manufacturing Systems. SMLC activities are built around industry-driven development, application and scaling of a shared infrastructure that will achieve economic-wide impact and manufacturing innovation. [i.11]

Table 5.2-4 Industrial Domain Research in SMLC [i.12]

| Mission & Scope | • To build a cloud-based, open-architecture platform that integrates existing and future plant level data, simulations and systems across manufacturing seams and orchestrate business real time action. |
| --- | --- |
| Technical Keywords | • Cloud-based networked data • Enterprise real-time • Plant level data • Information & action • Security |

Based on the information above and the current oneM2M architecture, the techology trends below are becoming more and more important:

- Data management and analytics

In some industrial organizations, data management  and data analytics are independent layers for data processing (such as filtering and catalogue management) and data analytics. Since large amounts of data are generated in industrial scenarios, further functionality design for data management and data analytics CSFs may need to be considered in oneM2M.

- Real-time command and control

M2M technologies enabled real-time response manufacturing practices in complex supplier networks. Realizing real-time command and control by highly available and time critical technologies will bring benefits to process automation and supply chains optimization. Use cases with real-time command and control features may need to be considered in oneM2M. Additionally, requirements from these use cases may need to be taken into consideration.

- Connectivity

Since connectivity in the industrial domain must co-exist and evolve with legacy protocols, legacy connectivity (both wired and wireless) and legacy wiring, connectivity for manufacturing processes need to be considered, which may have an impact on NSE functionalities.

- Security

Increased networking and wireless technologies are the main security concerns for industrial companies. Undoubtedly, the risk trade-off will not stop companies from manufacturing evolution. Thus a renewed risk for management and ensuring security for the industrial domain may need to be considered.

Meanwhile more trends, such as web services over M2M devices and protocols in industrial domain, will be further tracked and analyzed.

# 6	Use Cases

## 6.1	An industrial use case for on-demand data collection for factories

### 6.1.1	Description

In factories, a lot of data are created from Programmable logic controllers (PLCs) every second, and data are utilised to monitor production lines. These data are available via industrial bus systems, e.g. Real-time Ethernet. In order to monitor remotely, data are gathered by the M2M service platform that needs to interface with such industrial bus systems via M2M gateways. However, it is difficult to gather all data to the M2M service platform because sometimes more than 1M bit data are created per second. In such cases, only necessary data are gathered depending on situations and filtering / pre-processing of the raw data needs to be performed at the gateway.

This use case proposes that the oneM2M System offers pre-processing capabilities, e.g. rule-based collection policies (averages, thresholds …). These rules (e.g. in XML format) are called “data catalogues”.

### 6.1.2	Source

Hitachi

### 6.1.3	Actors

- PLC:  It controls sensors and devices in a production line according to embedded programs. It also has interface to Real-time Ethernet. It broadcasts data related to the production line to Real-time Ethernet.

- M2M Gateway: It provides an interface from the Real-time Ethernet to the OneM2M System. An application on the gateway collects necessary data from Real-time Ethernet according to the configuration called data catalogue, and send collected/pre-processed data to M2M service platform.

- M2M service platform: It stores data gathered from gateway(s), and provide data to applications. It also manages data catalogue in gateway(s).

- Application: An M2M Application in the Infrastructure Domain that monitors production lines by using collected data in M2M service platform, and send change request of data catalogue depending on situations.

- Real-time Ethernet: A technology standardized in IEC TC65. Ethernet is used at the physical layer, but upper protocol is designed for industry purpose. In this use case, broadcast protocol is assumed. On top of Ethernet cable, data are broadcast with ID. Address configuration is not necessary here.

- Internet connection: M2M service platform and gateway(s) are connected by the Internet physically.

### 6.1.4	Pre-conditions

- PLCs and gateway are connected to Real-time Ethernet. PLCs broadcast data to Real-time Ethernet. Gateway is configured to pick up necessary data from Real-time Ethernet.

- On top of the internet, VPN connection is established between the M2M service platform and gateway(s).

- The data catalogue is managed by the M2M service platform

### 6.1.5	Triggers

- Data catalogue is configured for the gateway to pick up data in the Real-time Ethernet

### 6.1.6	Normal Flow

- The Gateway picks up broadcasted data. It picks up only data that matches conditions described in the data catalogue. If data does not match the conditions, gateway ignores the data.

- The Gateway sends collected data to the M2M service platform.

- The M2M service platform receives data and stores it.

- Application utilise the data. For example, it monitors the status of the production line.

- If the application user finds some problems in a production line, he/she changes the data catalogue in the M2M service platform to collect all data related to the production line and sends the data catalogue to the target gateway.

### 6.1.7	Alternative Flow

None

### 6.1.8	Post-conditions

None

### 6.1.9	High Level Illustration

### 6.1.10	Potential requirements

- The gateway shall be able to collect data from the field area network (e.g. industrial bus systems) according to the data collection policy stored in the gateway.

- The data collection policy shall be manageable (configured, updated, deleted..) by M2M Applications on the M2M service platform

## 6.2	Integrity of Data Collection Monitoring

### 6.2.1      Description

In factories, a lot of data are created from Programmable logic controllers (PLCs) every second and data are utilised to monitor production lines. These data are available via industrial bus systems, e.g. Real-time Ethernet.

This type of data is called time series data which is a sequence of data points, typically consisting of successive measurements made over a time interval.

In order to monitor remotely, data are gathered by the oneM2M service platform that needs to interface with such industrial bus systems via the M2M gateway (MN)s.

However, when some of the data are lost due to some reasons, such as, damage of production line, temporal network delay, continuous network capacity overload and so on, action will be required immediately for safety. In addition, some considerations may be necessary, for example, switching to a new network service with more capacity, changing to the backup network or adjusting data collecting policy to address the original cause of data loss. Other considerations may be effective when remote monitoring application queries the oneM2M platform about condition of network traffic, e.g. temporal delay, continuous capacity overflow, or connection failure.

Similar to the remote monitoring application, the M2M gateway (MN) in each factory receives results  of analysis or some commands, which can be lost due to  for example, failure in analysis process, temporal network delay, or continuous network capacity overflow. M2M gateway (MN) can detect the loss when the analysis results or the commands in the form of time series data, or it can detect potential loss with the help of monitoring the condition of network traffic. When temporal network delay or continuous network capacity overflow occurs, analysis results or commands may be lost. This loss will also require immediate decision and addressing at the root cause.

This use case proposes that the oneM2M System shall be able to provide capability to collect, store time series data as well as monitor the integrity of data.

Moreover, the oneM2M System shall be able to provide the capability to monitor condition of network traffic.

### 6.2.2    Source

Hitachi and Huawei Technologies, CO., LTD

### 6.2.3     Actors

- PLC:  It controls sensors and devices in a production line according to embedded programs. It also has interface to Real-time Ethernet. It broadcasts data related to the production line to Real-time Ethernet.

- M2M Gateway (MN): It provides an interface from the Real-time Ethernet to the oneM2M System. The gateway collects and stores time series data from Real-time Ethernet then sends them to the oneM2M service platform. It also receives analysis results or commands from the oneM2M service platform. Furthermore, the gateway monitors the integrity of received analysis results or commands by monitoring the condition of the network. It can detect the loss when the analysis results or the commands in the form of time series data, or it can detect potential loss with the help of monitoring the condition of the network traffic when temporal network delay or continuous network capacity overflow occurs.

- oneM2M service platform: It stores data gathered from gateway(s), and provides data to applications.

- Application: An M2M Application in the Infrastructure Domain monitors production lines by using collected data in the oneM2M service platform and sends analysis results or commands depending on situations.

- Real-time Ethernet: A technology standardized in IEC TC65. Ethernet is used at the physical layer, however the upper protocol is designed for industry purposes. In this use case, broadcast protocol is assumed.

- Internet connection: the oneM2M service platform and gateway(s) are connected by the Internet physically.

### 6.2.4     Pre-conditions

- PLCs and the gateway are connected to the Real-time Ethernet. PLCs broadcast data to the Real-time Ethernet. The Gateway is configured to pick up necessary data from the Real-time Ethernet.

- On top of the internet, a VPN connection is established between the oneM2M service platform and gateway(s).

### 6.2.5     Triggers

- The gateway starts to receive time series data.

### 6.2.6      Normal Flow

The Gateway picks up time series data which is broadcasted at Real-time Ethernet. The gateway sends collected data to oneM2M service platform.

oneM2M service platform receives data, then stores it and sends it to the application.

The application monitors the integrity of the time series data which is sent from the gateway. If data loss occurs, the application user will decide to send a command for immediate control action. Then the application user will check the condition of the network traffic to determine which means are used to solve the data loss.

### 6.2.7     Alternative flow

None

### 6.2.8     Post-conditions

None

### 6.2.9     High Level Illustration

### 6.2.10     Potential requirements

- The oneM2M System shall be able to collect and store time series data, as well as monitor the integrity of this data.

- The oneM2M System shall be able to provide the capability to monitor the condition of the network traffic.

## 6.3	Data Process for Inter-factory manufacturing

### 6.3.1   	Description

To achieve remote manufacturing, numerous sensors are placed at product lines in factories and a significant amount of data is generated for monitoring purposes. This data is broadcast via an intra-factory network (e.g. real-time Ethernet) through PLCs or Distributed Control Systems (DCS), etc. For monitoring product lines efficiently and effectively, MNs (Middle Nodes which means the gateway) will selectively collect necessary data from intra-factory network, and then send this data to the oneM2M services platform for use by manufacturing control applications. The data collection policy (named data catalogue) utilized at the MNs is generated and managed by the application layer and may vary based on the specific monitoring purpose (e.g. collect only temperature data for the purpose of monitoring device temperature, or collect both humidity and gas data in order to monitor product quality).Data process functionality is also needed at the MNs in order to filter out error data or to summarize the percentage of data exceeding a threshold.

To respond rapidly to market changes, new factories may be needed to ensure sufficient productivity. Thus inter-factory collaboration provides significant benefit and efficiency which can be used in establishing a new factory where the reference data catalogue can be used to establish a new environment.

### 6.3.2        Actors

M2M Device: Sensors, controllers etc. located in factories (e.g. located at product lines) which measure and generate data.The PLC /DCS control sensors in production lines according to embedded programs. Both PLC and DCS can broadcast data related to production lines to intra-factory networks.

Intra-factory Network: In this use case, real-time Ethernet is assumed. It is standardized in IEC TC65 for which Ethernet is used at physical layer, but upper protocol is designed for industrial purposes. Meanwhile, broadcast protocol is assumed and data is broadcast with unique identifier/parameter (e.g. device ID).

oneM2M MN: Middle Node (gateway) provides an interface from the intra-factory network to the oneM2M System. The MN collects data from the intra-factory network according to the data catalogue, which is the data collection policy. The MN may process collected data and send the data to the oneM2M services platform through the underlying network.

oneM2M Services Platform: The oneM2M services platform stores data gathered from MNs, and provides data to applications.

oneM2M Application: A oneM2M application in the Infrastructure Domain that monitors production lines for remote manufacturing control by using collected data from the oneM2M services platform. For monitoring purposes, the M2M application defines and generates the data catalogue. Then, the application provides the data catalogue to MNs. The M2M application also shares data catalogue in the MNs with other factories.

### 6.3.3	Pre-conditions

The PLCs or DCSs control sensors in production lines according to embedded programs. Both PLC and DCS can broadcast data related to production lines into an intra-factory network.

Real-time Ethernet is assumed as the intra-factory network.Broadcast protocol is assumed and real-time Ethernet data is broadcast with a unique identifier/parameter (e.g. device ID).

### 6.3.4	Triggers

A remote manufacturing control application generates a data catalogue to collect data from product lines in factories.

### 6.3.5	Normal Flow

The normal flow steps are as follows:

- The application generates the data collection policy into a data catalogue, or updates the data catalogue when the monitoring purpose has changed. Then the application notifies the data catalogue to the MNs in a factory.

- The application providing the data catalogue to the MNs may include  the following condition:

- In some inter-factory collaboration cases, the application also provides the data catalogue to the MNs in other collaborating factories, e.g. a newly built factory.

- MNs start to selectively collect data from real-time Ethernet according to the data catalogue.

- The oneM2M services platform receives, stores and provides the data to manufacturing control applications.

- The application analyzes collected data for remote manufacturing control.

### 6.3.6	Post-conditions

The application utilizes (e.g. monitors and analyzes) the data collected according to the data catalogue.

### 6.3.7	High Level Illustration

### 6.3.8	Potential requirements

- The oneM2M system shall be able to share data collection policies among different applications.

## 6.4	Aircraft construction and maintenance

### 6.4.1      Description

In aircraft construction, there are precise regulations that specify the kind of screw and the amount of force that must be used to join specific parts. When it comes to passenger aircraft, there are thousands of such screws that must be tightened and precisely documented. Joints on the wings naturally require a different amount of force than those on an aircraft window.

The tools are wifi-enabled and can identify their precise location on the shop floor. The position of the aircraft in the hangar is also fixed. With fixed coordinates and wifi connectivity, we know, for example, that a particular tool is located at the vertical stabilizer. Instructions that specify the force it should use to tighten screws can automatically be sent to the tool.

Although this use case focuses on aircraft construction and maintenance, the connected power tool technology is expected to be effective in more applications:

- Safety-critical work processes are closely monitored and analyzed. Anomalies are automatically detected through the central processing, analysis, and visualization of production process data in near real time. Role-specific alerts can be triggered automatically.

- The power tool fleet manager has an exact overview of the power tool fleet status and utilization thanks to central access to process data. Organizational processes can be triggered automatically.

- Quality controls are automated and shifted to earlier stages of the production process. For example, hundreds of thousands of torque recordings are made available in their entirety for quality monitoring.

- Indoor geofencing alarms ensure that power tools are used according to regulations. Not all tools are allowed for all production and maintenance steps, e.g. in aircraft maintenance. As soon as power tools know their location, they can switch off when used in error.

### 6.4.2      Source

An article “First European testbed for the Industrial Internet Consortium” in Bosch’s ConnectedWorld Blog

http://blog.bosch-si.com/categories/manufacturing/2015/02/first-european-testbed-for-the-industrial-internet-consortium/

### 6.4.3      Actors

- Power tools: People in a shop floor utilize them to tighten screws of the airplane. They are wifi-enabled, and can identify their precise location on the shop floor. They receive instruction for the amount of power needed to tighten screws.

- Indoor localization technology: The technology is utilized to identify the location of the power tools.

- oneM2M service platform: oneM2M service platform receives location information of the power tools and sends them to the application. It also receives instructions for the amount of power needed to tighten the screws and sends them to each one of the power tools.

- Application: In this use case, the application is for aircraft construction and maintenance. It holds the position of the aircraft in the hangar and matches it to the location information of the power tools to calculate which power tool is utilized for which part of the aircraft. Then it gets instructions that specify the force a power tool should use to tighten screws by utilizing regulations that specify the kind of screw and the amount of force that must be used to join specific parts. Finally, it sends the instructions to each one of the power tools.

- Underline network: In this use case, wifi is assumed.

### 6.4.4      Pre-conditions

- The application holds the position of the aircraft in the hanger and the regulations that specify the kind of screw and the amount of force that must be used to joint specific parts.

- Power tools are wifi-enabled.

### 6.4.5      Triggers

- A person on a shop floor starts to use a power tool.

### 6.4.6      Normal Flow

- The power tool identifies precise location information with the use of indoor localization technology and sends the information to the oneM2M platform.

- The oneM2M service platform receives the location information and sends it to the application.

- The application matches the position of the aircraft in the hanger to the location information of the power tool to calculate for which part of the aircraft the power tool is used.

- The application gets instructions that specify the force the power tool should use to tighten screws by utilizing regulations that specify the kind of screw and the amount of force that must be used to join specific parts.

- The application sends the instruction to the oneM2M platform.

- The oneM2M platform receives the instruction and sends it to the power tool.

- The power tool uses the specified amount of force to tighten a screw.

### 6.4.7      Alternative flow

None

### 6.4.8     Post-conditions

None

### 6.4.9     High Level Illustration

### 6.4.10     Potential requirements

- The oneM2M System shall be able to support mechanism for the M2M Devices and/or Gateways to report their geographical location information to M2M Applications. (OSR-047)

- The oneM2M System shall support the ability for single or multiple M2M Applications to interact with a single or multiple M2M Devices/Gateways (application in the device/gateway) (OSR-009)

## 6.5	Real Time Data Collection

### 6.5.1	Description

In automated production with utilization of information and communication technology, behaviours of devices are controlled according to sensor values. In order to achieve adequate control, real-time Ethernet , with which sensors and devices are connected through controllers, are required to provide real-time transmission and a high-level of reliability. Real-time Ethernet is standardized in IEC TC65 and has characteristics such as the following.

- It enables real-time transmission by message priority control, according to the importance of the data (network re-configuration communication is first, time series data is second, controlling commands for devices are third, and information data is forth priority). “Information data” contains movie, audio data or objected sensor data. Higher priority data can be transmitted without interference from lower priority data. Priority of data depends not only on the kind of data but also on source node or destination node. For example, although controlling commands for devices shall not be delayed, log information for logging servers does not require strict real-time transmission. Therefore the data have a different value of priority.

- When real-time Ethernet is introduced, size and frequency of high priority data transmission are designed to ensure that the total volume of sent data does not exceed the capacity. Through this design process, real-time transmission is ensured and time series data and controlling commands are received within pre-defined intervals.

- It utilizes a duplex LAN, which consists of two physically independent network paths between end-nodes, or dual Node to provide high-level of reliability.

- It achieves a high level reliability on the basis of operations on ring based topologies and of following reconfiguration process. Each node connects to the ring via two ports. In the normal state, two neighboring nodes of the network become the Blocking State and cut off the connections logically between them to prevent loop. Each node monitors the neighboring segments including the blocking segment at all the time. If any segment of the network is disconnected, it will be back to a normal state within high-speed recovery time by automatically moving the blocking segments to each end of the faulty part thus isolating it.

- ・It broadcasts data to make each node have data, can enable autonomous de-centered distributed process where each node can keep working even if network failure has occurred somewhere, and  can achieve high-level reliability.

This use case describes the M2M Gateway which is a oneM2M MN and sends data received from real-time Ethernet.

The oneM2M MN should be able to cope with temporal performance degradation of underlying network, temporal increase of the amount of information data and with temporal increase of the amount of one data flow, which represents the collection of data that has the same source node, priority, and type of data. Therefore oneM2M MN shall be able to buffer data, transmit data in the order corresponding to priority and control transmission amount of data flow.

### 6.5.2	Source

None

### 6.5.3	Actors

- M2M Device: Sensors, controllers etc. located in factories (e.g. located at product lines) which measure and generate data. PLC/DCS control sensors in production lines according to embedded programs.

- Real-time Ethernet: In this use case, real-time Ethernet with the characteristics mentioned above is assumed. It is standardized in IEC TC65.

- oneM2M MN: Middle Node (gateway) provides an interface from the real-time Ethernet to the oneM2M System. The oneM2M MN is developed as a dual Node (primary and secondary) to achieve a high-level of reliability.

- oneM2M Services Platform: The oneM2M services platform stores data gathered from MNs and provides data to applications.

- oneM2M Application: A oneM2M application in the Industrial Domain

### 6.5.4	Pre-conditions

PLCs or DCSs send and receive data through real-time Ethernet.

### 6.5.5	Triggers

The primary MN and secondary MN receive data which is sent from PLCs or DCSs.

### 6.5.6	Normal Flow

- The primary MN receives data and buffers it. If the buffer is overloaded then the data with the lowest priority is discarded.

- The secondary MN also receives data and buffers it. If the buffer is overloaded then the   data with the lowest priority is discarded.

- The primary MN sends the buffered data with the highest priority to the oneM2M platform. If multiple data have the highest priority then the data flow of the least recent data transmitted is selected.

- The secondary MN confirms the status of the primary MN and, as the primary MN is active, it stops the secondary MN from sending buffered data.

- After a pre-defined time-interval the primary or secondary MN receives further data from either the PLCs or DCS.

### 6.5.7	Alternative flow

- When the secondary MN confirms that the primary MN is not working the secondary MN sends buffered data.

- After a pre-defined time-interval the secondary MN receives further data from either the PLCs or DCSs.

### 6.5.8	Post-conditions

When the sending data process is not completed within the time-interval, or the oneM2M platform does not receive the data, the primary or secondary MN sends the data again using buffered data.

### 6.5.9	High Level Illustration

Figure 6.5.10-1: TITLE

### 6.5.10	Potential requirements

- The oneM2M system shall be able to identify a series of data (e.g. time series data) and indicate the individual data belong to this series.

- The oneM2M system shall be able to transmit data according to priority (CRPR-003).

## 6.6	Data Encryption in Industrial Domain

### 6.6.1	Description

In smart factories data is essential for the execution of automation and efficient manufacturing. Data needs to be secured by measures such as authentication, authorization and encryption. As smart factories may connect to remote servers through multiple, partially external public networks, it cannot be assumed that these networks are secure. Encrypting data to avoid it being stolen in an external network and protecting the integrity of data to avoid it being modified is necessary for data security.

In the industrial domain the requirements of encryption are different (for exacmple, sensitive data such as confidential commercial secrets must be protected by strong encryption algorithms) and the capabilities of encryption/decryption vary for different devices (for example,  for some low-cost devices, encryption/decryption with a  long key may cause heavy loads to their poor processing units). Therefore each M2M application for smart factories needs proper encryption schemes to satisfy the various encryption requirements from application data while keeping the encryption/decryption loads acceptable to constrained devices.

M2M applications classify data into various levels based on the importance of the data and the capabilities of the devices. In the industrial domain the definition of sensitive data varies with products. For example, for manufactures of electric power grids, oil and gas, most data is sensitive since it includes confidential state secrets (required by governments to provide strong protection)while for makers of electronic products, information of product designs such as appearance or material is sensitive, since competitors stealing the product information will cause commercial damage. Based on common requirements from the industrial domain classification of application data is as follows, but not limit to, the categories below (dependent on M2M applications)

- Sensitive data: encryption strength is required (e.g. with longer key or electronic signature); sensitive data includes such as,

  - Confidential commercial secrets (e.g. customer information, intellectual properties, etc.).

  - Confidential data from infrastructure manufacturers (e.g. waveform data to diagnose, which is collected from devices in power grid).

  - Data for industrial control systems (including filed bus, SCADA/Supervisory Control And Data Acquisition, controllers as PLC/Programmable Logic Controller or DCS/Distributed Control System).

  - Keys transmitted for encryption algorithms.

- Normal data: encryption is recommended (optional) and proper applicable schemes should be adopted which are dependent on the capabilities of the devices; normal data includes such as,

  - Status data for product line and device monitoring (e.g. device availability) in normal products manufacture which do not include any commercial secrets.

  - Data for human resources monitoring and employee performance assessment (e.g. GPS information of workers collected from carried mobile tablets).

When various levels of application data in industrial domain is encrypted /decrypted based on its associated encryption scheme, the essential data is secured and the loads caused by normal data encryption are acceptable for constrained devices.

### 6.6.2	Actors

M2M Device: Machines, sensors, controllers etc. located in factories which measure and generate data. PLC /DCS control machines and sensors in production lines according to embedded programs.

Intra-factory Network: In this use case, the intra-factory network is assumed to be managed by factory owners (different from the Underlying Network operated by external parties).

oneM2M MN: Middle Node (gateway) collects data from the intra-factory network and sends the data to the oneM2M services platform through underlying network.

oneM2M Services Platform: Support secure data transmission and mapping of application data levels to applicable encryption schemes.

oneM2M Application: Classify data based on sensitivity.

### 6.6.3	Pre-conditions

The oneM2M Services Platform and security CSE inside M2M devices/MNs support various application data levels and mapping these levels to applicable encryption schemes.

### 6.6.4	Triggers

None.

### 6.6.5	Normal Flow

- The M2M application classifies data into various levels (e.g. into sensitive data and normal data).

- The oneM2M Services Platform or security CSE inside M2M devices/MNs map these levels to applicable encryption schemes.

- Each pair of transmitter and receiver performs preparation procedure (e.g. share symmetric key and store it in respective storage inside nodes) before sending encrypted data.

- The transmitter (e.g. a server located at one end of the application) prepares the data for sending; encrypts the data based on its associated security level (with respective encryption algorithm, key length etc.).

- The encrypted data is sent to the receiver through intra-factory network and public underlying networks.

- The receiver (e.g. a machine located at the other end of the application) receives the data from transmitter and decrypts the data.

### Post-conditions

The M2M application utilizes the decrypted data for smart manufacturing, such as executing orders, monitoring devices and diagnosis, monitoring workers’ location etc.

### 6.6.7	High Level Illustration

### 6.6.8	Potential requirements

- The oneM2M system shall support classification of application data by oneM2M applications into various security levels that are specified by oneM2M and support the mapping of these levels to applicable security capabilities.

## 6.7	QoS/QoI monitoring in industrial domain

### 6.7.1	Description

In factories a lot of data are generated from M2M devices (e.g. machines and program logic controllers) and the data are delivered to the M2M gateway via industrial bus systems, e.g. Real-time Ethernet. In addition, factory management applications can get factory status information through the oneM2M Service platform (Infrastructure Node) which gathers data from M2M gateways located in each factory domain.

In local industrial communications data packet transmission between M2M gateway and M2M devices has real-time transmission characteristic delivered over an Ethernet-based communication system. However to enable remote mechanisms (remote supervisory, operation, service), Wide Area Networks is composed of broad and heterogeneous communication technologies, e.g. digital wireless telecommunication systems (GSM-based, UMTS-based), digital wired telecommunication systems (ISDN, DSL).

In this environment the M2M gateway can use various telecommunication systems to send and receive data packets from the oneM2M service platform. In addition, according to industrial application service types, it requires hard real-time data delivery, soft real-time data delivery or real-time not requiring data delivery when it comes to communication between the M2M gateway and the oneM2M service platform.

If quality of service (QoS)required from the application could not be guaranteed, this situation limits service scenarios in industrial domains. In order to prevent this situation, the M2M gateway can decrease the volume of data needed to send the oneM2M platform via data processing based on data catalogue. At the same time, if the M2M gateway can monitor network environments, it can dynamically choose the network type or network provider who guarantees the required QoS.

In addition, to satisfy QoS, real-time data generated from M2M devices can be pre-processed/filtered, based on the data catalogue. In this situation, post-processed data is to include a kind of quality of information (QoI) and if QoI monitored and delivered to the oneM2M service platform, this information can be used  for further data processing in the oneM2M platform.

This use case proposes that the oneM2M system offers QoS/QoI Monitoring capabilities, which include data accuracy, data age, cost, communication, encryption etc.

### 6.7.2	Source

None.

### 6.7.3	Actors

- M2M Devices: Sensors, controllers etc. located in factories (e.g. located at product lines) which measure and generate data. PLC /DCS control sensors in production lines according to embedded programs.

- Real-time Ethernet: A technology standardized in IEC TC65 for use in industrial control system.

- MN Gateway (MN): It provides an interface from the Real-time Ethernet to the oneM2M system. The gateway collects data from M2M devices which are connected via Real-Time Ethernet communication technology. The gateway can conduct data pre-processing/filtering based on the data catalog delivered from the oneM2M service platform.

- oneM2M Service Platform (IN): It acts as a oneM2M Infrastructure Node. It communicates with MNs in the remote industrial domains and gathers the data from the MN gateway. The data in the oneM2M service platform can be delivered to the applications, e.g. factory monitoring application.

- Applications: An M2M application in the application service provider domain. It monitors production lines and sends analysis results or alert messages to the factory administrator

### 6.7.4	Pre-conditions

- Devices (e.g. PLC, Machines) and gateway are connected to Real-time Ethernet. PLCs broadcast data to Real-time Ethernet.

- The Gateway can have a capability of various network hardware interfaces (e.g. GSM-based, UMTS-based, ISDN, DSL) and can also use various network service providers who guarantee the required QoS level.

### 6.7.5	Triggers

- The Application initiates service which require QoS/QoI requirement (e.g. response time, data freshness).

- The Application sends the QoS/QoI requirement to oneM2M platform.

### 6.7.6	Normal Flow

- The oneM2M service platform requests QoS/QoI monitoring data from the Gateway (MN) and based on this information, the oneM2M service platform negotiates the supported QoS/QoI parameter with the application.

- To enable end-to-end services, the oneM2M service platform sends the QoS requirement to the Gateway (MN). Based on the QoS requirement, the Gateway dynamically chooses the network type and network service provider who guarantees the required QoS level.

- In the Gateway, the QoS/QoI monitoring function can annotate data with quality information.

- After receiving data from the Gateway (MN), the oneM2M service platform can further process the data referring to the quality attributes.

### 6.7.7	Alternative flow

None

### 6.7.8	Post-conditions

None

### 6.7.9	High Level Illustration

### 6.7.10	Potential requirements

- The oneM2M System shall support the inclusion of M2M Application’s QoS preference in service requests to Underlying Networks (OSR-038)

- The oneM2M System shall provide the capability to monitor and describe data streams with associated attributes, e.g. data freshness, accuracy, sampling rate, data integrity.

# 7	Overview of Potential Requirements

Potential requirements from all industrial use cases collected in this technical report are summarized as follows,

- The oneM2M System shall be able to collect data that is broadcast (e.g. in industrial bus systems) according to data collection policies. (OSR-081 [i.13])

Note: This requirement addresses the use case 6.1 “An industrial use case for on-demand data collection for factories”.

- The oneM2M system shall allow the update, modification, or deletion of data collection policies within an M2M application. (OSR-082 [i.13])

Note: This requirement addresses the use case 6.1.

- The oneM2M System shall be able to collect, store time series data. (OSR-075 [i.13])

Note: This requirement addresses the  use case 6.2 “Integrity of Data Collection Monitoring”.

- The oneM2M System shall be able to detect and report the missing data in time series. (OSR-076 [i.13])

Note: This requirement addresses the use case 6.2.

- The oneM2M System shall be able to support receipt of the status information of the Underlying Network if supported by the Underlying Network. (OPR-007 [i.13])

Note: This requirement addresses the use case 6.2.

- The oneM2M System shall be able to provide the M2M applications with status information received from the Underlying Network. (OPR-008 [i.13])

Note: This requirement addresses the use case 6.2.

- The oneM2M system shall be able to share data collection policies among multiple M2M Devices/Gateways within an M2M application service, or among different M2M application services. (OSR-097 [i.13])

Note: This requirement addresses the use case 6.3 “Data Process for Inter-factory manufacturing”.

- The oneM2M System shall be able to support mechanism for the M2M Devices and/or Gateways to report their geographical location information to M2M Applications. (OSR-047  [i.13])

Note: This requirement addresses the use case 6.4 “Aircraft construction and maintenance”

- The oneM2M System shall support the ability for single or multiple M2M Applications to interact with a single or multiple M2M Devices/Gateways (application in the device/gateway). (OSR-009 [i.13])

Note: This requirement addresses the use case 6.4.

- The oneM2M system shall be able to identify a series of data (e.g. time series data) and indicate individual data belong to this series. (CMR-015 [i.13])

Note: This requirement addresses the use case 6.5 “Real Time Data Collection”.

- The oneM2M system shall be able to transmit data according to priority.

Note: This requirement addresses the use case 6.5 and it is included in/supported by CMR-003 [i.13].

- The oneM2M system shall support classification of application data by oneM2M applications into various security levels that are specified by oneM2M and support the mapping of these levels to applicable security capabilities. (SER-045 [i.13])

Note: This requirement addresses the use case 6.6 “Data Encryption in Industrial Domain”.

- The oneM2M System shall support the inclusion of M2M Application’s QoS preference in service requests to Underlying Networks. (OSR-038 [i.13])

Note: This requirement addresses the use case 6.7 “QoS/QoI monitoring in industrial domain”.

- The oneM2M System shall provide the capability for monitoring and describing data streams with associated attributes e.g. data freshness, accuracy, sampling rate, data integrity. (OSR-092 [i.13])

Note: This requirement addresses the use case 6.7.

# 8	High Level Architecture

The seven use cases in the industrial domain discussed in this document are listed in Table 8-1.

Table 8-1: Use cases in the industrial domain

| Use case No. | Title | Description |
| --- | --- | --- |
| 1 | An industrial use case for on-demand data collection for factories | See clause 6.1 |
| 2 | Integrity of data collection monitoring | See clause 6.2 |
| 3 | Data process for inter-factory manufacturing | See clause 6.3 |
| 4 | Aircraft construction and maintenance | See clause 6.4 |
| 5 | Real Time Data Collection | See clause 6.5 |
| 6 | Data encryption in industrial domain | See clause 6.6 |
| 7 | QoS/QoI monitoring in industrial domain | See clause 6.7 |

The deployments which support these use cases require the use of M2M Devices which use broadcasting mode through the PLC or DCS.The following sections  provide the high level oneM2M architecture mapping for these deployments  .

## 8.1 Deployment Mapping Using IPE

Table 8.1-1 lists the mapping relationship between actors in industrial domain and nodes in oneM2M domain. Those devices under the M2M Gateway are non-oneM2M devices which can be mapped into the M2M Area Network. In this case ,the M2M Gateway which is mapped to the MN shall implement the Inter-working Proxy Application Entity (IPE).

Table 8.1-1: Mapping relationship 1

| Use case No. | Actors in the use case | oneM2M Node |
| --- | --- | --- |
| 1, 2, 3, 4, 5, 6, 7 | Application and M2M service platform | IN |
| 1, 2, 3, 5, 6, 7 | M2M Gateway | MN |
|  | M2M Devices (PLC/DCS, sensors) and Intra-factory network | Non-oneM2M Device in M2M Area Network |
| 4 | WiFi gateway | MN |
|  | Power tools and underlying network (WiFi) | Non-oneM2M Device in M2M Area Network |

Figure 8.1-1 illustrates Deployment Mapping Using  IPE.

Figure 8.1-1 Deployment Mapping Using IPE

## 8.2 Deployment Mapping Using Peer-to-Peer Communication

Table 8.2-1 lists the mapping relationship between actors in industrial domain  and nodes in oneM2M domain. In this case, all of the actors are assumed to be oneM2M compliant nodes.

Table 8.2-1: Mapping relationship 2

| Use case No. | Actors in the use case | oneM2M Node |
| --- | --- | --- |
| 1, 2, 3, 4, 5, 6, 7 | Application and M2M service platform | IN |
| 1, 2, 3, 5, 6, 7 | M2M Gateway | MN |
|  | M2M Devices (PLC/DCS, sensors) | ADN/ASN |
| 4 | WiFi gateway | MN |
|  | Power tools | ADN/ASN |

Figure 8.2-1 illustrates Deployment Mapping for Peer-to-Peer Communication. In the existing factory production line, M2M Devices with PLC/DCS will support peer-to-peer communication, which means data can be exchanged among nodes directly. Note that  peer-to-peer communication between CSEs of different ASNs is currently not supported by oneM2M architecture deployment (see 6.1 [i.14]) .

Figure 8.2-1 Deployment Mapping Using Peer-to-Peer Communication

## 8.2 Conclusion

Since M2M Devices with PLC/DCS need to support peer-to-peer communication,  the existing reference enhancement  between CSEs of different ASNs shall be considered in the future oneM2M releases.

# 9	Security Analysis

<Text> memo: Security analysis of above use cases

# 10	Conclusion

<Text> memo: summarize above contents and potential requirements

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A>:
Title of annex (style H9)

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V 0.1.0 | 24-Feb-2015 | Skeleton draft |
| V 0.2.0 | 15-Jun-2015 | REQ-2015-0526R01- Industrial Domain Overview REQ-2015-0532- Technology Trends in Industrial_Domain REQ-2015-0522R04- Integrity of Data Collection Monitoring REQ-2015-0551- CR to TR0018 Use Case 6 1 REQ-2015-0552R01- Data Process for Inter-factory manufacturing REQ-2015-0562R01- Aircraft Construction and Maintenance |
| V 0.3.0 | 14-Aug-2015 | REQ-2015-0600R02- Real Time Data Collection REQ-2015-0603R03- Data encryption in industrial domain REQ-2015-0606R01- QoS/QoI monitoring in industrial domain |
| V 0.4.0 | 05-Oct-2015 | REQ-2015-0640R01- TR-0018 Overview of Potential Requirements REQ-2015-0641R03- TR-0018- Industrial Domain Enablement Clause 8 High Level Architecture Editorial Updates |