| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0018-V-0.1.4 |
| Document Name: | Test Suite Structure and Test Purposes |
| Date: | 2016-November-01 |
| Abstract: | The Test Suite Structure and Test Purposes document for conformance testing consists of: - Defining the test suite structure by grouping the test purposes according to different criteria; - Specifing test purposes for conformance test. A test purpose is an informal description of the expected test behaviour. |
| Template Version:23 February 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2016, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	4

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	8

3.4	Acronyms	8

4	Conventions	8

5	Prerequisites and Test Configurations	9

5.1	Test Configurations	9

6	Test Suite Structure (TSS)	10

6.1	Structure for oneM2M tests	10

6.2	Test groups	10

6.2.1 	Functional Entity Groups	10

6.2.2 	Common Services Functions Subgroups	10

6.2.3 	oneM2M Resource Primitives specific Operations	11

6.3	Test Suite Structure (TSS) for oneM2M	11

7	Test Purposes (TP)	12

7.1	Introduction	12

7.1.1	TP definition conventions	12

7.1.2	TP Identifier naming conventions	12

7.1.3	Rules for the behaviour description	13

7.1.4	Source of TP definitions	13

7.1.5	ICS reference	13

7.2	Test Purposes for oneM2M Service Primitives	14

7.2.1.	Group AE	14

7.2.2.	Group CSE	15

7.2.2.1.	Registration (REG)	15

7.2.2.2.	Data Management and Repository (DMR)	20

7.2.2.3.	Subscription and Notification (SUB)	80

7.2.2.4.	Group Management (GMG)	102

7.2.2.5.	Discovery (DIS)	131

7.2.2.6.	Location (LOC)	139

7.2.2.7.	Device Management (DMG)	157

7.2.2.8.	Communication Management and Delivery Handling(CMDH)	157

7.2.2.9.	Security(SEC)	157

7.2.2.10.	Resource Announcement (ANNC)	158

Proforma copyright release text block	168

Annexes	168

Annex <A> Conformance Test Requirement	168

Annex <B>(Informative): TP template	171

Annex <y>: Bibliography	171

History	171

# 1	Scope

The present document …

EXAMPLE:	The present document provides the necessary adaptions to the endorsed document.

The Scope shall not contain requirements.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Clause 2.1 only shall contain normative (essential) references which are cited in the document itself. These references have to be publicly available and in English.

The following referenced documents are necessary for the application of the present document.

- Use the EX style, enclose the number in square brackets and separate it from the title with a tab (you may use sequence fields for automatically numbering references, see clause A.4: "Sequence numbering") (see example).

EXAMPLE:

[1]	 ETSI TR 102 473: "<Title>".

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

## 3.4	Acronyms

Acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Acronym format

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Prerequisites and Test Configurations

## 5.1	Test Configurations

Editor’s Note: Test configurations for AE are added in the future extensions.

IUT

TEST SYSTEM

AE

CSE

Mca

Figure 1. Test configuration 1 (CF01)

Mcc

IUT

TEST SYSTEM

AE

CSE

Mca

CSE

Figure 2. Test configuration 2 (CF02)

# 6	Test Suite Structure (TSS)

## 6.1	Structure for oneM2M tests

## 6.2	Test groups

The test groups are organized in 2 levels. The first level defines the functional entity of oneM2M. The second level defines the common services functions which are represented in oneM2M functional architecture.

### 6.2.1 	Functional Entity Groups

The functional entity groups identify the types of entity which are specified in oneM2M functional architecture(TS-0001).

- AE

- CSE

### 6.2.2 	Common Services Functions Subgroups

The common services functions subgroups identify the general procedures in each common services functions specified in oneM2M functional architecture(TS-0001).

- Registration

- Data Management and Repository

- Subscription and Notification

- Group Management

- Discovery

- Location

- Device Management

- Communication Management and Delivery Handling

- Security

### 6.2.3 	oneM2M Resource Primitives specific Operations

A group of resource type specific procedures are defined in oneM2M functional architecture (TS-0001) and service layer core protocol (TS-0004). Those procedures can be potentially abstracted with requirements for generating test purposes for conformance testing purpose. Those resource type specific procedures are the complementary procedures to the general procedures defined in oneM2M Common Service Functions (CSF) and will use for subgroups of structures for oneM2M tests. A list of those procedures are including:

- Resource announcement

- Editor’s Note: more procedures will be added for future tests extension

## 6.3	Test Suite Structure (TSS) for oneM2M

The Test Suite Structure (TSS) for oneM2M will be present as bellow:

- Group 1: AE(AE)

  - Subgroup 1.1: Registration(REG)

  - Subgroup 1.2: Data Management and Repository(DMR)

  - Subgroup 1.3: Subscription and Notification(SUB)

  - Subgroup 1.4: Group Management(GMG)

  - Subgroup 1.5: Discovery(DIS)

  - Subgroup 1.6: Location(LOC)

  - Subgroup 1.7: Device Management(DMG)

  - Subgroup 1.8: Communication Management and Delivery Handling(CMDH)

  - Subgroup 1.9: Security(SEC)

- Group 2: CSE(CSE)

  - Subgroup 2.1: Registration(REG)

  - Subgroup 2.2: Data Management and Repository(DMR)

  - Subgroup 2.3: Subscription and Notification(SUB)

  - Subgroup 2.4: Group Management(GMG)

  - Subgroup 2.5: Discovery(DIS)

  - Subgroup 2.6: Location(LOC)

  - Subgroup 2.7: Device Management(DMG)

  - Subgroup 2.8: Communication Management and Delivery Handling(CMDH)

  - Subgroup 2.9: Security(SEC)

  - Subgroup 2.10: Resource Announcement (ANNC)

# 7	Test Purposes (TP)

## 7.1	Introduction

### 7.1.1	TP definition conventions

The TP definition is constructed according to Test Suite Structure & Test Purposes(TSS&TP) in Testing Framework(TS-0015).

### 7.1.2	TP Identifier naming conventions

The identifier of the TP is constructed according to table 7.1.2.-1.

Table 7.1.2.-1: TP naming convention for oneM2M

| TP/<root>/<gr>/<sgr>/<xx>/<nnn> |  |  |
| --- | --- | --- |
| <root> = root | oneM2M | oneM2M |
| <gr> = group | AE | Application Entity |
|  | CSE | Common Services Entity |
| <sgr> = sub- group | REG | Registration |
|  | DMR | Data Management and Repository |
|  | SUB | Subscription and Notification |
|  | GMG | Group Management |
|  | DIS | Discovery |
|  | LOC | Location |
|  | DMG | Device Management |
|  | CMDH | Communication Management and Delivery Handling |
|  | SEC | Security |
|  | ANNC | Resource Announcement |
| <xx> = type of testing | BV | Valid Behaviour tests |
|  | BI | Invalid Behaviour tests |
| <nnn> = sequential number |  | 001 to 999 |

### 7.1.3	Rules for the behaviour description

### 7.1.4	Source of TP definitions

### 7.1.5	ICS reference

Editor note: the table 7.1.5.-1 and 7.1.5.-2 are for examples and are here for being referenced in the Table 7.1.5.-3: mnemonics for PICS reference. Table 7.1.5.-1 and 7.1.5.-2 should be included in the ICS TS-0017.

Table 7.1.5.-1: Node

| Item | Name of field | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | ASN (Application Service Node) | 6.1 | o.1 |  |
| 2 | ADN (Application Dedicated Node) | 6.1 | o.1 |  |
| 3 | IN (Infrastructure Node) | 6.1 | o.1 |  |
| 4 | MN (Middle Node) | 6.1 | o.1 |  |
| o.1: Exactly one item shall be supported |  |  |  |  |

Table 7.1.5.-2: Functional entity type

| Prerequisite: A.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Name of field | Reference | Status | Support |
| 1 | AE | 5.2.1 | c.1 |  |
| 2 | CSE | 5.2.1 | c.2 |  |
| c.1: If A.1-1 or A.1-2 then m else o c.2: If A.1-2 x else m |  |  |  |  |

Table 7.1.5.-3: Mnemonics for PICS reference

| Mnemonic | PICS item |
| --- | --- |
| PICS_AE | A.2/1 [PICS document] |
| PICS_CSE | A.2/2 [PICS document] |
| PICS_ASN | A.1/1 [PICS document] |
| PICS_ADN | A.1/2 [PICS document] |
| PICS_IN | A.1/3 [PICS document] |
| PICS_MN | A.1/4 [PICS document] |

## 7.2	Test Purposes for oneM2M Service Primitives

Editor Note: The following TPs contain some test variables (using upper-case and italic) which will be instantiated during test execution.

## Group AE

Editor Note: Test Purposes for testing AE will add there. Test Purposes for AE generat

## Group CSE

## Registration (REG)

| TP Id |  | TP/oneM2M/CSE/REG/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, C-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 - case C, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a ServiceSubscribedNode resource containing RuleLinks attribute pointing to a ServiceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing Resource-Type set to 2 (AE) and From set to empty and Content <AE> containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/REG/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed App-ID, C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 - case d, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a ServiceSubscribedNode resource containing RuleLinks attribute pointing to a ServiceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing Resource-Type set to 2 (AE) and From set to C-AE-ID-STEM and Content <AE> containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/REG/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (not allowed App-ID) |  |
| Reference |  | TS-0001 10.1.1.2.2 – step 3, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing no ServiceSubscribedNode resource containing RuleLinks attribute pointing to a ServiceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing Resource-Type set to 2 (AE) and Content <AE> containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/REG/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (allowed App-ID, not allowed C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 – step 3, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a ServiceSubscribedNode resource containing RuleLinks attribute pointing to a ServiceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and no allowedAEs attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing Resource-Type set to 2 (AE) and From set to C-AE-ID-STEM and Content <AE> containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/REG/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, S-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 - case a, 9.6.19 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a ServiceSubscribedNode resource containing RuleLinks attribute pointing to a ServiceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating a value starting with ‘S’ and containing wildcard } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing Resource-Type set to 2 (AE) and From set to empty and Content <AE> containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a CREATE request to the IN-CSE containing Resource-Type set to 10002 (AEAnnc) and From set to ‘/S’ and Content <AEAnnc> containing App-ID attribute set to APP-ID and nodeLink attribute set to ??? and Labels indicating ‘Credential-ID:None’ } |  | IUT  IN-CSE |

## Data Management and Repository (DMR)

| TP Id |  | TP/oneM2M/CSE/DMR/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully an empty content of TARGET_RESOURCE_ADDRESS resource when the ResultContent set to 0 (Nothing). |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and ResultContent set to 0 (Nothing) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and no Content attribute } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content ist set to 1 (attributes) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and Result Content set to 1 (attributes) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only hierarchichal address of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 2 (hierarchical address) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and Result Content set to 2 (hierarchical address and attributes) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing RESOURCE_TYPE resource containing no attributes and hierarchichal address and no child resources and no child resource references } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes and hierarchichal address of TARGET_RESOURCE_ADDRESS resource when Result Content is set 3 (attributes and hierarchichal address) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and Result Content set to 3 (attributes and hierarchichal address) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing RESOURCE_TYPE resource containing attributes and hierarchichal address and no child resources and no child resource references } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes and child resources of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 4 (attributes and child resources) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and Result Content set to 4 (attributes and child resources) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and child resources and no child resource references } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes and child resource references of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 5 (attributes and child resource references) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and Result Content set to 5 (attributes and child resource references) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and child resource references } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only child resource references of TARGET_RESOURCE_ADDRESS resource when Result Content is set to 6 (child resource references) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_IDENTIFIER and Result Content set to 6 (child resource references) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing RESOURCE_TYPE resource containing no attributes and no hierarchichal address and no child resources and child resource references } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with resource name not provided by AE |  |
| Reference |  | TS-0001 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and Resource-Type set to RESOURCE_TYPE and From set to AE-ID and no Name attribute and Content containing RESOURCE_TYPE resource containing no ResourceName attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Name and Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing ResourceName attribute } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/BV/001_01 | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/BV/001_02 | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/BV/001_03 | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/BV/001_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/BV/001_05 | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/BV/001_06 | TS-0001 10.2.11.2 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with a non-existing resource name provided by AE |  |
| Reference |  | TS-0001 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the AE containing no child resource containing name attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and Resource-Type set to RESOURCE_TYPE and From set to AE-ID and Name attribute set to NAME and Content containing RESOURCE_TYPE resource containing ResourceName attribute set to NAME } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/BV/002_01 | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/BV/002_02 | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/BV/002_03 | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/BV/002_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/BV/002_05 | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/BV/002_06 | TS-0001 10.2.11.2 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource with an existing resource name provided by AE |  |
| Reference |  | TS-0001 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the AE containing a child resource containing ResourceNamename attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and Resource-Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource containing ResourceName attribute set to NAME } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5106 (ALREADY_EXISTS) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/BV/003_01 | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/BV/003_02 | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/BV/003_03 | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/BV/003_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/BV/003_05 | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/BV/003_06 | TS-0001 10.2.11.2 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource when AE has no privileges |  |
| Reference |  | TS-0001 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE not having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to AE_RESOURCE_ADDRESS and Resource-Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/BV/004_01 | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/BV/004_02 | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/BV/004_03 | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/BV/004_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/BV/004_05 | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/BV/004_06 | TS-0001 10.2.11.2 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE request of <container> resource when the RW ATTRIBUTE is provided with an invalid value |  |
| Reference |  | TS-0001 10.2.4.1, TS-0004 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource-Type set to <container> type and no name attribute and Content attribute containing a RW ATTRIBUTE } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP ID | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/BV/005_01 | TS-0001 9.6.6, TS-0004 7.4.7.1 | MaxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/BV/005_02 | TS-0001 9.6.6, TS-0004 7.4.7.1 | MaxByteSize |
| TP/oneM2M/CSE/DMR/CRE/BV/005_03 | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxInstanceAge |

Editor’s note: When one or more than one of the ATTRIBUTE list as above is/are included in the <contianer> CREATE request, the IUT will respond with BAD_REQUEST error.

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns a default value to RW ATTRIBUTE in the response of a <container> CREATE request where no a RW ATTRIBUTE is provided by AE |  |
| Reference |  | TS-0001 10.2.4.1, TS-0004 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource-Type set to <container> type and no name attribute and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing ATTRIBUTE with assigned field value by the IUT } |  | IUT  AE |

| TP ID | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/BV/006_01 | TS-0001 9.6.6, TS-0004 7.4.7.1 | MaxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/BV/006_02 | TS-0001 9.6.6, TS-0004 7.4.7.1 | MaxByteSize |
| TP/oneM2M/CSE/DMR/CRE/BV/006_03 | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxInstanceAge |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with error when the newly created <contentInstance> results in that the field value of attribute currentNrOfInstances exceeds the field value of maxNrOfInstances in the parent <container> resource. |  |
| Reference |  | TS-0001 10.2.19.2, TS-0004 7.4.8.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource TARGET_RESOURCE_ADDRESS containing attribute currentNrOfInstances set to value MAX_NUMBER_OF_INSTANCES and attribute maxNrOfInstances set to value MAX_NUMBER_OF_INSTANCES and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource-Type set to <contentInstance> type and no name attribute and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with error when the newly created <contentInstance> results in the field value set in attribute currentByteSize exceeds the field value set in maxByteSize in the parent <container> resource. |  |
| Reference |  | TS-0001 10.2.19.2, TS-0004 7.4.8.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE a container resource TARGET_RESOURCE_ADDRESS containing attribute maxByteSize set to value MAX_BYTE_SIZE_OF_DATA and attribute currentByteSize set to value MAX_BYTE_SIZE_OF_DATA and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource-Type set to <contentInstance> type and no name attribute and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BV/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT increaments and then copies the field value of attribute stateTag in parent <container> resource when <contentInstance> resource is created as the direct child of the parent <container> |  |
| Reference |  | TS-0001 9.6.1.3.2, TS-0004 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE a container resource TARGET_RESOURCE_ADDRESS containing attribute stateTag set to COUNT_NUMBER and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource-Type set to <contentInstance> type and no name attribute and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing stateTag set to value (COUNT_NUMBER + 1) } |  | IUT  AE |

Editor’s Note: The stateTag gets increased whenever there are child resources created, updated or deleted.

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BI/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE request of a <latest> resource as a direct child of a <container> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.22, TS-0004 7.4.28.2.2 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> TARGET_RESOURCE_ADDRESS resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no name attribute and Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/BI/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE request of a <oldest> resource as a direct child of a <container> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.23, TS-0004 7.4.29.2.2 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> TARGET_RESOURCE_ADDRESS resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource-Type set to A_RESOURCE_TYPE_VALUE and no name attribute and Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing the resource TARGET_RESOURCE_ADDRESS } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BV/001_01 | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/BV/001_02 | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/BV/001_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/BV/001_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/BV/001_05 | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/BV/001_06 | TS-0001 10.2.11.3 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BO/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference |  | TS-0001 10.1.2 - item 13) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BO/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve the resource TARGET_RESOURCE_ADDRESS without having privileges for the RETRIEVE operation |  |
| Reference |  | TS-0001 10.1.2 - item 14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE not having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BO/003_01 | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/BO/003_02 | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/BO/003_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/BO/003_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/BO/003_05 | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/BO/003_06 | TS-0001 10.2.11.3 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource (“To” option) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_ATTRIBUTE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing attributeList containing ATTRIBUTE attribute } |  | IUT  AE |
| Notes: TARGET_ATTRIBUTE_ADDRESS = TARGET_RESOURCE_ADDRESS # ATTRIBUTE ATTRIBUTE = Common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BV/004_01 | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/BV/004_02 | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/BV/004_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/BV/004_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/BV/004_05 | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/BV/004_06 | TS-0001 10.2.11.3 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource (“Content” option) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing attributeList containing attribute containing name set to ATTRIBUTE and no value name } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing attributeList containing the attribute ATTRIBUTE attribute } |  | IUT  AE |
| Notes: ATTRIBUTE = Common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BV/005_01 | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/BV/005_02 | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/BV/005_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/BV/005_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/BV/005_05 | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/BV/005_06 | TS-0001 10.2.11.3 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing attributeList containing attribute containing name set to ATTRIBUTE_1 name and no value and attribute containing name set to AATTRIBUTE_2 name and no value } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing attributeList containing the attribute ATTRIBUTE_1 attribute and the attribute ATTRIBUTE_2 attribute } |  | IUT  AE |
| Notes: ATTRIBUTE_1 = Common attribute for all RESOURCE_TYPE resources listed in the table below ATTRIBUTE_2 = Common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BV/006_01 | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/BV/006_02 | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/BV/006_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/BV/006_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/BV/006_05 | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/BV/006_06 | TS-0001 10.2.11.3 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BO/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource (“single” attribute case) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing attributeList containing ATTRIBUTE name } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BO/007_01 | TS-0001 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/BO/007_02 | TS-0001 10.2.7.3 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/RET/BO/007_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/BO/007_04 |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/RET/BO/007_05 | TS-0001 10.2.13.3 | 15 (pollingChallenge) | labels |
| TP/oneM2M/CSE/DMR/RET/BO/007_06 | TS-0001 10.2.11.3 | 23 (subscription) | labels |

Editor Note: A common ATTRIBUTE has been chosen for all types of resource. A note could replace ATTRIBUTE column.

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BO/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource (“multiple” attributes case) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing no attribute ATTRIBUTE_1 and no attribute ATTRIBUTE_2 and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing attributeList containing attribute containing name set to ATTRIBUTE_1 name and no value and attribute containing name set to AATTRIBUTE_2 name and no value } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/BO/008_01 | TS-0001 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/BO/008_02 | TS-0001 10.2.7.3 | 9 (group) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/BO/008_03 | TS-0001 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/BO/008_04 |  | 18 (schedule) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/BO/008_05 | TS-0001 10.2.13.3 | 15 (pollingChallenge) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/BO/008_06 | TS-0001 10.2.11.3 | 23 (subscription) | labels | announceTo |

Editor Note: The references to the TS-0001 for the operations (CREATE, RETRIEVE) procedures of the resource type <schedule> have not been found. To check whether these procedures are missing or they have been added in newer versions (TS-0001 v1.12.0 as basis)

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BO/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the RETRIEVE request of a <latest> resource target to a <container> resource that has no direct child <contentInstance> resources. |  |
| Reference |  | TS-0001 10.2.22.1, TS-0004 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing no direct child <contentInstance> resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BV/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the RETRIEVE request of the oldest <contentInstance> target to a <container> resource |  |
| Reference |  | TS-0001 10.2.23.1, TS-0004 7.4.29.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing <contentInstance> resource(s) } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing Representation of <contentInstance> } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BO/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the RETRIEVE request of a <oldest> resource target to a <container> resource that has no direct child <contentInstance> resources. |  |
| Reference |  | TS-0001 10.2.23.1, TS-0004 7.4.29.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing no direct child <contentInstance> resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/RET/BV/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the RETRIEVE request of the latest <contentInstance> target to a <container> resource |  |
| Reference |  | TS-0001 10.2.22.1, TS-0004 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing a <contentInstance> resource containing attribute creationTime having the most rencent resource creation time } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing Representation of <contentInstance> } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW attribute ATTRIBUTE_NAME set to VALUE_1 and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and (optional) Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 | VALUE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BV/001_01 | TS-0001 10.2.4.3 | 3 (container) | labels | “VALUE_1” | “VALUE_2” |
| TP/oneM2M/CSE/DMR/UPD/BV/001_02 | TS-0001 10.2.7.4 | 9 (group) | labels | “VALUE_1” | “VALUE_2” |
| TP/oneM2M/CSE/DMR/UPD/BV/001_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels | “VALUE_1” | “VALUE_2” |
| TP/oneM2M/CSE/DMR/UPD/BV/001_04 |  | 18 (schedule) | labels | “VALUE_1” | “VALUE_2” |
| TP/oneM2M/CSE/DMR/UPD/BV/001_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels | “VALUE_1” | “VALUE_2” |
| TP/oneM2M/CSE/DMR/UPD/BV/001_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels | “VALUE_1” | “VALUE_2” |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT adds successfully the attribute ATTRIBUTE_NAME to the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing no optional RW attribute ATTRIBUTE_NAME and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and (optional) Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_1 } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BV/002_01 | TS-0001 10.2.4.3 | 3 (container) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BV/002_02 | TS-0001 10.2.7.4 | 9 (group) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BV/002_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BV/002_04 |  | 18 (schedule) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BV/002_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BV/002_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels | “VALUE_1” |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes successfully the attribute ATTRIBUTE_NAME from the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing an optional RW attribute ATTRIBUTE_NAME and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to NULL } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and (optional) Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BV/003_01 | TS-0001 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/BV/003_02 | TS-0001 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/BV/003_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/BV/003_04 |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/BV/003_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/BV/003_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates the value of the attribute ATTRIBUTE_NAME_1, and creates the ATTRIBUTE_NAME_2 and deletes the ATTRIBUTE_NAME_3 of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW attribute ATTRIBUTE_NAME_1 set to VALUE_1 and no optional RW attribute ATTRIBUTE_NAME_2 and an optional RW attribute ATTRIBUTE_NAME_3 and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME_1 set to VALUE_2 and ATTRIBUTE_NAME_2 set to VALUE_3 and ATTRIBUTE_NAME_3 set to NULL } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and (optional) Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME_1 set to VALUE_2 and ATTRIBUTE_NAME_2 set to VALUE_3 and ATTRIBUTE_NAME_3 } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME_1 | ATTRIBUTE_NAME_2 | ATTRIBUTE_NAME_3 | VALUE_1 | VALUE_2 | VALUE_3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BV/004_01 | TS-0001 10.2.4.3 | 3 (container) | labelsexpirationTime | maxNrOfInstancesaccessControlPolicyIDs | labelsmaxNrOfInstances | - | "20001231T012345" | 5 |
| TP/oneM2M/CSE/DMR/UPD/BV/004_02 | TS-0001 10.2.7.4 | 9 (group) | groupNamelabels | accessControlPolicyIDs | labelsgroupName | "VALUE_1" | "VALUE_2" |  |
| TP/oneM2M/CSE/DMR/UPD/BV/004_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | privilegeslabels | announceTo- | labels- | acos: 63 | acos: 61 | PX_URI_CSE |
| TP/oneM2M/CSE/DMR/UPD/BV/004_04 |  | 18 (schedule) | labelsscheduleElement | announceTo- | -labels | "0,0,0 1 2,1,1,*" | "1,1,1 1 2,1,1,*" | PX_URI_CSE |
| TP/oneM2M/CSE/DMR/UPD/BV/004_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels- | accessControlPolicyIDs | -labels | - | - |  |
| TP/oneM2M/CSE/DMR/UPD/BV/004_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels | accessControlPolicyIDs | expirationCounter | "VALUE_1" | "VALUE_2" |  |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BO/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update an attribute of a TARGET_RESOURCE_ADDRESS resource which does not exist |  |
| Reference |  | TS-0001 10.1.3 – item 15) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BO/005_01 | TS-0001 10.2.4.3 | 3 (container) | labelsexpirationCounter | 1 |
| TP/oneM2M/CSE/DMR/UPD/BO/005_02 | TS-0001 10.2.7.4 | 9 (group) | labelsexpirationCounter | 1 |
| TP/oneM2M/CSE/DMR/UPD/BO/005_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labelsexpirationCounter | 1 |
| TP/oneM2M/CSE/DMR/UPD/BO/005_04 |  | 18 (schedule) | expirationCounterlabels | 1 |
| TP/oneM2M/CSE/DMR/UPD/BO/005_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | expirationCounterlabels | 1 |
| TP/oneM2M/CSE/DMR/UPD/BO/005_06 | TS-0001 10.2.11.4 | 23 (subscription) | labelsmaxNrOfInstances | 5 |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BO/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update the attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource without having privileges for the UPDATE operation |  |
| Reference |  | TS-0001 10.1.3 – 16) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW attribute ATTRIBUTE_NAME set to VALUE_1 and the AE not having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BO/006_01 | TS-0001 10.2.4.3 | 3 (container) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BO/006_02 | TS-0001 10.2.7.4 | 9 (group) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BO/006_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BO/006_04 |  | 18 (schedule) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BO/006_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels | “VALUE_1” |
| TP/oneM2M/CSE/DMR/UPD/BO/006_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels | “VALUE_1” |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BO/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update a non-RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a non-RW attribute ATTRIBUTE_NAME and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 | VALUE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BO/007_01 | TS-0001 10.2.4.3 | 3 (container) | labels | labelscreationTime | "20001231T012345" |
| TP/oneM2M/CSE/DMR/UPD/BO/007_02 | TS-0001 10.2.7.4 | 9 (group) | labels | creationTimelabels | "20001231T012345" |
| TP/oneM2M/CSE/DMR/UPD/BO/007_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels | creationTimelabels | "20001231T012345" |
| TP/oneM2M/CSE/DMR/UPD/BO/007_04 |  | 18 (schedule) | labels | creationTimelabels | "20001231T012345" |
| TP/oneM2M/CSE/DMR/UPD/BO/007_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels | creationTimelabels | "20001231T012345" |
| TP/oneM2M/CSE/DMR/UPD/BO/007_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels | creationTimelabels | "20001231T012345" |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BO/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete a mandatory RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a mandatory RW attribute ATTRIBUTE_NAME and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to NULL } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BO/008_01 | TS-0001 10.2.4.3 | 3 (container) | labels | labelsexpirationTime |
| TP/oneM2M/CSE/DMR/UPD/BO/008_02 | TS-0001 10.2.7.4 | 9 (group) | labels | expirationTimelabels |
| TP/oneM2M/CSE/DMR/UPD/BO/008_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels | expirationTimelabels |
| TP/oneM2M/CSE/DMR/UPD/BO/008_04 |  | 18 (schedule) | labels | expirationTimelabels |
| TP/oneM2M/CSE/DMR/UPD/BO/008_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels | expirationTimelabels |
| TP/oneM2M/CSE/DMR/UPD/BO/008_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels | expirationTimeLabels |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BO/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update a RW attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource with an UNACCEPTABLE_VALUE |  |
| Reference |  | TS-0001 10.1.3 – item 17) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW attribute ATTRIBUTE_NAME set to VALUE_1 and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to UNACCEPTABLE_VALUE } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/BO/009_01 | TS-0001 10.2.4.3 | 3 (container) | labels |  |
| TP/oneM2M/CSE/DMR/UPD/BO/009_02 | TS-0001 10.2.7.4 | 9 (group) | labels |  |
| TP/oneM2M/CSE/DMR/UPD/BO/009_03 | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |  |
| TP/oneM2M/CSE/DMR/UPD/BO/009_04 |  | 18 (schedule) | labels |  |
| TP/oneM2M/CSE/DMR/UPD/BO/009_05 | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |  |
| TP/oneM2M/CSE/DMR/UPD/BO/009_06 | TS-0001 10.2.11.4 | 23 (subscription) | labels |  |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the stateTag attribute of a <Container> resource is increased when an update operation has been performed on its child resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <Container> resource containing CHILD_RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_1 and the AE having privileges to perform UPDATE operation on the <Container> child resource and the IUT having updated TARGET_CHILD_RESOURCE_ADDRESS resource} |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid RETRIEVE response containing PrimitiveContent containing RESOURCE_TYPE resource containing stateTag attribute set to 2 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE request of an existing <contentInstance> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.19.4, TS-0004 7.3.2.1 & 7.4.8.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing a <contentInstance> resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BI/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE request of a <latest> resource as a direct child of a <container> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.22, TS-0004 7.4.28.2.4 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing a <contentInstance> resource containing attribute creationTime set to the latest time } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no name attribute and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BI/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE request of a <oldest> resource as a direct child of a <container> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.23, TS-0004 7.4.29.2.4 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing <contentInstance> resource(s) } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no name attribute and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a RESOURCE_TYPE resource |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/BV/001_01 | TS-0001 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/BV/001_02 | TS-0001 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/BV/001_03 | TS-0001 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/BV/001_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/BV/001_05 | TS-0001 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/BV/001_06 | TS-0001 10.2.11.5 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS without having privileges for the DELETE operation |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE not having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/BV/002_01 | TS-0001 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_02 | TS-0001 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_03 | TS-0001 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_05 | TS-0001 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_06 | TS-0001 10.2.11.5 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when AE tries to delete a child resource of a RESOURCE_TYPE resource when this resource has been already deleted |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS and the AE having deleted the TARGET_RESOURCE_ADDRESS resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) }} |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/BV/002_01 | TS-0001 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_02 | TS-0001 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_03 | TS-0001 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_04 |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_05 | TS-0001 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/BV/002_06 | TS-0001 10.2.11.5 | 23 (subscription) |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the stateTag attribute of a <Container> resource is increased when a child resource is deleted |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <Container> resource containing a child resource and the AE having privileges to perform DELETE operation on the resource and the AE having deleted the TARGET_CHILD_RESOURCE_ADDRESS resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid RETRIEVE response containing PrimitiveContent containing RESOURCE_TYPE resource containing stateTag attribute set to 2 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT decreases the field value of attribute currentNrOfInstances and currentByteSize of direct parent <container> respectively when a latest <contentInstance> resource is deleted successfully |  |
| Reference |  | TS-0001 10.2.19.5, TS-0004 7.4.8.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing a latest <contentInstance> resource TARGET_RESOURCE_ADDRESS containing currentNrOfInstances set to NUMBER_OF_INSTANCES and currentByteSize set to BYTE_SIZE_OF_DATA } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and Content attribute containing currentNrOfInstances set to value (NUMBER_OF_INSTANCES – 1) and currentByteSize set to value (BYTE_SIZE_OF_DATA minus DELETED_DATA_BYTE_SIZE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the DELETE request of the latest <contentInstance> resource target to a <container> resource |  |
| Reference |  | TS-0001 10.2.22.2, TS-0004 7.4.28.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing a latest <contentInstance> resource TARGET_RESOURCE_ADDRESS containing currentNrOfInstances set to NUMBER_OF_INSTANCES and currentByteSize set to BYTE_SIZE_OF_DATA } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and Contribute attribute containing currentNrOfInstances set to value (NUMBER_OF_INSTANCES – 1) and currentByteSize set to value (BYTE_SIZE_OF_DATA minus DELETED_DATA_BYTE_SIZE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BO/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the DELETE request of a <latest> resource in a <container> resource that has no direct child <contentInstance> resources. |  |
| Reference |  | TS-0001 10.2.22.2, TS-0004 7.4.28.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing no <contentInstance> resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BV/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the DELETE request of the oldest <contentInstance> resource target to a <container> resource |  |
| Reference |  | TS-0001 10.2.23.2, TS-0004 7.4.29.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing a latest <contentInstance> resource TARGET_RESOURCE_ADDRESS containing currentNrOfInstances set to NUMBER_OF_INSTANCES and currentByteSize set to BYTE_SIZE_OF_DATA } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a DELETE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and Contribute attribute containing currentNrOfInstances set to value (NUMBER_OF_INSTANCES – 1) and currentByteSize set to value (BYTE_SIZE_OF_DATA minus DELETED_DATA_BYTE_SIZE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/BO/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the DELETE request of a <oldest> resource in a <container> resource that has no direct child <contentInstance> resources. |  |
| Reference |  | TS-0001 10.2.23.2, TS-0004 7.4.29.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a <container> resource containing no <contentInstance> resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

## Subscription and Notification (SUB)

| TP Id |  | TP/oneM2M/CSE/SUB/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the target subscribed-to RESOURCE_TYPE resource is not subscribable. |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT having a resource PARENT_RESOURCE_ADDRESS not being subscribable RESOURCE_TYPE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource-Type set to <subscription> and From set to AE-ID and Content <subscription> resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5203 (TARGET_NOT_SUBSCRIBABLE) and no Content attribute } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/SUB/BV/001_01 | TS-0001 9.6.7 | 4 (ContentInstance) |
| TP/oneM2M/CSE/SUB/BV/001_02 | TS-0001 9.6.8 | 23 (Subscription) |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the originator does not have privileges for retrieving the subscribed-to resource. |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS containing accessControlPolicyIDs pointing to accessControlPolicy resource containing Privileges containing accessControlRule containing accessControlOriginators indicating AE-ID and accessControlOperations not indicating RETRIEVE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource-Type set to <subscription> and From set to AE-ID and Content <subscription> resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) and no Content attribute } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT cannot send the Notify request to the notificationURI |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT not being able to send the Notify request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource-Type set to <subscription> and From set to AE-ID and Content <subscription> resource containing NotificationURI set to NOTI_URI_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5204 (SUBSCRIPTION_VERIFICATION_INITIATION_FAILED) and no Content attribute } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the Notify request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives the Notify response containing Response Status Code set to 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) and no Content attribute } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the Notify request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives the Notify response containing Response Status Code set to 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) and no Content attribute } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends Notify request to the subscriber resource when an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 10.2.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventType attribute is set to "Create_of_Direct_Child_Resource" and an create operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing eventType condition tag set to 3 (Create_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing CHILD_RESOURCE_TYPE resource containing ResourceName attribute set to NAME } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing CHILD_RESOURCE_TYPE resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventType attribute is set to "Delete_of_Resource" and an delete operation has been performed the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing eventType condition tag set to 2 (Delete_of_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventType attribute is set to "Delete_of_Direct_Child_Resource" and an delete operation has been performed on a child resource of the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing a CHILD_RESOURCE_ADDRESS child resource and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing eventType condition tag set to 4 (Delete_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE1 containing To set to CHILD_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing CHILD_RESOURCE_TYPE resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT stores Originator ID in the notification creator attribute when a <SUBSCRIPTION> creation request which needs verification is received and the notificationURI is not the Originator. |  |
| Reference |  | TS-0001 10.2.12.1 & TS-0004 7.4.9.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Resource-Type set to <SUBSCRIPTION> and PrimitiveContent <SUBSCRIPTION> resource containing NotificationURI set to AE2_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a Notify request containing PrimitiveContent attribute containing Notification message containing verificationRequest set to TRUE and creator set to AE1_RESOURCE_ADDRESS } |  | IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to “modified attributes” and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 2 (modified attributes) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to “ResourceID” and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 3 (ResourceID) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing subscribed-to ResourceID } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT decreases the expirationCounter attribute of a <Subscription> resource when the Hosting CSE of the subscribed-to resource successfully sends the notification request to subscriber resource(s) |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to MAX_NUMBER_NOTIFICATION and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request and the IUT decrease by one the expirationCounter attribute of the <Subscription> resource } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes the <Subscription> resource when the the expirationCounter meets zero |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to 1 and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify request and the IUT decrease by one the expirationCounter attribute of the <Subscription> resource and the IUT deletes the <Subscription> resource } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the “attribute” condition tag of eventNotificationCriteria attribute is set to ATTRIBUTE_NAME and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT doesn’t send a Notify request to the subscriber resource when the “attribute” condition tag of eventNotificationCriteria attribute is set to LIST_OF_ATTRIBUTE and doesn’t contain the ATTRIBUTE_NAME attribute updated. |  |
| Reference |  | TS-0001 9.6.8 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to LIST_OF_ATTRIBUTE containing no ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and the IUT doesn’t send a Notification message } |  | IUT  AE1 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the AE2_RESOURCE_ADDRESS of the subscriberURI attribute when the SUBSCRIPTION_RESOURCE_ADDRESS <subscription> resource is deleted |  |
| Reference |  | TS-0001 9.6.8 & TS-0004 7.5.1.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing <Subscription> resource containing subscriberURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE1 containing To set to SUBSCRIPTION_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing Notification message containing subscriptionDeletion set to TRUE and subscriptionReference set to AE2_RESOURCE_ADDRESS } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends aggregated notification to the subscriber resource when the number value of the batchNotify attribute is set to GROUP_LIMIT and when this number have been reached |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing number set to GROUP_LIMIT and duration set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (GROUP_LIMIT – 1) valid UPDATE requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE requests from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_N } |  | IUT  AE1 |
|  | then { the IUT sends a Response messages containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing AggregatedNotification message containing GROUP_LIMIT Notification messages } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends aggregated notification to the subscriber resource when the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing number set to GROUP_LIMIT and duration set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received NUMBER_OF_REQUEST valid UPDATE requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and NUMBER_OF_REQUEST < GROUP_LIMIT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the timer expires after the TIME_LIMIT duration value } |  | IUT  AE1 |
|  | then { The IUT sends a Notify Request contaning PrimitiveContent attribute containing AggregatedNotification message containing NUMBER_OF_REQUEST Notification messages } |  | IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the number value of the batchNotify attribute is set to GROUP_LIMIT and when this number have been reached |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing number set to GROUP_LIMIT and duration set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (GROUP_LIMIT – 1) valid UPDATE requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE requests from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and PrimitiveContent attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_N } |  | IUT  AE1 |
|  | then { the IUT sends a Response messages containing Response Status Code set to 2004 (UPDATED) and the IUT sends a Notify Request contaning PrimitiveContent attribute containing the latest Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

| TP Id |  | TP/oneM2M/CSE/SUB/BV/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and <Subscription> child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing number set to GROUP_LIMIT and duration set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received NUMBER_OF_REQUEST valid UPDATE requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and NUMBER_OF_REQUEST < GROUP_LIMIT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the timer expires after the TIME_LIMIT duration value } |  | IUT  AE1 |
|  | then { The IUT sends a Notify Request contaning PrimitiveContent attribute containing the latest Notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE2 |

## Group Management (GMG)

| TP Id |  | TP/oneM2M/CSE/GMG/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <group> resource when member ID exceed max number of members. |  |
| Reference |  | TS-0001 10.2.7.2 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content <group> resource containing memberIDs set to A_LIST_OF_MEMBER_RESOURCE_IDS and maxNrOfMembers set to AN_INTEGER_VALUE less than the numbers of memerIDs set in memberIDs attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <group> resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference |  | TS-0001 10.2.7.2 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation the sub-group members being in the “inital state” and the sub-group members having RESOURCE_TYPE containing accessControlPolicyIDs attribute pointing to accessControlPolicy resource containing Privileges attribute containing accessControlRule containing accessControlOriginators not indicating AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content <group> resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5105 (NO_PRIVILEGE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update of the <group> resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference |  | TS-0001 10.2.7.4 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform UPDATE operation the sub-group members being in the “inital state” and the sub-group members having RESOURCE_TYPE containing accessControlPolicyIDs attribute pointing to accessControlPolicy resource containing Privileges attribute containing accessControlRule containing accessControlOriginators not indicating AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content <group> resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 5105 (NO_PRIVILEGE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update of the <group> resource when member ID exceed max number of members. |  |
| Reference |  | TS-0001 10.2.7.4 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform UPDATE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content <group> resource containing memberIDs set to A_LIST_OF_MEMBER_RESOURCE_IDS and maxNrOfMembers set to AN_INTEGER_VALUE less than the numbers of memerIDs set in memberIDs attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of the RESOURCE_TYPE resource by using <fanOutPoint> in <group> resource. |  |
| Reference |  | TS-0001 10.2.7.7 & TS-0004 7.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a <group> resource containing the memberIDs set to List of member resource IDs and the IUT having a <group> resource allowing the AE to perform CREATE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to FANOUTPOINT_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content RESOURCE_TYPE } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing aggregatedResponse representation } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the resource of the group members by using <fanOutPoint> in <group> resource. |  |
| Reference |  | TS-0001 10.2.7.8 & TS-0004 7.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a <group> resource containing the memberIDs set to List of member resource IDs and the IUT having a <group> resource allowing the AE to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to FANOUTPOINT_ADDRESS and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing aggregatedResponse representation } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of the RESOURCE_TYPE resource by using <fanOutPoint> in <group> resource. |  |
| Reference |  | TS-0001 10.2.7.9 & TS-0004 7.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a <group> resource containing the memberIDs set to List of member resource IDs linked to group members with RESOURCE_TYPE and the IUT having a <group> resource allowing the AE to perform UPDATE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to FANOUTPOINT_ADDRESS and From set to AE-ID and Content RESOURCE_TYPE } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (CHANGED) and Content attribute containing aggregatedResponse representation } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT could delete all the member resources by using <fanOutPoint> in <group> resource. |  |
| Reference |  | TS-0001 10.2.7.10 & TS-0004 7.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions |  | with { the IUT being in the "initial state" and the IUT having a <group> resource containing the memberIDs set to List of member resource IDs and the IUT having a <group> resource allowing the AE to perform DELETE operation } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to FANOUTPOINT_ADDRESS and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) and Content attribute containing aggregatedResponse representation } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects the presence of duplicate member IDs during the creation of the <group> resource and removes the duplicate member IDs prior to creation of the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to <group> and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS, MEMBER_RESOURCE_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2001 (CREATED) Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS } |  | IUT  AE |

TARGET_RESOURCE_ADDRESS : resource type can be <CSEBase>, <AE>, <remoteCSE>

MEMBER_RESOURCE_ADDRESS: Any resource type

| TP Id |  | TP/oneM2M/CSE/GMG/BV/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT validates the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to <group> and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS memberType attribute set to RESOURCE_TYPE_1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2001 (CREATED) Content set to <group> resource containing memberTypeValidated attribute set to TRUE } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is SET_MIXED. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to <group> and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS memberType attribute set to RESOURCE_TYPE_2 consistencyStrategy attribute set to SET_MIXED } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2001 (CREATED) Content set to <group> resource containing memberTypeValidated attribute set to FALSE memberType attribute set to MIXED } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_MEMBER,. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to <group> and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 memberType attribute set to RESOURCE_TYPE_1 consistencyStrategy attribute set to ABANDON_MEMBER } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2001 (CREATED) Content set to <group> resource containing memberTypeValidated attribute set to TRUE memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_GROUP. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to <group> and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 memberType attribute set to RESOURCE_TYPE_1 consistencyStrategy attribute set to ABANDON_GROUP } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 6011(MEMBER_TYPE_INCONSISTENT) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects the presence of duplicate member IDs during an update of the <group> resource and removes the duplicate member IDs prior to updating the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2004 (UPDATED) Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |

TARGET_RESOURCE_ADDRESS : resource type can be <CSEBase>, <AE>, <remoteCSE>

MEMBER_RESOURCE_ADDRESS: Any resource type

| TP Id |  | TP/oneM2M/CSE/GMG/BV/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT validates the resource type during an UPDATE of the <group> resource when memberType attribute is not ‘mixed’. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2004 (UPDATED) Content set to <group> resource containing memberTypeValidated attribute set to TRUE } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is SET_MIXED. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 memberTypeValidated attribute set to TRUE consistencyStrategy attribute set to SET_MIXED and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2004 (UPDATED) Content set to <group> resource containing memberTypeValidated attribute set to FALSE memberType attribute set to MIXED } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_MEMBER. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 memberTypeValidated attribute set to TRUE consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT sends a valid Response message containing Response Status Code set 2004 (UPDATED) Content set to <group> resource containing memberTypeValidated attribute set to TRUE memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_GROUP. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 memberTypeValidated attribute set to TRUE consistencyStrategy attribute set to # (ABANDON_GROUP) and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2002 (DELETED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects when the number of memberIDs exceeds the limitation of maxNrOfMembers. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 maxNrOfMembers attribute set to 2 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_3 having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_3 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects when the value provided for maxNrOfMembers attribute is less than the value of the currentNrOfMembers attribute. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_3 maxNrOfMembers attribute set to 3 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_3 having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing maxNrOfMembers attribute set to 2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles validation of the resource type during an UPDATE of the <group> resource when memberIDs contains a sub-group on a temporarily unreachable Hosting CSE. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 memberTypeValidated attribute set to TRUE consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 of type RESOURCE_TYPE_1 and the remoteCSE having a resource at MEMBER_RESOURCE_ADDRESS_2 of type <group> containing memberType attribute set to RESOURCE_TYPE_1 and the remoteCSE having onlineStatus set to False } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <group> resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2004 (UPDATED) Content set to <group> resource containing memberTypeValidated attribute set to FALSE memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
| TP Id |  | TP/oneM2M/CSE/GMG/BV/022 |  |
| Test objective |  | Check that the IUT handles validation of the resource type during an UPDATE of the <group> resource when memberIDs contains a sub-group on a a previously unreachable Hosting CSE that has become reachable |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS of type <group> containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 memberType attribute set to RESOURCE_TYPE_1 memberTypeValidated attribute set to FALSE consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the AE having a subscription to TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 of type RESOURCE_TYPE_1 and the remoteCSE having a resource at MEMBER_RESOURCE_ADDRESS_2 of type <group> containing memberType attribute set to RESOURCE_TYPE_2 and the remoteCSE having onlineStatus set to False } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from remoteCSE containing To set to REMOTE_CSE_ADDRESS and From set to CSE-ID and Content set to <remoteCSE> resource containing onlineStatus attribute set to TRUE } |  | IUT  AE |
|  | then { the IUT sends a Notification message containing Content set to <group> resource containing memberTypeValidated attribute set to TRUE memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Allow a <group>/fanoutPoint OPERATION when the Originator has OPERATION_PERMISSION specified in membersAccessControlPolicyIDs in the group resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to allow the AE privileges to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION request from AE containing To set to TARGET_RESOURCE_ADDRESS/fanOutPoint and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set RESPONSE_STATUS_CODE Content set to aggregated_response containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/BV/023_1 | CREATE |
| TP/oneM2M/CSE/GMG/BV/023_2 | RETRIEVE |
| TP/oneM2M/CSE/GMG/BV/023_3 | UPDATE |
| TP/oneM2M/CSE/GMG/BV/023_4 | DELETE |

| TP Id |  | TP/oneM2M/CSE/GMG/BO/024 |  |
| --- | --- | --- | --- |
| Test objective |  | Deny a <group>/fanOutPoint OPERATION when the Originator does not have OPERATION_PERMISSION specified in membersAccessControlPolicyIDs in the group resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to allow the AE privileges to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION request from AE containing To set to TARGET_RESOURCE_ADDRESS/fanOutPoint and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to ORIGINATOR_HAS_NO_PRIVILEGE } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/BV/024_1 | CREATE |
| TP/oneM2M/CSE/GMG/BV/024_2 | RETRIEVE |
| TP/oneM2M/CSE/GMG/BV/024_3 | UPDATE |
| TP/oneM2M/CSE/GMG/BV/024_4 | DELETE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/025 |  |
| --- | --- | --- | --- |
| Test objective |  | Allow a <group>/fanoutPoint OPERATION when the Originator has OPERATION_PERMISSION specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to empty containing accessControlPolicyIDs set to allow the AE privileges to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION request from AE containing To set to TARGET_RESOURCE_ADDRESS/<group>/fanOutPoint and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set RESPONSE_STATUS_CODE Content set to aggregated_response containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

| TP Id | OPERATION | Response Status Code |
| --- | --- | --- |
| TP/oneM2M/CSE/GMG/BV/025_1 | CREATE | CREATED |
| TP/oneM2M/CSE/GMG/BV/025_2 | RETRIEVE | OK |
| TP/oneM2M/CSE/GMG/BV/025_3 | UPDATE | UPDATED |
| TP/oneM2M/CSE/GMG/BV/025_4 | DELETE | DELETED |

| TP Id |  | TP/oneM2M/CSE/GMG/BO/026 |  |
| --- | --- | --- | --- |
| Test objective |  | Deny a <group>/fanOutPoint OPERATION when the Originator does not have OPERATION_PERMISSION specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to empty containing accessControlPolicyIDs set to allow the AE privileges to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION request from AE containing To set to TARGET_RESOURCE_ADDRESS/fanOutPoint and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to ORIGINATOR_HAS_NO_PRIVILEGE } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/BV/026_1 | CREATE |
| TP/oneM2M/CSE/GMG/BV/026_2 | RETRIEVE |
| TP/oneM2M/CSE/GMG/BV/026_3 | UPDATE |
| TP/oneM2M/CSE/GMG/BV/026_4 | DELETE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/027 |  |
| --- | --- | --- | --- |
| Test objective |  | Generate a request primitive for each resource in memberIDs with no relative address appended to it. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to allow the AE privileges to perform CREATE containing memberType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType set to AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS/fanOutPoint and From set to AE-ID Content set to <container> resource containing resourceName attribute set to “myContainer” resourceType set to <container> } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2000 (OK) Content set to aggregated_response containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/028 |  |
| --- | --- | --- | --- |
| Test objective |  | Generate a request primitive for each resource in memberIDs with a relative address appended to it. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to allow the AE privileges to perform CREATE containing memberType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1/myCon containing resourceType set to container and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2/myCon containing resourceType set to container } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS/fanOutPoint/myCon and From set to AE-ID Content set to <contentInstance> resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2000 (OK) Content set to aggregated_response containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/GMG/BV/029 |  |
| --- | --- | --- | --- |
| Test objective |  | Generate a request primitive for each resource in memberIDs with a relative address appended to fanoOutPoint that includes a virtual resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs set to allow the AE privileges to perform RETRIEVE containing memberType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1/myCon containing resourceType set to container and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2/myCon containing resourceType set to container } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS/fanOutPoint/myCon/latest and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set 2000 (OK) Content set to aggregated_response containing Response for MEMBER_RESOURCE_ADDRESS1/myCon/latest and Response for MEMBER_RESOURCE_ADDRESS2/myCon/latest } |  | IUT  AE |

## Discovery (DIS)

Editor Note: all the uppercased italic strings denotes type of primitives defined in the oneM2M specification, for example, FILTER_CRITERIA_TYPE denotes the category of filter criteria, while the uppercased normal strings, e.g. ATTRIBUTE_VALUE denotes the user-input value.

| TP Id |  | TP/oneM2M/CSE/DIS/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list all discovered resource addresses |  |
| Reference |  | TS-0001 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS allowing the AE to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and FilterUsage set to 1 (Discovery Criteria) and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing URIList representation } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DIS/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully appropriate list of discovered resource when the filter criteria is provided in the request |  |
| Reference |  | TS-0001 8.1.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS allowing the AE to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request with specific filter criteria from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and FilterUsage set to 1 (Discovery Criteria) and FilterCriteria set to FILTER_CRITERIA_CONDITIONS and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing URIList representation } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DIS/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns the empty address list when no result matching with filter criteria is discovered |  |
| Reference |  | TS-0001 8.1.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS allowing the AE to perform RETRIEVE operation and FILTER_CRITERIA_CONDITIONS not being applicable to the IUT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request with specific filter criteria from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and FilterUsage set to 1 (Discovery Criteria) and FilterCriteria set to FILTER_CRITERIA_CONDITIONSand no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and no Content attribute } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DIS/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list of discovered resource addresses with Non-hierarchical addressing form when the Discovery Result Type is provided in the request |  |
| Reference |  | TS-0001 8.1.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS allowing the AE to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request with specific filter criteria from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and FilterUsage set to 1 (Discovery Criteria) and DiscoveryResultType set to 2 (unstructured) and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing URIList representation containing unstructured addresses of discovered resources } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DIS/BO/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the discovery requests to the resource TARGET_RESOURCE_ADDRESS when AE has no privilege to perform the discovery request for the resource TARGET_RESOURCE_ADDRESS |  |
| Reference |  | TS-0001 10.2.6.2 & 7.2.3.14, TS-0004 7.3.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE not having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and FilterUsage set to 1 (Discovery Criteria) and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

Editor Note: Inopportune Behavior (BO) test cases test against IUT in cases of after receipt of a syntactically correct request primitives but not expected in the actual message exchange or state.

| TP Id |  | TP/oneM2M/CSE/DIS/BO/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE sends requests to discover the resource TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference |  | TS-0001 10.2.6.2, TS-0004 7.3.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and FilterUsage set to 1 (Discovery Criteria) and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DIS/BI/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds the originator with an error when the originator sends a request including an invalid format of filter criteria to discover the resource TARGET_RESOURCE_ADDRESS |  |
| Reference |  | TS-0001 10.2.6, TS-0004 7.3.3.6 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions |  | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a resource TARGET_RESOURCE_ADDRESS allowing the AE to perform RETRIEVE operation } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and FilterUsage set to 1 (Discovery Criteria) and FilterCriteria set to FILTER_CRITERIA_CONDITIONS not being valid format and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/DIS/BI/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds the originator with an error when the originator sends a request to discover the resource TARGET_RESOURCE_ADDRESS including two conflicted filter criteria of different type FILTER_CRITERIA_CONDITION_1 and FILTER_CRITERIA_CONDITION_2 (E.G. createBefore < createdAfter) |  |
| Reference |  | TS-0001 6.2.5, TS-0004 7.3.3.17 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a resource TARGET_RESOURCE_ADDRESS allowing the AE to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and FilterUsage set to 1 (Discovery Criteria) and FilterCriteria set to FILTER_CRITERIA_CONDITIONS having conflict conditions and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE) } |  | IUT  AE |

| TP ID | REQ Reference | FILTER_CRITERIA_ CONDITION_1 | FILTER_CRITERIA_ CONDITION_2 |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DIS/BI/008 -1 | REQ-0004-07241 | createdBefore | createdAfter |
| TP/oneM2M/CSE/DIS/BI/008 -2 | REQ-0004-07242 | lastModifiedBefore | lastModifiedAfter |
| TP/oneM2M/CSE/DIS/BI/008-3 | REQ-0004-07243 | stateTagSmaller | stateTagBigger |
| TP/oneM2M/CSE/DIS/BI/008 -4 | REQ-0004-07244 | expireBefore | expireAfter |

## Location (LOC)

Editor Note: all  the uppercased strings using underscore to connect string, e.g. NON_ZERO_VALUE, TARGET_RESOURCE_ADDRESS denote the user-input value.

| TP Id |  | TP/oneM2M/CSE/LOC/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <locationPolicy> resource when locationSource attribute is set by Network Based and Location Server is not reachable. |  |
| Reference |  | TS-0001 8.5. & 10.2.10 & TS-0004 7.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having no connection to Location Server and the IUT having a CSEBase resource allowing the AE to perform CREATE operation and } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to CSEBase resource and From set to AE-ID and Content <locationPolicy> resource } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of locationPolicy resource with locationSource attribute and locationUpdatePeriod attribute provided. |  |
| Reference |  | TS-0001 10.2.10.1, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource-Type set to <locationPolicy> and From set to AE_ID and Content attribute containing LocationSource set to 1 (Network_based) and LocationUpdatePeriod set to NON_ZERO_VALUE and LocationTargetID set to LOCATION_TARGET_ID and LocationServer set to LOCATION_SERVER_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BO/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of <locationPolicy> resource when AE has no privilege to perform the creation request for the <locationPolicy> resource |  |
| Reference |  | TS-0001 10.2.10.1, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the AE not having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource-Type set to <locationPolicy> and From set to AE_ID and Content attribute containing LocationSource set to 1 (Network_based) and LocationUpdatePeriod set to NON_ZERO_VALUE and LocationTargetID set to LOCATION_TARGET_ID and LocationServer set to LOCATION_SERVER_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BO/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when receiving a creation request of <locationPolicy> resource target to the TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference |  | TS-0001 10.2.10.1, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource-Type set to <locationPolicy> and From set to AE_ID and Content attribute containing LocationSource set to 1 (Network_based) and LocationUpdatePeriod set to NON_ZERO_VALUE and LocationTargetID set to LOCATION_TARGET_ID and LocationServer set to LOCATION_SERVER_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BI/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects AE to update the <locationPolicy> resource identified with LOCATION_ID when receiving an update request containing an invalid attribute |  |
| Reference |  | TS-0001 10.2.10.1.3, TS-0004 7.3.3.4 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type <locationPolicy> containing an optional RW attribute LocationUpdatePeriod and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a UPDATE request with from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Contents attribute containing LocationUpdatePeriod set to INVALID_LOCATION_UPDATE_PERIOD } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with a <container> resource containing attribute locationID set to resourceID of the associated <locationPolicy> resource |  |
| Reference |  | TS-0001 10.2.10.1.1, TS-0004 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationContainerID set to TARGET_RESOURCE_ADDRESS and locationSource set to 1 (Network_based) and locationTargetID set to LOCATION_TARGET_ID and locationServer set to LOCATION_SERVER_ADDRESS and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type <container> containing locationID set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing <container> representation containing locationID set to LOCATIONPOLICY_ADDRESS } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts the creation of a <locationPolicy> resource with attribute locationSource set to Device-based and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having location positioning modules and the originator being a PX_ASN_AE and the IUT being a PX_ASN_CSE and the IUT having registered the PX_ASN_AE and the ASN-AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and ResourceType set to <locationPolicy> and From set to AE_ID and Content attribute containing locationSource set to 2 (Device_based) and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD } |  | IUT  ASN-AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing <locationPolicy> attributes representation } |  | IUT  ASN-AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <locationPolicy> resource with attribute locationSource set to Sharing-based |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the originator being a PX_ADN_AE and the IUT being a PX_MN_CSE and the ADN-AE being no location positioning modules deployed and the ADN-AE being no Network-based positioning capabilities and the IUT having registered the PX_ADN_AE and the ADN-AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and ResourceType set to <locationPolicy> and From set to AE_ID and Content attribute containing locationSource set to 3 (Sharing_based) } |  | IUT  ADN-AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing <locationPolicy> attributes representation } |  | IUT  ADN-AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notification message including the location Information to the originator when the originator sends a retrieve request target to the <latest> resource of CONTAINER_ADDRESS <container> associated with a <locationPolicy> resource |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1 & 10.2.10.2.2, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationContainerID set to CONTAINER_ADDRESS and locationUpdatePeriod set to ZERO (or NULL) and locationSource set to LOCATION_SOURCE and the IUT having created a resource CONTAINER_ADDRESS of type <container> containing locationID set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource CONTAINER_ADDRESS and the AE having created a resource of type <subscription> as a child of the resource CONTAINER_ADDRESS of type <container> } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to CONTAINER_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing notification representation containing notificationEvent attribute containing representation attribute containing representation of <contentInstance> including location information } |  | IUT  AE |

| TP ID | Reference | LOCATION_SOURCE |
| --- | --- | --- |
| TP/oneM2M/CSE/LOC/BV/006_01 | TS-0001 9.6.10 & 10.2.10 TS-0004 7.4.11 & 6.3.4.2.3 | 1 (Network_based) |
| TP/oneM2M/CSE/LOC/BV/006_02 |  | 2 (Device_based) |
| TP/oneM2M/CSE/LOC/BV/006_03 |  | 3 (Sharing_based) |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notification message including the location Information to the originator with frequency of POSITIVE_UPDATE_PERIOD set in field of locationUpdatePeriod |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1 & 10.2.10.2.2, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationContainerID set to CONTAINER_ADDRESS and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD and locationSource set to LOCATION_SOURCE and the IUT having created a resource CONTAINER_ADDRESS of type <container> containing locationID set to LOCATIONPOLICY_ADDRESS and the AE having created a resource of type <subscription> as a child of the resource CONTAINER_ADDRESS of type <container> and the IUT having privileges to retrieve the location information from the NSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a location response from NSE containing location-protocol-specific-result-code of positioning procedure execution and location-response containing location information } |  | IUT  NSE |
|  | then { the IUT transforms the location-response into a oneM2M Response primitive and sends the Response primitive containing Response Status Code set to 2000 (OK) and Content attribute containing notification representation containing notificationEvent attribute containing representation attribute containing representation of <contentInstance> including location information } |  | IUT  AE |
| Note that: the attribute location-protocol-specific-result-code and location-response are location acquisition protocol specific and the exact name of these two attributes have to follow the specification of corresponding location protocol e.g. OMA Mobile Location Protocol. |  |  |  |

Table Referenced values for parameter LOCATION_SOURCE

| TP ID | Reference | LOCATION_SOURCE |
| --- | --- | --- |
| TP/oneM2M/CSE/LOC/BV/007_01 | TS-0001 9.6.10 & 10.2.10 TS-0004 7.4.11 & 6.3.4.2.3 | 1 (Network_based) |
| TP/oneM2M/CSE/LOC/BV/007_02 |  | 2 (Device_based) |
| TP/oneM2M/CSE/LOC/BV/007_03 |  | 3 (Sharing_based) |

Table Referenced values for parameter location-protocol-specific-result-code and location-response

| location-protocol-specific-result-code | location-response | Location protocol | Reference |
| --- | --- | --- | --- |
| [Result code] | [Content] | OMA Mobile Location Protocol | Not Present |
| [HTTP Response code] | <TerminalLocation> representation containing attributes address indicating the address of targeted node where location information is retrieved, locationRetrievalStatus indicating the status of retrieval of the location information for the target node , and currentLocation containing the actual location data of the target node | OMA RESTful NetAPI | TS-0004 Annex G |
| [HTTP Response code] | <SubscriptionNotification> representation | OMA RESTful NetAPI | Location Server Open RESTful API Specification |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT creates a <container> resource with CONTAINER_NAME included in the associated <locationPolicy> resource that has been created resulted in a valid <locationPolicy> create request. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.2.1, TS-0004 7.3.3.3 & 7.4.11.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having no created container having resourceName set to CONTAINER_NAME and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationContainerName set to CONTAINER_NAME and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD and locationSource set to 1 (Network_based) and locationTargetID set to LOCATION_TARGET_ID and locationServer set to LOCATION_SERVER_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and ResourceType set to <container> and From set to AE_ID and Content attribute containing resourceName set to CONTAINER_NAME locationID set to LOCATIONPOLICY_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing <container> resource representation containing resourceName set to CONTAINER_NAME locationID set to resourceID of the associated <locationPolicy> resource } |  | IUT  AE |
| Note that the IUT sets the locationContainerID attribute in <locationPolicy> resource to the resourceID of the created <container> resource |  |  |  |
| TP Id |  | TP/oneM2M/CSE/LOC/BV/009 |  |
| Test objective |  | Check that the IUT creates a <subscription> resource with SUBSCRIPTION_NAME provided by the originator as a child resource of the <container> linked with a <locationPolicy> resource. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.2.1, TS-0004 7.3.3.3 & 7.4.11.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing resourceID set to NOTI_URI_ADDRESS and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD and locationSource set to 1 (Network_based) and locationTargetID set to LOCATION_TARGET_ID and locationServer set to LOCATION_SERVER_ADDRESS and locationContainerID set to CONTAINER_ADDRESS and and the IUT having created a resource CONTAINER_ADDRESS of type <container> containing locationID set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and ResourceType set to <subscription> and From set to AE_ID and Content attribute containing notificationURI set to NOTI_URI_ADDRESS and eventNotificationCriteria containing notificationEventType set to 3 (Create_of_Direct_Child_Resource) and notificationContentType set to 1 (All Attributes) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) and Content attribute containing <subscription> resource representation } |  | IUT  AE |
| Note that the IUT sets the locationContainerID attribute in <locationPolicy> resource to the resourceID of the created <container> resource |  |  |  |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with a <locationPolicy> resource containing attribute locationContainerID set to resourceID of the associated <container> resource |  |
| Reference |  | TS-0001 10.2.10.1.1, TS-0004 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type <locationPolicy> containing locationContainerID set to CONTAINER_ADDRESS and locationSource set to 1 (Network_based) and locationTargetID set to LOCATION_TARGET_ID and locationServer set to LOCATION_SERVER_ADDRESS and the IUT having created a resource CONTAINER_ADDRESS of type <container> containing locationID set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attributes representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing <locationPolicy> representation containing locationContainerID set to CONTAINER_ADDRESS } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sets the locationStatus attribute with the error status code in the <locationPolicy> resource when the location information of the requested node is failed to be retrieved due to NSE. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type <locationPolicy> and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing locationPolicy representation containing locationStatus set to LOCATION_PROTOCOL_SPECIFIC_RESULT_CODE } |  | IUT  AE |
| Note: the error status code LOCATION_PROTOCOL_SPECIFIC_RESULT_CODE is location protocol specific, e.g. in OMA Mobile Location Protocol the reslt code is set to 6 indicating POSITION METHOD FAILURE. Note that the failure is caused by the NSE such as location server. |  |  |  |

| TP Id |  | TP/oneM2M/CSE/LOC/BV/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notification message including the location Information to the originator with frequency of POSITIVE_UPDATE_PERIOD set in field of locationUpdatePeriod |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the originator being a ADN-AE and the IUT being a MN-CSE and the ADN-AE being no location positioning modules deployed and the ADN-AE being no Network-based positioning capabilities and the IUT having registered the ADN-AE and the IUT having created under the ADN-AE a child resource ORIGINATOR_LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationContainerID set to ORIGINATOR_CONTAINER_ADDRESS and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD and locationSource set to 3 (Sharing-based) and the IUT having created under the ADN-AE a child resource ORIGINATOR_CONTAINER_ADDRESS of type <container> containing locationID set to ORIGINATOR_LOCATIONPOLICY_ADDRESS and the IUT having created a resource of type <subscription> as a child resource of ORIGINATOR_CONTAINER_ADDRESS of type <container> and the IUT having registered a Node being a minimum hop with the originator compared to other nodes registered with the hosting CSE and the IUT having created under the Node a child resource CLOSEST_NODE_CONTAINER_ADDRESS of type <container> containing a latest <contentInstance> resource containing the latest location information and the IUT having privileges to perform RETRIEVE operation on the resource CLOSEST_NODE_CONTAINER_ADDRESS of type of <container> } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a <latest> resource of <container> from the CLOSEST_NODE_CONTAINER_ADDRESS containing latest location information and the IUT creates a <contentInstance> as a child resource of ORIGINATOR_CONTAINER_ADDRESS of type <container> containing the location information that retrieved } |  | IUT  AE |
|  | then { the IUT sends a Response primitive containing Response Status Code set to 2000 (OK) and Content attribute containing notification representation containing notificationEvent attribute containing representation attribute containing representation of <contentInstance> including location information } |  | IUT  ADN-AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BI/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <locationPolicy> resource when locationSource attribute is set to Network-Based and neither locationTargetID nor locationServer is provided. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a CREATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to <locationPolicy> and From set to AE_ID and Content attribute containing LocationSource set to 1 (Network_based) } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/LOC/BO/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error message “NOT_FOUND” when receiving a retrieve request target to an non-existing <locationPolicy> resource |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.2, TS-0004 7.3.3.2 & 7.4.11.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not created under the AE a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> and the IUT having privileges to perform RETRIEVE operation on <locationPolicy> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE request from AE containing To set to LOCATIONPOLICY_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

## Device Management (DMG)

Editor note:  Test Purposes for DMG will be updated here.

## Communication Management and Delivery Handling(CMDH)

Editor note:  Test Purposes for CMDH will be updated here.

## Security(SEC)

Editor node: Test Purposes for SEC will be updated here.

## Resource Announcement (ANNC)

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | AE or CSE initiates announcement of a resource to an announcement target CSE-ID when the resource’s parent resource is announced. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a RESPONSE message containing Response Status Code set to 2004 (UPDATED) announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Original resource hosting CSE initiates announcement of a resource to an announcement target CSE-ID when the resource’s parent resource is announced and the resource has an accessControlPolicyIDs attribute. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a CREATE request containing To set to remoteCSE1/ ANNOUNCED_PARENT_URI Content set to < RESOURCE_TYPE ANNC> resource containing Attributes marked with MA set to same value as original resource resourceType set to RESOURCE_TYPE ANNC expirationTime set to same value as original resource link set to TARGET_RESOURCE_ADDRESS (in SP-relative Resource-ID format or Absolute Resource-ID format. accessControlPolicyIDs set to same value as original resource labels set to same value as original resource } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Host of <resourceAnnc> returns the <resourceAnnc> representation in a RETRIEVE response |  |
| Reference |  | TS-0001 10.2.18.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE_ANNC> at TARGET_RESOURCE_ADDRESS containing link attribute set to remoteCSE1/ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the TARGET_RESOURCE_ADDRESS and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing attributes } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Host of <resourceAnnc> returns the original <resource> representation from the original resource hosting CSE in a RETRIEVE response when result content is set to “original-resource” |  |
| Reference |  | TS-0001 10.2.18.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE_ANNC> at TARGET_RESOURCE_ADDRESS containing link attribute set to remoteCSE1/ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the TARGET_RESOURCE_ADDRESS and the remoteCSE1 having onlineStatus set to True and the remoteCSE1 having <RESOURCE_TYPE> at ORIGINAL_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content attribute containing attributes from <RESOURCE_TYPE> at remoteCSE1/ORIGINAL_RESOURCE_ADDRESS } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Original resource hosting CSE initiates de-announcement of a resource at remoteCSE2 when remoteCSE2 is removed from the announceTO attribute. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having announced to the remoteCSE2 and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI1, remoteCSE2/ANNOUNCED_RESOURCE_URI2 announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET _RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a DELETE message containing To set to remoteCSE2/ANNOUNCED_RESOURCE_URI2 } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Host CSE returns announceTO attribute showing the list of remaining announcement target CSEs. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having announced to the remoteCSE2 and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI1, remoteCSE2/ANNOUNCED_RESOURCE_URI2 announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to TARGET _RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and Content attribute containing RESOURCE_TYPE resource containing announceTo attribute set to remoteCSE1/ ANNOUNCED_RESOURCE_URI1 } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Original resource hosting CSE initiates announcement of a resource to an announcement target CSE-ID when the announced resource does not have an accessControlPolicyIDs attribute. |  |
| Reference |  | TS-0001 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1 } |  | IUT  AE |
|  | then { the IUT sends a REQUEST message containing To set to remoteCSE1/ ANNOUNCED_PARENT_URI Content set to < RESOURCE_TYPE ANNC> resource containing Attributes marked with MA set to same value as original resource resourceType set to RESOURCE_TYPE ANNC expirationTime set to same value as original resource link set to TARGET_RESOURCE_ADDRESS (in SP-relative Resource-ID format or Absolute Resource-ID format. accessControlPolicyIDs set to same value as original resource’s parent accessControlPolicyIDs labels set to same value as original resource } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BO/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Host CSE of original resource removes an announcement target CSE from the announceTo attribute if it does not return successful CREATE response for the announced resource. |  |
| Reference |  | 10.2.18.4 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announceTo attribute set to remoteCSE1,remoteCSE2 } |  | IUT  CSE |
|  | then { the IUT sends a RESPONSE message containing Response Status Code set to 2004 (UPDATED) announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BV/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Original resource hosting CSE initiates de-announcement of a resource at remoteCSE2 when remoteCSE2 is removed from the announceTO attribute. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having announced to the remoteCSE2 and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI1, remoteCSE2/ANNOUNCED_RESOURCE_URI2 announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the remoteCSE1 having onlineStatus set to True and the IUT having received a valid UPDATE request message containing To set to TARGET_RESOURCE_ADDRESS and Content attribute containing RESOURCE_TYPE resource containing announceTo attribute set to remoteCSE2/ANNOUNCED_RESOURCE_URI2 and the IUT having sent a DELETE request message to remoteCSE1/ ANNOUNCED_RESOURCE_URI1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE response from remoteCSE1 containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) and Content attribute containing RESOURCE_TYPE resource containing announceTo attribute set to remoteCSE2/ANNOUNCED_RESOURCE_URI2 } |  | IUT  AE |

| TP Id |  | TP/oneM2M/CSE/ANNC/BI/010 |  |
| --- | --- | --- | --- |
| Test objective |  | HOST CSE of original resource returns INVALID response if announcedAttributes contains an attribute that is not marked OA. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an invalid UPDATE request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content set to <RESOURCE_TYPE> resource containing announcedAttributes attribute set to pi } |  | IUT  AE |
|  | then { the IUT sends a RESPONSE message containing Response Status Code set to 2004 (UPDATED) announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI } |  | IUT  AE |

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A> Conformance Test Requirement

A.1	MQTT Protocol Conformance Test Requirement

| Test Items |  | TS Chapter | Obligation | Condition for Optional Item | Checking point | Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| Commonly Repeated Items |  | TS-0010, Sec5.1 | Mandatory |  | "QoS=1" | Fail when QoS is other than “1” |
|  |  | TS-0010, Sec5.1 | Mandatory |  | "Retain" flag set as "0" | Fail if DUT sends with “Retain” flag=1 |
| Connection Items | Connect | TS-0010, Sec6.3 | Mandatory |  | Prefix A:: or C:: is added to the ID | Fail if Prefix A:: or C:: are not added to the ID |
|  |  | TS-0010, Sec6.3 | Mandatory |  | if Clean session (ex: Retain) flag is set to “False” in CONNECT | Fail when clean session is “True” when DUT want to be connected |
|  |  | TS-0010, Sec6.3 | Mandatory |  | if “Will Flag” is not set in CONNECT | Fail when will flag is enabled |
|  | Disconnect | TS-0010, Sec6.3 | Mandatory |  | if Clean session (ex: Retain) flag is set to “True” in CONNECT | Fail when clean session is “False” when DUT want to be disconnected |
| Subscription Items |  | TS-0010, Sec5.1 | Mandatory |  | if the message is set to Durable | Fail is the message is not Durable |
|  |  | TS-0010, Sec6.4.4 | Conditional | (for Initial Registration) when Originator doesn’t knows it’s ID | if DUT’s Initial ID assigning /oneM2M/reg_req/<originator>/<receiver> oneM2M/reg_resp/<originator>/<receiver> | Fail if DUT's ID assigning is other than this |
| Publish Item |  | TS-0010, Figure 6.4.5-2 | Conditional | when Originator knows it’s ID | if DUT’s Initial ID assigning oneM2M/req/<Originator>/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional | When Originator uses wild card | if DUT’s Initial ID assigning oneM2M/req/+/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | MQTT v3.1.1 | Conditional |  | if the has proper Primitive contents (“op”, “fr”, “to”, “ri”, “pc”, “ty”) | Fail if any of primitive contents feature wrong |
| Reception Item |  | TS-0010, Figure 6.4.5-2 | Conditional | when Originator knows it’s ID | if DUT’s Initial ID assigning oneM2M/resp/<Originator>/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional | When Originator uses wild card | if DUT’s Initial ID assigning oneM2M/resp/<Originator>/+ | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional |  | server Delete received contents properly not storing data | Fail if Receiver stores message |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional |  | If the receiver sends PubACK when receives messages | Fail if the receiver doesn’t send PubACK |
|  |  | MQTT v3.1.1 | Mandatory |  | if the has proper Primitive contents (“op”, “fr”, “to”, “ri”, “pc”, “ty”) | Fail if any of primitive contents feature wrong |

<PAGE BREAK>

Annex <B>(Informative): TP template

| TP Id |  |  |  |
| --- | --- | --- | --- |
| Test objective |  |  |  |
| Reference |  |  |  |
| Config Id |  |  |  |
| PICS Selection |  |  |  |
| Initial conditions |  |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { } |  | IUT  AE |
|  | then { } |  | IUT  AE |

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 22 Apil 2015 | Initial Draft Version (based on TST-2015-0071R01) |
| V.0.0.2 | 15 July 2015 | Add the agreed document from TST 17.3 (TST-2015-0115R01-Test_Purposes_contributions) |
| V.0.0.3 | 25 August 2015 | Add the agreed document from TST 18 (TST-2015-0138R03 TSS List and Test Purposes contributions) (TST-2015-0152R01 MQTT binding Requirement) |
| V.0.0.4 | 26 October 2015 | Add the agreed document from TST 19 (TST-2015-0184R02 TS-0018-TSS contribution) (TST-2015-0185R02 TS-0018-TP Definition conventions contribution) (TST-2015-0200 TS-0018-TP template) |
| V.0.0.5 | 14 January 2016 | Add the agreed document from TST 20 (TST-2015-0217R02-TS-0018-GMG_Test_Purposes_contribution) (TST-2015-0221R01-DIS_LOC_Test_Purpose_Contributions) (TST-2015-0222R01-SUB_Test_Purposes) (TST-2015-0234R01-TS-0018_DMR_Test_Purposes_with_Result_Content_param) |
| V.0.0.6 | 18 January 2016 | Add the agreed document from TST 19.2 (TST-2015-0212-TS-0018-Test_Purposes_contribution) |
| V.0.0.7 | 09 March 2016 | Add the agreed document from TST 21.2 (TST-2016-0034R01 TS-0018-Test Purposes contribution) (TST-2016-0035R01 Test_Purpose_GMG_CREATE) |
| V.0.0.8 | 02 May 2016 | Add the agreed document from TST 22 (TST-2016-0056-Test_Purpose_GMG_CREATE_crs_accepted) (TST-2016-0047R01-TS-0018-Test_Purposes_contribution) (TST-2016-0058-Test_Purposes_for_DIS) (TST-2016-0059-Test_Purposes_for_LOC) |
| V.0.1.0 | 16 May 2016 | Add the agreed document from TST 22.3 (TST-2016-0081-TS-0018-Test_purpose_contribution) |
| V.0.1.1 | 18 July 2016 | Add the agreed documents from TST 22.2 (TST-2016-0066R02-Test_Purpose_GMG_UPDATE) (TST-2016-0067R02-Test_Purpose_GMG_UPDATE_REMOTE) |
| V0.1.2 | 22 September | Add the agreed documents from TST 24 (TST-2016-0124R01-CR_TS-0018-Update_LOC_testcase) |
| V0.1.3 | 10 October | Add the agreed documents from TST 24.x (TST-2016-0134R02-TestPurposes_fanoutPoint_basic) (TST-2016-0144R01-CR_TS-0018-Update_GMG_Test_Purposes) |
| V0.1.4 | 1st November | Add the agreed documents from TST 25 (TST-2016-0154R01-LOC_Test_Purposes) (TST-2016-0166R03-announce_deannounce_test_purpose) (TST-2016-0173R01-DMR_Container_and_Latest_ContentInstance_Test_Purposes) (TST-2016-0177R01-TS-0018 Test purpose contribution) (TST-2016-0178-TS-0018 Test purpose contribution) |