| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0018-V-0.5.0 |
| Document Name: | Test Suite Structure and Test Purposes |
| Date: | 2017-September-11 |
| Abstract: | The Test Suite Structure and Test Purposes document for conformance testing consists of: - Defining the test suite structure by grouping the test purposes according to different criteria; - Specifing test purposes for conformance test. A test purpose is an informal description of the expected test behaviour. |
| Template Version:23 February 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2016, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	10

2	References	10

2.1	Normative references	10

2.2	Informative references	10

3	Definitions, symbols and abbreviations	10

3.1 	Definitions	10

3.2	Abbreviations	11

4	Conventions	11

5	Prerequisites and Test Configurations	11

5.1	Test Configurations	11

6 	Test Suite Structure (TSS)	14

6.1	Structure for oneM2M tests	14

6.2	Test groups	14

6.2.1 	Functional Entity Groups	14

6.2.2 	Common Services Functions Subgroups	14

6.2.3 	oneM2M Resource Primitives specific Operations	14

6.3	Test Suite Structure (TSS) for oneM2M	14

7	Test Purposes (TP)	16

7.1	Introduction	16

7.1.1	TP definition conventions	16

7.1.2	TP Identifier naming conventions	16

7.1.3	Rules for the behaviour description	16

7.1.4	Source of TP definitions	17

7.1.5	ICS reference	17

7.2    Test Purposes for oneM2M Service Primitives	18

7.2.1.1	General Capability (GEN)	18

CREATE Operation	18

TP/oneM2M/AE/GEN/CRE/001	18

TP/oneM2M/AE/GEN/CRE/002	19

UPDATE Operation	20

TP/oneM2M/AE/GEN/UPD/001	20

TP/oneM2M/AE/GEN/UPD/002	21

RETRIEVE Operation	22

TP/oneM2M/AE/GEN/RET/001	22

TP/oneM2M/AE/GEN/RET/002	22

DELETE Operation	24

TP/oneM2M/AE/GEN/DEL/001	24

TP/oneM2M/AE/GEN/DEL/002	24

7.2.1.2	Registration (REG)	25

CREATE Operation	25

TP/oneM2M/AE/REG/CRE/001	25

TP/oneM2M/AE/REG/CRE/002	25

DELETE Operation	26

TP/oneM2M/AE/REG/DEL/001	26

7.2.1.3	Data Management and Repository (DMR)	27

CREATE Operation	27

TP/oneM2M/AE/DMR/CRE/001	27

TP/oneM2M/AE/DMR/CRE/002	27

TP/oneM2M/AE/DMR/CRE/003	28

TP/oneM2M/AE/DMR/CRE/004	29

UPDATE Operation	30

TP/oneM2M/AE/DMR/UPD/001	30

TP/oneM2M/AE/DMR/UPD/002	31

TP/oneM2M/AE/DMR/UPD/003	31

RETRIEVE Operation	32

TP/oneM2M/AE/DMR/RET/001	32

TP/oneM2M/AE/DMR/RET/002	33

TP/oneM2M/AE/DMR/RET/003	33

TP/oneM2M/AE/DMR/RET/004	34

TP/oneM2M/AE/DMR/RET/005	34

DELETE Operation	35

TP/oneM2M/AE/DMR/DEL/001	35

TP/oneM2M/AE/DMR/DEL/002	35

TP/oneM2M/AE/DMR/DEL/003	36

TP/oneM2M/AE/DMR/DEL/004	36

7.2.1.4	Subscription and Notification (SUB)	37

CREATE Operation	37

TP/oneM2M/AE/SUB/CRE/001	37

TP/oneM2M/AE/SUB/CRE/002	37

NOTIFY Operation	38

TP/oneM2M/AE/SUB/NTF/001	38

TP/oneM2M/AE/SUB/NTF/002	38

7.2.1.5	Group Management (GMG)	39

7.2.1.6	Location (LOC)	39

7.2.1.7	Device Management (DMG)	39

7.2.1.8	Communication Management and Delivery Handling (CMDH)	39

7.2.1.9	Security (SEC)	39

7.2.2.1	General Capability (GEN)	40

CREATE Operation	40

TP/oneM2M/CSE/GEN/CRE/001	40

TP/oneM2M/CSE/GEN/CRE/002	41

UPDATE Operation	42

TP/oneM2M/CSE/GEN/UPD/001	42

TP/oneM2M/CSE/GEN/UPD/002	43

RETRIEVE Operation	44

TP/oneM2M/CSE/GEN/RET/001	44

TP/oneM2M/CSE/GEN/RET/002	45

DELETE Operation	46

TP/oneM2M/CSE/GEN/DEL/001	46

TP/oneM2M/CSE/GEN/DEL/002	46

7.2.2.2	Registration (REG)	47

RETRIEVE Operation	47

TP/oneM2M/CSE/REG/RET/001	47

TP/oneM2M/CSE/REG/RET/002	47

TP/oneM2M/CSE/REG/RET/003	48

TP/oneM2M/CSE/REG/RET/004	48

TP/oneM2M/CSE/REG/RET/005	49

TP/oneM2M/CSE/REG/RET/006	49

TP/oneM2M/CSE/REG/RET/007	50

TP/oneM2M/CSE/REG/RET/008	51

TP/oneM2M/CSE/REG/RET/009	51

TP/oneM2M/CSE/REG/RET/010	52

CREATE Operation	52

TP/oneM2M/CSE/REG/CRE/001	52

TP/oneM2M/CSE/REG/CRE/002	53

TP/oneM2M/CSE/REG/CRE/003	53

TP/oneM2M/CSE/REG/CRE/004	54

TP/oneM2M/CSE/REG/CRE/005	55

TP/oneM2M/CSE/REG/CRE/006	56

TP/oneM2M/CSE/REG/CRE/007	56

TP/oneM2M/CSE/REG/CRE/008	57

TP/oneM2M/CSE/REG/CRE/009	58

TP/oneM2M/CSE/REG/CRE/010	59

TP/oneM2M/CSE/REG/CRE/011	60

TP/oneM2M/CSE/REG/CRE/013	61

TP/oneM2M/CSE/REG/CRE/014	61

TP/oneM2M/CSE/REG/CRE/015	62

TP/oneM2M/CSE/REG/CRE/016	62

TP/oneM2M/CSE/REG/CRE/017	63

TP/oneM2M/CSE/REG/CRE/018	63

TP/oneM2M/CSE/REG/CRE/019	64

TP/oneM2M/CSE/REG/CRE/020	65

TP/oneM2M/CSE/REG/CRE/021	65

TP/oneM2M/CSE/REG/CRE/022	66

TP/oneM2M/CSE/REG/CRE/023	66

TP/oneM2M/CSE/REG/CRE/024	67

TP/oneM2M/CSE/REG/CRE/025	67

TP/oneM2M/CSE/REG/CRE/027	69

TP/oneM2M/CSE/REG/CRE/028	70

TP/oneM2M/CSE/REG/CRE/029	70

TP/oneM2M/CSE/REG/CRE/030	71

TP/oneM2M/CSE/REG/CRE/031	71

TP/oneM2M/CSE/REG/CRE/032	73

TP/oneM2M/CSE/REG/CRE/033	73

TP/oneM2M/CSE/REG/CRE/034	73

DELETE Operation	74

TP/oneM2M/CSE/REG/DEL/001	74

TP/oneM2M/CSE/REG/DEL/002	75

TP/oneM2M/CSE/REG/DEL/003	75

TP/oneM2M/CSE/REG/DEL/004	76

TP/oneM2M/CSE/REG/DEL/005	76

UPDATE Operation	77

TP/oneM2M/CSE/REG/UPD/001	77

TP/oneM2M/CSE/REG/UPD/002	77

TP/oneM2M/CSE/REG/UPD/003	78

TP/oneM2M/CSE/REG/UPD/004	79

7.2.2.3	Data Management and Repository Function (DMR)	80

RETRIEVE Operation	80

TP/oneM2M/CSE/DMR/RET/001	80

TP/oneM2M/CSE/DMR/RET/002	81

TP/oneM2M/CSE/DMR/RET/003	81

TP/oneM2M/CSE/DMR/RET/004	82

TP/oneM2M/CSE/DMR/RET/005	83

TP/oneM2M/CSE/DMR/RET/006	84

TP/oneM2M/CSE/DMR/RET/007	85

TP/oneM2M/CSE/DMR/RET/008	86

TP/oneM2M/CSE/DMR/RET/009	87

TP/oneM2M/CSE/DMR/RET/010	87

TP/oneM2M/CSE/DMR/RET/011	88

TP/oneM2M/CSE/DMR/RET/012	88

TP/oneM2M/CSE/DMR/RET/013	89

TP/oneM2M/CSE/DMR/RET/014	90

TP/oneM2M/CSE/DMR/RET/015	91

TP/oneM2M/CSE/DMR/RET/016	92

TP/oneM2M/CSE/DMR/RET/017	93

TP/oneM2M/CSE/DMR/RET/018	94

TP/oneM2M/CSE/DMR/RET/019	95

TP/oneM2M/CSE/DMR/RET/020	96

TP/oneM2M/CSE/DMR/RET/021	97

TP/oneM2M/CSE/DMR/RET/022	98

TP/oneM2M/CSE/DMR/RET/023	99

TP/oneM2M/CSE/DMR/RET/024	100

TP/oneM2M/CSE/DMR/RET/025	100

UPDATE Operation	102

TP/oneM2M/CSE/DMR/UPD/001	102

TP/oneM2M/CSE/DMR/UPD/002	103

TP/oneM2M/CSE/DMR/UPD/003	104

TP/oneM2M/CSE/DMR/UPD/004	105

TP/oneM2M/CSE/DMR/UPD/005	107

TP/oneM2M/CSE/DMR/UPD/014	107

TP/oneM2M/CSE/DMR/UPD/006	108

TP/oneM2M/CSE/DMR/UPD/007	109

TP/oneM2M/CSE/DMR/UPD/008	110

TP/oneM2M/CSE/DMR/UPD/009	111

TP/oneM2M/CSE/DMR/UPD/010	112

TP/oneM2M/CSE/DMR/UPD/011	112

TP/oneM2M/CSE/DMR/UPD/012	113

TP/oneM2M/CSE/DMR/UPD/013	113

TP/oneM2M/CSE/DMR/UPD/014	114

TP/oneM2M/CSE/DMR/UPD/015	115

TP/oneM2M/CSE/DMR/UPD/016	116

TP/oneM2M/CSE/DMR/UPD/017	117

TP/oneM2M/CSE/DMR/UPD/018	118

CREATE Operation	119

TP/oneM2M/CSE/DMR/CRE/001	119

TP/oneM2M/CSE/DMR/CRE/002	120

TP/oneM2M/CSE/DMR/CRE/003	121

TP/oneM2M/CSE/DMR/CRE/004	122

TP/oneM2M/CSE/DMR/CRE/005	123

TP/oneM2M/CSE/DMR/CRE/006	123

TP/oneM2M/CSE/DMR/CRE/007	125

TP/oneM2M/CSE/DMR/CRE/008	125

TP/oneM2M/CSE/DMR/CRE/009	126

TP/oneM2M/CSE/DMR/CRE/010	126

TP/oneM2M/CSE/DMR/CRE/011	127

TP/oneM2M/CSE/DMR/CRE/012	127

TP/oneM2M/CSE/DMR/CRE/013	128

TP/oneM2M/CSE/DMR/CRE/014	129

TP/oneM2M/CSE/DMR/CRE/015	130

TP/oneM2M/CSE/DMR/CRE/016	131

TP/oneM2M/CSE/DMR/CRE/017	132

TP/oneM2M/CSE/DMR/CRE/018	133

TP/oneM2M/CSE/DMR/CRE/019	133

TP/oneM2M/CSE/DMR/CRE/020	134

TP/oneM2M/CSE/DMR/CRE/021	134

DELETE Operation	136

TP/oneM2M/CSE/DMR/DEL/001	136

TP/oneM2M/CSE/DMR/DEL/002	137

TP/oneM2M/CSE/DMR/DEL/003	137

TP/oneM2M/CSE/DMR/DEL/004	138

TP/oneM2M/CSE/DMR/DEL/005	139

TP/oneM2M/CSE/DMR/DEL/006	139

TP/oneM2M/CSE/DMR/DEL/007	140

TP/oneM2M/CSE/DMR/DEL/008	140

TP/oneM2M/CSE/DMR/DEL/009	141

TP/oneM2M/CSE/DMR/DEL/010	141

7.2.2.4	Subscription and Notification (SUB)	142

CREATE Operation	142

TP/oneM2M/CSE/SUB/CRE/001	142

TP/oneM2M/CSE/SUB/CRE/002	143

TP/oneM2M/CSE/SUB/CRE/003	144

TP/oneM2M/CSE/SUB/CRE/004	144

TP/oneM2M/CSE/SUB/CRE/005	145

TP/oneM2M/CSE/SUB/CRE/006	145

DELETE Operation	147

TP/oneM2M/CSE/SUB/DEL/001	147

TP/oneM2M/CSE/SUB/DEL/002	148

TP/oneM2M/CSE/SUB/DEL/003	149

UPDATE Operation	150

TP/oneM2M/CSE/SUB/UPD/001	150

TP/oneM2M/CSE/SUB/UPD/002	151

TP/oneM2M/CSE/SUB/UPD/003	152

TP/oneM2M/CSE/SUB/UPD/004	153

TP/oneM2M/CSE/SUB/UPD/005	154

TP/oneM2M/CSE/GMG/UPD/003	154

TP/oneM2M/CSE/SUB/UPD/006	155

TP/oneM2M/CSE/SUB/UPD/007	156

TP/oneM2M/CSE/SUB/UPD/008	157

TP/oneM2M/CSE/SUB/UPD/009	158

NOTIFY Operation	159

TP/oneM2M/CSE/SUB/NTF/001	159

TP/oneM2M/CSE/SUB/NTF/002	160

TP/oneM2M/CSE/SUB/NTF/003	160

TP/oneM2M/CSE/SUB/NTF/004	161

TP/oneM2M/CSE/SUB/NTF/005	161

TP/oneM2M/CSE/SUB/NTF/006	162

7.2.2.5	Group Management (GMG)	163

CREATE Operation	163

TP/oneM2M/CSE/GMG/CRE/001	163

TP/oneM2M/CSE/GMG/CRE/002	164

TP/oneM2M/CSE/GMG/CRE/003	165

TP/oneM2M/CSE/GMG/CRE/004	166

TP/oneM2M/CSE/GMG/CRE/006	168

TP/oneM2M/CSE/GMG/CRE/007	169

TP/oneM2M/CSE/GMG/CRE/008	170

TP/oneM2M/CSE/GMG/CRE/009	171

UPDATE Operation	172

TP/oneM2M/CSE/GMG/UPD/001	172

TP/oneM2M/CSE/GMG/UPD/002	173

TP/oneM2M/CSE/GMG/UPD/003	174

TP/oneM2M/CSE/GMG/UPD/004	175

TP/oneM2M/CSE/GMG/UPD/005	176

TP/oneM2M/CSE/GMG/UPD/006	177

TP/oneM2M/CSE/GMG/UPD/007	178

TP/oneM2M/CSE/GMG/UPD/008	179

TP/oneM2M/CSE/GMG/UPD/009	180

TP/oneM2M/CSE/GMG/UPD/010	181

RETRIEVE Operation	182

TP/oneM2M/CSE/GMG/RET/001	182

BASIC OPERATION	183

TP/oneM2M/CSE/GMG/001	183

TP/oneM2M/CSE/GMG/002	184

TP/oneM2M/CSE/GMG/003	185

TP/oneM2M/CSE/GMG/004	186

7.2.2.6	Discovery (DIS)	187

RETRIEVE Operation	187

TP/oneM2M/CSE/DIS/001	187

TP/oneM2M/CSE/DIS/002	187

TP/oneM2M/CSE/DIS/003	188

TP/oneM2M/CSE/DIS/004	188

TP/oneM2M/CSE/DIS/005	189

TP/oneM2M/CSE/DIS/006	189

TP/oneM2M/CSE/DIS/007	190

TP/oneM2M/CSE/DIS/008	191

TP/oneM2M/CSE/DIS/009	192

7.2.2.7	Location (LOC)	193

CREATE Operation	193

TP/oneM2M/CSE/LOC/CRE/001	193

TP/oneM2M/CSE/LOC/CRE/002	193

TP/oneM2M/CSE/LOC/CRE/003	194

TP/oneM2M/CSE/LOC/CRE/004	194

TP/oneM2M/CSE/LOC/CRE/005	195

TP/oneM2M/CSE/LOC/CRE/006	196

TP/oneM2M/CSE/LOC/CRE/007	197

TP/oneM2M/CSE/LOC/CRE/008	198

TP/oneM2M/CSE/LOC/CRE/009	198

UPDATE Operation	199

TP/oneM2M/CSE/LOC/UPD/001	199

RETRIEVE Operation	200

TP/oneM2M/CSE/LOC/RET/001	200

TP/oneM2M/CSE/LOC/RET/002	200

TP/oneM2M/CSE/LOC/RET/003	202

TP/oneM2M/CSE/LOC/RET/004	203

TP/oneM2M/CSE/LOC/RET/005	204

NOTIFY Operation	205

TP/oneM2M/CSE/LOC/NTF/001	205

TP/oneM2M/CSE/LOC/NTF/002	206

7.2.2.8	Device Management (DMG)	207

7.2.2.9	Communication Management and Delivery Handling(CMDH)	207

7.2.2.10	Security(SEC)	207

CREATE Operation	207

TP/oneM2M/CSE/SEC/ACP/CRE/001	207

TP/oneM2M/CSE/SEC/ACP/CRE/002	208

TP/oneM2M/CSE/SEC/ACP/CRE/003	208

TP/oneM2M/CSE/SEC/ACP/CRE/004	209

UPDATE Operation	209

TP/oneM2M/CSE/SEC/ACP/UPD/001	209

TP/oneM2M/CSE/SEC/ACP/UPD/002	210

BASIC OPERATION	210

TP/oneM2M/CSE/SEC/ACP/001	210

TP/oneM2M/CSE/SEC/ACP/002	211

TP/oneM2M/CSE/SEC/ACP/003	211

TP/oneM2M/CSE/SEC/ACP/004	212

TP/oneM2M/CSE/SEC/ACP/005	212

TP/oneM2M/CSE/SEC/ACP/006	213

TP/oneM2M/CSE/SEC/ACP/007	213

TP/oneM2M/CSE/SEC/ACP/008	214

TP/oneM2M/CSE/SEC/ACP/009	215

TP/oneM2M/CSE/SEC/ACP/010	215

TP/oneM2M/CSE/SEC/ACP/011	216

TP/oneM2M/CSE/SEC/ACP/012	216

7.2.2.11	Resource Announcement (ANNC)	217

BASIC Operation	217

TP/oneM2M/CSE/ANNC/001	217

CREATE Operation	218

TP/oneM2M/CSE/ANNC/CRE/001	218

TP/oneM2M/CSE/ANNC/CRE/002	219

TP/oneM2M/CSE/ANNC/CRE/003	220

TP/oneM2M/CSE/ANNC/CRE/004	221

TP/oneM2M/CSE/ANNC/CRE/005	222

TP/oneM2M/CSE/ANNC/CRE/006	223

TP/oneM2M/CSE/ANNC/CRE/007	224

TP/oneM2M/CSE/ANNC/CRE/008	225

TP/oneM2M/CSE/ANNC/CRE/009	226

RETRIEVE Operation	227

TP/oneM2M/CSE/ANNC/RET/001	227

TP/oneM2M/CSE/ANNC/RET/002	228

UPDATE Operation	229

TP/oneM2M/CSE/ANNC/UPD/001	229

TP/oneM2M/CSE/ANNC/UPD/002	230

TP/oneM2M/CSE/ANNC/UPD/003	231

TP/oneM2M/CSE/ANNC/UPD/004	232

TP/oneM2M/CSE/ANNC/UPD/005	233

TP/oneM2M/CSE/ANNC/UPD/006	233

TP/oneM2M/CSE/ANNC/UPD/007	234

TP/oneM2M/CSE/ANNC/UPD/008	235

TP/oneM2M/CSE/ANNC/UPD/009	236

TP/oneM2M/CSE/ANNC/UPD/010	237

TP/oneM2M/CSE/ANNC/UPD/011	238

TP/oneM2M/CSE/ANNC/UPD/012	239

TP/oneM2M/CSE/ANNC/UPD/013	240

TP/oneM2M/CSE/ANNC/UPD/014	241

DELETE Operation	242

TP/oneM2M/CSE/ANNC/DEL/001	242

TP/oneM2M/CSE/ANNC/DEL/002	243

Proforma copyright release text block	244

Annexes	245

History	247

# 1	Scope

The present document specifies oneM2M test suite structure and test purposes that are designed to evaluate  the comformity of oneM2M implementations to the oneM2M specifications. It also specifies

- Guidelines for description of  test behaviours and definition of test purpose structure,

- Notations to define a test purpose for conformance testing, and

- Test configurations for conformance testing.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0001: "Functional Architecture".

[2]				 	oneM2M TS-0004: "Service Layer Core Protocol Specification".

[3]	oneM2M TS-0015: "Testing Framework".

[4]	oneM2M TS-0017: "Implementation Conformance Statements".

[5]	oneM2M TS-0031: "oneM2M Features".

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definitions, symbols and abbreviations

## 3.1 	Definitions

For the purposes of the present document, the terms and definitions given in TS-0015 Testing Framework and the following ones apply:

valid <OPERATION> Request: a oneM2M Request primitive that only contains all mandatory primitive parameters with correct values which are not required to be explicitly specified in the test purpose. If specific values need to be specified, parameters need to be explicitly indicated in the test purpose. Operation parameter is set to <OPERATION>.

valid Response: a oneM2M Response primitive that only contains all mandatory primitive parameters with correct values which are not required to be explicitly specified in the test purpose. If specific values need to be specified, parameters need to be explicitly indicated in the test purpose. Request Identifier parameter contains the Request Identifier value of the corresponding Request.

valid <ATTRIBUTE_NAME>: a correct value for the <ATTRIBUTE_NAME> attribute which is not required to be explicitly specified in the test purpose.

Editor’s Note:	This may contain additional information.

## 3.2	Abbreviations

For the purposes of the present document, abbreviations given in TS-0001[1] apply:

ADN 			Application Dedicated Node

ADN-AE 		AE which resides in the Application Dedicated Node

AE 				Application Entity

ASN 				Application Service Node

CSE 				Common Service Entity

CSF 				Common Service Function

IN 				Infrastructure Node

IN-CSE 			CSE which resides in the Infrastructure Node

IUT 				Implementation Under Test

MN 				Middle Node

SUT				System Under Test

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Prerequisites and Test Configurations

## 5.1	Test Configurations

Test configurations are defined to test different entities such as CSE and AE etc.

Figure 1 and Figure 2 show CSE test configurations where CSE as a IUT can be tested against TTCN-3 Test System, which is acting as AE or AE and CSE respectively. Test system may include as well other entities which are necessary for each configuration, i.e. a second AE (AE2) for receiving notifications These entities are represented with dashed rectangles.

Figure 3 shows AE test configuration (CF03) for ADN-AE testing scenario where ADN-AE is IUT and TTCN-3 Test System is acting as a CSE.

Figure 4 depicts test configurations between two CSEs, where one CSE is acting as a Test System, the other is SUT.

# 6 	Test Suite Structure (TSS)

## 6.1	Structure for oneM2M tests

## 6.2	Test groups

The test groups are organized in 2 levels. The first level defines the functional entity of oneM2M. The second level defines the common services functions which are represented in oneM2M functional architecture.

### 6.2.1 	Functional Entity Groups

The functional entity groups identify the types of entity which are specified in oneM2M functional architecture(TS-0001).

- AE

- CSE

### 6.2.2 	Common Services Functions Subgroups

The common services functions subgroups identify the general procedures in each common services functions specified in oneM2M functional architecture(TS-0001).

- Registration

- Data Management and Repository

- Subscription and Notification

- Group Management

- Discovery

- Location

- Device Management

- Communication Management and Delivery Handling

- Security

### 6.2.3 	oneM2M Resource Primitives specific Operations

A group of resource type specific procedures are defined in oneM2M functional architecture (TS-0001) and service layer core protocol (TS-0004). Those procedures can be potentially abstracted with requirements for generating test purposes for conformance testing purpose. Those resource type specific procedures are the complementary procedures to the general procedures defined in oneM2M Common Service Functions (CSF) and will use for subgroups of structures for oneM2M tests. A list of those procedures are including:

- Resource announcement

- Editor’s Note: more procedures will be added for future tests extension

## 6.3	Test Suite Structure (TSS) for oneM2M

The Test Suite Structure (TSS) for oneM2M will be present as bellow:

- Group 1: General Capability (GEN)

  - Subgroup 1.1:	Protocol Bindings

  - Subgroup 1.2: 	Serializations

- Group 2: AE(AE)

  - Subgroup 2.1: 	General Capability (GEN)

  - Subgroup 2.2: 	Registration(REG)

  - Subgroup 2.3: 	Data Management and Repository(DMR)

  - Subgroup 2.4: 	Subscription and Notification(SUB)

  - Subgroup 2.5: 	Group Management(GMG)

  - Subgroup 2.6: 	Discovery(DIS)

  - Subgroup 2.7: 	Location(LOC)

  - Subgroup 2.8: 	Device Management(DMG)

  - Subgroup 2.9:   Communication Management and Delivery Handling(CMDH)

  - Subgroup 2.10: Security(SEC)

- Group 3: CSE(CSE)

  - Subgroup 3.1  	General Capability (GEN)

  - Subgroup 3.2: 	Registration(REG)

  - Subgroup 3.3: 	Data Management and Repository(DMR)

  - Subgroup 3.4: 	Subscription and Notification(SUB)

  - Subgroup 3.5: 	Group Management(GMG)

  - Subgroup 3.6:	Discovery(DIS)

  - Subgroup 3.7: 	Location(LOC)

  - Subgroup 3.8: 	Device Management(DMG)

  - Subgroup 3.9: 	Communication Management and Delivery Handling(CMDH)

  - Subgroup 3.10: Security(SEC)

  - Subgroup 3.11: Resource Announcement (ANNC)

# 7	Test Purposes (TP)

## 7.1	Introduction

### 7.1.1	TP definition conventions

The TP definition is constructed according to Test Suite Structure & Test Purposes(TSS&TP) in Testing Framework(TS-0015).

### 7.1.2	TP Identifier naming conventions

The identifier of the TP is constructed according to table 7.1.2.-1.

Table 7.1.2.-1: TP naming convention for oneM2M

| Name | Option | Interpretation |
| --- | --- | --- |
| TP/<root>/<ent>/<gr>[/<sgr>][/<opr>]/<nnn>_<pmu> |  |  |
| <root> = root | oneM2M | oneM2M |
| <ent> = entity | AE | Application Entity |
|  | CSE | Common Services Entity |
| <gr> = group | GEN | Common Service Function |
|  | REG |  |
|  | DMR |  |
|  | SUB |  |
|  | GMG |  |
|  | DIS |  |
|  | LOC |  |
|  | DMG |  |
|  | CMDH |  |
|  | SEC |  |
|  | ANNC |  |
| <sgr> = sub-group | ACP | AccessControlPolicy |
| <opr> = operation | CRE | Create |
|  | UPD | Update |
|  | RET | Retrieve |
|  | DEL | Delete |
|  | NTF | Notify |
| <nnn> = sequential number | 001 to 999 | 023 |
| <pmu> = permutation part | May be number, short name of attribute or resource type depend on the permutated part in the TP. If there are more than one permutated variables, they should be separated by slash ‘/’. | Enumerated type attribute and its value is represented as <VARIABLE_1>/<VALUE>, e.g. TP/oneM2M/CSE/DMR/RET/020_RCN/0 When resourceType attribute is involved in a TP Id, the resourceType shall be put in first order followed by any other attribute(s), if any, <RESOURCE_TYPE>/<VARIABLE_1>/<VARIABLE_2> e.g. TP/oneM2M/CSE/DMR/CRE/015_CNT/ACPI |

### 7.1.3	Rules for the behaviour description

The description of the TP is built according to TS-0015 Testing Framework.

In addition, the following rules apply:

- Primitive parameters shall use their Parameter name as indicated in TS-0004 [2] table 8.2.2-1

- Resource attributes shall use their Attribute name as indicated in TS-0004 [2] tables 8.2.3 followed by “attribute”.

- When Content request parameter contains a resource respresentation, such representation shall contain all mandatory attributes and could contain some optional attributes. Only those attributes and/or child resources required by the test purpose will be explicitly indicated.

- When Content response parameter contains a resource respresentation, such representation shall contain attributes and/or child resources according to the Result Content parameter of the corresponding request. Only those attributes and/or child resources required by the test purpose will be explicitly indicated.

### 7.1.4	Source of TP definitions

### 7.1.5	ICS reference

Editor note: the table 7.1.5.-1 and 7.1.5.-2 are for examples and are here for being referenced in the Table 7.1.5.-3: mnemonics for PICS reference. Table 7.1.5.-1 and 7.1.5.-2 should be included in the ICS TS-0017.

Table 7.1.5.-1: Node

| Item | Name of field | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | ASN (Application Service Node) | 6.1 | o.1 |  |
| 2 | ADN (Application Dedicated Node) | 6.1 | o.1 |  |
| 3 | IN (Infrastructure Node) | 6.1 | o.1 |  |
| 4 | MN (Middle Node) | 6.1 | o.1 |  |
| o.1: Exactly one item shall be supported |  |  |  |  |

Table 7.1.5.-2: Functional entity type

| Prerequisite: A.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Name of field | Reference | Status | Support |
| 1 | AE | 5.2.1 | c.1 |  |
| 2 | CSE | 5.2.1 | c.2 |  |
| c.1: If A.1-1 or A.1-2 then m else o c.2: If A.1-2 x else m |  |  |  |  |

Table 7.1.5.-3: Mnemonics for PICS reference

| Mnemonic | PICS item |
| --- | --- |
| PICS_AE | A.2/1 [PICS document] |
| PICS_CSE | A.2/2 [PICS document] |
| PICS_ASN | A.1/1 [PICS document] |
| PICS_ADN | A.1/2 [PICS document] |
| PICS_IN | A.1/3 [PICS document] |
| PICS_MN | A.1/4 [PICS document] |

## 7.2    Test Purposes for oneM2M Service Primitives

    - Group AE (AE)

#### General Capability (GEN)

##### CREATE Operation

###### TP/oneM2M/AE/GEN/CRE/001

| TP Id |  | TP/oneM2M/AE/GEN/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the creation of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS } |  |  |
| ted behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid Container CREATE Request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | CSE  IUT |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/CRE/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/CRE/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/CRE/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/AE/GEN/CRE/002

| TP Id |  | TP/oneM2M/AE/GEN/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the creation of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to HIERARCHICAL_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | CSE  IUT |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/CRE/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/CRE/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/CRE/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### UPDATE Operation

###### TP/oneM2M/AE/GEN/UPD/001

| TP Id |  | TP/oneM2M/AE/GEN/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the update of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing container resource containing valid labels attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  CSE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/UPD/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/UPD/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/UPD/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/AE/GEN/UPD/002

| TP Id |  | TP/oneM2M/AE/GEN/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the update of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing container resource containing valid labels attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing Container resource containing valid labels attribute } |  | IUT  CSE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/UPD/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/UPD/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/UPD/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### RETRIEVE Operation

###### TP/oneM2M/AE/GEN/RET/001

| TP Id |  | TP/oneM2M/AE/GEN/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the retrieval of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/RET/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/RET/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/RET/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/AE/GEN/RET/002

| TP Id |  | TP/oneM2M/AE/GEN/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the retrieval of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/RET/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/RET/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/RET/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### DELETE Operation

###### TP/oneM2M/AE/GEN/DEL/001

| TP Id |  | TP/oneM2M/AE/GEN/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the deletion of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform DELETE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid DELETE request to CSE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/DEL/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/DEL/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/DEL/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/AE/GEN/DEL/002

| TP Id |  | TP/oneM2M/AE/GEN/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the deletion of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform DELETE operation on container resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid DELETE request to CSE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/DEL/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/DEL/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/AE/GEN/DEL/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

#### Registration (REG)

##### CREATE Operation

###### TP/oneM2M/AE/REG/CRE/001

| TP Id | TP/oneM2M/AE/REG//CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an AE initial registration request with no AE-ID-STEM provided when it is started |  |
| Reference | TS-0001 10.1.1.2.2 - case C, 9.6.19 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUT having got a valid APP-ID } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSE_RESOURCE_ADDRESS and Resource Type set to 2 (AE) and Content containing AE resource representation } | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing Resource Type set to 2 (AE) and To set to CSE_RESOURCE_ADDRESS and From set to empty and Content containing AE resource representation } | CSE  IUT |

###### TP/oneM2M/AE/REG/CRE/002

| TP Id |  | TP/oneM2M/AE/REG/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a registration CREATE Request with the value of the attribute ATTRIBUTE_NAME of the AE resource |  |
| Reference |  | TS-0004 7.4.6.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUT having got a valid APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 2 (AE) and Content containing AE resource containing ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing AE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT  CSE |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/REG/CRE/002_RN | TS-0004 2 | resourceName |
| TP/oneM2M/AE/REG/CRE/002_ET | TS-0004 2 | expirationTime |
| TP/oneM2M/AE/REG/CRE/002_LBL | TS-0004 2 | labels |
| TP/oneM2M/AE/REG/CRE/002_APN | TS-0004 2 | appName |
| TP/oneM2M/AE/REG/CRE/002_API | TS-0004 2 | App-ID |
| TP/oneM2M/AE/REG/CRE/002_POA | TS-0004 2 | pointOfAccess |
| TP/oneM2M/AE/REG/CRE/002_OR | TS-0004 2 | ontologyRef |
| TP/oneM2M/AE/REG/CRE/002_NL | TS-0004 2 | nodeLink |
| TP/oneM2M/AE/REG/CRE/002_RR | TS-0004 2 | requestReachability |
| TP/oneM2M/AE/REG/CRE/002_CSZ | TS-0004 2 | contentSerialization |

##### DELETE Operation

###### TP/oneM2M/AE/REG/DEL/001

| TP Id | TP/oneM2M/AE/REG/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends AE deregistration request to CSE |  |
| Reference | TS-0001 10.1.4.2.2 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being in the "initial state" the IUT having registered to CSE and the IUT having privileges to perform DELETE operation on the resource AE to CSE } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to AE_RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } | CSE  IUT |

#### Data Management and Repository (DMR)

##### CREATE Operation

###### TP/oneM2M/AE/DMR/CRE/001

| TP Id |  | TP/oneM2M/AE/DMR/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Container creation request when it is triggered |  |
| Reference |  | TS-0001 10.1.1.1 & 10.2.4.1, TS-0004 7.2.2.1 & 7.4.7.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource representation } |  | CSE  IUT |

###### TP/oneM2M/AE/DMR/CRE/002

| TP Id |  | TP/oneM2M/AE/DMR/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a ContentInstance creation request when it is triggered |  |
| Reference |  | TS-0001 10.1.1.1 & 10.2.4.1, TS-0004 7.2.2.1 & 7.4.8.2.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and Content containing contentInstance resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and From set to AE_ID and Content containing contentInstance resource representation } |  | CSE  IUT |

###### TP/oneM2M/AE/DMR/CRE/003

| TP Id |  | TP/oneM2M/AE/DMR/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a ContentInstance creation request with optional attribute ATTRIBUTE_NAME |  |
| Reference |  | TS-0001 10.1.1.1 & 10.2.4.1, TS-0004 7.2.2.1 & 7.4.8.2.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS through preconfiguration request and the IUT having privileges to perform CREATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and Content containing ContentInstance resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Resource Type set to 4 (contentInstance) and From set to AE_ID and Content containing ContentInstance resource containing valid ATTRIBUTE_NAME attribute } |  | CSE  IUT |

| TP Id | ATTRIBUTE_NAME |
| --- | --- |
| TP/oneM2M/AE/DMR/CRE/003_CNF | contentInfo |
| TP/oneM2M/AE/DMR/CRE/003_CONR | contentRef |
| TP/oneM2M/AE/DMR/CRE/003_RN | resourceName |
| TP/oneM2M/AE/DMR/CRE/003_ET | expirationTime |
| TP/oneM2M/AE/DMR/CRE/003_LBL | labels |
| TP/oneM2M/AE/DMR/CRE/003_CR | creator |

###### TP/oneM2M/AE/DMR/CRE/004

| TP Id |  | TP/oneM2M/AE/DMR/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Container creation request with optional attribute ATTRIBUTE_NAME when it is triggered |  |
| Reference |  | TS-0001 10.1.1.1 & 10.2.4.1, TS-0004 7.2.2.1 & 7.4.7.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to AE_RESOURCE_ADDRESS and Resource Type set to 3 (container) and From set to AE_ID and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | CSE  IUT |

| TP Id | ATTRIBUTE_NAME |
| --- | --- |
| TP/oneM2M/AE/DMR/CRE/004_ACPI | accessControlPolicyIDs |
| TP/oneM2M/AE/DMR/CRE/004_MNI | maxNrOfInstances |
| TP/oneM2M/AE/DMR/CRE/004_MBS | maxByteSize |
| TP/oneM2M/AE/DMR/CRE/004_MIA | maxInstanceAge |
| TP/oneM2M/AE/DMR/CRE/004_OR | ontologyRef |
| TP/oneM2M/AE/DMR/CRE/004_LI | locationID |
| TP/oneM2M/AE/DMR/CRE/004_RN | resourceName |
| TP/oneM2M/AE/DMR/CRE/004_ET | expirationTime |
| TP/oneM2M/AE/DMR/CRE/004_LBL | labels |
| TP/oneM2M/AE/DMR/CRE/004_CR | creator |

Editor’s Note: Combination of optional attributes that are included in the Content of container creation request can be extended to generate more test cases based on this test case.

##### UPDATE Operation

###### TP/oneM2M/AE/DMR/UPD/001

| TP Id |  | TP/oneM2M/AE/DMR/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the CSE having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW attribute ATTRIBUTE_NAME set to VALUE_1 and the IUT having privileges to perform UPDATE operation on resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  CSE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/001_AE/LBL | TS-0001 10.2.1.3 | 2 (AE) | labels |

###### TP/oneM2M/AE/DMR/UPD/002

| TP Id |  | TP/oneM2M/AE/DMR/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the AE resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered containing a RW attribute ATTRIBUTE_NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and Content containing AE resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing AE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT  CSE |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/002_ET | TS-0004 7.4.6.1 | expirationTime |
| TP/oneM2M/AE/DMR/UPD/002_APN | TS-0004 7.4.6.1 | appName |
| TP/oneM2M/AE/DMR/UPD/002_POA | TS-0004 7.4.6.1 | pointOfAccess |
| TP/oneM2M/AE/DMR/UPD/002_OR | TS-0004 7.4.6.1 | ontologyRef |
| TP/oneM2M/AE/DMR/UPD/002_NL | TS-0004 7.4.6.1 | nodeLink |
| TP/oneM2M/AE/DMR/UPD/002_RR | TS-0004 7.4.6.1 | requestReachability |
| TP/oneM2M/AE/DMR/UPD/002_CSZ | TS-0004 7.4.6.1 | contentSerialization |

###### TP/oneM2M/AE/DMR/UPD/003

| TP Id |  | TP/oneM2M/AE/DMR/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the <container> resource |  |
| Reference |  | TS-0004 7.4.7.2.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered containing the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid ATTRIBUTE_NAME attribute } |  | IUT  CSE |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/003_ET | TS-0004 7.4.7.2.3 | expirationTime |
| TP/oneM2M/AE/DMR/UPD/003_LBL | TS-0004 7.4.7.2.3 | labels |
| TP/oneM2M/AE/DMR/UPD/003_MNI | TS-0004 7.4.7.2.3 | maxNrOfInstances |
| TP/oneM2M/AE/DMR/UPD/003_MBS | TS-0004 7.4.7.2.3 | maxByteSize |
| TP/oneM2M/AE/DMR/UPD/003_MIA | TS-0004 7.4.7.2.3 | maxInstanceAge |

##### RETRIEVE Operation

###### TP/oneM2M/AE/DMR/RET/001

| TP Id |  | TP/oneM2M/AE/DMR/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE Request on the TARGET_RESOURCE_ADDRESS to CSE |  |
| Reference |  | TS-0001 10.1.2, TS-0004 7.2.2.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the CSE having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the IUT having privileges to perform RETRIEVE operation on resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to TARGET_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/RET/001_CB | TS-0001 10.2.3.2 | 5 (CSEBase) |
| TP/oneM2M/AE/DMR/RET/001_AE | TS-0001 10.2.1.2 | 2 (AE) |

###### TP/oneM2M/AE/DMR/RET/002

| TP Id |  | TP/oneM2M/AE/DMR/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE Request of oldest virtual resource. |  |
| Reference |  | TS-0001 10.2.23.1, TS-0004 7.4.29.2.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest and From set to AE_ID } |  | IUT  CSE |

###### TP/oneM2M/AE/DMR/RET/003

| TP Id |  | TP/oneM2M/AE/DMR/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE Request of latest virtual resource. |  |
| Reference |  | TS-0001 10.2.22.1, TS-0004 7.4.28.2.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/latest } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS/latest and From set to AE_ID } |  | IUT  CSE |

###### TP/oneM2M/AE/DMR/RET/004

| TP Id |  | TP/oneM2M/AE/DMR/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a <container> resource RETRIEVE Request to CSE |  |
| Reference |  | TS-0004 7.4.7.2.2 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on container resource } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

###### TP/oneM2M/AE/DMR/RET/005

| TP Id |  | TP/oneM2M/AE/DMR/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a <contentInstance> resource RETRIEVE Request to CSE |  |
| Reference |  | TS-0004 7.4.8.2.2 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  CSE |

##### DELETE Operation

###### TP/oneM2M/AE/DMR/DEL/001

| TP Id |  | TP/oneM2M/AE/DMR/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a DELETE Request of oldest virtual resource. |  |
| Reference |  | TS-0001 10.2.23.2, TS-0004 7.4.29.2.5 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest } |  | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest and From set to AE_ID } |  | IUT  CSE |

###### TP/oneM2M/AE/DMR/DEL/002

| TP Id |  | TP/oneM2M/AE/DMR/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a DELETE Request of latest virtual resource. |  |
| Reference |  | TS-0001 10.2.22.2, TS-0004 7.4.28.2.5 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/oldest } |  | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS/latest and From set to AE_ID } |  | IUT  CSE |

###### TP/oneM2M/AE/DMR/DEL/003

| TP Id | TP/oneM2M/AE/DMR/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <container> resource DELETE request to CSE |  |
| Reference | TS-0004 7.4.7.2.4 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and and the IUT being switched on and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform DELETE operation on container resource } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } | CSE  IUT |

###### TP/oneM2M/AE/DMR/DEL/004

| TP Id | TP/oneM2M/AE/DMR/DEL/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <contentInstance> resource DELETE request to CSE |  |
| Reference | TS-0004 7.4.8.2.4 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS } | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to CONTENTINSTANCE_RESOURCE_ADDRESS and From set to AE_ID } | CSE  IUT |

#### Subscription and Notification (SUB)

##### CREATE Operation

###### TP/oneM2M/AE/SUB/CRE/001

| TP Id |  | TP/oneM2M/AE/SUB/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a subscription creation request |  |
| Reference |  | TS-0001 10.1.1.1 & 10.2.4.1, TS-0004 7.2.2.1 & 7.4.9.2.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT being switched off and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing Resource Type set to 23 (subscription) and To set to AE_RESOURCE_ADDRESS and Content containing subscription resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing Resource Type set to 23 (subscription) and To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Content containing subscription resource representation } |  | CSE  IUT |

###### TP/oneM2M/AE/SUB/CRE/002

| TP Id |  | TP/oneM2M/AE/SUB/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a subscription creation request with optional attribute ATTRIBUTE_NAME when it is triggered |  |
| Reference |  | TS-0001 10.1.1.1 & 10.2.11.2, TS-0004 7.2.2.1 & 7.4.9.2.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT having privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and Content containing subscription resource containing notificationURI attribute and valid ATTRIBUTE_NAME attribute } |  | NA |
|  | then { the IUT sends a valid CREATE Request to CSE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE_ID and Content containing subscription resource containing notificationURI attribute and valid ATTRIBUTE_NAME attribute } |  | CSE  IUT |

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/SUB/CRE/002_ACPI | TS-0001 9.6.8, TS-0004 7.4.9.1 | accessControlPolicyIDs |
| TP/oneM2M/AE/SUB/CRE/002_ENC | TS-0001 9.6.8, TS-0004 7.4.9.1 | eventNotificationCriteria |
| TP/oneM2M/AE/SUB/CRE/002_EXC | TS-0001 9.6.8, TS-0004 7.4.9.1 | expirationCounter |
| TP/oneM2M/AE/SUB/CRE/002_NFU | TS-0001 9.6.8, TS-0004 7.4.9.1 | notificationForwardingURI |
| TP/oneM2M/AE/SUB/CRE/002_NCT | TS-0001 9.6.8, TS-0004 7.4.9.1 | notificationContentType |
| TP/oneM2M/AE/SUB/CRE/002_NEC | TS-0001 9.6.8, TS-0004 7.4.9.1 | notificationEventCat |
| TP/oneM2M/AE/SUB/CRE/002_SU | TS-0001 9.6.8, TS-0004 7.4.9.1 | subscriberURI |

Editor’s Note: Combination of optional attributes that are included in the Content of subscription creation request can be extended to generate more test cases based on this test case.

##### NOTIFY Operation

###### TP/oneM2M/AE/SUB/NTF/001

| TP Id |  | TP/oneM2M/AE/SUB/NTF/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify Response to the hosting CSE when receiving a Notify request containing a single notification |  |
| Reference |  | TS-0001 6.1.12, TS-0004 7.5.1.2 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT having been registered and the IUT having created subscription resource under the CSE and the IUT being reachable through a URL ACCESSIBLE_URL } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a NOTIFY Request from hosting CSE containing Content containing notification message representation } |  | IUT  CSE |
|  | then { the IUT sends a valid NOTIFY Response to the hosting CSE containing Response Status Code set to RESPONSE_STATUS_CODE } |  | CSE  IUT |

###### TP/oneM2M/AE/SUB/NTF/002

| TP Id |  | TP/oneM2M/AE/SUB/NTF/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify Response to the hosting CSE when receiving a Notify request containing aggregated notifications |  |
| Reference |  | TS-0001 6.2.12, TS-0004 7.5.1.2 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT having been registered and the IUT having created subscription resource under the CSE and the IUT being reachable through a URL ACCESSIBLE_URL } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a NOTIFY Request from hosting CSE containing Content containing aggregatedNotification message representation } |  | IUT  CSE |
|  | then { the IUT sends a valid Response to the hosting CSE containing Response Status Code set to RESPONSE_STATUS_CODE } |  | CSE  IUT |

#### Group Management (GMG)

#### Location (LOC)

#### Device Management (DMG)

#### Communication Management and Delivery Handling (CMDH)

#### Security (SEC)

    - Group CSE (CE)

#### General Capability (GEN)

##### CREATE Operation

###### TP/oneM2M/CSE/GEN/CRE/001

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/CRE/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/CRE/002

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/CRE/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### UPDATE Operation

###### TP/oneM2M/CSE/GEN/UPD/001

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/UPD/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/UPD/002

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing valid labels attribute } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/UPD/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### RETRIEVE Operation

###### TP/oneM2M/CSE/GEN/RET/001

| TP Id |  | TP/oneM2M/CSE/GEN/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content containing container resource representation } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/RET/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/RET/002

| TP Id |  | TP/oneM2M/CSE/GEN/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2000 (OK) and Content containing container resource representation } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/RET/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### DELETE Operation

###### TP/oneM2M/CSE/GEN/DEL/001

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/001_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/DEL/001_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/DEL/002

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE request from AE containing To set to HIERARCHICAL_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response message containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/002_SPR | SP_RELATIVE_RESOURCE_ID* |
| TP/oneM2M/CSE/GEN/DEL/002_ABS | ABSOLUTE_RESOURCE_ID* |
| * These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

#### Registration (REG)

##### RETRIEVE Operation

###### TP/oneM2M/CSE/REG/RET/001

| TP Id |  | TP/oneM2M/CSE/REG/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource and responds with all attributes that have multiplicity equals to 1. |  |
| Reference |  | TS-0001 10.2.3.2, TS-0001 9.6.3, TS-0004 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource representation } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/RET/002

| TP Id |  | TP/oneM2M/CSE/REG/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource with the optional ATTRIBUTE. |  |
| Reference |  | TS-0001 10.2.3.2, TS-0001 9.6.3, TS-0004 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource and the IUT having a CSEBase resource containing the valid ATTRIBUTE attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource containing valid ATTRIBUTE attribute } |  | IUT  AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/002_CST | TS-0001 9.6.3 | cseType |
| TP/oneM2M/CSE/REG/RET/002_NL | TS-0001 9.6.3 | nodeLink |

###### TP/oneM2M/CSE/REG/RET/003

| TP Id |  | TP/oneM2M/CSE/REG/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <AE> resource with attributes multiplicity equals to 1. |  |
| Reference |  | TS-0001 10.2.1.2, TS-0001 9.6.5, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the AE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing AE resource representation } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/RET/004

| TP Id |  | TP/oneM2M/CSE/REG/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <AE> resource with optional ATTRIBUTE. |  |
| Reference |  | TS-0001 10.2.1.2, TS-0001 9.6.5, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing valid ATTRIBUTE attribute and the AE having privileges to perform RETRIEVE operation on the AE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing AE resource containing ATTRIBUTE attribute } |  | IUT  AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/004_LBL | TS-0001 9.6.5 | labels |
| TP/oneM2M/CSE/REG/RET/004_APN | TS-0001 9.6.5 | appName |
| TP/oneM2M/CSE/REG/RET/004_POA | TS-0001 9.6.5 | pointOfAccess |
| TP/oneM2M/CSE/REG/RET/004_NL | TS-0001 9.6.5 | nodeLink |
| TP/oneM2M/CSE/REG/RET/004_CSZ | TS-0001 9.6.5 | contentSerialization |

###### TP/oneM2M/CSE/REG/RET/005

| TP Id |  | TP/oneM2M/CSE/REG/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource including the cseType attribute |  |
| Reference |  | TS-0001 10.2.3.2, TS-0001 9.6.3, TS-0004 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource containing cseType attribute set to 1 (IN_CSE) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/RET/006

| TP Id |  | TP/oneM2M/CSE/REG/RET/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <remoteCSE> resource. |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.4.2.1, TS-0001 9.6.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the ORIGINATOR having privileges to perform RETRIEVE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from ORIGINATOR containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT  ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing remoteCSE resource representation } |  | IUT  ORIGINATOR |

###### TP/oneM2M/CSE/REG/RET/007

| TP Id |  | TP/oneM2M/CSE/REG/RET/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <remoteCSE> resource with optional ATTRIBUTE |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.4.2.1, TS-0001 9.6.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the ORIGINATOR having privileges to perform RETRIEVE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from ORIGINATOR containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT  ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing remoteCSE resource containing ATTRIBUTE attribute } |  | IUT  ORIGINATOR |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/007_LBL | TS-0001 9.6.4 | labels |
| TP/oneM2M/CSE/REG/RET/007_CST | TS-0001 9.6.4 | cseType |
| TP/oneM2M/CSE/REG/RET/007_POA | TS-0001 9.6.4 | pointOfAccess |
| TP/oneM2M/CSE/REG/RET/007_NL | TS-0001 9.6.4 | nodeLink |

###### TP/oneM2M/CSE/REG/RET/008

| TP Id |  | TP/oneM2M/CSE/REG/RET/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a retrieval request of <CSEBase> resource and responds with supportedResourceTypes attribute containing a list of the supported resources and pointOfAccess containing the list of physical addresses to be used by Registree to connect to this CSE (e.g. IP address, FQDN). |  |
| Reference |  | TS-0001 10.2.3.2, TS-0001 9.6.3, TS-0004 7.4.3.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CSEBase resource containing supportedResourceType set to PX_SRT pointOfAccess set to PX_POA } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/RET/009

| TP Id |  | TP/oneM2M/CSE/REG/RET/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> retrieve request on TARGET_REMOTE_CSE_ADDRESS |  |
| Reference |  | TS-0001 10.2.2.2, TS-0004 7.4.5.2.3 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS } |  | NA |
|  | then { the IUT sends a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/RET/010

| TP Id |  | TP/oneM2M/CSE/REG/RET/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> retrieve request |  |
| Reference |  | TS-0001 10.2.2.2, TS-0004 7.4.5.2.3 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing remoteCSE resource representation } |  | IUT  CSE |

##### CREATE Operation

###### TP/oneM2M/CSE/REG/CRE/001

| TP Id |  | TP/oneM2M/CSE/REG/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned SP_relative_ AE_ID. |  |
| Reference |  | TS-0001 10.1.1.2.2 case b, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to SP_RELATIVE_AE_ID and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource representation } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/002

| TP Id |  | TP/oneM2M/CSE/REG/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned AE_ID of AE-ID-Stem format. |  |
| Reference |  | TS-0001 10.1.1.2.2 case d, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID_STEM and Resource Type set to 2 (AE) and Content containing AE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource representation } |  | IUT  AE |

Editor’s Note: Test purpose TP/oneM2M/CSE/REG/CRE/002 is duplicated with TP/oneM2M/CSE/REG/CRE/026 in terms of testing AE re-registration in TS-0001 10.1.1.2.2. case d. Therefore, test purpose TP/oneM2M/CSE/REG/CRE/026 is going to be removed.

###### TP/oneM2M/CSE/REG/CRE/003

| TP Id |  | TP/oneM2M/CSE/REG/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned SP_relative_AE_ID. |  |
| Reference |  | TS-0001 10.1.1.2.2 case b, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem and the IUT having deregistrated the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to SP_RELATIVE_AE_ID and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT  AE |
|  | then{ the IUT sends a valid UPDATE Request to IN-CSE containing To set to AEANNC_RESOURCE_ADDRESS and From set to IUT_CSE_ID and Content containing AEAnnc resource representation } |  | IUT  IN-CSE |

Editor’s Note: need to clarify if there is duplication.

###### TP/oneM2M/CSE/REG/CRE/004

| TP Id |  | TP/oneM2M/CSE/REG/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (allowed App-ID, not allowed C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 – step 3, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a serviceSubscribedNode resource containing ruleLinks attribute pointing to a serviceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and no allowedAEs attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to C-AE-ID-STEM and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT does not create the AE resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/005

| TP Id |  | TP/oneM2M/CSE/REG/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, S-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 case a, 9.6.19 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a serviceSubscribedNode resource containing ruleLinks attribute pointing to a serviceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating a value starting with ‘S’ and containing wildcard } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to empty and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a valid CREATE Request to the IN-CSE containing Resource Type set to 10002 (AEAnnc) and From set to ‘/S’ and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and nodeLink attribute set to ??? and labels indicating ‘Credential-ID:None’ } |  | IUT  IN-CSE |

Editor’s Note: Double check the value of attribute nodeLink!!

###### TP/oneM2M/CSE/REG/CRE/006

| TP Id |  | TP/oneM2M/CSE/REG/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided character ‘S’ in AE_ID_Stem ask for a SP_relative_AE_ID. |  |
| Reference |  | TS-0001 10.1.1.2.2 case a, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to character ‘S’ and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT  AE |
|  | then{ the IUT sends a valid CREATE Request to IN_CSE containing To set to IN_CSEBASE_RESOURCE_ADDRESS and From set to IUT_CSE_ID and Content containing AEAnnc resource containing link attribute set to ‘S’ } |  | IUT  IN-CSE |

###### TP/oneM2M/CSE/REG/CRE/007

| TP Id |  | TP/oneM2M/CSE/REG/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an response of AEANNC create request during AE registration with provided character ‘S’ in AE_ID_Stem and asked for a SP_relative_AE_ID. |  |
| Reference |  | TS-0001 10.1.1.2.2 case a, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem and the IUT having received a valid AE create request and the IUT having sent a valid AEANNC create to IN_CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when{ the IUT receives a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AEAnnc resource containing link attribute set to SP_RELATIVE_AE_ID } |  | IUT <- IN-CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing ResourceID attribute set to SP_RELATIVE_AE_ID and AE-ID attribute set to SP_RELATIVE_AE_ID } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/008

| TP Id |  | TP/oneM2M/CSE/REG/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer request to the IN-CSE |  |
| Reference |  | TS-0001 10.1.1.2.2 – case b, step 5, 9.6.19 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a serviceSubscribedNode resource containing ruleLinks attribute pointing to a serviceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating a value starting with ‘S’ and containing wildcard } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to S-AE-ID-STEM and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT sends a valid UPDATE Request to the IN-CSE containing Resource Type set to 10002 (AEAnnc) and To set to IN_CSE_RESOURCE_ADDRESS and From set to ‘/S’ and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and nodeLink attribute set to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS and labels indicating ‘Credential-ID:None’ } |  | IUT  IN-CSE |
| Note: AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS = /CSE_ID/ S-AE-ID-STEM |  |  |  |

Editor’s Note: TS-0001 defines the nodeLink attribute in AE resource is set to the resourceID of a <node> resource that stores the node specific information where this AE resides. If this is the case, it is not correct to set the nodeLink attribute to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS. Clarification is required here!!

###### TP/oneM2M/CSE/REG/CRE/009

| TP Id |  | TP/oneM2M/CSE/REG/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference |  | TS-0001 10.1.1.2.2 – case b, step 5-8 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the AE having already been registered and the AE having deregistered } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from the CSE containing Resource Type set to 10002 (AEAnnc) and To set to AEANNC_RESOURCE_ADDRESS and From set to ‘/S’ and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and nodeLink attribute set to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS and labels indicating ‘Credential-ID:None’ } |  | IUT MN-CSE |
|  | then { the IUT updates the AEANNC_RESOURCE_ADDRESS resource and the IUT sends a valid Response to CSE containing Response Status Code set to 2004 (UPDATED) and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and nodeLink attribute set to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS and labels indicating ‘Credential-ID:None’ } |  | IUT  MN-CSE |
| Note: AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS = /CSE_ID/ S-AE-ID-STEM |  |  |  |

Editor’s Note: TS-0001 defines the nodeLink attribute in AE resource is set to the resourceID of a <node> resource that stores the node specific information where this AE resides. If this is the case, it is not correct to set the nodeLink attribute to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS. Clarification is required here!!

###### TP/oneM2M/CSE/REG/CRE/010

| TP Id |  | TP/oneM2M/CSE/REG/CRE/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer response to the AE |  |
| Reference |  | TS-0001 10.1.1.2.2 – case b, step 8 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a serviceSubscribedNode resource containing ruleLinks attribute pointing to a serviceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating a value starting with ‘S’ and containing wildcard } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Response from IN-CSE containing Response Status Code set to 2004 (UPDATED) and Content containing AEAnnc resource containing App-ID attribute set to APP-ID and nodeLink attribute set to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS and labels indicating ‘Credential-ID:None’ } |  | IUT  IN-CSE |
|  | then { the IUT sends a valid Response to the AE containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT  AE |
| Note: AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS = /CSE_ID/ S-AE-ID-STEM |  |  |  |

Editor’s Note: TS-0001 defines the nodeLink attribute in AE resource is set to the resourceID of a <node> resource that stores the node specific information where this AE resides. If this is the case, it is not correct to set the nodeLink attribute to AE_UNSTRUCTURED_SP_RELATIVE_ADDRESS. Clarification is required here!!

###### TP/oneM2M/CSE/REG/CRE/011

| TP Id |  | TP/oneM2M/CSE/REG/CRE/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and asking an CSE relative AE_ID by indicating AE_ID_Stem with character ‘C’. |  |
| Reference |  | TS-0001 10.1.1.2.2 case c, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to ‘C’ and Resource Type set to 2(AE) and Content containing AE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing ResourceID attribute set to CSE_RELATIVE_AE_ID and AE-ID attribute set to SP_RELATIVE_AE_ID } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/013

| TP Id |  | TP/oneM2M/CSE/REG/CRE/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration with the optional attribute OPTIONAL_ATTRIBUTE provided |  |
| Reference |  | TS-0001 10.1.1.2.2, 9.6.5-2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to AE_ID and Content containing AE resource containing valid OPTIONAL_ATTRIBUTE attribute } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource containing valid OPTIONAL_ATTRIBUTE attribute } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/013_AE/LBL | TS-0001 9.6.5-2 | AE | labels |
| TP/oneM2M/CSE/REG/CRE/013_AE/APN | TS-0001 9.6.5-2 | AE | AppName |
| TP/oneM2M/CSE/REG/CRE/013_AE/POA | TS-0001 9.6.5-2 | AE | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/013_AE/NL | TS-0001 9.6.5-2 | AE | nodeLink |
| TP/oneM2M/CSE/REG/CRE/013_AE/OR | TS-0001 9.6.5-2 | AE | ontologyRef |

###### TP/oneM2M/CSE/REG/CRE/014

| TP Id |  | TP/oneM2M/CSE/REG/CRE/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the notification to the AE according to pointOfAccess in the <AE> resource. |  |
| Reference |  | TS-0001 9.6.5, TS-0004 7.3.3.9, TS-0001 9.3.2.3.1 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created the AE resource containing POINT_OF_ACCESS attribute and REQUEST_REACHABILITY set to true } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid NOTIFY from CSE containing To set to AE_RESOURCE_ADDRESS From set to CSE_ID } |  | IUT  CSE |
|  | then { the IUT sends the valid NOTIFY to POINT_OF_ACCESS } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/015

| TP Id | TP/oneM2M/CSE/REG/CRE/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the notification to the AE according to pointOfAccess in the <AE> resource and forward the response back to the CSE. |  |
| Reference | TS-0001 9.6.5, TS-0004 7.3.3.9, TS-0001 9.3.2.3.1 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received a valid NOTIFY from CSE and The IUT having sent the NOTIFY to the AE } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid NOTIFY RESPONSE from the AE } | IUT  AE |
|  | then { the IUT sends the valid NOTIFY RESPONSE to the CSE } | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/016

| TP Id |  | TP/oneM2M/CSE/REG/CRE/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <remoteCSE> resource with OPTIONAL_ATTRIBUTE. |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.4.2.1, TS-0001 9.6.4 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation containing OPTIONAL_ATTRIBUTE attribute } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  | IUT  CSE |

| TP Id | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/016_CSR/LBL | TS-0001 9.6.4 | remoteCSE | labels |
| TP/oneM2M/CSE/REG/CRE/016_CSR/CST | TS-0001 9.6.4 | remoteCSE | cseType |
| TP/oneM2M/CSE/REG/CRE/016_CSR/POA | TS-0001 9.6.4 | remoteCSE | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/016_CSR/NL | TS-0001 9.6.4 | remoteCSE | nodeLink |

###### TP/oneM2M/CSE/REG/CRE/017

| TP Id |  | TP/oneM2M/CSE/REG/CRE/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the request to the CSE according to pointOfAccess in the <remoteCSE> resource. |  |
| Reference |  | TS-0001 9.6.4, TS-0004 6.2.3, TS-0001 9.3.2.2.3 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE, PICS_SP_relative_Resource_ID, PICS_Absolute_Resource_ID |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created the remoteCSE resource containing POINT_OF_ACCESS attribute } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid REQUEST from Originator containing To set to RESOURCE_ADDRESS and From set to Originator_ID } |  | IUT  Originator |
|  | then { the IUT sends the REQUEST to POINT_OF_ACCESS } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/018

| TP Id |  | TP/oneM2M/CSE/REG/CRE/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT retargets the request to the CSE according to pointOfAccess in the <remoteCSE> resource and forward the response back to the originator. |  |
| Reference |  | TS-0001 9.6.4, TS-0004 6.2.3, TS-0001 9.3.2.2.3 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE , PICS_SP_relative_Resource_ID, PICS_Absolute_Resource_ID |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received a valid REQUEST from Originator and The IUT having sent the REQUEST to the remoteCSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RESPONSE from the remoteCSE } |  | IUT  CSE |
|  | then { the IUT sends the RESPONSE to the Originator } |  | IUT  Originator |

###### TP/oneM2M/CSE/REG/CRE/019

| TP Id |  | TP/oneM2M/CSE/REG/CRE/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (not allowed App-ID) |  |
| Reference |  | TS-0001 10.1.1.2.2 – step 3, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing no serviceSubscribedNode resource containing ruleLinks attribute pointing to a serviceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT does not create the AE resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/020

| TP Id |  | TP/oneM2M/CSE/REG/CRE/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 without preference of format of AE_ID. |  |
| Reference |  |  |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |

Editor’s Note: the case need to be added after the publication of updated R2 of TS-0001 where the case will be added.

###### TP/oneM2M/CSE/REG/CRE/021

| TP Id |  | TP/oneM2M/CSE/REG/CRE/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a create request of <AE> resource that doesn’t include the MANDATORY_ATTRIBUTE. |  |
| Reference |  | TS-0001 10.1.1.2.2, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 2(AE) and Content containing AE resource containing no MANDATORY_ATTRIBUTE attribute } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD Request) } |  | IUT  AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/021_API | TS-0001 9.6.5 | App-ID |
| TP/oneM2M/CSE/REG/CRE/021_RR | TS-0001 9.6.5 | requestReachability |

###### TP/oneM2M/CSE/REG/CRE/022

| TP Id |  | TP/oneM2M/CSE/REG/CRE/022 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <remoteCSE> resource with attributes multiplicity equals to 1. |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2, TS-0001 9.6.4 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/023

| TP Id |  | TP/oneM2M/CSE/REG/CRE/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a create request of <remoteCSE> resource with attributes multiplicity equals to 1 without the preconfigured CSE-ID. |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2, TS-0001 9.6.4 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and no From and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/024

| TP Id |  | TP/oneM2M/CSE/REG/CRE/024 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a response from IN_CSE on the success update of AEAnnc during AE registration with preprovisioned SP_relative_AE_ID. |  |
| Reference |  | TS-0001 10.1.1.2.2 case b, TS-0004 7.4.5.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received the AE creation request and the IUT having sent out the AE_ANNC Update request to IN_CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when{ the IUT receives a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing AEAnnc resource representation } |  | IUT <- IN-CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing AE resource representation } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/025

| TP Id |  | TP/oneM2M/CSE/REG/CRE/025 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the create request of <CSEBase> resource. |  |
| Reference |  | TS-0001 10.2.3.1, TS-0001 9.6.3, TS-0004 7.4.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and Resource Type set to 5 (CSEBase) and From set to AE-ID and Content containing CSEBase resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/027

| TP Id |  | TP/oneM2M/CSE/REG/CRE/027 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, C-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 - case C, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource containing a ServiceSubscribedProfile resource containing a serviceSubscribedNode resource containing ruleLinks attribute pointing to a serviceSubscribedAppRule resource containing applicableCredIDs attribute set to None and allowedApp-IDs attribute indicating APP-ID and allowedAEs attribute indicating C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and From set to empty and Content containing AE resource containing App-ID attribute set to APP-ID } |  | IUT  AE |
|  | then { the IUT creates the AE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/028

| - TP Id |  | TP/oneM2M/CSE/REG/CRE/028 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects registration of already registered AE (C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 10.1.1.2.2 – step 4, 9.6.19 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state"and the IUT having registered the AE with AE-ID attribute set to C-AE-ID-STEM } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and To set to TARGE_CSE_RESOURCE_ADDRESS and From set to C-AE-ID-STEM and Content containing AE resource representation } |  | IUT  AE |
|  | then { the IUT does not create the AE resource and the IUT sends a valid Response containing Response Status Code set to 4105 (CONFLICT) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/CRE/029

| TP Id |  | TP/oneM2M/CSE/REG/CRE/029 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with attributes multiplicity equals to 1 |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/030

| TP Id |  | TP/oneM2M/CSE/REG/CRE/030 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with attributes multiplicity equals to 1 |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation and the IUT creates the <remoteCSE> resource } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/031

| TP Id |  | TP/oneM2M/CSE/REG/CRE/031 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | IUT  CSE |

| TP Id | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/031_RN | TS-0001 9.6.4-2 | resourceName |
| TP/oneM2M/CSE/REG/CRE/031_ET | TS-0001 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/CRE/031_LBL | TS-0001 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/CRE/031_POA | TS-0001 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/031_NL | TS-0001 9.6.4-2 | nodeLink |

###### TP/oneM2M/CSE/REG/CRE/032

| TP Id |  | TP/oneM2M/CSE/REG/CRE/032 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with cseType attribute set to ‘MN_CSE’ |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) } |  | NA |
|  | then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/033

| TP Id |  | TP/oneM2M/CSE/REG/CRE/033 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with cseType attribute set to ‘MN_CSE’ |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and the IUT creates a <remoteCSE> resource } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/CRE/034

| TP Id |  | TP/oneM2M/CSE/REG/CRE/034 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE and the IUT creates a <remoteCSE> resource } |  | IUT  CSE |

| TP Id | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/034_RN | TS-0001 9.6.4-2 | resourceName |
| TP/oneM2M/CSE/REG/CRE/034_ET | TS-0001 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/CRE/034_LBL | TS-0001 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/CRE/034_POA | TS-0001 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/034_NL | TS-0001 9.6.4-2 | nodeLink |

##### DELETE Operation

###### TP/oneM2M/CSE/REG/DEL/001

| TP Id |  | TP/oneM2M/CSE/REG/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the delete request of <CSEBase> resource. |  |
| Reference |  | TS-0001 10.2.3.4, TS-0001 9.6.3, TS-0004 7.4.3.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/DEL/002

| TP Id |  | TP/oneM2M/CSE/REG/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an delete request of <remoteCSE> resource. |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.4.2.1, TS-0001 9.6.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the CSE and the CSE originator having privileges to perform DELETE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from CSE containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to CSE_ID and no Content } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |

Editor’s Note: The originator of <remoteCSE> delete request can only be CSE that is AE cannot initate a <remoteCSE> delete request.

###### TP/oneM2M/CSE/REG/DEL/003

| TP Id |  | TP/oneM2M/CSE/REG/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration. |  |
| Reference |  | TS-0001 10.1.4.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform DELETE operation on the resource AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the AE_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/DEL/004

| TP Id |  | TP/oneM2M/CSE/REG/DEL/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> delete request on TARGET_REMOTE_CSE_ADDRESS |  |
| Reference |  | TS-0001 10.2.2.4, TS-0004 7.4.5.2.5 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform DELETE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESSS } |  | NA |
|  | then { the IUT sends a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT  CSE |

###### TP/oneM2M/CSE/REG/DEL/005

| TP Id |  | TP/oneM2M/CSE/REG/DEL/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> delete request on TARGET_REMOTE_CSE_ADDRESS, and deletes the <remoteCSE> resource |  |
| Reference |  | TS-0001 10.2.2.4, TS-0004 7.4.5.2.5 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform DELETE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and no Content } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT deletes the remoteCSE resource } |  | IUT  CSE |

##### UPDATE Operation

###### TP/oneM2M/CSE/REG/UPD/001

| TP Id |  | TP/oneM2M/CSE/REG/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update request of <CSEBase> resource. |  |
| Reference |  | TS-0001 10.2.3.3, TS-0001 9.6.3, TS-0004 7.4.3.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE_ID and Content containing CSEBase resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/REG/UPD/002

| TP Id |  | TP/oneM2M/CSE/REG/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an update request of <remoteCSE> resource. |  |
| Reference |  | TS-0001 10.1.1.2.1, TS-0004 7.4.4.2.1, TS-0001 9.6.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the CSE originator having privileges to perform UPDATE operation on the remoteCSE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from ORIGINATOR containing To set to REMOTECSE_RESOURCE_ADDRESS and From set to CSE_ID and Content containing remoteCSE resource containing valid ATTRIBUTE attribute } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing remoteCSE resource representation containing valid ATTRIBUTE attribute } |  | IUT  CSE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/002_ET | TS-0001 9.6.4 | expirationTime |
| TP/oneM2M/CSE/REG/UPD/002_LBL | TS-0001 9.6.4 | labels |
| TP/oneM2M/CSE/REG/UPD/002_POA | TS-0001 9.6.4 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/002_NL | TS-0001 9.6.4 | nodeLink |

###### TP/oneM2M/CSE/REG/UPD/003

| TP Id |  | TP/oneM2M/CSE/REG/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> update request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 10.2.2.3, TS-0004 7.4.5.2.4 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS containing a RW OPTIONAL_ATTRIBUTE attribute set to VALUE_1 and the IUT having privileges to perform UPDATE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is triggered to send a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and Content containing remoteCSE resource containing attribute OPTIONAL_ATTRIBUTE set to VALUE_2 } |  | NA |
|  | then { the IUT sends a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and Content containing remoteCSE resource containing attribute OPTIONAL_ATTRIBUTE set to VALUE_2 } |  | IUT  CSE |

| TP Id | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/003_ET | TS-0001 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/UPD/003_LBL | TS-0001 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/UPD/003_POA | TS-0001 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/003_NL | TS-0001 9.6.4-2 | nodeLink |
| TP/oneM2M/CSE/REG/UPD/003_RR | TS-0001 9.6.4-2 | RequestReachability |

###### TP/oneM2M/CSE/REG/UPD/004

| TP Id |  | TP/oneM2M/CSE/REG/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> update request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 10.2.2.3, TS-0004 7.4.5.2.4 |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS containing a RW OPTIONAL_ATTRIBUTE attribute set to VALUE_1 and the IUT having privileges to perform UPDATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS and From set to CSE_ID and Content containing remoteCSE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE_2 } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing remoteCSE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE_2 } |  | IUT  CSE |

| TP Id | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/004_ET | TS-0001 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/UPD/004_LBL | TS-0001 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/UPD/004_POA | TS-0001 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/004_NL | TS-0001 9.6.4-2 | nodeLink |
| TP/oneM2M/CSE/REG/UPD/004_RR | TS-0001 9.6.4-2 | RequestReachability |

#### Data Management and Repository Function (DMR)

##### RETRIEVE Operation

###### TP/oneM2M/CSE/DMR/RET/001

| TP Id |  | TP/oneM2M/CSE/DMR/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/001_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/001_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/001_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/001_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/001_PCH | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/001_SUB | TS-0001 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/001_CIN | TS-0001 10.2.4.2 | 4 (contentInstance) |

###### TP/oneM2M/CSE/DMR/RET/002

| TP Id |  | TP/oneM2M/CSE/DMR/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference |  | TS-0001 10.1.2 - item 13) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/RET/003

| TP Id |  | TP/oneM2M/CSE/DMR/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a resource TARGET_RESOURCE_ADDRESS under AE without having privileges for the RETRIEVE operation |  |
| Reference |  | TS-0001 10.1.2 - item 14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/003_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/003_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/003_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/003_SCH | TS-0001 9.6.9 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/003_PCH | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/003_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/004

| TP Id |  | TP/oneM2M/CSE/DMR/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource (“To” option) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_ATTRIBUTE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT  AE |
| Notes: TARGET_ATTRIBUTE_ADDRESS is represented as TARGET_RESOURCE_ADDRESS#ATTRIBUTE where ‘#’ is present between TARGET_RESOURCE_ADDRESS and ATTRIBUTE. ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/004_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/004_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/004_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/004_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/004_PCH | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/004_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/005

| TP Id |  | TP/oneM2M/CSE/DMR/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource (“Content” option) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT  AE |
| Notes: ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/005_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/005_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/005_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/005_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/005_PCH | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/005_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/006

| TP Id |  | TP/oneM2M/CSE/DMR/RET/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containin ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_1 attribute and valid ATTRIBUTE_2 attribute and no other attribute } |  | IUT  AE |
| Notes: ATTRIBUTE_1 indicates common attribute for all RESOURCE_TYPE resources listed in the table below ATTRIBUTE_2 indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/006_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/006_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/006_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/006_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/006_PCH | TS-0001 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/006_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/007

| TP Id |  | TP/oneM2M/CSE/DMR/RET/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource (“single” attribute case) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource containing no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/007_CNT/LBL | TS-0001 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/007_GRP/LBL | TS-0001 10.2.7.3 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/RET/007_ACP/LBL | TS-0001 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/007_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/RET/007_PCH/LBL | TS-0001 10.2.13.3 | 15 (pollingChallenge) | labels |
| TP/oneM2M/CSE/DMR/RET/007_SUB/LBL | TS-0001 10.2.11.3 | 23 (subscription) | labels |

Editor Note: A common ATTRIBUTE has been chosen for all types of resource. A note could replace ATTRIBUTE column.

###### TP/oneM2M/CSE/DMR/RET/008

| TP Id |  | TP/oneM2M/CSE/DMR/RET/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attributes of the TARGET_RESOURCE_ADDRESS resource (“multiple” attributes case) |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource containing no ATTRIBUTE_1 attribute and no ATTRIBUTE_2 attribute and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/008_CNT/LBL/AT | TS-0001 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_GRP/LBL/AT | TS-0001 10.2.7.3 | 9 (group) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_ACP/LBL/AT | TS-0001 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_SCH/LBL/AT |  | 18 (schedule) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_PCH/LBL/AT | TS-0001 10.2.13.3 | 15 (pollingChallenge) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_SUB/LBL/AT | TS-0001 10.2.11.3 | 23 (subscription) | labels | announceTo |

Editor Note: The references to the TS-0001 for the operations (CREATE, RETRIEVE) procedures of the resource type <schedule> have not been found. To check whether these procedures are missing or they have been added in newer versions (TS-0001 v1.12.0 as basis)

###### TP/oneM2M/CSE/DMR/RET/009

| TP Id |  | TP/oneM2M/CSE/DMR/RET/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the RETRIEVE Request of a latest resource target to a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 10.2.22.1, TS-0004 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no direct child contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/RET/010

| TP Id | TP/oneM2M/CSE/DMR/RET/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the RETRIEVE Request of the oldest contentInstance target to a <container> resource |  |
| Reference | TS-0001 10.2.23.1, TS-0004 7.4.29.2.3 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform RETRIEVE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing contentInstance resource containing resourceName attribute set to CONTENT_INSTANCE_1 } | IUT  AE |

###### TP/oneM2M/CSE/DMR/RET/011

| TP Id |  | TP/oneM2M/CSE/DMR/RET/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the RETRIEVE Request of an oldest resource target to a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 10.2.23.1, TS-0004 7.4.29.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no direct child contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/RET/012

| TP Id |  | TP/oneM2M/CSE/DMR/RET/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the RETRIEVE Request of the latest contentInstance target to a <container> resource |  |
| Reference |  | TS-0001 10.2.22.1, TS-0004 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform RETRIEVE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response containing Response Status Code set to 2000 (OK) and Content containing contentInstance resource containing resourceName attribute set to CONTENT_INSTANCE_2 } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/RET/013

| TP Id |  | TP/oneM2M/CSE/DMR/RET/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource under CSEBase |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/013_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/013_AE | TS-0001 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/013_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/013_DLV | TS-0001 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/013_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/013_LCP | TS-0001 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/013_MGC | TS-0001 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/013_NOD | TS-0001 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/013_CSR | TS-0001 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/013_REQ | TS-0001 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/013_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/013_STCL | TS-0001 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/013_STCG | TS-0001 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/013_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/014

| TP Id |  | TP/oneM2M/CSE/DMR/RET/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a resource TARGET_RESOURCE_ADDRESS under CSEBase without having privileges for the RETRIEVE operation |  |
| Reference |  | TS-0001 10.1.2 - item 14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/014_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/014_AE | TS-0001 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/014_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/014_DLV | TS-0001 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/014_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/014_LCP | TS-0001 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/014_MGC | TS-0001 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/014_NOD | TS-0001 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/014_CSR | TS-0001 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/014_REQ | TS-0001 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/014_SCH | TS-0001 9.6.9 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/014_STCL | TS-0001 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/014_STCG | TS-0001 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/014_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/015

| TP Id |  | TP/oneM2M/CSE/DMR/RET/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource (“To” option) under CSEBase |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_ATTRIBUTE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT  AE |
| Notes: TARGET_ATTRIBUTE_ADDRESS is represented as TARGET_RESOURCE_ADDRESS # ATTRIBUTE ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/015_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/015_AE | TS-0001 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/015_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/015_DLV | TS-0001 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/015_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/015_LCP | TS-0001 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/015_MGC | TS-0001 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/015_NOD | TS-0001 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/015_CSR | TS-0001 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/015_REQ | TS-0001 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/015_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/015_STCL | TS-0001 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/015_STCG | TS-0001 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/015_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

Editor’s Note: Update the reference of schedule resource to TS-0001 9.6.9

###### TP/oneM2M/CSE/DMR/RET/016

| TP Id |  | TP/oneM2M/CSE/DMR/RET/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource (“Content” option) under CSEBase |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute and no other attribute } |  | IUT  AE |
| Notes: ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/016_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/016_AE | TS-0001 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/016_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/016_DLV | TS-0001 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/016_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/016_LCP | TS-0001 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/016_MGC | TS-0001 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/016_NOD | TS-0001 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/016_CSR | TS-0001 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/016_REQ | TS-0001 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/016_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/016_STCL | TS-0001 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/016_STCG | TS-0001 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/016_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/017

| TP Id |  | TP/oneM2M/CSE/DMR/RET/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource under CSEBase |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_1 attribute and valid ATTRIBUTE_2 attribute and no other attribute } |  | IUT  AE |
| Notes: ATTRIBUTE_1 indicates common attribute for all RESOURCE_TYPE resources listed in the table below ATTRIBUTE_2 indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/017_ACP | TS-0001 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/017_AE | TS-0001 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/017_CNT | TS-0001 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/017_DLV | TS-0001 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/017_GRP | TS-0001 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/017_LCP | TS-0001 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/017_MGC | TS-0001 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/017_NOD | TS-0001 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/017_CSR | TS-0001 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/017_REQ | TS-0001 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/017_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/017_STCL | TS-0001 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/017_STCG | TS-0001 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/017_SUB | TS-0001 10.2.11.3 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/018

| TP Id |  | TP/oneM2M/CSE/DMR/RET/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource (“single” attribute case) under CSEBase |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource containing no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/018_ACP/LBL | TS-0001 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/018_AE/LBL | TS-0001 10.2.1.2 | 2 (AE) | labels |
| TP/oneM2M/CSE/DMR/RET/018_CNT/LBL | TS-0001 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/018_DLV/LBL | TS-0001 10.2.5.3 | 6 (delivery) | labels |
| TP/oneM2M/CSE/DMR/RET/018_GRP/LBL | TS-0001 10.2.7.3 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/RET/018_LCP/LBL | TS-0001 10.2.10.1.2 | 10 (locationPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/018_MGC/LBL | TS-0001 10.2.8.3 | 12 (mgmtCmd) | labels |
| TP/oneM2M/CSE/DMR/RET/018_NOD/LBL | TS-0001 10.2.14.2 | 14 (node) | labels |
| TP/oneM2M/CSE/DMR/RET/018_CSR/LBL | TS-0001 10.2.2.2 | 16 (remoteCSE) | labels |
| TP/oneM2M/CSE/DMR/RET/018_REQ/LBL | TS-0001 10.2.20.2 | 17 (request) | labels |
| TP/oneM2M/CSE/DMR/RET/018_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/RET/018_STCL/LBL | TS-0001 10.2.15.11 | 21 (statsCollect) | labels |
| TP/oneM2M/CSE/DMR/RET/018_STCG/LBL | TS-0001 10.2.15.3 | 22 (statsConfig) | labels |
| TP/oneM2M/CSE/DMR/RET/018_SUB/LBL | TS-0001 10.2.11.3 | 23 (subscription) | labels |

###### TP/oneM2M/CSE/DMR/RET/019

| TP Id |  | TP/oneM2M/CSE/DMR/RET/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource (“multiple” attributes case) under CSEBase |  |
| Reference |  | TS-0001 10.1.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the <CSEBase> resource containing no ATTRIBUTE_1 attribute and no ATTRIBUTE_2 attribute and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing attributeList element containing ATTRIBUTE_1 name and ATTRIBUTE_2 name } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/019_ACP/LBL/AT | TS-0001 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_AE/LBL/AT | TS-0001 10.2.1.2 | 2 (AE) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_CNT/LBL/AT | TS-0001 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_DLV/LBL/AT | TS-0001 10.2.5.3 | 6 (delivery) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_GRP/LBL/AT | TS-0001 10.2.7.3 | 9 (group) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_LCP/LBL/AT | TS-0001 10.2.10.1.2 | 10 (locationPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_MGC/LBL/AT | TS-0001 10.2.8.3 | 12 (mgmtCmd) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_NOD/LBL/AT | TS-0001 10.2.14.2 | 14 (node) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_CSR/LBL/AT | TS-0001 10.2.2.2 | 16 (remoteCSE) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_REQ/LBL/AT | TS-0001 10.2.20.2 | 17 (request) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_SCH/LBL/AT |  | 18 (schedule) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_STCL/LBL/AT | TS-0001 10.2.15.11 | 21 (statsCollect) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_STCG/LBL/AT | TS-0001 10.2.15.3 | 22 (statsConfig) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_SUB/LBL/AT | TS-0001 10.2.11.3 | 23 (subscription) | labels | announceTo |

###### TP/oneM2M/CSE/DMR/RET/020

| TP Id |  | TP/oneM2M/CSE/DMR/RET/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a RETRIEVE Request target to TARGET_RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to RESULT_CONTENT } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) and no Content } |  | IUT  AE |

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/020_RCN/0 | TS-0004 6.3.4.2.7 | 0 (Nothing) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/2 |  | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/3 |  | 3 (attributes and hierarchichal address) |

###### TP/oneM2M/CSE/DMR/RET/021

| TP Id |  | TP/oneM2M/CSE/DMR/RET/021 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |  |  |  |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |  |  |  |  |  |
| Config Id |  | CF01 |  |  |  |  |  |  |
| PICS Selection |  | PICS_CSE |  |  |  |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |  |  |  |  |  |
| Expected behaviour | Test events |  |  |  | Direction |  |  |  |
|  |  |  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to 1 (attributes) } |  |  | IUT  AE |  |  |
|  |  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references } |  |  | IUT  AE |  |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/RET/021_CNT | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/021_GRP | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/021_ACP | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/021_SCH | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/021_PCH | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/021_SUB | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/022

| TP Id |  | TP/oneM2M/CSE/DMR/RET/022 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes and child resources of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 4 (attributes and child resources) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to 4 (attributes and child resources) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and child resources and no child resource references } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/RET/022_CNT | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/022_GRP | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/022_ACP | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/022_SCH | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/022_PCH | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/022_SUB | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/023

| TP Id |  | TP/oneM2M/CSE/DMR/RET/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes and child resource references of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 5 (attributes and child resource references) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to 5 (attributes and child resource references) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and child resource references } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/RET/023_CNT | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/023_GRP | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/023_ACP | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/023_SCH | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/023_PCH | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/023_SUB | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/024

| TP Id |  | TP/oneM2M/CSE/DMR/RET/024 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only child resource references of TARGET_RESOURCE_ADDRESS resource when Result Content is set to 6 (child resource references) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to 6 (child resource references) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing resourceRefList element containing child resource reference } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/RET/024_CNT | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/024_GRP | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/024_ACP | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/024_SCH | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/024_PCH | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/024_SUB | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/RET/025

| TP Id |  | TP/oneM2M/CSE/DMR/RET/025 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only child resources of TARGET_RESOURCE_ADDRESS resource when Result Content is set to 8 (child resources) |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content and Result Content set to 8 (child resources) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing CHILD_RESOURCE_TYPE resource representation } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/RET/025_CNT | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/025_GRP | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/025_ACP | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/025_SCH | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/025_PCH | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/025_SUB | 23 (subscription) |

##### UPDATE Operation

###### TP/oneM2M/CSE/DMR/UPD/001

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/001_CNT/LBL | TS-0001 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_GRP/LBL | TS-0001 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_ACP/LBL | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_PCH/LBL | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_SUB/LBL | TS-0001 10.2.11.4 | 23 (subscription) | labels |

###### TP/oneM2M/CSE/DMR/UPD/002

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT adds successfully the attribute ATTRIBUTE_NAME to the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing no optional RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 } |  | IUT  AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/002_CNT/LBL | TS-0001 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_GRP/LBL | TS-0001 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_ACP/LBL | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_PCH/LBL | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_SUB/LBL | TS-0001 10.2.11.4 | 23 (subscription) | labels |

###### TP/oneM2M/CSE/DMR/UPD/003

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes successfully the attribute ATTRIBUTE_NAME from the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing an optional RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to NULL } |  | IUT  AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to NULL } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/003_CNT/LBL | TS-0001 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_GRP/LBL | TS-0001 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_ACP/LBL | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_PCH/LBL | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_SUB/LBL | TS-0001 10.2.11.4 | 23 (subscription) | labels |

###### TP/oneM2M/CSE/DMR/UPD/004

| TP Id | TP/oneM2M/CSE/DMR/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the value of the attribute ATTRIBUTE_NAME_1, and creates the ATTRIBUTE_NAME_2 and deletes the ATTRIBUTE_NAME_3 of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 10.1.3 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME_1 attribute set to VALUE_1 and no optional RW ATTRIBUTE_NAME_2 attribute and an optional RW ATTRIBUTE_NAME_3 attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME_1 attibute set to VALUE_2 and ATTRIBUTE_NAME_2 attribute set to VALUE_3 and ATTRIBUTE_NAME_3 attribute set to NULL } | IUT  AE |
|  | then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME_1 attribute set to VALUE_2 and ATTRIBUTE_NAME_2 attribute set to VALUE_3 and ATTRIBUTE_NAME_3 attribute set to NULL } | IUT  AE |
| Editor’s Note: Clarification is required in term of the test objective!! |  |  |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME_1 | ATTRIBUTE_NAME_2 | ATTRIBUTE_NAME_3 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/004_CNT/ET/MNI/LBL | TS-0001 10.2.4.3 | 3 (container) | expirationTime | maxNrOfInstances | labels |
| TP/oneM2M/CSE/DMR/UPD/004_GRP/ET/GN/LBL | TS-0001 10.2.7.4 | 9 (group) | expirationTime | groupName | labels |
| TP/oneM2M/CSE/DMR/UPD/004_ACP/PV/AT/LBL | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | privileges | announceTo | labels |
| TP/oneM2M/CSE/DMR/UPD/004_SCH/SE/AT/LBL |  | 18 (schedule) | scheduleElement | announceTo | labels |
| TP/oneM2M/CSE/DMR/UPD/004_PCH/LBL/ACPI/LBL | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels | accessControlPolicyIDs | labels |
| TP/oneM2M/CSE/DMR/UPD/004_SUB/ET/LBL/EXC | TS-0001 10.2.11.4 | 23 (subscription) | expirationTime | labels | expirationCounter |

###### TP/oneM2M/CSE/DMR/UPD/005

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update an attribute of a TARGET_RESOURCE_ADDRESS resource which does not exist |  |
| Reference |  | TS-0001 10.1.3 – item 15) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/005_CNT/EXC | TS-0001 10.2.4.3 | 3 (container) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/005_GRP/EXC | TS-0001 10.2.7.4 | 9 (group) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/005_ACP/EXC | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/005_SCH/EXC |  | 18 (schedule) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/005_PCH/EXC | TS-0001 10.2.13.4 | 15 (pollingChannel) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/005_SUB/MNI | TS-0001 10.2.11.4 | 23 (subscription) | maxNrOfInstances |

###### TP/oneM2M/CSE/DMR/UPD/014

###### TP/oneM2M/CSE/DMR/UPD/006

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update the attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource without having privileges for the UPDATE operation |  |
| Reference |  | TS-0001 10.1.3 – 16) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having no privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/006_CNT/LBL | TS-0001 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_GRP/LBL | TS-0001 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_ACP/LBL | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_PCH/LBL | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_SUB/LBL | TS-0001 10.2.11.4 | 23 (subscription) | labels |

###### TP/oneM2M/CSE/DMR/UPD/007

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update a non-RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a non-RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE_NAME attribute } |  | IUT  AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/007_CNT/CT | TS-0001 10.2.4.3 | 3 (container) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_GRP/CT | TS-0001 10.2.7.4 | 9 (group) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_ACP/CT | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_SCH/CT |  | 18 (schedule) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_PCH/CT | TS-0001 10.2.13.4 | 15 (pollingChannel) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_SUB/CT | TS-0001 10.2.11.4 | 23 (subscription) | creationTime |

###### TP/oneM2M/CSE/DMR/UPD/008

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete a mandatory RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a mandatory RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to NULL } |  | IUT  AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/008_CNT/ET | TS-0001 10.2.4.3 | 3 (container) | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/008_GRP/ET | TS-0001 10.2.7.4 | 9 (group) | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/008_ACP/ET | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/008_SCH/ET |  | 18 (schedule) | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/008_PCH/ET | TS-0001 10.2.13.4 | 15 (pollingChannel) | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/008_SUB/ET | TS-0001 10.2.11.4 | 23 (subscription) | expirationTime |

###### TP/oneM2M/CSE/DMR/UPD/009

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to update a RW attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource with an UNACCEPTABLE_VALUE |  |
| Reference |  | TS-0001 10.1.3 – item 17) |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a valid RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to UNACCEPTABLE_VALUE } |  | IUT  AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a Response containing Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/009_CNT/LBL | TS-0001 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/009_GRP/LBL | TS-0001 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/009_ACP/LBL | TS-0001 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/009_SCH/LBL |  | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/009_PCH/LBL | TS-0001 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/009_SUB/LBL | TS-0001 10.2.11.4 | 23 (subscription) | labels |

###### TP/oneM2M/CSE/DMR/UPD/010

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the stateTag attribute of a container resource is increased when an update operation has been performed on its child resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CHILD_RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having privileges to perform UPDATE operation on the TARGET_CHILD_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID Content containing CHILD_RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE |
|  | then { the IUT increments the stateTag attribute of container resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/UPD/011

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/BV/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE Request of an existing contentInstance resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.19.4, TS-0004 7.3.2.1 & 7.4.8.2.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing a contentInstance resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing contentInstance representation } |  |  |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  |  |

###### TP/oneM2M/CSE/DMR/UPD/012

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE Request of a latest resource as a direct child of a <container> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.22, TS-0004 7.4.28.2.4 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing a contentInstance resource containing valid creationTime attribute set to the most recent resource creation time } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT does not update the CONTAINER_RESOURCE_ADDRESS/la resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/UPD/013

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the UPDATE Request of an oldest resource as a direct child of a <container> resource with error “OPERATION_NOT_ALLOWED” |  |
| Reference |  | TS-0001 10.2.23, TS-0004 7.4.29.2.4 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing contentInstance resource(s) } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT does not update the CONTAINER_RESOURCE_ADDRESS/ol resource and the IUT sends a Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/UPD/014

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the optional attribute OPTIONAL_ATTRIBUTE of the RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/ACPI | TS-0001 9.6.1.3.2-1 | container | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/ET | TS-0001 9.6.1.3.1-1 | container | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/LBL | TS-0001 9.6.1.3.2-1 | container | labels |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MNI | TS-0001 9.6.6-2 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MBS | TS-0001 9.6.6-2 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MIA | TS-0001 9.6.6-2 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/OR | TS-0001 9.6.6-2 | container | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/ET | TS-0001 9.6.1.3.1-1 | accessControlPolicy | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/LBL | TS-0001 9.6.1.3.2-1 | accessControlPolicy | labels |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/PV | TS-0001 9.6.2-2 | accessControlPolicy | privileges |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/PVS | TS-0001 9.6.2-2 | accessControlPolicy | selfPrivileges |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ACPI | TS-0001 9.6.1.3.2-1 | subscription | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ET | TS-0001 9.6.1.3.1-1 | subscription | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/LBL | TS-0001 9.6.1.3.2-1 | subscription | labels |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ENC | TS-0001 9.6.8-2 | subscription | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/EXC | TS-0001 9.6.8-2 | subscription | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/GPI | TS-0001 9.6.8-2 | subscription | groupID |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NFU | TS-0001 9.6.8-2 | subscription | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/BN | TS-0001 9.6.8-2 | subscription | batchNotify |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/RL | TS-0001 9.6.8-2 | subscription | rateLimit |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/PN | TS-0001 9.6.8-2 | subscription | pendingNotification |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NSP | TS-0001 9.6.8-2 | subscription | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/LN | TS-0001 9.6.8-2 | subscription | latestNotify |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NCT | TS-0001 9.6.8-2 | subscription | notificationContentType |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NEC | TS-0001 9.6.8-2 | subscription | notificationEventCat |
| TP/oneM2M/CSE/DMR/UPD/014_STCG/LBL | TS-0001 9.6.23-2 | statsConfig | labels |
| TP/oneM2M/CSE/DMR/UPD/014_STCG/ACPI | TS-0001 9.6.23-2 | statsConfig | accessControlPolicyIDs |

###### TP/oneM2M/CSE/DMR/UPD/015

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the announceTo optional attribute of the RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 10.2.18.4, TS-0001 9.6.1.3.2-1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource containing RESOURCE_TYPE resource containing announceTo attribute set to NULL and the IUT having announced the PARENT_RESOURCE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { the IUT sends a valid CREATE Request containing To set ANNC_TARGET_CSE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/UPD/015_ACP | accesControlPolicy |
| TP/oneM2M/CSE/DMR/UPD/015_CNT | container |
| TP/oneM2M/CSE/DMR/UPD/015_GRP | group |
| TP/oneM2M/CSE/DMR/UPD/015_LCP | locationPolicy |
| TP/oneM2M/CSE/DMR/UPD/015_MGO | mgmtObj |
| TP/oneM2M/CSE/DMR/UPD/015_NOD | node |
| TP/oneM2M/CSE/DMR/UPD/015_SCH | schedule |

###### TP/oneM2M/CSE/DMR/UPD/016

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the announcedAttribute optional attribute of the RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 10.2.18.8, TS-0001 9.6.1.3.2-1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to NULL and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to OA_ATTRIBUTE } |  | IUT  AE |
|  | then { The IUT sends a valid UPDATE Request containing To set to ANNC_TARGET_CSE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to VALUE } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/016_CNT/MBS | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/MT | group | memberType |
| TP/oneM2M/CSE/DMR/UPD/016_LCP/LOI | locationPolicy | locationContainerID |
| TP/oneM2M/CSE/DMR/UPD/016_MGO/DC | mgmtObj | description |
| TP/oneM2M/CSE/DMR/UPD/016_NOD/HCL | node | hostedCSELink |
| TP/oneM2M/CSE/DMR/UPD/016_SCH/SE | schedule | scheduleElement |

###### TP/oneM2M/CSE/DMR/UPD/017

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the optional attribute OPTIONAL_ATTRIBUTE of the RESOURCE_TYPE resource under CSEBase |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource containing no OPTIONAL_ATTRIBUTE attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT  AE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/017_CSR/LBL | TS-0001 9.6.4-2 | remoteCSE | labels |
| TP/oneM2M/CSE/DMR/UPD/017_CSR/POA | TS-0001 9.6.4-2 | remoteCSE | pointOfAccess |
| TP/oneM2M/CSE/DMR/UPD/017_CSR/NL | TS-0001 9.6.4-2 | remoteCSE | nodeLink |
| TP/oneM2M/CSE/DMR/UPD/017_NOD/LBL | TS-0001 9.6.18-2 | node | labels |
| TP/oneM2M/CSE/DMR/UPD/017_NOD/ACPI | TS-0001 9.6.18-2 | node | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/017_NOD/HCL | TS-0001 9.6.18-2 | node | hostedCSELink |
| TP/oneM2M/CSE/DMR/UPD/017_AE/LBL | TS-0001 9.6.5-2 | AE | labels |
| TP/oneM2M/CSE/DMR/UPD/017_AE/ACPI | TS-0001 9.6.5-2 | AE | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/017_AE/APN | TS-0001 9.6.5-2 | AE | AppName |
| TP/oneM2M/CSE/DMR/UPD/017_AE/POA | TS-0001 9.6.5-2 | AE | pointOfAccess |
| TP/oneM2M/CSE/DMR/UPD/017_AE/OR | TS-0001 9.6.5-2 | AE | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/LBL | TS-0001 9.6.6-2 | container | labels |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/ACPI | TS-0001 9.6.6-2 | container | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/MNI | TS-0001 9.6.6-2 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/MBS | TS-0001 9.6.6-2 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/MIA | TS-0001 9.6.6-2 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/OR | TS-0001 9.6.6-2 | container | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/017_CNT/LI | TS-0001 9.6.6-2 | container | locationID |
| TP/oneM2M/CSE/DMR/UPD/017_ACP/LBL | TS-0001 9.6.2-2 | accessControlPolicy | labels |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/ACPI | TS-0001 9.6.8-2 | subscription | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/LBL | TS-0001 9.6.8-2 | subscription | labels |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/ENC | TS-0001 9.6.8-2 | subscription | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/EXC | TS-0001 9.6.8-2 | subscription | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/GPI | TS-0001 9.6.8-2 | subscription | groupID |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/NFU | TS-0001 9.6.8-2 | subscription | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/BN | TS-0001 9.6.8-2 | subscription | batchNotify |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/RL | TS-0001 9.6.8-2 | subscription | rateLimit |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/PN | TS-0001 9.6.8-2 | subscription | pendingNotification |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/NSP | TS-0001 9.6.8-2 | subscription | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/LN | TS-0001 9.6.8-2 | subscription | latestNotify |
| TP/oneM2M/CSE/DMR/UPD/017_SUB/NEC | TS-0001 9.6.8-2 | subscription | notificationEventCat |
| TP/oneM2M/CSE/DMR/UPD/017_GRP/LBL | TS-0001 9.6.13-2 | group | labels |
| TP/oneM2M/CSE/DMR/UPD/017_GRP/ACPI | TS-0001 9.6.13-2 | group | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/017_GRP/MACP | TS-0001 9.6.13-2 | group | membersAccessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/017_GRP/GN | TS-0001 9.6.13-2 | group | groupName |

###### TP/oneM2M/CSE/DMR/UPD/018

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the attribute MANDATORY_ATTRIBUTE of the RESOURCE_TYPE resource under CSEBase |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing MANDATORY_ATTRIBUTE attribute set to VALUE } |  | IUT  AE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing MANDATORY_ATTRIBUTE attribute set to VALUE } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | MANDATORY_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/018_CSR/ET | TS-0001 9.6.4-2 | remoteCSE | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_CSR/RR | TS-0001 9.6.4-2 | remoteCSE | requestReachability |
| TP/oneM2M/CSE/DMR/UPD/018_NOD/ET | TS-0001 9.6.18-2 | node | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_NOD/NI | TS-0001 9.6.18-2 | node | nodeID |
| TP/oneM2M/CSE/DMR/UPD/018_AE/ET | TS-0001 9.6.5-2 | AE | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_AE/RR | TS-0001 9.6.5-2 | AE | requestReachability |
| TP/oneM2M/CSE/DMR/UPD/018_CNT/ET | TS-0001 9.6.6-2 | container | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_ACP/ET | TS-0001 9.6.2-2 | accessControlPolicy | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_ACP/PV | TS-0001 9.6.2-2 | accessControlPolicy | privileges |
| TP/oneM2M/CSE/DMR/UPD/018_ACP/PVS | TS-0001 9.6.2-2 | accessControlPolicy | selfPrivileges |
| TP/oneM2M/CSE/DMR/UPD/018_SUB/ET | TS-0001 9.6.8-2 | subscription | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_SUB/NU | TS-0001 9.6.8-2 | subscription | notificationURI |
| TP/oneM2M/CSE/DMR/UPD/018_SUB/NCT | TS-0001 9.6.8-2 | subscription | notificationContentType |
| TP/oneM2M/CSE/DMR/UPD/018_GRP/ET | TS-0001 9.6.13-2 | group | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/018_GRP/MNM | TS-0001 9.6.13-2 | group | maxNrOfMembers |
| TP/oneM2M/CSE/DMR/UPD/018_GRP/MID | TS-0001 9.6.13-2 | group | memberIDs |

##### CREATE Operation

###### TP/oneM2M/CSE/DMR/CRE/001

| TP Id | TP/oneM2M/CSE/DMR/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource on the TARGET_RESOURCE_ADDRESS of type PARENT_RESOURCE_TYPE with resource name not provided |  |
| Reference | TS-0001 10.1.1.1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to RESOURCE_TYPE and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing no resourceName attribute } | IUT  AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing Valid resourceName attribute } | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/CB | TS-0001 10.2.4.1 | 3 (container) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/AE | TS-0001 10.2.4.1 | 3 (container) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/CNT | TS-0001 10.2.4.1 | 3 (container) | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/CB | TS-0001 10.2.7.2 | 9 (group) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/AE | TS-0001 10.2.7.2 | 9 (group) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/CB | TS-0001 10.2.21.1 | 1 (accessControlPolicy) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/AE | TS-0001 10.2.21.1 | 1 (accessControlPolicy) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SCH/CB |  | 18 (schedule) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_SCH/AE |  | 18 (schedule) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SCH/SUB |  | 18 (schedule) | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/001_PCH/AE | TS-0001 10.2.13.2 | 15 (pollingChannel) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CB | TS-0001 10.2.11.2 | 23 (subscription) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/AE | TS-0001 10.2.11.2 | 23 (subscription) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CNT | TS-0001 10.2.11.2 | 23 (subscription) | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/ACP | TS-0001 10.2.11.2 | 23 (subscription) | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/SCH | TS-0001 10.2.11.2 | 23 (subscription) | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/GRP | TS-0001 10.2.11.2 | 23 (subscription) | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/001_NOD/CB | TS-0001 9.6.3-1 | 14 (node) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_MGC/CB | TS-0001 9.6.3-1 | 12 (mgmtCmd) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_LCP/CB | TS-0001 9.6.3-1 | 10 (locationPolicy) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_STCG/CB | TS-0001 9.6.3-1 | 22 (statsConfig) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_STCL/CB | TS-0001 9.6.3-1 | 21 (statsCollect) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_MSSP/CB | TS-0001 9.6.3-1 | 11 (m2mServiceSubscriptionProfile) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_ASAR/CB | TS-0001 9.6.3-1 | 19(serviceSubscribedAppRule) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CSR | TS-0001 9.6.4-1 | 23 (subscription) | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/CRE/001_SCH/CSR | TS-0001 9.6.4-1 | 18 (schedule) | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/AEA | TS-0001 9.6.5-1 | 23 (subscription) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/AEA | TS-0001 9.6.5-1 | 3 (container) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/AEA | TS-0001 9.6.5-1 | 9 (group) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/AEA | TS-0001 9.6.5-1 | 1 (accessControlPolicy) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_EVCF/STCG | TS-0001 9.6.23-1 | 7 (eventConfig) | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/STCG | TS-0001 9.6.23-1 | 23 (subscription) | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/CRE/001_CIN/CNT | TS-0001 10.2.4.1 | 4 (contentInstance) | 3 (container) |

###### TP/oneM2M/CSE/DMR/CRE/002

| TP Id | TP/oneM2M/CSE/DMR/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with a non-existing resource name provided by AE |  |
| Reference | TS-0001 10.1.1.1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the AE containing no child resource containing resourceName attribute set to NAME } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource containing resourceName attribute set to NAME } | IUT  AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/002_CNT | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/002_GRP | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/002_ACP | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/002_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/002_PCH | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/002_SUB | TS-0001 10.2.11.2 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/CRE/003

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource with an existing resource name provided by AE |  |
| Reference |  | TS-0001 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the AE containing a child resource having resourceName attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource containing resourceName attribute set to NAME } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5106 (ALREADY_EXISTS) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/003_CNT | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/003_GRP | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/003_ACP | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/003_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/003_PCH | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/003_SUB | TS-0001 10.2.11.2 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/CRE/004

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource when AE has no privileges |  |
| Reference |  | TS-0001 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having no privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource } |  | IUT  AE |
|  | then { the IUT does not create the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/004_CNT | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/004_GRP | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/004_ACP | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/004_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/004_PCH | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/004_SUB | TS-0001 10.2.11.2 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/CRE/005

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of container resource when the RW ATTRIBUTE is provided with an invalid value |  |
| Reference |  | TS-0001 10.2.4.1, TS-0004 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing an invalid RW ATTRIBUTE attribute } |  | IUT  AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/005_MNI | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/005_MBS | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/005_MIA | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxInstanceAge |

###### TP/oneM2M/CSE/DMR/CRE/006

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns a default value to RW ATTRIBUTE in the response of a <container> CREATE Request where no a RW ATTRIBUTE is provided by AE |  |
| Reference |  | TS-0001 10.2.4.1, TS-0004 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT creates the container resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing container resource containing valid ATTRIBUTE attribute } |  | IUT  AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/006_MNI | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/006_MBS | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/006_MIA | TS-0001 9.6.6, TS-0004 7.4.7.1 | maxInstanceAge |

###### TP/oneM2M/CSE/DMR/CRE/007

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with error when the newly created <contentInstance> results in that the field value of attribute currentNrOfInstances exceeds the field value of maxNrOfInstances in the parent container resource. |  |
| Reference |  | TS-0001 10.2.19.2, TS-0004 7.4.8.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource TARGET_RESOURCE_ADDRESS containing currentNrOfInstances attribute set to MAX_NUMBER_OF_INSTANCES and maxNrOfInstances attribute set to MAX_NUMBER_OF_INSTANCES and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 4 (contentInstance) and Content containing contentInstance resource representation } |  | IUT  AE |
|  | then { the IUT does not create the contentInstance resource and the IUT sends a valid Response containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/CRE/008

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Checks that the IUT accepts a newly created contentInstance when the currentByteSize exceeds the field value set in maxByteSize in the parent container resource by removing enough of the oldest <contentInstance> resources to allow the creation of the new <contentInstance> resource.. |  |
| Reference |  | TS-0001 10.2.19.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource TARGET_RESOURCE_ADDRESS containing maxByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and currentByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 4 (contentInstance) and Content containing contentInstance resource with size less than MAX_BYTE_SIZE_OF DATA } |  | IUT  AE |
|  | then { the IUT creates the contentInstance resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/CRE/009

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT increaments and then copies the field value of attribute stateTag in parent container resource when contentInstance resource is created as the direct child of the parent container |  |
| Reference |  | TS-0001 9.6.1.3.2, TS-0004 7.4.7.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource CONTAINER_RESOURCE_ADDRESS containing stateTag attribute and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 4 (contentInstance) and Content containing contentInstance resource representation } |  | IUT  AE |
|  | then { the IUT increments stateTag attribute of CONTAINER_RESOURCE_ADDRESS resource andthe IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/CRE/010

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of a container resource named “la” as a direct child of a container resource |  |
| Reference |  | TS-0001 10.2.22, TS-0004 7.4.28.2.2 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing Container resource containing resourceName attribute set to “la” } |  | IUT  AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/CRE/011

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of a container resource named “ol” as a direct child of a container resource |  |
| Reference |  | TS-0001 10.2.23, TS-0004 7.4.29.2.2 & 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID and Content containing container resource containing resourceName attribute set to “ol” } |  | IUT  AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/CRE/012

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully after a CREATE request an empty content of created resource when the ResultContent set to 0 (Nothing) |  |
| Reference |  | TS-0001 8.1.2, 8.1.3 &10.1.1.1, TS-0004 6.3.4.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Result Content set to 0 (nothing) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and no Content } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/012_CNT | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/012_GRP | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/012_ACP | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/012_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/012_PCH | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/012_SUB | TS-0001 10.2.11.2 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/CRE/013

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully after a CREATE request only hierarchichal address of the created resource when the Result Content is set to 2 (hierarchical address) |  |
| Reference |  | TS-0001 8.1.2, 8.1.3 &10.1.1.1, TS-0004 6.3.4.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Result Content set to 2 (hierarchical address) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing Hierarchical address of the RESOURCE_TYPE resource } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/013_CNT | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/013_GRP | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/013_ACP | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/013_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/013_PCH | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/013_SUB | TS-0001 10.2.11.2 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/CRE/014

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully after a CREATE request only attributes and hierarchichal address of the created resource when Result Content is set 3 (attributes and hierarchichal address) |  |
| Reference |  | TS-0001 8.1.2, 8.1.3 &10.1.1.1, TS-0004 6.3.4.2.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Result Content set to 3 (attributes and hierarchichal address) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing resource element containing Hierarchical address of the RESOURCE_TYPE resource and RESOURCE_TYPE resource representation } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/014_CNT | TS-0001 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/014_GRP | TS-0001 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/014_ACP | TS-0001 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/014_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/014_PCH | TS-0001 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/014_SUB | TS-0001 10.2.11.2 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/CRE/015

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute OPTIONAL_ATTRIBUTE provided |  |
| Reference |  | TS-0001 10.1.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing valid OPTIONAL_ATTRIBUTE attribute } |  | IUT  AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/ACPI | TS-0001 9.6.1.3.2-1 | 3 (container) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/ET | TS-0001 9.6.1.3.1-1 | 3 (container) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/LBL | TS-0001 9.6.1.3.2-1 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/MNI | TS-0001 9.6.6-2 | 3 (container) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/MBS | TS-0001 9.6.6-2 | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/MIA | TS-0001 9.6.6-2 | 3 (container) | maxInstanceAge |
| TP/oneM2M/CSE/DMR/CRE/015_CNT/OR | TS-0001 9.6.6-2 | 3 (container) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/015_CIN/ET | TS-0001 9.6.1.3.1-1 | 4 (contentInstance) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/015_CIN/LBL | TS-0001 9.6.1.3.2-1 | 4 (contentInstance) | labels |
| TP/oneM2M/CSE/DMR/CRE/015_CIN/CR | TS-0001 9.6.7-2 | 4 (contentInstance) | creator |
| TP/oneM2M/CSE/DMR/CRE/015_CIN/CNF | TS-0001 9.6.7-2 | 4 (contentInstance) | contentInfo |
| TP/oneM2M/CSE/DMR/CRE/015_CIN/OR | TS-0001 9.6.7-2 | 4 (contentInstance) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/015_ACP/ET | TS-0001 9.6.1.3.1-1 | 1 (accesControlPolicy) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/015_ACP/LBL | TS-0001 9.6.1.3.2-1 | 1 (accesControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/ACPI | TS-0001 9.6.1.3.2-1 | 23 (subscription) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/ET | TS-0001 9.6.1.3.1-1 | 23 (subscription) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/LBL | TS-0001 9.6.1.3.2-1 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/ENC | TS-0001 9.6.8-2 | 23 (subscription) | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/EXC | TS-0001 9.6.8-2 | 23 (subscription) | expirationCounter |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/GPI | TS-0001 9.6.8-2 | 23 (subscription) | groupID |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/NFU | TS-0001 9.6.8-2 | 23 (subscription) | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/BN | TS-0001 9.6.8-2 | 23 (subscription) | batchNotify |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/RL | TS-0001 9.6.8-2 | 23 (subscription) | rateLimit |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/PSN | TS-0001 9.6.8-2 | 23 (subscription) | preSubscriptionNotify |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/PN | TS-0001 9.6.8-2 | 23 (subscription) | pendingNotification |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/NSP | TS-0001 9.6.8-2 | 23 (subscription) | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/LN | TS-0001 9.6.8-2 | 23 (subscription) | latestNotify |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/NCT | TS-0001 9.6.8-2 | 23 (subscription) | notificationContentType |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/NEC | TS-0001 9.6.8-2 | 23 (subscription) | notificationEventCat |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/CR | TS-0001 9.6.8-2 | 23 (subscription) | creator |
| TP/oneM2M/CSE/DMR/CRE/015_SUB/SU | TS-0001 9.6.8-2 | 23 (subscription) | subscriberURI |
| TP/oneM2M/CSE/DMR/CRE/015_STCG/CR | TS-0001 9.6.23-2 | 22 (statsConfig) | creator |

###### TP/oneM2M/CSE/DMR/CRE/016

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute announceTo provided |  |
| Reference |  | TS-0001 10.2.18.4, TS-0001 9.6.1.3.2-1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to ANNC_TARGET_CSE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE | PICS |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/016_ACP | 1 (accesControlPolicy) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/016_CNT | 3 (container) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/016_CIN | 4 (contentInstance) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/016_GRP | 9 (group) |  |
| TP/oneM2M/CSE/DMR/CRE/016_LCP | 10 (locationPolicy) |  |
| TP/oneM2M/CSE/DMR/CRE/016_MGO | 13 (mgmtObj) |  |
| TP/oneM2M/CSE/DMR/CRE/016_NOD | 14 (node) |  |
| TP/oneM2M/CSE/DMR/CRE/016_SCH | 18 (schedule) |  |

###### TP/oneM2M/CSE/DMR/CRE/017

| TP Id | TP/oneM2M/CSE/DMR/CRE/017 |  |  |  |
| --- | --- | --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute announcedAttribute provided |  |  |  |
| Reference | TS-0001 10.2.18.4, TS-0001 9.6.1.3.2-1 & TS-0004 7.3.3.10 |  |  |  |
| Config Id | CF02 |  |  |  |
| PICS Selection | PICS_CSE |  |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announcedAttribute attribute set to OA_ATTRIBUTE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } | IUT  AE |  |
|  |  | then { The IUT sends a valid CREATE Request containing To set to ANNC_TARGET_CSE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes and OA_ATTRIBUTE attribute set to VALUE } | IUT  CSE |  |

| TP Id | RESOURCE_TYPE | PICS |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/017_CNT | 3 (container) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/017_CIN | 4 (contentInstance) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/017_GRP | 9 (group) |  |
| TP/oneM2M/CSE/DMR/CRE/017_LCP | 10 (locationPolicy) |  |
| TP/oneM2M/CSE/DMR/CRE/017_MGO | 13 (mgmtObj) |  |
| TP/oneM2M/CSE/DMR/CRE/017_NOD | 14 (node) |  |
| TP/oneM2M/CSE/DMR/CRE/017_SCH | 18 (schedule) |  |

###### TP/oneM2M/CSE/DMR/CRE/018

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a CREATE request when the ResultContent is set to RESULT_CONTENT |  |
| Reference |  | TS-0001 8.1.2 & TS-0004 7.3.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to RESULT_CONTENT } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/4 | TS-0004 6.3.4.2.7 | 4 (attributes and child resources) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/5 |  | 5 (attributes and child resources references) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/6 |  | 6 (child resrouces references) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/8 |  | 8 (child resources) |

###### TP/oneM2M/CSE/DMR/CRE/019

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource on the TARGET_RESOURCE_ADDRESS of type announced PARENT_RESOURCE_TYPE |  |
| Reference |  | TS-0001 9.6.5, |  |
| Config Id |  | CF04 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the CSE and the IUT having created the PARENT_RESOURCE_TYPE and the Originator having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing Resource Type set to RESOURCE_TYPE and To set to PARENT_RESOURCE_ADDRESS and From set to CSE-ID and Content containing RESOURCE_TYPE resource containing no resourceName attribute } |  | IUT  CSE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing Valid resourceName attribute } |  | IUT  CSE |

| TP Id | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/019_CNTA/AEA | 9.6.5-1 | (containerAnnc) | (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/019_GRPA/AEA | 9.6.5-1 | (groupAnnc) | (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/019_ACPA/AEA | 9.6.5-1 | (accessControlPolicyAnnc) | (AEannc) |

###### TP/oneM2M/CSE/DMR/CRE/020

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the CREATE Request of a contentInstance resource with error “NOT_ACCEPTABLE” when contentSize exceeds maxByteSize |  |
| Reference |  | TS-0001 10.2.19, TS-0004 7.4.7.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource having maxByteSize attribute set to MAX_BYTE_SIZE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to container_RESOURCE_ADDRESS and From set to AE_ID and Content containing contentInstance resource containing valid content with size larger than MAX_BYTE_SIZE } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/CRE/021

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates the currentByteSize attribute in a PARENT_RESOURCE_TYPE when a new RESOURCE_TYPE is created. |  |
| Reference |  | TS-0001 10.2.19.2, TS-0004 7.4.8.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource at PARENT_RESOURCE_ADDRESS containing maxByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and currentByteSize attribute set to 0 and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Content containing RESOURCE_TYPE resource containing valid content attribute with size equal to DATA_SIZE } |  | IUT  AE |
|  | then { the IUT increments currentByteSize attribute of PARENT_RESOURCE_ADDRESS resource by DATA_SIZE and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/021_CNT_CIN | container | contentInstance |

##### DELETE Operation

###### TP/oneM2M/CSE/DMR/DEL/001

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a RESOURCE_TYPE resource |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/001_CNT | TS-0001 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/001_GRP | TS-0001 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/001_ACP | TS-0001 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/001_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/001_PCH | TS-0001 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/001_SUB | TS-0001 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/001_STCG | TS-0001 10.2.15.5 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/DEL/001_CIN | TS-0001 10.2.4.1 | 4 (contentInstance) |

###### TP/oneM2M/CSE/DMR/DEL/002

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS without having privileges for the DELETE operation |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having no privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT does not delete the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/002_CNT | TS-0001 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/002_GRP | TS-0001 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/002_ACP | TS-0001 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/002_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/002_PCH | TS-0001 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/002_SUB | TS-0001 10.2.11.5 | 23 (subscription) |

###### TP/oneM2M/CSE/DMR/DEL/003

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT does not delete the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/DEL/004

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when AE tries to delete a child resource of a RESOURCE_TYPE resource when this resource has been already deleted |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS and the AE having deleted the TARGET_RESOURCE_ADDRESS resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/004_CNT | TS-0001 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/004_GRP | TS-0001 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/004_ACP | TS-0001 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/004_SCH |  | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/004_PCH | TS-0001 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/004_SUB | TS-0001 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/004_STCG | TS-0001 10.2.15.5 | 22 (statsConfig) |

###### TP/oneM2M/CSE/DMR/DEL/005

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the stateTag attribute of a container resource is increased when a child resource is deleted |  |
| Reference |  | TS-0001 10.1.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing a child resource and the AE having privileges to perform DELETE operation on the TARGET_CHILD_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_CHILD_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT increments the stateTag attribute of the container resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/DEL/006

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT decreases the field value of attribute currentNrOfInstances and currentByteSize of parent container when a latest contentInstance resource is deleted successfully |  |
| Reference |  | TS-0001 10.2.19.5, TS-0004 7.4.8.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing currentNrOfInstances attribute set to NR_OF_INSTANCES and currentByteSize attribute set to BYTE_SIZE_OF_DATA and a contentInstance resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT decreases the currentNrOfInstances attribute of container resource and the IUT decreases the currentByteSize attribute of container resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

Editor’s Note: Need to clarify if needed to split into two separate test purposes.

###### TP/oneM2M/CSE/DMR/DEL/007

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the DELETE Request of the latest contentInstance resource target to a container resource |  |
| Reference |  | TS-0001 10.2.22.2, TS-0004 7.4.28.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform DELETE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the CONTENT_INSTANCE_2_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/DEL/008

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the DELETE request of a latest resource in a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 10.2.22.2, TS-0004 7.4.28.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/la and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/DEL/009

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the DELETE Request of the oldest contentInstance resource target to a container resource |  |
| Reference |  | TS-0001 10.2.23.2, TS-0004 7.4.29.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_IN_PROFILE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform DELETE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT deletes the CONTENT_INSTANCE_1_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DMR/DEL/010

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the DELETE request of an oldest resource in a container resource that has no direct child contentInstance resources. |  |
| Reference |  | TS-0001 10.2.23.2, TS-0004 7.4.29.2.5 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no contentInstance resources } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

#### Subscription and Notification (SUB)

##### CREATE Operation

###### TP/oneM2M/CSE/SUB/CRE/001

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the target subscribed-to RESOURCE_TYPE resource is not subscribable. |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT having a resource PARENT_RESOURCE_ADDRESS not being subscribable RESOURCE_TYPE resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE-ID and Content containing subscription resource representation } |  | IUT  AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 5203 (TARGET_NOT_SUBSCRIBABLE) and no Content } |  | IUT  AE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/SUB/CRE/001_CIN | TS-0001 9.6.7 | 4 (contentInstance) |
| TP/oneM2M/CSE/SUB/CRE/001_SUB | TS-0001 9.6.8 | 23 (subscription) |

###### TP/oneM2M/CSE/SUB/CRE/002

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the originator does not have privileges for retrieving the subscribed-to resource. |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute pointing to accessControlPolicy resource containing privileges attribute containing accessControlRule attribute containing accessControlOriginators attribute indicating AE-ID and accessControlOperations attribute not indicating RETRIEVE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE-ID and Content containing subscription resource representation } |  | IUT  AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) and no Content } |  | IUT  AE |

###### TP/oneM2M/CSE/SUB/CRE/003

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT cannot send the Notify request to the notificationURI |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT not being able to send the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and From set to AE-ID and Content containg subscription resource containing notificationURI attribute set to NOTI_URI_ADDRESS } |  | IUT  AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 5204 (SUBSCRIPTION_VERIFICATION_INITIATION_FAILED) and no Content } |  | IUT  AE |

###### TP/oneM2M/CSE/SUB/CRE/004

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT stores Originator ID in the notification creator attribute when a <SUBSCRIPTION> creation request which needs verification is received and the notificationURI is not the Originator. |  |
| Reference |  | TS-0001 10.2.12.1 & TS-0004 7.4.9.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Resource Type set to 23 (subscription) and Content containing subscription resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid NOTIFY Request containing Content containing notification message containing verificationRequest attribute set to TRUE and creator attribute set to AE1_RESOURCE_ADDRESS } |  | IUT  AE2 |

###### TP/oneM2M/CSE/SUB/CRE/005

| TP Id |  | TP/oneM2M/CSE/SUB/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when notificationEventType attribute is set to "Create_of_Direct_Child_Resource" and a create operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 3 (Create_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing CHILD_RESOURCE_TYPE resource containing resourceName attribute set to NAME } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and the IUT sends a NOTIFY Request contaning Content containing Notification message containing notificationEvent attribute containing representation attribute containing CHILD_RESOURCE_TYPE resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/CRE/006

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a subscription resource with the eventNotificationCriteria attribute which includes a CONDITION_TAG condition provided |  |
| Reference |  | TS-0001 9.6.8 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing Resource Type set to subscription and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing supscription resource containing eventNotificationCriteria attribute containing valid CONDITION_TAG condition } |  | IUT  AE |
|  | then { the IUT creates the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

| TP Id | Reference | CONDITION_TAG |
| --- | --- | --- |
| TP/oneM2M/CSE/SUB/CRE/006_CRB | TS-0001 9.6.8 | createdBefore |
| TP/oneM2M/CSE/SUB/CRE/006_CRA | TS-0001 9.6.8 | createdAfter |
| TP/oneM2M/CSE/SUB/CRE/006_MS | TS-0001 9.6.8 | modifiedSince |
| TP/oneM2M/CSE/SUB/CRE/006_US | TS-0001 9.6.8 | unmodifiedSince |
| TP/oneM2M/CSE/SUB/CRE/006_STS | TS-0001 9.6.8 | stateTagSmaller |
| TP/oneM2M/CSE/SUB/CRE/006_STB | TS-0001 9.6.8 | stateTagBigger |
| TP/oneM2M/CSE/SUB/CRE/006_EXB | TS-0001 9.6.8 | expireBefore |
| TP/oneM2M/CSE/SUB/CRE/006_EXA | TS-0001 9.6.8 | expireAfter |
| TP/oneM2M/CSE/SUB/CRE/006_SZA | TS-0001 9.6.8 | sizeAbove |
| TP/oneM2M/CSE/SUB/CRE/006_SZB | TS-0001 9.6.8 | sizeBelow |
| TP/oneM2M/CSE/SUB/CRE/006_ET | TS-0001 9.6.8 | eventType |
| TP/oneM2M/CSE/SUB/CRE/006_OM | TS-0001 9.6.8 | operationMonitor |
| TP/oneM2M/CSE/SUB/CRE/006_ATR | TS-0001 9.6.8 | attribute |

##### DELETE Operation

###### TP/oneM2M/CSE/SUB/DEL/001

| TP Id |  | TP/oneM2M/CSE/SUB/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the AE2_RESOURCE_ADDRESS of the subscriberURI attribute when the SUBSCRIPTION_RESOURCE_ADDRESS <subscription> resource is deleted |  |
| Reference |  | TS-0001 9.6.8 & TS-0004 7.5.1.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription resource containing subscriberURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE1 containing To set to SUBSCRIPTION_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing subscriptionDeletion attribute set to TRUE and subscriptionReference attribute set to AE2_RESOURCE_ADDRESS } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/DEL/002

| TP Id |  | TP/oneM2M/CSE/SUB/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventType attribute is set to "Delete_of_Resource" and an delete operation has been performed the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 2 (Delete_of_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/DEL/003

| TP Id |  | TP/oneM2M/CSE/SUB/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when eventType attribute is set to "Delete_of_Direct_Child_Resource" and an delete operation has been performed on a child resource of the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8-3 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing a CHILD_RESOURCE_ADDRESS child resource and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 4 (Delete_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE1 containing To set to CHILD_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing CHILD_RESOURCE_TYPE resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

##### UPDATE Operation

###### TP/oneM2M/CSE/SUB/UPD/001

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends Notify request to the subscriber resource when an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 10.2.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/UPD/002

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to “modified attributes” and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 2 (modified attributes) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/UPD/003

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to “ResourceID” and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 3 (ResourceID) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing subscribed-to ResourceID } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/UPD/004

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT decreases the expirationCounter attribute of a subscription resource when the Hosting CSE of the subscribed-to resource successfully sends the notification request to subscriber resource(s) |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to MAX_NUMBER_NOTIFICATION and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT decreases the expirationCounter attribute of the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/UPD/005

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes the subscription resource when the the expirationCounter meets zero |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to 1 and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT decreases the expirationCounter attribute of the subscription resource and the IUT deletes the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/GMG/UPD/003

###### TP/oneM2M/CSE/SUB/UPD/006

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT doesn’t send a Notify request to the subscriber resource when the “attribute” condition tag of eventNotificationCriteria attribute is set to LIST_OF_ATTRIBUTE and doesn’t contain the ATTRIBUTE_NAME attribute updated. |  |
| Reference |  | TS-0001 9.6.8 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to LIST_OF_ATTRIBUTE containing no ATTRIBUTE_NAME attribute and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT doesn’t send a NOTIFY Request } |  | IUT  AE1 |

###### TP/oneM2M/CSE/SUB/UPD/007

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends aggregated notification to the subscriber resource when the number value of the batchNotify attribute is set to GROUP_LIMIT and when this number have been reached |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing number attribute set to GROUP_LIMIT and duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (GROUP_LIMIT – 1) valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_N } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing aggregatedNotification message containing GROUP_LIMIT notification messages } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/UPD/008

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the number value of the batchNotify attribute is set to GROUP_LIMIT and when this number has been reached |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing number attribute set to GROUP_LIMIT and duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (GROUP_LIMIT – 1) valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_N } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing the latest notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

###### TP/oneM2M/CSE/SUB/UPD/009

| TP Id |  | TP/oneM2M/CSE/SUB/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Notify request to the subscriber resource when the “attribute” condition tag of eventNotificationCriteria attribute is set to ATTRIBUTE_NAME and an update operation has been performed on the subscribed-to resource |  |
| Reference |  | TS-0001 9.6.8 & TS-0004 7.5.1.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and the IUT sends a NOTIFY Request contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE1 IUT  AE2 |

##### NOTIFY Operation

###### TP/oneM2M/CSE/SUB/NTF/001

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response containing Response Status Code set to 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) } |  | IUT  AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) and no Content } |  | IUT  AE |

###### TP/oneM2M/CSE/SUB/NTF/002

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE |  |
| Reference |  | TS-0001 10.2.11.2 & TS-0004 7.4.8.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response containing Response Status Code set to 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) } |  | IUT  AE |
|  | then { the IUT does not create the subscription resource and the IUT sends a valid Response containing Response Status Code set to 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) and no Content } |  | IUT  AE |

###### TP/oneM2M/CSE/SUB/NTF/003

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends aggregated notification to the subscriber resource when the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing number attribute set to GROUP_LIMIT and duration attrinbute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received NUMBER_OF_REQUEST valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and NUMBER_OF_REQUEST < GROUP_LIMIT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the timer expires after the TIME_LIMIT duration value } |  | IUT  AE1 |
|  | then { The IUT sends a NOTIFY Request contaning Content containing aggregatedNotification message containing NUMBER_OF_REQUEST notification messages } |  | IUT  AE2 |

###### TP/oneM2M/CSE/SUB/NTF/004

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference |  | TS-0001 10.2.12.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing number attribute set to GROUP_LIMIT and duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received NUMBER_OF_REQUEST valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and NUMBER_OF_REQUEST < GROUP_LIMIT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the timer expires after the TIME_LIMIT duration value } |  | IUT  AE1 |
|  | then { The IUT sends a NOTIFY Request contaning Content containing the latest notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT  AE2 |

###### TP/oneM2M/CSE/SUB/NTF/005

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the latest cached notification after IUT escapes from connectionless state when the pendingNotification is set to 1 (sendLatest) |  |
| Reference |  | TS-0001 9.6.8 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and pendingNotification attribute set to 1 (sendLatest) and the AE2 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT being in the “connectionless state” with AE2 and the IUT having frequently received from AE1 a valid UPDATE Requests to subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT escapes from the “connectionless state” } |  | NA |
|  | then { The IUT sends a valid NOTIFY Request contaning Content containing the latest notification message } |  | IUT  AE2 |

###### TP/oneM2M/CSE/SUB/NTF/006

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the aggregated notification of cached notification messages after IUT escapes from connectionless state when the pendingNotification is set to 2 (sendAllPending) |  |
| Reference |  | TS-0001 9.6.8 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and pendingNotification attribute set to 2 (sendAllPending) and the AE2 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT being in the “connectionless state” with AE2 and the IUT having frequently received from AE1 a valid UPDATE Requests to subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT escapes from the “connectionless state” } |  | NA |
|  | then { The IUT sends a valid NOTIFY Request contaning Content containing the aggregated notification message } |  | IUT  AE2 |

#### Group Management (GMG)

##### CREATE Operation

###### TP/oneM2M/CSE/GMG/CRE/001

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the group resource when member ID exceed max number of members. |  |
| Reference |  | TS-0001 10.2.7.2 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to LIST_OF_MEMBER_RESOURCE_IDS and maxNrOfMembers attribute set to MAX_NR_MEMBERS less than the number of memerIDs set in memberIDs attribute } |  | IUT  AE |
|  | then { the IUT does not create the group resource and the IUT sends a valid Response containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/CRE/002

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the group resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference |  | TS-0001 10.2.7.2 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the sub-group members being in the “inital state” and the AE not having privileges to perform RETRIEVE operation on the sub-group members } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing group resource representation containing memberIDs attribute containing sub-group members } |  | IUT  AE |
|  | then { the IUT does not create the group resource and the IUT sends a valid valid Response containing Response Status Code set to 5105 (NO_PRIVILEGE) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/CRE/003

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects the presence of duplicate member IDs during the creation of the <group> resource and removes the duplicate member IDs prior to creation of the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having created a resource at MEMBER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS, MEMBER_RESOURCE_ADDRESS } |  | IUT  AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS } |  | IUT  AE |

Editor’s Note: Resource type of TARGET_RESOURCE_ADDRESS can be <CSEBase>, <AE>, <remoteCSE> and resource type of MEMBER_RESOURCE_ADDRESS can be any oneM2M resource type.

###### TP/oneM2M/CSE/GMG/CRE/004

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT validates the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS and memberType attribute set to RESOURCE_TYPE } |  | IUT  AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) Content containing group resource containing memberTypeValidated attribute set to TRUE } |  | IUT  AE |

TP/oneM2M/CSE/GMG/CRE/005

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is SET_MIXED. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS and memberType attribute set to RESOURCE_TYPE_2 and consistencyStrategy attribute set to SET_MIXED } |  | IUT  AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberType attribute set to MIXED } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/CRE/006

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_MEMBER,. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and consistencyStrategy attribute set to ABANDON_MEMBER } |  | IUT  AE |
|  | then { the IUT creates the group resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/CRE/007

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the group resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_GROUP. |  |
| Reference |  | TS-0001 10.2.7.2, TS-0004 7.4.14.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 9 (group) and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and consistencyStrategy attribute set to ABANDON_GROUP } |  | IUT  AE |
|  | then { the IUT does not create the group resource and the IUT sends a valid Response containing Response Status Code set to 4110 (GROUP_MEMBER_TYPE_INCONSISTENT) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/CRE/008

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT generates a request primitive for each resource in memberIDs with no relative address appended to it. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE and memberType attribute set to AE AE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType set to AE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID and Resource Type set to 3 (container) and Content containing container resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/CRE/009

| TP Id |  | TP/oneM2M/CSE/GMG/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT generates a request primitive for each resource in memberIDs with a relative address appended to it. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE and memberType attribute set to AE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to container and resourceName attribute set to NAME and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to container and resourceName attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt/NAME and From set to AE-ID and Content containing contentInstance resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1 and Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

##### UPDATE Operation

###### TP/oneM2M/CSE/GMG/UPD/001

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the update of the group resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference |  | TS-0001 10.2.7.4 & TS-0004 7.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform UPDATE operation and the sub-group members being in the “inital state” and the AE not having privileges to perform RETRIEVE operation on the sub-group members } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing group resource representation } |  | IUT  AE |
|  | then { the IUT sends a valid valid Response containing Response Status Code set to 5105 (NO_PRIVILEGE) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/002

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects the presence of duplicate member IDs during an update of the <group> resource and removes the duplicate member IDs prior to updating the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 and the IUT having created a <group> resource at GROUP_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and the AE having privileges to perform UPDATE operation on the GROUP_RESOURCE_ADDRESS and the IUT having created a resource at MEMBER_RESOURCE_ADDRESS_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to GROUP_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/003

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT validates the resource type during an UPDATE of the <group> resource when memberType attribute is not ‘mixed’. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing group resource containing memberTypeValidated attribute set to TRUE } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/004

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the <group> resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is SET_MIXED. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to SET_MIXED and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberType attribute set to MIXED } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/005

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the group resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_MEMBER. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing group resource containing memberTypeValidated attribute set to TRUE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/006

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles unsuccessful validation of the resource type during the creation of the group resource when memberType attribute is not ‘mixed’ and the consistencyStrategy attribute is ABANDON_GROUP. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_GROUP and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/007

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects when the number of memberIDs exceeds the limitation of maxNrOfMembers. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and maxNrOfMembers attribute set to 2 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_3 having resourceType attribute set to RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_3 } |  | IUT  AE |
|  | then { the IUT does not update the group resource and the IUT sends a valid Response containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/008

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT detects when the value provided for maxNrOfMembers attribute is less than the value of the currentNrOfMembers attribute. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2, MEMBER_RESOURCE_ADDRESS_3 and maxNrOfMembers attribute set to 3 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_3 having resourceType attribute set to RESOURCE_TYPE_1 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing maxNrOfMembers attribute set to 2 } |  | IUT  AE |
|  | then { the IUT does not update the group resource and the IUT sends a valid Response containing Response Status Code set to 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/009

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles validation of the resource type during an UPDATE of the <group> resource when memberIDs contains a sub-group on a temporarily unreachable Hosting CSE. |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the remoteCSE having a group resource at MEMBER_RESOURCE_ADDRESS_2 containing memberType attribute set to RESOURCE_TYPE_1 and the remoteCSE having onlineStatus set to False } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing group resource containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |
|  | then { the IUT updates the group resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) Content containing group resource containing memberTypeValidated attribute set to FALSE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |

###### TP/oneM2M/CSE/GMG/UPD/010

| TP Id |  | TP/oneM2M/CSE/GMG/UPD/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT handles validation of the resource type during an UPDATE of the group resource when memberIDs contains a sub-group on a a previously unreachable Hosting CSE that has become reachable |  |
| Reference |  | TS-0001 10.2.7.4, TS-0004 7.4.14.2.4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to FALSE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the AE having a subscription to TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the remoteCSE having a group resource at MEMBER_RESOURCE_ADDRESS_2 containing memberType attribute set to RESOURCE_TYPE_2 and the remoteCSE having onlineStatus set to False } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from remoteCSE containing To set to REMOTE_CSE_ADDRESS and From set to CSE-ID and Content containing remoteCSE resource containing onlineStatus attribute set to TRUE } |  | IUT  AE |
|  | then { the IUT sends a NOTIFY Request containing Content containing group resource containing memberTypeValidated attribute set to TRUE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 } |  | IUT  AE |

##### RETRIEVE Operation

###### TP/oneM2M/CSE/GMG/RET/001

| TP Id |  | TP/oneM2M/CSE/GMG/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT generates a request primitive for each resource in memberIDs with a relative address appended to fanOutPoint that includes a virtual resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform RETRIEVE operation and memberType attribute set to 2 (AE) and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType set to 2 (AE) and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType set to 2 (AE) and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to container and resourceName attribute set to NAME and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to container and resourceName attribute set to NAME } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to ARGET_RESOURCE_ADDRESS/fopt/NAME/la and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1/NAME/la, Response for MEMBER_RESOURCE_ADDRESS2/NAME/la } |  | IUT  AE |

##### BASIC OPERATION

###### TP/oneM2M/CSE/GMG/001

| TP Id |  | TP/oneM2M/CSE/GMG/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT allows a <group>/fanoutPoint OPERATION when the Originator has OPERATION_PERMISSION specified in membersAccessControlPolicyIDs attribute in the group resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE and Content containing aggregatedResponse containing Response for MEMBER_RESOURCE_ADDRESS1, Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/GMG/001_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/GMG/001_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/GMG/001_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/GMG/001_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/GMG/002

| TP Id |  | TP/oneM2M/CSE/GMG/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT denies a <group>/fanOutPoint OPERATION when the Originator does not have OPERATION_PERMISSION specified in membersAccessControlPolicyIDs in the group resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/002_CRE | CREATE |
| TP/oneM2M/CSE/GMG/002_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/002_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/002_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/003

| TP Id |  | TP/oneM2M/CSE/GMG/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT allows a <group>/fanoutPoint OPERATION when the Originator has OPERATION_PERMISSION specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the <group> resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to empty and accessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set RESPONSE_STATUS_CODE Content containing aggregatedResponse message containing Response for MEMBER_RESOURCE_ADDRESS1, Response for MEMBER_RESOURCE_ADDRESS2 } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/GMG/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/GMG/003_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/GMG/003_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/GMG/003_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/GMG/004

| TP Id |  | TP/oneM2M/CSE/GMG/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT rejects a <group>/fanOutPoint OPERATION when the Originator does not have OPERATION_PERMISSION specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the group resource. |  |
| Reference |  | TS-0001 10.2.7.7 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to empty and accessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS/fopt and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/004_CRE | CREATE |
| TP/oneM2M/CSE/GMG/004_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/004_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/004_DEL | DELETE |

#### Discovery (DIS)

##### RETRIEVE Operation

###### TP/oneM2M/CSE/DIS/001

| TP Id |  | TP/oneM2M/CSE/DIS/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list all discovered resource addresses |  |
| Reference |  | TS-0001 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation } |  |  |

###### TP/oneM2M/CSE/DIS/002

| TP Id |  | TP/oneM2M/CSE/DIS/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully appropriate list of discovered resource when the filter criteria is provided in the request |  |
| Reference |  | TS-0001 8.1.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing filterUsage set to 1 (Discovery Criteria) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation } |  | IUT  AE |

###### TP/oneM2M/CSE/DIS/003

| TP Id |  | TP/oneM2M/CSE/DIS/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns the empty address list when no result matching with filter criteria is discovered |  |
| Reference |  | TS-0001 8.1.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform RETRIEVE operation and FILTER_CRITERIA_CONDITIONS not being applicable to the IUT } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and no Content } |  |  |

###### TP/oneM2M/CSE/DIS/004

| TP Id |  | TP/oneM2M/CSE/DIS /004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully a list of discovered resource addresses with Non-hierarchical addressing form when the Discovery Result Type is provided in the request |  |
| Reference |  | TS-0001 8.1.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Discovery Result Type set to 2 (unstructured) and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing URIList representation containing unstructured addresses of discovered resources } |  |  |

###### TP/oneM2M/CSE/DIS/005

| TP Id |  | TP/oneM2M/CSE/DIS/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the discovery requests to the resource TARGET_RESOURCE_ADDRESS when AE has no privilege to perform the discovery request for the resource TARGET_RESOURCE_ADDRESS |  |
| Reference |  | TS-0001 10.2.6.2 & 7.2.3.14, TS-0004 7.3.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

###### TP/oneM2M/CSE/DIS/006

| TP Id |  | TP/oneM2M/CSE/DIS/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE sends requests to discover the resource TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference |  | TS-0001 10.2.6.2, TS-0004 7.3.3.14 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a containe resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) } |  |  |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  |  |

###### TP/oneM2M/CSE/DIS/007

| TP Id |  | TP/oneM2M/CSE/DIS/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds the originator with an error when the originator sends a request including an invalid format of filter criteria to discover the resource TARGET_RESOURCE_ADDRESS |  |
| Reference |  | TS-0001 10.2.6, TS-0004 7.3.3.6 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions |  | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |
| Expected behaviour | Test events |  | Expected behaviour |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS not being valid format } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE) } |  | IUT  AE |

###### TP/oneM2M/CSE/DIS/008

| TP Id |  | TP/oneM2M/CSE/DIS/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds the originator with an error when the originator sends a request to discover the resource TARGET_RESOURCE_ADDRESS including two conflicted filter criteria of different type FILTER_CRITERIA_CONDITION_1 and FILTER_CRITERIA_CONDITION_2 (E.G. createBefore < createdAfter) |  |
| Reference |  | TS-0001 6.2.5, TS-0004 7.3.3.17 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform RETRIEVE operation } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Filter Criteria containing filterUsage set to 1 (Discovery Criteria) and having conflict conditions } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE) } |  | IUT  AE |

| TP Id | REQ Reference | FILTER_CRITERIA _CONDITION_1 | FILTER_CRITERIA _CONDITION_2 |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DIS/008_CRB/CRA | REQ-0004-07241 | createdBefore | createdAfter |
| TP/oneM2M/CSE/DIS/008_MS/US | REQ-0004-07242 | unmodifiedSince | modifiedAfter |
| TP/oneM2M/CSE/DIS/008_STS/STB | REQ-0004-07243 | stateTagSmaller | stateTagBigger |
| TP/oneM2M/CSE/DIS/008_EXB/EXA | REQ-0004-07244 | expireBefore | expireAfter |

###### TP/oneM2M/CSE/DIS/009

| TP Id |  | TP/oneM2M/CSE/DIS/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns the empty address list when resources match the filter criteria but they do not include DISCOVERY permission. |  |
| Reference |  | TS-0001 9.6.2.3 & 10.2.6 & TS-0004 7.2.3.13 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS not allowing the AE to perform DISCOVERY operation and FILTER_CRITERIA_CONDITIONS being applicable to the IUT } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing filterUsage set to 1 (Discovery Criteria) } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and no Content } |  | IUT  AE |

#### Location (LOC)

##### CREATE Operation

###### TP/oneM2M/CSE/LOC/CRE/001

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the locationPolicy resource when locationSource attribute is set by Network Based and Location Server is not reachable. |  |
| Reference |  | TS-0001 8.5. & 10.2.10 & TS-0004 7.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having no connection to Location Server and the IUT having a CSEBase resource allowing the AE to perform CREATE operation and } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE-ID and Content containing locationPolicy resource representation } |  | IUT  AE |
|  | then { the IUT does not create the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

###### TP/oneM2M/CSE/LOC/CRE/002

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of locationPolicy resource with locationSource attribute and locationUpdatePeriod attribute provided. |  |
| Reference |  | TS-0001 10.2.10.1, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 10 (locationPolicy) and From set to AE_ID and Content containing locationSource attribute set to 1 (Network_based) and valid locationUpdatePeriod attribute and valid locationTargetID attribute and valid locationServer attribute } |  | IUT  AE |
|  | then { the IUT creates the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/LOC/CRE/003

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of locationPolicy resource when AE has no privilege to perform the creation request for the locationPolicy resource |  |
| Reference |  | TS-0001 10.2.10.1, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the AE having no privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 10 (locationPolicy) and From set to AE_ID and Content containing locationSource attribute set to 1 (Network_based) and valid locationUpdatePeriod attribute and valid locationTargetID attribute and valid locationServer attribute } |  | IUT  AE |
|  | then { the IUT does not create the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

###### TP/oneM2M/CSE/LOC/CRE/004

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when receiving a creation request of locationPolicy resource target to the TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference |  | TS-0001 10.2.10.1, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 10 (locationPolicy) and From set to AE_ID and Content containing locationSource attribute set to 1 (Network_based) and valid locationUpdatePeriod attribute and valid locationTargetID attribute and valid locationServer attribute } |  | IUT  AE |
|  | then { the IUT does not create the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

###### TP/oneM2M/CSE/LOC/CRE/005

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT creates a container resource with CONTAINER_NAME included in the associated locationPolicy resource that has been created resulted in a valid locationPolicy create request. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.2.1, TS-0004 7.3.3.3 & 7.4.11.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having no created container having resourceName set to CONTAINER_NAME and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerName attribute set to CONTAINER_NAME and locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD and locationSource attribute set to 1 (Network_based) and locationTargetID attribute set to LOCATION_TARGET_ID and locationServer attribute set to LOCATION_SERVER_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and ResourceType set to 3 (container) and From set to AE_ID and Content containing resourceName attribute set to CONTAINER_NAME and locationID attribute set to LOCATIONPOLICY_ADDRESS } |  | IUT  AE |
|  | then { the IUT creates the container resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing container resource representation containing resourceName attribute set to CONTAINER_NAME and locationID attribute set to resourceID of the associated locationPolicy resource } |  | IUT  AE |
| Note that the IUT sets the locationContainerID attribute in locationPolicy resource to the resourceID of the created <container> resource |  |  |  |

###### TP/oneM2M/CSE/LOC/CRE/006

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT creates a subscription resource with SUBSCRIPTION_NAME provided by the originator as a child resource of the container linked with a locationPolicy resource. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.2.1, TS-0004 7.3.3.3 & 7.4.11.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing resourceID set to NOTI_URI_ADDRESS and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy> containing locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD and locationSource set to 1 (Network_based) and locationTargetID set to LOCATION_TARGET_ID and locationServer set to LOCATION_SERVER_ADDRESS and locationContainerID set to CONTAINER_ADDRESS and and the IUT having created a resource CONTAINER_ADDRESS of type <container> containing locationID set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and ResourceType set to 23 (subscription) and From set to AE_ID and Content containing notificationURI attribute set to NOTI_URI_ADDRESS and eventNotificationCriteria attribute containing notificationEventType attribute set to 3 (Create_of_Direct_Child_Resource) and notificationContentType attribute set to 1 (All Attributes) } |  | IUT  AE |
|  | then { the IUT creates the subscription resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing subscription resource representation } |  | IUT  AE |
| Note that the IUT sets the locationContainerID attribute in <locationPolicy> resource to the resourceID of the created <container> resource |  |  |  |

###### TP/oneM2M/CSE/LOC/CRE/007

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts the creation of a locationPolicy resource with attribute locationSource set to Device-based and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having location positioning modules and the originator being a PX_ASN_AE and the IUT being a PX_ASN_CSE and the IUT having registered the PX_ASN_AE and the ASN-AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 10 (locationPolicy) and From set to AE_ID and Content containing locationSource attribute set to 2 (Device_based) and locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD } |  | IUT  ASN-AE |
|  | then { the IUT creates the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing locationPolicy resource representation } |  | IUT  ASN-AE |

###### TP/oneM2M/CSE/LOC/CRE/008

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a locationPolicy resource with attribute locationSource set to Sharing-based |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the originator being a PX_ADN_AE and the IUT being a PX_MN_CSE and the ADN-AE being no location positioning modules deployed and the ADN-AE being no Network-based positioning capabilities and the IUT having registered the PX_ADN_AE and the ADN-AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 10 (locationPolicy) and From set to AE_ID and Content containing locationSource attribute set to 3 (Sharing_based) } |  | IUT  ADN-AE |
|  | then { the IUT creates the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing locationPolicy resource representation } |  | IUT  ADN-AE |

###### TP/oneM2M/CSE/LOC/CRE/009

| TP Id |  | TP/oneM2M/CSE/LOC/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of the locationPolicy resource when locationSource attribute is set to Network-Based and neither locationTargetID nor locationServer is provided. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to 10 (locationPolicy) and From set to AE_ID and Content containing locationSource attribute set to 1 (Network_based) } |  | IUT  AE |
|  | then { the IUT does not create the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

##### UPDATE Operation

###### TP/oneM2M/CSE/LOC/UPD/001

| TP Id |  | TP/oneM2M/CSE/LOC/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects AE to update the locationPolicy resource identified with LOCATION_ID when receiving an update request containing an invalid attribute |  |
| Reference |  | TS-0001 10.2.10.1.3, TS-0004 7.3.3.4 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type locationPolicy containing an optional RW attribute locationUpdatePeriod and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing locationUpdatePeriod attribute set to INVALID_LOCATION_UPDATE_PERIOD } |  | IUT  AE |
|  | then { the IUT does not update the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

##### RETRIEVE Operation

###### TP/oneM2M/CSE/LOC/RET/001

| TP Id |  | TP/oneM2M/CSE/LOC/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with a container resource containing attribute locationID set to resourceID of the associated locationPolicy resource |  |
| Reference |  | TS-0001 10.2.10.1.1, TS-0004 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to TARGET_RESOURCE_ADDRESS and locationSource attribute set to 1 (Network_based) and locationTargetID attribute set to LOCATION_TARGET_ID and locationServer attribute set to LOCATION_SERVER_ADDRESS and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type container containing locationID attribute set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing container resource containing locationID attribute set to LOCATIONPOLICY_ADDRESS } |  | IUT  AE |

###### TP/oneM2M/CSE/LOC/RET/002

| TP Id |  | TP/oneM2M/CSE/LOC/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notification message including the location Information to the originator when the originator sends a retrieve request target to the <latest> resource of CONTAINER_ADDRESS container associated with a <locationPolicy> resource |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1 & 10.2.10.2.2, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to CONTAINER_ADDRESS and locationUpdatePeriod attribute set to ZERO (or NULL) and locationSource attribute set to LOCATION_SOURCE and the IUT having created a resource CONTAINER_ADDRESS of type <container> containing locationID attribute set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource CONTAINER_ADDRESS and the AE having created a resource of type subscription as a child of the resource CONTAINER_ADDRESS of type container } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE Request from AE containing To set to CONTAINER_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid NOTIFY Response containing Response Status Code set to 2000 (OK) and Content containing notification attribute containing notificationEvent attribute containing representation attribute containing contentInstance resource containing location information } |  | IUT  AE |

| TP Id | Reference | LOCATION_SOURCE |
| --- | --- | --- |
| TP/oneM2M/CSE/LOC/RET/002_LOS/1 | TS-0001 9.6.10 & 10.2.10 TS-0004 7.4.11 & 6.3.4.2.3 | 1 (Network_based) |
| TP/oneM2M/CSE/LOC/RET/002_LOS/2 |  | 2 (Device_based) |
| TP/oneM2M/CSE/LOC/RET/002_LOS/3 |  | 3 (Sharing_based) |

###### TP/oneM2M/CSE/LOC/RET/003

| TP Id |  | TP/oneM2M/CSE/LOC/RET/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with a <locationPolicy> resource containing attribute locationContainerID set to resourceID of the associated <container> resource |  |
| Reference |  | TS-0001 10.2.10.1.1, TS-0004 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type locationPolicy containing locationContainerID attribute set to CONTAINER_ADDRESS and locationSource attribute set to 1 (Network_based) and locationTargetID attribute set to LOCATION_TARGET_ID and locationServer attribute set to LOCATION_SERVER_ADDRESS and the IUT having created a resource CONTAINER_ADDRESS of type container containing locationID attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing locationPolicy representation containing locationContainerID attribute set to CONTAINER_ADDRESS } |  | IUT  AE |

###### TP/oneM2M/CSE/LOC/RET/004

| TP Id |  | TP/oneM2M/CSE/LOC/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sets the locationStatus attribute with the error status code in the locationPolicy resource when the location information of the requested node is failed to be retrieved due to NSE. |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.4.11.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type locationPolicy and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing locationPolicy representation containing locationStatus attribute set to LOCATION_PROTOCOL_SPECIFIC_RESULT_CODE } |  | IUT  AE |
| Note: the error status code LOCATION_PROTOCOL_SPECIFIC_RESULT_CODE is location protocol specific, e.g. in OMA Mobile Location Protocol the reslt code is set to 6 indicating POSITION METHOD FAILURE. Note that the failure is caused by the NSE such as location server. |  |  |  |

###### TP/oneM2M/CSE/LOC/RET/005

| TP Id |  | TP/oneM2M/CSE/LOC/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error message “NOT_FOUND” when receiving a retrieve request target to an non-existing locationPolicy resource |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.2, TS-0004 7.3.3.2 & 7.4.11.2.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not created LOCATIONPOLICY_ADDRESS of type locationPolicy under the AE a resource and the IUT having privileges to perform RETRIEVE operation on locationPolicy resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a RETRIEVE Request from AE containing To set to LOCATIONPOLICY_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } |  | IUT  AE |

##### NOTIFY Operation

###### TP/oneM2M/CSE/LOC/NTF/001

| TP Id |  | TP/oneM2M/CSE/LOC/NTF/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notification message including the location Information to the originator with frequency of POSITIVE_UPDATE_PERIOD set in field of locationUpdatePeriod |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1, TS-0004 7.3.3.3 & 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the originator being a ADN-AE and the IUT being a MN-CSE and the ADN-AE being no location positioning modules deployed and the ADN-AE being no Network-based positioning capabilities and the IUT having registered the ADN-AE and the IUT having created under the ADN-AE a child resource ORIGINATOR_LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to ORIGINATOR_CONTAINER_ADDRESS and locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD and locationSource attribute set to 3 (Sharing-based) and the IUT having created under the ADN-AE a child resource ORIGINATOR_CONTAINER_ADDRESS of type container containing locationID attribute set to ORIGINATOR_LOCATIONPOLICY_ADDRESS and the IUT having created a resource of type subscription as a child resource of ORIGINATOR_CONTAINER_ADDRESS of type container and the IUT having registered a Node being a minimum hop with the originator compared to other nodes registered with the hosting CSE and the IUT having created under the Node a child resource CLOSEST_NODE_CONTAINER_ADDRESS of type <container> containing a latest contentInstance resource containing the latest location information and the IUT having privileges to perform RETRIEVE operation on the resource CLOSEST_NODE_CONTAINER_ADDRESS of type of container } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a latest resource of container from the CLOSEST_NODE_CONTAINER_ADDRESS containing latest location information and the IUT creates a contentInstance as a child resource of ORIGINATOR_CONTAINER_ADDRESS of type <container> containing the location information that retrieved } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing notification representation containing notificationEvent attribute containing representation attribute containing contentInstance resource containing location information } |  | IUT  ADN-AE |

###### TP/oneM2M/CSE/LOC/NTF/002

| TP Id |  | TP/oneM2M/CSE/LOC/NTF/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notification message including the location Information to the originator with frequency of POSITIVE_UPDATE_PERIOD set in field of locationUpdatePeriod |  |
| Reference |  | TS-0001 8.5.2 & 10.2.10.1.1 & 10.2.10.2.2, TS-0004 7.4.11.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to CONTAINER_ADDRESS and locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD and locationSource attribute set to LOCATION_SOURCE and the IUT having created a resource CONTAINER_ADDRESS of type container containing locationID attribute set to LOCATIONPOLICY_ADDRESS and the AE having created a resource of type subscription as a child of the resource CONTAINER_ADDRESS of type container and the IUT having privileges to retrieve the location information from the NSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from NSE containing location-protocol-specific-result-code of positioning procedure execution and location-response containing location information } |  | IUT  NSE |
|  | then { the IUT transforms the location-response into a oneM2M Response primitive and sends a valid NOTIFY Response containing Response Status Code set to 2000 (OK) and Content containing notification representation containing notificationEvent attribute containing representation attribute containing contentInstance representation } |  | IUT  AE |
| Note that: the attribute location-protocol-specific-result-code and location-response are location acquisition protocol specific and the exact name of these two attributes have to follow the specification of corresponding location protocol e.g. OMA Mobile Location Protocol. |  |  |  |

Table Referenced values for parameter LOCATION_SOURCE

| TP Id | Reference | LOCATION_SOURCE |
| --- | --- | --- |
| TP/oneM2M/CSE/LOC/NTF/010_LOS/1 | TS-0001 9.6.10 & 10.2.10 TS-0004 7.4.11 & 6.3.4.2.3 | 1 (Network_based) |
| TP/oneM2M/CSE/LOC/NTF/010_LOS/2 |  | 2 (Device_based) |
| TP/oneM2M/CSE/LOC/NTF/010_LOS/3 |  | 3 (Sharing_based) |

Table Referenced values for parameter location-protocol-specific-result-code and location-response

| location-protocol-specific-result-code | location-response | Location protocol | Reference |
| --- | --- | --- | --- |
| [Result code] | [Content] | OMA Mobile Location Protocol | Not Present |
| [HTTP Response code] | <TerminalLocation> representation containing attributes address indicating the address of targeted node where location information is retrieved, locationRetrievalStatus indicating the status of retrieval of the location information for the target node , and currentLocation containing the actual location data of the target node | OMA RESTful NetAPI | TS-0004 Annex G |
| [HTTP Response code] | <SubscriptionNotification> representation | OMA RESTful NetAPI | Location Server Open RESTful API Specification |

#### Device Management (DMG)

#### Communication Management and Delivery Handling(CMDH)

#### Security(SEC)

##### CREATE Operation

###### TP/oneM2M/CSE/SEC/ACP/CRE/001

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a accessControlPolicy resource with privileges attribute having multiple access control rules |  |
| Reference |  | TS-0001 9.6.2-2 & TS-0001 10.2.21 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing privileges attribute containing accessControlRule attribute containing ACCESS_CONTROL_RULE_1 and ACCESS_CONTROL_RULE_2 } |  | IUT  AE |
|  | then { the IUT creates the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/SEC/ACP/CRE/002

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a accessControlPolicy resource with selfPrivileges attribute having multiple access control rules |  |
| Reference |  | TS-0001 9.6.2-2 & TS-0001 10.2.21 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing selfPrivileges attribute containing accessControlRule attribute containing ACCESS_CONTROL_RULE_1 and ACCESS_CONTROL_RULE_2 } |  | IUT  AE |
|  | then { the IUT creates the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/SEC/ACP/CRE/003

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a accessControlPolicy resource with privileges attribute having empty rules |  |
| Reference |  | TS-0001 9.6.2 & TS-0001 10.2.21 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing privileges attribute set to empty list } |  | IUT  AE |
|  | then { the IUT creates the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE |

###### TP/oneM2M/CSE/SEC/ACP/CRE/004

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the creation of a accessControlPolicy resource with selfPrivileges attribute having empty rules |  |
| Reference |  | TS-0001 9.6.2 & TS-0001 10.2.21 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing accessControlPolicy resource containing selfPrivileges attribute set to empty list } |  | IUT  AE |
|  | then { the IUT does not create the accessControlPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  AE |

##### UPDATE Operation

###### TP/oneM2M/CSE/SEC/ACP/UPD/001

| TP Id | TP/oneM2M/CSE/SEC/ACP/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the accessControlPolicyIDs attribute of the <AE> resource when the AE has privileges for UPDATE operation in any selfPrivileges of the <accessControlPolicy> resources which this attribute originally indicates. |  |
| Reference | TS-0001 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing accessControlPolicyID attribute set to ACP_ID_1 and the AE having privileges to perform UPDATE operation on the AE_RESOURCE_ADDRESS in selfPrivileges } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID Content containing AE resource containing accessControlPolicyIDs attribute set to ACP_ID_2 } | IUT  AE |
|  | then { the IUT updates the accessControlPolicyIDs attribute of AE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing AE resource containing accessControlPolicyID attribute set to ACP_ID_2 } | IUT  AE |

###### TP/oneM2M/CSE/SEC/ACP/UPD/002

| TP Id | TP/oneM2M/CSE/SEC/ACP/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries an UPDATE operation on the accessControlPolicyIDs attribute without having privileges for such operation in any selfPrivileges of the <accessControlPolicy> resources which this attribute originally indicates. |  |
| Reference | TS-0001 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing accessControlPolicyID attribute set to ACP_ID_1 and the AE having no privileges to perform UPDATE operation on the AE_RESOURCE_ADDRESS in selfPrivileges } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID Content containing AE resource containing accessControlPolicyID attribute set to ACP_ID_2 } | IUT  AE |
|  | then { the IUT does not update the accessControlPolicyIDs attribute of AE resource and the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } | IUT  AE |

##### BASIC OPERATION

###### TP/oneM2M/CSE/SEC/ACP/001

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when an ORIGINATOR tries an OPERATION without having privileges for performing the OPERATION on the TARGET_RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the ORIGINATOR having no privileges to perform an OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to TARGET_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT  ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  ORIGINATOR |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/001_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/001_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/001_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/001_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/002

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on its <AE> resource which has multiple accessControlPolicyID attribute |  |
| Reference |  | TS-0001 9.6.1.3.2-1 & TS-0001 9.6.2.1-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <Container> resource containing accessControlPolicyID attribute containing ACP_ID_1 and ACP_ID_2 and the AE having privileges to perform an OPERATION on itself given by ACP_ID_2 only } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/002_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/002_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/002_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/002_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/003

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when an allowed DOMAIN tries an OPERATION on a AE_RESOURCE_TYPE resource. |  |
| Reference |  | TS-0001 9.6.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the DOMAIN having privileges to perform an OPERATION on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to DOMAIN } |  | IUT  DOMAIN |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  DOMAIN |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/003_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/003_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/003_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/004

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when any ORIGINATOR tries an OPERATION on a AE_RESOURCE_TYPE resource for which all originators are allowed |  |
| Reference |  | TS-0001 9.6.2.1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and any originator (*) having privileges to perform an OPERATION on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to ORIGINATOR } |  | IUT  ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  ORIGINATOR |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/004_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/004_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/004_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/004_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/005

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource and that the request is within accessControlTimeWindow context. |  |
| Reference |  | TS-0001 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within TIME_WINDOW } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID within TIME_WINDOW } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/005_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/005_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/005_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/005_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/006

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlTimeWindow context. |  |
| Reference |  | TS-0001 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within TIME_WINDOW } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID out of TIME_WINDOW } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id |  | OPERATION |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/006_CRE | CREATE |  |
| TP/oneM2M/CSE/SEC/ACP/006_RET | RETRIEVE |  |
| TP/oneM2M/CSE/SEC/ACP/006_UPD | UPDATE |  |
| TP/oneM2M/CSE/SEC/ACP/006_DEL | DELETE |  |

###### TP/oneM2M/CSE/SEC/ACP/007

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource and that the request is within accessControlLocationRegion context. |  |
| Reference |  | TS-0001 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a child locationPolicy resource containing locationContainerID attribute set to CONTAINER_ID and a child container resource containing locationID attribute set to LOCATION_POLICY_ID and contentInstance resource containing content attribute set to LOCATION and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within LOCATION_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and LOCATION within LOCATION_RESTRICTION } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/007_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/007_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/007_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/007_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/008

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlLocationRegion context. |  |
| Reference |  | TS-0001 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within LOCATION_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID out of LOCATION_RESTRICTION } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/008_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/008_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/008_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/008_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/009

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on a TARGET_RESOURCE_TYPE resource and that the request is within accessControlIpAddresses context. |  |
| Reference |  | TS-0001 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within IP_ADDRESS_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and AE_IP_ADDRESS within IP_ADDRESS_RESTRICTION } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/009_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/009_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/009_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/009_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/010

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlIpAddresses context. |  |
| Reference |  | TS-0001 9.6.2.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the AE_RESOURCE_ADDRESS within IP_ADDRESS_RESTRICTION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and AE_IP_ADDRESS out of IP_ADDRESS_RESTRICTION } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  AE |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/010_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/010_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/010_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/010_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/011

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when the AE tries an OPERATION on its child <container> resource whose accessControlPolicyID attribute is not set and AE has privileges for such OPERATION on its associated accessControlPolicy resource |  |
| Reference |  | TS-0001 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <Container> resource containing no accessControlPolicyID attribute and the AE having privileges to perform an OPERATION on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE_ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  AE |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/011_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/011_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/011_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/011_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/012

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when an ORIGINATOR tries an OPERATION on a <Container> resource with no accessControlPolicyID associated and such ORIGINATOR having no privileges for performing the OPERATION on the <Container> resource. |  |
| Reference |  | TS-0001 9.6.1.3.2-1 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE, PICS_ACP_SUPPORT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <Container> resource containing no accessControlPolicyID attribute and the ORIGINATOR having no privileges to perform an OPERATION on the <Container> resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to CONTAINER_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  | IUT  ORIGINATOR |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ACCESS_DENIED) } |  | IUT  ORIGINATOR |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/012_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/012_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/012_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/012_DEL | DELETE |

#### Resource Announcement (ANNC)

##### BASIC Operation

###### TP/oneM2M/CSE/ANNC/001

| TP Id |  | TP/oneM2M/CSE/ANNC/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT provides confirmation of the announcement of a resource to the originator of the initiating request after a successful resource announcement procedure |  |
| Reference |  | TS-0001 10.2.18.1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource containing RESOURCE_TYPE resource containing announcedTo attribute set to NULL and the IUT having announced the PARENT_RESOURCE resource and the AE having privileges to perform an OPERATION operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and The IUT having sent a valid CREATE Request containing To set ANNC_PARENT_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 2001 (CREATED) containing valid RESOURCE_TYPE announced variant resource representation } |  | IUT  CSE |
|  | then { The IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |

| TP Id | RESOURCE_TYPE | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/ANNC/001_ACP/UPD | 1 (accesControlPolicy) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_ACP/CRE | 1 (accesControlPolicy) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_CNT/UPD | 3 (container) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_CNT/CRE | 3 (container) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_GRP/UPD | 9 (group) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_GRP/CRE | 9 (group) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_LCP/UPD | 10 (locationPolicy) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_LCP/CRE | 10 (locationPolicy) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_MGO/UPD | 13 (mgmtObj) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_MGO/CRE | 13 (mgmtObj) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_NOD/UPD | 14 (node) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_NOD/CRE | 14 (node) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_SCH/UPD | 18 (schedule) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_SCH/CRE | 18 (schedule) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_CIN/CRE | 4 (contentInstance) | CREATE | 2001 (CREATED) |

##### CREATE Operation

###### TP/oneM2M/CSE/ANNC/CRE/001

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the accessControlPolicyIDs attribute is announced when accessControlPolicyIDs attribute of the original resource is not present |  |
| Reference |  | TS-0001 10.2.18.4, 9.6.26.3-1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing container resource containing no accessControlPolicyIDs attribute and no anouncedAttribute attribute and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_ANNC_RESOURCE_ADDRESS and Content containing containerAnnc resource containing MA (mandatory) attributes and valid accessControlPolicyIDs attribute } |  | IUT  CSE |

###### TP/oneM2M/CSE/ANNC/CRE/002

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and IUT having not been announced to announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the IUT having not been announced nor registered to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request containing Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid CREATE Request containing To set to ANNC_TARGET_CSE_ADDRESS and Content containing remoteCSEAnnc resource } |  | IUT  CSE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/002_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/002_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/002_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/002_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/CRE/003

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Host CSE of original resource removes an announcement target CSE from the announceTo attribute if it does not return successful CREATE response for the announced resource (for a CREATE initiating resource announcement) |  |
| Reference |  | 10.2.18.4 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the container resource and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having received a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE-ID and Content containing contentInstance resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and The IUT having sent a valid CREATE Request containing To set to ANNC_CONTAINER_RESOURCE_ADDRESS and Content containing contentInstanceAnnc resource containing MA (mandatory) attributes } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  CSE |
|  | then { the IUT creates the contentInstance resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing contentInstance resource containing announceTo attribute set to NULL } |  | IUT  AE |

###### TP/oneM2M/CSE/ANNC/CRE/004

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and its parent resource having been announced to the announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE resource and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to ANNC_PARENT_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/004_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/004_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/004_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/CRE/004_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/004_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/004_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/004_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/004_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/CRE/005

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and IUT having been already announced to announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource and the IUT having been announced to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/005_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/005_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/005_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/CRE/005_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/005_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/005_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/005_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/005_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/CRE/006

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and IUT has registered and created a <remoteCSE> resource to the announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource and the IUT having registered and created a remoteCSE resource to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/006_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/006_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/006_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/CRE/006_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/006_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/006_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/006_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/006_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/CRE/007

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and IUT having not been announced to announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource and the IUT having not been announced nor registered to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS and the IUT having received a valid CREATE Request containing Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid CREATE Request containing To set to ANNC_TARGET_CSE_ADDRESS and Content containing remoteCSEAnnc resource representation } |  | IUT  CSE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/007_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/007_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/007_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/CRE/007_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/007_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/007_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/007_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/007_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/CRE/008

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo and announcedAttribute optional attributes are provided and its parent resource having been announced to the announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.6 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announcedAttribute attribute set to OA_ATTRIBUTE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to ANNC_PARENT_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes and OA_ATTRIBUTE attribute set to VALUE } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/008_ACP/MBS | 1 (accesControlPolicy) | maxByteSyze |
| TP/oneM2M/CSE/ANNC/CRE/008_CNT/CNF | 3 (container) | contentInfo |
| TP/oneM2M/CSE/ANNC/CRE/008_CIN/MT | 4 (contentInstance) | memberType |
| TP/oneM2M/CSE/ANNC/CRE/008_GRP/LOI | 9 (group) | locationContainerID |
| TP/oneM2M/CSE/ANNC/CRE/008_LCP/DC | 10 (locationPolicy) | description |
| TP/oneM2M/CSE/ANNC/CRE/008_MGO/HCL | 13 (mgmtObj) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/CRE/008_NOD/SE | 14 (node) | scheduleElement |

###### TP/oneM2M/CSE/ANNC/CRE/009

| TP Id |  | TP/oneM2M/CSE/ANNC/CRE/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the labels attribute is announced when such attribute is present in the original resource |  |
| Reference |  | TS-0001 10.2.18.4, 9.6.26.3-1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing container resource containing no anouncedAttribute attribute and labels attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_ANNC_RESOURCE_ADDRESS and Content containing containerAnnc resource containing MA (mandatory) attributes and labels attribute set to VALUE } |  | IUT  CSE |

##### RETRIEVE Operation

###### TP/oneM2M/CSE/ANNC/RET/001

| TP Id |  | TP/oneM2M/CSE/ANNC/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Host of <resourceAnnc> returns the <resourceAnnc> representation in a RETRIEVE response |  |
| Reference |  | TS-0001 10.2.18.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having RESOURCE_TYPE announced variant resource containing link attribute set to remoteCSE/ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the RESOURCE_TYPE announced variant resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ANNC_ADDRESS and From set to AE-ID } |  | IUT  AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE announced variant resource representation } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/RET/001_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/RET/001_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/RET/001_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/RET/001_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/RET/001_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/RET/001_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/RET/001_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/RET/001_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/RET/002

| TP Id |  | TP/oneM2M/CSE/ANNC/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Host of <resourceAnnc> returns the original <resource> representation from the original resource hosting CSE in a RETRIEVE response when result content is set to “original-resource” |  |
| Reference |  | TS-0001 10.2.18.2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the remoteCSE having created RESOURCE_TYPE and the IUT having RESOURCE_TYPE announced variant resource containing link attribute set to remoteCSE/ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the RESOURCE_TYPE announced variant resource Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation and the IUT having received a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ANNC_ADDRESS and From set to AE-ID and Result Content set to 7 (original-resource) and no Content and the IUT having sent a a valid RETRIEVE Request from AE containing To set to remoteCSE/ORIGINAL_RESOURCE_ADDRESS and From set to AE-ID and no Content } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation } |  | IUT  CSE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource representation } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/RET/002_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/RET/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/RET/002_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/RET/002_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/RET/002_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/RET/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/RET/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/RET/002_SCH | 18 (schedule) |

##### UPDATE Operation

###### TP/oneM2M/CSE/ANNC/UPD/001

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and its parent resource having been announced to the announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource containing RESOURCE_TYPE resource containing announceTo attribute set to NULL and the IUT having announced the PARENT_RESOURCE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set ANNC_PARENT_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/001_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/001_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/001_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/001_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/001_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/001_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/001_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/UPD/002

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT initiates de-announcement of the RESOURCE_TYPE resource when a CSE-ID is deleted from the announceTo optional attribute |  |
| Reference |  | TS-0001 10.2.18.5 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announceTo attribute set to NULL } |  | IUT  AE |
|  | then { The IUT sends a valid DELETE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/002_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/002_GRP | 4 (group) |
| TP/oneM2M/CSE/ANNC/UPD/002_LCP | 9 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/002_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/UPD/003

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT provides confirmation of resource de-announcement to the originator of the initiating request after a successful resource de-announcement procedure originator (for an UPDATE initiating resource de-announcement) |  |
| Reference |  | TS-0001 10.2.18.3 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content attribute containing RESOURCE_TYPE resource containing announceTo attribute set to NULL and the IUT having sent a valid DELETE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |
|  | then { the IUT updates RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing announceTo attribute set to NULL } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/003_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/003_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/003_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/003_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/003_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/003_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/003_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/UPD/004

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/004 |  |
| --- | --- | --- | --- |
| Test objective |  | HOST CSE of original resource returns INVALID response if announcedAttributes contains an attribute that is not marked OA. |  |
| Reference |  | TS-0001 10.2.18.1, 10.2.18.4, TS-0004 7.3.1.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE1 and the IUT having registered the AE and the IUT having a resource of type <RESOURCE_TYPE> at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to remoteCSE1/ANNOUNCED_PARENT_URI and the remoteCSE1 having onlineStatus set to True } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives an invalid UPDATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announcedAttributes attribute set to pi } |  | IUT  AE |
|  | then { the IUT sends a Response containing Response Status Code set to 2004 (UPDATED) announceTo attribute set to remoteCSE1/ANNOUNCED_RESOURCE_URI } |  | IUT  AE |

###### TP/oneM2M/CSE/ANNC/UPD/005

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the labels attribute is de-announced when such attribute is deleted from the original resource |  |
| Reference |  | TS-0001 10.2.18.4, 9.6.26.3-1 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and valid labels attribute and the IUT having announced the container resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE-ID and Content containing container resource containing labels attribute set to NULL } |  | IUT  AE |
|  | then { The IUT sends a valid UPDATE Request containing To set to CONTAINER_ANNC_RESOURCE_ADDRESS and Content containing containerAnnc resource containing labels attribute set to NULL } |  | IUT  CSE |

###### TP/oneM2M/CSE/ANNC/UPD/006

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and IUT having been already announced to announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the IUT having been announced to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/006_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/006_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/006_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/006_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/006_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/006_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/006_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/UPD/007

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/007 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and IUT has registered and created a <remoteCSE> resource to the announcement target CSE |  |
| Reference |  | TS-0001 10.2.18.4 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the IUT having registered and created a remoteCSE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid CREATE Request containing To set to REMOTE_CSE_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing MA (mandatory) attributes } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/007_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/007_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/007_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/007_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/007_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/007_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/007_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/UPD/008

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT provides confirmation of the announcement of an attribute to the originator of the initiating request after a successful attribute announcement procedure |  |
| Reference |  | TS-0001 10.2.18.6 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to NULL and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to OA_ATTRIBUTE and the IUT having sent a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to VALUE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 2004 (UPDATED) containing valid RESOURCE_TYPE announced variant resource representation } |  | IUT  CSE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to OA_ATTRIBUTE } |  | IUT  AE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/008_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/008_GRP/MT | 9 (group) | memberType |
| TP/oneM2M/CSE/ANNC/UPD/008_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/008_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/008_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/008_SCH/SE | 18 (schedule) | scheduleElement |

###### TP/oneM2M/CSE/ANNC/UPD/009

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully RESOURCE_TYPE announced variant resource when a new OA attribute is provided to announcedAttribute attribute of the original resource |  |
| Reference |  | TS-0001 10.2.18.8 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to NULL and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to OA_ATTRIBUTE } |  | IUT  AE |
|  | then { The IUT sends a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to VALUE } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/009_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/009_GRP/MT | 9 (group) | memberType |
| TP/oneM2M/CSE/ANNC/UPD/009_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/009_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/009_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/009_SCH/SE | 18 (schedule) | scheduleElement |

###### TP/oneM2M/CSE/ANNC/UPD/010

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the RESOURCE_TYPE announced variant resource when a MA attribute of its original resource is updated. |  |
| Reference |  | TS-0001 10.2.18.10 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing expirationTime attribute set to VALUE_1 and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing expirationTime attribute set to VALUE_2 } |  | IUT  AE |
|  | then { The IUT sends a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing expirationTime attribute set to VALUE_2 } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/010_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/010_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/010_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/010_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/010_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/010_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/010_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/UPD/011

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/011 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the RESOURCE_TYPE announced variant resource when a optional attribute present in the announcedAttribute attribute of its original resource is updated. |  |
| Reference |  | TS-0001 10.2.18.10 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE_1 and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to OA_ATTRIBUTE and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE_2 } |  | IUT  AE |
|  | then { The IUT sends a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to VALUE_2 } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/011_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/011_GRP/MT | 9 (group) | memberType |
| TP/oneM2M/CSE/ANNC/UPD/011_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/011_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/011_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/011_SCH/SE | 18 (schedule) | scheduleElement |

###### TP/oneM2M/CSE/ANNC/UPD/012

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT de-announces successfully an announced attribute when such attribute is deleted from announcedAttribute attribute. |  |
| Reference |  | TS-0001 10.2.18.9 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to OA_ATTRIBUTE and valid OA_ATTRIBUTE attribute and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to NULL } |  | IUT  AE |
|  | then { The IUT sends a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to NULL } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/012_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/012_GRP/MT | 9 (group) | memberType |
| TP/oneM2M/CSE/ANNC/UPD/012_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/012_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/012_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/012_SCH/SE | 18 (schedule) | scheduleElement |

###### TP/oneM2M/CSE/ANNC/UPD/013

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT provides confirmation of the de-announcement of an attribute to the originator of the initiating request after a successful attribute de-announcement procedure |  |
| Reference |  | TS-0001 10.2.18.7 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to OA_ATTRIBUTE and valid OA_ATTRIBUTE attribute and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to NULL and The IUT having sent a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to NULL } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 2004 (UPDATED) containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to NULL } |  | IUT  CSE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to NULL } |  | IUT  AE |

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE | PICS |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/013_CNT/MBS | 3 (container) | maxByteSize | PICS_IN_PROFILE |
| TP/oneM2M/CSE/ANNC/UPD/013_GRP/MT | 9 (group) | memberType |  |
| TP/oneM2M/CSE/ANNC/UPD/013_LCP/LOI | 10 (locationPolicy) | locationContainerID |  |
| TP/oneM2M/CSE/ANNC/UPD/013_MGO/DC | 13 (mgmtObj) | description |  |
| TP/oneM2M/CSE/ANNC/UPD/013_NOD/HCL | 14 (node) | hostedCSELink |  |
| TP/oneM2M/CSE/ANNC/UPD/013_SCH/SE | 18 (schedule) | scheduleElement |  |

###### TP/oneM2M/CSE/ANNC/UPD/014

| TP Id |  | TP/oneM2M/CSE/ANNC/UPD/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Host CSE of original resource removes an announcement target CSE from the announceTo attribute if it does not return successful CREATE response for the announced resource (for an UPDATE initiating resource announcement) |  |
| Reference |  | 10.2.18.4 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the container resource and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE-ID and Content containing container resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and The IUT having sent a valid CREATE Request containing To set to ANNC_CONTAINER_RESOURCE_ADDRESS and Content containing containerAnnc resource containing MA (mandatory) attributes } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT  CSE |
|  | then { the IUT updates the container resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing container resource containing announceTo attribute set to NULL } |  | IUT  AE |

##### DELETE Operation

###### TP/oneM2M/CSE/ANNC/DEL/001

| TP Id |  | TP/oneM2M/CSE/ANNC/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT provides confirmation of resource de-announcement to the originator of the initiating request after a successful resource de-announcement procedure (for a DELETE initiating resource de-announcement) |  |
| Reference |  | TS-0001 10.2.18.3 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and the IUT having sent a valid DELETE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |
|  | then { the IUT deletes the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2002 (DELETED) } |  | IUT  AE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/DEL/001_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/001_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/DEL/001_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/DEL/001_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/001_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/DEL/001_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/DEL/001_SCH | 18 (schedule) |

###### TP/oneM2M/CSE/ANNC/DEL/002

| TP Id |  | TP/oneM2M/CSE/ANNC/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes all announced resources when the original resource is deleted. |  |
| Reference |  | TS-0001 9.6.26.1, TS-0001 10.2.18.5 & TS-0004 7.3.3.10 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS } |  | IUT  AE |
|  | then { The IUT sends a valid DELETE Request containing To set to TARGET_RESOURCE_ANNC_ADDRESS } |  | IUT  CSE |

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/DEL/002_ACP | 1(accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/DEL/002_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/DEL/002_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/DEL/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/DEL/002_SCH | 18 (schedule) |

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.s

Annex <A> Conformance Test Requirement

A.1	MQTT Protocol Conformance Test Requirement

| Test Items |  | TS Chapter | Obligation | Condition for Optional Item | Checking point | Fail condition |
| --- | --- | --- | --- | --- | --- | --- |
| Commonly Repeated Items |  | TS-0010, Sec5.1 | Mandatory |  | "QoS=1" | Fail when QoS is other than “1” |
|  |  | TS-0010, Sec5.1 | Mandatory |  | "Retain" flag set as "0" | Fail if DUT sends with “Retain” flag=1 |
| Connection Items | Connect | TS-0010, Sec6.3 | Mandatory |  | Prefix A:: or C:: is added to the ID | Fail if Prefix A:: or C:: are not added to the ID |
|  |  | TS-0010, Sec6.3 | Mandatory |  | if Clean session (ex: Retain) flag is set to “False” in CONNECT | Fail when clean session is “True” when DUT want to be connected |
|  |  | TS-0010, Sec6.3 | Mandatory |  | if “Will Flag” is not set in CONNECT | Fail when will flag is enabled |
|  | Disconnect | TS-0010, Sec6.3 | Mandatory |  | if Clean session (ex: Retain) flag is set to “True” in CONNECT | Fail when clean session is “False” when DUT want to be disconnected |
| Subscription Items |  | TS-0010, Sec5.1 | Mandatory |  | if the message is set to Durable | Fail is the message is not Durable |
|  |  | TS-0010, Sec6.4.4 | Conditional | (for Initial Registration) when Originator doesn’t knows it’s ID | if DUT’s Initial ID assigning /oneM2M/reg_req/<originator>/<receiver> oneM2M/reg_resp/<originator>/<receiver> | Fail if DUT's ID assigning is other than this |
| Publish Item |  | TS-0010, Figure 6.4.5-2 | Conditional | when Originator knows it’s ID | if DUT’s Initial ID assigning oneM2M/req/<Originator>/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional | When Originator uses wild card | if DUT’s Initial ID assigning oneM2M/req/+/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | MQTT v3.1.1 | Conditional |  | if the has proper Primitive contents (“op”, “fr”, “to”, “ri”, “pc”, “ty”) | Fail if any of primitive contents feature wrong |
| Reception Item |  | TS-0010, Figure 6.4.5-2 | Conditional | when Originator knows it’s ID | if DUT’s Initial ID assigning oneM2M/resp/<Originator>/<Receiver> | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional | When Originator uses wild card | if DUT’s Initial ID assigning oneM2M/resp/<Originator>/+ | Fail if DUT's ID assigning is other than this |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional |  | server Delete received contents properly not storing data | Fail if Receiver stores message |
|  |  | TS-0010, Figure 6.4.5-2 | Conditional |  | If the receiver sends PubACK when receives messages | Fail if the receiver doesn’t send PubACK |
|  |  | MQTT v3.1.1 | Mandatory |  | if the has proper Primitive contents (“op”, “fr”, “to”, “ri”, “pc”, “ty”) | Fail if any of primitive contents feature wrong |

<PAGE BREAK>

Annex <B>(Informative): TP template

| TP Id |  |  |  |
| --- | --- | --- | --- |
| Test objective |  |  |  |
| Reference |  |  |  |
| Config Id |  |  |  |
| PICS Selection |  |  |  |
| Initial conditions |  |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { } |  | IUT  AE |
|  | then { } |  | IUT  AE |

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 22 Apil 2015 | Initial Draft Version (based on TST-2015-0071R01) |
| V.0.0.2 | 15 July 2015 | Add the agreed document from TST 17.3 (TST-2015-0115R01-Test_Purposes_contributions) |
| V.0.0.3 | 25 August 2015 | Add the agreed document from TST 18 (TST-2015-0138R03 TSS List and Test Purposes contributions) (TST-2015-0152R01 MQTT binding Requirement) |
| V.0.0.4 | 26 October 2015 | Add the agreed document from TST 19 (TST-2015-0184R02 TS-0018-TSS contribution) (TST-2015-0185R02 TS-0018-TP Definition conventions contribution) (TST-2015-0200 TS-0018-TP template) |
| V.0.0.5 | 14 January 2016 | Add the agreed document from TST 20 (TST-2015-0217R02-TS-0018-GMG_Test_Purposes_contribution) (TST-2015-0221R01-DIS_LOC_Test_Purpose_Contributions) (TST-2015-0222R01-SUB_Test_Purposes) (TST-2015-0234R01-TS-0018_DMR_Test_Purposes_with_Result_Content_param) |
| V.0.0.6 | 18 January 2016 | Add the agreed document from TST 19.2 (TST-2015-0212-TS-0018-Test_Purposes_contribution) |
| V.0.0.7 | 09 March 2016 | Add the agreed document from TST 21.2 (TST-2016-0034R01 TS-0018-Test Purposes contribution) (TST-2016-0035R01 Test_Purpose_GMG_CREATE) |
| V.0.0.8 | 02 May 2016 | Add the agreed document from TST 22 (TST-2016-0056-Test_Purpose_GMG_CREATE_CRs_accepted) (TST-2016-0047R01-TS-0018-Test_Purposes_contribution) (TST-2016-0058-Test_Purposes_for_DIS) (TST-2016-0059-Test_Purposes_for_LOC) |
| V.0.1.0 | 16 May 2016 | Add the agreed document from TST 22.3 (TST-2016-0081-TS-0018-Test_purpose_contribution) |
| V.0.1.1 | 18 July 2016 | Add the agreed documents from TST 22.2 (TST-2016-0066R02-Test_Purpose_GMG_UPDATE) (TST-2016-0067R02-Test_Purpose_GMG_UPDATE_REMOTE) |
| V0.1.2 | 22 September | Add the agreed documents from TST 24 (TST-2016-0124R01-CR_TS-0018-Update_LOC_testcase) |
| V0.1.3 | 10 October | Add the agreed documents from TST 24.x (TST-2016-0134R02-TestPurposes_fanoutPoint_basic) (TST-2016-0144R01-CR_TS-0018-Update_GMG_Test_Purposes) |
| V0.1.4 | 1st November | Add the agreed documents from TST 25 (TST-2016-0154R01-LOC_Test_Purposes) (TST-2016-0166R03-announce_deannounce_test_purpose) (TST-2016-0173R01-DMR_Container_and_Latest_ContentInstance_Test_Purposes) (TST-2016-0177R01-TS-0018 Test purpose contribution) (TST-2016-0178-TS-0018 Test purpose contribution) |
| V0.1.5 | 6th January 2017 | Add the agreed documents from TST 25.1 & TST 26 (TST-2016-0211-TS-0018_ACP_Test_purposes_contribution) (TST-2016-0218R01-TS-0018-Test_Purpose_for_ADN_DMR) (TST-2016-0219R01-TS-0018-Test_Purpose_for_ADN2_SUB_update) (TST-2016-0231R04-TS-0018-Test_Purpose_for_ADN) (TST-2016-0232R01-TS-0018-Test_Purpose_for_IN-CSE) (TST-2016-0234R01-TS-0018-Test_Purpose_for_ADN) (TST-2016-0235R02-TS-0018-Test_Purpose_for_ADN_SUB) (TST-2016-0236R01-CR_TS-0018_Update_DMR) (TST-2016-0243R01-TS-0018_Test_purpose_contribution) (TST-2016-0245-TS-0018_Modification_of_TPs_for_Retrieve_operation_on_DMR) (TST-2016-0246-TS-0018_TPs_for_Retrieve_operation_of_resources_under_CSEBase) (TST-2016-0255R01-duplicate_test_removal) |
| V0.1.6 | 11 February 2017 | Add the agreed documents from TST 26.1 and TST 26.2 TST-2016-0253R03-TS-0018-Test_Purposes_contribution_REG TST-2017-0001-CR-TS-0018_GMG_Test_Purpose_Correction TST-2017-0004R01-TS-0018-Test_urposes_harmonization TST-2017-0008-TS-0018_New_TPs_for_optional_attributes_(IN-PROFILE) TST-2017-0009R01-TS-0018_New_TPs_for_Announcement TST-2017-0010-TS-0018_Review_of_ANNC_group_test_purposes TST-2017-0011-TS-0018_Review_of_CRE_group_test_purposes TST-2017-0012-TS-0018_Review_of_RET_group_test_purposes TST-2017-0013-TS-0018_Review_of_DEL_group_test_purposes TST-2017-0014-TS-0018_Review_of_DMR_group_test_purposes TST-2017-0015R01-TS-0018_Review_of_GMG_group_test_purposes TST-2017-0016R01-TS-0018-Test_purposes_improvements |
| V0.1.7 | 17 February 2017 | Add the agreed documents from TST 27 - TST-2017-0055-Removal_duplicate_test_purpose |
| V0.1.8 | 3 April 2017 | Incorporated agreed documents from TST 28 - TST-2017-0075R01-TS-0018_New_TPs - TST-2017-0077R01-New TPs contribution - TST-2017-0080R01-TS-0018_New_TPs_for_ResultContent_attribute - TST-2017-0081-TS-0018_Minor_changes_to_DMR_TPs - TST-2017-0083-TS-0018_Test_purposes_improvements - TST-2017-0085R02-TP_for_CE_REG_00001.DOC - TST-2017-0086R03-TP_for_CE_REG_00002.DOC - TST-2017-0090R02-TP_for_CE_REG_00004.DOC - TST-2017-0106-TS-0018_Update_CSE_DMR_CRE_BV_003 - TST-2017-0105R01-TS-0018_Update_CSE_DMR_CRE_BV_001 |
| V0.1.9 | 24 April 2017 | Incorporated agreed documents from TST#28.1 - TST-2017-0108R02-Restructure_of_TS-0018 - TST-2017-0118-TS-0018_Update_CSE_DMR_DEL_BV_002-4 - TST-2017-0119R01-TS-0018_Update_DMR_TPs |
| V0.2.0 | 29 May 2017 | Incorporated agreed documents from TST#29 - TST-2017-0141R02-Missing_TPs_REG.doc - TST-2017-0150R04-TDs_for_CSEBase - TST-2017-0151R02-TDs_for_remoteCSE - TST-2017-0153R02-TPs_for_ADN - TST-2017-0154-Add_new_Test_Configuration - TST-2017-0156-TS-0018_Modification_of_configurations - TST-2017-0158-TS-0018_Changes_from_STF531 - TST-2017-0159-TPs_for_ADN - TST-2017-0160-TPs_for_ADN |
| V0.3.0 | 26 June 2017 | Incorporated agreed documents from TST#29.1 - TST-2017-0129R03-Update_TP_ID_format - TST-2017-0177R01-TS-0018_New_TPs_for_resource_addressing_methods - TST-2017-0178-Minor_changes_for_ADN_TPs |
| V0.4.0 | 11 July 2017 | Incorporated agreed documents from TST#29.2, TST#30 - TST-2017-0170R02-Missing_TPs_contentInstance - TST-2017-0171R03-Missing_TPs_container - TST-2017-0185R01-TS-0018_Changes_from_STF531 - TST-2017-0190R01-CR-TS-0018_clarification_testpurpose_format_rules - TST-2017-0193R01-CR-TS-0018_clarification_test_purposes_test_behaviors - TST-2017-0194R03-TPs_for_ACP - TST-2017-0195R01-ADN_TPs_for_resource_identifier - TST-2017-0196R01-Add_Missing_test_purposes_for_remoteCSE_MN - TST-2017-0197R01-CR-TS-0018_correct_AE_triggering_test_bahaviors - TST-2017-0201R02-TPs_for_AE - TST-2017-0202- TPs_for_StatsConfig - TST-2017-0210R01-CR-TS-0018_update_TP_Id&structure - TST-2017-0211R02-TPs_for_SUB |
| V0.5.0 | 11 September 2017 | Incorporated agreed documents from TST#30.2 - TST-2017-0225R01 - TS-0018_Changes from STF531 |