| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0024-V-0.2.0 |
| Document Name: | 3GPP_Rel13_IWK |
| Date: | 2016-Feb-15 |
| Abstract: | The document is a study of interworking between oneM2M Architecture and 3GPP Rel-13 architecture for Service Capability Exposure as defined in TS 23.682 V13.2.0. |
| Template Version:03June 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Conventions,	6

5	Introduction to 3GPP Service Capability Exposure	6

5.1	oneM2M Underlying Network related requirements	6

5.2	3GPP Release 13 MTC features	7

5.3	3GPP architecture for Service Capability Exposure	7

5.4	OMA API Program	8

5.4.1  Overview	8

5.4.2  OMA work to be considered by oneM2M for 3GPP IWK	9

5.4.2.1 OMA Service Exposure Framework (ServiceExposure)	9

5.4.2.2 OMA Exposing Network Capabilities to M2M (ENCap-M)	9

6	Reference architecture	9

7	Potential impact for interworking with oneM2M	10

8         Potential solutions for interworking with oneM2M	11

8.1  Interworking Architecture with a 3GPP underlying network	11

8.1.1 Exclusive Support through 3GPP Reference Points	11

8.1.2 Exclusive Support through OMA API	11

8.1.3 Hybrid Mode	11

8.3 Proposed communication flows	12

8.3.1 Support for device trigger recall/replace procedure	12

Proforma copyright release text block	14

Annexes	14

Annex <y>: Bibliography	15

History	15

# 1	Scope

The present document is a study of interworking between oneM2M Architecture and 3GPP Rel-13 architecture for Service Capability Exposure as defined in TS 23.682 V13.2.0. The key objective and value is analyzed and described. The document also investigates the potential solution in oneM2M by evaluating the existing technical solutions.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

Not applicable.

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	oneM2M TS-0002: " Requirements".

[i.3]	3GPP TS 22.101 Service aspects; Service principles (Release 13).

[i.4]	3GPP TS 22.115 Service aspects; Charging and billing (Release 13).

[i.5]	3GPP TS 23.682 Architecture enhancements to facilitate communications with packet data networks and applications (Release 13).

[i.6]	OMA API Inventory (http://technical.openmobilealliance.org/Technical/technical-information/oma-api-program)

[i.7]	OMA Service Exposure Framework (http://member.openmobilealliance.org/ftp/Public_documents/ARCH/ServiceExposure)

[i.8]	OMA Exposing Network Capabilities to M2M (http://member.openmobilealliance.org/ftp/Public_documents/ARCH/ENCap-M2M)

[i.9]	oneM2M TS-0001: " Functional Architecture ".

# 3	Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

# 4	Conventions,

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Introduction to 3GPP Service Capability Exposure

Editor’sNote: The clause describes 3GPP Service Capability Exposure architecture and related features, and oneM2M Underlying Network related requirements.

## 5.1	oneM2M Underlying Network related requirements

Following requirements are defined in oneM2M TS-0002 [i.2], but not  implemented or partially implemented in release 1.

Most of these requirements except OSR-052 can be achieved through the 3GPP features addressed in the subsequent sections, with and without support by OMA API.

OSR-006		The oneM2M System shall be able to reuse the services offered by Underlying Networks to M2M Applications and/or M2M Services by means of open access models (e.g. OMA, GSMA OneAPI framework).Examples of available services are:

-	IP Multimedia communications.

-	Messaging.

-	Location.

-	Charging and billing services, including sponsoring data flows.

-	Device information and profiles, including configuring expected communication patterns.

-	Configuration and management of devices.

-	Triggering, monitoring of devices.

-	Small data transmission.

-	Group management and group messaging

-	Configuring QoS

-	Receiving Reports about the condition of the uderlying network

Partially implemented in Rel-1. (see note 10)

NOTE 10; Rel-1 covers: Location, Charging and billing services, Configuration and management of devices, Device information and profiles, Triggering.

OSR-045a		The oneM2M System shall be able to receive and utilize information provided by the Underlying Network about when an M2M Device can be reached.

Not implemented in Rel-1.

OSR-051		Depending on availability of suitable interfaces provided by the Underlying Network the oneM2M System shall be able to request the Underlying Network to broadcast / multicast data to a group of M2M Devices in a specified area.

Implemented in Rel-1.  -> Not implemented in Rel-1. ??

OSR-052		The oneM2M System shall be able to select an appropriate Underlying Network to broadcast or multicast data depending on the network’s broadcast/multicast support and the connectivity supported by the targeted group of M2M Devices/Gateways.

Not implemented in Rel-1.

OPR-004		When suitable interfaces are provided by the Underlying Network, the oneM2M System shall have the ability to schedule traffic via the Underlying Network based on instructions received from the Underlying Network.

Not implemented in Rel-1.

OPR-005		The oneM2M System shall be able to exchange information with M2M Applications related to usage and traffic characteristics of M2M Devices or M2M Gateways by the M2M Application. This should include support for the 3GPP feature called: “Time controlled” (see note 1).

Not implemented in Rel-1.

NOTE1:	“Time controlled” is equivalent to the MTC Features specified in clause 7.2 of 3GPP TS 22.368 [i.2].

OPR-006		Depending on availability of suitable interfaces provided by the Underlying Network the oneM2M System shall be able to provide information related to usage and traffic characteristics of M2M Devices or M2M Gateways to the Underlying Network.

Not implemented in Rel-1.

## 5.2	3GPP Release 13 MTC features

In 3GPP Release 13, requirements for “Service exposure with 3rd party service providers” features are specified in clause 29 of 3GPP TS 22.101 [i.3] and the “Charged party selection” feature is defined in sub-clause 5.1.3 of 3GPP TS 22.115 [i.4].

3GPP Release 13 architecture supports these features and they can be used to implement the oneM2M requirements as described in the previous clause.

These 3GPP features are not only intended for M2M communication, but also for human usable applications such as smartphone applications.

3GPP intends to expose these additional features through the 3GPP Service Capability Exposure Function (SCEF) as described in the following clause.

## 5.3	3GPP architecture for Service Capability Exposure

The 3GPP architecture for the Service Capability Exposure Function (SCEF) is defined in 3GPP TS 23.682 [i.5]. The specification includes two different architectures. One is for the “MTC Device triggering” feature and was specified in 3GPP release 11. The other one is for 3GPP Service exposure with 3rd party service providers features newly provided in Release 13 which is the focus of the present document. Refer to the following figure 5.2-1, taken from the release 13 version of 3GPP TS 23.682 [i.5].

Figure 5.3-1  3GPP Architecture for Service Capability Exposure

While 3GPP release 13 specifies the Service Capability Exposure Function (SCEF) as a 3GPP entity, residing in the trust domain of the 3GPP operator, 3GPP does not specify the APIs exposing these functions. Specification of these APIs is expected by external SDOs, e.g. OMA.  As described in 3GPP TS 23.682 [i.5]. the SCEF covers services such as the the ability to configure device communication patterns, the QoS of a data flow, sponsor a data flow, scheduling data transfers, monitor a device’s state, optimizing a device’s communication patterns for high latency applications, receive reports about the condition of the mobile core network, trigger devices, and send group messages via MBMS.

## 5.4	OMA API Program

### 5.4.1  Overview

The OMA API Program provides standardized interfaces to the service infrastructure residing within communication networks and on devices. Focused primarily between the service access layer and generic network capabilities, OMA API Program specifications allow operators and other service providers to expose device capabilities and network resources in an open and programmable way—to any developer community independent of the development platform. By deploying OMA APIs, fundamental capabilities such as SMS, MMS, Location Services, Payment and other core network assets are now exposed in a standardized way. Additional OMA APIs may be found in OMA API Inventory [i.6].

### 5.4.2  OMA work to be considered by oneM2M for 3GPP IWK

#### 5.4.2.1 OMA Service Exposure Framework (ServiceExposure)

OMA ARC WG is working to define the Service Exposure Framework specification [i.7] which covers non-functional capabilities that a network operator or a service provider should consider when it exposes the service capabilities through the Network APIs. Such non-functional capabilities implemented in the intermediation layer may include Authentication and Authorization, Infrastructural Policy, Business Policy, Assurance and Accounting.

OMA Service Exposure Framework can be considered as an OMA specified SCEF which can be used by oneM2M s platforms..

#### 5.4.2.2 OMA Exposing Network Capabilities to M2M (ENCap-M)

Recently, OMA ARC WG is developing new APIs for exposing network capabilities to M2M applications and/or M2M service platforms.

The OMA work item “Exposing Network Capabilities to M2M” [i.8] lists requirements on standard APIs derived from use cases in which third parties, such as oneM2M or any other can leverage network capabilities to enrich the services or to streamline the operation. It also includes a gap analysis to identify any missing Network APIs to address above requirements and use cases. This enables utilisation of the latest evolution in cellular networks, e.g. 3GPP.

# 6	Reference architecture

Editor’s Note: this clause describes the reference architecture of 3GPP interworking.

Figure 6-1 Interworking architecture

This architecture supports the following interworking modes:

- The NSSE invokes services of the underlying network directly via the reference points of the applicable nodes within the underlying network. This model is applicable to the case where the oneM2M service provider and the underlying network provider is the same or there is trust relation between both service providers if they are different.

- The NSSE exclusively invokes services of a 3GPP underlying network using OMA API.

- The NSSE invokes exclusively services of any underlying network using third party APIs.

- Any combination of the above, where some services are invoked using an API (OMA or third party depending on the underlying network) while other services are invoked directly with the underlying network using the applicable reference point.

The functionality supported by the NSSE is different depending on the interworking mode.

# 7	Potential impact for interworking with oneM2M

Editor’s Note: this clause propose  the enhancements based on  architecture defined by oneM2M. What mechanisms can be reused and what need to be newly defined.

There are specific high level functions defined in 3GPP TS23.682 [i.5] section 4.5, such as device triggering, information storage, group message delivery, monitoring, high latency communications, network status reporting, background data transfer, communication patterns parameters provisioning, session QoS setting up, chargeable party changing.

According to the end-to-end oneM2M functional architecture described in oneM2M TS-0001 [i.9], all Common Services Functions (CSFs) reside within CSE may support those functions defined by 3GPP TS23.682 [i.9] and no architecture functional changing.

The Network Service Exposure, Service Execution and Triggering (NSSE) CSF manages communications with the 3GPP MTC Release-13 Underlying Networks. The NSSE CSF may be deployed as SCEF using 3GPP defined interfaces (e.g. Rx, Tsp, etc.) bound to Mcn reference point. The NSSE CSF may also use OMA APIs or other APIs bound to Mcn reference point.

The Communication Management and Delivery Handling (CMDH) CSF may support those functions such as device triggering, group message delivery, monitoring, high latency communications, network status reporting, background data transfer, communication patterns parameters provisioning, session QoS setting up, chargeable party changing.

The Data Management and Repository (DMR) CSF may support those functions such as information storage, monitoring.

The Device Management (DMG) CSF may support monitoring function.

The Group Management (GMG) CSF may support group message delivery function.

The Location (LOC) CSF may support those functions such as monitoring, network status reporting.

Special authentication and authorization  mechanisms for 3GPP Underlying Network such as IMSI, ACL, profile managements, policy control may be supported by the Security (SEC) CSF.

The Service Charging and Accounting (SCA) CSF may support those functions such as monitoring, chargeable party changing.

For supporting those functions, oneM2M system may add new attributes in existing resource types and changing existing service flowsor create new resource types and new service flows. For detail, please refer to section 8 potential solutions for interworking with oneM2M.

# 8         Potential solutions for interworking with oneM2M

## 8.1  Interworking Architecture with a 3GPP underlying network

The following three architectural models can be supported.

### 8.1.1 Exclusive Support through 3GPP Reference Points

Figure 8.1.1-1 depicts this architectural model.

In this case 3GPP services capabilities are exclusively invoked via the 3GPP reference points for the applicable 3GPP node.

Figure 8.1.1-1: oneM2M interworking with a 3GPP underlying network via 3GPP Reference Points

### 8.1.2 Exclusive Support through OMA API

Figure 8.1.2-1 depicts this architectural model. In this case 3GPP services capabilities are exclusively invoked via the OMA API. Hence, the SCEF is fully implemented outside the oneM2M environment.

Figure 8.1.2-1: oneM2M interworking with a 3GPP underlying network via OMA API

### 8.1.3 Hybrid Mode

Figure 8.1.3-1 depicts this architectural model.

In this case 3GPP services capabilities are invoked on a per service basis which can include OMA API for some service, proprietary APIs for others and finally some services can be invoked directly using the 3GPP reference points.

Figure 8.1.3-1: oneM2M interworking with a 3GPP underlying network in a hybrid mode

## 8.3 Proposed communication flows

### 8.3.1 Support for device trigger recall/replace procedure

Figure 8.3.1-1 below depicts a generic procedure for device triggering recall/replace between oneM2M and 3GPP network.

Figure 8.3.1-1: General device triggering recall/replace procedure between oneM2M and 3GPP network

Pre-condition

The IN-CSE has already send device trigger request to 3GPP network and connectivity is not established yet.

Step-1: Device Trigger Recall/Replace request

IN-CSE issues the device trigger Recall/Replace request to 3GPP network.

Some information provided to 3GPP Network for device trigger recall/replace includes:

- M2M-Ext-ID associated with the ASN/MN-CSE as the target of the triggering request.

- IN-CSE ID which could be used by 3GPP network to authorize the IN-CSE for device trigger recall/replace.

- The old trigger reference number was assigned to the previously submitted trigger message that the IN-CSE wants to recall/replace .

- For trigger replace request, the new trigger reference number which is assigned by the IN-CSE to the newly submitted trigger message.

Step-2: 3GPP Network Device Trigger Recall/Replace procedure

Device Trigger Recall/Replace procedure is performed in 3GPP Network, which is specified in 3GPP TS23.682[i.5].

Step-3: Device Trigger Recall/Replace response

The IN-CSE receives a response for the Device Trigger Recall/Replace request via the Mcn reference point.

Step-4: For trigger replace request, deliver new trigger message.

For trigger replace request, the new trigger message will be delivered to the target Node as specified in 3GPP TS23.682[i.5].

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A>:
Title of annex (style H9)

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 10 Sep 2015 | skeletonnique report of 000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 |
| V.0.1.0 | 20 Nov 2015 | Incorporated contributions: ARC-2015-2214R02-Proposal_for_section_5_of_TR-0024_”Introduction_to_3GPP_Service Capability Exposure" ARC-2015-2220R03-TR0024_Reference_architecture ARC-2015-2221R02-TR0024_OMA_ARC_API ARC-2015-2234R04-3GPP-Interworking |
| V.0.2.0 | 15 Feb 2016 | ARC-2016-0013R01-TR0024_section 7 ARC-2016-0018R01-Proposal_for_section_8_of_TR-0024 |