| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0020-V-0.1.1 |
| Document Name: | WebSocket Protocol Binding |
| Date: | 2015-July-04 |
| Abstract: | WebSocket Protocol Binding TS |
| Template Version:23 February 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1.	Scope	4

2.	References	4

2.1.	Normative references	4

2.2.	Informative references	4

3.	Definitions, symbols and abbreviations	4

3.1.	Definitions	4

3.2.	Symbols	5

3.3.	Abbreviations	5

3.4.	Acronyms	5

4.	Conventions	5

5.	WebSocket Protocol Binding	5

5.1.	Introduction	5

5.2.	Registration procedure	6

5.3.	Sending Non-Registration Request	6

5.4.	Handling Non-Registartion Request	6

6.	Examples	6

Proforma copyright release text block	6

Annexes	7

Annex <y>: Bibliography	7

History	8

# Scope

The present document will cover the protocol specific part of WebSocket binding, which is used by oneM2M compliant system.

The scope of the present document is (not limited to as shown below):

- Binding oneM2M primitive operations in WebSocket binding.

- Procedures to establish/disconnect the WebSocket connection.

The present document is depending on Core Protocol specification (oneM2M TS-00004) for data types.

# References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## Normative references

The following referenced documents are necessary for the application of the present document.

[1]	 IETF RFC 6455 (December 2011): "The Web Socket Protocol"

## Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## Definitions

oneM2M WebSocket Client: An AE or a CSE which establish the communication channel over WebSocket.

oneM2M WebSocket Server: A CSE which accepts the request to establish the communication channel over WebSocket.

Upper WebSocket Server: An oneM2M WebSocket Server which is closer to the IN-CSE on the CSE topology.

  - Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

## Acronyms

Acronyms should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Acronym format

<ACRONYM1>	<Explanation>

<ACRONYM2>	<Explanation>

<ACRONYM3>	<Explanation>

# Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# WebSocket Protocol Binding

Present clause describes the protocol binding specification to use WebSocket protocol for transport of oneM2M Protocol.

- Introduction

The WebSocket protocol enables to establish two-way communication channel from the client to the server no matter if there is a firewall or NAT between them.

The establishing WebSocket communication channel is requested by upgrading an ordinary HTTP request following RFC6455([1]).

AE or CSE shall request to Register as first request to Upper WebSocket Server, and they shall not send further message until the Registration request was returned successfully.

Once Registration procedure was performed successfully, AE or CSE may send non-registration requests and responses bidirectionaly.

When the WebSocket connection is disconnected, the Upper WebSocket Server shall performe un-regstration procedure for the WebSocket Client.

Those procedures are illustration in Figure-X.

Editor's Note: redrawn Figure 5.1.1 to be here

Figure 5.1.1: Communication Flow of  WebSocket Binding

The oneM2M primitive request and response message shall be mapped to  "data frame" (e.g. non-control frame) in WebSocket protocol.

When the value of response Type primitive parameter was  'blockingRequest(3)', the request shall be handled as 'nonBlockingSyncRequest'.

Editor's Note: how to interpret 'blocking request' concept in WebSocket to be clarified.

The default serialization format of Each oneM2M primitive message shall be in form of "JSON serialized primitive message" which is sepecified in TS-0004.

Editor's Note: (illustration of above text to be added)

## Registration procedure

(TBD)

## Sending Non-Registration Request

(TBD)

## Handling Non-Registartion Request

(TBD)

# Examples

The present clause illustrates the example call flow for WebSocket protocol binding.

(TBD)

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A> (Informative/Normative):Remove Informative or Normative as appropriatTitle of annex (style H9)

<Text>

<PAGE BREAK>

Annex <B>(Informative/Normative): Remove Informative or Normative as appropriateTitle of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

The following text is to be used when appropriate:

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.1.0 | 06 Jun 2015 | Initial Draft Version (based on PRO-2015-0806-WebSocketBinding_Scope) |