| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0024-V3.1.0 |
| Document Name: | OCF Interworking |
| Date: | 2018-September-7 |
| Abstract: | This document specifies details on interworking between oneM2M-specified entitiesand OCF-specified clients and/or servers. |
| Template Version: 08 September 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2017, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSTDI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions and abbreviations	5

3.2	Abbreviations	5

4	Conventions	5

5	Introduction to OCF interworking	5

6	OCF interworking architecture	6

6.1	Exposure of OCF Functions to the oneM2M System	6

6.1.1	Summary of Interworking Architecture for exposure of OCF Functions	6

6.1.2	OCF-IPE Responsibilities to support exposure of OCF Functions to the oneM2M System	8

6.2	Exposure of native oneM2M services to an OCF Proximal IoT Network	9

6.2.1	Summary of Interworking Architecture for exposure of native oneM2M services	9

6.2.2	OCF-IPE Responsibilities to support exposure of oneM2M services to an OCF Proximal IoT Network	11

7	Representation of OCF and/or oneM2M functions	12

7.1	Representation of OCF functions by oneM2M resources	12

7.1.1	Representation of OCF Devices by oneM2M <node> resources	12

7.1.2	Representation of OCF Devices by oneM2M <AE> resources	14

7.1.2.1	OCF Clients	14

7.1.2.2	OCF Servers	15

7.1.3	Representation of OCF services by oneM2M <flexContainer> resources	16

7.2	Representation of oneM2M services by OCF resources	17

8	OCF Interworking Procedures	18

8.1	Procedures supporting exposure of OCF Functions to the oneM2M Sytsem	18

8.1.1 	Determination of OCF Functions to be exposed to the oneM2M System	18

8.1.1.1	Summary	18

8.1.1.2	Provisioning	18

8.1.1.3	Discovery	18

8.1.1.4	On demand determination	18

8.1.2	Creation / Deletion of oneM2M Resource representing exposed OCF Functions	18

8.1.3	Mirroring state of exposed OCF Functions in oneM2M Resources	19

8.1.4	Detection of requests to execute OCF Functions and invocation thereof	19

8.2	Procedures supporting exposure of native oneM2M services to an OCF Proximal IoT Network	19

8.2.1 	Determination of oneM2M services to be exposed to the OCF Proximal IoT Network	19

8.2.1.1	Summary	19

8.2.1.2	Provisioning	19

8.2.1.3	Discovery	19

8.2.1.4	On demand determination	19

8.2.2	Instantiation / removal of OCF Servers representing exposed oneM2M services	19

8.2.3	Mirroring state of oneM2M Resources representing exposed oneM2M services in OCF resources hosted on OCF Servers	19

8.2.4	Detection of requests to consume exposed oneM2M services and execution thereof	19

Annex A (Informative)	20

Examples	20

History	20

# 1	Scope

The present document specifies the detailed methods for oneM2M and OCF interworking using the architecture identified in TS-0033 [5] and annex F of oneM2M TS-0001 [2] for the following scenario:

Interworking with full mapping of the semantics of the non-oneM2M data model to Mca, see scenario number 1 listed in Clause F.2 of oneM2M TS-0001 [2]. This is also in line with the interworking concepts specified in TS-0033 [5].

- This interworking scenario allows for interworking between OCF devices and oneM2M entities purely based on the common understanding of aligned information models – such as the models defined in TS-0023 [6]. There is no limitation regarding the direction of exposure of services: Services provided by OCF devices (OCF servers) can be exposed to oneM2M entities or vice versa. The oneM2M entities do not need to be aware of any details of the OCF protocols or interfaces.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0011: "Common Terminology".

[2]	oneM2M TS-0001: "Functional Architecture".

[3]	OIC-Core-Specification-V1.0.0: "OIC Core Specification".

[4]	oneM2M TS-0003: "Security solutions".

[5]	oneM2M TS-0033: "Interworking Framework".

[6]		oneM2M TS-0023: “Home Appliances Information Model and Mapping”.

[7]	OCF-Core-Specification-V1.3.0.

NOTE:	Available at https://openconnectivity.org/specs/OCF_Core_Specification_v1.3.0.pdf

[8]	OCF Device-Specification-V1.3.0.

NOTE:	Available at https://openconnectivity.org/specs/OCF_Device_Specification_v1.3.0.pdf

[9]	OCF Security-Specification-V1.3.0.

NOTE:	Available at https://openconnectivity.org/specs/OCF_Security_Specification_v1.3.0.pdf

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

# 3	Definitions and abbreviations

For the purposes of the present document, the terms and definitions given in oneM2M TS-0011 [1], oneM2M TS-0001 [2], and oneM2M TS-0033 [5] apply.

OCF Client:	A logical entity that accesses a Resource on an OCF Server.

OCF Device: A logical entity that assumes one or more roles (e.g., OCF Client, OCF Server).

OCF Framework: A set of related functionalities and interactions defined in the OCF Core Specification [7], which enable interoperability across a wide range of networked devices, including the Internet of Things.

OCF-IPE: IPE providing interworking functions for OCF-oneM2M interworking.

OCF Physical Entity: An aspect of the physical world that is exposed through an OCF Device. An example of an OCF Physical Entity is an LED.

OCF Platform: a physical device containing one or more Devices

OCF Resource:	Represents an OCF Entity modelled and exposed by the OCF Framework

OCF Server:	An OCF Device with the role of providing access to OCF Resource state information and facilitating remote interaction with those resources.

OCF Functions: Services or Device information provided by one or more OCF Servers by exposing access to OCF Resources via OCF-specified interfaces.

NOTE:	A term defined in the present document takes precedence over the definition of the same term, if any, in oneM2M TS-0011 [1], oneM2M TS-0001 [2], and oneM2M TS-0033 [5].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACP	Access Control Policy

AE	Application Entity

AE-ID		Application Entity Identifier

CBOR	Concise Binary Object Representation

CMDH	Communication Management and Delivery Handling

CSE	Common Services Entity

IPE	Interworking Proxy Entity

JSON	JavaScript Object Notation

OIC	Open Interconnect Consortium

URI	Uniform Resource Identifier

XML	eXtensible Markup Language

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# 5	Introduction to OCF interworking

OCF specifies an architecture enabling resource-based interactions among OCF Devices, see OCF-Core-Specification-V1.0.0 [7]. OCF Devices can expose aspects of the physical world like a lightbulb and/or logical entities like an application. The OCF Devices in the OCF architecture can provide services – in that case they play the role of an OCF Server – and/or consume services – in that case they play the role of an OCF Client.

This present document specifies details of the interworking with OCF Devices on the information model layer, see oneM2M TS-0033 [5],  and the implications on how to represent and execute external OCF functions with means of resource instances in the oneM2M system and vice versa.

Service provided and/or consumed on either side (OCF versus oneM2M) are represented by resources hosted by service providers (OCF Servers or oneM2M CSEs) and accessible to service consumers (OCF Clients or oneM2M AEs/CSEs). In order to support seamless interworking across boundaries of OCF & oneM2M deployments, OCF and oneM2M have aligned several information models for such services so that they became technology agnostic. Definitions of such information models are contained in the following specifications:

- oneM2M TS-0023 [6].

- OCF Device-Specification-V1.3.0 [8].

In the remainder of this present document it is assumed that mutual exposure of services between oneM2M and OCF deployments is restricted to services represented by resource types defined in these specifications.

In line with oneM2M TS-0033 [5], this present document specifies two major directions of mutual OCF / oneM2M interworking:

- Expose OCF Functions provided by OCF Servers to oneM2M entities by using one or more OCF-IPE(s) which are acting as OCF Client(s). For this direction - OCF services exposed to oneM2M - the OCF-IPE(s) are creating and managing the oneM2M resources representing exposed services provided by OCF Servers and provide the required procedures to allow consumption of OCF services on the oneM2M side. Details on this exposure direction are specified in Claus 6.1 of the present document.

- Expose services provided by oneM2M entities by using one or more OCF-IPE(s) which are acting as OCF Server(s).  For this direction – oneM2M services provided to OCF – the OCF-IPE(s) are interacting with already created oneM2M resource representations that are offering the native oneM2M services and provide the required procedures to allow consumption of the oneM2M services on the OCF side. Details on this exposure direction are specified in Claus 6.1 of the present document.

While it is possible that a single OCF-IPE can simultaneously support exposure of OCF Functions to oneM2M and exposure of oneM2M services to OCF, it is not mandated to implement OCF-IPEs in that way. It is also possible to instantiate separate OCF-IPEs for the different exposure directions. This is an implementation choice. In case that exposure in one direction needs to be correlated with a different Service Subscription Profile than for the other direction, separate OCF-IPEs are required. Details on the representation of OCF Functions by oneM2M Resources are defined in Clause 7 of the present document.

# 6	OCF interworking architecture

## 6.1	Exposure of OCF Functions to the oneM2M System

### 6.1.1	Summary of Interworking Architecture for exposure of OCF Functions

An OCF-IPE exposing OCF Functions to the oneM2M System is responsible for the creation of oneM2M Resources representing the exposed OCF Functions on its own Registrar CSE. A single OCF-IPE may expose OCF Functions provided by one or more OCF Servers to the oneM2M System. A high-level summary of the relationship of OCF Servers providing OCF Functions to be exposed to oneM2M, one or more OCF-IPE(s) and sets of oneM2M Resources representing the exposed OCF Functions is depicted in Figure 6.1.1-1 below.

Figure 6.1.1-1: Summary of Interworking Architecture for Exposure OCF → oneM2M

More than one OCF-IPE may be instantiated for exposure of multiple non-overlapping sets of OCF Functions to the oneM2M System. In particular, if OCF Functions provided by different sets of OCF Servers shall be exposed to the oneM2M System and there is a deployment requirement to differentiate sets of OCF Functions provided by different OCF Servers – e.g. for the purpose of correlating them with different Service Subscription Profiles – then instantiating more than one OCF-IPE is required. Further details on the implications using more than one OCF-IPE for exposure of OCF Functions to the oneM2M System are defined in Clause 7.1.

A specific instance of an OCF-IPE exposing OCF Functions to the oneM2M System shall play the role of a single OCF Client on the OCF side. If there is a deployment requirement to differentiate multiple OCF Clients who act as interworking proxies for exposing a common set of OCF Functions to the oneM2M System – for instance due to the need to assign different access control properties to them on the OCF side – then the OCF-IPE exposing those OCF Functions would need to play the role of multiple OCF Clients. However, mapping rules between the identifiers of Originators on the oneM2M side who attempt to consume the exposed OCF Functions and OCF Client identifiers to use for triggering the execution of the exposed OCF Functions on the OCF side are not specified in this present document. Therefore, it is assumed that each OCF-IPE is only playing the role of a single OCF Client on the OCF side. This restriction implies that all requests originating from oneM2M entities which are successfully modifying the corresponding oneM2M Resources and then get translated by a specific OCF-IPE to the corresponding interactions with OCF Servers are treated on the OCF side as coming from one single OCF Client and, therefore, will be handled by OCF Servers with the same access control properties, irrespective of the identifier of the Originator on the oneM2M side. However, by means of setting appropriate Access Control Privileges on the oneM2M side, it is certainly possible to define which entities on the oneM2M side will get which mode of  access to the exposed OCF services. Support of multiple OCF Clients for a single OCF-IPE is for further study and might be subject of future releases of the present document.

After creation of oneM2M Resources representing exposed OCF Functions by a specific OCF-IPE, this particular OCF-IPE is also responsible for monitoring relevant parts of these created resources in order to detect any operations that need to be translated into an execution of corresponding OCF Functions. In case such an operation on the previously created oneM2M Resources gets detected, the corresponding Function on the OCF side shall be executed by the OCF-IPE.

State changes within the OCF Functions exposed by a specific OCF-IPE to the oneM2M System that are impacting the state of the corresponding oneM2M Resources which were previously created by that specific OCF-IPE need to be monitored on the OCF side and shall be translated by that specific OCF-IPE into corresponding state changes of the associated oneM2M Resources.

### 6.1.2	OCF-IPE Responsibilities to support exposure of OCF Functions to the oneM2M System

When exposing OCF Functions to the oneM2M System, an OCF-IPE shall be responsible to support the following procedures:

- Determination of OCF Functions to be exposed to the oneM2M System
The OCF-IPE needs to determine which OCF Functions need to be exposed to the oneM2M System. This determination can be done in different ways (e.g. through provisioning, discovery, on-demand signalling, or combinations thereof). Further details of this procedure are defined in Clause 8.1.1

- Creation / Deletion of oneM2M Resource representing exposed OCF Functions
The OCF-IPE needs to perform creation / deletion of resource instances representing OCF Functions according to the – possibly dynamically changing – need to expose them to the oneM2M System using resource types that are have been aligned between oneM2M and OCF in order to become technology independent. Further details on this procedure are defined in Clause 8.1.2 Resource types that meet the requirement to be technology agnostic between OCF and oneM2M are defined in:

  - OCF Device-Specification-V1.3.0 [8] and oneM2M TS-0023 [6].

- Mirroring state of exposed OCF Functions in oneM2M Resources
An OCF-IPE exposing OCF Functions provided by OCF Servers is responsible to modify the resource instances it has created in order to represent the exposed OCF Functions according to any state changes occurring in the corresponding OCF Servers. This implies that such an OCF-IPE shall monitor the state of the associated OCF Servers and upon detection of OCF Server state changes relevant for the exposed OCF Functions the OCF-IPE shall modify the previously created oneM2M Resources accordingly. Further details on this procedure are defined in Clause 8.1.3

- Detection of requests to execute OCF Functions and invocation thereof
The OFC-IPE is responsible for monitoring relevant changes in the resource instances it has previously created for the purpose of representing the exposed OCF Functions. Upon detection of any valid operation meant to trigger the execution of the exposed OCF Functions, the OCF-IPE is responsible for the invocation of the corresponding OCF Functions via its own OCF Client. Further details of this procedure are defined in Clause 8.1.4.

The set of responsibilities of the OCF-IPE when exposing OCF Functions to the oneM2M system is summarized in Figure 6.1.2-1 below. The dashed boxes describe optional/alternative means to determine the set of exposed OCF Functions. Note that, in this Figure one OCF-IPE is responsible for all interworking procedures to support exposure of OCF Functions to oneM2M. More than one OCF-IPE may be used to expose different sets of OCF Functions to oneM2M. Details on how to map exposed OCF Function into oneM2M Resources are defined in clause 7.

Figure 6.1.2-1: Exposure of OCF Functions to the oneM2M System

## 6.2	Exposure of native oneM2M services to an OCF Proximal IoT Network

### 6.2.1	Summary of Interworking Architecture for exposure of native oneM2M services

An OCF-IPE exposing oneM2M services to an OCF Proximal IoT Network is responsible for the creation of OCF Server instances in the OCF Proximal IoT Network representing the exposed oneM2M services. A single OCF-IPE may expose one or more oneM2M services to the OCF Proximal IoT Network. A high-level summary of the relationship of oneM2M Resources providing the services to be exposed to the OCF Proximal IoT Network, one or more OCF-IPE(s) and sets of OCF Servers representing the exposed oneM2M services is depicted in Figure 6.2.1-1 below.

Figure 6.2.1-1: Summary of Interworking Architecture for Exposure oneM2M → OCF

More than one OCF-IPE may be instantiated for exposure of multiple non-overlapping sets of oneM2M services to the OCF Proximal IoT Network. In particular, if there is a deployment requirement to differentiate the exposure of different sets of oneM2M services – e.g. for the purpose of correlating them with different Service Subscription Profiles – then instantiating more than one OCF-IPE is required. Further details on the implications using more than one OCF-IPE for exposure of oneM2M services to the OCF Proximal IoT Network are defined in Clause 7.1.

A specific oneM2M service shall only be exposed to a given OCF Proximal IoT Network by at most one instance of an OCF-IPE connected to that OCF Proximal IoT Network. If there is a deployment requirement to differentiate multiple AE instances who act as interworking proxies on behalf of OCF Clients for exposing a common set of oneM2M services to the OCF Proximal IoT Network – for instance due to the need to assign different access control privileges to them on the oneM2M side – then the exposure of that common set of oneM2M services would require multiple instances of OCF-IPEs with a shared set of OCF Servers exposing the same set of oneM2M services to a given OCF Proximal IoT Network. However, mapping rules between the identifiers of OCF Clients who attempt to consume the exposed oneM2M services and the respective OCF-IPE AE-IDs to use for triggering the execution of the exposed oneM2M services are not specified in this present document. Therefore, it is assumed that each exposed oneM2M service is only interacting with one OCF-IPE for a given OCF Proximal IoT Network. This restriction implies that all requests originating from OCF Clients received by any of the OCF Servers instantiated by a given OCF-IPE which are meant to be translated by the OCF-IPE to the corresponding operations on oneM2M Resources representing the exposed oneM2M services are treated on the oneM2M side as coming from one single oneM2M AE and, therefore, will be handled by oneM2M CSEs with the same access control privileges, irrespective of the identifier of the requesting OCF Client. However, by means of setting appropriate access control properties on the OCF side – i.e. access control governing the acceptance or requests at the OCF Servers instantiated by the OCF-IPE – it  is certainly possible to define which OCF Client will get which mode of  access to the exposed oneM2M services. Therefore, access control can be imposed on OCF Client basis if the appropriate access control properties are provisioned properly into the OCF Servers instantiated by the OCF-IPE. Support of multiple OCF-IPEs exposing the same set of oneM2M services is for further study and might be subject of future releases of the present document.

After creation of OCF Server instances representing the exposed oneM2M services by a specific OCF-IPE, this particular OCF-IPE is also responsible for monitoring incoming OCF Client requests reaching these created OCF Servers in order to detect any requests that need to be translated into a corresponding operation on oneM2M Resources representing the exposed oneM2M services. In case such an request directed to the previously created OCF Servers gets detected, the corresponding operation(s) on the oneM2M side shall be executed by the OCF-IPE.

State changes within the oneM2M Resources representing the oneM2M services exposed to the OCF Proximal IoT Network by a specific OCF-IPE that are impacting the state of the corresponding resources on any of the OCF Servers that were previously created by that specific OCF-IPE need to be detected on the oneM2M side and shall be translated by that specific OCF-IPE into corresponding state changes of the associated resources in the OCF Servers.

### 6.2.2	OCF-IPE Responsibilities to support exposure of oneM2M services to an OCF Proximal IoT Network

When exposing oneM2M services to an OCF Proximal IoT Network, an OCF-IPE shall be responsible to support the following procedures:

- Determination of oneM2M services to be exposed to the OCF Proximal IoT Network
The OCF-IPE needs to determine which oneM2M services need to be exposed to the OCF Proximal IoT Network. This determination can be done in different ways (e.g. through provisioning, discovery, on-demand signalling, or combinations thereof). Further details of this procedure are defined in Clause 8.2.1

- Instantiation / removal of OCF Servers representing exposed oneM2M services
The OCF-IPE needs to perform instantiation / removal of OCF Server instances representing oneM2M services according to the – possibly dynamically changing – need to expose them to the OCF Proximal IoT Network using OCF Servers hosting OCF resource types that are have been aligned between oneM2M and OCF in order to become technology independent. Further details on this procedure are defined in Clause 8.2.2. OCF resource types and equivalent oneM2M resource types that meet the requirement to be technology agnostic between OCF and oneM2M are defined in:

  - oneM2M TS-0023 [6] and OCF Device-Specification-V1.3.0 [8].

- Mirroring state of oneM2M Resources representing exposed oneM2M services in OCF resources hosted on OCF Servers
An OCF-IPE exposing oneM2M services to an OCF Proximal IoT Network is responsible to modify the resource instances in the OCF Servers it has instantiated in order to represent the exposed oneM2M services according to any state changes occurring in the corresponding oneM2M Resources. This implies that such an OCF-IPE shall monitor the state of the associated oneM2M Resources and upon detection of oneM2M Resource state changes relevant for the exposed oneM2M services, the OCF-IPE shall modify the corresponding resource state of OCF resources hosted in the previously instantiated OCF Servers accordingly. Further details on this procedure are defined in Clause 8.2.3

- Detection of requests to consume exposed oneM2M services and execution thereof
The OFC-IPE is responsible for monitoring relevant changes in the resources hosted by the OCF Servers the OCF-IPE has previously instantiated for the purpose of representing the exposed oneM2M servies. Upon detection of any valid operation on the OCF side meant to trigger the consumption of the associated exposed oneM2M services, the OCF-IPE is responsible for the requesting the corresponding oneM2M operation on the oneM2M Resources representing the exposed oneM2M service to be consumed. Further details of this procedure are defined in Clause 8.2.4.

The set of responsibilities of the OCF-IPE when exposing oneM2M services to an OCF Proximal IoT Network is summarized in Figure 6.2.2-1 below. The dashed boxes describe optional/alternative means to determine the set of exposed oneM2M services. Note that, in this Figure one OCF-IPE is responsible for all interworking procedures to support exposure of oneM2M services to an OCF Proximal IoT Network. More than one OCF-IPE may be used to expose different sets of oneM2M services to the same OCF Proximal IoT Network. Details on how to map exposed oneM2M services into OCF resources are defined in clause 7.

Figure 6.2.2-1: Exposure of native oneM2M functions to the OCF Proximal IoT Network

# 7	Representation of OCF and/or oneM2M functions

## 7.1	Representation of OCF functions by oneM2M resources

### 7.1.1	Representation of OCF Devices by oneM2M <node> resources

An OCF Device which provides functions that are exposed to the oneM2M system can be represented by a oneM2M <node> resource that contains device-specific information which can be used e.g. for the purpose of device management. Whether a specific OCF device shall be represented by a oneM2M <node> resource depends on the type of device:

- OCF Devices providing services that are exposed to the oneM2M system using specializations of  <flexContainer> resources as defined in oneM2M TS-0023 [6], see also clause 7.1.3 below, shall be represented by oneM2M <node> resources as well. Each instance of a <flexContainer> resource representing a service provided by a specific OCF device is linked via the nodeLink attribute to a specific <node> resource instance that represents this specific OCF device.

For OCF Devices that are not matching with any of the listed categories, the present document does not specify any normative procedure to represent such OCF Devices as oneM2M <node> resources.

When a <node> resource is used to represent an OCF Device, it actually needs to reflect the details of the OCF Platform that hosts the OCF Device, Attributes of the <node> resource listed in Table 7.1.1-1 below shall have the specified value settings in Table 7.1.1-1. All other attributes of the <node> resource shall be used as specified in oneM2M TS-0001 [2]. A <node> resource representing an OCF Device shall include exactly one [deviceInfo] child resource instance that represents specifics of the OCF Platform hosting the OCF Device. All other child resources of the <node> resource shall be used as specified in oneM2M TS-0001 [2]. Attributes of a [deviceInfo] resource listed in Table 7.1.1-2 below shall have the specified value settings in Table 7.1.1-2 for the [deviceInfo] child resource instance representing the OCF Platform of the OCF Device. All other attributes or child resources of the [deviceInfo] resource shall be used as specified in oneM2M TS-0001 [2].

Table 7.1.1-1: Attribute settings for <node> resources representing an OCF Platform.

| Attribute Name | Setting |
| --- | --- |
| resourceName | This attribute shall be set to the value of the “pi” property (Platform ID) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7]. |
| labels | This attribute shall include a Key:Value pair equal to “Iwked-Technology:OCF”. Other Key:Value pairs may also be present. |
| nodeID | This attribute shall be set to the value of the “pi” property (Platform ID) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7]. |
| hostedServiceLinks | This attribute shall contain a list of resource identifiers of <flexContainer> resources representing OCF services provided by OCF Servers which are hosted by the OCF Platform that is represented by this <node> resource. |

Table 7.1.1-2: Attribute settings for a [deviceInfo] child-resource of a <node> resources representing an OCF Platform.

| Attribute Name | Setting |
| --- | --- |
| labels | This attribute shall include a Key:Value pair equal to “Iwked-Technology:OCF”. Other Key:Value pairs may also be present. |
| deviceLabel | This attribute shall be set to the value of the “pi” property (Platform ID) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7]. |
| manufacturer | This attribute shall be set to the value of the “mnmn” property (Manufacturer Name) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| manufacturerDetailsLink | This attribute shall be set to the value of the “mnml” property (Manufacturer Details Link) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| manufacturingDate | This attribute shall be set to the value of the “mndt” property (Date of Manufacture) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| model | This attribute shall be set to the value of the “mnmo” property (Model Number) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| subModel | This attribute shall be set to the value of the “mnpv” property (Platform Version) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| deviceType | This attribute shall be set to the value “OCF Platform” |
| fwVersion | This attribute shall be set to the value of the “mnfv” property (Firmware Version) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| hwVersion | This attribute shall be set to the value of the “mnhw” property (Hardware Version) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| osVersion | This attribute shall be set to the value of the “mnos” property (OS Version) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| systemTime | This attribute shall expose the system time of the device in line with the value of the “st” property (SystemTime) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |
| supportURL | This attribute shall be set to the value of the “mnsl” property (Support link) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7], if present. |

### 7.1.2	Representation of OCF Devices by oneM2M <AE> resources

#### 7.1.2.1	OCF Clients

The present document does not specify any normative procedure to represent OCF Clients as oneM2M <AE> resources. Since the OCF specifications do not define any procedure for OCF Clients needing to register themselves with any other entity or needing to respond to any discovery procedures before trying to consume services provided by OCF Servers, it is not possible to determine which OCF Clients exist before they make any requests to consume services provided by OCF Servers. Therefore, it would be very complex to associate OCF Clients in an OCF Proximal IoT Network with oneM2M <AE> resources. In order to do that, the OCF-IPE would have to detect OCF Clients dynamically at the time when they try to request any services from that OCF-IPE and then create on the fly some corresponding <AE> resources. Furthermore, such dynamically created <AE> resources would also require adjustment of  access control privileges when the corresponding AEs would need to access oneM2M resources. For these reasons, there are no provisions in this present document to represent OCF Clients by oneM2M <AE> resources.

#### 7.1.2.2	OCF Servers

OCF Devices playing the role of an OCF Server can be represented by an <AE> resource that corresponds to an OCF-IPE, see Figure 6.1.1-1. Depending on implementation and deployment needs, one or more OCF Servers can be represented on the oneM2M side by a single OCF-IPE with a single <AE> resource.

In case an OCF Server is intended to be exposed to the oneM2M system, exactly one OCF-IPE shall be responsible for the exposure of services provided by that specific OCF Server in a given OCF Proximal Network. Therefore, a specific OCF Server shall be represented by at most one OCF-IPE, i.e. by at most one <AE> resource.

In case of OCF Servers, the details of the OCF Server can be determined in the OCF Proximal IoT Network by means of executing OCF discovery and introspection procedures. For an OCF-IPE representing a specific OCF-Server, any exposed services provided by that OCF Server that are meant to be exposed to the oneM2M System shall be represented by child resources of the <AE> resource representing that specific OCF-IPE, see Clause 7.1.3 below.

Representing one or more OCF Servers by the <AE> resource associated with an OCF-IPE has the following implications:

- The set of represented OCF Servers represented by a specific OCF-IPE can be associated with M2M Service Subscriptions just like any other oneM2M Application

- The OCF-IPE allowing interworking with specific type of OCF Servers can be registered in the App-ID registry

Attributes of  an <AE> resource listed in Table 7.1.2-1 below shall have the specified value settings in Table 7.1.2-1 when that <AE> resource represents and OCF-IPE. All other attributes of the <AE> resource shall be used as specified in oneM2M TS-0001 [2].

Table 7.1.2-1: Attribute settings for <AE> resources representing an OCF-IPE.

| Attribute Name | Setting |
| --- | --- |
| labels | In order to indicate the supported external technology and which specific OCF Server(s) are represented by a specific OCF-IPE, the labels attribute of the <AE> resource of an OCF-IPE exposing OCF Server Functions to oneM2M shall contain the following information: - A Key:Value pair set to “Iwked-Technology:OCF” indicating that this AE supports interworking with OCF entities. - A Key:Value pair indicating the specific OCF Servers being exposed to the oneM2M system by this specific OCF-IPE. The Key should be set to “Iwked-Entity-IDs”. The Value shall contain a coma-separated list of IDs in square brackets, e.g.. “[ID1, ID2, ID3]”, where each listed ID identifies one specific exposed OCF Server and is equal to the value of the “piid” property (Protocol Independent ID) of the device resource with the pre-defined URI “/oic/d” of the respective OCF Server as defined in the OCF Core Specification [7]. Other Key:Value pairs may also be present. |

### 7.1.3	Representation of OCF services by oneM2M <flexContainer> resources

Services provided by OCF Servers that are intended to be exposed to the oneM2M system shall be represented by specializations of <flexContainer> resources in order to be exposed to the oneM2M system. Services provided by a specific OCF Server and intended to be exposed to the oneM2M system shall be represented by <flexContainer> child resources of the <AE> resource instance that represents the OCP-IPE responsible for the exposure of services provided by that specific OCF Server to oneM2M.

Services provided by OCF Servers are characterized by the device types defined in the OCF Device Specifications [8]. For each device type there is a minimum set of OCF resources defined in the OCF Device Specifications [8]. Exposure of a specific service provided by an OCF Server is accomplished by representing the corresponding set of OCF resources hosted on the OCF Server with matching oneM2M <flexContainer> resources hosted on the Registrar-CSE of the OCF-IPE responsible for the exposure of that OCF Server as child resources of the <AE> resources representing the OCF-IPE.

In oneM2M TS-0023 [6] a set of information models is defined which describes the exposure of services provided by oneM2M Devices via  specializations of <felxContainer> resources. The specification oneM2M TS-0023 [6] also contains a mapping between OCF Device Types and the corresponding oneM2M Devices.

When exposing services of an OCF Server that complies with any of the OCF Device Types for which oneM2M TS-0023 [6] includes a normative mapping to oneM2M Devices, an instance of the respective <flexContainer> specialization as defined in oneM2M TS-0023 [6] and any required child resources shall be used to represent the exposed service provided by an OCF Server.

When exposing services of an OCF Server that does not comply with any of the OCF Device Types for which oneM2M TS-0023 [6] includes a normative mapping to oneM2M Devices, an instance of a customized <flexContainer> resource shall be used. In line with <flexContainer> resources defined in oneM2M TS-0023 [6], such customized <flexContainer> resources shall include a nodeLink attribute which links to a <node> resource that represents the OCF Platform hosting the exposed OCF Server. Further details of such customized  < flexContainer> resources are not in scope of this specification.

Attributes of  an <flexContainer> resource representing services provided by an OCF Server as listed in Table 7.1.3-1 below shall have the specified value settings in Table 7.1.3-1. All other attributes of the <flexContainer> resource shall be used as specified in oneM2M TS-0001 [2] or in oneM2M TS-0023 [6] if applicable.

Table 7.1.2-1: Attribute settings for <flexContainer> resources representing services provided by an OCF Server.

| Attribute Name | Setting |
| --- | --- |
| labels | In order to indicate the supported external technology and which specific OCF Server is represented by this <flexContainer> resource, the labels attribute of the <flexContainer> resource shall contain the following information: - A Key:Value pair set to “Iwked-Technology:OCF” indicating that this <flexContainer> supports interworking with OCF entities. - A Key:Value pair indicating the specific OCF Server being exposed to the oneM2M system by this specific <flexContainer>. The Key should be set to “Iwked-Entity-ID”. The Value shall be equal to the value of the “piid” property (Protocol Independent ID) of the device resource with the pre-defined URI “/oic/d” of the respective OCF Server as defined in the OCF Core Specification [7]. Other Key:Value pairs may also be present. |
| nodeLink | The resource identifier of a <node> resource that stores the node specific information of the node on which the OCF Server resides which provides services that are exposed by this <flexContainer> resource. See clause 7.1.1 for details on representing OCF Devices by a <node> resource |

The procedural aspects on how to create, delete and interact with <flexContainer> resources for the purpose of exposing OCF Functions to the oneM2M system are specified in Claus 8.1 of the present document.

## 7.2	Representation of oneM2M services by OCF resources

Services provided by oneM2M Devices can be exposed to an OCF Proximal IoT Network by an OCF-IPE acting as one or more OCF Servers – see clause 6.2.1.

For a service intended to be exposed to the OCF Proximal IoT Network and provided by a oneM2M Device represented by a <flexContainer> resource, the corresponding OCF Device Type shall be determined according to the normative mapping defined in the oneM2M specification(s)

- oneM2M TS-0023 [6].

Then for each OCF Device Type that has been determined in the mapping, the minimum set of resources for that OCF Device Type as specified in the OCF Device Specification [8] shall be exposed to the OCF Proximal IoT Network by the OCF-IPE responsible for the exposure. The OCF-IPE responsible for the exposure to the OCF Proximal IoT Network shall act as an OCF Server for each identified OCF Device Type.

Each OCF Server which is instantiated by an OCF-IPE needs to provide a minimum set of resources depending on the Device Type it represents. Common to all OCF Devices is the need to support the following mandatory core resources:

- “/oic/res” for discovery of resources hosted by the OCF Server

- “/oic/p” for discovery of platform specific parameters of the node hosting the OCF Server

- “/oic/d” for discovery of device information

Beyond these core resources, an OCF Server shall also support the resources required by the specific OCF Device Type. For the mandatory core resources supported by all OCF Servers, the property settings defined in Table 7.2-1 and Table 7.2-2 shall apply. Properties not listed in Table 7.2-1 and Table 7.2-2 may be present while the present specification does not define any settings for those properties.

Table 7.2-1: Property settings for a “/oic/p” resource provided by an OCF-IPE acting as an OCF Server.

| Property Name | Setting |
| --- | --- |
| pi | This property shall be generated in line with the OCF Core Specification [7]:Unique identifier for the physical platform (UIUID); this shall be a UUID in accordance with IETF RFC 4122. It is recommended that the UUID be created using the random generation scheme (version 4 UUID) specific in the RFC. If the OCF-IPE that instantiated this OCF Server is instantiating multiple OCF Servers, it shall use the same value for all pi properties of the /oic/p resources of all its instantiated OCF Servers. |
| mnmn | In case the <AEr> resource representing the OCF-IPE that instantiated this OCF Server is linked to a <node> resource that has a [deviceInfo] child resource that includes a manufacturer attribute, the value of that manufacturer shall be used for this mnmn property. Otherwise, the string to be used for this property is implementation dependent but needs to be present.. |

Table 7.2-2: Property settings for a “/oic/d” resource provided by an OCF-IPE acting as an OCF Server.

| Property Name | Setting |
| --- | --- |
| n | An implementation dependent OCF Device name prefixed by the string “oneM2M-“. |
| icv | Spec version of the OCF Core Specification this OCF Server instance of the OCF-IPE is implemented to, The syntax is "ocf.<major>.<minor>.<subversion>” where <major>, <minor>,and <sub-version> are the major, minor and sub-version numbers of the OCF Core Specification, respectively. |
| di | Unique identifier of the device in line with the requirements in the OCF Core Specifications [7]. |
| dmv | Version of the Resource Specification to which this OCF Server instance of the OCF-IPE is implemented. This needs to be in line with the requirements on the dmv property defined in the OCF Core Specifications [7]. |
| piid | A unique and immutable identifier for this OCF Server instance of the OCF-IPE generated in line with the OCF Core Specifications [7] |

# 8	OCF Interworking Procedures

## 8.1	Procedures supporting exposure of OCF Functions to the oneM2M System

### 8.1.1 	Determination of OCF Functions to be exposed to the oneM2M System

In an OCF Proximal IoT Network, the specific setup and security parameters to be used by OCF Clients or OCF Servers – such as credentials to be used – are provisioned during an onboarding procedure. This onboarding procedure also includes provisioning of credentials and roles etc. as described in OCF Security-Specification [9]. In order to perform that onboarding procedure, an onboarding tool is used. An OCF-IPE which intends to expose OCF Functions of OCF Servers in the OCF Proximal IoT Network to the oneM2M System needs to go through such an onboarding process as well. During the onboarding procedure, OCF-IPE is provisioned with appropriate credentials and parameters to interact with OCF Servers in the OCF Proximal IoT Network. For the OCF-IPE to properly support the intended exposure of OCF Services, it needs to be determined which particular OCF Servers are meant to be exposed via the OCF-IPE being onboarded. The set of OCF Servers to be exposed shall be identified by a set of “piid” property values (Protocol Independent ID) of the device resource with the pre-defined URI “/oic/d” of the respective OCF Servers, see OCF Core-Specification [7].

In what follows three different concepts of selecting the set of OCF-Servers to be exposed to the oneM2M System are outlined. However, it is not in the scope of the present specification to define details on how to implement these concepts:

- Pre-Provisioning: The set of OCF Servers to be exposed to the oneM2M System is determined before the OCF-IPE is initiated. The details of the selection procedure are not specified in the present specification. The resulting selection is provided to the OCF-IPE by means of configuration information – see circle termed “Configuration” in Figure 6.1.2-1 –  stored statically in storage accessible by the OCF-IPE such as a configuration file or a set of <contentInstance> resources. It is not in the scope of this specification to define any details of the storage mechanism such as storage location, format, serialization etc. of the selected set of OCF-Servers. Upon initiation of the OCF-IPE it shall act as an OCF Client and verify that the configured OCF Servers to be exposed to oneM2M are actually accessible on the OCF Proximal IoT Network by correlating the configured “piid” values with the results of a direct resource discovery procedure targeting the pre-defined URI “/oic/d” or a corresponding indirect discovery procedure targeting the the pre-defined URI “/oic/rd” in a multicast request in the OCF Proximal IoT Network, see clause 11.3 of OCF Core-Specification [7]. If configured OCF Servers are not discoverable in the OCF Proximal IoT Network, the corresponding entries need to be removed from the the set of OCF-Servers to be exposed to the oneM2M System  by the OCF-IPE

- Discovery: When the OCF-IPE is initiated, in will act as an OCF Client and trigger a discovery of OCF Servers in the OCF Proximal IoT Network. Among the discovered OCF Servers, a set of OCF Servers to be exposed will be selected by the stakeholder responsible for the OCF-IPE deployment. This selection may be implemented by a user-facing interface (GUI) or by other means and is not in the scope of the present specification. Discovery results may also get filtered by the OCF-IPE before selecting the OCF Servers to be exposed to oneM2M. For example, the set of discovered OCF Servers can be filtered in order to limit the exposure to oneM2M to a specific set of OCF Servers with a given manufacturer name or a specific model name as indicated by the “dmn” property (Manufacturer Name) or the “dmno” property (Model Number) of the OCF device discovery resource with the pre-defined URI “/oic/d” of the respective OCF Device as defined in [7]. In line with the described discovery and selection process, the OCF-IPE needs to be onboarded to the OCF Proximal IoT Network as an OCF Client with credentials that are sufficient to access the selected OCF Servers intended to be exposed to oneM2M.

- On demand determination: This selection concept is very similar to the one described in the previous paragraph, except that the discovery of OCF Servers to be exposed to the oneM2M system is triggered by means of changing the state of a oneM2M resource – see the dashed box termed “Resource instances to trigger discovery of OCF Functions to be exposed to oneM2M” in Figure 6.1.2-1. This specification does not define details for this triggering mechanism which is an implementation choice. In the remainder of this paragraph an example is given. For instance a <container> resource may get created by the OCF-IPE when it starts and a subscription to that <container> resource would be established for the OCF-IPE to get notified about creation of any new <contentInstance> resources in that <container> along with ACPs that would control which entities are authorized to trigger the OCF discovery. Upon creation of a new <contentInstance> child resource in that <container> resource, the OCF-IPE would get notified. The information in the content attribute of the new <contentInstance> resource may be used to define parameters for the discovery to be executed such as filtering OCF discovery with a specific “dmn” property (Manufacturer Name) or the “dmno” property (Model Number) of the OCF device discovery resource with the pre-defined URI “/oic/d” of the respective OCF Device as defined in [7]. The notification would then result in the OCF-IPE initiating a new discovery and selection procedure as described in the previous paragraph. After that is completed, the resource used to trigger the discovery and selection process may need to get cleaned up – i.e. the <contentInstance> resource may need to get removed.

Independent of which of the described selection concepts is implemented, the onboarding of the OCF-IPE as an OCF Client needs to result in appropriate credentials being provisioned to the OCF-IPE so that the OCF-IPE is authorized to interact as intended with the selected OCF Servers.

For instance if the OCF-IPE will act as an OCF Client with the intent to be able to switch on or off a set of lights implemented as OCF Servers, the onboarding procedure needs to result in provisioning of credentials that provide the OCF-IPE sufficient access rights to actually switch on or off the intended set of lights.

The present specification does not contain any details on how such consistency between the onboarding procedure for the OCF-IPE and the selection of the OCF Servers to interact with can be achieved since this is implementation dependent. The stakeholder responsible for deploying and onboarding the OCF-IPE needs to be aware that the selected set of OCF Servers for which the OCF-IPE will get provisioned with credentials that allow access to that set of servers will actually be exposed to the oneM2M system. On the oneM2M side, appropriate Access Control Privileges need to be setup in order to control which oneM2M entities are authorized to consume the services provided by the set of exposed OCF Servers.

In order to avoid a loop of exposing services from one oneM2M SP domain to an OCF Proximal IoT Network and back to the same oneM2M SP domain, the OCF-IPE needs to verify that none of the selected OCF Servers to be exposed to oneM2M is actually an instantiation of an OCF Server that was previously instantiated by an OCF-IPE interfacing to the same oneM2M SP domain. This can be identified by inspecting the “dmno” (Model Number) property of the pre-defined URI “/oic/d” of the respective OCF Device defined in [7]. As specified in clause 8.2.2 of the present specification, OCF Servers instantiated by OCF-IPEs shall use a value for the “dmno” (Model Number) property that starts with the string “oneM2M-“ followed by a concatenation of the M2M-SP-ID of the OCF-IPE that instantiated the OCF Server and a the value for a model number as detailed in clause 8.2.2. If any of the OCF Servers selected for exposure to oneM2M uses a value for the “dmno” (Model Number) property in its “/oic/d” resource that starts with “oneM2M-“ and is followed by an M2M-SP-ID which is the same as the M2M-SP-ID of the oneM2M SP domain of the OCF-IPE intended to expose that OCF Server, the OCF Server shall be removed from the set of OCF Servers to be exposed to oneM2M.

As a result of onboarding and selection of the set of OCF Servers to be exposed, it is assumed in the remainder of the present specification that the following applies:

- The OCF-IPE got onboarded as an OCF Client with appropriate credentials to access the set of OCF Servers selected by the stakeholder responsible for the OCF-IPE deployment for exposure to the oneM2M system

- A valid set of OCF Servers to be exposed to oneM2M has been determined including avoidance of loopback of services from a oneM2M SP domain to an OCF Proximal IoT Network and back to the same oneM2M SP domain

- The set of “piid” property values (Protocol Independent ID) of the OCF device resource with the pre-defined URI “/oic/d” of the respective OCF Servers to be exposed is known to the OCF-IPE.

### 8.1.2	Handling of oneM2M Resource representing exposed OCF Functions

#### 8.1.2.1		<AE> resource representing an OCF-IPE and the associated set of exposed OCF Servers

When an OCF-IPE completes registration with its Registrar CSE, an <AE> resource representing that OCF-IPE has been created as a result of that registration. As specified in clause 7.1.2.2, the labels attribute of this <AE> resource shall reflect the fact that this AE is an OCF-IPE by adding a Key:Value pair set to “Iwked-Technology:OCF”, see Table 7.1.2-1. The registration of the OCF-IPE including a successful creation of the <AE> representing the OCF-IPE is a pre-requisite for creating any other oneM2M Resources representing OCF Functions exposed by this specific OCF-IPE.

oneM2M resources representing services provided by exposed OCF Servers which are exposed by a specific OCF-IPE shall be created as child resources of the <AE> resource representing that OCF-IPE.

For each OCF Server that is exposed to oneM2M by a specific OCF-IPE, the value of the “piid” property (Protocol Independent ID) of the device resource with the pre-defined URI “/oic/d” of the respective OCF Server as defined in the OCF Core Specification [7] shall be added to the list of IDs under the Key “Iwked-Entity-IDs” in the labels attribute of the <AE> resource representing that OCF-IPE as defined in Table 7.1.2-1. The addition of the value of the “piid” property to the list of IDs under the Key “Iwked-Entity-IDs” in the labels attribute of the <AE> resource representing the OCF-IPE exposing a specific OCF Server shall be performed when all other resources and subscriptions for exposing that OCF Server to the oneM2M system have been established, see clauses 8.1.2.2, 8.1.2.3, 8.1.3 and 8.1.4 for more details. Step 002 below defines the pre-requisites in detail.

When the OCF-IPE detects that a specific OCF Server is not accessible any longer – e.g. when the device resource with the pre-defined URI “/oic/d” of the respective OCF Server cannot be accessed any longer because it was removed from the OCF Proximal Network – the OCF-IPE shall remove the “piid” property value (Protocol Independent ID) of that specific OCF Server from he list of IDs under the Key “Iwked-Entity-IDs” in the labels attribute of the <AE> resource representing that OCF-IPE. In this case the OCF-IPE shall also delete any child resources of its own <AE> resource representing services provided by the respective OCF Server that is not accessible any longer, see also clause 8.1.2.2. Before the deletion of such child resources is performed, linkage to the respective <node> resource shall be updated as defined in clause 8.1.2.3, including possible deletion of <node> resources. It is an implementation choice of the OCF-IPE to choose an appropriate timeout for detecting absence of an OCF Server after an attempt to access it has not resulted in a valid response. Furthermore, it is also an implementation choice of the OCF-IPE to re-try to discover and expose OCF Servers which have previously been exposed to oneM2M but got removed from exposure due to inability to access them.

When the OCF-IPE detects that an additional OCF Server has to be exposed to the oneM2M system – this might be triggered on demand or by entering discovery procedure on the OCF side from time to time as described in clause 8.1.1 – the OCF-IPE shall complete verification of access to the targeted OCF Server, creation of child and <node> resources as defined in clauses 8.1.2.2 and 8.1.2.3 as well as establishing the state monitoring functions as described in clauses 8.1.3 and 8.1.4 below. Only then, the OFC-IPE shall add the value of the “piid” property of the respective OCF Server to the list of IDs under the Key “Iwked-Entity-IDs” in the labels attribute of the <AE> resource representing that OCF-IPE.

When an OCF-IPE is going to terminate, it shall properly de-register with its Registrar CSE, i.e. it shall delete the corresponding <AE> resource representing the OCF-IPE. With that deletion any child resources of this <AE> resource will also be deleted, terminating the exposure of any of the exposed OCF Functions. Before de-registration is performed, the OFC-IPE shall update the linkage between any previously created <flexContainer> resources and the respective <node> resource as defined in clause 8.1.2.3, including possible deletion of <node> resources. Figure 8.1.2.1-1 depicts a high-level flow of events and processing steps throughout the OCF-IPE life cycle. Some of the processing steps consist of complex sub-procedures partially relying on functionality specified in other clauses of the present specification. Therefore, the description of the different steps include references to the relevant clauses of the present specification. For now it should just serve the purpose of explaining the handling of an <AE> resource representing an OCF-IPE and the various actions the OCF-IPE has to take in order to comply with the specified way to expose OCF functions to oneM2M. Details of the more complex processing steps are defined in subsequent clauses of this present specification.

Step 001:	Register OCF-IPE
The OCF-IPE registers with its Registrar CSE. The representation of the <AE> resource which is requested to be created shall contain a labels attribute which includes a “key:value” pair of “Iwked-Technology:OCF”

Step 002: Establish pre-requisites
In order to perform a proper exposure of OCF functions to the oneM2M system, some pre-requisites have to be met. Step 002 contains what is needed to establish those pre-requisites. In particular the following needs to be completed:

- The set of OCF Servers to be exposed to oneM2M has to be determined, see clause 8.1.1 for a description on how to accomplish that. It is necessary to ensure via onboarding that the OCF-IPE is provisioned with the proper security credentials to access the set of OCF Servers to be exposed. Besides proper onboarding and selection of the set of OCF Servers to be exposed, the OCF-IPE shall also verify that it is able to access each of the selected OCF Servers and discard OCF Servers for which access is not possible. The set of OCF Servers to be exposed shall be characterized by the set of values of the “piid” property (Protocol Independent ID) of the device resources with the pre-defined URI “/oic/d” of the respective OCF Servers as defined in the OCF Core Specification [7].

- Once the set of OCF Servers to be exposed is determined, the OCF-IPE shall create oneM2M resources to represent the exposed services and physical devices. Depending on the OCF Device types and the normative mapping to oneM2M information models and resource types as defined in OCF Device-Specification [8] and oneM2M TS-0023 [6], one <flexContainer> plus possibly required sub-ordinated child resources shall be created for each exposed OCF Server, see clause 8.1.2.2. It is recommended that Access Control Privileges are set such that only the OCF-IPE and possibly administration entities can access these <flexContainer> resources. After creation of the <flexContainer> resources representing exposed OCF Servers, <node> resources representing the physical OCF Devices hosting the exposed OCF Servers shall be linked with the corresponding <flexContainer> resources and – if not already present – shall be created by the OCF-IPE on its Registrar CSE, see clause 8.1.2.3. In order to be able to detect any requests on the oneM2M side to consume the exposed OCF services – which may require a state change on the corresponding OCF Server – the  OCF-IPE shall establish subscriptions to the previously created oneM2M resources which expose such OCF Servers. See clause 8.1.4 for more details.

- After creation of the oneM2M resources needed by the OCF-IPE to expose OCF Servers to oneM2M, the OCF-IPE shall establish monitoring of the state of OCF Servers which are to be exposed by using the observe mechanism defined in OCF Core Specification [7]. The OCF-IPE shall be able to receive observation of changes of state of the exposed OCF Servers. The OCF-IPE shall also ensure that this monitoring is continuously performed throughout the lifecycle of the OCF-IPE. This may require re-establishment of the observe mechanism as defined in OCF Core Specification [7], see also clause 8.1.3

- Upon reception of a state change observation from exposed OCF Servers, the OCF-IPE shall mirror that state change by updating the corresponding oneM2M resource(s) created earlier, see clause 8.1.3 for more details. This particular sub-step 002d – i.e. the task to mirror any reported state changes of exposed OCF Servers to the state of the corresponding oneM2M resources – is actually a procedure that can be triggered asynchronously throughout the lifetime of the OCF-IPE and shall be executed as many times as needed.

Step 003: Add list of exposed entities
The OCF-IPE has established all pre-requisites to start the exposure of the OCF Servers to oneM2M. Before doing so, the OCF-IPE needs to include a list of all “piid” property (Protocol Independent ID) values of the exposed OCF Servers in the labels attribute of its own <AE> resource as a “key:value” pair in line with the format “Iwked-Entity-IDs:[ID1, ID2, ID3]”, see clause 7.1.2.2.

Step 004: Start exposure
In order to start the actual service exposure of OCF Servers to oneM2M entities, the OCF-IPE needs to set the ACPs which govern the access to the oneM2M resources representing OCF Servers such that the intended oneM2M entities are authorized to access the respective oneM2M resources.

Steps 005a, 005b, 005c define conditional procedures which shall be carried out under the conditions defined in the specific step descriptions below. They can be triggered asynchronously – i.e. not in the order described in the present specification, not at any pre-defined time and not in any particular order of execution. The execution of the procedures defined for each of these steps shall be executed as many times as needed.

Step 005a: Process oneM2M request to consume OCF service
This processing step is triggered asynchronously when receiving a notification caused by a request to change state in oneM2M resources that are representing exposed OCF Servers due to the subscription(s) established in Step 002b. The OCF-IPE shall decide on the need for issuing a corresponding request for state change to the relevant OCF Server and execute the corresponding request on the OCF side if needed. The OCF-IPE shall conditionally adjust oneM2M resource state depending on the outcome of executed OCF operation(s) and send a response to the received notification. Details for this step are defined in clause 8.1.4. Since this step is triggered by occurrences of notifications which were generated by subscriptions established earlier, see step 002b, the OCF-IPE shall execute this step for each of these notifications it receives.

Step 005b: Expose additional OCF Server(s)
This processing step is triggered asynchronously when the OCF-IPE detects a need to expose additional OCF Server(s). As described in clause 8.1.1, the set of OCF Servers to be exposed to oneM2M may change dynamically (e.g. addition on demand or by asynchronous discovery on the OCF side). It is possible that additional OCF Servers are identified and are intended to be exposed via an already initialized OCF-IPE. In such a case all the pre-requisites listed for step 002 need to be established for each additional OCF Server to be exposed. After these pre-requisites are met, the OCF-IPE needs to update the list  of “piid” property (Protocol Independent ID) values of all exposed OCF Servers in the labels attribute of its own <AE> resource as a “key:value” pair in line with the format “Iwked-Entity-IDs:[ID1, ID2, ID3]”, see clause 7.1.2.2. Ultimately the OCF-IPE needs to adjust ACPs for the resources representing the additionally exposed OCF Servers so that the intended oneM2M entities can consume the offered services. Step 005b shall be executed when the OCF-IPE detects the need to expose additional OCF-Servers which are currently not exposed to the oneM2M side. Methods for detecting such conditions are out of the scope of the present specification.

Step 005c: Stop exposure of a specific OCF Server
Upon detection of need to stop exposure of a specific OCF Server- e.g. when its reachability stalled or on demand – the OCF-IPE needs to go through the following procedure. Initially, the OCF-IPE shall update ACPs of the oneM2M resources representing the OCF Server to stop access by other oneM2M entities to the exposed services provided by the considered OCF Server. After that, the OCF-IPE shall stop any monitoring and mirroring of state of the OCF Server established in steps 002c and 002d, see clause 8.1.3. Furthermore, the OCF-IPE shall remove linkage from the <node> resource representing the OCF Platform hosting the OCF Server to the <flexContainer> representing the OCF Server including a possible deletion of the <node> resource if needed. See clauses 8.1.2.2, 8.1.2.3. After that, the OCF-IPE shall remove the OCF Server’s “ppid” property from the list associated with “Iwked-Entity-IDs” key in the labels attribute of the OCF-IPE <AE> resource. Finally, the OCF-IPE shall delete the <flexContainer> representing the OCF Server including its children.

Step 006: Prepare termination
When the OCF-IPE intends to terminate, it shall stop any active procedures for monitoring and mirroring of state of any of the remaining exposed OCF Servers, see steps 002c and 002d as well as clause 8.1.3. In addition, the OCF-IPE shall remove any linkage from <node> resources representing OCF Platforms hosting any of the exposed OCF Servers to <flexContainer> child resources under this OCF-IPE’s own <AE> resource including deletion of <node> resources if needed. See clauses 8.1.2.3.

Step 007: De-register OCF-IPE
The OPC-IPE shall delete the <AE> resource representing this OCF-IPE which will result also in deletion of all its child resources – and therefore – the removal of all remaining oneM2M resources representing OCF Servers exposed by this OCF-IPE.

After successful completion of the de-registration, the OCF-IPE has terminated from a oneM2M perspective. It is an implementation choice of the OCF-IPE whether it is also terminating any remaining activity on the OCF side – such as listening to further on-boarding requests or performing further discovery procedures to discover OCF Servers to be exposed in the future. These implementation choices are not in scope of the present specification.

Figure 8.1.2.1-1: High-level flow of events and processing steps throughout the OCF-IPE life cycle.

#### 8.1.2.2		<flexContainer> resources representing services provided by exposed OCF Servers

This clause describes the procedures related to the use of <flexContainer> resources – and possibly any required child resources thereof – to represent exposed OCF Servers to the oneM2M system.

##### 8.1.2.2.1	Parent resource and dependency on OCF Device types

In line with TS-0033 [5] and clause 7.1.3 of the present specification, services provided by exposed OCF Servers shall be represented by <flexContainer> resources on the oneM2M side – and possibly by child resources thereof. Which particular specialization of <flexContainer> resources shall be used to represent an OCF Server complying with a given OCF Device type depends on the information model mapping for OCF Devices defined in TS-0023 [6]. Depending on the applicable information model and the optional resources and properties of the specific OCF Server, the top level <flexContainer> resource used to represent a specific OCF Server may require additional child resources. The <flexContainer> resources representing exposed OCF Servers – and accordingly their child resources when needed – shall be created by the OCF-IPE that is exposing those OCF Servers as descendants of its own <AE> resource.

##### 8.1.2.2.2	Creation of <flexContainer> resources used to represent OCF Servers

###### 8.1.2.2.2.1	Pre-requisites

A pre-requisite to create <flexContainer> resources used to represent OCF Servers is that the OCF-IPE has previously determined which specific OCF Servers have to be exposed, see the description in clause 8.1.1 and step 002a in clause 8.1.2.1.

Proper determination of which OCF Server to expose via the considered OCF-IPE will ensure that the OCF-IPE is provisioned with the proper security credentials to access the set of OCF Servers to be exposed. Furthermore, the OCF-IPE shall also verify that it is able to access each of the selected OCF Servers and discard OCF Servers for which access is not possible. After this proper determination of the set of OCF Servers to be exposed has been performed, the set of  values of the “piid” property (Protocol Independent ID) of the device resources with the pre-defined URI “/oic/d” of the respective OCF Servers as defined in the OCF Core Specification [7] will be known to the OCF-IPE. In summary, the following shall be completed for each OCF Server to be exposed to the oneM2M system before the OCF-IPE requests creation of corresponding <flexContainer> resources representing exposed OCF Servers on the oneM2M side:

- proper onboarding of the OCF-IPE

- selection of set of OCF Servers to be exposed

- verification of access to the selected OCF Servers

- determination of the set of “piid” property (Protocol Independent ID) for the respective OCF Servers

These pre-requisites for the creation of <flexContainer> resources used to represent OCF Servers have to be established for both, the initial set of OCF Servers to be exposed (see step 002a in clause 8.1.2.1) as well as for any additional OCF Servers which may get added to the exposed set of OCF Servers later on (see step 005b in clause 8.1.2.1).

###### 8.1.2.2.2.2	Creation process

Reasons for creation

Throughout the lifecycle of an OCF-IPE, <flexContainer> resources used to represent OCF Servers – and possibly required child resources thereof – shall be created in the following cases:

- when the OCF-IPE has registered and an initial set of OCF Servers to be exposed has been determined before exposure of any OCF services has started, see step 002a in clause 8.1.2.1.

- when an already active OCF-IPE which is already exposing a set of OCF services to oneM2M has detected that one or more new OCF Servers have to be added to the set of exposed OCF servers, see step 005b in clause 8.1.2.1.

<flexContainer> resources which are meant to represent OCF Servers – and their required child resources – shall only be created when the pre-requisites outlined in clause 8.1.2.2.2.1 are established.

Creation of top level <flexContainer> resource and descendant resources

For each OCF Server to be exposed to oneM2M, the OCF-IPE shall create one top level <flexContainer> resource in line with the appropriate device model according to the mapping defined in TS-0023 [6] as a direct child of its own <AE> resource. For a given OCF Server offering services in line with a specific OCF Device type, the OCF-IPE needs to determine which optional resources are supported by this OCF Server. Each of these optional resources on the OCF side will require additional creation of the corresponding optional descendant resources of the top level <flexContainer> resource in the device models defined in TS-0023 [6]. The OCF-IPE shall also create these required descendant resources of the top level <flexContainer> resource.

Linkage with <node> resources

According to TS-0023 [6] and in line with clause 7.1.3 of the present specification, the nodeLink attribute of the top level <flexContainer> resource representing a given OCF Server shall be set to the resource identifier of a <node> resource representing the physical hardware on which an OCF Server is hosted, which is termed OCF Platform in OCF specifications. A <node> resource representing a specific OCF Platform contains a nodeID attribute set to the “pi” property (Platform ID) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7]. If such a <node> resource does not yet exist, it shall be created by the OCF-IPE, see clause 8.1.2.3 for more details. The hostedServiceLinks attribute of the <node> resource representing the OCF Platform for a given OCF Server shall be present and include the resource identifier of the top level <flexContainer> resource representing that given OCF Server.

labels Attribute settings

In line with clause 7.1.3 the labels attribute of the <flexContainer> resource shall contain a Key:Value pair set to “Iwked-Technology:OCF” indicating that this <flexContainer> supports interworking with OCF entities. It shall also contain a Key:Value pair Iwked-Entity-ID:{piid}” indicating the specific OCF Server being exposed to the oneM2M system by this specific <flexContainer>, where {piid} is a placeholder for the value of the “piid” property (Protocol Independent ID) of the device resource with the pre-defined URI “/oic/d” of the respective OCF Server as defined in the OCF Core Specification [7].

Update of parent <AE> resource after creation completed

After the creation of a <flexContainer> resource – including any required descendants - and after all other pre-requisites for exposing a given OCF Server are established – see step 002 in clause 8.1.2.1 - the parent <AE> resource of the respective <flexContainer> resource used to represent the given OCF Server needs to be updated to include the value of the “piid” property (Protocol Independent ID) of the respective OCF Server in the labels attribute under the “Iwked-Entity-IDs” key, see clause 7.1.2.2 of the present specification.

Handling of Access Control Privileges

At time of creation of <flexContainer> resources and descendant resources to represent OCF Servers, it is advised to set Access Control Privileges such that other entities are not able yet to consume the OCF services which shall be exposed by these resources. The reason for that is: The OCF-IPE has to establish a few more pre-requisites before starting the actual exposure – see step 002 in clause 8.1.2.1. Only after all these pre-requisites are established and after the parent <AE> resource has been updated with the proper information in the labels attribute under the “Iwked-Entity-IDs” key, the OCF-IPE should set the Access Control Privileges of <flexContainer> resources and descendant resources to represent OCF Servers such that intended oneM2M entities could eventually consume the exposed services.

##### 8.1.2.2.3	Deletion of <flexContainer> resources used to represent OCF Servers

Reasons for deletion

An OCF-IPE shall explicitly delete <flexContainer> resources it is using to represent OCF Servers when the OCF-IPE detects that one or more OCF Servers among the set of exposed OCF Servers must not be exposed to the oneM2M side any longer (e.g. due to removal from the OCF Proximal IoT Network), see step 005c in clause 8.1.2.1.

When the OCF-IPE intends to de-register – i.e. when it is planning to delete its own <AE> resource – any remaining child <flexContainer> resources will be deleted implicitly.

However, in both cases, certain preparations have to be performed for a proper end of exposure of OCF Servers, see the following paragraphs.

Handling of Access Control Privileges

In order to avoid any inconsistencies between linked resources and to avoid inconsistent exposure of information on which OCF services are exposed, it is recommended that the exposure of OCF services should be stopped before any of the following actions are taken. This is accomplished by adjusting Access Control Privileges such that no new requests will proceed to consume the services which shall no longer be exposed.

Update of parent <AE> resource before <flexContainer> deletion

This only applies when the OCF-IPE is intending to stop exposure of OCF services for a sub-set of the exposed OCF Server it is handling so far. In case the OCF-IPE intends to de-register, this update action is not needed.

Before deletion of any <flexContainer> resources used to represent OCF Servers the OCF-IPE shall remove any “piid” property (Protocol Independent ID) values of the OCF Servers that are meant to be no longer exposed from the labels attribute under the “Iwked-Entity-IDs” key, see clause 7.1.2.2 of the present specification.

Cleaning up llinkage with <node> resources

Before deleting any <flexContainer> resources it is using to represent OCF Servers, an OCF-IPE shall remove any linkage from the hostedServiceLinks attribute of the <node> resource representing the OCF Platform that hosts a given OCF Server and the respective <flexContainer> representing that OCF Server. In case the hostedServiceLinks attribute of that <node> resource does not contain any other resource identifiers, the <node> resource itself should be deleted as well, however, this is an implementation choice.

When an OCF-IPE is preparing to de-register, it shall also perform the described removal of such linkage from <node> resources to <flexContainer> resources it is using to represent OCF Servers, including the potential deletion of the respective <node> resources in case no other <flexContainer> resources are linked via the hostedServiceLinks attribute.

Explicit or implicit deletion

After all the conditions outlined in this clause are met, an OCF-IPE can either explicitly delete <flexContainer> resources it is using to represent OCF Servers which are meant to be no longer exposed or it can de-register, which has the effect of implicitly deleting all remaining <flexContainer> resources the OCF-IPE is using to represent OCF Servers.

##### 8.1.2.3 	<node> and [deviceInfo] resources representing OCF Devices

##### 8.1.2.3.1 	Creation

The physical hardware on which an OCF Server is hosted is termed OCF Platform in OCF specifications. It shall be represented by a <node> resource and a linked [deviceInfo] resource in line with the attribute settings defined in clause 7.1.1. Therefore, for each OCF Server for which a <flexContainer> resource is getting created, see clause 8.1.2.2, the OCF-IPE shall check if its own Registrar CSE hosts a <node> resource matching with the OCF Platform of the represented OCF Server. A match can be detected by comparing the nodeID attribute of all candidate <node> resources against the “pi” property (Platform ID) of the OCF resource with the pre-defined URI “/oic/p” of the respective OCF Device as defined in the OCF Core Specification [7]. If no match is found, a new <node> resource and a new [deviceInfo] resource shall be created, jointly representing the OCF Platform. The attribute settings of this pair of new resources and the linkage among them are defined in detail in clause 7.1.1.

##### 8.1.2.3.2	Linkage with <flexContainer> resources

A proper linkage between <flexContainer> resources representing OCF Servers and a <node> resource representing the OCF Platform hosting the respective OCF Server shall be established. The nodeLink attribute of <flexContainer> resources representing OCF Servers shall be set to the resource identifier of the <node> resource representing the OCF Platform which hosts that OCF Server. The hostedServiceLinks attribute of a <node> resource representing an OCF Platform shall contain a list of resource identifiers of all <flexContainer> resources representing OCF Servers hosted on that OCF Platform. Note that more than one resource identifier can be present in the hostedServiceLinks attribute of the <node> resource when more than one OCF Server is hosted on the same OCF Platform.

##### 8.1.2.3.3	Deletion

As long as a <node> resource representing an OCF Platform is linked via its hostedServiceLinks attribute to at least one <flexContainer> resource representing an exposed OCF Server, the <node> resource shall not be deleted. When all links to <flexContainer> resource representing an exposed OCF Server have been removed from the hostedServiceLinks attribute, this attribute shall not be present in the respective <node> resource any longer. In that case the <node> resource should get deleted by the OCF-IPE, however, this is an implementation choice.

### 8.1.3	Mirroring state of exposed OCF Servers in oneM2M Resources

#### 8.1.3.1	Establishment of monitoring

Once the appropriate set of resources to represent an OCF Server on the oneM2M side has been created, it is important to reflect accurate state information in the oneM2M resources so that they are kept consistent with the actual state of the OCF Servers that they represent.

In order to do so, the OCF-IPE shall establish monitoring of the state of all exposed OCF Servers. Such monitoring shall be accomplished by initializing and subsequently maintaining the “observe” mechanism as defined in the OCF Core Specification [7] for the respective OCF resources hosted on the respective OCF Servers. Using this observe mechanism enables the OCF-IPE to be informed about any state changes occurring in the exposed OCF Servers. The establishment of this monitoring is a pre-requisite for exposing OCF Servers as outlined in step 002c of clause 8.1.2.1

Establishment of monitoring for all relevant OCF resources requires the OCF-IPE to issue a number of requests on the OCF side with an “observe” option to retrieve all OCF Resources implementing OCF Device functions on all exposed OCF Servers. Using this “observe” option means that in addition to an initial response, subsequent responses will follow on each and every change of state in the observed resource. Therefore, the OCF-IPE shall be able to receive these subsequent responses and process them according to clause 8.1.3.2 below.

#### 8.1.3.2	Mirroring of state information

Upon reception of state information of OCF resources via the previously established observe mechanism, the OCF-IPE shall identify the oneM2M resource that corresponds to the observed OCF resource and translate the received observation into the corresponding request to update the state of the oneM2M resource representing the modified OCF resource. This procedure corresponds to step 002d in clause 8.1.2.1.

In order to avoid excessive discovery requests to find the right oneM2M resources for mirroring state information, the OCF-IPE shall keep a record of the mapping between OCF resources it observes and the corresponding oneM2M resources.

#### 8.1.3.3	Stop monitoring

When an OCF Server must no longer be exposed to the oneM2M side – this happens when steps 005c and 006 in clause 8.1.2.1 occur – then the previously established monitoring of the resources hosted by this particular OCF Server using the OCF observe mechanism needs to be terminated to avoid any further responses being sent to the OCF-IPE when state changes of these resources happen. Therefore, the OCF-IPE needs to cancel the observation of the resources it was so far observing for the given OCF Server by sending it a request indicating cancellation of the respective observation process.

### 8.1.4	Detection of requests from oneM2M entities to execute exposed OCF services and invocation thereof

#### 8.1.4.1	Monitoring of oneM2M resources representing OCF Servers

In order to find out when a properly authorized oneM2M entity would like to consume an OCF Service, the OCF-IPE needs to rely on subscriptions to the oneM2M resources that are representing the exposed OCF Servers. Subscriptions shall be established by the OCF-IPE to get notified whenever an authorized oneM2M entity is modifying the state of a oneM2M resource representing and exposing an OCF Server, where that state change needs to be translated into a corresponding state change request on the OCF side. This mechanism is part of step 002a in clause 8.1.2.1.

Note that not all services provided by an OCF Server require a state change of resources hosted on that OCF Server by the entity that is intending to consume that service. For instance an OCF Server that only provides the status of a window contact sensor may be implemented using a read-only resource that reflects the current state of the contact sensor. In such a configuration, exposure of the service to oneM2M only requires state mirroring from the OCF side to the oneM2M side as already described in clause 8.1.3.

In general, however, services provided by an OCF Server – for instance a light – may require monitoring of state changes initiated on the oneM2M side in order to detect when a state change needs to be requested on the OCF side – for instance when a an OCF light needs to be switched on or off by a oneM2M entity. Therefore, the OCF-IPE needs to decide which resources representing OCF Servers on the oneM2M side it needs to subscribe to. This decision depends on the specific device model as defined in TS-0023 [6] that was used to create the respective oneM2M resources representing OCF Servers. Only those resources that are meant to be modified (updated or deleted) by the service consuming entity need to be subscribed to.

When supported by the hosting CSE, the OCF-IPE shall use subscriptions with the condition tag notificationEventType in the eventNotificationCriteria attribute of the <subscription> resources set to “G”. This type of subscription will block any update request to the subscribed-to resource while sending a notification to the OCF-IPE and waiting for a response from the OCF-IPE. Only after receiving a response, the hosting CSE will complete the requested update operation. The outcome of the update operation will then depend on the status indicated in the notification response. This allows the OCF-IPE to take appropriate actions on the OCF side, see clause 8.1.4.2, before actually allowing a state change on oneM2M resource which are actually representing state that resides in OCF resources.

When the hosting CSE does not support setting “G” for the condition tag notificationEventType in the eventNotificationCriteria attribute of the <subscription> resource – for instance due to being implemented against on older release of oneM2M specifications – then the OCF-IPE has to take case of any remaining state alignments needed after the OCF interaction completed, see also clause 8.1.4.2 below.

#### 8.1.4.2	Invocation of OCF services

The procedure outlined in this clause correspond to step 005a in clause 8.1.2.1.

When the OCF-IPE has successfully established all pre-requisites for exposing a set of OCF-Servers to oneM2M – see step 002 in clause 8.1.2.1 – and when it has initiated the exposure – see steps 003 and 004 in clause 8.1.2.1 – it needs to detect relevant state changes of oneM2M resources that need to be translated into OCF requests for corresponding state changes. As specified in clause 8.1.4.1 above, the OCF-IPE is supposed to receive notification on such events based on subscriptions established earlier.

However, when receiving notifications triggered by the subscriptions established according to clause 8.1.4.1, not all notifications received will need to be translated into corresponding OCF requests. For instance if a notification was sent because of an update request issued by the OCF-IPE itself due to an incoming OCF observation, it shall not translate that back into another OCF request to avoid loopbacks. Furthermore, update requests on the oneM2M side may trigger a notification request while the actual state of the resource is not altered – e.g. when switching power state from on to on. Also in that case, the OCF-IPE shall not trigger any action on the OCF side.

Therefore, notifications received by the OCF-IPE need to be evaluated first and the OCF-IPE needs to decide whether a state change request is needed on the OCF side. If so, the OCF-IPE shall issue the corresponding state change request on the OCF side. Which exact request that is and what changed state values to request depends on the specific device model as defined in TS-0023 [6] that was used to create the respective oneM2M resources representing OCF Servers.

Upon completion of the OCF interaction – in case the OCF decided to initiate one – or after deciding that no OCF interaction was needed after receiving a notification, the OCF-IPE shall send a notification response. The following cases need to be treated differently:

- Subscription was established with the condition tag notificationEventType in the eventNotificationCriteria attribute of the <subscription> resources set to “G”:

  - No operation on the OCF side was needed: Send back a notification response back indicating successful reception of the notification and successful completion of requested operation.

  - A request was issued on the OCF side: Send a notification response back indicating a successful reception of the notification and a status for the requested operation corresponding to the outcome of the requested OCF operation.

- Subscription was established with a different setting of the condition tag notificationEventType in the eventNotificationCriteria attribute of the <subscription> resources or with no setting:

  - No operation on the OCF side was needed: Send back a notification response back indicating successful reception of the notification.

  - A request was issued on the OCF side: Send a notification response back indicating a successful reception of the notification. If the requested operation on the OCF side resulted in a different state than the current state of the subscribed-to resource, re-adjust the state of the subscribed to resource accordingly.

In order to better understand case 2b above a short example is provided: a) oneM2M entity tries to switch an OCF light from on to off b) State in oneM2M resource representing OCF light gets changed. c) Notification request is sent to OCF-IPE. d) OCF-IPE request OCF Server to turn off light. e) OCF Server denies request (might be overridden by some locking). f) OCF-IPE sends back notification response. G) OCF-IPE reverts state of oneM2M resource back from off to on.

### 8.1.5	Overall flows for procedures supporting exposure of OCF Functions to the oneM2M System

In this clause the overall flow of events and processing steps is summarized as depicted in Figure 8.1.5-1 for the case that a static set of OCF Servers – only determined once during the life cycle of the OCF-IPE – is meant to be exposed to the oneM2M system. For enumerating steps carried out on the OCF side the letters A-D are used, for the steps on the oneM2M side numbers 1-9 are used. All the details of the flow are defined in the previous clauses 8.1.1 through 8.1.4. In essence Figure 8.1.5-1 is a more specific variant of Figure 8.1.2.1-1 outlining in more details which steps happen on the OCF side versus the oneM2M side.

Note that the processing steps within the boxes shaded in gray may occur multiple times and are triggered by OCF observe responses or oneM2M notifications, respectively.

Figure 8.1.5-1: Overall flow of processing steps when exposing a static set of OCF Servers to oneM2M entities

## 8.2	Procedures supporting exposure of native oneM2M services to an OCF Proximal IoT Network

### 8.2.1 	Determination of oneM2M services to be exposed to the OCF Proximal IoT Network

#### 8.2.1.1	Summary

TBD.

#### 8.2.1.2		Provisioning

TBD

#### 8.2.1.3	Discovery

TBD

#### 8.2.1.4	On demand determination

TBD

### 8.2.2	Instantiation / removal of OCF Servers representing exposed oneM2M services

TBD

### 8.2.3	Mirroring state of oneM2M Resources representing exposed oneM2M services in OCF resources hosted on OCF Servers

TBD

### 8.2.4	Detection of requests to consume exposed oneM2M services and execution thereof

TBD

Annex A (Informative)

# Examples

# History

| Publication history |  |  |
| --- | --- | --- |
| V2.0.0 | 30-Aug-2016 | Release 2 - Publication |
| V2.0.1 | July 2017 | Clean-up done by editHelp! e-mail: mailto:edithelp@etsi.org |
| V3.0.0 | 21-Dec-2017 | Initial Releae 3-Version Implementation of agreed CR in: ARC-2017-0410R01 |
| V3.1.0 | 07-Sept-2018 | Implementation of agreed CR in: ARC-2018-0036R03 |