| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0035-V-0.2.0 |
| Document Name: | Developer guide of device management |
| Date: | 2017-08-15 |
| Abstract: | The document provides example of device management in oneM2M. |
| Template Version: 08 September 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols and abbreviations	4

3.1	Definitions	4

3.2	Symbols	5

3.3	Abbreviations	5

4	Conventions,	5

5	Device management over the service layer	5

5.1	Introduction	5

5.2	Use case	6

5.3	Architecture	6

5.3.1	Management of ASN, MN and IN	6

5.3.2	Management of ADN	7

5.3.3	Management of NoDN	8

5.4	Procedures	9

5.4.1	Management of ASN, MN and IN	9

5.4.2	Management of ADN	10

5.4.3	Management of NoDN	11

5.5	Implementation	11

5.5.1	Introduction	11

5.5.2	Management of ASN, MN and IN	12

5.5.3	Management of ADN	12

5.5.4	Management of NoDN	13

6	Device management using external management technologies	13

6.1	Introduction	13

6.2	Use case	13

6.3	Architecture	14

6.4	Procedures	15

6.5	Implementation	15

Proforma copyright release text block	16

Annexes	16

Annex <y>: Bibliography	17

History	17

# 1	Scope

The present document gives example of message flow and procedures on how device management will be performed using oneM2M defined primitives and procedures.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

Not applicable.

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf)

[i.2]     oneM2M TS-0001: "Functional Architecture".

# 3	Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

ARIB	Association of Radio Industries and Businesses

ATIS	Alliance for Telecommunications Industry Solutions

CCSA	China Communications Standards Association

ETSI	European Telecommunications Standards Institute

TIA	Telecommunications Industry Association,

TSDSI	Telecommunications Standards Development Society

TTA	Telecommunications Technology Association

TTC	Telecommunication Technology Committee

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

# 4	Conventions,

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Device management over the service layer

## 5.1	Introduction

oneM2M uses <mgmtObj> resource for device management. For device management over the service layer, the <mgmtObj> that is used for the management of the managed entity is located at different places in case of the characteristic of the managed entity. In either of the case, there exists one driver or client on the managed entity that observes the update of the <mgmtObj> resource to perform management operation.

For ASN, MN and IN which has CSE, the <mgmtObj> is hosted on the corresponding ASN-CSE, MN-CSE and IN-CSE as child resource of the <node> resource that represents the ASN, MN and IN. The managed entity observes the <mgmtObj> directly to perform the management operation.

For ADN, the <mgmtObj> resource is hosted on the registrar CSE of the ADN-AE as child resource of the <node> resource that represents the ADN. The ADN-AE in this case should subscribe the <mgmtObj> resources under the corresponding <node> resource to receive notifications on any update of the <mgmtObj> resource. The ADN which is the managed entity in this case further perform the management operation based on the notifications received.

For NoDN, the <mgmtObj> resource is hosted on the CSE which is NoDN is connected to. As NoDN is not an oneM2M defined entity, the connection between NoDN and the CSE is out of scope of oneM2M and is implementation specific. In this case, the <mgmtObj> resource on the CSE under the corresponding <node> resource should be monitored to determine if any management operation need to be performed. This part is implementation specific and is out of scope of oneM2M as well.

## 5.2	Use case

An application want to retrieve the current available memory of the device. In this case, the specialization of <mgmtObj> memory is used Annex D.4 of [2]. In the memory specialization, memAvailable is the attribute that contains the required information. The application can get to know the available memory by retrieving the resource.

Figure: 5.2-1: Use case for management

## 5.3	Architecture

### 5.3.1	Management of ASN, MN and IN

Figure: 5.3.1-1: Architecture for management of IN,MN,ASN

In the architecture for management of IN, MN and ASN. As those Managed Entities have their own CSEBase resource, they have the capability to host oneM2M resources. When using device management over the service layer, the [memory] resource is directly hosting under the <node> resource of the CSEBase. In this case, the <node> resource represents the IN, MN, ASN themselves.

The Driver is the software that interacts with the memory of the device that acquires the total memory and available memory from the system.

In this case, the [memory] specialization is modified by Driver using internal interface. The modification may be done periodically or some other policies which is out of scope of oneM2M standard.

Whenever the AE issues retrieve to the [memory] specialization, the value from the resource is returned.

### 5.3.2	Management of ADN

Figure: 5.3.2-1: Architecture for management of ADN

In the architecture of management of ADN, the [memory] specialization is hosted on the Registrar CSE of the ADN. According to the supported configuration of oneM2M architecture, the Registrar CSE could be IN-CSE or MN-CSE.

In this case, the <node> resource hosted under the CSEBase of the Registrar CSE represents the ADN which is the managed entity. The driver gets the memory information using internal interfaces and make the ADN-AE updates the memory value to the [memory] specialization over the Mca reference point. The update over Mca is triggered periodically or based on the policy of the ADN which is implementation specific.

### 5.3.3	Management of NoDN

Figure: 5.3.3-1: Architecture for management of NoDN

In the architecture of management of NoDN, the [memory] specialization is hosted on the connection point of NoDN. The connection point is the entity that the NoDN is connected to. The entity has the adaptor that shares the same network protocol with NoDN.

In this case, the <node> resource hosted under the CSEBase represents the NoDN. And the connection between Driver and the CSE is out of scope of oneM2M. It may be Bluetooth, ZigBee or even proprietary.

The update of the memory information happens over the interface between driver and CSE which is out of scope of oneM2M.

## 5.4	Procedures

### 5.4.1	Management of ASN, MN and IN

Figure: 5.4.1-1: Procedures for management of IN, MN, ASN

001: The AE send an oneM2M Retrieve primitive to the ResourceID of [memory] resource.

002: The CSE receives the Retrieve primitive and triggers the Driver to acquire the current available memory value, update the [memory] resource.

003: The CSE responds the AE with the representation of the [memory] resource.

Note: The AE could also subscribe the [memory] resource to get the notification on the update of the [memory] resource.

The [memory] resource is this case is locally created before hand by the Hosting CSE using internal interfaces.

### 5.4.2	Management of ADN

Figure: 5.4.2-1: Procedures for management of ADN

001: The AE sends Retrieve request to Registrar CSE of ADN-AE. Requesting the [memory] resource.

003: The Registrar CSE responds with the representation of the [memory] resource.

002: The ADN internally acquires the available memory information.

004: The ADN-AE updateds the [memory] resource using Mca reference point.

005: Update successful

Note that 001, 003 and 002, 004 don’t have a specific order. The ADN is updating the [memory] resource periodically. In 003, the Registrar CSE is responding always the current resource.

Subscritpion and notification could also be used by the AE to monitor the current status of the [memory] resource.

The [memory] resource in this case is created by ADN-AE.

### 5.4.3	Management of NoDN

Figure: 5.4.3-1: Procedures for management of NoDN

001: The AE sends Retrieve request to connecton point of NoDN. Requesting the [memory] resource.

003: The CSE of the connection point responds with the representation of the [memory] resource.

002: The NoDN internally acquires the available memory information.

004: The NoDN updateds the [memory] resource using protocol that is non-oneM2M.

Note that 001, 003 and 002, 004 don’t have a specific order. The NoDN is updating the [memory] resource periodically. In 003, the Registrar CSE is responding always the current resource.

The [memory] resource in this case is created by NoDN via non oneM2M interface.

## 5.5	Implementation

### 5.5.1	Introduction

In this implementation,  the current document is using HTTP binding and JSON serialization as examples, and SP-relative-structured resource addressing format is used.

Following resource ID and/or entity ID is used:

- Memory Resource

  - Resource Name "memory001"

  - Resource ID "m343245334adf" represented in CSE-relative Unstructured-Resource-ID of the [memory] resource.

  - Parent Resource ID "n893051036jdg"

- AE-ID "C324352841DAS" of the managed entity

- Registrar/Hosting CSE

  - SP-relative CSE-ID "/CSE0034234"

  - CSE Resource Name "server"

- Host name of the Registrar/Hosting CSE "in.management.server.com".

### 5.5.2	Management of ASN, MN and IN

| Step | Message Example |
| --- | --- |
| 001 | HTTP Request GET /~/CSE0034234/server/memory001 HTTP/1.1 Host : in.management.server.com X-M2M-RI : 1234 X-M2M-Origin: C324352841DAS Accept : application/json |
| 002 | HTTP Response 200 OK X-M2M-RI : 1234 X-M2M-RSC : 2000 { "m2m:mgo" : { "ri":"m343245334adf", "pi":"n893051036jdg", "ct":"20161129T121550", "et":"20181231T235959", "lt":"20161130T152341", "ty":13, "rn":"memory001", "mgd":1003, "mma":800, "mmt":1024 } } |

### 5.5.3	Management of ADN

| Step | Message Example |
| --- | --- |
| 001 | Same as 001 in 5.5.2 |
| 002 | Same as 002 in 5.5.2 |
| 004 | HTTP Request PUT /~/CSE0034234/server/memory001?rcn=0 HTTP/1.1 Host : in.management.server.com X-M2M-RI : 1235 Content-Type: application/json X-M2M-Origin: C324352841DAS { "m2m:mgo":{ "mma":600 } } |
| 005 | HTTP Response 200 OK X-M2M-RI : 1235 X-M2M-RSC : 2004 |

### 5.5.4	Management of NoDN

001 and 003 is the same as in clause 5.5.2.

002 and 004is protocol specific and even proprietary, thus it’s out of scope of oneM2M.

# 6	Device management using external management technologies

## 6.1	Introduction

In the case of device management using external management technologies such as OMA DM, LWM2M or BBF TR069. All management is done through IN-CSE, since only the IN-CSE has the capability to talk to the Management Server. Therefore, in this case, all <node> resource that represents the managed entity is hosted under the <CSEBase> of IN-CSE. The manged entity may be IN, MN, ASN, ADN or NoDN. Then the <mgmtObj> that is used for management is hosted as the child resource of <node> resource. The attribute objectIDs and objectPaths of the <mgmtObj> are set in this case to map to the external management objects.

## 6.2	Use case

An application want to retrieve the current  avaiable memory of the device. In this case, the specialization of <mgmtObj> memory is used Annex D.4 of [2]. In the memory specialization, memAvailable is the attribute that contains the required information. The application can get to know the available memory by retrieving the resource.

Figure: 6.2-1: Use case for management

## 6.3	Architecture

Figure: 6.3-1: Architecture for management using external technology

In the case of management using external management technology, all management related resources are hosted on the IN-CSE. IN-CSE interacts with Management Server through ms interface and triggers the Management Server to send management commands to Managed Entity through the mc interface. mc and ms interface is defined by external management technology or implementation specific which is out of scope of oneM2M.

The Management Server may be deployed together with the IN-CSE, in this case, the IN-CSE directly sends out management commands through mc interface to the Managed Entity. The Management Server may also be deployed separately from the IN-CSE, in this case, the IN-CSE need to send commands to Management Server to trigger tell the Management Server to send out management commands to the Managed Entity. ms interface may be implementation specific.

## 6.4	Procedures

Figure: 6.4-1: Procedures for management to ADN

001: AE retrieves [memory] resource specialization from IN-CSE over mca interface.

002: On receiving the request, the IN-CSE sends management command to managed entity using external management technology implementation.

003: On the managed entity, the driver aquires the memory information.

004: The managed entity responds with aquired memory information over the mc interface.

005: The IN-CSE updates the [memory] resource and respond the RETRIEVE request with the representation of [memory] resource.

## 6.5	Implementation

In the implementation, the document is using LWM2M as the external management technology. Resource ID and/or entity ID that are defined in Section 5.5.1 is also applicable to current section.

In addition, host name of the managed entity is named as "adn.managed.entity.com", the memory resource name is "memory002", the memory resource ID is "m463245245afg", and the parent resource ID of the memory resource is  "n993051028asd". The request URL for the managed entity is assumed as "coap://adn.managed.entity.com/adn001/memory".

| Step | Message Example |
| --- | --- |
| 001 | HTTP Request GET /~/CSE0034234/server/memory002 HTTP/1.1 Host : in.management.server.com X-M2M-RI : 3454 X-M2M-Origin: C324352841DAS Accept : application/json |
| 002 | CoAP Request Method : 0.01 (GET) Uri-Host : adn.managed.entity.com Uri-Path : adn001 Uri-Path : memory oneM2M-FR : C324352841DAS oneM2M-RQI: 4553 Payload : empty |
| 004 | CoAP Response X-M2M-RSC : 2.05 (OK) oneM2M-RQI : 4553 Content-Format: 50 (application/json) Payload : {memory_object_serialized_in_JSON} |
| 005-2 | HTTP Response X-M2M-RI : 3454 X-M2M-RSC : 2000 { "m2m:mgo" : { "ri":"m463245245afg", "pi":"n993051028asd", "ct":"20161129T121550", "et":"20181231T235959", "lt":"20161130T152341", "ty":13, "rn":"memory002", "mgd":1003, "mma":800, "mmt":1024 } } |

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A>:
Title of annex (style H9)

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 09 Nov 2016 | Initial skeleton |
| V.0.1.0 | 9 Dec 2016 | Incoporated contributions: TST-2016-0229R03-Service_layer_management_developer_guide TST-2016-0230R02-underlying_network_management_developer_guide |
| V.0.2.0 | 15 Aug 2017 | Incoporated contributions: TST-2017-0175-CR-Update_TR-0035 |