| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0038-V-0.2.1 |
| Document Name: | Developer guide: Implementing security example |
| Date: | 2017-10-09 |
| Abstract: | The document provides a simple use case for guiding developers to implement security when developing applications using functionalities provided by a oneM2M service platform. |
| Template Version: 08 September 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2017, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Conventions	6

5	Use case	6

6	Functional architecture	7

7	Procedures and call flows	9

7.1	Security Association Establishment	9

7.1.1	Security Requirements	9

7.1.2	Provisioned Symmetric Key SAE between the Locks and the Home Gateway	9

7.1.3	Certificate-based SAE between Home Gateway and IN-CSE	11

7.1.4	MAF-based SAE between Smartphone and IN-CSE	11

7.1.5	Registration upon successful SAE	11

7.2	Authorisation	11

7.3	Secure communications	11

8	Implementation	11

8.1	Definition and assumption	11

8.2	Resource structures	11

8.3	Roles of entities	11

8.4	Procedures	12

9	Conclusions	12

Proforma copyright release text block	12

Annexes	12

Annex A:	12

Security Association Establishment Message Flows	12

A.1  Introduction	12

A.2  PSK-Based Security Association Establishment	13

Annex <y>: Bibliography	16

History	17

# 1	Scope

This Technical Report aims at providing guidelines to developers to implement security as specified by oneM2M TS-0003, using a simple use case as example. It addresses the initial security provisioning for enrolment with a Service Provider, and the operational phase relying on a Security Association Establishment process to implement secure connection and access control services for basic use cases.

As example, the considered use cases are implementing a home door lock service with:-

- Authentication

- Authorisation

- Integrity

- Confidentiality

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

Not applicable.

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf)

[i.2]		oneM2M TS-0001: "Functional Architecture".

[i.3]		oneM2M TS-0004: "Service Layer Core protocol Specification”.

[i.4]		oneM2M TS-0003: "Security Solutions".

[i.5]		oneM2M TS-0011: "Common Terminology".

[i.6]		oneM2M TR-0025: "Application Developer Guide"

# 3	Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability.

- A definition shall not take the form of, or contain, a requirement.

- The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

- The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

ARIB	Association of Radio Industries and Businesses

ATIS	Alliance for Telecommunications Industry Solutions

CCSA	China Communications Standards Association

ETSI	European Telecommunications Standards Institute

TIA	Telecommunications Industry Association,

TSDSI	Telecommunications Standards Development Society

TTA	Telecommunications Technology Association

TTC	Telecommunication Technology Committee

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Use case

This guide is based on a smart key use case involving front and back door locks in a home that can be remotely controlled by a user's smartphone leveraging the capabilities of oneM2M. For example, the user can remotely open the doors when friends, relatives, housekeeper or babysitter come to the user’s home. However, if a system of the use case is not secured, attackers can easily unlock the door locks by spoofing.

An overview of the use case is shown in figure 5.1-1 and the main components are introduced as follows:

- The door locks are deployed in a home and are connected to a home gateway.

- The home gateway communicates with a cloud service platform allowing the door locks to be controlled remotely by the smartphone.

- The cloud service platform supports a set of services to enable the smartphone to more easily control the door locks in the home. Some examples of services include registration, discovery, data management, group management, subscription/notification etc

- The smartphone hosts an application used to remotely control the door locks in the home and supports the following capabilities:

  - Discovery of door locks deployed in the home.

  - Sending commands to change door lock states i.e. LOCKED and UNLOCKED.

  - Retrieval of door lock states.

  - Receiving notifications when certain events occurred.

- M2M Authentication Function (MAF) is used when employing MAF-based Security Association Establishment (SAE) between field nodes and infrastructure nodes. When using Pre-Shared Key or Certificate-based SAE, the MAF is not required.

Figure 5.1-1 Overview of remote door locks control use case

# 6	Functional architecture

This clause describes how the different components of this use case can be represented by corresponding oneM2M architectural entities as shown in figure 6.1-1.

Figure 6.2-1 oneM2M functional architecture of remote door locks control use case

An IN-CSE is hosted in the cloud by the oneM2M Service Provider and a MN-CSE is hosted on the Home Gateway. Applications and MAF used in the current use case are classified as follows:

- ADN-AE1: an application embedded in Front Door Lock with capabilities to control Front Door Lock and interact with the home gateway MN-CSE through Mca reference point;

- ADN-AE2: an application embedded in Back Door Lock with capabilities to control Back Door Lock and interact with the home gateway MN-CSE through Mca reference point;

- ADN-AE3: an application embedded in the smartphone device with capabilities to interact directly with the oneM2M service platform IN-CSE through Mca reference point used to remotely control Front Door Lock and Back Door Lock;

- MN-AE: a gateway application embedded into the home gateway that interacts with the MN-CSE through Mca reference point.

- MAF: M2M Authentication Function assigns symmetric keys to MAF clients on the IN-CSE and the ADN-AE3 through Mmaf reference point.

# 7	Procedures and call flows

## 7.1	Security Association Establishment

### 7.1.1	Security Requirements

M2M services are offered by CSEs to AEs and/or other CSEs. To be able to use M2M services offered by one CSE, the AEs and/or CSEs need to be mutually identified and authenticated by that CSE, in order to provide protection from unauthorized access and Denial of Service attacks.

This mutual authentication enables to additionally provide encryption and integrity protection for the exchange of messages across a single Mca, Mcc or Mcc' reference point. In addition, communicating AEs that require similar protection for their own information exchanges can be provisioned to apply the same security method to their communications. This is the purpose of the Security Association Establishment (SAE) procedure.

When CoAP binding of oneM2M primitives is used, i.e. the Underlying Network communication uses UDP/IP transport, Authentication is performed by means of a DTLS Handshake.

When HTTP, MQTT or WebSocket binding of oneM2M primitives is used, i.e. the Underlying Network communication uses TCP/IP transport, Authentication is performed by means of a TLS Handshake.

For the use cases in this guideline document it is assumed that HTTP binding is employed between all applicable pairs of entities (see also TR-0025 [i.6])

In order to exemplify the use of all three Security Association Establishment Frameworks (SAEF) defined in TS-0003 [i.4] the following use cases are described:

- Provisioned Symmetric Key SAE between Door Locks and Home Gateway,

- Pre-provisioned Certificate Based SAE between Home Gateway and IN-CSE,

- MAF Based Symmetric Key SAEF between the smartphone and IN-CSE.

Communication between the MN-AE and MN-CSE internally to the Home Gateway is assumed to not require Security Association Establishment.

### 7.1.2	Provisioned Symmetric Key SAE between the Locks and the Home Gateway

In this example it is assumed that authentication between the Locks (ADN-AE1 and ADN-AE2) and the Home Gateway (MN-CSE) is performed using provisioned keys (Kpsa) and key identifiers (KpsaID).

Configuration of ADN-AE1 and ADN-AE2:

- The AEs are configured with the set of allowed TLS ciphersuites when using TLS-PSK as defined in clause 10.2.2 of TS-0003 [i.4]. The set of ciphersuites includes TLS_PSK_WITH_AES_128_CBC_SHA256.

- The AE is assumed to be configured with the CSE-ID of the Home Gateway which is a unique identifier within the M2M-SPs domain. The CSE-ID value is assumed as mn-cse-123456.

- The AE is assumed to be configured with a pair of credentials (psk, psk_identity) associated with the CSE-ID. An example of credential configuration is given in Table 7.1.2-1. The length of the keys Kpsa is not mandated by TS-0003 [i.4] and left to implementation. In this example the key length of 8 bytes (64 bits) is chosen. The key identifiers comply with the format specified in clause 10.5 of TS-0003 [i.4].

Table 7.1.2-1: Example Credentials configured on ADN-AE1 and ADN-AE2

| Entity | Kpsa (hex format) | KpsaID |
| --- | --- | --- |
| ADN-AE1 | 1a2b3c4d5e6f7a8b | AE123456789012-Lock@in.provider.com |
| ADN-AE2 | 12345678abcdefab | AE123456789015-Lock@in.provider.com |

Configuration of MN-CSE (Home Gateway):

- The MN-CSE is configured with the set of allowed TLS ciphersuites when using TLS-PSK as defined in clause 10.2.2 of TS-0003 [i.4]. The set of ciphersuites includes TLS_PSK_WITH_AES_128_CBC_SHA256.

- The MN-CSE is assumed to have a psk-lookup-table with columns for (client identity, psk, psk_identity), such that when a TLS client provides a particular psk_identity, then the MN-CSE uses the corresponding psk for establishing a TLS session, and the client identity is associated with the established TLS session. This needs to be integrated to the TLS server. Table 7.1.2-2 shows an example of credentials configured on the Home Gateway to serve ADN-AE1 and ADN-AE2, containing AE-ID, KpsaID, Kpsa. A new row would need to be added to this table for each additional AE allowed to register to the MN-CSE by using TLS_PSK.   .

NOTE: Some open source libraries, e.g. OpenSSL, do not provide a psk-lookup-table, but do indicate a spot in the source code where a psk-lookup could be implemented. The psk-look-up-table values could then be provided in a configuration file.

Table 7.1.2-2: Credentials configured on MN-CSE

| AE-ID | Kpsa (hex format) | KpsaID |
| --- | --- | --- |
| Clock-AE1 | 1a2b3c4d5e6f7a8b | AE123456789012-Lock@in.provider.com |
| Clock-AE2 | 12345678abcdefab | AE123456789015-Lock@in.provider.com |

Operation of ADN-AE1 and ADN-AE2

When the AE is triggered to establish a TLS-PSK session with the MN-CSE using some pair (Kpsa, KpsaID), the following should occur automatically based on the AE’s configuration:

- AE’s TLS Client is triggered to perform a TLS-PSK handshake with the TLS values (psk, psk_identity) set to the values of (Kpsa, KpsaID), and with the configured list of TLS ciphersuites.

- On completion of the TLS handshake, the AE associates the established TLS session with the MN-CSE’s CSE-ID.

Operation of MN-CSE

The MN-CSE’ TLS Server is listening on the TLS Server port and the following should occur automatically based on the MN-CSE’s configuration:

- A TLS handshake is started at the MN-CSE TLS Server on receiving a TLS handshake Client_Hello message. In the case of the AE, this includes the list of TLS-PSK ciphersuites supported by the AE for use with the MN-CSE. The MN-CSE will select a ciphersuite that is also in its configured list.

- A later TLS handshake message will include the psk_identity element set to KpsaID.

- The MN-CSE’s TLS Server looks up the psk-lookup-table using KpsaID as an index, and retrieves the AE’s (AE-ID, Kpsa). If AE-ID is not available, then the MN-CSE may query the node’s <serviceSubscribedAppRule> resource.

- The MN-CSE’s TLS client continues the TLS handshake with the TLS value psk set to the value of Kpsa.

- On completion of the TLS handshake, the MN-CSE associates the established TLS session with the AE’s AE-ID.

Annex A provides details for implementing the TLS handshake procedure.

### 7.1.3	Certificate-based SAE between Home Gateway and IN-CSE

<Text>

### 7.1.4	MAF-based SAE between Smartphone and IN-CSE

<Text>

### 7.1.5	Registration upon successful SAE

Editor’s note: this clause will provide an example of the registration procedure following successful Security Association Establishment. This procedure is independent of the SAE procedures described in clauses 7.1.2 to 7.1.4. It will also include an example of AE impersonation checking procedure.

<Text>

## 7.2	Authorisation

<Text>

## 7.3	Secure communications

<Text>

# 8	Implementation

## 8.1	Definition and assumption

<Text>

## 8.2	Resource structures

<Text>

## 8.3	Roles of entities

<Text>

## 8.4	Procedures

<Text>

# 9	Conclusions

<Text>

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

# Annex A:

# Security Association Establishment Message Flows

## A.1  Introduction

This Annex presents some example message flows which are useful to understand the operation of the oneM2M security establishment frameworks, to verify correct operation or to identify the cause of misbehavior.

Some details of TLS message flows and message content depend on the employed SSL/TLS implementation. Implementations of oneM2M entities will typically make use of SSL/TLS libraries to enable support of the required security functions specified in TS-0003. Examples of open source SSL/TLS libraries include OpenSSL, gnuSSL and mbed TLS.

Such SSL/TLS libraries implement the basic cryptographic functions and provide various utility functions such as e.g. TLS clients and servers which may be executed from a command line.

The message flows shown here have been produced using OpenSSL Version 1.1.1 on an Ubuntu 14.04 computer using the s_client and s_server utility functions, and employing Wireshark for capturing and analyzing the exchanged data packets.

The commands given in the subsections below may be used to reproduce these flows.

## A.2  PSK-Based Security Association Establishment

A typical flow of messages and actions for a successful PSK-Based Security Association Establishment is shown in figure A.1-1. The message content described in the steps below applies to the example described in clause 7.1.2.

Subsequent to TCP connection establishment (not shown in the Figure), the following messages are exchanged between ADN-AE1 and the MN-CSE:

- The TLS client on ADN-AE1 sends a Client Hello Handshake message which is encapsulated in a TLS Record layer frame. The record layer message includes the following fields:

- Record layer header fields:

      - Content type 0x16 (Handshake)

      - Version 0x0301 (indicating TLS 1.0)

      - Length of the message (2 bytes, value depending on the message content)

- Application data (handshake message):

      - Handshake Type 0x01 (Client Hello)

      - Length of the message (3 bytes, value depending on the message content)

      - Client Version 0x0303 (TLS 1.2)

      - (Client) Random (32 bytes, generated by the TLS client’s pseudo random number generator (PRNG))

      - Number of cipher suites supported by the client (value at least 1)

      - List of cipher suites. Must include identifier for TLS_PSK_WITH_AES_128_CBC_SHA256 (0x00ae)

      - Extension length and Extensions (irrelevant for this example)

- The TLS server handshake protocol responds with Server Hello and Server Hello Done messages. For the implementation employed here, each of these messages is encapsulated into a dedicated record layer frame.

- Record layer header fields:

      - Content type 0x16 (Handshake)

      - Version 0x0303 (indicating TLS 1.2)

      - Length of the application data field  (2 bytes, value depending on the message content)

- Application data (“Server Hello” handshake message):

      - Handshake Type 0x02 (Server Hello)

      - Length of the message (3 bytes, value depending on the message content)

      - Server version 0x0303 (indicating TLS 1.2)

      - (Server) Random (32 bytes, generated by the TLS server’s PRNG)

      - Session-Id length (0x00, no session ID supplied)

      - Number of cipher suites supported by the client (at least 1)

      - Cipher suite selected by the server is TLS_PSK_WITH_AES_128_CBC_SHA256 (0x00ae)

      - Compression method (null, no compression)

      - Extension length and Extensions (irrelevant for this example)

- Record layer header fields:

      - Same as in step 2.i

- Application data (“Server Hello Done” handshake message):

      - Handshake type 0x0e (Server Hello Done)

      - Length of the message (0x0000, message has no content)

- The TLS client responds with Client Key exchange, Change Cipher Spec, Finished messages. For the implementation employed here, each of these messages is encapsulated into a dedicated record layer frame.

- Record layer header fields:

      - Same as in step 2.i

- Application data (“Client Key Exchange” handshake message):

      - Handshake Type 0x10 (Client Key Exchange)

      - Length of the message (3 bytes, value depending on the message content)

      - PSK client parameters:

        - Identity length ( 0x00000f in this example)

        - PSK Identity (here binary equivalent of “Client_identity”)

- Record layer header fields:

      - Content type 0x14 (Change Cipher Spec)

      - Version 0x0303 (TLS 1.2)

      - Length of the message (0x0001)

- Application data (“Change Cipher Spec” message):

      - Change Cipher Spec message 0x01 (1 byte)

- Record layer header fields:

      - Same as in step 2.i

- Application data (encrypted “Finished” handshake message)

      - Handshake type 0x14 (Finished)

      - Length of the message 0x00000c (12)

      - Verify Data (12 bytes), see RFC 5246, section 7.4.9.

- The server retrieves Kpsa associated with the PSK Identity, computes the master secret and authenticates the client by validating Verify Data

- The TLS server responds with New Session Ticket, Change Cipher Spec, Finished messages. For the implementation employed here, each of these messages is encapsulated into a dedicated record layer frame.

- Record layer header fields:

      - Same as in step 2.i

- Application data (“New Session Ticket” handshake message):

      - Handshake Type 0x04 (New Session Ticket)

      - Length of the message (3 bytes: 0x0000b6)

      - Session Ticket:

        - Lifetime Hint ( 4 bytes: 0x00001c20, 7200 in this example)

        - Session Ticket Length  (2 bytes, 0x00b0, 176 in this example)

        - Session Ticket (176 bytes),   see RFC 4507, server session state enabling session resumption

- Record layer header fields:

      - Content Type 0x14 (Change Cipher Spec)

      - Version 0x0303 (TLS 1.2)

      - Length of the message (0x0001)

- Encrypted application data (“Change Cipher Spec” message):

      - Change Cipher Spec message 0x01 (1 byte)

- Record layer header fields:

      - Same as in step 2.i

- Application data (encrypted “Finished” handshake message, to verify that the key exchange and authentication processes were successful):

      - Handshake Type 0x14 (Finished)

      - Length of the message 0x00000c (12)

      - Verify Data (12 bytes), see RFC 5246, section 7.4.9.

- The client authenticates the server by validating Verify Data

- Application data encrypted by the TLS record layer is exchanged between ADN-AE1 and MN-CSE

Figure A.1-1: PSK-Based Security Association Establishment

The message flow described above (excluding step 7) can be reproduced with the following commands under Linux OS using localhost IP address and port 443:

TLS server on MN-CSE:

$ sudo openssl s_server -accept 443 -psk 1a2b3c4d5e6f7a8b

TLS Client on ADN-AE1:

$ openssl s_client -connect 0.0.0.0:443 -psk_identity Client_identity \

-psk 1a2b3c4d5e6f7a8b -cipher PSK-AES128-CBC-SHA256

NOTE: 	The OpenSSL  s_server utility does not support table lookup of pre-shared keys when using the option

-psk_identity AE123456789015-Lock@in.provider.com

as required for the example in clause 7.1.2. Therefore the above command line for the server includes the used PSK itself. The client command line provides the PSK identity “Client_identity” which is expected by the server for this PSK.

Note that in order to enable Wireshark to decrypt application data which has been encrypted by the TLS record layer, it must be configured as follows:

In the Wireshark configuration menu Edit -> Preferences -> Protocols -> SSL,

- In the “Pre-Shared-Key” field, enter Kpsa, i.e. 1a2b3c4d5e6f7a8b

- In the (Pre)-Master-Secret log filename field, enter the name of a text file which includes Client Random (32 bytes as 64 hex characters) and the Master Secret (48 bytes as 96 hex characters) as a text line as follows:

CLIENT_RANDOM <space> 64-characters-random <space> 96-characters-Master-Secret

The master secret is provided as log information in the terminal window, where s_client is started. The value of Client Random can be retrieved from the Wireshark packet capture in the Client Hello handshake message.

First the data captured with Wireshark must be stored into a file. Then, after configuring Wireshark as described above, the messages in the saved data file can be decrypted by Wireshark.

Editor’s note:  relation between credential identifiers, entity identifiers and service subscription information needs to be clarified

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.1 | 05 December 2016 | Initial skeleton |
| V0.1.0 | 21 February 2017 | Integration of contributions agreed during TP 27: SEC-2017-0009R02 SEC-2017-0020R02 SEC-2017-0021R02 |
| V0.2.0 | 05 April 2017 | Integration of contributions agreed during TP 28: TST-2017-0097R01 |
| V0.2.1 | 09 October 2017 | Integration of contributions agreed during TP 28: SEC-2017-0138R01 |