| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0053-V-0.3.0 |
| Document Name: | Lightweight oneM2M Services |
| Date: | 2019-06-05 |
| Abstract: | The document is a study of lightweight oneM2M services. Based on the result of the study, it identifies proposed optimizations and enhancements to the oneM2M system to streamline and optimize its features and services. |
| Template Version: January 2017 (Do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this report.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2017, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Conventions	6

5	Introduction	6

6	Analysis of oneM2M Functionality	6

6.1	Analysis of oneM2M Messages and Potential Requirement	6

6.1.1	Introduction of oneM2M Messages	7

6.1.2	Limitations of oneM2M Messages	8

6.1.3	Potential Requirements	11

6.1.4	Potential Solutions	11

6.2	Analysis of oneM2M Requests Targeted Towards AEs	12

6.2.1  Introduction of oneM2M Requests Targeted Towards AEs	12

6.2.2  Limitations of oneM2M Requests Targeted Towards AEs	13

6.2.3  Potential Requirements	17

6.2.4	Potential Solutions	17

6.2.4.1 Solution 1: Retargetting via a New Attribute resourceMappingRules	17

6.2.4.1.1 Definition of a New Attribute resourceMappingRules	17

6.2.4.1.2 Retargeting Procedure based on a New Attribute resourceMappingRules	20

6.2.4.2 Solution 2: Access Control for Retargeting Requests to an AE	22

6.2.4.3 Solution 3: Using Subscription/Notification to Retarget RETRIEVE Requests	23

7	Conclusions	25

Annexes	26

History	27

# Scope

This document is a study of lightweight oneM2M services. Based on the result of the study, it identifies proposed optimizations and enhancements to the oneM2M system to streamline and optimize its features and services.

# References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non- specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## Normative references

Normative references are not applicable in the present document.

## Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

# Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## Definitions

Clause numbering depends on applicability.

A definition shall not take the form of, or contain, a requirement.

The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABREVIATION1>	<Explanation>

<ABREVIATION2>	<Explanation>

<ABREVIATION3>	<Explanation>

# Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# Introduction

Editor’s Note:  This section provides background information, including: 1) primary features of constrained IoT devices and constrained IoT access networks; 2) the challenges for supporting oneM2M service functions on constrained IoT devices; 3) the benefits of designing lightweight oneM2M services.

# Analysis of oneM2M Functionality

Editor’s Note: This section analyses and summarizes potential limitations and/or optimizations to the existing oneM2M functionality (e.g. subscription and notification mechanisms, access control mechanisms, addressing schemes, request/response primitives and message exchange, resource discovery mechanism, registration procedures, CSE-to-AE re-targeting mechanisms, etc.) and potential requirements.

## Analysis of oneM2M Messages and Potential Requirement

### Introduction of oneM2M Messages

Figure 6.1.1-1 from oneM2M TS-0001 shows the general communication flow in oneM2M that governs the information exchange over Mca, Mcc, and Mcc’ reference points  (i.e. between an Application Entity (AE) and a Common Service Entity (CSE) or between two CSEs), which is based on the use of Request and Response messages. Such communications can be initiated either by the AEs or by the CSEs depending upon the operation in the Request message. For example, an AE can send a request message to a hosting CSE for application registration; then, the AE can send another request message to the hosting CSE to subscribe to certain resources; lastly, the hosting CSE may send request messages (i.e. notifications) to the AE when the status of subscribed-to resources has been changed.

Figure 6.1.1-1: General Flow in oneM2M

oneM2M provides several advanced features for processing a request message and generating a corresponding response message via some new request parameters as described in Table 6.1.1-1. For example, the parameter “Request Expiration Timestamp”, if included in a request message, indicates when the request message expires. Similarly, each response message contains some response parameters as listed in Table 6.1.1-2. For instance, the parameter “Result Expiration Timestamp ”, if contained in a response message, indicates when the response message expires. In addition, the parameter “Content” contains a resource representation, which can be present in a request message for creating/updating a resource or in a response message for retrieving a resource. The resource representation includes the attributes of the targeted resource.

Table  6.1.1-1: Summary of Request Parameters in oneM2M (Adapted from oneM2M TS-0001)

| Request Message Parameters |  |
| --- | --- |
| Mandatory | Operation - operation to be executed |
|  | To - the address of the target resource on the target CSE |
|  | From - the identifier of the message Originator |
|  | Request Identifier - uniquely identifies a Request message |
| Operation dependent | Content - to be transferred |
|  | Resource Type - of resource to be created |
| Optional | Originating Timestamp - when the message was built |
|  | Request Expiration Timestamp - when the request message expires |
|  | Result Expiration Timestamp - when the result message expires |
|  | Operational Execution Time - the time when the specified operation is to be executed by the target CSE |
|  | Response Type - type of response that shall be sent to the Originator |
|  | Result Persistence - the duration for which the reference containing the responses is to persist |
|  | Result Content - the expected components of the result |
|  | Event Category - indicates how and when the system should deliver the message |
|  | Delivery Aggregation - aggregation of requests to the same target CSE is to be used |
|  | Group Request Identifier - Identifier added to the group request that is to be fanned out to each member of the group |
|  | Filter Criteria - conditions for filtered retrieve operation |
|  | Discovery Result Type - format of information returned for Discovery operation |
|  | Security Info - information about the Content if it contains security parameters |
|  | Token Request Indicator - indicating that the Originator may attempt Token Request procedure (for Dynamic Authorization) if initiated by the Receiver |
|  | Tokens - for use in dynamic authorization |
|  | Token IDs - for use in dynamic authorization |
|  | Role IDs - for use in role based access control |
|  | Local Token IDs - for use in dynamic authorization |

Table 6.1.1-2: Summary of Response Parameters in oneM2M (Adapted from oneM2M TS-0001)

Response Message Parameters

Response Status Code - successful, unsuccessful, ack

Request Identifier - uniquely identifies a Request message

Content - to be transferred

To - the identifier of the Originator or the Transit CSE that sent the corresponding non-blocking request

From - the identifier of the Receiver

Originating Timestamp - when the message was built

Result Expiration Timestamp - when the message expires

Event Category - what event category shall be used for the response message

Security Info - information about the Content if it contains security protocols

### Limitations of oneM2M Messages

Request and response parameters as listed in Table 6.1.1-1 and Table 6.1.1-2 could make a oneM2M request or response message large. In the meantime, multiple consecutive request (or response) messages could contain the same request (or response) parameters with the same value; in another case, resource attributes contained in the “Content” parameter could be unexpected by the requestor or redundant in multiple consecutive messages. Such a large request or response message could be a burden for constrained IoT devices and/or networks with limited communication bandwidths. For example, the maximum layer-2 frame size for LoRaWAN at both US 915 MHz and EU 868 MHz band is 250 bytes, and the frame size in IEEE 802.15.4 networks is up to 127 bytes. The length of an oneM2M message if containing various request and response parameters could easily go beyond the maximum frame size which can be supported by the underlying Low-Power Wide Area Networks (LPWAN) like LoRaWAN or Low-Power Wireless Personal Area Networks (LoWPAN) such as IEEE 802.15.4.

Figure 6.1.2-1 illustrates a smart metering use case, where each cellular UE smart meter uses low-power wide-area access technologies such as 3GPP Narrow-Band Internet of Things (NB-IoT) to communicate with a Server where an IoT service layer resides for storing and managing meter data from various UEs; the Server could be deployed by an electricity company. Basically, there could be a smart meter application running on each UE to periodically send meter readings to the Server. In addition, multiple smart meters (e.g. deployed in the same community) may report their readings to the Server in the same way (e.g. reporting frequency, how request messages shall be processed by the Server, etc.). As such, each smart meter may repeatedly send similar request messages to the Server, and multiple meters may also send similar request messages to the Server at different times. These two aspects are abstracted and discussed furthermore in Figure 6.1.2-2 and Figure 6.1.2-3.

Figure 6.1.2-1: Smart Metering Use Case based on Cellular IoT

Figure 6.1.2-2 illustrates interactions between an application and a service layer. In this example, the application (e.g. smart meter application on a smart meter in Figure 6.1.2-1) repeatedly sends request messages to the service layer. Each request message contains a set of request parameters; likewise, the corresponding response message contains response parameters. In addition, the application may request the same services/resources from the service layer during certain time durations; thus, each repeated request message includes the same set of request parameters.

Figure 6.1.2-3 shows another example, where multiple applications (e.g. smart meter applications on smart meters in Figure 6.1.2-1) interact with the same service layer. In this scenario, although three (or more) applications could access different service/resources, they may instruct the service layer to process their request messages in the same way. For example, they may indicate to the service layer: the same request message expiration time, the same result expiration time, etc. Therefore, the request message from each application may contain the same set of request parameters.

Figure 6.1.2-2: Interactions between an Application and a Service Layer

Figure 6.1.2-3: Interactions between Multiple Applications and a Service Layer

The following limitations of oneM2M communication flow for constrained IoT devices are identified:

- If a  oneM2M request (or response) message contains too many request (or response) parameters, it cannot be sent in one layer-2 frame, which causes high message overhead and increases message transmission latency.

- If a oneM2M request (or response) message is contained in multiple layer-2 frames, the loss of one layer-2 frame will results in the failure of oneM2M message delivery.

- Even if a oneM2M request (or response) message can be completely contained in one layer-2 frame, the large size of the oneM2M message increases the message loss probability over the wireless channel.

- Parameters contained in multiple consecutive oneM2M request (or response) messages could be the same and  redundant, which increases message overhead unnecessarily.

- The resource representation in the “Content” parameter may contain some extra attributes which are not expected or required by the requestor.

- The resource representation in the “Content” parameter from multiple consecutive oneM2M request (or response) messages may contain the same or redundant attributes, which causes extra message overhead unnecessarily.

- The oneM2M service layer does not currently support any compression mechanisms and is unable to leverage existing header compression protocols (e.g. IETF RFC 3095 for robust header compression) which have been designed for network layer, transport layer, and application protocol layer.. None of these are directly applicable to the oneM2M service layer.

### Potential Requirements

- The oneM2M System shall support suitable request/response message interaction between a service layer and a constrained IoT device with low latency.

- The oneM2M System shall support suitable request/response message interaction between a service layer and a constrained IoT device with low communication overhead.

- The oneM2M System shall support suitable approaches for constrained IoT device to minimize request message size.

- The oneM2M System shall support suitable approaches for constrained IoT device to minimize response message size.

- The oneM2M System shall support suitable approaches for constrained IoT device to remove unrequired or redundant attributes from the resource representation as contained in the “Content” parameter.

### Potential Solutions

Editor’s Note:  The section describes potential solutions related to optimizing/enhancing the oneM2M function to address the identified limitations and requirements.

## Analysis of oneM2M Requests Targeted Towards AEs

### 6.2.1  Introduction of oneM2M Requests Targeted Towards AEs

oneM2M AEs currently rely on receiving oneM2M requests via the oneM2M subscription/notification framework.  Figure 6.2.1-1 illustrates how the oneM2M subscription/notification framework is used by an AE to receive a request originating from another AE.   For example, in a real world use case deployment, AE1 could function as a door lock device and AE2 could represent an application hosted on a smartphone that is used to lock and unlock the door.

In this use case, AE1 first creates a resource in the Hosting CSE for the purposes of receiving commands from other AEs in the system (e.g. AE2). AE1 then creates a subscription to this resource such that it is notified by the Hosting CSE if the resource is updated.  AE2 then performs an update of the resource which results in the Hosting CSE sending a notification to AE1.  The update response that is returned back to AE2 can be returned immediately or the Hosting CSE can wait until it receives a notification response from AE1.  This is determined by the type of subscription that AE1 creates in the Hosting CSE.

Figure 6.2.1-1: oneM2M AE Subscription/Notification-based Request Targeting

### 6.2.2  Limitations of oneM2M Requests Targeted Towards AEs

The oneM2M architecture currently does not support end-to-end re-targeting of requests such as RETRIEVE, UPDATE, DISCOVER, etc. to AEs as illustrated in Figure 6.2.2-1.  A oneM2M AE is currently limited to receiving requests only via the oneM2M subscription/notification framework.  As a result an AE can only receive oneM2M NOTIFY requests.  This limitation is a bi-product of the fact that the current oneM2M architecture requires oneM2M AEs to mirror their resources into CSE hosted resources in order for these resources to be made accessible to other oneM2M entities.

Figure 6.2.2-1: oneM2M Lacks End-to-End Re-targeting of Requests to AEs

Due to the lack of support for re-targeting non-NOTIFY requests to AEs in oneM2M there are several limitations and inefficiencies that can be problematic for certain types of real world deployments.

Limitation #1 – This limitation involves use cases requiring mirrored resources hosted in a CSE that need to be to be kept current (i.e. fresh) with the state of an AE. For example, the current state of a device needs to be known (i.e. what is the current location of the device).   Different types of use cases can have different freshness requirements for mirrored resources.  Based on the freshness requirements, an AE has to keep its mirrored resources fresh enough to meet these requirements.  This is done by issuing repeated update requests to the CSE on a frequent enough basis as shown in Figure 6.2.2-2.  The AEs consuming the information from these mirrored resources however may be issuing retrieve requests on a much less frequent basis even though they require the resources to be kept current with a certain freshness.  Since the AE mirroring its resources has no insight into when its mirrored resources are being accessed it has to continuously update them in case they are accessed.   This example demonstrates how, in some deployment scenarios, mirroring of resources is not always an efficient deployment model.  In this type of use case, re-targeting a request down to a device hosting an AE rather than mirroring of resources can be more efficient.

Figure 6.2.2-2: oneM2M AE Mirroring Inefficiencies

Limitation #2 – This limitation involves the oneM2M subscription/notification framework and how it is not an optimal mechanism for re-targeting requests to an AE hosted on a lightweight device.  Currently the oneM2M subscription/notification framework does support the capability to re-target UPDATE requests to an AE as illustrated in Figure 6.2.1-1.  This is done by sending a NOTIFY request to an AE when an UPDATE request is received to the AE’s mirrored resource.  Within the NOTIFY request, the updated representation of the mirrored resource is provided.  When this NOTIFY request is sent to the AE, the Hosting CSE holds-off sending the UPDATE response to the Originator until the Hosting CSE receives a NOTIFY response back from the AE.  When the AE receives the NOTIFY it can extract and process the resource representation from the payload.  The AE can then send a NOTIFY response back to the Hosting CSE indicating whether it successfully processed the updated resource representation and it is ok for the Hosting CSE to apply the update to the mirrored resource.  The Hosting CSE in turn processes the NOTIFY response, conditionally updates the mirrored resource, and returns a response back to the Originator.  The major drawback with  re-targeting UPDATE requests to an AE via the oneM2M subscription/notification framework is the extra overhead incurred with tunneling/encapsulation the updated resource representation with the NOTIFY request.  This tunneling/encapsulation adds extra overhead to the message size as well as message processing.

Other than UPDATE, the oneM2M subscription/notification framework does not currently support re-targeting other types of requests (e.g. RETRIEVE) to AEs.  Re-targeting these other types of requests to an AE can be very useful since it can allow an Originator’s RETRIEVE request to be forwarded to AE if/when the mirrored resource does not contain a representation that meets the Originator’s freshness requirements (i.e. it’s too stale).  Re-targeting a RETRIEVE via the oneM2M subscription/notification framework however is more complex and less efficient than the UPDATE and for this reason oneM2M has not defined support for this capability.  For example, if oneM2M were to entertain this idea, then it would most likely have to define something like what is shown in Figure 6.2.2-3.  The added complexity/inefficiency in this procedure is the additional UPDATE request from AE1 to the Hosting CSE when AE1 receives a NOTIFY request that a RETRIEVE to its mirrored resource in the Hosting CSE was received from AE2.  This UPDATE is needed to provide the Hosting CSE with an updated representation that the Hosting CSE can then use to update the representation of its mirrored resource and include in the response back to the Originator.   From this figure, one can see that re-targeting RETRIEVE requests to AEs via the oneM2M subscription/notification framework has its limitations and complexities and is not very efficient.  For this reason, oneM2M should analyze other alternatives.

Figure 6.2.2-3: oneM2M AE Subscription/Notification-based RETRIEVE Request Targeting

The following is a summarized list of limitations in the current oneM2M architecture regarding targeting requests towards oneM2M AEs:

- A oneM2M AE is currently limited to receiving only oneM2M NOTIFY requests.  An AE is not permitted to receive any other type of oneM2M request (i.e. CREATE, RETRIEVE, UPDATE, DELETE or DISCOVER).

- A oneM2M AE must mirror its locally hosted resources into CSE hosted resources (e.g. containers, flexContainers, timeSeries, mgmtObj, etc) such that they can be accessed by other entities in the oneM2M system.

- Mirrored resources containing AE state information may become stale depending on how often an AE updates the mirrored resources and therefore may not meet the freshness requirements of the other entities accessing these mirrored resources.

- oneM2M lacks the capability to initiate the re-fresh of a stale resource. oneM2M lacks the capability for a Hosting CSE to obtain an updated representation of a mirrored resource from an AE if/when a RETRIEVE to the mirrored resource requires a fresher representation

- For an AE to receive an updated representation of a mirrored resource in a Hosting CSE, it must first create and SUBSCRIBE to this resource such that updated resource representations can be encapsulated and tunnelled to the AE via a oneM2M NOTIFY request.  This tunnelling/encapsulation adds message latency and processing overhead to the AE as well as the end-to-end communication path.

### 6.2.3  Potential Requirements

- The oneM2M System shall support request/response message interaction with M2M Devices with minimal latency.

- The oneM2M System shall support request/response message interaction with M2M Devices with minimal number of request/response messages.

- The oneM2M System shall support request/response message interaction with M2M Devices with minimal request/response message size.

- The oneM2M System shall support the capability to initiate the update (i.e. refresh) of a resource by its creator if/when the representation of the resource is too old (i.e. stale) to meet the requirements of a requester.

### Potential Solutions

Editor’s Note:  The section describes potential solutions related to optimizing/enhancing the oneM2M function to address the identified limitations and requirements.

#### 6.2.4.1 Solution 1: Retargetting via a New Attribute resourceMappingRules

##### 6.2.4.1.1 Definition of a New Attribute resourceMappingRules

A new common attribute resourceMappingRules is proposed.  This attribute can be supported by various oneM2M resources such as <flexContainer>, <mgmtObj>, <container>, <timeSeries>, etc.  As illustrated in Figure 6.2.4.1.1-1, resourceMappingRules serves the following purposes:

- First, it indicates the mapping from a given oneM2M resource to local resource(s) hosted at an AE. Figure 6.2.4.1.1-1 presents an example, where an AE registers to a CSE and a <AE> resource is created. A <Rsrc-1> child resource of <AE> is also created which has some attributes and child resources as well. The AE hosts some local resources such as <localRsrc-1>, <localRsrc-2>, etc.  The resourceMappingRules contains mapping rules that indicate the relationship between oneM2M resources/attributes and locally hosted resources at the AE. Five mapping rule examples are illustrated in Figure 6.2.4.1.1-1. For instance, attribute-1 of <Rsrc-1> maps to <localRsrc-1> locally hosted at the AE. Not shown in Figure 6.2.4.1.1-1, but resourceMappingRules can also include other information such as retargeting filter criteria, retargeting schedule, retargeting policies, etc.

- The Hosting CSE can automatically and intelligently determine if and how a received CRUD request on a given resource/attribute should be retargeted to an AE based on information contained in resourceMappingRules. Still taking Figure 6.2.4.1.1-1 as an example, if the Hosting CSE receives a RETRIEVE request on attribute-1 of <Rsrc-1>, it will automatically retarget or forward the RETRIEVE request to <localRsrc-1> at the AE according to the MappingRule-1.  As also shown in Figure 6.2.4.1.1-1, resourceMappingRules can be applied to the given resource <Rsrc-1> and/or its child resources, which can be described as retargeting scope and included in resourceMappingRules as well.

In summary, this new common attribute resourceMappingRules can be introduced to various types of existing or future new oneM2M resources (e.g. <flexContainer>, <mgmtObj>, <container>, <timeSeries>, etc.) and it can be leveraged by the Hosting CSE to flexibly and efficiently determine and perform retargeting a received CRUD requests to an AE.

Figure 6.2.4.1.1-1: Mapping between oneM2M Resources/Attributes at a Hosting CSE and Local Resources at an AE

resourceMappingRules is formally defined in Table 6.2.4.1.1-1 as a common attribute.

Table 6.2.4.1.1-1: Definition of resourceMappingRules Attribute

| Attribute Name | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceMappingRules | 0..1 (L) | RW | This common attribute contains a list of rules for mapping the attributes and child resources for a resource on the Hosting CSE to corresponding resources and attributes locally hosted by an AE. Each mapping rule contains information for retargeting a CRUD request to an AE such as retargeting schedule, retargeting filter criteria, etc. Table 6.2.4.1.1-2 gives the definitions of the elements of this attribute. The mapping information stored in this attribute enables a Hosting CSE to access the services hosted and offered by an AE by retargeting a received oneM2M request primitive as a simplified CRUD request to an AE for processing. Once the AE finishes processing the retargeted CRUD request and returns a response, the Hosting CSE can use this information to update its local resource and formulate a oneM2M response primitive that it returns back to the Originator AE/CSE of the oneM2M request. |

Table 6.2.4.1.1-2 Definition of resourceMappingRules Elements

| Element | Multiplicity | Note |
| --- | --- | --- |
| name | 1 | This element is configured with a name of a resource attribute hosted by the Hosting CSE. Optionally a wildcard character is allowed to specify mapping rules applicable to all attributes (e.g. "*") of a resource. |
| mappedID | 1 | For the case when the name element of a mapping rule contains a single resource attribute name, this element contains the identifier of the mapped resource attribute hosted by an AE. For the case when a wildcard character is used in the name element of a mapping rule, this element contains the identifier of the mapped resource hosted by an AE. The identifier is configured with the identifier of an <AE> resource followed by a path to the corresponding resource and attribute hosted by the AE. The identifier of the <AE> resource can be formatted as a structured or unstructured CSE-Relative-Resource-ID, SP-Relative-Resource-ID or Absolute-Resource-ID. The following is an example. /CSE0001/AE0001/foo/bar/ Where /CSE0001/AE0001 is a structured SP-Relative-Resource-ID of an <AE> resource and foo/bar is the path to the resource attribute hosted by the AE. A Hosting CSE uses this information to determine the proper pointOfAccess and requestReqchability stored in the <AE> resource. Based on this information, the Hosting CSE can determine which underlying protocol, network address (e.g. IP or FQDN) and port to use when issuing requests to this AE resource attribute. Note, if a Hosting CSE needs to access multiple attributes of the same AE hosted resource, the Hosting CSE can issue a single request to the AE that performs a request on multiple attributes. Note, if a Hosting CSE needs to access multiple attributes of different AE hosted resources, the Hosting CSE issues different requests to each AE hosted resource. |
| retargetMode | 0..1 | Defines the mode of retargeting performed by the Hosting CSE when receiving a request targeting this resource attribute. See retargetMode definition in Table 6.2.4.1.1-3. |
| retargetFilterCriteria | 0..1 | Defines conditions for when the Hosting CSE will retarget received requests that target this attribute. |
| retargetSchedule | 0..1 | Defines a schedule for when the Hosting CSE will retarget received requests that target this attribute. |
| retargetScope | 0..1 | If this element is not included, then the scope of the retargeting rules apply to the parent resource of the resourceMappingRules attribute. For example, if resourceMappingRules is an attribute of a <flexContainer> resource, the Hosting CSE applies retargeting rules to the attributes of the <flexContaier> resource. If this element is included, then it is configured with the type of child resource that the Hosting CSE will apply the resourceMappingRules. For example, if resourceMappingRules is an attribute of a <container> resource and this element is configured with resourceType of a <contentInstance> resource, the Hosting will apply the retargeting rules to requests it receives that target child <contentInstance> resources of the <container>. |

Table 6.2.4.1.1-3: Definition of retargetMode

| Value | Interpretation |
| --- | --- |
| 0 | The Hosting CSE never retargets received requests targeting this attribute. This is the default mode. |
| 1 | The Hosting CSE always retargets received requests targeting this attribute. |
| 2 | The Hosting CSE conditionally retargets received requests targeting this attribute based on the specified retargetFilterCriteria and retargetSchedule elements of the attribute's resourceMappingRules. |

For example, assume: 1) a Hosting CSE hosts a <flexContainer> resource named “device1” that has custom attributes named “doorSensor” and “doorLock”; 2) An AE hosts a resource named “d1” that is mapped to this <flexContainer> resource and has corresponding attributes named “ds” and “dl” mapped to the “doorSensor” and “doorLock” attributes, respectively.

To map these resources with one another, a resourceMappingRules attribute is configured in the device1 <flexContainer> resource.  In this example, only the doorSensor and doorLock attributes are mapped.  Retargeting is enabled always for the doorLock attribute to allow interaction to take place. The resourceMappingRules attribute is configured as follows using JSON serialization, but XML or CBOR could also be used:

"resourceMappingRules": [	{"name": "doorSensor", "mappedID": "AE01/d1/ds", "retargetMode": 0}

{"name": "doorLock", "mappedID": "AE01/d1/dl", "retargetMode": 1}]

##### 6.2.4.1.2 Retargeting Procedure based on a New Attribute resourceMappingRules

The procedure for leveraging the new resourceMappingRules attribute to enable retargeting of CRUD requests to AE hosted resources is illustrated in Figure 6.2.4.1.2-1 and described in detail below.

Figure 6.2.4.1.2-1: Retargeting a CRUD Request to an AE based on resourceMappingRules

Step 000: An AE is registered to a Hosting CSE, and an <AE> resource is created for the Registree AE. In addition, one or more data sharing resources (e.g. <flexContainer>) are created for the Registree AE on the Hosting CSE.

Step 001: A request is sent to the Hosting CSE to configures the resouceMappingRules for a given resource (e.g. <flexContainer>).  The resouceMappingRules define rules that map to resource(s) hosted by the AE.

Editor’s Note: FFS - Investigate whether there should be restrictions or privileges enforced by a Hosting CSE to limit retargeting of a requests for a given attribute to only certain entities (e.g. creator or owner of a resource).

Step 002: The Hosting CSE configures resouceMappingRules for the given resource.

Step 003: The Hosting CSE returns a response.

Note: The Registree AE may not perform Steps 001-003 itself. Instead, another AE (e.g. a system administrator AE) on behalf of the Registree AE may perform these steps.

Step 004: An Originator AE (or CSE) sends an oneM2M request (e.g. RETRIEVE or UPDATE) to a given resource that has a resouceMappingRules attribute configured.

Step 005: The Hosting CSE processes the oneM2M request. The Hosting CSE checks whether it needs to retarget the request to mapped AE resource(s) based on the resourceMappingRules defined in the clause 6.2.4.1.1.  The Hosting CSE will perform the following:

- Based on the targeted resource or attribute(s) (e.g. device1 or device1/doorSensor or  device1/doorLock), the Hosting CSE will parse the resourceMappingRules attribute of the targeted resource for each applicable attribute and detect if retargeting the request is required.   This is done by checking the retargetMode setting for each attribute targeted by the request.

E.g. UPDATE CSE01/AE01/device1/doorLock "OPEN"

- If retargeting is required, the Hosting CSE figures out which Registree AE(s) to retarget request(s) to.  This is done by checking the pointOfAccess information from the <AE> resource(s) associated with the AE-ID(s) configured in the mappedID element(s) of the resourceMappingRules attribute.  The pointOfAccess will provide the Protocol to use, the IP address and the port.  Using this info the Hosting CSE appends the path info (e.g. "/d1/a") to form the URI of where to send a retargeted request.  For example,

E.g. coap://192.152.25.10:5683/d1/dl

Editor’s Note: FFS - Investigate how a target should be configured for MQTT and WebSockets retargeting.

Note: A single oneM2M resource can have attributes that are mapped to more than one resource locally hosted by one or more Registree AEs.  Hence a Hosting CSE may need to issue multiple retargeted requests to Registree AEs for a single request that it receives to a targeted resource.

Note: If a Hosting CSE receives a request that accesses multiple attributes that are mapped to a single AE hosted resource, the Hosting CSE can issue a single retargeted request to the Registree AE which accesses the multiple attributes.

- The Hosting CSE maps the operation of the incoming oneM2M request primitive to a corresponding operation supported by the AE’s underlying protocol binding defined by the AE’s pointOfContact.

| oneM2M | CoAP / HTTP |
| --- | --- |
| Create | POST |
| Retrieve | GET |
| Update | PUT |
| Delete | DELETE |

Editor’s Note: FFS -  Investigate whether oneM2M should support retargeting CREATE and DELETE requests. E.g. retargeting a contentInstance CREATE may make sense.

Editor’s Note: FFS – Also consider HTTP PATCH

Editor’s Note: FFS - Investigate how mapping oneM2M operations to MQTT and WebSockets should be handled for retargeting.

- The Hosting CSE gets the supported serialization format from the corresponding <AE> contentSerialization attribute and uses it to determine which serialization format to use when retargeting the request to the Registree AE.

- If applicable, the Hosting CSE generates the content of the request (e.g. for a UPDATE/PUT request).  Based on the resourceMappingRules of each applicable attribute, the Hosting CSE constructs the content consisting of serialized attributes and values in the serialization format specified by the contentSerialization attribute of the corresponding <AE> resource. Below is a JSON example,

{

"doorLock": "OPEN"

}

Note: the Hosting CSE does not format the content as a oneM2M Primtive Content parameter (i.e. no "m2m:" or "pc:" root elements are included in the serialization).  Instead, the Hosting CSE formats the content to include only a simplified payload consisting of the applicable attributes as specified in the resourceMappingRules and their corresponding values. This reduces complexity and makes interaction with AEs lighter weight.

Step 006: The Hosting CSE sends the retargeted request(s) to the mapped AE resource attribute(s).

Editor’s Note: FFS - if any additional oneM2M request header or content parameters are needed in the request  (e.g. From parameter for security purposes)

Step 007: The Registree AE receives and processes the retargeted request.

Step 008: The Registree AE sends a response back to the Hosting CSE.

Step 009: The Hosting CSE receives the response from the Registree AE and checks the status. If the status indicates a success, the Hosting CSE extracts the content (if present/applicable) from the response payload, parses the attributes and values within the content and based on the resourceMappingRules configures the values of any attributes in the content of the response into the Hosting CSE's locally mapped attributes.  If the status indicates an error, the Hosting CSE includes this error in the oneM2M response primitive that it forms for the Originator AE/CSE.

Step 010: The Hosting CSE completes processing of the request by returning the oneM2M response primitive to the Originator AE/CSE.

Editor’s Note: FFS – add further details regarding error conditions such as cases when retargeting is enabled but retargeting privileges are not configured for an Orignator of request, or a AE is not reachable by the Hosting CSE to receive and process a retargeted request, or the AE returns error responses to the Hosting CSE for a retargeted request, etc.

#### 6.2.4.2 Solution 2: Access Control for Retargeting Requests to an AE

With the exception of the NOTIFY access control privilege, existing oneM2M access control privileges are only applicable to the authorization of entities performing operations on resources hosted by a CSE.  A NOTIFY privilege however is applicable to the authorization of entities issuing notification requests to AEs.  For example, this allows an AE to configure which entities are authorized to send it notification requests.

Clause 6.2.1 of TR-0053 introduces new functionality that enables a Hosting CSE to retarget CRUD requests that it receives to AEs to process.  With this new proposal, corresponding enhancements are also needed to the existing oneM2M access control policies to provide the capability to authorize the retargeting of CRUD requests to AEs.  For example, adding this capability enables an AE to configure access control policies with privileges that grant entities with the rights to have their requests retargeted to the AE for processing by a Hosting CSE.

To provide this capability, the accessControlOperations parameter of the privileges attribute of the <accessControlPolicy> resource is extended to support a new RETARGET operation, as shown in Table 6.2.4.2-1.

For example, an <accessControlPolicy> resource having privileges that include a RETARGET accessControlOperations can be created as a child of an <AE> resource.  When a Hosting CSE receives a request that requires re-targeting towards the AE, the Hosting CSE can first check the <accessControlPolicy> child resource of the corresponding <AE>.  If the <accessControlPolicy> resource specifies RETARGET privileges for the Originator of the received request, the Hosting CSE retargets the request to the AE. Otherwise, the Hosting CSE does not retarget the request and returns an error to the Originator indicating access was denied.

Table 6.2.4.2-1: accessControlOperations attribute with a new operation “RETARGET”

| Name | Description |
| --- | --- |
| RETRIEVE | Privilege to retrieve the content of an addressed resource |
| CREATE | Privilege to create a child resource |
| UPDATE | Privilege to update the content of an addressed resource |
| DELETE | Privilege to delete an addressed resource |
| DISCOVER | Privilege to discover the resource |
| NOTIFY | Privilege to receive a notification |
| RETARGET | Privilege to retarget a request |

Editor’s Note: FFS – Compare pros/cons of RETARGET ACP vs. adding configuration to mapping rules in resource instead.

#### 6.2.4.3 Solution 3: Using Subscription/Notification to Retarget RETRIEVE Requests

Figure 6.2.4.3-1 illustrates a retargeting procedure, where a Hosting CSE retargets a RETRIEVE request to a Registree AE using an enhanced subscription/notification mechanism. In this example, the Hosting CSE maintains a given resource, but its representation may be missing or not up-to-date all the time. The original representation of the given resource is kept locally at the Registree AE and it is always up-do-date. An Originator AE or CSE aims to retrieve the latest representation of the given resource.

Figure 6.2.4.3-1: Retargeting a RETRIEVE request to an AE via a Notification Message

Step 000: A Registree AE and an Originator AE first register with the Hosting CSE. The Originator AE then discovers the Registree AE.   The details of these procedures have been specified in oneM2M TS-0001.

Steps 001a-c: The Registree AE makes a subscription to RETRIEVE operations that target the subscribed-to-resource.  This subscription indicates to the Hosting CSE that the Registree AE supports the capability to receive a notification from the Hosting CSE that it has a received a RETRIEVE request for the subscribed-to-resource.  A Hosting CSE can conditionally send a notification to the Registree AE indicating it has received a RETRIEVE request to the subscribed-to-resource and requires an updated representation of the subscribed-to-resource to process the RETRIEVE request.  For example, the Originator of the RETRIEVE request has specified a Filter Criteria such as modifiedSince that specifies a time value that is more recent that the lastModifiedTime of the current resource representation.

Editor’s Note: FFS –  It might make sense to limit a subscription to RETRIEVE to a subset of oneM2M resource types (e.g. <flexContainer>, <mgmtObj>, <container>, <timeSeries>, etc. )

Editor's Note: FFS - Investigate whether there should be restrictions or privileges enforced by a Hosting CSE to limit which entity(s) are allowed to create subscription to RETRIEVE operations on a resource (e.g. creator or owner of a resource) and also whether only a single instance of a subscription to RETRIEVE should be allowed for a given subscribed-to-resource.

Step 002: An Originator AE issues a RETRIEVE request to a given resource that the Registree AE has performed Steps 001a-c on.   The Originator includes a modifiedSince Filter Criteria parameter in the request that specifies a timestamp value.

Step 003: The Hosting CSE receives the request and compares the timestamp value configured in the modifiedSince Filter Criteria parameter of the request against the lastModifiedTime attribute of the subscribed-to-resource and detects that the value of the lastModifiedTime attribute is older than the specified modifiedSince value.  Since there is a subscription to RETRIEVE operations, the Hosting CSE generates a notification message that contains an indication that a RETRIEVE Request was received for the subscribed-to-resource and an updated representation of the resource is requested from the AE.

Editor’s Note: FFS – Investigate an optimal way within the notification request to indicate to the AE that a RETRIEVE request was received by the Hosting CSE which requires an updated representation of the subscribed-to-process from the AE.

Step 004: The Hosting CSE sends the notification message to the subscriber of the subscribed-to-resource (i.e. Registree AE).

Step 005: The Registree AE receives and processes the notification of the RETRIEVE request from the Hosting CSE.

Step 006: The Registree AE sends a response to the Hosting CSE, which contains the an updated representation of the subscribed-to-resource being retrieved.

Editor’s Note: FFS – Investigate optimal method to structure/encode, within the notification response, an updated representation of the subscribed-to-resource.

Step 007: Upon receiving the response from the Registree AE, the Hosting CSE may store/refresh the representation of the subscribed-to-resource.

Step 008: The Hosting CSE processes the RETRIEVE request using the updated resource representation and returns RETRIEVE response in Step 006 to the Originator AE.

In order to support the procedure illustrated in Figure 6.2.4.3-1, a new type of notification event is defined for the oneM2M subscription eventNotificationCriteria.  The new event is triggered when an attempt to retrieve a subscribed-to resource (e.g. the latest <contentInstance> of a subscribed-to <container> resource) and the resource is either not present, its creationTime is older than the time specified within the createdAfter Filter Criteria, or its lastModifiedTime is older than the time specified within the modifiedSince Filter Criteria of the retrieve request.  This is shown in Table 6.2.4.3-1 below.

Table 6.2.4.3-1:  New Types of Notification Events

| Condition Tag | Multiplicity | Description |
| --- | --- | --- |
| notificationEventType | 0..8 | Type H: Generate an event when a retrieve request is attempted to retrieve a subscribed-to-resource and its creationTime is older than the time specified within the createdAfter Filter Criteria, or its lastModifiedTime is older than the time specified within the modifiedSince Filter Criteria of the retrieve request. |

Editor’s Note: FFS – Whether this new proposed Type H notificationEventType should be harmonized with Type E.

In addition, an enhancement to the format of a notification request can be defined to indicate to a subscriber that a RETRIEVE request targeting the subscribed-to-resource has been received and an updated representation of the subscribed-to-resource is needed from the subscriber for the subscription Hosting CSE to process the request.  Li

Likewise, an enhancement to the format of a notification response can be defined to allow a subscriber to return an updated representation of the subscribed-to-resource to a subscription Hosting CSE such that it can use the representation to update the subscribed-to-resource and process the RETRIEVE request.

# Conclusions

Editor’s Note: This clause provides a summary of the conclusions drawn during the study

# Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A>:
Title of annex (style H9)

<Text>

<PAGE BREAK>

# History

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 2018-03-01 | Skeleton of the TR |
| V0.1.0 | 2018-05-02 | Update to include ARC-2018-0052R02-Limitations_of_oneM2M_Messages which was agreed at ARC34 |
| V0.2.0 | 2018-08-16 | Update to include ARC-2018-0111-Limitations_of_oneM2M_Requests_Targeted_Towards_AEs which was agreed at ARC36 |
| V0.3.0 | 2019-06-05 | Incorporated Agreed CRs: SDS-2019-0068R02-TR-0053_Retargeting_via_resourceMappingRules SDS-2019-0069R02-TR-0053_ACP_for_AE_Retargeting SDS-2019-0070R02-TR-0053_SubNotif_Enhancement_for_Retargeting_Retrieve_Requests |