| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0067-V-0.1.0 |
| Document Name: | Study on Management Object migration to SDT |
| Date: | 2020-02-04 |
| Abstract: | The document is a study of how SDT <flexContainer> type resources could replace <mgmtObj> resources in the future. Depending on the TR outcomes, it will be decided whether interworking with SDT model could be added in oneM2M specifications and whether <mgmtObj> should be removed or not in the future releases. |
| 'Template Version: January 2020 (do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this report.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2020, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	1

1	Scope	1

2	References	1

2.1	Normative references	1

2.2	Informative references	1

3	Definition of terms, symbols and abbreviations	1

3.1	Terms	1

3.2	Symbols	1

3.3	Abbreviations	1

4	Conventions	1

5	Introduction	1

5.1	Analysis Background	1

5.2	DM Architecture	1

5.3	Conclusion and Proposal	1

Annex A : Proposal for update of TS-0001	1

----------------------- Start of change 1 -------------------------------------------	1

9.6.1.1	Resource Type Summary	1

----------------------- End of change 1 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

9.6.18	Resource Type node	1

----------------------- End of change 2 -------------------------------------------	1

----------------------- Start of change 3 -------------------------------------------	1

10.2.8	Device management	1

10.2.8.1	Introduction	1

----------------------- End of change 3 -------------------------------------------	1

----------------------- Start of change 4 -------------------------------------------	1

10.2.8.22	Device management using [flexNode] and <flexContainer> resources	1

----------------------- End of change 4 -------------------------------------------	1

----------------------- Start of change 5 -------------------------------------------	1

D.12	Resource cmdhPolicy	1

----------------------- End of change  -------------------------------------------	1

Annex B : Proposal for update of TS-0003	1

Annex C : Proposal for update of TS-0004	1

----------------------- Start of change 1 -------------------------------------------	1

7.3.4	Management common operations	1

7.3.4.1	Identify the managed entity and the technology specific protocol	1

----------------------- End of change 1 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

7.4.18	Resource Type <node>	1

7.4.18.1	Introduction	1

----------------------- End of change 2 -------------------------------------------	1

Annex D : Proposal for update of TS-0005	1

----------------------- Start of change 1 -------------------------------------------	1

5.3	Mapping of <mgmtObj> resources	1

5.3.0	Introduction	1

----------------------- End of change 1 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

5.7	Mapping of <flexContainer> resources	1

5.7.0	Introduction	1

----------------------- End of change 2 -------------------------------------------	1

----------------------- Start of change 3 -------------------------------------------	1

6.2.2	Object identifier	1

6.2.3	Object Instance Identifier	1

6.3	Mapping of <mgmtObj> resources	1

6.2.0	Introduction	1

----------------------- End of change 3 -------------------------------------------	1

----------------------- Start of change 4 -------------------------------------------	1

6.8	Mapping of <flexContainer> resources	1

6.2.0	Introduction	1

6.9	Generic Guidelines for Mapping LwM2M Objects to oneM2M <flexContainer> Resources	1

----------------------- End of change 4 -------------------------------------------	1

Annex E : Proposal for update of TS-0006	1

----------------------- Start of change 1 -------------------------------------------	1

7	Mapping of <mgmtObj> resources	1

7.0	Introduction	1

----------------------- End of change 1 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

8	Mapping of procedures for management	1

8.0	Introduction	1

8.1	Resource Type primitive mappings	1

8.1.0	Introduction	1

----------------------- End of change 2 -------------------------------------------	1

----------------------- Start of change 3 -------------------------------------------	1

11	Mapping of <flexContainer> resources	1

11.0	Introduction	1

11.1	General mapping assumptions	1

11.1.0	Introduction	1

11.1.1	Mapping of Device Identifiers	1

11.1.2	Mapping of Embedded Devices	1

11.2	Resource [dmDeviceInfo]	1

----------------------- End of change 3 -------------------------------------------	1

Annex F : Proposal for update of TS-0014	1

----------------------- Start of change 1 -------------------------------------------	1

1	Scope	1

----------------------- End of change 1 -------------------------------------------	1

Annex G : Proposal for update of TS-0022	1

----------------------- Start of change 1 -------------------------------------------	1

6.1	Introduction	1

----------------------- End of change 1 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

6.2.1	Introduction	1

6.2.2	Information elements required for M2M Service Layer operation	1

6.2.2.1	Introduction	1

----------------------- End of change 2 -------------------------------------------	1

----------------------- Start of change 3 -------------------------------------------	1

7	Resource type and data format definitions	1

----------------------- End of change 3 -------------------------------------------	1

----------------------- Start of change 4 -------------------------------------------	1

7.4	<flexContainer> Resource type specializations	1

7.4.1	Introduction	1

7.4.2	Resource [dmRegistration]	1

----------------------- End of change 4 -------------------------------------------	1

Annex H : Proposal for update of TS-0023	1

----------------------- Start of change 1 -------------------------------------------	1

5.8.2 flexNode	1

----------------------- End of change 1 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

6.2.2 Resource mapping for Device model	1

----------------------- End of change 2 -------------------------------------------	1

----------------------- Start of change 2 -------------------------------------------	1

6.2.5 Resource mapping for Property	1

----------------------- End of change 3 -------------------------------------------	1

----------------------- Start of change 4 -------------------------------------------	1

A.1	Example for Device model ‘deviceAirConditioner'	1

----------------------- End of change 4 -------------------------------------------	1

----------------------- Start of change 5 -------------------------------------------	1

A.2	Example of ModuleClass 'binarySwitch'	1

----------------------- End of change 5 -------------------------------------------	1

History	1

# 1	Scope

The present document is the Technical Report which study the completion of SDT (Smart Device Template) using <flexContainer> resource specializations and the possible migration of the existing device management mdel using Management Object (<mgmtObj>).

This document is initiated in the context of the Management Object Migration [MOM] Work Item (WI-0099) having the following objectives (extract) :

In Release 4, SDT (Smart Device Template) has been extended to include device management functions in addition to the existing services. This justified extension creates a new way to perfom device management compared to the existing Device Management (DMG) Common Service Function (CSF) model using <mgmtObj>.

In order not to live with 2 solutions for the same purpose, the work item WI-0099 proposes to work on a transition phase allowing implementation using DMG <mgmtObj> to migrate towards SDT model.

The scope of the Work Item is to study the Device Management model based on Management Object (<mgmtObj>) model migration towards SDT model.

This action plan includes:

- Provide a temporary TR with a mapping between <mgmtObj> and the SDT flexContainer;

- List in this TR all the specifications and sections that will have to be updated when <mgmtObj> will be replaced by SDT;

- List in this TR the issues to be resolved by removing the <mgmtObj> after migration and the proposed solutions;

- Depending on the TR outcomes, decide whether <mgmtObj> should be removed or not in OneM2M Release 4.

New tasks may be added during the process if necessary.

This TR intends to provide the study as part of the action plan above.

# 2	References

## 2.1	Normative references

[1]	oneM2M TS-0023: SDT based Information Model & Mapping for Vertical Industries

[2]	oneM2M TS-0001: Functional Architecture

[3]	oneM2M TS-0004: Service Layer Core Protocol Specification

[4]	oneM2M TS-0005: Management Enablement (OMA)

[5]	oneM2M TS-0006: Management Enablement (BBF)

[6]	oneM2M TS-00014: LWM2M Interworking

[7]	oneM2M TS-00022: Field Device Configuration

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

BBF	BroadBand Forum

DM	Device Management

IPE	Interworking Proxy application Entity

LWM2M	LightWeight M2M

MO	Management Object

OMA	Open Mobile Alliance

SDT	Smart Device Template

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Introduction

In the Release 4 of the TS-0023 [1] specification (SDT-based Information Model and Mapping for Vertical Industries) was introduced a new approach for Device Management, based on Smart Device Template ModuleClasses, a concept that is mapped in oneM2M as <flexContainer> resource specializations.

The benefits of this approach are:

- Unified Device and Service Management of all nodes, including ADN or NoDN devices: it is possible to use the same type of resources for handling both the functional behaviour of devices and their remote management in the usual DM meaning (reboot, firmware update, configuration, log, etc.) Before this, the DM aspect was performed through <mgmtObj> resources, and the functional aspect through <container> or <flexContainer> resources.

- Enhanced expressivity. The SDT design allows a powerful information model to describe devices, with concepts such as Devices,  SubDevices, ModuleClasses, Actions, DataPoints and Properties. The <mgmtObj> has a poorer semantics.

- Standardized, flexible, extensible and incremental Data Model. Adding new Devices, ModuleClasses or DataPoints is an easy process, compared with <mgmtObj> resources.

- Standardized ontological model: the SDT Information Model is aligned with the oneM2M Base Ontology (see clause 8 in TS-0023 [1]).

- Ease of use. For oneM2M application developers, handling <flexContainers> that map ModuleClasses or Actions is quite simple and natural. For example, triggering a [reboot] SDT Action is very similar to calling a method, with a rebootType parameter, and more satisfying / less ambiguous than the [reboot] <mgmtObj> with its 2 writable attributes reboot and factoryReset.

This TR proposes to extend this approach to all <mgmtObj> resources.

## 5.1	Analysis Background

The table below lists specializations of <mgmtObj> resources that are currently specified in oneM2M Release 4 (referenced in TS-0001 [2] clause 9.6.1.2.1).

- The first group handles CMDH policy management (Communication Management and Delivery Handling), specified in TS-0001 [2] Annex D.12. As the Work Item WI-0096 is currently working on an evolution of CMDH based on flexContainers, these resources will not be considered in the present document.

- The second group handles Device Configuration in the Field Domain, defined in TS-0003 Security Solutions and specified in TS-0022 Field Device Configuration.

- The third group handles Device Management in the “classical” meaning, i.e. the remote management of devices (reboot, firmware/software management, configuration, logging, etc.) These <mgmtObj> already have been mapped to SDT <flexContainer> specializations in TS-0023 [1].

| Resource specialization | Short Description | Comment |
| --- | --- | --- |
| CMDH CSF (TS-0001 6.2.2 and Annex D.12) |  |  |
| activeCmdhPolicy | Provides a link to the currently active set of CMDH policies |  |
| cmdhBuffer | Defines CMDH buffer usage limits |  |
| cmdhDefaults | Defines CMDH default values |  |
| cmdhEcDefParamValues | Represent a specific set of default values for the CMDH related parameters |  |
| cmdhDefEcValue | Defines a value for the Event Category parameter of an incoming request when it is not defined |  |
| cmdhLimits | Defines limits for CMDH related parameter values |  |
| cmdhNetworkAccessRules | Defines rules for the usage of underlying networks |  |
| cmdhNwAccessRule | Defines a rule for the usage of underlying networks |  |
| cmdhPolicy | A set of rules defining which CMDH parameters will be used by default |  |
| Field Device Configuration (TS-0022) |  |  |
| registration | To convey the service layer configuration information |  |
| dataCollection | To convey the application configuration information |  |
| authenticationProfile | To convey the configuration information regarding establishing mutually-authenticated secure communications |  |
| myCertFileCred | To configure a certificate or certificate chain |  |
| trustAnchorCred | To identify a trust anchor certificate for validation of certificates |  |
| MAFClientRegCfg | To convey instructions regarding the MAF Client Registration procedure |  |
| MEFClientRegCfg | To convey instructions regarding the MEF Client Registration procedure |  |
| wifiClient | To set up configuration of WiFi connection on the client device. |  |
| DMG CSF (TS-0001 6.2.4 and Annex D.1 to D.11 & D.13) |  |  |
| areaNwkDeviceInfo | Provides information about the Node in the M2M Area Network | TS-0023 “5.8.11 dmAreaNwkDeviceInfo” |
| areaNwkInfo | Describes the list of Nodes attached behind the MN node and its physical or underlying relation among the nodes in the M2M Area Network | TS-0023 “5.8.10 dmAreaNwkInfo” |
| battery | Provides the power information of the node (e.g. remaining battery charge) | TS-0023 “5.3.1.10 battery”, referenced in TS-0023 “5.8.2 flexNode” |
| deviceCapability | Contains information about the capability supported by the Node | TS-0023 “5.8.12 dmDeviceCapability” |
| deviceInfo | Contains information about the identity, manufacturer and model number of the device | TS-0023 “5.8.4 dmDeviceInfo” |
| eventLog | Contains information about the log of events of the Node | TS-0023 “5.8.8 dmEventLog” |
| firmware | Provides information about the firmware of the Node (e.g. name, version) | TS-0023 “5.8.6 dmFirmware” |
| memory | Provides the memory (typically RAM) information of the node (e.g. the amount of total volatile memory) | TS-0023 “5.8.3 dmAgent” |
| reboot | Used to reboot or reset the Node | TS-0023 “5.8.3 dmAgent” |
| software | Provides information about the software of the Node | TS-0023 “5.8.7 dmSoftware” and TS-0023 “5.8.9 dmPackage” |
| storage | To manage available storage memory on the device | TS-0023 “5.8.13 dmStorage” |

## 5.2	DM Architecture

Currently, the DM Architecture is composed of two parts:

- The <node> and <mgmtObj> model defined in TS-0001 [2] clause 9.6.18.

- The [flexNode] and [DM Module classes] model defined in TS-0023 [1] clause 5.8.

These models coexisted, i.e. an IPE could create a device representation in SDT with a [flexNode] and/or a <node>. This choice was made in order not to modify the <node> resource, but it adds a lot of complexity.

We propose here to move the [flexNode] as child of the <node>, which supposes letting the <node> resource capable of having <flexContainer> children.

Figure 5.2-1: New DM architecture proposal

The resources in blue in the above figure are not supposed to coexist: either the <node> has <mgmtObj> children such as the [deviceInfo] specialization, or it has a [flexNode] child which is the root of DM <flexContainer> specializations such as [dmDeviceInfo].

This architecture presents three main benefits:

- It forbids having <mgmtObj> resources and their equivalent <flexContainers> at the same level under the <node> resource, which would bring some confusion.

- It maintains most of the current TS-0023 [1] DM architecture (the emplacing of the [flexNode] was not forced), just removing the notion of nodeFlexLink that linked an IPE to the associated [flexNode].

- It maintains the current <node> / <mgmtObj> DM architecture, allowing a smoother transition from <mgmtObj>-based to <flexContainer>-based implementations.

## 5.3	Conclusion and Proposal

In the rest of this TR will be presented an outline of the modifications that should be made to each oneM2M Technical Specification (TS) document that handles <mgmtObj> resources in order to take into account this proposed approach for Device Management.

Each Annex will be dedicated to an existing TS that deals with <mgmtObj> resources, and will propose a skeleton of the changes that could be made for using <flexContainers> instead.

# Annex A : Proposal for update of TS-0001

In this Annex, are presented the proposed changes to the TS-0001 [2] specification for flexContainer introduction for device management operations.

### ----------------------- Start of change 1 -------------------------------------------

#### 9.6.1.1	Resource Type Summary

TBD: Add <flexContainer> in <node>’s Child Resource Types and <node> in <flexContainer>’s Parent Resource Types.

### ----------------------- End of change 1 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

### 9.6.18	Resource Type node

The <node> resource represents specific information that provides properties of an M2M Node that can be utilized by other oneM2M operations. It contains resources that represent the Node's context information (e.g. memory and battery), network topology, device information, device capability etc. These resources can be mapped in two distinct models:

- Either the <node> resource has specialization of the <mgmtObj> as its child resources. The specialized <mgmtObj> resources are used to perform management of the Node.

- Or the <node> resource has the [flexNode] specialization of a <flexContainer> as child resource, and this [flexNode] has specialization of the <flexContainer> as its child resources. These <flexContainer> resources are used to perform management of the Node.

This node specific information stored in these resources, <mgmtObj> specializations such as [deviceInfo] and [firmware] or <flexContainer> specializations such as [dmDeviceInfo] or [dmFirmware],  can be obtained either by the existing device management technologies (OMA DM [i.3], BBF TR-069 [i.2]) or any other way (e.g. JNI [i.18]).

For the case when the <node> resource belongs to an ADN, please see figure 9.6.18-1 in conjunction with the description of nodeLink attribute in the <AE> resource (clause 9.6.5).

For the case when the <node> resource belongs to an NoDN and the applications that correspond to interworked devices are represented by <flexContainer>s please see figure 9.6.18-2.

Figure 9.6.18-1: Relationship between IN/MN and ADN

Figure 9.6.18-2: Relationship between IPE, interworked Services and NoDN

The <node> resource shall contain the child resources specified in table 9.6.18-1.

Table 9.6.18-1: Child resources of <node> resource

| Child Resources of <node> | Child Resource Type | Multiplicity | Description | <nodeAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <flexContainer> as defined in the specialization [flexNode] | 0..1 | This resource provides the root for SDT-based <flexContainers> that correspond to Device Management related ModuleClasses (see clause TS-0023 5.8). See Note. | <flexContainerAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [memory] | 0..1 | This resource provides the memory (typically RAM) information of the node. (E.g. the amount of total volatile memory), See clause D.4. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [battery] | 0..n | The resource provides the power information of the node. (E.g. remaining battery charge). See clause D.7. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [areaNwkInfo] | 0..n | This resource describes the list of Nodes attached behind the MN/ASN node and its physical or underlying relation among the nodes in the M2M Area Network. This attribute is defined in case the Node is MN/ASN. See clause D.5. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [areaNwkDeviceInfo] | 0..n | This resource describes the information about the Node in the M2M Area Network. See clause D.6. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [firmware] | 0..n | This resource describes the information about the firmware of the Node include name, version etc. See clause D.2. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [software] | 0..n | This resource describes the information about the software of the Node. See clause D.3. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [deviceInfo] | 0..n | The resource contains information about the identity, manufacturer and model number of the device. See clause D.8. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [deviceCapability] | 0..n | The resource contains information about the capability supported by the Node. See clause D.9. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [reboot] | 0..1 | The resource is the place to reboot or reset the Node. See clause D.10. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [eventLog] | 0..1 | The resource contains the information about the log of events of the Node. See clause D.11. | <mgmtObjAnnc> |
| [variable] | <mgmtObj> as defined in the specialization [cmdhPolicy] | 0..n | The resource(s) contain(s) information about CMDH policies that are applicable to the CMDH processing on the CSE hosted on the node represented by this <node> resource and identified by the hostedCSELink attribute of this <node> resource. See clause D.12. | NA |
| [variable] | <mgmtObj> as defined in the specialization [activeCmdhPolicy] | 0..1 | This resource defines which of the present [cmdhPolicy] resource(s) shall be active for the CMDH processing on the CSE hosted on the node represented by this <node> resource and identified by the hostedCSELink attribute of this <node> resource. See clause D.12. | NA |
| [variable] | <subscription> | 0..n | See clause 9.6.8. | <subscription> |
| [variable] | <schedule> | 0..n | See clause 9.6.9. | <scheduleAnnc> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.61 | None |
| NOTE: if the [flexNode] child is present, then all <mgmtObj> children from [memory] to [eventLog] cannot be present. |  |  |  |  |

The <node> resource shall contain the attributes specified in table 9.6.18-2.

Table 9.6.18-2: Attributes of <node> resource

| Attributes of <node> | Multiplicity | RW/ RO/ WO | Description | <nodeAnnc> attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| owner | 0..1 | RW | See clause 9.6.1.3 | NA |
| nodeID | 1 | RW | The M2M-Node-ID of the node which is represented by this <node> resource. | MA |
| nodeType | 0..1 | RW | Indicates the type of node. It shall have one of the following values: - IN - MN - ASN - ADN - NoDN | OA |
| hostedCSELink | 0..1 | RW | This attribute allows to find the <CSEBase> or <remoteCSE> resource representing the CSE that is residing on the node that is represented by this <node> resource. The attribute contains the resource ID of a resource where all of the following applies: The resource is a <CSEBase> resource or a <remoteCSE> resource. The resource represents the CSE which resides on the specific node that is represented by the current <node> resource. In case the node that is represented by this <node> resource does not contain a CSE, this attribute shall not be present. | OA |
| hostedAELinks | 0..1(L) | RW | This attribute allows to find the AEs hosted by the node that is represented by this <node> resource. The attribute shall contain a list of resource identifiers of <AE> resources representing the ADN-AEs residing on the node that is represented by the current <node> resource. In case the node that is represented by this <node> resource does not contain an AE, this attribute shall not be present. | OA |
| hostedServiceLinks | 0..1(L) | RW | This attribute allows to find <flexContainer> resources that have been created by an IPE to represent services hosted on a NoDN, the NoDN being represented by this <node> resource. If the NoDN hosts a set of services represented by <flexContainer>s, then the attribute shall contain the list of resource identifiers of these <flexContainer> resources. In case the node that is represented by this <node> resource does not contain an service that is represented by a <flexContainer>, this attribute shall not be present. | OA |
| mgmtClientAddress | 0..1 | RW | Represents the physical address of management client of the node which is represented by this <node> resource. This attribute is absent if management server is able to acquire the physical address of the management client. | OA |
| roamingStatus | 0..1 | RO | Indicates if the M2M Node is currently roaming from the perspective of the underlying network. The allowed values are “Yes” or “No”. | OA |
| networkID | 0..1 | RO | Configured with the identity of the underlying network which the M2M Node is currently attached to. | OA |

### ----------------------- End of change 2 -------------------------------------------

### ----------------------- Start of change 3 -------------------------------------------

### 10.2.8	Device management

#### 10.2.8.1	Introduction

This clause describes the procedures for managing device capabilities on MNs (e.g. M2M Gateways), ASNs and ADNs (e.g. M2M Devices), as well as devices that reside within an M2M Area Network.

Resources maintaining information and relationships that are specific to Device Management are termed Device Management Resources. This clause details the creation, retrieval, update and deletion of the information associated with the following Device Management Resources: <node> (clauses 10.2.8.2 to 10.2.8.6), <mgmtObj> (clauses 10.2.8.7 to 10.2.8.12), <mgmtCmd> and its child resource <execInstance> (clauses 10.2.8.13 to 10.2.8.21), [flexNode] and its child <flexContainer> child resources (clauses 10.2.8.22 to xxx).

These operations are used in both Device Management options available in oneM2M: one utilizing existing technology protocols (e.g. BBF TR-069 [i.2], OMA-DM [i.3], and LWM2M [i.4]) and another utilizing the native oneM2M protocols. Clause 6.2.4 details the Device Management (DMG) CSF supporting this functionality.

### ----------------------- End of change 3 -------------------------------------------

### ----------------------- Start of change 4 -------------------------------------------

### 10.2.8.22	Device management using [flexNode] and <flexContainer> resources

This clause describes procedures for managing device capabilities on devices that reside within an M2M Area Network, using <flexContainer> specializations. This is an alternative to the approach based on <node>, <mgmtObj> and <mgmtCmd> resources specified in clause 10.2.8.

### TBC…

### ----------------------- End of change 4 -------------------------------------------

### ----------------------- Start of change 5 -------------------------------------------

## D.12	Resource cmdhPolicy

TBD: Write DM <flexContainers> that correspond to the 8 [*cmdh*] <mgmtObj>.

Here or in TS-0023 5.8?

### ----------------------- End of change  -------------------------------------------

# Annex B : Proposal for update of TS-0003

In this Annex, are presented the proposed changes to the TS-0003 specification for flexContainer introduction for device management operations.

TBD: definition of <mgmtObj> to be replicated/replaced by DM <flexContainers>

# Annex C : Proposal for update of TS-0004

In this Annex, are presented the proposed changes to the TS-0004 [3] specification for flexContainer introduction for device management operations.

Need abstract type for [dm*] flexContainers, such as 6.3.5.34	m2m:mgmtResource in TS-0004?

### ----------------------- Start of change 1 -------------------------------------------

### 7.3.4	Management common operations

#### 7.3.4.1	Identify the managed entity and the technology specific protocol

Where a managed entity is being addressed via a <mgmtObj> or <flexContainer> resource, the Hosting CSE shall identify the managed entity via the <node> resource that is the parent resource of the <mgmtObj> or <flexContainer> resource. In case of a <mgmtCmd> resource the entity to be managed is indicated by its execTarget attribute. This addresses either a <node> resource or a group of resources of type <node>. Hence, in all cases the managed entity is ultimately identified through a <node> resource, from which the identifier of the device can be retrieved.

TBC…

### ----------------------- End of change 1 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

### 7.4.18	Resource Type <node>

#### 7.4.18.1	Introduction

The <node> resource represents specific information that provides properties of an oneM2M Node that can be utilized by other oneM2M operations. The <node> resource has either a [flexNode] <flexContainer> specialization or <mgmtObj> as its child resources.

Table 7.4.18.1-1: Data type definition of <node> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| node | CDT-node-v4_2_0.xsd |  |

Table 7.4.18.1-2: Universal/Common Attributes of <node> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.18.1-3: Resource Specific Attributes of <node> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| nodeID | M | O | m2m:nodeID |  |
| hostedCSELink | O | O | m2m:ID |  |
| hostedAELinks | O | O | m2m:listOfM2MID |  |
| hostedServiceLinks | O | O | m2m:listOfM2MID |  |
| mgmtClientAddress | O | O | xs:string |  |
| roamingStatus | NP | NP | xs:boolean | No default. True means that the Node is currently roaming. When this attribute is not present, it indicates that no information is available. |
| networkID | NP | NP | xs:string | No default. When this attribute is not present, it indicates that no information is available. |
| nodeType | O | O | m2m:nodeType | Default is UNSPECIFIED. |

Table 7.4.18.1-4: Child resources of <node> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <mgmtObj> | [variable] | 0..n | Clause Erreur ! Source du renvoi introuvable., and Erreur ! Source du renvoi introuvable. |
| <flexContainer> | [flexNode] | 0..1 |  |
| <subscription> | [variable] | 0..n | Clause Erreur ! Source du renvoi introuvable. |
| <semanticDescriptor> | [variable] | 0..n | Clause Erreur ! Source du renvoi introuvable. |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <schedule> | [variable] | 0..n | Clause 7.4.9 |

### ----------------------- End of change 2 -------------------------------------------

TBC…

# Annex D : Proposal for update of TS-0005

In this Annex, are presented the proposed changes to the TS-0005 [4] specification for flexContainer introduction for device management operations.

TBD: introduce the alternative approach <flexContainer> vs <mgmtObj>.

### ----------------------- Start of change 1 -------------------------------------------

## 5.3	Mapping of <mgmtObj> resources

### 5.3.0	Introduction

This clause describes how to map <mgmtObj> resources specified in annex D of [Erreur ! Source du renvoi introuvable.] to the relevant management objects as defined by OMA DM ([Erreur ! Source du renvoi introuvable.] and [Erreur ! Source du renvoi introuvable.]). Since OMA DM 1.3 and OMA DM 2.0 use the same management objects except standard management objects, the resource mappings can be considered regardless of the specific version of the OMA DM Protocol.

For mapping Device Management <flexContainer> specializations specified in TS-0023 clause 5.8, see clause 5.7.

### ----------------------- End of change 1 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

## 5.7	Mapping of <flexContainer> resources

### 5.7.0	Introduction

This clause describes how to map <flexContainer> resources specified in clause 5.8 of TS-0023 to the relevant management objects as defined by OMA DM ([Erreur ! Source du renvoi introuvable.] and [Erreur ! Source du renvoi introuvable.]). Since OMA DM 1.3 and OMA DM 2.0 use the same management objects except standard management objects, the resource mappings can be considered regardless of the specific version of the OMA DM Protocol.

TBC

### ----------------------- End of change 2 -------------------------------------------

### ----------------------- Start of change 3 -------------------------------------------

### 6.2.2	Object identifier

In OMA LwM2M [Erreur ! Source du renvoi introuvable.], each object is characterized by a unique identifier represented by an integer. This identifier is provided by OMNA (OMA Naming Authority) and is registered as a unique URN:

urn:oma:lwm2m:{oma,ext,x}:objectID[ :{version}] (e.g. the LwM2M 1.0  Device Object (ObjectID:3) is registered as urn:oma:lwm2m:oma:3).

The context of a given oneM2M <mgmtObj> Resource is represented by the objectIDs attribute which can contain several references to OMA LwM2M [Erreur ! Source du renvoi introuvable.] Object identifiers expressed as OMNA registered URN.

TBD for <flexContainer> resources. Need objectIDs custom attribute?

### 6.2.3	Object Instance Identifier

OMA LwM2M [Erreur ! Source du renvoi introuvable.] permits objects to have multiple object instances where each object instance is contained in the objectPath attribute of the <mgmtObj> Resource within the context of the Resource's objectIDs as described in previous clause.

The objectPath attribute in <mgmtObj> Resource contains one (or several) element(s) representing the local path(s) where the Object Instance(s) are located.

TBD for <flexContainer> resources. Need objectPath custom attribute?

## 6.3	Mapping of <mgmtObj> resources

### 6.2.0	Introduction

This clause describes how to map the <mgmtObj> Resources specified in the annex D of [Erreur ! Source du renvoi introuvable.] to the relevant Objects specified in OMA LwM2M [Erreur ! Source du renvoi introuvable.].

For mapping Device Management <flexContainer> specializations specified in TS-0023 clause 5.8, see clause 6.8.

### ----------------------- End of change 3 -------------------------------------------

### ----------------------- Start of change 4 -------------------------------------------

## 6.8	Mapping of <flexContainer> resources

### 6.2.0	Introduction

This clause describes how to map <flexContainer> resources specified in clause 5.8 of TS-0023 to the relevant Objects specified in OMA LwM2M [Erreur ! Source du renvoi introuvable.].

TBC…

## 6.9	Generic Guidelines for Mapping LwM2M Objects to oneM2M <flexContainer> Resources

Since there are several LwM2M defined objects and vendor defined LwM2M Objects that currently do not have a corresponding <flexContainer> specialization defined in this document, the following guidelines have been defined.  These guidelines provide interoperability and extensibility between the two standards and ensure seamless interworking of LwM2M clients and servers with oneM2M CSEs and AEs.

Below are the guidelines for generically mapping LwM2M Objects to oneM2M <flexContainer> resources.

TBC…

### ----------------------- End of change 4 -------------------------------------------

# Annex E : Proposal for update of TS-0006

In this Annex, are presented the proposed changes to the TS-0006 [5] specification for flexContainer introduction for device management operations.

TBD: introduce the alternative approach <flexContainer> vs <mgmtObj>.

### ----------------------- Start of change 1 -------------------------------------------

# 7	Mapping of <mgmtObj> resources

## 7.0	Introduction

This clause contains all information on how to map <mgmtObj> management resources from TS-0004 [Erreur ! Source du renvoi introuvable.] to managed objects and parameters as defined in the TR-181 [Erreur ! Source du renvoi introuvable.] data model or the Remote Procedure Calls (RPCs) in TR-069 [Erreur ! Source du renvoi introuvable.].

For mapping Device Management <flexContainer> specializations specified in TS-0023 clause 5.8, see clause 11.

### ----------------------- End of change 1 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

# 8	Mapping of procedures for management

## 8.0	Introduction

This clause contains all information on how to map management resource primitives from TS-0004 [Erreur ! Source du renvoi introuvable.] to the Remote Procedure Calls (RPCs) in TR-069 [Erreur ! Source du renvoi introuvable.].

## 8.1	Resource Type primitive mappings

### 8.1.0	Introduction

This clause contains all information on how to map Resource Type <mgmtObj> primitives from TS-0004 [Erreur ! Source du renvoi introuvable.], or Device Management <flexContainer> from TS-0023, to the Remote Procedure Calls (RPCs) in TR-069 [Erreur ! Source du renvoi introuvable.].

### ----------------------- End of change 2 -------------------------------------------

### ----------------------- Start of change 3 -------------------------------------------

# 11	Mapping of <flexContainer> resources

## 11.0	Introduction

This clause contains all information on how to map Device Management <flexContainer> resources from TS-0023 clause 5.8 to managed objects and parameters as defined in the TR-181 [Erreur ! Source du renvoi introuvable.] data model or the Remote Procedure Calls (RPCs) in TR-069 [Erreur ! Source du renvoi introuvable.].

## 11.1	General mapping assumptions

### 11.1.0	Introduction

TR-069 [Erreur ! Source du renvoi introuvable.] specifies a protocol for communication between a CPE (Customer Premises Equipment) and an ACS (Auto-Configuration Server). Any TR-069 enabled device has to follow the data model as described in the TR-106 [Erreur ! Source du renvoi introuvable.] and TR-181 [Erreur ! Source du renvoi introuvable.] as well as RPCs described in TR-069 [Erreur ! Source du renvoi introuvable.].

As TR-181 [Erreur ! Source du renvoi introuvable.] is the model that the Resources are mapped, all Resources shall have the objects of the TR-181 [Erreur ! Source du renvoi introuvable.] namespace (e.g. "urn:broadband-forum-org:tr-181-2-7-0").

### 11.1.1	Mapping of Device Identifiers

The Device identifiers for CPEs are mapped to the Resource Types [deviceInfo].

For CPE and Virtual Devices map their Device Identifiers (OUI-[PC-]SN) to the manufacturer, deviceType and deviceLabel attributes of the Resource [dmDeviceInfo].

For Embedded Devices, the ControllerID and ProxiedDeviceID parameters of the Device.ManagementServer.EmbeddedDevice.{i} object instance are mapped to the deviceLabel attribute of the Resource [dmDeviceInfo] as a comma separated list: "Device.ManagementServer.EmbeddedDevice.{i}.ControllerID, Device.ManagementServer.EmbeddedDevice.{i}.ProxiedDeviceID".

### 11.1.2	Mapping of Embedded Devices

The TR-181 [Erreur ! Source du renvoi introuvable.] specification does not provide a mechanism where Embedded Devices provide information related to the Device.DeviceInfo objects and sub-objects. Instead the TR-181 [Erreur ! Source du renvoi introuvable.] provides this information in a manner that is reliant on the Embedded Device's underlying technology (e.g. ZigBee®, UpnP).

## 11.2	Resource [dmDeviceInfo]

TBC

### ----------------------- End of change 3 -------------------------------------------

# Annex F : Proposal for update of TS-0014

In this Annex, are presented the proposed changes to the TS-0014 [6] specification for flexContainer introduction for device management operations.

Extend “type 3” interworking type, so that it is possible to use <flexContainer> the samew ay as <mgmtObj>.

### ----------------------- Start of change 1 -------------------------------------------

# 1	Scope

The present document specifies the interworking capabilities of the M2M Service Layer between ASN/IN/MN CSEs and LWM2M Endpoints using the architecture identified in Annex F of oneM2M TS-0001 [2] for the following interworking scenarios:

Interworking for transparent transport of encoded LWM2M Objects and commands in Content Sharing Resources between LWM2M Endpoints and M2M Applications.

Interworking with full mapping of LWM2M Objects in LWM2M Endpoints to semantically enabled Content Sharing Resources that are utilized by M2M Applications.

Interworking with one-to-one mapping of LWM2M Objects with oneM2M <mgmtObj> or <flexContainer> resources.

NOTE:	The present document limits Content Sharing Resources to <container> and <contentInstance> resources.

### ----------------------- End of change 1 -------------------------------------------

TBC…

# Annex G : Proposal for update of TS-0022

In this Annex, are presented the proposed changes to the TS-0022 [7] specification for flexContainer introduction for device management operations.

TBD: Write DM <flexContainers> that correspond to the 9 <mgmtObj>.

Or should we directly replace <mgmtObj> by <flexContainer>???

### ----------------------- Start of change 1 -------------------------------------------

## 6.1	Introduction

The information needed by the remote AE or CSE in the field domain to establish M2M Service Layer operation uses the architectural aspects of oneM2M TS-0001 [Erreur ! Source du renvoi introuvable.] in order to convey the information elements to the ASN/MN or ADN nodes that host the AE or CSE prior to or during M2M Service Layer operation and to the AE or CSE during M2M Service Layer operation.

Figure 6.1-1: Architectural Aspects for Configuration of ASN/MN and ADN Nodes

Figure 6.1-1 depicts three (3) methods, in which ADN or ASN/MN nodes are configured using the following:

Device Management technologies using the mc reference point defined in clause 6 of oneM2M TS-0001 [Erreur ! Source du renvoi introuvable.]. Using this method, the information that is used to configure the ASN/MN or ADN is described as <mgmtObj> or <flexContainer> resource types that are hosted in the IN-CSE.

oneM2M Mcc and Mca reference point when M2M Service Layer operation has been established to the AE or CSE. Establishment of the M2M Service Layer operation includes actions such as setting up security associations and registration of the M2M entities as per oneM2M TS-0003 [Erreur ! Source du renvoi introuvable.] and oneM2M TS-0001 [Erreur ! Source du renvoi introuvable.].

oneM2M IPE technology where the IPE interworks the information exchange between the ADN and ASN/MN and the IN-CSE. This type of IPE is called a Configuration IPE in order to depict the role and capabilities of the IPE related to the present document.

NOTE:	The reference point between the Configuration IPE and the ADN and ASN/MN is unspecified in the present document.

In addition, Figure 6.1-1 introduces an AE whose role is to configure the IN-CSE and nodes in the Field Domain with the information needed to establish M2M Service Layer operation. This type of AE is called a Configuration AE in order to depict the role and capabilities of the AE related to the present document.

The information that is used to configure the ASN/MN or ADN is described as <mgmtObj> or <flexContainer> resource types that are hosted in the IN-CSE.

### ----------------------- End of change 1 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

### 6.2.1	Introduction

The Configuration AE provisions the <mgmtObj> or <flexContainer> resource types in the IN-CSE and the IN-CSE then interacts with the DM Server, ADN or ASN/MN node or Configuration IPE in order to configure the AE or CSE on the nodes.

### 6.2.2	Information elements required for M2M Service Layer operation

#### 6.2.2.1	Introduction

The ASN/MN and ADN in the Field Domain should support the capability to be configured with the <mgmtObj> or <flexContainer> resource types defined in the present document prior to initial registration with a registrar CSE (enrolment phase). When the AE or CSE has established M2M Service Layer operation with a Registrar CSE (operational phase), the AE or CSE shall provide the capability to be configured with the <mgmtObj> or <flexContainer> resource types defined in the present document.

### ----------------------- End of change 2 -------------------------------------------

### ----------------------- Start of change 3 -------------------------------------------

# 7	Resource type and data format definitions

The management resources can be written in two distinct types: either as <mgmtObj> resource types under the <node> (see clause 7.1 for the definitions and clause 7.2 for the procedures), or as <flexContainer> under the [flexNode] child of the <node> (see clause 7.4 for the definitions and clause 7.5 for the procedures).

### ----------------------- End of change 3 -------------------------------------------

### ----------------------- Start of change 4 -------------------------------------------

## 7.4	<flexContainer> Resource type specializations

### 7.4.1	Introduction

The present clause specifies <flexContainer> resource specializations used to configure AEs or CSEs on ADN or ASN/MN nodes in the Field Domain in order to establish M2M Service Layer operation.

Table 7.4.1-1 shows a summary of <flexContainer> resource specializations defined in the present document.

Table 7.4.1-1: Summary of defined <mgmtObj> resources

| flexContainer | containerDefinition | Intended use | Note |
| --- | --- | --- | --- |
| dmRegistration | TBD | Service Layer Configuration information needed to register an AE or CSE with a Registrar CSE. | This is M2M Service Provider dependent. |
| dmDataCollection | TBD | Application Configuration information needed to establish collection of data within the AE and transmit the data to the Hosting CSE using <container> and <contentInstance> resource types. | This is M2M Application dependent. |
| dmAuthenticationProfile | TBD | Security information needed to establish mutually-authenticated secure communications |  |
| dmMyCertFileCred | TBD | Configuring a file containing a certificate and associated information |  |
| dmTrustAnchorCred | TBD | Identifies a trust anchor certificate and provides a URL from which the certificate can be retrieved. The trust anchor certificate can be used to validate a certificate which the Managed Entity uses to authenticate another entity. |  |
| dmMAFClientRegCfg | TBD | Instructions for performing the MAF Client Registration procedure with a MAF. Links to an Authentication Profile instance. |  |
| dmMEFClientRegCfg | TBD | Instructions for performing the MEF Client Registration procedure with a MEF. Links to an Authentication Profile instance. |  |

### 7.4.2	Resource [dmRegistration]

This specialization of <flexContainer> is used to convey the service layer configuration information needed to register an AE or CSE with a Registrar CSE.

TBC…

### ----------------------- End of change 4 -------------------------------------------

TBC…

- Annex H : Proposal for update of TS-0023

In this Annex, are presented the proposed changes to the TS-0023 [1] specification for flexContainer introduction for device management operations.

Note: the clause 5.8 of TS-0023 specifies the [flexNode] specialization and its DM-related child <flexContainer> resources.

Maybe it should be entirely moved to TS-0001?

### ----------------------- Start of change 1 -------------------------------------------

### 5.8.2 flexNode

This flexContainer specialization is the root for SDT-based Device Management modules.

The containerDefinition attribute of this specialization shall be “org.onem2m.management.device.flexNode”.

It is a <flexContainer> child of the <node> resource targeted by the nodeLink attribute of <flexContainer> SDT devices (see in 6.2.2 the rules 1-7).

Table 5.8.2-1: Child resources of [flexNode] resource

| Child Resources of [flexNode] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| dmAreaNwkInfo_<i> | [dmAreaNwkInfo] | 0..n | See clause 5.8.10 |
| dmAgent | [dmAgent] | 0..1 | See clause 5.8.3 |
| dmDeviceInfo | [dmDeviceInfo] | 1 | See clause 5.8.4 |
| dmDataModelIO_<i> | [dmDataModelIO] | 0..N | See clause 5.8.5 |
| dmFirmware_<i> | [dmFirmware] | 1..N | See clause 5.8.6 |
| dmSoftware_<i> | [dmSoftware] | 0..N | See clause 5.8.7 |
| dmEventLog_<i> | [dmEventLog] | 0..N | See clause 5.8.8 |
| dmPackage_<i> | [dmPackage] | 0..N | See clause 5.8.9 |
| battery_<i> | [battery] | 0..N | See clause 5.3.10 |
| dmCapability_<i> | [dmCapability] | 0..N | See clause 5.8.12 |
| dmStorage_<i> | [dmStorage] | 0..N | See clause 5.8.13 |

NOTES:

- the notation ‘_<i>’ for child resources indicates that the resource name is the name of the child ModuleClass or SubDevice flexContainer, appended with an underscore ‘_’ and an incrementing index so that it is unique in the [flexNode] children (e.g. “dmFirmware_0”, “dmFirmware_1”, etc.). The index shall not have leading 0’s.

- the current list of modules for Device Management is not fixed and can evolve with new optional features.

### ----------------------- End of change 1 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

### 6.2.2 Resource mapping for Device model

When the AE exposes a controlling interface for a home domain device which is specified as an information model in clause 5.5, a specialization of the <flexContainer> resource shall be created as the mapping of the model following conversion rules:

- Rule 1-1: Each Device model defined in clause 5.5 shall be mapped to a specialization of <flexContainer>. The containerDefinition attribute shall be set according to 6.4.2.

- Rule 1-2: Each entry in the 'Module' table shall be mapped to a child resource(s) which is mapped as a specialised <flexContainer> following the rule in clause 6.2.3.

- Rule 1-3: The specialized <flexContainer> resource of the Device model shall contain an attribute nodeLink (as defined in TS-0001[3] and in TS-0004[4]). The value of nodeLink shall be set to the resource identifier of a <node> resource described in Rule 1-5 below.

- Rule 1-4: XSD file for each Device model shall be named according to 6.5.2.

- Rule 1-5:  A <node> resource shall be created on the same hosting CSE as the <flexContainer> representing this Device model. If the <node> resource does not contain a [flexNode] child resource (see Rule 1.7), then it contains all the management information as specialized <mgmtObj> resources (e.g. [firmware]) about the Device model instance for device management purposes.

- Void.

- Rule 1-7: The <node> resource targeted by the nodeLink attribute may contain a [flexNode] specialization of a <flexContainer> resource. This [flexNode] resource contains all the Device Management information as specialized <flexContainer> resources defined in 5.8 (e.g. [dmFirmware]) about the device model instance for Device Management purposes.

- Void.

- Rule 1-9: Each entry in the 'SubDevice' table shall be mapped to a child resource(s) which is mapped as a specialised <flexContainer> following the rule in clause 6.2.7.

### ----------------------- End of change 2 -------------------------------------------

### ----------------------- Start of change 2 -------------------------------------------

### 6.2.5 Resource mapping for Property

When the Device model (in clause 5.5) or the ModuleClass model (in clause 5.3) is mapped to the <flexContainer> resource, and if the device supports a Property, the following rules shall be applied:

- Rule 4-1: Each entry of ‘Property’ table in ModuleClass model, shall be mapped to the [customAttribute] of <flexContainer> resource which is mapped from associated ModuleClass model, with its Property name with prefix 'prop'.

- Rule 4-2: Each ‘Property’ of a Device model is either mapped to a specialized [objectAttribute] of a [deviceInfo] <mgmtObj> resourcewhen the <node> resource targeted by the nodeLink attribute does not contain a [flexNode] child, or to a [customAttribute] of a [dmDeviceInfo] <flexContainer> resource, child of this [flexNode] resource if present.

- Rule 4-3: Each entry of ‘Property’ table in SubDevice model, shall be mapped to the [customAttribute] of <flexContainer> resource which is mapped from associated SubDevice model, with its Property name with prefix 'prop'.

### ----------------------- End of change 3 -------------------------------------------

### ----------------------- Start of change 4 -------------------------------------------

  - Example for Device model ‘deviceAirConditioner'

The present clause explains the creation process for the device typed 'deviceAirConditioner' (see clause 5.5.1.1 for device model definition of ‘deviceAirConditioner').

Using the definition, 'deviceAirConditioner' model is mapped to [deviceAirConditioner] resource which is a specialization of <flexContainer> resource (See Figure A.2-1).

Figure A.2-1: Structure of [deviceAirConditioner] resource

The AE creates the [deviceAirConditioner] specialization of <flexContainer> resource for the Device model [deviceAirConditioner] resource.

The [deviceAirConditioner] resource contains the child resource specified in Table A.2-2.

Table A.2-2: Child resources of [deviceAirConditioner] resource

| Child Resources of [deviceAirConditioner] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <flexContainer> as defined in the specialization [binarySwitch] | 0..1 | This resource is used to map 'binarySwith' ModuleClass defined in clause .5.3.1.12. |
| [variable] | <flexContainer> as defined in the specialization [runState] | 0..1 | This resource is used to map 'runState' ModuleClass defined in clause 5.3.1.75. |
| [variable] | <flexContainer> as defined in the specialization [airConJobMode] | 0..1 | This resource is used to map ‘airConJobMode’ ModuleClass defined in clause. Editor’s Note: airConJobMode is not a moduleclass. It is an instance of that. It is needed to fix. |
| [variable] | <flexContainer> as defined in the specialization [airConOperationMode] | 0..1 | This resource is used to map ‘airConOperationMode’ ModuleClass defined in clause 5.3.1.57. |
| [variable] | <flexContainer> as defined in the specialization [airCleanOperationMode] | 0..1 | This resource is used to map ‘airCleanOperationMode’ ModuleClass defined in clause 5.3.1.57. |
| [variable] | <flexContainer> as defined in the specialization [temperature] | 0..1 | This resource is used to map ‘temperature’ ModuleClass defined in clause 5.3.1.87. |
| [variable] | <flexContainer> as defined in the specialization [timer] | 0..1 | This resource is used to map 'timer' ModuleClass defined in clause 5.3.1.90. |
| [variable] | <flexContainer> as defined in the specialization [sleepTimer] | 0..1 | This resource is used to map 'sleepTimer' ModuleClass defined in clause 5.3.1.90. |
| [variable] | <flexContainer> as defined in the specialization [turbo] | 0..1 | This resource is used to map 'turbo' ModuleClass defined in clause 5.3.1.91. |
| [variable] | <flexContainer> as defined in the specialization [airFlow] | 0..1 | This resource is used to map 'airFlow' ModuleClass defined in clause 5.3.1.4. |
| [variable] | <flexContainer> as defined in the specialization [powerSave] | 0..1 | This resource is used to map 'powerSave' ModuleClass defined in clause 5.3.1.66. |
| [variable] | <flexContainer> as defined in the specialization [airQualitySensor] | 0..1 | This resource is used to map 'airQualitySensor' ModuleClass defined in clause 5.3.1.6. |
| [variable] | <flexContainer> as defined in the specialization [filterInfo] | 0..1 | This resource is used to map 'filterInfo' ModuleClass defined in clause 5.3.1.35. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in oneM2M TS-0001 [i.3] |

Editor’s Note: Above table should be updated compliant to present structure of deviceAirConditioner.

The [deviceAirConditioner] resource contains the attributes specified in Table A.2-3.

Table A.2-3: Attributes of [deviceAirConditioner] resource

| Attributes of [deviceAirConditioner] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [[i.3]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| labels | 0..1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| stateTag | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| creator | 0..1 | RW | See clause 9.6.35 in oneM2M TS-0001 [i.3] |
| containerDefinition | 1 | WO | The value is "org.onem2m.home.device.airconditioner" |
| ontologyRef | 0..1 | RW | See clause 9.6.35 in oneM2M TS-0001 [i.3]. |
| contentSize | 1 | RO | See clause 9.6.35 in oneM2M TS-0001 [i.3]. |
| nodeLink | 1 | RO | nodeLink attribute links to a <node> resource that is hosted on the same hosting CSE of the <flexContainer>. See clause 6.2.2 and 6.2.5 for more details. |

### ----------------------- End of change 4 -------------------------------------------

### ----------------------- Start of change 5 -------------------------------------------

  - Example of ModuleClass 'binarySwitch'

The [binarySwitch] resource is used to share information regarding the modeled binary switch module as a ModuleClass. The [binarySwitch] resource is a specialization of the <flexContainer> resource.

Figure A.3-1: Structure of [binarySwitch] resource

The [binarySwitch] resource contains the child resource specified in Table A.3-2.

Table A.3-2: Child resources of [binarySwitch] resource

| Child Resources of [binarySwitch] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <flexContainer> as defined in the specialization [toggle] | 0..1 | This resource is used to map 'toggle' Action defined in Clause 5.3.1.12. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in oneM2M TS-0001 [i.3] |

The [binarySwitch] resource contains the attributes specified in Table A.3-3.

Table A.3-3: Attributes of [binarySwitch] resource

| Attributes of [binarySwitch] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| expirationTime | 1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| labels | 0..1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| stateTag | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3] |
| creator | 0..1 | RW | See clause 9.6.35 in oneM2M TS-0001 [i.3] |
| containerDefinition | 1 | WO | The value is "org.onem2m.home.moduleclass.binaryswitch" |
| ontologyRef | 0..1 | RW | See clause 9.6.35 in oneM2M TS-0001 [i.3] |
| contentSize | 1 | RO | See clause 9.6.35 in oneM2M TS-0001 [i.3] |
| nodeLink | 0..1 | RW | Not applicable to a ModuleClass specialization. This attribute is not present in an instantiation of this resource. |
| dataGenerationTime | 0..1 | RO | See clause 6.2.3 |
| powerState | 1 | RW | See clause 5.3.1.12 |

- History

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 2020-10-19 | Initial draft of the TR. |
| V0.1.0 | 2021-02-04 | Updating the study with according to the latest version of TS-0023 v4.7.0 Including CR RDM-2021-0012R01 |